3GPP TSG RAN WG1 #106-e		R1-2107969

e-Meeting, August 16th – 20th, 2021

Title:	[Draft] Reply LS on Inter-donor migration

Response to:	R3-212981

Release:	Rel-17

Work Item:	NR_IAB_enh-Core

Source:	vivo [RAN1]

To:	RAN3

Cc:

# Contact Person:

# Name:	                 Huan Wang

# E-mail Address:	  wanghuan [at] vivo [dot] com

# 1. Overall Description:

RAN1 thanks RAN3 for the LS R3-212981 on Inter-donor migration. RAN3 provides the following two implementation alternatives, which involves creation of two logical IAB-DUs at the boundary IAB node, are to be further discussed in the scope of Full Migration:

- Alt1: the two logical DUs use separate physical cell resources

- Alt2: the two logical DUs use the same physical cell resources

Questions from RAN3

RAN3 would like to ask RAN1, RAN2, and RAN4 to provide feedback, e.g, any technical issue for the above Alt1 and Alt2?

For Alt2, RAN3 also has some concrete questions w.r.t., PCI/NCGI, i.e.,

- Q1: Whether the current specification enables a RRC CONNECTED UE remains connected, while observing the change of NCGI, and no change to the PCI?

- Q2: is it possible to use same PCI for cell1 and cell2, and support the HO from cell1 to cell2 without new impact to the UE (e.g. a legacy UE)?

- Q3: when cell1 and cell2 use different PCI/NCGI, is it possible to use one set of shared resource, without new impact to the UE?

Answer from RAN1 perspective

For Alt1, RAN1 understands that two logical DUs are created in different carriers of the boundary IAB node. RAN1 does not observe any technical issue for Alt1.

For Q2, RAN1 assumes to use different PCI for cell 1 and cell 2 during design of RAN 1 specification, thus it is not possible to use the same PCI for cell 1 and cell 2.

For Q3, RAN1 understands cell 1 and cell 2 can use the same carrier while using different PCIs. When gNB schedules transmissions in cell 1 and cell 2, orthogonal resources can be assigned in cell 1 and cell 2 by gNB implementation.

# 2. Actions:

# To RAN3

# ACTION: RAN1 respectfully requests RAN3 to take the above into account.

# 3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #106bis-e	October 11th – 19th, 2021	e-meeting

TSG RAN WG1 Meeting #107-e	November 11th – 19th, 2021	e-meeting

4. Reference:

[1] R2-2107862, “Discussion on Migration and Service Interruption”, RAN1#115e, 3GPP.