3GPP TSG RAN WG1 #106e	R1-2108012

e-Meeting, August 16th – 27th, 2021

Agenda item:	8.3.1.2

Source: 	ITRI

Title: 	Views for Increasing Granularity of Subband CQI

Document for:	Discussion/Decision

# 1 Introduction

RAN guidance on CSI feedback enhancement is provided as the following recommendations [1]:

Proposal: RAN confirms the following as a guidance to RAN1 for CSI enhancement in Enhanced URLLC/IIoT WI: RAN1 to further investigate the following for CSI enhancements for IIoT/URLLC: - Increasing the number of bits used for the reported subband CQI (3-bits differential subband CQI or 4-bits CQI) - Reporting of delta-MCS: - Report consists of delta-MCS for a TB received with MCS index IMCS: - delta-MCS is calculated from the difference between IMCS_tgt and IMCS, where IMCS_tgt is the largest MCS index such that the estimated BLER for a TB received with this MCS index would be smaller than or equal to a BLER target, and IMCS is the MCS index of the received TB.

In this contribution, we would like to share views for increasing granularity of subband CQI based on our simulation results. Different performance metrics are revealed with 2-bit, 3-bit differential subband CQI, and 4-bit subband CQI.

# 2 Increasing Granularity of Subband CQI

As defined in TS 38.214, UE report the subband differential CQI values to let gNB know the difference/offset between wideband CQI and each subband CQI. In order not to make subband CQI feedback a large overhead, it spends only two bits for sub-band differential CQI value per subband. 2-bit differential subband CQI only covers four values for offset level {≥2, 1, 0, ≤-1}. However, insufficient subband information may cause MCS prediction error and fail to achieve the requirements, especially in strict reliability requirements of URLLC use cases. Increasing granularity of subband CQI seems a potential solution to overcome the issues. Example of 3-bit differential subband CQI and 4-bit subband CQI is shown Table 1 and Table 2. Simulation assumptions are provided in Appendix A. In this contribution, the InF-DH scenario with DL SPS is evaluated.

- Case 1: Report 2-bit differential subband CQI (as defined in TS 38.214 Table 5.2.2.1-1)

- Case 2: Report 3-bit differential subband CQI (the offset level is between 4 and -3, shown as Table 1)

- Case 3: Report 4-bit CQI (as defined in TS 38.214 Table 5.2.2.1-4)

# 3 Simulation Results

In addition to the performance metrics agreed in RAN1#102-e, Appendix B, communication service availability (CSA) [2] is evaluated as well. CSA is one of the important performance metrics in 5G-ACIA (5G Alliance for Connected Industries and Automation) evaluation. The survival time and packet arrival interval are 2ms in this evaluation. Consecutive errors are not allowed based on the configuration.

The simulation result is shown in Table 3. For the percentage of UEs satisfying reliability and latency requirements, 3-bit differential subband CQI shows a 22% percentage increase of UEs and 4-bit subband CQI shows a 27% percentage increase of UEs. Performance of 4-bit CQI is slightly improved, compared to that of 3-bit differential subband CQI. For the percentage of UEs satisfying 99.9999% CSA and latency requirements, increasing granularity of subband CQI may not lead to obvious improvement. Chances of consecutive errors may be minor due to very low target BLER. 2-bits differential subband CQI already shows a good performance on CSA. It means that there is not much beneficial to increase granularity of subband CQI, such as 3-bit differential subband CQI. Similarly, 4-bit CQI obtain a minor improvement.

Observation: Differential subband CQI with increasing granularity improves the percentage of UEs satisfying requirements while maintaining a similar resource utilization.

Simulation results demonstrate that 3-bit differential subband CQI is sufficient to enhance the system performance, which performance is close to that of 4-bit subband CQI. Besides of the performance, more bits cause larger payload and increase resource utilization. We also think that the impact on specifications and the implementation complexity is quite low to introduce a new mapping table with 3-bit differential subband CQI. However, the detail of the mapping table may be further designed.

Proposal: A new mapping table with 3-bit differential subband CQI is sufficient to enhance the system performance for URLLC in Rel-17.

| Table 1: Example of 3-bit differential subband CQI. Sub-band differential CQI value; Offset level. 0; 0. 1; 1. 2; 2. 3; 3. 4; ≥4. 5; -1. 6; -2. 7; ≤-3. | Table 2: 4-bit CQI (TS 38.214 Table 5.2.2.1-4). CQI index; modulation; Code rate x 1024; efficiency. 0; out of range. 1; QPSK; 30; 0.0586. 2; QPSK; 50; 0.0977. 3; QPSK; 78; 0.1523. 4; QPSK; 120; 0.2344. 5; QPSK; 193; 0.377. 6; QPSK; 308; 0.6016. 7; QPSK; 449; 0.877. 8; QPSK; 602; 1.1758. 9; 16QAM; 378; 1.4766. 10; 16QAM; 490; 1.9141. 11; 16QAM; 616; 2.4063. 12; 64QAM; 466; 2.7305. 13; 64QAM; 567; 3.3223. 14; 64QAM; 666; 3.9023. 15; 64QAM; 772; 4.5234. |
| --- | --- |

Table 3: Simulation results (percentage of UEs satisfying requirements and RU).

|  | Resource Utilization | Percentage of UEs satisfying reliability and latency requirements | Percentage of UEs satisfying 99.9999% CSA and latency requirements |
| --- | --- | --- | --- |
| 2-bit differential subband CQI | 6.32% | 63.33% | 90.56% |
| 3-bit differential subband CQI | 6.94% | 85.00% | 95.00% |
| 4-bit CQI | 7.06% | 90.56% | 96.67% |

# 4 Design of 3-bit differential subband CQI offset mapping table

- In this section, we provide the performance with different 3-bit differential subband CQI offset mapping tables. Table 4 shows another design of differential CQI mapping. Performance is compared in Table 5 with 3-bit differential subband CQI offset mapping table, Table 1 and Table 5. The performance difference is minor. We suggest that the new mapping table with 3-bit differential subband CQI can be determined in the following meetings.

Table 4: Example of 3-bit differential subband CQI.

| Sub-band differential CQI value | Offset level |
| --- | --- |
| 0 | 0 |
| 1 | 1 |
| 2 | 2 |
| 3 | ≥3 |
| 4 | -1 |
| 5 | -2 |
| 6 | -3 |
| 7 | ≤-4 |

Table 5: Simulation results (percentage of UEs satisfying requirements and RU).

|  | Resource Utilization | Percentage of UEs satisfying reliability and latency requirements | Percentage of UEs satisfying 99.9999% CSA and latency requirements |
| --- | --- | --- | --- |
| 3-bit differential subband CQI (Table 1) | 6.94% | 85.00% | 95.00% |
| 3-bit differential subband CQI (Table 4) | 6.95% | 87.22% | 96.11% |

# 5 Conclusions

In this contribution, we provide simulation results for CSI feedback enhancement with increasing the number of bits used for the reported subband CQI and have the following observation and proposal:

Observation: Differential subband CQI with increasing granularity improves the percentage of UEs satisfying requirements while maintaining a similar resource utilization.

Proposal: A new mapping table with 3-bit differential subband CQI is sufficient to enhance the system performance for URLLC in Rel-17.

# 6 References

RP-211297

RP-201279

- Appendix A: Simulation assumptions

| Parameters | Value |
| --- | --- |
| Factory hall size | 120x60 m |
| Room height | 10 m |
| Inter-BS/TRP distance | 20 m |
| BS/ antenna height | 8m for InF-DH |
| Layout – BS/TRP deployment | 18 BS |
| Channel model | InF-DH |
| Carrier frequency and simulation bandwidth | TDD 4 GHz: 40 MHz |
| SCS | 30 kHz |
| TDD DL-UL configuration | {S}, S={D6,G2, U6} |
| Number of UEs per BS | 10 |
| Channel model | InF-DH in TR 38.901 |
| Traffic model | Periodic deterministic traffic model with arrival interval 2ms (32 bytes) |
| Latency requirement | 1 ms |
| Reliability requirement | 99.9999% |
| CSA requirement | 99.9999% |
| UE speed | 3 km/h |
| BS Tx Power | 27 [dBm] |
| BS Antenna Configuration | 4 Tx antenna ports (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) dH = dV = 0.5 λ |
| UE Antenna Configurationl | 4 Rx antenna ports (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) dH = dV = 0.5 λ |
| Receiver Noise Figure | 5dB for BS and 9dB for UE |
| Channel estimation | Realistic |
| Receiver | MMSE-IRC |
| Periodicity of P-CSI reporting | 5 ms |
| Delay of P-CSI reporting | 2 ms |
| Layer | 1 |

# Appendix B: Baseline SLS assumption agreed in RAN1#102-e

Agreements:

- Consider Table 1 as baseline assumption for system level simulation for evaluating CSI enhancement schemes

  - The uses cases in Table 1 is for simulation purposes and it does not preclude a CSI enhancement scheme which is beneficial for the other URLLC use cases

- No baseline assumption is used for link level simulation

  - Companies are encouraged to use one of LLS assumption tables in Section A.3 in TR38.824 for any link level simulation

Table 1. Baseline SLS assumption for CSI enhancement schemes in URLLC/IIoT

| Parameters | Values |
| --- | --- |
| Performance metric | Option-1 (section 5.1 of TR 38.824) Additional metrics (it is up to company to bring results with additional metric): - MCS prediction error (e.g., difference of a scheduled MCS and an ideal MCS) - DL/UL signaling overhead - CCDF of latency samples from all UEs - BLER of 1st transmission - Resource utilization - Spectral efficiency |
| Use cases | Following two use cases can be considered for new triggering method and new reporting. Companies are encouraged to evaluate the following cases in descending priority: - Rel-15 enabled use case (e.g. AR/VR) in TR 38.824 - Reliability: 99.999 - Latency: 4ms (200bytes) - Traffic mode: FTP model 3 (100p/s) - Factory automation in TR 38.824 - Reliability: 99.9999 - Latency: 1ms (32bytes) - Traffic mode: Periodic deterministic traffic model with arrival interval 2ms - Rel-15 enabled use case (e.g. AR/VR) in TR 38.824 - Reliability: 99.999 - Latency: 1ms (32bytes) - Traffic mode: FTP model 3 (100p/s) - Assumptions for eMBB and URLLC UEs sharing the same carrier is used (as in A2.5 of TR 38.824) |
| Simulation assumptions | Following simulation assumption is used based on the use case selected: - Rel-15 enabled use case with UMa (Table A.2.4-1 in TR 38.824) - Factory automation at 4GHz (Table A.2.2-1 in TR38.824) with following update: - Channel model is replaced with InF (InF-DH) in TR 38.901 - Companies can bring results with other InF scenarios additionally - Layout is replaced with BS deployment in Table 7.8-7 in TR 38.901 |
| Transmission scheme | Multiple antenna ports Tx scheme - Companies report the details of Tx scheme used |