3GPP TSG RAN WG1 Meeting #106-e		R1- 2108058

e-Meeting, Aug 16th – 27th, 2021

Agenda item:       8.1.4

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Enhancement on CSI measurement and reporting

Document for:     Discussion and Decision

# 1	Introduction

In this contribution we discuss the remaining aspects of the design for reciprocity-aided FDD CSI enhancement, and Multi-TRP CSI enhancement identified by the agreements in RAN1#105-e [2].

# 2	FDD CSI reporting with partial reciprocity

## 2.1	W1: remaining issues

In RAN1#105bis-e, the following candidate value set was agreed for the port selection parameter K1

Agreement

At least for rank 1, candidate values of K1  for port selection matrix W1 in NP*K1 are {2, 4, 8, 12, 16, 24, 32}.

- Note: for polarization-common based free-selection, it means to select the same L=K1/2 ports out of P/2 ports for both polarizations

It was also agreed to limit total parameter combinations to a number comparable to Rel-16 eType II:

Agreement

Further reduction for possible parameter combinations among codebook parameters of Rel-17 port selection codebook, e.g. {K1, Mv, Beta}, will be discussed jointly once candidate values are determined

- based on trade-off among UPT performance, feedback overhead, and complexity

- based on all supported ranks

- Limit total number of parameter combinations comparable to Rel-16 eType II

- Exact parameters (e.g. with 2 or 3 parameters) within each combination are FFS

- Other parameterizations of codebook parameter (e.g. alpha with K1= Alpha*# of CSI-RS ports and Alpha <=1) are not excluded

Besides, in RAN1#104-e it was also agreed that the identity matrix is a special configuration for W1, i.e., the parameter value K1=P should be configurable. One possible way to reduce the number of parameter combinations is to parametrise the value of K1 as a fraction of the number of ports, P, for example by using the formula: K1=αP with α≤1 as suggested in the agreement above.

For small number of ports, e.g., P≤12, only one value of α may be needed, with α=1, whereas for P>12, two values of α∈1,12 are useful to control the feedback overhead. The restriction in supporting α=1/2 only for P>12 avoids introducing the additional value of K1=6 for P=12, which was not agreed in the agreement of the candidate values above. Besides, for small number of ports selecting half the ports results in significant performance degradation.

- Regarding the port selection parameter K1, consider the parameter formula: K1=αP, with α=1 for P≤12 and α∈1,12 for P>12.

In Figure 1 and Figure 2, performance is evaluated for maximum rank 1 (MR1) and 2 (MR2), respectively, for different number of ports, P={8,16,32}, and parameter combinations, (α,Mν,β), where the number of selected ports K1 is parametrised as K1=αP, with α=1 for P≤12, and α=12,1, for P>12. The number of components of Wf is Mν∈{1,2}. The measurement window size for Wf is configured to N=Mν.

In general, we observe higher throughput gain relative to Rel-16 PS at lower overhead and higher gain for MR2 compared to MR1. For MR1, the relative gain for the best parameter combinations is about 50%, 11% and 10% for low, medium, and high overhead, respectively. For MR2, the relative gain stands at about 80%, 42%, 32% for low, medium, and high overhead, respectively.

Figure 1. Performance for maximum rank 1 (MR1), for CSI-RS ports P={8,16,32} and different parameter combinations: (α,Mν,β), with K1=αP, α∈12,1, Mν∈{1,2} and β∈14,12,34,1.

Figure 2. Performance for maximum rank 2 (MR2), for CSI-RS ports P={8,16,32} and different parameter combinations: (α,Mν,β), with K1=αP, α∈12,1, Mν∈{1,2} and β∈14,12,34,1.

For Rel-17 PS enhancement, it is also desirable to support rank extension to higher ranks 3 and 4, similarly to Rel-16 PS, by ensuring that the feedback overhead is comparable to that of rank 2.

- Support reporting of rank 3 and 4 by ensuring that feedback overhead is comparable to that of rank 2.

## 2.2	W2 quantisation and reporting

In RAN1#105-e, the design of the strongest coefficient indicator (SCI) was discussed and a number of alternatives were identified

Proposal

Study following alternatives for reporting the strongest coefficient indication (SCI) for Rel-17 port selection codebook in W2

- Alt 0 : Reporting of the position, [il*, fl*], of the strongest coefficient of layer l using log2K0 bits, where K0=βK1Mν

- Alt 1-1: Reporting of the position, [il*, fl*], of the strongest coefficient of layer l, using log2K1Mν or log2K1+log2Mν bits

- Alt 1-2: Reporting of the position, [il*, fl*], of the strongest coefficient of layer l, using log2K1Mν or log2K1+log2Mν bits, and shifting of the strongest coefficient to position fl*=0

- Alt 2: shifting the strongest coefficient to fl* = 0, and using log2N bits to indicate the shift quantity for l-th layer. The strongest coefficient is indicated by il*, using ceil log2K1 for l-th layer.

- Alt 3: SCI is not needed so that the SCI in R16 codebook is replaced with a strongest polarization indicator (1 bit)

- Considering further impact of UCI design for above Alternatives

We recall that in Rel-16 the SCI depends only on the beam/port index of the strongest coefficient, il*, because the strongest coefficient is shifted to position fl*=0, i.e., it is always found on the first column of W2 and the components of Wf, which is layer-specific in Rel-16, are shifted accordingly with a modulo-N3 operation.

In Rel-17, however, because of the window restriction of size N configured by the gNB, the SCI needs to be modified by adding information related to the FD component of the strongest coefficient, i.e., either n3,l(fl*) or fl*. The need for this modification can be illustrated, for example, in the case N=Mν=2, for which Wf=n30,n31=[0, 1] is layer-common and not reported.

Figure 3. Illustration of an example window configuration N=Mν=2. The strongest coefficient for layer 1 is found in position [i1*,f1*=1] and in position [i2*,f2*=0], for layer 2.

Figure 3 illustrates an example for this window configuration, in which the strongest coefficient of layer 1 and 2 are in two different components. Because Wf is layer common and not reported, the first component, n3(0)=0, may not correspond to the component of the strongest coefficient for some layer. Therefore, both coordinates [il*, fl*] of the strongest coefficient need to be reported for each layer l to be able to locate the strongest coefficient. Note that in this case, when N=Mν, the FD component index of the strongest coefficient, n3(fl*), and the column index of the strongest coefficient in W2, fl*, take the same values, i.e.,  n3(fl*)=fl*, hence fl* can be reported separately with log2Mν=log2N bits, or jointly with il*, by using log2K1Mν bits.

- In Rel-17, because of the window/set restriction of size N configured by the gNB, the SCI needs to be modified by adding information related to the FD component of the strongest coefficient for each layer.

In the current agreement regarding the relationship between N and Mν, there is also the possibility that N>Mν, in which case Wf needs reporting and it can be layer-common (Alt 2-1) or layer-specific (Alt 2-2).

Alt 1-2 and 2 define the FD component of the strongest coefficient as reference for reporting Wf, similarly to Rel-16, whereas in the other three alternatives no remapping operation is used, so one can assume that all Mν components of Wf need reporting.

- Regarding the SCI definition, if not shift is applied (Alt 1-1), one can assume that all Mν components of Wf need reporting, using log2NMν bits.

To minimise overhead and following Rel-16 design, we assume that in the case N>Mν, Wf is reported by a combinatorial coefficient using log2N-1Mν-1 bits, such that component 0 is always included in Wf and not reported. Because only Mν-1 FD components are reported, a reference component needs to be specified, which determines the modulo-N shift operation applied to the components of Wf before reporting.

In Rel-16, where Wf is layer-specific, the reference component is that of the strongest coefficient for a layer l, such that the component indices of Wf are remapped with respect to n3,l(fl*) as n3,lf=n3,lf-n3,lfl*mod N3. This choice of shift for Wf in Rel-16 allows to reduce overhead in the reporting of Wf as well as in the reporting of the SCI because only the first coordinate of the pair [il*, fl*] identifying the position of the strongest coefficient for layer l needs reporting. For this latter overhead saving to be possible in Rel-16 a second shift needs to be applied to the column indices of W2, which are remapped with respect to fl* as f=f-fl* mod Mν, such that the columns of W2 correspond to those of Wf after the shift.

In Rel-17, however, both coordinates of the SCI need reporting in all cases, including the case Wf is reported (for N>Mν) and the strongest coefficient is shifted to fl*=0. Note that if remapping is applied to f, the reported SCI component fl* is the one before the remapping. Hence, in case of reporting of Wf, a different shift from Rel-16 may be specified for the components of Wf, and the shift to the columns of W2 may not be applied.

- The additional indication of fl* in the SCI for each layer is needed both in the case Wf is not reported (N=Mν) and in the case Wf is reported (N>Mν).

In practice, there are two possible components that can be taken as reference for Wf: the component of the strongest coefficient as in Rel-16, or the ‘first’ selected component, i.e., the component with the lowest index value in the window: {0,1…,N-1}. Depending on the choice of layer-common or -specific Wf, the following mapping choices are available for Wf

- Layer-common Wf=[n30,…,n3(Mν-1)]

  - The reference is the component of the strongest coefficient for layer 1, n3(f1*). The FD components are remapped with respect to n3f1* as n3f=n3f-n3f1*mod N, such that n3f1*=0 after remapping.

  - The reference is the ‘first’ component, n3(0). The FD components are remapped with respect to n30 as n3f=n3f-n30, such that n30=0 after remapping.

- Layer-specific Wf=[n3,l0,…,n3,l(Mν-1)], for l=1,…,ν

  - The reference is the component of the strongest coefficient for layer l, n3,l(fl*). The FD components are remapped with respect to n3,lf1* as n3,lf=n3,lf-n3,lfl*mod N, such that n3,lfl*=0 after remapping.

  - The reference is the ‘first’ component, n3,l(0). The FD components are remapped with respect to n3,l0 as n3,lf=n3,lf-n3,l0, such that n3,l0=0 after remapping.

If no shift is specified for Wf a UE may understand that Wf=[0,n31,…, n3(Mν-1)], in the layer-common case, for example, i.e., it may assume that the first component, n3(0)=0 is fixed and only select and report Mν-1 nonzero components in the window of size N.

Regarding the shift applied to the columns of W2, there are in practice two possible options: no shift or shifting the index f with respect to fl* as f=f-fl* mod Mν , such that the index of the strongest coefficient is fl*=0 after remapping.

For all the above alternatives of index mapping of Wf and W2, reporting of the pair il*, fl* as SCI for layer l is enough to locate the strongest coefficient of W2 and to ensure correct reconstruction of Wf. Reporting the values il*, n3fl*, according to Alt 2 proposed in RAN1#105-e, also works but requires more feedback bits if N>Mν because n3fl*∈{0,1,…,N-1}, whereas fl*∈{0,1,…,Mν-1}.

To show that reporting fl* allows correct reconstruction, let us consider an example for the cases 1.a and 1.b above, with N=4>Mν=2 and with layer-common Wf=n30,n31=[1, 2]. Let us assume that, for layer 1, W2=acbd and the strongest coefficient is c in position i1*,f1*=[0,1]; for layer 2, W2=egfh and the strongest coefficient is f in position i2*,f2*=[1,0]. This example is illustrated in Figure 4. On the left-hand side the UE and gNB operations are shown in the case 1.a, wherein the strongest coefficient for each layer is shifted to FD component 0 and the shift applied to Wf follows that of layer 1. Case 1.b is shown on the right-hand side, wherein the strongest coefficient is not shifted and the shift applied to Wf is with respect to the selected component of lowest index.

In case 1.a, the components of Wf are remapped with respect to n3f1*=2 as n3f=n3f-2mod 4, f=0,1, such that Wf=[0,3] is the reported FD basis set. The index f is remapped with respect to f1*=1 as f=f-1 mod 2, for layer 1, and with respect to f2*=0, i.e., no shift is applied, for layer 2. The reported combination coefficient matrices are W2=cadb, egfh. At the gNB, using Wf=[0,3] in the precoder reconstruction would not be correct because the selected components to the left of n3(f1*), i.e., n30,…,n3(f1*-1) have been remapped inside the window by the modulo N, as n3Mν-f1*,…,n3(Mν-1). Hence, the gNB needs to remap the Mν components of Wf with respect to n3Mν-f1*, if f1*>0, as n3f=n3f-n3Mν-f1*mod N, such that n3Mν-f1*=0 after demapping. If f1*=0, no remapping is needed because no component of Wf was remapped by the modulo operation. An equivalent way to describe the demapping at the gNB is by remapping the components of Wf with respect to n3Mν-f1*mod Mν, as n3f=n3f-n3Mν-f1*mod Mνmod N, such that n3Mν-f1*mod Mν=0, after demapping. The gNB also de-maps f as f=f-(Mν-fl*) mod Mν=f+fl* mod Mν, for l=1,2 such that Wf=[0,1] and W2=acbd, egfh are used in the precoder reconstruction.

In case 1.b, the components of Wf are remapped with respect to n30=1 as n3f=n3f-1, f=0,1, such that Wf=[0,1] is the reported FD basis set. No mapping is applied to the index f, hence the combination coefficient matrices are reported without shifting the strongest coefficient to FD component 0, as W2=acbd, egfh. At the gNB, no de-mapping is required and the reported quantities for Wf and W2 are used in the precoder reconstruction.

This example illustrates that, no matter what solution is adopted for the representation of Wf and W2, the SCI can always be reported as the index pair il*,fl* for l=1,…,ν.

Figure 4. Example of operations for two possible mappings of layer-common Wf and W2: 1.a) the strongest coefficient is shifted to component 0 (left), or 1.b) the strongest coefficient is not shifted (right). In both cases the SCI can be reported as il*,fl*.

Table 1 and Table 2 summarise the mapping options discussed above for the components of Wf and the column index of W2 in case of layer-common and layer-specific Wf. Note that the mapping and de-mapping of Wf is only applicable for N>Mν, whereas for N=Mν, Wf is layer-common and fixed by configuration.

Table 1. Summary of UE’s mapping options for the components of Wf and column index of W2. In all cases the SCI can be reported as il*,fl* for layer l

|  |  | UE |  |  |
| --- | --- | --- | --- | --- |
|  | N>Mν (Wf reported) |  | N≥Mν |  |
|  | choice of shift for Wf and W2 | map of n3f/n3,l(f) (Wf) | map of f (W2) | SCI layer l |
| layer-common Wf | Strongest coefficient is shifted to fl*=0 | n3f=n3f-n3f1*mod N | f=f-fl* mod Mν | il*,fl* |
|  | Strongest coefficient is not shifted | n3f=n3f-n30 | - | il*,fl* |
| layer-specific Wf | Strongest coefficient is shifted to fl*=0 | n3,lf=n3,lf-n3,lfl*mod N | f=f-fl* mod Mν | il*,fl* |
|  | Strongest coefficient is not shifted | n3,lf=n3,lf-n3,l0 | - | il*,fl* |

Table 2. Summary of gNB’s de-mapping options for the components of Wf and column index of W2. In all cases the SCI can be reported as il*,fl* for layer l

|  |  | gNB |  |
| --- | --- | --- | --- |
|  | N>Mν (Wf reported) |  | N≥Mν |
|  | choice of shift for Wf and W2 | De-map of n3f/n3,l(f) (Wf) | De-map of f (W2) |
| layer-common Wf | Strongest coefficient is shifted to fl*=0 | n3f=n3f-n3Mν-f1*mod N,f1*>0n3f,f1*=0 | f=f+fl* mod Mν |
|  | Strongest coefficient is not shifted | - | - |
| layer-specific Wf | Strongest coefficient is shifted to fl*=0 | n3,lf=n3,lf-n3,lMν-fl*mod N,fl*>0n3,lf,fl*=0 | f=f+fl* mod Mν |
|  | Strongest coefficient is not shifted | - | - |

- For all possible index mapping of Wf and W2, reporting the position il*, fl* as SCI for layer l is enough to locate the strongest coefficient of W2 and to ensure correct reconstruction of Wf, in case Wf is reported.

Regarding the choice between separate reporting of the two SCI indices by using log2K1+log2Mν bits, or joint reporting, by using log2K1Mν bits, the latter is preferable because K1 may not be a power of 2, hence there is some overhead saving in encoding the two indices jointly, for example, as follows

| SCI=K1fl*+il*. | (1) |
| --- | --- |

There are two remaining alternative designs for the SCI that have been proposed: Alt 0 and Alt 3. In Alt 0, the SCI is calculated from the bitmap B=[bi,f], bi,f∈{0,1} and the coordinates of the strongest coefficient, il*,fl*, for example as follows:

| SCI=f=0fl*-1i=0K1bi,f+i=0il*bi,fl*-1 | (2) |
| --- | --- |

which is represented with log2(βK1Mν) bits. The main problem with this approach is that it introduces an unnecessary dependency between the SCI encoding and the bitmap, such that the strongest coefficient cannot be located by the gNB without decoding the bitmap first. However, it is desirable for the SCI to be mapped in the highest priority group together with W1 reporting to allow a precoder reconstruction in case one or both lower priority groups are omitted. On the other hand, the bitmap, which occupies larger payload size, can be assigned to lower priority groups together with the other W2 and Wf components. This principle was adopted in Rel-16 where the bitmap is split between Group 1 and 2 of UCI part 2, whereas the SCI is found in Group 0. Therefore, for Alt 0 to work in combination with the omission rules, the bitmap would need to be also mapped to Group 0, but in case Group 1 and 2 are omitted its only use in Group 0 is to allow the SCI decoding.

- Reporting the position il*, fl* as SCI for layer l using log2K0 bits (Alt 0) requires correct decoding of the bitmap before the strongest coefficient can be located. Therefore, the bitmap needs to be mapped in the same UCI Group 0 as the SCI and W1, whereas the nonzero coefficients are mapped in Groups 1 and 2.

One additional problem with making the encoding of the position il*,fl* dependent on the bitmap is that it is not possible to apply to the bitmap the same shift of the strongest coefficient such that fl*=0. Decoding fl* from (2) requires the bitmap columns to be in their original unshifted order, hence if the strongest coefficient is to be reported in position fl*=0, the shift of index f can only be applied to W2 but not the bitmap.

- Reporting the position il*, fl* as SCI for layer l using log2K0 bits (Alt 0) requires that the bitmap is reported without any shift that depends on fl*. If the strongest coefficient is shifted such that fl*=0, the shift can only be applied to the nonzero coefficients of W2 but not to the bitmap.

Finally, Alt 3 proposes not to report the SCI and instead report the differential amplitude and phase of the strongest coefficient for each layer, like for all the other nonzero coefficients. Besides, a new one-bit indicator is added to identify the polarisation corresponding to the weaker reported reference amplitude. This solution requires a redesign of the reporting mechanism and does not seem to provide any benefit. In fact, the solution requires 8 bits per layer for any value of K1 and Mν: 7 bits for the amplitude (3 bits) and phase (4 bits) quantisation of the strongest coefficient plus the additional 1 bit for the new polarisation indicator. Conversely, reporting the position of the strongest coefficient, il*,fl*, with log2K1Mν bits requires, at most, 6 bits for K1=32 and Mν=2.

- Replacing the SCI with a one-bit indicator of the stronger polarisation (Alt 3) requires larger overhead: 8 bits per layer for any value of K1 and Mν, compared to at most 6 bits for K1=32 and Mν=2 for the SCI reported with log2K1Mν bits.

- Regarding the bitmap and SCI reporting, support:

- Reporting of the position, [il*, fl*], of the strongest coefficient of layer l, for Mν>1, using log2K1Mν bits.

- Remapping the strongest coefficient to fl*=0 as in Rel-16.

In RAN1#105-e, a conclusion was reached to consider the possibility that the bitmap may not be reported in some special cases described in the following text.

For future RAN1 meeting:

Study whether/how the bitmap for indicating non-zero coefficients for W2 can be absent for CSI reporting

- FFS: applicable conditions of being absent, .e.g. Mv=1 and Beta =1 for rank 1 or higher ranks

- FFS: additional impact for reporting mechanism when/how the bitmap is absent

- Note: The principle of UE determining the real number of NZC (same as Rel-15 and Rel-16) is unchanged in Rel-17

based on trade-off among UPT performance, feedback overhead and complexity

The configurations for which the bitmap may not need reporting is when β=1, i.e., the maximum number of nonzero coefficients per layer is K0=K1Mν. This may only be possible for reported rank ν≤2, because for larger ranks some coefficients are not reported to reduce the feedback overhead, hence the location of the nonzero coefficients needs to be indicated. However, there are two complications: if the bitmap is absent in this case, a UE must report KNZ=K0 coefficients for rank 1 and KNZ=2K0 coefficients for rank 2, which implies that a UE cannot report fewer than the maximum number of nonzero coefficients and the KNZ indicator  in part 1 becomes redundant. Moreover, the amplitude quantisation levels do not comprise the zero value by construction because only the nonzero coefficients are reported. Hence if some of the coefficients are very close to zero, these would need to be reported anyway with a nonzero value, which may degrade performance and increase overhead unnecessarily.

- If the bitmap is absent from the CSI reporting for some configurations, e.g., Mν=1, β=1 and rank ν≤2, a UE cannot determine the real number of NZC, as the maximum number of NZC must be reported. In this case the indicator KNZ in part 1 CSI becomes redundant and coefficients that may be virtually zero are reported with a nonzero quantisation level.

- Regarding the reporting of the bitmap, if β=1 and a UE reports KNZ=K0 nonzero coefficients for ν=1 or KNZ=2K0 nonzero coefficients for ν=2, the bitmap can be assumed to be absent from the CSI report. In all other cases, support reporting of the bitmap.

Regarding the quantisation mechanism for the nonzero coefficients, in RAN1#105-e it was agreed to reuse the same mechanism as for Rel-16 eType II, as detailed in the following agreement:

Agreement

For the quantization of W2 coefficient, reusing following Rel-16 quantization mechanism for Rank1 at least:

- Two polarization-specific reference amplitudes:

  - for the polarization associated with the strongest coefficient, the reference amplitude is not reported

  - for the other polarization, reference amplitude is quantized to 4 bits

    - The alphabet is{1, 1/2)^(1/4), (1/4)^(1/4), (1/8)^(1/4), …, (1/2^14)^(1/4), [Reserved]} (-1.5dB step size)

- For coefficients other than the strongest coefficient

  - differential amplitude is calculated relative to the associated polarization-specific reference amplitude and quantized to 3 bits

    - The alphabet is {1, 1/sqrt(2), 1/2, 1/(2*sqrt(2)), 1/4, 1/(4*sqrt(2)), 1/8, 1/(8*sqrt(2))} (-3dB step size)

  - phase is quantized to 16PSK

- For the reserved state for reference amplitude, down-select one Alt

  - Alt 1: it is kept to be reserved

  - Alt 2: it is replaced as (1/2)^(15/4)

  - Alt 3: it is replaced as (1/2)^(3/8)

Note: whether/how SCI is supported for R17 codebook will be discussed separately

One open issue is whether the level ‘0’ for the reference amplitude quantisation should be mapped to a valid quantisation level or it should be kept ‘reserved’. It was proposed to replace this reserved value with an additional level at -1.5dB lower than the current lowest level, i.e., at -22.5dB (Alt 2) or with a value obtained by some further optimisation based on simulation-based distribution of the coefficient amplitudes (Alt 3). Whilst Alt 2 provides negligible performance improvement because of the very low quantisation value, Alt 3 depends on the assumed distribution of the coefficient amplitudes and may not be the best choice for other distributions.

- Regarding the quantisation mechanism for the reference amplitude, support Alt 1, i.e., keep the codepoint ‘0’ as reserved.

## 2.3	Wf configuration, selection and reporting

In RAN1#105-e there was a discussion on the length of the all-one vector when Wf is turned off by the gNB. This was an FFS point identified in RAN1#104-e. The following three alternatives were proposed in the last meeting.

Agreement

For Rel-17 port selection codebook, study following Alternatives and down-select in RAN1 106e:

- Alt 1: Wf OFF and Wf ON with Mv=1 are same, and Wf is an all-one vector of length N3. Wf as an all-one vector of length 1 is not needed

- Alt 2: Wf OFF and Wf ON with Mv=1 are same, and Wf is an all-one vector of length 1, i.e., a scalar. Wf as an all-one vector of length N3 is not needed.

- Alt 3: Keep both Wf OFF and Wf ON with Mv=1.

  - If PMI format is SB, Wf  is an all-one vector of length N3

    - Informative note: this case is considered as “Wf ON with Mv=1” in the agreement in RAN1 104e

  - If PMI format is WB, Wf is an all-one vector of length 1, i.e., a scalar

    - Informative note: this case is considered as “Wf OFF” in the agreement in RAN1 104e

- Note: N3 = NCQISubband*R.

- FFS: the case when no SB size is configured.

It was noted during the discussion that the UE behaviour in calculating the PMI is the same in the two cases described in Alt 1 and 2, i.e., in the case Mν=1 and Wf is an all-one vector of length N3 (Alt 1) and the case Wf is OFF and Wf is the scalar 1 (Alt 2). Therefore, supporting both configurations as proposed in Alt 3 is redundant from specifications perspective.

- Regarding the configuration with Wf OFF, support Alt 1, i.e., Wf OFF and Wf ON with Mν=1 are the same, and Wf is an all-one vector of length N3.

In a related discussion, it was proposed to extend support for Rel-17 PS codebook to BWP sizes smaller than 24 PRBs in wideband reporting mode.

Currently, there is no subband definition for BWP<24 PRB and only Rel-15 Type I and Type II CBs are supported for these bandwidth parts with wideband frequency granularity (pmi-FormatIndicator is set to 'widebandPMI'). Possible use cases for BWP<24 PRB involve RedCap and NR railway where smaller carrier bandwidths (CBW) are being considered, such as 3 and 3.6 MHz. However, currently, the smallest CBW of 5MHz is only supported at 15KHz SCS, hence there is no supported 5MHz UE channel bandwidth <24 PRB. The only few cases with 11PRBs and 18PRBs are with 60kHz SCS for 10MHz and 15MHz UE channel bandwidth (see tables 5.3.5-1 and 5.3.2-1 in TS 38.101-1), but 60kHZ is not a commonly used SCS.

In RedCap WI, there is an ongoing discussion on supporting subband CSI reporting for BWP<24, for example by introducing a SB size (e.g. 2 or 4) for BWP size < 24 PRBs (see P9 in R1-2105316 [9]). If a subband definition is introduced for BWP<24 PRB, both Rel-16 and Rel-17 CBs could be configured for these small BWP without further chances in the specifications.

- Regarding a possible extension to support Rel-17 CB to BWP<24 PRB, consider the outcome of the discussion in RedCap WI to extend the subband definition to BWP<24 PRB. If the subband definition is extended to BWP<24 PRB, no further changes are needed to support Rel-17 CB for such small BWPs.

Another open issue related to the definition of Wf for rank larger than 1 and Mν>1 is whether Wf should be layer-common or layer-specific. In Rel-16 eType II CBs, in a typical implementation, the “layer extraction” operation follows the port selection for W1 and precedes the FD basis selection for Wf and the nonzero coefficients selection and quantisation for W2, as illustrated, in principle, by the high-level block diagram of Figure 5. Naturally, the operations before the layer extraction are layer-common, whilst those performed after layer extraction are layer-specific. In Rel-16 design, of the codebook components, W1 is layer-common, whereas both Wf and W2 are layer-specific.

Figure 5. High-level diagram of the operations involved in Rel-16 PS CB showing the position of the “layer extraction” operation relative to the codebook components. Of these, W1 is layer-common, whilst Wf and W2 are layer-specific.

In Rel-17, however, there are strong motivations to define Wf as layer-common instead of layer-specific. The main advantage of a layer-common design for Wf is the lower complexity in the layer-extraction operation at the UE. As illustrated in the high-level block diagram of Figure 8. In Rel-17, because most of the FD compression is carried out at the gNB, it is possible to replace the N3 EVD operations, which calculate ν eigenvectors each, of length 2L=K1, with a single SVD that yields ν singular vectors. The length of these singular vectors depends on whether Wf can be assumed layer-common or layer-specific. In case of layer-common assumption, the singular vectors can be of length K1Mν, whereas in the layer-specific case the SVD need to be of size K1N3.

- Regarding the choice of layer-common vs layer-specific Wf for rank ν>1 and Mν>1, assuming Wf layer-common allows to reduce the size and complexity of the SVD operation from K1N3 to K1Mν.

The main reason for the network to configure a window of components for Wf is to provide robustness against imperfect UL-DL reciprocity of delays and any timing offset between delay estimation at gNB and UE. Because these impairments are common across receive antennas, it makes sense for the selection of Wf components at the UE to be layer common

- The main reason for the network to configure Mν>1 is to increase robustness against non-ideal delay reciprocity and timing offsets. Because these impairments are common across receive antennas, the selection of Wf components at the UE can be layer common.

Simulation results confirm that there is negligible performance loss in restricting Wf to be layer common. In Figure 6 and Figure 7 we compare performance between layer-common and layer-specific Wf for MR2, with Mν=2 and measurement window N=4 and for different parameter combinations (α,β), and number of ports P={8,16,32}.

| Figure 6. Performance for MR2 and layer-common Wf, with N=4 and Mν=2 | Figure 7. Performance for MR2 and layer-specific Wf, with N=4 and Mν=2 |
| --- | --- |

- Regarding the choice of layer-common vs layer-specific Wf for rank ν>1 and Mν>1, we observed no performance difference between the two assumptions in simulation results.

- Regarding the choice of layer-common vs layer-specific Wf for rank ν>1 and Mν>1, support layer-common Wf.

Figure 8. High-level diagram of the operations involved in Rel-17 PS CB showing the position of the “layer extraction” operation relative to the codebook components. Of these, it is proposed that both W1 and Wf are layer-common, whilst W2 remains layer-specific.

Another outstanding issue identified in RAN1#105-e is the relationship between the window size N and the number of components of Wf, Mν>1, as described in the following agreement.

Agreement

At least for rank 1 and 2 and Mv > 1, for relationship between N and Mv, study and down-select one alternative from following in RAN1#106-e

- Alt 1: N= Mv always, no UE reporting of Wf

- Alt 2-1: N >= Mv, Wf  is layer-common and reported by UE for N>Mv.

- Alt 2-2: N >= Mv, Wf is layer-specific and reported by UE for N>Mv.

Note: Wf is layer-common for N=Mv

Note: For all alternatives, a layer-common window/set of size N is configured.

Alt 2 provides a wider selection of candidate components for the UE to select, hence increased robustness against non-ideal delay reciprocity and timing offsets. Additionally, for Alt 2-1, for N>Mν, Wf is reported as layer-common, which only requires log2(N-1) bits to report 1 additional component besides component 0 for Mν=2.

- Regarding the relationship between N and Mν, support Alt 2-1, i.e., N≥Mν, with Wf layer-common and reported by a UE for N>Mν.

| Figure 9. Performance for MR2 and layer-common Wf, with N=Mν=2 | Figure 10. Performance for MR2 and layer-common Wf, with N=4 and Mν=2 |
| --- | --- |

Regarding the reporting of Wf, for N>Mν, the following simple solution requires the smallest overhead

- For N>Mν>1, support reporting of Mν-1 nonzero components of Wf using a combinatorial indicator of log2N-1Mν-1 bits.

Regarding the gNB configuration of the measurement window for Wf as a consecutive window of size N, the following working assumption was agreed in RAN1#105-e

Working assumption

At least for rank 1, FD bases used for Wf quantization are limited within a single window with size N configured to the UE whereas FD bases in the window must be consecutive from an orthogonal DFT matrix, i.e. Alt 1

- FFS: Further dependence/restriction, e.g. conditioned on N3 or the number of CSI-RS ports, can be applied to above design. If does, how to support a non-consecutive FD bases used for Wf quantization

- FFS: Whether to introduce thresholds for N3 and/or P

We recall that the main reason for the network to configure a window/set of components for Wf is to provide robustness against imperfect UL-DL reciprocity of delays and any timing offset between delay estimation at gNB and UE. For these use cases, configuring non-consecutive components does not seem to provide any advantage. In fact, if the components of the measurement set are selected freely by the gNB, based on delay estimation from SRS, this set would need to be updated dynamically, which requires significant extra signalling.

- Confirm the working assumption on a single measurement window for Wf, for all supported ranks, without further dependence/restriction on the values of N3 and/or P.

Regarding the supported values for the parameter R, the following was agreed in RAN1#104bis-e

Agreement

At least for rank 1, regarding the value(s) of R for Rel-17 PS codebook enhancement, study and down-select one or more than one Alternative (or a subset of corresponding values) in RAN1 105e:

- Alt 0:  R < 1 (e.g. 1/4, 1/2)

- Alt 1: R=1

- Alt 2: R=1 and 2

- Alt 3: R=1,2, 4, and 8

- Alt 4: R= {1,2,…, D*NPRBSB} whereas D is the density of CSI-RS in frequency domain

- FFS: applicable conditions: e.g. Wf turned ON/OFF and/or associated value of Mv

- FFS: Whether this applies when Wf is turned OFF

Note that “at least for rank 1” does not imply for the support of rank 1 only in Rel-17 or restrictions of supporting/not supporting additional alternatives for higher rank.

As shown in our previous contribution [8] and confirmed in results presented below, we observe best performance when parameter R is configured at the maximum value, for R=D/NPRBSB, i.e., when only one RB per PMI subband carries CSI-RS. Generally, we observe performance degradation for values of R smaller than the maximum, however there is at least a use case where values R smaller than the maximum may be beneficial, i.e., when the gNB has lower delay resolution due, for example to the SRS bandwidth being a fraction of that of CSI-RS.

- The minimum configurable value of R depends on the CSI-RS density D, according to the inequality: R≤D⋅NPRBSB, which states that the number of RBs carrying CSI-RS in a PMI subband, d⋅NPRBSBR, must be larger than 1. In simulations, we observe that throughput performance is best with equality, for R=D⋅NPRBSB, i.e., when only one RB per PMI subband carries CSI-RS. Values of R<D⋅NPRBSB may be beneficial for some special use cases, such as when the gNB has lower delay resolution due, for example to the SRS bandwidth being a fraction of the CSI-RS bandwidth.

- Regarding the values of R, support configurations with R=D⋅NPRBSB.

| Figure 11. Performance for MR1 and R=D⋅NPRBSB=4 and different parameter combinations. | Figure 12. Performance for MR1 and R=2<D⋅NPRBSB and different parameter combinations. |
| --- | --- |

| Figure 13. Performance for MR2 and R=D⋅NPRBSB=4 and different parameter combinations. | Figure 14. Performance for MR2 and R=2<D⋅NPRBSB and different parameter combinations. |
| --- | --- |

# 3	M-TRP CSI reporting enhancement

## 3.1	Remaining aspects of a Reporting Setting configuration

With regard to the configuration parameters for a single M-TRP CSI Reporting Setting, in the following we address the remaining open issues regarding the value of Ks,max and M and the mechanism to configure M (M≤ Ks) CMRs from the CSI-RS resource set for Single-TRP measurement hypotheses.

In RAN1#105-e, the following alternatives were identified for the maximum default value of Ks, i.e., the maximum number of CMR resources in a set that a UE can support by default, if M-TRP CSI reporting is supported.

Agreement

For a CSI-RS resource set with Ks NZP CSI-RS resources configured for CMR and N NZP CSI-RS resource pairs configured for NCJT measurement hypotheses, study following default value of Ks,max,

- Alt 1: Ks,max = 4

- Alt 2: Ks,max = 2

- Alt 3: Ks,max = 4 for FR2, and Ks,max = 2 for FR1

Note that default value means the minimal supported value for Ks,max in UE capability reporting, if UE support this feature.

Configuring different CMR resources for NCJT and S-TRP measurement is possible in both FR1 and FR2 without additional capability signalling. In fact, there exist use cases for which configuring different CMR resources for NCJT and single-TRP measurement hypotheses is beneficial, for example:

- The gNB may apply different beamforming to the CSI-RS resources intended for NCJT and single-TRP measurement. For example, the CMR resources to be measured for NCJT may be beamformed such that interference between two TRP is minimised for cell-edge UEs, whereas the CMR resources intended for single-TRP measurement are beamformed to maximise beamforming gain.

- In FR2, a UE may use different receive spatial filters to measure NCJT and single-TRP hypotheses. For example, when measuring a CMR resource for an NCJT hypothesis, the receive spatial filters may be optimised for single-panel reception, with two different panels measuring each CMR resource in the NCJT pair. Conversely, for a single-TRP measurement hypothesis the spatial filters may be optimised for two-panel reception, as both panels can receive a single TRP. Therefore, CMR measurement may be impacted if a single CMR resource is configured for both single-TRP and NCJT measurements if only one spatial filter can be used per measurement. In this case, it would be beneficial to configure different CMR resources for single-TRP and NCJT measurement hypotheses.

Therefore, it is beneficial for both network and UE to be able to configure Ks>2 CMR resources in a set and restrict single-TRP measurements, when applicable, to only a subset M≤Ks resources.

- For a multi-TRP CSI Reporting Setting, configuring Ks>2 CMR resources in a set and restricting single-TRP measurements to a subset, M≤Ks, of CMR resources allows the flexibility to configure different resources for NCJT and single-TRP measurements.

- For a multi-TRP CSI Reporting Setting, configuring different CMR resources for NCJT and single-TRP measurements allows the network to optimise transmit beamforming to minimise interference between TRPs (NCJT) or maximise beamforming gain (single-TRP). It also allows a UE to optimise receive spatial filters when measuring a CMR resource for NCJT or single-TRP hypothesis.

- Regarding the default value of Ks,max, support Alt 1, i.e., Ks,max=4.

Regarding the definition and value of M, i.e., the number of CMR resources configured for single-TRP measurement, this affects, for example, the CRI bitwidth for Option 2  the number of CSI-IM resources that can be configured, according to the following agreement:

Agreement

A CSI-IM resource is configured to be associated with either a CMR for Single-TRP measurement hypothesis or a CMR pair for NCJT measurement hypothesis:

- One-to-one mapping between M+N CSI-IM resources versus M NZP CSI-RS resources for single-TRP measurement hypothesis and N NZP CSI-RS resource pairs for NCJT measurement hypothesis configured in a CSI-RS resource set.

  - FFS the value/definition of M

Note: it is possible to configure the same value of CSI-IM resource ID for both NCJT and Single-TRP measurement hypotheses in FR1 and FR2, subject to QCL-Type D consistency between measurement hypotheses of the shared CMR in FR2

The definition of M depends on the mechanism used to configure the CMR for single-TRP measurement in a Reporting Setting. The following three alternatives were identified in RAN1#105-e:

Agreement

For CSI measurement associated with a CSI-ReportConfig for NC-JT, down-select one or more Alts in RAN1#106-e:

- Alt 2: additional RRC signalling is needed to configure M (M≤ Ks) CMRs from the CSI-RS resource set for CMR for Single-TRP measurement hypotheses

  - Example: For a given set of {{#0, #1}, {#2, #3}} with N=1, {#0, #2} are for NCJT measurement hypothesis. Additional RRC signaling may select {#0,#3} (if sharing is allowed), or {#1, #3} (if not allowed), or select any from the set for single-TRP measurement hypotheses.

- Alt 3: For CMRs configured in the CSI-RS resource set, support RRC signalling to enable/disable single-TRP measurement hypothesis using CMR configured within CMR pairs for NCJT measurement hypothesis

  - Example: For a given set of {{#0, #1}, {#2, #3}} with N=1, {#0, #2} are for NCJT measurement hypothesis. If gNB enables the sharing, {#0, #1, #2, #3} are for single-TRP measurement. If gNB disable the sharing, {#1, #3} are for single-TRP measurement hypotheses.

- Alt 4: CMR sharing between single-TRP measurement hypothesis and NCJT measurement hypothesis is realized by configuring the same value of CMR ID for single-TRP CMR and NCJT CMR pair.

  - Example: When the UE supports sharing, for a given set of {{#0, #0}, {#2, #3}} with N=1, {#0, #2} are for NCJT measurement hypotheses, the rest {#0, #3} are for single-TRP measurement hypotheses. The CMRs for STRP can be updated by re-configuring the CSI resource set.

Note that above examples are only for the purpose of illustrating/discussing Alternatives.

Alt 2 comprises two possible RRC signalling mechanisms to configure M:

- a bitmap of size Ks to select the CMR resources for single-TRP measurement

- indication of the value of M only, such that, for example, the first M≤Ks CMR resources in the set are configured for s-TRP measurement.

Note that the details of the RRC design for the CSI Reporting Setting are left to RAN2. However, it is worth clarifying that the configuration of the N NCJT pairs is such that any pair of CMR resources can be configured for NCJT measurement. Hence, there is no need to assume any predefined order of assignment of CMR resources to CMR groups, for example that the two CMR groups are formed by the first K1 and the last K2 CMR resources in the set, respectively. In fact, to reduce the RRC signalling overhead, one can assume, without loss of generality, that the first M CMR resources in the set are configured for single-TRP measurement hypotheses.

In the following, we illustrate an example of CSI Reporting Setting configuration according to Alt 2. In the CSI Reporting Setting the CMR resource set consists of Ks resources and the network can assign any K1 of these CMR resources to group 1 and the remaining K2=Ks-K1 to group 2. Moreover, the network can select N CMR pairs for NCJT measurement from any combination of CMRs from the two groups, by using a bitmap of size K1K2. As for the single-TRP measurement hypotheses, if the network wants to configure, say M≤Ks CMR resources for single-TRP measurement with M1≤K1 taken from group 1 and M2=M-M1≤K2 from group 2, it can simply arrange these M CMR resources at the beginning of the resource set list, without restricting the choice for the CMR group and NCJT pairs configuration. In practice, full flexibility in the configuration of NCJT and single-TRP hypotheses can be achieved with minimal signalling overhead by using a bitmap of size K1K2 for the NCJT pairs and configuring the first M CMR resources in the CSI-RS resource set list for single-TRP measurement.

The configuration of CMR groups, NCJT hypotheses and single-TRP hypotheses is shown in Figure 15, for an example with Ks=5, K1=3, K2=2, N=2. The example shows a possible RRC definition for M as well as the RRC parameter X∈{0,1,2}. Parameters M and X together determine the reporting quantities for both Option 1 and 2 reporting, as illustrated in the Figure.

Figure 15. CSI Reporting Setting configuration for multi-TRP showing the configuration parameters for the NCJT hypotheses (bitmap), single-TRP hypotheses (M) and single-TRP reported CSIs (X).

- Full flexibility in the configuration of NCJT and single-TRP hypotheses can be achieved by Alt 2 with minimal signalling overhead by using a bitmap of size K1K2 for the NCJT pairs and configuring the first M CMR resources in the CSI-RS resource set for single-TRP measurement.

Alt 3 proposes to use a single bit in RRC signalling to enable or disable the sharing of CMR resources between NCJT and single-TRP measurement. If CMR sharing is enabled, then M=Ks. However, if CMR sharing is disabled, determining the value of M is more complicated as it depends on whether some CMRs are referred to by two NCJT pairs, which is feasible in FR1. In this case, one has to count the unique CMR IDs that are configured in the NCJT pairs, say N'<2N, and the number of single-TRP hypotheses is given by Ks-N'. Moreover, with Alt 3, it is not possible to configure partial sharing of the NCJT CMRs, so, for example, in the case no CMR is referred to by two NCJT pairs, the only configurable values for M are M=Ks and M=Ks-2N, and the values Ks-2N<M<Ks are not configurable.

- Alt 3 is less flexible as it only allows to configure full sharing (M=Ks) or no sharing (M=Ks-2N) of CMRs between NCJT and single-TRP measurements. Besides, determining the value of M is complicated when sharing is disabled and some CMRs are referred to by two NCJT pairs.

On the other hand, Alt 4 assumes that the value of M is always M=Ks-2N and sharing of CMRs between NCJT and S-TRP is achieved by duplicating the IDs of the shared CMRs in the resource set. Compared to Alt3, Alt 4 offers more flexibility in allowing partial sharing of the NCJT CMRs at the cost of introducing unnecessary configuration redundancy, when compared to Alt 2. However, Alt 4 has the same complication as Alt 3 in determining the value of M when sharing is disabled and some CMRs are referred to by two NCJT pairs.

- Alt 4 assumes that M=Ks-2N in all cases. It offers more flexibility than Alt 3 as it allows to configure partial sharing of the NCJT CMRs at the cost of introducing unnecessary configuration redundancy, if compared to Alt 2. Alt 4 has the same complication as Alt 3 in determining the value of M when sharing is disabled and some CMRs are referred to by two NCJT pairs.

- Regarding the definition/value of M, support Alt 2, i.e., additional RRC signalling to configure the value of M≤Ks.

- Regarding the definition/value of M, further clarify that the CMRs for single-TRP measurement can be configured from the first M CMRs in the resource set, without loss of flexibility.

Regarding CBSR and RI combinations/restrictions the following was agreed in RAN1#104bis-e:

Agreement

Support the indication of following RI combinations by a joint RI field for a NCJT measurement hypothesis in CSI part 1, when the maximal transmission layers is less than or equal to 4:

- {1, 1}, {1, 2}, {2,1}, {2,2}

- FFS: CBSR and/or RI restrictions per TRP or across TRPs

In Rel-15/16 these configurations are included in CodebookConfig and CodebookConfig-r16, respectively, of CSI-ReportConfig. Therefore, currently these restrictions pertain to the codebook configuration and because a CSI-ReportConfig can include only a single codebook configuration, the same restrictions would apply to all measurement hypotheses. Because there is no TRP association to CMR resources, it is not possible to configure restrictions for individual TRPs. However, it is possible to have a CodebookConfig configuration in the Reporting Setting with two CBSR and RI restrictions, one for each CMR resource group.

- Regarding CBSR and RI restriction, support a single CodebookConfig configuration for a CSI Reporting Setting with two CMBS and RI restrictions, one for each CMR group.

## 3.2	Dynamic update of configuration parameters

In RAN1#105-e, it was agreed to study whether to support dynamic updating of a number of configuration parameters, as indicated in the following agreement:

Agreement

For CSI measurement associated with a CSI-ReportConfig for NC-JT, study whether/how to support following dynamic updating on, e.g. by MAC-CE

- Alt 1: CMR pairs for NCJT measurement hypotheses

- Alt 2: CMRs for Single-TRP measurement hypotheses

- Alt 3: TCI states in CMRs

- Alt 4: the number of single-TRP CSIs (i.e. X=0/1/2) in a NCJT CSI report

Regarding dynamic updating of configuration parameters, there are several use cases benefitting from dynamically updating the NCJT pairs, the number of single-TRP measurements, and the number X∈{0,1,2} of single-TRP CSIs, via MAC-CE. This MAC-CE update is applicable to semi-persistent and aperiodic multi-TRP reporting.

In one use case, if a number of CPUs are already occupied, for example by other simultaneous periodic CSI reports, the gNB may need to schedule, for example, an aperiodic multi-TRP report with reduced number of CSI calculations to avoid CPU overbooking. This can be done by reducing the number of single-TRP measurements or removing some NCJT pairs. In another use case, the gNB may dynamically update the NCJT pairs based on some prior information of the channel propagating conditions or some other CSI scheduling constraints. For example, the gNB may decide to remove an NCJT hypothesis that is unlikely to be selected and adjust the number of single-TRP hypotheses. In a third use case, being able to dynamically adjust the number X of single-TRP CSIs allows the gNB to better control the feedback overhead and avoid partial omission of part 2 of the CSI report, if the resources available on PUSCH are limited.

On the other hand, dynamic updating of TCI states for CMR resources is already supported for aperiodic and semi-persistent reporting by means of multiple trigger states.

- There are some use cases benefitting from dynamically updating the NCJT pairs, the number of single-TRP measurements, and the number X∈{0,1,2} of single-TRP CSIs, via MAC-CE:

- Controlling the number of CSI calculations to avoid CPU overbooking

- Updating the NCJT pairs based on some prior information of the channel propagating conditions or some other CSI scheduling constraints

- Dynamically adjusting the number X of single-TRP CSIs allows the gNB to better control the feedback overhead and avoid partial omission of part 2 of the CSI report, if the resources available on PUSCH are limited.

- Regarding the dynamic updating of configuration parameters, support Alt 1,2 and 4, by MAC-CE indication of the N NCJT pairs and/or the value of M and/or the value of X.

- Support the use of parameters M and X to configure Option 1/Option 2 reporting: M=X=0 configures Option 2 with 1 CSI (best of N+M hypotheses); otherwise Option 1 is configured with X+1 CSI(s).

## 3.3	CRI mapping

In RAN1#105-e, a number of open items where identified for additional study, which have potential impact on specifications. Amongst them, CBSR and RI restriction was discussed in the previous section. Hereafter, we address the CRI codepoint mapping as indicated by the following agreement:

Agreement

For Rel-17 Multi-TRP CSI enhancement, companies are encouraged to study following potential specification impact:

- CRI codepoint mapping order with CMRs and CMR pairs

- Whether/how to configure RI restriction/CBSR configuration for NCJT CSI measurement

- Whether/how to enhance the CSI updating rule to address CPU overbooking

- Whether/how to introduce new CSI computation delay requirement for NCJT CSI calculation

- Whether/how to support wideband CSI report

For Option 1, one question is whether the X+1 CRI(s) should be jointly encoded as a single CRI report or separately encoded as separate CRI reports in the CSI report, because a single CSI report is associated with a Reporting Setting.

In case of joint encoding, a single CRI report has bit width log2N, for X=0,  log2(NM)=log2N+log2M, for X=1, and log2(NM1M2)=log2N+log2M1+log2M2, for X=2, where M1 and M2 are the number of single-TRP hypotheses configured from CMR group 1 and 2, respectively and M=M1+M2.

In case of separate encoding, CRI 1 is associated to the NCJT CSI and has bit width log2N, CRI 2 is associated with the single-TRP CSI for X=1 and has bit width log2M, whereas for X=2, CRI 2 and 3 are associated with the single-TRP CSI for CMR group 1 and 2, respectively, and have bit width log2M1 and log2M2, respectively. Note that if one or more values of N, M1, M2 equal 1, the corresponding CRI does not need reporting.

Although, in general, x+y≤x+y, in practice, for most applicable combinations of N, M1, M2, there is no difference in the number of bits required by joint and separate encoding. Therefore, it is more straightforward to associate a separate CRI to each reported CSI, as illustrated in Figure 16, in a general case, for Option 1.

- For Option 1, when X=1, joint CRI reporting requires log2N+log2M bits, whereas separate CRI reporting requires  log2N+log2M bits. When X=2, joint CRI reporting requires log2N+log2M1+log2M2, whereas separate CRI reporting requires log2N+log2M1+log2M2, where M1 and M2 are the number of single-TRP hypotheses configured from CMR group 1 and 2, respectively.

- For Option 1, in all practical configurations, the bit width required by joint and separate CRI reporting is the same.

- For Option 1, support separate X+1 CRI(s), with bit width log2N for X=0,  log2N and log2M, for X=1, and log2N, +log2M1 and log2M2 for X=2, where M1 and M2 are the number of active single-TRP hypotheses from CMR group 1 and 2, respectively, with M=M1+M2.

Figure 16. Mapping of CRI(s) to measurement hypotheses for Option 1, in a general case.

For Option 2, there is a single CRI being reported with bit width log2(N+M) and a possible simple mapping mechanism is illustrated, in a general case, in Figure 17.

- For Option 2, support a CRI mapping with log2(N+M) bits, where the first N codepoints are associated to the N configured NCJT hypotheses and the last M codepoints are associated to the first M CMR resources in the CSI-RS resource set.

Figure 17. Mapping of the CRI to measurement hypotheses for Option 2, in a general case.

## 3.4	CSI priority, omission, and CPU occupancy

In RAN1#105-e it was agreed to study the need for prioritisation of the CSIs within a report in case of Option 1 with 1 or 2 single-TRP CSIs, as described in the following conclusion:

For future RAN1 meeting:

For a CSI report setting with Option 1 and X=1 or 2, study prioritizing CSI associated with reported CSI hypotheses within a CSI Reporting Setting

- FFS potential impact for UCI payload generation

- FFS whether/how to update CSI priority formula, and additional specification impact due to updated formula

- FFS whether/how to update CSI omission rules for Part 2 CSI based on prioritized CSI

- FFS: whether the X+1 CSI hypotheses per CSI Reporting Setting are mapped to a single CSI report or X+1 CSI reports

- Companies are encouraged to discuss and justify purposes of prioritizing CSI associated with reported CSI hypotheses.

In the current specifications [TS 38.214, 5.2.5], the priority function is defined for each CSI report and it depends on the reportConfigID, the serving cell index, whether the reports include L1-RSRP or L1-SINR, as reporting quantities, or not, the time-domain reporting behaviour and the uplink channel used for reporting.

The CSI priority function impacts both UCI mapping and related omission rules in case of insufficient PUCCH/PUSCH resources, and the “soft” formula that controls which CSI reports are not updated in case of CPU overbooking.

Changing the priority function Prii,CSI(y,k,c,s) to include the X+1 CSIs reported in a single M-TRP CSI report would complicate significantly the rules for CPU overbooking because, for example, only some of the CSIs in the report would not be updated in case the CPU counts exceeds the UE capability. This would add to the already complex handling of CSI reporting capabilities for the maximum number of resources/ports and CPUs.

The problem of CPU overbooking can be mitigated more easily, for example by allowing dynamic updating of the configuration parameters N, M and X, which control the number of CSI measurements, hence the number of occupied CPUs.

- Changing the priority function Prii,CSI(y,k,c,s) to include the X+1 CSIs reported in a single M-TRP CSI report would further complicate the rules for CPU calculation. The problem of CPU overbooking can be mitigated more easily by allowing dynamic updating of the configuration parameters N, M and X, which control the number of CSI measurements.

Regarding UCI mapping of the CSIs, for a UE configured with Option 1 and X=1,2, multiple CSIs would be reported with the same reportConfigID. As the measurement hypotheses do not have any impact on the priority function, single-TRP and NCJT CSIs will have the same priority.

This means that, in case of CSI omission, the UE does not distinguish between reports based on their assumed transmission hypotheses. If omission is decided for a given reportConfigID, all associated CSIs would be impacted.

While the problem of CSI omission can be reduced by optimizing CSI reporting configuration so that instances of colliding reports are minimized as much as possible, a prioritization based on the CSI measurement hypothesis would be beneficial. One simple way of achieving this is to assign different priority reporting levels to the CSIs configured in the report, such that the NCJT CSI of CSI report n has priority 0, the single-TRP CSI, in case X=1, and the single-TRP CSI for CMR group 1, in case X=2, have priority 2n-1, the single-TRP CSI for CMR group 2, in case X=2, has priority 2n.

- Regarding the UCI mapping of the CSIs in a M-TRP CSI report n, support a simple solution that assigns reporting priority 0 to the NCJT CSI, priority 2n-1 to the single-TRP CSI, for Option 1 with X=1, and to the single-TRP CSI for CMR group 1, for Option 1 with X=2, priority 2n to the single-TRP CSI for CMR group 2, for Option 1 with X=2.

## 3.5	PMI/RI sharing

In RAN1#105-e, a proposal was discussed (see [10]) to share PMI and RI reporting between NCJT and single-TRP CSIs when a UE is configured to report X=2 single-TRP CSIs in addition to the NCJT CSI. The following was study was agreed:

Agreement

For Option 1 CSI reporting associated with NCJT and X single-TRP measurement hypotheses, study whether to support following PMI/RI sharing mechanisms between NCJT CSI and single-TRP CSI(s):

- Enabling/Disabling PMI, RI sharing via higher-layer configuration

- Dynamic indication of PMI, RI sharing in the CSI report

- FFS: other details

- FFS: applicable conditions/restrictions of CMR sharing among Single-TRP and NCJT hypotheses, if above PMI/RI sharing mechanism can be applied

Sharing of PMI and/or RI among the single-TRP and NCJT hypotheses would reduce UCIs overhead. However, in order to avoid performance excessive performance degradation two additional CQIs need to be reported, which correspond to the transmission hypotheses in which only one TRP is transmitting using the PMI and RI indicated in the NCJT CSI.

Nonetheless, while the NCJT report provides feasible PMIs for single-TRP transmission hypothesis, they will be suboptimal as the number of their layers was restricted due to the limited number of rank combination for NCJT hypothesis. A PMI computation with a prior assumption of single-TRP transmission could have resulted in a PMI with a higher rank, achieving better performance.

Some questions remain on the applicability of the sharing scheme, the implications for the UE meaasruments and the complications it introduces for CPU calculation, UCI mapping and omissions. For example, if PMI/RI sharing is enabled, only the NCJT pairs need to be measured to calculate the PMI and RI, because these quantities are derived from the best NCJT hypothesis. However, in cases other than full CMR sharing between NCJT and single-TRP hypotheses, it is not clear which resources should be used for the calculation of the additional CQIs based on single-TRP transmission. Therefore, it seems that the only relevant configuration for the PMI/RI sharing scheme is when M=0, i.e., no single-TRP measurement hypotheses are configured. However, M=0 is also configured for Option 1 reporting with X=0, hence Option 1 with X=2, M=0 and PMI/RI sharing enabled, looks very similar to Option 1 with X=0. The only difference is in reporting two additional CQIs.

The similarities between X=2 with PMI/CQI sharing and X=0 also apply to CPU occupancy and CRI report, for which, in case of X=2, different rules need to be considered if PMI/CQI sharing is enabled.

Supporting PMI/CQI sharing as a special case of Option 1, X=2, requires additional RRC signalling and specific rules for CMR measurements, CRI mapping, UCI mapping and CPU calculation. This would, in practice, introduce a fifth reporting option, in addition to Option 1, X=0,1,2 and Option 2, with considerable additional specification effort.

- Regarding the study of PMI/RI sharing mechanisms, consider the completion of specifications for the already agreed Option 1, X=0,1,2 and Option 2 with higher priority.

# 4	Conclusion

Hereafter is a summary of observations and proposals for FDD CSI enhancement

Observation 1	In Rel-17, because of the window/set restriction of size N configured by the gNB, the SCI needs to be modified by adding information related to the FD component of the strongest coefficient for each layer.

Observation 2	Regarding the SCI definition, if not shift is applied (Alt 1-1), one can assume that all Mν components of Wf need reporting, using log2NMν bits.

Observation 3	The additional indication of fl* in the SCI for each layer is needed both in the case Wf is not reported (N=Mν) and in the case Wf is reported (N>Mν).

Observation 4	For all possible index mapping of Wf and W2, reporting the position il*, fl* as SCI for layer l is enough to locate the strongest coefficient of W2 and to ensure correct reconstruction of Wf, in case Wf is reported.

Observation 5	Reporting the position il*, fl* as SCI for layer l using log2K0 bits (Alt 0) requires correct decoding of the bitmap before the strongest coefficient can be located. Therefore, the bitmap needs to be mapped in the same UCI Group 0 as the SCI and W1, whereas the nonzero coefficients are mapped in Groups 1 and 2.

Observation 6	Reporting the position il*, fl* as SCI for layer l using log2K0 bits (Alt 0) requires that the bitmap is reported without any shift that depends on fl*. If the strongest coefficient is shifted such that fl*=0, the shift can only be applied to the nonzero coefficients of W2 but not to the bitmap.

Observation 7	Replacing the SCI with a one-bit indicator of the stronger polarisation (Alt 3) requires larger overhead: 8 bits per layer for any value of K1 and Mν, compared to at most 6 bits for K1=32 and Mν=2 for the SCI reported with log2K1Mν bits.

Observation 8	If the bitmap is absent from the CSI reporting for some configurations, e.g., Mν=1, β=1 and rank ν≤2, a UE cannot determine the real number of NZC, as the maximum number of NZC must be reported. In this case the indicator KNZ in part 1 CSI becomes redundant and coefficients that may be virtually zero are reported with a nonzero quantisation level.

Observation 9	Regarding the choice of layer-common vs layer-specific Wf for rank ν>1 and Mν>1, assuming Wf layer-common allows to reduce the size and complexity of the SVD operation from K1N3 to K1Mν.

Observation 10	The main reason for the network to configure Mν>1 is to increase robustness against non-ideal delay reciprocity and timing offsets. Because these impairments are common across receive antennas, the selection of Wf components at the UE can be layer common.

Observation 11	Regarding the choice of layer-common vs layer-specific Wf for rank ν>1 and Mν>1, we observed no performance difference between the two assumptions in simulation results.

Observation 12	The minimum configurable value of R depends on the CSI-RS density D, according to the inequality: R≤D⋅NPRBSB, which states that the number of RBs carrying CSI-RS in a PMI subband, d⋅NPRBSBR, must be larger than 1. In simulations, we observe that throughput performance is best with equality, for R=D⋅NPRBSB, i.e., when only one RB per PMI subband carries CSI-RS. Values of R<D⋅NPRBSB may be beneficial for some special use cases, such as when the gNB has lower delay resolution due, for example to the SRS bandwidth being a fraction of the CSI-RS bandwidth.

Proposal 1	Regarding the port selection parameter K1, consider the parameter formula: K1=αP, with α=1 for P≤12 and α∈1,12 for P>12.

Proposal 2	Support reporting of rank 3 and 4 by ensuring that feedback overhead is comparable to that of rank 2.

Proposal 3	Regarding the bitmap and SCI reporting, support:

- Reporting of the position, [il*, fl*], of the strongest coefficient of layer l, for Mν>1, using log2K1Mν bits.

- Remapping the strongest coefficient to fl*=0 as in Rel-16.

Proposal 4	Regarding the reporting of the bitmap, if β=1 and a UE reports KNZ=K0 nonzero coefficients for ν=1 or KNZ=2K0 nonzero coefficients for ν=2, the bitmap can be assumed to be absent from the CSI report. In all other cases, support reporting of the bitmap.

Proposal 5	Regarding the quantisation mechanism for the reference amplitude, support Alt 1, i.e., keep the codepoint ‘0’ as reserved.

Proposal 6	Regarding the configuration with Wf OFF, support Alt 1, i.e., Wf OFF and Wf ON with Mν=1 are the same, and Wf is an all-one vector of length N3.

Proposal 7	Regarding a possible extension to support Rel-17 CB to BWP<24 PRB, consider the outcome of the discussion in RedCap WI to extend the subband definition to BWP<24 PRB. If the subband definition is extended to BWP<24 PRB, no further changes are needed to support Rel-17 CB for such small BWPs.

Proposal 8	Regarding the choice of layer-common vs layer-specific Wf for rank ν>1 and Mν>1, support layer-common Wf.

Proposal 9	Regarding the relationship between N and Mν, support Alt 2-1, i.e., N≥Mν, with Wf layer-common and reported by a UE for N>Mν.

Proposal 10	For N>Mν>1, support reporting of Mν-1 nonzero components of Wf using a combinatorial indicator of log2N-1Mν-1 bits.

Proposal 11	Confirm the working assumption on a single measurement window for Wf, for all supported ranks, without further dependence/restriction on the values of N3 and/or P.

Proposal 12	Regarding the values of R, support configurations with R=D⋅NPRBSB.

Hereafter is a summary of observations and proposals for M-TRP CSI enhancement.

Observation 13	For a multi-TRP CSI Reporting Setting, configuring Ks>2 CMR resources in a set and restricting single-TRP measurements to a subset, M≤Ks, of CMR resources allows the flexibility to configure different resources for NCJT and single-TRP measurements.

Observation 14	For a multi-TRP CSI Reporting Setting, configuring different CMR resources for NCJT and single-TRP measurements allows the network to optimise transmit beamforming to minimise interference between TRPs (NCJT) or maximise beamforming gain (single-TRP). It also allows a UE to optimise receive spatial filters when measuring a CMR resource for NCJT or single-TRP hypothesis.

Observation 15	Full flexibility in the configuration of NCJT and single-TRP hypotheses can be achieved by Alt 2 with minimal signalling overhead by using a bitmap of size K1K2 for the NCJT pairs and configuring the first M CMR resources in the CSI-RS resource set for single-TRP measurement.

Observation 16	Alt 3 is less flexible as it only allows to configure full sharing (M=Ks) or no sharing (M=Ks-2N) of CMRs between NCJT and single-TRP measurements. Besides, determining the value of M is complicated when sharing is disabled and some CMRs are referred to by two NCJT pairs.

Observation 17	Alt 4 assumes that M=Ks-2N in all cases. It offers more flexibility than Alt 3 as it allows to configure partial sharing of the NCJT CMRs at the cost of introducing unnecessary configuration redundancy, if compared to Alt 2. Alt 4 has the same complication as Alt 3 in determining the value of M when sharing is disabled and some CMRs are referred to by two NCJT pairs.

Observation 18	There are some use cases benefitting from dynamically updating the NCJT pairs, the number of single-TRP measurements, and the number X∈{0,1,2} of single-TRP CSIs, via MAC-CE:

- Controlling the number of CSI calculations to avoid CPU overbooking

- Updating the NCJT pairs based on some prior information of the channel propagating conditions or some other CSI scheduling constraints

- Dynamically adjusting the number X of single-TRP CSIs allows the gNB to better control the feedback overhead and avoid partial omission of part 2 of the CSI report, if the resources available on PUSCH are limited.

Observation 19	For Option 1, when X=1, joint CRI reporting requires log2N+log2M bits, whereas separate CRI reporting requires  log2N+log2M bits. When X=2, joint CRI reporting requires log2N+log2M1+log2M2, whereas separate CRI reporting requires log2N+log2M1+log2M2, where M1 and M2 are the number of single-TRP hypotheses configured from CMR group 1 and 2, respectively.

Observation 20	For Option 1, in all practical configurations, the bit width required by joint and separate CRI reporting is the same.

Observation 21	Changing the priority function Prii,CSI(y,k,c,s) to include the X+1 CSIs reported in a single M-TRP CSI report would further complicate the rules for CPU calculation. The problem of CPU overbooking can be mitigated more easily by allowing dynamic updating of the configuration parameters N, M and X, which control the number of CSI measurements.

Proposal 13	Regarding the default value of Ks,max, support Alt 1, i.e., Ks,max=4.

Proposal 14	Regarding the definition/value of M, support Alt 2, i.e., additional RRC signalling to configure the value of M≤Ks.

Proposal 15	Regarding the definition/value of M, further clarify that the CMRs for single-TRP measurement can be configured from the first M CMRs in the resource set, without loss of flexibility.

Proposal 16	Regarding CBSR and RI restriction, support a single CodebookConfig configuration for a CSI Reporting Setting with two CMBS and RI restrictions, one for each CMR group.

Proposal 17	Regarding the dynamic updating of configuration parameters, support Alt 1,2 and 4, by MAC-CE indication of the N NCJT pairs and/or the value of M and/or the value of X.

Proposal 18	Support the use of parameters M and X to configure Option 1/Option 2 reporting: M=X=0 configures Option 2 with 1 CSI (best of N+M hypotheses); otherwise Option 1 is configured with X+1 CSI(s).

Proposal 19	For Option 1, support separate X+1 CRI(s), with bit width log2N for X=0,  log2N and log2M, for X=1, and log2N, +log2M1 and log2M2 for X=2, where M1 and M2 are the number of active single-TRP hypotheses from CMR group 1 and 2, respectively, with M=M1+M2.

Proposal 20	For Option 2, support a CRI mapping with log2(N+M) bits, where the first N codepoints are associated to the N configured NCJT hypotheses and the last M codepoints are associated to the first M CMR resources in the CSI-RS resource set.

Proposal 21	Regarding the UCI mapping of the CSIs in a M-TRP CSI report n, support a simple solution that assigns reporting priority 0 to the NCJT CSI, priority 2n-1 to the single-TRP CSI, for Option 1 with X=1, and to the single-TRP CSI for CMR group 1, for Option 1 with X=2, priority 2n to the single-TRP CSI for CMR group 2, for Option 1 with X=2.

Proposal 22	Regarding the study of PMI/RI sharing mechanisms, consider the completion of specifications for the already agreed Option 1, X=0,1,2 and Option 2 with higher priority.

# References

- RP-202024, Samsung, “Revised WID: Further enhancements on MIMO for NR,” RAN#89e, Sep. 2020.

- RAN1 Chair’s Notes, RAN1#105-e.

- R1-2106070, Huawei, “Summary of CSI enhancements for MTRP and FDD (Round 0)”, RAN1 #105-e.

- R1-2106071, Huawei, “Summary of CSI enhancements for MTRP and FDD (Round 1)”, RAN1 #105-e.

- R1-2106072, Huawei, “Summary of CSI enhancements for MTRP and FDD (Round 2)”, RAN1 #105-e.

- R1-2106194, Huawei, “Summary of CSI enhancements for MTRP and FDD (Round 3 and NWM)”, RAN1#105-e.

- R1-2103371, Nokia, Nokia Shanghai Bell, “Enhancement on CSI measurement and reporting”, RAN1 #104bis-e.

- R1-2105277, Nokia, Nokia Shanghai Bell, “Enhancement on CSI measurement and reporting”, RAN1 #105-e.

- R1-2105316, Samsung, “Bandwidth reduction for RedCap UEs”, RAN1#105-e.

- R1-2106077, Lenovo, Motorola Mobility, Ericsson, Intel Corporation, vivo, “On PMI sharing for CSI enhancements under multi-TRP framework”, RAN1#105-e

# Appendix

| Parameter | Value |
| --- | --- |
| Scenario | Dense Urban (Macro only) |
| Frequency Range | 2GHz with duplexing gap of 200MHz between DL and UL |
| Inter-BS distance | 200m |
| Antenna setup and port layouts at gNB | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |
| BS Tx power | 44dBm for 20MHz |
| Simulation bandwidth | 20 MHz for 15kHz |
| Channel model for reciprocity | Alt1: based on Section 5.3 of TR 36.897 |
| CSI-RS overhead | All DL RS overhead is included in the DL throughput calculation |
| Traffic model | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) | ~70% for SU/MU-MIMO with rank adaptation |
| UE distribution | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver | MMSE-IRC as the baseline receiver |
| Feedback assumption | Realistic |
| Channel estimation | Realistic |
| SRS error model and configuration | SRS error model in Table A.1-2 in 36.897 with Δ=9 dB (SRS period, comb, # OFDM symbols, # UEs) = (5ms, 4, 4, 8) |
| Calibration error model | HUL=HUL⋅E=HUL⋅a1ejθ1⋯0⋮⋱⋮0⋯aNejθN Amplitude error (expressed in decibel of x=20log10a) and phase error have normal distribution with 0.7dB and 5 degrees standard deviation, respectively. |