3GPP TSG RAN WG1 Meeting #106-e	R1-2108095

E-meeting, August 16 – August 27, 2021

Agenda Item:	8.15.1

Source:	Guangdong Communications and Networks Institute

Title:	Applicable scenarios to NB-IoT in NTN

Document for:	Discussion

Introduction

The new Study Item-”IoT Non Terrestrial Network” was proposed at the RAN#86, and RAN#91 has revised it [1]. In RAN#91-e, companies agreed to accelerate the study of IoT over NTN. At RAN#91-e GTW, a Way Forward Proposal in [3] on email discussion on [50][New_proposals_approval] has reached to an agreement. This included guidance from RAN Chairman [4] for NTN NR and NTN IoT as follows

- RAN#92E (June) to finalize the scope and project plan to deliver the essential minimum functionality of both NTN NR and NTN IoT (both NB-IoT and eMTC) within the existing TU allocations

- Detailed scoping exercise (NTN NR WID revision, NTN IoT WID approval) to be undertaken at RAN#92E (June)

In this contribution, we provide add a new use case based on [6], and listed the maximum Doppler shift in different orbit height. The high Doppler shift may lead to additional requirement on repetition for IoT UE.

Discussion

Use cases and performance requirement

At RAN#91-e, intermittent delay-tolerant small packet transmission was widely supported. Detailed performance requirements is significant in the intermittent delay-tolerant small packet transmission. In TS 22.261 [2], the performance requirements for satellite access are defined. In [6], the UL and DL data rate for IoT NTN had listed in a Table. However, some of the application scenarios haven’t been concluded.

With the continuous development of marine satellite communication, satellite remote sensing, underwater acoustic telemetry and sensors, marine monitoring has gradually entered the era of three-dimensional monitoring in space, land, sea and underwater. Among them, as a widely used passive monitoring carrier, marine buoy has the characteristics of small volume, easy deployment, all-weather, long-term continuous and fixed-point monitoring. It has been widely used in China's marine stations, marine engineering, marine investigation and other fields, and plays an irreplaceable role in other marine monitoring methods. In the Table 1, we added one use case - ”ocean buoy”.

Proposal 1: Some typical use cases and their performance requirements in IoT over NTN should to be enriched.

Table 1 Performance requirements for satellite access

| Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) (note 1) | Area traffic capacity (UL) (note 1) | Overall user density | Activity factor | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pedestrian (note 2) | [1] Mbit/s | [100] kbit/s | 1,5 Mbit/s/km2 | 150 kbit/s/km2 | [100]/km2 | [1,5] % | Pedestrian | Handheld |
| Public safety | [3,5] Mbit/ss | [3,5] Mbit/s | TBD | TBD | TBD | N/A | 100 km/h | Handheld |
| Vehicular connectivity (note 3) | 50 Mbit/s | 25 Mbit/s | TBD | TBD | TBD | 50 % | Up to 250 km/h | Vehicle mounted |
| Airplanes connectivity (note 4) | 360 Mbit/s/ plane | 180 Mbit/s/ plane | TBD | TBD | TBD | N/A | Up to 1000 km/h | Airplane mounted |
| Stationary | 50 Mbit/s | 25 Mbit/s | TBD | TBD | TBD | N/A | Stationary | Building mounted |
| Narrowband IoT connectivity | [2] kbit/s | [10] kbit/s | 8 kbit/s/km2 | 40 kbit/s/km2 | [400]/km2 | [1] % | [Up to 100 km/h] | IoT |
| Ocean buoy | ≤16kbit/s | 32kbit/s | TBD | TBD | TBD | N/A | Stationary | Ocean |
| Note 1: Area capacity is averaged over a satellite beam. Note 2: Data rates based on Extreme long-range coverage target values in clause 6.17.2. User density based on rural area in Table 7.1-1. Note 3: Based on Table 7.1-1 Note 4: Based on an assumption of 120 users per plane 15/7.5 Mbit/s data rate and 20 % activity factor per user Note 5: All the values in this table are targeted values and not strict requirements. Note 6: Performance requirements for all the values in this table should be analyzed independently for each scenario. |  |  |  |  |  |  |  |  |

Repetitions for improved link budget

In satellite mobile communication system, not only the movement of mobile terminal will cause Doppler effect, but also the movement of satellite will introduce Doppler frequency shift. In geostationary satellite communication system, Doppler frequency shift mainly comes from the movement of mobile terminal; In the non geostationary orbit satellite system, the moving speed of the satellite is much faster than that of the terminal. Therefore, it can be considered that the high-speed movement of the satellite relative to the ground user is the main component of Doppler frequency shift. Besides, as mentioned in [7], “Observation 7: High doppler shift, NTN channel condition and additional loss may lead to additional requirement on repetition for IoT UE.” Thus, Table 2 described the Doppler frequency shift in different orbit height.

Proposal 2: Different orbit altitude will lead to different Doppler shift, and the high Doppler shift should taken into account in link budget.

Table 2 The maximum Doppler frequency shift in different orbit height

| Orbit altitude （/km） | Tangential direction of satellite motion (km/s) | Maximum Doppler frequency shift (fc = 1.6 GHz)（kHz） | Maximum Doppler frequency shift (fc = 30 GHz)（kHz） |
| --- | --- | --- | --- |
| 200 | 7.797 | 39.7 | 744 |
| 400 | 7.681 | 38 | 711 |
| 800 | 7.463 | 34.9 | 654 |
| 1400 | 7.168 | 30.9 | 579 |
| 5000 | 5.924 | 17.6 | 330 |
| 10000 | 4.936 | 10.4 | 195 |
| 20000 | 3.889 | 5.05 | 102 |

Conclusions

In summary, we discuss on the use cases and present the maximum Doppler shift under different orbit altitude. The following proposals are made:

Proposal 1: Some typical use cases and their performance requirements in IoT over NTN should to be enriched.

Proposal 2: Different orbit altitude will lead to different Doppler shift, and the high doppler shift should taken into account in link budget.

- References

[1]RP-210868, “New Study WID on NB-IoT/eTMC support for NTN”, MediaTek, RAN#91-e, March 2021

[2]RP-210915, “Moderator's summary for email discussion [91E][42][NTN_IoT_roadmap]”, Ericsson (RAN1 Vice-Chair), RAN#91-e, March 2021

- [3]RP-210906, Way forward on new proposals, Nokia (RAN Chair), RAN#91-e, March 2021

[4]RAN1#104-bis-e Chairman's Notes.

[5]3GPP TS 22.261, “Service requirements for the 5G system”.

[6]R1-2104258, “Application scenarios of IoT in NTN”, Huawei, HiSilicon

[7]R1-2105404,“Link budget evaluations and deployment for NB-IoT/eMTC over NTN”