3GPP TSG RAN WG1 #106-e		R1- 2108100

E-Meeting, August 16th – August 27th, 2021

Source:	China Unicom

Title:	Initial evaluation results for XR

Agenda Item:	8.14.3

Document for:	Discussion and Decision

Introduction

In RAN1 #104 [1], #104b [2] and #105 [3] e-meetings, basic traffic model and evaluation methodology for XR evaluation has been agreed. Based on the agreements, we simulate the single DL video stream with SU-MIMO.

In this contribution, we provide our evaluation results focus on XR capacity.

Discussion

2.1 Traffic model

In this section, the network deployment was Urban Macro. Statistical traffic model is used for the capacity simulation, and packet arrival interval for each stream is set to 60fps. Both XR and CG services were evaluated, detailed parameters are shown in the following table.

Table 1: Traffic Model Simulation Assumptions

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| Services | AR/VR |  | CG |  |
| Average data rate | 30Mbps | 45Mbps | 30Mbps | 8Mbps |
| Mean Packet size | 62.5K Bytes | 93.75K Bytes | 62.5K Bytes | 16.67K Bytes |
| Packet distribution | Truncated Gaussian Distribution: - STD: 0.105*mean - Max: 1.5*mean - Min: 0. 5*mean |  |  |  |
| Periodicity | 60fps |  |  |  |
| PDB | 10 ms |  | 15 ms |  |
| Jitter | Truncated Gaussian Distribution: - Mean: 0ms - STD: 2ms - Range: 2ms |  |  |  |

2.2 Baseline capacity results

In this section, we present the XR capacity results for FR1 Urban Macro. The system level simulation assumptions are listed in Appendix A. For the definition of a satisfied UE, we use the agreement that more than 99% of packets are successfully delivered within the given air interface PDB. The (PSR, PDB) combinations in this section is (99, 10) for VR/AR, and (99, 15) for CG.

Figure 1. Capacity results of the single-stream DL model

Figure 1 shows the percentage of satisfied UE under the Urban Macro scenario. We use traffic_model - Maen_packet_size - PDB to distinguish the different configurations. As can be seen, UE satisfaction rate gradually declines with the increase of the UE numbers per cell. The system capacity is obtained under the definition that 90% of users being satisfied. It is observed that the capacity of XR system for Urban Macro scenario is listed in Table 2.

Table 2: Capacity results for Urban Macro

| Traffic Model | AR/VR-45 | AR/VR-30 | CG-30 | CG-8 |
| --- | --- | --- | --- | --- |
| Capacity | 4.6 | 5.5 | 7.9 | >30 |

Observation 1： In Urban Macro scenario and single DL video stream with SU-MIMO, the average number of supported users per cell is:

- For traffic model of AR/VR-45Mbps-10ms, XR capacity is 4.6 UEs per cell.

- For traffic model of AR/VR-30Mbps-10ms, XR capacity is 5.5 UEs per cell.

- For traffic model of CG-30Mbps-15ms, XR capacity is 7.9 UEs per cell.

- For traffic model of CG-8Mbps-15ms, XR capacity is above 30 UEs per cell.

Conclusions

According to the discussion, following proposals and observations are provided:

Observation 1：In Urban Macro scenario and single DL video stream with SU-MIMO, the average number of supported users per cell is:

- For traffic model of AR/VR-45Mbps-10ms, XR capacity is 4.6 UEs per cell.

- For traffic model of AR/VR-30Mbps-10ms, XR capacity is 5.5 UEs per cell.

- For traffic model of CG-30Mbps-15ms, XR capacity is 7.9 UEs per cell.

- For traffic model of CG-8Mbps-15ms, XR capacity is above 30 UEs per cell.

# References

- 3GPP RAN1#104-e Chairman’s Notes, February 2021.

- 3GPP RAN1#104bis-e Chairman’s Notes, April 2021.

- 3GPP RAN1#105 Chairman’s Notes, May 2021.

# Appendix A – System level Simulation assumptions

Table A-1. Simulation assumption for FR1

| Parameter | Value |
| --- | --- |
| Scenarios | Urban Macro |
| Layout | 21cells with wraparound |
| Channel Model | UMa |
| Carrier frequency | 4.0 GHz |
| Bandwidth | 100 MHz |
| Subcarrier spacing | 30 KHz |
| Frame structure | [DDDSU DDDSU] Detailed S slot format is 10D:2F:2U. |
| Inter Site Distance | 500m |
| BS Antenna Height | 25m |
| BS Antennas | Option 1: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |
| CSI-RS Port | 32 |
| UE Antenna Height | General equation: hUT=3(nfl – 1) + 1.5 - nfl for outdoor UEs: 1 - nfl for indoor UEs: nfl ~ uniform(1, Nfl ) where Nfl ~ uniform(4,8) |
| UE Antennas | Baseline: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ |
| BS antenna pattern | 3-sector antenna radiation pattern, 8 dBi |
| UE antenna pattern | Omnidirectional, 0 dBi |
| TX Power | BS : 49 dBm per 20MHz |
| UE MAX Power | 23dBm |
| Noise Figure | BS:5 dB, UE:9 dB |
| Scheduler | SU-MIMO Proportional Fair |
| MCS | Up to 64QAM |
| UE distribution | 80% indoor, 20% outdoor |
| UE speed | 3 km/h |
| Channel Estimation | Ideal |
| Downtilt | 12 degree |
| CSI acquisition | CSI-RS: Period=5ms, Density=1, feedback delay=1ms, 4bit quantization, and modelling the error SRS: Period=5ms |
| Transmission scheme | Close loop rank adaptation |
| PHY processing delay | UE PDSCH processing Capability #1 |
| PDCCH overhead | 2 symbols |
| Target BLER | 10% |
| Max HARQ transmission | 4 |