3GPP TSG RAN WG1 #106-e                                              R1-2108280

e-Meeting, August 16th – 27th, 2021

Source: 	Moderator (Nokia)

Title: 	Feature Lead Summary #1 for Potential multipath/NLOS mitigation

Agenda item:	8.5.5

Document for: 	Discussion and Decision

# Introduction

In the WID, [1], for ePos the following objective was added at RAN#91:

- Study and specify, if agreed, the enhancements of information reporting from UE and gNB for multipath/NLOS mitigation [RAN1, RAN2, RAN3]

In this contribution, we provide a summary of the potential enhancements for information reporting from UE and gNB for multipath/NLOS mitigation proposed by companies in contributions [2]-[22]. We also make some initial proposals to facilitate RAN1 discussion. This document provides the summary of the following email discussion in RAN1#106-e:

[106-e-NR-ePos-05] Email discussion/approval on potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation with checkpoints for agreements on August 19, 24 and 27 – Ryan (Nokia)

Overview of proposals in contributions

The following list of proposed enhancements/areas was identified based on submitted contributions [2]-[22]:

- LoS/NLoS Indicators

- Indicators Values

- LoS/NLoS Identification methods

- Multipath reporting enhancements

- UE to LMF multipath reporting enhancements

- gNB/TRP to LMF multipath reporting enhancements

- UL-AoA Related Topics

- DL-AoD Related Topics

- Others

Issues for discussion

## Issue #1: LoS/NLoS Indicator

One issue discussed by many companies is the introduction of an indicator of a LoS/NLoS indicator. These indicators could be reported by the UE and/or TRP to the LMF when making positioning measurement reports. During RAN1#105-e the following agreement was reached:

Agreement:

- Study reporting of LoS/NLoS indicators for DL, UL, and DL+UL positioning measurements taken at both UE and TRP at least for UE assisted positioning.

- Study the following options (or combinations of the following options) for LoS/NLoS indicators

  - Option 1: Binary (i.e., hard) value indicators

  - Option 2: Soft value indicators (i.e., [0,1]).

    - FFS: Format and criteria for determination

  - FFS: additional information or options

- FFS: LoS/NLoS indicators for UE-based positioning

In this section we list the specific proposals from other companies related to this agreement and in particular related to the first bullet. Some other proposals seem to indicate that the company is also supportive of indicators while other companies have observations in which they appear opposed to indicators. We list the most relevant proposals here:

- [2]

  - Proposal 1:  Support UE to report the LOS/NLOS indicators of the measurement results for the DL and DL+UL positioning methods.

  - Proposal 2:  Support gNB to report the LOS/NLOS indicators of the measurement results for the UL and DL+UL positioning methods.

- [4]

  - Proposal 3: The validity of LoS/NLoS indicators with different LoS detection methods should be studied further.

  - Proposal 4: How LMF utilizes the reported measurements associated with LoS/NLoS indicators needs to be further studied.

- [5]

  - Proposal 1: Support UE positioning measurement report with LOS/NLOS identification. It can be reported as soft value indicators (i.e., [0,1]).

- [8]

  - Proposal 1: Support LoS/NLoS indicator reporting from the UE/TRP to the LMF during at least UE-A positioning. Details of the indicator (e.g., binary/soft) can be FFS.

- [9]

  - Proposal 1: To improve positioning accuracy by regularization techniques, use of LOS indicators as soft values for each link for UE-assisted and UE-based positioning should be supported.

  - Proposal 1a: For UE-assisted positioning, the determination of the LOS indicator can be done at the gNB/LMF or at the UE. For the latter, the LOS indicator is additionally reported back to the gNB/LMF.

  - Proposal 1b: For UE-assisted positioning, at least support the baseline case where needed measurements are reported back to the gNB/LMF for the determination of the LOS indicator at the gNB/LMF. The algorithm to compute the LOS indicator hence becomes a matter of network implementation.

  - Proposal 1c: For UE-based positioning, the LOS indicator can be signaled to the UE. The UE can request the LOS indicator for specific path(s) associated with a gNB or TRP to be signaled to the UE by the LMF/gNB.

- [14]

  - Proposal 1: Support reporting of LOS/NLOS indicator for DL, UL, and DL + UL positioning measurements taken at both UE and TRP in the following format:

    - Soft value indicator distributed in the range [0, 1] showing the probability of NLOS link detection, where 0 indicates pure LOS channel, and 1 indicates pure NLOS channel

- [17]

  - Proposal 1: Support introducing binary value LOS/NLOS indicator.

- [19]

  - Proposal 4: LMF provides the UE with information on the channel model or channel parameters to enable channel state identification.

- [20]

  - Proposal 2: Support UE reporting of RSTD, UE Rx-Tx time difference and/or DL-PRS RSRP associated with Option 1 (binary metrics) or Option 2(soft metrics) LOS/NLOS indicators on a per beam basis.

  - Proposal 3: Specify a separate LOS/NLOS measurement window, where the applicable LOS/NLOS positioning measurement is considered valid. This motivation of the LOS/NLOS measurement window is separate from the measurement timestamp reporting in AI 8.5.1. FFS further details such as length of the measurement window(s), configuration of periodic/aperiodic measurement window(s).

- [21]:

  - Proposal 1: Support NLOS/LOS detection mechanisms in release 17

### Round #1 Discussion

Feature Lead View

Many companies seem interested in introducing LoS/NLoS indicators calculated by the measuring nodes (UE/TRP) and reporting to LMF as part of location reports. Some companies propose additional details related with these indicators and those are discussed in subsequent sections. There are some companies that seem to feel that other solutions (e.g., multipath reporting) appear better in some scenarios. With that in mind the follow proposal may be a good starting ground:

Proposal 1.1

- Support LoS/NLoS indicators which are reported for DL, UL, and DL+UL positioning measurements taken at UE or TRP, at least for UE assisted positioning.

  - FFS: UE based positioning

Companies views:

| Company Name | Comments |
| --- | --- |
| Qualcomm | Not support |
| Huawei | Support. LoS/NLoS indicaotor reporting does not implicitly means that the implementation-based methods would not be used, such as RAIM/RANSAC. Actually, in reality, it can be jointly utilized with the RAIM/RANSAC. In addition, LoS/NLoS indicator does not implicitly excludes the NLOS measurements, but provides the assistant information that how much this link is like to a LoS or NLoS. Otherwise, why would the NLOS measurement be reported if it is flagged as NLOS. |
| vivo | We are not in favour of this proposal before we study it clearly. As mentioned in our contribution, the correct rate of LoS detection is dependent on the scenario and LoS identification methods, it is better to clarify the effective LoS detection methods correspond with different LoS/NLoS information reporting, and evaluate the corresponding validity of LoS/NLoS identification before we decide which LoS/NLoS information can be reported. In addition, how LMF utilizes the reported measurements associated with LoS/NLoS indicators when calculating the position is also a problem. Would the NLoS measurements be discarded or mitigated, or would they also be used? It is difficult for us to standardize ‘LoS/NLoS indicator’ before the above issues have been studied clearly. |
| China Telecom | Support. |
| Intel | Support. Agree with the comments provided by Huawei. |
| Xiaomi | Support. |
| ZTE | Support. Fully agree with Huawei’s comments. In addition, we think how UE/gNB derives the NLOS indicator should also be reported to LMF. Otherwise, how RAN4 can design the test case for the NLOS indicator? For example, UE/gNB reports a NLOS indicator, UE should also indicates which channel parameter has been used to derive the NLOS indicator. The channel parameters can be Ricean factor, coherence bandwidth or other parameters as mentioned in Issue#3. We would like to add another FFS, FFS: The report should be associated with at least a channel parameter that is used to derive the NLOS indicator |
| SONY | Support and we have similar view as Huawei. |
| CATT | Support. We share the same view with Huawei and Intel that LoS/NLoS indicaotor can be jointly utilized with the RAIM/RANSAC. The use of reporting of LOS/NLOS indication information together with RANSAC algorithm can improve significantly the positioning accuracy with given computational complexity. As shown in the figure below, - CASE 1 (Baseline): selecting 4 TRPs of minimum TOA error from 18 TRPs. - CASE 2: Pre-selecting 8 TRPs according to LOS identification information and processing with RANSAC(C84). It is shown that the positioning error for CASE2 is 0.63m which is close to the baseline. It demonstrates that the proposed “joint LoS/NLoS indicaotor + RANSAC” algorithm mitigates most of NLOS influence and almost obtains the required accuracy (detailed simulation information can be obtained from our contribution R1-2106975). |
| Huawei, HiSilicon2 | To vivo: Regarding the comment from vivo: As mentioned in our contribution, the correct rate of LoS detection is dependent on the scenario and LoS identification methods, it is better to clarify the effective LoS detection methods correspond with different LoS/NLoS information reporting, and evaluate the corresponding validity of LoS/NLoS identification before we decide which LoS/NLoS information can be reported. As vivo and many other companies have evaluated, by LoS/NLoS detection the positioning accuracy could be effectively improved. In addition, the correct detection rate could achieve up to 99%. Regarding the comment from vivo: In addition, how LMF utilizes the reported measurements associated with LoS/NLoS indicators when calculating the position is also a problem. There may be multiple ways how LMF could ulitize the NLoS indicator, and it is LMF implementation. - Method #1: The measurments even with low LoS probability can be used via labeling different measuremnts with different weight factors based on the LoS probabilities. - Method #2: The LMF could use the NLoS indicator as the processing priority indictor for a TRP, which can be jointly used for RAIM algorithm. For example, the TRP measurement with higher probability would be selected first and cross-check whether the less LOS-likely TRPs should be added in the location fix. - Method #3: The LMF could use the measurement associated with lower LOS probability in case there is a small number of TRPs available for a location fix, and also provide the location fix with a larger uncertainty. We do not understand why this is problematic.. |
| Nokia/NSB | Support. Agree in general with Huawei. To vivo, it is obvious that NLOS is a huge problem for positioning. It has been studied for decades in the literature and is clear from simulation/real world data. The error rate is clearly dependent on the environment and specific method but to simply not do anything is not acceptable to us. |
| OPPO | We share the same view as QC and vivo and we do not support this proposal. Indeed NLOS is a big trouble for positioning. However, reporting “NLOS indicator” does not help the current positioning methods since all the positioning so far is based on the distance or transmission time assumption. Instead of reporting the identification of NLOS to system, a better method for a UE is to deal with the NLOS at the UE side by implementation. Particually, we share the same understanding as vivo, it seem not possible to define the LOS indicator technically. For instance, if we use one bit value to indicate whether it is LOS or NLOS. If a UE reports a NLOS, what the LMF would do? Discard the corresponding results or use it? |
| Fraunhofer | Support. LOS/NLOS enables the LMF to weigh the multiple received measurements. The realibility of the LOS/NLOS indicators depends on the applied method (TRP/UE implementation) and required AD for an effective channel state identification. |
| Huawei, HiSilicon | To OPPO: May I understand the following contents as to only say for UE-based methods, it is not needed to report NLOS indicator? How about UE assisted case? How about UL methods? Instead of reporting the identification of NLOS to system, a better method for a UE is to deal with the NLOS at the UE side by implementation. For the following comments: it seem not possible to define the LOS indicator technically. For instance, if we use one bit value to indicate whether it is LOS or NLOS. If a UE reports a NLOS, what the LMF would do? Discard the corresponding results or use it? Have we defined RSTD measurement quality? Even for location fix report, UE could report the uncertainty shape and confidence, why would that be possible? For using the NLoS indicator, I think in the previous comments, companies already expressed how LMF could use the soft value. I do not think it is a constructive comment to simply repeat the same thing (which has already been addressed) again and again. |
| LG | We fully agree with the intention of reporting LoS/NLoS indicators and we only support the proposal under the condition that implementation-based methods are used to derive it. As many other companies have been introduced their own’s algorithms, we have concern about how to compare them effectively in the short time available. We prefer that there is no spec impact except for LoS/NLoS indicator. |
| Futurewei | In our view, an implementation-based approach (e.g. outlier rejection) at the UE for UE-based positioning is comparable to using an equivalent ‘soft indicator/algorithm’ at the UE to reject a specific measurements (e.g. RSTD) due to NLOS degradation , as an example. The specific approach here of course is implementation choice. We need to extend this capability also for UE-assisted positioning where positioning is computed at the LMF. One approach is to provide the gNB/LMF a similar ‘soft indicator’ e.g. a [x]-bits feedback to the gNB/LMF to support UE-assisted positioning. Hence, we have specs support for UE-assisted positioning as well, and not just an implementation approach that works for UE-based positioning. Perhaps a way to achieve middle ground and make some progress would be that for now we generalize this indicator as provide some ‘quality measurements’ that includes at least form for of indication of LOS or NLOS feedback, instead of stating now that it must be a LOS/NLOS indicator. However, prior to that, we would like to understand better the position taken by companies that think implementation-based UE approach is already sufficient: - Do you think such approach to address NLOS effects should be extended also to UE-assisted positioning where positioning is computed at the LMF and currently we don’t have means to support this? - If no, can you explain why not? If yes, please share some description of your proposed approach. |
| Lenovo, Motorola Mobility | Support, these indicators, essentially serve as additional assistance information to the LMF (UE-assisted) and can be complementary to the implementation-based approaches as highlighted by Huawei. |
| Ericsson | For the LMF, the (soft) indicator solution could be a complement to multiplath reporting, to enable LOS identification at the LMF. However it is not enough to rely on the gNB/UE implementation to produce a LOS/LNOS indicator. The LMF can fuse all measurements from both UE and gNB to extract the LOS/NLOS information. Therefore we think the priority is to enable LOS detection at the LMF, and if time allows specify LOS/NLOS indication to the LMF from gNB/UE To FL The proposal quoted from our contribution [7] was in support of enabling NLOS/LOS detection in general, but did not refer to the NLOS/LOS indication solution. Thus it can be removed from the list of supporting proposals for issue #1. |
| vivo2 | Actually, companies propose LOS/NLOS enables the LMF to weigh the multiple received measurements in the implemation algorithm. But we would like to confirm whether the path RSRP will be introduced in Rat dependent methods. If it is, it is natural to use path RSRP as weight instead of using an indication that performance cannot be guaranteed. Otherwise, we should get the conclusion first that no path-RSRP is needed in R17 except AoD positioning. |

## Issue #2: Indicator Values

One issue discussed by many companies is the value of LoS/NLoS indicators if introduced. During RAN1#105-e the following agreement was reached:

Agreement:

- Study reporting of LoS/NLoS indicators for DL, UL, and DL+UL positioning measurements taken at both UE and TRP at least for UE assisted positioning.

- Study the following options (or combinations of the following options) for LoS/NLoS indicators

  - Option 1: Binary (i.e., hard) value indicators

  - Option 2: Soft value indicators (i.e., [0,1]).

    - FFS: Format and criteria for determination

  - FFS: additional information or options

- FFS: LoS/NLoS indicators for UE-based positioning

In this section we list the specific proposals from other companies related to this agreement here:

- [2]

  - Proposal 3:  Support the LOS/NLOS indicator is reported with soft value between 0 and 1.

- [5]

  - Proposal 1: Support UE positioning measurement report with LOS/NLOS identification. It can be reported as soft value indicators (i.e., [0,1]).

- [6]

  - Proposal 1: The NLOS/LOS indicator is a soft decision (i.e., [0,1]).

- [9]

  - Proposal 1: To improve positioning accuracy by regularization techniques, use of LOS indicators as soft values for each link for UE-assisted and UE-based positioning should be supported.

- [10]

  - Proposal 6: Binary value should be used for LOS/NLOS indicators for both UE-A and UE-B positioning methods.

- [13]

  - In order to increase the positioning accuracy, LOS-likelyhood value (option 2) can be reported in conjunction with positioning measurement.

    - UE does not necessarily to report positioning measurement (e.g., RSTD(s), UE Rx-Tx time difference, and etc.) corresponding to certain TRP determined with NLOS.

- [14]

  - Proposal 1: Support reporting of LOS/NLOS indicator for DL, UL, and DL + UL positioning measurements taken at both UE and TRP in the following format:

    - Soft value indicator distributed in the range [0, 1] showing the probability of NLOS link detection, where 0 indicates pure LOS channel, and 1 indicates pure NLOS channel

- [17]

  - Proposal 1: Support introducing binary value LOS/NLOS indicator.

- [18]:

  - Proposal 2: Slightly prefer Option 2 with soft value indicators for LoS/NLoS identification if no other assisted information is specified.

  - Proposal 3: Support to select hard LoS/NLoS indicator or assisted information to be reported to LMF for LoS/NLoS identification depending on the probability of LoS/NLoS.

- [19]:

  - Support a soft value for the channel state indicator according to option 2. The channel state indicator shall include following information:

    - Channel state: LOS, NLOS

    - Confidence: an indication of the confidence in the provided channel state

    - FAP Quality:  an indication of the quality of the FAP (OLOS)

    - Secondary channel state (optional): LOS, NLOS

    - Secondary channel confidence (optional): an indication of the confidence in the secondary channel state

    - Occurrence (optional): provides the percentage that a channel state occurred over a period of time. This field is present for the case the channel state is not reported per measurement (i.e. reported for multiple measurements).

- [20]

  - Proposal 1: Consider the use of an adaptive threshold to enable LOS/NLOS classification using either Option 1: Binary indicators and/or Option 2: Soft metrics.

  - Proposal 2: Support UE reporting of RSTD, UE Rx-Tx time difference and/or DL-PRS RSRP associated with Option 1 (binary metrics) or Option 2(soft metrics) LOS/NLOS indicators on a per beam basis.

  - Proposal 3: Specify a separate LOS/NLOS measurement window, where the applicable LOS/NLOS positioning measurement is considered valid. This motivation of the LOS/NLOS measurement window is separate from the measurement timestamp reporting in AI 8.5.1. FFS further details such as length of the measurement window(s), configuration of periodic/aperiodic measurement window(s).

- [22]:

  - Proposal 1: The indication of soft value in interval [0, 1] should be reported from UE to the LMF for all the positioning methods.

### Round #1 Discussion

Feature Lead View

Most companies seem interested introducing “soft” LoS/NLoS indicators. With that in mind the follow proposal may be a good starting ground:

Proposal 2.1

- If LoS/NLoS indicators are supported in Rel-17, their value is in the interval [0, 1].

Companies views:

| Company Name | Comments |
| --- | --- |
| Huawei | Support. Soft value provides more degree of freedom such that the LMF would be aware of which link has higher LoS/NLoS probability. On the other hand, it could be left to the LMF to do the hard decision. |
| vivo | This proposal can be further discussed if conclusion of Proposal 1.1 is achieved. |
| China Telecom | We prefer the LoS/NLoS indicator in binary value, and the assistant information indicating the uncertainty of the indicatior can be reported at the same time. Since the identification methods for can be implement based, it’s hard to find a criteria for deciding the specific soft value for all the methods. However, we are also fine with the softvalue in the interval [0, 1] if a common criteria can be reached. |
| Intel | Support |
| Xiaomi | Support. Soft value can provide more information about the probability of the LoS/NLoS. |
| ZTE | Support. |
| SONY | Support |
| CATT | Support. The soft value can provide more information to LMF than the hard decision. |
| Nokia/NSB | Support |
| OPPO | Have concern on this proposal because it is not possible to standardize soft value. What does it mean if UE reports 0.6? Different UE might implement different method to calculate such a value and there seems not possible for RAN4 to test it. |
| Fraunhofer | Support. |
| Huaweei, HiSilicon | To OPPO: How about the quality of RSTD/Rx – Tx time difference/UL RTOA/UL AOA? |
| LG | We have the same view with vivo. we think that the proposal varys depending on the result of Proposal 1.1. So, we prefer to deal the proposal 2.1 as a lower priority than proposal 1.1. |
| Futurewei | Support and also agree that 2.1 is something that comes after 1.1 |
| Lenovo, Motorola Mobility | Support, however it would be beneficial to have further clarity on whether separate indicators for LOS and NLOS will be signalled. |
| FL | To Lenovo, others, The intention of the proposal in my understanding is that a single indicator is reported and that 1 = LOS and 0 = NLOS. To OPPO, The intention in my understanding is that e.g. 0.6 would mean that the UE feels it is 60% chance the measurement/link is LOS. Same understanding as Huawei that a soft value has zero impact on RAN4 tests. There many parameters that fall into this category. |
| NTT DOCOMO | Considering LoS/NLoS detection method is up to UE/TRP, binary value may be enough to report LoS/NLoS status. However, we can accept the proposal (i.e. introducing soft value indicator). |
| Samsung | Generally fine. But we feel this indication should be together with UE capability, not only on whether UE can report the identification; but also on whether UE can only report the hard decision or soft decision, e.g., for some UE with certain alrgorthim, it may only derive 0, 1 without reliable soft probability. So when LMF gets the value, e.g., 1; LMF needs to know the “1” means the UE is fully confident or the UE is just capable of 0, 1. We believe there will be difference for LMF to treat the value. |

## Issue #3: LoS/NLoS identification methods

One issue discussed by many companies is which LoS/NLoS identification method(s) should be used. During RAN1#105-e the following agreement was reached:

Agreement:

As part of studying LoS/NLoS information reporting, study at least the following options for information to enable/assist LoS/NLoS detection:

- Option 1: Polarization information reporting from UE/gNB to LMF.

- Option 2: Coherence bandwidth information reporting from UE/gNB to LMF.

- Option 3: Propagation time difference information reporting from UE/gNB to LMF.

- Option 4: RSRP reporting from UE/gNB to LMF with finer granularity

- Option 5: Ricean factor and the variance of Channel Frequency Response (CFR) information reporting from UE/gNB to LMF

- Option 6: No specification impact outside of LoS/NLoS reporting

Note: Companies are encouraged to identify differences in information reporting and any performance gains compared with multipath information reporting

In this section we list the specific proposals from other companies related to this agreement here:

- [2]

  - Proposal 4:  The LOS/NLOS identification methods are left up UE/gNB implementation.

- [3]

  - Proposal 1：Support UE /TRP to report coherence bandwidth information to LMF for NLOS mitigation, where the coherence bandwidth information can be hard indicator, soft indicator or measurement reporting derived from coherence bandwidth.

- [4]

  - Implementation-based solution should be considered to solve NLoS problems.

  - Before we decide which LoS/NLoS information can be reported, verify the effectiveness of LoS detection methods correspond with different LoS/NLoS information reporting.

  - Whether different LoS identification method should be applied to different positioning method should be confirmed.

- [5]

  - Proposal 2: The method/techniques of obtaining LOS/NLOS identification is left for UE implementation

  - Proposal 3: Support gNB to provide gNB/TRP antenna polarization to LMF and subsequently, LMF to provide such information to UE.

- [6]

  - Proposal 3: For LOS/NLOS information reporting, adopt option 6, i.e., there is no specification impact outside of LoS/NLoS reporting.

- [7]

  - Proposal 1: For the LOS/NLOS identification, support UE/gNB to report at least the Ricean factor and the variance of Channel Frequency Response (CFR).

- [8]

  - Proposal 2: How a UE/TRP determines a LoS/NLoS indicator is left to implementation (i.e., option 6 of prior agreement).

  - Proposal 6: RAN1 to study LOS/NLOS identification methods computed in LMF localization processing.

  - Proposal 7: Introduce LoS/NLoS identification assistance information for both DL and UL channels from LMF to UE/gNB.

    - Define both NRPPa and LPP messages

    - Format of the assistance information/status report is FFS (i.e. hard bit indication, quality metric or probability)

    - PHY/L1 can monitor CIR corresponding to the assistance information and report it back to LMF optionally.

    - Support the LoS/NLoS identification for both UE-based and UE-assisted positioning methods.

- [10]

  - Proposal 1: The K factor derived from RSRP of multipath should be used for LOS/NLOS identification.

  - Proposal 2: The angle, relative timing of additional paths can be used for LOS/NLOS identification as auxiliary information.

- [11]

  - Proposal 1: Rel-17 supports implementation-based solutions for NLOS mitigation.

- [13]

  - For LOS/NLOS identification, RAN1 needs to consider at least following method based on:

    - Polarization characteristic

    - Propagation time difference threshold/window between a reference and a target TRP.

- [15]

  - Proposal 1: Support Option 4 (RSRP reporting from UE/gNB to LMF with finer granularity) to enable/assist LoS/NLoS detection

- [19]

  - Proposal 4: LMF provides the UE with information on the channel model or channel parameters to enable channel state identification.

- [21]

  - Following measurements should be specified in Rel-17 to support signature-based methods. These measurements can be part of rich reporting.

    - Delay and magnitude of the first peak.

    - Delay and magnitude of the highest peak.

    - Components of PDP/CIR around first/highest peak.

  - A LOS/NLOS indicator reported by a UE/TRP should be coupled with an information element that states what kind of measurement or analysis that was used to obtain it.

### Round #1 Discussion

Feature Lead View

Summary of support for the various options is:

- Option 1: LGE, Sony(?)

- Option 2: ZTE

- Option 3: LGE

- Option 4: IDC

- Option 5: CATT, China Telecom

- Option 6: Huawei, Sony, Samsung, Nokia, Intel

There is also a proposal from Ericsson to include in the LOS/NLOS indicator report information about method used. These details could be discussed during UE capability signaling or after making progress on indicators in general. We have only proposed to continue discussion on the options which received at least two companies support in the contributions. With that in mind the follow proposal may be a good starting ground:

Proposal 3.1

- Down select between the following alternatives for LoS/NLoS detection method:

  - Alternative 1: Support Ricean factor and the variance of Channel Frequency Response (CFR) information reporting from UE/gNB to LMF

  - Alternative 2: Support polarization information reporting from UE/gNB to LMF.

  - Alternative 3: No specification impact outside of LoS/NLoS indicator reporting.

Companies views:

| Company Name | Comments |
| --- | --- |
| Qualcomm | Alternative 3 |
| Huawei | Support Alt. 3. The LoS/NLoS detection is performed at UE (DL), TRP (UL), or both UE and TRP (DL+UL), the specific method is up to the UE/TRP implementation without spec impacts. |
| vivo | We are okay to study to down select associated LOS/NLOS identification method(s) based on the error rate. But we have some concerns about specific information reporting (especially for polarization information). In addition, in our evaluation, the correct rate of LoS detection varies with the scenarios. Especially in the DH scene, the accuracy deteriorates seriously. Whether need RAN4 involvement to guarantee the worst performance? |
| China Telecom | We proposed the Alt 1in our contribution, but we are fine with Alt 3. The implementation based method can be more flexible according to the scenario. |
| Intel | Alt. 3. Support soft value indicator, [0, 1]. |
| Xiaomi | Support Alt 3 |
| ZTE | Although we may not get consensus to support measurement reporting. We want to emphasize that measurement reporting is superior to NLOS indicator in following aspects, - The NLOS indicator mean UE/gNB has to scale or normalize the measurement results locally, which assumes that the measurement result is linear distribution. However, some of channel parameters, for example Ricean factor, which normally belongs to lognormal distribution. - Measurement reporting scheme has the best performance among the three LOS identification schemes since priori statistical information of the channel parameter (e.g. the distribution of the channel parameter) can be utilized. |
| SONY | We consider there is no spec impact on how the UE performs the calculation. However, the UE can be assisted. Hence, we propose Alt.4: Assistance information on polarization information to facilitate LOS/NLOS indicator reporting is provided to UE. Here, UE can be assisted to facilitate LOS/NLOS indicator reporting (for DL-based positioning). The way how UE compute/calculate LOS/NLOS is UE implementation. |
| CATT | We support Alt.1. According to our simulation results, using Alt.1 method in InF-DH scenario, the success rate of selecting 4 LOS TRPs out of 18 TRPs is close to 96% based on NLOS indication information derived from the Ricean factor and the variance of Channel Frequency Response (CFR) information reporting. If we choose Alt.3 and without specification supporting, we worried about the performance will degrade much. In our point of view, proper UE/gNB reporting to LMF will benefit the performance of NLOS identifications. Table 1: Simulation result of LOS identification probability (ρ) CASEs Scenarios; Identification Probability (selecting 4 TRPs from 18 TRPs). InF-SH; 100%. InF-DH; 96%. |
| Nokia/NSB | Support Alt. 3. |
| OPPO | Alt.3. How to detect/identify NLOS and how to suppress the impact of NLOS is up to UE implementation. |
| Fraunhofer | We support a different proposal, subject to progress on issue 1: - Support LMF to provide the UE with information to enable LOS/NLOS idincators reporting. - FFS: Details on the information (channel model, channel parameters …) |
| LG | In the previous meeting, we already agreed to FFS about various options as shown above. But, we think that some of options are not reflected in this proposal. Frankly, even though we prefer to preserve the option 3 into the proposal, we also agree with Alt.3 for the smooth progress. |
| Futurewei | Alt 3. |
| Lenovo, Motorola Mobility | Support Alt 3. |
| FL | To LG, The options included in the proposal are only the ones that received at least 2 companies supporting it in contributions. We have to downselect in my understanding and this seems a reasonable way to do so. To vivo, I don’t understand how your comment impacts the proposal. Could you elaborate? Clearly companies looked at the error rate of the different techniques when they decided which ones to support. The specific rate depends on the environment but I am not clear how that has any impact on the potential specification impact of different methods. |
| Ericsson | Alt 3 |
| NTT DOCOMO | Support Alt.3. We think Los/NLoS detection method can be left to UE/TRP implementation. |
| vivo | To FL We mean UE doesn't know where it is, but error rate of Los detection for some methods depends on where it is (e.g. error rate varies for some methods in SH and DH scenario). Especially in DH scenario, the R17 targect may not be achieved based on some methods. So we are worried whether the UE can choose a suitable NLOS identification method without the information about its environment or scenario. |

## Issue #4: Enhanced multipath reporting

Many companies also propose enhancing the reporting from UE/TRP during positioning sessions in order to enable the LMF to determine the LoS/NLoS status or to perform outlier rejection. Some companies seem to view this as an alternative to UE/TRP reporting LoS/NLoS indicators while other companies support both enhancements. At RAN1#

Agreement:

- Study multipath reporting enhancements for DL, UL, and DL+UL positioning to enable LoS/NLoS/multipath identification and mitigation at the LMF for UE-assisted positioning.

  - FFS: Details of the enhancements.

Some specific proposals are included in other sub-sections, in particular those aimed at specific positioning methods, while other more general ones are captured here:

- [2]

  - Proposal 8:  Extend the number of additional paths for a measurement to 8.

- [4]

  - For multipath reporting enhancements to enable LoS/NLoS identification, the method and effectiveness to do LoS/NLoS identification with multipath information should be confirmed firstly.

- [8]

  - Proposal 3: Support NLOS/multipath enhanced reporting from the UE/TRP to the LMF to enable the LMF to calculate the probability of NLOS.

  - Proposal 4: Support multipath reporting enhancements from TRP to LMF of angle, timing, and power for the additional N paths. FFS value of N.

- [21]

  - For rich multipath reporting, it shall be unambiguously defined what additional paths a UE shall report.

### Round #1 Discussion

Feature Lead View

Overall, many companies seem to propose additional reporting elements as part of UE/TRP reports to LMF which will enable the LMF to determine LoS/NLoS status and/or perform outlier rejection. The specific elements may need more time to converge and the details could potentially be discussed as part of some subsequent proposals in this document.

- Proposal 4.1

- Multipath reporting enhancements for DL, UL, and DL+UL positioning to enable LoS/NLoS/multipath identification and mitigation at the LMF are supported, for at least UE-assisted positioning.

  - FFS: Details of the enhancements.

Companies views:

| Company Name | Comments |
| --- | --- |
| Qualcomm | Generally supportive, but it appears to me that it is unclear how this proposal interacts with the detailed proposals shown in Section 3.5 and 3.6. |
| Huawei | In general, we think that multi-path reporting is beneficial. On whether to use it for NLOS identification could be up to LMF. |
| vivo | It is unclear for us since 3 paths reporting have been supported. For us, the benefit of more than 3 path reporting is limited in the equation-based positioning methods since no matter how many paths are reported, only one best path is helpful for a TRP in the equation. |
| China Telecom | Support in general. The details need to be further discussed. |
| Intel | We suggest to discuss more detail proposals provided below. In our view this agreement is not necessary. |
| Xiaomi | We suggest to discuss the details first. |
| ZTE | The proposal is too general. Prefer to discuss the issue one by one in the following sections. |
| SONY | It is too high level, not sure if we need to make such agreements. |
| CATT | We prefer to discuss the details of enhanced multipath reporting in section 3.5 and 3.6. |
| Nokia/NSB | Support. Okay to directly discuss more details agreements if that is the preference from companies. |
| OPPO | Support in principle |
| InterDigital | We support the FL’s proposal. |
| LG | We generally agree to the intention of the proposal. But, since the proposal looks so general, we think that more detailed descriptions need to be added. |
| Lenovo, Motorola Mobility | Ok to discuss |
| FL | Okay, seems like consensus is to take the detailed proposals directly. I propose to close discussion on this issue then. |

## Issue #5: UE to LMF multipath reporting enhancements

Many companies discuss multipath reporting enhancements from the UE to the LMF. The relevant agreements from RAN1#105-e are copied here and the proposals in company contributions are copied below it.

Agreement:

For multipath reporting enhancements, study reporting from UE to LMF, relative timing of additional paths (additional to the first path) and the power (at least relative power) at least per DL PRS resource per additional path for at least DL-AoD reporting (the number of paths is part of the study).

Agreement:

- Study whether to support up to N>2 additional paths in the measurement reports from UE to LMF for at least DL-TDOA and multi-RTT,

  - FFS: Exact value of N.

  - FFS: reporting the power of the paths in addition to the timing.

  - FFS: LMF requesting additional M non-distinct paths corresponding to the first path.

- Note 1: This agreement applies to N additional paths (i.e., not including the “first” path).

- Note 2: Rel-16 supports N=2 already.

RAN1#106-e company proposals:

- [2]

  - Extend the number of paths for a measurement to 8 and support reporting for each path

    - TOA information defined relative to the first path (only for the additional paths)

    - A Rx beam index

    - A list of path powers measured from different PRS resources for the path measured via the Rx beam indicated by the Rx beam index

  - Proposal 9:  Support the following reporting of each path

    - Single TOA Information (relative TOA to first path for additional path)

    - Single power information  (relative strength to first path for additional path)

    - Phase information

  - Proposal 10:  Support the CIR reporting (including phase and amplitude) with non-distinctive paths only associated with the first path.

- [3]

  - Proposal 2: Postpone the discussions on path-specific RSRP of additional paths and timings of N>2 additional paths

- [4]

  - N=2 additional paths in the measurement reports from UE to LMF are enough at this stage for positioning.

- [6]

  - Proposal 2: Support the indication of the uncertainty range of the first arrival LOS path to UE/TRP.

- [7]

  - Proposal 2: Support UE and/or gNB to report the multipath information measured from multiple antennas for multipath/NLOS mitigation. These multipath information may include the CIR, CFR, pseudo-spectrum, etc., obtained from the first N>=1 most powerful paths of each antenna.

- [8]

  - Proposal 5: Support multipath reporting enhancements from UE to LMF of relative timing of additional paths for N> 2 and the relative power of those paths.

- [10]

  - Proposal 8: UE should report angle, timing and power(RSRP) per DL PRS resource per additional path to LMF at least for DL-AoD.

- [11]

  - Proposal 2: In DL-AoD measurement reporting, the UE reports the relative time-of-arrival of each reported DL PRS resource in addition to the RSRP report.

  - Proposal 3: In DL TDOA and Multi-RTT measurement report, the UE reports the RSRP measurement of each reported additional path and the maximum number of additional paths remains to be N = 2.

- [12]

  - Proposal 1: Support a UE to report to the LMF additional time-domain paths (beyond 2 paths which is already specified) and their corresponding relative powers associated to a PRS resource

    - Applicable to both RTT and DL-TDOA methods

    - Support at least [8] total paths to be provided per PRS resource

- [14]

  - Support relative power PR in [dBm] and phase φ measurement in [deg] reporting associated with the relative time delay of additional path with the total number of additional paths equal to N (N > 2) for DL, UL, and DL + UL positioning methods:

    - PR,n = 10 × lg(Pn/P) indicates the relative power measurement in [dBm] for the nth path, where Pn is the power of the nth path, P is the total receive power corresponding to the measured DL PRS-RSRP or UL SRS-RSRP value within the considered measurement frequency bandwidth

      - The PR,n parameter is defined in the finite range [-X, 0], where X defines some threshold in [dBm] identifying the level of sensitivity for receive power

      - The granularity (or step size) of the measured PR,n parameter is set equal to Y in [dBm], identifying the sufficient level of accuracy, required in the measurements

    - φ indicates the phase measurement in [deg] for each additional path, that can be defined with the resolution of 1 [deg] or 0.1 [deg]

    - FFS: X in [dBm], Y in [dBm] (up to RAN4 discussion)

    - FFS: exact value of N

- [15]

  - Proposal 2: A UE reports multiple UE Rx-Tx differences associated with a PRS resource in the presence of multiple paths between TRP and UE, where each UE Rx-Tx difference is associated with each detected path.

- [16]

  - Proposal 2: For both UE-based and UE-assisted methods, the relative power of the first detected path to the measured RSRP is also measured and reported.

- [18]

  - Proposal 1: To indicate the first arrival path by reporting the arrival time in the PRS measurement report.

  - Proposal 4: Support to reuse PRS for identifying LoS/NLoS.

- [19]

  - For multipath mitigating, study signaling and reporting enhancements based on the characteristics of the first lobe. The enhancements can be coupled with measurement reliability indicators derived from the parameters of the correlation function.

  - Proposal 2:   Support LMF requesting non-distinct path reporting as “Equal spaced path reporting” (ESPR) for the power and timing of M paths at a sampling frequency is defined by the numerology.

    - FFS: value of M and additional message

  - Proposal 6: Support the UE to measure and report phase information over multiple time instants.

- [21]:

  - The number of additional paths N should be large to enable accurate LOS/NLOS detection.

  - For each additional path, report at least:

    - the path power,

    - the path delay.

- [22]

  - Proposal 2: For multipath mitigation,reporting of path specific RSRP, RTOA/TDOA and angle of arrival for both UL and DL based schemes for every SRS/PRS resource configured.

  - Proposal 3: For multipath mitigation , reporting of up to 16 additional paths should be supported for measurement reports from UE to LMF and from TRP to LMF.

### Round #1 Discussion

Feature Lead View

Many companies propose many different enhancements though there are clear majorities on some topics. As such we have split them into the following proposals for discussion:

Proposal 5.1

- For multipath reporting enhancements, support reporting from UE to LMF, relative timing and the relative power of N additional paths for at least DL-AoD reporting.

  - FFS: Value of N.

Companies views:

| Company Name | Comments |
| --- | --- |
| Huawei | Support N = 8. |
| vivo | Firstly, we don’t think timing reporting should be discussed here since it is not supported in AoD before. In addition, for additional path RSRP reporting, we evaluated the performance gain of DL-AoD with RSRP information for the additional paths, where 3 additional paths are reported and the one with strongest RSRP is chosen to calculate location. From the evaluation results, we can see that with an accurate first path selection method, reporting more additional paths have no performance gain, which can only approach the accuracy of the first path. |
| Xiaomi | Support in pricinple. We suggest to make it more clear. Proposal 5.1 - For multipath reporting enhancements, in addition to the first path, support reporting from UE to LMF, relative timing and the relative power of N additional paths to that of the first path for at least DL-AoD reporting. - FFS: the maximum value of N. |
| ZTE | We prefer to agree the first path RSRP and first path timing in DL-AOD agenda before we discuss additional paths. |
| CATT | Support in principle. We think UE reporting of relative timing/power, phase and CIR/CFR information of additional paths to LMF will benefits the NLOS detection and improving the positioning accuracy. We prefer the following revision with one additional FFS: Updated Proposal 5.1 - For multipath reporting enhancements, support reporting from UE to LMF, relative timing and the relative power of N additional paths for at least DL-AoD reporting. - FFS: Value of N. - FFS: Other reporting information from UE to LMF, such as phase information, CIR and CFR, etc. |
| Nokia/NSB | Support |
| OPPO | Support, and N can be FFS. However, N = 8 seem to be too large. |
| InterDigital | This topic can be discussed along with the definition related to first-path RSRP in AoD AI. |
| LG | For clear understanding, we prefer Xiaomi’s version and we also agree with the principle. |
| Qualcomm | We think it is lower priority for DL-AoD to have a multipath/PDP reporting. DL-AoD is a method that uses power information, having the relative difference between paths is not as usefulas for the timing methods. We suggest to make first progress in the TDOA/M-RTT/UL-AoA, and if we have agreements there, we can come back to this one. |
| Ericsson | Support. For the value of N, we think the value can be larger depending on UE capability. We also propose that all N values across methods should be the same. The rewording of Xiaomi is ok with us. We also should discuss how the UE select/is requested to choose the repored paths (strongest peak, etc. ). This can be left FFS. |

Proposal 5.2

- For up to N additional paths, support reporting relative timing and power in the measurement reports from UE to LMF for at least DL-TDOA and multi-RTT

- Support one of the following options for maximum value of N

  - Option 1: N = 4

  - Option 2: N = 8

  - Option 3: N = 16

Companies views:

| Company Name | Comments |
| --- | --- |
| Fraunhofer | We are fine with the first bullet for N>2 while adding an FFS on the definition of the N additional paths. Unlike N=2; the N reported additional paths is inconclusive for the LMF. Assuming that it is up to UE implementation to select the N paths, the usage by the LMF for the additional paths differs for different reporting, such as - N strongest paths - Permanent N paths over multiple instants - “N”/M non-distinct paths - First N detectable paths - First N paths following a reference path - … The N additional paths shall not unambiguous as proposed in [21]. The values on N is subject to the usage Rel-17 would introduce from such a reporting. |
| Qualcomm | Support this proposal, assuming that the corresponding Proposal 6.1 is also agreed. For M-RTT, both are needed, and we prefer to be added all together. We are OK to leave it up the value “N” and decide next meeting. |
| Huawei | Support. |
| China Telecom | Support. |
| Xiaomi | Does it mean that the value of N will be discussed separately for different positioning scheme? |
| ZTE | As mentioned by some other companies, the path-specific RSRP and N>2 additional paths may be used for Artificial Intelligence (AI). During the RAN plenary workshop for Rel-18, a number of contributions discussed that the positioning can be a use case for AI. Hence, we suggest to discuss the path-specific RSRP in future release since we haven’t agreed the methodology to evaluate AI based positioning. We propose to postpone the discussions on path-specific RSRP of additional paths and timings of N>2 additional paths. |
| CATT | We are fine with the proposal. |
| Nokia/NSB | Support. We are okay with the FFS suggested by Fraunhofer. |
| OPPO | Suport and N can be FFS. And the maximum value of N shall be a UE capability |
| FL | To Xiaomi, looks like companies prefer to discuss N for all methods together which is okay from my side. If N is the same or different for different methods was an open question to be discussed in my view. |
| Ericsson | Ok to discuss for all methods. |
| vivo | we have similar view as ZTE |

Proposal 5.3

- Support reporting phase information for additional paths from UE to LMF as part of DL measurement reports

  - FFS: Details of phase information (e.g., CIR)

  - FFS: first path only or also additional paths

Companies views:

| Company Name | Comments |
| --- | --- |
| Qualcomm | We think it is too late to support this aspect now. We suggest to down prioritize it. |
| Huawei | Support. In general, we think that the phase reporting should be applicable for both the first path and the additional paths. The main bullet says “additional paths”, but the second subbullet “first path only or also additional paths”. |
| Intel | Support, we think that it is beneficial to support phase reporting of the first path and per additional path. |
| ZTE | Postpone the discussion. |
| CATT | Support the reporting of phase information. |
| Nokia/NSB | Don’t support. We agree with QC that it is too late for this feature and adding a new measurement (in addition to path RSRP) is very concerning from our side in terms of being able to complete the release. In addition, we don’t see phase information as critical for NLOS detection/mitigation. |
| OPPO | Do not support the proposal 5.3. Considering all the phase impairment caused by Tx and Rx chain, reporting a phase information of each path does not provide any valid information. |
| Ericsson | Do not support. Agree with qualcomm that time is running out for new features. |

## Issue #6: gNB/TRP to LMF multipath reporting enhancements

Many companies discuss multipath reporting enhancements from the UE to the LMF. The relevant agreements from RAN1#105-e are copied here and the proposals in company contributions are copied below it.

Agreement:

For multipath reporting enhancements, study reporting from TRP to LMF, angle, timing, phase (of additional paths) and power for the additional N paths (value of N is part of the study).

- Note: Companies are not obligated to provide inputs for all parameters in their study

RAN1#106-e company proposals:

- [2]

  - Proposal 6:  Extend the number of paths for a TRP measurement to 8, in which each path may be associated with its

    - TOA

    - Single value for the path strength (Path RSRP)

    - Single or Multiple AoA values

- [10]

  - Proposal 7: TRP should report angle, timing and power(RSRP) per additional path to LMF..

- [12]

  - Proposal 2: Support a gNB to report to the LMF additional time-domain paths (beyond 2 paths which is already specified) and their corresponding relative powers associated to an SRS resource

    - Applicable to both RTT and UL-TDOA methods

    - Support at least [8] total paths to be provided per SRS resource

  - Proposal 3: Support a gNB to report multiple tuples (UL-AoA, UL-RSRP, RTOA/gNB Rx-Tx) within a single report, such that

    - The UL-RSRP corresponds to a relative RSRP associated to the reported path in the angle/delay domain.

    - The RTOA/gNB-Rx-Tx corresponds to the delay of the associated reported path in the angle/delay domain

    - The UL-AoA corresponds to the received angle (potentially 2-dimensional) of the associated reported path in the angle/delay domain

    - Support at least [8] (UL-AoA, UL-RSRP, RTOA/gNB Rx-Tx) tuples that can be sent in a single report

  - Proposal 4: In the multipath reporting framework, a UE/gNB may also include an indication of which additional path is the strongest path measured per PRS/SRS resource.

- [14]

  - Support relative power PR in [dBm] and phase φ measurement in [deg] reporting associated with the relative time delay of additional path with the total number of additional paths equal to N (N > 2) for DL, UL, and DL + UL positioning methods:

    - PR,n = 10 × lg(Pn/P) indicates the relative power measurement in [dBm] for the nth path, where Pn is the power of the nth path, P is the total receive power corresponding to the measured DL PRS-RSRP or UL SRS-RSRP value within the considered measurement frequency bandwidth

      - The PR,n parameter is defined in the finite range [-X, 0], where X defines some threshold in [dBm] identifying the level of sensitivity for receive power

      - The granularity (or step size) of the measured PR,n parameter is set equal to Y in [dBm], identifying the sufficient level of accuracy, required in the measurements

    - φ indicates the phase measurement in [deg] for each additional path, that can be defined with the resolution of 1 [deg] or 0.1 [deg]

    - FFS: X in [dBm], Y in [dBm] (up to RAN4 discussion)

    - FFS: exact value of N

- [21]

    - The number of additional paths N should be large to enable accurate LOS/NLOS detection.

    - For each additional path, report at least:

      - the path power,

      - the path delay.

- [22]

  - Proposal 2: For multipath mitigation,reporting of path specific RSRP, RTOA/TDOA and angle of arrival for both UL and DL based schemes for every SRS/PRS resource configured.

  - Proposal 3: For multipath mitigation , reporting of up to 16 additional paths should be supported for measurement reports from UE to LMF and from TRP to LMF..

### Round #1 Discussion

Feature Lead View

Many companies propose many different enhancements though there are clear majorities on some topics. As such we have split them into the following proposals for discussion:

Proposal 6.1

- For multipath reporting enhancements, support reporting from TRP to LMF, angle, timing, and power for an additional N paths.

- Down select between the following options for N:

  - Option 1: N = 4

  - Option 2: N = 8

  - Option 3: N = 16

Companies views

| Company Name | Comments |
| --- | --- |
| Qualcomm | Support this proposal, assuming that the corresponding Proposal 6.1 is also agreed. For M-RTT, both are needed, and we prefer to be added all together. We are OK to leave it up the value “N” and decide next meeting. |
| Huawei | Support N =8 |
| Xiaomi | We want to clarify that is it the value of N or the maximum value of N? |
| ZTE | Similar view as Proposal 5.2. |
| CATT | We are fine with the proposal. |
| Nokia/NSB | Support. Our preference is N =4 but we are okay to decide at the next meeting. |
| OPPO | Support in principle. |
| LG | Agree. |
| FL | To Xiaomi, N is the maximum value of the number of additional paths to be reported. |
| Ericsson | Support. We have a similar comment than for UE reporting. For the value of N, we think the value can be larger. |

Proposal 6.2

- For multipath reporting enhancements, support reporting from TRP to LMF, phase information for the additional N paths.

  - FFS: Details of phase information (e.g., CIR)

Companies views

| Company Name | Comments |
| --- | --- |
| Qualcomm | We think it is too late to support this aspect now. We suggest to down prioritize it. |
| Huawei | Support. |
| China Telecom | Support. |
| Intel | Support, we think that it is beneficial to support phase reporting of the first path and per additional path. |
| ZTE | Postpone the discussion |
| CATT | Support the reporting of phase information. |
| Nokia/NSB | Don’t support. We agree with QC that it is too late for this feature and adding a new measurement (in addition to path RSRP) is very concerning from our side in terms of being able to complete the release. In addition, we don’t see phase information as critical for NLOS detection/mitigation. |
| OPPO | Do not support. The absolute phase information has impairement caused in Tx and Rx chain. It does not provide valid information. |
| Fraunhofer | Support |
| ericsson | Same comment as for UE reporting. Do not support. |

## Issue #7: UL-AoA Related Topics

A few companies brought proposals specific to enhancing reporting of UL-AoA for at least LoS/NLoS detection. Some alignment may be needed with AI 8.5.2. The specific proposals were:

- [2]

  - Proposal 5:  Support the same number of UL-AoA measurements per additional path..

- [5]

  - Proposal 4: Support UL-AoA measurement report from gNB to LMF that contain LOS/NLOS indicator (e.g., based on the statistical property / standard deviation of AoA) of the measured AoA for multipath/NLOS mitigation.

- [8]

  - Proposal 9: Further clarify and enhance the prior agreement of multiple measurements of M > 1 UL-AOA (AoA/ZoA) measurement values associated with the first arrival path and corresponding to the same timestamp by stating that:

    - M is the number of UL-AoA (AoA/ZoA) measuremnets that a UL receiver can measure in the same time stamp.

    - A UL receiver measures UL-AoA (AoA/ZoA) on a first arrival path at a measurement timing.

    - Corresponding to one UL-AoA measurement, a UL receiver may be requested to report additional information such as ToA of the measured path or beamforming to LMF.

    - Multiple measurements at a same time stamp are requested up to gNB measurement capability

- [9]

  - Proposal 4: NR supports reporting to LMF of N ≥ 1 UL-AOA measurement values per additional path for the same timestamp.

### Round #1 Discussion

Feature Lead View

At least two companies seem to propose that for the additional paths reporting that multiple AoA values per path are also support. With that in mind we make the following proposal:

- Proposal 7.1

- If additional paths for the UL-AoA measurement are supported, reporting multiple UL-AoA values per additional path is also supported.

  - Note: maximum number should be the same as the number eventually supported for the first path.

Companies views:

| Company Name | Comments |
| --- | --- |
| Qualcomm | Support |
| Huawei | Support |
| China Telecom | Support. |
| Xiaomi | Support |
| ZTE | Support |
| Sony | In the last meeting, we agreed to study reporting of LoS/NLoS indicators for DL, UL, and DL+UL positioning measurements taken at both UE and TRP at least for UE assisted positioning. In this context, the gNB can also report LOS/NLOS indicators (e.g. soft values (0,1] ). |
| CATT | Support the main bullet. But for the note, we prefer to leave it FFS, so we update the proposal as follows, - Updated Proposal 7.1 - If additional paths for the UL-AoA measurement are supported, reporting multiple UL-AoA values per additional path is also supported. - FFS: maximum number of UL-AoA values per additional path. |
| Nokia/NSB | Okay. |
| OPPO | We are ok with the update by CATT |
| LG | Support. |
| Ericsson | Support. |
| NTT DOCOMO | Support |

## Issue #8: DL-AoD Related Topics

Some companies brought proposals specific to enhancing reporting of DL-AoD for at least LoS/NLoS detection. Some alignment may be needed with AI 8.5.3. The specific proposals brought were:

- [9]

  - Proposal 2: At least the gNB beam/antenna bores-sight information can be provided to the LMF by the gNB and to the UE for UE-based positioning.

  - Proposal 3: UE can be requested to measure and report on specific PRS resources by the gNB.

- [16]

  - Proposal 1: Support PRS-RSRP measurement within a configured time window wherein the power of paths out of the window, if any, does not contribute in PRS-RSRP.

    - Alternatively or additionally, support PRS-RSRP for the first arrival path only that is measured within a configured time window.

### Round #1 Discussion

Feature Lead View

The proposals from Futurewei seems to already be under discussion in AI 8.5.3 as part of prior agreements. Suggest to take that discussion under 8.5.3. The proposal from Apple also seems quite related to ongoing discussion in AI 8.5.3 on the first path RSRP. Also suggest to take that discussion under AI 8.5.3.

Companies views:

| Company Name | Comments |
| --- | --- |

## Issue #9: Others

A few proposals from a single company are included here which don’t seem to fall under one of the specific topics above. Related proposals:

- [8]

  - Proposal 8: Support relative angle measurements between Rx beams used by UE for reception of DL PRS resources in Rel-17 at least for NLoS/LoS detection and mitigation.

- [19]

  - Proposal 4: LMF provides the UE with information on the channel model or channel parameters to enable channel state identification.

  - Proposal 5:   For NLOS mitigation, support the UE to provide the LMF with motion information reports with the same timestamps as the measurements or transmitted SRS for NR methods.

    - Send this agreement in an LS to RAN2.

- [20]

  - Proposal 3: Specify a separate LOS/NLOS measurement window, where the applicable LOS/NLOS positioning measurement is considered valid. This motivation of the LOS/NLOS measurement window is separate from the measurement timestamp reporting in AI 8.5.1. FFS further details such as length of the measurement window(s), configuration of periodic/aperiodic measurement window(s).

- [21]

  - The LMF can provide the UE/TRP a configuration for FFT window placement or alternatively provide an indication that a reconfiguration is needed.

### Round #1 Discussion

Feature Lead View

It is unclear to FL if there is any consensus on these topics. Suggest the proponent to explain further the motivation and any supporting companies to also comment such. If any consensus appears explicit proposals can be added for future discussion in this email discussion.

Companies views:

| Company Name | Comments |
| --- | --- |
| Fraunhofer | Proposal 4 in [19], is related to discussion in Issues 1 and 3: the UE might require assistance information from the LMF for a more reliable channel state detection. The channel state detection proposed by different contributions depends on the comparing an measured channel indicator with the expected one. These indicators (DS, K-factor, …) depends on the channel as we know from TR38.901. The question is how can a UE would identify if the DL-PRS corresponds to UMi, InF-SH/DH … to estimate a channel state? |
| ZTE | Proposal 8 in [8] could be useful for LOS identification. We also have interest of Proposal 4 in [19], if LMF has some priori information of the channel, LMF can provide the information to UE/gNB to facilitate the LOS determination. |
| Lenovo, Motorola Mobility | P3 in [20] relates to Issue#1 and Issue#2 in terms of the meta information configured by the LMF in order to accurately capture the short-term/long-term LOS/NLOS measurement behaviour depending on the length of the measurement window in which the measurement is deemed to be LOS/NLOS. Perhaps this could be added as added FFS point on any additional configuration related parameters to LOS/NLOS indicators under Issue#2. |
| FL | To Fraunhofer, thanks for the clarification. I will add the proposal under those issues. To ZTE, thanks for the comment. If additional companies also support this issue we can create a new section to discuss it. To Lenovo, thanks for clarification. I will add the proposal under those issues. |

Proposals for GTW

Suggested Proposals for 1st GTW

Proposal 5.2.1

- For up to N additional paths, support reporting relative timing and power in the measurement reports from UE to LMF for at least DL-TDOA and multi-RTT

  - FFS: Definition of additional path

- Support one of the following options for maximum value of N at RAN1#106-b:

  - Option 1: N = 4

  - Option 2: N = 8

  - Option 3: N = 16

  - Option 4: N = 32

Proposal 6.1.1

- For multipath reporting enhancements, support reporting from TRP to LMF, angle, timing, and power for up to additional N paths.

  - FFS: Definition of additional path

- Down select between the following options for N at RAN1#106-b:

  - Option 1: N = 4

  - Option 2: N = 8

  - Option 3: N = 16

  - Option 4: N = 32

Proposal 1.1

- Support LoS/NLoS indicators which are reported for DL, UL, and DL+UL positioning measurements taken at UE or TRP, at least for UE assisted positioning.

  - FFS: UE based positioning

Proposal 2.1

- If LoS/NLoS indicators are supported in Rel-17, their value is in the interval [0, 1].

Proposal 5.1.1

- For multipath reporting enhancements, support reporting from UE to LMF, relative timing and the relative power of up to N additional paths for at least DL-AoD reporting.

  - FFS: Value of N.

Conclusion

In this contribution, we provided review of the submitted contributions for NR Positioning AI 8.5.5 on potential enhancements for information reporting from UE and gNB for multipath/NLOS mitigation and prepared an initial set of proposals to facilitate further discussion/decision by RAN1 during the RAN1#106–e meeting.

References

- RP-210903, Revised WID on NR Positioning Enhancements, CATT, Intel Corporation, Ericsson.

- R1-2106453, Enhancements to support multi-path and NLOS mitigation, Huawei, HiSilicon.

- R1-2106553, Enhancements on NLOS and multi-path mitigation for NR positioning, ZTE.

- R1-2106599, Discussion on potential enhancements for multipath/NLOS mitigation, vivo.

- R1-2106813, Discussion on enhanced reporting from UE and gNB for Multipath/NLOS mitigation, Sony.

- R1-2106892, Discussion on potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation, Samsung.

- R1-2106975, Discussion on potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation, CATT.

- R1-2107061, Views on LoS/NLoS Identification and Mitigation, Nokia, Nokia Shanghai Bell.

- R1-2107096, NLOS Mitigation Enhancements, Futurewei.

- R1-2107135, Discussion on multipath/NLOS identification and mitigation for positioning enhancement, China Telecom.

- R1-2107217, Discussion on multipath/NLOS mitigation for NR positioning, OPPO.

- R1-2107349, Multipath Reporting in NR Positioning, Qualcomm Incorporated.

- R1-2107546, Discussion on multipath/NLOS mitigation for positioning, LG Electronics.

- R1-2107594, Solutions for mitigation of NLOS Problem for NR Positioning, Intel Corporation.

- R1-2107648, Discussion on multipath/NLOS mitigation for positioning, InterDigital Inc.

- R1-2107744, Views on potential enhancements for NLOS mitigation in Rel-17 positioning, Apple.

- R1-2107862, Discussion on multipath/NLOS mitigation for NR positioning, NTT DOCOMO, INC.

- R1-2107924, Potential enhancements for multipath/NLOS mitigation, Xiaomi.

- R1-2108104, Potential positioning enhancements for multipath/NLOS mitigation, Fraunhofer IIS, Fraunhofer HHI.

- R1-2108145, Accuracy enhancements based on NLOS/Multipath Information Reporting, Lenovo, Motorola Mobility.

- R1-2108168, Potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation, Ericsson.

- R1-2108175, Discussion on enhancements of multipath/NLOS reporting from UE and gNB, CEWiT.