## NR Multicast and Broadcast Services

Please refer to RP-201038 for detailed scope of the WI

### Mechanisms to support group scheduling for RRC_CONNECTED UEs

R1-2106438	Resource configuration and group scheduling for RRC_CONNECTED UEs	Huawei, HiSilicon, CBN

R1-2106623	Discussion on mechanisms to support group scheduling for RRC_CONNECTED Ues	vivo

R1-2106662	Group Scheduling Mechanisms to Support 5G Multicast / Broadcast Services for RRC_CONNECTED Ues	Nokia, Nokia Shanghai Bell

R1-2106716	Discussion on MBS group scheduling for RRC_CONNECTED UEs	Spreadtrum Communications

R1-2106745	Discussion on Mechanisms to Support Group Scheduling for RRC_CONNECTED UEs	ZTE

R1-2106820	Considerations on MBS group scheduling for RRC_CONNECTED UEs	Sony

R1-2106912	Support of group scheduling for RRC_CONNECTED Ues	Samsung

R1-2106945	Discussion on group scheduling mechanism for RRC_CONNECTED UEs in MBS	CATT

R1-2106996	Common frequency resource configuration for multicast of RRC_CONNECTED Ues	ETRI

R1-2107093	Group Scheduling Aspects for Connected UEs	FUTUREWEI

R1-2107137	Discussion on Group Scheduling Mechanisms for RRC_CONNECTED Ues	NEC

R1-2107160	On group scheduling mechanism for NR MBS	Lenovo, Motorola Mobility

R1-2107201	Discussion on group scheduling mechanisms for RRC_CONNECTED UEs	Potevio Company Limited

R1-2107229	Discussion on group Scheduling mechanism for RRC_CONNECTED UEs	OPPO

R1-2107369	Views on group scheduling for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

R1-2107382	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	Google Inc.

R1-2107424	Summary#1 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS	Moderator (CMCC)

R1-2107425	Discussion on group scheduling mechanisms	CMCC

R1-2107456	Support of group scheduling for RRC_CONNECTED UEs	LG Electronics

R1-2107514	Discussion on NR MBS group scheduling for RRC_CONNECTED UEs	MediaTek Inc.

R1-2107611	NR-MBS Group Scheduling for RRC_CONNECTED UEs	Intel Corporation

R1-2107763	Discussion on group scheduling mechanism for RRC_CONNECTED Ues	Apple

R1-2107881	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	NTT DOCOMO, INC.

R1-2107902	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UE	Xiaomi

R1-2107950	Group scheduling related discussion for RRC_CONNECTED UEs	CHENGDU TD TECH LTD.

R1-2108026	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	Convida Wireless

R1-2108046	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs	ASUSTeK

R1-2108170	Mechanisms to support MBS group scheduling for RRC_CONNECTED Ues	Ericsson

R1-2108308

R1-2108368

[106-e-NR-MBS-01] Email discussion/approval on mechanisms to support group scheduling for RRC_CONNECTED UEs with checkpoints for agreements on August 19, 24 and 27 – Fei (CMCC)

Agreement:

Confirm the working assumption with the following update:

Option 2B for CFR associated with UE active BWP other than initial DL BWP is supported at least for multicast of RRC-CONNECTED UEs.

- FFS: CFR associated with initial BWP

- FFS: CFR larger than initial BWP

Note: The deleted FFSs can be discussed in another AI.

Proposal:

Align the size of the first DCI format with DCI format 1_0 with CRC scrambled by C-RNTI monitored in CSS (when the size budget of 3 DCI formats scrambled by C-RNTI is exceeded).

Agreement:

For multicast of RRC-CONNECTED UEs, align the size of the first DCI format for GC-PDCCH with DCI format 1_0 with CRC scrambled by C-RNTI monitored in CSS.

Agreement:

Confirm the following working assumption:

The maximum number of CORESETs per BWP is not increased for support of MBS, and the number of CORESETs configured within the CFR is left to gNB implementation.

Agreement:

For indication of the starting PRB and the length of PRBs of CFR for multicast of RRC-CONNECTED UEs,

- the starting PRB is referenced to Point A, i.e., the starting PRB is a PRB determined by subcarrierSpacing of the associated BWP and offsetToCarrier corresponding to this subcarrier spacing, similar as how locationAndBandwidth of a BWP is indicated as described in TS 38.331.

- FFS: Indication mechanism.

Agreement:

For LBRM and TBS determination for GC-PDSCH:

- The maximum number of layers can be provided by maxMIMO-Layers in PDSCH-Config for MBS in CFR; if not provided, a default value is defined.

  - FFS the default value.

- The maximum modulation order can be determined from mcs-Table in PDSCH-Config for MBS in CFR;

  - FFS: if mcs-Table in PDSCH-Config for MBS is not configured in CFR, a value determined from mcs-Table in PDSCH-Config for unicast in the active DL BWP is used; if the mcs-Table in PDSCH-Config for unicast is not configured, Table 5.1.3.1-1 in TS38.214 is used (similar as the default value in R16).

- xOverhead can be provided in PDSCH-Config for MBS in CFR; if not provided, a default value of zero is used.

- The number of PRBs is determined based on the size of CFR.

Agreement:

The first DCI format for GC-PDCCH uses the same fields as DCI format 1_0 with CRC scrambled by C-RNTI with the following modifications:

- At least ‘Identifier for DCI formats’ is not needed.

  - FFS: Whether the field should be ignored and reserved, or should be removed.

- For FDRA determination, down-select from following options:

  - Option 1:

    - NRBDL,BWP is given by

      - the size of CORESET 0 if CORESET 0 is configured for the cell; and

      - the size of initial DL bandwidth part if CORESET 0 is not configured for the cell.

    - For resource indication value (RIV) of downlink resource allocation type 1, the resource blocks that can be indicated are

      - the resource blocks in the CORESET 0 if CORESET 0 is configured for the cell; and

      - the resource blocks in the initial DL bandwidth part if CORESET 0 is not configured for the cell.

  - Option 2:

    - NRBDL,BWP is given by

      - the size of CORESET 0 if CORESET 0 is configured for the cell; and

      - the size of initial DL bandwidth part if CORESET 0 is not configured for the cell.

    - For resource indication value (RIV) of downlink resource allocation type 1, the similar scheme as for the case that the DCI size for DCI format 1_0 in USS is derived from the size of DCI format 1_0 in CSS but applied to an active BWP is used.

      - FFS details, e.g., if the size of CFR (i.e. ) is larger than the size of CORESET0/initial DL bandwidth part, the resource indication value (RIV) is defined as in section 5.1.2.2.2 in TS38.214, where K is the maximum value from set {1, 2, 4, 8} which satisfies ;otherwise,

  - Option 3: NRBDL,BWP is given by the size of CFR in the active DL BWP

Agreement:

The second DCI format for GC-PDCCH uses the same fields as DCI format 1_1 with the following modifications:

- At least ‘Identifier for DCI formats’ and ‘SRS request’ are not needed.

  - FFS whether the fields should be ignored and reserved, or should be removed.

- Note: At least the configurable fields in DCI format 1_1 remain configurable for the second DCI format

Proposal:

For initializing scrambling sequence generator for GC-PDCCH with the second DCI format,

- equals the higher layer parameter pdcch-DMRS-ScramblingID if it is configured in the CORESET in a CFR used for the GC-PDCCH;  otherwise.

- FFS  is given by

  - Alt1: G-RNTI used for the GC-PDCCH.

  - Alt2: 0

### Mechanisms to improve reliability for RRC_CONNECTED UEs

R1-2106439	Mechanisms to improve reliability for RRC_CONNECTED UEs	Huawei, HiSilicon, CBN

R1-2106624	Discussion on mechanisms to improve reliability for RRC_CONNECTED Ues	vivo

R1-2106663	Reliability Improvements for RRC_CONNECTED UEs	Nokia, Nokia Shanghai Bell

R1-2106717	Mechanisms to improve MBS reliability for RRC_CONNECTED Ues	Spreadtrum Communications

R1-2106746	Discussion on Mechanisms to Improve Reliability for RRC_CONNECTED UEs	ZTE

R1-2106913	On mechanisms to improve reliability for RRC_CONNECTED UEs	Samsung

R1-2106946	Discussion on reliability improvement mechanism for RRC_CONNECTED UEs in MBS	CATT, CBN

R1-2106997	HARQ-ACK feedback scheme for multicast of RRC_CONNECTED Ues	ETRI

R1-2107094	Further Discussions on Reliability for RRC_CONNECTED UEs	FUTUREWEI

R1-2107138	Discussion on Reliability Improvements for RRC_CONNECTED UEs	NEC

R1-2107161	On reliability improvement for RRC-CONNECTED UEs	Lenovo, Motorola Mobility

R1-2107230	UL feedback for RRC-CONNECTED UEs in MBS	OPPO

R1-2107370	Views on UE feedback for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

R1-2107383	Discussion on MBS reliability for RRC_CONNECTED UEs	Google Inc.

R1-2107426	Discussion on reliability improvement	CMCC

R1-2107457	Mechanisms to improve reliability of Broadcast/Multicast service	LG Electronics

R1-2107515	Discussion on mechanisms to improve reliability for RRC_CONNECTED UEs	MediaTek Inc.

R1-2107612	Mechanisms to Improve Reliability of NR-MBS for RRC_CONNECTED UEs	Intel Corporation

R1-2107764	Discussion on MBS reliability improvement for RRC_CONNECTED UEs	Apple

R1-2107882	Discussion on mechanisms to improve reliability for multicast for RRC_CONNECTED UEs	NTT DOCOMO, INC.

R1-2107951	Reliability related discussion for RRC_CONNECTED UEs	CHENGDU TD TECH LTD.

R1-2108027	Discussion on reliability enhancement for RRC_CONNECTED UEs	Convida Wireless

R1-2108171	Mechanisms to improve reliability for RRC_CONNECTED Ues	Ericsson

R1-2108332

[106-e-NR-MBS-02] Email discussion/approval on mechanisms to improve reliability for RRC_CONNECTED UEs with checkpoints for agreements on August 19, 24 and 27 – Jinhuan (Huawei)

Agreement:

For UE supporting both unicast and multicast, the pdsch-HARQ-ACK-Codebook/pdsch-HARQ-ACK-CodebookList can be separately configured for multicast from that for unicast.

Agreement:

- When UE is configured Type-1 codebooks for unicast and multicast with different priorities, respectively, the UE separately generates each of the Type-1 codebooks.

- FFS: How UE is configured one codebook for unicast and one codebook for multicast and the two codebooks are of different priorities.

Proposal:

If UE reports the capability of supporting the FDM-ed unicast and multicast in the same slot, UE can be configured to receive FDM-ed unicast and multicast:

- If UE is not configured to receive FDM-ed unicast and multicast, Type-1 HARQ codebook is generated as the agreement for TDM-ed unicast and multicast.

- If UE is configured to receive FDM-ed unicast and multicast, Type-1 HARQ codebook is generated as the agreement for FDM-ed unicast and multicast.

### Basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs

R1-2106440	Discussion on UE receiving broadcast in RRC IDLE/INACTIVE state	Huawei, HiSilicon, CBN

R1-2106625	Discussion on basic functions for broadcast multicast for RRC_IDLE RRC_INACTIVE UEs	vivo

R1-2106664	Basic Functions for Broadcast / Multicast for RRC_IDLE / RRC_INACTIVE Ues	Nokia, Nokia Shanghai Bell

R1-2106718	Basic Functions for Broadcast or Multicast for RRC_IDLE or RRC_INACTIVE UEs	Spreadtrum Communications

R1-2106747	Discussion on basic Functions for Broadcast or Multicast for RRC_IDLE or RRC_INACTIVE UEs	ZTE

R1-2106821	Considerations on MBS functions for RRC_IDLE/INACTIVE UEs	Sony

R1-2106914	On basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs	Samsung

R1-2106947	Discussion on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs	CATT, CBN

R1-2107095	MBS Support for RRC IDLE/INACTIVE UEs	FUTUREWEI

R1-2107162	Basic functions for broadcast/multicast in idle/inactive states	Lenovo, Motorola Mobility

R1-2107165	Search Space and DCI Design for MBS in IDLE and INACTIVE states	TCL Communication Ltd.

R1-2107231	Discussion on basic functions for RRC_IDLE/RRC_INACTIVE UEs	OPPO

R1-2107371	Views on group scheduling for Broadcast RRC_IDLE/INACTIVE UEs	Qualcomm Incorporated

R1-2107384	Discussion on MBS for RRC_IDLE/RRC_INACTIVE UEs	Google Inc.

R1-2107427	Discussion on NR MBS in RRC_IDLE/ RRC_INACTIVE states	CMCC

R1-2107458	Basic function for broadcast/multicast	LG Electronics

R1-2107516	Discussion on MBS for RRC_IDLE/INACTIVE UEs	MediaTek Inc.

R1-2107613	NR-MBS for RRC_IDLE/INACTIVE UEs	Intel Corporation

R1-2107765	Discussion on MBS for RRC_IDLE and RRC_INACTIVE UEs	Apple

R1-2107883	Discussion on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs	NTT DOCOMO, INC.

R1-2107952	NR MBS related discussion for RRC_IDLE/RRC_INACITVE UEs	CHENGDU TD TECH LTD.

R1-2108028	Discussion on MBS for RRC_IDLE/RRC_INACTIVE UEs	Convida Wireless

R1-2108172	Support for NR multicast reception in RRC Inactive/Idle	Ericsson

R1-2108228

[106-e-NR-MBS-03] Email discussion/approval on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs and the LS in R1-2106410 (from AI5) including any reply LS as necessary with checkpoints for agreements on August 19, 24 and 27 – David (BBC)

Proposal:

GC-PDCCH/PDSCH carrying MCCH or MTCH for broadcast reception with UEs in RRC IDLE/INACTIVE state can use a configured/defined CFR with larger size than CORESET0. In Rel-17, at least support Case-C.

- FFS: whether signalling to enable Case-C is included/extended as part of SIB1 or other SIB, whether signalling needs to use configured BWP framework, whether CFR configuration for MTCH can be provided by MCCH (e.g. if different CFR configuration is needed for MTCH), or whether it is up to RAN2 to ensure adequate signalling.

- FFS: support of Case D and/or Case E.

Alternate proposal:

GC-PDCCH/PDSCH carrying MCCH or MTCH for broadcast reception with UEs in RRC IDLE/INACTIVE state can use a configured/defined CFR with a larger size than CORESET0 and equal to the initial BWP configured to an MBS capable UE. In Rel-17, at least support Case-C.

- FFS: Whether the initial BWP configured to an MBS capable UE is the same as that configured to a legacy UE.

- FFS: Signaling details for enabling Case C.

- FFS: Support of Case D and/or Case E.

Agreement:

From RAN1 perspective, the CFR for broadcast reception of RRC_IDLE/INACTIVE UEs, includes at least the following configurations:

- One set of parameters configured for PDSCH for broadcast reception with GC-PDSCH

- One set of parameters configured for PDCCH for broadcast reception with GC-PDCCH

- FFS: whether some parameters configured for PDSCH/PDCCH are optional/needed for the supported cases of CFR.

- FFS: If necessary, depending on the cases supported, starting PRB and the number of PRBs

  - The reference for starting PRB is Point A. (Following the same approach to determine reference for starting PRB as that defined in AI8.12.1.)

### Others

R1-2106626	Other issues for Rel-17 MBS	vivo

R1-2106665	Other Aspects for Rel-17 MBS	Nokia, Nokia Shanghai Bell

R1-2106748	Consideration on potential further enhancement for MBS	ZTE

R1-2106948	Discussion on other issues in Rel-17 MBS	CATT

R1-2107379	Other aspects of NR Multicast	Qualcomm Incorporated

Withdrawn

R1-2107459	Other aspects for MBS	LG Electronics

R1-2107662	Impact from MCCH and MTCH on broadcast reception	Huawei, HiSilicon

R1-2107953	Other aspects for NR MBS	CHENGDU TD TECH LTD.

R1-2108173	Assumptions for performance evaluations of NR-MBS	Ericsson