## New Bands and Bandwidth Allocation for LTE based 5G Terrestrial Broadcast

Please refer to RP-211144 for detailed scope of the WI

R1-2106555	TEI-17 proposal on NR codeword mapping	ZTE

Withdrawn

R1-2106762	Work Plan for New Bands and Bandwidth Allocation for LTE based 5G Terrestrial Broadcast			Qualcomm Incorporated, EBU

### PMCH allocation of 6/7/8 MHz and corresponding MBSFN reference signals

R1-2106560	On PMCH allocation of 6/7/8 MHz and corresponding reference signals	Huawei, HiSilicon

R1-2106752	Discussion on PMCH allocation and corresponding MBSFN reference signals	ZTE

R1-2106763	PMCH allocation of 6/7/8MHz	Qualcomm Incorporated

R1-2108211	Feature lead summary on New bands and bandwidth allocation for LTE based 5G terrestrial broadcast		Moderator (Qualcomm)

R1-2108324	Feature lead summary #2 on New bands and bandwidth allocation for LTE based 5G terrestrial broadcast		Moderator (Qualcomm)

[106-e-LTE-5G-Terr-Bcast-01] Email discussion/approval on PMCH allocation of 6/7/8 MHz and corresponding MBSFN reference signals with checkpoints for agreements on August 19, 24 and 27 – Alberto (Qualcomm)

Agreement:

For supporting 6/7/8MHz PMCH:

- The bandwidth of CAS (system bandwidth indicated in MIB) is set to 25PRBs (5MHz).

  - FFS: whether it can be set to 6 and/or 15 PRBs.

- The bandwidth for PMCH ( is indicated by another parameter in system information if the center frequency of bandwidth for CAS and bandwidth for PMCH is aligned.

  - FFS: Details

Agreement:

It is supported that the center frequency of system bandwidth and center frequency of PMCH bandwidth are aligned

- FFS: Other cases (non-aligned center frequencies)

Agreement:

For PMCH allocation of 6/7/8 MHz on MBMS-dedicated cells, mapping the MBSFN reference signal to REs is based on the PMCH bandwidth of 6/7/8 MHz instead of NRBDL.

Agreement:

For PMCH allocation of 6/7/8 MHz on MBMS-dedicated cells, the TBS determination is based on the PMCH bandwidth of 6/7/8 MHz instead of NRBDL.

Conclusion:

It is RAN1’s understanding that 15kHz SCS is currently supported for MBMS dedicated cells.

- RAN1 to further discuss how to handle 15kHz SCS for 6/7/8 MHz PMCH bandwidth

Agreement:

The following PMCH bandwidth values are supported:

- 8MHz:

- 7MHz:

- 6MHz:

Agreement:

The signalling of PMCH bandwidth is to be selected to be one of the following:

- Alt 1: Per cell

- Alt 2: Per MBSFN area

Agreement:

For CAS bandwidth values, 25PRBs (5MHz) and 15 PRBs (3MHz) are supported.

Conclusion:

Non-aligned PMCH bandwidth and system bandwidth is not supported (this resolves an FFS item in a prior agreement).

Agreement:

For 6/7/8MHz PMCH bandwidth with 15kHz SCS:

- Alt 1: The control region in MBSFN subframes with 15kHz SCS has the same bandwidth as CAS. The UE is not required to process the control region.

- Alt 2: The MBSFN subframes with 15kHz SCS do not have control region.

- Alt 3: The MBSFN subframes with 15 kHz SCS have a control region but its content is not defined

Agreement:

The signalling of PMCH bandwidth is per MBSFN area.

### Others

R1-2106753	Considerations on bandwidth changing in MBSFN subframe for 15KHz subcarrier spacing	ZTE

R1-2107685	Numerologies supported for MBMS-dedicated cell	Huawei, HiSilicon