## NR Positioning Enhancements

Please refer to RP-210903 for detailed scope of the WI

The FCC’s quiet period for Auction 110 (the 3.45 GHz spectrum) is currently in effect. Accordingly, no discussions or questions relating to the auction, bids, bidding strategy, or post-auction market structure will be invited or permitted today or at any time until the quiet period has ended.

### Accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays

Including DL, UL and DL+UL positioning methods, and UE-based and UE-assisted positioning solutions.

R1-2106449	Enhancement to mitigate gNB and UE Rx/Tx timing error	Huawei, HiSilicon

R1-2106549	Positioning accuracy improvement by mitigating timing delay	ZTE

R1-2106595	Discussion on  potential enhancements for RX/TX timing delay mitigating	vivo

R1-2106809	On mitigating Rx/Tx timing delays	Sony

R1-2106888	Discussion on accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays	Samsung

R1-2106971	Discussion on mitigating UE and gNB Rx/Tx timing errors	CATT

R1-2107057	Views on mitigating UE and gNB Rx/Tx timing errors	Nokia, Nokia Shanghai Bell

R1-2107213	Enhancement of timing-based positioning by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays	OPPO

R1-2107345	Enhancements on Timing Error Mitigations for improved Accuracy	Qualcomm Incorporated

R1-2107403	Discussion on mitigation of gNB/UE Rx/Tx timing errors	CMCC

R1-2107542	Discussion on accuracy improvement by mitigating UE Rx/Tx and gNB Rx/Tx timing delays	LG Electronics

R1-2107590	Details of UE/gNB RX/TX Timing Errors Mitigation	Intel Corporation

R1-2107643	Discussion on accuracy improvements by mitigating timing delays	InterDigital, Inc.

R1-2107740	Positioning accuracy enhancements under timing errors	Apple

R1-2107822	Mitigation of RX/TX timing delays for higher accuracy	MediaTek Inc.

R1-2107858	Discussion on mitigating UE and gNB Rx/Tx timing delays	NTT DOCOMO, INC.

R1-2108101	On methods for Rx/Tx timing delays mitigation	Fraunhofer IIS, Fraunhofer HHI

R1-2108142	Enhancements for mitigation of Tx/Rx Delays	Lenovo, Motorola Mobility

R1-2108164	Techniques mitigating Rx/Tx timing delays	Ericsson

[106-e-NR-ePos-01] Email discussion/approval on accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays with checkpoints for agreements on August 19, 24 and 27 – Ren Da (CATT)

R1-2108241	FL Summary for accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays	Moderator (CATT)

Agreement:

- Subject to UE capability, support a UE to include one UE Rx TEG ID for the RSTD reference time and one UE Rx TEG ID for each DL RSTD measurement (including each additional DL RSTD measurement), in a DL TDOA measurement report. These UE Rx TEG IDs can be the same or different.

- Note: RSTD reference time is related to the DL_PRS_Reference_Info IE

Working assumption:

For mitigating UE Tx timing errors for UL TDOA, support the following:

- Subject to UE’s capability, support LMF to request and a UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF if the UE has multiple Tx TEGs.

- FFS: Support LMF to forward the association information provided by the UE to the serving and neighboring gNBs

- FFS: Option 2:

  - Subject to UE’s capability, support serving gNB to request and a UE providing the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB if the UE has multiple Tx TEGs.

  - Support the serving gNB to forward the association information provided by the UE to the LMF

  - FFS: Support LMF to forward the association information from the serving gNB for the UE to the neighboring gNBs

- FFS: UE should be able to report capability information related to Tx TEGs to LMF via LPP signalling

- FFS: the need or the benefits of defining static and dynamic association information of UL SRS resources for positioning with Tx TEGs

- FFS: The details of the signalling and the procedures

- Send LS to RAN2 and check if they have any concerns

Alternate Proposal:

- For mitigating UE Tx timing errors for UL TDOA, support the following:

- Subject to UE’s capability, support the network to request and a UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the network if the UE has multiple Tx TEGs.

- RAN2 or RAN3 to decide whether to use LPP or RRC signaling and what signaling is needed between the network nodes

- Send LS to RAN2 and RAN3 informing them of the agreement

### Accuracy improvements for UL-AoA positioning solutions

R1-2106450	Enhancement for UL AoA positioning	Huawei, HiSilicon

R1-2106550	Accuracy improvement for UL-AoA positioning solutions	ZTE

R1-2106596	Discussion on potential enhancements for UL-AoA method	vivo

R1-2106810	Considerations on UL-AoA enhancements	Sony

R1-2106889	Discussion on accuracy improvements for UL-AoA positioning solutions	Samsung

R1-2106972	Discussion on enhancements for UL-AoA positioning method	CATT

R1-2107058	Views on enhancing UL AoA	Nokia, Nokia Shanghai Bell

R1-2107214	Enhancements for UL AoA Positioning	OPPO

R1-2107346	Potential Enhancements on UL-AOA positioning	Qualcomm Incorporated

R1-2107404	Discussion on UL-AoA enhancements	CMCC

R1-2107543	Discussion on accuracy improvement for UL-AoA positioning	LG Electronics

R1-2107591	Remaining Details of NR Positioning UL-AoA Enhancements	Intel Corporation

R1-2107645	Discussion on enhancements for UL-AoA positioning solutions	InterDigital, Inc.

R1-2107741	Positioning Accuracy enhancements for UL-AoA	Apple

R1-2107859	Discussion on UL-AoA positioning enhancements	NTT DOCOMO, INC.

R1-2108102	UL-AoA positioning enhancements	Fraunhofer IIS, Fraunhofer HHI

R1-2108165	Enhancements of UL-AoA positioning solutions	Ericsson

R1-2108176	Discussion on enhancements for UL AoA positioning	CEWiT

[106-e-NR-ePos-02] Email discussion/approval on accuracy improvements for UL-AoA positioning solutions, with checkpoints for agreements on August 19, 24 and 27 – Alexey (Intel)

R1-2108289	Feature Lead Summary#1 for E-mail Discussion [106-e-NR-ePos-02]		Moderator (Intel)

Agreement:

The maximum number of UL-AOAs values (pair of AOA & ZOA values) to be reported per SRS resource for the first arrival path corresponding to the same timestamp is 8.

Conclusion:

It is up to RAN3 to decide how to support indication of UL AoA/ZoA assistance information in LCS for LCS to GCS translation

### Accuracy improvements for DL-AoD positioning solutions

Including UE-based and network-based (including UE-assisted) positioning solutions

R1-2106451	Enhancement for DL AoD positioning	Huawei, HiSilicon

R1-2106551	Accuracy improvement for DL-AoD positioning solutions	ZTE

R1-2106597	Discussion on potential enhancements for DL-AoD method	vivo

R1-2106811	Considerations on DL-AoD enhancements	Sony

R1-2106890	Discussion on accuracy improvements for DL-AoD positioning solutions	Samsung

R1-2106973	Discussion on enhancements for DL-AoD positioning method	CATT

R1-2107059	Views on enhancing DL AoD	Nokia, Nokia Shanghai Bell

R1-2107169	Discussion on enhancements for DL-AoD positioning	CAICT

R1-2107215	Enhancements for DL-AoD positioning	OPPO

R1-2107347	Potential Enhancements on DL-AoD positioning	Qualcomm Incorporated

R1-2107405	Discussion on DL-AoD enhancements	CMCC

R1-2107544	Discussion on accuracy improvement for DL-AoD positioning	LG Electronics

R1-2107592	DL-AoD Enhancements for Precise NR Positioning	Intel Corporation

R1-2107646	Discussion on enhancements for DL-AoD positioning solutions	InterDigital, Inc.

R1-2107742	Positioning Accuracy enhancements for DL-AoD	Apple

R1-2107823	Accuracy enhancement for DL-AOD technique	MediaTek Inc.

R1-2107860	Discussion on DL-AoD positioning enhancements	NTT DOCOMO, INC.

R1-2107922	Accuracy improvements for DL-AoD positioning solutions	Xiaomi

R1-2108103	DL-AoD positioning enhancements	Fraunhofer IIS,  Fraunhofer HHI

R1-2108143	Discussion on DL-AoD Positioning Enhancements	Lenovo, Motorola Mobility

R1-2108166	Enhancements of DL-AoD positioning solutions	Ericsson

R1-2108174	Discussion on enhancements for DL-AoD positioning	CEWiT

[106-e-NR-ePos-03] Email discussion/approval on accuracy improvements for DL-AoD positioning solutions with checkpoints for agreements on August 19, 24 and 27 – Florent (Ericsson)

R1-2108311	FL summary #1 for AI 8.5.3 Accuracy improvements for DL-AoD positioning solutions	Moderator (Ericsson)

Proposal:

For definition of the path PRS RSRP,

- Option 1: the path PRS RSRP correspond to the power of the channel impulse response at a certain path delay

- Option 2: the path PRS RSRP correspond to the accumulated power of the channel impulse response over a time duration corresponding to the given path delay

  - FFS: how is the window conveyed to the UE (i.e. fixed in specification or configured in measurement request)

- FFS further details of the definition

- Send LS to RAN4 informing them of this agreement

Alternate Proposal:

The measured path PRS RSRP is the power of the channel impulse response at a certain path delay.

- Up to RAN4 to define any test/requirement for the measurement.

- Send LS to RAN4 informing them of this agreement after completion of further discussion.

### Latency improvements for both DL and DL+UL positioning methods

R1-2106452	Positioning latency enhancements	Huawei, HiSilicon

R1-2106552	Discussion on latency reduction for NR positioning	ZTE

R1-2106598	Discussion on latency enhancement for NR positioning	vivo

R1-2106812	Considerations on latency improvements for positioning	Sony

R1-2106891	Discussion on latency improvements for both DL and DL+UL positioning methods	Samsung

R1-2106974	Discussion on latency reduction for NR positioning	CATT

R1-2107060	Views on PHY Latency Reductions	Nokia, Nokia Shanghai Bell

R1-2107134	Discussion on latency improvements for positioning methods	China Telecom

R1-2107216	Enhancements on Latency Reduction in NR Positioning	OPPO

R1-2107348	Enhancements for Latency Improvements for Positioning	Qualcomm Incorporated

R1-2107406	Discussion on latency improvement for positioning	CMCC

R1-2107545	Discussion on latency improvements for NR positioning	LG Electronics

R1-2107593	Latency Reduction Solutions for NR Positioning	Intel Corporation

R1-2107647	Discussion on latency improvements for DL and DL+UL positioning methods	InterDigital, Inc.

R1-2107743	Views on Rel-17 positioning latency reduction	Apple

R1-2107828	Aspects of physical latency improvement	MediaTek Inc.

R1-2107861	Discussion on latency improvements for both DL and DL+UL positioning methods	NTT DOCOMO, INC.

R1-2107923	Latency improvements for both DL and DL+UL positioning method	Xiaomi

R1-2108144	Positioning Latency Reduction Enhancements	Lenovo, Motorola Mobility

R1-2108167	Latency improvements for both DL and DL+UL positioning methods	Ericsson

[106-e-NR-ePos-04] Email discussion/approval on latency improvements for both DL and DL+UL positioning methods with checkpoints for agreements on August 19, 24 and 27 – Su (Huawei)

### Potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation

R1-2106453	Enhancements to support multi-path and NLOS mitigation	Huawei, HiSilicon

R1-2106553	Enhancements on NLOS and Multi-path mitigation for NR positioning	ZTE

R1-2106599	Discussion on potential enhancements for multipath/NLOS mitigation	vivo

R1-2106813	Discussion on enhanced reporting for multipath/NLOS mitigation	Sony

R1-2106892	Discussion on potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation	Samsung

R1-2106975	Discussion on information reporting from UE and gNB for multipath/NLOS mitigation	CATT

R1-2107061	Views on LoS/NLoS Identification and Mitigation	Nokia, Nokia Shanghai Bell

R1-2107096	NLOS Mitigation Enhancements	FUTUREWEI

R1-2107135	Discussion on multipath/NLOS identification and mitigation for positioning enhancement	China Telecom

R1-2107217	Discussion on multipath/NLOS mitigation for NR positioning	OPPO

R1-2107349	Multipath Reporting in NR Positioning	Qualcomm Incorporated

R1-2107546	Discussion on multipath/NLOS mitigation for positioning	LG Electronics

R1-2107594	Solutions for Mitigation of NLOS Problem for NR Positioning	Intel Corporation

R1-2107648	Discussion on multipath/NLOS mitigation for positioning	InterDigital, Inc.

R1-2107744	Views on potential enhancements for NLOS mitigation in Rel-17 positioning	Apple

R1-2107862	Discussion on multipath/NLOS mitigation for NR positioning	NTT DOCOMO, INC.

R1-2107924	Potential enhancements for multipath/NLOS mitigation	Xiaomi

R1-2108104	Potential positioning enhancements for multipath/NLOS mitigation	Fraunhofer IIS, Fraunhofer HHI

R1-2108145	Accuracy enhancements based on NLOS/Multipath Information Reporting	Lenovo, Motorola Mobility

R1-2108168	Potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation	Ericsson

R1-2108175	Discussion on enhancements of multipath/NLOS reporting from UE and gNB	CEWiT

[106-e-NR-ePos-05] Email discussion/approval on potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation with checkpoints for agreements on August 19, 24 and 27 – Ryan (Nokia)

### Others

Including aspects for RAN2-led on-demand transmission and reception of DL PRS for DL and DL+UL positioning for UE-based and UE-assisted positioning solutions, and RAN2-led methods/measurements/signalling and procedures to support positioning for UEs in RRC_ INACTIVE state, for UE-based and UE-assisted positioning solutions

R1-2106554	Discussion on items led by RAN2 for NR positioning	ZTE

R1-2106600	Discussion on inactive state positioning and on-demand PRS	vivo

R1-2106814	Considerations on positioning in RRC Inactive and on-demand PRS	Sony

R1-2106893	Discussion on on demand positioning and positioning in inactive state	Samsung

R1-2106976	Discussion on on-demand DL PRS and positioning for UEs in RRC_ INACTIVE state	CATT

R1-2107062	Additional views on Inactive Mode Positioning and on-demand PRS	Nokia, Nokia Shanghai Bell

R1-2107170	Discussion on other enhancements for on-demand PRS and INACTIVE mode positioning	CAICT

R1-2107218	Discussion on positioning for UE in RRC_INACTIVE and on-demand PRS	OPPO

R1-2107350	Enhancements Related to On Demand PRS And Positioning in RRC Inactive State	Qualcomm Incorporated

R1-2107407	Discussion on RAN2-led items for positioning	CMCC

R1-2107595	On-demand DL PRS Signalling and NR Positioning for UEs in RRC-INACTIVE state	Intel Corporation

R1-2107649	Discussion on on-demand PRS and INACTIVE mode positioning	InterDigital, Inc.

R1-2107664	Discussion on RAN2 led objectives for NR positioning	Huawei, HiSilicon

R1-2107830	Potential physical layer impact to the RAN2-led topics	MediaTek Inc.

R1-2107831	Discussion on other enhancements for positioning	LG Electronics

R1-2107863	Discussion on positioning for UEs in RRC_INACTIVE state	NTT DOCOMO, INC.

R1-2107925	On-demand PRS and positioning for in-active state UE	Xiaomi

R1-2108105	Considerations on SRS transmission for positioning in RRC_INACTIVE state	Fraunhofer IIS, Fraunhofer HHI

R1-2108146	Discussion on On-Demand PRS and RRC_INACTIVE Positioning	Lenovo, Motorola Mobility

R1-2108169	On-demand transmission and reception of DL PRS for DL and DL+UL positioning	Ericsson

[106-e-NR-ePos-06] Email discussion/approval on issues in the Others section including the LSs in R1-2106411 and R1-2106412 from AI5 and any reply LSs necessary, with checkpoints for agreements on August 19, 24 and 27 – Alexey (Intel)

[106-e-NR-ePos-07] Email discussion/approval on LS in R1-2106435 from AI5 and any reply LS as necessary, with checkpoints for agreements until August 24 – Ericsson (Florent)