3GPP TSG RAN WG1 #106b-e																   R1-2108815

e-Meeting, October 11th – 19th, 2021

Agenda Item:		8.1.5

Source:				InterDigital, Inc.

Title:	Performance of Multi-Panel Multi-TRP MIMO

Document for:		Discussion and Decision

# Introduction

The work on the WID for Rel-17 eMIMO [1] kicked off in the RAN1 #102-e meeting. EVM assumptions were agreed during RAN1 #102-e and the assumptions for the baseline scenarios were captured in [2]:

The three proposals on R1-2007151 on the evaluation methodology for multi-beam enhancement are agreed.

In the previous meetings, we provided a study on the potential gains achievable by using multi-panel UEs in a multi-TRP transmission scenario [3]. In this contribution, we provide our views based on our simulation results and on the progress achieved so far in Rel-17 feMIMO.

# Background

The use cases of MIMO evolve as new scenarios emerge. For example, FD-MIMO was introduced in LTE by employing two-dimensional antenna arrays. By giving a TRP the ability to steer its beams, FD-MIMO provides a better spatial coverage of UEs located in a 3D space. The work in NR MIMO is built upon the significant progress achieved with FD-MIMO that is being extended to operate in the FR2 range. This means that new UE architectures with multi-panel capability should be further studied to better understand their potential use cases and benefits.

The shift in NR towards higher operational frequencies (e.g. FR2 and beyond 52 GHz) to support higher bandwidths enables manufacturers to pack more panels on a UE. This means that multi-panel UE are expected to become more and more mainstream. They enable the UE to offset the performance loss due to poor channel conditions on a panel by providing a different panel where channel conditions are better. Moreover, having multiple panels on different orientations enable the UE to mitigate MPR/MPE effects because a UE can dynamically switch to a panel which is unobstructed and can transmit at its highest power.

In this contribution, we provide some system level simulation results in different NR MIMO FR2 deployments where UEs are equipped with multi-panels. We also study the impact of using multi-TRPs with different assumptions on UE panel usage.

# Simulation assumptions

We use the agreed simulation assumptions from RAN1 #102-e meeting [2]. The deployments are evaluated at the carrier frequency of 30 GHz. The UEs are uniformly distributed within the deployment area. UEs are assumed to be equipped with four antenna panels that are placed on front, left, back, and right of the device. In this contribution, we provide new results with up to 4 antenna panels with the effect of blocking simulated by randomly choosing a panel and adding a 10 dB blocking loss. We evaluate cases when 1, 2, 3, or 4 panels are active. Employed antenna arrays are based on uniformly distributed structures. Cases with single and two TRPs are considered, and the effect of combining is also studied.

Both UEs and TRPs are assumed to have analog beamforming capability covering multiple spatial directions. UE and TRP sweep through all possible TX/RX beam-pairs to find the one which yields the highest RSRP. We consider several cases for pairing a UE with a TRP:

Single TRP

Case 1:

- UE activates only one panel. The selection of the panel is random to simulate random holding/orientation by/of the user.

- UE finds the TRP that results in the best beam-pair to the activated panel.

Case 2:

- UE activates more than one panel.

- UE finds the TRP that results in best beam-pairs per panel, but only selects the best of the two beam-pairs.

Case 2a:

- UE activates more than one panel.

- UE finds the TRP that results in best beam-pairs per panel, UE combines the signal received by the panels.

Multiple TRP

Case 3:

- UE activates only one panel. The selection of the panel is random to simulate random holding/orientation by/of the user.

- UE finds the two TRPs that result in the best beam-pair for the activated panel. UE receives and processes the over-the-air combined signals.

Case 3a:

- The UE activates more than one panel.

- UE finds the two TRPs and the panel that results in the best beam-pair for the activated panel. UE receives and processes the over-the-air combined signals.

Case 4:

- The UE activates more than one panel.

- UE finds the two TRPs that results in best beam-pairs per panel, UE receives and processes the signal received by the panels.

A detailed list of simulation assumptions is available in the Appendix.

# Evaluation results

In Figure 1, we show the results for cases with a single TRP in an open office scenario, and compare the performance with different assumptions on activated panels and combining mechanism. We notice that case 2 with 2 panels provides a substantial 10-12 dB dB gain over case 1 because both panels are active and allow the UE to search for the best possible beam-pair. This shows that beam diversity through multiple active panels is an effective way to mitigate the significant losses due to blocking in FR2. With 3 and 4 panels, the gains are even more substantial due to more available choices for beam diversity. In an open office scenario, the TRPs are densely packed within a room which gives several potential candidate TRPs. Nevertheless, being able to find the best panel orientation towards a serving TRP makes a significant difference. In Case 2a, where multiple panels are activated and combining is used, an additional 2-3 dB gain is obtained. Given the panel placement at the UE, it is always likely that one of the panels is facing more towards a serving TRP than the other ones. The panel with less favorable orientation might be still helpful in reception, but obviously its benefit is limited due to its direction. With 3 and 4 panels, the combining gain is more favourable than 2 panels due to better choices for combining. We notice that the more substantial gain is from 1 to 2 panels, the gains diminish with the additional panels. Figure 3 shows the same cases as Figure 1assuming UMa scenario. We notice similar gains from using the best panel pair over using a random panel pair.

Observation 1: Multiple activated panels yield significant gain when single TRP is used.

In Figure 2, we plot the cases with multi-TRP in an open office scenario. We notice that similar to the single TRP case, case 3a with best panel to different TRPs provides significant gains of ~10 dB compared to random panel activation. Given the density of cells, it is easy for a UE to find two TRPs with the best possible panel orientation and proper beam directions. Case 4 shows that using another panel may help in some cases to find a different TRP than the serving TRP for the other panel. However, in some cases, there are UEs for which both best beam-pairs are on the same panel. Comparing to Figure 1, we see that case 2a with single TRP and multi-panel yields almost the same performance as case 3a and 4. Substantial gains can be achieved when both panels are activated to search for best beam-pairs without needing a second TRP. A similar observation is made in the UMa scenario between Figures 3 and 4.

Observation 2: Similar gains can be achieved with single TRP multi-panel compared to multiple TRPs.

In Figure 4, we plot the same cases as Figure 2 but assuming the UMa scenario. The best panel activated yields ~10 dB gain over random activation. The TRPs are spaced wide apart, resulting a small number of TRPs compared to the open office. This results in having fewer suitable beam-pair combinations than InH. Then, compared to the open office case, it is hard for a UE to find a second TRP that can significantly help the serving cell. Therefore, there may not be a beam-pair with a significantly better performance than the others. So the gain from single panel to multi-panel is significant but not as large as in the InH scenario. As shown in Figure 4, case 4 performs slightly better than case 3a, because of using another active panel could provide some gain for most UEs. With a sparse TRP deployment and less beam options, it’s less likely to find two good beam-pairs from one panel other than the best one to the closest TRP. Then a second panel with a different orientation helps to find alternative TRPs that are in other directions. We notice that the use of 3 and 4 panels compared to 2 gives an additional gain and, similarly to InH, the more substantial gains are obtained when going from 1 panel to 2 panels.

Observation 3: Multi-panel is beneficial for multi-TRP when TRPs are sparsely deployed.

Figure 1																Figure 2

Figure 3 																Figure 4

In Figure 5-8, we show the gain of single and multi-TRP over the baseline Case 1 for the 5,50, and 95%-tile points in InH and UMa scenarios.

In Figure 5-6, the performance for InH is shown. The gains for different UE percentiles are similar. In contrast, in the UMa case figure 7-8, the gains increase going from 5 to 50 to 95%-tile UEs. The TRPs are distant so cell-edge UEs have poor signal quality to any TRP. Using multiple panels helps the cell-edge UEs find a better beam pair but the improvement is limited because the next best beam pair is still towards a TRP far from the UE. For the 50 and 95%-tile UEs, the multi-panel can make a significant difference to find the best beam pair to a TRP that is relatively close to the UE.

Observation 4: With sparse TRP deployment, the multi-panel gains are larger for 50 and 95%-tile UEs compared to 5%-tile.

Figure 5 – Single TRP												Figure 6 – Multi-TRP

Figure 7 – Single TRP 												Figure 8 – Multi-TRP

Based on these simulation results, it is apparent that multiple panels on a UE provide increased diversity. It is therefore our preference that the specification introduce changes to maximize the potential of multi-panel UEs. Introducing a panel ID is our preferred solution to enable UE and NW to align their operations on a per panel basis. For example, a CSI-RS resource can be associated to a panel ID so that the UE can perform a CSI report per panel. Alternatively, the TCI may include a panel ID to enable the NW to control panel selection and activation.

Proposal 1: Support introducing a panel ID.

The choice of panel also determines the optimal beamformer. The TRP-panel pair used for a transmission affects the choice of PMI. The transmission mode also affects the optimal beamformer. As part of Rel-17 CSI enhancements, a single CSI report settings can be configured with different measurement hypothesis to support sTRP and mTRP feedback. For example, a UE determines the optimal choice of PMI in a sTRP transmission mode; however, this PMI is not always the same for a mTRP transmission scheme such as NCJT. The optimal sTRP PMI may be suboptimal for NCJT due to inter-layer interference. Then it is best if the UE reports separate PMIs for each measurement hypothesis. In some cases, the same PMI can be optimal or near optimal for both sTRP and NCJT; then to avoid the excess overhead, the UE can report a single PMI which is applicable to both transmission schemes.

Proposal 2: In a single CSI report setting, support the option to report per TRP PMI for sTRP and NCJT measurement hypothesis.

Furthermore, to better support multi-panel UE operation, TCI state group could be considered. For example, two TCI state groups can be supported and each of the TCI state groups are associated with a different panel. Based on the configured TCI state groups, gNB can indicate a TCI state group for UE operation. The actual TCI state to be used for UE operation can be determined based on UE reporting for MPE mitigation. For example, a TCI state, associated with reported SSBRI(s)/CRI(s), in the indicated TCI state group can be used.

Proposal 3: Introduce a TCI state group-based indication and UE-centric determination of a TCI state in the indicated TCI state group.

# Conclusions

This contribution evaluated system level performance of NR MIMO in an FR2 deployment with multi-panel UEs and multi-TRP. Based on the presented discussion, we make the following observations and proposals:

Observation 1: Multiple activated panels yield significant gains when single TRP is used.

Observation 2: Similar gains can be achieved with single TRP multi-panel compared to multiple TRPs.

Observation 3: Multi-panel is beneficial for multi-TRP when TRPs are sparsely deployed.

Observation 4: With sparse TRP deployment, the multi-panel gains are larger for 50 and 95%-tile UEs compared to 5%-tile.

Proposal 1: Support introducing a panel ID.

Proposal 2: In a single CSI report setting, support the option to report per TRP PMI for sTRP and NCJT measurement hypothesis.

Proposal 3: Introduce a TCI state group-based indication and UE-centric determination of a TCI state in the indicated TCI state group.

# References

- RP-193133, “New WI: Further enhancements on MIMO for NR”, Samsung, 3GPP RAN Meeting #86, Sitges, Spain, December 9-12, 2019

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #102-e, e-Meeting, August 2020

- R1-2106647, “Performance of Multi-Panel Multi-TRP MIMO, InterDigital, 3GPP TSG RAN WG1 Meeting #106-e, e-Meeting, August 2021

# Appendix

Table 1 InH Open Office deployment

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 30 GHz |
| Channel Model | Indoor in TR 38.901 |
| TXRU mapping to antenna elements | One TXRU per panel per polarization |
| TXRU mapping weights | 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT, i.e., 2D sub-array partition model defined in TR36.897. |
| Criteria for beam selection for serving TRP | Select the best beam-pair among the limited set of DFT beams, based on the criteria of maximizing receive power after beamforming. |
| Constraints for the range of selective beams per TRP sector | With analog TX/RX beamforming, using a single digital TX/RX port. The first TXU/RXU in a single fixed panel is used with single-pol. - Beam directions for TRP: - Azimuth angle [pi/16:pi/8:15*pi/16] - Zenith angle [pi/8 3*pi/8 5*pi/8 7*pi/8] - Beam directions for UE: - Azimuth angle [pi/2:pi/30:2*pi/3] |
| ISD | 20m |
| BS Tx power | 23dBm |
| Number of sites | 12 |
| BS Antenna Configuration | (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2). (dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (2.0, 4.0) λ |
| UE Configuration | Number/location of panels: up to 4 panels (left, right, back, and front) (M, N, P, Mg, Ng) = (1, 4, 2, 1, 3); (dH) = (0.5)λ. (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+90; Ω0,2=Ω0,0+180; Ω0,3=Ω0,0+270 The polarization angles are 0 and 90 |
| UE array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 0 degree, ΩUT, = 0 degree |
| BS antenna pattern | See wall-mount in Table A.2.1-7 in TR 38.802 |
| UE antenna pattern | See Table A.2.1-8 in TR 38.802 |
| BS antenna height | 3m |
| UE antenna height | 1.5 m |
| UE antenna gain | 5dBi |
| Noise figure for BS | 7dB |
| UE receiver noise figure | 10dB |
| UE distribution | 100% Indoor, 3km/h, 10 users per BS |
| Blockage | Only one panel is blocked. The blocked panel is randomly selected at each drop. - Blocking entails an additional pathloss of 10 dB. |
| Metric | CDF of RSRP |

Table 2 UMa deployment

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 30 GHz |
| Channel Model | UMa in TR 38.901 |
| TXRU mapping to antenna elements | One TXRU per panel per polarization |
| TXRU mapping weights | 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT, i.e., 2D sub-array partition model defined in TR36.897. |
| Criteria for beam selection for serving TRP | Select the best beam-pair among the limited set of DFT beams, based on the criteria of maximizing receive power after beamforming. |
| Constraints for the range of selective beams per TRP sector | With analog TX/RX beamforming, using a single digital TX/RX port. The first TXU/RXU in a single fixed panel is used with single-pol. - Beam directions for TRP: - Azimuth angle [-5*pi/16:pi/8:5*pi/16] - Zenith angle [5*pi/8 7*pi/8] - Beam directions for UE: - Azimuth angle [pi/2:pi/30:2*pi/3] |
| ISD | 200m |
| BS Tx power | 40dBm |
| Number of sites | 7 |
| BS Antenna Configuration | (M,N,P,Mg,Ng) = (4,8,2,2,2), (dH,dV) = (0.5, 0.5)λ , (dg,H,dg,V) = (4.0, 2.0)λ |
| BS array orientation | azimuth 0 degree; mechanic downtilt: 0 degree |
| UE Configuration | Number/location of panels: up to 4 panels (left, right, back, and front) (M, N, P, Mg, Ng) = (1, 4, 2, 1, 3); (dH) = (0.5)λ. (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+90; Ω0,2=Ω0,0+180; Ω0,3=Ω0,0+270 The polarization angles are 0 and 90 |
| UE array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 0 degree, ΩUT, = 0 degree |
| BS antenna pattern | See Table A.2.1-6 in TR 38.802 |
| UE antenna pattern | See Table A.2.1-8 in TR 38.802 |
| BS antenna height | 25m |
| UE antenna height | Same as 3D-UMa in TR36.873 |
| UE antenna gain | 5dBi |
| Noise figure for BS | 7dB |
| UE receiver noise figure | 10dB |
| UE distribution | 100% Outdoor in cars: 30km/h, 10 users per TRP |
| Blockage | Only one panel is blocked. The blocked panel is randomly selected at each drop. - Blocking entails an additional pathloss of 10 dB. |
| Metric | CDF of RSRP |