3GPP TSG RAN WG1 Meeting #106bis-e	R1-2108834

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.17.2

Source:	Futurewei

Title:	UE features for supporting NR from 52.6 GHz to 71 GHz

Document for:	Discussion/Decision

# Introduction

This contribution discusses the preliminary RAN1 UE features for Rel-17 NR provided from WI rapporteurs in R1-2108426.

The following changes were proposed and marked in red in the table:

- Change the Feature group 24-1 title from “General” to “Basic” given that this feature will be pre-requisite for all other features, and is mandatory when FR2-2 is supported

- Move Feature 24-2 into Feature 24-1. We consider the support 120 kHz SSB for initial access as basic, because 120kHz SCS is mandatory (per WID) if FR2-2 is supported. While the initial version suggests that this should be mandatory only in stand alone deployment, we note that the other SCS (480kHz, 960 kHz) are both optional, therefore no necessary supported for NSA deployment.

- Indication for all features as “Optional with capability signaling”, similar as in other features groups, for instance 10-xx

- Indication where the support of the features needs to be known by gNB

- Indication of Mandatory for FR2-2 support for the LBT modes given that LBT is mandatory supported in some regions

- Indication of type support (per UE/peband, etc.)

- Addition of the agreements for the support of PRACH sequences, and multi-slot PDCCH monitoring

RAN1 should consider identifying whether the use cases scenarios specific for FR2-2 are necessary to be added to this table, after the channel access is futher refined, similar as in FG 10 NR-Unlicensed.

# References

[1] R1-2108426, Updated RAN1 UE features list for Rel-16 NR after RAN1#106-e

- NR_ext_to_71GHz

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-1 | General Basic FR2-2 support | 1. Support 120KHz SCS transmission and reception 2. Support multi-RB PUCCH format 0/1/4 3. PRACH with 120KHz SCS and length 139/571/1151 4. Support 120 kHz SSB for initial access |  |  |  | FR2-2 is not supported | Per UE |  |  |  |  | Optional with capability signaling This feature is the basic feature for the support of FR2-2 |
| 24. NR_ext_to_71GHz | 24-2 | 120KHz SSB based stand-alone support | 1. Support 480KHz 120 kHz SSB for initial access | 24-1 |  |  |  |  |  |  |  |  |  |
| 24. NR_ext_to_71GHz | 24-3 24-2 | 480KHz SSB based stand-alone support | 1. Support 480KHz SSB for initial access | 24-1, 24-3 24-2, 24-4 | Yes |  |  | Per UE |  |  |  | From WID: - In addition to 120kHz, support 480 kHz SSB for initial access with support of CORESET#0/Type0-PDCCH configuration in the MIB with following constraints: - Note: 480 kHz is an optional SSB numerology for initial access for the UE. A UE supporting a band in 52.6-71 GHz must at least support 120 kHz SCS (for initial access and after initial access) | Optional with capability signaling |
| 24. NR_ext_to_71GHz | 24-4 24-3 | 480KHz SCS support | 1. 480KHz SCS for UL transmission 2. 480KH SCS for DL reception 3. 480KHz for SSB monitoring 4. Multiple-slot PDCCH monitoring for 480KHz with X=4 5. PRACH with 480KHz and length 139/[571] | 24-1 | Yes |  |  | Per UE |  |  |  | From WID: In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported. Agreement: A UE supporting 480 kHz SCS supports multi-slot PDCCH monitoring for 480 kHz SCS. Agreement: Do not support PRACH length L=571, 1151 for 960kHz PRACH and at least L =1151 for 480kHz PRACH. | Optional with capability signaling |
| 24. NR_ext_to_71GHz | 24-5 24-4 | 960KHz SCS support | 1. 960KHz SCS for UL transmission 2. 960KH SCS for DL reception 3. 960KHz for SSB monitoring 4. Multiple-slot PDCCH monitoring for 960KHz with X=8 5. PRACH with 960KHz and length 139 | 24-1 | Yes |  |  | Per UE |  |  |  | From WID In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported Agreement: A UE supporting 960 kHz SCS supports multi-slot PDCCH monitoring for 960 kHz SCS. Agreement: Do not support PRACH length L=571, 1151 for 960kHz PRACH and at least L =1151 for 480kHz PRACH. | Optional with capability signaling |
| 24. NR_ext_to_71GHz | 24-? | Cat 3 or Cat 4 LBT support for FR2-2 unlicensed operation | 1. Cat 3 or 4 LBT support (not agreed yet if CW is supported, so it can be either Cat 3 or Cat 4 LBT for now. Will update when we have agreement) | 24-1 |  |  |  | Per band/ Per cell |  |  |  |  | Optional with capability signaling This feature is a basic feature for FR2-2 support where required by the regulators |
| 24. NR_ext_to_71GHz | 24-? | Cat 2 LBT support for FR2-2 unlicensed operation | 1. Support Cat 2 LBT | 24-1 |  |  |  |  |  |  |  |  | Optional with capability signaling This feature is a basic feature for FR2-2 support where required by the regulators |