3GPP TSG RAN WG1 #106bis-e                             R1-2108970

e-Meeting, October 11th – 19th, 2021

Source: 	vivo

Title:  	Discussion on propagation delay compensation enhancements

Agenda Item:	8.3.4

Document for:	Discussion and Decision

Introduction

In RAN1#106 e-meeting, the equations on RTT-based method are discussed and the following agreements are achieved[1].

Agreement

Take the following two alternatives as the equation for evaluation of the overall time synchronization error for RTT-based propagation delay compensation. RAN1 to select one of the alternatives in RAN1#106bis-e.

- Alt. 1:

  - errorRxTxDiff, indication is to reflect the error due to indication granularity of Rx-Tx time difference

  - errorgNB,RxTxDiff and errorUE, RxTxDiff reflects the measurement inaccuracy of gNB Rx-Tx time difference, and the measurement inaccuracy of UE Rx-Tx time difference, respectively.

  - Note: The equation may be updated after clarification on the gNB TX-RX timing difference and UE TX-RX timing difference

- Alt. 2:

  - errorRxTxDiff,indication is to reflect the error due to indication granularity of Rx-Tx time difference

  - Note: Alt.2 assumes that gNB can coordinate the time of TA procedure and the time of PD compensation, so that the DL frame timing error and BS transmit timing error for propagation delay estimation is correlated to (e.g. the same as) that for the transmission of RRC signaling carrying the reference time clock

Note: FFS whether / how to handle inconsistent RTT measurement in gNB and UE due a change of uplink TX timing

Agreement

If RTT-based propagation delay compensation is supported,

- CSI-RS for tracking (TRS) can be used for Rx – Tx time difference estimation at UE side, if PRS is not configured for the UE.

- PRS can be used for Rx – Tx time difference estimation at UE side, if PRS is configured for the UE.

Agreement

SRS can be used for Rx – Tx time difference estimation at gNB side for RTT-based propagation delay compensation, if RTT-based propagation delay compensation is supported.

Agreement

Support the following configurations for RTT-based propagation delay compensation, if RTT-based propagation delay compensation is supported.

- At least one CSI-RS for tracking (TRS) configuration for Rx – Tx time difference estimation at UE side if PRS is not configured

- At least one SRS configuration for Rx – Tx time difference estimation at gNB side

According to LS from RAN2, the suggested single Uu interface budgets are listed in the following table [2]

| Scenario | Single Uu interface Budget |
| --- | --- |
| Control-to-Control | ±145ns to ±275ns |
| Smart Grid | ±795ns to ±845ns |

In this contribution, we share our views on propagation delay compensation enhancements.

RTT-based method

Details for RTT-based method

TA-based method has been studied and applied widely in NR and previous system. For RTT-based method, CSI-RS, SRS and PRS (if configured) can be used for Rx – Tx time difference estimation. The achievable accuracy and corresponding RS bandwidth need RAN4 evaluation and input. Generally, the large bandwidth or density of reference signal is beneficial to guarantee accuracy. In IIOT scenario, the larger bandwidth or density of reference signal may be required for all UEs if high accuracy delay compensation is needed. This leads to the higher overhead of reference signal, especially for UE specific reference signal.

Observation 1: For RTT-based method, the achievable accuracy and corresponding RS bandwidth need RAN4 evaluation and input.

RTT-based timing error evaluation

For RTT-based method, propagation delay compensation can be performed at gNB side or at UE side. There is no obvious difference between these two methods. For compensation at gNB side, UEs report Rx-Tx time difference to gNB and gNB executes PDC, which may have the potential RAN3 impact. For compensation at UE side, UEs receive signalling of Rx-Tx time difference from gNB and execute PDC. The compensation at UE side has no RAN 3 impact compared to compensation at gNB side.

In this section, taken RTT-based gNB compensation as an example, the total error for RTT-based PDC is

errortotal, RTT=errorBS, DL, TX+errorPDL,  RTT+errorUE, DL, RX

The relation of transmitting and receiving signals for gNB and UE is shown in Figure 1.

Figure 1 The relation of transmitting and receiving signals between gNB and UE

In Figure 1,

∆t1+∆t2 =TBS,Rx-TBS,Tx)+(TUE,Rx-TUE,Tx+errorUE,DL,Rx+errorBS,DL,Tx+ errorBS,UL,Rx+errorUE,UL,Tx

Where, errorUE,DL,Rx+errorUE,UL,Tx can be represented by Rx-Tx time difference inaccuracy at UE side, i.e., errorUE, RxTxDiff. errorBS,DL,Tx+ errorBS,UL,Rx can be represented by Rx-Tx time difference inaccuracy at gNB, i.e., errorBS,RxTxDiff.

When using d=(∆t1+∆t2)2 to estimate propagation delay, the uncertainties for PL are

errorPDL,  RTT=errorBS,RxTxDiff+errorUE, RxTxDiff+errorRxTxDiff, indication2

Where errorRxTxDiff, indicationis the error caused by the reporting granularity.

Then, the total uncertainties for RTT-based method can be derived by:

errortotal, RTT=errorBS, DL, Tx+errorUE,DL,Rx+errorBS,RxTxDiff+errorUE, RxTxDiff+errorRxTxDiff, indication2               (1)

According to the conclusion in RAN 4[3], ±[137+]ns is applied for errorUE, RxTxDiff for 15kHz in fading channel based on TDL-A (30 ns delay spread, 5Hz) channel for FR1 and the discussion on  value is still open. We assume the PDC evaluation can base on FR1 firstly. However, even for FR1, the error is not determined for 30kHz case.

For errorBS,RxTxDiff, only the accuracy requirements for AWGN propagation conditions are provided for both 15kHz and 30kHz. There are no available values for fading channel.

For errorRxTxDiff, indication, the error caused by the reporting granularity is from Tc to 32Tc according to TS38.133. Here we use 32Tc as errorRxTxDiff, indication for evaluation. The inaccuracy due to the granularity of reporting the Rx-Tx time difference = 32*Tc = 16 ns.

Parameters for RTT-based method are shown in Table 1 for both control-to-control and smart grid use case.

Table 1: RTT-based timing error in clock synchronisation

| Factors | Timing error type | Requirement for different SCS (kHz) (unit: ns) |  |
| --- | --- | --- | --- |
|  |  | 15 kHz | 30 kHz |
| 1.1 | Frame alignment error of gNB transmitter (errorBS, DL, Tx) | 32.5 | 32.5 |
| 1.2 | Inaccuracy caused by UE detection (errorUE,DL,Rx) | 100 | 100 |
| 1.3 | Difference between Rx and Tx at UE side (errorUE,RxTx diff) | [137+], is FFS | TBD in fading channel |
| 1.4 | Difference between Rx and Tx at gNB side (errorBS,RxTx diff) | TBD in fading channel | TBD in fading channel |
| 1.6 | Inaccuracy of reporting errorRxTxDiff, indication | 16 | 16 |

According to Table 1, we can observe some parameters are still not available for evaluation of RTT-based PDC, e.g.,

  - errorUE,RxTx diff for 30kHz and  value for 15kHz

  - errorBS,RxTx diff for fading channel

Observation 2: For evaluation of RTT-based PDC, the following parameters should be determined:

  - errorUE,RxTx diff for 30kHz and  value for 15kHz

  - errorBS,RxTx diff for fading channel

Conclusions

Based on the analysis given above, we have the following Observations:

Observation 1: For RTT-based method, the achievable accuracy and corresponding RS bandwidth need RAN4 evaluation and input.

Observation 2: For evaluation of RTT-based PDC, the following parameters should be determined:

  - errorUE,RxTx diff for 30kHz and  value for 15kHz

  - errorBS,RxTx diff for fading channel

Reference

[1] Chairman notes RAN1#106 e-meeting

[2] R2-2010837 Reply LS on propagation delay compensation enhancement

[3] R4-2108300 Big CR 38133-NR positioning RRM perf_R16