3GPP TSG RAN WG1 Meeting #106bis-e	R1-2109146

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.17.4

Source:	Huawei, HiSilicon

Title:	Rel-17 UE features for NR NTN

Document for:	Discussion and Decision

- Introduction

In RAN#106-e, the Rel-17 UE features for NR NTN was included in the preliminary RAN1 UE features for Rel-17 NR in [1].

- Discussion

For FG 26-3 in [1], the feature of “Start of RAR window” is in the scope of the “enhancement on the timing” and  all the agreed timing relationship should be contained as component as follows：

- Enhancement on the timing relationship

  - estimate of UE-gNB RTT

  - delaying the start of ra-ResponseWindow by UE-gNB RTT

  - delaying the start of msgB-ResponseWindow by UE-gNB RTT

  - delaying the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource,  transmission of aperiodic SRS with cell-specific Koffset

  - delaying the UE action and assumption on a downlink configuration carried by MAC CE command by Kmac if it is indicated

In addition, one UE feature on UE-specific K_offset is added.

- Reception of UE-specific K_offset via MAC-CE

- delaying the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource,  transmission of aperiodic SRS with UE-specific Koffset

Proposal 1: Enhancement on the timing relationship is added as a UE feature and all the agreed time relationship enhancements should be listed as the components.

Proposal 2: UE-specific K_offset is added as a UE feature which allows UE-specific K_offset reception and application.

For FG 26-1, TA pre-compensation for preamble transmission is not considered and according to the agreement of RAN1#106-e, a validity timer for satellite ephemeris data or common TA if provided should be included in “Uplink Time pre-compensation” as follows:

- Uplink Time pre-compensation

  - UE specific TA calculation in RRC_IDLE and RRC_INACTIVE states based on its GNSS-acquired position and the serving satellite ephemeris.

  - UE specific TA calculation in RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris.

  - UE applies common TA according to the parameters provided by the network (if any)

  - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops

  - UE acquire the serving satellite ephemeris and common TA upon the expiration of the corresponding validity timer.

Proposal 3: For the feature of “Uplink Time pre-compensation”, include “UE acquire the serving satellite ephemeris and common TA upon the expiration of the corresponding validity timer” as one additional component.

According to the agreement of HARQ enhancement in RAN1#102-e, the feature of HARQ feedback disabling should also be taken into consideration as follows:

- Enabling/disabling on HARQ feedback

  - Enabling/disabling on HARQ feedback for downlink transmission should be at least configurable per HARQ process via UE specific RRC signaling.

Proposal 4: “Disabling of HARQ feedback” is listed as one feature with component of “Disabling on HARQ feedback for downlink transmission should be at least configurable per HARQ process via UE specific RRC signaling”.

For 26-8, although the enhancement of Type-2 HARQ codebook and Type-1 HARQ codebook are in the scop of HARQ codebook enhancement, they are two independent feature as they can be supported by the UE separately, i. e. a UE can only support Type-2 HARQ codebook or only support Type-1 HARQ codebook or support both.

Proposal 5: Enhancement of Type-2 HARQ codebook and enhancement of Type-1 HARQ codebook are listed as separate UE feature group.

For 26-8, the component of polarization signaling should take the RAN1#106-e agreement into consideration and the updated component is as follows:

- Support of polarization signaling in NR NTN

  - Support polarization indication reception in SIB indicating DL and/or UL polarization information using respective polarization type parameters to indicate: RHCP or LHCP or linearFFS: whether polarization signaling is per SSB

Proposal 6: For the feature of “Support of polarization signaling in NR NTN”, update the component according to the agreement of RAN1#106-e.

For FG 26-5 increasing the number of HARQ processes, it should be note that it was agreed that this feature is also beneficial for other features such as abve 52.5 GHz WI. Hence an independent feature can be defined without coupling it NTN or other features.

Proposal 7: For the feature of increasing the number of HARQ processes, an independent feature is defined which is not under NTN or any other Rel-17 features.

Regarding whether a FG is mandatory or optional, all NTN features should be defined as optional with capability signaling. However, for FG 26-1, 26-2 and 26-3, they can be viewed as the basic features for NTN since the UEs supporting NTN would need these features to get access to the system. For UE supporting NR NTN, it must indicate these FGs are supported.

Proposal 8: For UEs supporting NR NTN, they must indicate FG 26-1, 26-2 and 26-3 are supported.

The proposed relationship of UE features for NR NTN is given in Appendix.

Conclusion

In this contribution, we discuss the Rel-17 UE features for NR NTN. The following proposals are presented:

Proposal 1: Enhancement on the timing relationship is added as a UE feature and all the agreed time relationship enhancements should be listed as the components.

Proposal 2: UE-specific K_offset is added as a UE feature which allows UE-specific K_offset reception and application.

Proposal 3: For the feature of “Uplink Time pre-compensation”, include “UE acquire the serving satellite ephemeris and common TA upon the expiration of the corresponding validity timer” as one additional component.

Proposal 4: “Disabling of HARQ feedback” is listed as one feature with component of “Disabling on HARQ feedback for downlink transmission should be at least configurable per HARQ process via UE specific RRC signaling”.

Proposal 5: Enhancement of Type-2 HARQ codebook and enhancement of Type-1 HARQ codebook are listed as separate UE feature group.

Proposal 6: For the feature of “Support of polarization signaling in NR NTN”, update the component according to the agreement of RAN1#106-e.

Proposal 7: For the feature of increasing the number of HARQ processes, an independent feature is defined which is not under NTN or any other Rel-17 features.

Proposal 8: For UEs supporting NR NTN, they must indicate FG 26-1, 26-2 and 26-3 are supported.

References

- R1-2108679, “Preliminary RAN1 UE features list for Rel-17 NR”.

# Appendix

Proposed Rel-17 UE features for NR_NTN_solutions

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26. NR_NTN_solutions | 26-1 | Uplink Time pre-compensation | - UE specific TA calculation in RRC_IDLE and RRC_INACTIVE states based on its GNSS-acquired position and the serving satellite ephemeris. - UE specific TA calculation in RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris. - UE applies common TA according to the parameters provided by the network (if any) - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops - UE (re)acquire the serving satellite ephemeris and common TA upon the expiration of the corresponding validity timer. |  | No | No | Release 17 UE cannot access NTN |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 | An NTN UE is required to at least support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris. | Mandatory Optional with capability signalling For UE supports NR NTN, UE must indicate this FG is supported. |
| 26. NR_NTN_solutions | 26-2 | Uplink Frequency/Doppler pre-compensation | - In RRC_IDLE and RRC_INACTIVE states calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. - In RRC_CONNECTED states, calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. |  | No | No | Release 17 UE cannot access NTN |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 | An NR NTN UE shall be capable of at least using its acquired GNSS position and satellite ephemeris to calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. | Mandatory Optional with capability signalling. For UE supports NR NTN, UE must indicate this FG is supported. |
| 26. NR_NTN_solutions | 26-3 | Start of RAR window Enhancement on the timing relationship | - estimate of UE-gNB RTT - delaying the start of ra-ResponseWindow by UE-gNB RTT - delaying the start of msgB-ResponseWindow by UE-gNB RTT - delaying the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS with cell-specific K_offset - delaying the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated |  | No | No | Due to large propagation delay in NTN the UE RAR is not expected within a few milliseconds like in TN. The UE does not need to monitor the PDCCH for RAN right after transmitting RA Preamble. |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 |  | Mandatory Optional with capability signalling For UE supports NR NTN, UE must indicate this FG is supported. |
| 26. NR_NTN_solutions | 26-3a | UE-specific K_offset | - Reception of UE-specific K_offset via MAC-CE - delaying the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS with UE-specific Koffset | 26-3, 26-4 | Yes | No |  |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 |  | Optional with capability signalling |
| 26. NR_NTN_solutions | 26-4 | UE reporting of information about the UE specific TA pre-compensation | - The exact content of UE reporting of information about the UE specific TA pre-compensation - frequency of the reports - granularity of the reported content | 26-2 | Yes | No |  |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 |  | [Optional with capability signaling] |
| 26. NR_NTN_solutions | 26-5 | Increasing the number of HARQ processes | - The maximal supported HARQ process number is 32 for both UL and DL - Support on the maximal HARQ process number is up to UE capability |  | Yes | No | Increasing the number of HARQ processes avoids HARQ stalling, |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 |  | Optional with capability signalling |
| 26. NR_NTN_solutions | 26-6 | Disabling of HARQ feedback | - Disabling on HARQ feedback for downlink transmission should be at least configurable per HARQ process via UE specific RRC signaling. |  | Yes | No |  |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 |  | Optional with capability signalling |
| 26. NR_NTN_solutions | 26-7 | HARQ CODEBOOK Enhancement Type-1 HARQ codebook enhancement | - Enhancement on Type-2 HARQ codebook in NTN - Enhancement on Type-1 HARQ codebook in NTN | 26-6 | Yes | No |  |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 |  | Optional with capability signalling |
| 26. NR_NTN_solutions | 26-8 | Type-2 HARQ codebook enhancement | - Enhancement on Type-2 HARQ codebook in NTN | 26-6 | Yes | No |  |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 |  | Optional with capability signalling |
| 26. NR_NTN_solutions | 26-9 | NTN Performance enhancement | - The maximum number of supported aggregation factor (i.e., pdsch-AggregationFactor) for DL PDSCH is [X] FFS: X = 8, 16 or 32 |  | Yes | No | Particularly beneficial in case HARQ feedback is disabled |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 |  | Optional with capability signalling |
| 26. NR_NTN_solutions | 26-10 | Support of polarization signalling reception in NR NTN | - UE polarization capability (RHCP, LHCP, Linear) - Support polarization indication reception in SIB indicating DL and/or UL polarization information using respective polarization type parameters to indicate: RHCP or LHCP or linear |  | No | No |  |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 |  | Optional with capability signalling |