3GPP TSG RAN WG1 #106bis-e                            		R1-2109225

e-Meeting, October 11th– 19th, 2021

Source:	CATT

Title:	Further discussion on enhancements for UL-AoA positioning method

Agenda item:	8.5.2

Document for:	Discussion and Decision

Introduction

Most of the issues related to the enhancements for the UL-AoA positioning method were resolved in previous meetings. In this contribution, we further discuss the remaining issues related to the enhancements of UL-AoA (AoA/ZoA) measurement reporting for UL-AoA positioning.

Association of UL measurements with ARP

The following agreement was made in RAN1#106-e[1]:

Agreement: Further study and conclude whether the association of UL-AOA UL-TDOA, Multi-RTT measurements with ARP (Antenna Reference Point) information is supported at RAN1#106bis-e.

For supporting the use of the UL measurements to calculate the UE position, the TRP ID is currently provided together with measurement results in the MEASUREMENT RESPONSE message when gNB reports the positioning measurements for the target UE [2]. In addition, the gNB can also provide the Geographical Coordinates associated with the TRP ID to the LMF via TRP Information. In this way, when the LMF obtains the UL measurements, it has the information of the TRP that obtains the measurement and also the TRP’s coordinates, which are needed in positioning calculation.

A TRP may have multiple antennas (or antenna panels), which may have different antenna reference points (ARPs) with different geographical coordinates. In this case, if only the TRP ID with its coordinates is reported with the measurements for the position calculation, then the positioning accuracy will be impacted by the inaccuracy of the coordinates of the Rx antenna.

Thus, it is required for the gNB to report correctly the geographical coordinates of the ARP of the Rx antenna together with the UL measurements. In our view, this can be done in either of the following two options:

- Option 1: The gNB will assign a single TRP ID for each ARP of the Rx antenna panels in a cell. Given that TRP ID has a value range of [1 – 65535], there are enough TRP IDs to be used for each ARP of the Rx panels in a cell.

- Option 2: Reporting the ARP information, i.e., the coordinates of the ARP relative to the coordinate of a TRP, in the UL measurements (Note: This is similar to the DL PRS transmission, where the relative coordinates of the ARP to the TRP are provided by the gNB to the LMF).

For Option 1, there is no need to change the current specs.  For Option 2, there will be a small impact on the specs by adding the reporting of the ARP information associated with the UL measurements. In our view, Option 2 is preferred for the consistency of the DL and UL implementation.

- Proposal 1: It is preferable to support gNB reporting the ARP (Antenna Reference Point) information associated with UL-AoA, UL-TDOA, Multi-RTT measurements for the consistency of the DL and UL design.

- Note: The ARP information includes the coordinates of the ARP relative to the coordinate of a TRP as defined in TS 38.455.

UL-AoA measurements enhancements

The following agreements relating to the reporting of the UL measurements of the first arrival path in RAN1#106e[1]:

Agreement: Reporting of one UL-RTOA and multiple UL-AOAs measurements for the first arrival path per SRS resource for positioning and per SRS resource for MIMO in a single gNB report to LMF is supported - The above measurements are associated with SRS resource ID which is also reported to LMF - FFS: Reporting of RSRP for the first arrival path - Note: The use of SRS for MIMO resource is transparent to the UE - FFS: Reporting of gNB Rx-Tx Agreement: Reporting of one gNB Rx-Tx time difference and multiple UL-AOAs measurements for the first arrival path per SRS resource for positioning in a single gNB report to LMF is supported - The above measurements are associated with SRS resource ID which is also reported to LMF - FFS: Reporting of RSRP for the first arrival path

In TS 38.455, the TRP Measurement Result IE is defined as follows [2]:

| IE/Group Name | Presence | Range | IE Type and Reference |
| --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas=16384> |  |
| >CHOICE Measured Results Value | M |  |  |
| >>UL Angle of Arrival | M |  | 9.2.38 |
| >>UL SRS-RSRP | M |  | INTEGER (0..126) |
| >>UL RTOA | M |  | 9.2.39 |
| >>gNB Rx-Tx Time Difference | M |  | 9.2.40 |
| >Time Stamp | M |  | 9.2.42 |
| >Measurement Quality | O |  | 9.2.43 |
| >Measurement Beam Information | O |  | 9.2.57 |

In Rel-16, for the existing TRP Measurement Result IE, each TRP Measurement Result IE only reports one UL AoA, or one UL SRS-RSRP, or one UL RTOA, or one gNB Rx-Tx Time Difference, for the first path.

From the agreements of the last meeting, the following reporting {multiple UL AoAs, one UL RTOA} and {multiple UL AoAs, one gNB Rx-Tx Time Difference} were supported in Rel-17. It is still FFS on whether to support reporting these measurements together with SRS-RSRP, and it is also FFS on whether UL RTOA and gNB Rx-Tx Time Difference can be reported together. In our view, since all these measurements can be obtained from the same SRS resources, we don’t see any reason for not supporting the reporting of any of the combinations between {multiple UL AoAs, one UL SRS-RSRP, one UL RTOA, and one gNB Rx-Tx Time Difference} in the one single gNB report to the LMF.

Proposal 2: Support reporting any of the combinations of {multiple UL-AoAs measurements, one UL SRS-RSRP, one UL RTOA, one gNB Rx-Tx Time Difference} for the first arrival path per positioning SRS resource in a single gNB report to LMF.

- The above measurements are associated with positioning SRS resource ID that is also reported to LMF

Proposal 3: Support reporting any of the combinations of {multiple UL-AoAs measurements, one UL SRS-RSRP, one UL RTOA} for the first arrival path per MIMO SRS resource in a single gNB report to LMF.

- The above measurements are associated with MIMO SRS resource ID that is also reported to LMF

Conclusions

In this contribution, we discussed the remaining issues related to the enhancements for UL-AoA positioning with the following proposal.

- Proposal 1: It is preferable to support gNB reporting the ARP (Antenna Reference Point) information associated with UL-AoA, UL-TDOA, Multi-RTT measurements for the consistency of the DL and UL design.

- Note: The ARP information includes the coordinates of the ARP relative to the coordinate of a TRP as defined in TS 38.455.

Proposal 2: Support reporting any of the combinations of {multiple UL-AoAs measurements, one UL SRS-RSRP, one UL RTOA, one gNB Rx-Tx Time Difference} for the first arrival path per positioning SRS resource in a single gNB report to LMF.

- The above measurements are associated with positioning SRS resource ID that is also reported to LMF

Proposal 3: Support reporting any of the combinations of {multiple UL-AoAs measurements, one UL SRS-RSRP, one UL RTOA} for the first arrival path per MIMO SRS resource in a single gNB report to LMF.

- The above measurements are associated with MIMO SRS resource ID that is also reported to LMF

References

- 3GPP TSG RAN WG1 Meeting #106-e, RAN1 Chairman’s Notes.

- TR 38.455, NG-RAN; NR Positioning Protocol A (NRPPa)