3GPP TSG RAN WG1 #106bis-e	                                                                                              R1-2109548

e-Meeting, October 11th – 19th, 2021

Agenda Item: 8.1.4

Source: MediaTek Inc.

Title: CSI enhancement for NCJT and FR1 FDD reciprocity

Document for: Discussion & Decision

# Introduction

The objective for this agenda item, stated in [1], is given by

Enhancement on CSI measurement and reporting:

a.	Evaluate and, if needed, specify CSI reporting for DL multi-TRP and/or multi-panel transmission to enable more dynamic channel/interference hypotheses for NCJT, targeting both FR1 and FR2

b.	Evaluate and, if needed, specify Type II port selection codebook enhancement (based on Rel.15/16 Type II port selection) where information related to angle(s) and delay(s) are estimated at the gNB based on SRS by utilizing DL/UL reciprocity of angle and delay, and the remaining DL CSI is reported by the UE, mainly targeting FDD FR1 to achieve better trade-off among UE complexity, performance and reporting overhead

In this contribution, we continue the discussion of CSI enhancement based on the outcome of the previous meetings.

# CSI Enhancement: Multi-TRP

## CSI measurement/processing

In the RAN1#106e meeting, we agreed to study the time restriction for two CMRs in the same CMR pair [2]:

Agreement For CSI measurement associated with a CSI-ReportingConfig for NC-JT, study following restriction(s) for two CMRs within the same CMR pair configured for NCJT measurement hypothesis: - FFS: two resources are restricted within the same DL slot - FFS: two resources are restricted with the same CDRX active time

In order to reduce the overall CSI latency, ensure reduced impact of time-varying interference and guarantee high coherency, the two CMR resources configured for the NCJT measurement should not experience a large gap in time domain, preferably within the same DL slot.

Proposal 1: For CSI measurement associated with a CSI-ReportingConfig for NCJT, for two CMRs within the same CMR pair configured for NCJT measurement hypothesis two resources are restricted within the same DL slot.

In the RAN1#106e meeting, we agreed to study the power control offset between CSI-RS and PDSCH [2]:

Agreement For a CMR pair configured for a NCJT measurement hypothesis, study following Alternatives: - Alt 1: a separate powerControlOffset (Pc ratio) shall be configured for the NCJT measurement hypothesis by re-defining such Pc ratio as 10log10(P_PDSCH/P_CSIRS) dB, whereas - P_PDSCH is the energy of PDSCH ports with a same TCI state as the CMR on one subcarrier of one OFDM symbol - P_CSIRS is the energy of all CSI-RS ports of the CMR multiplexed on one subcarrier of one OFDM symbol - Alt 2: re-interpret two Pc ratios configured for the CMR pair for the NCJT measurement hypothesis, FFS detailed impact of specification - Alt 3: No change to definition or configuration of Pc ratio - Note that other solutions are not excluded.

As previously agreed powerControlOffset is the assumed ratio of PDSCH EPRE to NZP CSI-RS EPRE when UE derives CSI feedback. In order to keep the consistency of the powerControlOffset definition when applying to NCJT scenario we need to re-interpret the PDSCH EPRE and NZP CSI-RS EPRE to the following:

  - P_PDSCH is the energy of PDSCH ports with a same TCI state as the CMR on one subcarrier of one OFDM symbol

  - P_CSIRS is the energy of all CSI-RS ports of the CMR multiplexed on one subcarrier of one OFDM symbol

Proposal 2: For a CMR pair configured for a NCJT measurement hypothesis a separate powerControlOffset (Pc ratio) shall be configured for the NCJT measurement hypothesis by re-defining such Pc ratio as 10log10(P_PDSCH/P_CSIRS) dB, whereas

- P_PDSCH is the energy of PDSCH ports with a same TCI state as the CMR on one subcarrier of one OFDM symbol

- P_CSIRS is the energy of all CSI-RS ports of the CMR multiplexed on one subcarrier of one OFDM symbol

In the RAN1#106e meeting, we agreed to study CSI computation delay requirement [2]:

Agreement For CSI computation delay requirement associated with a CSI-ReportingConfig for a NCJT measurement hypothesis, study following alternatives: - Alt1: introducing new/relaxed values on Z and Z’, FFS exact values or other conditions - Alt2: No changes of values on Z and Z’

Additional CSI processing may be required at the UE side for NCJT hypothesis, as opposed to single TRP case. Hence, new processing timeline values, i.e., Z and Z’, needs to be introduced to reflect the new requirements.

Proposal 3: For CSI computation delay requirement associated with a CSI-ReportingConfig for a NCJT measurement hypothesis, new/relaxed values on Z and Z’ should be introduced. FFS exact values or other conditions

## CSI reporting

In the RAN1#106e meeting, it was agreed to further study UCI payload construction for reported CSIs [2]:

Agreement To confirm the order of UCI payload construction for reported CSIs, study following Alternatives and down-select one or more Alternative(s) for required specification changes in RAN1 106bis: - Alt 1: modify priority equation, i.e., Section 5.2.5 in 38.214. - Alt 2: modify the table of priority reporting levels for Part 2 CSI, i.e., Table 5.2.3-1 in 38.214. - Alt 4: modify mapping order of CSI fields of one CSI report, i.e., Table 6.3.2.1.2-3/4/5 in 38.212

First, it is desirable to have finer granularity for CSI omission so that gNB can at least acquire some partial CSI that are useful for scheduling. Second, considering limited schedule for R17, minimum specification impact is preferable. We propose that each CSI measurement hypothesis is mapped to a distinct CSI report. Comparing with mapping all hypotheses to a single CSI report, RAN1 does not need to further discuss how to divide the CSI into groups and then design new CSI omission rules accordingly.

Proposal 4: For Option 1 with X = 1, 2, each CSI measurement hypothesis is mapped to a distinct CSI report.

Then, the CSI priority formula needs to be updated. The following two CSI priority formulas are reasonable to us: Either

PriiCSIy,k,c,s,h=2∙Ncells∙Ms∙Nhypo∙y+Ncells∙Ms∙Nhypo∙k+Ms∙Nhypo∙c+Nhypo∙s+h,

or

PriiCSIy,k,c,s,h=2∙Ncells∙Ms∙Nhypo∙y+Ncells∙Ms∙Nhypo∙k+Ms∙Nhypo∙c+Ms∙h+s,

where

- y=0 for aperiodic CSI reports to be carried on PUSCH, y=1 for semi-persistent CSI reports to be carried on PUSCH, y=2 for semi-persistent CSI reports to be carried on PUCCH and y=3 for periodic CSI reports to be carried on PUCCH;

-	k=0 for CSI reports carrying L1-RSRP or L1-SINR and k=1  for CSI reports not carrying L1-RSRP or L1-SINR;

-	c is the serving cell index and Ncells is the value of the higher layer parameter maxNrofServingCells;

-	s is the reportConfigID and Msis the value of the higher layer parameter maxNrofCSI-ReportConfigurations

-	h=0 for the NCJT measurement hypothesis, h=1 for the first single-TRP measurement hypothesis, if reported, and h=2 for the second single-TRP measurement hypothesis, if reported. Nhypo is the number of CSI reports in a CSI reporting setting. If UE is configured to report one single CSI report for a CSI reporting setting, then h=0 and Nhypo=1.

Proposal 5: The CSI priority formula is updated as: Either

PriiCSIy,k,c,s,h=2∙Ncells∙Ms∙Nhypo∙y+Ncells∙Ms∙Nhypo∙k+Ms∙Nhypo∙c+Nhypo∙s+h,

or

PriiCSIy,k,c,s,h=2∙Ncells∙Ms∙Nhypo∙y+Ncells∙Ms∙Nhypo∙k+Ms∙Nhypo∙c+Ms∙h+s,

where h=0 for the NCJT measurement hypothesis, h=1 for the first single-TRP measurement hypothesis, if reported, and h=2 for the second single-TRP measurement hypothesis, if reported. Nhypo is the number of CSI reports in a CSI reporting setting. If UE is configured to report one single CSI report for a CSI reporting setting, then h=0 and Nhypo=1.

In the RAN1#106e meeting, we have the following agreement on RI restriction [2]:

Agreement For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, support RI restriction by selecting at most one alternative from the following in RAN1#106bis-e: - Alt 1: One RI restriction is configured per CodebookConfig, whereas the RI restriction is applied to both Single-TRP and NCJT measurement hypotheses. - If rank restriction of X is configured, reported rank is X for a Single-TRP measurement hypothesis and sum of two reported ranks is X for a Multi-TRP measurement hypothesis. - Alt 2: Two RI restrictions can be configured per CodebookConfig, whereas one RI restriction is applied to one CMR group in a CMR resource set respectively, i.e. per TRP. - If rank restriction of (X, Y) is configured, reported rank is X for the CMR in the first CMR group and Y for the CMR in the second CMR group, regardless single-TRP and NCJT measurement hypotheses. - Alt 3: Multiple RI restrictions can be configured per CodebookConfig, whereas RI restriction is applied to per each CMR in CMR pair for NCJT and per each CMR for Single-TRP. - Alt 4: Two RI restrictions can be configured per CodebookConfig, whereas one RI restriction is applied to all Single-TRP measurement hypotheses, and another one is applied to all NCJT measurement hypotheses. - If rank restriction of (X, Y) is configured, reported rank is X for all single-TRP measurement hypotheses and reported rank (1 out of 4 possible rank combinations) is Y for all NCJT measurement hypotheses. - Alt 5: Three RI restrictions can be configured per CodebookConfig, whereas two RI restrictions are applied to two CMR groups in a CMR resource set respectively for Single-TRP measurement hypothesis, and the third one is applied to all NCJT measurement hypotheses. - If rank restriction of (X1, X2, Y) is configured, reported rank is X1, X2 for each CMR group respectively for single-TRP measurement hypotheses and reported rank (1 out of 4 possible rank combinations) is Y for all NCJT measurement hypotheses. - Alt 6: Switch between Alt 4 and Alt 5 where gNB can configure via RRC signaling which alternative to use Note that if none of above Alternatives is agreed in Rel-17, RI restriction is only applied for Single-TRP measurement hypotheses and no RI restriction is applied for Multi-TRP measurement hypotheses.

For Alt. 1, as RI restriction is applied to the sum of two reported ranks, the RI combinations (1, 2) and (2, 1) require X=3 and the RI combination (2, 2) require X=4. However, since X is also applied to single-TRP measurement hypotheses, UE is forced to calculate CSI for RI=3 or RI=4 even if gNB knows that each channel between UE and one TRP can support up to two layers. For Alt. 3, we do not see the necessity of such flexibility, which is not even supported in R15/R16 for single-TRP measurement hypotheses. For Alt. 4 and Alt. 5, we fail to identify a circumstance that gNB would know how to configure joint RI restriction. Unless there is a convincing use case for joint RI restriction, we think per-TRP RI restriction is more reasonable.

Proposal 6: For a CSI report associated with an NCJT measurement hypothesis configured by single CSI reporting setting, support RI restriction with Option 2, i.e., two RI restrictions can be configured per CodebookConfig, whereas one RI restriction is applied to one CMR group in a CMR resource set respectively.

In the RAN1#106e meeting, we have the following agreements on non-PMI based CSI reporting [3]:

Agreement For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT measurement hypothesis, study whether to support non-PMI CSI reporting with reportQuantity set to "CRI-RI-CQI" in Rel-17 - Related details, if needed, are to be discussed in RAN1#106bis. - Interested companies are encouraged to share details and related specification impact if support

For TDD bands, based on channel reciprocity, DL CSI can be measured through UL measurements using SRS. Since gNB can determine adequate precoders from UL measurements, it suffices to ask UE to report its desired RI and CQI. Thus, for NCJT CSI, non-PMI CSI reporting with reportQuantity set to "CRI-RI-CQI" should be supported.

Proposal 7: Non-PMI CSI reporting with reportQuantity set to "CRI-RI-CQI" is supported for NCJT CSI.

If the UE is not configured with higher layer parameter non-PMI-PortIndication, we may consider the following extension for NCJT CSI:

Proposal 8: If the UE is not configured with higher layer parameter non-PMI-PortIndication, for two CMRs configured in a CMR pair as an NCJT measurement hypothesis, the CSI-RS port indices (p0v1, …, pv1-1v1)=(0,…,v1-1) of the CMR in CMR group 1 and the CSI-RS port indices (p0v2, …, pv2-1v2)=(0,…,v2-1) of the CMR in CMR group 2 are associated with rank combinations v1, v2∈{1, 1, 1, 2, 2, 1, (2, 2)}. The UE does not expect that an associated rank is larger than the number of ports of the corresponding CMR.

Next, we consider the case where the higher layer parameter non-PMI-PortIndication is configured. We note that the existing R15 design supports port indication per rank per CMR. Besides, when reportQuantity is set to "CRI-RI-CQI", gNB is supposed to know the precoders and certain estimate of inter-layer interference for an NCJT measurement hypothesis. Thus, it is reasonable that the port indication for NCJT measurement hypotheses is configured per rank combination per CMR pair. Besides, RI restriction can be supported by not indicating porting indication for a rank combination.

For example, denoting by p a port of the first CMR in a CMR pair and by q a port of the second CMR in the same CMR pair. Then, the CMR pair can be configured with the following port indications for each rank combination:

(1, 1): p0(1,1), q0(1,1)

(1, 2): p0(1,2), q0(1,2), q1(1,2)

(2, 1): p0(2,1), p1(2,1), q0(2,1)

(2, 2): p0(2,1), p1(2,1), q0(2,1), q1(2,2)

In addition, pi(v1,v2), qi(v1,v2) can be either values from {0, 1} for 2 ports or from {0, 1, 2, 3} for 4 ports.

Proposal 9: If the UE is configured with higher layer parameter non-PMI-PortIndication, the port indication for NCJT measurement hypotheses is configured per rank combination per CMR pair. The UE shall only report RI combination corresponding to the configured fields.

Finally, it is straightforward to extend the descriptions of CQI and precoder scaling for NCJT CSI:

Proposal 10: For NCJT CSI, when calculating the CQI for a rank combination (v1,v2), the UE shall use the ports indicated for that rank combination for the selected CMR pair. The precoders for the indicated ports shall be assumed to be the identity matrix scaled by 1v1+v2.

# CSI Enhancement: FR1 FDD Reciprocity

In RAN1 #106-e, the following agreements were made for Rel-17 port selection codebook enhancement in FR1 FDD [2]:

Agreement - Following working assumption is confirmed (with revision in RED): - At least for rank 1 and 2, FD bases used for Wf quantization are limited within a single window with size N configured to the UE whereas FD bases in the window must be consecutive from an orthogonal DFT matrix, i.e. Alt 1. - FFS other restrictions, e.g. value(s) of N, if the value of N3 is small - FFS other restrictions, e.g. when the number of CSI-RS ports is small - For Rel-17 PS codebook, the reserved state for reference amplitude is to be reserved as Rel-16 PS codebook. - For Rel-17 PS codebook, support reporting of the position, [il*, fl*], of the strongest coefficient (SCI) of layer l, using ceil(log2(K1*Mv)) bits. - For Rel-17 PS codebook, support layer-common port selection for rank 2. - Support parameter combinations represented by (alpha, Mv, beta) with K1 = alpha*P for Rel-17 PS codebook - The candidate values of alpha are {1/2, 3/4, 1} - Note that exact parameter combination will be discussed from RAN1 106bis: - based on trade-off among UPT performance, feedback overhead, and complexity - based on all supported ranks - Limit total number of parameter combinations comparable to Rel-16 eType II - Mv={1, 2} and beta = {[1/4], 1/2, 3/4, 1} are from previous agreements - For Rel-17 PS codebook with Rank 2, support layer-specific bitmap for indicating non-zero coefficient selection of W2 - Support rank 3 and 4 for Rel-17 PS codebook with following - Supporting ranks 3 and 4 is optional with separate UE capability (same as Rel-16 PS codebook) - The maximal CSI overhead of rank 3 and 4 is comparable to rank 2 - FFS: use a smaller K1 (or alpha) or beta for ranks 3 and 4, or limit the maximum number of non-zero coefficients across all layers to 2K0 and per layer to K0 with the same beta - FFS: limit Mv=1 for ranks 3 and 4 PMI - At least for rank 1/2 and Mv > 1, for relationship between N and Mv, support following alternative - Alt 2-1: N >= Mv, Wf is layer-common and reported by UE for N>Mv. - For Mv=2, N=2 and one value from {3, 4, 5} - RAN1 to select one value from {3, 4, 5} in RAN1#106bis-e - FFS: how to report Wf in terms of reporting mechanism and associated bits when Mv=2 and N=one value from {3, 4, 5} - Note: Wf is layer-common for N=Mv - Note: For all alternatives, a layer-common window/set of size N is configured. - If a bitmap for indicating non-zero coefficients can be absent, down-select one Alt from the following for Rel-17 PS codebook: - Alt 1: At least for rank 1 PMI, the bitmap of indicating non-zero coefficients is not needed if Mv=1 and Beta=1. - FFS the need for Mv>1 and/or Beta<1 - Alt 2: For rank 1 /2 PMI, the bitmap(s) of indicating non-zero coefficients for corresponding layer(s) is absent if reported KNZ=K1*Mv*rank - Where KNZ is the number of non-zero coefficients - Alt 3: In addition to Alt 2, additional field is reported by UE to inform whether the bitmap of indicating non-zero coefficients for specific layer is absent if rank>1. - Alt 4: The bitmap of indicating non-zero coefficients is not needed if the number of coefficients is sufficiently small, i.e. K1Mv ≤ δ - Note: If none of above Alternative is agreed in RAN1#106bis-e, the bitmap for indicating non-zero coefficient is always present by default - For Rel-17 PS codebook, following values of R are supported: - R = 1 and - At most one value from {2, D* NPRBSB} - FFS: which one is to be decided in RAN1#106bis if support, and applicable conditions, e.g. whether the support of this feature when Mv=1 - D is the density of CSI-RS in frequency domain and NPRBSB is the subband size in PRBs - Note that this R is optional if supported Conclusion: For Rel-17 PS codebook, there is no consensus on the support of Mv>2 for Wf

In this correspondence, we provide our views on some remaining design issues of the Rel-17 port selection (PS) codebook - namely, codebook parameter combinations including potential extension to rank 3,4, regarding the value of R (number of PMI subbands per CQI subbands), and reporting of frequency domain (FD) component in Wf.

## Discussion on codebook parameter combinations

In the last meeting, it was agreed to support a triplet (α,β,Mv) of parameter combinations for the Rel-17 PS codebook. The candidate values for each of these parameters are as follows:

- Port selection coefficient α∈12,34,1

- Compression coefficient β∈14,12,34,1

- Number of FD basis vectors Mv∈{1,2}

A brute force enumeration of the candidate values results in 3×4×2=24 parameter combinations. It is not desirable to support such a large number of combinations, and also it is observed that some combinations result in a similar performance-overhead tradeoff. Therefore it is agreed that the total number of combinations will be comparable to that in Rel-16 codebook, taking into account potential extension of Rel-17 codebook to rank 3,4. Towards deciding the specific combinations, we present simulation results of rank 1,2 first. Discussion on rank 3,4 is presented later.

The simulation assumption follows the agreed EVM assumption in RAN1 #102-e meeting [4]. NT=32 antenna ports are used at the gNB and 2 antenna ports are used at the UE. The detailed assumptions are shown in Appendix. The baseline performance is R16 Type II PS codebook with paramCombination-r16 = 1. Traffic load with 70% RU and SU/MU-MIMO adaptation with up to rank 2 is adopted. In all cases, CSI-RS is transmitted every 5ms whenever there is an active user. The average CSI-RS overhead per cell per 5ms is calculated by normalizing the total number of CSI-RS ports, which is the total number of active users times the number of CSI-RS ports per user, by the number of cells and CSI-RS periodicity. The obtained average CSI-RS overhead is taken into account for throughput calculation.

A polarization common, free port selection matrix W1 selects a fraction K1=αP of the P beamformed CSI-RS ports. For the quantization of coefficients in W2, we use the baseline scheme of Rel-16. βK1Mv of the K1Mv coefficients are assumed to be non-zero. The non-zero coefficients are indicated by a polarization specific and layer specific bitmap of size K1×Mv. For a P port CSI-RS, P SD-FD bases are estimated jointly at the gNB from oversampled DFT basis for SD and regular DFT basis for FD using the uplink channel measured from SRS. The NT×P precoder for a P port CSI-RS in PMI subband n=0, …N3-1 is of the form

WDn=si1fnj1*si2fnj2*⋯siPfnjP*,

where ip,jp, p=1,…P are the indices of the P dominant SD, FD bases si∈CNT×1 and fj∈CN3×1 respectively. The granularity (length) of FD bases is N3=NSB×R, where NSB  is the number of CQI subbands, and R is the number of PMI subbands per CQI subband. R=1 and CSI-RS density = 1 is considered for the simulation. For Mv>1, a continuous window of N=Mv DFT basis vectors starting from 0 is configured to the UE.

The CSI feedback overhead consists of the layer common port selection bits, the layer specific bitmap, strongest coefficient indicator, reference amplitude of the weaker polarization, and the remaining quantized linear combination coefficients, so that the total overhead is given by

log2P2K12+rank×K1Mv+ log2K1Mv+4+4+3βK1Mv-1 bits.

In Fig.1 we present two scenarios which represent the performance trends for CSI-RS ports P=8,16 with Mv=1. The first scenario fixes α=1 and varies β from ¼ to 1 (black colored curve). The second scenario fixes β=1 and varies α from ½ to 1 (red colored curve). It is seen that the UPT and overhead increase monotonously in both these scenarios. It is also observed that α=1 yields a higher performance than β=1 for the same feedback overhead. This is since all the SD-FD pairs are selected with α=1. Therefore, it can be assumed that the low feedback overhead – low performance regime corresponds to the parameter combinations with low values of α, the intermediate feedback overhead – intermediate performance regime corresponds to intermediate values of α and β, and the high feedback overhead – high performance regime corresponds to higher values of α and β.

| (a) | (b) |
| --- | --- |

Fig. 1 UPT vs overhead for (a) 8 port and (b) 16 port CSI-RS

Observation 1: The low, intermediate, and high feedback overhead regimes translate to corresponding low, intermediate and high candidate values of α and β.

Based on the above observation, in Fig. 2 we present the simulation results for 16 port CSI-RS with the following 5 parameter combination triplets α,β,Mv=12,12,1, 34,12,1, 1,12,1, 34,1,1, 1,1,1. It is seen that both the UPT performance and feedback overhead range from low to high for these combinations.

Fig. 2 UPT vs overhead for 16 port CSI-RS with the five parameter combinations in Table 1

The five parameter combinations (PC) are tabulated below for convenience:

| PC | α | β | Mv |
| --- | --- | --- | --- |
| 1 | ½ | ½ | 1 |
| 2 | ¾ | ½ | 1 |
| 3 | 1 | ½ | 1 |
| 4 | ¾ | 1 | 1 |
| 5 | 1 | 1 | 1 |

Table 1 Parameter combinations with Mv = 1

Proposal 11: For Mv=1 and rank 1,2, support the parameter combinations shown in Table 1, i.e., α,β,Mv=12,12,1, 34,12,1, 1,12,1, 34,1,1, 1,1,1.

Mv=2 is a UE optional feature which is desired to yield higher UPT at the cost of higher feedback overhead. This means that with Mv=2, the UPT and overhead are expected to be higher than those obtained using the five combinations proposed in Table 1. This is only possible with higher values of α and β for Mv=2. The rationale for having higher α with Mv=2 is evident from the fact that non-selected SD-FD pairs with low α do not contribute to the additional configured FD basis and therefore do not contribute to performance enhancement. Regarding the values of β, we refer to the following Fig. 3 from our contribution in the last meeting [3]. This figure shows the performance of Mv>1 for 8 port CSI-RS.

Fig. 3 UPT vs overhead for 8 CSI-RS ports with Mv = 2

Here, all the curves correspond to α=1 and β varying from ¼ to 1. It is observed that for the same β, Mv=2 shows a performance improvement over Mv=1 with increased overhead. However, for low values of β  (¼  and ½) with Mv=2, a similar, or slightly higher performance can be obtained with Mv=1 and higher β  (¾  and 1), as shown by the green coloured oval. This makes having lower β with Mv=2 irrelevant.

Observation 2: With Mv=2, low candidate values of α and β are irrelevant since a similar performance-overhead tradeoff can be obtained with Mv=1 and higher values of α and β.

We therefore propose the following three parameter combinations which can be optionally supported by the UE:

| PC | α | β | Mv |
| --- | --- | --- | --- |
| 6 | 1 | ½ | 2 |
| 7 | ¾ | 1 | 2 |
| 8 | 1 | 1 | 2 |

Table 2 Parameter combinations with Mv = 2

These parameter combinations are the same as combinations 3,4,5 shown in Table 1 with the exception of Mv=2.

Proposal 12: For Mv=2 and rank 1,2, support the parameter combinations shown in Table 2, i.e., α,β,Mv=1,12,2, 34,1,2, 1,1,2.

Extension to rank 3, 4

In the last meeting, it is agreed to extend Rel-17 codebook design to rank 3,4 with a separate UE capability. The general design principle follows the Rel-16 eType II in trying to have the CSI feedback overhead of rank 3,4 comparable to rank 1,2. The main contributor to the CSI feedback overhead is the number of non-zero coefficients K0=βK1Mv=αβPMv per layer. For rank 1,2 the total number of non-zero coefficients (NZC) can be up to 2K0. A straightforward extension to rank 3,4 with the same parameter combinations would result in up to 4K0 NZC. To have feedback overhead comparable to rank 1,2, it is therefore necessary to lower one or more of the parameters α, β, or Mv. It is to be noted that in Rel-16 eType II codebook, the frequency domain compression coefficient pv is halved for rank 3,4 compared to rank 1,2, while retaining the same number of SD components L, and compression coefficient β.

For the Rel-17 PS codebook, α represents the fraction of chosen SD-FD pairs. To support rank 3,4 it is necessary that the spatial resolution of the beamformed channel after port selection is high. Reducing α reduces the number of SD components, and makes it difficult to find a precoder for rank 3,4 with good inter-layer interference reducing capability. This is also evident from the previous discussion for rank 1,2 that low α results in poor UPT performance.

Proposal 13: For the Rel-17 codebook extension to rank 3,4, the port selection coefficient α should be kept the same as that for rank 1,2.

For the Rel-17 PS codebook, β represents the fraction of non-zero linear combination coefficients after the dominant SD, FD pairs are selected via port selection matrix W1 and FD bases matrix Wf. To reduce the number of NZC and therefore the feedback overhead, values of β can be halved for rank 3,4 compared to rank 1,2. Although there is performance degradation due to lower β, a higher SD resolution can be maintained, which is useful for obtaining a good precoder for rank 3,4.

Proposal 14: For the Rel-17 codebook extension to rank 3,4, the compression coefficient β can be halved compared to that for rank 1,2.

Regarding the support of Mv=2 for rank 3,4, there are two issues to consider in our opinion. The first issue is the fact that Mv=2 improves the frequency selectivity of the precoder and helps to accommodate UL/DL delay mismatch. However, a higher rank precoder is related more to the spatial resolution of the channel rather than frequency resolution. The second issue is that even if Mv=2 is supported for rank 3,4, the compression coefficient β is halved compared to rank 1,2. As discussed earlier, having lower β with Mv=2 results in a similar performance overhead trade-off as Mv=1 with a higher β. Therefore, Mv=2 need not be supported for rank 3,4.

Observation 3: Supporting Mv=2 does not improve the spatial resolution of the channel, hence may not be useful to design a higher rank precoder.

Observation 4: A lower value of compression coefficient β for rank 3,4 results in the performance overhead trade-off for Mv=2 being similar to Mv=1 with a higher β.

Proposal 15: For Rel-17 PS codebook, Mv=2 need not be supported for rank 3,4.

In summary, we propose the following parameter combination table for Rel-17 PS codebook considering extension to rank 3,4; with the blue coloured cells being UE optional features.

| PC | α | β |  | Mv | Restriction, if any |
| --- | --- | --- | --- | --- | --- |
|  |  | RI 1-2 | RI 3-4 |  |  |
| 1 | ½ | ½ | ¼ | 1 |  |
| 2 | ¾ | ½ | ¼ | 1 | Not applicable for P=4, 12 |
| 3 | 1 | ½ | ¼ | 1 |  |
| 4 | ¾ | 1 | ½ | 1 | Not applicable for P=4, 12 |
| 5 | 1 | 1 | ½ | 1 |  |
| 6 | 1 | ½ | - | 2 |  |
| 7 | ¾ | 1 | - | 2 | Not applicable for P=4, 12 |
| 8 | 1 | 1 | - | 2 |  |

Table 3 Parameter combinations for Rel-17 PS codebook

Proposal 16: For the Rel-17 PS codebook, support the 8 parameter combinations in Table 3.

For combinations 2, 4, and 7, we see that the port selection coefficient α=3/4 causes K1=3 and K1=9 for 4 port and 12 port CSI-RS, respectively. This makes it impossible to select K12 ports for each polarization. Therefore, these parameter combinations cannot be supported for 4 and 12 CSI-RS ports. All the other CSI-RS ports supported for Rel-16 codebook, namely, P=8, 16, 24 and 32, can be supported for Rel-17 codebook without further restriction.

Proposal 17: RAN1 should further discuss the applicability of α=34 for P=4 and 12 CSI-RS ports.

## Regarding the value(s) of R

In the last meeting, it was agreed that a single additional value of R, other than R=1, if supported, will be down selected among R=2 or R=D×NPRBSB.

In Rel-16 codebook, the intention of R>1 is to increase PMI granularity by having more than one frequency domain precoders per subband. In Rel-17 codebook, however, PMI granularity can be increased by RB-level frequency domain beamforming of CSI-RS followed by configuration of density D=1 CSI-RS or RB-level channel interpolation by UE. Further, the linear combination coefficients may be computed by the UE using SB level Wf and reported to the gNB, along with the indices of the Mv FD bases. The gNB may use RB-level FD bases of the same indices to reconstruct RB-level PMI. As shown by some companies in the last meeting, such interpolation causes very little performance degradation for bandwidths of practical interest. Therefore, there is no need of specifying a value of R>1 for the Rel-17 codebook.

Observation 5: Frequency domain PMI granularity for the Rel-17 codebook can be improved by RB-level FD beamforming of CSI-RS and RB-level PMI reconstruction by gNB.

Proposal 18: There is no need of specifying R>1 for the Rel-17 codebook to increase PMI granularity.

In the ideal scenario, when the gNB performs FD beamforming corresponding to the strongest delays in the UL channel, it is expected that the strongest delay component observed in the effective downlink channel is at FD basis 0 (delay 0). However, in practical scenarios of UL/DL delay mismatch, the UE may first need to search for the strongest FD basis (for Mv=1) or strongest window (for Mv>1) over the frequency domain granularity used in FD beamforming. In other words, UE may need to perform search operation of the kind

m=argmaxm=0,…N3-1n=0N3-1HnWDne-j2πN3mnF2

for Mv=1 and use this strongest component to form the effective channel H for computation of linear combination coefficients

H=n=0N3-1HnWDne-j2πN3mn

For Mv=2, UE needs to perform this search over a window of N FD bases,

m=argmaxm=0,…N3-1n=0N3-1HnWDne-j2πN3mn⋯e-j2πN3(m+N-1)nF2

Specifying a value of R>1 increases the DFT size and search space of the above operation and increases UE complexity.

Observation 6: Specifying a value of R>1 increases the DFT size, space of the search for strongest FD basis and increases UE complexity.

Proposal 19: R>1 should not be supported for Rel-17 codebook.

## Regarding reporting of FD component

In the last meeting, it was agreed to support layer common UE reporting of FD bases in Wf when N>Mv. Although it is hard to observe performance gain of N>Mv in simulations as seen in previous meeting, it may be worthwhile in practical scenarios for the UE to select the best Mv FD bases within a small window of N bases. Since gNB has identified the strongest FD bases from the UL channel and performed FD beamforming, it is not expected that the size of the configured window, N will be very large. Therefore, it is preferred to have N=4 for all configurable CSI-RS ports and ranks (including rank 3 and 4). The window size will upper bounded by the number of PMI subbands N3.

Observation 7: Since gNB performs beamforming using strongest FD bases, by (partial) delay reciprocity it is not expected that the size of the window by which the UL/DL delays maybe perturbed is large.

Proposal 20: N=min⁡{4,N3} is enough for all configurable CSI-RS ports and ranks (including rank 3 and 4).

In Rel-16 PS codebook, CSI-RS is beamformed only in the spatial domain. The UE needs to compute frequency dependent precoders and consequently perform frequency domain compression on the space-frequency precoder matrix for each layer. Therefore, the FD bases used for compression maybe different for each layer. However, in Rel-17 PS codebook, CSI-RS is SD-FD beamformed. The FD bases in Wf serve a different purpose than in Rel-16 codebooks. Here, the FD bases serve to accommodate UL/DL delay mismatch and/or indicate additional delays for channel measurement, and are therefore a property of the channel, and not of the individual layers in a precoder matrix. Layer common selection of Mv out of N FD bases should therefore be extended to rank 3,4 if supported.

Observation 8: In Rel-17 codebook, the FD bases in Wf serve to accommodate UL/DL delay mismatch and/or indicate additional delays for channel measurement, and are therefore a property of the channel.

Proposal 21: Support layer common selection of Mv out of N FD bases for rank 3 and 4.

Another issue is regarding the mechanism of reporting Mv out of N FD bases. Considering Mv=2 and N=4 as an example, we see that a free selection of 2 out 4 bases results in 42=6 combinations, namely – (0,1), (0,2), (0,3), (1,2), (1,3) and (2,3). However, the combinations (1,2) and (2,3) are equivalent to the combination (0,1) in the sense that the reconstructed subband precoders using these combinations differ from the precoders reconstructed using (0,1) by only a phase rotation ej2πN3kn common across all ports. k=1 for the combination (1,2) and k=2 for the combination (2,3). This phase rotation does not affect the CQI and therefore need not be reported. Similarly, the combination (1,3) is equivalent to (0,2). Therefore we see that for N=4, the UE only needs to report among the combinations (0,1), (0,2) and (0,3) using log23=2 bits. In general, for N bases, the UE needs to report one among 0,1, 0,2, …(0,N-1) combinations using log2N-1 bits.

Proposal 22: To report Mv=2 out of N FD bases, the UE only needs to report one among 0,1, 0,2, …(0,N-1) combinations using log2N-1 bits.

The final issue we discuss is regarding the reporting of strongest coefficient. It was agreed in the last meeting to report both the row and column index of the SCI using log2K1Mv bits unlike the SCI of Rel-16 codebook which is reported only using the row index. The mechanism which enables such reporting in Rel-16 codebook is the shifting/remapping operation for the linear combination coefficient matrix W2 and the FD basis matrix Wf. This mechanism has the dual benefits of saving overhead of log2Mv bits and also being robust to UCI omission of FD indicator, since the gNB may use the SCI along with FD basis 0 to reconstruct a coarse precoder. However, the UE incurs an additional complexity of performing the remapping operations. For the Rel-17 codebook, the overhead saving of log2Mv=1 bit is negligible and so the SCI reporting using log2K1Mv bits is agreed. However, the problem of accounting for UCI omission still persists. To do away with the remapping operations of W2 and Wf in this case, we propose that the FD indicator of log2N-1 bits be made part of UCI Group 0. In conjunction with the SCI report of log2K1Mv bits and port selection indicator in Group 0, this will enable the gNB to reconstruct a reasonable precoder in case of UCI omission. In our opinion, since CSI-RS is SD-FD precoded in Rel-17 PS codebook, it makes sense to include both SD and FD indicators in Group 0. This is in line with the design of Rel-16 regular and PS codebooks wherein CSI-RS is not precoded or SD precoded, respectively, and therefore SD indicators are given the highest priority.

Observation 9: Placing the FD indicator in UCI Group 0 will enable doing away with the remapping operations for W2 and Wf.

Observation 10: The port selection indicator, SCI, and FD indicator in Group 0 will together help reconstruct a reasonable precoder in case of UCI omission.

Proposal 23: For the Rel-17 PS codebook, port selection indicator of log2P2K12 bits, SCI of log2K1Mv bits, and FD indicator of log2N-1 bits should be made part of UCI Group 0.

# Conclusion

In summary, based on the above discussion we have the following observations and proposals:

Observation 1: The low, intermediate, and high feedback overhead regimes translate to corresponding low, intermediate and high candidate values of α and β.

Observation 2: With Mv=2, low candidate values of α and β are irrelevant since a similar performance-overhead tradeoff can be obtained with Mv=1 and higher values of α and β.

Observation 3: Supporting Mv=2 does not improve the spatial resolution of the channel, hence may not be useful to design a higher rank precoder.

Observation 4: A lower value of compression coefficient β for rank 3,4 results in the performance overhead trade-off for Mv=2 being similar to Mv=1 with a higher β.

Observation 5: Frequency domain PMI granularity for the Rel-17 codebook can be improved by RB-level FD beamforming of CSI-RS and RB-level PMI reconstruction by gNB.

Observation 6: Specifying a value of R>1 increases the DFT size, space of the search for strongest FD basis and increases UE complexity.

Observation 7: Since gNB performs beamforming using strongest FD bases, by (partial) delay reciprocity it is not expected that the size of the window by which the UL/DL delays maybe perturbed is large.

Observation 8: In Rel-17 codebook, the FD bases in Wf serve to accommodate UL/DL delay mismatch and/or indicate additional delays for channel measurement, and are therefore a property of the channel.

Observation 9: Placing the FD indicator in UCI Group 0 will enable doing away with the remapping operations for W2 and Wf.

Observation 10: The port selection indicator, SCI, and FD indicator in Group 0 will together help reconstruct a reasonable precoder in case of UCI omission.

Proposal 1: For CSI measurement associated with a CSI-ReportingConfig for NCJT, for two CMRs within the same CMR pair configured for NCJT measurement hypothesis two resources are restricted within the same DL slot.

Proposal 2: For a CMR pair configured for a NCJT measurement hypothesis a separate powerControlOffset (Pc ratio) shall be configured for the NCJT measurement hypothesis by re-defining such Pc ratio as 10log10(P_PDSCH/P_CSIRS) dB, whereas

- P_PDSCH is the energy of PDSCH ports with a same TCI state as the CMR on one subcarrier of one OFDM symbol

- P_CSIRS is the energy of all CSI-RS ports of the CMR multiplexed on one subcarrier of one OFDM symbol

Proposal 3: For CSI computation delay requirement associated with a CSI-ReportingConfig for a NCJT measurement hypothesis, new/relaxed values on Z and Z’ should be introduced. FFS exact values or other conditions

Proposal 4: For Option 1 with X = 1, 2, each CSI measurement hypothesis is mapped to a distinct CSI report.

Proposal 5: The CSI priority formula is updated as: Either

PriiCSIy,k,c,s,h=2∙Ncells∙Ms∙Nhypo∙y+Ncells∙Ms∙Nhypo∙k+Ms∙Nhypo∙c+Nhypo∙s+h,

or

PriiCSIy,k,c,s,h=2∙Ncells∙Ms∙Nhypo∙y+Ncells∙Ms∙Nhypo∙k+Ms∙Nhypo∙c+Ms∙h+s,

where h=0 for the NCJT measurement hypothesis, h=1 for the first single-TRP measurement hypothesis, if reported, and h=2 for the second single-TRP measurement hypothesis, if reported. Nhypo is the number of CSI reports in a CSI reporting setting. If UE is configured to report one single CSI report for a CSI reporting setting, then h=0 and Nhypo=1.

Proposal 6: For a CSI report associated with an NCJT measurement hypothesis configured by single CSI reporting setting, support RI restriction with Option 2, i.e., two RI restrictions can be configured per CodebookConfig, whereas one RI restriction is applied to one CMR group in a CMR resource set respectively.

Proposal 7: Non-PMI CSI reporting with reportQuantity set to "CRI-RI-CQI" is supported for NCJT CSI.

Proposal 8: If the UE is not configured with higher layer parameter non-PMI-PortIndication, for two CMRs configured in a CMR pair as an NCJT measurement hypothesis, the CSI-RS port indices (p0v1, …, pv1-1v1)=(0,…,v1-1) of the CMR in CMR group 1 and the CSI-RS port indices (p0v2, …, pv2-1v2)=(0,…,v2-1) of the CMR in CMR group 2 are associated with rank combinations v1, v2∈{1, 1, 1, 2, 2, 1, (2, 2)}. The UE does not expect that an associated rank is larger than the number of ports of the corresponding CMR.

Proposal 9: If the UE is configured with higher layer parameter non-PMI-PortIndication, the port indication for NCJT measurement hypotheses is configured per rank combination per CMR pair. The UE shall only report RI combination corresponding to the configured fields.

Proposal 10: For NCJT CSI, when calculating the CQI for a rank combination (v1,v2), the UE shall use the ports indicated for that rank combination for the selected CMR pair. The precoders for the indicated ports shall be assumed to be the identity matrix scaled by 1v1+v2.

Proposal 11: For Mv=1 and rank 1,2, support the parameter combinations shown in Table 1, i.e., α,β,Mv=12,12,1, 34,12,1, 1,12,1, 34,1,1, 1,1,1.

Proposal 12: For Mv=2 and rank 1,2, support the parameter combinations shown in Table 2, i.e., α,β,Mv=1,12,2, 34,1,2, 1,1,2.

Proposal 13: For the Rel-17 codebook extension to rank 3,4, the port selection coefficient α should be kept the same as that for rank 1,2.

Proposal 14: For the Rel-17 codebook extension to rank 3,4, the compression coefficient β should be halved compared to that for rank 1,2.

Proposal 15: For Rel-17 PS codebook, Mv=2 need not be supported for rank 3,4.

Proposal 16: For the Rel-17 PS codebook, support the 8 parameter combinations in Table 3.

Proposal 17: RAN1 should further discuss the applicability of α=34 for P=4 and 12 CSI-RS ports.

Proposal 18: There is no need of specifying R>1 for the Rel-17 codebook to increase PMI granularity.

Proposal 19: R>1 should not be supported for Rel-17 codebook.

Proposal 20: N=min⁡{4,N3} is enough for all configurable CSI-RS ports and ranks (including rank 3 and 4).

Proposal 21: Support layer common selection of Mv out of N FD bases for rank 3 and 4.

Proposal 22: To report Mv=2 out of N FD bases, the UE only needs to report one among 0,1, 0,2, …,(0,N-1) combinations using log2N-1 bits.

Proposal 23: For the Rel-17 PS codebook, port selection indicator of log2P2K12 bits, SCI of log2K1Mv bits, and FD indicator of log2N-1 bits should be made part of UCI Group 0.

# References

- RP-193133, “Further enhancements on MIMO for NR,” Samsung.

- Chairman’s Notes, RAN1 #106e, e-Meeting, Aug. 16–27, 2021.

- R1-2107490, CSI Enhancement for NCJT and FR1 FDD reciprocity, MediaTek Inc., RAN1 #106e, e-Meeting, Aug. 16–27, 2021.

- R1-2006973, Discussion Summary for CSI enhancements MTRP and FR1 FDD reciprocity, RAN#102e, E-meeting, Aug. 17-28, 2020.

# Appendix

SLS assumptions for FR1 FDD reciprocity

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense urban macro |
| Frequency Range |  | 2 GHz with duplexing gap of 200 MHz between DL and UL |
| Inter-BS distance |  | 200 m |
| Channel model |  | Based on TR 38.901 with the reciprocity model of DL/UL channel in Section 5.3 of TR 36.897 |
| Antenna setup and port layouts at gNB |  | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ 100 deg subarray downtilt (zenith angle) |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1) 4RX: (2,1,2,1,1,1,1) |
| BS Tx power |  | 44 dBm |
| BS antenna height |  | 25 m |
| UE antenna height & gain |  | According to TR36.873 |
| UE receiver noise figure |  | 9 dB |
| Modulation |  | Up to 256QAM |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Simulation bandwidth |  | 20 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO adaptation with up to 2 rank |
| CSI feedback |  | CSI feedback periodicity: 5 ms Scheduling delay: 4 ms |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | 70% |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |
| Baseline for performance evaluation |  | Rel-16 PS eTypeII |
| SRS modeling for UL channel estimation |  | SRS periodicity = 5 ms SRS error modelling according to Table A.1-2 in TR 36.897 with Δ = 9 dB |