3GPP TSG RAN WG1#106-bis-e      	        	                                   R1-2109775    E-meeting October.11th-19th, 2021

Agenda Item:					    8.1.2.4

Source:	Sony

Title:	Enhancement on HST-SFN deployment

Document for:	Discussion and decision

# Introduction

In RANP#86 meeting, the WID on multi-TRP operation of Rel.17 was established as below. In this contribution, we present our considerations on HST-SFN for multi-TRP scenario.

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: - Identify and specify features to improve reliability and robustness for channels other than PDSCH (that is, PDCCH, PUSCH, and PUCCH) using multi-TRP and/or multi-panel, with Rel.16 reliability features as the baseline - Identify and specify QCL/TCI-related enhancements to enable inter-cell multi-TRP operations, assuming multi-DCI based multi-PDSCH reception - Evaluate and, if needed, specify beam-management-related enhancements for simultaneous multi-TRP transmission with multi-panel reception - Enhancement to support HST-SFN deployment scenario: - Identify and specify solution(s) on QCL assumption for DMRS, e.g. multiple QCL assumptions for the same DMRS port(s), targeting DL-only transmission - Evaluate and, if the benefit over Rel.16 HST enhancement baseline is demonstrated, specify QCL/QCL-like relation (including applicable type(s) and the associated requirement) between DL and UL signal by reusing the unified TCI framework

# UE-based SFN scheme

In some regions/countries, NR network deployment for high speed train (HST) scenario takes significant role for user experience and vertical usage. One common operation to provide robust DL coverage is the so-called single frequency network (SFN). Taking the case in Figure 1 as an example, one UE located in a HST can be served by multiple TRPs simultaneously in a SFN manner. Specifically, two TRPs connecting to one BBU from Cell B transmits to the UE with same control information and data occupying the same time-frequency resource. In following sections, we will bring next-level details for discussion.

- Paradigm of high-speed train and single frequency network (HST-SFN)

## SFN schemes

In RAN1#102e, two schemes for DL SFN transmission were designed. After a few rounds of discussions, in RAN1#104e, scheme 1 was supported as described in following agreements.

Agreement For the discussion purpose consider the following categorization of the enhanced DL transmission schemes - Scheme 1: - TRS is transmitted in TRP-specific / non-SFN manner - DM-RS and PDCCH/PDSCH from TRPs are transmitted in SFN manner - Scheme 2: - TRS and DM-RS are transmitted in TRP-specific / non-SFN manner - PDSCH from TRPs is transmitted in SFN manner Agreement Scheme 1 is supported in Rel-17 - TRS is transmitted in TRP-specific / non-SFN manner - DM-RS and PDCCH/PDSCH from TRPs are transmitted in SFN manner - FFS other details

Assuming two TRPs serving a UE in SFN mode, we have illustrated these two schemes in Figure 2 where the red ellipse and blue ellipse represent non-SFN and SFN transmission respectively. From the description of both schemes, one may easily find out that the common part of two schemes is that TRS is always transmitted in TRP-specific fashion, i.e. non-SFN manner. It therefore facilitates UE to carry out time/frequency tracking from each transmitting TRP. The differential part of two schemes is whether DMRS associated with PDCCH/PDSCH can be transmitted in SFN manner or not.

- Illustration of scheme 1 (left) and scheme 2 (right)

From the discussion of RAN1#103e, the drawbacks of scheme 2 were revealed. Given DMRS transmitted in non-SFN manner, it implies that PDCCH DMRS port is transmitted from single TRP. It results in that the single port PDCCH cannot be transmitted via SFN. Obviously, the DL performance relies on both scheduling PDCCH and scheduled PDSCH. Clearly without a decoded DCI, UE would not be able to receive its scheduled PDSCH.

In addition, since different DMRS ports are transmitted from different TRPs, channel estimation complexity would be increased when compared with that of scheme 1. There will be double DMRS overhead when compared with scheme 1.

- : For scheme 2, the reliability and robust of PDCCH cannot be enhanced by SFN transmission.

- It is not necessary to additional support scheme 2.

## QCL assumption

In RAN1#104bis-e, the following working assumption on QCL assumptions were achieved. Specifically, the QCL source RS defined for Rel.16 M-TRP can be reused for Rel.17 SFN as source RS.

Working Assumption All QCL source RS resource types as defined in TCI state for Rel-16 multi-TRP are supported for scheme 1

Moreover, it could be beneficial for a UE located in an HST to be served not only by TRPs in its serving cell(s), but also TRPs associated with nearby non-serving cell(s). This operation aligns with the spirit of SFN operation. From implementation perspective, it implies that UE doesn’t need to know where the DL transmission actually comes from, if the UE knows how to carry out time/freq. tracking and how to retune its Rx beam(s).

From NW configuration point of view, the QCL assumption from TRPs in non-serving cell should be allowable to UE. In our accompany paper [2], the concept of the so-called “universal” TCI states has been discussed and proposed. It extends the QCL assumption from the TCI states configured in serving cell(s) to TCI states associated with non-serving cell(s). Note that a UE may use PCI to differential non-serving cell(s) from serving cell(s).

- To further facilitate HST-SFN operation, support to extend the QCL assumption of PDCCH/PDSCH DMRS from its serving cell(s) to non-serving cell(s).

# NW-aided SFN

In HST scenario, a UE served by multiple TRPs in an SFN manner would inevitably experience the Doppler effect. When the UE approaches toward a TRP, the Doppler effect brings positive frequency offset. At the same time, the UE may also leave away from another TRP, so the Doppler effect introduces negative frequency offset. Combining above two Doppler effect, the total frequency offsets could be comparable to the applied SCS. Let’s take a FR2 example here. Assume train speed up to 500km/h and operating @39GHz with SCS 120kHz or 240kHz, the Doppler offset in extreme cases could be up to 180kHz.

In RAN1#102e, the 3-step scheme of frequency offset pre-compensation was established for discussion purpose. But in RAN1#104e due to lack of evaluation results, RAN1 cannot achieve consensus on whether to support TRP pre-compensation in specification. Consequently, a conclusion was made to encourage companies to evaluate the performance associated with TRP pre-compensation considering the simulation assumptions in EVM and then move forward anyway.

Agreement For discussion purpose consider the following three steps for TRP-based frequency offset pre-compensation scheme: - 1st step: Transmission of the TRS resource(s) from TRP(s) without pre-compensation - 2nd step: Transmission of the uplink signal(s)/channel(s) with carrier frequency determined based on the received TRS signals in the 1st step - 3rd step: Transmission of the PDCCH/PDSCH from TRP(s) with frequency offset pre-compensation determined based on the received signal/channel in the 2nd step Note: A second set of TRS resource(s) may be transmitted at 3rd step. Conclusion The decision on support of specification based TRP pre-compensation scheme for HST-SFN scenario to be made in RAN1#104-e-bis meeting. To facilitate RAN1 decision, companies are encouraged to provide evaluation results according to the agreed evaluation assumptions. The evaluations not compliant with agreed assumptions will not be considered by RAN1 in the decision process.

In RAN1#104bis-e, there was great progress on TRP-based Doppler frequency shift pre-compensation as in following agreement. It was clearly stated that rather than implementation based Doppler shift pre-compensation, specification-based Doppler pre-compensation was supported in Rel.17.

Agreement Specification-based TRP Doppler pre-compensation scheme is supported in Rel-17 for FR1 with one or both: - UL RS based Doppler estimation by gNB - FFS: Details including UL RS enhancement - DL RS based Doppler feedback by UE - FFS: Details - FFS: Whether UE capability needs to be introduced - Whether to support one or both will be decided later

So next we will discuss the related standard impact of TRP based Doppler shift pre-compensation.

## QCL-Type of TRS

In Rel.15/16, TRS is allowed to serve as QCL source for PDCCH/PDSCH DMRS. For DL channels, normally TRS provides QCL information in either QCL-TypeA or QCL-TypeA + QCL-TypeD (when QCL-TypeD is applicable). Take the case of Doppler offset pre-compensation in Figure 3 as an example, NW selects TRP#0 as an anchor TRP and allows TRP#1 to pre-compensate its DL RF to the anchor frequency. Note that the anchor frequency is the frequency in which UE experiences from the un-compensated TRS#0.

According to the 3-step pre-compensation scheme, TRS#0 is not pre-compensated. With TRP pre-compensation, PDCCH/PDSCH are received at the anchor frequency. TRS#0 is applicable to serve as source RS for QCL-TypeA + QCL-TypeD. But as for TRS#1, since it is not pre-compensated to the anchor frequency, the UE cannot assume TRS#1 and DMRS of PDCCH/PDSCH have the same characters of Doppler effect in frequency domain. So TRS#1 from TRP#1 (non-anchor TRP) cannot serve as source RS in some of existing QCL-types, i.e. QCL-TypeA, QCL-TypeB or QCL-TypeC which contain Doppler-related large scale parameter(s).

- Multi-TRP Doppler offset pre-compensation to an anchor frequency

- : For TRS transmitted by non-anchor TRP, it is no longer applicable to serve as source RS for conducting QCL-TypeA, QCL-TypeB or QCL-TypeC for PDCCH/PDSCH DMRS transmitted in SFN manner.

In RAN1#103e, the following agreement on variants of new QCL-Type and UE behavior was made

Agreement When the same DMRS port(s) are associated with two TCI states containing TRS as source reference signal, at least one variant is supported for Rel-17 HST-SFN scenario based on further evaluations - Variant A: One of the TCI state can be associated with {average delay, delay spread} and another TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA) - Variant B: One of the TCI state can be associated with {average delay, delay spread} and another TCI state with {Doppler shift, Doppler spread} (i.e., QCL-TypeB) - Variant C: One of the TCI state can be associated with {delay spread} and another TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA) - Variant E: Both TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA) - FFS: Indication method to apply QCL, e.g., via new QCL-type, or reuse existing QCL-type while UE to ignore certain QCL properties - Note: Each TCI state in the above variants may be additionally associated with {Spatial Rx parameter} (i.e., QCL-TypeD) - Note: Companies are encouraged to provide evaluation results for the above variants based on agreed EVM from RAN1#102e meeting - Note: Above variants are applicable to scheme 1 and/or TRP based pre-compensation as a reference for evaluation. - This agreement is for the purpose of evaluation and does not imply the support or lack of support of scheme 1 and/or TRP based pre-compensation

For UE receiving PDCCH/PDSCH from non-anchor TRP(s), the QCL information in time domain i.e. delay spread and average delay and in spatial domain, i.e. spatial Rx filter, are still valid and valuable. To solve this QCL problem, one alternative is to define a new QCL type which only contains {delay spread, average delay} which is included in Variant A/B in above agreement.

In RAN1#105e and RAN1#106e, RAN1 achieved agreement on the principle of QCL parameters dropping for TRP-based pre-compensation and Variant A with FFS on Variant B. Let’s take TRS with existing QCL-TypeA as example, UE may choose to drop part of QCL parameters depending whether pre-compensated or not.

Agreement - For TRP-based pre-compensation QCL assumptions is provided to the UE by using the existing QCL type(s) with certain QCL parameters dropped from the indicted QCL type - FFS rule or signalling to determine which TCI state with dropped QCL parameters - UE does not expect to be configured different SFN schemes (scheme 1 or TRP pre-compensation) for both PDCCH and PDSCH. - FFS whether this restriction is per UE or per CC - UE does not expect to be configured different SFN schemes (scheme 1 or TRP pre-compensation) for different CORESETs. - FFS whether this restriction is per UE or per CC Agreement Confirm working assumption from RAN1#105e meeting without modification: For TRP-based pre-compensation, Variant A (based on RAN1#103-e meeting agreement) is supported as QCL types/assumption, when the same DMRS port(s) are associated with two TCI states. - FFS: Support of Variant B

If the dropping rule or signaling can be designed and specified, i.e. UE knows how to pick up QCL parameters, e.g. {average delay, delay spread} and drop others from QCL-TypeA, then we think any variant, e.g. Variant A or B can be obtained without further confirm the working assumption. Hence, we would like to share

- : For TRP-specific pre-compensation, Variant A or Variant B can be equivalently obtained from UE dropping QCL parameter based on Variant E.

In our understanding, when Variant E is applied (QCL-TypeA + QCL-TypeA in TCI states configured to a UE), the UE cannot deduct which TRP is the anchor TRP and which TRP(s) is/are not. Since above-scheme is built on allowing UE to drop the frequency domain QCL assumption from non-anchor TRP, this kind of information is important to UE. And it can be done by RRC configuration and/or MAC CE as in Rel.15/16.

- NW should explicitly configure or implicitly indicate to UE from which TRP the frequency-domain QCL assumption can be ignored.

# Conclusions

Finally, allow us to repeat our proposals to draw attention.

- It is not necessary to additional support scheme 2.

- To further facilitate HST-SFN operation, support to extend the QCL assumption of PDCCH/PDSCH DMRS from its serving cell(s) to non-serving cell(s).

- NW should explicitly configure or implicitly indicate to UE from which TRP the frequency-domain QCL assumption can be ignored.

# Reference

- RP-193133, “New WID: Further enhancements on MIMO for NR”, Samsung

- R1-2005560, Considerations on the enhancement of multi-beam operation, Sony