3GPP TSG RAN WG1 #106bis-e		R1-2109785

e-Meeting, October 11th – 19th, 2021

Agenda Item: 	8.3.3

Source: 	Sony

Title: 	Considerations on intra-UE UL multiplexing

Document for: 	Discussion / decision

# Introduction

In RAN1#106e, we agreed and concluded the following:

Agreement

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17,

- HP A/N reuses rate matching equation, and RE mapping rules in Rel-15 for A/N+CSI-1.

- LP A/N reuses rate matching equation, and RE mapping rules in Rel-15 for CSI-2.

Above applies at least for PUCCH format 3 and 4.

Agreement

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, an additional maxCodeRate for LP HARQ-ACK can be configured in the second PUCCH-Config per PUCCH format.

Conclusion

Simultaneous PUCCH/PUSCH transmission on the same cell is not supported in Rel-17.

Agreement

In NR Rel-17, [at least] 2 new set of beta offset values can be configured to the UE to indicate separate beta_offset values for the following cases:

- Multiplexing LP HARQ-ACK on HP PUSCH

- Multiplexing HP HARQ-ACK on LP PUSCH

Working Assumption

For handling overlapping PUCCHs/PUSCHs with different priorities in R17

- Step 1: Resolve overlapping PUCCHs and/or PUSCHs with the same priority

- Step 2: Resolve overlapping PUCCHs and/or PUSCHs with different priorities

Note: Avoid recursive pseudo-code to implement this procedure

Note: It is expected that Rel-15 intra-UE UCI multiplexing timeline will be applicable

Agreement

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17,

- PUCCH resource set determination is based on: UCI payload size = the number of HP UCI bits + the number of LP UCI bits.

- FFS PRB number determination for HP A/N and LP A/N, e.g. based on their coding rates.

- FFS the impact to the number of LP UCI bits due to missed DCI and potential solutions

- Note: the number of LP UCI bits in the above agreement does may not necessarily mean the actual number of LP UCI bits until the second FFS is resolved

This contribution discusses some remaining issues on intra-UE multiplexing.

# UCI Multiplexing in PUCCH

## Enabling/Disabling UCI Multiplexing

It was agreed that the gNB indicates whether to multiplex or prioritize the UCI bits from PUCCH of different priorities.  For gNB scheduler flexibility, a dynamic indicator is beneficial.  A new DCI field, i.e. Multiplexing Indicator Mux, can be introduced in the DL Grant to indicate whether an HP PUCCH can accept multiplexing from LP UCIs.

Proposal 1: The gNB dynamically enables/disables multiplexing in an HP PUCCH by using a new Multiplexing Indicator in the DL Grant scheduling the HP PUCCH.

## Misalignment of Number of LP HARQ-ACKs

The PUCCH resource is determined by the total number of UCI bits, NUCI.  For HARQ-ACK multiplexing of different L1 priorities, NUCI = NHP + NLP, where NHP and NLP are the number of HARQ-ACK bits for HP and LP respectively.  An issue was raised in previous meetings that miss detection of DL Grants scheduling LP PUCCH would leading to misalignment of NLP and this would cause the UE to select a different PUCCH resource to the one expected by the gNB, which may impact the decoding and reliability of the HP HARQ-ACKs.

Firstly, misalignment of NLP is not an issue for Type 1 HARQ-ACK Codebook since the number of HARQ-ACKs is semi-statically configured.  Hence, misalignment of NLP may only be an issue in Type 2 HARQ-ACK Codebook (Type 2 CB).

Observation 1: Misalignment on the number of LP HARQ-ACK NLP between UE and gNB is not an issue for Type 1 HARQ-ACK Codebook.

For Type 2 CB, Downlink Assignment Index (DAI) is introduced to mitigate against DL Grant misdetections.  However, DAI can fail in the following two scenarios, for a C-DAI of 2 bits [1]:

- Scenario 1: The UE misses 4 consecutive DL Grants thereby causing the DAI counter to wrap around.

- Scenario 2: The UE misses the last DL Grant associated with the HARQ-ACK PUCCH

Observation 2: Misdetection of DL Grant for Type 2 HARQ-ACK Codebook is mitigated using DAI.

Observation 3: A DAI with 2 bits can fail in the following scenarios:

- Scenario 1: The UE misses 4 consecutive DL Grants thereby causing the DAI counter to wrap around.

- Scenario 2: The UE misses the last DL Grant associated with the HARQ-ACK PUCCH

The PDCCH BLER target for eMBB or the DL Grant scheduling a LP PUCCH is 10-2.  Hence, misalignment of NLP due to Scenario 1, i.e. UE missing 4 consecutive DL Grants has a probability of approximately 10-8, which is significantly lower than the 10-5 or 10-6 PDCCH target BLER for URLLC.  Hence, misalignment of NLP for Type 2 CB is likely caused by the UE missing the last DL Grant associated with the LP PUCCH.

Observation 4: The probability of missing 4 consecutive DL Grants scheduling a LP PUCCH is approximately 10-8, which is significantly lower than the PDCCH target BLER for URLLC.  Hence, this scenario is highly unlikely to occur.

Observation 5: For Type 2 HARQ-ACK Codebook, misalignment on the number of LP HARQ-ACK NLP between UE & gNB is likely caused by the UE missing the last DL Grant associated with the LP PUCCH.

The following options were considered for the misalignment of NLP [2]:

- Option 1: Configure a dedicated PUCCH resource for HP+LP in the second PUCCH-Config

- Option 2: PRI+x in the HP DCI is used to implicitly determine an extended PUCCH resource

- Option 3a: The LP type 2 codebook size is quantized/rounded up to a nearest reference size. FFS reference size granularity.

- Option 3b: Configuration of semi-static size reservation for LP HARQ-ACK payload is provided by RRC. LP HARQ-ACK semi-static size reservation is used instead of determined LP HARQ-ACK codebook size when selecting the PUCCH resource set.

- Option 4: Additional DCI field in DCI corresponding HP HARQ-ACK or HP PUSCH for determining the number of LP HARQ-ACK bits multiplexed on PUCCH/PUSCH.

- Option 5: Provide indication on at least the number of RBs and/or PUCCH resource set index to be used in the PUCCH transmission, where the indication is included in the high-priority DL assignment.

In Option 1, a separate PUCCH resource is used to carry the multiplexed HARQ-ACKs instead of a PUCCH resource from the HP PUCCH.  It is argued in [3] that missing part of the LP HARQ-ACKs is not an issue as the gNB could easily recover from it via the PUCCH’s DMRS.  Hence, the issue is when the UE misses ALL the DL Grants and therefore Option 1 reduces the gNB blind decoding to only 2 candidates, i.e. the new PUCCH resource with when LP HARQ-ACK is multiplexed and the HP PUCCH resource when there is no LP HARQ-ACK multiplexing.  However, the probability for the UE missing 3 or more consecutive DL Grants is 10-6 which is equivalent to the PDCCH target BLER for URLLC and hence unlikely.  Therefore, Option 1 would cater for the case where the NLP ≤ 2, which has limited benefit.  Option 2 is an implementation of Option 1 and hence has the same limitation.

Observation 6: Option 1 and Option 2 of using a dedicated PUCCH resource for multiplexing of HP & LP HARQ-ACK is only beneficial when the UE misses ALL the DL Grants associated to the LP PUCCH, which is only limited to the case where the number of LP HARQ-ACK NLP ≤ 2.

Observation 7: The gNB can recover from the UE missing a subset of LP HARQ-ACK from the PUCCH DMRS.

Option 3a and 3b proposed that the number of LP HARQ-ACK NLP can only use predetermined size, i.e. from a set of reference NLP.  Firstly, semi-statically configuring the size of NLP defeats the purpose of using a dynamic HARQ-ACK codebook like Type 2 CB, and hence would just increase the overhead of the PUCCH which may degrade the reliability of the PUCCH carrying the HP & LP HARQ-ACKs.  Hence, Option 3a and 3b can already be implemented using Type 1 CB.  Secondly, Option 3a and 3b attempt to solve the consequences of NLP misalignment rather than address the root cause of the problem, i.e. misdetection of DL Grants, which leads to an inefficient solution as a higher PUCCH overhead is always used.

Observation 8: Option 3a and Option 3b that forces the number of LP HARQ-ACK NLP, into a reference set of values that are semi-statically configured would defeat the purpose of using dynamic Type 2 CB and increase the overhead of the PUCCH carrying the multiplexed HARQ-ACKs, which would then impact the reliability of the HP HARQ-ACKs.

Observation 9: Option 3a and Option 3b do not address the root cause of the problem, i.e. misdetection of DL Grant, but instead attempt to solve the consequence of the problem, which is inefficient.

Option 4 addresses the root cause of the problem, i.e. misdetection of DL Grant, by providing information regarding NLP in the DL Grant scheduling the HP PUCCH.  In [5], it is proposed to include the DAI associated with the LP PUCCH in the HP DL Grant, which would cost 2 additional DCI bits but it addresses the problem of the UE missing the last DL Grant associated with the LP PUCCH as described above.  However, this would increase the overhead of the DL Grant scheduling the HP PUCCH.

Observation 10: Option 4 proposes to indicate information regarding the number of LP HARQ-ACK NLP, e.g. the DAI, in the DL Grant scheduling the HP PUCCH, which addresses the root cause of the problem direction but would incur additional DCI overhead.

Option 5 proposes to indicate, in the DL Grant associated with the HP PUCCH, the number of RBs used in the PUCCH carrying the multiplexed HARQ-ACKs.  This may incur a significant number of bits in the DL Grant scheduling the HP PUCCH especially if the range of possible RBs is large.  Furthermore Option 5 does not address the root cause of the problem, i.e. missing DL Grant, directly.

Observation 11: Option 5 that proposes to indicate the number of RBs for the PUCCH carrying the multiplexed HARQ-ACKs may incur significant overhead to the DL Grant scheduling the HP PUCCH and it does not address the root cause of the problem, i.e. missing DL Grant.

Our view is, none of the proposed solutions are satisfactory, as they have limited benefit such as Option 1 & Option 2, increase PUCCH overhead such as Option 3a & Option 3b or increase DCI overhead such as Option 4 & Option 5.  Also, most of the options do not address the root cause of the problem, i.e. missing DL Grant, except for Option 4.  The increase in either PUCCH or DCI overhead does not justify trying to solve an issue that is unlikely to occur and even if it does, for example when the UE misses some of the DL Grants, it could be recovered by the gNB via DMRS detection of the PUCCH as described in [3].  However, if companies insist on addressing this issue, we prefer that any solution should address the problem directly, i.e. the case where the UE misses the last DL Grant associated with the LP PUCCH, with no overhead in either the PUCCH and DCI.

Proposal 2: Since misalignment on the number of LP HARQ-ACK NLP for Type 2 HARQ-ACK Codebook is caused by the UE missing the last DL Grant associated with the LP PUCCH, any proposed solution should address this issue directly with no overhead to either the PUCCH or DCI.

One way is to use the 1 bit Multiplexing Indicator Mux, in the DL Grant, that is used to enable/disable UCI multiplexing of different priorities as in Proposal 1.  For LP DL Grant (Priority Indicator = “0”), this Multiplexing Indicator is used to indicate whether a DL Grant is the last DL Grant associated with a LP PUCCH (Mux=“1”) or not (Mux=“0”).  For HP DL Grant (Priority Indicator = “1”), this Multiplexing Indicator will indicate whether to perform multiplexing (Mux=“1”) or not (Mux=“0”).  Hence, the UE would only multiplex the LP HARQ-ACK with the HP HARQ-ACK if the UE detects Mux=“1” in one of the LP DL Grants (indicating that this is the last DL Grant) and Mux=“1” in at least one of the HP DL Grant.  An example is shown in Figure 1, where DCI#1, DCI#2, DCI#3, DCI#4 and DCI#5 schedule PDSCH#1, PDSCH#2, PDCSH#3, PDSCH#4 and PDSCH#5 respectively with their HARQ-ACKs scheduled in LP PUCCH#1.  DCI#6 and DCI#7 schedule PDSCH#6 and PDSCH#7 respectively with their HARQ-ACKs scheduled in HP PUCCH#2.  In this example, the UE misses DCI#2 and so at time t3, the UE assumes wrongly that NLP =1 but since the C-DAI in DCI#3 indicates 3 PDSCH, the UE can then correct NLP=3.  DCI#5 is the last LP DL Grant and so Mux=“1”.  DCI#7 which is a HP DL Grant indicates Mux=“1” thereby enabling UCI multiplexing.  Here the UE misses DCI#5 which is the last LP DL Grant and hence would wrongly assume that NLP=4 instead of 5, but since the UE did not detect Mux= “1” in any of the LP DL Grants, the UE would not perform UCI multiplexing and drops PUCCH#1.  That is, this mechanism stops UCI multiplexing if the UE fails to receive the last DL Grant associated with the LP PUCCH, thereby avoiding NLP misalignment.  This method also does not introduce any additional overhead to the DCI since the Multiplexing Indicator (if introduced) is reused.  The gNB may have to perform blind decoding but only for two candidates, i.e. multiplexing with the correct NLP or no multiplexing at all.

Figure 1: Using Multiplexing Indicator to indicate the last LP DL Grant

Proposal 3: Introduce a 1-bit “last DL Grant” indicator in the DL Grant to indicate whether a DL Grant is the last DL Grant associated with a LP PUCCH.

Proposal 4: The last DL Grant indicator can reuse the Multiplexing Indicator field, such that;

- If Priority Indicator = “1”, then the Multiplexing Indicator indicates whether UCI multiplexing of different L1 priority in a PUCCH is enabled or disabled

- If Priority Indicator = “0”, then the Multiplexing Indicator indicates whether or not the DL Grant is the last DL Grant associated with a LP PUCCH .

Proposal 5: The UE performs UCI multiplexing if it detects a positive Multiplexing Indicator in one (i.e. the last) of the LP DL Grants and a positive multiplexing Indicator in at least one of the HP DL Grants, otherwise the UE drops the LP PUCCH.

## High Priority SR & Low Priority HARQ-ACK Multiplexing

Three multiplexing scenarios involving High Priority (HP) SR and Low Priority (LP) HARQ-ACK multiplexing using PUCCH Format 0 (PF0) and PUCCH Format 1 (PF1) were agreed in RAN1#104e [5].  We will discuss the behaviour for these 3 scenarios in the following sections.

### HP SR PF0 + LP HARQ-ACK PF0

For the multiplexing of HP SR carried by PF0 with LP HARQ-ACK carried by PF0, the following options were agreed for further consideration:

- Opt.1: The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource.

  - Opt.1a: The UE does not transmit negative SR.

  - Opt.1b: For negative SR, the UE transmit only HARQ-ACK on the HARQ-ACK resource.

  - Opt.1c: For negative SR, the UE transmits SR and HARQ-ACK on the SR resource

  - FFS: whether with power boost to transmit multiplexed payload or not.

- Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource.

  - Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource.

  - Opt.2b: Using 4 CS values as for SR+1-bit HARQ-ACK in Rel-15/16. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit.

  - Opt.2c: If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.

- Opt.3: No enhancement over Rel-16.

In Rel-15, when a positive SR carried by PF0 collides with HARQ-ACK carried by PF0, the multiplexed UCI (SR + HARQ-ACK) is transmitted using HARQ-ACK PUCCH resource.  This behaviour can be reused for the case where SR & HARQ-ACK have different L1 priorities without modification, which would reduce specification impact.  Hence, we have a preference for Opt.2c.

Proposal 6: When HP SR using PF0 multiplexes with LP HARQ-ACK using PF0:

- If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15.

- If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.

### HP SR PF0 + LP HARQ-ACK PF1

For the multiplexing of HP SR carried by PF0 with LP HARQ-ACK carried by PF1, the following options were agreed for further consideration:

- Opt.1: The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource.

  - Opt.1a: The UE does not transmit negative SR.

  - Opt.1b: For negative SR, the UE transmit only HARQ-ACK on the HARQ-ACK resource.

  - Opt.1c: For negative SR, the UE transmits SR and HARQ-ACK on the SR resource

  - FFS: whether with power boost to transmit multiplexed payload or not.

- Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource.

  - Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource.

  - Opt.2b: Applying QPSK for SR+1-bit HARQ-ACK. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit.

  - FFS on conditions of multiplexing.

- Opt.3: For positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource.

- Opt.4: For positive SR, transmit SR on the SR resource and drop HARQ-ACK. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource.

- Opt.5: No enhancement over Rel-16.

In Rel-15, when a positive SR carried by PF0 collides with HARQ-ACK carried by PF1, the SR is dropped and the HARQ-ACK carried by PF1 is transmitted.  For HP SR, Rel-15 behaviour may not meet the URLLC low latency requirement.  Since PF0 can carry SR and up to 2 HARQ-ACKs, PF0 can be used to transmit multiplexed HP SR and LP HARQ-ACK.  For negative SR, the HARQ-ACK carried by PF1 is transmitted as per Rel-15 behaviour.  That is, we prefer Opt.1b.

Proposal 7: When HP SR using PF0 multiplexes with LP HARQ-ACK using PF1:

- The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource

- For negative SR, the UE transmits only HARQ-ACK on the HARQ-ACK resource.

### HP SR PF1 + LP HARQ-ACK PF0

For the multiplexing of HP SR carried by PF1 with LP HARQ-ACK carried by PF0, the following options were agreed for further consideration:

- Opt.1: The SR and HARQ-ACK are multiplexed and transmitted on the SR resource.

  - Opt.1a: For positive SR, the UE transmits the PUCCH in the resource using PUCCH format 1 for SR. The value of cyclic shift of sequence, i.e., , of this PUCCH format 1 is determined by HARQ-ACK, and the bit, i.e., b(0), of this PUCCH format 1 is determined by SR. For negative SR, the UE transmits only a PUCCH with HARQ-ACK information and drops the PUCCH with negative SR.

  - Opt.1b: SR and HARQ-ACK are multiplexed and modulated to be transmitted on the SR resource

- Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource.

  - Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource.

  - Opt.2b: Using 4 CS values as for SR+1-bit HARQ-ACK in Rel-15/16. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit.

  - Opt.2c: If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.

  - Opt.2d: HP SR and LP HARQ-ACK are multiplexed by the Rel-15 cyclic shift only if latency requirement for HP SR is met. Otherwise, drop the LP HARQ-ACK and only transmit the HP SR on its resource.

- Opt.3: For positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource.

- Opt.4: No enhancement over Rel-16.

Similar to the case of HP SR PF0 + LP HARQ-ACK PF0, in this scenario, the positive SR is multiplexed with HARQ-ACK and carried by the HARQ-ACK resource.  That is, we have a preference for Opt.2c.  This will make the UE implementation easier where for HARQ-ACK carried by PF0: regardless of whether the positive SR is carried by PF0 or PF1, the multiplexed UCI is always carried by the HARQ-ACK PF0 resource.

Proposal 8: When HP SR using PF1 multiplexes with LP HARQ-ACK using PF0:

- If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15.

- If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.

# UCI Multiplexing in PUSCH

## a offsets

The a offset is RRC configured and is a scaling factor that determines the maximum percentage of PUSCH REs that can be used for UCI.  Separate a offsets should be used depending on the L1 priorities of the UCI and PUSCH.  For example, if an HP UCI is multiplexed into an LP PUSCH, a larger portion of the PUSCH can be made available for the HP UCI compared to when multiplexing an LP UCI and therefore a different a should be applied.

Proposal 9: For multiplexing of UCI into PUSCH of different L1 priorities, the gNB is able to configure separate a offsets for different PUSCH L1 priorities.

When multiplexing HARQ-ACK and CSI bits into a PUSCH, if there are insufficient REs to carry these UCI bits, the CSI bits are dropped.  A similar mechanism can be used in multiplexing UCI bits, specifically HARQ-ACK bits, of different L1 priorities.  That is, if there are insufficient REs to carry the encoded UCI bits of different L1 priorities, the LP UCI bits are dropped.

Proposal 10: When multiplexing UCI bits into PUSCH of different L1 priorities, if there are insufficient REs in a PUSCH to carry the UCI bits, the LP UCI bits are dropped.

## Enabling/Disabling UCI Multiplexing into a PUSCH

It was agreed that the gNB can indicate whether to enable or disable multiplexing of UCI bits of different L1 priorities into a PUSCH.  For scheduling flexibility, it is beneficial that such indication is made dynamically using a DCI.

Proposal 11: The gNB dynamically indicates whether to enable/disable multiplexing of UCI bits into PUSCH of different L1 priorities.

Some companies proposed to use b =0 to indicate no multiplexing [2] but this does not work for the case where the UCI is HP, since this would lead to the HP UCI being dropped.  Hence, a different indicator is required.

Observation 12: Disabling UCI multiplexing by indicating b =0 does not work when the UCI has High L1 Priority.

An alternative that is applicable to both HP UCI and LP UCI is to reuse the “beta_offset indicator” in the UL Grant scheduling the PUSCH, where some of the values indicate not to multiplex any UCI, i.e. a non-numerical b value.  This is also applicable for the case for HP UCI and LP PUSCH, where Rel-16 prioritisation is used, i.e. dropping the LP PUSCH.

Proposal 12: The “beta_offset indicator” DCI field in the UL Grant scheduling the PUSCH is used to enable/disable multiplexing of UCI bits into PUSCH, where some of the indices have non-numerical values, i.e. “NOT MULTIPLEX”, to indicate that multiplexing is not used and that the UE performs prioritisation.  That is:

- If beta_offset indicator is numerical then:

  - LP UCI is multiplexed into HP PUSCH using the indicated b offset value

  - HP UCI is multiplexed into LP PUSCH using the indicated b offset value

- If beta_offset indicator = “NOT MULTIPLEX” or non-numerical then:

  - For the case of LP UCI & HP PUSCH, the LP UCI is dropped and HP PUSCH is transmitted

  - For HP UCI & LP PUSCH, the LP PUSCH is dropped and HP UCI is transmitted on PUCCH

# Conclusion

In this contribution, we discuss some remaining issues on intra-UE multiplexing.  We observe the following

Observation 1: Misalignment on the number of LP HARQ-ACK NLP between UE and gNB is not an issue for Type 1 HARQ-ACK Codebook.

Observation 2: Misdetection of DL Grant for Type 2 HARQ-ACK Codebook is mitigated using DAI.

Observation 3: A DAI with 2 bits can fail in the following scenarios:

- Scenario 1: The UE misses 4 consecutive DL Grants thereby causing the DAI counter to wrap around.

- Scenario 2: The UE misses the last DL Grant associated with the HARQ-ACK PUCCH

Observation 4: The probability of missing 4 consecutive DL Grants scheduling a LP PUCCH is approximately 10-8, which is significantly lower than the PDCCH target BLER for URLLC.  Hence, this scenario is highly unlikely to occur.

Observation 5: For Type 2 HARQ-ACK Codebook, misalignment on the number of LP HARQ-ACK NLP between UE & gNB is likely caused by the UE missing the last DL Grant associated with the LP PUCCH.

Observation 6: Option 1 and Option 2 of using a dedicated PUCCH resource for multiplexing of HP & LP HARQ-ACK is only beneficial when the UE misses ALL the DL Grants associated to the LP PUCCH, which is only limited to the case where the number of LP HARQ-ACK NLP ≤ 2.

Observation 7: The gNB can recover from the UE missing a subset of LP HARQ-ACK from the PUCCH DMRS.

Observation 8: Option 3a and Option 3b that forces the number of LP HARQ-ACK NLP, into a reference set of values that are semi-statically configured would defeat the purpose of using dynamic Type 2 CB and increase the overhead of the PUCCH carrying the multiplexed HARQ-ACKs, which would then impact the reliability of the HP HARQ-ACKs.

Observation 9: Option 3a and Option 3b do not address the root cause of the problem, i.e. misdetection of DL Grant, but instead attempt to solve the consequence of the problem, which is inefficient.

Observation 10: Option 4 proposes to indicate information regarding the number of LP HARQ-ACK NLP, e.g. the DAI, in the DL Grant scheduling the HP PUCCH, which addresses the root cause of the problem direction but would incur additional DCI overhead.

Observation 11: Option 5 that proposes to indicate the number of RBs for the PUCCH carrying the multiplexed HARQ-ACKs may incur significant overhead to the DL Grant scheduling the HP PUCCH and it does not address the root cause of the problem, i.e. missing DL Grant.

Observation 12: Disabling UCI multiplexing by indicating b =0 does not work when the UCI has High L1 Priority.

We therefore propose the following:

Proposal 1: The gNB dynamically enables/disables multiplexing in an HP PUCCH by using a new Multiplexing Indicator in the DL Grant scheduling the HP PUCCH.

Proposal 2: Since misalignment on the number of LP HARQ-ACK NLP for Type 2 HARQ-ACK Codebook is caused by the UE missing the last DL Grant associated with the LP PUCCH, any proposed solution should address this issue directly with no overhead to either the PUCCH or DCI.

Proposal 3: Introduce a 1-bit “last DL Grant” indicator in the DL Grant to indicate whether a DL Grant is the last DL Grant associated with a LP PUCCH.

Proposal 4: The last DL Grant indicator can reuse the Multiplexing Indicator field, such that;

- If Priority Indicator = “1”, then the Multiplexing Indicator indicates whether UCI multiplexing of different L1 priority in a PUCCH is enabled or disabled

- If Priority Indicator = “0”, then the Multiplexing Indicator indicates whether or not the DL Grant is the last DL Grant associated with a LP PUCCH .

Proposal 5: The UE performs UCI multiplexing if it detects a positive Multiplexing Indicator in one (i.e. the last) of the LP DL Grants and a positive multiplexing Indicator in at least one of the HP DL Grants, otherwise the UE drops the LP PUCCH.

Proposal 6: When HP SR using PF0 multiplexes with LP HARQ-ACK using PF0:

- If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15.

- If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.

Proposal 7: When HP SR using PF0 multiplexes with LP HARQ-ACK using PF1:

- The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource

- For negative SR, the UE transmits only HARQ-ACK on the HARQ-ACK resource.

Proposal 8: When HP SR using PF1 multiplexes with LP HARQ-ACK using PF0:

- If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15.

- If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.

Proposal 9: For multiplexing of UCI into PUSCH of different L1 priorities, the gNB is able to configure separate a offsets for different PUSCH L1 priorities.

Proposal 10: When multiplexing UCI bits into PUSCH of different L1 priorities, if there are insufficient REs in a PUSCH to carry the UCI bits, the LP UCI bits are dropped.

Proposal 11: The gNB dynamically indicates whether to enable/disable multiplexing of UCI bits into PUSCH of different L1 priorities.

Proposal 12: The “beta_offset indicator” DCI field in the UL Grant scheduling the PUSCH is used to enable/disable multiplexing of UCI bits into PUSCH, where some of the indices have non-numerical values, i.e. “NOT MULTIPLEX”, to indicate that multiplexing is not used and that the UE performs prioritisation.  That is:

- If beta_offset indicator is numerical then:

  - LP UCI is multiplexed into HP PUSCH using the indicated b offset value

  - HP UCI is multiplexed into LP PUSCH using the indicated b offset value

- If beta_offset indicator = “NOT MULTIPLEX” or non-numerical then:

  - For the case of LP UCI & HP PUSCH, the LP UCI is dropped and HP PUSCH is transmitted

  - For HP UCI & LP PUSCH, the LP PUSCH is dropped and HP UCI is transmitted on PUCCH

# References

[1] R1-2106804, “Considerations on UL Intra-UE Multiplexing,” Sony, RAN1#106e

[2] R1-2108556, “Summary#4 of email thread [106-e-NR-R17-IIoT-URLLC-04],” Moderator (OPPO), RAN1#106e

[3] R1-2106492, “Intra-UE multiplexing enhancements,” Huawei, HiSilicon, RAN1#106e

[4] R1-2107073, “Intra-UE multiplexing and prioritization,” InterDigital, RAN1#106e

[5] R1-2102281, “Report of RAN1#104-e meeting,” ETSI MCC, RAN1#104bis-e