3GPP TSG RAN WG1 #106bis-e 	R1-2110235

e-Meeting, October 11th – 19th, 2021

Source:              Qualcomm

Title:                   Rel-17 TEI Topics

Agenda Item:     8.18

Document for:   Discussion and Decision

# Introduction

In this paper we describe three potential items for TEI NR Rel-17:

- Improved Frequency-Domain Interleaving

- Enhancements to Short PUCCH Format

- DRX enhancements

  - Periodic SRS transmission outside DRX active time

  - Joint configuration of DRX groups and Rel-16 power saving features.

# Improved Frequency-Domain Interleaving

Starting from NR Rel-15, it was identified that at large BWs and high data rates and high rank, several codeblocks (~12) can be mapped onto any single OFDM symbol. When this happens, even though interleaving exists within any given code block, the frequency diversity of each codeblock can be relatively small since each CB occupies only a small set of PRBs. VRB-to-PRB interleaved mapping was introduced to distribute codeblocks across frequency.

Unfortunately, several limitations of the NR Rel-15 VRB-to-PRB solution were identified in practice:

- Small performance gains are observed since CBs are only distributed along two sub-bands that are diverse in frequency. Much larger gains can be achieved with higher-depth interleavers having more diversity.

- VRB-to-PRB interleaved mapping is happening within the BWP and not within the UE’s scheduled allocation which limits the ability to multiplex UE’s with different BWP configurations. VRB-to-PRB mapping may preclude coexistence of different UE’s with BWP switching for power savings.

- There is no mode of CSI reporting which assumes VRB-To-PRB interleaved mapping, while dynamic switching between the interleaved and non-interleaved mapping is specified. The scheduler does not have an indication from the UE whether, in any given conditions, it will be beneficial to dynamically switch ON/OFF the interleaved mapping.

As a simple simulation example, we consider the case of high throughput / high spectral efficiency (where the interleaving was supposed to provide most of the gains):  Rank 4, 100MHz BW, 30kHz SCS, TDL-A 30 nsec with MCS 13, 19 which correspond to the following cases:

| MCS | Rate | Modulation Order | # of CBs | # of CBs / Symbol |
| --- | --- | --- | --- | --- |
| 19 | 0.85 | 6 | 79 | 7.9 |
| 13 | 0.55 | 6 | 51 | 5.1 |

The gains in SINR over NR Rel-15 options to reach 90% throughput is shown below:

| Interleaver | Delta in dB, MCS = 19, MMSE | Delta in dB, MCS = 13, MMSE |
| --- | --- | --- |
| Rel-15 (No-ILV) | 0 | 0 |
| Rel-15 (VRB2PRB ILV) | 1.0 | 0.6 |
| 8-Row | 6.5 | 2.8 |

Based on the above observations, we make the following proposals:

Proposal 1: For the VRB-To-PRB interleaved mapping:

- Increase the depth of the interleaver (e.g. 4 or 8 rows instead of 2 rows in NR Rel-15)

- Perform the interleaved mapping within the scheduled allocations and not within the active BWP

# Enhancements to Short PUCCH Format

Short PUCCH format, specifically, PUCCH Format 2 spans one to two OFDM symbols and is restricted to use CP-OFDM waveform. The table below summarizes the configurations available for various PUCCH formats. The lack of DFT-S-OFDM support for short PUCCH Format is a significant shortcoming that we wish to address.

Table 1 Configurations for different PUCCH Formats

Short PUCCH formats have found significant use in FR2 deployments where a large number of analog antenna beams are used to serve users in uplink/downlink. Due to large number of antenna beams, long PUCCH formats are not favored since it’s easier to support beam sweeping operations using short PUCCH formats. Further since certain beams are intended to provide cell-edge coverage while certain beams are intended for cell-center UEs, beam-specific PUCCH configurations are ideally desired. However, defining beam-specific PUCCH formats is a tedious effort and adds to overall network configuration complexity. Therefore, short PUCCH formats are typically configured across all beams. Additional details on these aspects are presented in [1].

Further, with uplink being a typical bottleneck in NR deployments, it is worth considering enhancements that help improve uplink control coverage. It is well known that DFT-S-OFDM waveforms have a smaller PAPR compared to CP-OFDM, and this enables them to be transmitted at a higher power.

Using power class 3 UE as a motivating example, Table 6.2.2-1 of 38.101-1 as provided in Table 2 specifies a set of power reduction values dependent on RB allocation and modulation order for power class 3 UEs. The power back off values are then used by the UE to calculate the lower bound on its PCMAX,f,c value.

Table 2 MPR Table from 38.101-1

Note that DFT-S-OFDM with pi/2 BPSK has two sets of values defined, one for the case where the 0 dB MPR is in reference to 23 dBm and another where the 0 dB MPR is in reference to 26 dBm. This change in reference power to 26 dBm is permitted when UE is operating in TDD mode with less than 40% of the slots in a radio frame being used for uplink transmission.

It is thus seen that for a wide range of RB allocations, DFT-S-OFDM waveforms can be transmitted at a transmit power that is 2 dB higher than that possible for CP-OFDM waveforms. This motivates us to make the following proposal:

Proposal 2: Support transmitting PUCCH Format 2 using DFT-S-OFDM waveform.

Introducing DFT-S-OFDM for short PUCCH format requires a careful consideration of how the resources are split between DMRS and data. To support single symbol PUCCH transmission, it is required that DMRS and data be multiplexed on the same symbol. One option is to multiplex DMRS and data in time domain before the DFT operation [2], [3].

Transmit-side operations

The proposed scheme multiplexes data and reference signal within one symbol duration by virtual TDM. The time domain signal before DFT-spread and the transmitter block diagram is shown in Figure 1.

Figure 1. Transmitter for Virtual TDM of Reference Signal and Data

The first part of the pre DFT-spread time-domain signal is the reference signal. It is preferable for the reference signal to have low PAPR property on both time and frequency domain to keep the PAPR of the final DFT-s-OFDM waveform low and at the same time make the frequency domain channel estimation efficient.

To reduce inter-symbol interference, an additional virtual CP for reference signal can be optionally added at the beginning of the pre DFT-spread time-domain signal by copying the last symbols of the reference signal.

The reference signal symbols are followed by data symbols to form the pre DFT-spread time domain sequence s(n). The pre DFT-s sequence s(n) goes through the conventional DFT-s-OFDM waveform synthesis to generate the final time domain waveform.

Denote the signals in Figure 1 as follows:

a(n) : pre DFT-s Reference signal with length m1

b(n) : pre DFT-s Data signal with length m2

c(n) : pre DFT-s Virtual Cyclic Prefix for Reference Signal with length m3

s(n) : pre DFT-s Time-domain signal with length M

From the above discussion, we can see that M should be m1+m2+m3, and sn should be

sn=[am1-m3, …,am1-1cn : m3  Symbolsa0, …,am1-1an: m1  Symbolsb0, …,bm2-1bn: m2  Symbols].

Receive-side operations

Figure 2 shows the receiver block diagram for the virtual TDM shown in Figure 1. Except the channel estimation block, the receiver is essentially equivalent to the conventional DFT-s-OFDM receiver. After FFT and tone demapper, the extracted tones are equalized and go through IDFT to obtain M time domain symbols. Then, m2 data symbols are extracted for the decoding.

Figure 2. Receiver for Virtual TDM of Reference Signal and Data

There can be multiple options for the channel estimator. Figure 3 shows a channel estimator for the virtual TDM of reference signal and data. After FFT and tone demapper, the extracted tones go through IDFT to obtain M time domain symbols. Denote the discrete-time equivalent channel between the Tx antenna and Rx antenna for the M time domain symbols as h(n). When the CP length for Reference signal m3 is chosen longer than the propagation delay of h(n), the reference signal is protected from inter-symbol interference and circular convolution is preserved. Therefore, the extracted RS symbols in Figure 3 can be represented as hn⊗m1an where ⊗m1 denotes the m1-point circular convolution. The channel hn can be obtained by converting the extracted reference signal symbols to frequency domain by m1-point DFT. Finally, the estimated channel for m1 tones can be upsampled by a factor of M/m1 to obtain the channel estimation for M tones, which can be used for the channel equalization in the receiver of Figure 2.

Figure 3. Channel Estimator for Virtual TDM of Reference Signal and Data – Option A.

Alternatively, the upsampling block can be further removed by using M-point DFT. Figure 4 shows an alternative option for the channel estimator. The extracted tones go through M-point IDFT to obtain M time domain symbols. Then, the data symbols are replaced by zeros, and the modified M time domain symbols converted to the frequency domain by M-point DFT. Finally, the M channel tones can be estimated in the frequency domain.

Figure 4. Channel Estimator for Virtual TDM of Reference Signal and Data – Option B.

Clearly, this proposed transmission scheme can provide the multiplexing of reference signal and data with arbitrary pilot ratio while keeping the low PAPR property of DFT-s-OFDM waveform.

Simulation Results

In this section, we simulate and compare the link performances of the proposed virtual TDM scheme and compare with that of OFDM where the reference signal and data is FDMed.

Figure 5 presents two plots that illustrate the characteristics and the performance of the DFT-S-OFDM waveform. First, Figure 5 shows that DFT-s-OFDM waveform has at least 2 to 2.5 dB PAPR gain over CP-OFDM --- this is a reasonably well known result. Second, Figure 5 also shows that when comparing the link level performance between CP-OFDM (with data-RS FDM) and DFT-S-OFDM, it is observed that there is little to no difference at least for small payloads. For the link level performance, a three-bit payload is considered, and transmitted over 2 RB. The additional virtual CP length for RS is set to be zero, and the pilot ratio is chosen as 50% for both cases. Thus, taking both these observations into account, we see that the proposed scheme can provide up to 2 dB better performance than a CP-OFDM-based approach.

Figure 5 PAPR for CP-OFDM and DFT-s-OFDM waveforms (on the left) and link level performance comparison between DFT-S-OFDM and CP-OFDM (on the right).

Based on the discussion above and the simulation results, we make the following proposal:

Proposal 3: Consider pre-DFT data-DMRS multiplexing to enable DFT-S-OFDM waveform for PUCCH Format 2.

# DRX Enhancements

## Periodic SRS transmission outside DRX active time

According to NR Rel-15, when a UE is configured with DRX operation, the UE is not required to measure and report periodic and semi-persistent CSI outside the DRX active time. In Rel-16 UE power saving WI, as an enhancement for the DRX operation, a PDCCH-based wake-up signal (WUS), i.e., DCI format 2-6, has been introduced, based on which, the network can indicat the UE whether to start or skip a drx-onDurationTimer for a DRX cycle. In the later stage of Rel-16 discussion, an issue was identified with the periodic and semi-persistent CSI reproting when both DRX and DCI format 2_6 are configured: if the UE is not indicated to wake-up by the network for a long time, e.g., due to DL traffic inactivity, the UE needs to stay outside DRX active time during at least a few DRX cycles, and cannot get a chace to measure and report CSI during that time. Thus, when a new DL traffic arrives later and the UE is woken-up by the network, even the most recent CSI report from the UE is a few DRX cycles ago and may already be stale. This may result in an increased decoding error rate of earlier data packets, until the CSI at the network is updated by a new CSI report from the UE.

To address this issue, in Rel-16, when both DRX and DCI format 2_6 are configured, it was agreed to allow measurement and reporting for periodic CSI during the time duration indicated by drx-onDurationTimer outside DRX active time. Two new higher layer parameters, ps-TransmitPeriodicL1-RSRP-r16 and ps-TransmitOtherPeriodicCSI-r16, are introduced for separately enabling CSI reporting for L1-RSRP (i.e., cri-RSRP and ssb-Index-RSRP) and other report quantities, respectively, outside DRX active time.

Like periodic and semi-persistent CSI reporting, in Rel-15, the UE is not required to transmit periodic SRS and semi-persistent SRS outside the DRX active time. Thus, when the UE is configured with DRX and DCI format 2_6, the same issue aforementioned for CSI reporting persists for SRS transmission; the UE may not get an opportunity to transmit SRS for a very long time outside DRX active time. When SRS is used for either DL or UL channel sounding, this may impact the overall system performance. In Rel-16, nevertheless, only the issue of CSI reporting outside DRX active time was addressed, while the issue with SRS transmission was overlooked due to lack of time.

Although periodic CSI reporting outside active time can help keep the CSI updated, it may not be sufficient in some case. For example, without channel reciprocity, the network should rely on SRS to assess UL channels. With channel reciprocity, relying on SRS for DL channel sounding may be more power efficient from the UE perspective, since the UE is not required to measure CSI-RS and compute the CSI report. Also, for SUL, SRS may be the only resource that the network can assess the UL channel. Therefore, it seems necessary to allow a UE to transmit SRS outside DRX active time, when the UE is configured to monitor DCI format 2_6.

Proposal 4: When UE is configured with DRX and to monitor DCI format 2_6, it can also be configured to transmit at least periodic SRS outside DRX active time during the time duration indicated by drx-onDurationTimer.

## Joint configuration of DRX groups and Rel-16 power saving features

The feature of DRX groups was discussed under TEI16 in RAN2 as a solution to reduce power consumption when UE is configured with FR1+FR2 CA, and agreed in RAN#88-e. During the discussion, RAN4 confirmed that there is minimal impact on their specs [4]. On the other hand, RAN1 could not reach a consensus [5] on whether it may have any impact on other Rel-16 power saving features. As a way-forward, it was agreed that in Rel-16 DRX groups cannot be jointly configured with WUS or SCell dormancy.

Later, in RAN #90-e, it was further discussed whether to continue the discussion on the enhancement of DRX groups in Rel-17 UE power saving WI, focusing on the joint configuration with WUS or SCell dormancy. However, due to the concern on the limited TU for Rel-17 UE power saving WI, no consensus was made in RAN #90-e.

In our view, the discussion on the enhancement of DRX groups should be continued due to the evident power saving benefits, and Rel-17 TEI should handle it. Although the discussion was initiated in RAN2, we do not think this is a cross-WG issue, since there is no RAN2 or RAN4 impact with the joint configuration and can solely be handled by RAN1. More in-depth discussion follows below.

Joint configuration of DRX group and WUS

It is easily expected that additional power can be saved if DRX group and WUS can be configured together. For example, suppose WUS configured on SpCell indicates to UE whether it should wake up for next on duration or not. Then skipping on durations when there is no data can help UE save extra power on top of savings enabled by DRX groups, in the same way as how WUS saves UE power if there is only single DRX group. In Appendix A.2 of [6], we provide a quantitative analysis on the power savings that can be achieved by joint configuration, compared with the baseline in which WUS is not configured. The analysis shows that ~82% more power can be saved per DRX cycle than the baseline when there is no data and ~18% when there is data.

Observation 1: If WUS and DRX groups are jointly configured, UE can save extra ~82% power per DRX cycle when there is no data and ~18% when there is data.

If we have to minimize the impact of joint configuration of DRX group and WUS in RAN1, then the existing UE behaviors need to be reused as much as possible. More specifically,

- WUS should be configured only on SpCell, as in legacy;

- Conditions for WUS monitoring is completely determined by DRX state of SpCell and independent from DRX state of the secondary DRX group. For example, UE monitors WUS if SpCell is not in DRX active time, even if secondary DRX group is in DRX active time at the same time. This requirement avoids changes to the RAN1 spec;

- If WUS is not received or does not indicate wakeup, none of UE’s carriers should wake up, as in legacy;

- If a WUS occasion is not monitored (e.g., SpCell is already in DRX active time) or WUS indicates wakeup, UE should start DRX on duration timers of both DRX groups at their respective next occurrence. This behavior can be captured in RAN2 MAC specification. Note that this behavior works even in the corner case where FR1 (SpCell) is outside DRX active time but FR2 is within DRX active time.

As one may see from the above, no new PHY-layer behaviors need to be defined. We only need to add the following clarifications to the RAN1 standards:

- Clarify that, if secondary DRX group is configured, DRX active time for a serving cell refers to DRX active time of its associated DRX group;

- Clarify that DRX on-duration timer refers to those of all DRX groups in the text on WUS procedure.

Text proposal for the above clarifications can be found in [7].

Observation 2: Joint configuration between WUS and DRX groups can be supported with minimal change to RAN1 specs.

Joint configuration with SCell dormancy

In legacy, there are two scenarios in which SCell dormancy indication can be sent:

- Case 1. In a WUS occasion outside UE’s DRX active time, it can be sent together with WUS to indicate which SCell dormancy group(s) should switch to dormant BWP;

- Case 2. When UE is in DRX active time, it can be sent in a non-fallback DCI to indicate which SCell dormancy group(s) should switch to dormant BWP.

Case 1 requires joint configuration with WUS. In case secondary DRX group is configured, it effectively overrides DRX state of a SCell. For example, if a FR2 carrier is in a SCell dormancy group and receives dormancy indication, then it does not need to monitor PDCCH until the next DRX cycle, i.e., before receiving the next WUS. Therefore, network can take advantage of this property and use SCell dormancy indication to selectively wakeup secondary DRX group. In Appendix A.2.2 of [6], we provide a quantitative analysis on the power saving gains that can be achieved in this scenario. Our analysis shows that ~18% more power can be saved than the baseline.

In this case, because SCell dormancy indication is sent together with WUS, we do not expect much changes to RAN1/2 standards other than those described above for WUS.

Observation 3: If SCell dormancy is jointly configured with DRX groups, dormancy indication sent outside DRX active time can help save ~18% power.

In Case 2, if secondary DRX group is also configured, we think SCell dormancy operation and DRX operation can be independent from each other. More specifically,

- If both DRX groups are in DRX active time, SCell dormancy procedure can be performed exactly the same as in legacy (i.e. only a single DRX group is configured);

- If the secondary DRX group is outside DRX active time, UE can still switch active BWPs of any carriers in that DRX group according to the received indication (i.e. either from dormant to non-dormant BWP or from non-dormant to dormant BWP).  It is only an implementation matter that UE first stores the new active BWP indication for a carrier and then uses it after the carrier starts the next DRX active time.

It is straightforward to see that this case also requires no spec changes. Even though joint configuration in this case may not enable extra power savings, we think it is still beneficial for operators if the two features can co-exist. Otherwise, it would not be desirable if operators are forced to choose one feature over the other. For example, DRX groups may be deployed earlier than other power saving features, including SCell dormancy, because operators typically have more field experience with DRX. Then the artificial exclusivity imposed by the current Rel-16 agreement could delay the deployment of SCell dormancy, which clearly is not desirable for both operators and UEs.

Observation 4: Joint configuration between SCell dormancy and DRX groups can be supported without any change to RAN1 specs.

Based on the above analysis, we propose to discuss the following proposal in Rel-17 TEI:

Proposal 5: Support joint configuration between DRX groups and WUS, SCell dormancy, or both, without changes to their PHY-layer configurations and procedures.

# Conclusions

We make the following proposals.

On improved frequency-domain interleaving:

Proposal 1: For the VRB-To-PRB interleaved mapping:

- Increase the depth of the interleaver (e.g. 4 or 8 rows instead of 2 rows in NR Rel-15)

- Perform the interleaved mapping within the scheduled allocations and not within the active BWP

On Short PUCCH Enhancements:

Proposal 2: Support transmitting PUCCH Format 2 using DFT-S-OFDM waveform.

Proposal 3: Consider pre-DFT data-DMRS multiplexing to enable DFT-S-OFDM waveform for PUCCH Format 2.

On DRX enhancements:

Observation 1: If WUS and DRX groups are jointly configured, UE can save extra ~82% power per DRX cycle when there is no data and ~18% when there is data.

Observation 2: Joint configuration between WUS and DRX groups can be supported with minimal change to RAN1 specs.

Observation 3: If SCell dormancy is jointly configured with DRX groups, dormancy indication sent outside DRX active time can help save ~18% power.

Observation 4: Joint configuration between SCell dormancy and DRX groups can be supported without any change to RAN1 specs.

Proposal 4: When UE is configured with DRX and to monitor DCI format 2_6, it can also be configured to transmit at least periodic SRS outside DRX active time during the time duration indicated by drx-onDurationTimer.

Proposal 5: Support joint configuration between DRX groups and WUS, SCell dormancy, or both, without changes to their PHY-layer configurations and procedures.

# References

- R1-2008560, “Potential techniques for PUCCH coverage enhancements”, NTT DOCOMO, INC., 3GPP TSG RAN WG1 Meeting #103e.

- R1-1704783, “Pre-DFT Multiplexing of RS and UCI/Data for Short Duration PUCCH”, IITH, Reliance Jio, IITM, Tejas Networks, CEWiT, 3GPP TSG RAN WG1 Meeting #88Bis. Spokane

- R1-1708615, “Channelization of 1-symbol short PUCCH with more than 2 bits payload”, Qualcomm Inc, 3GPP TSG RAN WG1 Meeting #89, Hangzhou.

- R4-2005296, “LS on secondary DRX group for FR1+FR2 CA”, LS out, To: RAN2, RAN4, RAN4#94bis-e.

- R1-2002961, “LS response on secondary DRX”, LS out, To: RAN2, Cc: RAN4, RAN1#100bis-e.

- RP-202407, “Enhancement to DRX groups in R17 power saving WI”, Qualcomm Inc, 3GPP TSG RAN #90-e.

- R1-2002555, “Remaining issues for PDCCH-based power saving channel design”, Qualcomm Inc, 3GPP TSG RAN1 #100bis-e.