3GPP TSG-RAN WG1 Meeting #106-bis-e	R1-2110353

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.5.5

Source:	Ericsson

Title:	Potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation

Document for:	Discussion, Decision

# Introduction

The updated WID for NR positioning enhancements  includes the following objective

- Study and specify, if agreed, the enhancements of information reporting from UE and gNB for multipath/NLOS mitigation [RAN1, RAN2, RAN3]

This contribution is towards the above objective. In this contribution, we propose a few ways to enable LOS/NLOS detection in the network.

# Further details on multipath reporting

## Types of path to be reported

While release 16 provides a good suite of reference signals for positioning, the measurements specified in release 16 needs to be enriched through the incorporation ofadditional useful information for positioning.

Rich multipath reporting have been discussed in release 16, and there is support for up to two additional paths to be reported with the timing measurements (RSTD, UE RxTX Time Diff, gNB RxTx Time Diff and RTOA). However, in release 17, to make rich reporting useful and to meet stringent accuracy requirements, we need to increase the number of additional paths reported and unambiguously define what additional paths and what metrics the UE shall report.

Rich multipath reporting can include many parameters of the received signal which provides additional information helpful for positioning. Such parameters can be magnitudes of peaks SNRs of the peaks, Doppler frequencies, angle of arrival etc. of every reported multipath.

In order for rich reporting of additional paths to be at all useful, it’s necessary to define unambiguously what additional paths the UE should report.

- First path. Gives the TOA of the LOS path in case of LoS and the TOA closest to LOS in case of NLOS.

- Strongest path. Gives information that can be used for LOS detection (If the first path isn’t also the strongest path it can be assumed to be NLOS). It can be measured with best precision, and is useful for fingerprinting techniques.

- N strongest paths with shorter delay than the strongest path. Gives useful information in case the first path is misdetected with noise, interference or an out of range path. This is useful also for fingerprinting techniques.

- If the first path is the strongest, then N first path should be reported.

- For rich multipath reporting, it shall be unambiguously defined what additional paths a UE shall report. The following paths should be reported. First path, strongest path, N-2 paths between first and strongest paths, if first and strongest paths are same then first N paths.

## Reporting of path power for NLOS/LOS identification

The following was agreed during RAN1#106e:

Agreement: - For up to N>2 additional paths, support reporting relative timing (to the first detected path) in the measurement reports from UE to LMF for at least DL-TDOA and multi-RTT - FFS: Definition of additional paths for N>2 - FFS: Whether power is additionally reported and if reported whether power is relative to first detected path or total power - Support one of the following options for maximum value of N at RAN1#106-b (any further criteria for selection to be discussed during RAN1#106): - Option 1: N = 4 - Option 2: N = 8 - Option 3: N = 16 - Option 4: N = 32 Agreement: - For multipath reporting enhancements, support reporting from TRP to LMF, angle, timing, for up to additional N>2 paths for at least UL-TDOA and multi-RTT. - FFS: Definition of additional paths for N>2 - FFS: Whether power is additionally reported and if reported whether power is relative to first detected path or total power - Down select between the following options for N at RAN1#106-b (any further criteria for selection to be discussed during RAN1#106): - Option 1: N = 4 - Option 2: N = 8 - Option 3: N = 16 - Option 4: N = 32

In signature-based LOS/NLOS classification, we propose detecting specific signatures(patterns) in the measurements to detect LOS/NLOS. Below we motivate a LOS/NLOS detection method inspired from a real-world measurement dataset in LOS/NLOS scenarios . Figure 2 shows power delay profile in indoor factory like scenario in LOS/NLOS conditions. The zoomed version of the power delay profile is shown on the right in Fig.2 in relevant regions of the measurement.

Figure 2 Power delay profile in LOS/NLOS scenarios;  zoomed version of the PDP is shown on the right

It can be seen from above figures that in LOS scenario the highest peak of the measurement is very prominent. It is the first very clear peak arising from the measurement noise floor. Whereas, in the NLOS case, the highest peak of the measurement data is very gradual. The nature of LOS peak can be attributed to the fact that in the LOS conditions, all scattered signals arrive after the LOS component of the delay profile. Magnitudes of arriving scattered signals are always lower than the LOS reception due to higher path losses and other loss attributing effects. In NLOS cases, the highest peak can arrive possibly after many other scattering peaks. Even when the NLOS peak is prominent, typically it may be received after diffuse scattering components. Receiving diffuse scattering components along and before the prominent NLOS peak makes the rise of the peak very gradual, as evident from. Based on above discussion, following observations can be made.

LOS peak is always the first peak in power delay profile of the received signal for a LOS link.

A NLOS peak can also be a first prominent peak in a delay profile. But it appears along with smaller surrounding peaks and other components of delay profile generated from diffusely scattered signal before and after the NLOS prominent peak.

Rise of LOS highest peak is very sharp. Rise of NLOS highest peak is typically very gradual.

Figure 4 shows a possible implementation for detecting LOS/NLOS based on analyzing power delay profile of the received signal.

Figure 3 LOS/NLOS detection algorithm based on analyzing power delay profile of the received signal.

Based on the above discussion, we make the following proposal:

- Following measurements should be specified in Rel-17 to support signature-based methods. These measurements can be part of rich reporting.

  - Delay and magnitude/power of the first peak.

  - Delay and magnitude/power of the highest peak.

  - Components of PDP/CIR around first/highest peak.

Regarding “FFS Whether power is additionally reported and if reported whether power is relative to first detected path or total power”: Aside from NLOS mitigation, multipath reporting can be used for more advanced positioning techniques, for instance in combination with a 3D map. In  such techniques are used to “turn multipath channels from foe to friend” increasing accuracy and enabling indoor positioning from a single TRP. One of the key results is that the positioning robustness can be increased when the SINR of each path is reported. Consequently we strongly think that the power/magnitude should be reported for each additional path.

Support reporting the path power for the first and additional path for all positioning methods.

## Number of additional paths: Performance evaluations with ML based NLOS detection

### Machine learning example

The following was agreed during RAN1#106e:

Agreement: - For multipath reporting enhancements, support reporting from TRP to LMF, angle, timing, for up to additional N>2 paths for at least UL-TDOA and multi-RTT. - FFS: Definition of additional paths for N>2 - FFS: Whether power is additionally reported and if reported whether power is relative to first detected path or total power - Down select between the following options for N at RAN1#106-b (any further criteria for selection to be discussed during RAN1#106): - Option 1: N = 4 - Option 2: N = 8 - Option 3: N = 16 - Option 4: N = 32 Agreement: - For up to N>2 additional paths, support reporting relative timing (to the first detected path) in the measurement reports from UE to LMF for at least DL-TDOA and multi-RTT - FFS: Definition of additional paths for N>2 - FFS: Whether power is additionally reported and if reported whether power is relative to first detected path or total power - Support one of the following options for maximum value of N at RAN1#106-b (any further criteria for selection to be discussed during RAN1#106): - Option 1: N = 4 - Option 2: N = 8 - Option 3: N = 16 - Option 4: N = 32

Machine Learning (ML) classifiers can be trained for LOS/NLOS detection. We have studied the performance of different ML classifiers that take as inputs the value and delay time of varying number of peaks in the power delay profile (PDP). The classifiers were trained and evaluated on real 5G measurement data from a macro deployment with carrier frequency 2.66 GHz and PDP obtained from reference signals with 20 MHz bandwidth. The best performance was obtained with a K-nearest neighbor method and those results are presented here.

Test cases

10 test cases were evaluated, with increasing number of reported peaks:

For case 1 to 7:

- The LOS/NLOS classifier inputs are the (i) peak value and (ii) delay time of the highest peaks in the PDP.

- The number of peaks used are: 5, 10, 15, 20, 30, 40 or 50 (Test case 1 to 7)

For case 8 to 10:

- The LOS/NLOS classifier inputs are the first X number of samples of the measured PDP.

- Using X= 100 (case 8), X=150 (case 9), and all samples in the PDP (case 10)

The K-nearest neighbor method is always used as it gives the best performance.

Results

Table 2 summarize the results of the evaluation. Since Test case 10 makes use of the full PDP, we interpret the performance results obtained there as an upper bound for what can be achieved by this method on a subset of the PDP. We can make a number of observations:

The more PDP peaks (corresponding to paths) that are used by the classifier, the better becomes the classifier performance.

Table 2: ML results for test cases 1-10. The true LOS/NLOS status of the channel is on the vertical axis, and the ML-estimate is on the horizontal axis. Consequently, the diagonal elements shows the fraction of correct NLOS resp. LOS classifications and the off-diagonal elements show false negatives and false positives.

### Measurement classification example

Classification of measurements as LOS or NLOS and mitigation of the effects of NLOS can sometimes be done algorithmically with methods for robust estimation. The input to this kind of algorithms is typically a set of observations, like for instance TOA or TDOA measurements between different TRPs and a UE. NLOS measurements are considered as contaminated observations or outliers. While e.g. estimation with a least-squares method is highly sensitive to outliers, robust estimation methods are not. The methods only work when there is an excess of observations from which outliers can be pruned, hence algorithmic classification is particularly interesting for positioning in e.g. industrial scenarios where there is a dense deployment of TRPs. It should be stressed that the applicability of algorithmic LOS/NLOS classification to macro deployments (that are sparse) is very limited.

In this section we show results for an adaptation of the RANdom SAmple Consensus (RANSAC) algorithm proposed in  to the TOA/TDOA positioning method. The performance of robust estimation methods degrades as the fraction of contaminated observations increases. RANSAC can cope with a relatively large fraction of contaminated data. Algorithmic LOS/NLOS classification can benefit from rich reporting and be used in combination with e.g. signature-based LOS/NLOS detection to obtain even better performance. Here we show one such example where RANSAC is combined with a LOS/NLOS detector that compares the strength of different peaks in the power delay profile: If the first peak is the strongest, then it is likely LOS.

We evaluate the positioning performance on the InF-DH scenario specified by 3GPP in TR 38.901 with model parameters as given in TR 38.857. There are 18 TRPs in a 120x60 room which are mounted on a high level and 800 UEs are uniformly distributed over the deployment area. Each TRP broadcast DL-PRS that the UEs perform TOA measurements on by peak detection in an estimated channel impulse response. The TRPs are assumed synchronized with each other but not with the UE. The bandwidth and carrier frequency of the reference signal is 100 MHz respectively 3.5 GHz. With 800 UEs and 18 TRPs there in total 14400 TOA measurements. Figure 4 shows the TOA error distribution for LOS resp. NLOS measurements. The UE/TRP clock synchronization errors are not included in these figures, meaning that what is shown are the measurement errors and NLOS excess delays.

For LOS measurements the error distribution is well approximated by a Student´s t distribution with location, scale and shape parameters μ= 0, σ= 0.05 and ν= 1, respectively. The accuracy of these measurements in the order of decimeters, likely thanks to the high bandwidth of the DL-PRS signal. The NLOS excess delay is modeled with a log-normal distribution in the InF-DH scenario. This distribution is also plotted in Figure 4 (b) and its agreement with the simulated data is very good. Notice that the NLOS TOA errors are generally much larger than the LOS TOA errors, this is an important property for the performance of an algorithmic LOS/NLOS classifier.

Figure 4

Figure 5 shows the UE positioning error CDFs that are obtained with two different RANSAC algorithms. The figure also includes the results obtained with a least squares estimator, serving as a baseline performance reference. Moreover, the "LOS Genie'' results are obtained with a least-squares estimation based on the actual LOS measurements. Hence, these results indicate an upper limit for the accuracy that can theoretically be achieved by an estimator with a non-ideal LOS classifier.

The performance of the least-squares estimator is bad because of the large fraction of NLOS measurements in this scenario. Already the basic RANSAC algorithm gives very good improvements as compared to the LS estimator. The RANSAC+SFP algorithm combines RANSAC with the output of a signature-based LOS/NLOS classifier. The combination gives an even better positioning performance, close to the performance of “LOS Genie”.

Robust estimation methods can drastically improve the positioning performance in dense deployments with many NLOS measurements. However, since they require an excess of measurements, they cannot be used in sparse deployments.

Robust estimation methods can be combined with signature-based LOS/NLOS detectors to achieve even better positioning performance.

Figure 5

In Figure 6 we group the total 800 UEs according to how many LOS measurements there are for each of them and show how many of the UEs that different algorithms locate to a position within 1 m from the true position.

We observe a gradually reduced performance for both the robust estimation methods as the number of LOS measurements reduce. However, the combined RANSAC+SFP method performs better when there are very few LOS measurements.

The performance of robust estimation methods reduces gradually as the fraction of LOS measurements decrease.

The use of signature-based LOS/NLOS detectors becomes more important when there are only a few LOS measurements.

Figure 6

The next table summarizes the LOS/NLOS classification performance of the methods. SFP is the detector that compares the strengths of the pdp-peaks.

The SFP classifier has a much larger fraction of false LOS classifications than the other methods. In this dense deployment scenario, algorithmic LOS/NLOS classification with RANSAC works well. However, the best performance is obtained with the combined algorithmic and signature-based detector RANSAC+SFP.

Note: In our implementation, RANSAC+SFP will always “inherit” the NLOS classifications of the SFP detector and possibly also add more. However, this need not be the case depending on how the detectors are combined.

The LOS/NLOS classification performance of robust estimation methods is good in dense deployments.

The LOS/NLOS classification performance can be further improved by combining signature-based classifiers with robust estimation techniques.

Table 2 LOS/NLOS detector statistics. In total 14400 TOA measurements are simulated according to the Inf-DH scenario, 7524 of them are in LOS conditions and the other 6876 are in NLOS conditions

| Detector | LOS |  | NLOS |  |
| --- | --- | --- | --- | --- |
|  | Correct | False | Correct | False |
| SFP | 7136 | 2250 | 4626 | 388 |
| RANSAC | 7197 | 249 | 6627 | 327 |
| RANSAC+SFP | 7016 | 32 | 6844 | 508 |
| LOS Genie | 7524 | 0 | 6876 | 0 |

When performing TOA measurements on an estimated PDP, it is not always obvious which peak to report for, for instance if the first peak is not the strongest. In this situation the first peak could be a noise peak or it can correspond to the LOS path which is attenuated from e.g. penetration of a wall. However, since algorithmic LOS/NLOS classification can resolve such confusions, it is best to always report both the TOA for the first peak and additionally the N strongest peaks.

To support outlier detection types of methods (e.g. RANSAC) The UE could report TOA for the first (earliest) detected peak and additionally the N strongest detected peaks.

The more PDP peaks (corresponding to paths) that are used by the classifier, the better becomes the classifier performance.

### proposal

Motivated by these results we think that the number N of additional paths should be as large as possible.

The number of additional paths N should be large to enable accurate LOS/NLOS detection.

# Conclusion

In the previous sections we made the following observations:

Observation 1	LOS peak is always the first peak in power delay profile of the received signal for a LOS link.

Observation 2	A NLOS peak can also be a first prominent peak in a delay profile. But it appears along with smaller surrounding peaks and other components of delay profile generated from diffusely scattered signal before and after the NLOS prominent peak.

Observation 3	Rise of LOS highest peak is very sharp. Rise of NLOS highest peak is typically very gradual.

Observation 4	The more PDP peaks (corresponding to paths) that are used by the classifier, the better becomes the classifier performance.

Observation 5	Robust estimation methods can drastically improve the positioning performance in dense deployments with many NLOS measurements. However, since they require an excess of measurements, they cannot be used in sparse deployments.

Observation 6	Robust estimation methods can be combined with signature-based LOS/NLOS detectors to achieve even better positioning performance.

Observation 7	The performance of robust estimation methods reduces gradually as the fraction of LOS measurements decrease.

Observation 8	The use of signature-based LOS/NLOS detectors becomes more important when there are only a few LOS measurements.

Observation 9	The LOS/NLOS classification performance of robust estimation methods is good in dense deployments.

Observation 10	The LOS/NLOS classification performance can be further improved by combining signature-based classifiers with robust estimation techniques.

Observation 11	To support outlier detection types of methods (e.g. RANSAC) The UE could report TOA for the first (earliest) detected peak and additionally the N strongest detected peaks.

Observation 12	The more PDP peaks (corresponding to paths) that are used by the classifier, the better becomes the classifier performance.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For rich multipath reporting, it shall be unambiguously defined what additional paths a UE shall report. The following paths should be reported. First path, strongest path, N-2 paths between first and strongest paths, if first and strongest paths are same then first N paths.

Proposal 2	Following measurements should be specified in Rel-17 to support signature-based methods. These measurements can be part of rich reporting.

a.	Delay and magnitude/power of the first peak.

b.	Delay and magnitude/power of the highest peak.

c.	Components of PDP/CIR around first/highest peak.

Proposal 3	Support reporting the path power for the first and additional path for all positioning methods.

Proposal 4	The number of additional paths N should be large to enable accurate LOS/NLOS detection.

# Bibliography