3GPP TSG RAN WG1 Meeting #106bis-e	R1-2110371

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.16.1

Source:	Huawei, HiSilicon

Title:	Flexible PMCH bandwidth allocation for the 15KHz subcarrier spacing

Document for:	Discussion and Decision

Introduction

RAN1 related part of the work item “new bands and bandwidth allocation for 5G terrestrial broadcast” [1] was discussed in RAN1#106-e meeting and the agreements reached are recorded in the chair notes [2].

One issue discussed in the last meeting was whether/how to handle the control region with 15 kHz subcarrier spacing in the MBSFN subframes. It was agreed the UE is not required to process the control region as follows with the minimal impact to the specification.

Agreement:

The UE is not required to process the control region in MBSFN subframes for 6/7/8MHz PMCH bandwidth with 15kHz SCS

- Note: This does not preclude future removal of the control region after completion of this WI.

- Note: The agreement is made with the intention to capture the agreement with minimal impact to the specifications.

Discussion on specification impact

We are concerned at the possibility of any specification impact to MBSFN subframes with 15 kHz subcarrier spacing, which is also used by other features, e.g., TM9/10, other than for the purpose of MBMS transmission, since any change to MBSFN subframes with 15 kHz subcarrier spacing is intended for a specific newly introduced feature, i.e., flexible PMCH bandwidth allocation of 6/7/8 MHz in Rel-17.

Since UE is not required to process the control region in MBSFN subframes for 6/7/8 MHz PMCH bandwidth with 15 kHz SCS, the minimum specification change to MBSFN subframes with 15 kHz subcarrier spacing wherever it appears in the specification can be ZERO. From network perspective, the bandwidth of the control region can be up to network, i.e., either 6/7/8MHz as PMCH channel or 
3/5 MHz as CAS, because UE is not required to process this region.

The following are the instances of MBSFN subframes with 15 kHz subcarrier spacing in the current specifications of TS 36.211 and TS 36.213 regarding the control region.

< 36.211> ## 6.1 Overview The smallest time-frequency unit for downlink transmission is denoted a resource element and is defined in clause 6.2.2. A subset of the downlink subframes in a radio frame can be configured as MBSFN subframes by higher layers. For Δf≈0.37 kHz the MBSFN region is defined as one slot of 3 ms. Except for Δf≈0.37 kHz, each MBSFN subframe is divided into a non-MBSFN region and an MBSFN region. - For subframes using , the non-MBSFN region spans the first one or two OFDM symbols in an MBSFN subframe where the length of the non-MBSFN region is given according to Clause 6.7. - For subframes using , Δf=2.5 kHz, , or slots using Δf≈0.37 kHz, the non-MBSFN region is of zero size. - The MBSFN region in an MBSFN subframe is defined as the OFDM symbols not used for the non-MBSFN region. …. ## 6.5 Physical multicast channel The physical multicast channel shall be processed and mapped to resource elements as described in clause 6.3 with the following exceptions: - No transmit diversity scheme is specified. - Layer mapping and precoding shall be done assuming a single antenna port and the transmission shall use antenna port 4. - The PMCH can only be transmitted in the MBSFN region. For PMCH with Δf = 15 kHz, the index in the first slot in the MBSFN subframe fulfils where is equal to the value given by the higher layer parameter non-MBSFNregionLength [9]. … ## 6.7 Physical control format indicator channel The physical control format indicator channel carries information about the number of OFDM symbols used for transmission of PDCCHs in a subframe. The set of OFDM symbols possible to use for PDCCH in a subframe is given by Table 6.7-1. Table 6.7-1: Number of OFDM symbols used for PDCCH Subframe; Number of OFDM symbols for PDCCH when; Number of OFDM symbols for PDCCH when. Subframe 1 and 6 for frame structure type 2 or a subframe for frame structure type 3 with the same duration as the DwPTS duration of a special subframe configuration; 1, 2; 2. MBSFN subframes with and configured with 1 or 2 cell-specific antenna ports; 1, 2; 2. MBSFN subframes with and configured with 4 cell-specific antenna ports; 2; 2. MBSFN subframes with ∆f∈7.5, 2.5, 1.25 kHz or MBSFN slots with ∆f≈0.37 kHz; 0; 0. Non-MBSFN subframes (except subframe 6 for frame structure type 2) configured with positioning reference signals; 1, 2, 3; 2, 3. All other cases; 1, 2, 3; 2, 3, 4. The UE may assume the PCFICH is transmitted when the number of OFDM symbols for PDCCH is greater than zero unless stated otherwise in [4, clause 12]. < 36.213 > ## 11.1 UE procedure for receiving the PMCH UE is not expected to receive PMCH with (defined in [3]) other than in an MBSFN subframe with non-zero-size non-MBSFN region. A UE is not expected to receive PMCH with in an MBSFN subframe with zero-size non-MBSFN region. The UE shall decode the PMCH when configured by higher layers. The UE may assume that an eNB transmission on the PMCH is performed according to Clause 6.5 of [3]. …

According to the agreement that the UE is not required to process the control region in MBSFN subframes for 6/7/8 MHz PMCH bandwidth with 15 kHz SCS with the intention to capture the agreement with minimal impact to the specifications, the above specification cited regarding the control region within the MBSFN subframes with 15kHz subcarrier spacing can be unchanged.

Observation: The specification in TS 36.211/TS 36.213 regarding the control region within the MBSFN subframes with 15kHz subcarrier spacing can be unchanged to support flexible PMCH bandwidth allocation of 6/7/8HMz.

Conclusions

This contribution discusses the specification impact to MBSFN subframes with 15kHz subcarrier spacing for support of flexible PMCH bandwidth allocation of 6/7/8HMz in Rel-17. The following observation is derived based on the discussion:

Observation: The specification in TS 36.211/TS 36.213 regarding the control region within the MBSFN subframes with 15kHz subcarrier spacing can be unchanged to support flexible PMCH bandwidth allocation of 6/7/8HMz.

- References

RP-211144, Revised WID: New bands and bandwidth allocation for LTE based 5G terrestrial broadcast - part 1, EBU, Qualcomm, RAN#92-e, June 14-18, 2021.

RAN1#106e Chair notes. RAN1#106-e, Aug. 16-27, 2021.

R1-2108360, Feature lead summary #3 on New bands and bandwidth allocation for LTE based 5G terrestrial broadcast, Moderator (Qualcomm Incorporated), RAN1#106-e, Aug. 16-27th, 2021.