3GPP TSG-RAN WG1 Meeting #106bis-e	R1- 2110541

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.4.1

Source:	Moderator (Ericsson)

Title:	Feature lead summary#4 on timing relationship enhancements

Document for:	Discussion

# 1	Introduction

At RAN1#106bis-e, a first round of email discussion on timing relationship enhancements for NTN has been conducted. This contribution presents proposals that can be considered for discussion and potential endorsement at the GTW session on Monday, October 18, 2021.

# 2	Updated proposal based on company views (2nd round of email discussion)

Proposal 2 – v0

Use cell-specific K_offset in the enhanced PDCCH ordered PRACH timing relationship.

Proposal 4 – v0:

- For the reference subcarrier spacing value for the unit of K_mac in FR1, a value of 15 kHz is used.

- FFS: FR2

Proposal 5 – v1:

Take the following as a working assumption:

For defining value range(s) of K_offset, down-select one option from below:

| Option | Value range | Step size |
| --- | --- | --- |
| Option 1: One value range of K_offset covering all scenarios. | [0] – [542] ms | Same as the unit of K_offset |
| Option 2: Different value ranges of K_offset for different scenarios. | LEO: [0] – [49] ms MEO: [93] – [395] ms GEO: [477] – [542] ms FFS: ATG and HAPS FFS: How to determine the scenarios | Same as the unit of K_offset |
| Note: If deemed necessary, numbers in bracket can be further updated at RAN1#107-e. |  |  |

Proposal 6 – v1:

Take the following as a working assumption:

For defining value range(s) of K_mac, down-select one option from below:

| Option | Value range | Step size |
| --- | --- | --- |
| Option 1: One value range of K_mac covering all scenarios. | [1] – [271] ms | Same as the unit of K_mac |
| Option 2: Different value ranges of K_mac for different scenarios. | LEO: [1] – [25] ms MEO: [1] – [198] ms GEO: [1] – [271] ms FFS: ATG and HAPS FFS: How to determine the scenarios | Same as the unit of K_mac |
| Note 1: If deemed necessary, numbers in bracket can be further updated at RAN1#107-e. Note 2: Note that it was agreed already that when UE is not provided by network with a K_mac value, UE assumes K_mac = 0. |  |  |