3GPP TSG RAN WG1 Meeting #107-e	R1-2110858

e-Meeting, November 11th – November 19th, 2021

Agenda Item:	8.9.2

Source:	Huawei, HiSilicon

Title:	Support of 14-HARQ processes in DL for HD-FDD MTC UEs

Document for:	Discussion and Decision

Introduction

The WID for Rel-17 enhancements for LTE-MTC [1] includes an objective to support 14 HARQ for increasing the peak data rate for HD-FDD MTC UEs.

In RAN1 #106bis-e, the following agreements of HARQ-ACK delay solution were achieved:

Working Assumption For the joint encoding of “PDSCH Scheduling delay” and “HARQ-ACK delay” when Alt-2e is configured, the HARQ-ACK delay set has a size of: - Alt-C: - 12 elements: HARQ-ACK delay set = {a, b, c, d, e, f, g, h, i, j, k, l} for the PDSCH Scheduling delay expression associated to the delay of 2. - 10 elements: HARQ-ACK delay set = {o, p, q, r, s, t, u, v, x, w} for the two PDSCH Scheduling delay expressions associated to the delay of 7. - FFS: The values of {a, b, c, d, e, f, g, h, i, j, k, l}, {o, p, q, r, s, t, u, v, x, w} where some of these elements may share the same value. Conclusion: How to implement/describe the states, e.g., table, resulting from the joint encoding solution of Alt-2e is left up to the Editor, based on the agreements for the PDSCH scheduling delay, HARQ-ACK delay and the WA confirmed for Alt-2e. Agreement The Rel-17 14 HARQ processes feature only applies to User Specific Search Space (USS) Agreement In Rel-17, for the 14 HARQ processes feature the “HARQ-ACK process number” field uses 4-bits. Agreement In Rel-17, for the 14 HARQ processes feature the following updates on the technical specification are to be performed. - The maximum number of received PDSCH receptions pending HARQ-ACK is set to W = 12 (in Sect. 7.3.1 of TS 36.213) when the UE is configured with 14 HARQ processes. Agreement In Rel-17, one option will be downselected from Opt-2 and Opt-3 for the 14 HARQ processes feature the “Repetition number” field in RAN1#107e:  Opt-2: 0-bits when the 14 HARQ processes feature is configured (i.e., 2-bits from this field become available for jointly-encoding purposes).  Opt-3: 2-bits as in legacy.

In this contribution, we discuss the Alt-2e HARQ-ACK delays and DCI design  for the 14 HARQ processes feature to support 14 HARQ processes for HD-FDD MTC UEs.

Discussion

Values of Alt-2e HARQ-ACK delays

The HARQ-ACK delays are evaluated for a typical case as shown in Figure 1, with 20% invalid DL subframes and 20% invalid UL subframes. It can be observed that the HARQ-ACK values are {5,6,7,9,10,12,13,14,15,17} for the first-round HARQ-ACK processes and are {4,5,6,7,8,9,10,12,13,14,15} for the second-round HARQ-ACK processes. Thus, the HARQ-ACK delay values for the two-round HARQ-ACK processes are the union of these two sets, i.e. {4,5,6,7,8,9,10,12,13,14,15,17}, which are totally 12 values.

As can be observed, PDSCH scheduling delay {2} corresponds to HARQ-ACK delays {4,5,6,7,8,9,10,12,13,14}of 10 values, PDSCH scheduling delay {7-type I} corresponds to HARQ-ACK delays {15,17} of 2 values and PDSCH scheduling delay {7-type II} corresponds to HARQ-ACK delay {14,15} of 2 values.

To resolve the above issue, different set of HARQ-ACK delay values can be used for different PDSCH scheduling delays, 2 and 7. It can be easily analysed that the number of HARQ-ACK delay values corresponding to PDSCH scheduling delay 2 should be more than the the number of HARQ-ACK delay values corresponding to PDSCH scheduling delay 7.

- First round

- Second round

Figure 1 14-HARQ processes with 20% invalid DL and UL subframes

The number of HARQ-ACK delay values required for scheduling delay of 2 is much more than that for scheduling delay 7.

Observation 1: The number of HARQ-ACK delay values required for scheduling delay of 2 is much more than that for scheduling delay 7.

In RAN1#106bis-e, the working assumption was reached that 12 HARQ-ACK values is associated to delay 2 and 10 HARQ-ACK values is associated to delay 7. For 14-HARQ process scenarios, the typical percentage for invalid DL subframes and invalid UL subframes can be assumed as DL≤20% and UL≤20%. Consequently, the HARQ-ACK delay values for PDSCH scheduling delay 2 can range from 4 to 13,15,17 including 12 values. The HARQ-ACK delay values for PDSCH scheduling delay 7 can range from 12 to 21 including 10 values. In this way, most of the scenarios with different reserved subframes can be covered.

Proposal 1: The set of HARQ-ACK delay values associated with PDSCH scheduling delay 2 is from 4 to 13, 15, 17, and the set of HARQ-ACK delay values associated with PDSCH scheduling delay 7 is from 12 to 21 if Alt-2e is configured.

DCI design

In last meeting, the following proposals were discussed:

Agreement In Rel-17, one option will be downselected from Opt-2 and Opt-3 for the 14 HARQ processes feature the “Repetition number” field in RAN1#107e:  Opt-2: 0-bits when the 14 HARQ processes feature is configured (i.e., 2-bits from this field become available for jointly-encoding purposes).  Opt-3: 2-bits as in legacy. Potential Working Assumption#3: In Rel-17, for the 14 HARQ processes feature the “HARQ-ACK delay” field is:  Opt-1: 0-bits when the "HARQ-ACK bundling flag" is set to 1 (i.e., 3-bits from this field become available e.g., for jointly-encoding purposes)  Opt-2: 0-bits when the 14 HARQ processes feature is configured (i.e., 3-bits from this field become available e.g., for jointly-encoding purposes).

As the motivation for 14-HARQ procedure is to increase peak data rate of UEs in good channel conditions, the PDSCH repetition is not needed if 14-HARQ is configured. We don’t see the necessity to dynamically adjust the repetition number field. And obviously, the 3-bits HARQ-ACK delay field is not needed either as it’s indicated by the joint-encoding field. Therefore, Opt-2 is preferred.

Proposal 2: For the 14 HARQ processes feature, Opt-2 is supported for the “Repetition number” field for the 14 HARQ processes feature.

Proposal 3: For the 14 HARQ processes feature the “HARQ-ACK delay” field is 0-bits when the 14 HARQ processes feature is configured (i.e., 3-bits from this field become available e.g., for jointly-encoding purposes).

Conclusions

In this contribution, we provide discussions on introducing 14 HARQ for HD-FDD MTC UEs, and the following observation and proposals are given:

Observation 1: The number of HARQ-ACK delay values required for scheduling delay of 2 is much more than that for scheduling delay 7.

Proposal 1: The set of HARQ-ACK delay values associated with PDSCH scheduling delay 2 is from 4 to 13, 15, 17, and the set of HARQ-ACK delay values associated with PDSCH scheduling delay 7 is from 12 to 21 if Alt-2e is configured.

Proposal 2: For the 14 HARQ processes feature, Opt-2 is supported for the “Repetition number” field for the 14 HARQ processes feature.

Proposal 3: For the 14 HARQ processes feature the “HARQ-ACK delay” field is 0-bits when the 14 HARQ processes feature is configured (i.e., 3-bits from this field become available e.g., for jointly-encoding purposes).

- References

RP-201306, “WID revision: Additional enhancements for NB-IoT and LTE-MTC”, Huawei, HiSilicon, RAN#88e, E-meeting, June 2020.