3GPP TSG-RAN WG1 Meeting #107-e	R1-2111415

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.4.3

Source:	Ericsson

Title:	On HARQ enhancements for NTN

Document for:	Discussion

# Introduction

The work item on solutions for NR to support non-terrestrial networks (NTN) was approved at RAN#86 and the work item description (WID) is updated in [1]. One objective is to specify HARQ enhancements for NTN. An overview of 3GPP non-terrestrial networks can be found in [8] and an overview of state of the art in LEO satellite access can be found in [9].

In this contribution, we discuss HARQ enhancements for NTN per the WID objective:

- HARQ

  - Number of HARQ process [RAN1]

  - Enabling / disabling of HARQ feedback as described in the TR 38.821 [RAN1&2]

# Parameter configuration for feedback enabled/disabled HARQ processes

At RAN1#102-e, the following agreement was made for enabling / disabling HARQ feedback for downlink transmission.

RAN1#102-e:

Agreement:

Enabling/disabling on HARQ feedback for downlink transmission should be at least configurable per HARQ process via UE specific RRC signaling

Given that enabling / disabling of HARQ feedback should be configurable per HARQ process, it is pointed out in Section 7.2.1.4 in TR 38.821 [6] that “If the feedback is disabled for a selective number (i.e. not all) of HARQ processes, the configuration parameters for different HARQ processes may need to be different.”

Disabling HARQ feedback may increase the residual Block Error Rate (BLER) since retransmissions based on HARQ feedback are not possible anymore. For robustness and to minimize the need of RLC AM retransmission, the BLER target of the first transmission should be reduced. If the feedback is disabled for some HARQ processes but not for the others, it is not desirable to apply the same configuration. For example, for a HARQ process with feedback enabled, aggregation factor may not be needed, while for a HARQ process with feedback disabled, aggregation factor may be needed.

RAN1 to discuss what parameters need to be configured differently for HARQ processes with feedback and HARQ processes without feedback. One example parameter is aggregation factor.

# HARQ codebook for enabled/disabled HARQ

With the UE being configured with both feedback disabled HARQ processes and feedback enabled HARQ processes, UE procedures on how to determine the entries in the HARQ codebooks need to be defined. For instance, a UE may receive PDSCHs associated with either a feedback disabled HARQ process or a feedback enabled HARQ process.

## Type-1 HARQ codebook

### Only DCI carrying feedback-disabled processes detected

At RAN1#106-e, the following agreement was made:

Agreement:

For Type-1 HARQ codebook, if only DCI carrying feedback-disabled HARQ process is detected by UE, one of following options should be supported:

- Option-1: The UE’s behavior is same as the case if DCIs carrying the feedback-disabled and feedback-enabled HARQ processes are detected by UE

- Option-2: The UE should skip the codebook feedback at least when the feedback is carried by PUCCH

  - FFS: the case that feedback is carried by PUSCH.

If no DCI scheduling a feedback-enabled DL HARQ process has been received in any of the slots associated with a Type-1 HARQ codebook, the UE can omit sending the HARQ-ACK feedback to reduce UL interference and UE battery consumption. Note that this may be a common scenario in case HARQ feedback is disabled for most HARQ processes.

If DCI scheduling feedback-enabled DL HARQ processes is transmitted but the UE fails to decode it for all slots associated with a Type-1 HARQ codebook, no HARQ feedback will be sent by the UE, even though the network expects HARQ feedback. The network will then know from the absense of HARQ feedback that DCI reception (and therefore also PDSCH reception) failed for all HARQ processes with feedback enabled and can interpret that as implicit NACK for these HARQ processes.

In case of Type-1 HARQ codebook, when the UE has not decoded DCI for a feedback-enabled HARQ process in any slot associated with the HARQ codebook, the UE omits sending the HARQ-ACK feedback when it is not multiplexed with other feedback in the same UCI.

If DCI scheduling feedback-enabled DL HARQ processes is transmitted but no Type-1 HARQ codebook is transmitted by the UE, the network can interpret that as implicit NACK for the feedback-enabled HARQ processes.

### DCI carrying feedback-disabled and enabled processes detected

At RAN1#106-e, the following agreement was made:

Agreement:

For Type-1 HARQ codebook, if DCIs carrying the feedback-disabled and feedback-enabled HARQ processes are detected by UE, one of following options should be supported:

- Option-1: The UE will report NACK only for the feedback-disabled HARQ process regardless of decoding results of corresponding PDSCH

- Option-2: The UE will report NACK/ACK for the feedback-disabled HARQ process depending on the decoding results of corresponding PDSCH

- FFS: Other cases, e.g., if only DCI carrying feedback-disabled HARQ process is detected by UE

One approach is to insert NACKs in the positions corresponding to PDSCHs associated with feedback disabled HARQ processes. Another approach is to insert ACK or NACK also in the positions corresponding to feedback-disabled processes, depending on the decoding outcome. While this feedback is not used to trigger HARQ retransmissions from the network, it may still be useful for other purposes such as BLER-based link adaptation for feedback-disabled HARQ processes.

In case of Type-1 HARQ codebook, when the UE has decoded DCI for both feedback-enabled and feedback-disabled HARQ process in slots associated with the same HARQ codebook , RAN1 to downselect among the following:
1) the UE inserts NACK in positions corresponding to PDSCHs associated with feedback disabled HARQ processes.
2) the UE inserts ACK or NACK in positions corresponding to PDSCHs associated with feedback disabled HARQ processes, depending on the decoding outcome.

## Type-2 HARQ codebook

At RAN1#104-e it was agreed to reduce the Type-2 HARQ codebook size by only including HARQ-ACK of PDSCH with feedback-enabled HARQ processes. At RAN1#105-e, it was agreed that for DCI of PDSCH with feedback-enabled HARQ processes, the C-DAI and T-DAI are the count of only feedback-enabled processes. The details of C-DAI and T-DAI counting for DCI of PDSCH with feedback /disabled HARQ processes were left for further study. At RAN1#106-e, the following agreement was made:

Agreement:

For Type-2 HARQ codebook, for the DCI of PDSCH with feedback-disabled HARQ processes:

- Option-1: The C-DAI and T-DAI are the count of feedback-enabled processes, despite they are not incremented, and are taken into account by the UE

- Option-2: The C-DAI and T-DAI are ignored by the UE regardless of the value

Option 1 is preferred. The UE should use the DAI in DCI of PDSCH with feedback-disabled HARQ process to detect if a previous DCI of PDSCH with feedback-enabled HARQ processes has been missed. In such case, the HARQ codebook size can be correctly chosen based on the DAI in the DCI of the feedback-disabled HARQ process, and the feedback-enabled HARQ process detected to be missing can be NACKed.

In case of Type-2 HARQ codebook:
- When HARQ processes are enabled/disabled on a per HARQ process basis, C-DAI/T-DAI count PDSCH with feedback-enabled HARQ processes. In the DCI, the C-DAI and T-DAI give the count of feedback-enabled HARQ processes regardless of whether a feedback-enabled or feedback-disabled HARQ process is scheduled.
- The UE should use the DAI in DCI of feedback-disabled HARQ process to detect if a previous DCI of feedback-enabled HARQ processes has been missed. In such case, the UE should chose the HARQ codebook size based on the DAI in DCI of the feedback-disabled HARQ process, and the feedback-enabled HARQ process detected to be missed should be NACKed.

## Type-3 HARQ codebook

Type-3 HARQ codebook is introduced in Rel-16, motivated by NR operation in unlicensed spectrum that often requires listen-before-talk (LBT) to assess the availability of the carrier since the carrier may be shared with other networks and may not be available at any given time. It was agreed at RAN1#103-e that Type-3 HARQ codebook can be applied also to licensed spectrum. This is in line with general NR design principle that NR is a toolbox of features and each feature should not be limited to a certain use case or deployment and it is up to implementation to use it as fit. In particular, the Type-3 HARQ codebook can be used to increase robustness of HARQ operation in any network and can be useful for NTN as well. There should not be some artificial restriction that Type-3 HARQ codebook is not applicable to NTN.

RAN1 already agreed that Type-3 HARQ codebook can be applied in licensed spectrum.

NR is a toolbox of features. Each feature should not be limited to a certain use case or deployment and it is up to implementation to use it as fit.

There should not be some artificial restriction that Type-3 HARQ codebook is not applicable to NTN.

In the case of the NR Type-3 HARQ codebook, feedback is provided on all HARQ processes in one shot. In order to reduce unnecessary overhead one simple approach is to only provide ACK/NACK information in the Type-3 codebook for those HARQ processes that are enabled. This allows for an optimized codebook size that does not vary dynamically. The Type 3 codebook can thus be used to increase reliability in some situations.

In the case of the NR Type-3 HARQ codebook, when HARQ processes are enabled/disabled on a per HARQ process basis, the codebook size is dimensioned to include ACK/NACK information only for HARQ processes that are enabled.

# Out-of-order HARQ feedback scheduling restriction for PDSCHs

In the specification, transmission of PDSCHs with different HARQ process IDs is subject to the so-called “out-of-order HARQ restriction” according to clause 5.1 in [7]:

In a given scheduled cell, the UE is not expected to receive a first PDSCH in slot i, with the corresponding HARQ-ACK assigned to be transmitted in slot j, and a second PDSCH starting later than the first PDSCH with its corresponding HARQ-ACK assigned to be transmitted in a slot before slot j.

This is shown in Figure 1.

Figure 1: Out-of-order HARQ feedback scheduling restriction for PDSCH

However, in Rel-17 NTN WI it is agreed that HARQ-ACK feedback can be disabled per HARQ process. In this case, the restriction of PDSCH scheduling when one or both of the HARQ processes are disabled is not clear. This is important because the timing restriction is defined based on the UE processing capability. Therefore, it is necessary to discuss how to adapt the timing restriction for PDSCH scheduling when the HARQ feedback is disabled.

Currently out-of-order HARQ timing restriction is defined for PDSCH scheduling based on the HARQ-ACK feedback in TS 38.214. It is not clear what UE should expect regarding PDSCH scheduling timing when HARQ feedback is disabled. Therefore, it is necessary to discuss the scheduling restriction when HARQ feedback is disabled.

Depending on which one of the HARQ processes is disabled in the above situation, 3 different cases can be considered. One way to resolve the specification issue is to preserve the same “out-of-order restriction” for the diabled HARQ process. The drawback with this approach is putting unnecessary restriction on the scheduler since there is no need for UE to send feedback when HARQ is disabled. Since for disabled HARQ processes, the UE does is not required to send back the HARQ-ACK feedback, another approach would be to remove the “out-of-order restriction” for the disabled HARQ processes. According to this solution, there is no need to indicate a K1 value for the disabled HARQ process to the UE, either it is not sent in DCI or it is ignored by the UE. Relaxing the restriction in this way gives the scheduler more flexibility and also reduces the latency for PDSCH transmission.

Regarding the “out-of-order restriction” for transmission of different HARQ processes corresponding to different PDSCHs, RAN1 to down-select between following alternatives:
Alt-1: Preserve the same “out-of-order restriction” for (enabled) HARQ processes in specification for the disabled HARQ processes
Alt-2: the “out-of-order restriction” for (enabled) HARQ processes does not apply to disabled HARQ processes.

# SPS activation

HARQ feedback is enabled/disabled per HARQ process by the network. For SPS, a subset of the HARQ processes is configured to be used in a given SPS configuration. A straightforward way for the network to get confirmation of SPS activation is to enable HARQ feedback for one or more of the HARQ processes associated with the SPS allocation.

It has been proposed to introduce an exception that HARQ feedback is mandated for SPS activation. However, we prefer keeping the solution simple by avoiding introduction of exceptions. In addition to the SPS activation case, other cases such as Scell dormancy have been identified for which similar exceptions would be required.

For SPS activation, HARQ feedback is not sent if the network has disabled HARQ feedback for the used HARQ process.

# Combination of HARQ retransmissions with disabled feedback

At RAN1#106b-e, the capability of the UE to combine HARQ retransmissions of a feedback-disabled HARQ process was discussed.

At RAN2#112-e , RAN2 agreed the following:

”From a RAN2 perspective, for DL, HARQ feedback can be enabled/disabled in Rel-17 NTN, but HARQ processes remain configured. The criteria and decision to enable/disable HARQ feedback is under network control and is signalled to the UE via RRC in a semi-static manner.”

Note that it is the HARQ feedback that is disabled, not the HARQ process. The capability of the UE to combine HARQ retransmissions should not depend on whether HARQ feedback is enabled or disabled, or retransmissions are sent blindly or sent based on HARQ feedback.

Also, feedback-enabling/disabling is configured by the network. The UE must support the case that all HARQ processes are feedback-enabled, i.e., have sufficient soft-buffer for all HARQ processes. Thus, there is no benefit in not using the soft-buffer for feedback-disabled HARQ processes.

Since feedback-enabling/disabling of HARQ processes is configured by the network, the UE must support the case that all HARQ processes are feedback-enabled, i.e., have sufficient soft-buffer for all HARQ processes. Thus, there is no benefit in not using the soft-buffer if HARQ retransmissions are sent for feedback-disabled HARQ processes.

Even if HARQ feedback is disabled by the network, the HARQ process remains configured and HARQ retransmissions may occur.

RAN1 should conclude that the configuration of HARQ feedback as enabled or disabled does not impact the UE capability to buffer and combine HARQ retransmissions.

# Performance enhancement

It has been proposed to support aggregation factor 16 or 32 for PDSCH. However, simulations in [5] show that with low SE MCS table, AF=8 is sufficient even if BLER target is as low as 0.1%. Support for lower BLER target than 0.1% does not seem justified.

Further, the PDSCH is not necessarily the bottleneck for coverage. Coverage enhancement, including repetition enhancements and other enhancements, has been proposed in RAN plenary for an NR NTN evolution WI in Rel-18 [2]. It is premature to single out one specific enhancement for one specific channel in Rel-17 without proper evaluation of the system level impact on coverage. Therefore, we do not support this enhancement in Rel-17.

Increased aggregation factor for PDSCH has not been justified.

Do not increase the aggregation factor for PDSCH.

# RRC parameters

The outcome of the intial discussion on RRC parameters related to HARQ enhancements is summarized in [4].

Several companies are proposing in the UE capability discussion to have separate DL and UL capabilities for 32 HARQ processes in NR NTN. Given this, we strongly recommend adding an RRC parameter “nrofHARQ-ProcessesForPUSCH-r17”, to respect the RAN2 guideline (as documented in [3] and copied below):

5          Avoid defining functionality that has no RRC configuration but is dependent on capability bits.

The specification should not be written so that the network determines what configuration it can use for a UE implicitly by the reported UE capabilities. Instead, the gNB should always configure the UE explicitly by DL RRC signalling, respecting  the reported capabilities.

Add RRC parameter nrofHARQ-ProcessesForPUSCH-r17 indicating the number of HARQ processes to be used on the PUSCH of a serving cell if separate UE capabilities are defined for supported number of HARQ processes in UL and DL.

For the parameters indicating the size of the HARQ process ID field in DCI formats 0_1 and 1_1 – HARQ-ProcessNumberSizeDCI-0-1-r17 and HARQ-ProcessNumberSizeDCI-1-1-r17, the value range is said to be 0..5. However, the agreement for DCI format 0_1 and 1_1 is only to extend the size up to 5 bits, and since the current field is fixed to 4 bits, the value range should be {4,5}.

For RRC parameters HARQ-ProcessNumberSizeDCI-0-1-r17 and HARQ-ProcessNumberSizeDCI-1-1-r17, the value range should be {4,5}.

# Conclusion

In the previous sections, we discuss HARQ enhancements for NTN. We made the following observations:

Observation 1	If DCI scheduling feedback-enabled DL HARQ processes is transmitted but no Type-1 HARQ codebook is transmitted by the UE, the network can interpret that as implicit NACK for the feedback-enabled HARQ processes.

Observation 2	RAN1 already agreed that Type-3 HARQ codebook can be applied in licensed spectrum.

Observation 3	NR is a toolbox of features. Each feature should not be limited to a certain use case or deployment and it is up to implementation to use it as fit.

Observation 4	There should not be some artificial restriction that Type-3 HARQ codebook is not applicable to NTN.

Observation 5	Currently out-of-order HARQ timing restriction is defined for PDSCH scheduling based on the HARQ-ACK feedback in TS 38.214. It is not clear what UE should expect regarding PDSCH scheduling timing when HARQ feedback is disabled. Therefore, it is necessary to discuss the scheduling restriction when HARQ feedback is disabled.

Observation 6	Since feedback-enabling/disabling of HARQ processes is configured by the network, the UE must support the case that all HARQ processes are feedback-enabled, i.e., have sufficient soft-buffer for all HARQ processes. Thus, there is no benefit in not using the soft-buffer if HARQ retransmissions are sent for feedback-disabled HARQ processes.

Observation 7	Even if HARQ feedback is disabled by the network, the HARQ process remains configured and HARQ retransmissions may occur.

Observation 8	Increased aggregation factor for PDSCH has not been justified.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN1 to discuss what parameters need to be configured differently for HARQ processes with feedback and HARQ processes without feedback. One example parameter is aggregation factor.

Proposal 2	In case of Type-1 HARQ codebook, when the UE has not decoded DCI for a feedback-enabled HARQ process in any slot associated with the HARQ codebook, the UE omits sending the HARQ-ACK feedback when it is not multiplexed with other feedback in the same UCI.

Proposal 3	In case of Type-1 HARQ codebook, when the UE has decoded DCI for both feedback-enabled and feedback-disabled HARQ process in slots associated with the same HARQ codebook , RAN1 to downselect among the following: 1) the UE inserts NACK in positions corresponding to PDSCHs associated with feedback disabled HARQ processes. 2) the UE inserts ACK or NACK in positions corresponding to PDSCHs associated with feedback disabled HARQ processes, depending on the decoding outcome.

Proposal 4	In case of Type-2 HARQ codebook: - When HARQ processes are enabled/disabled on a per HARQ process basis, C-DAI/T-DAI count PDSCH with feedback-enabled HARQ processes. In the DCI, the C-DAI and T-DAI give the count of feedback-enabled HARQ processes regardless of whether a feedback-enabled or feedback-disabled HARQ process is scheduled. - The UE should use the DAI in DCI of feedback-disabled HARQ process to detect if a previous DCI of feedback-enabled HARQ processes has been missed. In such case, the UE should chose the HARQ codebook size based on the DAI in DCI of the feedback-disabled HARQ process, and the feedback-enabled HARQ process detected to be missed should be NACKed.

Proposal 5	In the case of the NR Type-3 HARQ codebook, when HARQ processes are enabled/disabled on a per HARQ process basis, the codebook size is dimensioned to include ACK/NACK information only for HARQ processes that are enabled.

Proposal 6	Regarding the “out-of-order restriction” for transmission of different HARQ processes corresponding to different PDSCHs, RAN1 to down-select between following alternatives: Alt-1: Preserve the same “out-of-order restriction” for (enabled) HARQ processes in specification for the disabled HARQ processes Alt-2: the “out-of-order restriction” for (enabled) HARQ processes does not apply to disabled HARQ processes.

Proposal 7	For SPS activation, HARQ feedback is not sent if the network has disabled HARQ feedback for the used HARQ process.

Proposal 8	RAN1 should conclude that the configuration of HARQ feedback as enabled or disabled does not impact the UE capability to buffer and combine HARQ retransmissions.

Proposal 9	Do not increase the aggregation factor for PDSCH.

Proposal 10	Add RRC parameter nrofHARQ-ProcessesForPUSCH-r17 indicating the number of HARQ processes to be used on the PUSCH of a serving cell if separate UE capabilities are defined for supported number of HARQ processes in UL and DL.

Proposal 11	For RRC parameters HARQ-ProcessNumberSizeDCI-0-1-r17 and HARQ-ProcessNumberSizeDCI-1-1-r17, the value range should be {4,5}.

# References

RP-211784, “Solutions for NR to support non-terrestrial networks (NTN),” 3GPP TSG RAN #93-e, Sep 2021.

RP-212713, ”New WI: NR NTN (Non-Terrestrial Networks) enhancements”, RAN Vice-Chair (AT&T), December 2021

R1-2001513, Guidelines for UE capability definitions

R1-2110624, ”Summary of [106bis-e-R17-RRC-NR-NTN] Email discussion on Rel-17 RRC parameters for NR to support NTN”, Moderator (Thales), Oct 2021

R1-2109812, ”Discussion on HARQ Enhancements for NTN”, ETRI, Oct 2021

TR 38.821, “Solutions for NR to support non-terrestrial networks”.

TS 38.214, “NR Physical layer procedures for data”, v16.4.0, Dec, 2020.

X. Lin, S. Rommer, S. Euler, E. A. Yavuz, and R. S. Karlsson, “5G from space: An overview of 3GPP non-terrestrial networks,” to appear in IEEE Communications Standards Magazine, March 2021. Available at arXiv preprint arXiv:2103.09156.

X. Lin, S. Cioni, G. Charbit, N. Chuberre, S. Hellsten, and J.-F. Boutillon, “On the path to 6G: Embracing the next wave of low Earth orbit satellite access,” to appear in IEEE Communications Magazine, April 2021. Available at arXiv preprint arXiv:2104.10533.