3GPP TSG RAN WG1 #107-e                                                      R1-2111529

e-Meeting, November 11th – 19th, 2021

Source: 	Intel Corporation

Title: 	UE Features for NR Positioning Enhancements

Agenda item:	8.16.5

Document for: 	Discussion and Decision

# Introduction

In this contribution, we provide our initial views on UE features for NR positioning enhancements based on input provided in [1] and considering the following agreement made at RAN1#106bis-e [2] (please refer to conclusion section):

Agreement Agree the following table as baseline for further discussions during RAN1 #107-e, incl. the yellow highlighting and the revisions marked with chromatic formatting

Note: table referred in agreement is provided in Annex A for reader convenience.

# Views on UE Features for NR Positioning Enhancements

## Latest Version of UE Features for NR Positioning Enhancements

The initial (non-endorsed) version of UE features for NR sidelink enhancements was captured in [1] and is expected to be used as a starting point for further discussion (please refer to Table 1):

Table 1: Initial feature groups for NR positioning enhancements from source - R1-2110587

| 27. NR_pos_enh | 27-1-1 | Maximum number of UE-RxTEGs [for UE-assisted DL TDOA and/or Multi-RTT positioning] |
| --- | --- | --- |
| 27. NR_pos_enh | 27-1-2 | Maximum number of UE-TxTEGs [for UL TDOA and/or Multi-RTT positioning] |
| 27. NR_pos_enh | 27-1-3 | Maximum number of UE-RxTxTEGs for Multi-RTT |
| 27. NR_pos_enh | 27-1-4 | The maximum Number of UE Rx TEGs for measuring the same DL PRS resource |
| 27. NR_pos_enh | 27-2-1 | [UE-assisted] DL PRS RSRP of the first [or additional] path [for DL-AoD] |
| 27. NR_pos_enh | 27-2-2 | DL PRS RSRP reporting for more than 8 measurements for UE-assisted DL-AoD positioning |
| 27. NR_pos_enh | 27-3-1 | M-sample measurements [of DL PRS measurement on single DL PRS period/occasion] |
| 27. NR_pos_enh | 27-3-2 | DL PRS measurement outside MG [and in a PRS processing priority window] - processing types |
| 27. NR_pos_enh | 27-3-3 | DL PRS Processing Capability outside MG - buffering capability |
| 27. NR_pos_enh | 27-4-1 | LOS/NLOS Indicator for UE-assisted positioning |
| 27. NR_pos_enh | 27-5-1 | [UE-initiated] on-demand PRS |

The above table is not aligned yet with the latest agreements made by RAN1 and needs further revision to introduce new FGs as discussed in the next section and corresponding sub-sections of this document.

## Feature Groups for NR Positioning Enhancements

### FGs for NR Positioning Latency Reduction

RAN1 agreed to introduce a new UE capability on lower RX beam sweeping factor (<8) to reduce the PRS measurement latency for FR2 positioning frequency layers.

Support of reduced RX beam sweeping factor

RAN1 agreed to introduce DL MAC CE for MG activation for the purpose of DL PRS measurements. The following new FG can be introduced to reflect it:

Support of low latency MG activation request from LMF (DL MAC CE for MG activation request)

Finally, RAN1 agreed to introduce UL MAC CE for MG activation request by UE. The following new FG can be introduced to reflect it:

Support of low latency MG activation request from UE (UL MAC CE for MG activation request)

Additional FGs may need to be defined for NR latency reduction support, once more progress on design options is made in RAN1.

Define the following new FGs for NR positioning with lower latency

  - 27-3-2: Support of reduced RX beam sweeping factor for FR2

    - Support of RX beam sweeping factor < 8

  - 27-3-3: Support of low latency MG activation request from LMF

    - Support of DL MAC CE for MG activation request

  - 27-3-4: Support of low latency MG activation request from UE

    - Support of UL MAC CE for MG activation request

In addition, the extension of the set of existing values for DL PRS processing capabilities within MG, in particular the processing window T can be beneficial for latency reduction and set of values {1 ms, 2 ms, 4 ms} can be introduced.

Introduce additional values {1ms, 2ms, 4 ms} for parameter T defined for UE DL PRS processing capabilities within MG

Add new values for R17 UEs for Rel-16 FG 13-1 Common DL PRS Processing Capability

  - Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE.

  - T: {1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms

### FGs for NR Positioning LOS/NLOS Mitigation

LOS/NLOS indication

Additional FGs need to be introduced reflecting LOS/NLOS mitigation framework developed by RAN1. At least the following FGs need to be specified.

- LoS/NLoS indicator for DL-TDOA positioning / DL PRS RSTD measurement

- LoS/NLoS indicator for DL-AOD positioning / DL PRS RSRP measurement

- LoS/NLoS indicator for Multi-RTT

Each of the above FGs can be further split into two FGs related to soft / hard value for LOS/NLOS metric. In addition, dedicated FG needs to be introduced for LoS/NLoS assistance information to support UE-based positioning.

  - Replace FG 27-4-1 with multiple FGs associated with NR positioning LOS/NLOS mitigation capabilities per positioning method

    - LoS/NLoS soft/hard value indicator for DL-TDOA positioning / DL PRS RSTD measurement

    - LoS/NLoS soft/hard value indicator for DL-AOD positioning / DL PRS RSRP measurement

    - LoS/NLoS soft/hard value indicator for Multi-RTT

  - Introduce additional FG indicating that UE can use LoS/NLoS assistance information for UE based positioning

    - LoS/NLoS assistance information for UE-based positioning

DL RPS RSRP for first and additional paths

It was agreed by RAN1 to support reporting the path RSRP for the first path and for additional paths as part of DL-TDOA, UL-TDOA, and multi-RTT reporting enhancements. This agreement require definition of at least 4 new FGs:

  - Define FGs associated with the per path DL PRS RSRP measurement support instead of single FG 27-2-1

    - First path DL PRS RSRP for DL-TDOA positioning

    - First path DL PRS RSRP for DL-AOD positioning

    - First path DL PRS RSRP for Multi-RTT positioning

    - Additional path DL PRS RSRP for DL-TDOA positioning

    - Additional path DL PRS RSRP for DL-AoD positioning

    - Additional path DL PRS RSRP for Multi-RTT positioning

### FGs for NR Positioning in RRC_INACTIVE State

UE NR Positioning Measurements in RRC_INACTIVE state

Support of NR positioning by RRC_INACTIVE UEs has been added in Rel.17 for DL, UL and DL+UL positioning methods. The corresponding FGs need to be added to UE feature list.

The first family of the FGs that needs to be added is associated with NR positioning measurement capabilities for UEs in RRC_INACTIVE state.

  - Define FGs associated with NR positioning measurements in RRC-INACTIVE state

    - FG 27-x2-1: DL PRS measurement for DL-AoD in RRC_INACTIVE state

    - FG 27-x2-2: DL PRS measurement for DL-TDoA in RRC_INACTIVE state

    - FG 27-x2-3: UE Rx-Tx measurement for Multi-RTT in RRC_INACTIVE state

    - FG 27-x2-4: SS-RSRP RRM measurements for NR E-CID positioning in RRC_INACTIVE state

    - FG 27-x2-5: SS-RSRQ RRM measurements for NR E-CID positioning in RRC_INACTIVE state

    - FG 27-x2-6: CSI-RSRP RRM measurements for NR E-CID positioning in RRC_INACTIVE state

    - FG 27-x2-7: CSI-RSRQ RRM measurements for NR E-CID positioning in RRC_INACTIVE state

    - FG 27-x2-8: Simultaneous DL-AoD and DL-TDoA processing in RRC_INACTIVE state

    - FG 27-x2-9: Simultaneous DL-AoD and Multi-RTT processing in RRC_INACTIVE state

In Rel.16, the FGs for DL PRS resources for various NR positioning techniques were defined [3]. The same FGs are in general applicable for NR positioning by RRC-INACTIVE UEs.

  - The Rel.16 NR positioning FGs (13-2/2a/2b, 13-3/3a/3b, 13-4/4a/4b) for DL PRS resources and different NR positioning techniques are reused and defined as pre-requisites for FGs associated with NR positioning measurements by RRC-INACTIVE UEs

SRS for positioning transmission by RRC_INACTIVE UEs

RAN1 has agreed on SRS for positioning transmission by RRC_INACTIVE UEs. The discussion on type of SRS for positioning to be supported (except periodic SRS for positioning) is still open. The support of OLPC and spatial relationship was agreed to be reused from Rel.16. The support of SRS for positioning by RRC_INACTIVE UEs should be reflected in UE features. gNB should be at least aware about support of SRS for positioning transmission in RRC_INACTIVE state. Another aspect that needs to be discussed is that for RRC_CONNECTED UEs SRS for positioning is configured within active UL BWP.  RAN1 agreed that SRS for positioning in RRC-INACTIVE state can be configured outside of initial UL-BWP and have different numerology settings. This may require redefinition of the FGs for SRS for positioning resources for RRC-INACTIVE UEs (as there is no motivation to introduce parameters that are defined per UL BWP). Based on discussion, we have following proposal.

  - Define new FGs for SRS for positioning support in RRC_INACTIVE state:

    - FG 27-x3-1: SRS for positioning transmission in RRC_INACTIVE state

    - FG 27-x3-2: Support of SRS for positioning resources in RRC_INACTIVE state

    - FG 27-x3-3: [Support of periodic SRS for positioning resources in RRC_INACTIVE]

    - FG 27-x3-4: [Support of semi-persistent SRS for positioning resources in RRC_INACTIVE]

  - Further discuss whether to reuse R16 FGs for SRS for positioning OLPC (FGs 13-9/9a/9b/9e/9f) and spatial relation (FGs 13-10/10a/10b/10d/10e/10f) or define new FGs for support of SRS for positioning in RRC_INACTIVE state (considering R16 FGs defined for RRC_CONNECTED state), i.e.:

    - OLPC

      - FG 27-x4-1: OLPC for SRS for positioning based on DL PRS from serving cell

      - FG 27-x4-2: OLPC for SRS for positioning based on DL PRS from neighboring cells

      - FG 27-x4-3: OLPC for SRS for positioning based on SSB from serving cells

      - FG 27-x4-4: Pathloss estimate maintenance for serving cell

      - FG 27-x4-5: Pathloss estimate maintenance across all cells

    - Spatial relation

      - FG 27-x5-1: Spatial relation for SRS for positioning based on SSB from serving cell

      - FG 27-x5-2: Spatial relation for SRS for positioning based on CSI-RS from serving cell

      - FG 27-x5-3: Spatial relation for SRS for positioning based on DL PRS from serving cell

      - FG 27-x5-4: Spatial relation for SRS for positioning based on SSB from non-serving cells

      - FG 27-x5-5: Spatial relation for SRS for positioning based on DL PRS from non-serving cells

      - FG 27-x5-6: Spatial relation maintenance

### FGs for On-Demand DL PRS Support

The UE-initiated on-demand DL PRS request from UE to LMF was agreed. The support of this functionality requires definition of a new FG family. Given that LMF initiated on-demand DL PRS request is also supported it is recommended to update FG 27-5-1.

  - Update FG 27-5-1 as follows: UE initiated on-demand DL PRS request

# Conclusions

In this contribution, we have provided our initial views on UE features for R17 WI on NR positioning enhancements. In summary, we have following proposals:

Define the following new FGs for NR positioning with lower latency

  - 27-3-2: Support of reduced RX beam sweeping factor for FR2

    - Support of RX beam sweeping factor < 8

  - 27-3-3: Support of low latency MG activation request from LMF

    - Support of DL MAC CE for MG activation request

  - 27-3-4: Support of low latency MG activation request from UE

    - Support of UL MAC CE for MG activation request

Introduce additional values {1ms, 2ms, 4 ms} for parameter T defined for UE DL PRS processing capabilities within MG

Add new values for R17 UEs for Rel-16 FG 13-1 Common DL PRS Processing Capability

  - Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE.

  - T: {1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms

  - Replace FG 27-4-1 with multiple FGs associated with NR positioning LOS/NLOS mitigation capabilities per positioning method

    - LoS/NLoS soft/hard value indicator for DL-TDOA positioning / DL PRS RSTD measurement

    - LoS/NLoS soft/hard value indicator for DL-AOD positioning / DL PRS RSRP measurement

    - LoS/NLoS soft/hard value indicator for Multi-RTT

  - Introduce additional FG indicating that UE can use LoS/NLoS assistance information for UE based positioning

    - LoS/NLoS assistance information for UE-based positioning

  - Define FGs associated with the per path DL PRS RSRP measurement support instead of single FG 27-2-1

    - First path DL PRS RSRP for DL-TDOA positioning

    - First path DL PRS RSRP for DL-AOD positioning

    - First path DL PRS RSRP for Multi-RTT positioning

    - Additional path DL PRS RSRP for DL-TDOA positioning

    - Additional path DL PRS RSRP for DL-AoD positioning

    - Additional path DL PRS RSRP for Multi-RTT positioning

  - Define FGs associated with NR positioning measurements in RRC-INACTIVE state

    - FG 27-x2-1: DL PRS measurement for DL-AoD in RRC_INACTIVE state

    - FG 27-x2-2: DL PRS measurement for DL-TDoA in RRC_INACTIVE state

    - FG 27-x2-3: UE Rx-Tx measurement for Multi-RTT in RRC_INACTIVE state

    - FG 27-x2-4: SS-RSRP RRM measurements for NR E-CID positioning in RRC_INACTIVE state

    - FG 27-x2-5: SS-RSRQ RRM measurements for NR E-CID positioning in RRC_INACTIVE state

    - FG 27-x2-6: CSI-RSRP RRM measurements for NR E-CID positioning in RRC_INACTIVE state

    - FG 27-x2-7: CSI-RSRQ RRM measurements for NR E-CID positioning in RRC_INACTIVE state

    - FG 27-x2-8: Simultaneous DL-AoD and DL-TDoA processing in RRC_INACTIVE state

    - FG 27-x2-9: Simultaneous DL-AoD and Multi-RTT processing in RRC_INACTIVE state

  - The Rel.16 NR positioning FGs (13-2/2a/2b, 13-3/3a/3b, 13-4/4a/4b) for DL PRS resources and different NR positioning techniques are reused and defined as pre-requisites for FGs associated with NR positioning measurements by RRC-INACTIVE UEs

  - Define new FGs for SRS for positioning support in RRC_INACTIVE state:

    - FG 27-x3-1: SRS for positioning transmission in RRC_INACTIVE state

    - FG 27-x3-2: Support of SRS for positioning resources in RRC_INACTIVE state

    - FG 27-x3-3: [Support of periodic SRS for positioning resources in RRC_INACTIVE]

    - FG 27-x3-4: [Support of semi-persistent SRS for positioning resources in RRC_INACTIVE]

  - Further discuss whether to reuse R16 FGs for SRS for positioning OLPC (FGs 13-9/9a/9b/9e/9f) and spatial relation (FGs 13-10/10a/10b/10d/10e/10f) or define new FGs for support of SRS for positioning in RRC_INACTIVE state (considering R16 FGs defined for RRC_CONNECTED state), i.e.:

    - OLPC

      - FG 27-x4-1: OLPC for SRS for positioning based on DL PRS from serving cell

      - FG 27-x4-2: OLPC for SRS for positioning based on DL PRS from neighboring cells

      - FG 27-x4-3: OLPC for SRS for positioning based on SSB from serving cells

      - FG 27-x4-4: Pathloss estimate maintenance for serving cell

      - FG 27-x4-5: Pathloss estimate maintenance across all cells

    - Spatial relation

      - FG 27-x5-1: Spatial relation for SRS for positioning based on SSB from serving cell

      - FG 27-x5-2: Spatial relation for SRS for positioning based on CSI-RS from serving cell

      - FG 27-x5-3: Spatial relation for SRS for positioning based on DL PRS from serving cell

      - FG 27-x5-4: Spatial relation for SRS for positioning based on SSB from non-serving cells

      - FG 27-x5-5: Spatial relation for SRS for positioning based on DL PRS from non-serving cells

      - FG 27-x5-6: Spatial relation maintenance

  - Update FG 27-5-1 as follows: UE initiated on-demand DL PRS request

# References

- R1-2110587, Updated RAN1 UE features list for Rel-17 NR after RAN1 #106bis-e, Moderators (AT&T, NTT DOCOMO, INC.), October 2021

- R1-2109915, Summary of UE features for NR positioning enhancements, Moderator (AT&T)

- R1-2102006, Updated RAN1 UE features list for Rel-16 NR after RAN1#104-e, Moderators (AT&T, NTT DOCOMO, INC.)

# Annex A: UE features for NR positioning enhancements from [R1-2110587] – After RAN1#106bis-e

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-1-1 | Maximum number of UE-RxTEGs [for UE-assisted DL TDOA and/or Multi-RTT positioning] | The maximum number of UE-RxTEG, which is supported and reported by UE [for DL TDOA and/or Multi-RTT positioning] FFS: the values (>1) FFS: whether to have a value=1 to indicate UE Rx timing errors is well calibrated FFS: whether to have separate values/FGs for DL TDOA and/or Multi-RTT positioning [If UE supports this capability with the values > 1, the UE supports including one UE Rx TEG ID for the RSTD reference time and one UE Rx TEG ID for each DL RSTD measurement (including each additional DL RSTD measurement), in a DL TDOA measurement report] | [13-3] | No |  | Mitigation of UE Rx timing delays is not supported | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1,2,4,6,8,12,16,24,32}] Need for location server to know if the feature is supported FFS: Separate row for “Support of UE-RxTEG reporting for DL-TDOA”, and “Support of UE-RxTEG reporting for M-RTT” | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-2 | Maximum number of UE-TxTEGs [for UL TDOA and/or Multi-RTT positioning] | The maximum number of UE-TxTEG, which is supported and reported by UE [for UL TDOA and/or Multi-RTT positioning] FFS: the values (>1) FFS: whether to have a value=1 to indicate UE Tx timing errors is well calibrated FFS: whether to have different values/FGs for UL TDOA and/or Multi-RTT positioning FFS: Separate row for “Support of UE-TxTEG reporting for MRTT”, and a separate row for the “maximum number of TxTEGs for RTT” [If UE supports this capability with the values > 1, the UE supports to provide the association information of UL SRS resources for positioning with Tx TEGs to the LMF. - FFS: Whether the association information is sent directly from UE to LMF, or is first provided to gNB and then forwarded to LMF] | [13-4, 13-8] | FFS |  | Mitigation of UE Tx timing delays is not supported | FFS: Per UE or per band or per FS | n/a | n/a | n/a | [The candidate values are {1, 2, 4, 8, 16, 32}] Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-3 | Maximum number of UE-RxTxTEGs for Multi-RTT | The maximum number of UE-RxTxTEG, which is supported and reported by UE for Multi-RTT positioning FFS: the values (>1) FFS: whether to have a value=1 to indicate UE RxTx timing errors is well calibrated [If a UE support this capability with the values > 1, the UE supports reporting of UE RxTx TEG ID with UE Rx-Tx time difference measurements for Multi-RTT positioning] | [13-4, 13-8] | No |  | Mitigation of UE RxTx timing delays is not supported | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1, 2, 4, 6, 8, 12, 16, 24, 32, 64, [128]}] Need for location server to know if the feature is supported FFS: Separate row for “Support of UE-RxTxTEG reporting for Multi-RTT” | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-4 | The maximum Number of UE Rx TEGs for measuring the same DL PRS resource | The maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP. FFS: The values (>1) | 27-1-1 | No |  | Mitigation of UE Rx timing delays by using different Rx TEGs are not supported | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1, 2, 4, 8}] Need for location server to know if the feature is supported [Note: UE measures the same instance of the DL PRS with multiple RX TEGs] FFS: Separate row for “Support measuring the same DL PRS of a TRP with different UE-RxTEGs” FFS: Separate row for “The maximum Number of UE Rx TEGs for measuring the same DL PRS resource simultaneously” | Optional with capability signaling |
| 27. NR_pos_enh | 27-2-1 | [UE-assisted] DL PRS RSRP of the first [or additional] path [for DL-AoD] | Support of [measuring and reporting the] PRS RSRP of the first path [or additional] [for DL-AoD positioning method] [Note: Applicable for DL-TDOA and Multi-RTT] | [13-2 or 13-3] | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-2-2 | DL PRS RSRP reporting for more than 8 measurements for UE-assisted DL-AoD positioning | Support reporting K> 8 DL PRS RSRP measurements per TRP. FFS: the values of K Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. FFS: Additional capability may be added to limit the maximum number of DL PRS RSRP associated with the same Rx beam index | [13-5] | No |  | UE report of more than 8 DL PRS-RSRP is not supported. | FFS: Per UE or per band | n/a | FFS | n/a | [The candidate values are {12, 16, 32, 64}] Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-3-1 | M-sample measurements [of DL PRS measurement on single DL PRS period/occasion] | The capability to support reporting a measurement based on measuring M samples (instances) of a DL PRS resource set M=1[,2-3]. FFS: other values. If the UE does not provide the capability, the UE [is assumed to] support M=4 only. | [13-1] | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1, 2, 3}] Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-3-2 | DL PRS measurement outside MG [and in a PRS processing priority window] - processing types | 1. Supported PRS processing types subject to the UE determining that DL PRS to be higher priority for PRS measurement outside MG [and in a PRS processing priority window] Candidate values: {Type 1A, Type 1B, Type 2}. Note: - Type 1A refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from all DL CCs (per UE) are affected. - Type 1B refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from certain DL CCs are affected. - Type 2 refers to DL PRS being prioritized over other DL signals/channels only in DL PRS symbols within the PRS processing priority window. Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options [Note: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP] | [13-1] | FFS |  |  | FFS: Per UE or per band | n/a | n/a | n/a | Need for location server to know if the feature is supported FFS: Separate feature group for a UE to declare support of each of the Type-1A, Type-1B, Type-2” capabilities | Optional with capability signaling |
| 27. NR_pos_enh | 27-3-3 | DL PRS Processing Capability outside MG - buffering capability | 1. DL PRS buffering capability: Type 1 or Type 2 a) T: [{8, 16, 20, 30, 40, 80, 160, 320, 640, 1280}] ms b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) Type 1 – sub-slot/symbol level buffering b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz | 27-3-2 | No |  |  | Per band | n/a | n/a | n/a | FFS: Separate feature group for a UE to declare PRS processing capabilities of each of the Type-1A, Type-1B, Type-2” capabilities | Optional with capability signaling |
| 27. NR_pos_enh | 27-4-1 | LOS/NLOS Indicator for UE-assisted positioning | Support reporting LoS/NLoS indicator to LMF [for RSTD and UE Rx-Tx time difference measurements to LMF for DL and DL+UL positioning] FFS: whether to have separate capability component/FG for RSTD and UE Rx-Tx time difference measurements FFS: whether to have separate capability component for hard and soft indication |  | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate value are [0,1]] Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-5-1 | [UE-initiated] on-demand PRS | UE’s capability to support UE-initiated on-demand DL PRS [request signalling] | [13-1] | No |  |  | Per UE | n/a | n/a | n/a | FFS: Need for location server to know if the feature is supported | Optional with capability signaling |