3GPP TSG RAN WG1 #107-e		R1-2111644

e-meeting, November 11th – 19th, 2021

Source:	Lenovo, Motorola Mobility

Title: 	PDSCH/PUSCH scheduling enhancements for NR from 52.6 GHz to 71GHz

Agenda Item:   8.2.5

Document for:	Discussion

# 1	Introduction

In RAN#93-e, a revised work item for supporting NR from 52.6 GHz to 71 GHz in NR Rel. 17 has been approved with following objectives for PDSCH/PUSCH scheduling in [1]:

- Physical layer aspects including [RAN1]:

  - In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported.

Note: Except for timing line related aspects, a common design framework shall be adopted for 480kHz to 960kHz

  - Time line related aspects adapted to 480kHz and 960kHz, e.g., BWP and beam switching timeing, HARQ timing, UE processing, preparation and computation timelines for PDSCH, PUSCH/SRS and CSI, respectively.

  - Support enhancements for multi-PDSCH/PUSCH scheduling and HARQ support with a single DCI

Note: coverage enhancement for multi-PDSCH/PUSCH scheduling is not pursued

  - Evaluate, and if needed, specify the PTRS enhancement for 120kHz SCS, 480kHz SCS and/or 960kHz SCS, as well as DMRS enhancement for 480kHz SCS and/or 960kHz SCS.

In this contribution, we provide our views on following enhancements for NR operation between 52.6 GHz to 71 GHz with newly agreed SCS values including 480kHz and 960kHz:

- Multi-PDSCH/PUSCH scheduling by single DCI

  - Scheduling restrictions over multiple slots

  - HARQ-ACK timing

  - Time-domain bundling for HARQ codebook

- PDSCH/PUSCH enhancements for new SCS

  - CSI processing and CPU availability

  - UE-assisted numerology adaptation

# 2	Multi-PDSCH/PUSCH scheduling by single DCI

## 2.1 Scheduling restrictions over multiple slots

In RAN1#106bis-e [2], following agreement has been made related to multi-PDSCH scheduling for 120 kHz:

Agreement:

Confirm the working assumption from RAN1#106-e with the following modification.

Working assumption: (RAN1#106-e)

Scheduling multiple PDSCHs by single DL DCI applies to 120 kHz in addition to 480 and 960 kHz at least in FR2-2.

- FFS: Further limitations on maximum number of PDSCHs

- Note: Further limitations (in addition to what was agreed earlier) on the maximum number of PDSCHs or PUSCHs can be separately discussed for all SCSs.

Furthermore, following agreement has also been made in RAN1#106-e [3] related to maximum number of PDSCH/PUSCH that can be scheduled by single DCI.

Agreement:

- The maximum number of PDSCHs/PUSCHs that can be scheduled with a single DCI in Rel-17 is 8 for SCS of 120, 480 and 960 kHz.

- FFS: Whether UE capability is introduced for restricting the maximum number of PDSCHs or PUSCHs that can be scheduled with a single DCI

In our view, this discussion on restricting the maximum number of PDSCHs/PUSCHs is tied to the PDCCH multi-slot monitoring discussion. The maximum duration for both SCS 480 kHz and 120 kHz should be at least equal to the duration of multi-slot PDCCH monitoring. From this point of view, for 120 kHz and 480kHz SCS values, a UE capability should be introduced for indicating such restriction.

Proposal 1: For NR operation between 52.6 GHz and 71 GHz with 480 kHz, support UE capability to indicate any restriction related to maximum number of PDSCHs/PUSCHs that can be scheduled by single DCI

Regarding the time domain resource allocation for PDSCH/PUSCH with both contiguous and non-contiguous slots, three aspects need to be addressed including:

- maximum allowed gap between two adjacent PDSCHs/PUSCHs

- Maximum allowed gap between the first and last PDSCH/PUSCH

- Signaling for indicating the gap

In our view, the most straightforward method should be to define maximum allowed gap between the first and last PDSCH/PUSCH. Once that is defined, we don’t see the need to additionally define maximum allowed gap between two adjacent PDSCHs/PUSCHs. It should be up to the network implementation that the PDSCHs/PUSCHs are allocated in time domain such that all the scheduled PDSCHs/PUSCHs are contained within the maximum gap between first and last PDSCH/PUSCH.  This will not require any additional signaling.

Proposal 2: For NR operation between 52.6 GHz and 71 GHz, when multiple PDSCHs/PUSCHs can be scheduled by a single DCI, then only the maximum allowed gap between first and last PDSCH/PUSCH is defined, or alternatively, maximum duration to contain all the scheduled PDSCHs/PUSCHs can be defined (in case of non-contiguous allocation)

- Maximum allowed gap between two adjacent PDSCHs/PUSCHs need not be defined as network implementation can handle it under the constraint that all the scheduled PDSCHs/PUSCHs are contained within the maximum allowed gap between first and last PDSCH/PUSCH

In terms of the exact values to be defined for maximum allowed gap between first and last PDSCH/PUSCH, one value for each of the supported SCS can be defined. For example, maximum gap of 16 slots between first and last PDSCH/PUSCH can be defined for 960kHz, where the maximum number of scheduled slots is 8 (as already agreed). Similarly, for 480kHz and 120kHz, one value each (possibly smaller values) can be defined.

Proposal 3: For NR operation between 52.6 GHz and 71 GHz, when multiple PDSCHs/PUSCHs can be scheduled by a single DCI, one value to define the maximum allowed gap between first and last PDSCH/PUSCH for each of the SCS value can be defined

## 2.2 HARQ-ACK timing

For HARQ-ACK transmission corresponding to multiple PDSCHs scheduled by single DCI, following agreement was made in RAN1#104-e [4]:

Agreement:

- For a DCI scheduling multiple PDSCHs, HARQ-ACK information corresponding to PDSCHs scheduled by the DCI is multiplexed with a single PUCCH in a slot that is determined based on K1,

  - where K1 (indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ_feedback timing indicator field is not present in the DCI) indicates the slot offset between the slot of the last PDSCH scheduled by the DCI and the slot carrying the HARQ-ACK information corresponding to the scheduled PDSCHs.

    - It is noted that granularity of K1 can be separately discussed.

- FFS: If needed, further discuss whether or not HARQ-ACK information corresponding to different PDSCHs scheduled by the DCI can be carried by different PUCCH(s)

On the point whether or not HARQ-ACK information corresponding to different PDSCHs scheduled by DCI can be carried by different PUCCH(s), we think that such flexibility should be supported. It could be useful to carry HARQ-ACK separately corresponding to each of PDSCH for the case when non-continuous transmission of multiple PDSCHs is scheduled by single DCI. Therefore, even before the transmission of some of the later PDSCHs, the HARQ-ACK for at least some of the previous PDSCHs can be sent on PUCCH in the middle when UL symbols are available between non-contiguous PDSCHs.

Proposal 4: For NR operation between 52.6 GHz and 71 GHz, for HARQ-ACK information corresponding to PDSCHs scheduled by the DCI, different PUCCH(s) can be used where the PUCCH carrying the HARQ-ACK can be transmitted in the middle of non-contiguous PDSCHs transmissions to allow earlier/faster transmission of HARQ-ACK associated with earlier PDSCHs

## 2.3 Time-domain bundling for HARQ codebook

In RAN1#106bis-e, following two options related to time-domain bundling for HARQ codebook have been captured in [5]:

For type-1 HARQ-ACK codebook:

- Option 1: Time domain bundling is performed across all PDSCHs scheduled by a DCI and pruning procedure is based on the last SLIV

- Option 2: Time domain bundling is performed across subset of PDSCHs scheduled by a DCI, FFS for pruning procedure

For type-2 HARQ-ACK codebook:

- Option 1: Time domain bundling is performed across all PDSCHs scheduled by a DCI and corresponding HARQ-ACK bit belongs to the first sub-codebook.

- Option 2: Time domain bundling is performed across subset of PDSCHs scheduled by a DCI, FFS for how to determine the subset of scheduled PDSCHs

In our view, option 2 is more suitable for both cases as it allows more flexibility to limit the size depending on number of PDSCHs scheduled by a DCI. For larger number of PDSCHs such as 8, it might be more optimal to perform time-bundling across a small set of PDSCHs for better reliability in comparison to applying time bundling across all the 8 PDSCHs together. For example, 2 bundles could be applied, where the first 4 PDSCHs are bundled together in terms of HARQ-ACK codebook and the last 4 PDSCHs can be bundled together.

Proposal 5: For NR operation between 52.6 GHz and 71 GHz, for HARQ-ACK information corresponding to PDSCHs scheduled by the DCI, time domain bundling should be supported for both type-1 and type-2 HARQ-ACK codebook, where the time domain bundling is performed across a subset of PDSCHs scheduled by a DCI.

# 3	PDSCH/PUSCH enhancements for new SCS

## 3.1 CSI processing and CPU availability enhancements

Currently, the CSI processing timeline is also specified where the processing delay is defined in terms of number of symbols. The symbols are depending up on the subcarrier spacing applied for CSI-RS transmission, CSI reporting and PDCCH triggering the CSI report (for aperiodic or semi-persistent reporting). Depending up on the UE capability in terms of number of CSI processing units (CPUs) in each symbol (parallel processing capability), not all the triggered reports are always processed. If all the CPUs are occupied on a given symbol, then the remaining low priority reports are not required by UE to be processed. However, from a UE perspective, a symbol can correspond to different durations depending up on the corresponding numerologies for each CSI report. For lower subcarrier spacing, a symbol might only have CPU occupied partially when the CPU is used for CSI processing associated with very high subcarrier spacing. Therefore, it is not optimal to disregard the processing of CSI reporting in such scenarios. When a UE is expected to process/prepare for transmissions associated with different subcarrier spacings ranging from 15 kHz to possibly 960 kHz in a parallel manner, then enhancements could be considered on how to efficiently utilize UE’s limited processing capability to reduce latency and handle processing/preparation of transmissions associated with multiple numerologies. Figure 1 illustrates an example where CPU availability is checked for multiple CSI reports associated multiple SCS values. For SCS value 15 kHz, CPU availability is checked at beginning of symbol N, and the corresponding CSI report is not processed since at that point there is no availability. UE waits until next symbol (corresponding 15 kHz SCS value) to check the availability again. CPU is occupied to process CSI report associated with SCS value of 480kHz. However, the CPU is released once the processing is done for CSI report associated with 480kHz SCS value and it becomes available for processing other CSI reports. But based on current specification, UE would still wait for the beginning of next symbol corresponding to 15 kHz SCS value to check availability, although the CPU became available somewhere in the middle of the symbol where it initially checked. This is inefficient method.

One simple possibility could be having a reference symbol duration as the lowest value corresponding to highest SCS value for checking CPU availability for any CSI report regardless of the associated SCS value. For example, a reference symbol duration corresponding to 480kHz could be used for checking CPU availability for different CSI reports corresponding to SCS values of 15kHz, 60kHz, 120kHz and 480kHz.

Proposal 6: For supporting NR between 52.6 GHz and 71 GHz with high subcarrier spacing values including 480kHz and 960kHz, following enhancements should be supported to efficiently utilize UE’s limited processing capability to reduce latency and efficiently handle processing/preparation of CSI reports associated with multiple numerologies in parallel:

- Same reference symbols duration (possibly the shortest duration corresponding to maximum supported SCS value) could be used for checking CPU availability corresponding to different CSI reports associated with different SCS values

Figure 1: Example of current CSI Processing Unit availability check procedure

## 3.2 UE-assisted numerology (BWP) adaptation

Higher SCS values including 120kHz, 480kHz and 960kHz are agreed to be supported for NR operation between 52.6 GHz and 71 GHz. However, the selection of SCS value based on the frequency range should not be the only criteria. As can be seen in Figure 2, the channel condition and selection of MCS plays an important role in determining the SCS value. When the channel condition is not so good (lower SNR range), lower SCS value of 120kHz is better as noise is more pronounced than the inter-carrier interference caused by phase noise. When channel condition is moderate (mid SNR range), then slightly higher SCS value such 480kHz is better. For very good channel conditions (high SNR), then 960kHz SCS value is better (phase noise dominates and so reduction in phase noise interference by increased subcarrier spacing has better performance (provided the CP is sufficient)), all considering similar mobile speed. In addition, the performance requirements in terms of target BLER and/or throughput are also critical in determining the suitable value of SCS along with other parameters such as MCS, rank, etc.

Figure 2. PDSCH with 400MHz BW for 60GHz carrier frequency and channel delay spread of 10ns

Observation 1: For supporting NR between 52.6 GHz and 71 GHz with high subcarrier spacing values including 480kHz and 960kHz, the selection of SCS value should not limited based on the frequency range .Other factors of channel conditions such as phase noise, ICI, Doppler, CQI, etc. plays an important role in determining the SCS value:

- For DL channel, UE has all the required estimates related to channel, receiver phase noise and other impairments, etc.

For DL channels, the UE estimates channel conditions such as phase noise, ICI, Doppler, CQI etc., and some other impairments based on the supported receiver algorithm. Hence, UE is in a better position compared to gNB to make corresponding decision on which SCS value is more suitable. One possibility could be that UE reports all these measurements to gNB but reporting a large set of measurement to gNB will have quite large overhead. Therefore, based on these measurements at UE, it is a simpler solution for a UE to recommend a suitable SCS value or a corresponding BWP. Based on this recommendation, gNB could indicate a BWP switching that is in line with the suggested SCS value.

Proposal 7: For supporting NR between 52.6 GHz and 71 GHz with high subcarrier spacing values including 480kHz and 960kHz, UE assistance for SCS/BWP selection should be supported to take in to account all the channel measurements and receiver impairments that are more prominent at higher frequency range

- CSI reporting should be enhanced to report back suitable subcarrier spacing value based on measurements and receiver characteristics at UE

# 4	Conclusion

In this contribution, we discussed enhancements for PDSCH/PUSCH scheduling for NR operation between 52.6 GHz to 71 GHz with high SCS values and provided following observations/proposals:

Observation 1: For supporting NR between 52.6 GHz and 71 GHz with high subcarrier spacing values including 480kHz and 960kHz, the selection of SCS value should not limited based on the frequency range .Other factors of channel conditions such as phase noise, ICI, Doppler, CQI, etc. plays an important role in determining the SCS value:

- For DL channel, UE has all the required estimates related to channel, receiver phase noise and other impairments, etc.

Proposal 1: For NR operation between 52.6 GHz and 71 GHz with 480 kHz, support UE capability to indicate any restriction related to maximum number of PDSCHs/PUSCHs that can be scheduled by single DCI

Proposal 2: For NR operation between 52.6 GHz and 71 GHz, when multiple PDSCHs/PUSCHs can be scheduled by a single DCI, then only the maximum allowed gap between first and last PDSCH/PUSCH is defined, or alternatively, maximum duration to contain all the scheduled PDSCHs/PUSCHs can be defined (in case of non-contiguous allocation)

- Maximum allowed gap between two adjacent PDSCHs/PUSCHs need not be defined as network implementation can handle it under the constraint that all the scheduled PDSCHs/PUSCHs are contained within the maximum allowed gap between first and last PDSCH/PUSCH

Proposal 3: For NR operation between 52.6 GHz and 71 GHz, when multiple PDSCHs/PUSCHs can be scheduled by a single DCI, one value to define the maximum allowed gap between first and last PDSCH/PUSCH for each of the SCS value can be defined

Proposal 4: For NR operation between 52.6 GHz and 71 GHz, for HARQ-ACK information corresponding to PDSCHs scheduled by the DCI, different PUCCH(s) can be used where the PUCCH carrying the HARQ-ACK can be transmitted in the middle of non-contiguous PDSCHs transmissions to allow earlier/faster transmission of HARQ-ACK associated with earlier PDSCHs

Proposal 5: For NR operation between 52.6 GHz and 71 GHz, for HARQ-ACK information corresponding to PDSCHs scheduled by the DCI, time domain bundling should be supported for both type-1 and type-2 HARQ-ACK codebook, where the time domain bundling is performed across a subset of PDSCHs scheduled by a DCI.

Proposal 6: For supporting NR between 52.6 GHz and 71 GHz with high subcarrier spacing values including 480kHz and 960kHz, following enhancements should be supported to efficiently utilize UE’s limited processing capability to reduce latency and efficiently handle processing/preparation of CSI reports associated with multiple numerologies in parallel:

- Same reference symbols duration (possibly the shortest duration corresponding to maximum supported SCS value) could be used for checking CPU availability corresponding to different CSI reports associated with different SCS values

Proposal 7: For supporting NR between 52.6 GHz and 71 GHz with high subcarrier spacing values including 480kHz and 960kHz, UE assistance for SCS/BWP selection should be supported to take in to account all the channel measurements and receiver impairments that are more prominent at higher frequency range

- CSI reporting should be enhanced to report back suitable subcarrier spacing value based on measurements and receiver characteristics at UE

# 5	References

[1]	3GPP RP-212637, “Revised WID:  Extending current NR operation to 71 GHz”, Qualcomm, Intel

[2] 	3GPP RAN1#106bis-e, “Chairman Notes”

[3] 	3GPP RAN1#106-e, “Chairman Notes”

[4] 	3GPP RAN1#104-e, “Chairman Notes”

[5] 	R1-2110640, “Summary#3 of PDSCH/PUSCH enhancements (Scheduling/HARQ)”, Moderator (LG Electronics)