3GPP TSG RAN WG1 #107-e	R1-2111782

e-Meeting, November 11th – 19th, 2021

Agenda item:	8.16.16

Source: 	Samsung

Title: 				     UE features for DL 1024QAM

Document for:	Discussion and Decision

Introduction

This contribution discussed UE features for DL 1024QAM for NR FR1.

Discussion

In RAN1#106bis-e, following was agreed in [1]. Based on this, we provides our views directly (in highlighted yellow) on remaining fields in the table.

Agreement: Agree the following table as baseline for further discussions during RAN1 #107-e, incl. the yellow highlighting and the revisions marked with chromatic formatting

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36. NR_DL1024QAM_FR1 | 36-1 | 1024QAM for PDSCH for FR1 | Support 1024QAM for PDSCH for FR1 including 1024QAM modulation scheme as defined in TS 38.211, MCS and CQI feedback tables based on 1024QAM modulation order as defined in TS 38.214. | 1-4 [pdsch-256QAM-FR1] 1-4 of 4.3(RF and RRM features) is preferred | Yes | N/A | No support of 1024 QAM for PDSCH | [Per Band/per FSPC] Per Band is preferred | N/A | No Applicable only to FR1 | N/A | Note from WI objective: DL PDSCH 1024QAM for FR1 should be defined as a per-band UE capability | Optional with capability signalling |
| 36. NR_DL1024QAM_FR1 | [36-2] For 36-2, we are generally fine to have the feature. However, it should be further discussed together with existing UE feature [supportedModulationOrderDL] by adding 1024QAM condition | [scalingFactor for 1024QAM] prefer to remove bracket | [Indicates the scaling factor to be applied to the band in the max data rate calculation as defined in 4.1.2 when support of 1024-QAM is signalled for the band. ] remove bracket | 36-1 | Yes | N/A |  | Per Band | N/A | Applicable only to FR1 | N/A | Supported values [0.4, 0.75, 0.8, 1.0] prefer to remove bracket | Optional with capability signaling should be confirmed |

Conclusion

This contribution considered the UE features list for Rel-17 DL 1024QAM for NR FR1. Proposals are included in above table.

Reference

[1] R1-2109919, Summary of UE features for DL 1024QAM for NR FR1