3GPP TSG RAN WG1 #107-e			R1- 2112119

e-Meeting, November 11th – 19th, 2021

Source:	NTT DOCOMO, INC.

Title:	Enhancements on PUSCH repetition type A

Agenda Item:	8.8.1.1

Document for: 	Discussion

- Introduction

At the RAN#90-e meeting, a new WID [1] on “NR coverage enhancements” was approved. In this contribution, we discuss on the enhancements on PUSCH repetition type A for coverage enhancements.

- Discussion on Enhancements on PUSCH repetition type A

  - Increasing the maximum number of repetitions

At the RAN1#104-e meeting, enhancements on PUSCH repetition Type A was discussed and following agreements were made [2].

Agreement

The maximum number of repetitions for DG-PUSCH is also applicable to CG-PUSCH.

Agreement

Rel-17 PUSCH repetition Type A supports the increase of maximum number of repetitions with repetition factors configured in a TDRA list with a row index indicated either by the configured grant configuration or by TDRA field in a DCI.

FFS: increasing the maximum number of repetitions with repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig.

Agreement

The maximum number of repetitions for DG-PUSCH is also applicable to CG-PUSCH.

Agreement

Rel-17 PUSCH repetition Type A supports the increase of maximum number of repetitions with repetition factors configured in a TDRA list with a row index indicated either by the configured grant configuration or by TDRA field in a DCI.

FFS: increasing the maximum number of repetitions with repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig.

The maximum number of repetitions is increased with TDRA list, and next discussion point is whether the maximum number of repetitions configured in PUSCH-Config IE and ConfiguredGrantConfig IE is increased or not. The number of repetitions configured in the IEs is available if numberOfRepetitions in PUSCH-TimeDomainResourceAllocation IE is not configured. Therefore, increasing the maximum number of repetitions for the IEs is not necessary. On the other hands, numberOfRepetitions is not available for Type 1 CG-PUSCH based on the current specification. Therefore, increasing the number of repetitions in ConfiguredGrantConfig IE (RepK) is a sufficient way with considering the specification impact. Therefore, we conclude a following proposal.

Proposal 1: Increase the maximum number of repetitions in ConfiguredGrantConfig IE for Type 1 CG-PUSCH.

- Conclusion

In this contribution, we discussed on the enhancements on PUSCH repetition type A for coverage enhancements. Based on the discussion we made following proposal.

Proposal 1: Increase the maximum number of repetitions in ConfiguredGrantConfig IE for Type 1 CG-PUSCH.

References

- 3GPP, RP-202928, “New WID on NR coverage enhancements”, Dec. 2020.

- 3GPP, RAN1#104e, RAN1 Chairman’s Notes, Feb. 2021.