3GPP TSG RAN WG1 #107-e			R1-2112256

November 11th – 19th, 2021

Agenda item:	8.17.11

Source: 	Qualcomm

Title: 	UE Features for Sidelink Enhancements

Document for:	Discussion/Decision

- Introduction

In this contribution, we propose a set of UE features for NR Sidelink Enhancements based on the preliminary feature list provided in [1].

In general, many of the new features are impacted by the currently supported featureset, e.g. the ability to process the additional information required for inter-UE coordination could be impacted by support of operations in other bands that would increase the baseband processing effort. Similarly transmitting or receiving such information would be impacted by other reception and transmission. Another example is sensing and partial sensing, which both require baseband processing taking away processing resources from other operators. Therefore, we propose that the feature be defined per featureset.

Proposal 1: UE features for sidelink enhancements are defined per featureset.

A second general note is that the new features are enhancements that could be implemented separately to a large degree. The UE ability to utilize inter-UE coordination information is not related to its ability to perform partial sensing for example. Therefore, we propose to have all the features as optional without being required for a UE that supports NR sidelink. Furthermore, a Release-16 UE that supports NR sidelink but not any of the Release-17 features is still capable of successfully performing sidelink communications and should not be impacted by the introduction of those new features.

Proposal 2: UE features for sidelink enhancements are not basic features and are not required of a UE that supports NR sidelink.

The rest of this section provides discussion for some of the changes in the feature list included in the next section.

In [1], FG 32-2 combined support for receiving PSFCH and S-SSB. However, the two serve different purposes and have different implementation requirements. Therefore, there is no need to group them. We propose to separate them into two FGs. While the gNB would benefit from knowing whether a UE can receive PSFCH or not, the same does not hold for S-SSB reception.

A new FG corresponding to the random selection capability in the following agreement from RAN1 #106bis-e is introduced:

Agreement

Following Tx capabilities are used as FGs for Rel-17 SL

- mode 2 with random resource selection

- mode 2 with partial sensing

- FFS: TX capabilities with more than one sensing schemes (e.g., {full sensing, partial sensing, random selection}, {partial sensing, random selection})

It was already agreed to support reevaluation and pre-emption checking. Two new FGs are introduced for these features.

A single FG is used for all inter-UE coordination features in [1]. However, the ability to utilize preferred resource information is unrelated to the ability to utilize non-preferred resources. Therefore, the FGs for Scheme 1 with preferred resource indication, for Scheme 1 with non-preferred resource indication, and Scheme 2 are separated. Moreover, a UE that utilizes coordination information does not necessarily need to be able to generate coordination information. Hence, those two aspects are listed in separate FGs for each variant.

- NR_SL_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-2a | Receiving NR sidelink of PSFCH | 1) UE can receive NR PSFCH. | None | [Yes] | [Yes] |  | [Per FS] | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| 32. NR_SL_enh | 32-2b | Receiving NR sidelink of S-SSB | 1) UE can receive NR S-SSB. | None | [No] | [No] |  | [Per FS] | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| 32. NR_SL_enh | 32-4a | Transmitting NR sidelink mode 2 with random selection | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with random selection configured by NR Uu or preconfiguration. |  | [Yes] | [Yes] | The UE does not support transmission using random selection | [Per FS] | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |
| 32. NR_SL_enh | 32-4 | Transmitting NR sidelink mode 2 with partial sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with partial sensing configured by NR Uu or preconfiguration. 2) UE can perform periodic-based partial sensing and resource allocation operation. 3) UE can perform contiguous partial sensing and resource allocation operation. | None | [No] | [Yes] | The UE does not support trasmissoin according to the partial sensing and resource allocation | [Per FS] | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| 32. NR_SL_enh | 32-6 | Reevaluation of selected resources | - UE can perform reevaluation checking for selected but not reserved resources. | None | No | No |  | Per FS | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |
| 32. NR_SL_enh | 32-7 | Preemption checking for reserved resources | - UE can perform preemption checking for selected resources. | None | No | Yes |  | Per FS | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |
| 32. NR_SL_enh | 32-5a | Inter-UE coordination preferred resource transmission | 1) UE can generate and transmit inter-UE coordination information of preferred resource set in NR sidelink mode 2. 2) UE can receive an explicit request for inter-UE coordination information | None | Yes | Yes |  | Per FS | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |
| 32. NR_SL_enh | 32-5b | Inter-UE coordination preferred resource reception | 1) UE can receive inter-UE coordination information of preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit an explicit request for inter-UE coordination information | None | Yes | Yes |  | Per FS | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |
| 32. NR_SL_enh | 32-5c | Inter-UE coordination non-preferred resource transmission | 1) UE can generate and transmit inter-UE coordination information of non-preferred resource set in NR sidelink mode 2. | None | Yes | Yes |  | Per FS | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |
| 32. NR_SL_enh | 32-5d | Inter-UE coordination preferred nonresource reception | 1) UE can receive inter-UE coordination information of non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. | None | Yes | Yes |  | Per FS | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |
| 32. NR_SL_enh | 32-5e | Inter-UE coordination expected conflict indication transmission | 1) UE can detect the presence of expected/potential resource conflict and transmit an indication for inter-UE coordination in NR sidelink mode 2. | None | Yes | Yes |  | Per FS | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |
| 32. NR_SL_enh | 32-5f | Inter-UE coordination expected conflict indication reception | 1) UE can receive inter-UE coordination information indicating the presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. | None | Yes | Yes |  | Per FS | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |

# Conclusion

In addition to the changes in Section 23, we proposed the following:

Proposal 1: UE features for sidelink enhancements are defined per featureset.

Proposal 2: UE features for sidelink enhancements are optional and not required of a UE that supports NR sidelink.

Proposal 3: Any Release-17 UE that is capable of Mode 2 sidelink transmission supports random resource selection.

Proposal 4: Release-17 sidelink related FGs in the LTE UE feature list can be discussed after the NR FGs are finalized.

# References

[1]  R1-2108679, “Preliminary RAN1 UE features list for Rel-17 NR,” Moderators  (AT&T, NTT DOCOMO, INC.), RAN1 #106-e.

[2]  R1-2108678, “Preliminary RAN1 UE features list for Rel-17 LTE,” Moderators  (AT&T, NTT DOCOMO, INC.), RAN1 #106-e.