3GPP TSG-WG1 Meeting #107	R1-2112919

e-meeting, November 11 – 19, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR | 0080 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of MIMO enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Core |  |  |  |  |  | Date: |  |  | 2021-12-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Addition of partial frequency sounding for SRS and multi-TRP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding partial frequency sounding and multi-TRP support. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for partial frequency sounding or multi-TRP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1.4.2, 6.4.1.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | 38.212, 38.213, 38.214 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.4.1.4	Sounding reference signal

##### 6.4.1.4.1	SRS resource

An SRS resource is configured by the SRS-Resource IE or the SRS-PosResource IE and consists of

-	NapSRS∈1,2,4 antenna ports pii=0NapSRS-1, where the number of antenna ports is given by the higher layer parameter nrofSRS-Ports if configured, otherwise NapSRS=1, and pi=1000+i when the SRS resource is in a SRS resource set with higher-layer parameter usage in SRS-ResourceSet not set to 'nonCodebook', or determined according to [6, TS 38.214] when the SRS resource is in a SRS resource set with higher-layer parameter usage in SRS-ResourceSet set to 'nonCodebook'

-	NsymbSRS∈1,2,4,8,12 consecutive OFDM symbols given by the field nrofSymbols contained in the higher layer parameter resourceMapping

-	l0, the starting position in the time domain given by l0=Nsymbslot-1-loffset where the offset loffset∈0,1,…,13 counts symbols backwards from the end of the slot and is given by the field startPosition contained in the higher layer parameter resourceMapping and loffset≥NsymbSRS-1

-	k0, the frequency-domain starting position of the sounding reference signal

##### 6.4.1.4.2	Sequence generation

The sounding reference signal sequence for an SRS resource shall be generated according to

r(pi)n,l'=ru,v(αi,δ)n

0≤n≤Msc,bSRS-1

l'∈0,1,…,NsymbSRS-1

where Msc,bSRS is given by clause 6.4.1.4.3, ru,vα,δ(n) is given by clause 5.2.2 with δ=log2KTC and the transmission comb number KTC∈2,4,8 is contained in the higher-layer parameter transmissionComb. The cyclic shift αi for antenna port pi is given as

αi=2πnSRScs,inSRScs,max
nSRScs,i=nSRScs+nSRScs,maxpi-10002NapSRS2 mod nSRScs,maxif NapSRS=4 and nSRScs,max=6nSRScs+nSRScs,maxpi-1000NapSRS mod nSRScs,maxotherwise

where nSRScs∈0,1,…,nSRScs,max-1 is contained in the higher layer parameter transmissionComb. The maximum number of cyclic shifts nSRScs,max are given by Table 6.4.1.4.2-1.

The sequence group u=fghns,fμ,l'+nIDSRS mod 30 and the sequence number v in clause 5.2.2 depends on the higher-layer parameter groupOrSequenceHopping in the SRS-Resource IE or the SRS-PosResource IE. The SRS sequence identity nIDSRS is given by the higher layer parameter sequenceId in the SRS-Resource IE, in which case nIDSRS∈0, 1, …, 1023, or the SRS-PosResource-r16 IE, in which case nIDSRS∈0, 1, …, 65535. The quantity l'∈0,1,…,NsymbSRS-1 is the OFDM symbol number within the SRS resource.

-	if groupOrSequenceHopping equals 'neither', neither group, nor sequence hopping shall be used and

-	if groupOrSequenceHopping equals 'groupHopping', group hopping but not sequence hopping shall be used and

where the pseudo-random sequence ci is defined by clause 5.2.1 and shall be initialized with cinit=nIDSRS at the beginning of each radio frame.

-	if groupOrSequenceHopping equals 'sequenceHopping', sequence hopping but not group hopping shall be used and

where the pseudo-random sequence  ci is defined by clause 5.2.1 and shall be initialized with cinit=nIDSRS at the beginning of each radio frame.

Table 6.4.1.4.2-1: Maximum number of cyclic shifts nSRScs,max as a function of KTC.

| KTC | nSRScs,max |
| --- | --- |
| 2 | 8 |
| 4 | 12 |
| 8 | 6 |

##### 6.4.1.4.3	Mapping to physical resources

When SRS is transmitted on a given SRS resource, the sequence rpi(n,l') for each OFDM symbol l' and for each of the antenna ports of the SRS resource shall be multiplied with the amplitude scaling factor  in order to conform to the transmit power specified in [5, 38.213] and mapped in sequence starting with  to resource elements  in a slot for each of the antenna ports  according to

The length of the sounding reference signal sequence is given by

Msc,bSRS=mSRS,bNscRBKTCPF KTC

where mSRS,b is given by a selected row of Table 6.4.1.4.3-1 with  where  is given by the field b-SRS contained in the higher-layer parameter freqHopping if configured, otherwise BSRS=0. The row of the table is selected according to the index  given by the field c-SRS contained in the higher-layer parameter freqHopping. The quantity PF is given by the higher-layer parameter FreqScalingFactor if configured, otherwise PF=1.

The frequency-domain starting position k0(pi) is defined by

k0(pi)=k0(pi)+noffsetFH+noffsetRPFSb=0BSRSKTCMsc,bSRSnb

where

k0pi=nshiftNscRB+kTCpi+koffsetl' mod KTC
kTCpi=kTC+KTC2 mod KTCif NapSRS=4, pi∈1001, 1003, and nSRScs,max=6 kTC+KTC2 mod KTC if NapSRS=4, pi∈1001, 1003, and nSRScs∈nSRScs,max2, …, nSRScs,max-1kTCotherwise
noffsetFH=b=0BSRSKTCMsc,bSRSnb
kTC(pi)=kTC+KTC2 mod KTC if nSRScs∈nSRScs,max2, …, nSRScs,max-1 and NapSRS=4 and pi∈1001, 1003kTCotherwise
noffsetRPFS=NscRBmSRS,BSRSkF+khopmod PFPF

and

-	kF∈0,1,…,PF-1 is given by the higher-layer parameter StartRBIndex if configured, otherwise kF=0;

-	khop is given by Table 6.4.1.4.3-3 with

khop=nSRSb'=bhopBSRSNb' mod PF
Nbhop=1

if the higher-layer parameter EnableStartRBHopping is configured, otherwise khop=0.

Editor’s note: the applicability of RPFS to the non-FH case is to be discussed in RAN1

If NBWPstart≤nshift the reference point for k0(pi)=0 is subcarrier 0 in common resource block 0, otherwise the reference point is the lowest subcarrier of the BWP.

If the SRS is configured by the IE SRS-PosResource, the quantity koffsetl' is given by Table 6.4.1.4.3-2, otherwise koffsetl'=0.

The frequency domain shift value nshift adjusts the SRS allocation with respect to the reference point grid and is contained in the higher-layer parameter freqDomainShift in the SRS-Resource IE or the SRS-PosResource IE. The transmission comb offset kTC∈0,1,…,KTC-1 is contained in the higher-layer parameter transmissionComb in the SRS-Resource IE or the SRS-PosResource IE and nb is a frequency position index.

Frequency hopping of the sounding reference signal is configured by the parameter bhop∈0,1,2,3, given by the field b-hop contained in the higher-layer parameter freqHopping if configured, otherwise bhop=0.

If bhop≥BSRS, frequency hopping is disabled and the frequency position index  remains constant (unless re-configured) and is defined by

for all NsymbSRS OFDM symbols of the SRS resource. The quantity  is given by the higher-layer parameter freqDomainPosition if configured, otherwise nRRC=0, and the values of mSRS,b and Nb for b=BSRS are given by the selected row of Table 6.4.1.4.3-1 corresponding to the configured value of .

If bhop<BSRS, frequency hopping is enabled and the frequency position indices nb are defined by

nb=4nRRCmSRS,b mod Nbb≤bhopFbnSRS+4nRRCmSRS,b mod Nbotherwise

where Nb is given by Table 6.4.1.4.3-1,

and where Nbhop=1 regardless of the value of Nb. The quantity nSRS counts the number of SRS transmissions. For the case of an SRS resource configured as aperiodic by the higher-layer parameter resourceType, it is given by nSRS=l'R within the slot in which the NsymbSRS symbol SRS resource is transmitted. The quantity R≤NsymbSRS is the repetition factor given by the field repetitionFactor if configured, otherwise R=NsymbSRS.

For the case of an SRS resource configured as periodic or semi-persistent by the higher-layer parameter resourceType, the SRS counter is given by

for slots that satisfy . The periodicity  in slots and slot offset  are given in clause 6.4.1.4.4.

Table 6.4.1.4.3-1: SRS bandwidth configuration.

| 0 | 4 | 1 | 4 | 1 | 4 | 1 | 4 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 | 1 | 4 | 2 | 4 | 1 | 4 | 1 |
| 2 | 12 | 1 | 4 | 3 | 4 | 1 | 4 | 1 |
| 3 | 16 | 1 | 4 | 4 | 4 | 1 | 4 | 1 |
| 4 | 16 | 1 | 8 | 2 | 4 | 2 | 4 | 1 |
| 5 | 20 | 1 | 4 | 5 | 4 | 1 | 4 | 1 |
| 6 | 24 | 1 | 4 | 6 | 4 | 1 | 4 | 1 |
| 7 | 24 | 1 | 12 | 2 | 4 | 3 | 4 | 1 |
| 8 | 28 | 1 | 4 | 7 | 4 | 1 | 4 | 1 |
| 9 | 32 | 1 | 16 | 2 | 8 | 2 | 4 | 2 |
| 10 | 36 | 1 | 12 | 3 | 4 | 3 | 4 | 1 |
| 11 | 40 | 1 | 20 | 2 | 4 | 5 | 4 | 1 |
| 12 | 48 | 1 | 16 | 3 | 8 | 2 | 4 | 2 |
| 13 | 48 | 1 | 24 | 2 | 12 | 2 | 4 | 3 |
| 14 | 52 | 1 | 4 | 13 | 4 | 1 | 4 | 1 |
| 15 | 56 | 1 | 28 | 2 | 4 | 7 | 4 | 1 |
| 16 | 60 | 1 | 20 | 3 | 4 | 5 | 4 | 1 |
| 17 | 64 | 1 | 32 | 2 | 16 | 2 | 4 | 4 |
| 18 | 72 | 1 | 24 | 3 | 12 | 2 | 4 | 3 |
| 19 | 72 | 1 | 36 | 2 | 12 | 3 | 4 | 3 |
| 20 | 76 | 1 | 4 | 19 | 4 | 1 | 4 | 1 |
| 21 | 80 | 1 | 40 | 2 | 20 | 2 | 4 | 5 |
| 22 | 88 | 1 | 44 | 2 | 4 | 11 | 4 | 1 |
| 23 | 96 | 1 | 32 | 3 | 16 | 2 | 4 | 4 |
| 24 | 96 | 1 | 48 | 2 | 24 | 2 | 4 | 6 |
| 25 | 104 | 1 | 52 | 2 | 4 | 13 | 4 | 1 |
| 26 | 112 | 1 | 56 | 2 | 28 | 2 | 4 | 7 |
| 27 | 120 | 1 | 60 | 2 | 20 | 3 | 4 | 5 |
| 28 | 120 | 1 | 40 | 3 | 8 | 5 | 4 | 2 |
| 29 | 120 | 1 | 24 | 5 | 12 | 2 | 4 | 3 |
| 30 | 128 | 1 | 64 | 2 | 32 | 2 | 4 | 8 |
| 31 | 128 | 1 | 64 | 2 | 16 | 4 | 4 | 4 |
| 32 | 128 | 1 | 16 | 8 | 8 | 2 | 4 | 2 |
| 33 | 132 | 1 | 44 | 3 | 4 | 11 | 4 | 1 |
| 34 | 136 | 1 | 68 | 2 | 4 | 17 | 4 | 1 |
| 35 | 144 | 1 | 72 | 2 | 36 | 2 | 4 | 9 |
| 36 | 144 | 1 | 48 | 3 | 24 | 2 | 12 | 2 |
| 37 | 144 | 1 | 48 | 3 | 16 | 3 | 4 | 4 |
| 38 | 144 | 1 | 16 | 9 | 8 | 2 | 4 | 2 |
| 39 | 152 | 1 | 76 | 2 | 4 | 19 | 4 | 1 |
| 40 | 160 | 1 | 80 | 2 | 40 | 2 | 4 | 10 |
| 41 | 160 | 1 | 80 | 2 | 20 | 4 | 4 | 5 |
| 42 | 160 | 1 | 32 | 5 | 16 | 2 | 4 | 4 |
| 43 | 168 | 1 | 84 | 2 | 28 | 3 | 4 | 7 |
| 44 | 176 | 1 | 88 | 2 | 44 | 2 | 4 | 11 |
| 45 | 184 | 1 | 92 | 2 | 4 | 23 | 4 | 1 |
| 46 | 192 | 1 | 96 | 2 | 48 | 2 | 4 | 12 |
| 47 | 192 | 1 | 96 | 2 | 24 | 4 | 4 | 6 |
| 48 | 192 | 1 | 64 | 3 | 16 | 4 | 4 | 4 |
| 49 | 192 | 1 | 24 | 8 | 8 | 3 | 4 | 2 |
| 50 | 208 | 1 | 104 | 2 | 52 | 2 | 4 | 13 |
| 51 | 216 | 1 | 108 | 2 | 36 | 3 | 4 | 9 |
| 52 | 224 | 1 | 112 | 2 | 56 | 2 | 4 | 14 |
| 53 | 240 | 1 | 120 | 2 | 60 | 2 | 4 | 15 |
| 54 | 240 | 1 | 80 | 3 | 20 | 4 | 4 | 5 |
| 55 | 240 | 1 | 48 | 5 | 16 | 3 | 8 | 2 |
| 56 | 240 | 1 | 24 | 10 | 12 | 2 | 4 | 3 |
| 57 | 256 | 1 | 128 | 2 | 64 | 2 | 4 | 16 |
| 58 | 256 | 1 | 128 | 2 | 32 | 4 | 4 | 8 |
| 59 | 256 | 1 | 16 | 16 | 8 | 2 | 4 | 2 |
| 60 | 264 | 1 | 132 | 2 | 44 | 3 | 4 | 11 |
| 61 | 272 | 1 | 136 | 2 | 68 | 2 | 4 | 17 |
| 62 | 272 | 1 | 68 | 4 | 4 | 17 | 4 | 1 |
| 63 | 272 | 1 | 16 | 17 | 8 | 2 | 4 | 2 |

Table 6.4.1.4.3-2: The offset koffsetl' for SRS as a function of KTC and l'.

| KTC | koffset0, …, koffsetNsymbSRS-1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | NsymbSRS=1 | NsymbSRS=2 | NsymbSRS=4 | NsymbSRS=8 | NsymbSRS=12 |
| 2 | 0 | 0,1 | 0,1,0,1 | - | - |
| 4 | - | 0, 2 | 0, 2, 1, 3 | 0, 2, 1, 3, 0, 2, 1, 3 | 0, 2, 1, 3, 0, 2, 1, 3, 0, 2, 1, 3 |
| 8 | - | - | 0, 4, 2, 6 | 0, 4, 2, 6, 1, 5, 3, 7 | 0, 4, 2, 6, 1, 5, 3, 7, 0, 4, 2, 6 |

Table 6.4.1.4.3-3: The quantity khop as a function of khop.

| khop | khop |  |  |
| --- | --- | --- | --- |
|  | PF=1 | PF=2 | PF=4 |
| 0 | 0 | 0 | 0 |
| 1 | - | 1 | 2 |
| 2 | - | - | 1 |
| 3 | - | - | 3 |

#### 7.3.1.6	Mapping from virtual to physical resource blocks

The UE shall assume the virtual resource blocks are mapped to physical resource blocks according to the indicated mapping scheme, non-interleaved or interleaved mapping. If no mapping scheme is indicated, the UE shall assume non-interleaved mapping.

For non-interleaved VRB-to-PRB mapping, virtual resource block  is mapped to physical resource block , except for PDSCH transmissions scheduled with DCI format 1_0 in a common search space in which case virtual resource block n is mapped to physical resource block n+NstartCORESET where NstartCORESET is the lowest-numbered physical resource block in the control resource set where the corresponding DCI was received. When two PDCCH candidates from two linked search space sets as indicated by the higher-layer parameter searchSpaceLinking are detected, and the two linked search space sets are associated with different control resource sets, the control resource set with the lowest number among the two linked control resource sets is used to determine NstartCORESET.

For interleaved VRB-to-PRB mapping, the mapping process is defined by:

-	Resource block bundles are defined as

-	for PDSCH transmissions scheduled with DCI format 1_0 with the CRC scrambled by SI-RNTI in Type0-PDCCH common search space in CORESET 0, the set of NBWP,initsize resource blocks in CORESET 0 are divided into Nbundle=NBWP,initsizeL resource-block bundles in increasing order of the resource-block number and bundle number where L=2 is the bundle size and NBWP,initsize is the size of CORESET 0.

-	resource block bundle Nbundle-1 consists of NBWP,initsize mod L resource blocks if NBWP,initsize mod L>0 and L resource blocks otherwise,

-	all other resource block bundles consists of L resource blocks.

-	for PDSCH transmissions scheduled with DCI format 1_0 in any common search space in bandwidth part i with starting position NBWP,istart, other than Type0-PDCCH common search space in CORESET 0, the set of NBWP,initsize virtual resource blocks 0,1,…,NBWP,initsize-1, where NBWP,initsize is the size of CORESET 0 if CORESET 0 is configured for the cell and the size of initial downlink bandwidth part if CORESET 0 is not configured for the cell, are divided into Nbundle virtual resource-block bundles in increasing order of the virtual resource-block number and virtual bundle number and the set of NBWP,initsize physical resource blocks NstartCORESET, NstartCORESET+1,…,NstartCORESET+NBWP,initsize-1 are divided into Nbundle physical resource-block bundles in increasing order of the physical resource-block number and physical bundle number, where Nbundle=NBWP,initsize+NBWP,istart+NstartCORESET mod LL, L=2 is the bundle size, and NstartCORESET is the lowest-numbered physical resource block in the control resource set where the corresponding DCI was received. When two PDCCH candidates from two linked search space sets as indicated by the higher-layer parameter searchSpaceLinking are detected, and the two linked search space sets are associated with different control resource sets, the control resource set with the lowest number among the two linked control resource sets is used to determine NstartCORESET.

-	resource block bundle 0 consists of L-NBWP,istart+NstartCORESET mod L resource blocks,

-	resource block bundle Nbundle-1 consists of NBWP,initsize+NBWP,istart+NstartCORESET mod L resource blocks if NBWP,initsize+NBWP,istart+NstartCORESET mod L>0 and L resource blocks otherwise,

-	all other resource block bundles consists of L resource blocks.

-	for all other PDSCH transmissions, the set of NBWP,isize resource blocks in bandwidth part  with starting position NBWP,istart are divided into Nbundle=NBWP,isize+NBWP,istart mod LiLi resource-block bundles in increasing order of the resource-block number and bundle number where  is the bundle size for bandwidth part  provided by the higher-layer parameter vrb-ToPRB-Interleaver and

-	resource block bundle 0 consists of  resource blocks,

-	resource block bundle  consists of  resource blocks if  and  resource blocks otherwise,

-	all other resource block bundles consists of  resource blocks.

-	Virtual resource blocks in the interval  are mapped to physical resource blocks according to

-	virtual resource block bundle  is mapped to physical resource block bundle

-	virtual resource block bundle  is mapped to physical resource block bundle  where

-	The UE is not expected to be configured with Li=2 simultaneously with a PRG size of 4 as defined in [6, TS 38.214]

The UE may assume that the same precoding in the frequency domain is used within a PRB bundle and the bundle size is determined by clause 5.1.2.3 in [6, TS 38.214]. The UE shall not make any assumption that the same precoding is used for different bundles of common resource blocks.