3GPP TSG RAN WG1 #107bis-e		R1-2200099

e-Meeting, January 17th – 25th, 2022

Source:	vivo

Title:	Discussion on UE features for NR operation from 52.6-71GHz

Agenda Item:	8.15.2

Document for:	Discussion and Decision

- Introduction

After RAN1 #107-e meeting, the Rel-17 UE features are provided in [1]. In this contribution, our views on NR 52.6GHz part are provided.

- Discussion

With/without shared spectrum access:

After RAN1#107-e, it is not decided yet applicable spectrum type of the following feature groups, i.e. with/without shared spectrum access. The motivation of wideband PRACH and multi-RB PUCCH is mainly from PSD limitation on unlicensed band. Therefore, there is no need to extend them to licensed band.

| Index | Feature group | Components | Prerequisite feature groups | Type |
| --- | --- | --- | --- | --- |
| 24-1b | Wideband PRACH [with/without shared spectrum channel access] | Enhanced PRACH design for operation by adopting a single long ZC sequence, with ZC sequence equal to 1151 for 120kHz and ZC sequence equal to 571 for 120kHz |  |  |
| 24-1c | Multi-RB support PUCCH format 0/1/4 for 120 kHz [with/without shared spectrum channel access] | 1. Support multi-RB PUCCH format 4 for 120 kHz 2. Support multi-RB PUCCH format 0/1 for 120 kHz |  |  |
| 24-4b | Wideband PRACH for 480 kHz [with/without shared spectrum channel access] | PRACH with 480 kHz and length 571 |  |  |
| 24-5c | Multi-RB PUCCH format 0/1/4 for 960 kHz [with/without shared spectrum channel access] | Support multi-RB PUCCH format 0/1/4 for 960 kHz |  |  |

Proposal 1: FG 24-1b, 24-1c, 24-4b and 24-5c are only applicable to the scenarios without shared spectrum access.

Multi-PDSCH scheduling:

For multi-PDSCH scheduling and corresponding HARQ enhancements, it is not decided yet whether it is a separate FG or a component of 480/960 kHz support. In our view, there is no critical requirement for multi-PDSCH scheduling even when multi-slot PDCCH monitoring is used for 480/960 kHz. The only drawback is data rate loss brought by multi-slot PDCCH monitoring but the system still works. Therefore, separate optional multi-PDSCH scheduling capability is more flexible. In this way, to enhance data rate further, UE has flexibility to increase PDCCH monitoring complexity with smaller X value or support multi-PDSCH scheduling.

| Index | Feature group | Components | Prerequisite feature groups | Type |
| --- | --- | --- | --- | --- |
| 24-4 | 480KHz SCS support for DL | 1. 480KH SCS for DL data and control channels, SSB, and reference signal reception in FR2-2 for non-initial access 2. Multiple-slot PDCCH monitoring for 480KHz with X=4 slots FFS: 3. Multi-PDSCH scheduling by single DCI for the operation with 480 kHz SCS and corresponding HARQ enhancements |  |  |
| 24-5 | 960KHz SCS support for DL | 1. 960KHz SCS for DL data and control channels, SSB, and reference signal reception in FR2-2 for non-initial access 2. Multiple-slot PDCCH monitoring for 960KHz with X=8 slots FFS: 3. Multi-PDSCH scheduling by single DCI for the operation with 960 kHz SCS and corresponding HARQ enhancements |  |  |

Proposal 2: List multi-PDSCH scheduling by single DCI as a separate FG from 24-4 and 24-5.

- Conclusion

In this contribution, we focus on UE feature discussion for operation from 52.6-71GHz, and have the following proposals:

Proposal 1: FG 24-1b, 24-1c, 24-4b and 24-5c are only applicable to the scenarios without shared spectrum access.

Proposal 2: List multi-PDSCH scheduling by single DCI as a separate FG from 24-4 and 24-5.

- References

- R1-2112902, Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e, Moderators (AT&T, NTT DOCOMO, INC.)