3GPP TSG RAN WG1 Meeting #107bis-e	R1-2200647

e-Meeting, January 17th – 25th, 2022

Agenda Item:	5

Source:	Huawei, HiSilicon, CBN

Title:	Discussion on MBS broadcast reception on SCell and non-serving cell

Document for:	Discussion and Decision

Introduction

The RAN2 LS sent to RAN1 in [1] was received in the last RAN1 meeting and only first round comments were collected then with no further progress due to limited time.

Discussion

The LS indicates that RAN2 discussed MBS broadcast reception on SCell and non-serving cell respectively by the UE in RRC Connected state and asks RAN1 to check the feasibility of MBS broadcast reception on SCell and non-serving cell taking the following agreements from RAN2 into account:

From RAN2 point of view, the UE may receive MBS broadcast service from SCell in intra-PLMN case and if supported this may be a separate UE capability. Send an LS to RAN1 to ask to check the feasibility of MBS broadcast reception on SCell. From RAN2 point of view, the connected UE may if supported receive MBS broadcast service from non-serving cell in intra-PLMN case, under the condition this does not have any impact to operation on serving cell(s). This may be a separate UE capability. Send an LS to RAN1 to ask to check the feasibility.

Different from some comments from other companies in the last meeting [2], we assume receiving MBS broadcast on non-serving cell does not require UE to obtain the related configuration from the serving cell. For receiving multicast on SCell, UE also needs to report the capability of receiving multicast on SCell in addition to reporting that it has capability of CA.

In our view, UE receiving MBS broadcast on SCell and non-serving cell are both feasible as long as UE has such capabilities. RRC_IDLE/INACTIVE state can directly receive it on non-serving cell and UE in RRC_CONNECTED state can report such a capability to network. Note that, in LTE for SC-PTM, receiving broadcast on SCell and non-serving cell as separate capabilities are defined and can be reported to network as follows:

4.3.22.3 scptm-SCell-r13 This parameter defines whether UEs supporting SC-PTM support in RRC_CONNECTED, MBMS reception via SC-PTM on a frequency indicated in an MBMSInterestIndication message, when an SCell is configured on that frequency (regardless of whether the SCell is activated or deactivated), as specified in TS 36.331 [5]. 4.3.22.4 scptm-NonServingCell-r13 This parameter defines whether UEs supporting SC-PTM support in RRC_CONNECTED, MBMS reception via SC-PTM on a frequency indicated in an MBMSInterestIndication message, where (according to supportedBandCombination and to network synchronization properties) a serving cell may be additionally configured, as specified in TS 36.331 [5]. If this is supported, the UE shall also support MBMS reception via SC-PTM on a frequency when an SCell is configured on that frequency (regardless of whether the SCell is activated or deactivated), as specified in TS 36.331 [5].

NR MBS broadcast reception on SCell and non-serving cell can take the similar approach.

Proposal 1: Reply LS to RAN2 to confirm that MBS broadcast on SCell and non-serving cell are both feasible.

Proposal 2: Support separate UE capabilities for UEs supporting MBS broadcast on SCell and for UEs supporting MBS broadcast on non-serving cell, respectively. The UE capabilities can be defined by RAN2.

Conclusions

This contribution provides our views of UE features for Rel-17 NR MBS with the proposals summarized as follows:

Proposal 1: Reply LS to RAN2 to confirm that MBS broadcast on SCell and non-serving cell are both feasible.

Proposal 2: Support separate UE capabilities for UEs supporting MBS broadcast on SCell and for UEs supporting MBS broadcast on non-serving cell, respectively. The UE capabilities can be defined by RAN2.

- References

R1-2112637, LS on MBS broadcast reception on SCell and non-serving cell, RAN1#107-e, Nov. 11th -19th, 2021.

R1-2112830, Summary# on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS, Moderator (CMCC), RAN1#107-e, Nov. 11th -19th, 2021.