3GPP TSG-RAN WG1 #107bis-e 																	R1-2200685

e-Meeting, January 17th – 25th, 2021

Agenda item:			8.8.1.1

Source:	Moderator (Sharp)

Title:	FL Summary #1 on Enhancements on PUSCH repetition type A

Document for:	Discussion and Decision

Introduction

For PUSCH enahancements the following objectives are described in the Coverage Enhancement WID.

- Specification of PUSCH enhancements [RAN1, RAN4]

  - Specify the following mechanisms for enhancements on PUSCH repetition type A [RAN1]

    - Increasing the maximum number of repetitions up to a number to be determined during the course of the work.

    - The number of repetitions counted on the basis of available UL slots.

This document is intended to facilitate view exchange and discussions on the enhancements on PUSCH repetition type A, for the following assigned email discussion.

[107bis-e-R17-CovEnh-01] Email discussion regarding enhancements for PUSCH repetition type A – Toshi (Sharp)

- 1st check point: January 20

- Final check point: January 25

Discussion

RRC parameter related issues

For this meeting, the following remaining issues which have impacts on RRC parameters have been raised.

- Issue#1-1: Whether to support pusch-AggregationFactor-r17

- Issue#1-2: AvailableSlotCounting for CG-PUSCH

[Open] Issue#1-1: Whether to support pusch-AggregationFactor-r17

We have discussed whether to introduce pusch-AggregationFactor-r17 to support up-to-32 repetitions, for quite some times. However, no consensus to support it was made until now.

For RAN1#107bis-e, OPPO [7] and Intel [8] are proposing the introduction of pusch-AggregationFactor-r17 which support up-to-32 repetitions. Meanwhile, Apple [9] is proposing no use of pusch-AggregationFactor to configure the 32 repetitions in Rel-17

Since this issue has RRC impact, we have to make a decision shortly. Moreover, all the arguments have been presented during the past discussions already. Therefore, FL would like to suggest quickly checking companies’ opinions, and if no consensus is made, this issue will be considered closed and no further discussion will be held.

1st round (Issue#1-1)

Companies are asked to express their position on whether pusch-AggregationFactor-r17 supporting up-to-32 repetitions is introduced in Rel-17 or not.

|  | Company name |
| --- | --- |
| Introduce pusch-AggregationFactor-r17 | Nokia, NSB, Panasonic, Intel, Samsung, China Telecom, OPPO, Lenovo, Motorola Mobility |
| Do not introduce pusch-AggregationFactor-r17 | CATT, Panasonic, QC, vivo, ZTE, Sharp, Apple, DOCOMO,Xiaomi, Ericsson,TCL, LG |

1st round summary (Issue#1-1)

9 companies prefer introducing pusch-AggregationFactor-r17 supporting up-to-32 repetitions, while 12 companies do not think it as necessary. Therefore, FL would like to propose the following conclusion.

Proposed conclusion:

No consensus to introduce pusch-AggregationFactor-r17.

[Pending] Issue#1-2: AvailableSlotCounting for CG-PUSCH

There is an ongoing discussion in UE features on whether a single FG or separate FGs are defined for DG-PUSCH and CG-PUSCH with the available slot counting. Samsung [5] mentions that, if separate FGs are defined, the parameter AvailableSlotCounting needs to be additionally included in ConfiguredGrantConf, as described in the consolidated higher layers parameter list for Rel-17.

| WI code | Sub-feature group | RAN1 specification | Section | RAN2 Parant IE | RAN2 ASN.1 name | Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_cov_enh-Core | Enhancement on PUSCH repetition Type A |  |  |  |  | AvailableSlotCounting | new |  | Enabling PUSCH repetitions counted on the basis of available slots | ENUMERATED {enabled, disable } |  | in PUSCH-Config Note: if separate FGs are defined for DG-PUSCH and CG-PUSCH, this field for CG-PUSCH may need to be added in ConfiguredGrantConf, too. | UE-specific | 38.331 | Agreement: • Each available slot identified by the UE is considered as a transmission occasion for PUSCH repetition. o RV is cycled across transmission occasions, irrespective of whether PUSCH transmission in the transmission occasion is further omitted or not. Note: if separate FGs are defined for DG-PUSCH and CG-PUSCH, this field may be necessary for each of them. |

This issue should be addressed after the discussion on single FG or separate FGs for DG-PUSCH and CG-PUSCH with the available slot counting concludes in UE feature AI.

[Open] Issue#1-3: Other RRC parameter related issues

1st round (Issue#1-3)

If there is any other issue which is not captured in this document yet but needs be discussed for the completion of the RRC parameter list in this meeting, please comment below

| Company | Comments |
| --- | --- |

Remaining issues

For this meeting, the following remaining issues have been raised.

- Issue#2-1: Consideration of Rel-17 PUSCH dropping rules

- Issue#2-2: Use of SSBs across multiple TRPs for the available slot determination

- Issue#2-3: Use of SSBs of other serving cells in half duplex CA operation for the available slot determination

- Issue#2-4: Available slot counting for DG-PUSCH scheduled by DCI format 0_0 with other RNTI than TC-RNTI

- Issue#2-5: Available slot counting for PUSCH repetition Type A with K=1

[Open] Issue#2-1: Consideration of Rel-17 PUSCH dropping rules

In RAN1#106-e, the following 2-step procedure for PUSCH repetition Type A with available slot counting was agreed. This agreement includes one FFS point about applicability of Rel-17 PUSCH dropping rules introduced in other WI(s).

Agreement Take Option 1-B as an agreement for the procedure of Rel-17 PUSCH repetitions counted on the basis of available slots. - Alt 1-B consisting of two steps - Step 1: Determine available slots for K repetitions based on RRC configuration(s) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI - Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions. - FFS: Rel-17 PUSCH dropping rules are also applied if introduced in other WI(s)

In Rel-17 eIIoT/URLLC WI, following PUSCH cancellation rules have been made in RAN1#106bis-e and RAN1#107-e meeting.

RAN1#106bis-e: Agreement For collision between HP CG PUSCH and LP DG PUSCH, if MAC delivers two MAC PDUs to PHY, PHY layer can make the prioritization so that the UE is expected to transmit the CG PUSCH and cancel the DG PUSCH at latest from the first symbol that is overlapping with the CG PUSCH. - Note: For the DG PUSCH, it is up to UE implementation to handle OFDM symbols of the DG PUSCH before the start of HP CG PUSCH which are nonoverlapping with the HP CG PUSCH. - FFS: How to handle the collision when there is repetition for CG and/or DG PUSCH RAN1#107-e Agreement For collision of LP DG-PUSCH and HP CG-PUSCH of different priorities, the cancellation is applied per actual repetition, if LP DG-PUSCH and/or HP CG-PUSCH is repeated. Agreement For the overlapping between LP CG and HP DG, if MAC delivers two MAC PDUs to PHY, PHY layer can make the prioritization so that the UE is expected to cancel the overlapping low priority CG PUSCH by the first overlapping symbol at the latest. - On top of Rel-16 cancellation time (N2+d1) for PUCCH/PUCCH or PUCCH/PUSCH collision, additional time d3 is needed (which results N2+d1+d3 in total cancellation time) for LP CG-PUSCH and HP DG-PUSCH collision resolution. - (Working assumption) d3 = {0, 1,…,2μ+1}symbol(s) upon UE capability report, where μ=0,1,2,3 for SCS=15/30/60/120kHz, respectively. Agreement For collision of HP DG-PUSCH and LP CG-PUSCH, the cancellation is applied per actual repetition, if HP DG-PUSCH and/or LP CG-PUSCH is repeated.

vivo [2] is proposing that the cancellation of LP PUSCH introduced in Rel-17 eIIoT/URLLC WI should be applied in Step 2 in the same way as Rel-15/16 PUSCH dropping. More specifically, LP DG-PUSCH is cancelled when colliding with HP CG-PUSCH, and LP CG-PUSCH is cancelled when colliding with HP DG-PUSCH. In both cases, the cancellation is applied per actual repetition. Note that this cancellation is already captured in Clause 9 of TS38.213v17.0.0, and the PUSCH dropping procedure in Clause 6.1.2.1 and Clause are saying that a PUSCH transmission in a slot of a multi-slot PUSCH transmission is omitted according to the conditions in Clause 9, Clause 11.1 and Clause 11.2A of [6, TS 38.213]. Therefore, no specification change would be necessary for supporting of this behaviour.

1st round (Issue#2-1)

Do you agree on the following proposal?

FL proposal to Issue#2-1:

- The cancellation of LP PUSCH (introduced in Rel-17 eIIoT/URLLC WI) is applied in Step 2 of the previously agreed 2-step procedure of Rel-17 PUSCH repetitions counted on the basis of available slots (i.e., Option 1-B).

  - No specification impact is expected.

| Company | Comments |
| --- | --- |
| Nokia/NSB | As explained by the FL, the current specifications have already captured the above proposal. Therefore, we don’t see the need to make an agreement on this issue. |
| CATT | Agree. This may be a conclusion if it is already captured. |
| Panasonic | Agree. |
| QC | Agree |
| Intel | We are fine with the proposal. |
| vivo1 | Agree. |
| ZTE | Fine with the proposal. |
| Samsung | It seems not necessary to have this agreement now. |
| Sharp | Agree. |
| Apple | Fine with the proposal. |
| China Telecom | Agree. Also agree with Nokia the agreement is not necessary. |
| OPPO | Agree. |
| Huawei/HiSilicon | OK. |
| Xiaomi | Agree |
| Ericsson | Support the proposal, since it is aligned with the FFS: Rel-17 PUSCH dropping rules are also applied if introduced in other WI(s). |
| Lenovo, Motorola Mobility | Agree |
| TCL | Agree. |
| LG | We support the proposal. |

1st round summary (Issue#2-1)

No companies objected the proposal. Several companies do not see the need to take it as an agreement. FL would like to suggest taking the following conclusion.

Proposed conclusion:

- The cancellation of LP PUSCH (introduced in Rel-17 eIIoT/URLLC WI) is applied in Step 2 of the previously agreed 2-step procedure of Rel-17 PUSCH repetitions counted on the basis of available slots (i.e., Option 1-B).

  - No specification impact is expected.

[Open] Issue#2-2: Use of SSBs across multiple TRPs for the available slot determination

In NR Rel-17 feMIMO WI, it was agreed to support different SSB sets with different PCIs transmitted from mTRPs in a same serving cell. In RAN1#107-e, we discussed whether to consider more than one SSB sets for available slot counting for ConEnc, and no consensus to support more than one SSB sets for available slot counting was made.

vivo [2] is raising three possible options to handle the SSBs with PCIs different from the serving cell PCI, and they prefers Options 2 and 3. Also, some other companies are providing their views on this issues as shown below. Ericsson [13] is raising the point that, if a UE considers the SSB configurations of multiple TRPs in the first step of determining of available slots, the TRP that schedules and receives PUSCH needs to know the SSB configuration of other TRP so as to decode a PUSCH repetition correctly. For this issue, Interdigital [11] is suggesting that whether an inter-cell multi-TRP scenario can be considered as a practical use case for coverage enhancement should be discussed before considering SSBs from multiple TRPs to determine available slots, and that would also have some impact on other AIs, such as joint channel estimation.

- Select only one option from:

  - Option 1: SSBs with PCI different from the serving cell PCI are only used for actual PUSCH repetition determination after the available slots are determined

    - ZTE [3],

  - Option 2: Both SSBs with serving cell PCI and SSBs with PCI different from the serving cell PCI are used for the available slot determination

    - vivo [2], Samsung [5], Qualcomm [6], CMCC [12]

  - Option 3: SSBs with the same PCI as the SSB used to determine the spatial relation of the PUSCH, are used to determine the available slots. Other set of SSBs are used for actual PUSCH repetition determination after the available slots are determined.

    - vivo [2], Nokia/Nokia Shanghai Bell [4] , CMCC [12], Ericsson [13]

- Discuss first whether an inter-cell multi-TRP scenario can be considered as a practical use case for coverage enhancement.

    - Interdigital [11]

FL’s understanding is that many companies think multi-TRP scenario can be considered as a practical use case for coverage enhancement. At the same time, multiple SSB configurations of multiple TRPs affect not only Rel-17 PUSCH repetitions with available slot counting but also the legacy PUCCH repetition procedure, because the slots for the legacy PUCCH repetitions are also determined by referring to SSB positions. When we designed the available slot counting for PUSCH repetition Type A, many companies had the alignment with the legacy PUCCH repetitions in mind, and consequently the procedures of PUSCH repetition Type A with the avaialble slot counting is well-aligned to the legacy PUCCH repetition procedure. Therefore, from the FL perspective, it is better to also check with MIMO experts how PUCCH repetitions work with the SSB configurations of multiple TRPs.

1st round (Issue#2-2)

Q1: Any views on how the PUCCH repetitions work with the SSB configurations of inter-cell multiple TRPs? (Note that this question is related to feMIMO WI rather than CovEnh WI.)

Q2: Please provide your preferences/views/clarifications on the following options.

- Option 1: SSBs with PCI different from the serving cell PCI are only used for actual PUSCH repetition determination after the available slots are determined

- Option 2: Both SSBs with serving cell PCI and SSBs with PCI different from the serving cell PCI are used for the available slot determination

- Option 3: SSBs with the same PCI as the SSB used to determine the spatial relation of the PUSCH, are used to determine the available slots. Other set of SSBs are used for actual PUSCH repetition determination after the available slots are determined.

| Company | Comments |
| --- | --- |
| Nokia/NSB | Q1: It is worth clarifying that the case of multiple TRPs (mTRPs) with separate SSB configurations is inter-cell mTRPs with multiple DCIs. Hence, from our understanding, how the legacy UL transmission works with inter-cell multi-DCI mTRPs has not been discussed in feMIMO WI. Q2: Option 3. |
| CATT | Q1: Open to hear other companies’ views. Q2: Option 3 sounds reasonable. If PUSCH repetition is received by one or multiple TRPs, it is natural that the SSB set(s) from related TRPs should be used to determine available slot. The SSBs of irrelevant TRPs does not need to be considered during available slot determination. It may be OK to consider them in Step 2 (actual repetition determination), to avoid potential inter-UE interference, or just simply ignore them to provide more opportunities for actual transmission. |
| FL | As mentioned by Nokia, this issue is about inter-cell mTRPs. |
| QC | Q1: The same issues are being discussed in feMIMO WI, but no conclusions have been drawn. One option for us would be to wait until feMIMO WI makes more progress. The WI is not handled in this meeting. We are okay to mirror the solution that feMIMO WI adopts to handle PUCCH repetitions. Q2: Prefer Option 2. It is information that is available to the UE beforehand and the collision rules dictate that if we don’t take it into account, the transmission will get dropped. Option 2 ensures more repetitions are actually transmitted. |
| Intel | Q1: Suggest to treat this in MIMO session, but not CovEnh session Q2: We prefer Option 3. The SSBs from other TRPs do not need to be considered for available slot determination. |
| vivo1 | Q1: Agree with feature leader that PUCCH repetition related changes are related to feMIMO WI, we should solve the PUSCH available slot determination issue here and it’s up to MIMO topic to decide whether the option we agreed here should be applied to PUCCH as well. Q2: Since option 1 may provide less number of actual repetitions compared to option 2 and option 3, either option 2 or option 3 is fine from our side. From coverage perspective to allow more actual repetitions, option 2 is a bit more preferred as a PUSCH transmission would not be allowed anyway when colliding with SSBs with serving cell PCI according to current spec. in TDD case. |
| ZTE | Q1: In MIMO session, the collision handling between UL channels/signals and non-serving cell SSB is not concluded yet. There are two options (with no difference between different UL channel/signal) on the table for now. We agree that the decision should be made in MIMO session. - Alt 1: The UE does not transmit any UL channel/signal (no matter associated with serving cell PCI or non-serving cell PCI). - Alt 2: The UE can only transmit UL signal/channel associated with the serving cell TRP. Q2: Option 1 and Option 3. We don’t see the difference between Option 1 and Option 3. For Option 1, it means SSBs with PCI the same as the serving cell PCI will be used to determine the available slots, and other set of SSBs are not. Could FL or proponents of Option 3 clarify the difference with Option 1? |
| Samsung | Q1: This should not be discussed in CovEnh – should be in MIMO maintenance. Q2: This should also be discussed in MIMO maintenance. |
| Sharp | Q1: Share the views from other companies. That discussion has not been concluded in feMIMO WI yet. Q2: We prefer waiting the discussions in feMIMO AI on how the legacy PUSCH/PUCCH repetitions work with inter-cell multi-TRPs. |
| Apple | Q1: According to our understanding, there is no conclusion in MIMO discussion on the UL transmission colliding with SSB for M-TRP operation. Q2: For three options, Option 3 is simple and preferred. the wording could be updated. The spatial relation of PUSCH and SSB is only applied to FR2, maybe coresetpoolindex could be used to indicate the SSB. |
| China Telecom | Q1: A unified solution for PUSCH and PUCCH is preferred. Q2: Option 3. |
| OPPO | Q1: We are open on the discussion. Q2: We prefer Option 3. |
| NTT DOCOMO | Q2 : We prefer Option2. In our understanding, whether UL transmission with the symbols overlapping with SSBs of different PCI is allowed or not is not discussed in M-TRP discussion. So that we may wait for the conclusion of M-TRP discussion or we assume one option that UL transmission is not allowed with the symbol since same PDCCH/PDSCH/PUCCH/PUSCH-Config is assumed for the multiple TRPs (In this case Option 2 is preferred.). |
| Xiaomi | Q1: discuss it in MIMO session if it has been concluded. Q2: we prefer Option 3, we agree that SSBs from other TRPs should not be considered for available slot determination. |
| Ericsson | Q1: In our view Rel-17 inter-cell multiple TRPs deals with multi-DCI Multi-PDSCH. Rel-17 Multi-TRP PUCCH repetition is for reliability, where multiple TRPs are in one serving cell. In this case, there is no separate SSB configurations associated with different TRPs, and a UE needs to consider all SSBs of mTRPs in the cell. Q2: With the same logic as Q1, if SSBs with different PCIs are considered, we prefer Option 2. |
| Lenovo, Motorola Mobility | Q1: Should be discussed in MIMO session Q2: Prefer option 3 as SSBs from other TRPs should be considered for available slot determination on one TRP |
| TCL | Q1: We are open on the discussion. Q2: We prefer Option 3. The SSB from other TRPs should not be considered for available slot determination. |
| LG | Q1: The discussion will be handled in feMIMO WI, but it would be better if the aligned solution are adopted for PUCCH and PUSCH. Q2: We prefer to wait the decision in feMIMO. Regarding three options, we are not sure if Option 3 has a benefit compared to Option 1. Option 2 seems to be a solution with both gain (PUSCH can be transmitted in more slots) and loss (latency increases due to postpone). Therefore, Option 1 is preferred, which uses SSB of the serving cell for available slot determination without changing the spec. |

1st round summary (Issue#2-2)

In feMIMO session the collision handling between UL channels/signals and multiple SSBs for inter-cell mTRPs has not been concluded yet, where it includes the collision handling between the legacy PUCCH repetitions and multiple SSBs for inter-cell mTRPs and the collision handling between the legacy PUSCH repetitions (i.e., PUSCH repetition Type A with the physical slot counting) and multiple SSBs for inter-cell mTRPs. The decision on these issues should be made in feMIMO session, taking into account mTRP aspects, e.g., what information is shared by multiple TRPs, etc.

Such mTRP aspects would highly affects the discussion on available slot determination in CovEnh as well. Moreover, several companies expressed that we can wait for the conclusion in feMIMO discussion. For the FL perspective, it is suggested waiting further progress in feMIMO session.

Proposed conclusion:

- The CovEnh discussion on the available slot counting for inter-cell mTRPs is deferred until further progress on the collision handling between UL channels/signals and multiple SSBs for inter-cell mTRPs is made in feMIMO session.

For the reference for the future discussion, the companies’ current positions are summarized as follows:

- Option 1: SSBs with PCI different from the serving cell PCI are only used for actual PUSCH repetition determination after the available slots are determined

  - ZTE, LG

- Option 2: Both SSBs with serving cell PCI and SSBs with PCI different from the serving cell PCI are used for the available slot determination

  - QC, vivo, NTT DOCOMO, Ericsson

- Option 3: SSBs with the same PCI as the SSB used to determine the spatial relation of the PUSCH, are used to determine the available slots. Other set of SSBs are used for actual PUSCH repetition determination after the available slots are determined.

  - Nokia/NSB, CATT, Intel, Apple, China Telecom, OPPO, Xiaomi, Lenovo/Motorola Mobility, TCL

[Open] Issue#2-3: Use of SSBs of other serving cells in half duplex CA operation for the available slot determination

For CA case, the collision handling is performed according to following text in the same section of 38.213 v17.0.0.

If a UE - is configured with multiple serving cells and is provided with directionalCollisionHandling-r16 = 'enabled' for a set of serving cell(s) among the multiple serving cells, and - indicates support of half-DuplexTDD-CA-SameSCS-r16 capability, and - is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells, for a set of symbols of a slot that are indicated to the UE for reception of SS/PBCH blocks in a first cell of the multiple serving cells by ssb-PositionsInBurst in SystemInformationBlockType1 or by ssb-PositionsInBurst in ServingCellConfigCommon, when provided to the UE, the UE does not transmit PUSCH, PUCCH, or PRACH in the slot if a transmission would overlap with any symbol from the set of symbols, and the UE does not transmit SRS in the set of symbols of the slot in - any of the multiple serving cells if the UE is not capable of simultaneous transmission and reception as indicated by simultaneousRxTxInterBandCA among the multiple serving cells, and - any one of the cells corresponding to the same band as the first cell, irrespective of any capability indicated by simultaneousRxTxInterBandCA.

For RAN1#107-e, we discussed whether half duplex CA UEs need to take configurations from different DL carriers into account when determining the available slots. For the FL perspective, the agreement in RAN1#106bis-e means only tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst of the serving cell where the PUSCH repetitions are performed are considered for the determination of available slots. However, due to the lack of time, FL suggested revisiting this issue in the CR phase.

According to contributions for RAN1#107bis-e, companies’ views for half duplex CA operation are summarized as follows.

- Only SSB in the serving cell where the PUSCH is transmitted is considered for available slot determination for the Type A PUSCH repetition transmissions. SSBs in different serving cells can be considered during PUSCH dropping after the available slot determination.

  - vivo [2], ZTE [3], Nokia/Nokia Shanghai Bell [4], CMCC [12], Ericsson [13]

- SSBs in different serving cells are also considered for the available slot determination.

  - Qualcomm [4]

- UL CA scenario is deprioritized in Rel-17 Coverage Enhancement WI.

  - Samsung [5]

1st round (Issue#2-3)

Do you agree on the following proposal?

FL proposal to Issue#2-3:

- For half duplex CA operation, select only one from:

  - Alt 1: Only SSB in the serving cell where the PUSCH is transmitted is considered for available slot determination for the Type A PUSCH repetition transmissions. SSBs in different serving cells can be considered during PUSCH dropping after the available slot determination.

  - Alt 2: SSBs in different serving cells are also considered for the available slot determination.

| Company | Comments |
| --- | --- |
| Nokia/NSB | Alt. 1 |
| CATT | Fine with the proposal. Prefer Alt1. |
| Panasonic | We agree to the FL proposal. Our preference is Alt.1. |
| QC | Agree |
| Intel | For coverage enhancement, our view is that CA operation should be deprioritized. Even for HD-FDD case, it is mainly for single cell operation. We do not see the need to discuss this issue. |
| vivo1 | Fine. |
| ZTE | Support Alt 1, which aligns with previous agreements. |
| Samsung | We don’t agree. It was already discussed and there was no consensus in considering UL CA scenario, the reason being that UL CA is not a realistic use case for a UE in need of coverage enhancement. No need to split power, and also lose several dB due to MPR on top, when the UE is coverage limited. Also, phase consistency and DMRS bundling gains will be lost. Overall, that is a scenario that does not justify any further consideration at this stage. |
| Sharp | Support the proposal. We prefer Alt 1. |
| Apple | The proposal is for TDD half duplex CA UE, this is not the typical coverage enhancement scenario. In case of DL CA, the simple solution is enough. We prefer Alt1. |
| China Telecom | Support Alt 1. |
| OPPO | We prefer Alt1. |
| Huawei/HiSilicon | Alt.1. In our understanding, in the CR phase, spec impact is the focus. Considering that Alt 1 has been reflected in the current spec, we could directly discuss whether the additional spec impact required by Alt2 is necessary. We would like to suggest to rephrase the proposal to whether additional spec impact for Alt 2 is necessary. |
| Xiaomi | We prefer alt 1. |
| Ericsson | Support Alt1. |
| Lenovo, Motorola Mobility | Support Alt 1 |
| TCL | Alt 1 is prefer |
| LG | We prefer Alt 1. |

1st round summary (Issue#2-3)

Companies’ views are summarized as follows:

- Fine with the proposal

  - Prefer Alt 1: Nokia/NSB, CATT, Panasonic, ZTE, Sharp, Apple, China Telecom, OPPO, Huawei/HiSIlicon, Xiaomi, Ericsson, Lenovo/Motorola Mobility, TCL, LG

  - QC, vivo

- Do not discuss this issue

  - Intel, Samsung

The large majority prefers Alt 1. At the same time, Huawei provided a good suggestion rephrasing the proposal to whether additional spec impact for Alt 2 is necessary. Considering this and the 1st round inputs, FL would like to make the following proposal.

Updated FL proposal to Issue#2-3:

- For half duplex CA operation, no additional spec impact is necessary in terms of consideration of SSB(s) for the available slot determination.

[Open] Issue#2-4: Available slot counting for DG-PUSCH scheduled by DCI format 0_0 with other RNTI than TC-RNTI

So far, we have not made any agreement on whether available slot counting is applied to DG-PUSCH scheduled by DCI format 0_0 with other RNTIs than TC-RNTI. In the current TS38.214, the behaviors related to the available slot counting is specified for DCI format 0_1 and 0_2 but not for DCI format 0_0. Sharp [10] is proposing that available slot counting is NOT applied to DG-PUSCH scheduled by DCI format 0_0 with other RNTIs than TC-RNTI even when AvailableSlotCounting is enabled.

1st round (Issue#2-4)

Do you agree on the following proposal?

FL proposal to Issue#2-4:

- A slot for PUSCH scheduled by DCI format 0_0 with C-RNTI / CS-RNTI with NDI=1 / MCS-C-RNTI is determined by physical slots (not available slots), irrespective of whether AvailableSlotCounting is enabled or not.

  - No specification impact is expected.

| Company | Comments |
| --- | --- |
| Nokia/NSB | Given that there is no specification impact, we don’t see the need to make an agreement on this issue. |
| CATT | Agree. |
| Panasonic | Agree. |
| QC | Legacy R15/R16 spec does not support PUSCH repetitions when triggering PUSCH using DCI Format 0_0. R17 made no changes in this regard. If repetitions cannot be triggered, then the question of available slot-based counting does not arise. We don’t think any clarification on this is necessary. |
| Intel | Given that there is no spec impact, we think it is more appropriate to capture this as conclusion, rather than proposal. |
| vivo1 | Agree. |
| ZTE | Fine with the proposal. |
| Samsung | No need for this agreement. UE behavior for the cases that the new feature applies (for DCI format 0_1 and 0_2) is already captured in the specifications. |
| Sharp | Agree. |
| Apple | Fine with the proposal. |
| OPPO | Agree. |
| Huawei/HiSilicon | Since no spec impact, we prefer not to spend time on this. |
| Xiaomi | Agree |
| Ericsson | Agree. |
| Lenovo, Motorola Mobility | Agree |
| TCL | Agree. |
| LG | We are not sure whether the clarification is necessary, but ok to make a conclusion. |

1st round summary (Issue#2-4)

No company objected the proposal. Several companies do not see the need to take it as an agreement. Therefore, FL would suggest taking it as a conclusion.

Proposed conclusion:

- A slot for PUSCH scheduled by DCI format 0_0 with C-RNTI / CS-RNTI with NDI=1 / MCS-C-RNTI is determined by physical slots (not available slots), irrespective of whether AvailableSlotCounting is enabled or not.

  - No specification impact is expected.

[Open] Issue#2-5: Available slot counting for PUSCH repetition Type A with K=1

So far, we have not explicitly made any agreement on whether available slot counting is applied to PUSCH with K=1. Note that, in Rel-16, the PUSCH repetition Type A concept covers both K=1 and K>1. Sharp [10] is proposing that available slot counting is applied to DG-PUSCH and CG-PUSCH with K=1 when AvailableSlotCounting is enabled.

1st round (Issue#2-5)

Do you agree on the following proposal?

FL proposal to Issue#2-5:

- A slot for DG-PUSCH repetition Type A with K=1 is determined by available slots when AvailableSlotCounting is enabled.

  - This includes the case of DG-PUSCH with CSI only.

  - No specification impact is expected.

- A slot for CG-PUSCH repetition Type A with K=1 is determined by available slots when AvailableSlotCounting is enabled.

| Company | Comments |
| --- | --- |
| Nokia/NSB | Given that there is no specification impact, we don’t see the need to make an agreement on this issue. |
| CATT | For PUCCH, available counting happens only when NPUCCHrepeat>1, - 9.2.6 PUCCH repetition procedure … For NPUCCHrepeat>1, - the UE repeats the PUCCH transmission with the UCI over NPUCCHrepeat slots - a PUCCH transmission in each of …. If PUCCH is not repeated, available counting is not applied, and gNB should indicate a usable slot/symbol groups for PUCCH transmission. So, if we prefer that PUSCH strictly follows PUCCH mechanism, then when K=1, available slot counting should not be used. The gNB should indicate a correct slot (not overlapped with DL or SSB) by TDRA indication. But we are open to consider different behavior between PUSCH and PUCCH, if it is beneficial. |
| Panasonic | Agree |
| QC | Its not clear why AvailableSlotCounting would come into play for a dynamically triggered PUSCH with no repetitions. The slot where the first transmission is to be carried out should be clear from the grant itself. Even for CG-PUSCH with K=1, we think the gNB should be able to configure in such a manner that the first slot is available for transmission (gNB knows the tdd configs and ssb positions). We don’t see the value to this relaxation. |
| Intel | It is not clear to us whether this is needed. The enhancement on PUSCH repetition type A is intended for the case when repetition is applied for the PUSCH type A. K = 1 is clearly not for the repetition. We do not see the need to capture the proposal. |
| vivo1 | For the first bullet, the single slot for DG PUSCH normally can be always available slot which can be controlled by network. For the 2nd bullet, this seems to be not necessary. For coverage limited scenario, network will not configure repK=1. If latency is important, DG PUSCH will be used. So in such case, PUSCH transmission based on physical slot may be enough. |
| ZTE | We have similar view as CATT. In case of K=1, available slot counting should not be used to follow similar rules of PUCCH repetition. In addition, UE behavior would be also aligned with that of PUSCH scheduled by DCI format 0_0 discussed above. |
| Samsung | No need to introduce applicability of AvailableSlotCounting with no repetitions. |
| Sharp | Although this is our proposal, we also understand the point raised by CATT that available counting is not applied to the legacy PUCCH without repetitions. So, we are open to further discuss this issue. |
| Apple | Seems current specification already support k=1, or repK=1. If proposal is not agreeable, the related configuration need to be restricted. |
| China Telecom | Not necessary support AvailableSlotCounting for K=1. |
| OPPO | It is not necessary to capture this proposal. |
| Huawei/HiSilicon | For any K, the gNB should ensure the first indicated slot is an available slot. However, the only value provided by the proposal is to allow a gNB to indicate an unavailable slot as the first slot when K=1. It is unclear why it is beneficial. Therefore, for the simplicity of spec, we don’t feel the second bullet is necessary. |
| Xiaomi | It seems to be not necessary to introduce the AvailableSlotCounting when K=1 |
| Ericsson | For CG-PUSCH, it is hard to say if the available slot counting framework is beneficial CG-PUSCH with K=1. A Rel-15/16 UE transmits CG-PUSCH in a flexible slot, only if it is indicated as UL by dynamic SFI. If a flexible slot is indicated as DL by dynamic SFI, a legacy UE doesn't transmit PUSCH in the slot and looks for another UL slot or flexible slot, which is indicated as UL by dynamic SFI. But if a Rel-17 UE applies the rule "flexible symbols are available for CG-PUSCH" to determine available slots, the PUSCH transmission in the flexible slot, which is indicated by DL by dynamic SFI, will be dropped. We prefer that PUSCH repetition based on available slot applies to PUSCH transmission with K>1. |
| Lenovo, Motorola Mobility | Not needed to capture this proposal |
| TCL | For DG-PUSCH transmission with no repetition, gNB can dynamic indicated an available slot to UE, AvailableSlotCounting is no need. For CG-PUSCH transmission with no repetition, an available slot for UE could be also configured by gNB. |
| LG | No need to agree on this proposal. |

1st round summary (Issue#2-5)

Companies’ views are summarized as follows:

- Available slot counting is applicable to PUSCH repetition Type A with K=1

  - Sharp, Panasonic, vivo (for DG-PUSCH)

- Available slot counting is not applicable to PUSCH repetition Type A with K=1

  - CATT, QC, Intel, vivo (for CG-PUSCH), ZTE, Samsung, China Telecom, OPPO, Huawei/HiSilicon (for CG-PUSCH), Xiaomi, Ericsson, Lenovo/Motorola Mobility, TCL, LG

Companies have different views. In addition, several companies prefer different counting schemes for DG-PUSCH and CG-PUSCH when K=1. Therefore, FL suggests having further discussions.

Updated FL proposal to Issue#2-5:

For DG-PUSCH when AvailableSlotCounting is enabled, select one from:

- A slot for DG-PUSCH repetition Type A with K=1 is determined by physical slots.

- A slot for DG-PUSCH repetition Type A with K=1 is determined by available slots.

For CG-PUSCH when AvailableSlotCounting is enabled, select one from:

- A slot for CG-PUSCH repetition Type A with K=1 is determined by physical slots.

- A slot for CG-PUSCH repetition Type A with K=1 is determined by available slots.

Correction proposals

For this meeting, the following proposals for corrections on Rel-17 RAN1 specifications have been raised.

- Issue#3-1: Corrections on the definition of available slots

- Issue#3-2: Corrections on frequency hopping

- Issue#3-3: Corrections on available slot counting for FDD

- Issue#3-4: Corrections on available slot counting for TBoMS

[Open] Issue#3-1: Corrections on the definition of available slots

Agreement For DG-PUSCH with counting based on the available slots, count of available slots continues until satisfying the conditions defined for DG-PUSCH repetition Type A in Rel-16.

In RAN1#106-e, it was agreed that for DG-PUSCH, the count of available slots continues until satisfying the conditions defined for DG-PUSCH repetition Type A in Rel-16, and this agreement means the K available slots are the earliest available slots. Huawei/HiSilicon [1] is raising the issue that this is not correctly captured in TS38.214v17.0.0. In order to fix this issue, Huawei/HiSilicon [1] is providing the following TP.

TP in R1-2200051

<Unchanged parts are omitted – 38.214> 6.1.2.1 Resource allocation in time domain <Unchanged parts are omitted – 38.214> For unpaired spectrum: - When AvailableSlotCounting is enabled, the UE determines the earliest N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, based on tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst, and the TDRA information field value in the DCI format 0_1 or 0_2. - A slot is not counted in the number of N∙K slots for PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. - Otherwise, the UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. <Unchanged parts are omitted – 38.214>

1st round (Issue#3-1)

Companies are invited to provide their views on the above TP in R1-2200051.

| Company | Comments |
| --- | --- |
| CATT | Even without the modification, the definition is still clear to us. But we are fine with the TP if it can improve the reading. |
| Panasonic | We are fine with the above TP. |
| QC | Its should be clear to all companies that we have been referring to the earliest K or N*K slots. Don’t think there are any divergent views here. If we make this change here, and leave all other instances where we refer to K slots or N*K slots unchanged, it can create some confusion. The language is rather consistent all through the spec. Assuming “earliest” is the common understanding, we prefer to leave things unchanged. |
| Intel | Do not support the TP. The existing spec is already clear. |
| vivo1 | The TP is not necessary. For DG PUSCH the first slot can be always controlled by network and can be configured to be on available slot in our view (and we cannot find a reason why a network would dynamically schedule a first repetition to be on unavailable slot) . And after the first slot, the remaining available slot will be the latest available slots anyway with current wording. |
| ZTE | It’s not clear for us why such modification is needed. Further clarification is needed. |
| Samsung | It seems no need to add it as the UE behavior is already clear. Also there are other instances of “UE determines N∙K slots for a PUSCH transmission” when AvailableSlotCounting is enabled throughout 38.214. |
| Sharp | We are fine with the TP, but also agree with other companies that the current text is interpreted as “earliest”. In fact, we already have the following agreement: Agreement - For the K repetitions of DG-PUSCH, Step 1 of the previously agreed two-step procedure (i.e., Alt 1-B) determines the K earliest available slots no earlier than the slot which is determined by the slot offset K2. - No RAN1 spec impact is expected in terms of the relation with the slot which is determined by the slot offset K2. - Note: The available slot determination is to be specified. - For the K repetitions of CG-PUSCH, Step 1 of the previously agreed two-step procedure (i.e., Alt 1-B) determines the K earliest available slots no earlier than the first slot which is determined by at least ConfiguredGrantConfig. - No RAN1 spec impact is expected in terms of the relation with the first slot which is determined by at least ConfiguredGrantConfig. - Note: The available slot determination is to be specified.. |
| Apple | Seems current spec is clear enough. |
| OPPO | The definition of the existing spec seems to be already clear. |
| Huawei/HiSilicon | Agree. Thus we can correctly capture the previous agreement. |
| Ericsson | No need for the change, because there is no specification to allow UE to skip an available slot. |
| Lenovo, Motorola Mobility | Current spec seems to be correct |
| TCL | We are fine with the TP. |
| LG | The existing specification seems clear. |

1st round summary (Issue#3-1)

Companies’ views are summarized as follows:

- Accept the TP

  - CATT (if it can improve the reading.), Panasonic, Sharp, Huawei/HiSilicon, TCL

- The TP is not necessary

  - CATT, Intel, vivo, ZTE, Samsung, Apple, OPPO, Ericsson, Lenovo/Motorola Mobility, LG

Since companies have different views, FL suggest continuing the discussion.

[Open] Issue#3-2: Corrections on frequency hopping

For TS38.214v17.0.0, Apple [9] is raising the issue that, for PUSCH repetition type A frequency hopping, the current descriptions limit the PUSCH is scheduled by DCI format 0_1 or 0_2, RAR UL grant or DCI format 0_0 with TC-RNTI. However, intra-slot frequency hopping is also applicable to a single slot PUSCH transmission, for example, the case when PUSCH is scheduled by DCI format 0_0 with C-RNTI. In order to resolve this issue, Apple [9] is providing the following TP.

TP in R1-2200420

- 6.3.1 Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots For PUSCH repetition Type A and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in configuredGrantConfig for configured PUSCH transmission. For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. One of two frequency hopping modes can be configured: - Intra-slot frequency hopping, applicable to single slot and multi-slot configured PUSCH transmission and multi-slot PUSCH transmission scheduled by DCI format 0_1 or 0_2 and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission.

1st round (Issue#3-2)

Companies are invited to provide their views on the above TP in R1-2200420.

| Company | Comments |
| --- | --- |
| Nokia/NSB | The later sentence in the first paragraph quoted above should clarify the concern, i.e., “For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively” |
| CATT | We have the same understanding with Nokia. |
| FL | FL’s above explanation was wrong. This issue seems not about RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI but related to a single slot PUSCH transmission (e.g., PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI). Therefore, I made updates accordingly. Sorry for the confusion. |
| QC | This change seems reasonable. |
| Intel | We do not support this. For Msg3 initial and retransmission, it was captured under “For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI,…”. |
| vivo1 | Agree that something is wrong when both Msg3 repetition and TBoMS were captured in this section for Rel-17. However, simply removing the text (scheduled by DCI format 0_1 or 0_2) as proposed by Apple may not solve this issue as this would mean that FH of Msg3 repetition will also rely on the parameter frequencyHopping provided in pusch-Config to indicate which FH type (intra or inter-slot FH) to use which is wrong since we agreed that only inter-slot FH is supported when Msg3 is repeated meaning that no intra/inter-slot FH type indication is needed. For PUSCH repetition Type A other than the PUSCH scheduled by 0_1 or 0_2 RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in configuredGrantConfig for configured PUSCH transmission. For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. One of two frequency hopping modes can be configured: - Intra-slot frequency hopping, applicable to single slot and multi-slot configured PUSCH transmission and multi-slot PUSCH transmission scheduled by DCI format 0_1 or 0_2 and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission.. |
| ZTE | We suggest to directly add the missing case (i.e., PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI) or choose the proposed TP from vivo. |
| Sharp | We support the proposal, as the proposed change would more aligned with Rel-16 descriptions, in terms of a single slot PUSCH transmission. |
| Apple | As indicated by ZTE, adding missing part is another choice. For PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 or 0_0 with CRC scrambled by C-RNTI and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, |
| Ericsson | Support Vivo’s TP. |
| Lenovo, Motorola Mobility | We are fine with th TP |

1st round summary (Issue#3-2)

All the companies that mentioned the support of a single slot PUSCH were thinking the specification should be fixed, though several variants of the TP were provided during the 1st round. Therefore, FL would like to suggest agreeing on the following proposal and then exact TP will be discussed in the next round.

FL proposal to Issue#3-2:

- Clause 6.3.1 of TS38.214v17.0.0 is updated such that it covers single slot PUSCH transmissions.

  - FFS: exact TP.

[Open] Issue#3-3: Corrections on available slot counting for FDD

For TS38.214v17.0.0, Sharp [10] is pointing an inconsistency between the main bullet and its sub-bullet, because main bullet says “N∙K consecutive slots” but the sub-bullet implies “N∙K slots” may be non-consecutive. In order to resolve this issue, Sharp [10] is providing the following TP.

TP in R1-2200500

- 6.1.2.1 Resource allocation in time domain [Omitted] For paired spectrum and SUL band: - For the case of non reduced capability half-duplex UE irrespective of whether AvailableSlotCounting is enabled or not, or the case of reduced capability half-duplex UE and when AvailableSlotCounting is not enabled, the UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. - For the case of reduced capability half-duplex UE, and when AvailableSlotCounting is enabled, he UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. A slot is not counted in the number of N∙K slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, based on the TDRA information field value in the RAR UL grant. - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, based on the TDRA information field value in the DCI scheduling the PUSCH. [Omitted] 6.1.2.3.1 Transport Block repetition for uplink transmissions of PUSCH repetition Type A with a configured grant [Omitted] For both Type 1 and Type 2 PUSCH transmissions with a configured grant, when K > 1, - For unpaired spectrum: - If AvailableSlotCounting is enabled, the UE shall repeat the TB across the N∙K slots determined for the PUSCH transmission applying the same symbol allocation in each slot. - A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. - Otherwise, the UE shall repeat the TB across the N∙K consecutive slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots and cg-nrofPUSCH-InSlot, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration. - For paired spectrum: - In case of non reduced capability half-duplex UE irrespective of whether AvailableSlotCounting is enabled or not, or in case of reduced capability half-duplex UE when AvailableSlotCounting is not enabled, the UE shall repeat the TB across the N∙K consecutive slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots and cg-nrofPUSCH-InSlot, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration. - If AvailableSlotCounting is enabled, and in case of reduced capability half-duplex UE, the UE shall repeat the TB across the N∙K slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots and cg-nrofPUSCH-InSlot, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration. A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

1st round (Issue#3-3)

Companies are invited to provide their views on the above TP in R1-2200500.

| Company | Comments |
| --- | --- |
| Nokia/NSB | Adding “For the case of non reduced capability half-duplex UE…” in the main bullet and making equal bullet level makes these paragraphs look like focusing on half-duplex UE only. In fact, the current structure of the specification is clearer since the sub-bullet can be seen as an exception of the main bullet. |
| CATT | OK. |
| Panasonic | We are fine with the above TP. |
| QC | It is correct that for redcap HD UEs, the N*K slots need not be consecutive. This needs to be fixed. |
| Intel | We do not see the need to change the current spec. The existing spec is already clear. |
| vivo1 | Seems fine except a typo “the UE”. |
| ZTE | Fine with the TP. Two minor editorial comments: 1) ‘he’ should be ‘the’ for the second sub-bullet of the TP in section 6.1.2.1, and 2) the last sub-bullet of the TP in section 6.1.2.3.1 should have the same level with the sub-bullet above. |
| Sharp | Support the TP with the corrections suggested by vivo and ZTE. We do not agree on Nokia’s statement “the sub-bullet can be seen as an exception of the main bullet”. For the paired spectrum, people can see almost the same structure of the main bullet and the sub-bullet (see below), where the sub-bullet is the further clarification of the main-bullet, but not an exception. Two parts with the same bullet-point structure leading to two different interpretations would cause unnecessary confusion and have to be avoided. - When AvailableSlotCounting is enabled, the UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, based on tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst, and the TDRA information field value in the DCI format 0_1 or 0_2. - A slot is not counted in the number of N∙K slots for PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.. |
| Apple | According to our understanding, HD-FDD UE supporting TBoMS is still open for now. |
| OPPO | OK with the TP. |
| Xiaomi | Fine with the TP |
| Ericsson | Agree with ZTE’s comments. |
| Lenovo, Motorola Mobility | Fine with the TP |
| TCL | Agree with ZTE’s comments. |

1st round summary (Issue#3-3)

Companies’ views are summarized as follows:

- Support the TP (with a couple of typos fixed)

  - CATT, Panasonic, QC, vivo, ZTE, Sharp, OPPO, Xiaomi, Ericsson, Lenovo/Motorola Mobility, TCL

- The TP is not necessary

  - Nokia/NSB, Intel

Since companies have different views, FL suggest continuing the discussion.

[Pending] Issue#3-4: Corrections on available slot counting for TBoMS

For TS38.214v17.0.0, Sharp [10] is pointing out that the definition of available slots for TBoMS with a configured grant is missing. In order to resolve this issue, Sharp [10] is providing the following TP.

TP in R1-2200500

##### 6.1.2.3.3 Transport Block repetition for uplink transmissions of TB processing over multiple slots with a configured grant The procedures described in this clause apply to PUSCH transmissions of TB processing over multiple slots with a Type 2 configured grant. The higher layer parameter repK-RV defines the redundancy version pattern to be applied to the repetitions. If the parameter repK-RV is not provided in the configuredGrantConfig, the redundancy version for uplink transmissions with a configured grant shall be set to 0. If the parameter repK-RV is provided in the configuredGrantConfig, the nth transmission occasion among N∙K transmissions occasions, n=0,1, …, N∙K -1, is associated with (mod((n-mod(n, N))/N,4)+1)th value in the configured RV sequence. When K=1, or when K>1 and the configured grant configuration is configured with startingFromRV0 set to 'off', the initial transmission of the transport block may only start at the first transmission ocasion of the N∙K transmission occasions. Otherwise, the initial transmission of the transport block may start at - The first transmission occasion of the N∙K transmission occasions if the configured RV sequence is {0,2,3,1}. - Any transmission occasion n associated with RV=0, and for which n mod N =0, if the configured RV sequence is {0,3,0,3} or {0,0,0,0}. If the UE determines that, for a transmission occasion, the number of symbols available in a slot for the PUSCH transmission of TB processing over multiple slots is smaller than transmission duration L, the UE does not transmit the PUSCH in the transmission occasion. For unpaired spectrum: - The UE determines N∙K slots for a PUSCH transmission of a TB processing over multiple slots, based on tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst. - A slot is not counted in the number of N∙K slots for PUSCH transmission of TB processing over multiple slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. For paired spectrum and SUL band: - For the case of non reduced capability half-duplex UE, the UE determines N∙K consecutive slots for a PUSCH transmission of a TB processing over multiple slots. - For the case of reduced capability half-duplex UE, the UE determines N∙K slots for a PUSCH transmission of a TB processing over multiple slots. A slot is not counted in the number of N∙K slots for a PUSCH transmission of a TB processing over multiple slots, if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. For Type 2 PUSCH transmission with a configured grant of TB processing over multiple slots, the UE shall transmit the TB across the N∙K slots determined for the PUSCH transmission applying the same symbol allocation in each slot. A Type 2 PUSCH transmission with a configured grant of TB processing over multiple slots is omitted in a slot according to the conditions in clause 9, clause 11.1 and clause 11.2A of [6, TS 38.213].

The same TP is also proposed in AI 8.8.1.2. Since this proposal is more related to TBoMS, FL suggests discussing this proposal in AI 8.8.1.2.

References

- R1-2200051	Discussion on coverage enhancements for PUSCH repetition type A	Huawei, HiSilicon

- R1-2200086	Remaining issues on enhancement for PUSCH repetition type A	vivo

- R1-2200111	Discussion on remaining issues for enhanced PUSCH repetition type A	ZTE

- R1-2200160	Enhancements on PUSCH repetition type A	Nokia, Nokia Shanghai Bell

- R1-2200205	Enhancements on PUSCH repetition type A	Samsung

- R1-2200301	Enhancements on PUSCH Repetition Type A	Qualcomm Incorporated

- R1-2200334	Enhancements on PUSCH repetition type A	OPPO

- R1-2200379	Remaining details of enhancements on PUSCH repetition type A	Intel Corporation

- R1-2200420	Discussion on PUSCH repetition type A enhancement	Apple

- R1-2200500	Enhancements on PUSCH repetition type A	Sharp

- R1-2200518	Type-A PUSCH repetition for coverage enhancement	InterDigital, Inc.

- R1-2200588	Remaining issues on PUSCH repetition type A enhancement	CMCC

- R1-2200655	Remaining Issues for PUSCH Repetition Type A Enhancement	Ericsson

List of agreements

Agreements in RAN1#104-e

Agreements:

Select one of the following alternatives, considering the aspect whether or not the determination of all the available slots should be done prior to the first actual transmission of the repetitions (other alternatives are not precluded)

-        Alt1: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and does not depend on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC).

-        Alt2: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and also depends on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC).

Agreements:

The maximum number of repetitions for DG-PUSCH is also applicable to CG-PUSCH.

Agreements:

For defining available slots: a slot is determined as unavailable if at least one of the symbols indicated by TDRA for a PUSCH in the slot overlaps with the symbol not intended for UL transmissions

- FFS details

Agreements:

Rel-17 PUSCH repetition Type A supports the increase of maximum number of repetitions with repetition factors configured in a TDRA list with a row index indicated either by the configured grant configuration or by TDRA field in a DCI.

- FFS: increasing the maximum number of repetitions with repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig.

Conclusion:

Discuss further to select one of the following alternatives:

- Alt-a: The determination of all the available slots has to be done prior to the first actual transmission of the repetitions.

- Alt-b: The determination of all the available slots does not have to be done prior to the first actual transmission of the repetitions. The timeline requirement is per repetition basis.

Agreements in RAN1#105-e

Agreement:

- RV cycling is based on available slot for the Type A PUSCH repetition enhancement with repetitions counted based on available slot in Rel-17

Agreement:

- Down-selection in RAN1#106-e:

- Alt 1: The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is 32, irrespective of counting method,

- Alt 2: The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is: 32 for the counting based on physical slots; and 16 (i.e. no change from Rel-16) for the counting based on available slots.

Conclusion:

- The following agreement in RAN1#104-e is applied to all slots including special slots.

Agreements: For defining available slots: a slot is determined as unavailable if at least one of the symbols indicated by TDRA for a PUSCH in the slot overlaps with the symbol not intended for UL transmissions. - FFS details

Agreement:

In addition to {1, 2, 3, 4, 7, 8, 12, 16} and {32}, the following additional value set for repetition factor is supported in Rel-17.

- {20, 24, 28}

Agreement:

- Each available slot identified by the UE is considered as a transmission occasion for PUSCH repetition.

  - RV is cycled across transmission occasions, irrespective of whether PUSCH transmission in the transmission occasion is further omitted or not.

Agreement:

- If PUSCH symbol in a slot overlaps with flexible symbol(s) with SSB transmission, the slot is determined as not available during the counting of repetitions. As there is no PUSCH in the slot, no PUSCH omission applies to the slot.

Agreement:

Select one from the following (further refinement of the alternatives can be further discussed), for the procedure of Rel-17 PUSCH repetition Type A (other alternatives are not precluded)

- Alt 1-B consisting of two steps

  - Step 1: Determine available slots for K repetitions based on RRC configuration(s) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI

  - Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

- Alt 1-B’ consisting of two steps

  - Step 1: Determine K repetitions based on available slots, where the available slot is the UL slot and flexible slot indicated by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated.

  - Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

  - FFS: handling of dynamic signaling (e.g. UL CI, DCI for high priority channel), e.g., UE without CI capability

- Alt 2-A consisting of a single step

  - Step 1: Determine available slots for K repetitions based on RRC configuration(s) and dynamic signaling (e.g. SFI, UL CI, DCI for high priority channel) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI

- Alt 2-B consisting of two steps

  - Step 1: Determine available slots for K repetitions based on RRC configuration(s) and dynamic SFI in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI

    - FFS timeline for the dynamic signalling

  - Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

Agreements in RAN1#106-e

Agreement:

- For Rel-17 PUSCH repetition Type A without joint channel estimation, no new inter-slot frequency hopping mechanism is introduced.

Agreement

Take Option 1-B as an agreement for the procedure of Rel-17 PUSCH repetitions counted on the basis of available slots.

- Alt 1-B consisting of two steps

- Step 1: Determine available slots for K repetitions based on RRC configuration(s) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI

- Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

- FFS: Rel-17 PUSCH dropping rules are also applied if introduced in other WI(s)

Agreement

For PUSCH repetition Type A for Rel-17 CG-PUSCH, semi-static flexible symbol is considered as available.

Agreement

For PUSCH repetition Type A for Rel-17 DG-PUSCH, semi-static flexible symbol is considered as available.

Note: The applicability for Msg 3 is to be discussed in 8.8.3

Agreement

- DCI format 0_1 and DCI format 0_2 support Rel-17 PUSCH repetition Type A with the increased maximum repetition numbers configured in TDRA lists.

Agreement

- For DG-PUSCH with counting based on the available slots, count of available slots continues until satisfying the conditions defined for DG-PUSCH repetition Type A in Rel-16.

Working Assumption

The maximum number of repetitions accounted for available slots supported by Rel-17 PUSCH repetition Type A is 32

Agreements in RAN1#106bis-e

Working Assumption is confirmed

Working Assumption

The maximum number of repetitions accounted for available slots supported by Rel-17 PUSCH repetition Type A is 32

Conclusion:

For CG-PUSCH repetitions counted on the basis of available slots, all the K transmission occasions including the 1st transmission occasion are determined on the basis of available slots.

Agreement

For CG-PUSCH repetition Type A with the counting based on available slots, the R16 existing restrictions as defined in Clause 6.1.2.3.1 of TS38.214 at least on the initial transmission of a transport block are applied, assuming the K repetitions of R17 determined based the rule of counting available slots.

Observation

- Whether or not the counting based on available slots is applicable only to unpaired spectrum is not discussed under AI 8.8.1.1 in RAN1#106bis-e. Discussions on how HD-FDD RedCap UEs support the available slot counting may take place in AI 8.8.1.1 in RAN1#107-e, depending on the progress of RedCap WI discussions.

Agreement

- For the K repetitions of DG-PUSCH, Step 1 of the previously agreed two-step procedure (i.e., Alt 1-B) determines the K earliest available slots no earlier than the slot which is determined by the slot offset K2.

  - No RAN1 spec impact is expected in terms of the relation with the slot which is determined by the slot offset K2.

  - Note: The available slot determination is to be specified.

- For the K repetitions of CG-PUSCH, Step 1 of the previously agreed two-step procedure (i.e., Alt 1-B) determines the K earliest available slots no earlier than the first slot which is determined by at least ConfiguredGrantConfig.

  - No RAN1 spec impact is expected in terms of the relation with the first slot which is determined by at least ConfiguredGrantConfig.

  - Note: The available slot determination is to be specified.

Agreement

- Only tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst are considered for the determination of available slots.

  - Any other RRC configuration is not considered for the determination of available slots.

Agreement

- The existing restriction “The UE is not expected to be configured with the time duration for the transmission of K repetitions larger than the time duration derived by the periodicity P” applies to both the counting based on physical slots and the counting based on available slots.

- The above “the time duration for the transmission of K repetitions” means the time duration between the start of the 1st slot of the K repetitions and the end of the last slot of the K repetitions for any instance of a CG period.

Agreements in RAN1#107-e

Agreement

- The counting based on available slots is applicable to unpaired spectrum, paired spectrum and SUL

  - For paired spectrum and SUL except HD-FDD, all slots are considered as available slots in the first step of determining the available slots.

Agreement

- For HD-FDD RedCap Ues supporting the counting based on available slots.

  - For CG-PUSCH, ssb-PositionsInBurst is used in the first step of determining of available slots.

    - A slot is not counted in the number of available slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionInBurst.

  - FFS: For DG-PUSCH

  - Note: Neither tdd-UL-DL-ConfigurationCommon nor tdd-UL-DL-ConfigurationDedicated is configured for FDD.

Agreement

- Rel-17 does not support numberOfRepetitions-r17 for DG-PUSCH scheduled by DCI format 0_0 and for Type 2 CG-PUSCH activated by DCI format 0_0.

- repK-r17 supporting up-to-32 repetitions is introduced and is applicable to Type 1 CG-PUSCH and Type 2 CG-PUSCH (irrespective of the activating DCI format).

  - Note: No RAN1 spec impact is expected.

  - The possible values of repK-r17 includes 16 and 32. FFS: other values.

- numberOfRepetitions-r17 is not applicable to Type 1 CG-PUSCH repetition type A.

Agreement

- All the following combinations support the counting based on available slots.

  - DG-PUSCH with Rel-15 repetition factor

  - Type-1 CG-PUSCH with Rel-15 repetition factor

  - Type-2 CG-PUSCH with Rel-15 repetition factor

  - DG-PUSCH with Rel-16 repetition factor

  - Type-2 CG-PUSCH with Rel-16 repetition factor

  - DG-PUSCH with Rel-17 repetition factor

  - Type-1 CG-PUSCH with Rel-17 repetition factor, if supported in Issue#1-1

  - Type-2 CG-PUSCH with Rel-17 repetition factor

Conclusion

  - Rel-17 PUSCH repetition Type A with K>1 does not support PUSCH transmission without UL-SCH.

Agreement

  For repK-r17,

  The value range of repK-17 is {1, 2, 4, 8, 12, 16, 24, 32}.

  repK-r17 is included in ConfiguredGrantConfig.

  When repK-r17 is provided, the legacy repK is not provided.

Agreement

  For HD-FDD RedCap Ues supporting the counting based on available slots.

  For DG-PUSCH, ssb-PositionsInBurst is used in the first step of determining of available slots.

  A slot is not counted in the number of available slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionInBurst.

  Note: Neither tdd-UL-DL-ConfigurationCommon nor tdd-UL-DL-ConfigurationDedicated is configured for FDD.