3GPP TSG RAN WG1 #107bis-e			R1-2200744

e-Meeting, January 17th – 25th, 2022

Agenda Item:	8.2.5

Source: 	Moderator (LG Electronics)

Title: 	Summary #2 of PDSCH/PUSCH enhancements (Scheduling/HARQ)

Document for:	Discussion and decision

Introduction

This is the summary document for 8.2.5 on PDSCH/PUSCH enhancements (especially for scheduling and HARQ) for NR above 52.6 GHz, based on the contributions listed in reference section.

The following email thread is assigned for discussion of this topic:

[107bis-e-R17-52-71GHz-06] Email discussion/approval on scheduling particularly w.r.t. multi-PDSCH/PUSCH with a single DCI, HARQ – Seonwook (LGE)

- 1st check point: January 20

- Final check point: January 25

Among text proposals in the contributions, the ones that seem to be able to be directly discussed without agreeing on the related functionality are listed up in Section 4 while the other text proposals can be further discussed after more generic agreement is made in this meeting. Based on companies’ comments, 4 TPs that seem stable are provided in Section 5.

Agreements, conclusions and endorsed TPs made in RAN1#107bis-e meeting are provided in Section 6.

Multi-PDSCH/PUSCH scheduling

[Closed] 2-TB transmission

| Company | Views |
| --- | --- |
| [1] Futurewei | Proposal 1. Prefer that TB-disabling (if supported) applies to all scheduled TBs, and the need of further optimization over the Rel-15 mechanism can be discussed. |
| [4] vivo | Proposal 6: Regarding TB disabling for multi-PDSCH scheduling, when two codeword transmission is configured, for a DCI format scheduling more than one PDSCH, a given TB can be disabled for each scheduled PDSCH individually, by setting IMCS = 26 and the 1-bit RV for a scheduled PDSCH to a predefined value, e.g. ‘1’, to indicated the given TB for the scheduled PDSCH is disabled. |
| [6] CATT | Proposal 7: Both All-PDSCHs based and per-PDSCH based TB disable/enable mechanisms are supported, it depends on gNB configuration. Proposal 8: For the scheme 1 on TB disable, to enable flexible gNB scheduling, the following points can be considered: - Only all M of RV bit(s) are set to 1 (e.g. 11111111) represent the TB disable, even if the number of scheduled PDSCH is less than M. M is maximum number of PDSCHs can be scheduled configured by TDRA parameter. - If M PDSCH(s) are scheduled, and two TBs are set as MCS=26/RV=all “1”, UE assume that only TB1 is disabled. M is maximum number of PDSCHs can be scheduled configured by TDRA parameter. |
| [7] Nokia | Proposal 1: The combination MCS=26 and ‘1’ for RV bit fields for all PDSCHs associated with the TB is used to indicate the second TB is disabled. |
| [9] NTT DOCOMO | Proposal 2: When multiple PDSCHs are scheduled, dynamic 2-TB disabling/enabling is determined for each PDSCH separately. For each PDSCH, the corresponding 1 bit RV field is used. If IMCS=26 and rvid=2, 2-TB transmission is disabled for the PDSCH. Otherwise, 2-TB transmission is enabled for the PDSCH. Otherwise, 2-TB transmission is enabled for the PDSCH. Proposal 3: For antenna port field indication by multi-PDSCH scheduling DCI, two antenna port(s) fields are included in multi-PDSCH scheduling DCI. One antenna port field is applied to PDSCHs with only one codeword enabled, while the other antenna port field is applied to PDSCHs with two codeword enabled. |
| [12] Qualcomm | Proposal 8: To indicate that the second TB is disabled for a certain DCI that schedules multiple PDSCHs, use a combination of MCS and rvid such that rvid bit of PDCSH i-1 is the complement of the one of PDSCH i for i=1 : number of maximum PDSCHs -1. |
| [16] Apple | Proposal 9: RAN1 should update the RV design to support the TB-disabling mechanism for multi-PDSCH with a single scheduling DCI |
| [20] LG Electronics | Proposal #8: Consider one of the following methods to disable one of 2 TBs if 2-TB is enabled and more than one PDSCH is scheduled by multi-PDSCH scheduling DCI. - Method 1: Set all ‘1’s for RV bits corresponding to the TB of all scheduled PDSCHs and set MCS=26. - Method 2: Set ‘1’ for RV bit corresponding to the TB of a PDSCH and set MCS=26 (e.g., by reinterpreting value ‘1’ for RV field as RV index #1 when multiple PDSCHs are scheduled and MCS=26). |

- Issue 2.1) TB-disabling mechanism:

Motivation:

- In Rel-16, if 2-TB transmission is enabled, a TB signalled with MCS=26 and rvid =1 is disabled.

- In Rel-17, if DCI format 1_1 schedules more than one PDSCH, one-bit RV field can indicate rvid =0 or 2 and cannot indicate rvid =1, which disallows TB-disabling mechanism.

Company views on TB-disabling mechanism:

- Alt 1: TB-disabling for all of PDSCHs scheduled by a single DCI

  - Supported by Futurewei, CATT, Nokia, Qualcomm, LG Electronics

  - Alt 1-1: Set all ‘1’s to RV bits

    - Supported by CATT, Nokia, LG Electronics

  - Alt 1-2: rvid bit of PDCSH i-1 is the complement of the one of PDSCH i for i=1 : number of maximum PDSCHs -1 (e.g., 101010…)

    - Supported by Qualcomm

- Alt 2: TB-disabling individually for each of PDSCHs scheduled by a single DCI

  - Supported by vivo, NTT DOCOMO, Apple, LG Electronics,CATT

  - NTT DOCOMO: If Alt 2 is adopted, two DCI fields for antenna port(s) indication can be needed such that one is for 2-TB PDSCH and the other is for 1-TB PDSCH.

Companies are encouraged to provide views on the above alternatives (or any other alternatives) to disable a TB.

| Company | Views |
| --- | --- |
| Qualcomm | We support Alt1-2 as mentioned in our contribution, this is unlikely rvid vector to be used. Unlike, the single PDSCH DCI, for multi-PDSCH DCI, rvid can only take two values, if we scheduled a 8 PDSCHs via DCI with MCS 26 and rvid 00000000, if a retransmission is needed, a reasonable choice is use different rvid 11111111 |
| vivo | We support Alt 2 with more flexibility. We are also fine with Alt 1-1 if majority supports |
| Panasonic | Although Alt. 2 provides fully flexibility of TB-disabling, we do not see a strong motivation for it. This is because gNB decides how many actual TBs are scheduled by a DCI, i.e., it is not clear why gNB plans to schedule N TBs, then to disable at least one of them, especially for short slot duration of high SCSs. Therefore, we support direction of Alt. 1. Our preference is Alt. 1-1. |
| DOCOMO | We support Alt 2. It is more flexible considering that some PDSCHs may be used for retransmission, while some other PDSCHs are used for initial transmission. |
| CATT | We think both alt-1 and alt2 can be supported. gNB configuration is used for the tradeoff. |
| Intel | We prefer Alt. 1-1 as this is simple extension of existing mechanism. |
| Lenovo, Motorola Mobility | We support Alt 1-1 |
| Fujitsu | We slightly prefer Alt 1, considering that the flexibility of Alt 2 may be at the cost of more consequent issues to be resolved. |
| Samsung | We prefer Alt 1. Considering symbol durations and slot durations for 480/960kHz SCS, all PDSCHs scheduled by a single DCI may experience similar wireless channels so that individual TB-disabling would not be beneficial. |
| Nokia, NSB | We support Alt 1-1 |
| Apple | We support Alt-2 for more flexibility. |
| Futurewei | We prefer Alt-1. |
| ZTE, Sanechips | We support Alt 1-1 for simplicity. |
| MediaTek | We support Alt1-1 |
| Ericsson | We support Alt1-1. We don't think the flexibility is needed of having different # of TBs for different PDSCHs scheduled by the same DCI. |
| OPPO | We support Alt 1-1. |
| Huawei, HiSilicon | Alt 1-1 is preferred. |
| Moderator | - Alt 1: TB-disabling for all of PDSCHs scheduled by a single DCI - Supported by Futurewei, CATT, Nokia, Qualcomm, LG Electronics, vivo (acceptable), Panasonic, Intel, Lenovo, Fujitsu, Samsung, ZTE, MediaTek, Ericsson, OPPO, Huawei - Alt 1-1: Set all ‘1’s to RV bits - Supported by CATT, Nokia, LG Electronics, vivo (acceptable), Panasonic, Intel, Lenovo, ZTE, MediaTek, Ericsson, OPPO, Huawei - Alt 1-2: rvid bit of PDCSH i-1 is the complement of the one of PDSCH i for i=1 : number of maximum PDSCHs -1 (e.g., 101010…) - Supported by Qualcomm - Alt 2: TB-disabling individually for each of PDSCHs scheduled by a single DCI - Supported by vivo, NTT DOCOMO, Apple, LG Electronics, CATT - Configurable between Alt 1 and Alt 2 - Supported by CATT Given that clear majority companies support Alt 1 (compared to Alt 2) and Alt 1-1 (compared to Alt 1-2), the following proposal can be taken based on Alt 1-1. |

Proposal #2.1 (TB-disabling):

- If the higher layer parameter maxNrofCodeWordsScheduledByDCI indicates that two codeword transmission is enabled and more than one PDSCH is scheduled by a multi-PDSCH scheduling DCI,

  - One of the two transport blocks of all scheduled PDSCHs is disabled by the DCI if IMCS = 26 and if RV bit(s) is(are) set to ‘1’ for the corresponding transport block of all scheduled PDSCHs.

Companies are encouraged to provide views on Proposal #2.1.

| Company | Views |
| --- | --- |

On 1/19 GTW session, the following agreement was made:

- Agreement:

- If the higher layer parameter maxNrofCodeWordsScheduledByDCI indicates that two codeword transmission is enabled and more than one PDSCH is scheduled by a multi-PDSCH scheduling DCI,

  - Either the first or the second transport block of all scheduled PDSCHs is disabled by the DCI if IMCS = 26 and if RV bits are set to ‘1’ for the corresponding transport block of all scheduled PDSCHs (i.e. irrespective of whether this is a valid PDSCH).

Out-of-order handling

| Company | Views |
| --- | --- |
| [2] Huawei | Proposal 6: UE does not expect any of the scheduled/SPS PDSCHs and the resource for the HARQ-ACK transmission to lead to out-of-order scheduling. |
| [3] InterDigital | Proposal 2: For the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, UE doesn’t expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCI lead to out-of-order scheduling. Observation 1: For the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, any further specification is not needed. As per Rel-16, it is considered as an out-of-order scheduling and not supported. Proposal 3: A UE doesn’t expect the case of two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans where span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV. |
| [4] vivo | Observation 1: The case where two DCIs, each of which schedules a multi-slot PDSCH (or multi-slot PUSCH), end in the same symbol but the two scheduled multi-slot PDSCHs (or multi-slot PUSCHs) have overlapping spans, where the span is defined from the beginning of the first repetition till the end of the last repetition for a PDSCH/PUSCH, is allowed in Rel-15/16. Proposal 4: For multi-PDSCH/PUSCH scheduling, DCI-to-PDSCH/PUSCH out-of-order scheduling is defined as NR Rel-15/16 without any exception. Proposal 5: For multi-PDSCH scheduling, UE does not expect any of the scheduled PDSCH(s)/SPS PDSCH and the PUCCH resource for the HARQ-ACK transmission to lead to out-of-order HARQ-ACK reporting. |
| [8] Samsung | Proposal 8: For single PDSCH (or PUSCH) scheduling DCIs and multi-PDSCH (or multi-PUSCH) scheduling DCIs, UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCIs to lead to out-of-order scheduling. Observation 1: SPS PDSCH reception has large scheduling restriction on multi-PDSCH scheduling. Proposal 9: UE is not expected to receive a SPS PDSCH if the SPS PDSCH is configured to be received between a PDCCH with a DCI scheduling multiple PDSCHs and the last PDSCH scheduled by the DCI. |
| [9] NTT DOCOMO | Proposal 1: The following two cases are OoO scheduling, and should not be allowed: - the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH). - the case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV. |
| [10] ZTE | Proposal 1: It is not recommended to introduce additional specification impact to handle the two cases listed in FFS except existing OOO rule. |
| [11] Panasonic | Proposal 3: For the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, UE does not expect any of the scheduled PDSCHs (or PUSCHs) to lead to out-of-order scheduling. Proposal 4: For the case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol, but two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans, UE does not expect any of the scheduled PDSCHs (or PUSCHs) to lead to out-of-order scheduling. Proposal 5: For multi-PDSCH scheduling, UE does not expect any of the scheduled/SPS PDSCHs and the resource for the HARQ-ACK transmission lead to out-of-order scheduling. |
| [12] Qualcomm | Proposal 10: The UE does not expect to be scheduled with two DCIs that schedule DL (UL) data allocations with overlapping spans, where the span of the allocations scheduled by one DCI is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV by the same DCI. |
| [13] OPPO | Proposal 1: Support scheduling of the following case: - One multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH), and the single-PDSCH (or single-PUSCH) is transmitted at least later than the first PDSCH (or PUSCH) of the multi-PDSCH (or multi-PUSCH). Proposal 2: Do not specify the following case: - Two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV. |
| [14] Intel | Proposal 4 For two PDCCHs and the associated PDSCH(s) and PUSCH(s), if at least one PDCCH is scheduling multiple PDSCHs/PUSCHs, referring to Figure 1, - Case A/B/C/D are invalid; - The existing specification should be updated to reflect that Case A/C are invalid - Agree on the TP 2 on OOO handling between two PDCCHs and the associated PDSCH(s) and PUSCH(s) For two sets of PDSCHs and associated PUCCH, if at least one set of PDSCHs is of multiple PDSCHs/PUSCHs, referring to Figure 2, - Case E/F are valid, and Case G/H is invalid. - The existing specification is sufficient for cases E/F/G/H. - No TP is needed on OOO handling between two sets of PDSCHs and associated PUCCH. ------------------------------ TP#2: TS 38.214 ----------------------------------- *** Unchanged text is omitted *** - 5.1 UE procedure for receiving the physical downlink shared channel *** Unchanged text is omitted *** For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start receiving a first PDSCH starting in symbol j by a PDCCH ending in symbol i, the UE is not expected to be scheduled to receive a PDSCH starting earlier than the end of the first PDSCH with a PDCCH that ends later than symbol i. In a given scheduled cell, if the UE is scheduled to receive multiple PDSCHs, with the first PDSCH starting in symbol j and the last PDSCH ending in symbol k, by a first PDCCH ending in symbol i, the UE is not expected to be scheduled to receive a PDSCH starting no earlier than symbol j and no later than symbol k with a second PDCCH. *** Unchanged text is omitted *** - 6.1 UE procedure for transmitting the physical uplink shared channel *** Unchanged text is omitted *** Except for the case when a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet for the active BWP of a serving cell and PDCCHs that schedule two non-overlapping in time domain PUSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex, - for any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission starting in symbol j by a PDCCH ending in symbol i, the UE is not expected to be scheduled to transmit a PUSCH starting earlier than the end of the first PUSCH by a PDCCH that ends later than symbol i. - in a given scheduled cell, if the UE is scheduled to transmit multiple PUSCHs, with the first PUSCH starting in symbol j and the last PUSCH ending in symbol k, by a first PDCCH ending in symbol i, the UE is not expected to be scheduled to transmit a PUSCH starting no earlier than symbol j and no later than symbol k with a second PDCCH. *** Unchanged text is omitted ***. |
| [15] Ericsson | Proposal 2 Out-of-Order scheduling is allowed for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH). Proposal 3 Out-of-Order scheduling is NOT allowed for the case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV. |
| [16] Apple | Proposal 3: To simplify UE implementation, we propose that for the DCI-to-data out of order issue, the UE does not expect any out-of-order scheduling for the following cases: - for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH). - for the case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV. This applies also when one of two DCIs is single-PDSCH (or single-PUSCH) scheduling DCI Proposal 4: For the PDSCH-to-HARQ-ACK out-of-order issue, - for multi-PDSCH scheduling, the UE does not expect any of the scheduled/SPS PDSCHs and the resource for the HARQ-ACK transmission to lead to out-of-order scheduling in the case of a PDSCH scheduled by multi-PDSCH scheduling DCI and other unicast PDSCH scheduled by single-PDSCH scheduling DCI |
| [19] MediaTek | Proposal 4: For the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, UE doesn’t expect any of the scheduled PDSCHs(or PUSCHs) and the scheduling DCI lead to out-of-order scheduling. Proposal 5: For multi-PDSCH scheduling, UE doesn’t expect any of the scheduled PDSCHs and the resource for the HARQ-ACK transmission lead to out-of-order scheduling. |
| [20] LG Electronics | Proposal #4: For one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCI to lead to out-of-order scheduling. Proposal #5: For the case where two DCIs end at the same symbol but two DCIs have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV, UE drops the PDSCHs scheduled by one of the two DCIs in the overlapping duration. |
| [22] WILUS | Proposal 1: We propose to support the followings: - A UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCI to lead to out-of-order scheduling, also for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH) - The case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV, is considered as out-of-order scheduling and is not expected by UE. - This applies also when one of two DCIs is single-PDSCH (or single-PUSCH) scheduling DCI. |

- Issue 2.2-1) DCI-to-data out-of-order issue:

TS 38.214 For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start receiving a first PDSCH starting in symbol j by a PDCCH ending in symbol i, the UE is not expected to be scheduled to receive a PDSCH starting earlier than the end of the first PDSCH with a PDCCH that ends later than symbol i. … For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission starting in symbol j by a PDCCH ending in symbol i, the UE is not expected to be scheduled to transmit a PUSCH starting earlier than the end of the first PUSCH by a PDCCH that ends later than symbol i.

Agreement: (RAN1#106bis-e)

For two multi-PDSCH (or two multi-PUSCH) scheduling DCIs, UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCI to lead to out-of-order scheduling.

- FFS: whether to allow OOO scheduling for the following two cases:

  - for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH)

  - for the case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV

- Note: The above FFS aspect applies only to multi-PDSCH and multi-PUSCH scheduling with single DCI

Company views on DCI-to-data out-of-order issue:

- For the first case of above highlighted FFS,

  - Considered as OOO scheduling: Huawei, InterDigital, vivo, Samsung, NTT DOCOMO, ZTE, Panasonic, Apple, MediaTek, LG Electronics, WILUS,CATT

  - Can be allowed: OPPO (if single PXSCH DCI is transmitted later than the first PXSCH scheduled by multi-PXSCH DCI), Ericsson

- For the second case of above highlighted FFS,

  - Considered as OOO scheduling: InterDigital, NTT DOCOMO, Intel, Ericsson, Apple, WILUS,CATT

  - Can be allowed: vivo, ZTE

[Moderator’s note] Considering the majority view, the following proposal can be made.

Proposal #2.2-1 (DCI-to-data OOO):

- UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCI to lead to out-of-order scheduling, also for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH).

  - This may not have specification impact.

- The case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV, is considered as out-of-order scheduling and is not expected by UE.

  - This applies also when one of two DCIs is single-PDSCH (or single-PUSCH) scheduling DCI.

Companies are encouraged to provide views on Proposal #2.2-1.

| Company | Views |
| --- | --- |
| Xiaomi | Agree with the proposal in general. |
| Qualcomm | We agree with the proposal |
| InterDigital | We agree with the proposal. |
| vivo | We agree with the first bullet of the proposal. We don’t agree with the second bullet. Regarding the second bullet, in our understanding, for Rel-15/16 PDSCH/PUSCH repetition operation with pdsch-AggregationFactor/pusch-AggregationFactor or repetitionNumber-r16/numberOfRepetitions-r16, overlapping spans is allowed, where the span can be regarded as from the beginning of the first repetition till the end of the last repetition belonging to a given PDSCH/PUSCH with repetitions, as long as any two repetitions for the involved two PDSCHs/PUSCHs don’t overlap each other. Based on this understanding, we don’t see strong motivation to regard the case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but have overlapping spans as out-of-order scheduling. We prefer to have aligned behaviour between Rel-15/16 and Rel-17 unless there is strong motivation/significant benefit. |
| Panasonic | Support proposal #2.2-1 |
| DOCOMO | Support Proposal #2.2-1. |
| CATT | We agree with the proposal. |
| Intel | We support the FL proposal |
| Lenovo, Motorola Mobility | We agree with Proposal #2.2-1 |
| Fujitsu | We are generally fine with the proposal. But we would like to suggest the following modification for the first bullet. “where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one valid PDSCH (or PUSCH).” |
| Samsung | We agree with the intention of this proposal. But, there is still a pending issue whether configured SLIV or valid SLIV is used for OoO. So, we suggest to discuss Issue 2.4-2 first and, if needed, add the following sentence for the proposal #2.2-1: It is separately discussed that the scheduled PDSCHs (or PUSCHs) is based on configured SLIV or valid SLIV. |
| Nokia, NSB | We support Moderator’s proposal |
| Apple | We support the proposal. |
| Futurewei | We support Proposal #2.2-1. |
| ZTE, Sanechips | We agree with the proposal. |
| MediaTek | We support the proposal |
| Ericsson | We understand that we are in the minority, so for the sake of progress, we can support Proposal #2.2-1. However, we don't agree to changing "scheduled" to "valid," as proposed by Fujitsu. Using "scheduled" is more in-line with current specifications, and we think is the simpler option. |
| OPPO | For sake of progress, we are fine with Proposal #2.2-1. |
| Huawei, HiSilicon | Support proposal #2.2-1. |
| Moderator | It is observed that all companies can accept the first bullet (thanks for being flexible!) and all but one company supports the second bullet. @ vivo, Could you accept this proposal? At least from my understanding, one difference between Rel-15/16 and this proposal is that the same TB is repeated for Rel-15/16 while individual TBs are transmitted for Rel-17, which may require somehow more complicated UE implementation. One remaining issue is to consider whether OOO rule is based on configured or valid SLIV, which is being discussed in Issue 2.4-2. Therefore, we can add a note as suggested from Samsung to clarify that “based on configured or valid SLIV” can be separately discussed. |

Proposal #2.2-1a (DCI-to-data OOO):

- UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCIs to lead to out-of-order scheduling, also for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH).

  - This may not have specification impact.

- The case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but these two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV, is considered as out-of-order scheduling and is not expected by UE.

  - This applies also when one of two DCIs is single-PDSCH (or single-PUSCH) scheduling DCI.

- Note: It is separately discussed whether the scheduled PDSCHs (or PUSCHs or SLIV) is based on configured SLIV or valid SLIV.

Companies are encouraged to provide views on Proposal #2.2-1a.

| Company | Views |
| --- | --- |
| Xiaomi | Support Proposal 2.2-1a |
| vivo | Thanks moderator for the explanation. I know the difference lies in that the TB is same or different. However, I don’t quite understand the extra implementation complexity brought by this difference. It is good that the supporting company can clarify this. Besides, there is still one question we are not clear on this proposal. According to moderator’s explanation, the following understanding applies to Rel-17: For two multi-slot PDSCH/PUSCH scheduling DCIs, overlapping span is allowed, i.e. not considered as out-of-order scheduling; For two multi-PDSCH/PUSCH scheduling DCIs, overlapping span is not allowed, i.e. considered as out-of-order scheduling. How about the case that one multi-PDSCH/PUSCH scheduling DCI and one multi-slot PDSCH/PUSCH? Is overlapping span is allowed for this case? |
| Moderator | @ vivo, How about the case that one multi-PDSCH/PUSCH scheduling DCI and one multi-slot PDSCH/PUSCH? Is overlapping span is allowed for this case? [Moderator’s note] In this case, overlapping span is not allowed according to the note “This applies also when one of two DCIs is single-PDSCH (or single-PUSCH) scheduling DCI” in the proposal. |
| vivo | We are not sure if the note saying “single-PDSCH (or single-PUSCH)” covers multi-slot PDSCH/PUSCH case. Actually, there is no span definition for multi-slot PDSCH/PUSCH. Namely, for multi-slot PDSCH/PUSCH, how to understand the span definition in the proposal, i.e. from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV? There may be two interpretations: Alt. 1: For multi-slot PDSCH/PUSCH, the span is from the beginning of the first repetition till the end of the last repetition; Alt. 2: For multi-slot PDSCH/PUSCH, the span is only the scheduled SLIV with the first repetition. Besides, we still can’t understand the motivation to have such a proposal. For example, there are the following two cases: Case 1: DCI 1 schedules multi-slot PDSCH (blue) + DCI 2 schedules single PDSCH (green) Case 2: DCI 1 schedules multi-slot PDSCH (blue) + DCI 2 schedules multi-PDSCH (green+light blue) According to current proposal, Case 1 is allowed and Case 2 is considered as out-of-order. However, what’s the extra complexity for Case 2 over Case 1? |
| Huawei,HiSilicon | Support the proposal #2.2-1a with some editorial changes in red for easy understanding - UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCIs to lead to out-of-order scheduling, also for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH). - This may not have specification impact. - The case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but these two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV, is considered as out-of-order scheduling and is not expected by UE. |
| ZTE, Sanechips | Support Proposal 2.2-1a and agree to separately discuss whether the scheduled PDSCHs are based on configured SLIV or valid SLIV |
| Moderator | @ Huawei, Two editorial changes are reflected now. @ vivo, I tend to agree with your arguments and now I’m getting confused on Rel-15/16 out-of-order rule especially for slot-aggregated PXSCH case. Perhaps, any of proponents of the second bullet of Proposal #2.2-1a can respond to the question raised by vivo. Otherwise, I will try to make a convergence only for the first FFS case, since it seems that all companies are OK with the first and third bullets. @ all, As I mentiond above, I would like to ask any of proponents for the second bullet to respond to the question raised by vivo. Otherwise, I will not pursue the second bullet in this meeting, since it seems that all companies are OK with the first and third bullets. |
| Intel | We don’t fully understand the figures of Case 1/2 from vivo. However, according to our understanding, both two cases are invalid schedulings. NR only supports PDSCH repetitions in consecutive slots. If Case 1/2 happens, it means gNB schedule a PDSCH repetition by DCI 1 and another single/multiple PDSCHs by DCI 2 in the same slot. However, it conflicts with a conclusion made in RAN1#96. Conclusion: - It is understood that if a UE is configured with pdsch-AggregationFactor, according to current spec, the UE is not expected to be scheduled to receive more than one unicast PDSCH per slot per carrier. - No spec update is necessary Having said above, our original understanding on ‘single-PDSCH (or single-PUSCH)’ in the note of the second bullet is not related to multi-slot PDSCH/PUSCH. If companies want to extend the second bullet to multi-slot PDSCH/PUSCH, we are also fine with it. If there is still a concern, we may add FFS for multi-slot PDSCH/PUSCH in the note. |
| Moderator | @ Intel, Thanks for sharing the previous discussions. It seems that we need further discussion on the second bullet especially for the case of multi-slot PDSCH/PUSCH. @ all, As I commented earlier, I would like to suggest focusing on non-controversial bullets at this stage, so the updated proposal can be made as follows. |

Proposed Conclusion #2.2-1b (DCI-to-data OOO):

- UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCIs to lead to out-of-order scheduling, also for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH).

  - This may not have specification impact.

- Note: It is separately discussed whether the scheduled PDSCHs (or PUSCHs or SLIV) is based on configured SLIV or valid SLIV.

Companies are encouraged to provide views on Proposed Conclusion #2.2-1b.

| Company | Views |
| --- | --- |
| Moderator | Based on comments fo far, this proposed conclusion seems acceptable to all companies. Please comment if you have a strong concern. |
| ZTE, Sanechips | We agree with the moderator’s proposal, first focus on non-controversial technical point. |
| DOCOMO | Support Conclusion #2.2-1b. |
| Ericsson | As we commented earlier, for the sake of progress we can agree to Proposed Conclusion #2.2-1b, and we can further discuss the controversial point next meeting. |
| Qualcomm | We prefer version 2.2-1a, there is very limited progress made by this proposal |
| Futurewei | We agree with the FL to first proceed with Proposed Conclusion #2.2-1b that does not have a controversial part, though the version 2.2-1a looks ok. |
| Moderator | @ Qualcomm, I agree that it is a little bit small progress but considering the remaining meeting time, that is the best we can do. I don’t think Qualcomm opposes to this proposed conclusion itself. @ all, This proposed conclusion seems stable, so it can be reported for email endorsement. |

During email discussion, the following conclusion was made:

- Conclusion:

- UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCIs to lead to out-of-order scheduling, also for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH).

  - This may not have specification impact.

- Note: It is separately discussed whether the scheduled PDSCHs (or PUSCHs or SLIV) is based on configured SLIV or valid SLIV.

- [Closed] Issue 2.2-2) PDSCH-to-HARQ-ACK out-of-order issue:

TS 38.214 In a given scheduled cell, the UE is not expected to receive a first PDSCH in slot i, with the corresponding HARQ-ACK assigned to be transmitted in slot j, and a second PDSCH starting later than the first PDSCH with its corresponding HARQ-ACK assigned to be transmitted in a slot before slot j.

Company views on PDSCH –to-HARQ-ACK out-of-order issue:

- For multi-PDSCH scheduling, UE does not expect any of the scheduled/SPS PDSCHs and the resource for the HARQ-ACK transmission to lead to out-of-order scheduling.

  - Supported by Huawei, vivo, Panasonic, Intel, Apple, MediaTek

- UE is not expected to receive a SPS PDSCH if the SPS PDSCH is configured to be received between a PDCCH with a DCI scheduling multiple PDSCHs and the last PDSCH scheduled by the DCI.

  - Supported by Samsung

[Moderator’s note] Considering the majority view, the following proposal can be made.

Proposed Conclusion #2.2-2 (PDSCH-to-HARQ-ACK OOO):

- UE does not expect any of the scheduled/SPS PDSCHs and the resource for the HARQ-ACK transmission to lead to out-of-order scheduling, for any scheduling DCIs (including multi-PDSCH scheduling DCI).

Companies are encouraged to provide views on Proposed Conclusion #2.2-2.

| Company | Views |
| --- | --- |
| Xiaomi | Agree with the proposal in general. |
| Qualcomm | We agree with the proposal |
| InterDigital | We agree with the proposal |
| vivo | Support the proposed conclusion. |
| Panasonic | Support the proposed conclusion #2.2-2. |
| DOCOMO | Support the conclusion. |
| CATT1 | We agree with the proposal |
| Intel | We support the FL proposal |
| Lenovo, Motorola Mobility | We agree with Proposal #2.2-2 |
| Fujitsu | We agree with the proposal. |
| Samsung | We have still a concern on the proposed conclusion. We think it is too restrictive for multi-PDSCH scheduling and not acceptable. For example, up to 8 SPS configurations can be activated in a cell and each SPS configuration has separate periodicity and HARQ-ACK timing. In this case, it is not clear how gNB schedules multi-PDSCH scheduling under OoO restriction. Our preference is to give higher priority to multi-PDSCH scheduling and drop SPS PDSCHs if it leads OoO scheduling. In addition, in current spec, “receive a PDSCH” refers to valid PDSCH, the wording “scheduled” in the proposed conclusion #2.2-2 is not clear to us. |
| Nokia, NSB | We support Moderator’s proposal |
| Apple | We are fine with the proposal. |
| Futurewei | We support Proposal #2.2-2. |
| ZTE, Sanechips | We agree with the proposal |
| MediaTek | We support the proposal |
| Ericsson | Support Proposed Conclusion #2.2-2 |
| OPPO | We support the conclusion. |
| Huawei, HiSilicon | We support the conclusion. |
| Moderator | @ Samsung, Even though 8 SPS configurations can be activated in FR2-2, still gNB can make UE cancel SPS PDSCH reception by scheduling PDSCH in a slot where SPS PDSCH is configured. That is, gNB can manage a level of restriction. Considering the majority view, could we move forward? To address Samsung’s comment, “scheduled” can be slightly revised accordingly, as follows. |

Proposed Conclusion #2.2-2a (PDSCH-to-HARQ-ACK OOO):

- UE does not expect any of the received PDSCHs (including SPS PDSCH) and the resource for the HARQ-ACK transmission to lead to out-of-order scheduling, for any scheduling DCIs (including multi-PDSCH scheduling DCI).

Companies are encouraged to provide views on Proposed Conclusion #2.2-2a.

| Company | Views |
| --- | --- |
| Xiaomi | Support Proposal 2.2-2a |
| vivo | Support Proposal 2.2-2a |
| Intel | Support Proposal 2.2-2a |
| Lenovo, Motorola Mobility | Support the Proposed Conclusion |
| Samsung | @FL It is unclear that “gNB can make UE cancel SPS PDSCH reception by scheduling PDSCH in a slot where SPS PDSCH is configured” Our understanding is gNB should schedule DG PDSCH overlapping with SPS PDSCH to cancel the SPS PDSCH so that it might bring large scheduling restriction on multi-PDSCH scheduling. That is, gNB is forced to use a SLIV depending on SPS PDSCH. Even though we still see the benefit of cancelling SPS PDSCH leading OoO, given the situation, we can accept the proposed conclusion. |
| Huawei, HiSilicon |  |
| ZTE, Sanechips | Support Proposal 2.2-2a |
| Futurewei | Support the Proposed Conclusion #2.2-2a. |
| Moderator | @ Samsung, I agree with your statements and many thanks for being flexible! @ all, I assume this proposal is acceptable to all (although Huawei’s response is blank ), so please comment if you have a concern. |
| Huawei, HiSilicon | Sorry for missing the content. We support the proposal. |
| Ericsson | Support Proposed Conclusion #2.2-2a |
| Futurewei | We support the Proposed Conclusion. |
| Moderator | This proposed conclusion seems stable, so it can be reported for email endorsement. |

During email discussion, the following conclusion was made:

- Conclusion:

- UE does not expect any of the received PDSCHs (including SPS PDSCH) and the resource for the HARQ-ACK transmission to lead to out-of-order scheduling, for any scheduling DCIs (including multi-PDSCH scheduling DCI).

[Closed] Maximum gap between PDSCHs/PUSCHs

| Company | Views |
| --- | --- |
| [1] Futurewei | Proposal 4. It is recommended to specify the maximum value of the gap between the first scheduled PxSCH and the last scheduled PxSCH for simpler UE implementation. |
| [2] Huawei | Proposal 1: Support proposed conclusion#2.7 in RAN1#107-e that for multi-PDSCH or multi-PUSCH scheduling DCI, the following maximum gap values are not specified and are up to the gNB implementation. - The maximum value of the gap between two consecutively scheduled PDSCHs or between two consecutively scheduled PUSCHs - The maximum value of the gap between the first scheduled PDSCH and the last scheduled PDSCH or between the first scheduled PUSCH and the last scheduled PUSCH. |
| [3] InterDigital | Proposal 1: The maximum value of the gap between the first scheduled PDSCH and the last scheduled PDSCH and the gap between the first scheduled PUSCH and the last scheduled PUSCH should be 8 X,X∈{2,3}. |
| [11] Panasonic | Proposal 1: For multi-PDSCH or multi-PUSCH scheduling DCI, not to specify the following maximum value of a gap other than the values of the scheduling offset K0 (or K2) - A maximum value of the gap between two consecutively scheduled PDSCHs or between two consecutively scheduled PUSCHs - A maximum value of the gap between the first scheduled PDSCH and the last scheduled PDSCH or between the first scheduled PUSCH and the last scheduled PUSCH. |
| [15] Ericsson | Proposal 1 Do not introduce constraints on maximum value of the gap between two consecutively scheduled PDSCHs/PUSCHs or maximum value of the gap between the first and the last scheduled PDSCH/PUSCH other than that inherently provided by the range of K0/K2 value. |
| [16] Apple | Proposal 2: On the maximum gap for PxSCH transmission: - The maximum gap between the first and last PxSCH transmissions should be selected based on the maximum values of k0 and k2 i.e., 128 slots. - The maximum gap between two consecutive transmissions, can be set to the maximum value between the first and the last transmission in a 2 PDSCH/PUSCH transmission i.e., 128 slots. |
| [19] MediaTek | Proposal 3: For multi-PDSCH scheduling, if M PDSCHs are scheduled by a DCI, the M PDSCHs should be contained within at most M consecutive slots |
| [20] LG Electronics | Observation #1: Adjustment of the gap between PDSCHs (or PUSCHs) for multi-PDSCH (or multi-PUSCH) scheduling DCI can be left up to network implementation. |

- Summary on the maximum gap between scheduled PDSCHs/PUSCHs:

Agreement: (RAN1#105-e)

For TDRA in a DCI that can schedule multiple PDSCHs (or PUSCHs),

- A row of the TDRA table can indicate PDSCHs (or PUSCHs) that are in consecutive or non-consecutive slots.

  - FFS: The maximum value of the gap between two consecutively scheduled PDSCHs or between two consecutively scheduled PUSCHs

  - FFS: The maximum value of the gap between the first scheduled PDSCH and the last scheduled PDSCH or between the first scheduled PUSCH and the last scheduled PUSCH

  - FFS: Details to introduce the gap between PDSCHs or between PUSCHs

Company views on the maximum gap between scheduled PDSCHs/PUSCHs:

- Between two consecutively scheduled PDSCHs or between two consecutively scheduled PUSCHs

  - No additional impact on specification: Huawei, Panasonic, Ericsson, LG Electronics,CATT

  - To be specified: Apple

- Between the first scheduled PDSCH and the last scheduled PDSCH or between the first scheduled PUSCH and the last scheduled PUSCH

  - No additional impact on specification: Huawei, Panasonic, Ericsson, LG Electronics, CATT

  - To be specified: Futurewei, InterDigital, Apple, MediaTek (M PDSCHs are confined within at most M consecutive slots)

[Moderator’s note] In general, company views are divided into two categories where one is to suggest specifying a certain value to restrict the maximum gap between PDSCHs or PUSCHs and the other is not to further specify the maximum gap between PDSCHs or PUSCHs. In addition, even for proponents suggesting to specify the maximum gap between shared channels, the exact values for the gap are not aligned. Therefore, since it seems hard to pick an agreeable value for each gap, it is proposed not to specify the gap between PXSCHs.

Proposed Conclusion #2.3 (Max gap):

- For multi-PDSCH or multi-PUSCH scheduling DCI, the following maximum value of a gap is not specified in Rel-17 and up to gNB scheduler.

  - The maximum value of the gap between two consecutively scheduled PDSCHs or between two consecutively scheduled PUSCHs

  - The maximum value of the gap between the first scheduled PDSCH and the last scheduled PDSCH or between the first scheduled PUSCH and the last scheduled PUSCH

Companies are encouraged to provide views on Proposed Conclusion #2.3.

| Company | Views |
| --- | --- |
| Xiaomi | Agree with the proposal. |
| Qualcomm | We can be okay with proposal given that there will be no overlapping spans as discussed in OoO discussion |
| InterDigital | As pointed out in our contribution, we prefer to restrict the maximum gap between two PDSCHs/PUSCHs due to (i) using the same MCS for all the PDSCHs/PUSCHs, (ii) to avoid HARQ process starvation, (iii) the reason for supporting non-consecutive PDSCHs/PUSCHs is to accommodate UL/DL switching and to this end having 2-3 slots are sufficient. |
| Vivo | We support the proposed conclusion |
| Panasonic | Support the proposed conclusion #2.3. |
| DOCOMO | Support the conclusion. |
| CATT | Agree with the proposal. |
| Intel | We are fine with the Proposed Conclusion #2.3 |
| Lenovo, Motorola Mobility | In our view, we think that the maximum gap between the first scheduled PDSCH/PUSCH and the last scheduled PDSCH/PUSCH should be specified to limit the overall duration of transmission in case of non-contiguous transmission. The gap between two consecutive transmissions need not be specified then However, if majority is to support the proposal, we would be okay. |
| Fujitsu | We agree with the proposal. Even if there is potential MCS/HARQ process issue in some cases, it can be avoided by gNB implementation. So, we do not see the necessity to introduce the restriction on gap. |
| Samsung | We agree with the proposal |
| Nokia, NSB | We support the Proposed Conclusion |
| Apple | We can support the proposal. |
| Futurewei | We can accept no specification but think there is merit to specify the maximum gap. |
| ZTE, Sanechips | We support the proposal conclusion. |
| MediaTek | We don’t support the proposal. With large gap between PDSCHs, the scheduling can be impacted by no out-of-order and the HARQ ID starvation issue is elevated. |
| Ericsson | Support Proposed Conclusion #2.3. We think that the value range for K0 and K2 inherently sets an upper limit on scheduling gaps, and this is sufficient. Agree with the comment from Fujitsu on gNB implementation. |
| OPPO | We support the conclusion. |
| Huawei, HiSilicon | The configurable range of K0 and K2 already set the restriction of the gap. No additional restriction on the gap is necessary. |
| Moderator | - Supported by Xiaomi, Qualcomm, vivo, Panasonic, NTT DOCOMO, CATT, Intel, Lenovo (acceptable), Fujitsu, Samsung, Nokia, Apple, Futurewei (acceptable), ZTE, Ericsson, OPPO, Huawei - Objected by InterDigital, MediaTek @ InterDigital, MediaTek, As commented earlier, if we decide to specify a limitation on interval between PXSCHs, consequent discussion is needed to determine a specific value. In addition, as many companies pointed out, OOO scheduling or HARQ starvation issue can be left up to gNB’s implementation. In that sense, would it be possible to agree on Proposed Conclusion #2.3? |

During email discussion, the following conclusion was made:

- Conclusion:

For multi-PDSCH or multi-PUSCH scheduling DCI, the following maximum value of a gap is not specified in Rel-17 and up to gNB scheduler.

- The maximum value of the gap between two consecutively scheduled PDSCHs or between two consecutively scheduled PUSCHs

- The maximum value of the gap between the first scheduled PDSCH and the last scheduled PDSCH or between the first scheduled PUSCH and the last scheduled PUSCH

Handling of collision with semi-static DL/UL/flexible symbols

| Company | Views |
| --- | --- |
| [1] Futurewei | Proposal 2. For multiple PDSCH/PUSCH, the NDI/RV fields are based the maximum number of schedulable SLIVs; for RV the bit-width between 1 bit and 2 bits is based on the maximum number of schedulable PUSCH; for the CSI-request, the number M needs to be determined based on the number a valid SLIVs; whether CBG field is determined based on number of configured SLIVs or valid SLIVs is not relevant; valid SLIVs should be used for OOO scheduling. Proposal 5. On handling of collision between PUSCH and CORESET#0 for Rel-17 multiple PUSCHs scheduled by a single DCI, update the agreement by removing [or symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set]. |
| [5] Fujitsu | Proposal 3: For a first DCI scheduling multiple PDSCHs and providing an inapplicable value of k1 in its PDSCH-to-HARQ_feedback timing indicator filed, to multiplex the corresponding HARQ-ACK information in a PUCCH or PUSCH in a slot indicated by the PDSCH-to-HARQ_feedback timing indicator filed in a second DCI, only the valid PDSCHs scheduled by the first DCI are considered for definition of the corresponding timeline requirements. |
| [6] CATT | Proposal 5: NDI/RV fields for both valid and invalid PXSCHs are present in multi-PXSCH scheduling DCI. Proposal 6: For out-of-order scheduling, the rule for OOO case is determined based on valid SLIV(s) |
| [9] NTT DOCOMO | Proposal 4: If multiple PUSCHs are scheduled by single DCI, and there is at least one PUSCH collides with semi-static DL symbol, and/or symbol configured for SSB or CORESET#0 reception, - OoO scheduling limitation is based on valid PUSCHs. - If CBG based transmission is configured, CBGTI field is present in DCI for the case when multiple PUSCHs are scheduled but only one PUSCH is valid. - A-CSI reporting triggered by multi-PUSCH scheduling DCI is based on valid PUSCHs. When the A-CSI triggering DCI schedules N valid PUSCHs, the PUSCH that carries the aperiodic CSI feedback is N-th valid PUSCH for N <= 2, or (N-1)-th valid PUSCH for N > 2. - When timeline is satisfied, the CG PUSCH overlapping with the cancelled DG PUSCH can be transmitted. - When timeline is satisfied, the CG PUSCH with same HARQ process ID as the cancelled DG PUSCH can be transmitted. Proposal 5: If multiple PDSCHs are scheduled by single DCI, and there is at least one PDSCH collides with semi-static UL symbol, - OoO scheduling limitation is based on valid PDSCHs. - If CBG based transmission is configured, CBGTI/CBGFI fields are present in DCI for the case when multiple PDSCHs are scheduled but only one PDSCH is valid. - DCI scheduling multiple PDSCHs but with only one valid PDSCH is included in the first sub-codebook. - When timeline is satisfied, the SPS PDSCH overlapping with the cancelled DG PDSCH can be received. |
| [10] ZTE | Proposal 2: Unnecessary optimization should not be introduced for “scheduled PXSCH”. - NDI/RV/CBGTI field is determine based on the number of configured SLIVs. - gNB should guarantee the assigned PUSCH carrying the A-CSI is valid. - Only valid PXSCH should be considered in out-of-order scheduling. |
| [12] Qualcomm | Proposal 7: In the case of multi-PDSCH scheduling via a single DCI with ‘tdmSchemeA’, consider one of the following options to handle the overlap with semi-static UL symbols - Option 1: If one of the repetitions of the PDSCH collides with semi-static UL symbols, the corresponding PDSCH is considered as not valid - Option 2: If the first repetition of the PDSCH collides with semi-static UL symbols, the corresponding PDSCH is considered as not valid - On the other hand, if only the second repetition of the PDSCH collides with semi-static UL symbol, the PDSCH is still considered valid Proposal 9: For a single DCI that schedules multi-PDSCH/PUSCH, the NDI/ RV should be ehaviour per SLIV, i.e., a single bit will be assumed for each SLIV in NDI or RV vector even if the corresponding PDSCH/PUSCH is not valid. |
| [13] OPPO | Proposal 6: Clarify whether one PDSCH/PUSCH of the multi-PDSCH/PUSCH scheduled by a single DCI can be cancelled by a dynamic indication. |
| [14] Intel | Proposal 1 - If a PUSCH is collided with symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, the HARQ process number increment is not skipped for the PUSCH. - No TP is needed for HARQ process number increment for invalid PUSCH. Proposal 5: The following ehaviour should be defined based on the configured SLIVs of a TDRA row, - NDI/RV field in the DCI format - A-CSI multiplexing on the multiple PUSCHs scheduled by a DCI - OOO handling - No TP is needed as operation based on configured SLIVs is the default behavior |
| [20] LG Electronics | Proposal #1: If a PDSCH among multiple PDSCHs that are scheduled by a single DCI is collided with uplink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, NDI/RV fields corresponding to the PDSCH are absent in the DCI. Proposal #2: If a PUSCH among multiple PUSCHs that are scheduled by a single DCI is collided with downlink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or collided with SSB symbols indicated by ssb-PositionsInBurst, NDI/RV fields corresponding to the PUSCH are absent in the DCI. Proposal #3: Considering that M-th or (M-1)-th scheduled PDSCH (which would carry aperiodic CSI report as per previous agreement) can be cancelled due to the collision with semi-static DL symbols or SSB, when the DCI schedules M PUSCHs and K (<=M) PUSCHs are actually transmitted, the PUSCH that carries the aperiodic CSI feedback is K-th transmitted PUSCH for K <= 2, or (K-1)-th transmitted PUSCH for K > 2. Proposal #6: Do not consider any invalid PDSCH (which is collided with semi-static UL symbol(s)) to check out-of-order scheduling. |
| [22] WILUS | Proposal 2: If one of multiple PUSCHs is collided with symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, the HARQ process number increment is skipped for the PUSCH. |

- [Closed] Issue 2.4-1) How to handle collision between PUSCH and CORESET#0:

Agreement: (RAN1#106bis-e)

For multiple PDSCHs (or PUSCHs) scheduled by a single DCI,

- Rel-15/16 behavior that is described in TS 38.213 Clauses 11 and 11.1 for a PDSCH (or PUSCH) indicated by DCI also applies for multiple PDSCHs (or PUSCHs) schedule by a single DCI.

- If one of multiple PDSCHs (or PUSCHs) scheduled by the DCI collides with a flexible symbol (indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated),

  - If that PUSCH is collided with SSB symbols indicated by ssb-PositionsInBurst [or symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set], the HARQ process number increment is skipped for the PUSCH.

  - Otherwise, the HARQ process number increment is not skipped for that PDSCH (or PUSCH).

Company views on highlighted part above:

- If that PUSCH is collided with SSB symbols indicated by ssb-PositionsInBurst or symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, the HARQ process number increment is skipped for the PUSCH.

  - Supported by NTT DOCOMO, WILUS

  - Objected by Futurewei, Intel

[Moderator’s note] Given a small number of inputs, it is encouraged for companies to provide views on the above proposals, if any.

| Company | Views |
| --- | --- |
| Xiaomi | Agree with the highlighted part |
| Qualcomm | We don’t believe PUSCH should be cancelled if it is colliding with type0 PDCCH monitoring occasion, given there can be two occasions for the SSB and the gNB may choose one of them to use |
| vivo | Agree with Qualcomm. Besides, there is no such rule in NR Rel-15 if a PUSCH collides with CORESET#0 symbol. |
| DOCOMO | Support the highlighted part. In Rel-16 PUSCH repetition type B, symbols indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set are equally handled as semi-static Dlsymbols. Therefore, we think similar principle can be followed. |
| CATT | Agree with the highlighted part. |
| Intel | We do not support to consider CORESET0 with Type0-PDCCH CSS set for HPN determination. Based on Rel-15/16 spec as captured below, NB scheduler needs to ensure that there is no collision between scheduled PUSCH, and flexible symbols indicated for CORESET with Type0-PDCCH CSS set. In this case, UE can still transmit the PUSCH on the flexible symbols which are indicated for CORESET with Type0-PDCCH CSS set. We do not need to change existing behavior. |
| Fujitsu | We could accept the highlighted part if it is the majority view. |
| Nokia, NSB | We object this proposal. It should be possible for gNB to prioritize PUSCH against PDCCH for SIB1 |
| Apple | This can be handled by gNB implementation as mentioned by multiple companies. |
| Futurewei | Agree that no collision can be ensured by gNB implementation. |
| ZTE, Sanechips | We support removing the part of highlighted in yellow. In the current spec, the UE does not expect the set of symbols to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated. Therefore, for multiple PUSCH scheduling by single DCI, it should be also left to the gNB scheduler implementation to resolve the possible collision between the scheduled PUSCH and flexible symbols indicated for CORESET with Type0-PDCCH CSS set. |
| Ericsson | We do not support specifying the highlighted part for HPN determination; any potential collisions can be avoided by gNB implementation. |
| OPPO | We think the highlighted part can be removed. |
| Huawei, HiSilicon | The highlighted part is not necessary. gNB can avoid such collision by correct scheduling. |
| Moderator | As for skipping HARQ process number increment of PUSCH collided with CORESET#0, - Supported by Xiaomi, NTT DOCOMO, CATT - Objected by Qualcomm, vivo, Intel, Nokia, Apple, Futurewei, ZTE, Ericsson, OPPO, Huawei Based on the majority view, it is suggested to deprioritize this issue in Rel-17, which implies that current specification can be kept as is and HARQ process number increment won’t be skipped for PUSCH collided with CORESET#0. Please provide comments if any concerns. |

- Issue 2.4-2) Clarification on whether “scheduled PXSCH” in previous agreements implies valid PXSCH or not:

Agreement: (RAN1#104-bis)

For a DCI that can schedule multiple PDSCHs,

- NDI for the 1st TB: This is signaled per PDSCH and applies to the first TB of each PDSCH

- RV for the 1st TB: This is signaled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the first TB of each PDSCH

Conclusion: (RAN1#105-e)

For a DCI that can schedule multiple PUSCHs,

- CSI-request: When the DCI schedules M PUSCHs, the PUSCH that carries the aperiodic CSI feedback is M-th scheduled PUSCH for M <= 2, or (M-1)-th scheduled PUSCH for M > 2.

Agreement: (RAN1#105-e)

- At least for 120 kHz SCS, for a DCI that can schedule multiple PUSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs,

  - If CBG-based (re)transmission is configured, CBGTI field is not present when more than one PUSCHs are scheduled, but is present when a single PUSCH is scheduled, as in Rel-16.

Agreement: (RAN1#106bis-e)

For two multi-PDSCH (or two multi-PUSCH) scheduling DCIs, UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCI to lead to out-of-order scheduling.

Agreement: (RAN1#106bis-e)

For a DCI that can schedule multiple PDSCHs, and if RRC parameter configures that two codeword transmission is enabled,

- NDI for the 2nd TB: This is ignalled per PDSCH and applies to the 2nd TB of each PDSCH

- RV for the 2nd TB: This is ignalled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the 2nd TB of each PDSCH

Company views on whether “scheduled PXSCH” in previous agreements implies valid PXSCH or not:

- Case 1: For NDI/RV, are NDI/RV fields for invalid PXSCHs present in multi-PXSCH scheduling DCI?

  - Based on configured SLIVs: Futurewei, CATT, ZTE, Qualcomm, Intel

  - Based on valid SLIVs: LG Electronics

- Case 2: For RV field, is the bit-width between 1 bit and 2 bits determined based on the number of configured SLIVs or valid SLIVs?

  - Based on configured SLIVs: Futurewei, ZTE, Qualcomm, Intel,CATT

  - Based on valid SLIVs

- Case 3: For CSI-request, is the number M determined based on the number of configured SLIVs or valid SLIVs?

  - Based on configured SLIVs: ZTE, Intel,CATT

  - Based on valid SLIVs: Futurewei, NTT DOCOMO, LG Electronics

- Case 4: For CBGTI field, is the presence of CBGTI field determined based on the number of configured SLIVs or valid SLIVs?

  - Based on configured SLIVs: ZTE,CATT

  - Based on valid SLIVs: NTT DOCOMO

- Case 5: For out-of-order scheduling, is the rule for OOO scheduling determined based on configured SLIVs or valid SLIVs?

  - Based on configured SLIVs: Intel

  - Based on valid SLIVs: Futurewei, CATT, NTT DOCOMO, ZTE, LG Electronics

- Case 6: For a first DCI scheduling multiple PDSCHs and providing an inapplicable value of k1 in its PDSCH-to-HARQ_feedback timing indicator filed, to multiplex the corresponding HARQ-ACK information in a PUCCH or PUSCH in a slot indicated by the PDSCH-to-HARQ_feedback timing indicator filed in a second DCI, only the valid PDSCHs scheduled by the first DCI are considered for definition of the corresponding timeline requirements.

  - Supported by Fujitsu

- Case 7: In the case of multi-PDSCH scheduling via a single DCI with ‘tdmSchemeA’, cancel both of two repeated PDSCHs if at least one of repeated PDSCHs collides with semi-static UL symbols or determine the validity rule for each of repeated PDSCHs

  - Supported by Qualcomm

Companies are encouraged to provide views on the above cases.

| Company | Views |
| --- | --- |
| Example | Case 1 (NDI/RV): Based on configured or valid SLIVs Case 2 (RV bit-width): Based on configured or valid SLIVs Case 3 (CSI-request): Based on configured or valid SLIVs Case 4 (CBGTI): Based on configured or valid SLIVs Case 5 (OOO): Based on configured or valid SLIVs Case 6 (NN-K1): Any views? Case 7 (tdmSchemeA): Any views? |
| Xiaomi | Case 1 (NDI/RV): Based on configured SLIVs Case 2 (RV bit-width): Based on configured SLIVs Case 3 (CSI-request): Based on configured SLIVs Case 4 (CBGTI): Based on configured SLIVs Case 5 (OOO): Based on valid SLIVs Case 6: based on valid SLIVs |
| Qualcomm | We support making the DCI related fields based on the configured SLIVs to simplify the DCI processing at the UE Case 1 (NDI/RV): Based on configured SLIVs Case 2 (RV bit-width): Based on configured SLIVs Case 3 (CSI-request): Based on configured SLIVs Case 4 (CBGTI): Based on configured SLIVs Case 5 (OOO): Based on valid SLIVs Case 6 (NN-K1): Based on valid SLIVs Case 7 (tdmSchemeA): As mentioned in our paper, if the first of the SLIV of a PDSCH is invalid, we should skip this the PDSCH and its repetition, while if the second SLIV (repetition) is invalid, we can either skip the repetition or the PDSCH and its repetition. |
| Vivo | Case 1 (NDI/RV): Based on configured SLIVs for simplicity. The signaling overhead is the same irrespective of which option is selected. Case 2 (RV bit-width): Based on configured SLIVs. The settings of DCI fields are only based on the indicated TDRA row where one or more SLIVs are configured, therefore the UE can decode a DCI format supporting multi-PDSCH/PUSCH scheduling with less processing time. Case 3 (CSI-request): Based on configured SLIVs. No additional rule will be introduced, and it is up to gNB implementation to ensure the PUSCH conveying a A-CSI is valid. Case 4 (CBGTI): Based on configured SLIVs. It is related to the above Case 2, and same rule(s) should be applied for all DCI fields. Case 5 (OOO): Based on valid SLIVs. In our opinion, an invalid PDSCH/PUSCH can be regarded as not scheduled by the gNB, and OoO rules are only applied to cases where PDSCH(s)/PUSCH(s) is(are) actually scheduled based on semi-static configuration(s)/indication(s). Case 6 (NN-K1): Based on valid SLIVs Case 7 (tdmSchemeA): from the two options provided by QC, option 2 is preferred, i.e., if the first repetition of the PDSCH collides with semi-static UL symbols, the corresponding PDSCH is considered as not valid. |
| DOCOMO | Case 1 (NDI/RV): Based on valid SLIVs Case 2 (RV bit-width): Based on valid SLIVs Case 3 (CSI-request): Based on valid SLIVs Case 4 (CBGTI): Based on valid SLIVs Case 5 (OOO): Based on valid SLIVs Case 6 (NN-K1): Based on valid SLIVs Case 7 (tdmSchemeA): Share similar view as Qualcomm that the simplest way is to skip the PDSCH if the first repetition is invalid. |
| CATT | Case 1 (NDI/RV): Based on configured SLIVs for simplicity. Case 2 (RV bit-width): Based on configured SLIVs. Case 3 (CSI-request): Based on configured SLIVs. Case 4 (CBGTI): Based on configured SLIVs. Case 5 (OOO): Based on valid SLIVs. The purpose of specification to define OOO scenario is that gNB shall avoid OOO case when gNB schedules one or more PDSCHs. For multi- PDSCHs/PUSCHs scheduling, when UE receives scheduling signals, UE assumes that OOO case rarely happens or does not occur at all. So it is reasonable that the rule for OOO scheduling is determined based on valid SLIVs. Case 6 (NN-K1): support the proposal Case 7 (tdmSchemeA): Ok with cancel both for simplicity. |
| Intel | We prefer to define a unified rule, i.e., always applying the configured SLIVs in Case 1-6 and potentially other cases not identified yet. - Case 1 (NDI/RV): Based on configured SLIVs - Case 2 (RV bit-width): Based on configured SLIVs - Case 3 (CSI-request): Based on configured SLIVs. gNB can guarantee the associated PUSCH is available CSI transmission. - Case 4 (CBGTI): Based on configured SLIVs for PUSCH scheduling. It is not needed for PDSCH scheduling - Case 5 (OOO): Based on configured SLIVs. An operation based on configured SLIVs is the most robust behavior and the default behavior. On the other hand, if interpretation based on ‘valid SLIVs’ is adopted, we are afraid exhausted checking on other related operations becomes necessary. This should be avoided in the maintenance phase. - Case 6 (NN-K1): Based on configured SLIVs - Case 7 (tdmSchemeA): It is preferred to first clarify whether M-TRP operation with tdmSchemeA is supported or not for multi-PDSCH scheduling. If it is supported, how to interpret the SLIVs of a TDRA row? For example, if N SLIVs is configured for a row, is it to schedule N TBs or N/2 TBs? |
| Fujitsu | Case 1 (NDI/RV): Based on configured SLIVs Case 2 (RV bit-width): Based on configured SLIVs Case 3 (CSI-request): Based on configured SLIVs Case 4 (CBGTI): Based on configured SLIVs Case 5 (OOO): Based on valid SLIVs Case 6 (NN-K1): Based on valid SLIVs Case 7 (tdmSchemeA): okey with the majority view |
| Samsung | Case 1 (NDI/RV): Based on configured SLIVs Case 2 (RV bit-width): Based on configured SLIVs Case 3 (CSI-request): Based on configured SLIVs Case 4 (CBGTI): Based on configured SLIVs Case 5 (OOO): Based on valid SLIVs Case 6 (NN-K1): Based on valid SLIVs Case 7 (tdmSchemeA): Based on valid SLIVs We would like to comment general principle. For any bit-field in DCI format, our preference is to use configured SLIVs since the payload size of DCI format is determined based on the maximum number of configured SLIVs. Thus, even though some bits for invalid SLIVs are excluded from a DCI field in a DCI format, but the payload size of the DCI format is not changed. Regarding the collision handling with other channels, we prefer a unified solution. The collision cases include the following - DG PDSCH overlaps with SPS PDSCH or CG PUSCH or semi-static PUCCH (SPS HARQ-ACK, SR, P/SP-CSI) on a same cell. - DG PUSCH overlaps with SPS PDSCH or CG PUSCH on a same cell. - DG PUSCH overlaps with PUCCH - OOO scheduling for both DL and UL - OOO HARQ for DL We propose: UE first resolves the collision of PDSCHs/PUSCHs and semi-static UL/DL symbols and then UE resolves the collision among PDSCHs, PUSCHs and PUCCHs |
| Nokia, NSB | Case 1 (NDI/RV): Based on configured SLIVs Case 2 (RV bit-width): Based on configured SLIVs Case 3 (CSI-request): Based on valid SLIVs Case 4 (CBGTI): Based on valid SLIVs Case 5 (OOO): Based on valid SLIVs Case 6 (NN-K1): Based on valid SLIVs Case 7 (tdmSchemeA): we have similar view with Intel, if this is the valid scenario. Since tdmSchemeA is intended for reliable transmission, skipping both are not comply with the main motivation. But, we are fine with the majority view. |
| Apple | Case 1 (NDI/RV): Based on configured SLIVs Case 2 (RV bit-width): Based on configured SLIVs Case 3 (CSI-request): Based on configured SLIVs Case 4 (CBGTI): Based on configured SLIVs Case 5 (OOO): Based on valid SLIVs Case 6 (NN-K1): Based on valid SLIVs Case 7 (tdmSchemeA): Agree with Intel |
| Futurewei | Case 1 (NDI/RV): Based on configured SLIVs Case 2 (RV bit-width): Based on configured SLIVs Case 3 (CSI-request): Based on configured SLIVs Case 4 (CBGTI): Based on configured SLIVs Case 5 (OOO): Based on valid SLIVs Case 6 (NN-K1): Based on valid SLIVs Case 7 (tdmSchemeA): Clarification needed as raised by company |
| ZTE, Sanechips | Case 1 (NDI/RV): Based on configured SLIVs Case 2 (RV bit-width): Based on configured SLIVs Case 3 (CSI-request): Based on configured SLIVs Case 4 (CBGTI): Based on configured SLIVs Case 5 (OOO): Based on valid SLIVs Case 6 (NN-K1): Based on valid SLIVs Case 7 (tdmSchemeA): Cancel both of two repeated PDSCHs if at least one of repeated PDSCHs collides with semi-static UL symbols considering self-decoding of PDSCH and simplicity. |
| MediaTek | Case 1 (NDI/RV): Based on configured SLIVs Case 2 (RV bit-width): Based on configured SLIVs Case 3 (CSI-request): Based on configured SLIVs Case 4 (CBGTI): Based on configured SLIVs Case 5 (OOO): Based on valid SLIVs to avoid the impact on Type-1 bundling operation Case 6 (NN-K1): Based on valid SLIVs Case 7 (tdmSchemeA): share the same view with Qualcomm |
| Ericsson | Case 1 (NDI/RV): Based on configured SLIVs Case 2 (RV bit-width): Based on configured SLIVs Case 3 (CSI-request): Based on valid SLIVs to enable more scheduling flexibility. Case 4 (CBGTI): Based on configured SLIVs Case 5 (OOO): Based on configured SLIVs. We share the same view as Intel. Case 6 (NN-K1): Based on configured SLIVs. Case 7 (tdmSchemeA): We think the simplest approach is to skip both repetitions if either overlaps semi-statically configured UL symbols. This corresponds to Qualcomm’s Option 1. Regarding Intel’s question on tdmSchemeA, “If N SLIVs is configured for a row, is it to schedule N TBs or N/2 TBs?” we don’t think this is relevant. Clearly it would correspond to N TBs, and the operation of tdmSchemeA for multi-PDSCH is transparent due to the following from 38.214 Section 5.1: - When two TCI states are indicated in a DCI and the UE is set to ‘tdmSchemeA’, the UE shall receive two PDSCH transmission occasions of the same TB with each TCI state associated to a PDSCH transmission occasion which has non-overlapping time domain resource allocation with respect to the other PDSCH transmission occasion and both PDSCH transmission occasions shall be received within a given slot as described in Clause 5.1.2.1. Hence, for each of the N SLIVs there are 2 repetitions in the same slot. Furthermore, the resource allocation for the repetitions is common to both as specified in 38.214 Section 5.1.2.1 (except the starting symbol for the 2nd occasion): When a UE is configured by the higher layer parameter repetitionScheme set to ‘tdmSchemeA’ and indicated DM-RS port(s) within one CDM group in the DCI field ‘Antenna Port(s)’, the number of PDSCH transmission occasions is derived by the number of TCI states indicated by the DCI field ‘Transmission Configuration Indication’ of the scheduling DCI. - If two TCI states are indicated by the DCI field ‘Transmission Configuration Indication’, the UE is expected to receive two PDSCH transmission occasions, where the first TCI state is applied to the first PDSCH transmission occasion and resource allocation in time domain for the first PDSCH transmission occasion follows Clause 5.1.2.1. The second TCI state is applied to the second PDSCH transmission occasion, and the second PDSCH transmission occasion shall have the same number of symbols as the first PDSCH transmission occasion. If the UE is configured by the higher layers with a value K in StartingSymbolOffsetK, it shall determine that the first symbol of the second PDSCH transmission occasion starts after K symbols from the last symbol of the first PDSCH transmission occasion. If the value K is not configured via the higher layer parameter StartingSymbolOffsetK, K = 0 shall be assumed by the UE. The UE is not expected to receive more than two PDSCH transmission layers for each PDSCH transmission occasion. For two PDSCH transmission occasions, the redundancy version to be applied is derived according to Table 5.1.2.1-2, where n=0, 1 applied respectively to the first and second TCI state. The UE expects the PDSCH mapping type indicated by DCI field ‘Time domain resource assignment’ to be mapping type B, and the indicated PDSCH mapping type is applied to both PDSCH transmission occasions. - Otherwise, the UE is expected to receive a single PDSCH transmission occasion, and the resource allocation in the time domain follows Clause 5.1.2.1. |
| OPPO | Case 1 (NDI/RV): Based on configured SLIVs Case 2 (RV bit-width): Based on configured SLIVs Case 3 (CSI-request): Based on valid SLIVs Case 4 (CBGTI): Based on configured SLIVs Case 5 (OOO): Based on valid SLIVs Case 6 (NN-K1): Based on configured SLIVs Case 7 (tdmSchemeA): We are fine with skip both repetitions for simplicity. |
| Huawei, HiSilicon | Case 1(NDI/RV): Configured SLIV. The overhead of NDI/RV depend on the maximum SLIV configured in TDRA table. Based on configured SLIV is straightforward without performance penalty. Case 2(RV bit): Configured SLIV. The DCI size is not able to change dynamically according to the validation of SLIV. Case 3(CSI request): Valid SLIV. The issue for configured SLIV is that CSI cannot be feedback if the M-1 SLIV is invalid. Case 4(CBGTI): Configured SLIV. Case 5(OOO): Valid SLIV. It has less restriction of scheduling caused by OOO. Case 6(NN-K1): Valid SLIV. It is one of the OOO in NRU, the behavior should be consistent with other OOO cases. Case 7(tdmSchemeA): For multi PDSCH/PUSCH scheduling, RAN1 had agreement in RAN1#104 that the case of “Single DCI to schedule N TBs (N>1) where a TB can be repeated over multiple slots (or mini-slots)” is not supported. Does it contradict with the agreement? |
| Moderator | Case 1 (NDI/RV) - Based on configured SLIVs: Xiaomi, Qualcomm, vivo, CATT, Intel, Fujitsu, Samsung, Nokia, Apple, Futurewei, ZTE, MediaTek, Ericsson, OPPO, Huawei - Based on valid SLIVs: NTT DOCOMO Case 2 (RV bit-width) - Based on configured SLIVs: Xiaomi, Qualcomm, vivo, CATT, Intel, Fujitsu, Samsung, Nokia, Apple, Futurewei, ZTE, MediaTek, Ericsson, OPPO, Huawei - Based on valid SLIVs: NTT DOCOMO Case 3 (CSI-request) - Based on configured SLIVs: Xiaomi, Qualcomm, vivo, CATT, Intel, Fujitsu, Samsung, Apple, Futurewei, ZTE, MediaTek, Ericsson - Based on valid SLIVs: NTT DOCOMO, Nokia, OPPO, Huawei Case 4 (CBGTI) - Based on configured SLIVs: Xiaomi, Qualcomm, vivo, CATT, Intel, Fujitsu, Samsung, Apple, Futurewei, ZTE, MediaTek, Ericsson, OPPO, Huawei - Based on valid SLIVs: NTT DOCOMO, Nokia Case 5 (OOO) - Based on configured SLIVs: Intel, Ericsson - Based on valid SLIVs: Xiaomi, Qualcomm, vivo, NTT DOCOMO, CATT, Fujitsu, Samsung, Nokia, Apple, Futurewei, ZTE, MediaTek, OPPO, Huawei Case 6 (NN-K1) - Based on configured SLIVs: Intel, Ericsson, OPPO - Based on valid SLIVs: Xiaomi, Qualcomm, vivo, NTT DOCOMO, CATT, Fujitsu, Samsung, Nokia, Apple, Futurewei, ZTE, MediaTek, Huawei Case 7 (tdmSchemeA) - Option 1 (cancel both of two repeated PDSCHs if at least one of repeated PDSCHs collides with semi-static UL symbols): Qualcomm, vivo, NTT DOCOMO, CATT, ZTE, Ericsson, OPPO - Option 2 (cancel one or both of two repeated PDSCHs depending on collision with semi-static UL symbols): Qualcomm, MediaTek @ Samsung, As to collision resolution step, it seems that the issue needs to be commonly resolved for all cases. In that sense, URLLC WI could be the right place to determine collision resolution step. @ Intel, My understanding is aligned with Ericsson’s, that is, tdm scheme A is applied for each of N SLIVs scheduled by a DCI. For example, if a DCI schedules PDSCH #A/#B at slot #1/#2, then repeated PDSCHs #A-1/#A-2 are received at slot #1 and repeated PDSCHs #B-1/#B-2 are received at slot #2. @ Huawei, Under 8.2.4 agenda item, we made the following agreement in RAN1#106bis-e and the corresponding RAN1 agreement made in RAN1#104 can be understood from single TRP perspective. Agreement: The working assumption in RAN1#106-e is confirmed with the following update: For multi-PDSCH scheduling for multi-TRPs, support a single DCI field ‘Transmission Configuration Indication’ as in Rel-16 TCI state indication mechanism for multi-TRPs - The single DCI field ‘Transmission Configuration Indication’ indicates one or two TCI states associated with a code point for single DCI based multi-TRP mechanism - When two TCI states are indicated, reuse Rel-16 association rules to apply the two TCI states for each PDSCH scheduled by a multi-PDSCH scheduling DCI - The single DCI field ‘Transmission Configuration Indication’ indicates only one TCI state associated with a code point for multi-DCI based multi-TRP mechanism - Reuse Rel-16 RRC configuration and MAC CE activation/deactivation methods for the one or two TCI states - FFS: Details of multiple TCI state association with multiple PDSCHs - Within the TDRA table for multi-PDSCH scheduling, the UE does not expect to be configured with the higher layer parameter repetitionNumber @ all, Majority view is marked as bold text. Please provide comments if there is a strong concern in direction with the majority view. |
| Intel | Case 5 (OOO) is timeline related. If valid SLIV is concluded, it should applies other timeline related behavior too. For example, the PUSCH preparation time needs to be defined as the delay between PDCCH and the first valid PUSCH, which means the following section in 38.214 should be updated. - 6.4 UE PUSCH preparation procedure time If the first uplink symbol in the PUSCH allocation for a transport block, including the DM-RS, as defined by the slot offset K2 and the start S and length L of the PUSCH allocation indicated by ‘Time domain resource assignment’ of the scheduling DCI and including the effect of the timing advance, is no earlier than at symbol L2, where L2 is defined as the next uplink symbol with its CP starting Tproc,2=max(N2+d2,1+d2)(2048+144)⋅κ2-μ⋅TC+Text+Tswitch,d2,2 after the end of the reception of the last symbol of the PDCCH carrying the DCI scheduling the PUSCH, then the UE shall transmit the transport block. Another example is the minimum PDCCH to PDSCH delay Npdsch which applies in cross-carrier scheduling with different SCS. If timeline checking is done with valid SLIVs, does it mean the delay between PDCCH and the first PDSCH (collided with UL symbols) can be < Npdsch? Not sure whether this is the intenteded ignalled for the supporting companies of using valid SLIVs. If valid SLIV is agreed for Case 5, we expect there will be a seriers of companion TPs/CRs in future meetings, which is not desired. For Case 7 (tdmSchemeA), thanks for explanation from moderator. We agree it is a most reasonable interpretation on SLIVs of TDRA, assuming mTRP ‘tdmSchemeA’ is supported in multi-PDSCH transmission. However, we found around 5 companies still have concerns whether mTRP ‘tdmSchemeA’ is allowed/conflicted with existing agreement or not. Therefore, it would be helpful to first clarify the support of mTRP ‘tdmSchemeA’ in multi-PDSCH scheduling |
| Samsung | We are fine with Case 1-6. For Case 7, what is a technical benefit and concern of option 1? Since two repeated PDSCHs carry the same TB, it would be beneficial to receive at least one “valid” repeated PDSCH such as Rel-15 PDSCH slot aggregation. To FL, we don’t think the issue should be discussed under URLLC WI, the issue is caused because of M-PDSCH/PUSCH scheduling and should be handled here. In fact, if we use valid PDSCH/PUSCH for collision handling, it means we check semi-static DL/UL conflicts first, we just prefer a unified solution for all related cases. |
| Huawei, HiSilicon | We observed there were 5 companies prefer “valid” for case 3. To our understanding, multiplexing CSI feedback based on valid SLIV can maximize the probability of CSI feedback feedback while introducing scheduling restriction due to potential collsion with DL. |
| Moderator | @ Samsung, What is the defined rule for CG-PUSCH in URLLC WI? Even for that case, after semi-static DL/UL conflict for CG-PUSCH is checked, then UCI is multiplexed to non-collided PUSCH? Again, I don’t think we need to make a somewhat different rule for multi-PXSCH scheduling case, from other cases. Please let me know if I’m missing something. @ all, - For Case 5 (OOO) (maybe also for Case 6), Intel raised a concern if we go with majority (based on valid SLIV) - For Case 7 (tdmSchemeA), Samsung raised a concern if we go with majority (based on Option 1) - For Case 3 (CSI-request), Huawei raised a concern if we go with majority (based on configured SLIV) Therefore, I suggest the following only for Cases 1/2/4 and we can discuss further other cases in the next meeting. |
| Samsung | Semi-static DL collision for CG PUSCH is discussing under Rel-17 URLLC WI, the details can be found in [107bis-e-R17-IIoT-URLLC-03] . For Rel-16 URLLC, in the last meeting, we made following agreement. Conclusion In the Rel-16 multiplexing/prioritization procedures described in TS 38.213 section 9, the UE is expected to apply the procedures to the PUSCH(s) for which a transport block is delivered by MAC, while the PUSCH(s) for which a transport block is not delivered is ignored. If a CG PUSCH collides with semi-static DL symbols, in our understanding MAC should not deliver a MAC PDU for the CG PUSCH. Thus, CG PUSCH is differently handled from DG PUSCH in URLLC. For multi-PUSCH scheduling, it cannot simply follow CG PUSCH or single DG PUSCH scheduling, so that separate discussion is necessary. |
| Moderator | @ Samsung, If I understood correctly, semi-static DL collusion rule for CG PUSCH is still under discussion and I don’t think we need a separate discussion here. For the second issue, I don’t see a critical problem if we follow Rel-16 MAC PDU generation procedure. Multi-PUSCH scheduling should be treated same as in DG PUSCH in Rel-16. Could you elaborate on what the problem is? |
| DOCOMO | Regarding Intel’s concern about case 5 (OoO), we understand that timeline can be relaxed if valid PUSCH is applied. But “can be relaxed” does not necessariliy mean “need to be relaxed”. In our understanding, the existing timeline still works. Regarding Huawei’s concern about case 3 (CSI request), we are not sure about what additional scheduling limitation in Huawei’s mind. Could you elaborate it more? Regarin case 7 (tdmSchemeA), we now have same feeling as Intel that it seems tdmSchemeA has conflict with previous agreements. Agreement (@RAN1#104-e): - For a UE and for a serving cell, scheduling multiple PDSCHs by single DL DCI and scheduling multiple PUSCHs by single UL DCI are supported. - Each PDSCH or PUSCH has individual/separate TB(s) and each PDSCH/PUSCH is confined within a slot. - FFS: The maximum number of PDSCHs or PUSCHs that can be scheduled with a single DCI - FFS: Whether multiple PDSCH scheduling applies to 120 kHz in addition to 480 and 960 kHz - At least for 120 kHz SCS, single-slot scheduling with slot-based monitoring will still be supported as specified in Rel-15/Rel-16 - The followings will not be considered in this WI. - Single DCI to schedule both PDSCH(s) and PUSCH(s) - Single DCI to schedule one or multiple TBs where any single TB can be mapped over multiple slots, where mapping is not by repetition - Single DCI to schedule N TBs (N>1) where a TB can be repeated over multiple slots (or mini-slots) |
| Samsung | @FL, As we clarified, URLLC WI does not include the case of M-PUSCH scheduling. We don’t think it is the common understanding that we would follow the rules for CG PUSCH. In addition, M-PDSCHs are not covered in URLLC either. For the 2nd issue, we don’t think M-PUSCH scheduling should be treated same as in DG PUSCH in Rel-16, for invalid PUSCH, there is no assigned HARQ ID, how can MAC generate a MAC PDU without a HARQ ID, could FL clarify a bit more? |
| Moderator | @ NTT DOCOMO, Regarding case 7 (tdmSchemeA), under agenda item 8.2.4, we already agreed to support m-TRP + multi-PDSCH scheduling + tdmSchemeA, with the clarification that the agreement made in RAN1#104-e corresponds to the single TRP case. @ Samsung, I understand that multi-PUSCH scheduling case is not covered in URLLC session. However, if we change the step for UCI multiplexing in case of multi-PUSCH scheduling, it may have an impact for other aspects for UCI multiplexing/prioritization whatever. The motivation to check semi-static DL collision for CG-PUSCH is similar to that for multi-PUSCH scheduling case, do you agree? So, I don’t think it is too late if we can come back this issue after semi-static DL collision issue with CG-PUSCH is resolved in URLLC session. Regarding the second issue, I might be wrong, but whether MAC PDU for multi-PUSCH DCI is treated as legacy DG PUSCH or CG PUSCH should be discussed in RAN2. What is the related RAN1 topic? |

Proposal #2.4-2 (configured vs. valid SLIV):

- For a DCI that can schedule multiple PDSCHs,

  - It is clarified that NDI/RV fields in the following previous agreements correspond to scheduled PDSCHs.

Agreement: (RAN1#104-bis) For a DCI that can schedule multiple PDSCHs, - NDI for the 1st TB: This is signaled per PDSCH and applies to the first TB of each PDSCH - RV for the 1st TB: This is signaled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the first TB of each PDSCH Agreement: (RAN1#106bis-e) For a DCI that can schedule multiple PDSCHs, and if RRC parameter configures that two codeword transmission is enabled, - NDI for the 2nd TB: This is ignalled per PDSCH and applies to the 2nd TB of each PDSCH - RV for the 2nd TB: This is ignalled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the 2nd TB of each PDSCH

  - The following example change to 38.214 Section 5.1.3 can be recommended to the editor of 38.214 to use at the editor’s discretion

5.1.3	Modulation order, target code rate, redundancy version and transport block size determination

=============================== Unchanged Text Omitted ===================================

When the UE is scheduled with multiple PDSCHs by a DCI, as described in clause 5.1.2.1, the bits of rv field and NDI field, respectively, in the DCI are one-to-one mapped to the scheduled PDSCH(s) with the corresponding transport block(s) in the scheduled order, where the LSB bits of the rv field and NDI field, respectively, correspond to the last scheduled PDSCH.

Companies are encouraged to provide views on Proposal #2.4-2. It is noted that the text proposal above is similar to the case for multi-PUSCH scheduling in current 214 specification.

| Company | Views |
| --- | --- |
| Futurewei | Support Proposal #2.4-2. |
| Intel | We support the TP. Is this agreement also applicable to multi-PUSCH scheduling? It would be good to also update the spec for PUSCH case. |
| Huawei, HiSlicon | I am not sure whether the wording of “scheduled” may still cause confusion between “configured” vs “valid”. Maybe “PDSCH(s) indicated by TDRA” is more straightforward to reflect the meaning of “configured”. |
| Samsung | We tend to agree with HW’s suggestion. “scheduled” may result in potential confusion. |
| Xiaomi | Share similar view as HW |
| Moderator | As suggested by Huawei, now updated as Proposal #2.4-2a. |

Proposal #2.4-2a (configured vs. valid SLIV):

- For a DCI that can schedule multiple PDSCHs or multiple PUSCHs,

  - It is clarified that NDI/RV fields in the following previous agreements correspond to scheduled PDSCHs indicated by the TDRA information field.

Agreement: (RAN1#104-bis) For a DCI that can schedule multiple PDSCHs, - NDI for the 1st TB: This is signaled per PDSCH and applies to the first TB of each PDSCH - RV for the 1st TB: This is signaled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the first TB of each PDSCH Agreement: (RAN1#106bis-e) For a DCI that can schedule multiple PDSCHs, and if RRC parameter configures that two codeword transmission is enabled, - NDI for the 2nd TB: This is signalled per PDSCH and applies to the 2nd TB of each PDSCH - RV for the 2nd TB: This is signalled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the 2nd TB of each PDSCH

  - Above clarification also applies to the DCI scheduling multiple PUSCHs, i.e., NDI/RV fields in the DCI correspond to scheduled PUSCHs indicated by the TDRA information field..

  - The following example change to 38.214 Sections 5.1.3 and 6.1.4 can be recommended to the editor of 38.214 to use at the editor’s discretion

5.1.3	Modulation order, target code rate, redundancy version and transport block size determination

=============================== Unchanged Text Omitted ===================================

When the UE is scheduled with multiple PDSCHs by a DCI, as described in clause 5.1.2.1, the bits of rv field and NDI field, respectively, in the DCI are one-to-one mapped to the scheduled PDSCH(s) indicated by the TDRA information field with the corresponding transport block(s) in the scheduled order, where the LSB bits of the rv field and NDI field, respectively, correspond to the last scheduled PDSCH indicated by the TDRA information field.

=============================== Unchanged Text Omitted ===================================

6.1.4	Modulation order, redundancy version and transport block size determination

=============================== Unchanged Text Omitted ===================================

When the UE is scheduled with multiple PUSCHs by a DCI, as described in clause 6.1.2.1, the bits of rv field and NDI field, respectively, in the DCI are one to one mapped to the scheduled PUSCH(s) indicated by the TDRA information field with the corresponding transport block(s) in the scheduled order where the LSB bits of the rv field and NDI field, respectively, correspond to the last scheduled PUSCH indicated by the TDRA information field.

Companies are encouraged to provide views on Proposal #2.4-2a.

| Company | Views |
| --- | --- |
| ZTE, Sanechips | Support proposal #2.4-2a |
| vivo | We support the proposal |
| DOCOMO | Fine with the proposal #2.4-2a. |
| Samsung | We support the prposal |
| Ericsson | Support Proposal #2.4-2a. |
| Qualcomm | We support the proposal |
| Futurewei | We support Proposal #2.4-2a. |
| Moderator | This proposal seems stable, so it can be reported for email endorsement. |

During email discussion, the following agreement was made:

- Agreement:

For a DCI that can schedule multiple PDSCHs or multiple PUSCHs,

- It is clarified that NDI/RV fields in the following previous agreements correspond to scheduled PDSCHs indicated by the TDRA information field.

Agreement: (RAN1#104-bis) For a DCI that can schedule multiple PDSCHs, - NDI for the 1st TB: This is signaled per PDSCH and applies to the first TB of each PDSCH - RV for the 1st TB: This is signaled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the first TB of each PDSCH Agreement: (RAN1#106bis-e) For a DCI that can schedule multiple PDSCHs, and if RRC parameter configures that two codeword transmission is enabled, - NDI for the 2nd TB: This is signalled per PDSCH and applies to the 2nd TB of each PDSCH - RV for the 2nd TB: This is signalled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the 2nd TB of each PDSCH

- Above clarification also applies to the DCI scheduling multiple PUSCHs, i.e., NDI/RV fields in the DCI correspond to scheduled PUSCHs indicated by the TDRA information field.

- The following example change to 38.214 Sections 5.1.3 and 6.1.4 can be recommended to the editor of 38.214 to use at the editor’s discretion

5.1.3	Modulation order, target code rate, redundancy version and transport block size determination

=============================== Unchanged Text Omitted ===================================

When the UE is scheduled with multiple PDSCHs by a DCI, as described in clause 5.1.2.1, the bits of rv field and NDI field, respectively, in the DCI are one-to-one mapped to the scheduled PDSCH(s) indicated by the TDRA information field with the corresponding transport block(s) in the scheduled order, where the LSB bits of the rv field and NDI field, respectively, correspond to the last scheduled PDSCH indicated by the TDRA information field.

=============================== Unchanged Text Omitted ===================================

6.1.4	Modulation order, redundancy version and transport block size determination

=============================== Unchanged Text Omitted ===================================

When the UE is scheduled with multiple PUSCHs by a DCI, as described in clause 6.1.2.1, the bits of rv field and NDI field, respectively, in the DCI are one to one mapped to the scheduled PUSCH(s) indicated by the TDRA information field with the corresponding transport block(s) in the scheduled order where the LSB bits of the rv field and NDI field, respectively, correspond to the last scheduled PUSCH indicated by the TDRA information field.

Proposed conclusion #2.4-3 (configured vs. valid SLIV):

- It is clarified that the absence or presence of CBGTI field in the following previous agreement is determined based on scheduled PUSCHs indicated by the TDRA information field (i.e. irrespective of whether this is a valid PUSCH).

Agreement: (RAN1#105-e) - At least for 120 kHz SCS, for a DCI that can schedule multiple PUSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs, - If CBG-based (re)transmission is configured, CBGTI field is not present when more than one PUSCHs are scheduled, but is present when a single PUSCH is scheduled, as in Rel-16.

Companies are encouraged to provide views on Proposed conclusion #2.4-3. It is proposed as conclusion since no specification impact is foreseen, but please comment if you have any different views.

| Company | Views |
| --- | --- |
| Futurewei | Support the Proposal Conclusion #2.4-3. |
| Intel | We agree to make it a conclusion |
| Huawei, HiSilicon | General fine with the intention. Similar as the previous comment, the wording of “scheduled” may still cause confusion between “configured” vs. “valid”. Suggest to rewording as “are scheduled” -> “are indicated in TDRA” |
| Samsung | As as in the previous comments, we prefer HW’s suggestion. |
| Moderator | Huawei’s suggestion is reflected. Please note that in current 212 specificaion, this aspect is clearly captured as follows. ----------------------------------------------------------------------------------------------- - CBG transmission information (CBGTI) – 0 bit if higher layer parameter codeBlockGroupTransmission for PUSCH is not configured or if the number of scheduled PUSCH indicated by the Time domain resource assignment field is larger than 1; otherwise, 2, 4, 6, or 8 bits determined by higher layer parameter maxCodeBlockGroupsPerTransportBlock for PUSCH. |
| ZTE, Sanechips | We support the updated proposal |
| vivo | We support the proposal |
| DOCOMO | Fine with the conclusion. |
| Samsung | Support the proposed conclusion. |
| Ericsson | Support Proposed Conclusion #2.4-3 |
| Qualcomm | We support the proposal |
| Futurewei | We support Proposed Conclusion #2.4-3. |
| Moderator | This proposed conclusion seems stable, so it can be reported for email endorsement. |

During email discussion, the following conclusion was made:

- Conclusion:

It is clarified that the absence or presence of CBGTI field in the following previous agreement is determined based on scheduled PUSCHs indicated by the TDRA information field (i.e. irrespective of whether this is a valid PUSCH).

Agreement: (RAN1#105-e) - At least for 120 kHz SCS, for a DCI that can schedule multiple PUSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs, - If CBG-based (re)transmission is configured, CBGTI field is not present when more than one PUSCHs are scheduled, but is present when a single PUSCH is scheduled, as in Rel-16.

SPS/CG-related issues

| Company | Views |
| --- | --- |
| [1] Futurewei | Proposal 6. For multi-PxSCH, the Rel-15/16 rule to handle collision with SPS/CG HPN can be reused. The corresponding Proposal #2.9-1 can be agreed with a note on it is up to the gNB implementation to avoid ignallinge cases that end up with data retransmission. Proposal 7. For multi-PxSCH, the Option 1 that allows only single SLIV-based activation for SPS/CG should be adopted, with a note to clarify that activation of SPS/CG is not supported if there is no row containing a single SLIV in the configured multi-PxSCH TDRA table. |
| [4] vivo | Proposal 3: For activation/de-activation of SPS/CG by using multi-PDSCH/PUSCH scheduling DCI, the first (valid) SLIV in the row indicated by an activation/de-activation DCI is used for determining SPS/CG occasions. |
| [5] Fujitsu | Proposal 2: For SPS activation/retransmission via DCI format 1_1 when multi-PDSCH scheduling is supported, the following 3 options can be considered, and Option 2 is slightly preferred for a well trade-off between flexibility and standardization effort. - Option 1: Allow only single SLIV-based (de)activation - Option 2: Based on the last configured SLIV - Option 3: Based on the first (valid) SLIV |
| [6] CATT | Proposal 1: When Activation of DL SPS is indicated by DCI format 1_1, if the time domain resource assignment indicates a multiple SLIV(s) entry, the first (valid) SLIV is used to compute PDSCH time domain resource allocation and slot of PUCCH carrying HARQ-ACK. Proposal 2: When Activation of UL CG is indicated by DCI format 0_1, if the time domain resource assignment indicates a multiple SLIV(s) entry, the first (valid) SLIV is used to compute PUSCH time domain resource allocation Proposal 3: At least for PUSCH transmission, for special HARQ process ID(s) that are assigned to GC PUSCH by RRC, UE shall skip these HARQ process IDs if the dynamic scheduling ignalling indicates the same ID(s). |
| [8] Samsung | Proposal 4: If a CG PUSCH is configured to be transmitted between the first scheduled PUSCH and the last scheduled PUSCH by a single DCI scheduling multiple PUSCHs, HARQ process number increment is skipped for the HARQ ID used for the CG PUSCH when determining the HARQ ID of the multiple scheduled PUSCHs. Proposal 5: For a DCI capable of scheduling multi-PDSCH/PUSCHs, gNB can only indicate a row with single SLIV for SPS PDSCH/CG PUSCH activation and retransmission. |
| [11] Panasonic | Proposal 2: For an activation of SPS (or CG) by using multi-PDSCH (or multi-PUCH) scheduling DCI, support to use a solution based on the last or first (valid) SLIV in order to reuse SLIV entry for multi-PDSCH/PUSCH scheduling. |
| [14] Intel | Proposal 2 - A HARQ process number configured for SPS PDSCH/CG PUSCH can be allocated to a PDSCH/PUSCH of multi-PDSCH/PUSCH scheduling, as long as the timeline is met. - No TP is needed for HARQ process number collision handling between SPS PDSCH/CG PUSCH and DG PDSCH/PUSCH. Proposal 3 - For activation of SPS-PDSCH or Type-2 CG-PUSCH by using multi-PDSCH or multi-PUSCH scheduling mechanism, respectively, only single SLIV-based activation is allowed. - Agree on TP1 for activation of SPS-PDSCH or Type-2 CG-PUSCH by using multi-PDSCH or multi-PUSCH scheduling mechanism, respectively. |
| [15] Ericsson | Proposal 4 Support the FL proposal #2.9-1 (SPS/CG HPN) on how to handle HARQ process number when it collides with that assigned for SPS or CG. I.e., HARQ process number configured for SPS PDSCH (or CG PUSCH) can be allocated to a PDSCH (or PUSCH) of multi-PDSCH (or multi-PUSCH) scheduling, as long as the timeline condition defined in Rel-15/16 is met. Observation 1 With the current specification regarding PDCCH validation for DL SPS and UL grant Type 2, it is not possible that a multiple scheduling DCI indicates SPS/CG activation/release while schedules normal PDSCH/PUSCH transmission at the same time. Proposal 5 For SPS/CG activation/release by using multi-PDSCH/PUSCH scheduling DCI, support Option 1. I.e., SPS/CG activation/release is indicated by a DCI that indicates a single SLIV. |
| [20] LG Electronics | Proposal #7: If a DCI that indicates a row index of the TDRA table associated with multiple SLIVs can be used for SPS PDSCH (or CG PUSCH) (de)activation, determine TDRA or PUCCH resource corresponding to SPS (or CG) based on the last SLIV value in the indicated TDRA row index. |
| [22] WILUS | Proposal 3: We propose to support that - If the timeline condition defined in Rel-15/16 is met, HARQ process number configured for SPS PDSCH (or CG PUSCH) can be allocated to a PDSCH (or PUSCH) of multi-PDSCH (or multi-PUSCH) scheduling. - Otherwise, HARQ process number increment for a PDSCH(or PUSCH) of multi-PDSCH (or multi-PUSCH) scheduling is skipped for the HARQ process number configured for SPS PDSCH (or CG PUSCH). |

- [Closed] Issue 2.5-1) How to handle HARQ process number when it collides with that assigned for SPS or CG:

Company views on how to handle HARQ process number when it collides with that assigned for SPS or CG:

- Option 1: Skip HARQ process number(s) pre-configured for SPS or CG when any of scheduled PDSCHs (or PUSCHs) is overlapped with an SPS PDSCH (or CG PUSCH), and/or when HARQ process numbers for PDSCHs (or PUSCHs) scheduled by a single DCI collide with HARQ process number(s) configured for the SPS PDSCH (or CG PUSCH).

  - Supported by CATT, Samsung

- Option 2: HARQ process number configured for SPS PDSCH (or CG PUSCH) can be allocated to a PDSCH (or PUSCH) of multi-PDSCH (or multi-PUSCH) scheduling, as long as the timeline is met.

  - Supported by Futurewei, Intel, Ericsson, WILUS

Timeline for PDSCH:

TS 38.214 5.1 UE procedure for receiving the physical downlink shared channel ……………………………………..<omitted>……………………………………… The UE is not expected to decode a PDSCH in a serving cell scheduled by a PDCCH with C-RNTI, CS-RNTI or MCS-C-RNTI and one or multiple PDSCH(s) required to be received according to this Clause in the same serving cell without a corresponding PDCCH transmission if the PDSCHs partially or fully overlap in time except if the PDCCH scheduling the PDSCH ends at least 14 symbols before the earliest starting symbol of the PDSCH(s) without the corresponding PDCCH transmission, where the symbol duration is based on the smallest numerology between the scheduling PDCCH and the PDSCH, in which case the UE shall decode the PDSCH scheduled by the PDCCH.

Timeline for PUSCH:

TS 38.214 6.1 UE procedure for transmitting the physical uplink shared channel ……………………………………..<omitted>……………………………………… A UE is not expected to be scheduled by a PDCCH ending in symbol i to transmit a PUSCH on a given serving cell overlapping in time with a transmission occasion, where the UE is allowed to transmit a PUSCH with configured grant according to [10, TS38.321], starting in a symbol j on the same serving cell if the end of symbol i is not at least N2 symbols before the beginning of symbol j. The value N2 in symbols is determined according to the UE processing capability defined in Clause 6.4, and N2 and the symbol duration are based on the minimum of the subcarrier spacing corresponding to the PUSCH with configured grant and the subcarrier spacing of the PDCCH scheduling the PUSCH.

[Moderator’s note] Although the number of inputs is small, given the wide support of Option 2 in the last meeting, the following proposal can be made with a note (from Futurewei’s proposal) added.

Proposed Conclusion #2.5-1 (SPS/CG HPN):

- HARQ process number configured for SPS PDSCH (or CG PUSCH) can be allocated to a PDSCH (or PUSCH) of multi-PDSCH (or multi-PUSCH) scheduling, as long as the timeline condition defined in Rel-15/16 is met.

  - Note: It is up to gNB implementation whether/how to avoid UL data retransmission due to HARQ process index collision and flushed HARQ transmit buffer.

Companies are encouraged to provide views on Proposed Conclusion #2.5-1.

| Company | Views |
| --- | --- |
| Xiaomi | Agree with proposal |
| Qualcomm | We are okay with the proposal |
| vivo | Support the proposed conclusion |
| Panasonic | We are fine with the proposed conclusion #2.5-1. |
| DOCOMO | Support the proposal. And we think no specification impact is needed. |
| CATT1 | We still prefer the option 1 for simplicity. Otherwise, what is behavior when the timeline condition is not met? |
| Intel | We support Proposed Conclusion #2.5-1 |
| Fujitsu | We are fine with the proposal. |
| Samsung | We still have a concern on the proposed conclusion #2.5-1 . It brings additional scheduling restrictions for OOO HARQ of the same HARQ process ID. TS 38.214 When HARQ feedback for the HARQ process ID is not disabled, the UE is not expected to receive another PDSCH for a given HARQ process until after the end of the expected transmission of HARQ-ACK for that HARQ process, where the timing is given by Clause 9.2.3 of [6, TS 38.213].. |
| Nokia, NSB | Support Proposed Conclusion |
| Apple | Prefer Option 1 |
| Futurewei | Support Proposal #2.5-1. |
| ZTE, Sanechips | We support the proposed conclusion. |
| Ericsson | Support Proposed Conclusion #2.5-1 |
| OPPO | Support conclusion #2.5-1 |
| Huawei, HiSilicon | We are fine with the conclusion |
| Moderator | @ CATT, The intention is UE does not expect that timeline condition is not met, which is error case. @ Samsung, Proposed Conclusion #2.5-1 intends not to have a specification impact. It would be appreciated if you could elaborate on what would be the additional scheduling restriction. @ CATT, Samsung, Please provide comments if there is a strong concern with Proposed Conclusion #2.5-1. |
| Samsung | We still see technical benefits on HARQ ID collision handling between SPS/CG and DG multi-PUSCH/PDSCH. As we proposed, HARQ ID increments for multi-PUSCH/PDSCH can be taken into account SPS/CG’s HARQ ID collision. We believe that it can provide more flexibility on multi-PUSCH/PDSCH scheduling as well as prevent HARQ buffer from being cleared by the HARQ ID collision. However, given the situation, we can live with the proposed conclusion #2.5-1 for the sake of progress. |

During email discussion, the following conclusion was made:

- Conclusion:

HARQ process number configured for SPS PDSCH (or CG PUSCH) can be allocated to a PDSCH (or PUSCH) of multi-PDSCH (or multi-PUSCH) scheduling, as long as the timeline condition defined in Rel-15/16 is met.

- Note: It is up to gNB implementation whether/how to avoid UL data retransmission due to HARQ process index collision and flushed HARQ transmit buffer.

- [Closed] Issue 2.5-2) (De)Activation of SPS (or CG) by using multi-PDSCH (or multi-PUCH) scheduling DCI:

Company views on the issue for (de)activation of SPS (or CG) by using multi-PDSCH (or multi-PUCH) scheduling DCI:

- Option 1: Allow only single SLIV-based activation

  - Supported by Futurewei, Samsung, Intel, Ericsson

- Option 2: Based on the last (valid) SLIV

  - Supported by Fujitsu, Panasonic, LG Electronics

- Option 3: Based on the first (valid) SLIV

  - Supported by vivo, CATT, Panasonic

[Moderator’s note] Given diverged view among three options, it is encouraged for companies to provide views on the above options, if any.

| Company | Views |
| --- | --- |
| Qualcomm | Our first preference is Option 2, then Option 3. We believe that option 2 is slightly simpler |
| InterDigital | We are fine with Option 2 or Option 3. |
| Vivo | We are fine with Option 2 or Option 3. Option 1 is too restricted and Option 2/3 has more flexibility. |
| DOCOMO | We support Option 1. Option 1 is the simplest way. The benefit of option 2 or option 3 is not clear. |
| Intel | We prefer Option 1. It is not clear to us the motivation to activate the SPS-PDSCH or Type 2 CG-PUSCH in a row with more than one SLIVs by multi-PDSCH/PUSCH scheduling. As defined in Rel-15/16, DCI format 0_0 can be used to activate SPS-PDSCH or Type 2 CG-PUSCH. In our view, Option 1 is preferred, i.e., only single SLIV-based activation is allowed |
| Fujitsu | Our 1st preference is Option 2. In addition, to avoid any impact on Type-1 HARQ-ACK codebook, we think the last configured SLIV should be used. Option 2 and Option 3 could bring more flexibility than Option 1, and Option 2 has nearly same level of spec impact as Option 1. Therefore, Option 2 is the best choice in our understanding. |
| Samsung | We prefer option 1 for SPS/CG activation. Also, we suggest to discuss whether SPS/CG retransmission can be scheduled by multi-PDSCH/PUSCH scheduling by a single DCI. |
| Nokia, NSB | We support Option 1. |
| Apple | We prefer Option 1 |
| Futurewei | Support Option 1. |
| ZTE, Sanechips | We support option 1. |
| MediaTek | We support option1 |
| Ericsson | We support Option 1 for simplicity (preferred in maintenance) |
| OPPO | Support Option 1. |
| Huawei, HiSilicon | We prefer option 1 due to its simplicity. |
| Moderator | - Option 1: Allow only single SLIV-based activation - Supported by Futurewei, Samsung, Intel, Ericsson, Nokia, Apple, ZTE, MediaTek, OPPO, Huawei - Option 2: Based on the last (valid) SLIV - Supported by Fujitsu, Panasonic, LG Electronics, Qualcomm (1st), vivo - Option 3: Based on the first (valid) SLIV - Supported by vivo, CATT, Panasonic, Qualcomm (2nd) Given the majority view, the following proposal can be made. |

Proposal #2.5-2 (SPS/CG (de)activation):

- A UE validates, for scheduling activation or scheduling release, a DL SPS assignment PDCCH or a configured UL grant Type 2 PDCCH if the PDCCH indicates a TDRA row index including only one SLIV.

Companies are encouraged to provide views on Proposal #2.5-2.

| Company | Views |
| --- | --- |
| vivo | We don’t support the proposal. When multi-PDSCH/PUSCH table is configured for 480K and 960KHz SCS, it is quite possible that the number of TDRA row index including only one SLIV is 0 or very small due to multi-slot PDCCH monitoring. In this sense, the number of SLIVs available for SPS/CG activation is quite restricted. Moreover, if gNB configures a multi-PDSCH/PUSCH table without a row index including only one SLIV, how can SPS/CG activation is performed if following this proposal? |
| Moderator | @ vivo, Still SPS/CG (de)activation can be performed by DCI format x_0 and/or x_2. |
| Fujitsu | We still believe Option 2 is the best way to go. Option 1 will limit flexibility, as commented by other companies. In addition, for the case where gNB configures multi-PDSCH/PUSCH table without a single-SLIV row, UE will be required to monitor more DCIs or SPS/CG activation/release, which may unnecessarily increase UE’s PDCCH monitoring burden. Option 2 can have more flexibility with similar specification effort. But we could accept Option 1 as a compromise if it is still the majority view after more discussions. |
| Vivo | @Moderator: Agree that DCI x_0 and x_2 could be used However, DCI x_2 may not exist for FR 2-2 case. If using DCI x_0 for SPS/CG activation, there will be several limitations such as resource allocation type, no application of slot aggregation, number of SLIVs and etc. Option 2 or 3 provides significant flexibility with similar spec impact as Fujitsu indicates. |
| Samsung | We are fine with the proposal. For completeness, the followings might be considered further. - For SPS/CG release, if the intention is to determine field size for NDI and RV, we suggest to make conclusion for both SPS/CG release and Scell dormancy indication together. Similairy, for Scell dormancy indication, the field size for NDI and RV depends on a TDRA row so that we need to discuss whether a TDRA row with multiple SLIV can be used or not. - For SPS/CG retransmission (scheduled by DCI format 1_1/0_1 with a CRC scrambled by CS-RNTI), it is unclear whether TDRA row with multiple SLIVs can be used or not. It would be better to make a decision for both SPS/CG activation and retransmssion together. |
| Moderator | @ vivo, Understood, let’s see other companies’ views. @ Samsung, For Scell dormancy case, I tend to agree that we need to figure out that issue and if generalized, also need to figure out the case of DCI format not scheduling PDSCH such as TCI state update. However, those issues could be handled once this proposal will be agreed. For SPS/CG retransmission DCI, I don’t see any problem since the DCI is treated as dynamic DL/UL DCI. |
| Intel | We are fine with the proposal. |
| Huawei, HiSlicon | Support the proposal |
| Samsung | @FL. Thanks for the response. I agree with you that we can further discuss DCI format not scheduling PDSCH separately in the next meeting. For SPS/CG retransmission DCI (DCI format x_1 with a CRC scrambled by CS-RNTI), we have some concerns on multi-PDSCH/PUSCH scheduling. Since the dedicated HARQ process IDs for SPS/CG retransmission is not all of HARQ process IDs configured for a cell, the HARQ process increment rule for multi-PDSCHs for SPS/CG retransmission should be further clarified. If not, UE behaviors for SPS/CG retransmssion with not dedicated HARQ process IDs are unclear. A potential solution is that a UE does not receive/transmit PDSCH/PUSCH corresponding to a SLIV for not dedicated HARQ process IDs for SPS/CG. From our perspective, we don’t prefer to specify complicated UE behaviors in this stage so that we slightly prefere to not support multi-PDSCH/PUSCH scheduling for SPS/CG retransmssion. |
| Xiaomi | OK with the proposal. |
| Moderator | @ Samsung, I agree that we need to refrain from making UE behavior complicated in this maintenance stage. From my understanding, SPS/CG retransmission is informed to UE with CRC scrambled with CS-RNTI and NDI set to ‘1’. Even in Rel-16, is the current standard specifying the UE behavior if UE receives DCI (with CRC scrambled with CS-RNTI and NDI set to ‘1’) indicating HARQ process which is not allocated to SPS/CG? If not, I don’t think we need to specify the corresponding UE behavior when gNB mistakenly indicates HARQ process index (not dedicated to SPS/CG) in SPS/CG retransmission DCI (scheduling multi-PXSCHs). |
| ZTE, Sanechips | Support the proposal |
| Samsung | @FL. Yes, for single PDSCH/PUSCH scheduling, the specification does not specified UE behaviors for a HARQ process index not decidated to SPS/CG. It is because there is no reason for gNB to indicate the HARQ process index not dedicated SPS/CG. For multi-PDSCH/PUSCH scheduling for SPS/CG retransmission, there are potential possibilities for a UE to determine the HARQ process index not dedicated to SPS/CG due to the HARQ process increment rule. Consider two cases. - Case 1: a UE is configured with a SPS configuration with HARQ process IDs of 0,1,2, and 3. If a UE fails to received SPS PDSCHs with HARQ process IDs of 0 and 2, then how does gNB retransmit these two SPS PDSCHs by using multi-PDSCH scheduling? Is it allowed to schedule 3 SPS PDSCHs with HARQ process IDs of 0, 1, and 2? If allowed, what is a UE behaviour for SPS PDSCH with HARQ process ID of 1? - Case 2: a UE is configured with two SPS configurations, where the first SPS configuration has HARQ process IDs of 0,1 and the second SPS configuration has HARQ process IDs of 3,4. If a UE fails to receive SPS PDSCHs with HARQ process IDs of 1 and 3, again how does gNB retransmit the two SPS PDSCHs by using multi-PDSCH scheduling? Also, is it allowed to schedule 3 SPS PDSCHs with HARQ process IDs of 1, 2, and 3? If allowed, what is a UE behaviour for SPS PDSCH with HARQ process ID of 2? For two cases, our preference is to not allow multi-PDSCH scheduling for simplicity. So, we would clarify it, for example, - A UE does not expect to be receive a DCI format scheduling multiple SPS PDSCHs (or CG PUSCHs) retransmsission with a HARQ process ID not dedicated for any SPS configurations (CG configurations). Another way is to allow the multi-PDSCH scheduling for two cases but clarify UE behaviors, for example, - A UE does not receive (or transmit) a SPS PDSCH (or CG PUSCH) retransmsission with a HARQ process ID not dedicated for any SPS configurations (CG configurations). As we mentioned, the latter option seems more complicated than the former option. Also, it was discussed in [103-e-NR-NRU-06] that whether multi-PUSCH based CG PUSCH retransmission is allowed or not. For this discussion, RAN1 did not adopt TPs for multi-PUSCH based CG PUSCH retransmission. Then, what’s difference between Rel-16 multi-PUSCH scheduling and Rel-17 multi-PUSCH scheduled in terms of CG PUSCH retransmission. If we support multi-PUSCH based SPS/CG transmission, further specification works are expected to clarify “NDI=1” since there are more than one bits for NDI field. So, it should be replaced with “NDI = all 1s” when multi-PDSCH/PUSCH scheduling is indicated. |
| Ericsson | Support Proposal #2.5-2 |
| Futurewei | We are OK with Proposal #2.5-2. |
| Moderator | @ Samsung, My point is that, in your examples, why gNB chooses a multi-PXSCH scheduling DCI even though it knows the DCI may cause UE’s confusion. gNB may have other choices, e.g., by indicating other TDRA rows in the multi-PXSCH scheduling DCI or by using other DCI format, not to give a mis-configuration to UE. Anyway, this issue is still discussed between us . We may discuss further on this issue next meeting, if companies also bring up this issue as well. @ vivo, So far, I don’t see any negative feedback of this proposal other than vivo. Given this situation, could you accept this proposal? |

During email discussion, the following agreement was made:

- Agreement:

A UE validates, for scheduling activation or scheduling release, a DL SPS assignment PDCCH or a configured UL grant Type 2 PDCCH if the PDCCH indicates a TDRA row index including only one SLIV.

Other issues

| Company | Views |
| --- | --- |
| [8] Samsung | Proposal 6：For single TRP or multi-TRP operation, for 480/960 kHz SCS, - A UE does not expect to receive more than one unicast PDSCH in a slot on a serving cell from the same TRP. - A UE does not expect to transmit more than one PUSCH in a slot on a serving cell from the same TRP. Proposal 7: For resolving collision of overlapping PDSCHs and/or PUSCHs and/or PUCCHs in case of M-PDSCH/M-PUSCH scheduling, UE first resolves the collision of PDSCHs/PUSCHs and semi-static UL/DL symbols and then UE resolves the collision among PDSCHs, PUSCHs and PUCCHs. Proposal 10: Clarify that for Scell dormancy indication, a UE repurposes Npdsch,max-bit NDI and Npdsch,max-bit RV fields if TDRA indicates multi-PDSCH scheduling or 1-bit NDI and 2-bit RV fields if TDRA indicates single-PDSCH scheduling. - If Npdsch,max-bit NDI and Npdsch,max-bit RV fields are repurposed, the sequence order for a bitmap is 5-bit MCS, Npdsch,max-bit NDI, Npdsch,max-bit RV, HPN, antenna port(s), and DMRS sequence initialization fields |
| [12] Qualcomm | Proposal 6: In the case of multi-PDSCH scheduling via a single DCI with ‘tdmSchemeA’, consider one of the following options - Option 1: UE assumes PDSCH mapping Type B for first and second repetitions of each TB regardless of the mapping type for each SLIV of the indicated TDRA row. - Option 2: UE applies the mapping type of each SLIV as indicated by the TDRA assignment field to the first repetition of the corresponding PDSCH and assumes PDSCH mapping Type B for the second repetition of each PDSCH. |
| [17] Xiaomi | Proposal 4: If Type 1 or Type 3 channel access mechanism is indicated, Type 1 or Type 3 channel access can be applied to each transmission burst among the multiple scheduled PUSCHs. If Type 2 channel access mechanism is indicated, Type 2 channel access can be applied to the first transmission burst, and Type 1 channel access can be for the subsequent bursts, if any. |
| [18] NEC | Proposal 4: Consider the impact of minimum applicable scheduling offset when multiple-PXSCH scheduling and cross-slot scheduling are enable simultaneously. |

- Summary on other aspects for multi-PDSCH/PUSCH scheduling:

The following issues are brought up by several companies:

- Samsung: Clarification on TDMed PXSCH, collision resolution step, and SCell dormancy indication of multi-PDSCH scheduling DCI

- Qualcomm: Clarification on PDSCH mapping type for TDM scheme A with m-TRP case

- Xiaomi: Interpretation of channel access type indication

- NEC: Clarification on the combination of minK0/minK2 with multi-PXSCH scheduling

[Moderator’s note] Please feel free to express views on above issues, if any.

| Company | Views |
| --- | --- |
| Qualcomm | Restricting all the SLIVs to have mapping type-B as legacy to allow TDM scheme A will limit the scheduling flexibility instead, we can choose one of the two options - Option 1: UE assumes PDSCH mapping Type B for first and second repetitions of each TB regardless of the mapping type for each SLIV of the indicated TDRA row. - Option 2: UE applies the mapping type of each SLIV as indicated by the TDRA assignment field to the first repetition of the corresponding PDSCH and assumes PDSCH mapping Type B for the second repetition of each PDSCH. |
| Samsung | We suggest to discuss the following in Section 2.5 SPS/CG-related issues. If there is SPS PDSCH/CG PUSCH in the slot, UE behaviour is not clear. For example, whether UE can receive a unicast PDSCH and a SPS PDSCH in a slot? The intention of the agreement is to restrict that there is up to 1 PDSCH/PUSCH per slot. Same rule should apply to SPS PDSCH/CG PUSCH. In addition, whether the scheduled PDSCH/PUSCH is a valid one should be clarified. Proposal 6：For single TRP or multi-TRP operation, for 480/960 kHz SCS, - A UE does not expect to receive more than one unicast PDSCH in a slot on a serving cell from the same TRP. - A UE does not expect to transmit more than one PUSCH in a slot on a serving cell from the same TRP. |
| NEC | Suggest to consider the impact of minimum applicable scheduling offset when multiple-PXSCH scheduling and cross-slot scheduling are enable simultaneously. |

HARQ

PUCCH power control

| Company | Views for type-1 HARQ-ACK codebook |
| --- | --- |
| [4] vivo | Proposal 9: For multi-PDSCH scheduling and Type-1 codebook, consider the TP2 in TS38.213 for the calculation of nHARQ-ACK. --------------------------------------------------Start TP2------------------------------------------------------- 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel …… If OACK+OSR+OCSI≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as described in clause 7.2.1, as nHARQ-ACK=c=0NcellsDL-1m=0Mc-1Nm,creceived+c=0NcellsDL-1m=0Mc-1Nm,creceived,CBG where - NcellsDL are all DL cells where the UE is configured to receive unicast or multicast PDSCHs - Mc is the cardinality for the union of all sets MA,c of occasions for unicast or multicast PDSCH receptions or SPS PDSCH releases for serving cell c - if enableTimeDomainHARQ-Bundling is not provided, Nm,creceived is the number of transport blocks the UE receives in PDSCH reception occasion m for serving cell c if harq-ACK-SpatialBundlingPUCCH and PDSCH-CodeBlockGroupTransmission are not provided, or the number of transport blocks the UE receives in PDSCH reception occasion m for serving cell c if PDSCH-CodeBlockGroupTransmission is provided and the PDSCH reception is scheduled by a DCI format that does not support CBG-based PDSCH receptions, or the number of PDSCH receptions if harq-ACK-SpatialBundlingPUCCH is provided or SPS PDSCH release or TCI state update in PDSCH reception occasion m for serving cell c and the UE reports corresponding HARQ-ACK information in the PUCCH. - if enableTimeDomainHARQ-Bundling is provided, Nm,creceived is the number of enabled transport blocks in a DCI format scheduling a PDSCH associated with PDSCH reception occasion m for serving cell c if harq-ACK-SpatialBundlingPUCCH is not provided, or the number of DCI formats scheduling a PDSCH associated with PDSCH reception occasion m for serving cell c if harq-ACK-SpatialBundlingPUCCH is provided or SPS PDSCH release or TCI state update in PDSCH reception occasion m for serving cell c and the UE reports corresponding HARQ-ACK information in the PUCCH. - Nm,creceived,CBG is the number of CBGs the UE receives in a PDSCH reception occasion m for serving cell c if PDSCH-CodeBlockGroupTransmission is provided and the PDSCH reception is scheduled by a DCI format that supports CBG-based PDSCH receptions and the UE reports corresponding HARQ-ACK information in the PUCCH. ---------------------------------------------------End TP2------------------------------------------------------- |
| [8] Samsung | Proposal 16: Adopt TP#6 in Appendix for TS38.213 ====================== Start of TP #6 for TS 38.213 ======================== 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel ====================== Unchanged Text Omitted ========================== If OACK+OSR+OCSI≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as described in clause 7.2.1, as nHARQ-ACK=c=0NcellsDL-1m=0Mc-1Nm,creceived+c=0NcellsDL-1m=0Mc-1Nm,creceived,CBG where - NcellsDL are all DL cells where the UE is configured to receive unicast or multicast PDSCHs - Mc is the cardinality for the union of all sets MA,c of occasions for unicast or multicast PDSCH receptions or SPS PDSCH releases for serving cell c - Nm,creceived is the number of transport blocks the UE receives in PDSCH reception occasion m for serving cell c if harq-ACK-SpatialBundlingPUCCH and PDSCH-CodeBlockGroupTransmission are not provided, or the number of transport blocks the UE receives in PDSCH reception occasion m for serving cell c if PDSCH-CodeBlockGroupTransmission is provided and the PDSCH reception is scheduled by a DCI format that does not support CBG-based PDSCH receptions, or the number of PDSCH receptions if harq-ACK-SpatialBundlingPUCCH is provided or SPS PDSCH release or TCI state update in PDSCH reception occasion m for serving cell c and the UE reports corresponding HARQ-ACK information in the PUCCH. - if enableTimeDomainHARQ-Bundling is provided for a serving cell c, for a DCI format indicating a TDRA row that includes more than one SLIV entry on the serving cell c, a PDSCH associated with the last SLIV is considered as received, a PDSCH other than the PDSCH associated with the last SLIV is considered as not received - Nm,creceived,CBG is the number of CBGs the UE receives in a PDSCH reception occasion m for serving cell c if PDSCH-CodeBlockGroupTransmission is provided and the PDSCH reception is scheduled by a DCI format that supports CBG-based PDSCH receptions and the UE reports corresponding HARQ-ACK information in the PUCCH. ====================== End of TP #6 for TS 38.213 ========================= |
| [20] LG Electronics | Proposal #9: For type-1 and type-2 HARQ-ACK codebooks, consider multi-PDSCH scheduling and time domain bundling configuration to derive nHARQ-ACK for PUCCH power control when UCI payload size is equal to or less than 11 bits. |
| Company | Views for type-2 HARQ-ACK codebook |
| [4] vivo | Proposal 11: For multi-PDSCH scheduling and Type-2 codebook, consider the TP3 in TS38.213 for the calculation of nHARQ-ACK. |
| [20] LG Electronics | Proposal #9: For type-1 and type-2 HARQ-ACK codebooks, consider multi-PDSCH scheduling and time domain bundling configuration to derive nHARQ-ACK for PUCCH power control when UCI payload size is equal to or less than 11 bits. |

- [Closed] Issue 3.1) How to calculate nHARQ-ACK for PUCCH power control when UCI payload size is equal to or less than 11 bits

[Moderator’s note] Several companies (vivo, Samsung, and LG Electronics) brought up a new issue for PUCCH power control when UCI payload size is equal to or less than 11 bits. Although a few companies raised this issue, it is the moderator’s understanding that the current specification for nHARQ-ACK calculation should be corrected considering at least the followings:

- Type-1 HARQ-ACK codebook: For a serving cell c configured with time domain bundling, whether the PDSCH received in PDSCH reception occasion m is associated with the last SLIV or not

- Type-2 HARQ-ACK codebook: Whether a serving cell c is configured with time domain bundling or not, and whether two sub-codebooks are generated or not

Companies are encouraged to provide views on above moderator’s note. Once we reach a consensus on the necessity of this issue, we can focus on how to figure it out.

| Company | Views |
| --- | --- |
| vivo | For Type-1 codebook, when the number of UCI information bits is less than or equal to 11, the Rel-16 nHARQ-ACK calculation formula for it can work well when time domain bundling is not enabled. Nevertheless, when time domain bundling is enabled, it cannot indicate the number of valid HARQ-ACK information bits contained in the Type-1 codebook precisely. Therefore, the transmit power of PUCCH transmission conveying the Type-1 codebook, set based on the calculated nHARQ-ACK, may not match the required one exactly. For Type-2 codebook, in TS38.213 v17.0.0, when the number of UCI information bits is less than or equal to 11, the calculation of nHARQ-ACK for the second HARQ-ACK sub-codebook for multi-PDSCH scheduling is missing. Furthermore, when time domain bundling is enabled for a serving cell, and the number of bundling groups is 1, then HARQ-ACK for PDSCHs scheduled by a DCI scheduling more than one PDSCH will be bundled and contained in the first HARQ-ACK sub-codebook. Then the calculation of nHARQ-ACK for the first HARQ-ACK sub-codebook should also be adjusted to accommodate this case. |
| DOCOMO | We agree that time domain bundling should be considered for nHARQ-ACK calculation. More discussions are needed for TP. |
| Intel | We agree the proposed CRs are necessary in principle. - For Type1 HARQ-ACK codebook, Samsung’s CR is clearer. Alternatively, if ‘the PDSCH received in PDSCH reception occasion m is associated with the last SLIV’ is captured somewhere in the specification, other CR can be considered too. For Type2 HARQ-ACK codebook, we share the view that the configuration of time bundling and the handling of two sub-codebooks need to be considered in nHARQ-ACK calculation |
| Fujitsu | We share the similar view that more discussions on the TPs are needed. |
| Samsung | We share the same view with FL that nHARQ-ACK calculation should be updated when time-domain bundling is configured. For type 1 CB, we suggest to use the last PDSCH regardless of its validity. In other words, the last PDSCH is regarded as “received” whenever at least one PDSCH scheduled by a single DCI (scheduling the last PDSCH as well) is received. |
| Nokia, NSB | We share Moderator’s note that corrections for the calculation of nHARQ-ACK are necessary when HARQ ACK time domain bundling is configured |
| ZTE, Sanechips | We agree with moderator that time domain bundling should be considered for nHARQ-ACK calculation. |
| Ericsson | We share Moderator’s view that updates are needed for the calculation of nHARQ-ACK when HARQ ACK time domain bundling is configured |
| OPPO | We share the same view with Moderator. More discussions are needed. |
| Huawei, HiSilicon | The the calculation of nHARQ-ACK should be updated if time bundling is enabled. Detail TP can be further discussed. |
| Moderator | It seems that all companies agree corrections for nHARQ-ACK calculation are necessary both for type-1 and type-2 HARQ-ACK codebooks. Then, I will prepare for corresponding text proposals. |

Proposal #3.1-1 (Type-1 CB PUCCH power control):

- For type-1 HARQ-ACK codebook, if OACK+OSR+OCSI≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as follows.

  - For a serving cell c configured with enableTimeDomainHARQ-Bundling, and for a DCI format indicating a TDRA row that includes more than one SLIV entry on the serving cell c, the UE considers a PDSCH (which carries one or two transport blocks enabled by the DCI format irrespective of whether the PDSCH is valid or not) only associated with the last SLIV as received, to determine Nm,creceived.

- The following example change to 38.213 Section 9.1.2.1 can be recommended to the editor of 38.213 to use at the editor’s discretion

9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

=============================== Unchanged Text Omitted ===================================

If OACK+OSR+OCSI≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as described in clause 7.2.1, as nHARQ-ACK=c=0NcellsDL-1m=0Mc-1Nm,creceived+c=0NcellsDL-1m=0Mc-1Nm,creceived,CBG where

-	NcellsDL are all DL cells where the UE is configured to receive unicast or multicast PDSCHs

-	Mc is the cardinality for the union of all sets MA,c of occasions for unicast or multicast PDSCH receptions or SPS PDSCH releases for serving cell c

-	Nm,creceived is the number of transport blocks the UE receives in PDSCH reception occasion m for serving cell c if harq-ACK-SpatialBundlingPUCCH and PDSCH-CodeBlockGroupTransmission are not provided, or the number of transport blocks the UE receives in PDSCH reception occasion m for serving cell c if PDSCH-CodeBlockGroupTransmission is provided and the PDSCH reception is scheduled by a DCI format that does not support CBG-based PDSCH receptions, or the number of PDSCH receptions if harq-ACK-SpatialBundlingPUCCH is provided or SPS PDSCH release or TCI state update in PDSCH reception occasion m for serving cell c and the UE reports corresponding HARQ-ACK information in the PUCCH.

- if enableTimeDomainHARQ-Bundling is provided for a serving cell c, for a DCI format indicating a TDRA row that includes more than one SLIV entry on the serving cell c, a PDSCH associated with the last SLIV is considered as received, a PDSCH other than the PDSCH associated with the last SLIV is considered as not received.

-	Nm,creceived,CBG is the number of CBGs the UE receives in a PDSCH reception occasion m for serving cell c if PDSCH-CodeBlockGroupTransmission is provided and the PDSCH reception is scheduled by a DCI format that supports CBG-based PDSCH receptions and the UE reports corresponding HARQ-ACK information in the PUCCH.

Companies are encouraged to provide views on Proposal #3.1-1.

| Company | Views |
| --- | --- |
| Intel | We are supportive to the TP |
| vivo | Support the proposal in principle. It should be clarified when the PDSCH associated with the last SLIV is invalid, no transport block will be carried by the PDSCH, i.e., the number of transport blocks the UE receives in the PDSCH is 0, so the Nm,creceived as the number of transport blocks the UE receives in PDSCH reception occasion m for serving cell c if harq-ACK-SpatialBundlingPUCCH and PDSCH-CodeBlockGroupTransmission are not provided will be incorrect accordingly. To resolve the above issue, the following modification can be considered. - For type-1 HARQ-ACK codebook, if OACK+OSR+OCSI≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as follows. For a serving cell c configured with enableTimeDomainHARQ-Bundling, and for a DCI format indicating a TDRA row that includes more than one SLIV entry on the serving cell c, the UE considers a PDSCH associated with the last SLIV as received, and carring one or two transport blocks enabled by the DCI format irrespective of whether the PDSCH is valid or not, to determine Nm,creceived. |
| Samsung | We support the proposal as proponent. |
| Huawei, HiSilicon | Support the TP and fine with the modification by VIVO. |
| Futurewei | We support Proposal #3.1-1 with added text from vivo. |
| Fujitsu | We have a question for clarification. In the above specification without the Proposal/TP, what is the common understanding here on “the number of transport blocks the UE receives in PDSCH reception occasion m”? is it include TBs corresponding to all SLIVs or only the last SLIV? |
| Moderator | @ vivo, Suggeted wording (with slight modification based on my understanding) is now reflected. @ Fujitsu, Just to understand the question, is there any case where a TDRA row index includes more than one SLIV, in Rel-16? |
| Fujitsu | @Moderator Of cource no in Rel-16. 😊 We are not objecting the TP. By asking the question, we are just trying to make sure we have same understanding as other companies on the the main bullet regarding Nm,creceived. It seems the TP assumes “the number of transport blocks the UE receives in PDSCH reception occasion m” including all SLIVs of a TDRA row. Is it the correct understanding? |
| Samsung | We slightly prefer the original one. Regarding the wording suggested by vivo, we think it is already reflected in “a PDSCH associated with the last SLIV is considered as received.” If a UE assume a PDSCH is received, the number of TBs in the PDSCH is determined by MCS and RV field in a DCI format. The interpretation that the PDSCH assumed to be received has 0 TB is not aligned to the agreements on 2-TB disabling rule (TB-disabling for all of PDSCHs scheduled by a single DCI). @FL. Is your intention to add vivo’s modification to the proposal only and not to the TP? If it is right, we are fine with the proposal. @Fujitsu. Regarding your question, our understanding is following: - if time domain bundling is provided, a UE assumes a PDSCH associated with the last SLIV is received (regardless valid or invalid) (as per the proposed TP) - If the PDSCH is received in PDSCH reception occasion m, then the UE counts the number of TBs in the PDSCH. Here, the number of TBs in the PDSCH is determined by MCS and RV field. |
| Moderator | @ Samsung, YES, that is the intention, just to make the proposal aligned with TP. @ Fujitsu, I’m on the same page with Samsung’s understanding. |
| Fujitsu | Thanks for responses from Samsung and Moderator. We understand the intention of the proposal/TP is to only count the TBs of the last PDSCH (regardless of valid/invalid). And we are fine with the intention. But it seems we still need to further elaborate our thinking. Our question is kind of about how we should interpret the number of TBs/PDSCH in a PDSCH reception occasion in case of time domain bundling. That may have impact on the wording of the TP. If the “the number of transport blocks the UE receives in PDSCH reception occasion m” only includes the total TBs of all PDSCHs of a row, the current wording is fine. But if it includes TBs of only the last PDSCH, it seems unnecessary and misleading to mention the PDSCHs other than the last PDSCH in the TP. Then, it may be better to remove that part as below. - if enableTimeDomainHARQ-Bundling is provided for a serving cell c, for a DCI format indicating a TDRA row that includes more than one SLIV entry on the serving cell c, a PDSCH associated with the last SLIV is considered as received, a PDSCH other than the PDSCH associated with the last SLIV is considered as not received. |
| ZTE, Sanechips | We agree to add vivo’s modification, but it seems that updated proposal is inconsistent with the current TP. TP don’t reflect new added modification from vivo on “whether PDSCH is valid or not”. |
| vivo | @Moderator: Thanks for reflecting our comment. We still prefer to reflect the updated text in the TP as ZTE also indicates. It seems that we share the same understanding and several companies also indicates that the clarification is needed. We don’t agree with Samsung that current spec is clear enough on this by the wording “a PDSCH associated with the last SLIV is considered as received” only. Actually, how to determine the number of TBs is not clear especially when the last PDSCH is invalid with no actual TB received. Based on this, we suggest to reflect the updated text in the TP as well to avoid potential confusing. |
| Moderator | @ Fujitsu, ZTE, vivo, and all, TP is proposed as an example to be able to relieve editor’s efforts. However, in case we cannot converge on the TP, it would be better to focus only on Proposal. As long as the proposal (without TP) is agreeable, we can take the proposal (without TP) as an agreement and leave the TP up to the editor. That is, I would suggest to exclude the TP in this proposal and please provide comments if any concerns only for the proposal. |
| DOCOMO | We are fine with the proposal. Vivo’s modification is also fine for us. |
| Samsung | @Fujitsu, Thanks for the update. If we remove “a PDSCH other than the PDSCH associated with the last SLIV is considered as not received” a PDSCH not associated with the last SLIV is used to count Nm,creceived (since the UE can receive the PDSCH not associated with the last SLIV). This is a reason why we add this sentence. @FL. We are fine with your suggestion that take the proposal without TP and leave the TP up to the editor. We slightly change the wording below to remove the case that a PDSCH not associated with the last SLIV is used to count Nm,creceived. - For type-1 HARQ-ACK codebook, if OACK+OSR+OCSI≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as follows. - For a serving cell c configured with enableTimeDomainHARQ-Bundling, and for a DCI format indicating a TDRA row that includes more than one SLIV entry on the serving cell c, the UE considers a PDSCH associated with the last SLIV as received only, to determine Nm,creceived. - Note that the PDSCH associated with the last SLIV carries one or two transport blocks enabled by the DCI format irrespective of whether the PDSCH is valid or not |
| Ericsson | We support Proposal #3.1-1 without the TP. The TP can be left to the editor. |
| Moderator | @ Samsung, I reflected your comment by adding “only” with yellow highlighted. The other part seems not critical in my view. @ all, Based on Samsung’s comment, to remove the case that a PDSCH not associated with the last SLIV is used to count Nm,creceived, “only” with yellow highlighted is added on top of the previous version. |

During email discussion, the following agreement was made:

- Agreement:

For type-1 HARQ-ACK codebook, if , the UE determines a number of HARQ-ACK information bits  for obtaining a transmission power for a PUCCH, as follows.

- For a serving cell c configured with enableTimeDomainHARQ-Bundling, and for a DCI format indicating a TDRA row that includes more than one SLIV entry on the serving cell c, the UE considers a PDSCH (which carries one or two transport blocks enabled by the DCI format irrespective of whether the PDSCH is valid or not) only associated with the last SLIV as received, to determine .

- Proposal #3.1-2 (Type-2 CB PUCCH power control):

- For type-2 HARQ-ACK codebook, if OACK+OSR+OCSI≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as follows.

  - For a serving cell c configured with numberOfHARQ-BundlingGroups and with NHARQ-ACKTBG,max=1, Nm,creceived in nHARQ-ACK,TB formula can be determined based on the number of DCI formats or the number of transport blocks.

    - FFS details on Nm,creceived determination

  - If the UE is provided numberOfHARQ-BundlingGroups with NHARQ-ACKTBG,max>1 for a serving cell c, or the UE is not provided numberOfHARQ-BundlingGroups but configured with multi-PDSCH scheduling for a serving cell c, nHARQ-ACK is the summation of nHARQ-ACK,TB and nHARQ-ACK,multi where nHARQ-ACK,multi can be determined based on multi-PDSCH scheduling DCI.

    - FFS details on nHARQ-ACK,multi determination

[Moderator’s note] Instead of diving into text proposal, it would be better to have an intermediate proposal so that we can discuss further details in the next meeting. Companies are encouraged to provide views on Proposal #3.1-2.

| Company | Views |
| --- | --- |
| Intel | We are supportive to the proposal |
| vivo | Support the proposal in principle. For the first sub-bullet, “the number of transport blocks” may be misleading. Since time domain bundling is applied, the granunarity of HARQ-ACK reporting should be a bundling group other than a transport block carried by a valid PDSCH. The following modification can be considered. - For a serving cell c configured with numberOfHARQ-BundlingGroups and with NHARQ-ACKTBG,max=1, Nm,creceived in nHARQ-ACK,TB formula can be determined based on the number of DCI format(s) detected in PDCCH monitoring occasion m or the number of enabled transport blocks based on DCI format(s) detected in PDCCH monitoring occasion m. FFS details on Nm,creceived determination |
| Samsung | We support that type-2 CB power control should be updated to cover time-domain bundling case. But, we suggest to defer the dicussion to the next meeting. Specification impacts is unclear to us because the proposal has FFS points to be address (it is not preferred to agree the proposal with FFS in the maintanence phase). Also, companies does not prepare TP for this issue. There are several remaining issues to be discussed over ~3 meetings (for example, OoO, SPS/CG, collision handling, configured vs valid). We need to conclude these issues with higher priority. Since this issues is independent to RRC, it would be safer to be further checked in the next meeting. |
| Futurewei | The main bullets of this proposal look okay. But we agree that it is better to focus on other essential issues for this meeting instead of seeking an agreement for this one with an FFS during the maintenance phase. The issue may be deferred into the next meeting. |
| ZTE, Sanechips | Support the proposal |
| Ericsson | We also think it would be better to have a complete solution (without FFSs), hence it might be better to revisit this next meeting. |
| Moderator | Let’s revisit this issue in the next meeting. |

Time domain bundling

| Company | Views for type-1 HARQ-ACK codebook |
| --- | --- |
| [2] Huawei | Proposal 3: There is no need to further restrict on last SLIV of multi PDSCH scheduling when enableTimeDomainHARQ-Bundling is configured for type-1 HARQ codebook. |
| [4] vivo | Proposal 7: For multi-PDSCH scheduling, when configured with Type-1 codebook and time domain bundling is enabled, UE does not expect more than one TDRA rows mapped to a same candidate PDSCH reception occasion is actually scheduled. |
| [5] Fujitsu | Proposal 1: When time domain bundling for Type-1 HARQ-ACK codebook is enabled, it is unnecessary to introduce the restriction that UE does not expect the last scheduled SLIV overlaps with a semi-static UL symbol. |
| [7] Nokia | Proposal 2: Last scheduled SLIV can overlap with a semi-static UL symbol also when parameter enableTimeDomain-HARQ-Bundling is configured. |
| [9] NTT DOCOMO | Proposal 6: The restriction “UE does not expect the last scheduled SLIV overlaps with a semi-static UL symbol when parameter enableTimeDomainHARQ-Bundling is configured” is not needed. |
| [10] ZTE | Proposal 3: In order not to introduce scheduling flexibility restrictions, the pruning procedure can be optimized when time domain bundling for Type 1 HARQ-ACK codebook is enabled. - The last configured invalid SLIV can be remove from the set of SLIVs used to determine the set of candidate PDSCH reception occasion and each of the removed last configured invalid SLIV corresponds one candidate PDSCH reception occasion. - After removing the last configured invalid SLIV, pruning procedure of Rel-16 can be reused to determine the remaining candidate PDSCH reception occasion. |
| [13] OPPO | Proposal 3: Remove the last bullet in the agreement, i.e., “FFS: UE does not expect the last scheduled SLIV overlaps with a semi-static UL symbol when parameter enableTimeDomainHARQ-Bundling is configured.” |
| [14] Intel | Proposal 6: If time bundling is configured, - For Type1 HARQ-ACK codebook, it is allowed that the last scheduled SLIV overlaps with a semi-static UL symbol. - For Type2 HARQ-ACK codebook, the bundling groups are allocated based on the configured SLIVs of the indicated TDRA row. - Agree on the TP 3 to handle the HARQ-ACK bundling when Type-2 HARQ-ACK codebook is configured. |
| [15] Ericsson | Proposal 6 Remove the entire FFS bullet in the agreement for Type-1 HARQ-ACK codebook enhancement for multi-PDSCH scheduling with a single DCI. |
| [16] Apple | Proposal 5: The following FFS is not needed in the agreement in RAN1 #107-e: UE does not expect the last scheduled SLIV overlaps with a semi-static UL symbol when parameter enableTimeDomainHARQ-Bundling is configured. |
| Company | Views for type-2 HARQ-ACK codebook |
| [2] Huawei | Proposal 4: Support to allocate PDSCHs corresponding to configured SLIVs in a TDRA row index indicated by multi-PDSCH scheduling DCI to the bundling groups. |
| [4] vivo | Proposal 10: For multi-PDSCH scheduling, when configured with Type-2 codebook and time domain bundling is enabled, division of bundling groups is based on valid SLIVs of the indicated TDRA row index. |
| [7] Nokia | Proposal 4: In HARQ-ACK time domain bundling for type-2 codebook, allocation of PDSCHs corresponding to the configured SLIVs in a TDRA row indicated by the scheduling DCI to the transport block groups is slightly preferred. |
| [8] Samsung | Proposal 17: Support to use valid PDSCH-based grouping for Type-2 HARQ-ACK CB with time-domain bundling |
| [12] Qualcomm | Proposal 3: For generating type-2 HARQ-ACK codebook, the formation of the bundling groups should be based on the valid SLIVs Proposal 4: Allowing different numbers of A/N bits per multi-PDSCH grant, such that for each A/N occasion all the corresponding multi-PDSCH grants will have the same A/N bits, however, from one A/N occasion to another we can allow different number A/N bits per grant - If time domain bundling is enabled, then the bundling pattern can be changed from one A/N occasion to another. - Time-domain bundling patterns to be defined via RRC configuration and the active pattern can be changed by MAC-CE or PDCCH. |
| [14] Intel | Proposal 6: If time bundling is configured, - For Type1 HARQ-ACK codebook, it is allowed that the last scheduled SLIV overlaps with a semi-static UL symbol. - For Type2 HARQ-ACK codebook, the bundling groups are allocated based on the configured SLIVs of the indicated TDRA row. - Agree on the TP 3 to handle the HARQ-ACK bundling when Type-2 HARQ-ACK codebook is configured. |
| [15] Ericsson | Proposal 7 For configurable time domain bundling for Type-2 HARQ-ACK codebook, where the number of HARQ bundling groups is indicated by a RRC parameter, grouping of PDSCHs is based on valid SLIVs. |
| [16] Apple | Proposal 6: For multi-PDSCH scheduling with a single DCI and Type 2 HARQ-ACK codebook, bundling is based on the number of configured PDSCHs and not valid PDSCHs. |
| [18] NEC | Proposal 3: The PDSCHs corresponding to valid SLIVs in a TDRA row index indicated by multi-PDSCH scheduling DCI are allocated to the bundling groups |
| [19] MediaTek | Proposal 1: For Type-2 codebook construction with bundling groups, the PDSCHs corresponding to configured SLIVs in a TDRA row index indicated by multi-PDSCH scheduling DCI are allocated to the bundling groups by reusing CBG grouping method. |
| [20] LG Electronics | Proposal #10: The PDSCHs corresponding to “configured” SLIVs in a TDRA row index indicated by multi-PDSCH scheduling DCI are allocated to the bundling groups. |
| Company | Views for type-3 HARQ-ACK codebook |
| [14] Intel | Proposal 8 Time domain bundling can be applied to Type-3 HARQ-ACK codebook. - HARQ-ACK bits of two or more consecutive HARQ processes that are scheduled by the same DCI can be bundled. |

- [Closed] Issue 3.2-1) Remaining issue on type-1 HARQ-ACK codebook with time domain bundling

Company views on remaining issue of type-1 HARQ-ACK codebook with time domain bundling:

Agreement (RAN1#107-e)

For multi-PDSCH scheduling with a single DCI

- Introduce a new RRC parameter, e.g., enableTimeDomainHARQ-Bundling, to enable time domain bundling operation for type-1 HARQ-ACK codebook per serving cell.

  - If the RRC parameter enables time domain bundling operation,

    - To determine the set of candidate PDSCH reception occasions,

      - A row index is removed if at least one symbol of every PDSCH associated with the row index is configured as semi-static UL. (NOTE: This is similar to the case of slot aggregated PDSCH in Rel-16)

      - Pruning procedure in Rel-16 is performed based on the last configured SLIV of each row index.

    - Logical AND operation is applied across all valid PDSCHs associated with a determined candidate PDSCH reception occasion, at least for 1-TB case.

    - FFS: UE does not expect the last scheduled SLIV overlaps with a semi-static UL symbol when parameter enableTimeDomainHARQ-Bundling is configured

- Remove the entire bullet “FFS: UE does not expect the last scheduled SLIV overlaps with a semi-static UL symbol when parameter enableTimeDomainHARQ-Bundling is configured” in the above previous agreement

  - Supported by Huawei, vivo, Fujitsu, Nokia, NTT DOCOMO, OPPO, Intel, Ericsson, Apple

Based on the large number of supports to remove the whole FFS bullet in previous agreement, the following proposal can be made.

Proposal #3.2-1 (Time domain bundling for type-1 codebook):

- Update the previous agreement made in RAN1#107-e, as follows:

Agreement (RAN1#107-e)

For multi-PDSCH scheduling with a single DCI

- Introduce a new RRC parameter, e.g., enableTimeDomainHARQ-Bundling, to enable time domain bundling operation for type-1 HARQ-ACK codebook per serving cell.

  - If the RRC parameter enables time domain bundling operation,

    - To determine the set of candidate PDSCH reception occasions,

      - A row index is removed if at least one symbol of every PDSCH associated with the row index is configured as semi-static UL. (NOTE: This is similar to the case of slot aggregated PDSCH in Rel-16)

      - Pruning procedure in Rel-16 is performed based on the last configured SLIV of each row index.

    - Logical AND operation is applied across all valid PDSCHs associated with a determined candidate PDSCH reception occasion, at least for 1-TB case.

    - FFS: UE does not expect the last scheduled SLIV overlaps with a semi-static UL symbol when parameter enableTimeDomainHARQ-Bundling is configured

Companies are encouraged to provide views on Proposal #3.2-1.

| Company | Views |
| --- | --- |
| Xiaomi | Agree with the proposal |
| Qualcomm | We agree with the proposal |
| InterDigital | We are fine with this proposal. |
| Vivo | We support the proposal |
| Panasonic | We are fine with the proposal #3.2-1. |
| DOCOMO | Support the proposal. |
| CATT | We support the proposal |
| Intel | We support the FL proposal |
| Lenovo, Motorola Mobility | We support the Proposal #3.2-1 |
| Fujitsu | We agree with the proposal. |
| Samsung | Fine with the proposal |
| Nokia, NSB | We support the proposal |
| Apple | We are fine with the proposal. |
| Futurewei | We support Proposal #3.2-1. |
| ZTE，Sanechips | We agree with the proposal. However, the scheduling flexibility also should be considered. For multi-PDSCH scheduling with a single DCI, the last configured SLIV of each row index which contains at least one valid SLIV is participate in the pruning procedure even though the last configured SLIV is invalid and the last configured SLIV is not transmitted actually by gNB. According to the Figure 1 mentioned in RAN1 #107-e email discussion, gNB cannot schedule both row 1 and row 2 according to the pruning procedure based on the last configured SLIV even though gNB can schedule both row 1 and row 2. Figure 1 example for pruning procedure based on the last configured SLIV In order not to introduce scheduling flexibility restrictions, the pruning procedure can be optimized when time domain bundling for Type 1 HARQ-ACK codebook is enabled and the last configured SLIV is invalid. The last configured invalid SLIV can be removed from the set of SLIVs used to determine the set of candidate PDSCH reception occasion and each of the removed last configured invalid SLIV corresponds to one candidate PDSCH reception occasion. After removing the last configured invalid SLIV, pruning procedure of Rel-16 can be reused to determine the remaining candidate PDSCH reception occasion. |
| MediaTek | We support the proposal. To ZTE: In our view, the issue can be resolved if the OOO restriction is applied for all the configured PDSCHs. In that case, then the example in the figure is an error case. |
| Ericsson | Support Proposal #3.2-1 |
| OPPO | We support Proposal #3.2-1. |
| Huawei, HiSilicon | Support the proposal |
| Moderator | @ ZTE, I agree with ZTE in that there could be a scheduling restriction, but at the same time, it seems optimization since it may not be a problem if gNB configures another row 2 that can schedule slots n-3 and n-2 (not collided with UL symbols). Proposal #3.2-1 seems stable and will be reported for email approval. |

During email discussion, the following agreement was made:

- Agreement:

- Update the previous agreement made in RAN1#107-e, as follows:

Agreement (RAN1#107-e)

For multi-PDSCH scheduling with a single DCI

- Introduce a new RRC parameter, e.g., enableTimeDomainHARQ-Bundling, to enable time domain bundling operation for type-1 HARQ-ACK codebook per serving cell.

  - If the RRC parameter enables time domain bundling operation,

    - To determine the set of candidate PDSCH reception occasions,

      - A row index is removed if at least one symbol of every PDSCH associated with the row index is configured as semi-static UL. (NOTE: This is similar to the case of slot aggregated PDSCH in Rel-16)

      - Pruning procedure in Rel-16 is performed based on the last configured SLIV of each row index.

    - Logical AND operation is applied across all valid PDSCHs associated with a determined candidate PDSCH reception occasion, at least for 1-TB case.

    - FFS: UE does not expect the last scheduled SLIV overlaps with a semi-static UL symbol when parameter enableTimeDomainHARQ-Bundling is configured

- Issue 3.2-2) Remaining issue on type-2 HARQ-ACK codebook with time domain bundling

Agreement (RAN1#107-e)

For multi-PDSCH scheduling with a single DCI

- Introduce a new RRC parameter, e.g., numberOfHARQ-BundlingGroups, to configure the number of HARQ bundling groups with value range {1, 2, 4} for type-2 HARQ-ACK codebook per serving cell.

  - If the RRC parameter is not configured for a serving cell, time domain bundling for type-2 HARQ-ACK codebook is not enabled for the serving cell.

  - The maximum number of PDSCHs allocated to each bundling group is ceil(NPDSCH,MAX/NHBG) where NHBG is the number of bundling groups configured by numberOfHARQ-BundlingGroups for a serving cell and NPDSCH,MAX is the maximum configured number of PDSCHs for the serving cell.

  - The PDSCHs corresponding to [configured or valid] SLIVs in a TDRA row index indicated by multi-PDSCH scheduling DCI are allocated to the bundling groups, e.g., if NHBG =4, NPDSCH,MAX =8, and 5 PDSCHs are scheduled, then 2/1/1/1 PDSCHs are assigned to each group, by reusing CBG grouping method.

    - For a group that is empty or is filled with only invalid PDSCH(s), HARQ-ACK bits for the bundling group is set to NACK (same principle as when no time bundling configured)

    - Logical AND operation is applied to across all valid PDSCHs within the same bundling group to generate 1 HARQ-ACK bit per group, at least for 1-TB case

  - If the number of HARQ bundling groups is configured as 1 for a serving cell, HARQ-ACK bits corresponding to any DCI for the serving cell belong to the first sub-codebook.

  - At least for 1-TB case, if the number of HARQ bundling groups is configured as larger than 1 for a serving cell, HARQ-ACK bits corresponding to multi-PDSCH scheduling case (which implies a multi-PDSCH DCI schedules more than one PDSCH) for the serving cell belong to the second sub-codebook,

    - Where the number of HARQ-ACK bits corresponding to a multi-PDSCH DCI is determined based on the maximum of Q value across all serving cells within the same PUCCH cell group, and Q=maximum configured number of PDSCHs for a cell without numberOfHARQ-BundlingGroups configured or Q=number of configured HARQ bundling groups for a cell with numberOfHARQ-BundlingGroups configured

Company views on between configured and valid in the above agreement:

- Construction of bundling group based on “configured” SLIVs

  - Supported by Huawei, Nokia, Intel, Apple, MediaTek, LG Electronics

- Construction of bundling group based on “valid” SLIVs

  - Supported by vivo, Samsung, Qualcomm, Ericsson, NEC,CATT

[Moderator’s note] The following points can be observed from Moderator’s point of view:

- “Based on configured SLIVs” is supported by 6 companies while “Based on valid SLIVs” is supported by 5 companies

- Proponents of “Based on configured SLIVs” prefer the common rule with HARQ-ACK bit ordering (which is also based on configured SLIV position) for simple UE implementation.

- Proponents of “Based on valid SLIVs” point out that it takes advantage of reducing the number of unnecessary retransmissions.

Given diverged view between two options, it is encouraged for companies to provide views on the above options, if any.

| Company | Views |
| --- | --- |
| Xiaomi | Prefer “Based on configured SLIVs”. And can go with majority |
| Qualcomm | We prefer considering only the valid SLIVs due to its technical benefit |
| Panasonic | Our preference is “Based on configured SLIVs”. |
| DOCOMO | We prefer “Based on valid SLIVs” due to higher efficiency. |
| CATT | We prefer valid SLIV |
| Intel | We prefer to use ‘configured SLIVs’ for simplicity. With a fixed pattern of bundling groups by ‘configured SLIVs’, gNB can control a reasonable distance between the two PDSCHs in the same bundling group in the configuration of TDRA. However, if valid SLIVs are used, the distance of the two PDSCHs in the same bundling group may be quite large depending on the dynamic scheduling. Note: the lager the distance, the lower the correlation between decoding results of the two PDSCHs which may impact the efficiency for PDSCH transmission. |
| Lenovo, Motorola Mobility | We prefer “valid’ SLIVs |
| Fujitsu | Prefer “based on configured SLIVs”. We do not quite understand the issue of unnecessary retransmissions raised by some companies. According to the agreement, it seems only valid PDSCHs are considered for generating bundled HARQ-ACK information bits. Even if HARQ-ACK information of invalid PDSCH(s) is bundled with that of valid PDSCHs, we could simply assume ACK for the invalid PDSCH(s). |
| Samsung | Prefer to use valid SLIVs due to its technical benefit @Fujitsu. Please check the following toy example. Consider 8 configured SLIVs {1,2,3,4,5,6,7,8} but last 2 SLIVs {7,8} are invalid. If the number of bundling group is configured with 4 then, UE makes the following bundling groups - Based on configured SLIVs: {1,2}, {3,4}, {5,6}, {7,8} - Based on valid SLIVs: {1,2}, {3,4}, {5}, {6} If a PDSCH corresponding to SLIV5 is NACKed, then gNB may re-transmit - Based on configured SLIVs: two PDSCHs corresponding to SLIV5 and SLIV6 since the 3rd HARQ-ACK bit for {5,6} is NACK - Based on valid SLIVs; one PDSCH corresponding to SLIV5 since the 3rd HARQ-ACK bits for {5} is NACK. |
| Nokia, NSB | We expect that HARQ-ACK time domain bundling is used primarily when the time domain bundling causes only negligible increase in unnecessary retransmissions when all configured SLIVs are also valid SLIVs – otherwise the configuration would be just hurting the performance. Hence, we do not expect any tangible benefit from the use of valid SLIVs and, hence, prefer the simpler option of configured SLIVs |
| Apple | Prefer based on “configured SLIVs” due to simplicity. |
| Futurewei | We slightly prefer “based on configured SLIVs” for simplicity. |
| ZTE, Sanechips | We slightly prefer to support “Based on configured SLIV” |
| MediaTek | We support the proposal of configured SLIV. If valid PDSCH is considered, some specification efforts are needed to implement the bundling operation based on valid PDSCHs. At least, the ordering of the HARQ-ACK information corresponding to the valid PDSCHs should be determined. For example, when a DCI schedules four PDSCHs and the third one is invalid, if the number of bundling group is 4, assuming all the three valid PDSCHs are decoding correctly, then there are at least two possible HARQ-ACK information reporting: Alt1:(ACK, ACK,ACK,NACK) if only the valid PDSCHs are considered for HARQ bit ordering or Alt2:(ACK,ACK,NACK,ACK) if both valid and invalid PDSCHs are considered for HARQ bit ordering. If Alt1 is adopted, then it will not aligned with the agreement RAN1 made in RAN1#107-e meeting when no bundling is considered: Agreement - For type-2 HARQ-ACK codebook generation, HARQ-ACK bit ordering is based on configured SLIV position in the indicated TDRA row index, regardless of the validity of each scheduled PDSCH. If Alt2 is adopted, then the Rel-15/16 CBG grouping method can’t be directly reused and some enhancement is necessary |
| Ericsson | We prefer based on valid SLIVs due to the technical merit that the re-transmission efficiency is better without any cost. We don't agree that MediaTek's Alt-1 goes against the RAN1#107-e agreement since that agreement corresponds to the case of no bundling. We don't understand MediaTek's Alt-2 – this is not the way grouping based on valid SLIVs is intended to work. The HARQ bundling groups are filled with ACK/NACK due to valid SLIVs first starting from the lowest indexed bundling group. |
| NEC | We prefer “Based on valid SLIVs” due to higher transmission efficiency. |
| vivo | We prefer “Based on valid SLIVs” When division of bundling groups is based on valid SLIVs, valid SLIVs can be divided into different bundling groups more even, therefore number of valid SLIVs divided into any bundling group may be smaller, compared to division based on configured SLIVs. Consequently, division of bundling groups based on valid SLIVs may offer better re-transmission efficiency. From the perspective of re-transmission performance, dividing bundling groups based on valid SLIVs is preferred. |
| OPPO | Prefer “based on configured SLIVs”. |
| Fujitsu2 | Thanks for the clarification from Samsung. Based on the discussion, we slightly prefer “based on configured SLIVs” for simplicity and considering it may well be a corner case that a part of bundled valid PDSCHs is NACK. |
| Huawei, HiSilicon | We prefer “configured SLIV” due to its relative fixed HARQ bit allocation which facilitate implementation. |
| Moderator | - Construction of bundling group based on “configured” SLIVs - Supported by (12) Huawei, Nokia, Intel, Apple, MediaTek, LG Electronics, Xiaomi, Panasonic, Fujitsu, Futurewei, ZTE, OPPO - Construction of bundling group based on “valid” SLIVs - Supported by (8) vivo, Samsung, Qualcomm, Ericsson, NEC, CATT, NTT DOCOMO, Lenovo Thanks for the technical discussions. I think two options have pros and cons, depending on which SLIVs are invalid. - Samsung’s example - Consider 8 configured SLIVs {1,2,3,4,5,6,7,8} but last 2 SLIVs {7,8} are invalid. If the number of bundling group is configured with 4 then, UE makes the following bundling groups - Based on configured SLIVs: {1,2}, {3,4}, {5,6}, {7,8} - Based on valid SLIVs: {1,2}, {3,4}, {5}, {6} - If a PDSCH corresponding to SLIV5 is NACKed, then gNB may re-transmit - Based on configured SLIVs: two PDSCHs corresponding to SLIV5 and SLIV6 since the 3rd HARQ-ACK bit for {5,6} is NACK - Based on valid SLIVs; one PDSCH corresponding to SLIV5 since the 3rd HARQ-ACK bits for {5} is NACK. - [Moderator’s note] Since 4 bundling groups are configured, in most cases, SLIV5 and SLIV6 may have the same decoding results, i.e., all ACK or all NACK. - Another example - Consider 8 configured SLIVs {1,2,3,4,5,6,7,8} but 2 SLIVs {2,3} are invalid. If the number of bundling group is configured with 4 then, UE makes the following bundling groups - Based on configured SLIVs: {1,2}, {3,4}, {5,6}, {7,8} - Based on valid SLIVs: {1,4}, {5,6}, {7}, {8} - [Moderator’s note] Since SLIV1 and SLIV4 are included in the same group but the distance between them becomes larger than that between SLIV1 and SLIV2, the probability of having different decoding results for SLIV1 and SLIV4 could be increased. @ all, Considering above examples, slightly majority view, and the simplicity, could we go with “based on configured SLIV”? |
| Xiaomi | Yes, we support “configured SLIV” |
| DOCOMO | We can accept “based on configured SLIV” for progress. |

- Proposal #3.2-2 (Time domain bundling for type-2 codebook):

- Update the previous agreement made in RAN1#107-e, as follows:

Agreement (RAN1#107-e)

For multi-PDSCH scheduling with a single DCI

- Introduce a new RRC parameter, e.g., numberOfHARQ-BundlingGroups, to configure the number of HARQ bundling groups with value range {1, 2, 4} for type-2 HARQ-ACK codebook per serving cell.

  - If the RRC parameter is not configured for a serving cell, time domain bundling for type-2 HARQ-ACK codebook is not enabled for the serving cell.

  - The maximum number of PDSCHs allocated to each bundling group is ceil(NPDSCH,MAX/NHBG) where NHBG is the number of bundling groups configured by numberOfHARQ-BundlingGroups for a serving cell and NPDSCH,MAX is the maximum configured number of PDSCHs for the serving cell.

  - The PDSCHs corresponding to [configured or valid] SLIVs in a TDRA row index indicated by multi-PDSCH scheduling DCI are allocated to the bundling groups, e.g., if NHBG =4, NPDSCH,MAX =8, and 5 PDSCHs are scheduled, then 2/1/1/1 PDSCHs are assigned to each group, by reusing CBG grouping method.

    - For a group that is empty or is filled with only invalid PDSCH(s), HARQ-ACK bits for the bundling group is set to NACK (same principle as when no time bundling configured)

    - Logical AND operation is applied to across all valid PDSCHs within the same bundling group to generate 1 HARQ-ACK bit per group, at least for 1-TB case

  - If the number of HARQ bundling groups is configured as 1 for a serving cell, HARQ-ACK bits corresponding to any DCI for the serving cell belong to the first sub-codebook.

  - At least for 1-TB case, if the number of HARQ bundling groups is configured as larger than 1 for a serving cell, HARQ-ACK bits corresponding to multi-PDSCH scheduling case (which implies a multi-PDSCH DCI schedules more than one PDSCH) for the serving cell belong to the second sub-codebook,

    - Where the number of HARQ-ACK bits corresponding to a multi-PDSCH DCI is determined based on the maximum of Q value across all serving cells within the same PUCCH cell group, and Q=maximum configured number of PDSCHs for a cell without numberOfHARQ-BundlingGroups configured or Q=number of configured HARQ bundling groups for a cell with numberOfHARQ-BundlingGroups configured

Companies are encouraged to provide views on Proposal #3.2-2.

| Company | Views |
| --- | --- |
| vivo | We don’t support this proposal. As many companies discussed, ‘based on valid PDSCHs’ has benefits over ‘based on configured PDSCHs’ on retransmission efficiency. However, we don’t see and hear any benefit for ‘based on configured PDSCHs’. |
| Moderator | @ vivo, Please see my notes above Proposal #3.2-2. @ all, I’ve checked the current 213 specification and it seems to be based on “configured” SLIV. If a UE is provided numberOfHARQ-BundlingGroups for a serving cell c, the UE generates HARQ-ACK information over transport block groups (TBGs) for PDSCH receptions where, for a maximum number of NPDSCHmax PDSCH receptions scheduled by a DCI format on the serving cell, a maximum number of TBGs NHARQ-ACK,cTBG,max is provided by numberOfHARQ-BundlingGroups. If the UE detects a DCI format scheduling NPDSCH,c PDSCH receptions on the serving cell c, the UE generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for first TBs and, if applicable, generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for second TBs in the NPDSCH PDSCH receptions as described in clause 9.1.1 by setting NHARQ-ACKCBG/TB,max=NHARQ-ACK,cTBG,max and C=NPDSCH,c. |
| Intel | We prefer bundling group generation by configured SLIVs. As we commented in GTW session, the typical use case for multiple bundling groups should be that the correlation between decoding of the scheduled PDSCHs are not that large, otherwise, gNB can simply configure single bundled bit for all scheduled PDSCHs. Therefore, it becomes quite important to provide gNB a means to fine control the bundling operation since it impacts the correlation of PDSCHs in a budnling group. With configured SLIVs based bundling group division, the distance among PDSCHs in a bundling group is fixed and is not impacted by dynamic scheduling configuring TDD UL-DL configuraitons. On the other hand, as analysized in the second example from moderator, the distance between PDSCHs in a bundling group may become quite larger which results in bad correclation and increased PDSCH retransmissions. |
| Lenovo, Motorola Mobility | As as compromise, we are fine to agree with the proposal stating “configured SLIVs”. Both options seem to work and with the understanding that current spec suggests “configured” SLIV as pointed out by moderator, we are fine. |
| vivo | Thanks moderator for more explanation. On performance aspect, we agree that in the 2nd exmpale ‘based on configured SLIV’ is better than ‘based on valid SLIV’ if SLIV 1 and SLIV4 have different decoding result. However, in the example, ‘based on valid SLIV’ is better than ‘based on configured SLIV’ if SLIV 7 and SLIV 8 have different decoding result. So for the 2nd example, the two alternatives is similar with respective pros and cons. However, for the 1st example, ‘based on valid SLIV’ is better than ‘based on configured SLIV’ in all cases. On spec impact aspect, we don’t agree that the spec capature all aspects for ‘based on configured SLIV’. The generation of HARQ-ACK is referred to CBG section 9.1.1 in current spec. However, CBG group doesn’t consider invalid CBG. So at least the following cases should be reflected in spec: Case 1: all PDSCHs in one bundling group are invalid Case 2: at least one of the PDSCHs in one bundling group is invalid and how to calculate the bundling HARQ ACK If ‘based on valid PDSCH’, the only change is to add valid before PDSCH receptions. In general, ‘based on valid SLIV’ has slight better performance than ‘based on configured SLIV’ with less spec impact. |
| Samsung | Regarding the current 213 specification, the specification says “scheduling NPDSCH,c PDSCH receptions”. As we discussed in GTW, “scheduling” is not clear so that if we agree to support either of “configured SLIV” or “valid SLIV”, we expect further specification works are needed. Note that some companies already submited a TP for “configured SLIV”. In our tdoc (R1-2200196), we also provide a TP for “valid SLIV” as follows: “If the UE detects a DCI format scheduling NPDSCH,c PDSCH receptions, not overlapping with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, on the serving cell c, the UE generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for first TBs and, if applicable, generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for second TBs in the NPDSCH,c PDSCH receptions as described in clause 9.1.1 by setting NHARQ-ACKCBG/TB,max=NHARQ-ACK,cTBG,max and C=NPDSCH,c.” Regading the moderator’s note, please check our understanding. The 1st moderator’s note: in general, two adjacent SLIVs may experience similar channel so that the moderator argued that the decoding results of two adjacent SLIVs are highly correlated. But, if we consider that there are no agreements to limit a gap between two adgacent SLIVs, the decoding results of two adjacent SLIVs might be less correlated. Furthermore, if we consider initially transmitted PDSCH and retransmitted PDSCH, the decoding results of two PDSCHs (even if two are consecutive) are less correlated. So, valid SLIV based construction can prevent gNB from re-transmitting unnessary PDSCHs. The 2nd moderator’s note: please consider all cases not for a corner case. We analzed all cases with 2 consecutive invalid SLIVs and 3 consecutive invalid SLIVs in our tdoc (R1-2200196). For reference, the “yellow” marked bundling group is empty as a result of configured SLIV based construction and the “cyan” marked bundling group is a group with non-consecutive SLIVs. Based on these analysis, we observe that - For 2 consecutive invalid PDSCHs, 4 cases include empty bundling group, but 2 cases include non-consecutive SLIVs - For 3 consecutive invalid PDSCHs, 6 cases include empty bundling group, but only 1 case includes non-consecutive SLIVs Also, even though two consecutive SLIVs in the same bundling group, we mention in the 1st moderator’s note, decoding results of two consecutive SLIVs can be less correlated. So, for 2 or 3 consecutive invalid PDSCHs, in average sense, the valid SLIV is not worse than the configured one. Also, its benefit becomes significant when more SLIVs are invalid. |
| Huawei, HiSilicon | From performance perspective, “valid” and “configured” has different advantage scenario and difficult to say which one better. From implementation perspective, “configured” solution is simpler as it does not depend on the validation of SLIV which might be dynamcically changed. According to editor note, the current spec assume “configured”. Based on the above 3 points, we prefer to general time bundling base on the “configured SLIV” |
| Moderator | - Alt 1: Construction of bundling group based on “configured” SLIVs - Supported by (13) Huawei, Nokia, Intel, Apple, MediaTek, LG Electronics, Xiaomi, Panasonic, Fujitsu, Futurewei, ZTE, OPPO, Lenovo (acceptable) - Alt 2: Construction of bundling group based on “valid” SLIVs - Supported by (8) vivo, Samsung, Qualcomm, Ericsson, NEC, CATT, NTT DOCOMO, Lenovo Thanks Lenovo for your spirit to progress! The followings can be observed based on discussions so far. - Either of alternatives can work and system is not broken. - Technical benefit: Two alternatives may have a benefit in some scenarios, but overall, Alt 2 can provide advantage in more cases. Companies have different views on whether this advantage is marginal or not. - Specification impact: It is clarified that the current 213 spec is written based on Alt 1 but either of alternatives may require additional specification work. - UE implementation: Alt 1 can bring an advantage in terms of simplified UE implementation, since UE doesn’t need to check the validity of each PDSCH and can have a common imiplementation irrespective of whether time bundling is configured or not. Considering the above all aspects, I couldn’t conclude that an alternative is absolutely more beneficial than the other alternative. With that, I encourage companies to converge on Proposal #3.2-2, as it is, based on slight majority view. Otherwise, I’m not sure we can make a consensus on this issue in this meeting. Please provide comments if you have a strong concern on Proposal #3.2-2. In that case, please also provide any potential way forward for us to be able to converge. |
| vivo | Thanks moderator for the summary. We agree with the first two bullets for the observations. For the 3rd bullet, we think Alt. 1 requires more spec impact. Agree with Samsung that, scheduling in spec is not clear and both alternatives need to change the ‘scheduling’ part. For Alt. 1, as we mentioned in previous comment, the generation of HARQ-ACK is referred to CBG section 9.1.1 in current spec. However, CBG group doesn’t consider invalid CBG. So at least the following cases should be reflected in spec: Case 1: all PDSCHs in one bundling group are invalid Case 2: at least one of the PDSCHs in one bundling group is invalid and how to calculate the bundling HARQ ACK For the 4th bullet, a UE should check whether the PDSCH is valid or invalid anyway when receiving. We don’t think Alt. 2 may bring extra complexity. As a way forward, we suggest to provide complete TPs for both alternatives and then decide which one should be adopted. |
| Samsung | @FL. We appreciate your great efforts on this issue. Unfortunately, we still see the benefit of valid SLIV for bundling group. For the observations, please see our understanding. - Yes. We agree with this. Both options can work. - We provided a concrete analysis on configured SLIV and valid SLIV in the last comments, and hope see an analysis from the proponent of configured SLIV in terms of overall performance, not for a particular case. Also, we would hear why the proponent of configured SLIV think the performance benefit is marginal. - Even though the current specification is written based on Alt 1, it is just text for a placeholder by 213 editor and it should not affect the decision. Clearly, it is an open issue to be decided by RAN1, not decided by editor. We agree that either of alternatives may require addional specification works. We provided the TP for valid SLIV but we cannot see the TP for configured SLIV. For completeness, it is highly preferred to dicusse the TP at this late stage. If the TP for configured SLIV is not ready, it would be better to defer the discussion till the TP for configured SLIV is ready. - For HARQ process ID determination, a UE first determines which SLIVs are valid regardless of two options. After determining the valid SLIVs, two options may have different complexity. For valid SLIVs, the HARQ-ACK information for valid SLIVs is considered to apply HARQ-ACK bundling and any invalid SLIVs are not considered. However, for configured SLIVs, the HARQ-ACK information for both valid SLIVs and invalid SLIVs is considered to apply HARQ-ACK bundling. Also, for configured SLIVs, when applying the HARQ-ACK bundling, the HARQ-ACK information for the invalid SLIV is considered as ACK if at least one SLIV in the bundling group is valid. Otherwise (i.e., all SLIVs in the bundling group are invalid), the HARQ-ACK information for the invalid SLIV is considered as NACK. So, it is not clear Alt1 is more simpler in terms of UE implementation. As a potential way forward, we also suggest to discuss the proposal with TPs for both options and finalize the issue based on overall performance, specification impacts and complexity. |
| Moderator | It seems hard to converge on this issue this meeting. As suggested by Samsung and vivo, it could be better to revisit this issue next meeting, especially with complete TPs for both alternatives. |

[Closed] Remaining issues of Type-2 (dynamic) HARQ-ACK codebook

| Company | Views |
| --- | --- |
| [4] vivo | Proposal 12: For multi-PDSCH scheduling and Type-2 codebook, consider the TP4 in TS38.213 for the case when spatial bundling is configured. |
| [8] Samsung | Observation 3: Including HARQ-ACK bits for 2 PDSCHs scheduled by a DCI in the first HARQ-ACK sub-codebook complicates the specification with marginal gain. Proposal 19: when a UE supports UE capability type2-HARQ-ACK-Codebook (FG 18-9), and there are >1 DCIs belonging to the same Mos and scheduling PDSCHs to the same serving cell. And these DCIs are configured to be able to schedule multiple PDSCHs. The counting procedure for the PDSCHs scheduled by these DCIs are: - PDSCHs are separated into different sets and each set of PDSCHs are scheduled by the same DCI. PDSCHs are counted separately for different sets. - The counting order between different sets of PDSCHs are based on the reception time of the first PDSCH in each set. |
| [9] NTT DOCOMO | Proposal 5: If multiple PDSCHs are scheduled by single DCI, and there is at least one PDSCH collides with semi-static UL symbol, - OoO scheduling limitation is based on valid PDSCHs. - If CBG based transmission is configured, CBGTI/CBGFI fields are present in DCI for the case when multiple PDSCHs are scheduled but only one PDSCH is valid. - DCI scheduling multiple PDSCHs but with only one valid PDSCH is included in the first sub-codebook. - When timeline is satisfied, the SPS PDSCH overlapping with the cancelled DG PDSCH can be received. |
| [12] Qualcomm | Proposal 5: If all PDSCHs scheduled by a DCI that schedules multi-PDSCHs (TDRA row has multiple SLIVs) except one PDSCH will not be transmitted due to overlap with semi-static UL symbols, then A/N bit of the valid PDSCH will be carried in the codebook of fallback and single-PDSCH grants. |
| [13] OPPO | Proposal 4: Spatial domain bundling operation should be applied before time domain bundling operation for 2-TB case. Proposal 5: HARQ-ACK bits corresponding to multi-PDSCH belong to the second sub-codebook only if the number of HARQ bundling groups is configured as larger than 2 for 2-TB case. |
| [14] Intel | Proposal 7: In HARQ-ACK codebook generation, - Spatial bundling and time bundling can be independently configured and enabled/disabled in HARQ-ACK transmission. - The agreement on Type-2 HARQ-ACK codebook generation with single TB per PDSCH applies per TB for a serving cell configured with two TBs per PDSCH. - Agree on the TP 4 to generate the Type-2 HARQ-ACK codebook depending on the configuration of spatial bundling. |
| [18] NEC | Proposal 1: - Handling of spatial bundling is an open question to be discussed for multiple-PDSCH transmission. - Only one function is applied for harq-ACK-SpatialBundling and enableTimeDomainHARQ-Bundling. |
| [20] LG Electronics | Proposal #11: If a UE is provided with numberOfHARQ-BundlingGroups, the UE can be also configured with harq-ACK-SpatialBundlingPUCCH or harq-ACK-SpatialBundlingPUSCH. |

- Issue 3-4) Remaining issues of Type-2 (dynamic) HARQ-ACK codebook

Company views on remaining issues of Type-2 (dynamic) HARQ-ACK codebook:

- Issue 1: Allow separate configuration of spatial bundling from time domain bundling configuration

  - Supported by vivo, Intel, LG Electronics

  - Objected by NEC, OPPO?

- Issue 2: Include HARQ-ACK bits for 2 PDSCHs scheduled by a DCI in the first HARQ-ACK sub-codebook

  - Supported by NTT DOCOMO, Qualcomm

  - Objected by Samsung

[Moderator’s note] Issue 2 had been discussed in previous meetings but majority view was that previous agreement was clear enough to contain HARQ-ACK bits for 2 PDSCHs into the second HARQ-ACK sub-codebook. So, it is proposed to deprioritize Issue 2 in Rel-17.

On the other hand, for Issue 1, RAN1 decision seems necessary. It is observed that current specification already supports separate configuration of time domain bundling and spatial bundling for type-1 HARQ-ACK codebook. Thus, similar to type-1 HARQ-ACK codebook design and considering time domain bundling and spatial bundling can provide two independent functionalities, the following proposal can be made.

Proposal #3.4 (Spatial bundling):

- For multi-PDSCH scheduling with a single DCI and for type-2 HARQ-ACK codebook generation,

  - Time domain bundling and spatial bundling can be independently configured.

Companies are encouraged to provide views on Proposal #3.4.

| Company | Views |
| --- | --- |
| Xiaomi | Agree with proposal. |
| Qualcomm | Agree with the proposal |
| vivo | Support the proposal for flexibility. |
| DOCOMO | Fine with the proposal. |
| CATT1 | Agree with proposal. |
| Intel | We support the FL proposal |
| Lenovo, Motorola Mobility | We support the Proposal #3.4 |
| Fujitsu | We are fine with the proposal. |
| Samsung | Agree |
| Nokia, NSB | We support Moderator’s proposal |
| Apple | Fine with the proposal |
| Futurewei | Ok with Proposal #3.4. |
| ZTE, Sanechips | We support the proposal. |
| Ericsson | Suppoert Proposal #3.4 |
| NEC | We still have a concern on the proposed conclusion. If time domain bundling and spatial bundling are independently configured, then there will be a case that these two functions are simultaneously enabled, and the PDSCH transmission efficiency will be heavily affected, since one NACK will cause re-transmission for all the PDSCHs in the time bundling window. Therefore, it’s better to avoid independently configuring time domain bundling and spatial bundling and enable only one function at a time. |
| OPPO | We are in general fine with the proposal. But we have one clarification question: for 2-TB case, if time domain bundling is configured and spatial domain bundling is not configured, what operation is assumed at UE side? |
| Huawei, HiSilicon | Support the proposal |
| Moderator | @ NEC, Such kind of inefficiency can be overcome by gNB’s configuration (i.e., by not configuring spatial bundling and time domain bundling simultaneously). Since those two functionalies seem to be independent, could we go with majority view? @ OPPO, If time domain bundling is configure but spatial bundling is not configured, UE performs time domain bundling for each TB. In other words, if N bundling groups are configured, N bits per TB are generated and 2N bits are generated in total for a single DCI. I’m not sure if all other companies have the same understanding. |
| OPPO | We are fine with the proposal as long as UE behavior is clear. |

During email discussion, the following agreement was made:

- Agreement:

For multi-PDSCH scheduling with a single DCI and for type-2 HARQ-ACK codebook generation,

- Time domain bundling and spatial bundling can be independently configured.

[Closed] HARQ process

| Company | Views |
| --- | --- |
| [1] Futurwei | Observation 1. Though introduction of 32 HARQ processes with UE capability does not hurt, it is recommended that more discussion on whether ‘uniform design’ alone is a strong enough reason to spend effort on introducing such feature, even if it is subject to UE capability. |
| [2] Huawei | Proposal 2: Support 32 as the maximum number of HARQ processes for DL and UL for 120 kHz SCS, subject to UE capability. |
| [8] Samsung | Observation 4. nrofHARQ-ProcessesForPDSCH (or nrofHARQ-ProcessesForPUSCH) are applied to all UE’s DL BWPs (or UL BWPs) of a serving cell Proposal 21: If nrofHARQ-ProcessesForPDSCH (or nrofHARQ-ProcessesForPUSCH) is configured for a cell, RAN1 decides one of two options: Option 1. - For a DL BWP (or UL BWP) with μ≤3 in a cell, the number of HARQ processes the UE may assume is 16 if the value of nrofHARQ-ProcessesForPDSCH (or nrofHARQ-ProcessesForPUSCH) is larger than 16. - For a DL BWP (or UL BWP) with μ=4 or μ=5 in a cell, the number of HARQ processes the UE may assume is provided nrofHARQ-ProcessesForPDSCH (or nrofHARQ-ProcessesForPUSCH). Option 2. - For all DL BWPs (or UL BWPs) in a cell, the number of HARQ processes the UE may assume is provided nrofHARQ-ProcessesForPDSCH (or nrofHARQ-ProcessesForPUSCH) Proposal 22: If option 1 is supported, adopt TP#9 in Appendix for TS38.214. If option 2 is supported, adopt TP#10 in Appendix for TS38.214 Proposal 23: When (enhanced) Type-3 HARQ-ACK is generated, the number of HARQ processes is determined by the number of HARQ processes configured in the active BWP of a serving cell. |
| [11] Panasonic | Proposal 6: For NR FR2-2 for 120 kHz SCS (in addition to 480/960 kHz), support 32 as the maximum number of HARQ processes for DL and UL, subject to UE capability. |
| [12] Qualcomm | Proposal 1: In case of BWP switching between SCS 120kHz, and 480/960kHz and when different numbers of HARQ processes are configured, consider one of the following options: - Option 1: No retransmission can be allowed over different SCSs. - Option 2: No soft combining is assumed between retransmissions over different SCSs. Proposal 2: To define different numbers of HARQ processes for 480/960kHz SCS and 120kHz SCS, consider one of the following options: - Option 1: Reuse the same parameter in PDSCH-ServingCellConfig and add more values, e.g., 24 and 32. - If UE is configured with more than 16 HARQs and the operating SCS is 120kHz or less, it will assume that number of HARQ processes is 16. - Option 2: Introduce new parameter(s) for SCSs 480kHz/960kHz. |
| [14] Intel | Proposal 10 - Up to 32 HARQ processes can be configured for SCS 120kHz. - The same configured number of HARQ processes applies to all possible SCSs for a FR2-2 cell. - No TP is needed since same number of HARQ processes for BWPs with different numerologies can be configured. |
| [15] Ericsson | Proposal 8 For Type-1 and Type-2 HARQ-ACK codebook generation for multi-PDSCH scheduling, if time domain HARQ bundling is not configured, the UE should report NACK for the feedback-disabled HARQ processes regardless of the decoding results of the corresponding PDSCHs. Proposal 9 For Type-1 and Type-2 HARQ-ACK codebook generation for multi-PDSCH scheduling, if time domain HARQ bundling is configured, - For a group with only feedback-disabled PDSCH(s), HARQ-ACK bits for the bundling group is set to NACK - Logical AND operation is applied across all valid feedback-enabled PDSCHs within the same bundling group, if any, to generate 1 HARQ-ACK bit per group Note: For Type-1 HARQ-ACK codebook, all PDSCHs scheduled by a DCI belong to a single bundling group. Proposal 10 For Type-2 HARQ-ACK codebook, the DAI counters in a DCI that schedule multiple PDSCHs still need to be incremented if any of the PDSCHs scheduled by the DCI are not feedback-disabled. Proposal 11 For Type-2 HARQ-ACK codebook generation for multi-PDSCH scheduling, if all PDSCHs scheduled by a DCI are feedback-disabled, HARQ-ACK feedback for the scheduling should be skipped, regardless of whether time domain bundling is configured or not. |
| [20] LG Electronics | Proposal #12: For 120 kHz SCS (in addition to 480/960 kHz), support 32 as the maximum number of HARQ processes for DL and UL, subject to UE capability. |

- Issue 3-5-1) Increased maximum number of HARQ processes for 120 kHz SCS

Agreement: (RAN1#106bis-e)

For NR FR2-2 at least for 480/960 kHz SCS, support 32 as the maximum number of HARQ processes for DL and UL, subject to UE capability.

- Note: Up to 32 maximal supported HARQ process number is already agreed in Rel-17 NTN WI.

- Working assumption: The same solution to support up to 32 HARQ process number in Rel-17 NTN WI is reused for NR FR2-2.

Company views on supporting 32 as the maximum number of HARQ processes for 120 kHz SCS:

- Supported by Huawei, Panasonic, Intel, LG Electronics,CATT

- Objected by Samsung, Qualcomm

If 32 maximum HARQ processes are not supported for 120 kHz SCS, at least the following issues seems to be resolved, in case a BWP in a serving cell is configured with 120 kHz SCS and another BWP in the serving cell is configured with 480/960 kHz SCS:

- Issue 1: Whether the number of HARQ processes is configured per cell (as in Rel-16) or per BWP/SCS

- Issue 2: Whether to perform data soft combining after BWP switching

- Issue 3: The number of HARQ processes for the serving cell to generate type-3 HARQ-ACK codebook

[Moderator’s note] In order not to create new issues in maintenance phase, and considering a UE capable of 32 HARQ processes for 480/960 kHz SCS would be capable of 32 HARQ processes for 120 kHz SCS as well in FR2-2, it is proposed to support 32 as the maximum number of HARQ processes for 120 kHz SCS. This proposal is indicated as “HIGH” since it might have an impact on RRC parameter.

[HIGH] Proposal #3.5-1 (HARQ process):

- For NR FR2-2 for 120 kHz SCS (in addition to 480/960 kHz), support 32 as the maximum number of HARQ processes for DL and UL, subject to UE capability.

Companies are encouraged to provide views on Proposal #3.5-1.

| Company | Views |
| --- | --- |
| Xiaomi | Agree with the proposal. |
| Qualcomm | We do not see the technical need to have 32HARQ processes for 120kHz, also, the issue of having different numbers of HARQ processes for different SCS will not be solved by this proposal unless the UE capability supporting X HARQ processes is unified over all SCSs. |
| InterDigital | We do not support this proposal. |
| vivo | We do not support the proposal since there is no technical motivation for 120KHz. |
| Panasonic | We support the proposal #3.5-1. |
| DOCOMO | We are open to discuss whether to extend 32 HARQ processes to 120kHz SCS in FR2-2. But we believe that such extension should not be applied to 120kHz SCS in FR 1. |
| CATT | Support the proposal |
| Intel | We support the FL proposal |
| Lenovo, Motorola Mobility | We support the proposal #3.5-1 |
| Fujitsu | We are fine with the proposal. |
| Samsung | For unified structure, we are ok to support 32 HARQ processes for 120kHz SCS in FR2_2. However, it is unclear that the unified structure should be prioritized over the motivation of more than 16 HARQ process for 120kHz SCS. So, we suggest to identify and discuss the problems when 32 HARQ processes for 120kHz SCS are not supported. If its specification impacts and UE complexity is too big to be solved, then we can support the proposal |
| Nokia, NSB | We support Moderator’s proposal |
| Apple | We do support that if there are up to 32 HARQ processes for 120 kHz, it should be subject to UE capability but we agree with QC that this proposal does not solve the issue if a UE is only capable of 16 HARQ processes for 120 kHz and 32 HARQ processes for 480 kHz and 960 kHz. |
| Futurewei | We can accept the proposal, while suggest deferring the discussion if no consensus for the relevant ongoing discussions in other AIs. |
| ZTE, Sanechips | We support the proposal. |
| Ericsson | Support Proposal #3.5-1 |
| OPPO | We do not support this proposal. |
| Huawei, HiSilicon | We support the proposal. It solve the HARQ process starvation problem brought up by multiple PDSCH scheduling. The ambiguity of HARQ process when BWP switching occurs between different SCS can be resolved as well. The issue might be solved together in UE feature discussion together with NTN and MBS. |
| Moderator | - Supported by Xiaomi, Panasonic, CATT, Intel, Lenovo, Fujitsu, Samsung?, Nokia, Apple, Futurewei, ZTE, Ericsson, Huawei - Objected by Qualcomm, InterDigital, vivo, OPPO As commented by Qaulcomm and Apple that making a separate UE capability for 32 HARQ processes for 120 kHz SCS from 480/960 kHz SCS may not figure out the listed issues, this proposal can be modified as follows. |

[HIGH] Proposal #3.5-1a (HARQ process):

- In NR FR2-2, a UE supporting 32 maximum number of HARQ processes for 480/960 kHz SCS for DL (or for UL) shall support 32 as the maximum number of HARQ processes for 120 kHz SCS for DL (or UL), subject to UE capability.

Companies are encouraged to provide views on Proposal #3.5-1a, and please opponents of Proposal #3.5-1a explain how to resolve the following identified issues if this proposal is not supported.

- If 32 maximum HARQ processes are not supported for 120 kHz SCS, at least the following issues would have to be resolved, in case a BWP in a serving cell is configured with 120 kHz SCS and another BWP in the serving cell is configured with 480/960 kHz SCS:

  - Issue 1: Whether the number of HARQ processes is configured per cell (as in Rel-16) or per BWP/SCS

  - Issue 2: Whether to perform data soft combining after BWP switching

  - Issue 3: The number of HARQ processes for the serving cell to generate type-3 HARQ-ACK codebook

| Company | Views |
| --- | --- |
| Xiaomi | Fine with the proposal |

On 1/19 GTW session, the following agreement was made:

- Agreement:

- In NR FR2-2, a UE supporting 32 maximum number of HARQ processes for 480/960 kHz SCS for DL (or for UL) shall support 32 as the maximum number of HARQ processes for 120 kHz SCS for DL (or UL), subject to UE capability.

- Issue 3-5-2) Whether to combine HARQ-disabling feature introduced in Rel-17 NTN with multi-PDSCH scheduling

[Moderator’s note] Ericsson brought up several issues when HARQ-disabling feature introduced in Rel-17 is also applicable to the serving cell configured with multi-PDSCH scheduling. From the moderator’s perspective, it should be first discussed whether this combination can be allowed or not. In that sense, it is encouraged for companies to provide views on this issue, if any.

| Company | Views |
| --- | --- |
| Qualcomm | HARQ disabling feature was specifically introduced with NTN in mind. We believe we should discuss if that feature can be supported for non-NTN first, before we discuss if it can be supported for FR2-2 |
| vivo | Agree with Qualcomm. |
| DOCOMO | We share similar view as Qualcomm, and we think this issue can be deprioritized in current stage. |
| CATT | Agree this should be deferred. |
| Intel | This question can be discussed in UE feature discussion of NTN WI |
| Lenovo, Motorola Mobility | Agree with QC |
| Samsung | We share the view with QC that it should be discussed whether HARQ-ACK disabling feature can be applied to non-NTN use case. If it is concluded that the feature can be applied to non-NTN, we are ok to support the feature for FR2_2 at least for single-PDSCH scheduling. For multi-PDSCH scheduling, more specification works are expected so that we need more discussion. |
| Apple | Agree with QC that there needs ot be a discussion on this. |
| Futurewei | Agree with Qualcomm that discussion is needed. |
| ZTE, Sanechips | Agree with Qualcomm. |
| Ericsson | We are fine to further discuss whether or not this feature is relevant to FR2-2 (non-NTN). The purpose of the proposals in our paper is to show that there doesn't seem to be a fundamental conflict between feedback disabled HARQ processes and multi-PDSCH scheduling with/without time domain bundling. It seems quite straight forward. |
| OPPO | Agree with Qualcomm. |
| Huawei, HiSilicon | It can be deprioritized. |
| Moderator | It is suggested to deprioritize this issue in this meeting. |

Other issues

| Company | Views |
| --- | --- |
| [14] Intel | Proposal 9 - For Type-1 HARQ-ACK codebook, if time bundling is not configured, a PDSCH can be scheduled by DCI format 1_0 if the indicated K1 belongs to the intersection of the extended set of K1 values for DCI format 1_1/1_2 and the predefined set of K1 values for DCI format 1_0. - Agree on the TP 5 to determine the allowed K1 values for DCI format 1_0 |
| [16] Apple | Proposal 7: RAN1 should support a single HARQ-ACK feedback for multi-PDSCH transmissions within a single COT only. Proposal 8: In the case of BWP switching during multi-PxSCH transmission the UE does not expect an UL or DL BWP change on the serving cell after the DCI scheduling the multi-PDSCH transmission and until the PUCCH is transmitted. |
| [17] Xiaomi | Proposal 2: For multi-slot PDSCH scheduling, the PDSCH(s) exceeding the COT are regarded as valid PDSCH(s) and the HARQ process is/are still reserved for those PDSCH(s). Proposal 3: For multi-slot PDSCH scheduling, the HARQ-ACK PUCCH resource for the scheduled multi-slot PDSCH is determined by the last PDSCH among the multiple PDSCHs scheduled by a single DCI, even if the last PDSCH exceeds the COT. |
| [19] MediaTek | Proposal 2: The UCI information bits including HARQ-ACK information bits should reuse the existing PUCCH payload size limit 1706. |

- Summary on other aspects for HARQ operation:

The following issues are brought up by several companies:

- Intel: To apply extended K1 set values also to DCI format 1_0.

- Xiaomi and Apple: Relationship between HARQ-ACK transmission and COT

- Apple: Clarification on BWP switching during multi-PDSCH reception (or multi-PUSCH transmission)

- MediaTek: Reuse the existing PUCCH payload size limit 1706.

[Moderator’s note] Please feel free to express views on above issues, if any.

| Company | Views |
| --- | --- |
| Xiaomi | Multi-PDSCH scheduling exceeding COT should be considered |
| Apple | The Relationship between HARQ-ACK transmission and COT and the effect of BWP switching on multi-PxSCH transmission should be discussed. |

[Closed] TPs

TP#A (was TP#1 from [4] vivo)

--------------------------------------------Start of TP#A for TS 38.213 Clause 9.1.2.1 ------------------------------------------------

If a UE is not provided ca-SlotOffset for any serving cell of PDSCH receptions and for the serving cell of corresponding PUCCH transmission with HARQ-ACK information

while k<CK1

if modnU-K1,k+1,max2μUL-μDL,1=0 or subslotLengthForPUCCH is provided for the HARQ-ACK codebook

Set nD=0 – index of a DL slot overlapping with an UL slot

Set Nk to a number of DL slots overlapping with UL slot nU-K1,k if subslotLengthForPUCCH is provided for the HARQ-ACK codebook; otherwise, Nk=max2μDL-μUL,1

while nD<Nk

……

if slot nU starts at a same time as or after a slot for an active DL BWP change on serving cell c or an active UL BWP change on the PCell and slot n0,k+nD is before the slot for the active DL BWP change on serving cell c or the active UL BWP change on the PCell, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and slot n0,k+nD overlaps with UL slot nU-K1,k-1, k>0, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k,

nD=nD+1;

else

while r<CR

if the UE is not provided enableTimeDomainHARQ-Bundling and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1, or if HARQ-ACK information for PDSCH time resource derived by row r in slot nU-K1,k⋅2μDL-μUL+nD cannot be provided in slot nU

R=R\r;

elseif the UE is provided enableTimeDomainHARQ-Bundling and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot nU-K1,k⋅2μDL-μUL+nD- ∆K0,r(d), at least one symbol of each of the one or more PDSCH time resources derived by row r of set R' is configured as UL, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r.

R=R\r;

R'=R'\r;

else

r=r+1;

end if

end while

……

nD=nD+1;

end if

end while

end if

k=k+1;

end while

else

……

end if

------------------------------------------------------------End of TP#A----------------------------------------------------------------

[Moderator’s note] TP#A is to consider the case when more than one PDSCH time resource can be configured within one DL slot based on a TDRA row for 120 kHz SCS.

Companies are encouraged to provide views on TP#A. It is noted that if it is agreeable, the same change is also needed for the case when UE is provided with ca-offset.

| Company | Views |
| --- | --- |
| vivo | Support the TP#A as proponent |
| Samsung | Not support The current specification is clear enough (“for each slot”) |
| ZTE, Sanechips | Support the proposed TP#A. |
| vivo | @Samsung: each slot is not enough. As moderator also indicates, TP#A is to deal with the case when more than one PDSCH time resource can be configured within one DL slot based on a TDRA row for 120 kHz SCS. |
| Huawei,HiSilicon | Support the TP |
| Moderator | @ Samsung, Could you accept this TP based on vivo’s explanation? |
| Samsung | We are generally fine with the TP after further checking vivo’s explanation. To address the concerns raised by vivo, we can accept the following change for simplicity (taking into account more than one SLIV for each slot). “One or more” seems not essential. for each slot nU-K1,k⋅2μDL-μUL+nD- ∆K0,r(d), at least one symbol of theeach PDSCH time resource derived by row r of set R' is configured as UL |
| vivo | We are fine with either our TP or Samsung’s revised TP. |
| Moderator | @ Samsung, and all, Thanks for the discussion. Let’s take Samsung’s suggestion which is compact. |

TP#B (was from [7] Nokia)

--------------------------------------------Start of TP#B for TS 38.213 Clause 9.1.2.1 ------------------------------------------------

9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

For a serving cell c, an active DL BWP, and an active UL BWP, as described in clause 12, the UE determines a set of MA,c occasions for candidate PDSCH receptions for which the UE can transmit corresponding HARQ-ACK information in a PUCCH in slot nU. If serving cell c is deactivated, the UE uses as the active DL BWP for determining the set of MA,c occasions for candidate PDSCH receptions a DL BWP provided by firstActiveDownlinkBWP-Id. The determination is based:

a)	on a set of slot timing values K1 associated with the active UL BWP

-	If the UE is configured to monitor PDCCH for DCI format 1_0 and is not configured to monitor PDCCH for either DCI format 1_1 or DCI format 1_2 for serving cell c, K1 is provided by the slot timing values {1, 2, 3, 4, 5, 6, 7, 8} for SCS configuration of PUCCH transmission μ≤3, {7, 8, 12, 16, 20, 24, 28, 32} for μ=5, and {13, 16, 24, 32, 40, 48, 56, 64} for μ=6.

-	If the UE is configured to monitor PDCCH for DCI format 1_1 and is not configured to monitor PDCCH for DCI format 1_2 for serving cell c, K1 is provided by dl-DataToUL-ACK

-	If the UE is configured to monitor PDCCH for DCI format 1_2 and is not configured to monitor PDCCH for DCI format 1_1 for serving cell c, K1 is provided by dl-DataToUL-ACK-DCI-1-2

-	If the UE is configured to monitor PDCCH for DCI format 1_1 and DCI format 1_2 for serving cell c, K1 is provided by the union of dl-DataToUL-ACK and dl-DataToUL-ACK-DCI-1-2

-	If the UE is configured to monitor PDCCH for multicast DCI formats for serving cell c

-	if the UE is not provided type1-Codebook-Generation-Mode = 'mode1', K1 is additionally provided by the union of dl-DataToUL-ACK-ForDCI Format4_1

-	if the UE is not provided dl-DataToUL-ACK-ForDCI Format4_1, K1 is provided by the slot timing values {1, 2, 3, 4, 5, 6, 7, 8}

-	if the UE is provided type1-Codebook-Generation-Mode = 'mode1', the UE

-	determines a first K1,UM set as K1∩K1,M, where K1,M is a set of slot timing values for the multicast DCI formats, a second K1,U\M set as K1\K1,UM, and a third K1,M\U set as K1,M\K1,UM

b)	on a set of row indexes R of a table that is associated with the active DL BWP and defining respective sets of slot offsets K0, start and length indicators SLIV, and PDSCH mapping types for PDSCH reception as described in [6, TS 38.214], where the row indexes R of the table are provided by

-	the union of row indexes of time domain resource allocation tables for DCI formats the UE is configured to monitor PDCCH for serving cell c if the UE is not configured to monitor PDCCH for multicast DCI formats for serving cell c, or is not provided type1-Codebook-Generation-Mode = 'mode1', or, if any, for the first K1,UM set

-	the union of row indexes of time domain resource allocation tables for DCI format 1_0 and/or DCI format 1_1 and/or DCI format 1_2 for serving cell c for the second K1,U\M set, if any

-	the union of row indexes of time domain resource allocation tables for multicast DCI formats the UE is configured to monitor PDCCH for serving cell c for the third K1,M\U set, if any

-	if the UE is provided referenceOfSLIVDCI-1-2, for each row index with slot offset K0=0 and PDSCH mapping Type B in a set of row indexes of a table for DCI format 1_2 [6, TS 38.214], for any PDCCH monitoring occasion in any slot where the UE monitors PDCCH for DCI format 1_2 and with starting symbol S0>0, if S+S0+L≤14 for normal cyclic prefix and S+S0+L≤12  for extended cyclic prefix, add a new row index in the set of row indexes of the table by replacing the starting symbol S of the row index by S+S0

c)	on the ratio 2μDL-μUL between the downlink SCS configuration μDL and the uplink SCS configuration μUL provided by subcarrierSpacing in BWP-Downlink and BWP-Uplink for the active DL BWP and the active UL BWP, respectively

d)	if provided, on tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated as described in clause 11.1

e)	if ca-SlotOffset is provided, on Nslot,offset,cDL and μoffset,DL,c provided by ca-SlotOffset for serving cell c, or on Nslot,offsetUL  and μoffset,UL provided by ca-SlotOffset for the primary cell, as described in [4, TS 38.211].

If a UE

-	is not provided coresetPoolIndex or is provided coresetPoolIndex with a value of 0 for first CORESETs on active DL BWPs of serving cells, and

-	is provided coresetPoolIndex with a value of 1 for second CORESETs on active DL BWPs of the serving cells, and

-	is provided ackNackFeedbackMode = joint

where

-	a serving cell is placed in a first set S0 of NcellsDL,0 serving cells if the serving cell includes a first CORESET, and

-	a serving cell is placed in a second set S1 of NcellsDL,1 serving cells if the serving cell includes a second CORESET, and

-	serving cells are placed in a set according to an ascending order of a serving cell index

the UE generates a Type-1 HARQ-ACK codebook for the set S0 and the set S1 of serving cells separately by setting NcellsDL=NcellsDL,0 and NcellsDL=NcellsDL,1 in the following pseudo-code. The UE concatenates the HARQ-ACK codebook generated for the set S0 followed by the HARQ-ACK codebook generated for the set S1 to obtain a total number of OACK HARQ-ACK information bits.

If a UE is provided fdmed-Reception-Multicast and the UE is configured to monitor PDCCH for detection of unicast DCI formats and to monitor PDCCH for detection of multicast DCI formats

-	a serving cell is placed in a first set SU of NcellsDL,U serving cells if the UE is configured to monitor PDCCH for DCI formats 1_0/1_1/1_2 for scheduling on serving cell c, and

-	a serving cell is placed in a second set SM of NcellsDL,M serving cells if the UE is configured to monitor PDCCH for detection of DCI format 4_1/4_2 for scheduling on serving cell c, and

-	serving cells are placed in a set according to an ascending order of a serving cell index

the UE generates a Type-1 HARQ-ACK codebook for the set SU and the set SM of serving cells separately by setting NcellsDL=NcellsDL,U and NcellsDL=NcellsDL,M in the following pseudo-code. The UE concatenates the HARQ-ACK codebook generated for the set SU followed by the HARQ-ACK codebook generated for the set SM to obtain a total number of OACK HARQ-ACK information bits.

If the UE is configured to monitor PDCCH for DCI formats with CRC scrambled by G-RNTI or G-CS-RNTI and is provided type1-Codebook-Generation-Mode = 'mode1', the UE separately applies the following pseudo-code for each of the first K1,UM set, the second K1,U\M set, and third K1,M\U set as the set of slot timing values K1, and for the corresponding sets of row indexes as R to obtain first, second, and third Type-1 HARQ-ACK sub-codebooks, and concatenates the first, second, and third, Type-1 HARQ-ACK sub-codebooks to obtain the Type-1 HARQ-ACK codebook.

If enableTimeDomainHARQ-Bundling is provided

-	set RT'=R

-	set RT to the set of rows that include the last SLIV of each row of set RT'

------------------------------------------------------------End of TP#B----------------------------------------------------------------

[Moderator’s note] TP#B is to clarify the definition of RT as “set of rows that include the last SLIV of each row of set RT'”.

Companies are encouraged to provide views on TP#B.

| Company | Views |
| --- | --- |
| vivo | Support the TP#B. |
| Samsung | Not needed. Based on the definition “b) on a set of row indexes R of a table”, the set RT includes row indexes. |
| ZTE, Sanechips | Support proposed TP#B. |
| Huawei, HiSilicon | Support the TP. To our understanding, RT is the set of last SLIV. |
| Moderator | @ Samsung, Could you accept this TP based on Huawei’s explanation? |
| Samsung | As we mentioned, the definition of the set R is “a set of row indexes.” If the TP is adopted, the set RT' is also a set of row indexes, but RT is a set of rows. It is unclear that why we need to have two different definitions on the sets R’T and RT. Our understanding is there is no ambiguity to define RT as a set of row indexes of last SLIV. |
| Moderator | This TP can be deprioritized in this meeting. |

TP#C (was TP#2 from [8] Samsung)

---------------------------------------------Start TP#C for TS 38.214 Clause 5.1.2.1.1 ------------------------------------------------

Table 5.1.2.1.1-1: Applicable PDSCH time domain resource allocation for DCI formats 1_0, 1_1, 4_0, 4_1 and 4_2

| RNTI | PDCCH search space | SS/PBCH block and CORESET multiplexing pattern | PDSCH-ConfigCommon includes pdsch-TimeDomainAllocationList | PDSCH-Config includes pdsch-TimeDomainAllocationList | PDSCH-Config-MCCH includes pdsch-TimeDomainAllocationList or PDSCH-Config-MTCH includes pdsch-TimeDomainAllocationList or pdsch-Config-Multicast includes pdsch-TimeDomainAllocationList | PDSCH-Config includes pdsch-TimeDomainAllocationListForMultiPDSCH-r17 | PDSCH time domain resource allocation to apply |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SI-RNTI | Type0 common | 1 | - | - | - | - | Default A for normal CP |
|  |  | 2 | - | - | - | - | Default B |
|  |  | 3 | - | - | - | - | Default C |
| SI-RNTI | Type0A common | 1 | No | - | - | - | Default A |
|  |  | 2 | No | - | - | - | Default B |
|  |  | 3 | No | - | - | - | Default C |
|  |  | 1,2,3 | Yes | - | - | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
| RA-RNTI, MSGB-RNTI, TC-RNTI | Type1 common | 1, 2, 3 | No | - | - | - | Default A |
|  |  | 1, 2, 3 | Yes | - | - | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
| P-RNTI | Type2 common | 1 | No | - | - | - | Default A |
|  |  | 2 | No | - | - | - | Default B |
|  |  | 3 | No | - | - | - | Default C |
|  |  | 1,2,3 | Yes | - | - | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
| MCCH-RNTI | Type 0/0B common for broadcast | 1 | No | - | No | - | Default A |
|  |  | 2 | No | - | No | - | Default B |
|  |  | 3 | No | - | No | - | Default C |
|  |  | 1,2,3 | Yes | - | No | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
|  |  | 1,2,3 | No/Yes | - | Yes | - | pdsch-TimeDomainAllocationList provided in pdsch-Config-MCCH |
| G-RNTI for broadcast | Type 0/0B common for broadcast | 1 | No | - | No | - | Default A |
|  |  | 2 | No | - | No | - | Default B |
|  |  | 3 | No | - | No | - | Default C |
|  |  | 1,2,3 | Yes | - | No | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
|  |  | 1,2,3 | No/Yes | - | Yes | - | TimeDomainAllocationList provided in PDSCH-Config-MCCH |
|  |  | 1,2,3 | No/Yes | - | Yes | - | pdsch-TimeDomainAllocationList provided in PDSCH-Config-MTCH |
| C-RNTI, MCS-C-RNTI, CS-RNTI | Any common search space associated with CORESET 0 | 1, 2, 3 | No | - | - | - | Default A |
|  |  | 1, 2, 3 | Yes | - | - | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
| C-RNTI, MCS-C-RNTI, CS-RNTI | Any common search space not associated with CORESET 0 UE specific search space | 1,2,3 | No | No | - | - | Default A |
|  |  | 1,2,3 | Yes | No | - | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
|  |  | 1,2,3 | No/Yes | Yes | - | - | pdsch-TimeDomainAllocationList provided in PDSCH-Config |
|  |  | 1,2,3 | No/Yes | - | - | Yes | pdsch-TimeDomainAllocationListForMultiPDSCH-r17 provided in PDSCH-Config (Note 2) |
| G-RNTI, G-CS-RNTI (for multicast) | Type-X common search space for multiast | 1,2,3 | No | - | No | - | Default A |
|  |  | 1,2,3 | Yes | - | No | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon (Note 1) |
|  |  | 1,2,3 | No/Yes | - | Yes | - | pdsch-TimeDomainAllocationList provided in pdsch-Config-Multicast (Note 1) |
| Note 1: For a UE that supports multicast, the same TDRA table applies to all G-RNTIs (configured for multicast) if configured on a given serving cell. Note 2: If pdsch-TimeDomainAllocationListForMultiPDSCH-r17 is provided, it is applicable to DCI format 1_1 only. |  |  |  |  |  |  |  |

------------------------------------------------------------End TP#C----------------------------------------------------------------

[Moderator’s note] TP#C is 1) to restore some rows which seem to be accidently lost in Rel-17 specification (compared to Rel-16 specification), and 2) to clarify that multi-PDSCH scheduling is not applicable to DCI format 1_0 by adding a Note 2.

Companies are encouraged to provide views on TP#C.

| Company | Views |
| --- | --- |
| vivo | Support the TP#C. |
| Samsung | We support TP#C. Without note 2, DCI format 1_0 uses TDAR table configured by pdsch-TimeDomainAllocationListForMultiPDSCH-r17 provided in PDSCH-Config. |
| ZTE, Sanechips | Support the proposed TP#C. |
| Ericsson | We support addition of Note 2. We think the "accidental loss of rows" can be handled by the spec editor without a TP from this WI. |
| Huawei, HiSilicon | Support the TP |
| Moderator | Agree with Ericsson’s comments. We can focus on adding Note 2 in this TP. Regarding the addition of Note 2, it seems agreeable, please comment if any concerns. |
| Samsung | We are fine with focusing on Note 2. |
| Moderator | Only Note 2 will be pursued and let me (or Samsung ^^) directly inform spec editor about the “accidental loss of rows”. |

TP#D (was TP#3 from [8] Samsung)

-----------------------------------------------------Start of TP#D for TS 38.214 ---------------------------------------------------------

5.1.2.1	Resource allocation in time domain

=============================== Unchanged Text Omitted ===================================

If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH-r17, the UE does not expect to be configured with higher layer parameter repetitionNumber in pdsch-TimeDomainAllocationListForMultiPDSCH-r17.

If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH-r17 on a DL BWP of a serving cell, the UE does not apply pdsch-AggregationFactor in PDSCH-config, if configured, to DCI format 1_1 on the DL BWP of the serving cell.

=============================== Unchanged Text Omitted ===================================

6.1.2.1	Resource allocation in time domain

If a UE is configured with pusch-TimeDomainAllocationListForMultiPUSCH-r17 on a UL BWP of a serving cell, the UE does not apply pusch-AggregationFactor, if configured, to DCI format 0_1 on the UL BWP of the serving cell and the UE does not expect to be configured with numberOfRepetitions in pusch-TimeDomainAllocationListForMultiPUSCH-r17.

------------------------------------------------------------End of TP#D----------------------------------------------------------------

[Moderator’s note] TP#D is mainly to remove “in which one of more rows contain multiple SLIVs for PDSCH (PUSCH)” which seems redundant.

Companies are encouraged to provide views on TP#D.

| Company | Views |
| --- | --- |
| vivo | We don’t see the necessity to remove the aforementioned descriptions. |
| Samsung | We support TP#D. |
| ZTE, Sanechips | We support the proposed TP#D or we can revise the description of “in which one or more rows contain multiple SLIVs for PDSCH (PUSCH)” as “in which at least one or more rows contains multiple SLIVs for PDSCH (PUSCH)” |
| Ericsson | Support TP#D. It would be strange if all rows of pdsch-TimeDomainAllocationListForMultiPDSCH-r17 contained only a single SLIV. |
| Huawei, HiSilicon | The TP looks cleaner. It depends on the editor. |
| Moderator | @ vivo, Could you accept this TP based on other companies’ explanation? |
| Futurewei | TP#D is acceptable for its conciseness and agree that it is unlikely all rows contain only single SLIV, though necessity of the change could be a valid concern. |
| vivo | We don’t see critical need to remove the sentence at current stage. As long as it will be clearly captured in 331, we are fine with the proposed TP. |
| Modeartor | This TP can be deprioritized in this meeting. |

TP#E (was TP#4 from [8] Samsung)

--------------------------------------------Start of TP#E for TS 38.213 Clause 9.1.2.1 ------------------------------------------------

9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

=============================== Unchanged Text Omitted ===================================

while r<CR

if the UE is not provided enableTimeDomainHARQ-Bundling and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1, or if HARQ-ACK information for PDSCH time resource derived by row r in slot n0,knU-K1,k⋅2μDL-μUL+nD cannot be provided in slot nU

R=R\r;

elseif the UE is provided enableTimeDomainHARQ-Bundling and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot n0,knU-K1,k⋅2μDL-μUL+nD- ∆K0,r(d), at least one symbol of the PDSCH time resource derived by row r of set R' is configured as UL, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r and for each slot from n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r of set R is configured as UL

R=R\r;

R'=R'\r;

else

r=r+1;

end if

end while

------------------------------------------------------------End of TP#E----------------------------------------------------------------

[Moderator’s note] As shown below excerpt from [8], TP#E is to allow the case when time domain bundling from DCI format 1_1 and PDSCH repetition from DCI format 1_2.

If time domain bundling is not configured (see the green highlight below), PDSCH occasions are generated by taking into account multi-PDSCH scheduling and PDSCH repetition. Note that for multi-PDSCH scheduling, the row r of the set R contains single SLIVs decomposing multiple SLIVs in a TDRA table (i.e., SLIV decomposition) and for PDSCH repetition, the single SLIV is assumed to be repeated over NPDSCHrepeat,max slots. Therefore, some of rows associated with DCI format 1_1 does not need to be repeated but all of rows are assumed to be repeated over NPDSCHrepeat,max slots, which make some overhead in type-1 HARQ-ACK CB. However, it can be acceptable since it does not bring any scheduling restrictions. If time domain bundling is configured (see the yellow highlight below), PDSCH occasions are generated by taking into account multi-PDSCH scheduling only, but not PDSCH repetition. As a results, the type-1 HARQ-ACK CB with time domain bundling does not includes PDSCH occasions for PDSCH repetition. Therefore, gNB may not schedule PDSCH repetitions by using DCI format 1_2. while r<CR if the UE is not provided enableTimeDomainHARQ-Bundling and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1, or if HARQ-ACK information for PDSCH time resource derived by row r in slot nU-K1,k⋅2μDL-μUL+nD cannot be provided in slot nU R=R\r; elseif the UE is provided enableTimeDomainHARQ-Bundling and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot nU-K1,k⋅2μDL-μUL+nD- ∆K0,r(d), at least one symbol of the PDSCH time resource derived by row r of set R' is configured as UL, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r. R=R\r; R'=R'\r; else r=r+1; end if end while. Observation 2: If time domain bundling is configured, Type-1 HARQ-ACK CB does not cover PDSCH repetitions scheduled by DCI format 1_2. To address this issue, we suggest to consider the following two options. - Option 1. A row r is removed when two conditions are met - Condition 1 for multi-PDSCH scheduling: each SLIVs of the TDRA row r overlapped with a semi-static UL symbol - Condition 2 for PDSCH repetition: the last SLIV of the TDRA row r over K slots overlapped with a semi-static UL symbol. - Note that the last SLIV is used for PDSCH repetition since the TDRA row r may include more than one SLIVs. Note that this may result in some overhead because the TDRA rows only for multi-PDSCH scheduling are also used for PDSCH repetition. - Option 2. Treat it as an error case. i.e., a UE does not expect to be configured with multi-PDSCH scheduling with time domain bundling and pdsch-AggregationFactor at the same time. Since type-1 HARQ-ACK CB is mainly used for a coverage limited scenario where a PDCCH scheduling PDSCH may be missed often, support of PDSCH repetitions scheduled by a DCI format 1_2 would be beneficial. Therefore, we prefer to support that type-1 HARQ-ACK CB covers both multi-PDSCH scheduling and PDSCH repetition. Proposal 13: To support multi-PDSCH scheduling by DCI format 1_1 and PDSCH repetition by DCI format 1_2 in type-1 HARQ-ACK CB, a row r in the set R’ and the set R is removed when the both conditions are met - Condition 1 for multi-PDSCH scheduling) each SLIVs of the TDRA row r overlapped with a semi-static UL symbol - Condition 2 for PDSCH repetition) the last SLIV of the TDRA row r over K slots overlapped with a semi-static UL symbol. Proposal 14: Adopt TP#4 in Appendix for TS38.213

Companies are encouraged to provide views on TP#E.

| Company | Views |
| --- | --- |
| vivo | More discussion may be needed, e.g. on whether such joint operation is desirable or not. |
| Samsung | We support TP#E. The case where multi-PDSCH scheduling is configured for DCI format 1_1 and PDSCH repetition is configured for DCI for 1_2 at the same time is not precluded in the agreements we made in the last RAN1 meeting. But, the current type-1 CB does not cover the case. |
| ZTE, Sanechips | More discussion may be needed |
| Huawei, HiSilicon | Support TP#E |
| Moderator | More discussem seems necessary, so we can discuss about this TP in the next meeting. |
| Samsung | We are fine to discuss this TP in the next meeting. We cannot agree that it is a new joint operation since we already made agreements that multi-PDSCH scheduling and pdsch-AggregationFactor can be configured at the same time, where pdsch-AggregationFactor is applied to DCI format 1_2 only. |

TP#F (was TP#5 from [8] Samsung)

--------------------------------------------Start of TP#F for TS 38.213 Clause 9.1.2.1 ------------------------------------------------

9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

=============================== Unchanged Text Omitted ===================================

while r<CR

if the UE is not provided enableTimeDomainHARQ-Bundling and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1, or if pdsch-TimeDomainAllocationListForMultiPDSCH-r17 is provided and HARQ-ACK information for PDSCH time resource derived by row r in slot nU-K1,k⋅2μDL-μUL+nD cannot be provided in slot nU

R=R\r;

elseif the UE is provided enableTimeDomainHARQ-Bundling and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot nU-K1,k⋅2μDL-μUL+nD- ∆K0,r(d), at least one symbol of the PDSCH time resource derived by row r of set R' is configured as UL, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r.

R=R\r;

R'=R'\r;

else

r=r+1;

end if

end while

------------------------------------------------------------End of TP#F----------------------------------------------------------------

[Moderator’s note] TP#F is to clarity that K1 set extension is only for the case when multi-PDSCH scheduling is configured.

Companies are encouraged to provide views on TP#F.

| Company | Views |
| --- | --- |
| vivo | Support the TP#F. |
| Samsung | We support TP#F. |
| ZTE, Sanechips | Support the TP#F. |
| ASUSTeK | Support the TP#F. |
| Huawei, HiSilicon | Support the TP |
| Moderator | Seems stable, please comment if any concerns. |

TP#G (was TP#8 from [8] Samsung)

----------------------------------------------Start of TP#G for TS 38.214 Clause 6.1 --------------------------------------------------

6.1	UE procedure for transmitting the physical uplink shared channel

=============================== Unchanged Text Omitted ===================================

For uplink, 16 HARQ processes per cell are supported by the UE, or subject to UE capability, a maximum of 32 HARQ processes per cell for the cases of = 5 or = 6. The number of processes the UE may assume will at most be used for the uplink is configured to the UE for each cell separately by higher layer parameter nrofHARQ-ProcessesForPUSCH, and when no configuration is provided the UE may assume a default number of 16 processes.

------------------------------------------------------------End of TP#G----------------------------------------------------------------

[Moderator’s note] TP#G is to capture that the maximum number of HARQ processes for UL is configurable by nrofHARQ-ProcessesForPUSCH and its default value is 16.

Companies are encouraged to provide views on TP#G.

| Company | Views |
| --- | --- |
| vivo | Support the TP#G |
| Samsung | We support TP#G |
| ZTE, Sanechips | Support the proposed TP#G. |
| Ericsson | Support TP#G |
| ASUSTeK | Support the TP#G |
| Huawei, HiSilicon | Support the TP |
| Futurewei | Support the TP#G. |
| Moderator | Seems stable, please comment if any concerns. |

TP#H (was TP#1 from [18] NEC)

------------------------------------------Start of TP#H for TS 38.212 Clause 7.3.1.2.2 ----------------------------------------------

7.3.1.2.2	Format 1_1

=============================== Unchanged Text Omitted ===================================

-	CBG transmission information (CBGTI) – 0 bit if higher layer parameter codeBlockGroupTransmission for PDSCH is not configured or if the number of scheduled PDSCH indicated by the Time domain resource assignment field is larger than 1; otherwise, 2, 4, 6, or 8 bits as defined in Clause 5.1.7 of [6, TS38.214], determined by the higher layer parameters maxCodeBlockGroupsPerTransportBlock and maxNrofCodeWordsScheduledByDCI for the PDSCH.

If higher layer parameter priorityIndicatorDCI-1-1 is configured, if the bit width of the CBG transmission information in DCI format 1_1 for one HARQ-ACK codebook is not equal to that of the CBG transmission information in DCI format 1_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller CBG transmission information until the bit width of the CBG transmission information in DCI format 1_1 for the two HARQ-ACK codebooks are the same.

-	CBG flushing out information (CBGFI) – 1 bit if higher layer parameter codeBlockGroupFlushIndicator is configured as "TRUE" and if the number of scheduled PDSCH indicated by the Time domain resource assignment field is 1, 0 bit otherwise.

If higher layer parameter priorityIndicatorDCI-1-1 is configured, if the bit width of the CBG flushing out information in DCI format 1_1 for one HARQ-ACK codebook is not equal to that of the CBG flushing out information in DCI format 1_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller CBG flushing out information until the bit width of the CBG flushing out information in DCI format 1_1 for the two HARQ-ACK codebooks are the same.

------------------------------------------------------------End of TP#H----------------------------------------------------------------

[Moderator’s note] TP#H is to capture the previous agreement on configuration of CBG operation and multi-PDSCH scheduling.

Companies are encouraged to provide views on TP#H.

| Company | Views |
| --- | --- |
| vivo | We don’t support the TP#H. Regarding the CBGTI field, when multi-PDSCH scheduling is configured, the higher layer parameter codeBlockGroupTransmission for PDSCH should not be configured, and there is no need to check the number of scheduled PDSCHs indicated by the Time domain resource assignment field. Similarly, regarding the CBGFI field, when the higher layer parameter codeBlockGroupFlushIndicator is configured as "TRUE", multi-PDSCH scheduling should not be configured, therefore there is no need to check the number of scheduled PDSCHs indicated by the Time domain resource assignment field as well. Agreement - For a DCI that can schedule multiple PDSCHs, CBGTI and CBGFI fields are not present in the DCI. - UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the serving cell with a Type 1 codebook. - Confirm the working assumption from RAN1#106bis-e with the following modification. Working assumption: (RAN1#106bis-e) - UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group with a Type 2 codebook. If time bundling operation is supported, this working assumption can be revisited |
| Samsung | Discuss Issue 2.4-2 first |
| ZTE, Sanechips | Discuss Issue 2.4-2 first |
| Ericsson | @vivo (or Moderator) Can you confirm where the following is captured in 38.213: "when multi-PDSCH scheduling is configured, the higher layer parameter codeBlockGroupTransmission for PDSCH should not be configured"? |
| vivo | @Ericsson: Our understanding is that it will be captured in TS 38.331 |
| Huawei, HiSilicon | The TP is not necessary. According to the agreement, CBG retransmission will not be configured when DCI can schedule multiple PDSCH. |
| Moderator | More discussion seems necessary, so we can deprioritize this TP in this meeting. |

TP#I (was from [21] ASUSTeK)

--------------------------------------------Start of TP#I for TS 38.214 Clause 6.1.2.1 ------------------------------------------------

6.1.2.1	Resource allocation in time domain

=============================== Unchanged Text Omitted ===================================

For PUSCH repetition Type A, when transmitting PUSCH scheduled by DCI format 0_1 or 0_2 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1, the number of repetitions K is determined as

-	if numberOfRepetitions is present in the resource allocation table, the number of repetitions K is equal to numberOfRepetitions;

-	elseif the UE is configured with pusch-AggregationFactor and the transmitting PUSCH is scheduled by DCI format 0_2, the number of repetitions K is equal to pusch-AggregationFactor;

-	elseif the UE is configured with pusch-AggregationFactor, (and the transmitting PUSCH is scheduled by DCI format 0_1) and not configured with pusch-TimeDomainAllocationListForMultiPUSCH-r17, the number of repetitions K is equal to puschAggregationFactor;

-	otherwise K=1.

-	the number of slots used for TBS determination N is equal to 1.

For PUSCH repetition type A, when transmitting PUSCH scheduled by RAR UL grant, the 2 MSBs of the MCS information field of the RAR UL grant provide a codepoint to determine the number of repetitions K according to Table 6.1.2.1-1A, based on whether or not the higher layer parameter numberOfMsg3Repetitions is configured.

For PUSCH repetition type A, when transmitting PUSCH scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, the 2 MSBs of the MCS information field of the DCI format 0_0 with CRC scrambled by TC-RNTI provide a codepoint to determine the number of repetitions K according to Table 6.1.2.1-1A, based on whether or not the higher layer parameter numberOfMsg3Repetitions is configured.

------------------------------------------------------------End of TP#I----------------------------------------------------------------

[Moderator’s note] TP#I is to clarify that UE does not apply pusch-AggregationFactor to DCI format 0_1 (can scheduling more than one PDSCH) and the number of repetitions K is 1.

Companies are encouraged to provide views on TP#I.

| Company | Views |
| --- | --- |
| vivo | Support the TP#I. |
| Samsung | No needed. TS38.214 already capture the following. If a UE is configured with pusch-TimeDomainAllocationListForMultiPDSCH-r17 in which one or more rows contain multiple SLIVs for PUSCH on a UL BWP of a serving cell, the UE does not apply pusch-AggregationFactor, if configured, to DCI format 0_1 on the UL BWP of the serving cell and the UE does not expect to be configured with numberOfRepetitions in pusch-TimeDomainAllocationListForMultiPDSCH-r17. |
| ZTE, Sanechips | Support the proposed TP#I. |
| Ericsson | Do not support TP#I for the reason pointed out by Samsung (already captured in specs) |
| ASUSTeK | As proponent, we support TP#I. Regaring paragraph pointed out by Samsung, we agree it specify “the UE does not apply pusch-AggregationFactor, if configured, to DCI format 0_1” which captures previous RAN1 agreement. However, repetitions K determination from another paragraph seems not aligned with the paragraph pointed out from Samsung since it specify K is equal to pusch-AggregationFactor regardless of checking pusch-AggregationFactor is applied or not applied (i.e. elseif the UE is configured with pusch-AggregationFactor, the number of repetitions K is equal to pusch-AggregationFactor). |
| Huawei, HiSilicon | Support the TP#I. The TP clarify the behavior when pusch-AggregationFactor should be applied. The behavior for DCI 0-1 and 0-2 are different and should be described separately. |
| Moderator | More discussion seems necessary, so we can discuss about this TP in the next meeting. |

Stable TPs

TP#A1 (was revised from TP#A)

-------------------------------------------Start of TP#A1 for TS 38.213 Clause 9.1.2.1 -----------------------------------------------

If a UE is not provided ca-SlotOffset for any serving cell of PDSCH receptions and for the serving cell of corresponding PUCCH transmission with HARQ-ACK information

while k<CK1

if modnU-K1,k+1,max2μUL-μDL,1=0 or subslotLengthForPUCCH is provided for the HARQ-ACK codebook

Set nD=0 – index of a DL slot overlapping with an UL slot

Set Nk to a number of DL slots overlapping with UL slot nU-K1,k if subslotLengthForPUCCH is provided for the HARQ-ACK codebook; otherwise, Nk=max2μDL-μUL,1

while nD<Nk

……

if slot nU starts at a same time as or after a slot for an active DL BWP change on serving cell c or an active UL BWP change on the PCell and slot n0,k+nD is before the slot for the active DL BWP change on serving cell c or the active UL BWP change on the PCell, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and slot n0,k+nD overlaps with UL slot nU-K1,k-1, k>0, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k,

nD=nD+1;

else

while r<CR

if the UE is not provided enableTimeDomainHARQ-Bundling and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1, or if HARQ-ACK information for PDSCH time resource derived by row r in slot nU-K1,k⋅2μDL-μUL+nD cannot be provided in slot nU

R=R\r;

elseif the UE is provided enableTimeDomainHARQ-Bundling and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot nU-K1,k⋅2μDL-μUL+nD- ∆K0,r(d), at least one symbol of each PDSCH time resource derived by row r of set R' is configured as UL, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r.

R=R\r;

R'=R'\r;

else

r=r+1;

end if

end while

……

nD=nD+1;

end if

end while

end if

k=k+1;

end while

else

……

end if

------------------------------------------------------------End of TP#A1----------------------------------------------------------------

TP#C1 (was revised from TP#C)

--------------------------------------------Start TP#C1 for TS 38.214 Clause 5.1.2.1.1 -----------------------------------------------

Table 5.1.2.1.1-1: Applicable PDSCH time domain resource allocation for DCI formats 1_0, 1_1, 4_0, 4_1 and 4_2

| MCCH-RNTI | Type 0/0B common for broadcast | 1 | No | - | No | - | Default A |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2 | No | - | No | - | Default B |
|  |  | 3 | No | - | No | - | Default C |
|  |  | 1,2,3 | Yes | - | No | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
|  |  | 1,2,3 | No/Yes | - | Yes | - | pdsch-TimeDomainAllocationList provided in pdsch-Config-MCCH |
| G-RNTI for broadcast | Type 0/0B common for broadcast | 1 | No | - | No | - | Default A |
|  |  | 2 | No | - | No | - | Default B |
|  |  | 3 | No | - | No | - | Default C |
|  |  | 1,2,3 | Yes | - | No | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
|  |  | 1,2,3 | No/Yes | - | Yes | - | TimeDomainAllocationList provided in PDSCH-Config-MCCH |
|  |  | 1,2,3 | No/Yes | - | Yes | - | pdsch-TimeDomainAllocationList provided in PDSCH-Config-MTCH |
| C-RNTI, MCS-C-RNTI, CS-RNTI | Any common search space associated with CORESET 0 | 1, 2, 3 | No | - | - | - | Default A |
|  |  | 1, 2, 3 | Yes | - | - | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
| C-RNTI, MCS-C-RNTI, CS-RNTI | Any common search space not associated with CORESET 0 UE specific search space | 1,2,3 | No | No | - | - | Default A |
|  |  | 1,2,3 | Yes | No | - | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
|  |  | 1,2,3 | No/Yes | Yes | - | - | pdsch-TimeDomainAllocationList provided in PDSCH-Config |
|  |  | 1,2,3 | No/Yes | - | - | Yes | pdsch-TimeDomainAllocationListForMultiPDSCH-r17 provided in PDSCH-Config (Note 2) |
| G-RNTI, G-CS-RNTI (for multicast) | Type-X common search space for multiast | 1,2,3 | No | - | No | - | Default A |
|  |  | 1,2,3 | Yes | - | No | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon (Note 1) |
|  |  | 1,2,3 | No/Yes | - | Yes | - | pdsch-TimeDomainAllocationList provided in pdsch-Config-Multicast (Note 1) |
| Note 1: For a UE that supports multicast, the same TDRA table applies to all G-RNTIs (configured for multicast) if configured on a given serving cell. Note 2: If pdsch-TimeDomainAllocationListForMultiPDSCH-r17 is provided, it is applicable to DCI format 1_1 only. |  |  |  |  |  |  |  |

------------------------------------------------------------End TP#C1----------------------------------------------------------------

TP#F (was TP#5 from [8] Samsung)

--------------------------------------------Start of TP#F for TS 38.213 Clause 9.1.2.1 ------------------------------------------------

9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

=============================== Unchanged Text Omitted ===================================

while r<CR

if the UE is not provided enableTimeDomainHARQ-Bundling and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1, or if pdsch-TimeDomainAllocationListForMultiPDSCH-r17 is provided and HARQ-ACK information for PDSCH time resource derived by row r in slot nU-K1,k⋅2μDL-μUL+nD cannot be provided in slot nU

R=R\r;

elseif the UE is provided enableTimeDomainHARQ-Bundling and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot nU-K1,k⋅2μDL-μUL+nD- ∆K0,r(d), at least one symbol of the PDSCH time resource derived by row r of set R' is configured as UL, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r.

R=R\r;

R'=R'\r;

else

r=r+1;

end if

end while

------------------------------------------------------------End of TP#F----------------------------------------------------------------

TP#G (was TP#8 from [8] Samsung)

----------------------------------------------Start of TP#G for TS 38.214 Clause 6.1 --------------------------------------------------

6.1	UE procedure for transmitting the physical uplink shared channel

=============================== Unchanged Text Omitted ===================================

For uplink, 16 HARQ processes per cell are supported by the UE, or subject to UE capability, a maximum of 32 HARQ processes per cell for the cases of = 5 or = 6. The number of processes the UE may assume will at most be used for the uplink is configured to the UE for each cell separately by higher layer parameter nrofHARQ-ProcessesForPUSCH, and when no configuration is provided the UE may assume a default number of 16 processes.

------------------------------------------------------------End of TP#G----------------------------------------------------------------

Conclusion

Agreement

- In NR FR2-2, a UE supporting 32 maximum number of HARQ processes for 480/960 kHz SCS for DL (or for UL) shall support 32 as the maximum number of HARQ processes for 120 kHz SCS for DL (or UL), subject to UE capability.

Agreement

- If the higher layer parameter maxNrofCodeWordsScheduledByDCI indicates that two codeword transmission is enabled and more than one PDSCH is scheduled by a multi-PDSCH scheduling DCI,

  - Either the first or the second transport block of all scheduled PDSCHs is disabled by the DCI if IMCS = 26 and if RV bits are set to ‘1’ for the corresponding transport block of all scheduled PDSCHs (i.e. irrespective of whether this is a valid PDSCH).

Conclusion

For multi-PDSCH or multi-PUSCH scheduling DCI, the following maximum value of a gap is not specified in Rel-17 and up to gNB scheduler.

- The maximum value of the gap between two consecutively scheduled PDSCHs or between two consecutively scheduled PUSCHs

- The maximum value of the gap between the first scheduled PDSCH and the last scheduled PDSCH or between the first scheduled PUSCH and the last scheduled PUSCH

Conclusion

HARQ process number configured for SPS PDSCH (or CG PUSCH) can be allocated to a PDSCH (or PUSCH) of multi-PDSCH (or multi-PUSCH) scheduling, as long as the timeline condition defined in Rel-15/16 is met.

- Note: It is up to gNB implementation whether/how to avoid UL data retransmission due to HARQ process index collision and flushed HARQ transmit buffer.

Agreement

- Update the previous agreement made in RAN1#107-e, as follows:

Agreement (RAN1#107-e)

For multi-PDSCH scheduling with a single DCI

- Introduce a new RRC parameter, e.g., enableTimeDomainHARQ-Bundling, to enable time domain bundling operation for type-1 HARQ-ACK codebook per serving cell.

  - If the RRC parameter enables time domain bundling operation,

    - To determine the set of candidate PDSCH reception occasions,

      - A row index is removed if at least one symbol of every PDSCH associated with the row index is configured as semi-static UL. (NOTE: This is similar to the case of slot aggregated PDSCH in Rel-16)

      - Pruning procedure in Rel-16 is performed based on the last configured SLIV of each row index.

    - Logical AND operation is applied across all valid PDSCHs associated with a determined candidate PDSCH reception occasion, at least for 1-TB case.

    - FFS: UE does not expect the last scheduled SLIV overlaps with a semi-static UL symbol when parameter enableTimeDomainHARQ-Bundling is configured

Agreement

For multi-PDSCH scheduling with a single DCI and for type-2 HARQ-ACK codebook generation,

- Time domain bundling and spatial bundling can be independently configured.

Conclusion

- UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCIs to lead to out-of-order scheduling, also for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH).

  - This may not have specification impact.

- Note: It is separately discussed whether the scheduled PDSCHs (or PUSCHs or SLIV) is based on configured SLIV or valid SLIV.

Conclusion

UE does not expect any of the received PDSCHs (including SPS PDSCH) and the resource for the HARQ-ACK transmission to lead to out-of-order scheduling, for any scheduling DCIs (including multi-PDSCH scheduling DCI).

Agreement

For a DCI that can schedule multiple PDSCHs or multiple PUSCHs,

- It is clarified that NDI/RV fields in the following previous agreements correspond to scheduled PDSCHs indicated by the TDRA information field.

Agreement: (RAN1#104-bis) For a DCI that can schedule multiple PDSCHs, - NDI for the 1st TB: This is signaled per PDSCH and applies to the first TB of each PDSCH - RV for the 1st TB: This is signaled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the first TB of each PDSCH Agreement: (RAN1#106bis-e) For a DCI that can schedule multiple PDSCHs, and if RRC parameter configures that two codeword transmission is enabled, - NDI for the 2nd TB: This is signalled per PDSCH and applies to the 2nd TB of each PDSCH - RV for the 2nd TB: This is signalled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the 2nd TB of each PDSCH

- Above clarification also applies to the DCI scheduling multiple PUSCHs, i.e., NDI/RV fields in the DCI correspond to scheduled PUSCHs indicated by the TDRA information field.

- The following example change to 38.214 Sections 5.1.3 and 6.1.4 can be recommended to the editor of 38.214 to use at the editor’s discretion

---------------------------Start of TP for TS 38.214 Clause 5.1.3 -----------------------------------------------

5.1.3	Modulation order, target code rate, redundancy version and transport block size determination

================ Unchanged Text Omitted =======================

When the UE is scheduled with multiple PDSCHs by a DCI, as described in clause 5.1.2.1, the bits of rv field and NDI field, respectively, in the DCI are one-to-one mapped to the scheduled PDSCH(s) indicated by the TDRA information field with the corresponding transport block(s) in the scheduled order, where the LSB bits of the rv field and NDI field, respectively, correspond to the last scheduled PDSCH indicated by the TDRA information field.

---------------------------------------------- End of TP --------------------------------------------------------------

---------------------------Start of TP for TS 38.214 Clause 6.1.4 -----------------------------------------------

6.1.4	Modulation order, redundancy version and transport block size determination

================ Unchanged Text Omitted =======================

When the UE is scheduled with multiple PUSCHs by a DCI, as described in clause 6.1.2.1, the bits of rv field and NDI field, respectively, in the DCI are one to one mapped to the scheduled PUSCH(s) indicated by the TDRA information field with the corresponding transport block(s) in the scheduled order where the LSB bits of the rv field and NDI field, respectively, correspond to the last scheduled PUSCH indicated by the TDRA information field.

---------------------------------------------- End of TP --------------------------------------------------------------

Conclusion

It is clarified that the absence or presence of CBGTI field in the following previous agreement is determined based on scheduled PUSCHs indicated by the TDRA information field (i.e. irrespective of whether this is a valid PUSCH).

Agreement: (RAN1#105-e) - At least for 120 kHz SCS, for a DCI that can schedule multiple PUSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs, - If CBG-based (re)transmission is configured, CBGTI field is not present when more than one PUSCHs are scheduled, but is present when a single PUSCH is scheduled, as in Rel-16.

Agreement

A UE validates, for scheduling activation or scheduling release, a DL SPS assignment PDCCH or a configured UL grant Type 2 PDCCH if the PDCCH indicates a TDRA row index including only one SLIV.

Agreement

For type-1 HARQ-ACK codebook, if OACK+OSR+OCSI≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as follows.

- For a serving cell c configured with enableTimeDomainHARQ-Bundling, and for a DCI format indicating a TDRA row that includes more than one SLIV entry on the serving cell c, the UE considers a PDSCH (which carries one or two transport blocks enabled by the DCI format irrespective of whether the PDSCH is valid or not) only associated with the last SLIV as received, to determine Nm,creceived.

The TP below for TS38.213v17.0.0 is endorsed.

-------------------------------------------Start of TP#A1 for TS 38.213 Clause 9.1.2.1 -----------------------------------------------

If a UE is not provided ca-SlotOffset for any serving cell of PDSCH receptions and for the serving cell of corresponding PUCCH transmission with HARQ-ACK information

while k<CK1

if modnU-K1,k+1,max2μUL-μDL,1=0 or subslotLengthForPUCCH is provided for the HARQ-ACK codebook

Set nD=0 – index of a DL slot overlapping with an UL slot

Set Nk to a number of DL slots overlapping with UL slot nU-K1,k if subslotLengthForPUCCH is provided for the HARQ-ACK codebook; otherwise, Nk=max2μDL-μUL,1

while nD<Nk

……

if slot nU starts at a same time as or after a slot for an active DL BWP change on serving cell c or an active UL BWP change on the PCell and slot n0,k+nD is before the slot for the active DL BWP change on serving cell c or the active UL BWP change on the PCell, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and slot n0,k+nD overlaps with UL slot nU-K1,k-1, k>0, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k,

nD=nD+1;

else

while r<CR

if the UE is not provided enableTimeDomainHARQ-Bundling and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1, or if HARQ-ACK information for PDSCH time resource derived by row r in slot nU-K1,k⋅2μDL-μUL+nD cannot be provided in slot nU

R=R\r;

elseif the UE is provided enableTimeDomainHARQ-Bundling and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot nU-K1,k⋅2μDL-μUL+nD- ∆K0,r(d), at least one symbol of each PDSCH time resource derived by row r of set R' is configured as UL, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r.

R=R\r;

R'=R'\r;

else

r=r+1;

end if

end while

……

nD=nD+1;

end if

end while

end if

k=k+1;

end while

else

……

end if

------------------------------------------------------------End of TP#A1----------------------------------------------------------------

The TP below for TS38.214v17.0.0 is endorsed.

--------------------------------------------Start TP#C1 for TS 38.214 Clause 5.1.2.1.1 -----------------------------------------------

Table 5.1.2.1.1-1: Applicable PDSCH time domain resource allocation for DCI formats 1_0, 1_1, 4_0, 4_1 and 4_2

| MCCH-RNTI | Type 0/0B common for broadcast | 1 | No | - | No | - | Default A |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2 | No | - | No | - | Default B |
|  |  | 3 | No | - | No | - | Default C |
|  |  | 1,2,3 | Yes | - | No | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
|  |  | 1,2,3 | No/Yes | - | Yes | - | pdsch-TimeDomainAllocationList provided in pdsch-Config-MCCH |
| G-RNTI for broadcast | Type 0/0B common for broadcast | 1 | No | - | No | - | Default A |
|  |  | 2 | No | - | No | - | Default B |
|  |  | 3 | No | - | No | - | Default C |
|  |  | 1,2,3 | Yes | - | No | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
|  |  | 1,2,3 | No/Yes | - | Yes | - | TimeDomainAllocationList provided in PDSCH-Config-MCCH |
|  |  | 1,2,3 | No/Yes | - | Yes | - | pdsch-TimeDomainAllocationList provided in PDSCH-Config-MTCH |
| C-RNTI, MCS-C-RNTI, CS-RNTI | Any common search space associated with CORESET 0 | 1, 2, 3 | No | - | - | - | Default A |
|  |  | 1, 2, 3 | Yes | - | - | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
| C-RNTI, MCS-C-RNTI, CS-RNTI | Any common search space not associated with CORESET 0 UE specific search space | 1,2,3 | No | No | - | - | Default A |
|  |  | 1,2,3 | Yes | No | - | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
|  |  | 1,2,3 | No/Yes | Yes | - | - | pdsch-TimeDomainAllocationList provided in PDSCH-Config |
|  |  | 1,2,3 | No/Yes | - | - | Yes | pdsch-TimeDomainAllocationListForMultiPDSCH-r17 provided in PDSCH-Config (Note 2) |
| G-RNTI, G-CS-RNTI (for multicast) | Type-X common search space for multiast | 1,2,3 | No | - | No | - | Default A |
|  |  | 1,2,3 | Yes | - | No | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon (Note 1) |
|  |  | 1,2,3 | No/Yes | - | Yes | - | pdsch-TimeDomainAllocationList provided in pdsch-Config-Multicast (Note 1) |
| Note 1: For a UE that supports multicast, the same TDRA table applies to all G-RNTIs (configured for multicast) if configured on a given serving cell. Note 2: If pdsch-TimeDomainAllocationListForMultiPDSCH-r17 is provided, it is applicable to DCI format 1_1 only. |  |  |  |  |  |  |  |

------------------------------------------------------------End TP#C1----------------------------------------------------------------

The TP below for TS38.213v17.0.0 is endorsed.

--------------------------------------------Start of TP#F for TS 38.213 Clause 9.1.2.1 ------------------------------------------------

9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

=============================== Unchanged Text Omitted ===================================

while r<CR

if the UE is not provided enableTimeDomainHARQ-Bundling and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1, or if pdsch-TimeDomainAllocationListForMultiPDSCH-r17 is provided and HARQ-ACK information for PDSCH time resource derived by row r in slot nU-K1,k⋅2μDL-μUL+nD cannot be provided in slot nU

R=R\r;

elseif the UE is provided enableTimeDomainHARQ-Bundling and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot nU-K1,k⋅2μDL-μUL+nD- ∆K0,r(d), at least one symbol of the PDSCH time resource derived by row r of set R' is configured as UL, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r.

R=R\r;

R'=R'\r;

else

r=r+1;

end if

end while

------------------------------------------------------------End of TP#F----------------------------------------------------------------

The TP below for TS38.214v17.0.0 is endorsed.

----------------------------------------------Start of TP#G for TS 38.214 Clause 6.1 --------------------------------------------------

6.1	UE procedure for transmitting the physical uplink shared channel

=============================== Unchanged Text Omitted ===================================

For uplink, 16 HARQ processes per cell are supported by the UE, or subject to UE capability, a maximum of 32 HARQ processes per cell for the cases of = 5 or = 6. The number of processes the UE may assume will at most be used for the uplink is configured to the UE for each cell separately by higher layer parameter nrofHARQ-ProcessesForPUSCH, and when no configuration is provided the UE may assume a default number of 16 processes.

------------------------------------------------------------End of TP#G----------------------------------------------------------------

Reference

- R1-2200025	On several study points for PDSCH/PUSCH enhancements for Beyond 52.6GHz	FUTUREWEI

- R1-2200048	Remaining issues of PDSCH/PUSCH enhancement for 52-71GHz spectrum	Huawei, HiSilicon

- R1-2200064	Remaining issues for PDSCH/PUSCH enhancements to supporting 52.6-71 GHz band in NR	InterDigital, Inc.

- R1-2200078	Remaining issues on PDSCH/PUSCH enhancements for NR operation from 52.6GHz to 71GHz	vivo

- R1-2200124	Remaining issues of multi-PDSCH scheduling via a single DCI	Fujitsu

- R1-2200145	Remaining issues on PDSCH/PUSCH enhancements for up to 71GHz operation	CATT

- R1-2200187	PDSCH/PUSCH enhancements	Nokia, Nokia Shanghai Bell

- R1-2200196	Maintenance on PDSCH/PUSCH enhancements for NR from 52.6 GHz to 71 GHz	Samsung

- R1-2200230	Remaining issues on PDSCH/PUSCH enhancements for NR in FR2-2	NTT DOCOMO, INC.

- R1-2200263	Remaining issues on the data channel enhancements for 52.6 to 71GHz	ZTE, Sanechips

- R1-2200267	Discussion on PDSCH/PUSCH enhancements for NR 52.6-71 GHz	Panasonic Corporation

- R1-2200292	PDSCH/PUSCH enhancements for NR in 52.6 to 71GHz band	Qualcomm Incorporated

- R1-2200328	Discussion on remaining issue for PDSCH/PUSCH enhancements	OPPO

- R1-2200370	Discussion on PDSCH/PUSCH enhancements for extending NR up to 71 GHz	Intel Corporation

- R1-2200405	PDSCH-PUSCH Enhancements	Ericsson

- R1-2200412	On remaining issues for PDSCH/PUSCH Enhancements	Apple

- R1-2200461	Remaining issues on PDSCH and PUSCH enhancements for NR 52.6-71GHz	xiaomi

- R1-2200508	Remaining issues on PDSCH/PUSCH enhancement for NR operation from 52.6GHz to 71GHz	NEC

- R1-2200542	Remaining discussion on multi-PDSCH scheduling design for 52.6-71 GHz NR operation	MediaTek Inc.

- R1-2200568	PDSCH/PUSCH enhancements to support NR above 52.6 GHz	LG Electronics

- R1-2200631	Discussion on multi-PUSCH scheduling	ASUSTeK

- R1-2200632	Remaining issues on PDSCH/PUSCH enhancement for NR from 52.6GHz to 71GHz	WILUS Inc.

- Appendix: Previous agreements

- RAN1#104-e

Agreement:

- For a UE and for a serving cell, scheduling multiple PDSCHs by single DL DCI and scheduling multiple PUSCHs by single UL DCI are supported.

  - Each PDSCH or PUSCH has individual/separate TB(s) and each PDSCH/PUSCH is confined within a slot.

  - FFS: The maximum number of PDSCHs or PUSCHs that can be scheduled with a single DCI

  - FFS: Whether multiple PDSCH scheduling applies to 120 kHz in addition to 480 and 960 kHz

  - At least for 120 kHz SCS, single-slot scheduling with slot-based monitoring will still be supported as specified in Rel-15/Rel-16

- The followings will not be considered in this WI.

  - Single DCI to schedule both PDSCH(s) and PUSCH(s)

  - Single DCI to schedule one or multiple TBs where any single TB can be mapped over multiple slots, where mapping is not by repetition

  - Single DCI to schedule N TBs (N>1) where a TB can be repeated over multiple slots (or mini-slots)

- Note: This does not imply that existing slot aggregation and/or repetition for PDSCH and PUSCH by single DCI is precluded for the serving cell.

Agreement:

- For a DCI scheduling multiple PDSCHs, HARQ-ACK information corresponding to PDSCHs scheduled by the DCI is multiplexed with a single PUCCH in a slot that is determined based on K1,

  - where K1 (indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ_feedback timing indicator field is not present in the DCI) indicates the slot offset between the slot of the last PDSCH scheduled by the DCI and the slot carrying the HARQ-ACK information corresponding to the scheduled PDSCHs.

    - It is noted that granularity of K1 can be separately discussed.

- FFS: If needed, further discuss whether or not HARQ-ACK information corresponding to different PDSCHs scheduled by the DCI can be carried by different PUCCH(s)

Agreement:

For generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the following alternatives can be considered to DAI counting and will be down-selected in RAN1#104bis-e.

- Alt 1: C-DAI/T-DAI is counted per DCI.

- Alt 2: C-DAI/T-DAI is counted per PDSCH.

- Alt 3: C-DAI/T-DAI is counted per M scheduled PDSCH(s), where M is configurable (e.g., 1, 2, 4, …).

- FFS: Codebook generation details

- FFS: How to signal DAI values (e.g., increase of DAI bits for Alt 2 and Alt 3)

- FFS: Whether to apply time domain bundling of HARQ-ACK feedback

Agreement:

The multi-PUSCH scheduling defined in Rel-16 NR-U is the baseline for multi-PUSCH scheduling in Rel-17.

- FFS: Applicability to multi-PDSCH scheduling.

Agreement:

- For the multi-PUSCH scheduling in Rel-17, study the enhancement of the following in addition to Rel-16 multi-PUSCH scheduling.

  - CBGTI: Whether or not CBG (re)transmission is supported when more than one PUSCHs are scheduled (Already supported when only one PUSCH is scheduled).

  - CSI-request: Whether to apply same or different rule compared to Rel-16 (e.g., the PUSCH that carries the AP-CSI feedback is the first PUSCH that satisfies the multiplexing timeline).

  - TDRA: Down-select among

    - Alt 1: TDRA table is extended such that each row indicates up to [X, FFS for X] multiple PUSCHs (continuous in time-domain). Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is signalled by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI.

    - Alt 2: TDRA table is extended such that each row indicates up to [X, FFS for X] multiple PUSCHs (that can be non-continuous in time-domain). Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is signalled by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI.

    - Alt 3: TDRA table is extended such that each row indicates up to 8 multiple PUSCH groups (that can be non-continuous between PUSCH groups). Each PUSCH group has a separate SLIV, mapping type and number of slots/PUSCHs N. Within each PUSCH group, N PUSCHs occupy the same OFDM symbols indicated by the SLIV and mapping type. The number of scheduled PUSCHs is the sum of number of PUSCHs in all PUSCH groups in the row of the TDRA table signalled in DCI.

  - FDRA: Whether/how to enhance FDRA e.g., by increasing RBG size or changing allocation granularity

  - Frequency hopping: Whether/how to support frequency hopping for scheduled PUSCHs, e.g., inter-PUSCH/intra-PUSCH hopping

  - URLLC related fields such as priority indicator and open-loop power control parameter set indication: Whether/how to apply URLLC related fields for scheduled PUSCHs

  - Applicability to multi-PDSCH scheduling in Rel-17.

  - Note: Other enhancements are not precluded.

- RAN1#104bis-e

Agreement:

- The maximum number of PDSCHs that can be scheduled with a single DCI in Rel-17 is 8 for SCS of 480 and 960 kHz.

  - FFS: Further restrictions for 480 kHz to 4

  - FFS: A UE capability to select between 4 and 8 for 480 kHz SCS

  - Note: Multi-PDSCH scheduling for the case of 120 kHz SCS is still FFS as per prior agreement. This case can be addressed after this FFS has been decided.

- The maximum number of PUSCHs that can be scheduled with a single DCI in Rel-17 is 8.

  - FFS: Further restrictions for 120 kHz and 480 kHz SCS

  - FFS: A UE capability to select between different values for 120 kHz and 480 kHz SCS

Agreement:

For a DCI that can schedule multiple PDSCHs,

- MCS for the 1st TB: This appears only once in the DCI and applies commonly to the first TB of each PDSCH

- NDI for the 1st TB: This is signaled per PDSCH and applies to the first TB of each PDSCH

- RV for the 1st TB: This is signaled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the first TB of each PDSCH

- HARQ process number: This applies to the first scheduled PDSCH and is incremented by 1 for subsequent PDSCHs (with modulo operation, if needed)

- FFS:

  - MCS/NDI/RV for the 2nd TB for each PDSCH, including whether scheduling of the 2nd TB for each PDSCH can be supported or not

  - Details of resource allocation related fields such as VRB-to-PRB mapping, PRB bundling size indicator, rate matching indicator, and ZP CSI-RS trigger

  - Whether/how to signal CBGFI/CBGTI if CBGFI/CBGTI is supported for multi-PDSCH scheduling

  - Details of fields that are common with multi-PUSCH scheduling, e.g., TDRA, FDRA, priority indicator, including potential enhancements

Agreement:

- For a DCI that can schedule multiple PUSCHs,

  - TDRA: Alt 2 (TDRA table is extended such that each row indicates up to 8 multiple PUSCHs (that can be non-continuous in time-domain). Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is implicitly indicated by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI.), as per agreement made in RAN1#104-e

    - FFS: signaling details

  - Note: Alt 2 does not preclude continuous resource allocation in time-domain.

- For a DCI that can schedule multiple PDSCHs,

  - TDRA: TDRA table is extended such that each row indicates up to 8 multiple PDSCHs (that can be non-continuous in time-domain). Each PDSCH has a separate SLIV and mapping type. The number of scheduled PDSCHs is implicitly indicated by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI.

    - FFS: signaling details

  - Note: This does not preclude continuous resource allocation in time-domain.

  - Note: Multi-PDSCH scheduling for the case of 120 kHz SCS is still FFS as per prior agreement. This case can be addressed after this FFS has been decided.

Agreement:

For enhancements of generating type-1 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the following options can be considered,

- Option 1: The set of candidate PDSCH reception occasions is determined according to each SLIV of each row in the TDRA table and based on extension of K1 set

- Option 1a: The set of candidate PDSCH reception occasions is determined according to each SLIV of each row in the TDRA table

- Option 2: The set of candidate PDSCH reception occasions is determined according to the last SLIV of each row in the TDRA table

- FFS: Codebook generation details, including how to handle the collision with TDD DL/UL configuration and whether/how to extend K1 set based on K1 and slot offset between last PDSCH and other PDSCHs in a row in the TDRA table

Conclusion:

The following is observed for alternative 1 from prior agreement.

- For Alt 1 (C-DAI/T-DAI is counted per DCI) of generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs,

  - C-DAI/T-DAI in DL DCI: Same DAI overhead with Rel-16 single-PDSCH DCI

  - T-DAI in UL DCI:

    - In case of single codebook handling feedback for both single and multi-PDSCH scheduling, same DAI overhead with Rel-16 UL DCI

    - In case of separate sub-codebooks, need additional DAI field (with same bit-width of DAI with Rel-16 UL DCI), in UL DCI for all serving cells including a serving cell not configured with multi-PDSCH DCI

      - Note that DAI field increment for this case is similar for the case in Rel-15 where CBG is configured

  - HARQ-ACK codebook generation:

    - A separate sub-codebook can be generated when multi-PDSCH DCI is configured for a serving cell, similar to the way as 2nd sub-codebook is defined to handle CBG-based scheduling

      - FFS: whether single codebook or separate sub-codebooks is(are) generated when multi-PDSCH DCI is configured for a serving cell

      - FFS: how many sub-codebooks are generated when multi-PDSCH DCI is configured for a serving cell and CBG is configured for the serving cell and/or the other serving cell(s)

    - HARQ-ACK payload size is increased compared to single PDSCH scheduling only, since the number of HARQ-ACK bits corresponding to each DAI of the (sub-)codebook for multi-PDSCH DCI in case of separate sub-codebooks (or for all DL DCIs in case of single codebook) depends on the maximum configured number of PDSCHs for multi-PDSCH DCI across serving cells belonging to the same PUCCH cell group.

    - The number of HARQ-ACK bits for multi-PDSCH DCI in case of separate sub-codebooks, or for all DL DCIs in case of single codebook, does not depend on the number of actually scheduled PDSCHs, rather, it is fixed as the maximum configured number of PDSCHs.

    - FFS: time domain bundling of HARQ-ACK feedback, as per agreement in RAN1#104-e

  - Note that multi-PDSCH DCI refers to a DL DCI where at least one entry of the TDRA table allows scheduling more than one PDSCH

Conclusion:

The following is observed for alternative 2 from prior agreement.

- For Alt 2a (C-DAI/T-DAI is counted per PDSCH with a single codebook) of generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs,

  - C-DAI/T-DAI in DL DCI: Bit-width can be increased (FFS: by how much), in DL DCI not only for multi-PDSCH DCI but also for single-PDSCH DCI for all serving cells including a serving cell not configured with multi-PDSCH DCI

  - T-DAI in UL DCI: Bit-width can be increased (FFS: by how much), in UL DCI for all serving cells including a serving cell not configured with multi-PDSCH DCI

  - C-DAI/T-DAI in DL DCI and T-DAI in UL DCI shall be designed such that at most 3 consecutive DCI missing can be resolved, same as in Rel-15/16 NR.

    - FFS: details on increment of DAI field size

    - FFS: whether/how to handle the case where different DCI formats (e.g., DCI format 1_0 and DCI format 1_1) have different field sizes for C-DAI/T-DAI

  - HARQ-ACK codebook generation:

    - The number of HARQ-ACK bits depends on the number of scheduled PDSCHs.

    - FFS: ordering of the PDSCHs for DAI counting

    - FFS: time domain bundling of HARQ-ACK feedback, as per agreement in RAN1#104-e

  - Note that multi-PDSCH DCI refers to a DL DCI where at least one entry of the TDRA table allows scheduling more than one PDSCH

Conclusion:

The following is observed for alternative 3 from prior agreement.

- For Alt 3 (C-DAI/T-DAI is counted per M scheduled PDSCH(s), where M is configurable) of generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs,

  - If M equals to the maximum configured number of PDSCHs, Alt 3 is the same with Alt 1, if the same number of codebooks is assumed.

  - Else if M equals to 1, Alt 3 is the same with Alt 2.

  - Otherwise (i.e., 1<M<the maximum configured number of PDSCHs), Alt 3 is similar to Alt 2, except that

    - The number of HARQ-ACK bits corresponding to each DAI increases by M times.

    - NACK bits may be padded if the number of scheduled PDSCHs is not an integer multiple of M.

    - FFS: details on DAI field size

    - FFS: whether single codebook or separate sub-codebooks is(are) generated when multi-PDSCH DCI is configured for a serving cell

  - In addition, new RRC parameter to configure M needs to be introduced.

  - Note that multi-PDSCH DCI refers to a DL DCI where at least one entry of the TDRA table allows scheduling more than one PDSCH

- RAN1#105-e

Agreement:

- Do not use fallback DCI (i.e., DCI formats 0_0 and 1_0) for multi-PDSCH/PUSCH scheduling.

- Use DCI format 0_1 to schedule multiple PUSCHs with a single DCI.

- Use DCI format 1_1 to schedule multiple PDSCHs with a single DCI.

Conclusion:

For a DCI that can schedule multiple PUSCHs,

- CSI-request: When the DCI schedules M PUSCHs, the PUSCH that carries the aperiodic CSI feedback is M-th scheduled PUSCH for M <= 2, or (M-1)-th scheduled PUSCH for M > 2.

Agreement:

- If a PDSCH among multiple PDSCHs that are scheduled by a single DCI is collided with uplink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, the UE does not receive the PDSCH.

  - FFS on how to handle HARQ-related issue for the PDSCH (e.g., HARQ process numbering)

- The UE does not expect to be scheduled with multiple PDSCHs by a single DCI, where every PDSCH is collided with uplink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

- If a PUSCH among multiple PUSCHs that are scheduled by a single DCI is collided with downlink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, the UE does not transmit the PUSCH.

  - FFS on how to handle HARQ-related issue for the PUSCH (e.g., HARQ process numbering)

- The UE does not expect to be scheduled with multiple PUSCHs by a single DCI, where every PUSCH is collided with downlink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

Agreement:

For TDRA in a DCI that can schedule multiple PDSCHs (or PUSCHs),

- A row of the TDRA table can indicate PDSCHs (or PUSCHs) that are in consecutive or non-consecutive slots.

  - FFS: The maximum value of the gap between two consecutively scheduled PDSCHs or between two consecutively scheduled PUSCHs

  - FFS: The maximum value of the gap between the first scheduled PDSCH and the last scheduled PDSCH or between the first scheduled PUSCH and the last scheduled PUSCH

  - FFS: Details to introduce the gap between PDSCHs or between PUSCHs

Agreement:

For enhancements of generating type-1 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the set of candidate PDSCH reception occasions corresponding to a UL slot with HARQ-ACK transmission is determined based on a set of DL slots and a set of SLIVs corresponding to each DL slot belonging to the set of DL slots.

- The set of DL slots includes all the unique DL slots that can be scheduled by any row index r of TDRA table in DCI indicating the UL slot as HARQ-ACK feedback timing.

- The set of SLIVs corresponding to a DL slot (belonging to the set of DL slots) at least include all the SLIVs that can be scheduled within the DL slot by any row index r of TDRA table in DCI indicating the UL slot as HARQ-ACK feedback timing.

  - FFS: details of further pruning of the set of SLIVs

  - FFS: impact if receiving more than one PDSCH in a slot is allowed, e.g., handling of overlapped SLIVs from different rows in the same and different DL slot

  - FFS impact of time domain bundling, if supported

Agreement:

- At least for 120 kHz SCS, for a DCI that can schedule multiple PUSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs,

  - If CBG-based (re)transmission is configured, CBGTI field is not present when more than one PUSCHs are scheduled, but is present when a single PUSCH is scheduled, as in Rel-16.

- FFS:

  - For 480/960 kHz SCS, whether to apply the same behavior with 120 kHz SCS or not to support CBGTI field configuration in the DCI that can schedule multiple PUSCHs

  - For a DCI that can schedule multiple PDSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs, whether/how to configure CBGTI/CBGFI fields

Agreement:

If Alt 1 (C-DAI/T-DAI is counted per DCI) is adopted for generating type-2 HARQ-ACK codebook corresponding to a DCI that can schedule multiple PDSCHs,

- At least two sub-codebooks are generated for a PUCCH cell group where

  - The first sub-codebook is for the following cases:

    - Any DCI that is not configured with CBG-based scheduling and is configured with TDRA table containing rows each with a single SLIV

    - Any DCI that is not configured with CBG-based scheduling and is configured with TDRA table containing at least one row with multiple SLIVs and schedules only a single PDSCH

  - The second sub-codebook is for the following case:

    - Any DCI that is configured with TDRA table containing at least one row with multiple SLIVs and schedules multiple PDSCHs

      - FFS: Methods (if needed) to align the size of HARQ-ACK feedback corresponding to different DCIs

      - FFS: Whether HARQ-ACK bits for 2 PDSCHs scheduled by this DCI can be included in the first sub-codebook in some cases

  - FFS: SPS PDSCH release, SCell dormancy indication without scheduled PDSCH

- FFS: 2 or 3 sub-codebooks if CBG is configured for a serving cell in the PUCCH cell group

- FFS: impact of time domain bundling, if supported, e.g., the number of sub-codebooks including single codebook if all A/N bits are bundled into a single bit per DCI

Agreement:

If Alt 2 (C-DAI/T-DAI is counted per PDSCH) is adopted for generating type-2 HARQ-ACK codebook corresponding to a DCI that can schedule multiple PDSCHs,

- PDSCH(s) scheduled by a single DCI is counted firstly, serving cell(s) in the same PUCCH cell group and same PDCCH monitoring occasion is counted secondly, and PDCCH monitoring occasion(s) is counted thirdly.

- The bit width of counter DAI field in fallback DCI (i.e., DCI formats 0_0 and 1_0) remains the same as in Rel-15 NR.

- Note: The DAI bit width and number of sub-codebooks shall ensure that at most 3 consecutive missed DCIs can be resolved, same as in Rel-15/16 NR

  - This shall not impose additional gNB’s scheduling restriction.

- In case where CBG retransmission is not configured for any serving cell in a same PUCCH cell group, the number of bits for each of counter DAI and total DAI in non-fallback DCI is extended (if needed) at least based on

  - The number of SLIVs associated with the row indexes in TDRA table

    - FFS: details

- FFS: the case with configuration of CBG retransmission

- FFS: the number of sub-codebooks

- FFS: for the UE indicating by type2-HARQ-ACK-Codebook support for more than one PDSCH reception on a serving cell that are scheduled from a same PDCCH monitoring occasion

- RAN1#106-e

Working assumption:

Scheduling multiple PDSCHs by single DL DCI applies to 120 kHz in addition to 480 and 960 kHz at least in FR2-2.

- FFS: Further limitations on maximum number of PDSCHs

Agreement:

Adopt Alt 1 (C-DAI/T-DAI is counted per DCI) for generating type-2 HARQ-ACK codebook corresponding to a DCI that can schedule multiple PDSCHs.

Agreement:

- The maximum number of PDSCHs/PUSCHs that can be scheduled with a single DCI in Rel-17 is 8 for SCS of 120, 480 and 960 kHz.

- FFS: Whether UE capability is introduced for restricting the maximum number of PDSCHs or PUSCHs that can be scheduled with a single DCI

Agreement:

If a scheduled PDSCH/PUSCH is dropped due to collision with UL/DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, HARQ process number increment is skipped for the PDSCH/PUSCH and applied only for valid PDSCH(s)/PUSCH(s).

- FFS: HARQ process number determination for the case where a scheduled PDSCH/PUSCH collides with a flexible symbol (indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated) if the UE is configured to monitor DCI format 2_0.

Agreement:

- For a DCI that can schedule multiple PUSCHs,

  - Priority indicator and open loop power control parameter set indication fields are applied to all of scheduled PUSCHs.

- For a DCI that can schedule multiple PDSCHs,

  - Priority indicator field is applied to all of scheduled PDSCHs.

Agreement:

For TDRA in a DCI that can schedule multiple PDSCHs (or PUSCHs),

- A row of the TDRA table can indicate PDSCHs (or PUSCHs) that are in consecutive or non-consecutive slots, by configuring {SLIV, mapping type, scheduling offset K0 (or K2)} for each PDSCH (or PUSCH) in the row of TDRA table.

- Note: Whether and how to reduce RRC overhead is left to RAN2.

Agreement:

For a DCI that can schedule multiple PDSCHs,

- Each of VRB-to-PRB mapping, PRB bundling size indicator, ZP-CSI-RS trigger, and rate matching indicator fields appears only once in the DCI.

- VRB-to-PRB mapping and PRB bundling size indicator fields are applied to all the PDSCHs scheduled by the DCI.

- For ZP-CSI-RS trigger field, the triggered aperiodic ZP CSI-RS is applied to all the slot(s) in which the PDSCH(s) scheduled by the DCI are contained.

- When receiving a PDSCH scheduled by the DCI, the REs corresponding to configured resources in rateMatchPatternGroup1 or rateMatchPatternGroup2 (according to indication of rate matching indicator field) are not available for the scheduled PDSCH.

Working assumption:

For NR FR2-2, two codeword transmission is supported, subject to UE capability.

- RRC parameter configures whether two codeword transmission is enabled or disabled.

  - FFS: Details on signaling of MCS/NDI/RV for the second TB in a DCI that can schedule multiple PDSCHs when two codeword transmission is enabled

  - FFS: Whether unified or separate parameter to enable/disable 2-TB for single and for multiple PDSCH scheduling

  - Strive to minimize the increase in the number of bits in the DCI needed to support this feature

Agreement:

- For single TRP operation, for 480/960 kHz SCS,

  - FFS: A UE does not expect to be scheduled with more than one PDSCH in a slot, by a single DCI or multiple DCIs.

  - FFS: A UE does not expect to be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs.

- For single TRP operation, for 120 kHz SCS (same as current specification for FR2-1 for PUSCH),

  - Subject to UE capability, a UE can be scheduled with more than one PDSCH in a slot, by a single DCI or multiple DCIs.

  - Subject to UE capability, a UE can be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs.

- FFS for multi-TRP operation

- Note: The optimization of HARQ codebook size for Type 1 or Type 2 codebook design is considered as a low priority in Rel-17 (this does not preclude HARQ ACK bundling in time domain).

- The agreement made in RAN1#105-e is revised as follows.

Agreement: (RAN1#105-e) For enhancements of generating type-1 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the set of candidate PDSCH reception occasions corresponding to a UL slot with HARQ-ACK transmission is determined based on a set of DL slots and a set of SLIVs corresponding to each DL slot belonging to the set of DL slots. - The set of DL slots contains all the unique DL slots determined by considering all combinations of the configured K1 values and the configured rows of the TDRA table. - The set of SLIVs corresponding to a DL slot (belonging to the set of DL slots) contains all the SLIVs for that slot determined by considering all combinations of the configured K1 values and the configured rows of the TDRA table. - The Rel-16 procedure is reused for determining the candidate PDSCH reception occasions for the set of SLIVs corresponding to each DL slot belonging to the set of DL slots - Note: The Rel-16 procedure already handles pruning of multiple SLIVs corresponding to a DL slot, for both UEs that are and are not capable of receiving multiple PDSCHs per slot - FFS impact of time domain bundling, if supported

Agreement:

Consider the following options to construct type-2 HARQ-ACK codebook when CBG operation is configured, and down-select to one of the following options in RAN1#106bis-e.

- Option 1: HARQ-ACK bits corresponding to CBG-based PDSCH reception and multi-PDSCH reception are merged into the same sub-codebook.

- Option 2: HARQ-ACK bits corresponding to CBG-based PDSCH reception and HARQ-ACK bits corresponding to multi-PDSCH reception are contained in separate sub-codebooks.

- Option 3: UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group.

- Note: Multi-PDSCH reception refers to the case where multiple PDSCHs are scheduled by a DCI that is configured with TDRA table containing at least one row with multiple SLIVs.

Agreement:

For NR FR2-2 at least for 480/960 kHz SCS, support 32 as the maximum number of HARQ processes for DL and UL, subject to UE capability.

- Note: Up to 32 maximal supported HARQ process number is already agreed in Rel-17 NTN WI.

- Working assumption: The same solution to support up to 32 HARQ process number in Rel-17 NTN WI is reused for NR FR2-2.

- RAN1#106bis-e

Agreement:

Confirm the working assumption from RAN1#106-e with the following modification.

Working assumption: (RAN1#106-e)

Scheduling multiple PDSCHs by single DL DCI applies to 120 kHz in addition to 480 and 960 kHz at least in FR2-2.

- FFS: Further limitations on maximum number of PDSCHs

- Note: Further limitations (in addition to what was agreed earlier) on the maximum number of PDSCHs or PUSCHs can be separately discussed for all SCSs.

Working assumption:

UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group with a Type 2 codebook.

- If time bundling operation is supported, this working assumption can be revisited

Agreement:

For a PDSCH that is scheduled by multi-PDSCH scheduling DCI and is skipped due to collision with semi-static UL symbol(s),

- For Type-1 HARQ-ACK codebook generation, the PDSCH is not considered and the HARQ-ACK bit corresponding to the PDSCH is not reported by UE.

  - Note: Rel-16 procedure can be reused to handle this case.

- For Type-2 HARQ-ACK codebook generation, UE reports NACK for the PDSCH.

  - FFS on HARQ-ACK bit ordering

- Note: Codebook generation in case time domain bundling is enabled can be separately discussed if time domain bundling is supported.

Agreement:

For generating type-2 HARQ-ACK codebook corresponding to a DCI that can schedule multiple PDSCHs,

- HARQ-ACK bit corresponding to SPS PDSCH release or SCell dormancy indication without scheduled PDSCH, belongs to the first sub-codebook (which is defined in the previous agreement made in RAN1#105-e)

Agreement:

For two multi-PDSCH (or two multi-PUSCH) scheduling DCIs, UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCI to lead to out-of-order scheduling.

- FFS: whether to allow OOO scheduling for the following two cases:

  - for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH)

  - for the case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV

- Note: The above FFS aspect applies only to multi-PDSCH and multi-PUSCH scheduling with single DCI

Agreement:

For multiple PDSCHs (or PUSCHs) scheduled by a single DCI,

- Rel-15/16 behavior that is described in TS 38.213 Clauses 11 and 11.1 for a PDSCH (or PUSCH) indicated by DCI also applies for multiple PDSCHs (or PUSCHs) schedule by a single DCI.

- If one of multiple PDSCHs (or PUSCHs) scheduled by the DCI collides with a flexible symbol (indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated),

  - If that PUSCH is collided with SSB symbols indicated by ssb-PositionsInBurst [or symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set], the HARQ process number increment is skipped for the PUSCH.

  - Otherwise, the HARQ process number increment is not skipped for that PDSCH (or PUSCH).

Conclusion:

For a DCI that can scheduled multiple PDSCHs (or PUSCHs), HARQ process number indicated in the DCI is applied to the first valid PDSCH (or PUSCH).

- Note: This is the consequence of previous agreements.

Agreement:

For single TRP operation, for 480/960 kHz SCS,

- A UE does not expect to be scheduled with more than one unicast PDSCH in a slot, by a single DCI or multiple DCIs.

- A UE does not expect to be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs.

Agreement:

For a DCI that can schedule multiple PDSCHs, and if RRC parameter configures that two codeword transmission is enabled,

- MCS for the 2nd TB: This appears only once in the DCI and applies commonly to the 2nd TB of each PDSCH

- NDI for the 2nd TB: This is signaled per PDSCH and applies to the 2nd TB of each PDSCH

- RV for the 2nd TB: This is signaled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the 2nd TB of each PDSCH

- FFS: the maximum number of PDSCHs when 2 TB is enabled or when 2 TB is scheduled

- RAN1#107-e

Agreement

- For multi-PDSCH or multi-PUSCH scheduling DCI, FDRA enhancement is deprioritized in Rel-17.

Agreement

- For multi-TRP operation, for 480/960 kHz SCS,

  - A UE does not expect to be scheduled with more than one unicast PDSCH in a slot, by a single DCI or multiple DCIs, from the same TRP.

  - A UE does not expect to be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs, from the same TRP.

  - Note: This does not preclude a UE being scheduled with two PDSCHs (or two PUSCHs) in the same slot from two different TRPs for multi-DCI based multi-TRP mechanism.

Agreement

- For a DCI that can schedule multiple PDSCHs, CBGTI and CBGFI fields are not present in the DCI.

- UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the serving cell with a Type 1 codebook.

- Confirm the working assumption from RAN1#106bis-e with the following modification.

Working assumption: (RAN1#106bis-e)

- UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group with a Type 2 codebook.

  - If time bundling operation is supported, this working assumption can be revisited

Agreement

For 480/960 kHz SCS, CBG-based HARQ cannot be configured for uplink and downlink.

Agreement

- The maximum number of PDSCHs that can be scheduled with a single DCI in Rel-17 is also 8 when 2 TB is enabled or when 2 TB is scheduled, for SCS of 120, 480 and 960 kHz.

  - Note: This is to handle FFS (the maximum number of PDSCHs when 2 TB is enabled or when 2 TB is scheduled) in previous agreement in RAN1#106bis-e.

Agreement

For multi-PUSCH scheduling DCI in Rel-17, support intra-slot frequency hopping which is applicable to each of multiple PUSCH transmissions scheduled by the DCI, and do not support inter-slot frequency hopping.

Agreement

For multi-PDSCH scheduling with a single DCI

- Introduce a new RRC parameter, e.g., enableTimeDomainHARQ-Bundling, to enable time domain bundling operation for type-1 HARQ-ACK codebook per serving cell.

  - If the RRC parameter enables time domain bundling operation,

    - To determine the set of candidate PDSCH reception occasions,

      - A row index is removed if at least one symbol of every PDSCH associated with the row index is configured as semi-static UL. (NOTE: This is similar to the case of slot aggregated PDSCH in Rel-16)

      - Pruning procedure in Rel-16 is performed based on the last configured SLIV of each row index.

    - Logical AND operation is applied across all valid PDSCHs associated with a determined candidate PDSCH reception occasion, at least for 1-TB case.

    - FFS: UE does not expect the last scheduled SLIV overlaps with a semi-static UL symbol when parameter enableTimeDomainHARQ-Bundling is configured

Agreement

- If a UE is configured with a TDRA table in which one or more rows contain multiple SLIVs for PDSCH for DCI format 1_1, the UE does not expect to be configured with repetitionNumber for the TDRA table, and if pdsch-AggregationFactor is configued in PDSCH-config, it does not apply to DCI format 1_1.

  - Note: repetitionNumber cannot be configured with pdsch-TimeDomainAllocationListDCI-1-2 as in Rel-16.

  - Note: Under agenda item 8.2.4, in RAN1#106-bis, it was already agreed that within the TDRA table for multi-PDSCH scheduling, the UE does not expect to be configured with the higher layer parameter repetitionNumber.

  - Note: These does not preclude pdsch-AggregationFactor can be configured and applies to DCI format 1_2

- If a UE is configured with a TDRA table in which one or more rows contain multiple SLIVs for PUSCH for DCI format 0_1, the UE does not expect to be configured with numberOfRepetitions for the TDRA table, and if pusch-AggregationFactor is configued in PUSCH-config, it does not apply to DCI format 0_1.

  - Note: These does not preclude numberOfRepetitions is configured for TDRA table corresponding to DCI format 0_2

  - Note: These does not preclude pusch-AggregationFactor can be configured and applies to DCI format 0_2

Agreement

- For type-2 HARQ-ACK codebook generation, HARQ-ACK bit ordering is based on configured SLIV position in the indicated TDRA row index, regardless of the validity of each scheduled PDSCH.

Agreement

- There is no consensus in RAN1 to support that HARQ-ACK information corresponding to different PDSCHs scheduled by a single DCI is carried over multiple PUCCHs in Rel-17.

Agreement

For multi-PDSCH scheduling with a single DCI

- Introduce a new RRC parameter, e.g., numberOfHARQ-BundlingGroups, to configure the number of HARQ bundling groups with value range {1, 2, 4} for type-2 HARQ-ACK codebook per serving cell.

  - If the RRC parameter is not configured for a serving cell, time domain bundling for type-2 HARQ-ACK codebook is not enabled for the serving cell.

  - The maximum number of PDSCHs allocated to each bundling group is ceil(NPDSCH,MAX/NHBG) where NHBG is the number of bundling groups configured by numberOfHARQ-BundlingGroups for a serving cell and NPDSCH,MAX is the maximum configured number of PDSCHs for the serving cell.

  - The PDSCHs corresponding to [configured or valid] SLIVs in a TDRA row index indicated by multi-PDSCH scheduling DCI are allocated to the bundling groups, e.g., if NHBG =4, NPDSCH,MAX =8, and 5 PDSCHs are scheduled, then 2/1/1/1 PDSCHs are assigned to each group, by reusing CBG grouping method.

    - For a group that is empty or is filled with only invalid PDSCH(s), HARQ-ACK bits for the bundling group is set to NACK (same principle as when no time bundling configured)

    - Logical AND operation is applied to across all valid PDSCHs within the same bundling group to generate 1 HARQ-ACK bit per group, at least for 1-TB case

  - If the number of HARQ bundling groups is configured as 1 for a serving cell, HARQ-ACK bits corresponding to any DCI for the serving cell belong to the first sub-codebook.

  - At least for 1-TB case, if the number of HARQ bundling groups is configured as larger than 1 for a serving cell, HARQ-ACK bits corresponding to multi-PDSCH scheduling case (which implies a multi-PDSCH DCI schedules more than one PDSCH) for the serving cell belong to the second sub-codebook,

    - Where the number of HARQ-ACK bits corresponding to a multi-PDSCH DCI is determined based on the maximum of Q value across all serving cells within the same PUCCH cell group, and Q=maximum configured number of PDSCHs for a cell without numberOfHARQ-BundlingGroups configured or Q=number of configured HARQ bundling groups for a cell with numberOfHARQ-BundlingGroups configured