3GPP TSG RAN WG1 #107bis-e		                                   					       												                 R1-2200765

e-Meeting, January 17th – 25th, 2022

Agenda Item:	8.15.1

Source:	Moderator (AT&T)

Title:	Summary of UE features for further enhancements on NR-MIMO

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email discussion/approval [107bis-e-R17-UE-features-MIMO-01] during RAN1 #107bis-e. According to the Chairman’s Notes:

[107bis-e-R17-UE-features-MIMO-01] Email discussion UE features for further enhancements on NR-MIMO – Ralf (AT&T) - 1st check point: January 20 - Final check point: January 25

The following was discussed and/or agreed during RAN1 #107bis-e within the scope of [107bis-e-R17-UE-features-MIMO-01]. All proposals are based on the latest RAN1 UE features list for Rel-17 NR in [1].

# Summary of Contributions Submitted to RAN1 #107bis-e

The following is the moderator’s summary of contributions submitted to RAN1 #107bis-e in this agenda item.

The following was agreed in RAN1 #107-e [21].

Agreement: - The following are the components of the feature “joint TCI for intra-cell operation” - Joint DL/UL TCI update with their components: (configuration mechanism, [QCL rules,] applicable source and target signals, MAC-CE -based TCI] activation assignment) - Common cross multi-CC TCI update and activation (involving RRC common TCI state pool) - Association between TCI state and UL PC settings except for PL RS [for PUCCH, PUSCH, and SRS] - Beam misalignment between the DL source RS in the TCI state to provide spatial relation indication and the PL-RS - [MAC-CE+DCI-based TCI state indication (including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)] - [TCI states pool for configured reference BWP/CC shared by a set of BWP/CC] - [Maximum number of CCs configured with BFR] - [R17 mechanism reusing R15/16 signalling to indicate R17 TCI for individual DL channel/RS that cannot share the same unified TCI as UE-dedicated PDCCH/PDSCH] - [Configuration of both R17 TCI and R15/16 TCI and spatial relation] - The maximum number of configured joint TCI state pools across all BWPs and all CCs in a band - [The maximum number of configured joint TCI states across all BWPs and all CCs in a band] - [The maximum number of configured joint TCI states per CC [in a band]] - [Support The maximum number of MAC-CE activated joint TCI states [per BWP per CC/across all BWPs and all CCs in a band]] - [Support any Whether a particular DL RS that is a valid target DL RS of a Rel-15/16 TCI state based on the Rel-15/16 QCL rules can be configured as a target DL RS of Rel-17 DL TCI (hence the Rel-17 DL TCI state pool)] - [The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS] - [The minimum time gap between the beam indication PDCCH and first slot where beam is applied] - This list gives no guidance as to how to group these components into FGs. They may be mapped to one or more FGs and one or more of these FGs may be agreed as basic. Specifically, different FGs can be grouped together through notes “a UE supporting this FG must also support FG 23-x-y”. This allows to create “basic” FGs while allowing, e.g., different types for different components/candidate values. - The purpose of this list is to find the individual components without having to worry about the signalling structure

| 23. NR_FeMIMO | 23-1-1 | Unified TCI for [intra- and inter-cell] beam management | [For both intra- and inter-cell beam management:] 1. Support of joint DL/UL TCI update [and separate DL/UL TCI update] with their components (configuration mechanism, QCL rules, applicable source and target signals) [and Common cross-CC TCI update and activation (involving RRC common TCI state pool)] 2. Support of association between TCI state and UL PC settings [for PUCCH, PUSCH, and SRS] [ (PLRS and other, including handling of beam [alignment/misalignment] for PLRS)] 3. Supported mode of [MAC-CE/MAC-CE+DCI]-based TCI state indication [(including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)] [4. The maximum number of configured TCI state pools across all BWPs and all CCs in a band] [5. The maximum number of configured TCI states across all BWPs and all CCs in a band] [6. The maximum number of configured TCI states per CC in a band] [7. Support number of MAC-CE activated joint TCI states across all BWPs and all CCs in a band] [8. The maximum number of MAC-CE activated separate DL and UL TCI states across all BWPs and all CCs in a band] [9. Support of association between a TCI state and PL-RS, where the “beam alignment” between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is assumed by the UE [10. Support any DL RS that is a valid target DL RS of a Rel-15/16 TCI state based on the Rel-15/16 QCL rules can be configured as a target DL RS of Rel-17 DL TCI (hence the Rel-17 DL TCI state pool) [11. The minimum beam application time in Y symbols] [12. The maximum number of PCI(s) different from serving cell PCI that can be associated with activated TCI states] |  |  |  |  | [per band] |  |  |  | [4. candidate values {1, …, 32} 5. candidate values {64, 72, 80, 96, 128, 192, 256, 128*4*32} 6. candidate values {8, 16, 32, 64, 128, 256} 3. candidate values FFS 7. candidate values {1, 2, 3, 4, 5, 6, 7, 8} 8. candidate values {1, 2, 3, 4,5,6,7, 8} Note 1: for component 7 & 8, if two TCI states are in one codepoint, it should be counted as 2.] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | Unified TCI framework It was agreed in RAN#106-e that the maximum number of configured TCI state pools across BWPs and CCs in a band is reported as UE capability. Agreement (RAN1#106-e) On Rel.17 unified TCI framework, confirm the following working assumption as an agreement with a minor refinement highlighted in red For common TCI state ID update and activation to provide common QCL information at least for UE-dedicated PDCCH/PDSCH and/or common UL TX spatial filter(s) at least for UE-dedicated PUSCH/PUCCH across a set of [configured] CCs/BWPs: - RRC-configured TCI state pool(s) can be configured in the PDSCH configuration (PDSCH-Config) for each BWP/CC as in Rel-15/16 - Note: Such RRC-configured TCI state pool(s) configuration doesn’t imply that separate DL/UL TCI state pool is excluded or supported - RRC-configured TCI state pool(s) can be absent in the PDSCH configuration (PDSCH-Config) for each BWP/CC, and replaced with a reference to RRC-configured TCI state pool(s) in a reference BWP/CC - In the PDSCH configuration (PDSCH-Config) of the reference BWP/CC, RRC-configured TCI state pool(s) shall be configured - For a BWP/CC where the PDSCH configuration contains a reference to the RRC-configured TCI state pool(s) in a reference BWP/CC, the UE applies the RRC-configured TCI state pool(s) in the reference BWP/CC - When the BWP/CC ID (i.e. bwp-Id or cell) for QCL-Type A/D source RS in a QCL-Info of the TCI state is absent, the UE assumes that QCL-Type A/D source RS is in the BWP/CC to which the TCI state applies - Introduce a UE capability to report maximum number of TCI state pools it can support across BWPs and CCs in a band, and the candidate value at least includes 1 - FFS: Introduce a UE capability to report maximum number of configured TCI states that it can support across BWPs and CCs in a band - FFS: How to define reference BWP/CC In addition, to reduce memory consumption at UE, similar to legacy UE feature 2-4/2-59 in Rel-15/16, it should be supported to report maximum number of configured unified TCI states across BWPs/CCs in a band in a band combination and per CC in a band in a band combination. With above analysis, we propose the following: Proposal 2-1: Revise component 4/5/6 in FG 23-1-1 as follows and remove the corresponding brackets - 4. The maximum number of configured TCI state pools across all BWPs and all CCs in a band in a band combination - 5. The maximum number of configured TCI states across all BWPs and all CCs in a band in a band combination - 6. The maximum number of configured TCI states per CC in a band in a band combination It was agreed in RAN#105-e that the maximum number of activated UL TCI states or joint TCI state is reported as UE capability. Agreement (RAN1#105-e) On path-loss measurement for Rel.17 unified TCI framework, a PL-RS (configured for path-loss calculation) is either included in UL TCI state or (if applicable) joint TCI state or associated with UL TCI state or (if applicable) joint TCI state. - Whether a UE supports “beam misalignment or not” (detailed definition FFS) between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is a UE capability - Note: The term “beam misalignment” is for discussion purpose only - Whether it is ‘included in’ or ‘associated with’ (including the manner it is performed and the signaling) is up to RAN2 - The UE maintains the PL-RS of the activated UL TCI state or (if applicable) joint TCI state - The maximum number of activated UL TCI states or (if applicable) joint TCI states per band per cell is a UE capability - FFS: detailed aspects of PL-RS, e.g. CSI-RS type(s), restriction on configuration - FFS: For the definition of “beam misalignment or not”, at least consider the case where the periodic DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication is configured/associated as the PL-RS - Note: PL-RS is assumed to be periodic With above analysis, we propose the following: Proposal 2-2: Revise component 7/8 in FG 23-1-1 as follows and remove the corresponding brackets - 7. The maximum number of MAC-CE activated joint TCI states across all BWPs and all CCs in a band in a band combination - 8. The maximum number of MAC-CE activated separate DL and UL TCI states across all BWPs and all CCs in a band in a band combination 23. NR_FeMIMO; 23-1-1; Unified TCI for [intra- and inter-cell] beam management; [For both intra- and inter-cell beam management:] 1. Support of joint DL/UL TCI update [and separate DL/UL TCI update] with their components (configuration mechanism, QCL rules, applicable source and target signals) [and Common cross-CC TCI update and activation (involving RRC common TCI state pool)] 2. Support of association between TCI state and UL PC settings [for PUCCH, PUSCH, and SRS] [ (PLRS and other, including handling of beam [alignment/misalignment] for PLRS)] 3. Supported mode of [MAC-CE/MAC-CE+DCI]-based TCI state indication [(including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)] [4. The maximum number of configured TCI state pools across all BWPs and all CCs in a band in a band combination] [5. The maximum number of configured TCI states across all BWPs and all CCs in a band in a band combination] [6. The maximum number of configured TCI states per CC in a band in a band combination] [7. Support number of MAC-CE activated joint TCI states across all BWPs and all CCs in a band in a band combination] [8. The maximum number of MAC-CE activated separate DL and UL TCI states across all BWPs and all CCs in a band in a band combination] [9. Support of association between a TCI state and PL-RS, where the “beam alignment” between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is assumed by the UE [10. Support any DL RS that is a valid target DL RS of a Rel-15/16 TCI state based on the Rel-15/16 QCL rules can be configured as a target DL RS of Rel-17 DL TCI (hence the Rel-17 DL TCI state pool) [11. The minimum beam application time in Y symbols] [12. The maximum number of PCI(s) different from serving cell PCI that can be associated with activated TCI states]; [per band]; [4. candidate values {1, …, 32} 5. candidate values {64, 72, 80, 96, 128, 192, 256, 128*4*32} 6. candidate values {8, 16, 32, 64, 128, 256} 3. candidate values FFS 7. candidate values {1, 2, 3, 4, 5, 6, 7, 8} 8. candidate values {1, 2, 3, 4,5,6,7, 8} Note 1: for component 7 & 8, if two TCI states are in one codepoint, it should be counted as 2.]; Optional with capability signalling. |
| ZTE [3] | Firstly, in the main feature, we need to explicit that the following functionalities are supported as part of the basic feature in 23-1-1: - The common TCI state ID update and activation for CA - Beam alignment for PL-RS - MAC-CE-only based TCI state indication Then, a new UE feature of beam misalignment has been agreed to be introduced as a sub-feature for 23-1-1, and by default the UE should mandatorily support the case of beam alignment. Also, TCI application timing for DCI based TCI state indication should be introduced as well. After that, we have the following comments for other potential UE features captured by the moderator. - Considering that only up to 128 TCI state is supported for unified TCI framework, we think that the further UE feature about maximum number of configured TCI state and TCI state pool cross CC is unnecessary at all (for component-4/5/6). - Regarding activated TCI state (component-7/8), we can support to have UE capability signaling of maximum number of joint, separate DL and separate UL TCI states, respectively. - Regarding QCL rules (component-10), we think that it should be supported by default considering that Rel-15 QCL rule is mandatory feature without capability signaling for NR UE. - Regarding maximum number of PCI (component-12), we think that we need to discuss this issue in the agenda of inter-cell mTRP firstly. We may just reuse the corresponding conclusion/UE feature. Proposal 1: For unified TCI for intra- and inter-cell beam management, the following modification in red is proposed. 23-1-1; Unified TCI for [intra- and inter-cell] beam management; [For both intra- and inter-cell beam management:] 1. Support of joint DL/UL TCI update [and separate DL/UL TCI update] with their components (configuration mechanism, QCL rules, applicable source and target signals) [and Common cross-CC TCI update and activation (involving RRC common TCI state pool)] 2. Support of association between TCI state and UL PC settings [for PUCCH, PUSCH, and SRS] [ (PLRS and other, including handling of beam [alignment/misalignment] for PLRS)] 3. Supported mode of [MAC-CE/MAC-CE+DCI]-based TCI state indication [(including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)] [4. The maximum number of configured TCI state pools across all BWPs and all CCs in a band] [5. The maximum number of configured TCI states across all BWPs and all CCs in a band] [6. The maximum number of configured TCI states per CC in a band] [7. Support number of MAC-CE activated joint TCI states in an active BWP in a CC across all BWPs and all CCs in a band] [8a. The maximum number of MAC-CE activated separate DL and UL TCI states in an active BWP in a CC 8b. The maximum number of MAC-CE activated UL TCI states in an active BWP in a CC across all BWPs and all CCs in a band] [9. Support of association between a TCI state and PL-RS, where the “beam alignment” between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is assumed by the UE [10. Support any DL RS that is a valid target DL RS of a Rel-15/16 TCI state based on the Rel-15/16 QCL rules can be configured as a target DL RS of Rel-17 DL TCI (hence the Rel-17 DL TCI state pool) [11. The minimum beam application time in Y symbols] [12. The maximum number of PCI(s) different from serving cell PCI that can be associated with activated TCI states]. 23-1-1a; Support of beam misalignment; Support of beam misalignment for PLRS. 23-1-1b; TCI application time; X symbols after transmitting HARQ-ACK, the indicated TCI state is applied to a downlink or uplink signal, in the case of DCI based TCI state indication. |
| vivo [4] | For component 1, there are still some yellow parts that need to be confirmed for QCL rule and MAC CE based update. It has already been agreed that DCI based beam indication is a separate UE capability previously and thus only MAC CE based beam indicated should be included in component1. Proposal 1-1: Update component 1 by combining component 8 as following and consider only the following component 1 as the basic feature for unified TCI for intra- and inter-cell beam management - Joint DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals, MAC-CE -based TCI activation assignment) and R17 mechanism reusing R15/16 RRC and MAC CE signalling to indicate R17 TCI for individual DL channel/RS that cannot share the same unified TCI as UE-dedicated PDCCH/PDSCH For components other than component 1, some redundant component should be deleted and they should all be grouped separately from component 1. Proposal 1-2: For components other than component 1, they can be updated as following and they are all grouped as separate UE feature from component 1: - Common cross multi-CC TCI update and activation (involving RRC common TCI state pool) - Association between TCI state and UL PC settings except for PL RS for PUCCH, PUSCH, and SRS - Beam misalignment between the DL source RS in the TCI state to provide spatial relation indication and the PL-RS - MAC-CE+DCI-based TCI state indication (including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment) - MAC-CE+DCI-based TCI state indication (including TCI state activation, use of DCI formats 1_1/1_2 without DL assignment) - The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS - TCI states pool for configured reference BWP/CC shared by a set of BWP/CC - [Maximum number of CCs configured with BFR] - [Configuration of both R17 TCI and R15/16 TCI and spatial relation] - The maximum number of configured joint TCI state pools across all BWPs and all CCs in a band - The maximum number of configured joint TCI states across all BWPs and all CCs in a band - [The maximum number of configured joint TCI states per CC [in a band]] - The maximum number of MAC-CE activated joint TCI states across all BWPs and all CCs in a band - [Support any Whether a particular DL RS that is a valid target DL RS of a Rel-15/16 TCI state based on the Rel-15/16 QCL rules can be configured as a target DL RS of Rel-17 DL TCI (hence the Rel-17 DL TCI state pool)] For inter-cell beam management an additional UE capability for association of TCI state with PCIs different from serving cell PCI should also be defined. Proposal 1-3: For inter-cell beam management, a separate UE feature 23-1-1c should be defined. - Component x: Support of TCI state association with PCIs different from serving cell PCI. |
| CATT [5] | Considering the basic feature for joint TCI for intra-cell operation, we prefer to remove component 7 and component 16 in the above agreement. Component 7 ([Maximum number of CCs configured with BFR]) is introduced considering that it is no need to configure BFR on every CC, if all CCs sharing the same analog beam. However, although the same beam is used across CCs, the interference may be different on each CC. BFR of one CC would not be applied to the other CCs. The restriction on the number of CCs configured with BFR would impact the UE performance. Regarding component 16 ([The minimum time gap between the beam indication PDCCH and first slot where beam is applied]), the corresponding UE capability (timeDurationForQCL) has been defined in Rel-15. It is not necessary to define a new UE capability in Rel-17. Proposal-3: For joint TCI for intra-cell operation, the following components should be removed: - [Maximum number of CCs configured with BFR] - [The minimum time gap between the beam indication PDCCH and first slot where beam is applied] For the the feature ‘joint TCI for inter-cell operation’, we only need to focus on the different components to those of intra-cell operation case. We prefer to remove component 22, as the definition of overlapping is not clear. For components 17,18, we prefer to remove ‘MAC-CE activated’ to be aligned with the agreement. Proposal-4: For joint TCI for inter-cell operation, the following component should be removed: - [Maximum number of overlapped SSBs in one SSB resource for L1-RSRP measurement] Proposal-5: For joint TCI for inter-cell operation, the following components should be updated as follows: - [The number of [RRC-configured/ MAC-CE activated] PCI(s) different from serving cell PCI for beam measurement in FR1] - [The number of [RRC- configured/ MAC-CE activated] PCI(s) different from serving cell PCI for beam measurement in FR2] |
| Samsung [6] | Based on the discussion in RAN1#107-e, there are 12 components in FG 23-1-1 which is a basic feature that enables the unified TCI framework for both intra-cell and inter-cell beam management. Hence, we support to remove bracket for “[intra- and inter-cell]” in the name of FG and “[For both intra- and inter-cell beam management:]” to confirm that this FG can enable basic functionalities for both intra- and inter-cell beam management for Unified TCI. Proposal 1: Remove the bracket for “[intra- and inter-cell]” in the name of FG and “[For both intra- and inter-cell beam management:]” to support functionalities for both intra- and inter-cell beam management for Unified TCI based on FG 23-1-1. 1. Support of joint DL/UL TCI update [and separate DL/UL TCI update] with their components (configuration mechanism, QCL rules, applicable source and target signals) [and Common cross-CC TCI update and activation (involving RRC common TCI state pool)]. Regarding the description of Component 1 for FG 23-1-1 as above, we are fine for both joint DL/UL TCI update as well as separate DL/UL TCI update. To avoid unnecessary complication/fragmentation, all Rel-17 UEs support both joint TCI and separate DL/UL TCI (i.e. there shouldn’t be separate UE capabilities for the two). Proposal 2: Remove the bracket for “[and separate DL/UL TCI update]” to support both joint and separate DL/UL TCI update based on FG 23-1-1. Proposal 3: Support “and Common cross-CC TCI update and activation (involving RRC common TCI state pool)”. 2. Support of association between TCI state and UL PC settings [for PUCCH, PUSCH, and SRS] [(PLRS and other, including handling of beam [alignment /misalignment] for PLRS)] [9. Support of association between a TCI state and PL-RS, where the “beam alignment” between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is assumed by the UE. Regarding the description of Component 2 and 9 for FG 23-1-1 as above, we do not support to include beam alignment related parameters/proposals for PLRS until we have a RAN1 agreement. Hence, in Component 2, we would like to remove “including handling of beam [alignment /misalignment] for PLRS)]”. Similarly, in Component 9, we prefer to remove “where the “beam alignment” between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is assumed by the UE” and only support “Support of association between a TCI state and PL-RS”. Also, in RAN1, the PL-RS can be associated with or included in a TCI state, we would like to suggest that the description of Component 9 should include not only association but also inclusion of PL-RS in TCI state. Proposal 4: Do not support to include beam alignment related parameters/proposals for PLRS for both Component 2 and 9 (i.e., remove “including handling of beam [alignment /misalignment] for PLRS)]” in Component 2 and “where the “beam alignment” between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is assumed by the UE” in Component 9) and support to revise the description of Component 9 as “Support of association between a TCI state and PL-RS or inclusion of PL-RS in TCI state”. 3. Supported mode of [MAC-CE/MAC-CE+DCI]-based TCI state indication [(including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)]. Regarding the description of Component 3 for FG 23-1-1 as above, we support MAC-CE/DCI based TCI state indication including all valid DCI formats (1_1/1_2 with and without DL assignment). We are against separating different types of beam indication mechanisms into different FGs, i.e. they should all be supported together. Hence, we prefer to revise Component 3 as “Supported mode of MAC-CE/MAC-CE+DCI-based TCI state indication (including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)”. Also, a UE that supports Rel-17 unified TCI framework supports at least 4 activated TCI states by MAC-CE+DCI-based TCI indication with and without DL assignment for both intra-cell and inter-cell beam management. Proposal 5: Revise the description of Component 3 as “Supported mode of MAC-CE/MAC-CE+DCI-based TCI state indication (including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)” Proposal 6: Support 4 activated TCI states by MAC-CE+DCI-based TCI indication with and without DL assignment for both intra-cell and inter-cell beam management. [11. The minimum beam application time in Y symbols]. Regarding Component 11 for FG 23-1-1 as above, we support the component for minimum beam activation latency (Y symbols). The candidate values for the minimum Y per SCS can be discussed further. Note that Y is configurable by the NW. Proposal 7: Support Component 11, i.e., “The minimum beam application time in Y symbols”, and the candidate values per SCS can be discussed further. [4. The maximum number of configured TCI state pools across all BWPs and all CCs in a band] [5. The maximum number of configured TCI states across all BWPs and all CCs in a band] [6. The maximum number of configured TCI states per CC in a band] [7. Support number of MAC-CE activated joint TCI states across all BWPs and all CCs in a band] [8. The maximum number of MAC-CE activated separate DL and UL TCI states across all BWPs and all CCs in a band]. For FG 23-1-1, in order to avoid duplicated functionality for the “counting” features as above components from 4 to 8, e.g., if UE supports N TCI states for intra-cell, it should support N for inter-cell as well. Proposal 10: Support same value for “counting” features for both intra-cell and inter-cell beam management. For both FG 23-1-1 and 23-1-2, which are considered as basic features for Rel-17 TCI framework, we would like to propose the signalling granularity as per band based on two reasons. First reason is that based on working assumption in RAN1#107-e, the UE is not expected to be configured with Rel-15/Rel-16 TCI/SpatialRelationInfo if the UE is configured with Rel-17 TCI in any CC in a band. Second reason is that according to RAN2 guidance, the UE features which have “per-band combination” or “per-band per band combination” should be minimized. Hence, “per band” granularity is quite natural starting point to consider for those basic features. Proposal 11: Support FG 23-1-1 and FG 23-1-2 as basic features and per-band granularity. |
| NTT DOCOMO, INC. [7] | We believe FG23-1-1 and FG23-1-2 should be reported per band. For FG23-1-1, we have the following suggestion: - Component 2: “[, SS-RSRP]” is not necessary in addition to “SSBRI-RSRP”, and hence we can delete it. - Component 3: MAC CE-based TCI state indication should be basic feature of FG23-1-1. Hence, the component values should be {“MAC CE”, “MAC CE+DCI”}. - Component 4: The number of TCI state pools across all BWPs/CCs should not be limited by UE capability. In Rel.15, TCI state pool (list) is configured per BWP in a CC, and we should not restrict gNB to configure one TCI state pool per BWP in a CC by UE capability. - Component 5/6: In Rel.15, the number of configured TCI state for PDSCH per CC is reported, however, the number of configured TCI states across all BWPs/CCs is not reported. Hence, only component 6 should be reported. - For the candidate values of component 6, in Rel.15, UE is mandated to report the number of RRC configured TCI states as “64” in FR2 and “the maximum number of allowed SSBs” in FR1. Since Rel.17 unified TCI framework replaces all of Rel.15/16 beam indication mechanism, the mandatory value of Rel.15/16 TCI state should be supported in Rel.17 TCI state. That is, UE is mandated to report the number of RRC configured TCI states per BWP in a CC as “64” in FR2 and “the maximum number of allowed SSBs” in FR1. - Component 7/8: In Rel.15, the number of activated TCI state for PDSCH per BWP in a CC is reported, however, the number of activated TCI states across all BWPs/CCs is not reported. Hence, component 7/8 should be reported per BWP in a CC. - Component 10: Component 10 should be a basic feature of FG23-1-1. In RAN1#107e, we made the following WA, which means once Rel.17 TCI state is configured, gNB cannot configure Rel.15/16 TCI state/QCL in the same band. Hence, component 10 cannot be an optional feature. Working Assumption The UE is not expected to be configured with Rel-15/Rel-16 TCI/SpatialRelationInfo if the UE is configured with Rel-17 TCI in any CC in a band - The CC list for Rel-16 multi-CC beam indication should not contain any CC configured with Rel-17 TCI. - Component 12: In RAN1#106e meeting, the number of RRC configured PCIs per CC for M-TRP inter cell was agreed as X<=7. We believe UE would report the same/similar value for L1/L2 inter cell mobility and M-TRP inter cell. Hence, we suggest to add candidate value of component 12 as {1, …, 7} per CC. 23. NR_FeMIMO; 23-1-1; Unified TCI for [intra- and inter-cell] beam management; For both intra- and inter-cell beam management: 1. Support of joint DL/UL TCI update [and separate DL/UL TCI update] with their components (configuration mechanism, QCL rules, applicable source and target signals) [and Common cross-CC TCI update and activation (involving RRC common TCI state pool)] 2. Support of association between TCI state and UL PC settings for PUCCH, PUSCH, and SRS (PLRS and other, including handling of beam alignment/misalignment for PLRS) 3. Supported mode of [MAC-CE/MAC-CE+DCI-based TCI state indication (including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment) 6. The maximum number of configured TCI states per CC in a band 7. Support number of MAC-CE activated joint TCI states across per BWP in a CC 8. The maximum number of MAC-CE activated separate DL and UL TCI states across per BWP in a CC 9. Support of association between a TCI state and PL-RS, where the “beam alignment” between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is assumed by the UE 11. The minimum beam application time in Y symbols 12. The maximum number of PCI(s) different from serving cell PCI that can be associated with activated TCI states per CC; per band; 5. candidate values {64, 72, 80, 96, 128, 192, 256, 128*4*32} 6. candidate values {8, 16, 32, 64, 128, 256} 3. candidate values {“MAC CE”, “MAC CE+DCI”} 12. candidate values {1, … , 7} Note 1: for component 7 & 8, if two TCI states are in one codepoint, it should be counted as 2.] For component 6, UE supports FG23-1-1 shall report at least “64” in FR2 and “the maximum number of allowed SSBs” in FR1.; Optional with capability signalling. |
| Spreadtrum Communications [8] | Agreement in RAN1#106b-e The following working assumption is confirmed with revision in RED. On Rel.17 unified TCI framework, for any DL RS that does not share the same indicated Rel-17 TCI state(s) as UE-dedicated reception on PDSCH and for UE-dedicated reception on all or subset of CORESETs in a CC, but can be configured as a target DL RS of a Rel-17 DL TCI (hence the Rel-17 DL TCI state pool), Rel-17 mechanism(s) which reuse the Rel-15/16 TCI state update signalling/configuration design(s) are used to update/configure such DL RS(s) with Rel-17 TCI state(s). - Applies for both intra-cell and inter-cell beam indication Working Assumption in RAN1#107-e The UE is not expected to be configured with Rel-15/Rel-16 TCI/SpatialRelationInfo if the UE is configured with Rel-17 TCI in any CC in a band - The CC list for Rel-16 multi-CC beam indication should not contain any CC configured with Rel-17 TCI Regarding component 8, one related agreement has been achieved in RAN11#106b-e shown above. In our understanding, for any DL channel/RS does not share the common beam, in general, there are two ways to configure and indicate the TCI state for these Dl channels/RSs : one way is shown as the agreement where R17 mechanism reusing R15/16 signalling to indicate R17 TCI, another way is to reuse R15/16 signalling to indicate R15/R16 TCI. However, in RAN1#107e meeting, one working assumption is achieved where it states that Rel-15/Rel-16 TCI/SpatialRelationInfo and Rel-17 TCI would not be configured simultaneously in any CC in a band. Thus, in light of the working assumption, the only way to configure/indicate TCI state for any DL RS does not share the common beam is to reuse R15/16 signalling to indicate a R17 TCI state. For component 8, we guess the intention is to express whether to support DL channel/RS not sharing the common beam, not how to configure/indicate TCI state for any DL channel/RS does not share the common beam. Thus, based on above, to avoid confusion, we have the following proposal: Proposal 1: For the feature group “joint TCI for intra-cell operation”, suggest to revise component 8 as ‘Support DL channel/RS not sharing the common indicated Rel-17 TCI state’. Proposal 2: For the feature group “joint TCI for intra-cell operation”, suggest to remove component 9. |
| Qualcomm Incorporated [9] | Proposal 2-1: For the joint TCI for intra-cell operation, suggest to adopt the following changes. - For component 1 - suggest to remove bracket of “QCL rules” and “MAC-CE -based TCI”, which are must-have function - suggest to add “MAC-CE+DCI-based TCI state indication using DCI formats 1_1/1_2 with DL assignment for UE supporting more than 1 active joint TCI”, which is must-have function - For component 3, prefer to remove the bracket around “PUCCH, PUSCH, and SRS”, which is the definition in WI agreement - For component 5, suggest to remove the bracket but strike out the “with DL assignment”, which is moved to component 1 and should be must-have function for UE supporting more than 1 active joint TCI - For component 6, we need to keep this, which is NOT must-have function. More importantly, the NO support of component 6 may not be indicated by max # of configured TCI pools in component 10. For example, UE reporting max # of configured TCI pool # of 5 does not mean UE can support configuring common TCI pool on a reference BWP/CC. - For component 7, support remove the bracket. No need to configure BFR on every CC if all CCs sharing the same analog beam. - For component 8, support remove the bracket and suggest to replace “that cannot” with “to not”, since “that cannot” may refer to channels/RSs that can NEVER share the same unified TCI, while the intention here is for channels/RSs that is eligible to share but NW configured them not to share - For component 9, it has been agreed in WI and should be deleted - For component 11, suggest to remove the bracket - For component 12, suggest to remove the bracket and add “per BWP” - For component 13, suggest to remove all brackets - For component 14, suggest to delete, since agreed target DL RS should be sufficient - For component 15, it can be replaced by component 16, which is the exact agreed definition - For component 16, suggest to add “per SCS” at the end, since the time gap is SCS dependent - The following are the components of the feature “joint TCI for intra-cell operation” - Joint DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals, MAC-CE -based TCI activation, MAC-CE+DCI-based TCI state indication using DCI formats 1_1/1_2 with DL assignment for UE supporting more than 1 active joint TCI) - Common multi-CC TCI update and activation - Association between TCI state and UL PC settings except for PL RS [for PUCCH, PUSCH, and SRS] - Beam misalignment between the DL source RS in the TCI state to provide spatial relation indication and the PL-RS - [MAC-CE+DCI-based TCI state indication (including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)] - [TCI states pool for configured reference BWP/CC shared by a set of BWP/CC] - [Maximum number of CCs configured with BFR] - [R17 mechanism reusing R15/16 signalling to indicate R17 TCI for individual DL channel/RS that cannot to not share the same unified TCI as UE-dedicated PDCCH/PDSCH] - [Configuration of both R17 TCI and R15/16 TCI and spatial relation] - The maximum number of configured joint TCI state pools across all BWPs and all CCs in a band - [The maximum number of configured joint TCI states across all BWPs and all CCs in a band] - [The maximum number of configured joint TCI states per BWP per CC in a band] - [Support Number of MAC-CE activated joint TCI states [per BWP per CC/across all BWPs and all CCs in a band]] - [Support any Whether a particular DL RS that is a valid target DL RS of a Rel-15/16 TCI state based on the Rel-15/16 QCL rules can be configured as a target DL RS of Rel-17 DL TCI (hence the Rel-17 DL TCI state pool)] - [The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS] - [The minimum time gap between the beam indication PDCCH and first slot where beam is applied per SCS] Proposal 2-2: For the joint TCI for inter-cell operation, suggest to adopt the following changes. - Suggest to replace component 1-10 with the revised/merged component 11-18 as in the group of joint TCI for intra-cell operation - Suggest to move all measurement related components 19-30 except for component 24 to FG 23-1-2, because the measurement related capability is common for both inter-cell BM and inter-cell mTRP, but inter-cell mTRP cannot use the joint or separate DL/UL TCI as in the title of this group - Suggest to replace component 31-36 with the revised component 37-41 as in the group of joint TCI for intra-cell operation - The following are the components of the feature “joint TCI for inter-cell operation” - [Support of joint DL/UL TCI update with their components (configuration mechanism[, QCL rules], applicable source and target signals including non-serving SSB as direct/indirect QCL source)] - [Common cross-CC TCI update and activation per SCS (involving RRC common TCI state pool)] - [Association between joint TCI state and UL PC settings except for PL RS [for PUCCH, PUSCH, and SRS]] - [Beam misalignment between the DL source RS in the joint TCI state to provide spatial relation indication and the PL-RS] - [MAC-CE+DCI-based TCI state indication [(including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)]] - [MAC-CE-based TCI state indication] - [TCI states pool for configured reference BWP/CC shared by a set of BWP/CC] - [Maximum number of CCs configured with BFR] - [R15/16 signalling to indicate R17 TCI for individual DL channel/RS that can share the same unified TCI as UE-dedicated PDCCH/PDSCH] - [Configuration of both R17 TCI and R15/16 TCI and spatial relation] - Joint DL/UL TCI update with their components: (configuration mechanism, [QCL rules,] applicable source and target signals, including non-serving SSB as direct/indirect QCL source, MAC-CE -based TCI activation, MAC-CE+DCI-based TCI state indication using DCI formats 1_1/1_2 with DL assignment for UE supporting more than 1 active joint TCI) - Common multi-CC TCI update and activation - Association between TCI state and UL PC settings except for PL RS [for PUCCH, PUSCH, and SRS] - Beam misalignment between the DL source RS in the TCI state to provide spatial relation indication and the PL-RS - [MAC-CE+DCI-based TCI state indication (including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)] - [TCI states pool for configured reference BWP/CC shared by a set of BWP/CC] - [Maximum number of CCs configured with BFR] - [R17 mechanism reusing R15/16 signalling to indicate R17 TCI for individual DL channel/RS that cannot to not share the same unified TCI as UE-dedicated PDCCH/PDSCH] - [The maximum number of PCI(s) different from serving cell PCI that can be configured for measurement] - [The maximum number of PCI(s) different from serving cell PCI for which the active RS (refer to def. in 38.214) UE is conducting measurement on in FR1] - [The maximum number of PCI(s) different from serving cell PCI for which the active RS (refer to def. in 38.214) UE is conducting measurement on in FR2] - [ [L1-RSRP/beam] measurement and report on SSB(s) with PCI(s) different from serving cell PCI] - [ [L1-RSRP/beam] reporting of Kmax SSBRI-RSRP[,SS-RSRP] [pairs/beams], with at least one [pair/beam] associated with a PCI different from serving cell PCI [in one report instance]] - The maximum number of PCI(s) different from serving cell PCI that can be associated with activated TCI states - [The number of [RRC-configured/ MAC-CE activated] PCI(s) different from serving cell PCI for beam measurement in FR1] - [The number of [RRC- configured/ MAC-CE activated] PCI(s) different from serving cell PCI for beam measurement in FR2] - [The max number of SSB resources configured to measure L1-RSRP within a slot across all PCI(s) different from serving cell PCI] - [The max number of SSB resources configured to measure L1-RSRP across all PCI(s) different from serving cell PCI] - Support that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement] - [Maximum number of overlapped SSBs in one SSB resource for L1-RSRP measurement] - [The maximum number of configured joint TCI state pools across all BWPs and all CCs in a band] - [The maximum number of configured joint TCI states across all BWPs and all CCs in a band] - [The maximum number of configured joint TCI states per CC [in a band]] - [Support number of MAC-CE activated joint TCI states across all BWPs and all CCs in a band] - [Support any DL RS that is a valid target DL RS of a Rel-15/16 TCI state based on the Rel-15/16 QCL rules can be configured as a target DL RS of Rel-17 DL TCI (hence the Rel-17 DL TCI state pool)] - [The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS] - The maximum number of configured joint TCI state pools across all BWPs and all CCs in a band - [The maximum number of configured joint TCI states across all BWPs and all CCs in a band] - [The maximum number of configured joint TCI states per BWP per CC in a band] - [Support Number of MAC-CE activated joint TCI states [per BWP per CC/across all BWPs and all CCs in a band]] - [The minimum time gap between the beam indication PDCCH and first slot where beam is applied per SCS] Proposal 2-3: For the separate DL/UL TCI, suggest to further split this group into two groups, for intra-cell and inter-cell operations, respectively - The following are the components of the feature “separate DL/UL TCI operation for intra-cell operation” - Support of separate DL/UL TCI update (configuration mechanism, applicable source and target signals) - … - The following are the components of the feature “separate DL/UL TCI operation for inter-cell operation” - Support of separate DL/UL TCI update (configuration mechanism, applicable source and target signals, including non-serving SSB as direct/indirect QCL source) - … Proposal 2-4: In joint TCI for inter-cell operation, all measurement related components 19-30 except for component 24 should be moved to FG 23-1-2, because they are common for both inter-cell BM and inter-cell mTRP, but inter-cell mTRP cannot use the joint or separate DL/UL TCI - [The maximum number of PCI(s) different from serving cell PCI that can be configured for measurement] - [The maximum number of PCI(s) different from serving cell PCI for which the active RS (refer to def. in 38.214) UE is conducting measurement on in FR1] - [The maximum number of PCI(s) different from serving cell PCI for which the active RS (refer to def. in 38.214) UE is conducting measurement on in FR2] - [ [L1-RSRP/beam] measurement and report on SSB(s) with PCI(s) different from serving cell PCI] - [ [L1-RSRP/beam] reporting of Kmax SSBRI-RSRP[,SS-RSRP] [pairs/beams], with at least one [pair/beam] associated with a PCI different from serving cell PCI [in one report instance]] - The maximum number of PCI(s) different from serving cell PCI that can be associated with activated TCI states - [The number of [RRC-configured/ MAC-CE activated] PCI(s) different from serving cell PCI for beam measurement in FR1] - [The number of [RRC- configured/ MAC-CE activated] PCI(s) different from serving cell PCI for beam measurement in FR2] - [The max number of SSB resources configured to measure L1-RSRP within a slot across all PCI(s) different from serving cell PCI] - [The max number of SSB resources configured to measure L1-RSRP across all PCI(s) different from serving cell PCI] - Support that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement] - [Maximum number of overlapped SSBs in one SSB resource for L1-RSRP measurement] |
| OPPO [10] | On each of the components included in previous agreement, we have the following discussion: - For component 1: the QCL rule shall be removed. Rel-17 does not change the QCL rule definition. So there is no need to include the QCL rule in UE feature. It is preferred to keep the MAC CE-based TCI indication there. - For component 5: it is preferred to include component 5 in FG. The UE can report the supporting of MAC CE-based TCI state indication or MAC CE+ DCI-based TCI state indication. - For component 6: it shall be kept. Sharing TCI state pool configured in a reference BWP/CC shall be optional feature for multi-CC TCI state indication. - For component 7: we propose to remove it because that is not part of the functions designed in rel17 unified TCI framework. - For component 8: we propose to keep it. Reusing the existing beam indication mechanism to indicate a rel17 TCI state to PDCCH or CSI-RS resource is new function that is introduced in rel17. Accordingly, new UE feature component shall be introduced for that. - For component 9: It needs more discussion. At least, it shall be discussed together with component 8 because both of them are about the TCI state on DL channel/CSI-RS that do not share the rel17 unified TCI state. - For components 11, 12 and 13: we prefer to keep them. - For component 14: it shall be discussed together with component 8 because both of them are about the TCI state for DL channel or CSI-RS that do not share the rel17 unified TCI state. - For component 16: it shall be kept due to the RAN1 agreement: Agreement On Rel-17 DCI-based beam indication, regarding application time of the beam indication, the first slot that is at least X ms or Y symbols after the last symbol of the acknowledgment of the joint or separate DL/UL beam indication. - Note: The gap between the last symbol of the beam indication DCI and that first slot shall satisfy the UE capability. If the common understanding on this is that rel.15 UE capability of timeDurationForQCL is applied here, then we can change the wording of component 16 to the following to clarify this common understanding: - The minimum time gap between the beam indication PDCCH and first slot where beam is applied, here the rel15 UE capability timeDurationForQCL is reused. Proposal 1: On UE features for unified TCI framework: - For component 1: remove ‘QCL’ and keep ‘MAC-CE based TCI’ - Keep components 5, 6, 11, 12 and 13. - For components 8,9 and 14: they shall be discussed together. - Remove component 7. - Keep and revise component 16 to: “16.The minimum time gap between the beam indication PDCCH and first slot where beam is applied, here the rel15 UE capability timeDurationForQCL is reused” to clarify that rel15 timeDurationFor QCL is reused here. |
| Intel Corporation [11] | Based on the above agreement we proposed the following modifications. - For the 1st component. “QCL rules” and “MAC CE activation” should be part of the 1st component (considering support of the UE with single active TCI state) and the corresponding brackets should be removed. - For the 3rd component. The brackets in “[for PUCCH, PUSCH, and SRS]” should be removed. It should be also clarified that “no association” is part of the basic component, e.g., the 1st component. - For the 5th component. The brackets in “[MAC-CE+DCI-based TCI state indication (including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)]” should be removed. The corresponding component should be mandatory for the UE capable of multiple active TCI states. - For the 6th component. This component may not be needed and replaced by the component 10 “maximum number of configured joint TCI state pools across all BWPs and all CCs in a band” with the value set to 0 or not reported. - For the 7th component. We don’t see necessity to indicate the number of CCs as part of this capability. The BFR using new TCI framework can be assumed supported by the UE capable of corresponding feature from previous NR releases, i.e. Rel-15/16 SpCell BFR, Rel-16 CBRA based SpCell BFR, and Rel-16 SCell BFR. - For the 8th component. It should be part of the basic component. - For the 9th component. May not be needed. FFS positioning. - For the 14th component. The intention of the component is not clear. It should be further clarified the purpose of this FG and its candidate values. For inter-cell measurement and reporting: - For components 5, 6 and 7, further discussion is required regarding necessity of the corresponding capabilities, taking into account RAN4 inputs / assumptions on the corresponding issues. For MPUE support for UL: - The UE capability report should consist of a set of values for maximum number of SRS ports which can differentiate different panels with different capabilities. Also, as per agreement, the value sets should contain at least one different value in each set. 23-1-1; Unified TCI for [intra- and inter-cell] beam management; [For both intra- and inter-cell beam management:] 1. Support of joint DL/UL TCI update [and separate DL/UL TCI update] with their components (configuration mechanism, QCL rules, applicable source and target signals, MAC CE based activation) [and Common cross-CC TCI update and activation (involving RRC common TCI state pool)] 1a Support of separate DL/UL TCI with their components (configuration mechanism, QCL rules, applicable source and target signals) 2. Support of association between TCI state and UL PC settings [for PUCCH, PUSCH, and SRS] [ (PLRS and other, including handling of beam [alignment/misalignment] for PLRS)] 3. Supported mode of [MAC-CE/MAC-CE+DCI]-based TCI state indication [(including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)] [4. The maximum number of configured TCI state pools across all BWPs and all CCs in a band] [5. The maximum number of configured TCI states across all BWPs and all CCs in a band] [6. The maximum number of configured TCI states per CC in a band] [7. Support The maximum number of MAC-CE activated joint TCI states across all BWPs and all CCs in a band (MAC CE +DCI based beam indication)] [8. The maximum number of MAC-CE activated separate DL and UL TCI states across all BWPs and all CCs in a band (MAC CE+DCI based beam indication)] [9. Support of association between a TCI state and PL-RS, where the “beam alignment” between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is assumed by the UE [10. Support any DL RS that is a valid target DL RS of a Rel-15/16 TCI state based on the Rel-15/16 QCL rules can be configured as a target DL RS of Rel-17 DL TCI (hence the Rel-17 DL TCI state pool) [11. The minimum beam application time in Y symbols] [12. The maximum number of PCI(s) different from serving cell PCI that can be associated with activated TCI states]. |
| Apple [12] | For the remaining issues on joint TCI for intra-cell operation, our view is provided as follows: 1. QCL rule can be removed, since some QCL rules should be invalid if UE only supports common TCI update, i.e. UE does not support component 8 and 17; MAC CE based TCI indication can be removed, since it is covered in component 5. 2. RRC common TCI pool can be removed, since it is covered in component 6. 3. Currently PL-RS in TCI is common for all channels, so PUCCH/PUSCH/SRS can be removed 4. This has already been agreed 5. It has been agreed that TCI update based on DCI format 1_1 with data and without data is optional 6. For UE that does not support this feature, R16 behavior can be reused, and it can be combined with component 10. 7. Since common beam is used for multi-CC beam indication, to support separate BFR per CC would significantly increase UE power consumption. 8. This feature is important for UE to avoid potential beam collision 9. This feature does not exist, as it has been agreed spatial relation and R17 TCI cannot be configured in a band 10. This has been agreed 11. This is helpful to identify the upper bound for UE memory to store number of TCI states 12. This is helpful to identify the upper bound for UE memory to store number of TCI states when it does not support TCI pool sharing 13. This has been agreed 14. We do not understand motivation for this component, some clarification is needed 15. This has been agreed 16. We do not understand motivation for this component, some clarification is needed 17. This is a new feature based on agreement from RAN1 #107 Proposal 1.1-1: Support the following change for the remaining issues for “joint TCI for intra-cell operation” - Joint DL/UL TCI update with their components: (configuration mechanism, applicable source and target signals) - Common multi-CC TCI update and activation - Association between TCI state and UL PC settings except for PL RS - Beam misalignment between the DL source RS in the TCI state to provide spatial relation indication and the PL-RS - TCI indication mode (MAC CE, MAC CE + DCI format 1_1 with data, MAC CE + DCI format 1_1 with and without data) - TCI states pool for configured reference BWP/CC shared by a set of BWP/CC - Maximum number of CCs among the CCs configured for common multi-CC TCI update and activation configured with BFR - R17 mechanism reusing R15/16 signalling to indicate R17 TCI for individual DL channel/RS that cannot share the same unified TCI as UE-dedicated PDCCH/PDSCH - void - The maximum number of configured joint TCI state pools across all BWPs and all CCs in a band - The maximum number of configured joint TCI states across all BWPs and all CCs in a band - The maximum number of configured joint TCI states per BWP - The maximum number of MAC-CE activated joint TCI states across all CCs in a band - [Support any Whether a particular DL RS that is a valid target DL RS of a Rel-15/16 TCI state based on the Rel-15/16 QCL rules can be configured as a target DL RS of Rel-17 DL TCI (hence the Rel-17 DL TCI state pool)] - The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS - [The minimum time gap between the beam indication PDCCH and first slot where beam is applied] - Separate TCI indication for SRS for CB/NCB/BM/AS For separate TCI for intra-cell operation, similar to joint TCI, the following is proposed: Proposal 1.1-2: Support the following feature list for separate TCI for intra-cell operation 1. Separate DL/UL TCI update with their components 2-10. Reuse component 2,3,4,5,6,7,8, 15,17 from feature list for joint TCI for intra-cell operation 11. The maximum number of configured DL TCI state pools across all BWPs and all CCs in a band 12. The maximum number of configured UL TCI state pools across all BWPs and all CCs in a band 13. The maximum number of configured DL TCI states across all BWPs and all CCs in a band 14. The maximum number of configured UL TCI states across all BWPs and all CCs in a band 15. The maximum number of configured DL+UL TCI states across all BWPs and all CCs in a band 16. The maximum number of configured DL TCI states per BWP 17. The maximum number of configured UL TCI states per BWP 18. The maximum number of configured DL+UL TCI states per BWP 19. The maximum number of MAC CE activated DL TCI states across all CCs in a band 20. The maximum number of MAC CE activated UL TCI states across all CCs in a band 21. The maximum number of MAC CE activated DL+UL TCI states across all CCs in a band, and a DL and UL TCI is counted as 1 if they share the same QCL source (reuse the same rule for FG 2-62) For join TCI for inter-cell operation, the following is proposed Proposal 1.1-3: Support the following feature list for joint TCI for inter-cell operation 1-16. Reuse 1-17 from joint TCI for intra-cell operation except for component 9 17. Support of a CORESET associated with at least a Type3 CSS/USS and at least a Type 0/0a/1/2 CSS 18. Support of a PDSCH scheduled by PDCCH in Type3 CSS or USS with scheduling offset smaller than threshold reported in FG 2-2 19. Support of aperiodic CSI-RS with scheduling offset smaller than threshold reported in FG 2-28 For separate TCI for inter-cell operation, the following is proposed. Proposal 1.1-4: Support the following feature list for separate TCI for inter-cell operation 1-21. Reuse 1-21 for separate TCI for intra-cell operation in proposal 2 22-24. Reuse 17-19 for joint TCI for inter-cell operation in proposal 3 |
| FUTUREWEI [13] |  |
| xiaomi [14] | According to the agreements above, there are so many components for this feature. In order to make it clearer and simpler, it is better to group them into sub-groups listed as below: - MAC-CE based TCI state indication - DCI based TCI state indication - Unified TCI for CA - TCI state pool configuration - Beam alignment for PL-RS - Uplink power control for unified TCI - BFR with unified TCI Proposal 1: Group components in FG23-1-1 into more sub-groups. According to the agreement in RAN1-104b e-meeting [5] listed as below, support DCI format 1_1/1_2 for beam indication only means support DCI with DL assignment and additional capability is necessary to support DCI without DL assignment. Thus it is reasonable to separate the capability to support DCI format 1_1/1_2 for beam indication with and without DL assignment. RAN1-104b e-meeting-Agreement For beam indication with Rel-17 unified TCI, support DCI format 1_1/1_2 without DL assignment: - Support UE to report whether or not to support TCI update by DCI format 1_1/1_2. - For a UE supporting TCI update by DCI format 1_1/1_2, it must support TCI update by using DCI 1_1/1_2 with DL assignment, and support of the above feature for TCI update by DCI format 1_1/1_2 without DL assignment is UE optional Proposal 2: Split the capability of support DCI format 1_1/1_2 for beam indication with and without DL assignment into two components. |
| LG Electronics [15] | - Component 2: Remove the bracket for PUCCH/PUSCH/SRS - Component 1 and 3: Support of DCI-based TCI indication with/without DL assignment were agreed as optional. Hence, MAC-CE based TCI indication is a basic component and UE signals whether to support DCI-based TCI indication with and/or without DL assignment additionally, with respective components according to the following agreement. One way can be to define two candidate values, i.e. DCI with DL assignment and DCI with and without DL assignment to capture the agreement correctly. Agreement For beam indication with Rel-17 unified TCI, support DCI format 1_1/1_2 without DL assignment: - Support UE to report whether or not to support TCI update by DCI format 1_1/1_2. - For a UE supporting TCI update by DCI format 1_1/1_2, it must support TCI update by using DCI 1_1/1_2 with DL assignment, and support of the above feature for TCI update by DCI format 1_1/1_2 without DL assignment is UE optional - Component 4/5: If we define component 6, the necessity of component 4/5 is unclear since TCI state pool/state is configured per BWP in a CC currently. If these are meant to report CC group that can be shared by common TCI or common TCI pool, it needs to be rephrased into such direction rather than reporting the total number of TCI states per band. We suggest to delete these for now. - Component 7/8: UE can report whether to support joint TCI or separate TCI per CC basis so these components do not seem to be needed. - Component 9: Support this component to capture the following agreement. Agreement On path-loss measurement for Rel.17 unified TCI framework, a PL-RS (configured for path-loss calculation) is either included in UL TCI state or (if applicable) joint TCI state or associated with UL TCI state or (if applicable) joint TCI state. - Whether a UE supports “beam misalignment or not” (detailed definition FFS) between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is a UE capability - Component 10: Having configurability of DL RSs for unified TCI is for NW flexibility and not related to UE capability. Suggest to delete this component. - Component 11/12: Support these since supporting these UE capability report were part of RAN1 agreements. Proposal 1: Adopt the following table for Rel-17 multi-beam 23-1-1. 23. NR_FeMIMO; 23-1-1; Unified TCI for [intra- and inter-cell] beam management; [For both intra- and inter-cell beam management:] 1. Support of joint DL/UL TCI update [and separate DL/UL TCI update] with their components (configuration mechanism, QCL rules, applicable source and target signals, MAC-CE based TCI state indication) [and Common cross-CC TCI update and activation (involving RRC common TCI state pool)] 2. Support of association between TCI state and UL PC settings for PUCCH, PUSCH, and SRS [ (PLRS and other, including handling of beam [alignment/misalignment] for PLRS)] 3. Supported mode of MAC-CE+DCI-based TCI state indication [(including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)] 6. The maximum number of configured TCI states per CC in a band 9. Support of association between a TCI state and PL-RS, where the “beam alignment” between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is assumed by the UE 11. The minimum beam application time in Y symbols 12. The maximum number of PCI(s) different from serving cell PCI that can be associated with activated TCI states; [per band]; [4. candidate values {1, …, 32} 5. candidate values {64, 72, 80, 96, 128, 192, 256, 128*4*32} 6. candidate values {8, 16, 32, 64, 128, 256} 3. candidate values {DCI with DL assignment, DCI with and without DL assignment} 7. candidate values {1, 2, 3, 4, 5, 6, 7, 8} 8. candidate values {1, 2, 3, 4,5,6,7, 8} Note 1: for component 7 & 8, if two TCI states are in one codepoint, it should be counted as 2.]; Optional with capability signalling. |
| Lenovo/Motorola Mobility [16] | In RAN1#106bis-e [1] and RAN1#107-e [2], the following preliminary set of UE features related to multi-beam operation was provided, with a brief version of the table containing the corresponding feature groups provided below: 23-1-1; Unified TCI for [intra- and inter-cell] beam management; [For both intra- and inter-cell beam management:] 1. Support of joint DL/UL TCI update [and separate DL/UL TCI update] with their components (configuration mechanism, QCL rules, applicable source and target signals) [and Common cross-CC TCI update and activation (involving RRC common TCI state pool)] 2. Support of association between TCI state and UL PC settings [for PUCCH, PUSCH, and SRS] [ (PLRS and other, including handling of beam [alignment/misalignment] for PLRS)] 3. Supported mode of [MAC-CE/MAC-CE+DCI]-based TCI state indication [(including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)] [4. The maximum number of configured TCI state pools across all BWPs and all CCs in a band] [5. The maximum number of configured TCI states across all BWPs and all CCs in a band] [6. The maximum number of configured TCI states per CC in a band] [7. Support number of MAC-CE activated joint TCI states across all BWPs and all CCs in a band] [8. The maximum number of MAC-CE activated separate DL and UL TCI states across all BWPs and all CCs in a band] [9. Support of association between a TCI state and PL-RS, where the “beam alignment” between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is assumed by the UE [10. Support any DL RS that is a valid target DL RS of a Rel-15/16 TCI state based on the Rel-15/16 QCL rules can be configured as a target DL RS of Rel-17 DL TCI (hence the Rel-17 DL TCI state pool) [11. The minimum beam application time in Y symbols] [12. The maximum number of PCI(s) different from serving cell PCI that can be associated with activated TCI states] [13: The maximum number of PCI(s) different from serving cell PCI that can be associated with configured TCI states]; 4. candidate values {1, …, 32} 5. candidate values {64, 72, 80, 96, 128, 192, 256, 128*4*32} 6. candidate values {8, 16, 32, 64, 128, 256} 3. candidate values FFS 7. candidate values {1, 2, 3, 4, 5, 6, 7, 8} 8. candidate values {1, 2, 3, 4,5,6,7, 8}. Separate values for DL TCIs and UL TCIs 12. candidate values {01,2,3,…} Note 1: for component 7 & 8, if two TCI states are in one codepoint, it should be counted as 2.]; Optional with capability signalling. 23-1-2; Inter-cell measurement and reporting (for inter-cell BM [and mTRP]); 1. Support of [L1-RSRP/beam] measurement and report on SSB(s) with PCI(s) different from serving cell PCI 2. Support of [L1-RSRP/beam] reporting of Kmax SSBRI-RSRP[,SS-RSRP] [pairs/beams], with at least one [pair/beam] associated with a PCI different from serving cell PCI [in one report instance] [3. The number of [RRC-configured/ MAC-CE activated] PCI(s) different from serving cell PCI for beam measurement] [in FR1] [4. The number of [RRC- configured/ MAC-CE activated] PCI(s) different from serving cell PCI for beam measurement] [in FR2] [5. The max number of SSB resources configured to measure L1-RSRP within a slot across all PCI(s) different from serving cell PCI] [6. The max number of SSB resources configured to measure L1-RSRP across all PCI(s) different from serving cell PCI] [7. Support on that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement]; [2-24, 2-29]; {Support, Not support} [2. Candidate value of {1,2,3,4}] [3. candidate values {1, 2, 4, 6}] [4. candidate values {1, 2, 4}] [5. candidate values: {1, 2, 4, 8, FFS}] [6. candidate values: {4, 8, 16, 32, 64, FFS}] [7. candidate values {not support}]; Optional with capability signalling. 23-1-3; MPE mitigation; 1. Support of enhanced [PHR] reporting which includes pairs of (P-MPR, SSBRI/CRI) 2. Maximum number of reported P-MPR and SSBRI/CRI pairs 3. Maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation; 2. Candidate value of {1,2,3, 4} 3. Candidate valueFFS; Optional with capability signalling. 23-1-4; MPUE support for UL; [depending on agreement] [1. The number of UE capability value set] [2. The maximal supported number of SRS port for each UE capability value set]; [1. Candidate values: {1.2. FFS}] [2. Candidate values: {1,2,4}] Note: The value of the maximal supported number of SRS port for each UE capability value set should be different.. On the component 12 of FG 23-1-1, the agreement says, “Support a UE feature on how many physical cell IDs (including that of the serving cell) can be associated with the activated TCI states” with a note “The above does not necessarily mean that more than 1 physical cell ID that is not serving cell in RRC”, so it should allow the UE to report a value of 0 for component 12. Besides, like for inter-cell multi-TRP operation, another capability regarding the maximum number of PCI(s) different from serving cell PCI that can be associated with configured TCI states should also be reported. The value of component 12 of FG 23-1-1 should include 0 Add component 13 of FG 23-1-1, as follows: [13: The maximum number of PCI(s) different from serving cell PCI that can be associated with configured TCI states] |
| Ericsson [17] | The work in the multi-beam agenda targets to develop a more efficient TCI framework. Unnecessary flexible configurations are removed, and the signalling mechanisms for the critical parts are streamlined. However, this work has led to that some Rel-15/16 functionality has been replicated, and based on this replicated functionality, enhancements have been added. It may be tempting to define a basic level of functionality that contains only the replicated functionality. This would bring the Rel-17 TCI framework on par with the Rel-15/16 functionality and would lead to a functional feature. However, there is no incentive for an operator to deploy only the replicated functionality, and there is no incentive for a NW vendor to implement only the replicated functionality: There is no incentive for a NW vendor to implement only the parts of the Rel-17 TCI framework that replicated the Rel-15/16 functionality. Based on Observation 1, we conclude that a UE that supports the Rel-17 TCI framework must also support enhancements on top of the replicated functionality: A UE that supports the Rel-17 TCI framework must support enhancements relative the Rel-15/16 TCI framework: it is not sufficient to support only the replicated functionality. To reiterate: no NW will implement only the duplicated functionality. There is no point that a UE only implements and advertises support for the replicated functionality. This would be a useless paper product. A UE that supports the Rel-17 TCI framework would thus have to also support enhancements. Overall, the Rel-17 TCI framework supports two main performance enhancements: - DCI-based TCI state update for all channels - Inter-cell beam management To make the Rel-17 TCI framework attractive for an operator to deploy and attractive for a NW vendor to implement, the basic level of UE support should include both DCI-based TCI state update and inter-cell beam management: A UE that supports the Rel-17 TCI framework should support both DCI-based TCI state update and inter-cell beam management. The support described in Proposal 1 could be on a minimum level: for example, DCI-based TCI state update requires that the UE supports two active TCI states – but not more than that. Inter-cell beam management would require that the UE can measure on SSBs associated with one PCI different from the serving cell PCI – but not more than one. More advanced UEs could then advertise larger values for these capabilities. The initial discussion on the UE features for the Rel-17 TCI framework [4] was quite scattered. In an attempt to structure the discussion, the topic was split in three subtopics: - Joint TCI for intra-cell operation - Joint TCI for inter-cell operation - Separate UL/DL TCI The target of the separation was to define and agree on components. Agreeing on a component does not mean that the UE would signal it separately. This separation led to one agreement for 1, whereas for 2 and 3, proposals were captured in [3]. We will discuss the topics separately. ### Joint TCI for intra-cell operation The agreement on joint TCI for intra-cell operation was captured in [1], and is included in the appendix for ease of reference. Most of the agreed components are clear, and we therefore provide comments for each of the components: - Joint DL/UL TCI update with their components: (configuration mechanism, [QCL rules,] applicable source and target signals, MAC-CE -based TCI] activation assignment) - Remove the highlighting and brackets for “QCL rules” – the statement “applicable source and target signals” is difficult to understand without it. - Remove “MAC-CE-based TCI”. Signaling is described in component 5. - Common cross multi-CC TCI update and activation (involving RRC common TCI state pool) - This is OK. Our understanding is that this is merely the Rel-16 functionality applied to the Rel-17 TCI states - Association between TCI state and UL PC settings except for PL RS [for PUCCH, PUSCH, and SRS] - Remove the highlighting for PUCCH and PUSCH, keep for SRS. The power control for SRS is still discussed. - Beam misalignment between the DL source RS in the TCI state to provide spatial relation indication and the PL-RS - OK. - [MAC-CE+DCI-based TCI state indication (including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)] - Remove highlighting. As previously discussed, this is part of the basic functionality - [TCI states pool for configured reference BWP/CC shared by a set of BWP/CC] - Remove the component. This is covered by component 10: a UE that does not report the support of component 10 will not support the reference CC functionality. As this only applies only to CA, which in itself is an optional feature, this cannot be part of the basic functionality. - [Maximum number of CCs configured with BFR] - Remove the component. This is covered by the UE feature maxNumberSCellBFR-r16, and it is not more complex to perform beam failure detection for the Rel-17 TCI framework. - [R17 mechanism reusing R15/16 signalling to indicate R17 TCI for individual DL channel/RS that cannot share the same unified TCI as UE-dedicated PDCCH/PDSCH] - Remove the component. The Rel-16 behaviour is kept, there is no need for a capability. The statement “configuration mechanism” in component 1 covers also the signaling described here. - [Configuration of both R17 TCI and R15/16 TCI and spatial relation] - Remove the component. There will no such support. - The maximum number of configured joint TCI state pools across all BWPs and all CCs in a band - OK - [The maximum number of configured joint TCI states across all BWPs and all CCs in a band] - OK – remove highlighting and brackets. Assuming that the capability is signalled per band, there is no need to include “per band” in the description. - [The maximum number of configured joint TCI states per CC [in a band]] - OK – remove highlighting and brackets. Assuming that the capability is signalled per band, there is no need to include “per band” in the description. - [Support The maximum number of MAC-CE activated joint TCI states [per BWP per CC/across all BWPs and all CCs in a band]] - ‘per BWP is unnecessary: there can only be activated TCI states for one (the active) BWP. Split in two components: - 13a The maximum number of MAC-CE activated joint TCI states per CC - 13b The maximum number of MAC-CE activated joint TCI states across all CCs - [Support any Whether a particular DL RS that is a valid target DL RS of a Rel-15/16 TCI state based on the Rel-15/16 QCL rules can be configured as a target DL RS of Rel-17 DL TCI (hence the Rel-17 DL TCI state pool)] - Remove. Part of component 1 – configuration mechanism and QCL rules - [The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS] - OK – remove highlighting and brackets - [The minimum time gap between the beam indication PDCCH and first slot where beam is applied] - Remove. The only feature related to timing is component 15, and the UE could use component 15. ### Joint TCI for inter-cell operation No components were agreed for inter-cell operation. Instead, a list of proposed components was included in [3]. In contrast to the components discussed in section 1.1.1, many of these components are unclear and partly overlapping. Directly commenting on the proposed components is not so useful. On high level, the proposed components fall in three categories: - Components that define inter-cell operation: support for SSB as a direct/or indirect QCL source and support of L1-RSRP reporting for SSB associated with a different PCI. - Components that are copies of features for intra-cell operation. - Components that are specific to inter-cell operation, mainly regarding measurements. As previously stated, inter-cell beam management should be part of the basic support. Referring to [3], components 15 and 17 define inter-cell beam management, and here we have the following comments: 13. [Support of joint DL/UL TCI update with their components (configuration mechanism [, QCL rules], applicable source and target signals including non-serving SSB as direct/indirect QCL source)] - Remove the highlighting and the brackets 15. [Support of [L1-RSRP/beam] measurement and report on SSB(s) with PCI(s) different from serving cell PCI] - Remove the highlighting and the brackets Since this is part of the basic functionality, component 13 can be combined with component 1 in section 2.1.1. Components 17 in [3] is related to the beam indication: 17. The maximum number of PCI(s) different from serving cell PCI that can be associated with activated TCI states The idea is to limit the UE complexity. In an attempt to reduce the complexity even further, we propose the following modification: 17. The maximum number of PCI(s) that can be associated with activated TCI states With this modification, the UE can advertise that all activated TCI states must be associated with one PCI. This would mean that MAC CE+DCI must be used to indicate a different cell: DCI alone cannot be used. There are several components in [3] that are related to measurements, but many of them are unclear and partly overlapping. Here we present a systematic approach that was previously used during the Rel-16 discussion. The UE complexity of performing measurements is related to two aspects: memory and processing requirements. To reduce requirements on memory, limitations on the configuration, e.g., the maximum number of CSI-RS configured for measurements across all CCs, is introduced. To reduce the processing requirements, limitations on the number of measurements per slot is introduced. We will now discuss these two aspects separately. There are already features specified in 38.306 that limit the memory requirements of the UE. For example, the UE feature beamManagementSSB-CSI-RS was designed for this purpose. Clearly, that UE feature can also be applied to inter-cell beam measurements. It could be argued that the memory consumption related to the inter-cell beam measurements is higher than that of the corresponding intra-cell beam measurements. However, we have this agreement from RAN1#107-e: Agreement On Rel-17 enhancements for inter-cell beam management and inter-cell mTRP, a CSI-SSB-ResourceSet configured for L1-RSRP measurement/reporting includes at least a set of SSB indices where PCI indices are associated with the set of SSB indices, respectively. The PCI indices refer to PCIs within the set of PCIs configured for inter-cell beam management or inter-cell multi-TRP. - The additionalInfo associated with SSB(s) with PCI(s) different from the serving cell agreed in RAN1 Agenda Item 8.1.2.2 is also applicable to inter-cell BM - Detailed signaling design is up to RAN2 - FFS (to be concluded in RAN1#107-e): Whether the above L1-RSRP measurement/reporting also includes group-based beam report for inter-cell mTRP Since the PCI index is included in the CSI-SSB-ResourceSet, the additional memory consumption is small: assuming Nmax=7, only three bits are used to represent the other PCI and is added to each CSI-SSB-ResourceSet. Typically, an CSI-SSB-ResourceSet contains many (up to 64) SSB indices and the additional three bits can be neglected. We hence observe: The memory consumption of the RRC configuration of intra-cell and inter-cell beam measurement is very similar. Based on this, we propose No UE feature is defined to signal the number of supported inter-cell SSB measurements – the UE features related to SSB measurements defined in Rel-15/16 apply. With Proposal 2, it is also clear that all previous UE features defined for beam measurements, e.g., maxNumberResAcrossCC-AcrossFR-r16 and maxNumberResAcrossCC-OneFR-r16, also applies for inter-cell beam measurements – since intra-cell and inter-cell beam measurement are the same. A similar situation applies for the processing requirements. There are already UE features defined for beam measurements. For example maxNumberSSB-CSI-RS-ResourceOneTx indicates maximum total number of configured one port NZP CSI-RS resources and SS/PBCH blocks that are supported by the UE to measure L1-RSRP as specified in TS 38.215 [13] within a slot and across all serving cells (see NOTE). On FR2, it is mandatory to report >=8; On FR1, it is mandatory with capability signalling to report >=8. Clearly, this feature is applicable also for inter-cell beam measurements. If a separate feature is defined, the relation between the two features must be clarified. However, in contrast to the memory consumption, it is not clear if the processing requirements for inter-cell beam measurements are the same. Still, as a starting point, it seems reasonable to assume that the Rel-15 feature maxNumberSSB-CSI-RS-ResourceOneTx is applicable also to the inter-cell case, and a UE that can perform a certain number of SSB measurements on the serving cell can share that number of measurements between the serving cell(s) and cells with another PCI: The Rel-15/16 features related to processing requirements for SS-RSRP measurements apply also to inter-cell beam measurements. However, there is a complication related to asynchronous measurements if SSBs from different cells are received at the UE with different timing. In this case, it may be challenging for the UE to perform measurements on these SSBs at the same time. Note that this is a very realistic case: in case the propagation delay is different, the receive timing will be different, and SSBs from different cells will always be overlapping. Defining inter-cell beam measurements assuming that SSBs from different cells are transmitted in different slots is unrealistic. This is a known issue for L3 measurements, where RAN4 has defined a framework that limits the number of inter-cell neighbours the UE has to measure over a certain period. It should also be noted that for periodic measurements (without measurement restriction), there is no need for the UE to measure a certain SSB in a certain slot: the UE can choose in which slot it would perform the measurement, as long as the RAN4 requirements are fulfilled. This is an important observation in the present context: There is no requirement that the UE performs a measurement on a periodic reference signal in a specific slot. For inter-cell beam measurements, this means that the UE does not have to perform measurements on SSB with different timings in the same slot: the UE can perform the SSB measurements for different cells in different slots. However, it must still be possible to perform the SSB measurements for all cells over a certain time window, and this is what the RAN4 requirements would describe: The RAN4 requirements will limit the number of cells the UE is required to perform measurements. It would be premature to add any UE feature to control this. Summing up, we propose that No UE feature is specified related to how many SSBs or PCIs can be measured per slot. In [3], the following proposal was brought forward: [21a. Support that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement] The idea with this proposal is to define a baseline mode of operation where only the PCIs that are configured for L3 mobility measurements can be reported in an inter-cell beam report. In this baseline mode of operation, the UE could reuse the measurements it anyway performs for L3 mobility and report only those over PUSCH. However, the default operation in NR (and LTE) is that the UE is not configured with specific PCIs for L3 mobility. Instead, the measurement object contains only an ARFCN, and the UE finds all the relevant PCIs. For this normal configuration, the proposed component is not operational With inter-cell beam management, the UE may communicate with a TRP broadcasting another PCI than that of the serving cell. Needless to say, this requires implementation on the UE side: the UE would have to derive QCL properties from an SSB associated with another PCI. However, most of the procedures are agnostic to the serving cell PCI. This means that the components defined for intra-cell operation are also valid for inter-cell operation. For example, the number of TCI state pools (component 10 in section 2.1.1) is appliable also for inter-cell operation. This is true for most components for inter-cell operation: Most components defined for intra-cell are applicable also for inter-cell operation, and there is no need to define a separate component for those. If there is a clear motivation, a separate inter-cell component can be defined, but so far, no such motivation has been presented. Referring to the proposals in [3], the following components are simply copies of the intra-cell versions and are unnecessary: 14. [Common cross-CC TCI update and activation per SCS (involving RRC common TCI state pool)] [14a. Association between joint TCI state and UL PC settings except for PL RS [for PUCCH, PUSCH, and SRS]] [14b. Beam misalignment between the DL source RS in the joint TCI state to provide spatial relation indication and the PL-RS] [14c. FFS: [MAC-CE/MAC-CE+DCI]-based TCI state indication [(including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)]] 14d. [TCI states pool for configured reference BWP/CC shared by a set of BWP/CC] [14e. Maximum number of CCs configured with BFR] [14f. R15/16 signalling to indicate R17 TCI for individual DL channel/RS that can share the same unified TCI as UE-dedicated PDCCH/PDSCH] 14g. [Configuration of both R17 TCI and R15/16 TCI and spatial relation] 21c. [The maximum number of configured joint TCI state pools across all BWPs and all CCs in a band] 21d. [The maximum number of configured joint TCI states across all BWPs and all CCs in a band] [21e. The maximum number of configured joint TCI states per CC [in a band]] 21f. [Support number of MAC-CE activated joint TCI states across all BWPs and all CCs in a band] 21g. [Support any DL RS that is a valid target DL RS of a Rel-15/16 TCI state based on the Rel-15/16 QCL rules can be configured as a target DL RS of Rel-17 DL TCI (hence the Rel-17 DL TCI state pool)] [21h. The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS] In our understanding, these can be removed. ### Separate UL/DL TCI No components were agreed for separate UL/DL TCI. Instead, a list of proposed components was included in [3]. The components in this category are less mature than those discussed in section 2.1.1 and 2.1.2. The component that defines operation with separate UL/DL TCI is this component: 22. Support of separate DL/UL TCI update (configuration mechanism, applicable source and target signals, including non-serving SSB as direct/indirect QCL source) Note that SRS is a valid source RS for the UL TCI. Clearly, SRS cannot be used as a PL RS, and defining beam alignment between SRS and a DL RS is non-trivial, since in the most realistic case, the SRS is configured without any relation to a DL RS. Based on this argument, we assume that a UE that supports separate UL/DL TCI would have to also support an arbitrary PL RS: A UE that supports separate UL/DL TCI would also support an arbitrary PL RS. A large part of the proposals in [3] are related to the number of UL/DL TCI states, both configured and activated. No consideration is taken to how many joint TCI states the UE support, even though there should be a clear relation: from a configuration point of view, a joint TCI state is identical to a DL TCI state. We should also note that from a functionality point of view, unfortunately the introduction of UL TCI states does not lead any reduction in the number of required DL TCI states. Based on this argument, we propose A UE that supports separate UL/DL TCI states supports the same number of configured DL TCI states as configured joint TCI states. A UE that supports separate UL/DL TCI states reports how many configured UL TCI states it supports in addition to the number of joint TCI states. The same is true for the activated TCI states: the number of activated DL TCI states that are needed is not reduced with the introduction of UL TCI states. Hence, we have the corresponding proposals for the activated TCI states: A UE that supports separate UL/DL TCI states supports the same number of activated DL TCI states as activated joint TCI states. A UE that supports separate UL/DL TCI states reports how many activated UL TCI states it supports in addition to the number of activated joint TCI states. Since it is not clear how the UL TCI states will be configured, it is premature to define a UE feature that limits the number of UL TCI states per BWP or cell. A suitable UE feature can be defined when RAN2 has defined the signalling. In [3], there are several proposals that are related to a pool of UL TCI states, mirroring the UE feature for the joint TCI states. Here we note that there is no agreement on any pool of UL TCI states, nor any agreement on a reference CC for UL TCI states. Hence, these proposals are irrelevant: There is currently no agreement on any UL TCI state pool, and it is premature to define any UE features related to such a pool. Finally, we realize that some proposed components are simply copies of the corresponding components for joint TCI: 25. [Common cross-CC TCI update and activation per SCS] 26. [Association between joint TCI state and UL PC settings except for PL RS [for PUCCH, PUSCH, and SRS]] 27. FFS: [MAC-CE/MAC-CE+DCI]-based TCI state indication [(including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)] 23. [The maximum number of MAC-CE activated separate DL and UL TCI states in a BWP in a CC across all BWPs and all CCs in a band] 29. [R15/16 signalling to indicate R17 TCI for individual DL channel/RS that can share the same unified TCI as UE-dedicated PDCCH/PDSCH] 30. [Configuration of both R17 TCI and R15/16 TCI and spatial relation] 38. [The maximum number of PCI(s) different from serving cell PCI that can be associated with activated TCI states] 43. [Support any DL RS that is a valid target DL RS of a Rel-15/16 TCI state based on the Rel-15/16 QCL rules can be configured as a target DL RS of Rel-17 DL TCI (hence the Rel-17 DL TCI state pool)] 44. [The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS] Unless there is a clear motivation to have these as separate components, we propose to delete them. |
| MediaTek Inc. [18] | Regarding the components of the feature “joint TCI for intra-cell operation” agreed in RAN1-107 meeting as follows, our views inline bellow. Agreement: - The following are the components of the feature “joint TCI for intra-cell operation” - Joint DL/UL TCI update with their components: (configuration mechanism, [QCL rules,] applicable source and target signals, MAC-CE -based TCI] activation assignment) Proposal 1: For Component 1 of the feature “joint TCI for intra-cell operation”, support and suggest to remove the brackets from “QCL rules” as follow: - Joint DL/UL TCI update with their components: (configuration mechanism, [QCL rules,] applicable source and target signals, MAC-CE -based TCI] activation assignment) - Common cross multi-CC TCI update and activation (involving RRC common TCI state pool) Proposal 2: Support Component 2 of the feature “joint TCI for intra-cell operation” - Association between TCI state and UL PC settings except for PL RS [for PUCCH, PUSCH, and SRS] Proposal 3: For Component 3 of the feature “joint TCI for intra-cell operation”, support and suggest to remove the brackets from “PUCCH, PUSCH, and SRS” as follow: - Association between TCI state and UL PC settings except for PL RS [for PUCCH, PUSCH, and SRS] - Beam misalignment between the DL source RS in the TCI state to provide spatial relation indication and the PL-RS Proposal 4: Support Component 4 of the feature “joint TCI for intra-cell operation” - [MAC-CE+DCI-based TCI state indication (including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)] Proposal 5: For Component 5 of the feature “joint TCI for intra-cell operation”, support and suggest to remove “TCI state activation” since it is already captured in component 1. - [MAC-CE+DCI-based TCI state indication (including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)] On component 5, note that according to RAN1 agreement, if UE supports DCI-based TCI update, DCI-based TCI update w/ DLA should be the baseline, and support of DCI-based TCI update w/o DLA is UE optional. - MAC-CE-based + DCI-based w/ DLA - MAC-CE-based + DCI-based w/ and w/o DLA Agreement For beam indication with Rel-17 unified TCI, support DCI format 1_1/1_2 without DL assignment: - … - Support UE to report whether or not to support TCI update by DCI format 1_1/1_2. - For a UE supporting TCI update by DCI format 1_1/1_2, it must support TCI update by using DCI 1_1/1_2 with DL assignment, and support of the above feature for TCI update by DCI format 1_1/1_2 without DL assignment is UE optional. In order to let UE report the supported mechanism(s) for DCI-based TCI update, we suggest to introduce a set of candidate values including: - DCI w/ DLA - DCI w/ and w/o DLA Proposal 6: Introduce a set of candidate values as follows for Component 5 of the feature “joint TCI for intra-cell operation” - MAC-CE-based + DCI-based w/ DLA - MAC-CE-based + DCI-based w/ and w/o DLA - [TCI states pool for configured reference BWP/CC shared by a set of BWP/CC] Proposal 7: Support Component 6 of the feature “joint TCI for intra-cell operation” - [Maximum number of CCs configured with BFR] Proposal 8: Not support Component 7 of the feature “joint TCI for intra-cell operation”, we don't see the need and suggest to remove it. - [R17 mechanism reusing R15/16 signalling to indicate R17 TCI for individual DL channel/RS that cannot share the same unified TCI as UE-dedicated PDCCH/PDSCH] This should be supported as baseline. For DL RS that cannot share the same unified TCI as UE-dedicated PDCCH/PDSCH, e.g., periodic/semi-persistent CSI-RS, how to provide TCI state for it if this component is not supported? Proposal 9: Not support Component 8 of the feature “joint TCI for intra-cell operation”, we don't see the need and suggest to remove it. - [Configuration of both R17 TCI and R15/16 TCI and spatial relation] There is working assumption to preclude this configuration. Proposal 10: Not support Component 9 of the feature “joint TCI for intra-cell operation”, we don't see the need and suggest to remove it. Working Assumption The UE is not expected to be configured with Rel-15/Rel-16 TCI/SpatialRelationInfo if the UE is configured with Rel-17 TCI in any CC in a band - The CC list for Rel-16 multi-CC beam indication should not contain any CC configured with Rel-17 TCI. - The maximum number of configured joint TCI state pools across all BWPs and all CCs in a band Proposal 11: Support Component 10 of the feature “joint TCI for intra-cell operation”. - This should be reported only if UE supports component 6. - [The maximum number of configured joint TCI states across all BWPs and all CCs in a band] Proposal 12: Not support Component 11 of the feature “joint TCI for intra-cell operation”, we don't see the need and suggest to remove it. - [The maximum number of configured joint TCI states per CC [in a band]] Following R15/16 principle, the support number of configured TCI states should be counted per BWP per CC. It is unclear what “in a band” means here, thus we suggest to remove it. Proposal 13: Support Component 12 of the feature “joint TCI for intra-cell operation” with the following change: - [The maximum number of configured joint TCI states per BWP per CC [in a band]] On component 12, according to the following RAN1 agreement, the maximum candidate value should be 128. In addition to 128, 4, 8, 16, 32 and 64 could be another candidate value. Since UE is mandated to support at least the max number of allowed SSB in the band in FR1 and 64 in FR2 in Rel-15/16, Rel-17 TCI framework can follow the same principle. Agreement On Rel.17 unified TCI framework, for Rel-17 unified TCI, the largest number of configured TCI states is given as follows (following Rel-15/16 principles): - When a UE is configured with joint DL/UL TCI: the largest number of configured TCI states for joint DL/UL TCI state update is 128 per BWP per CC - Further discuss and decide in RAN1#106bis-e when a UE is configured with separate DL/UL TC. Proposal 14: Introduce candidate values {4, 8 ,16, 32, 64, 128} for Component 12 of the feature “joint TCI for intra-cell operation” - For FR2, UE is mandated to report 64 - For FR1, UE is mandated to report at least the max number of allowed SSB in the band - [Support The maximum number of MAC-CE activated joint TCI states [per BWP per CC/across all BWPs and all CCs in a band]] Following R15/16 principle, the support number of activated TCI states should be counted per BWP per CC. On the other hand, the number of MAC-CE activated TCI states should include not only the TCI states activated for UE-dedicated PDCCH/PDSCH and PUSCH/PUCCH, but also the TCI state(s) activated for PDCCH/PDSCH that doesn't share the same TCI state for UE-dedicated PDCCH/PDSCH and PUSCH/PUCCH, e.g., for non-UE-dedicated PDCCH/PDSCH. We suggest to clarify this in the component. Proposal 15: Support Component 13 of the feature “joint TCI for intra-cell operation” with the following change: - The maximum number of MAC-CE activated joint TCI states [per BWP per CC/across all BWPs and all CCs in a band]] including TCI state(s) activated for UE-dedicated PDCCH/PDSCH and PUSCH/PUCCH and TCI state(s) activated for PDCCH/PDSCH that doesn't share the same TCI state for UE-dedicated PDCCH/PDSCH and PUSCH/PUCCH On component 13, according to the following RAN1 agreement, the number of codepoints activated via MAC-CE-based TCI state activation cannot be larger than 8, thus the maximum candidate value of component 13 shall not exceed 8. Agreement On Rel.17 unified TCI framework, for Rel-17 unified TCI: - For the number of codepoints in the TCI field for DCI-based beam indication (hence the number of codepoints activated via MAC-CE-based TCI state activation), the largest value is 8 - Further discuss and finalize in RAN1#106bis-e: the largest number of configured TCI states (including joint TCI state(s), DL-only TCI state(s), and/or UL-only TCI state(s)). On the other hand, if UE supports only one active TCI state, it means beam update always need to be based on MAC-CE, and SSB measurement and processing time are always needed for every beam switching according to RAN4 requirement. Since Rel-17 unified TCI framework is intended for beam update latency reduction, we don't prefer one single active TCI state as baseline capability. Otherwise, there is no much difference compared to Rel-15/16 TCI framework. In summary, we propose candidate values {2, 4, 8} for Component 13 of the feature “joint TCI for intra-cell operation”. Proposal 16: Introduce candidate values {2, 4, 8} for Component 13 of the feature “joint TCI for intra-cell operation” - [Support any Whether a particular DL RS that is a valid target DL RS of a Rel-15/16 TCI state based on the Rel-15/16 QCL rules can be configured as a target DL RS of Rel-17 DL TCI (hence the Rel-17 DL TCI state pool)] This should be supported as baseline. Since Rel-15/16 TCI cannot be configured if Rel-17 TCI is configured, any DL RS should be able to be configured as a target DL RS of Rel-17 DL TCI. Proposal 17: Not support Component 14 of the feature “joint TCI for intra-cell operation”, we don't see the need and suggest to remove it. - [The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS] Support but the wording is incorrect. The definition of Y symbols is not the offset between PUCCH of ACK and the first slot. In order to avoid confusion, we prefer to directly capture the component according to the following agreement. Agreement On Rel-17 DCI-based beam indication, regarding application time of the beam indication, the first slot to apply the indicated TCI is at least Y symbols after the last symbol of the acknowledgment of the joint or separate DL/UL beam indication. - Note: The Y symbols are configured by the gNB based on UE capability, which is also reported in units of symbols.. Proposal 18: Support Component 15 of the feature “joint TCI for intra-cell operation” with the following change: - [The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS] - [The minimum time gap between the beam indication PDCCH and first slot where beam is applied] Proposal 19: Not support Component 16 of the feature “joint TCI for intra-cell operation”, we don't see the need and suggest to remove it. - This list gives no guidance as to how to group these components into FGs. They may be mapped to one or more FGs and one or more of these FGs may be agreed as basic. Specifically, different FGs can be grouped together through notes “a UE supporting this FG must also support FG 23-x-y”. This allows to create “basic” FGs while allowing, e.g., different types for different components/candidate values. - The purpose of this list is to find the individual components without having to worry about the signalling structure. In RAN1#107, it was agreed that the new beam update after UE receives the BFRR from NW is applied to all PDSCH /PDCCH receptions and all PUSCH transmissions and all of PUCCH resources if Rel-17 unified TIC is configured. This should be captured as one component. Agreement from RAN1#107 On Rel-17 unified TCI framework, for intra-cell beam management, after X symbols from the UE receives the BFRR from NW, the UE assumes the same QCL parameter as the ones associated with the index q new for all PDSCH /PDCCH receptions in a CC, as well as other signals/channels configured to sharing the same indicated Rel-17 TCI state as PDSCH /PDCCH reception. - The above applies to Rel-15 SpCell BFR , Rel-16 CBRA based SpCell BFR , and Rel-16 SCell BFR - Note: q new is a candidate beam identified by the UE in set q1. q1 is the set of candidate beams Agreement from RAN1#107 On Rel-17 unified TCI framework, after X symbols from the UE receives the BFRR from NW, the UE uses the same UL spatial filter as the one associated with the index q new or the last PRACH transmission for all PUSCH transmissions and all of PUCCH resources in a CC, as well as other signals/channels configured to sharing the same indicated Rel-17 TCI state as PUSCH and all of PUCCH resources. - The above applies to Rel-15/16 SpCell BFR , Rel-16 CBRA based SpCell BFR, and Rel-16 SCell BFR - Note: q new is a candidate beam identified by the UE in set q1. q1 is the set of candidate beams - FFS : UL PC control including q u , q d , and closed loop index. Proposal 20: Introduce a new component “Update new beam for all PDSCH /PDCCH receptions and all PUSCH transmissions and all of PUCCH resources in a CC after receives the BFRR from NW” for the feature “joint TCI for intra-cell operation” When Rel-17 unified TCI is configured, we see all PDSCH/PDCCH reception sharing the same indicated TCI state should be supported as baseline. Thus, the support PDCCH/PDSCH that doesn’t share the same indicated the same indicated TCI state for UE-dedicated PDCCH/PDSCH should be an optional feature, at least for intra-cell case. Proposal 21: Introduce a new component “PDCCH/PDSCH that doesn’t share the same indicated the same indicated TCI state for UE-dedicated PDCCH/PDSCH” for the feature “joint TCI for intra-cell operation” For the feature “Separate TCI for intra-cell operation”, we see most of the components of the feature “joint TCI for intra-cell operation” can be replicated, including: - Common multi-CC TCI update and activation - Association between TCI state and UL PC settings except for PL RS for PUCCH, PUSCH, and SRS - Beam misalignment between the DL source RS in the TCI state to provide spatial relation indication and the PL-RS - MAC-CE+DCI-based TCI state indication (including use of DCI formats 1_1/1_2 with and without DL assignment) - The maximum number of MAC-CE activated TCI states per BWP per CC including TCI state(s) activated for UE-dedicated PDCCH/PDSCH and PUSCH/PUCCH and TCI state(s) activated for PDCCH/PDSCH that doesn't share the same TCI state for UE-dedicated PDCCH/PDSCH and PUSCH/PUCCH - TCI states pool for configured reference BWP/CC shared by a set of BWP/CC - The minimum Y symbols per SCS - Update new beam for all PDSCH /PDCCH receptions and all PUSCH transmissions and all of PUCCH resources in a CC after receives the BFRR from NW - PDCCH/PDSCH that doesn’t share the same indicated the same indicated TCI state for UE-dedicated PDCCH/PDSCH Proposal 22: For the feature “Separate TCI for intra-cell operation”, support the following components common to “joint TCI for intra-cell operation”: - Common multi-CC TCI update and activation - Association between TCI state and UL PC settings except for PL RS for PUCCH, PUSCH, and SRS - Beam misalignment between the DL source RS in the TCI state to provide spatial relation indication and the PL-RS - MAC-CE+DCI-based TCI state indication (including use of DCI formats 1_1/1_2 with and without DL assignment) - The maximum number of MAC-CE activated TCI states per BWP per CC including TCI state(s) activated for UE-dedicated PDCCH/PDSCH and PUSCH/PUCCH and TCI state(s) activated for PDCCH/PDSCH that doesn't share the same TCI state for UE-dedicated PDCCH/PDSCH and PUSCH/PUCCH - TCI states pool for configured reference BWP/CC shared by a set of BWP/CC - The minimum Y symbols per SCS - Update new beam for all PDSCH /PDCCH receptions and all PUSCH transmissions and all of PUCCH resources in a CC after receives the BFRR from NW - PDCCH/PDSCH that doesn’t share the same indicated the same indicated TCI state for UE-dedicated PDCCH/PDSCH In addition to the common components, the following components need to be supported as well for the feature “separate TCI for intra-cell operation”: - Separate DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals, MAC-CE-based TCI activation) - The maximum number of configured [pairs of DL and UL][separate] TCI state pools across all BWPs and all CCs in a band Since whether DL and UL share a same configured TCI state pool or not is still under discussion in RAN2, there could be a pair of DL and UL TCI state pools or one single TCI state pool for separate TCI. Conclusion from RAN1#106bis On Rel.17 unified TCI framework, in case of separate DL/UL TCI, it is up to RAN2 whether UL TCI shares the same TCI state pool as joint DL/UL TCI or UL TCI uses a separate TCI state pool from joint DL/UL TCI - Note: By previous agreements, DL TCI shares the same TCI state pool as joint DL/UL TCI. - The maximum number of configured DL TCI states per BWP per CC with candidate values {4, 8, 16, 32, 64, 128} - The maximum number of configured UL TCI states per BWP per CC with candidate values {4, 8, 16, 32, 64} According to the following RAN1 agreement, UE can report separate capabilities for the maximum number of configured DL TCI states including at least {64, 128}, and the maximum number of configured UL TCI states including at least {32, 64}. In addition, smaller candidate values should be introduced for FR1 use case. Agreement from RAN1#107 On Rel.17 unified TCI framework, for Rel-17 unified TCI, when a UE is configured with separate DL/UL TCI - The number of configured TCI states a UE can support is a UE capability including the following candidate values per BWP per CC: - DL TCI: 64, 128 - UL TCI: 32, 64 - Note: This doesn’t imply that UL TCI shares the same TCI state pool as or uses a different TCI state pool from joint DL/UL TCI.. Proposal 23: For the feature “separate TCI for intra-cell operation”, support the following components: - Separate DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals, MAC-CE-based TCI activation) - The maximum number of configured [pairs of DL and UL][separate] TCI state pools across all BWPs and all CCs in a band - The maximum number of configured DL TCI states per BWP per CC - Candidate values {4, 8, 16, 32, 64, 128} - The maximum number of configured UL TCI states per BWP per CC - Candidate values {4, 8, 16, 32, 64} For the feature “Joint TCI for inter-cell operation”, UE should support the feature “Joint TCI for intra-cell operation” as prerequisite. Proposal 24: For the feature “Joint TCI for inter-cell operation”, UE should support the feature “Joint TCI for intra-cell operation” as prerequisite. In addition to those components of the feature “Joint TCI for intra-cell operation”, we see at least the following components need to be supported for the feature “Joint TCI for inter-cell operation”: - Support of SSB associated with PCI different from serving cell PCI is used as an indirect QCL reference for DL TCI (in case of separate DL/UL TCI) or joint TCI, or an indirect/direct QCL reference for UL TCI (in case of separate DL/UL TCI) - The maximum number of PCI(s) (including that of the serving cell) that can be associated with activated TCI states for UE-dedicated PDCCH/PDSCH and PUSCH/PUCCH - The minimum Y symbols per SCS for inter-cell beam indication Agreement from RAN1#106 On Rel.17 beam indication enhancements for inter-cell beam management, for the supported Rel-17 MAC-CE-based and/or DCI-based beam indication (at least using DCI formats 1_1/1_2 with and without DL assignment including the associated MAC-CE-based TCI state activation): - Support a UE feature on how many physical cell IDs (including that of the serving cell) can be associated with the activated TCI states - FFS: If UE is configured for only one physical cell ID, decide between the following two options: - Opt1: the NW can activate TCI states associated with either the same physical cell ID as that of the serving cell or a different physical cell ID from that of the serving cell - Opt2: the NW can only activate TCI states associated with the same physical cell ID as that of the serving cell Note: The above does not necessarily mean that more than 1 physical cell ID that is not serving cell in RRC. Proposal 25: For the feature “Joint TCI for inter-cell operation”, support the following components: - Support of SSB associated with PCI different from serving cell PCI is used as an indirect QCL reference for DL TCI (in case of separate DL/UL TCI) or joint TCI, or an indirect/direct QCL reference for UL TCI (in case of separate DL/UL TCI) - The maximum number of PCI(s) (including that of the serving cell) that can be associated with activated TCI states for UE-dedicated PDCCH/PDSCH and PUSCH/PUCCH - The minimum Y symbols per SCS for inter-cell beam indication For the feature “Separate TCI for inter-cell operation”, UE should support the feature “Separate TCI for intra-cell operation” as prerequisite Proposal 26: For the feature “Separate TCI for inter-cell operation”, UE should support the feature “Separate TCI for intra-cell operation” as prerequisite. Similar to the feature of “Separate TCI for inter-cell operation” , we see at least the following components need to be supported for the feature “Joint TCI for inter-cell operation”: - Support of SSB associated with PCI different from serving cell PCI is used as an indirect QCL reference for DL TCI (in case of separate DL/UL TCI) or joint TCI, or an indirect/direct QCL reference for UL TCI (in case of separate DL/UL TCI) - The maximum number of PCI(s) (including that of the serving cell) that can be associated with activated TCI states for UE-dedicated PDCCH/PDSCH and PUSCH/PUCCH - The minimum Y symbols per SCS for inter-cell beam indication Proposal 27: For the feature “Separate TCI for inter-cell operation”, support the following components: - Support of SSB associated with PCI different from serving cell PCI is used as an indirect QCL reference for DL TCI (in case of separate DL/UL TCI) or joint TCI, or an indirect/direct QCL reference for UL TCI (in case of separate DL/UL TCI) - The maximum number of PCI(s) (including that of the serving cell) that can be associated with activated TCI states for UE-dedicated PDCCH/PDSCH and PUSCH/PUCCH - The minimum Y symbols per SCS for inter-cell beam indication |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] | Before getting into the details of the components listed above, it is useful to have a short review on what are the main elements of unified TCI framework, so that one can ensure that the FGs are meaningful and correspond to implementable features in the end, both in UE and network sides. Basic unified TCI state operation is described below, and illustrated in Figure 1: - RRC configures set of joint and/or separate TCI states - Different needs, e.g. to address MPE issue, beam resource allocation flexibility at gNB - MAC activation of up to 8 joint or separate TCI states - DCI indicates one of the activated TCI states/codepoints to be the indicated TCI state ( = common TCI state) - Before first indication, the first activated TCI state is the current indicated TCI state - DCI format 1_1/1_2 with and without DL assignment - Indication confirmed by HARQ-ACK by UE - Application time of the beam indication - One beam application time (BAT) for a given SCS Figure 1: Basic operation of unified TCI states Another essential functionality of unified TCI states is the support for common cross-CC TCI update, described briefly below and exemplified in Figure 2: - Two cases - RRC-configured TCI state pool(s) can be configured in the PDSCH configuration (PDSCH-Config) for each BWP/CC as in Rel-15/16 - Source RS in each CC to provide a reference signal for the quasi-colocation for DM-RS of PDSCH, DM-RS of PDCCH and CSI-RS in the CC, and a reference, if applicable, for determining UL TX spatial filter for dynamic-grant and configured-grant based PUSCH and SRS in the CC is associated to source RS of the indicated TCI-State - RRC-configured TCI state pool(s) can be absent in the PDSCH configuration (PDSCH-Config) for each BWP/CC, and replaced with a reference to RRC-configured TCI state pool(s) in a reference BWP/CC - Source RS of the indicated TCI-State provides a reference signal for the quasi-colocation for DM-RS of PDSCH, DM-RS of PDCCH and CSI-RS in the set of configured CCs, and a reference, if applicable, for determining UL TX spatial filter for dynamic-grant and configured-grant based PUSCH and SRS in the set of configured CCs (a). (b). Figure 2: Cross-CC TCI state update: (a) RRC-configured TCI state pool(s) for each BWP/CC and (b) RRC-configured TCI state pool(s) in a reference BWP/CC. Finally, UL power control is another key aspect of unified TCI state operation, briefly described below: - A PL-RS (configured for path-loss calculation) is either included in UL TCI state or (if applicable) joint TCI state or associated with UL TCI state of (if applicable) joint TCI state - For each of PUSCH and PUCCH and SRS, the UL control parameters can be associated with UL TCI state or (if applicable) joint TCI state per BWP In addition, while it is true that joint TCI state update provides optimized signaling and room for exta TCI states, all use cases for joint TCI state updates can be provided by separate TCI state updates directly. Hence, separate TCI states should be considered as a baseline. Based on the discussion above, the following mapping can be considered for the essential unified TCI state FGs for joint DL/UL TCI update: Index; Feature group; Components; Pre-requisites; Notes. 23-1-1; Unified TCI for intra-cell beam management with joint DL/UL TCI update; - Joint DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals, MAC-CE -based TCI activation - Association between TCI state and UL PC settings except for PL RS [for PUCCH, PUSCH, and SRS] - R17 mechanism reusing R15/16 signalling to indicate R17 TCI for individual DL channel/RS that cannot share the same unified TCI as UE-dedicated PDCCH/PDSCH - The maximum number of configured joint TCI states per CC [in a band] - The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS - Beam misalignment between the DL source RS in the TCI state to provide spatial relation indication and the PL-RS - MAC-CE+DCI-based TCI state indication (including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment) - The maximum number of MAC-CE activated joint TCI states [per BWP per CC; 23-1-1c; Separate TCI state update needs to be supported by a UE supporting FG 23-1-1. Note: Components 2, 3, 5, 6, 7 are common with 23-1-1c (for separate TCI states), and they do not need to be repeated here if 23-1-1c is a pre-requisite to 23-1-1. 23-1-1a; Multi-CC unified TCI state operation with joint DL/UL TCI update and separate TCI state for each BWP/CC; - Common cross multi-CC TCI update and activation - The maximum number of configured joint TCI state pools across all BWPs and all CCs in a band; 23-1-1; Note: Must be supported if UE supports 23-1-1 and more than 1 CC/BWP. 23-1-1b; Multi-CC unified TCI state operation with joint DL/UL TCI update and reference BWP/CC; - TCI states pool for configured reference BWP/CC shared by a set of BWP/CC - The maximum number of configured joint TCI state pools for configured reference BWP/CC shared by a set of BWP/CC across all BWPs and all CCs in a band; 23-1-1a. For separate UL/DL TCI state update, the following principles should be followed: - Maximise utilization of FGs defined for joint TCI update case as pre-requisites where applicable - Support at least one DL and one UL TCI state needs to be ensured if UE supports separate TCI state updates Index; Feature group; Components; Pre-requisites; Notes. 23-1-1c; Unified TCI for intra-cell beam management with separate DL/UL TCI update; - 1. Support of separate DL/UL TCI update (configuration mechanism, applicable source and target signals, including non-serving SSB as direct/indirect QCL source) Association between TCI state and UL PC settings except for PL RS [for PUCCH, PUSCH, and SRS] - The maximum number of configured separate TCI states per CC [in a band]. - The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS - Beam misalignment between the DL source RS in the TCI state to provide spatial relation indication and the PL-RS - MAC-CE+DCI-based TCI state indication (including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment) - The maximum number of MAC-CE activated TCI states [per BWP per CC], where at least 1 DL and 1 UL TCI activated states need to be supported [per BWP per CC]; Separate TCI state update needs to be supported by a UE supporting FG 23-1-1.. 23-1-1d; Multi-CC unified TCI state operation with separate DL/UL TCI update and separate TCI state for each BWP/CC; - Common cross multi-CC TCI update and activation - The maximum number of configured separate TCI state pools across all BWPs and all CCs in a band; 23-1-1c; Note: Must be supported if UE supports 23-1-1c and more than 1 CC/BWP. 23-1-1e; Multi-CC unified TCI state operation with separate DL/UL TCI update and reference BWP/CC; - TCI states pool for configured reference BWP/CC shared by a set of BWP/CC - The maximum number of configured separate TCI state pools for configured reference BWP/CC shared by a set of BWP/CC across all BWPs and all CCs in a band; 23-1-1d. Finally, for inter-cell TCI state update, the structures above can be used as baseline one they are stable. In addition the following principles need to be followed for better organization of the FGs and to avoid complexity on network and UEs alike: - Intra-cell operation be pre-requisite to inter-cell, as it is not sensible to imagine UEs that can support Rel-17 TCI state updates only for inter-cell scenarios. - Counting to be done consistently for both intra- and inter-cell cases, e.g. N TCI states type of features One specific aspect of inter-cell TCI state updates are the related measurements. Based on discussions in RAN1#107-e, the following candidate components can be considered further: Potential components based on RAN1#107-e discussions; Comments. - [The maximum number of PCI(s) different from serving cell PCI that can be configured for measurement]; Needed, but there is little value in too fine granularity of values. Potential range {1, 3, 7}.. - [The maximum number of PCI(s) different from serving cell PCI for which the active RS (refer to def. in 38.214) UE is conducting measurement on in FR1]; No clear need for this measurement. - [The maximum number of PCI(s) different from serving cell PCI for which the active RS (refer to def. in 38.214) UE is conducting measurement on in FR2]; No clear need for this measurement. - [ [L1-RSRP/beam] measurement and report on SSB(s) with PCI(s) different from serving cell PCI]; Should be part of a basic FG. Can be merged with 15.. - [ [L1-RSRP/beam] reporting of Kmax SSBRI-RSRP[,SS-RSRP] [pairs/beams], with at least one [pair/beam] associated with a PCI different from serving cell PCI [in one report instance]]; Should be part of a basic FG. Can be merged with 14.. - The maximum number of PCI(s) different from serving cell PCI that can be associated with activated TCI states; Needed. |

| 23. NR_FeMIMO | 23-1-2 | Inter-cell measurement and reporting (for inter-cell BM [and mTRP]) | 1. Support of [L1-RSRP/beam] measurement and report on SSB(s) with PCI(s) different from serving cell PCI 2. Support of [L1-RSRP/beam] reporting of Kmax SSBRI-RSRP[,SS-RSRP] [pairs/beams], with at least one [pair/beam] associated with a PCI different from serving cell PCI [in one report instance] [3. The number of [RRC-configured/ MAC-CE activated] PCI(s) different from serving cell PCI for beam measurement] [in FR1] [4. The number of [RRC- configured/ MAC-CE activated] PCI(s) different from serving cell PCI for beam measurement] [in FR2] [5. The max number of SSB resources configured to measure L1-RSRP within a slot across all PCI(s) different from serving cell PCI] [6. The max number of SSB resources configured to measure L1-RSRP across all PCI(s) different from serving cell PCI] [7. Support on that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement] | [2-24, 2-29] |  |  |  |  |  |  |  | [2. Candidate value of {1,2,3,4}] [3. candidate values {1, 2, 4, 6}] [4. candidate values {1, 2, 4}] [5. candidate values: {2, 4, 8, FFS}] [6. candidate values: {4, 8, 16, 32, 64, FFS}] [7. candidate values {not support}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | Inter-cell measurement and reporting As discussed in RAN1#106bis-e, regarding the number of RRC-configured PCI(s) different from serving cell PCI for beam measurement, it is agreed that two independent X values (X1, X2) are reported as UE capability for two different assumptions on additional SSB time domain position and periodicity with respect to serving cell SSB. In addition, it was agreed that this UE capability has FR1 and FR2 differentiation. Considering UE implementation complexity, the candidate values for X1 and X2 are recommended to be extended to {0, 1, 2, 3, 4, 5, 6, 7}. Proposal 2-3: Revise component 3/4 in FG 23-1-2 on RRC-configured PCI(s) different from serving cell PCI as follows - 3. The maximum number of configured additional PCIs is X1 when time domain positions and periodicity of configured SSBs with additional PCIs are the same as time domain positions and periodicity of the serving cell SSBs, with candidate values {0, 1, 2, 3, 4, 5, 6, 7}; - 4. The maximum number of configured additional PCIs is X2 when time domain positions and periodicity of configured SSBs with additional PCIs are different with time domain positions and periodicity of the serving cell SSBs, with candidate values {0, 1, 2, 3, 4, 5, 6, 7}; Regarding the number of SSBs with PCI(s) different from serving cell PCI for L1 beam measurement, to keep the memory consumption and computing complexity at UE under control, and also to put a limit on total number of SSB resources with PCI(s) same as or different from serving cell PCI, we propose the following: Proposal 2-4: Revise component 5/6 in FG 23-1-2 as follows and remove the corresponding brackets - The max number of SSB resources configured to measure L1-RSRP within a slot with PCI(s) same as or different from serving cell PCI - The max number of SSB resources configured to measure L1-RSRP with PCI(s) same as or different from serving cell PCI In addition, to mitigate UE complexity increase from L1 inter-cell beam measurement on top of L3 mobility measurement, it would be beneficial to allow UE to report not supporting measurement on SSBs with PCIs that are not included in L3 mobility measurement. We propose the following: Proposal 2-5: Support component 7 in FG 23-1-2 and remove the corresponding brackets - Support on that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement 23. NR_FeMIMO; 23-1-2; Inter-cell measurement and reporting (for inter-cell BM [and mTRP]); 1. Support of [L1-RSRP/beam] measurement and report on SSB(s) with PCI(s) different from serving cell PCI 2. Support of [L1-RSRP/beam] reporting of Kmax SSBRI-RSRP[,SS-RSRP] [pairs/beams], with at least one [pair/beam] associated with a PCI different from serving cell PCI [in one report instance] 3. The maximum number of configured additional PCIs is X1 when time domain positions and periodicity of configured SSBs with additional PCIs are the same as time domain positions and periodicity of the serving cell SSBs 4. The maximum number of configured additional PCIs is X2 when time domain positions and periodicity of configured SSBs with additional PCIs are different with time domain positions and periodicity of the serving cell SSBs [3. The number of [RRC-configured/ MAC-CE activated] PCI(s) different from serving cell PCI for beam measurement] [in FR1] [4. The number of [RRC- configured/ MAC-CE activated] PCI(s) different from serving cell PCI for beam measurement] [in FR2] [5. The max number of SSB resources configured to measure L1-RSRP within a slot across all with PCI(s) same as or different from serving cell PCI] [6. The max number of SSB resources configured to measure L1-RSRP across all with PCI(s) same as or different from serving cell PCI] [7. Support on that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement]; [2-24, 2-29]; {Support, Not support} [2. Candidate value of {1,2,3,4}] [3. candidate values {0, 1, 2, 3, 4, 5, 6, 7}] [4. candidate values {0, 1, 2, 3, 4, 5, 6, 7}] [5. candidate values: {2, 4, 8, FFS}] [6. candidate values: {4, 8, 16, 32, 64, FFS}] [7. candidate values {not support}]; Optional with capability signalling. |
| ZTE [3] | Firstly, regarding PCI(s) different from serving cell PCI, we prefer to reuse the legacy UE feature of [2-24] SSB/CSI-RS for beam measurement as a starting point. Then, based on already agreement, the maximum value of supported K is a UE capability, and then the corresponding candidate value should be provided, e.g., {1, 2, 4} as legacy beam reporting. After that, we have the following comments for other potential UE features captured by the moderator. - Regarding #. of PCI in FR1/FR2 (component-3/4), as we mentioned before, we suggest that this feature should be discussed in inter-cell mTRP firstly, and then we can further review whether we need to introduce the same/similar UE capability signaling accordingly. - Regarding #. of SSB associated with different PCI from serving cell (component-5/6), we do not identify the necessary of introducing the corresponding UE feature, considering that it is just the normal L1-measurement and there is no additional complexity for UE implementation. - Regarding the indication whether SSB for L1 measurement can be different from L3 measurement (component-7), we think that it should be supported by default. No UE feature is needed. Proposal 2: For inter-cell measurement and reporting (for inter-cell BM and mTRP), the following modification is proposed in red 23-1-2; Inter-cell measurement and reporting (for inter-cell BM [and mTRP]); 1. Support of [L1-RSRP/beam] measurement and report on SSB(s) with PCI(s) different from serving cell PCI 2. Support of [L1-RSRP/beam] reporting of Kmax SSBRI-RSRP[,SS-RSRP] [pairs/beams], with where at least one [pair/beam] associated with a PCI different from serving cell PCI can be reported [in one report instance] [3. The number of [RRC-configured/ MAC-CE activated] PCI(s) different from serving cell PCI for beam measurement] [in FR1] [4. The number of [RRC- configured/ MAC-CE activated] PCI(s) different from serving cell PCI for beam measurement] [in FR2] [5. The max number of SSB resources configured to measure L1-RSRP within a slot across all PCI(s) different from serving cell PCI] [6. The max number of SSB resources configured to measure L1-RSRP across all PCI(s) different from serving cell PCI] [7. Support on that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement]. |
| vivo [4] | The following RAN4 LS indicates that if the inter-cell measurement is not limited within SMTC, there would be additional UE effort to measure. New UE capability should be defined for such inter-cell measurement outside SMTC. Moreover, for measurement over overlapped SSB, it should also noticed that this involves additional UE efforts to measure multiple SSBs for a single measurement. - If measurement for L1-RSRP is not limited within SMTC - Impact on UE measurement burden is expected. For FR2, scheduling restriction may be expected due to non-collocation cell measurement. - Network may have full flexibility in configuration of the RS periodicity and offset. As for the maximum number of RRC-configured PCIs different from the serving cell for measurement/reporting, it was agreed in RAN1#106bis-e meeting that it is up to UE capability with candidate values of at least 1 and X, and the upper bound for X is same as that in the inter-cell MTRP case. Agreement On Rel-17 enhancements for inter-cell beam management and inter-cell mTRP, NMAX (the maximum number of RRC-configured PCIs different from the serving cell for measurement/reporting) is up to UE capability with candidate values of at least 1 and X. - Note: The upperbound for X as agreed in AI 8.1.2.2 - When NMAX is configured to be X, the UE is RRC-configured for L1-RSRP measurement with up to X PCIs different from the serving cell PCI - Additional restriction may be added by RAN4 FFS: UE measurement behaviour when SSBs associated with different PCIs overlap, including whether this is up to UE capability. Proposal 2-1: Support to update FG 23-1-2 as following: 23. NR_FeMIMO; 23-1-2; Inter-cell measurement and reporting (for inter-cell BM [and mTRP]); 1. Support of [L1-RSRP/beam] measurement and report on SSB(s) with PCI(s) different from serving cell PCI 2. Support of [L1-RSRP/beam] reporting of up to Kmax=4 SSBRI-RSRP[,SS-RSRP] [pairs/beams], with at least one [pair/beam] associated with a PCI different from serving cell PCI [in one report instance] [3. The maximum number of RRC-configured PCI(s) different from serving cell PCI for beam measurement in FR1] [4. The maximum number of RRC- configured PCI(s) different from serving cell PCI for beam measurement in FR2] 5. Supported mode inter-cell measurement: {inside SMTC, both inside and outside SMTC} 6. Supported mode of measurement over overlapped SSBs: {overlapped, both overlapped and non-overlapped}. |
| CATT [5] | For inter-cell beam management and inter-cell mTRP, K is defined as the number of beams associated at least with non-serving cell(s) reported in a single CSI reporting instance. The maximum value of supported K is a UE capability and K is configured by NW based on the UE capability. Therefore, component 2 should be the maximum value of K. In addition, since there is no definition of beam in the spec, we prefer to remove the corresponding description. Proposal-6: For inter-cell measurement and reporting, the UE feaure 23-1-2 is revised as follows: 23-1-2; Inter-cell measurement and reporting (for inter-cell BM [and mTRP]); 1. Support of [L1-RSRP/beam] measurement and report on SSB(s) with PCI(s) different from serving cell PCI 2. Support of [L1-RSRP/beam] reporting of Kmax SSBRI-RSRP[,SS-RSRP] [pairs/beams], with at least one [pair/beam] associated with a PCI different from serving cell PCI can be reported [in one report instance] [3. The number of [RRC-configured/ MAC-CE activated] PCI(s) different from serving cell PCI for beam measurement] [in FR1] [4. The number of [RRC- configured/ MAC-CE activated] PCI(s) different from serving cell PCI for beam measurement] [in FR2] [5. The max number of SSB resources configured to measure L1-RSRP within a slot across all PCI(s) different from serving cell PCI] [6. The max number of SSB resources configured to measure L1-RSRP across all PCI(s) different from serving cell PCI] [7. Support on that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement]; [2-24, 2-29]; Per band; {Support, Not support} [2. Candidate value of {1,2,3,4}] [3. candidate values {1, 2, 4, 6}] [4. candidate values {1, 2, 4}] [5. candidate values: {2, 4, 8, FFS}] [6. candidate values: {4, 8, 16, 32, 64, FFS}] [7. candidate values {not support}]; Optional with capability signalling. |
| Samsung [6] | Regarding FG 23-1-2, this feature should cover both inter-cell beam measurement and reporting. Since the components which are relevant to inter-cell beam management are already included in FG 23-1-1, we would like to rename this feature to support of inter-cell beam measurement and reporting. Proposal 8: Rename FG 23-1-2 to “support of inter-cell beam measurement and reporting”. 1. Support of [L1-RSRP/beam] measurement and report on SSB(s) with PCI(s) different from serving cell PCI 2. Support of [L1-RSRP/beam] reporting of Kmax SSBRI-RSRP[,SS-RSRP] [pairs/beams], with at least one [pair/beam] associated with a PCI different from serving cell PCI [in one report instance]. Regarding Components 1 and 2 for FG 23-1-2 as above, we support beam measurement and reporting on SSBs of cells with PCI different from that of the serving cell. We can include a UE capability on maximum number of cells with PCI(s) different from that of the serving cell the UE can measure. Also, we support beam reporting for up to Kmax beams (SSBRI-RSRP pairs) including beams with cells with PCI(s) different from that of the serving cell. Proposal 9: Support Components 1 and 2 in FG 23-1-2. For both FG 23-1-1 and 23-1-2, which are considered as basic features for Rel-17 TCI framework, we would like to propose the signalling granularity as per band based on two reasons. First reason is that based on working assumption in RAN1#107-e, the UE is not expected to be configured with Rel-15/Rel-16 TCI/SpatialRelationInfo if the UE is configured with Rel-17 TCI in any CC in a band. Second reason is that according to RAN2 guidance, the UE features which have “per-band combination” or “per-band per band combination” should be minimized. Hence, “per band” granularity is quite natural starting point to consider for those basic features. Proposal 11: Support FG 23-1-1 and FG 23-1-2 as basic features and per-band granularity. |
| NTT DOCOMO, INC. [7] | We believe FG23-1-1 and FG23-1-2 should be reported per band. For FG23-1-2, we have the following suggestion: - Component 3-4: We believe the number of PCIs for beam measurement should be common for both FR1 and FR2. - Component 5-7: We don’t think we should introduce new UE capability to report the number of SSBs for inter-cell beam reporting. For FG23-1-3, we have the following suggestion - Component 1: Regarding the text in bracket, we think it should be kept. From our perspective, it is better to clarity that the report is included in PHR report. - Component 2: Considering multiple entry PHR, PHRs and PMPRs of multiple CCs are included in one MAC CE, it is better to clarify the maximum number of reported P-MPR and SSBRI/CRI pairs is per CC or across all CCs. In our view, the maximum number of reported P-MPR and SSBRI/CRI pairs should be per CC. - Component 3: Similar as component 2, it is better to clarify the maximum number of candidate RS(s) is per CC or across all CCs. In our view, the maximum number of candidate RS(s) is related to the number of RS(s) that UE can measure. Thus, the maximum number of candidate RS(s) can be across all CCs. Regarding candidate value, similar as DL beam measurement, {4,8,16,32,64} can be considered. 23. NR_FeMIMO; 23-1-2; Inter-cell measurement and reporting (for inter-cell BM [and mTRP]); 1. Support of L1-RSRP measurement and report on SSB(s) with PCI(s) different from serving cell PCI 2. Support of L1-RSRP reporting of Kmax SSBRI-RSRP beams, with at least one beam associated with a PCI different from serving cell PCI 3. The number of RRC-configured PCI(s) different from serving cell PCI for beam measurement; [2-24, 2-29]; per band; {Support, Not support} [2. Candidate value of {1,2,3,4}] 3. candidate values {1, …, 7}; Optional with capability signalling. |
| Spreadtrum Communications [8] |  |
| Qualcomm Incorporated [9] |  |
| OPPO [10] |  |
| Intel Corporation [11] | 23-1-2; Inter-cell measurement and reporting (for inter-cell BM [and mTRP]); 1. Support of [L1-RSRP/beam] measurement and report on SSB(s) with PCI(s) different from serving cell PCI 2. Support of [L1-RSRP/beam] reporting of Kmax SSBRI-RSRP[,SS-RSRP] [pairs/beams], with at least one [pair/beam] associated with a PCI different from serving cell PCI [in one report instance] [3. The number of [RRC-configured/ MAC-CE activated] PCI(s) different from serving cell PCI for L1-RSRP beam measurement and reporting] [in FR1] [4. The number of [RRC- configured/ MAC-CE activated] PCI(s) different from serving cell PCI for L1-RSRP beam measurement and reporting] [in FR2] [5. The max number of SSB resources configured to measure L1-RSRP within a slot across all PCI(s) different from serving cell PCI] [6. The max number of SSB resources configured to measure L1-RSRP across all PCI(s) different from serving cell PCI] [7. Support on that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement]; [2-24, 2-29]. |
| Apple [12] | Proposal 1.2-1: Support the following change for the remaining issues on FG 23-1-2 for inter-cell beam measurement and report - Support of L1-RSRP measurement and report on SSB(s) with PCI(s) different from serving cell PCI - Support of L1-RSRP reporting of up to Kmax=4 SSBRI-RSRP, with at least one [pair/beam] associated with a PCI different from serving cell PCI in one report instance - Maximum number of Kmax - The maximum number of RRC-configured PCI(s) different from serving cell PCI for periodic beam measurement - The maximum number of MAC-CE activated PCI(s) different from serving cell PCI for semi-persistent beam measurement - The max number of SSB resources configured to measure L1-RSRP within a slot across all PCI(s) across all CCs - The max number of SSB resources configured to measure L1-RSRP across all PCI(s) across all CCs - Support on that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement - Maximum number of overlapped SSBs in one SSB resource for L1-RSRP measurement - The number of RRC-configured PCI(s) different from serving cell PCI for aperiodic beam measurement - The maximum total number of SSB/CSI-RS/CSI-IM resources configured to measure within a slot across all CCs in one frequency range for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, RLM and new beam identification for both intra-cell and inter-cell measurement (Similar to FG 16-1g) - The maximum total number of SSB/CSI-RS/CSI-IM resources configured across all CCs in one frequency range for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, RLM and new beam identification for both intra-cell and inter-cell measurement (Similar to FG 16-1g) 23. NR_FeMIMO; 23-1-2; Inter-cell measurement and reporting (for inter-cell BM [and mTRP]); - Support of L1-RSRP measurement and report on SSB(s) with PCI(s) different from serving cell PCI - Support of L1-RSRP reporting of up to Kmax=4 SSBRI-RSRP, with at least one [pair/beam] associated with a PCI different from serving cell PCI in one report instance - Maximum number of Kmax - The maximum number of RRC-configured PCI(s) different from serving cell PCI for periodic beam measurement - The maximum number of MAC-CE activated PCI(s) different from serving cell PCI for semi-persistent beam measurement - The max number of SSB resources configured to measure L1-RSRP within a slot across all PCI(s) across all CCs - The max number of SSB resources configured to measure L1-RSRP across all PCI(s) across all CCs - Support on that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement - Maximum number of overlapped SSBs in one SSB resource for L1-RSRP measurement - The number of RRC-configured PCI(s) different from serving cell PCI for aperiodic beam measurement - The maximum total number of SSB/CSI-RS/CSI-IM resources configured to measure within a slot across all CCs in one frequency range for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, RLM and new beam identification for both intra-cell and inter-cell measurement (Similar to FG 16-1g) - The maximum total number of SSB/CSI-RS/CSI-IM resources configured across all CCs in one frequency range for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, RLM and new beam identification for both intra-cell and inter-cell measurement (Similar to FG 16-1g); [2-24, 2-29]; {Support, Not support} 2. Candidate value of {1,2,3,4} 3. candidate values {1, 2, 3, 7} 4. candidate values {0, 1, 2, 3, 7} 5. candidate values: {0, 1, 2, 3, 7} 6. candidate values: {8, 16, 32, 64, FFS} 7. candidate values {not support} 8. candidate values {1, 2, 3, 4, 5, 6, 7, 8} 9. candidate values {1, 2, 3, 4} 10. candidate values {0, 1, 2, 3, 7} 11. candidate value set is {2, 4, 8, 12, 16, 32, 64, 128} 12. candidate value set is {2, 4, 8, 12, 16, 32, 40, 48, 64, 72, 80, 96, 128, 256}; Optional with capability signalling. |
| FUTUREWEI [13] |  |
| xiaomi [14] | About the number of MAC-CE activated PCI(s) different from serving cell PCI, it was agreed that only 1 additional PCI can be associated with the active TCI States for inter-cell mTRP as shown in the agreement below [7]. RAN1 106-e meeting-Agreement Rel. 17 inter-cell MTRP, the maximum number of additional RRC -configured PCIs per CC is denoted X and can be reported as a UE capability - For the report value of X, multiple candidate values including 1 is supported. - FFS: Which values to support other than 1. - Values larger than 7 are precluded - RAN1 needs to agree on value(s) of X other than 1 - Down-select one of the following alternatives: - Alt 1: A single value of X is reported as UE capability for any possible SSB time domain position and periodicity - Alt 3: At least Two independent X values (X1, X2) are reported as a UE capability for at least two different assumptions on SSB time domain position and periodicity with respect to serving cell SSB - The serving cell PCI is always associated with active TCI states, only 1 additional PCI can be associated with the active TCI States Therefore, the “MAC CE activated” part in components 3 and 4 in FG23-1-2 should be removed. If the number of MAC-CE activated PCI(s) different from serving cell PCI for inter-cell beam management needs to be decided, it should be discussed separately. Proposal 3: Remove the “MAC CE activated” part in component 3 and 4 in FG23-1-2. For inter-cell beam management, we really need to decide how many physical cell IDs (including that of the serving cell) can be associated with the activated TCI states because of the following agreement [7]. RAN1-106-e-agreement On Rel.17 beam indication enhancements for inter-cell beam management, for the supported Rel-17 MAC-CE-based and/or DCI-based beam indication (at least using DCI formats 1_1/1_2 with and without DL assignment including the associated MAC-CE-based TCI state activation): - Support a UE feature on how many physical cell IDs (including that of the serving cell) can be associated with the activated TCI states - FFS: If UE is configured for only one physical cell ID, decide between the following two options: - Opt1: the NW can activate TCI states associated with either the same physical cell ID as that of the serving cell or a different physical cell ID from that of the serving cell - Opt2: the NW can only activate TCI states associated with the same physical cell ID as that of the serving cell Note: The above does not necessarily mean that more than 1 physical cell ID that is not serving cell in RRC Accordingly, it is better to add a new feature group to deal with it. Proposal 4: Add another feature group about the beam indication for inter-cell beam management to discuss the supported number of MAC-CE activated PCI(s) different from serving cell PCI. Features; Index; Feature group; Components; Note; Mandatory/Optional. 23. NR_FeMIMO; 23-1-2; Inter-cell measurement and reporting (for inter-cell BM [and mTRP]); 1. Support of [L1-RSRP/beam] measurement and report on SSB(s) with PCI(s) different from serving cell PCI 2. Support of [L1-RSRP/beam] reporting of up to Kmax=4 SSBRI-RSRP[,SS-RSRP] [pairs/beams], with at least one [pair/beam] associated with a PCI different from serving cell PCI [in one report instance] [3. The number of [RRC-configured/ MAC-CE activated] PCI(s) different from serving cell PCI for beam measurement] [in FR1] [4. The number of [RRC- configured/ MAC-CE activated] PCI(s) different from serving cell PCI for beam measurement] [in FR2] [5. The max number of SSB resources configured to measure L1-RSRP within a slot across all PCI(s) different from serving cell PCI] [6. The max number of SSB resources configured to measure L1-RSRP across all PCI(s) different from serving cell PCI] [7. Support on that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement]; {Support, Not support} [2. Candidate value of {1,2,3,4}] [3. candidate values {1, 2, 4, 6}] [4. candidate values {1, 2, 4}] [5. candidate values: {2, 4, 8, FFS}] [6. candidate values: {4, 8, 16, 32, 64, FFS}] [7. candidate values {not support}]; Optional with capability signalling. 23. NR_FeMIMO; inter-cell beam indication for inter-cell BM; 1. The number of MAC-CE activated PCI(s) different from serving cell PCI for beam indication.. |
| LG Electronics [15] | - FG description: Suggest to clarify that this feature is of inter-cell ‘beam’ management/reporting. - Component 1: For UE feature description, it may be better to use the term ‘beam’ measurement/report to align with the description of the feature group. - Component 2: Suggest to use the ‘beam’ reporting on Kmax SSBRI-RSRP ‘pairs’, and [,SS-RSRP] is not needed in addition to SSBRI-RSRP. The description ‘in one report instance’ is preferred to be kept for clarity. For candidate values, based on the below conclusion, the brackets on the candidate value should be removed. Conclusion (106-e) On Rel.17 L1-RSRP multi-beam measurement/reporting enhancements for inter-cell beam management and inter-cell mTRP, there is no consensus in supporting additional value(s) of KMAX other than 4 - Component 3/4: Unclear to define two separate components for this. In addition, the number of PCIs should be RRC-configured as agreed below. Agreement (106bis-e) On Rel-17 enhancements for inter-cell beam management and inter-cell mTRP, NMAX (the maximum number of RRC-configured PCIs different from the serving cell for measurement/reporting) is up to UE capability with candidate values of at least 1 and X. - Note: The upper bound for X as agreed in AI 8.1.2.2 - When NMAX is configured to be X, the UE is RRC-configured for L1-RSRP measurement with up to X PCIs different from the serving cell PCI - Additional restriction may be added by RAN4 - FFS: UE measurement behaviour when SSBs associated with different PCIs overlap, including whether this is up to UE capability Proposal 2: Adopt the following table for Rel-17 multi-beam 23-1-2. 23. NR_FeMIMO; 23-1-2; Inter-cell beam measurement and reporting; 1. Support of beam measurement and report on SSB(s) with PCI(s) different from serving cell PCI 2. Support of beam reporting of Kmax SSBRI-RSRP pairs, with at least one pair associated with a PCI different from serving cell PCI in one report instance 3. The number of RRC-configuredPCI(s) different from serving cell PCI for beam measurement [5. The max number of SSB resources configured to measure L1-RSRP within a slot across all PCI(s) different from serving cell PCI] [6. The max number of SSB resources configured to measure L1-RSRP across all PCI(s) different from serving cell PCI] [7. Support on that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement]; [2-24, 2-29]; {Support, Not support} 2. Candidate value of {1,2,3,4} [3. candidate values {1, 2, 4, 6}] [4. candidate values {1, 2, 4}] [5. candidate values: {2, 4, 8, FFS}] [6. candidate values: {4, 8, 16, 32, 64, FFS}] [7. candidate values {not support}]; Optional with capability signalling. |
| Lenovo/Motorola Mobility [16] | For inter-cell beam management and inter-cell mTRP, there is no discussion on the component 7, based on our understanding, all the SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report should be included in the SSBs with PCIs configured for L3 mobility measurement to limit UE implementation complexity. Therefore, we propose to remove component 7 of FG 23-1-2. Remove component 7 of FG 23-1-2 |
| Ericsson [17] |  |
| MediaTek Inc. [18] | First, we see FG 23-1-2 should be used for both inter-cell BM and inter-cell mTRP. However, it may not need to mention this in the FG since this FG should be the prerequisite FG for inter-cell BM and inter-cell MTRP. On the other hand, in order to avoid confusion, we prefer to clarify this feature is “beam” measurement/reporting instead of “cell” measurement/reporting. Proposal 28: Update the title of FG 23-1-2 as follows: - Inter-cell beam measurement and reporting (for inter-cell BM [and mTRP]) Proposal 29: FG 23-1-2 should be the prerequisite FG of the following FGs: - FG for “Joint TCI for inter-cell operation” - FG for “Separate TCI for inter-cell operation” - FG for “IntCell-mTRP” On component #1, we prefer to clarify only L1-RSRP measurement/reporting is supported in this feature. Proposal 30: Update component #1 in FG 23-1-2 as follows: - Support of [L1-RSRP/beam] measurement and reporting on SSB(s) with PCI(s) different from serving cell PCI On component #2, we think this one should be the reporting of the maximum number of SSBRI-SSRSRP pairs that UE can supported in one reporting instance with candidate values {1, 2, 3, 4}. Thus, we suggest the following update. Proposal 31: Update component #2 in FG 23-1-2 as follows: - Support of [L1-RSRP/beam] reporting of up to Kmax=4 The maximum number of SSBRI-SSRSRP [pairs/beams] in one reporting instance, with at least one [pair/beam] associated with a PCI different from serving cell PCI [in one report instance] On components #3~#7, we are fine to further discuss the details. Proposal 32: Support components #3~#7 in FG 23-1-2, further discuss the details. 23. NR_FeMIMO; 23-1-2; Inter-cell beam measurement and reporting (for inter-cell BM [and mTRP]); 1. Support of [L1-RSRP/beam] measurement and reporting on SSB(s) with PCI(s) different from serving cell PCI 2. Support of [L1-RSRP/beam] reporting of up to Kmax=4 The maximum number of SSBRI-SSRSRP [pairs/beams] in one reporting instance, with at least one [pair/beam] associated with a PCI different from serving cell PCI [in one report instance] [3. The number of [RRC-configured/ MAC-CE activated] PCI(s) different from serving cell PCI for beam measurement] [in FR1] [4. The number of [RRC- configured/ MAC-CE activated] PCI(s) different from serving cell PCI for beam measurement] [in FR2] [5. The max number of SSB resources configured to measure L1-RSRP within a slot across all PCI(s) different from serving cell PCI] [6. The max number of SSB resources configured to measure L1-RSRP across all PCI(s) different from serving cell PCI] [7. Support on that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement]; [2-24, 2-29]; {Support, Not support} [2. Candidate value of {1,2,3,4}] [3. candidate values {1, 2, 4, 6}] [4. candidate values {1, 2, 4}] [5. candidate values: {2, 4, 8, FFS}] [6. candidate values: {4, 8, 16, 32, 64, FFS}] [7. candidate values {not support}]; Optional with capability signalling. |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] |  |

| 23. NR_FeMIMO | 23-1-3 | MPE mitigation | 1. Support of enhanced [PHR] reporting which includes pairs of (P-MPR, SSBRI/CRI) 2. Maximum number of reported P-MPR and SSBRI/CRI pairs 3. Maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation |  |  |  |  |  |  |  |  | 2. Candidate value of {1,2,3, 4} 3. Candidate valueFFS | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| ZTE [3] | Based on the following agreement, the N P-MPR values and corresponding SSBRI(s)/CRI(s) can be reported. In our views, M should be fixed to 1, and if sharing the same MPE impacts, the UE can provide same P-MPR values for different SSBRI(s)/CRI(s), respectively, in a report instance. Therefore, a new sub-feature of maximum number of P-MPR and SSBRI/CRI pairs can be introduced, and reusing legacy candidate values for normal beam report can be considered as a starting point, i.e., {1, 2, 4}. Then, based on the following highlighted part, we need to further consider another new UE feature of maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation. As a candidate value, we think that at least the value of ‘64’ should be supported. Agreement On Rel.17 enhancements to facilitate MPE mitigation, confirm the following working assumption (in the midst of the previous agreement) as an agreement with the following refinement (highlighted in red): On Rel.17 enhancements to facilitate MPE mitigation, support the following enhancement on the Rel-16 event-triggered P-MPR-based reporting (included in the PHR report when a threshold is reached, reported via MAC-CE): - In addition to the existing field in the PHR MAC-CE, N≥1 P-MPR values can be reported - The N P-MPR values are reported together with the following: - (Working Assumption) For each P-MPR value, up to M SSBRI(s)/CRI(s), where the SSBRI(s)/CRI(s) is selected by the UE from a candidate SSB/CSI-RS resource pool (FFS: how to perform the selection) - Support M=1 - FFS: The supported value(s) of M - FFS: Additional reporting quantities, e.g. SSBRI/CRI, MPR+DL RSRP, or modified virtual PHR - FFS: additional signaling (e.g. CSI triggering) from the NW. Proposal 3: For MPE mitigation, the following modification is proposed in red 23-1-3; MPE mitigation; 1. Support of enhanced [PHR] reporting which includes pairs of (P-MPR, SSBRI/CRI) 2. Maximum number of reported P-MPR and SSBRI/CRI pairs 3. Maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation Candidate value comprises at least 64.. |
| vivo [4] | Agreement: Adopt the following changes highlighted in chromatic formatting 23. NR_FeMIMO; 23-1-3; MPE mitigation; 1. Support of enhanced [PHR] reporting for MPE mitigation (with which includes pairs of (P-MPR,SSBRI/CRI and [associated] P-MPR in PHR reporting) [2. Support of reporting up to Nmax Maximum number of reported P-MPR and SSBRI/CRI pairs] [3. Maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation]; [2. Candidate value of {1,2,3,4}] [3. Candidate value FFS at least includes {64}]; Optional with capability signalling. The components for 23-1-3 are relatively stable. The candidate value for component 3 should include similar candidate values for L1-RSRP report. Proposal 3-1: Confirm the following components of FG 23-1-3 1. Support of enhanced PHR reporting which includes pairs of P-MPR,SSBRI/CRI 2. Maximum number of reported P-MPR and SSBRI/CRI pairs 3. Maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation Proposal 3-2: The following candidate values should be supported for component 3 of FG 23-1-3: {4,8, 16,32, 64}. |
| CATT [5] |  |
| Samsung [6] |  |
| NTT DOCOMO, INC. [7] | 23. NR_FeMIMO; 23-1-3; MPE mitigation; 1. Support of enhanced PHR reporting which includes pairs of (P-MPR, SSBRI/CRI) 2. Maximum number of reported P-MPR and SSBRI/CRI pairs per CC 3. Maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation across all CCs; 2. Candidate value of {1,2,3, 4} 3. Candidate value {4,8,16,32,64}; Optional with capability signalling. |
| Spreadtrum Communications [8] |  |
| Qualcomm Incorporated [9] |  |
| OPPO [10] |  |
| Intel Corporation [11] | 23-1-3; MPE mitigation; 1. Support of enhanced [PHR] reporting which includes pairs of (P-MPR, SSBRI/CRI) 2. Maximum number of reported P-MPR and SSBRI/CRI pairs 3. Maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation. |
| Apple [12] | 23. NR_FeMIMO; 23-1-3; MPE mitigation; 1. Support of enhanced reporting for MPE mitigation (with SSBRI/CRI and [associated] P-MPR in PHR reporting) 2. Support of reporting up to Nmax P-MPR and SSBRI/CRI pairs 3. Maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation; {Support, Not support} [2. Candidate value of {1,2 3,,4}] [3. Candidate value at least includes {4}]; Optional with capability signalling. |
| FUTUREWEI [13] |  |
| xiaomi [14] |  |
| LG Electronics [15] | - Component 1: Remove the brackets on PHR for clarity since it is based on existing PHR report with pairs of P-MPR and SSBRI/CRI. Proposal 3: Adopt the following table for Rel-17 multi-beam 23-1-3. 23. NR_FeMIMO; 23-1-3; MPE mitigation; 1. Support of enhanced PHR reporting which includes pairs of (P-MPR, SSBRI/CRI) 2. Maximum number of reported P-MPR and SSBRI/CRI pairs 3. Maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation; 2. Candidate value of {1,2,3,4} 3. Candidate value FFS; Optional with capability signalling. |
| Lenovo/Motorola Mobility [16] |  |
| Ericsson [17] | This FG describes enhancements to the P-MPR reporting. The basic capability would include support for a N (P-MPR,SSBRI/CRI) pairs in a new PHR MAC CE. The current version looks OK. We propose the following editorial change: 1. Support of enhanced [PHR] reporting which includes pairs of (P-MPR,SSBRI/CRI 2. Maximum number of reported P-MPR and SSBRI/CRI pairs 3. Maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation The reason to remove “in a RRC pool” is that it is unclear. In addition, there is a need to decide on the candidate values for the number of candidate RSs. This feature resembles beam failure recovery, and the corresponding beam failure recovery feature is maxNumberCSI-RS-SSB-CBD INTEGER (1..256) However, the corresponding BFR feature is defined across all CCs, which motivates the large value range. We note that for the feature to be of any use, if must be possible to include all the SSBs configured in the cell in the candidate list. Since the number of SSBs per cell can be between 1 and 64, one option could be to have the same value range for this capability. However, in current FR2 deployments many SSBs is used, and for the feature to be useful, that number must be matched. For example, if the UE reports that it only supports four candidate RSs, the NW will not enable the feature: a larger number is required. Therefore, we propose The candidate values for the maximum number of candidate RSs is (8,16,32,64) In accordance with the RAN2 guidance to minimize the number of features that are reported per band combination, we propose FG 23-1-3 is defined per band. We note that the corresponding BFR feature is also defined per band. |
| MediaTek Inc. [18] |  |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] | - Component 2 candidate values: It was agreed that UE can report up to 4 pairs of P-MPR and SSBRI/CRI values - Component 3 candidate values: In order to enable the UE to find an new candidate beam, the pool of configured RS should be as large as possible to enable UE to find new candidate beams, i.e. 64 RS. |

| 23. NR_FeMIMO | 23-1-4 | MPUE support for UL | [depending on agreement] |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | MP-UE The following WA on UE capability reporting for MP-UE was established in RAN1#107-e, which should be captured in UE feature list. In addition, regarding the issue on ‘Whether the UE capability value set can be common across all BWPs/CCs in same band or BC’, we suggest assigning a report type of per FSPC. In addition, we suggest considering maximum 4 UE capability sets, each with up to 4-port SRS resource for Rel-17. Working Assumption Support the UE reporting a list of UE capability value sets - Each UE capability value set comprises the max supported number of SRS ports - For any two different value sets, at least one capability value needs to be different - FFS: If in addition also identical value sets are allowed. - Whether the UE capability value set can be common across all BWPs/CCs in same band or BC can be discussed in UE feature session With above analysis, we propose the following: Proposal 2-6: Add the following components as FG 23-1-4 with report type of ‘per FSPC’ - 1. Supported UE capability value sets and corresponding max number of SRS ports for each UE capability value set 23. NR_FeMIMO; 23-1-4; MP-UE support; [depending on agreement] 1. Supported UE capability value sets and corresponding max number of SRS ports for each UE capability value set; FSPC; 1.Candidate values: Up to 4 values each with one value of {1,2,3,4}; Optional with capability signalling. |
| ZTE [3] | In RAN1#107, the following agreement on UE capability set has been agreed, and then we can firstly capture the corresponding feature of number of SRS ports accordingly. Then, by default, we need to have a component of supporting the beam reporting along with UE capability set. Working Assumption Support the UE reporting a list of UE capability value sets - Each UE capability value set comprises the max supported number of SRS ports - For any two different value sets, at least one capability value needs to be different - FFS: If in addition also identical value sets are allowed. - Whether the UE capability value set can be common across all BWPs/CCs in same band or BC can be discussed in UE feature session Agreement On Rel.17 enhancements to facilitate UE -initiated panel activation and selection via UE reporting a list of UE capability value sets, the correspondence between each reported CSI-RS and/or SSB resource index and one of the UE capability value sets in the reported list is determined by the UE (analogous to Rel-15/16) and is informed to NW in a beam reporting instance. - The Rel-15/16 beam reporting framework is used, i.e. the index of corresponding UE capability value set is reported along with the pair of SSBRI/CRI and L1-RSRP/SINR (up to 4 pairs, with 7-bit absolute and 4-bit differential) in the beam reporting UCI and down select (maintenance) between the following two options: - Option 1: UE can report one index for all the reported CRIs/SSBRIs in one beam reporting - Option 2: UE can report one index for each reported CRI/SSBRI in one beam reporting. - FFS: whether/how to take DL-only panel into account in the report - FFS: Time-domain behaviour, e.g. the support periodic, semi-persistent, and aperiodic reporting - FFS: Semi-persistent and/or aperiodic reporting is triggered only when periodic reporting is configured - (Working assumptions): Support acknowledgement mechanism of the reported correspondence from NW to UE, which doesn't preclude reusing/reinterpreting existing signaling/procedure - FFS (maintenance): the application time for the reported correspondence (if any), the exact acknowledgement mechanism and whether spec impact is needed, e.g. based on TCI state update, BFR response like mechanism, including the application time for the reported correspondence, if any - No new DCI format and no new RNTI are introduced for this function.. Proposal 4: For MPUE support for UL, the following modification is proposed in red 23-1-4; MPUE support for UL; [depending on agreement] 1. Support of enhanced L1-RSRP/SINR reporting which includes the index of corresponding UE capability value set; 2. A list of UE capability sets, each of which comprises maximum numbers of supported number of SRS ports Candidate value comprises up to X UE capability sets (X = 3), and the candidate value for maximum number of supported number of SRS ports comprises {1, 2, 4}.. |
| vivo [4] |  |
| CATT [5] | For uplink panel selection of MP UE, it was agreed that UE reporting a list of UE capability value sets, and the UE capability value set refers to the max supported number of SRS ports. Following the same rule as SP UE where the max supported number of SRS ports is reported per BWP, the UE capability value set for MP UE can be reported per BWP. Proposal-7: For MP UE support for UL, UE feature 23-1-4 can be revised as 23-1-4; MP UE support for UL; - Support UE reporting a list of UE capability value sets. - The UE capability value set refers to the max supported number of SRS ports.; Per BWP; Optional with capability signalling. |
| Samsung [6] | Pending progress in AI 8.1.1 in RAN1, we can consider FG 23-1-4 for enhanced UE multi-panel support and discuss further after finalizing the feature. We can start from defining a list of UE capability value sets which each UE capability value set comprises the max supported number of SRS ports. Proposal 12: Consider FG 23-1-4 for enhanced UE multi-panel support which includes a list of UE capability value sets comprising at least the max supported number of SRS ports. |
| NTT DOCOMO, INC. [7] |  |
| Spreadtrum Communications [8] |  |
| Qualcomm Incorporated [9] |  |
| OPPO [10] |  |
| Intel Corporation [11] | 23-1-4; MPUE support for UL; [depending on agreement] Supported combinations of the maximum number of SRS ports with candidate values - {1,2}, {1,4}, {2,4}, {1,2,4}. |
| Apple [12] | The following is proposed based on the agreed working assumption in last meeting. Proposal 1.3-1: Support the following element for FG 23-1-4 (MPUE support for UL) 1. Maximum number of SRS ports for each capability set 2. Capability set index report in a periodic beam report instance 3. Capability set index report in a semi-persistent beam report instance 4. Capability set index report in an aperiodic beam report instance 23. NR_FeMIMO; 23-1-4; MPUE support for UL; 1. Maximum number of SRS ports for each capability set 2. Capability set index report in a periodic beam report instance 3. Capability set index report in a semi-persistent beam report instance 4. Capability set index report in an aperiodic beam report instance; {Support, Not support} 1. candidate value {1_2} 2. {support, not support} 3. {support, not support} 4. {support, not support}; Optional with capability signalling. |
| FUTUREWEI [13] |  |
| xiaomi [14] | According to the agreement in RAN1-107 e-meeting [4] listed as below, as for MPUE support for UL, it is better to describe the component as “Support maximum number of UE capability value sets”, and the {Support, Not support} should be updated to “candidate values {1, 2}”. Proposal 5: Update FG23-1-4 as below 23. NR_FeMIMO; 23-1-4; MPUE support for UL; [depending on agreement] Support maximum number of UE capability value sets; {Support, Not support} Candidate values {1, 2}; Optional. |
| LG Electronics [15] | - In RAN1#107-e, it was agreed to support a UE reporting of a list of UE capability value sets where each value set could correspond to one or more UE panels. Working Assumption (107-e) Support the UE reporting a list of UE capability value sets - Each UE capability value set comprises the max supported number of SRS ports - For any two different value sets, at least one capability value needs to be different - FFS: If in addition also identical value sets are allowed. - Whether the UE capability value set can be common across all BWPs/CCs in same band or BC can be discussed in UE feature session Proposal 4: Adopt the following table for Rel-17 multi-beam 23-1-4. 23. NR_FeMIMO; 23-1-4; MPUE support for UL; 1. Total number of UE capability value sets 2. Maximum supported number of SRS ports per resource per UE capability value set for codebook based UL; 1. candidate values: FFS 2. candidate values: [1,2,4] for each UE capability value set; Optional with capability signalling. |
| Lenovo/Motorola Mobility [16] | The following working assumption was achieved in RAN1#107 Working Assumption Support the UE reporting a list of UE capability value sets - Each UE capability value set comprises the max supported number of SRS ports - For any two different value sets, at least one capability value needs to be different - FFS: If in addition also identical value sets are allowed. - Whether the UE capability value set can be common across all BWPs/CCs in same band or BC can be discussed in UE feature session We propose the following components of FG 23-1-4. Define the components of FG 23-1-4 as follows: - Comment 1: The number of UE capability value set - Comment 2: The maximal supported number of SRS port for each UE capability value set |
| Ericsson [17] |  |
| MediaTek Inc. [18] |  |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] | Confirm the FG. RAN1 already agreed that UE reports a list of UE capability value sets, the correspondence between each reported CSI-RS and/or SSB resource index and one of the UE capability value sets in the reported list is determined by the UE (analogous to Rel-15/16) and is informed to NW in a beam reporting instance. |

| 23. NR_FeMIMO | 23-2-1 | [Multi-TRP] PDCCH repetition | 1. Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) [with non-SFN scheme TDM and FDM] [including PDCCH repetition for Type 3 CSS] 2. Support of reporting one [or more] number(s) as required number of BDs for the two PDCCH candidates 3. [If 3 or {2, 3} is reported in component 2, support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 3 BDs] [If 2 or {2,3} is reported in component 2, the individual candidate is monitored when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 2 BDs.] FFS: 4. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier |  |  |  |  |  |  |  |  | Component1: details FFS Component2: details FFS Component3: details FFS Component4: details FFS [Note: UE supports PDCCH repetition for the following (basic) PDCCH monitoring capability: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | Multi-TRP PDCCH repetition Regarding the multi-TRP PDCCH repetition enhancement, three schemes have been agreed SFN, TDM and FDM in Rel-17. Therefore, the combinations of supported schemes should be a UE capability. For capability on SFN PDCCH, this has been captured as UE capability in HST-SFN feature. Proposal 3-1: Remove the brackets around the component 1 in FG 23-2-1, - Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) with non-SFN scheme TDM and FDM including PDCCH repetition for Type 3 CSS. The details on capability reporting of whether the individual candidate is monitored or not is TBD. In our view, a typical implementation of 2 BDs is that UE decodes the linked candidates individually. And for soft-combining that requires further processing, UE can report a capability of 3 BDs to relax the requirement. And if selective decoding is assumed, the individual candidate can naturally be monitored, while for soft combining, whether the individual candidate can be monitored or not is reported. In that case, gNB may get accurate resource allocation according to the capability reporting for the linked candidates. Proposal 3-3: Remove the bracket in components 3 in 23-2-1, i.e., when an individual candidate uses the same set of CCEs as one of a linked PDCCH candidates, and they both are associated with the same DCI size, scrambling, and CORESET, - if 2 BDs are reported for the linked two PDCCH candidates, the individual candidate is assumed to be monitored. - if 3 BDs are reported for the linked two PDCCH candidates, whether the individual candidate is assumed to be monitored or not should be reported by the UE. 23. NR_FeMIMO; 23-2-1; [Multi-TRP] PDCCH repetition; 1. Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) [with non-SFN scheme TDM and FDM including PDCCH repetition for Type 3 CSS] 2. Support of reporting one [or more] number(s) as required number of BDs for the two PDCCH candidates 3. [If 3 or {2, 3} is reported in component 2, support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 3 BDs.] [If 2 or {2,3} is reported in component 2, the individual candidate is monitored when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 2 BDs.] FFS: 4. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier; Component1: details FFS Component2: details FFS Component3: details FFS Component4: details FFS [Note: UE supports PDCCH repetition for the following (basic) PDCCH monitoring capability: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot]; Optional with capability signalling. |
| ZTE [3] | Three UE features (23-2-1, 23-2-1, and 23-2-3) are listed for MTRP PDCCH enhancements in [2]. We have some comments on these FGs as below. For the third bullet in FG 23-2-1, based on previous RAN1 discussion, when 2 BDs are counted for the two linked PDCCH candidates, UE can do ether individual detection or soft combining. And when 3 BDs are counted for the linked candidates, it is assumed that UE has to do both individual detection and soft combining. That’s why the total number of BDs is 3 which is larger than 2. Therefore, it is clear that the component 3 is only needed when 2 BDs are counted. In addition, there is a conclusion made in RAN1#106bis-e meeting, i.e. there is no consensus to introduce RRC configuration for the number of BDs. Hence, reporting more than one BD numbers is precluded for component 2. Otherwise, gNB cannot configure which one is used if UE reports two BD numbers. For component 1, we think it should be used for both USS and CSS type 3 as there is no much difference for PDCCH decoding with repetition for USS and CSS from UE complexity perspective. Proposal 5: For FG 23-2-1, - Component 1 is for both USS and CSS type 3. - For component 2, support of reporting one number as required number of BDs for the two PDCCH candidates - Component 3 is only for the case of BD = 2. 23-2-1; Multi-TRP PDCCH repetition; 1. Support of PDCCH repetition based on two linked SS sets associated with corresponding CORESETs 2. Support of reporting one number as required number of BDs for the two PDCCH candidates 3. If 2 is reported in component 2, report whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling 4. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier. |
| vivo [4] | - Regarding component 1, based on RAN1 discussion, non-SFN based PDCCH repetitions can be associated with same CORESET (i.e. in case of Single-TRP), it is better to revise CORESETs with CORESET(s). Furthermore, [TDM and FDM] and [Type3 CSS] is unnecessary which can be deleted. - Regarding component 2, since reporting more than one number of BD value was not agreed, so the bracket should be removed. - Regarding component 3, as per some comments from few companies in previous meetings, UE can report 3BDs by performing only one individual decoding and soft bits combining decoding. Therefore, the individual monitoring capability is also applied in case of 3 BDs. Proposal 4-1: 23. NR_FeMIMO; 23-2-1; [Multi-TRP] PDCCH repetition; 1. Support of non-SFN based PDCCH repetition (based on two linked SS sets associated with corresponding CORESET(s)) [with non-SFN scheme TDM and FDM] [including PDCCH repetition for Type 3 CSS] 2. Support of reporting one [or more] number(s) as required number of BDs for the two PDCCH candidates 3. [If 3 or {2, 3} is reported in component 2, support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 3 BDs] [If 2 or {2,3} is reported in component 2, the individual candidate is monitored when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 2 BDs.] FFS: 4. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier. |
| CATT [5] | For Component 1, according to the following agreement and conclusion, both SFN scheme and non-SFN scheme TDM and FDM are supported for PDCCH repetition. Since SFN based PDCCH repetition is captured as UE capability in HST-SFN feature (23-6-1), only non-SFN scheme is considered in FG 23-2-1. Therefore, non-SFN scheme shall be basic feature and the first two square brackets in Component 1 shall be removed. Agreement For PDCCH reliability enhancements, support SFN scheme + Alt 1-1. - FFS: TCI state activation for CORESET, impact on default beam, BFD resource for BFR Conclusion The agreed PDCCH repetition framework (Option 2 + Case 1 + Alt3) supports both TDM and FDM multiplexing schemes.. It was agreed that PDCCH repettion can be performed for both USS and Type 3 CSS. In our view, compared with PDCCH repetition for USS, PDCCH repetition for Type 3 CSS will not require additional complexity. Therefore, there is no need to introduce additional capability for Type 3 CSS PDCCH repetition. One option is to remove the related brackets for Type 3 CSS PDCCH repetition. In another option, both brackets and description related to Type 3 CSS PDCCH repetition can be deleted. Both options are acceptable to us. Based on the above analysis, we have the following proposal: Proposal-8: For multi-TRP PDCCH repetition, UE feature 23-2-1 is revised as follows: 23-2-1; [Multi-TRP] PDCCH repetition; 1. Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) [with non-SFN scheme TDM and FDM] [including PDCCH repetition for Type 3 CSS] 2. Support of reporting one [or more] number(s) as required number of BDs for the two PDCCH candidates 3. [If 3 or {2, 3} is reported in component 2, support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 3 BDs] [If 2 or {2,3} is reported in component 2, the individual candidate is monitored when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 2 BDs.] FFS: 4. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier; Component1: details FFS Component2: details FFS Component3: details FFS Component4: details FFS [Note: UE supports PDCCH repetition for the following (basic) PDCCH monitoring capability: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot]; Optional with capability signalling. |
| Samsung [6] | PDCCH enhancement Based on the discussion in RAN1#106b-e, there are 4 components including FFS point in FG 23-2-1 which is a basic feature for multi-TRP PDCCH repetition. Regarding FG 23-2-1 for supporting PDCCH repetition, the name of the FG would be PDCCH repetition (i.e., removing “Multi-TRP”) since our view is that single-TRP based PDCCH repetition can be supported based on the definition of PDCCH repetition (two linked SS sets associated with corresponding CORESET(s) and the corresponding CORESET(s) can be both same and different CORESETs). Proposal 13: Modify the name of FG 23-2-1 as PDCCH repetition (i.e., removing “Multi-TRP”). 1. Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) [with non-SFN scheme TDM and FDM] [including PDCCH repetition for Type 3 CSS] Component1: details FFS. Regarding the description of Component 1 for FG 23-2-1 as above, we are fine the wording only “with TDM and FDM” since it seems “non-SFN scheme” and “TDM and FDM” are duplication. Also, we are fine “including PDCCH repetition for Type 3 CSS” and prefer to remove FG 23-2-3 which defines support of PDCCH repetition for Type 3 CSS. Proposal 14: Support “with TDM and FDM” and “including PDCCH repetition for Type 3 CSS” in the description of Component 1 in FG 23-2-1 and remove FG 23-2-3. 2. Support of reporting one [or more] number(s) as required number of BDs for the two PDCCH candidates Component2: details FFS. Regarding the description and the candidate values of Component 2 for FG 23-2-1 as above, we support to delete “[or more]” since there is no consensus to introduce RRC configuration for the number of BDs, which means that gNB should follow UE capability reporting, i.e., one of 2 or 3. Hence, reporting more than 1 value for the number of BDs is not needed. As a consequence, the candidate values of Component 2 can be only 2 or 3. Proposal 15: Support to delete “[or more]” in the description and support the candidate values 2 or 3 for Component 2 in FG 23-2-1. 3. [If 3 or {2, 3} is reported in component 2, support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 3 BDs] [If 2 or {2,3} is reported in component 2, the individual candidate is monitored when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 2 BDs.] Component3: details FFS. Regarding the description of Component 3 for FG 23-2-1 as above, we support to delete “or {2, 3}” due to the reason mentioned in Proposal 14. Also, during the discussion in AI 8.1.2.1 PDCCH enhancement, if a UE reports the number of BDs as 3, an individual PDCCH candidate can be monitored in case of overlapping (when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET) since 3 BDs may include at least two separate decoding of each linked PDCCH candidate. but for the case of 2 BDs, whether an individual PDCCH candidate can be monitored or not depends on UE capability since there can be a case when 2 BDs may not include two separate decoding of each linked PDCCH candidate. However, the current description for Component 3 seems opposite way, i.e., whether individual PDCCH candidate is monitored can be determined by UE capability in case of 3 BDs but it is monitored in case of 2 BDs. Proposal 16: Support to delete “or {2, 3}” in the description for Component 3 in FG 23-2-1. Regarding the granularity of UE capability reporting, the reporting level can be per FSPC due to the complex functionality and buffer burden of multi-TRP PDCCH repetition for UE. Proposal 17: Support FG 23-2-1 per FSPC. |
| NTT DOCOMO, INC. [7] | - Component 1: regarding the text in bracket, we think it should be kept. In our view, if a UE supports PDCCH repetition, it supports both TDM and FDM repetition and also supports repetition for Type3 CSS. There is no need to have separate capability for FDM or TDM or Type3 CSS, and it is better to have the text in bracket to make it clear. - Component 2: since it was concluded that there is no consensus to support RRC configuration of number of BDs, to align the understand of number of BDs between gNB and UE, only one number of BDs should be reported by UE. Thus, the text in bracket, i.e., [or more], should be removed. - Component 3: similar as component 2, only one number of BDs should be reported by UE, the text “or {2,3}” should be removed. - Component 4: since it was agreed that UE capability for such overlap is introduced, we support to have component 4. Meanwhile, we think a note should be added to clarify how the number of overlaps is counted when N PDCCH candidates are overlapped, i.e., counted as one overlap or counted as N overlaps. In our understanding, if N PDCCH candidates are overlapped, the number is counted as one. Proposal 2: Adopt the following for Rel-17 M-TRP PDCCH 23. NR_FeMIMO; 23-2-1; [Multi-TRP] PDCCH repetition; 1. Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) with non-SFN scheme TDM and FDM including PDCCH repetition for Type 3 CSS 2. Support of reporting one number(s) as required number of BDs for the two PDCCH candidates 3. If 3 is reported in component 2, support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 3 BDs If 2 is reported in component 2, the individual candidate is monitored when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 2 BDs. 4. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier; Component2: {2,3} Component4: details FFS [Note: UE supports PDCCH repetition for the following (basic) PDCCH monitoring capability: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot] Note: for component4, if N PDCCH candidates are overlapped, the number of overlaps is counted as one.. |
| Spreadtrum Communications [8] |  |
| Qualcomm Incorporated [9] | In Rel-15, PDCCH monitoring capability is defined for different cases including Case 1-1 for basic capability (FG 3-1), Case 1-2 for single occasion monitoring (FG 3-2), and Case 2 for PDCCH monitoring with span gap (FG 3-5b). Furthermore, Rel-16 introduced additional capabilities (FG 11-2 family). The support of Rel-17 mTRP PDCCH repetition should be separately indicated for each of these PDCCH monitoring capabilities. In other words, the basic FG for Rel-17 mTRP PDCCH repetition should only include the basic PDCCH monitoring capability. This is because PDCCH repetition can significantly increase the UE complexity and some of the advanced PDCCH monitoring features that the UE is able to support in the absence of PDCCH repetition need a separate capability when combined with PDCCH repetition. Furthermore, for the proposed FGs 23-2-1b and 23-2-1c, UE should be able to indicate the supported mode of PDCCH repetition wrt intra-span versus inter-span. In fact, such capability is already agreed: Agreement When 3 BDs are counted for two linked candidates - The third BD is counted in the later span for inter-span PDCCH repetition when r16monitoringcapablityis configured. - Note: Inter-span repetition is UE optional Additionally, in RAN1 #107-e, the following was agreed to handle the UE complexity / memory requirement, which needs to be captured entirely by UE capability spec, and is specific to inter-span PDCCH repetition Agreement To handle UE complexity / memory requirements for linked PDCCH candidates, address the issue by UE capability, where UE indicates a limit (X) associated with the total number of linked candidates of which the first candidate is received and the second one has not been received at any given span. - The limit X is indicated as a total count assuming count 1 for AL=1; 2 for AL=2; 4 for AL=4 or 8 or 16. - The limit X is indicated per CC and also across all CCs - Note: “received” and “not been received” is wrt the end of the corresponding span of PDCCH candidate. - Above is applicable at least for the inter-span case (FFS: intra-span case) Based on the discussions above, and the agreements during the WI mentioned bove, we propose the following: Proposal 3-1: - The basic FG for Rel-17 mTRP PDCCH repetition (FG 23-2-1) should only include the basic PDCCH monitoring capability, i.e., the highlighted note in square bracket is required (square bracket should be removed). - PDCCH repetition for more advanced PDCCH monitoring capabilities need a separate capability for each of the following - Add FG 23-2-1a for “PDCCH repetition with PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot” - Add FG 23-2-1b for “PDCCH repetition for Case 2 PDCCH monitoring with a span gap” - Add a component for “Supported mode of PDCCH repetition” with candidate values {intra-span, inter-span, both} - Add a component for “X per CC” and “X across all CCs”, where X is defined in the agreement above applicable to inter-span PDCCH repetition - Add FG 23-2-1c for “PDCCH repetition for Rel-16 PDCCH monitoring” - Add a component for “Supported mode of PDCCH repetition” with candidate values {intra-span, inter-span, both} - Add a component for “X per CC” and “X across all CCs”, where X is defined in the agreement above applicable to inter-span PDCCH repetition Regarding components 3 and 4 of FG 23-2-1, we think there should not be any dependency on component 2. This is because for whether the UE performs soft-combining or not and the relationship to number of BDs was extensively discussed in RAN1 and no consensus was achived. Hence, component 3 does not need to depend on what UE indicates for component 2. Furthermore, component 4 is already agreed as shown below, and it needs to be included that this max number is per scheduled CC per slot. However, this number (component 4) should be per scheduled CC “per span” for Rel-16 span-based PDCCH monitoring since BDs and CCEs are also defined per span in this case. Hence, a similar component 4 is also needed for FG 23-2-1c. Agreement When one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET - Interpretation of the detected DCI is based on Rel. 17 PDCCH repetition rules (wrt reference PDCCH candidate). - Whether the individual candidate is monitored or not is determined by a UE capability - FFS (In UE feature session): The details including reusing the reported number of BDs for this purpose, or relation to reported number of BDs - In both cases, the individual candidate is not counted toward the BD limit. - UE capability for max number of such overlaps is introduced - FFS: Value of 0 is included as a candidate value for the UE capability - The details to be discussed as part of UE capability discussions - FFS: When the individual candidate is monitored, the scenario where the other linked candidate is also “overlapping” (same CORESET, DCI size, CCEs, scrambling) with a second individual candidate Hence, we propose the following: Proposal 3-2: - Coponent 3 of FG 23-2-1 should be “Support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET” with candidate values of {monitored, not monitored}. - Component 4 of FG 23-2-1 should be “Supported max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier per slot” - A similar component (component 5 in the table below) is needed for FG 23-2-1c as “Supported max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier per span” Based on the above discussions, we propose the following FGs for mTRP PDCCH: Proposal 3-3: Adopt the following for Rel-17 mTRP PDCCH UE features (modifications in red). 23. NR_FeMIMO; 23-2-1; PDCCH repetition; 1. Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) 2. Support of reporting one number as required number of BDs for the two PDCCH candidates 3. Support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET 4. Supported max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier per slot; Per FS; Note: UE supports PDCCH repetition for the following (basic) PDCCH monitoring capability: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot. Component2: {2, 3} Component3: {monitored, not monitored} Component4: {2,4,6,10,20,40}; Optional. 23. NR_FeMIMO; 23-2-1a; PDCCH repetition with PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot; Support of PDCCH repetition for PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot; FG 3-2 FG 23-2-1; Per Band; FR1 only; Optional. 23. NR_FeMIMO; 23-2-1b; PDCCH repetition for Case 2 PDCCH monitoring with a span gap; 1. Support of PDCCH repetition for PDCCH monitoring of any occasions with span gap as defined in FG 3-5b. 2. Supported mode of PDCCH repetition 3. X per CC 4. X across all CCs; FG 3-5b FG 23-2-1; Per FS; This capability is necessary for each SCS. Component2: {intra-span, inter-span, both} Component3: FFS candidate values Component 4: FFS candidate values Note: - Components 3 and 4 are reported only if UE supports inter-span PDCCH repetition. - The limit (X) is associated with the total number of linked candidates of which the first candidate is received and the second one has not been received at any given span, where “received” and “not been received” is wrt the end of the corresponding span of PDCCH candidate. - The limit X is indicated as a total count assuming count 1 for AL=1; 2 for AL=2; 4 for AL=4 or 8 or 16.; Optional. 23. NR_FeMIMO; 23-2-1c; PDCCH repetition for Rel-16 PDCCH monitoring; 1. Support of PDCCH repetition with Rel-16 PDCCH monitoring capability as defined in FG 11-2 family. 2. Supported mode of PDCCH repetition 3. X per CC 4. X across all CCs 5. Supported max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier per span; FG 11-2 FG 23-2-1; Per FS; This capability is signalled for SCS 15 kHz and 30 kHz. Component2: {intra-span, inter-span, both} Component3: FFS candidate values Component 4: FFS candidate values Note: - Components 3 and 4 are reported only if UE supports inter-span PDCCH repetition. - The limit X is associated with the total number of linked candidates of which the first candidate is received and the second one has not been received at any given span, where “received” and “not been received” is wrt the end of the corresponding span of PDCCH candidate. - The limit X is indicated as a total count assuming count 1 for AL=1; 2 for AL=2; 4 for AL=4 or 8 or 16. Component5: {1,2,3,5,10,20}; Optional. |
| OPPO [10] |  |
| Intel Corporation [11] | - for FG 23-2-1 component-1, mention intra-slot which forms a basic FG, inter-span could be optional - for FG 23-2-1 component-1, FDM multiplexing in FR2 would need reception of 2 QCL Type-D beams - for FG 23-2-1 component-1, support of Type 3 CSS could be part of basic FG - for FG 23-2-1 components-2, 3, support UE reporting either 2 or 3 BDs but not both - for FG 23-2-1 component-4, remove as there is no such RAN1 agreement for intra-slot case - for FG 23-2-1a add component indicating total number of linked candidates of which the first candidate is received and the second one has not been received at any given span as per RAN1#107-e agreement 23-2-1; [Multi-TRP] PDCCH repetition; 1. Support of intra-slot PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) [with non-SFN scheme TDM and FDM (except FR2)] [including PDCCH repetition for Type 3 CSS] 2. Support of reporting one [or more] number(s) as required The required number of BDs for the two PDCCH candidates 3. [If 3 or {2, 3} is reported in component 2, support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 3 BDs] [If 2 or {2,3} is reported in component 2, the individual candidate is monitored when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 2 BDs.] FFS: 4. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier. 23-2-1a; [Multi-TRP] PDCCH inter-span repetition; 1. Indicates support of PDCCH inter-span repetition 2. The total number of linked candidates of which the first candidate is received and the second one has not been received at any given span (X1 per CC, X2 across CCs); 23-2-1. |
| Apple [12] | With regard to additional processing delay for soft combining, we suggest we add an element to report additional delay required for mTRP PDCCH decoding as the fifth element. Meanwhile, for the second element, currently only 2 BD or 3 BDs are supported, where 2+3 is precluded since no RRC configuration for BD is supported. Proposal 1.4-1: Support the following change for FG 23-2-1 (mTRP PDCCH) - Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) with non-SFN scheme TDM and FDM [including PDCCH repetition for Type 3 CSS] - Support of reporting one number as required number of BDs for the two PDCCH candidates - support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET - Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduling scheduled component carrier - Additional delay (number of symbols) for mTRP PDCCH processing 23. NR_FeMIMO; 23-2-1; [Multi-TRP] PDCCH repetition; 1. Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) [with non-SFN scheme TDM and FDM] [including PDCCH repetition for Type 3 CSS] 2. Support of reporting one number as required number of BDs for the two PDCCH candidates 3. support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET 4. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduling scheduled component carrier 5. Additional delay (number of symbols) for mTRP PDCCH processing; Component1: {Support, Not support} Component2: {2, 3} Component3: {monitored, not monitored} Component4: {0, 1, 2, 3} Component5: {7, 14}; Optional with capability signalling. |
| FUTUREWEI [13] |  |
| xiaomi [14] | As for component 2 ‘Support of reporting one [or more] number(s) as required number of BDs for the two PDCCH candidates’ in FG23-2-1, according to the agreement in RAN1-104b [5] and RAN1-106bis e-meeting [6] listed as below, since there is no RRC configuration for the number of BDs, it is unnecessary to report more numbers as required number of BDs for the two PDCCH candidates. RAN1-104b e-meeting-Agreement For number of BDs corresponding to two PDCCH candidates that are linked for PDCCH repetition, support - UE reports one [or more] number(s) as required number of BDs for the two PDCCH candidates - Candidate values: 2, 3. - FFS: Default behaviour - FFS: Whether one of the candidate values imply that UE supports soft combining - FFS: Whether additional candidate values are supported (e.g. non-integer numbers) - FFS: RRC configuration based on reported UE capability RAN1-106b e-meeting-Conclusion There is no consensus to introduce RRC configuration for the number of BDs. Proposal 6: Update component 2 in FG23-2-1 by removing the text in bracket. - Support of reporting one [or more] number(s) as required number of BDs for the two PDCCH candidates As for component 3 ‘3. [If 3 or {2, 3} is reported in component 2, support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 3 BDs] [If 2 or {2,3} is reported in component 2, the individual candidate is monitored when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 2 BDs.]’ and component 4 ‘FFS: 4. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier’ in FG23-2-1, according to the agreement in RAN1-106 e-meeting [7] listed as below, it is better to report the candidate values of supported maximum number of individual candidate that can be monitored. RAN1-106 e-meeting-Agreement When one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET - Interpretation of the detected DCI is based on Rel. 17 PDCCH repetition rules (wrt reference PDCCH candidate). - Whether the individual candidate is monitored or not is determined by a UE capability - FFS (In UE feature session): The details including reusing the reported number of BDs for this purpose, or relation to reported number of BDs - In both cases, the individual candidate is not counted toward the BD limit. - UE capability for max number of such overlaps is introduced - FFS: Value of 0 is included as a candidate value for the UE capability - The details to be discussed as part of UE capability discussions - FFS: When the individual candidate is monitored, the scenario where the other linked candidate is also “overlapping” (same CORESET, DCI size, CCEs, scrambling) with a second individual candidate Proposal 7: update component 3&4 in FG23-2-1 as below. - [If 3 or {2, 3} is reported in component 2, support of whether maximum number of the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 3 BDs] [If 2 or {2,3} is reported in component 2, support maximum number of the individual candidate is monitored when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 2 BDs.] - FFS: 4. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier 23. NR_FeMIMO; 23-2-1; [Multi-TRP] PDCCH repetition; 1. Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) [with non-SFN scheme TDM and FDM] [including PDCCH repetition for Type 3 CSS] 2. Support of reporting one [or more] number(s) as required number of BDs for the two PDCCH candidates 3. [If 3 or {2, 3} is reported in component 2, support maximum number of of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 3 BDs] [If 2 or {2,3} is reported in component 2, support maximum number of the individual candidate is monitored when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 2 BDs.] FFS: 4. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier; Component1: details FFS {Support, Not support} Component2: {2, 3} Component3: {0, ……} Component4: details FFS [Note: UE supports PDCCH repetition for the following (basic) PDCCH monitoring capability: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot]; Optional with capability signalling. |
| LG Electronics [15] | - Component 2: it is beneficial to report multiple BD numbers and configure one of them by RRC considering decoding performance and PDCCH overbooking. - Component 3: if 2 BD is configured by RRC, then individual candidate should not be monitored considering possibility of combining based BD implementation. On the other hand, if 3 BD is configured, individual candidate is monitored since UE conducts separate BD for each of linked candidate in case of 3 BD. Proposal 5: Adopt the following table for Rel-17 M-TRP PDCCH. 23. NR_FeMIMO; 23-2-1; [Multi-TRP] PDCCH repetition; 1. Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) [with non-SFN scheme TDM and FDM] [including PDCCH repetition for Type 3 CSS] 2. Support of reporting one or more number(s) as required number of BDs for the two PDCCH candidates 3. Support to monitor the individual candidate when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET, if 3 is reported in component 2 and configured by RRC FFS: 4. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier; Component1: details FFS Component2: details FFS Component3: details FFS Component4: details FFS [Note: UE supports PDCCH repetition for the following (basic) PDCCH monitoring capability: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot]; Optional with capability signalling. |
| Lenovo/Motorola Mobility [16] | For item 1 of Feature group 23-2-1, the text “with non SFN scheme TDM and FDM” is proposed to provide detail information for repetition schemes. And, the text “including PDCCH repetition for Type 3 CSS” is proposed to clearly show supporting PDCCH with repetition for Type 3 CSS. We agree this is aligned with available agreement. If the key point for “with non SFN scheme TDM and FDM” is used to make differentiation between SFN and non-SFN scheme, it may be implicitly derived by text “(based on two linked SS sets associated with corresponding CORESETs)” in the proposal. On PDCCH repetition for Type 3 CSS, there is already listed feature group 23-2-3. Therefore, we prefer not include text “with non SFN scheme TDM and FDM” and “including PDCCH repetition for Type 3 CSS” on account of simplicity. Remove the text “[with non SFN scheme TDM and FDM]” and “[including PDCCH repetition for Type 3 CSS]” in item 1 of FG 23-2-1 for simplicity For item 2 of feature group 23-2-1, we think UE can support blind detection with number of BD as 2 if UE reports number of BDs as 3 for the two PDCCH candidates. Furthermore, we have conclusion in RAN1 #106b e-meeting that there is no consensus to introduce RRC configuration for the number of BDs. So we suggest to remove text “[or more]” in item 2 of FG 23-2-1. Remove text “[or more]” in item 2 of FG 23-2-1 For item 3 and 4, it is related with two FFS issues for UE monitoring behaviour when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET. From UE capability view, we think if UE reports 3 BDs, the individual candidate can be supported for monitoring and thus UE indication may be not needed; if UE reports 2 BDs, UE need indicates whether the individual candidate is monitored or not. Moreover, value 0 can be considered as candidate value for maximum number of overlapping candidate if UE has limited capability and this does not bring big issue to PDCCH candidate allocation for gNB. Keep the text in the bracket “[If 2 is reported in component 2, support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 2 BDs]” for item 3 of FG 23-2-1 |
| Ericsson [17] |  |
| MediaTek Inc. [18] | We propose to remove the square brackets in the description of component 1 of FG 23-2-1, i.e., [with non-SFN scheme TDM and FDM] [including PDCCH repetition for Type 3 CSS]. Firstly, the new FG 23-2-3 already captures Type3 CSS repetition supports separately, secondly, PDCCH repetition and SFN are not be confused, hence, it needs to be removed from the description. Proposal 33: Remove the square brackets from description of FG 23-2-1 component 1, to: - Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) Regarding FG 23-2-1 component 3 we propose to revert to the original wording (prior to the changes made as outcome of RAN-106-e). The current wording for this component is rather confusing and creates unnecessary controversy. Consequently, candidate values for this FG will also revert to {monitored, not monitored}. Proposal 34: Revert the description of FG 23-2-1 component 3 to its original wording, i.e., - Support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET With candidate values {monitored, not monitored}. Due to dependency of component 4 in FG 23-2-1 on support of component 3, i.e., component is only applicable for UEs which support component 3, we propose to have a separate FG capturing this feature. This new structure will allow for reduced signalling overhead. Proposal 35: Make a separate FG for component 4 of FG 23-2-1, i.e., FG 23-2-1a. The candidate values can be {4, 8, 16, 32}. 23. NR_FeMIMO; 23-2-1; [Multi-TRP] PDCCH repetition; 1. Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) [with non-SFN scheme TDM and FDM] [including PDCCH repetition for Type 3 CSS] 2. Support of reporting one [or more] number(s) as required number of BDs for the two PDCCH candidates 3. [If 3 or {2, 3} is reported in component 2, support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 3 BDs] [If 2 or {2,3} is reported in component 2, the individual candidate is monitored when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 2 BDs.] FFS: 4. Maximum number of such overlaps in 3 Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduling scheduled component carrier; Component1: details FFS {Support, Not support} Component2: details FFS {2, 3, [{2,3}]} Component3: details FFS {monitored, not monitored} Component4: details FFS [Note: UE supports PDCCH repetition for the following (basic) PDCCH monitoring capability: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot]; Optional with capability signalling. 23. NR_FeMIMO; 23-2-1a; PDCCH repetition with overlapped CCEs; Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduling scheduled component carrier; 23-2-1 Component 3; {4, 8, 16, 32}; Optional with capability signalling. |
| CMCC [19] | Basically, the number of components is proper for PDCCH repetition. However, for component 2, currently, we have only agreed that UE can report one number as required number of BDs for the two PDCCH candidates and reporting more numbers hasn’t been agreed. Therefore, we suggest the “or more” in the square blankets in component 2 should be deleted. Agreement @RAN1#104b-e For number of BDs corresponding to two PDCCH candidates that are linked for PDCCH repetition, support - UE reports one [or more] number(s) as required number of BDs for the two PDCCH candidates - Candidate values: 2, 3. - FFS: Default behaviour - FFS: Whether one of the candidate values imply that UE supports soft combining - FFS: Whether additional candidate values are supported (e.g. non-integer numbers) - FFS: RRC configuration based on reported UE capability. Proposal 1: Delete the “or more” in the square blankets in component 2 for FG 23-2-1. For the component 3 in FG 23-2-1, we think the number of BDs should not be tied with whether the individual PDCCH candidate is monitored or not for the case of overlapping. Because even if 3 BDs is reported in component 2, it can not be mandatory that the UE can perform two individual decoding and one soft combing. Some UE may still need 3BDs for one individual decoding and one soft combing, so whether the individual candidate can be monitored or not should be reported even if 3BDs is reported as required number of BDs for the two PDCCH candidates. Therefore, the wording of component 3 could be simple and the description of BD should be deleted. For example, “Support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET” is enough for component 3. Proposal 2: The description of BD in component 3 should be deleted for FG 23-2-1. |
| Nokia/Nokia Shanghai Bell [20] | - No independent signaling for component 1, as it is not allowed to individually disable components of an FG. No need to separate the FG though, support to component 1 is given by support to the FG itself. - Component 3 is not needed, these relations are expected to be described in specifications instead |

| 23. NR_FeMIMO | 23-2-2 | Two QCL TypeD for [CORESET monitoring in] PDCCH [repetition] | Support of determining two QCL-TypeD for overlapping CORESETs in the same CC or for intra-band CA [when UE is configured with PDCCH repetition] | [23-2-1, 23-6-1, 23-6-2] |  |  |  |  |  |  | FR2 only |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | We have been agreed that the condition of determining two QCL-TypeD for multiple overlapping CORESETs is PDCCH repetition, so the bracket for 23-2-2 should be removed. Agreement For a UE supporting reception with two different beams and configured with PDCCH repetitions, for determination of two QCL-TypeD properties for multiple monitored overlapping CORESETs, support - Alt2: Reuse legacy priority rule to identify the first QCL-TypeD property, and then, identify the second QCL-TypeD according to one of the SS sets that is linked with a SS set with the first QCL-TypeD (among the multiple overlapping CORESETs) - In the case of multiple such SS set pairs, Rel. 15 priority order is followed for the second QCL-TypeD determination - In the case of no such SS set pair, a second QCL-TypeD is not determined - Note 1: simultaneous two beam reception for PDCCH repetition is UE optional - Note 2: It can be separately discussed whether/how this feature interacts with multi-DCI based mTRP or with SFN PDCCH Proposal 3-2: Remove the bracket in feature group and components in 23-2-2. - Support to determine two QCL Type D when UE is configured with PDCCH repetition. 23. NR_FeMIMO; 23-2-2; Two QCL TypeD for [CORESET monitoring in] PDCCH [repetition]; Support of determining two QCL-TypeD for overlapping CORESETs in the same CC or for intra-band CA [when UE is configured with PDCCH repetition]; [23-2-1, 23-6-1, 23-6-2]; FR2 only; Optional with capability signalling. |
| ZTE [3] | 23-2-2; Two QCL TypeD for CORESET monitoring in PDCCH repetition; Support of determining two QCL-TypeD for overlapping CORESETs in the same CC or for intra-band CA when UE is configured with PDCCH repetition. |
| vivo [4] | The following agreement was achieved in RAN1#104-e meeting for CORESET monitoring. Agreement For a UE supporting reception with two different beams, support identifying two QCL-TypeD properties for multiple overlapping CORESETs - FFS: How to enhance existing QCL-TypeD priority rules for overlapping CORESETs - Note: The primary goal of this enhancement for the purpose of this sub-AI is to support time-overlapping PDCCH repetitions in FR2.. It can be seen that the primary goal of this enhancement for the purpose of this sub-AI is to support time-overlapping PDCCH repetitions in FR2. Thus, the condition of ‘when UE is configured with PDCCH repetition’ shall be kept. Proposal-9: For two QCL TypeD for CORESET monitoring in PDCCH repetition, UE feature 23-2-2 is revised as follows: 23-2-2; Two QCL TypeD for [CORESET monitoring] in PDCCH repetition; Support of determining two QCL-TypeD for overlapping CORESETs in the same CC or for intra-band CA [when UE is configured with PDCCH repetition]; [23-2-1, 23-6-1, 23-6-2]; FR2 only; Optional with capability signalling. |
| CATT [5] |  |
| Samsung [6] | Regarding a new required UE feature identifying two QCL-TypeD properties for SFNed PDCCH, since there are ongoing separate discussions for PDCCH repetition and SFNed PDCCH in PDCCH enhancement and HST-SFN agendas, respectively, but it is expected to make an aligned solution for both, our view is that FG 23-2-2 in PDCCH enhancement can be reused for the purpose of identifying two QCL-TypeD properties to receive SFNed PDCCH which two TCIs have been activated on a CORESET. Proposal 24: Re-use FG 23-2-2 as identifying two QCL-TypeD properties for SFNed PDCCH. |
| NTT DOCOMO, INC. [7] | For FG23-2-2, we think 23-6-1 and 23-6-2 should be removed from the prerequisite FG, because in our view, this FG is only related to non-SFN PDCCH repetition and is not related to SFN PDCCH. Meanwhile, we support to keep the text in bracket. 23. NR_FeMIMO; 23-2-2; Two QCL TypeD for CORESET monitoring in PDCCH repetition; Support of determining two QCL-TypeD for overlapping CORESETs in the same CC or for intra-band CA when UE is configured with PDCCH repetition; 23-2-1; FR2 only. |
| Spreadtrum Communications [8] |  |
| Qualcomm Incorporated [9] | 23. NR_FeMIMO; 23-2-2; Two QCL TypeD for PDCCH repetition; Support of determining two QCL-TypeD for overlapping CORESETs in the same CC or for intra-band CA when UE is configured with PDCCH repetition; FG 23-2-1; Per Band; FR2 only; Optional. |
| OPPO [10] | For 23-2-2 on determining two QCL-TypeD for overlapping CORESETs in the same CC or for intra-band CA which is introduced to support PDCCH repetition from multi-TRP. Hence it is reasonable to remove the bracket in FG 23-2-2. Proposal 2: Support of determining two QCL-TypeD for overlapping CORESETs in the same CC or for intra-band when UE is configured with PDCCH repetition. |
| Intel Corporation [11] | - for FG 23-2-2, clarify that overlap is in time dimension 23-2-2; Two QCL TypeD for [CORESET monitoring in] PDCCH [repetition]; Support of determining two QCL-TypeD for overlapping in time CORESETs in the same CC or for intra-band CA [when UE is configured with PDCCH repetition]; [23-2-1, 23-6-1, 23-6-2]. |
| Apple [12] |  |
| FUTUREWEI [13] |  |
| xiaomi [14] |  |
| LG Electronics [15] |  |
| Lenovo/Motorola Mobility [16] | For feature group 23-2-2, we think [when UE is configured with PDCCH repetition] is necessary since this feature is not for Rel.15/16 PDCCH and SFN based PDCCH. For two QCL TypeD monitoring for SFN scenario, another similar feature can be introduced in FG 23-6 to avoid confusion. So, we suggest to remove the bracket and keep the text “when UE is configured with PDCCH repetition”. Keep the text “when UE is configured with PDCCH repetition” for FG 23-2-2 |
| Ericsson [17] |  |
| MediaTek Inc. [18] |  |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] |  |

| 23. NR_FeMIMO | 23-2-3 | PDCCH repetition for Type3 CSS | Support of PDCCH repetition for Type3 CSS |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| ZTE [3] | For 23-2-3 on support of PDCCH repetition for Type 3 CSS, as aforementioned, we think this extra FG is unnecessary. The main FG 23-2-1 is sufficient from our view as there is no much difference between USS and CSS to process PDCCH repetition operation from UE side. No matter two PDCCH is from USS or CSS, the decoding of two PDCCH repetition is the same, i.e. separate decoding or soft combining. Proposal 6: Remove FG 23-2-3 on support of PDCCH repetition for Type 3 CSS |
| vivo [4] |  |
| CATT [5] | As we discussed in FG 23-2-1 part, PDCCH repetition for Type 3 CSS can be included in FG 23-2-1. Therefore, FG 23-2-3 can be removed. Proposal-10: FG 23-2-3 can be removed. 23-2-3; PDCCH repetition for Type3 CSS; Support of PDCCH repetition for Type3 CSS; Optional with capability signalling. |
| Samsung [6] |  |
| NTT DOCOMO, INC. [7] | For FG23-2-3, we think this FG should be removed. As analyzed above, there is no need to have separate capability for Type3 CSS. |
| Spreadtrum Communications [8] |  |
| Qualcomm Incorporated [9] | 23. NR_FeMIMO; 23-2-3; PDCCH repetition for Type3 CSS; Support of PDCCH repetition for Type3 CSS; FG 23-2-1; Per Band; Optional. |
| OPPO [10] |  |
| Intel Corporation [11] | 23-2-3; PDCCH repetition for Type3 CSS; Support of PDCCH repetition for Type3 CSS. |
| Apple [12] |  |
| FUTUREWEI [13] |  |
| xiaomi [14] |  |
| LG Electronics [15] |  |
| Lenovo/Motorola Mobility [16] |  |
| Ericsson [17] |  |
| MediaTek Inc. [18] |  |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] | Confirm the FG |

| 23. NR_FeMIMO | 23-3-1 | Multi-TRP PUSCH repetition (type A) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) [2. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A] [3. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2] [4. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.)] [5. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.)] [6. support of sequential mapping for single for single DCI based M-TRP PUSCH repetition Type A] [7. The maximum number of PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission)] [8. Support of A-CSI report on two PUSCH repetitions] [9. Support of SP-CSI report on two PUSCH repetitions] [10: Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission] [11. Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources] |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | Multi-TRP PUSCH repetition In order to reduce the implementation complexity for a UE for the PHR calculation and report in MTRP across all CCs in a band, the max number of PHR report across CCs should be a UE capability. Proposal 3-4: Support the component 7 in FG 23-3-1 and remove the corresponding bracket, - The maximum number of supported PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission) is a UE capability, FFS on candidate value, which is no more than 2*the number of supported CCs. 23. NR_FeMIMO; 23-3-1; Multi-TRP PUSCH repetition (type A); 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) [2. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A] [3. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2] [4. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.)] [5. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.)] [6. support of sequential mapping for single for single DCI based M-TRP PUSCH repetition Type A] [7. The maximum number of PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission)] [8. Support of A-CSI report on two PUSCH repetitions] [9. Support of SP-CSI report on two PUSCH repetitions] [10: Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission] [11. Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources]; Optional with capability signalling. |
| ZTE [3] | On component 6, it means sequential mapping should be mandatory to MTRP PUSCH scheme. However, both sequential mapping and cyclic mapping for MTRP PDSCH scheme in Rel-16 are NOT explicitly specified as the UE basic feature, because both of them are supported by the UE in default according to RAN1 specifications. Likewise, it is unnecessary to specify sequential mapping as one UE feature, hence we propose to remove component 6. On component 7, it is NOT in line with any agreement in the previous RAN1 meetings so far, hence we propose to remove component 7. On component 10, it means STRP/MTRP dynamic switching can be support or not supported to the UE. If so, when the UE cannot support this component, it means the UE can only be scheduled to always transmit PUSCH to multiple TRP, which is an error case of UE design in reality. Hence we propose to remove component 10. On component 11, we fail to see the necessity for the number of SRS resources when CB and NCB schemes, which is the same as Rel-15/16 and can be captured by RAN1 specifications. Besides, RAN1 has agreed that the number of SRS resources in two SRS resource sets should be the same, so it is unclear to the intention of this component. Hence we propose to remove component 11. Proposal 8: For FG 23-3-1, remove components 6, 7 10 and 11. 23-3-1; Multi-TRP PUSCH repetition (type A); 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) [2. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A] [3. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2] [4. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.)] [5. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.)] [8. Support of A-CSI report on two PUSCH repetitions] [9. Support of SP-CSI report on two PUSCH repetitions]. |
| vivo [4] |  |
| CATT [5] | According to the agreements in previous RAN1 meetings, component 2, 3, 4, 5, 8 and 9 in FG 23-3-1 are optional UE features, therefore they should be sperated from FG 23-3-1. Since component 7 is not in line with any agreement in the previous RAN1 meetings, it should be removed from FG 23-3-1. On the number of SRS resources, UE features on supported max number of SRS resource per set for CB based and NCB based PUSCH have been defined in Rel-15, respectively. It is our view that they can be applied to M-TRP PUSCH transmission directly. Therefore component 11 can be removed. Since single DCI based M-TRP PUSCH repetition schemes are based on Rel-16 PUSCH repetition Type A and Type B per agreements, the prerequisite feature groups for M-TRP PUSCH repetition Type A and Type B can be FG 11-6( pusch-RepetitionTypeA-r16) and FG 11-5(pusch-RepetitionTypeB-r16), respectively. Two TPC filed in DCI format 0_1/0_2 is needed only when two different TPC loops for PUSCH is supported by a UE, therefore the prerequisite feature groups for the feature for two TPC field for M-TRP PUSCH repetition should include FG 8-8(UL power control with 2 PUSCH closed loops). Based on the discussions above, we have following proposal on UE features for M-TRP PUSCH repetition: Proposal-11: For M-TRP PUSCH repetition, adopt the following: 23-3-1; Multi-TRP PUSCH repetition (type A); 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) [2. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A] [3. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2] [4. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.)] [5. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.)] [62. sSupport of sequential mapping for single for single DCI based M-TRP PUSCH repetition Type A] [7. The maximum number of PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission)] [8. Support of A-CSI report on two PUSCH repetitions] [9. Support of SP-CSI report on two PUSCH repetitions] [10: 3. Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission] [11. Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources]; FG 11-6. 23-3-1-1a; Multi-TRP PUSCH repetition (type B); Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) FFS: discuss details (to be discussed after type A is stable; FG 11-5 and FG 23-3-1. 23-3-1b; CG based Multi-TRP PUSCH repetition; Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.); FG 23-3-1. 23-3-1c; Multi-TRP PUSCH repetition with cyclic mapping; Support of cyclic mapping when the number of repetitions is larger than 2; FG 23-3-1. 23-3-1d; Multi-TPC in UL DCI; Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2; FG 8-8 and FG 23-3-1. 23-3-1e; Multi-PHR reporting for PUSCH; Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.); FG 23-3-1. 23-3-1f; A-CSI on two PUSCH repetitions; Support of A-CSI report on two PUSCH repetitions; FG 23-3-1. 23-3-1g; SP-CSI on two PUSCH repetitions; Support of SP-CSI report on two PUSCH repetitions; FG 23-3-1. |
| Samsung [6] | In our view, similar to single-DCI based mTRP PDSCH repetition (TDM schemes) in Rel-16, UE capability signalling for single-DCI based PUCCH/PUSCH repetition can be supported per band level. Proposal 19: Support FG 23-3-1 and FG 23-3-1-1 multi-TRP PUSCH repetition type A and B, respectively, per band level. In RAN1#106-e, per TRP PHR reporting is UE optional capability in Rel-17. Therefore, we can split component#4 in FG 23-3-1 as optional capability and add new UE capability signalling for that. And also prerequisite feature group of new UE capability signalling for enhanced PHR can be FG 23-3-1. Proposal 20: Split Component 4 (per TRP PHR reporting) of FG 23-3-1 as separate FG which is per UE signaling. |
| NTT DOCOMO, INC. [7] | For M-TRP PUSCH repetition, in our view, M-TRP PUSCH repetition type A and M-TRP PUSCH repetition type B can be merged into one FG, because in our understanding, other components in this FG are common for repetition type A and repetition type B, e.g., if UE supports cyclic mapping, it supports for both repetition type A and repetition type B, thus, there is no need to have separate FGs for repetition type A and type B. Furthermore, CB based and NCB based operation can be split into two components since UE may have different capability of whether to support CB or NCB PUSCH operation. And regarding number of SRS resources, it can be clarified that UE supports two CB or NCB SRS resource sets. Proposal 3: Adopt the following for Rel-17 M-TRP PUSCH 23. NR_FeMIMO; 23-3-1; Multi-TRP PUSCH repetition (type A); 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) 2. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) 3. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A 4. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 5. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) 6. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.) 7. support of sequential mapping for single for single DCI based M-TRP PUSCH repetition Type A 8. The maximum number of PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission) 9. Support of A-CSI report on two PUSCH repetitions 10. Support of SP-CSI report on two PUSCH repetitions 11: Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission 12. Support multi-TRP PUSCH repetition with CB based PUSCH operations and corresponding parameters including two CB SRS resource sets 13. Support multi-TRP PUSCH repetition with NCB based PUSCH operations and corresponding parameters including two NCB SRS resource sets; Optional with capability signalling. |
| Spreadtrum Communications [8] |  |
| Qualcomm Incorporated [9] | The following aspects need to be considered when designing UE capability for mTRP PUSCH repetitions: - Separate capabilities are required for codebook-based and non-codebook based mTRP PUSCH. - For codebook-based mTRP PUSCH, the structure of Rel-15 capability (FG 2-14) including indication on max number of SRS resources need to be taken in to account, and should be extended for the case that two SRS resource sets are configured. - For non-codebook based mTRP PUSCH, the structure of Rel-15 capability (FGs 2-15, 2-15a, 2-15b) including indication on max number of SRS resources and various capabilities for associated CSI-RS (for precoding calculation) need to be taken in to account, and should be extended for the case that two SRS resource sets / two associated CSI-RS resources are configured. - Separate capabilities for mTRP PUSCH repetition are required for repetition Type A and repetition Type B. - Reporting two PHRs per CC should be a separate UE capability as already agreed below: Agreement For PHR reporting related to M-TRP PUSCH repetition, support Option 4 as UE optional capability for a UE that supports mTRP PUSCH, - Option 4: Calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs - Capabilities are needed for transmitting AP/SP-CSI on two PUSCH repetitions as already agreed: Agreement For mTRP PUSCH repetition, introduce a RRC parameter to enable/disable the new behavior of transmitting AP/SP-CSI on the first PUSCH repetitions with the first beam and the second beam. The new parameter can be per CSI-AperiodicTriggerState or CSI-SemiPersistentOnPUSCH-TriggerState. - Transmitting AP/SP-CSI on two PUSCH repetitions with different beams is UE optional capability - Per TRP default power control pramaters or per TRP OLPC set indication features require UE capability. From our perspective, the first and most important aspect is to have separate capabilities consistent with Rel-15 for codebook-based and non-codebook based mTRP PUSCH. Based on the above discussions, we propose the following FGs for mTRP PUSCH: Proposal 4-1: Adopt the following for Rel-17 mTRP PUSCH UE features. 23. NR_FeMIMO; 23-3-1a; Multi-TRP PUSCH repetition for codebook based; 1. Support of codebook-based multi-TRP PUSCH repetition for repetition Type A 2. Support of two SRS resource sets with usage set to codebook 3. Supported number of SRS resources in one SRS resource set 4. Supported number of SRS resources across two SRS resource sets; FG 2-14; Per FSPC; Note: For more than 2 repetitions, sequential beam mapping is supported as part of this FG. Component 3: {1,2,4} Component 4: {2,3,4,5,6,8}; Optional. 23. NR_FeMIMO; 23-3-1b; Multi-TRP PUSCH repetition for non-codebook based; 1. Support of non-codebook-based multi-TRP PUSCH repetition for repetition Type A 2. Support of two SRS resource sets with usage set to noncodebook 3. Supported number of SRS resources in one SRS resource set 4. Supported number of SRS resources across two SRS resource sets; FG 2-15; Per FSPC; Note: For more than 2 repetitions, sequential beam mapping is supported as part of this FG. Component 3: {1,2,3,4} Component 4: {2,3,4,5,6,7,8}; Optional. 23. NR_FeMIMO; 23-3-1b-1; Two associated CSI-RS resources; Support of up to two CSI-RS resources associated with the two SRS resource sets for non-codebook-based mTRP PUSCH; FG 2-15a FG 23-3-1b; Per Band; Optional. 23. NR_FeMIMO; 23-3-1b-2; CSI-RS processing framework for SRS with two associated CSI-RS resources; 1. Maximum number of periodic SRS resources associated with first or second CSI-RS per BWP 2. Maximum number of aperiodic SRS resources associated with first or second CSI-RS per BWP 3. Maximum number of semi-persistent SRS resources associated with first or second CSI-RS per BWP 4. UE can process Y SRS resources associated with first or second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS.; FG 2-15b FG 23-3-1b; Per Band; Component 1: {1 to 8} Component 2: {1 to 8} Component 3: {1 to 8} Component 4: {1 to 16}; Optional. 23. NR_FeMIMO; 23-3-1c; Multi-TRP PUSCH repetition for repetition Type B; Support of multi-TRP PUSCH repetition for repetition Type B; FG 11-5 FG 23-3-1a or 23-3-1b; Per FS; Optional. 23. NR_FeMIMO; 23-3-1d; Two TPC fields in UL DCI; Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2; FG 8-8 FG 23-3-1a or 23-3-1b; Per Band; Optional. 23. NR_FeMIMO; 23-3-1e; Cyclic beam mapping; Support of cyclic mapping when the number of repetitions is larger than 2; FG 23-3-1a or 23-3-1b; Per Band; Optional. 23. NR_FeMIMO; 23-3-1f; Two PHRs per CC; Support of reporting two PHRs per CC (for a CC configured with mTRP PUSCH repetition); FG 23-3-1a or 23-3-1b; Per Band; Optional. 23. NR_FeMIMO; 23-3-1g; Multi-TRP PUSCH repetition for configured grant; 1. Support of multi-TRP PUSCH repetition for configured grant 2. Supported type of configured grant for multi-TRP PUSCH repetition; FG 23-3-1a or 23-3-1b; Per Band; Component 2: {Type1, Type2, both}; Optional. 23. NR_FeMIMO; 23-3-1h; AP-CSI on two PUSCH repetitions; Support of transmitting AP-CSI on two PUSCH repetitions; FG 23-3-1a or 23-3-1b; Per Band; Optional. 23. NR_FeMIMO; 23-3-1i; SP-CSI on two PUSCH repetitions; Support of transmitting SP-CSI on two PUSCH repetitions; FG 23-3-1a or 23-3-1b; Per Band; Optional. 23. NR_FeMIMO; 23-3-1j; Per TRP OLPC set configurations; Support configurations of two (per TRP) p0-PUSCH-SetList; FG 11-8 FG 23-3-1a or 23-3-1b; Per Band; Optional. 23. NR_FeMIMO; 23-3-1k; Per TRP default power control parameters; Support of two (per TRP) sets of default power control parameters when SRI fields are absent from the DCI; FG 23-3-1a or 23-3-1b; Per Band; Optional. |
| OPPO [10] | In component 6, we think support of sequential mapping for single DCI based M-TRP PUSCH repetition Type A is mandatory and it should be removed from UE feature. Proposal 3: For FG 23-2-1, suggest to remove component 6“ support of sequential mapping for single DCI based M-TRP PUSCH repetition Type A”. |
| Intel Corporation [11] | - for FG 23-3-1 component 1, clarify support of sequential and cyclical mapping - for FG 23-3-1 components 6, 7 should be part of basic FG - for FG 23-3-1 components 8, 9 can be optional - for FG 23-3-1 component 10 should be part of basic FG - for FG 23-3-1 component 11, CB based should be part of basic FG - for FG 23-3-1 add component 12: For NCB based PUSCH support of simultaneous precoding calculation for different associated NZP-CSI-RS within a CC (down-selection from RAN1#106b-e agreement) 23-3-1; Multi-TRP PUSCH repetition (type A); 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type Aand B) - sequential mapping for any number of repetitions - cyclic mapping for 2 repetitions 2. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A and B, 3. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 4. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) [5. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.)] [6. support of sequential mapping for single for single DCI based M-TRP PUSCH repetition Type A] [7. The maximum number of PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission)] [8. Support of A-CSI report on two PUSCH repetitions] [9. Support of SP-CSI report on two PUSCH repetitions] [10: Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission] [11. Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources] 12. For NCB based PUSCH support of simultaneous precoding calculation for two different associated NZP-CSI-RS within a CC. |
| Apple [12] | With regard to the potential UE complexity to handle the overlapped CSI-RS+SRS pairs for non-codebook SRS transmission, we suggest adding additional elements 12 and 13. Proposal 1.5-1: Support the following change for FG 23-3-1 (mTRP PUSCH) - Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A and B) - Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A and B - Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 - Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) - Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.) - support of sequential mapping for single for single DCI based M-TRP PUSCH repetition Type A - The maximum number of PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission) - Support of A-CSI report on two PUSCH repetitions - Support of SP-CSI report on two PUSCH repetitions - Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission - Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources - Maximum number of overlapped associated CSI-RS + SRS for NCB pairs in a CC - Maximum number of overlapped associated CSI-RS + SRS for NCB pairs across CCs in a band 23. NR_FeMIMO; 23-3-1; Multi-TRP PUSCH repetition (type A); 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A and B) 2. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A and B 3. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 4. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) 5. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.) 6. support of sequential mapping for single for single DCI based M-TRP PUSCH repetition Type A 7. The maximum number of PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission) 8. Support of A-CSI report on two PUSCH repetitions 9. Support of SP-CSI report on two PUSCH repetitions 10: Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission 11. Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources 12. Maximum number of overlapped associated CSI-RS + SRS for NCB pairs in a CC 13. Maximum number of overlapped associated CSI-RS + SRS for NCB pairs across CCs in a band; 1-10. {Support, Not support} 11. {1, 2, 3, 4} 12. {1, 2, 3, 4}; Optional with capability signalling. |
| FUTUREWEI [13] |  |
| xiaomi [14] | - On components 2 and 6, the related agreement was made in RAN1#104bis-e [5] as below. Agreement Confirm the following working assumption (with removing the last bullet): For single DCI based M-TRP PUSCH repetition Type A and B, it is possible to configure either cyclic mapping or sequential mapping of UL beams. - The support of cyclic mapping can be optional UE feature for the cases when the number of repetitions is larger than 2. - FFS: Support of half-half mapping. - FFS: Additional considerations on mapping patterns (including required beam switching gaps) Based on this agreement, the cyclic mapping can be optional UE feature for the cases when the number of repetitions is larger than 2, and the sequential mapping is always supported. In our opinion, the square brackets of component 2 should be removed. The sequential mapping should be basic UE feature and we propose to remove component 6. - On component 3, the related agreement was made in RAN1#106-e [7] as below. Agreement For mTRP PUCCH (or PUSCH) repetitions schemes, - When the second TPC field is configured and the indicated PUCCH transmission in DCI formats 1_1/1_2 (or PUSCH transmission in DCI formats 0_1/0_2) is associated with the same “closedLoopIndex” value for mutli-TRP tranmission, the other TPC field associated with the other “closedLoopIndex” value is unused. - Note: When the other TPC field associated with the other “closedLoopIndex” value is unused, the unused TPC field is not applied for any legacy procedures of calculating sum of TPC command values. Based on this agreement, the square brackets of component 3 should be removed. - On component 4, the related agreement was made in RAN1#106-e [7] as below. Agreement For PHR reporting related to M-TRP PUSCH repetition, support Option 4 as UE optional capability for a UE that supports mTRP PUSCH, - Option 4: Calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs. Based on this agreement, the square brackets of component 4 should be removed. - On component 5, the related agreement was made in RAN1#104-e [9] as below. Agreement Support CG PUSCH transmission towards M-TRPs using a single CG configuration. - Use same beam mapping principals as dynamic grant PUSCH repetition scheme. - FFS: Required changes on CG parameters (ConfiguredGrantConfig) - The feature is UE optional Based on this agreement, the square brackets of component 5 should be removed. - On component 7, the related agreement was made in RAN1#106-e [7] as below. Agreement For PHR reporting related to M-TRP PUSCH repetition, support Option 4 as UE optional capability for a UE that supports mTRP PUSCH, - Option 4: Calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs. From our perspective, component 7 is not in line with any agreement in the previous RAN1 FeMIMO discussion, and it can be modified as “The maximum number of PHR reporting per CC (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission)”. - On components 8 and 9, the related agreement was made in RAN1#107-e [4] as below, Agreement For mTRP PUSCH repetition, introduce a RRC parameter to enable/disable the new behavior of transmitting AP/SP-CSI on the first PUSCH repetitions with the first beam and the second beam. The new parameter can be per CSI-AperiodicTriggerState or CSI-SemiPersistentOnPUSCH-TriggerState. - Transmitting AP/SP-CSI on two PUSCH repetitions with different beams is UE optional capability. According to this agreement, the components 8 and 9 should be agreed as UE capability, and the square brackets should be removed. - On component 10, from our perspective, this component indicates that the dynamic switching between S-TRP and M-TRP is supported or not. If the UE cannot support this component, then M-TRP transmission is always scheduled. So, we think component 10 should be basic FG and propose to remove this component. - As for component 11, we suggest to remove it. Proposal 8: The proposed FG23-3-1 for M-TRP PUSCH repetition type A is show as follows. 23. NR_FeMIMO; 23-3-1; Multi-TRP PUSCH repetition (type A); 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) [2. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A] [3. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2] [4. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.)] [5. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.)] [6. support of sequential mapping for single for single DCI based M-TRP PUSCH repetition Type A] [76. The maximum number of PHR reporting across all CCs per CC (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission)] [7. Support of A-CSI report on two PUSCH repetitions] [8. Support of SP-CSI report on two PUSCH repetitions] [10: Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission] [11. Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources]; Optional with capability signalling. |
| LG Electronics [15] | - Except for component 1 and 6, each component should be defined as a separated sub-FG so that UE is able to report whether to support each of them, separately. - Component 6: sequential mapping should be a component of this FG. On the other hand, cyclic mapping is agreed as optional so component 2 should be a separate FG. - Component 11: it should be removed since MTRP PUSCH repetition cannot be supported without supporting PUSCH operations. Proposal 6: Adopt the following table for Rel-17 M-TRP PUSCH. 23. NR_FeMIMO; 23-3-1; Multi-TRP PUSCH repetition (type A); 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) 2. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A 3. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 4. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) 5. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.) 6. support of sequential mapping for single for single DCI based M-TRP PUSCH repetition Type A 7. The maximum number of PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission) 8. Support of A-CSI report on two PUSCH repetitions 9. Support of SP-CSI report on two PUSCH repetitions 10: Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission; Optional with capability signalling. |
| Lenovo/Motorola Mobility [16] |  |
| Ericsson [17] | Regarding component 6, our recollection is that this should be supported by the UE. Then, do we need to mention this as a component in UE features? Our understanding is that ‘sequential mapping’ will be supported in RAN1 specifications, and even if we don’t mention it as a component of the basic feature, the UE still supports it. So, we don’t see the need to include component 6, and propose to remove it. Regarding component 7, we think that this component does not correspond to any agreement during FeMIMO WI. Hence, component 4 should be enough for PHR, and we propose to remove component 7. On component 10, the intention seems to be to introduce {support/not support} for ‘dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission’ as proposed by the proponent of this component. If a UE is not capable of dynamic switching between single TRP and multi-TRP PUSCH, is this a UE that can only be scheduled to always transmit to multiple TRPs? Not sure how realistic such a UE is. So, we prefer to remove component 10. Regarding component 11, the need for this component is unclear. Furthermore, ‘PUSCH operations: CB based and NCB based and corresponding parameters including SRS resources’ is too broad to be captured as a component. Hence, we prefer to remove component 11. Remove Components 6, 7, 10, and 11 from FG 23-3-1. |
| MediaTek Inc. [18] | We propose to have separate capabilities for codebook-based and non-codebook based mTRP PUSCH, hence, new FG is needed to capture this as opposed to only having component 11 within 23-3-1 to determine if a UE supports codebook/non-codebook based PUSCH. Having separate FG for codebook and non-codebook uplink capabilities is inline with R15 design and we propose to keep the same consistency. Proposal 37: Have separate FG for codebook and non-codebook based mTRP PUSCH for both Type A and Type B. Many components in FG 23-3-1 are agreed as UE optional features of multi-TRP PUCCH/PUSCH repetitions, so these features should be listed as separate feature groups. Proposal 38: Move Components 2, 3, 4, 5, 8, 9 in FG 23-3-1 into distinct feature groups FG 23-3-1c, FG 23-3-1d, FG 23-3-1e, FG 23-3-1f, FG 23-3-1g, FG 23-3-1h. 23. NR_FeMIMO; 23-3-1; Multi-TRP PUSCH repetition (type A) – Codebook Based; 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A and B) [2. support of sequential mapping for single for single DCI based M-TRP PUSCH repetition Type A] [3. The maximum number of PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission)] [4. Support of A-CSI report on two PUSCH repetitions] [5. Support of SP-CSI report on two PUSCH repetitions] [6: Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission] [7. Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources]; Optional with capability signalling. 23. NR_FeMIMO; 23-3-1a; Multi-TRP PUSCH repetition (type A) – Non-codebook Based; 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) [2. support of sequential mapping for single for single DCI based M-TRP PUSCH repetition Type A] [3. The maximum number of PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission)] [4: Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission] [5. Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources]; Optional with capability signalling. 23. NR_FeMIMO; 23-3-1b; Multi-TRP PUSCH repetition (type B)-Non-Codebook Based; 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) FFS: discuss details (to be discussed after type A is stable_; Optional with capability signalling. 23. NR_FeMIMO; 23-3-1c; Multi-TRP PUSCH repetition (type A) with cyclic mapping; Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A; 23-3-1; {Supported, Not supported}; Optional with capability signalling. 23. NR_FeMIMO; 23-3-1d; Multi-TRP PUSCH repetition (type A) with second TPC field; Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2; 23-3-1; {Supported, Not supported}; Optional with capability signalling. 23. NR_FeMIMO; 23-3-1e; Multi-TRP PUSCH repetition (type A) support of PHR reporting; Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.); 23-3-1; {Supported, Not supported}; Optional with capability signalling. 23. NR_FeMIMO; 23-3-1f; Multi-TRP PUSCH repetition (type A) CG PUSCH transmission; Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.); 23-3-1; {Supported, Not supported}; Optional with capability signalling. 23. NR_FeMIMO; 23-3-1g; A-CSI multiplexing; Support of A-CSI report on two PUSCH repetitions; 23-3-1; {Supported, Not supported}; Optional with capability signalling. 23. NR_FeMIMO; 23-3-1h; SP-CSI multiplexing; Support of SP-CSI report on two PUSCH repetitions; 23-3-1; {Supported, Not supported}; Optional with capability signalling. |
| CMCC [19] | For the UE capability reporting for cyclic or sequential mapping for PUSCH repetition, the following agreement is achieved: Agreement @RAN1#104b-e Confirm the following working assumption (with removing the last bullet): For single DCI based M-TRP PUSCH repetition Type A and B, it is possible to configure either cyclic mapping or sequential mapping of UL beams. - The support of cyclic mapping can be optional UE feature for the cases when the number of repetitions is larger than 2. - FFS: Support of half-half mapping. - FFS: Additional considerations on mapping patterns (including required beam switching gaps) From our understanding, the support of sequential mapping is a basic capability if the UE supporting multi-TRP PUSCH repetition type A, so component 6 should be one of the basic components for FG 23-3-1. However, the support of cyclic mapping is optional UE feature for the cases when the number of repetitions is larger than 2, in other words, UE may support multi-TRP PUSCH repetition type A but cannot support cyclic mapping, so component 2 should be separated from FG 23-3-1. Proposal 3: Separate component 2 from FG 23-3-1. |
| Nokia/Nokia Shanghai Bell [20] |  |

| 23. NR_FeMIMO | 23-3-1-1 | Multi-TRP PUSCH repetition (type B) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) FFS: discuss details (to be discussed after type A is stable |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| ZTE [3] |  |
| vivo [4] |  |
| CATT [5] |  |
| Samsung [6] | In our view, similar to single-DCI based mTRP PDSCH repetition (TDM schemes) in Rel-16, UE capability signalling for single-DCI based PUCCH/PUSCH repetition can be supported per band level. Proposal 19: Support FG 23-3-1 and FG 23-3-1-1 multi-TRP PUSCH repetition type A and B, respectively, per band level. |
| NTT DOCOMO, INC. [7] |  |
| Spreadtrum Communications [8] |  |
| Qualcomm Incorporated [9] |  |
| OPPO [10] |  |
| Intel Corporation [11] | 23-3-1-1; Multi-TRP PUSCH repetition (type B); 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) FFS: discuss details (to be discussed after type A is stable. |
| Apple [12] |  |
| FUTUREWEI [13] |  |
| xiaomi [14] | Proposal 9: The proposed FG23-3-1-1 for M-TRP PUSCH repetition type B is show as follows. 23. NR_FeMIMO; 23-3-1-1; Multi-TRP PUSCH repetition (type B); 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) 2. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type B 3. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 4. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) 5. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.) 6. The maximum number of PHR reporting per CC (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission) 7. Support of A-CSI report on two actual PUSCH repetitions 8. Support of SP-CSI report on two actual PUSCH repetitions; Optional with capability signalling. |
| LG Electronics [15] |  |
| Lenovo/Motorola Mobility [16] |  |
| Ericsson [17] |  |
| MediaTek Inc. [18] |  |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] | Confirm the FG. Details to be further discussed based on structure of 23-3-1. |

| 23. NR_FeMIMO | 23-3-2 | Multi-TRP PUCCH repetition | 1. Support of PUCCH repetition scheme 1 (inter-slot repetition) 2. Support of PUCCH repetition scheme 3 (intra-slot repetition) 3. Support of cyclic mapping for beam mapping/power control parameter set mapping [when the number of repetitions is larger than 2] for PUCCH repetitions scheme 1 and 3 when the number of repetitions is larger than 2 4. Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats [1_0 /] 1_1 / 1_2 |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| ZTE [3] | FG 23-3-2: Multi-TRP PUCCH repetition On component 3, we agree to add the condition “when the number of repetitions is larger than 2” for cyclic mapping, and we are general fine with either way to add the text in component 3, e.g. removing the text in the brackets. Proposal 9: For FG 23-3-2, support to add the text “when the number of repetitions is larger than 2” in component 3. 23-3-2; Multi-TRP PUCCH repetition; 1. Support of PUCCH repetition scheme 1 (inter-slot repetition) 2. Support of PUCCH repetition scheme 3 (intra-slot repetition) 3. Support of cyclic mapping for beam mapping/power control parameter set mapping for PUCCH repetitions scheme 1 and 3 when the number of repetitions is larger than 2 4. Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats [1_0 /] 1_1 / 1_2. |
| vivo [4] |  |
| CATT [5] | Since repetition for PUCCH format 0 or 2 are not supported in Rel-16 and M-TRP PUCCH repetition are supported for all PUCCH format in Rel-17, we prefer FG 23-3-2 to be a UE feature indenpendent of Rel-16 UE features for PUCCH repetition. A UE support FG 23-3-2 should support PUCCH format 1 or 3 or 4 over multiple slots with the repetition factor 2, 4 or 8 and PUCCH format 0 or 2 over multiple slots with the repetition factor 2. PUCCH repetition scheme 3 is an optional UE feature according to previous meeting agreements, therefore it should be sperated from FG 23-3-2. Similarly as that for M-TRP PUSCH repetition, sequential mapping for M-TRP PUCCH repetition is a basic feature and the following two features are optional features that should be separated from FG 23-3-2: - Cyclic mapping for the cases when the number of repetitions is larger than 2; - Second TPC filed for per TRP closed loop power control. On the details of the the feature group for two TPC fields in DL DCI, since there is no agreement on the supportive of two TPC fields in DCI format 1_0, DCI format 1_0 should be removed. As two TPC fileds in DCI format 1_1/1_2 is needed only when two different TPC loops for PUCCH is supported by a UE, the prerequisite feature groups for the feature for two TPC field in DL DCI should include FG 8-9(UL power control with 2 PUCCH closed loops). In RAN1 #106-e meeting, grouping of PUCCH resources in Rel-17 multi-TRP PUCCH repetition schemes was supported, with the following details: - Support MAC-CE activating two spatial relation info’s (for FR2) for a group of PUCCH resources in a CC. - Support MAC-CE activating two sets of power control parameters (for FR1) for a group of PUCCH resources in a CC. - When the PUCCH resource is indicated with two spatial relation info’s or two sets of power control parameters (via a MAC-CE that activating two spatial relation info’s or a MAC-CE that activating two sets of power control parameters for a group of PUCCH resources, respectively), the other PUCCH resources in the group also get updated to have the same two spatial relation info’s or two sets of power control parameters. - When the PUCCH resource is indicated with one spatial relation info or one set of power control parameters (via a MAC-CE that activating single spatial relation info or a MAC-CE that activating single set of power control parameters for a group of PUCCH resources, respectively), then the other PUCCH resources in the group also get updated to have the same spatial relation info or the same set of power control parameters. PUCCH grouping in Rel-16 is an optional feature, it is natural to support grouping PUCCH resources with multi-TRP PUCCH repetition as an optional feature in Rel-17. Based on the discussions above, we have following proposal on UE features for M-TRP PUCCH repetition: Proposal-12: For M-TRP PUCCH repetition, adopt the following: 23-3-2; Multi-TRP PUCCH repetition; 1. Support of PUCCH repetition scheme 1 (inter-slot repetition) 2. Support of sequential mapping for beam mapping/power control parameter set mapping for PUCCH repetitions scheme 1 Support of PUCCH repetition scheme 3 (intra-slot repetition) 3. Support of cyclic mapping for beam mapping/power control parameter set mapping [when the number of repetitions is larger than 2] for PUCCH repetitions scheme 1 and 3 when the number of repetitions is larger than 2 4. Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats [1_0 /] 1_1 / 1_2. 23-3-2a; Multi-TRP PUCCH repetition based on miniSlot; 1. Support of PUCCH repetition scheme 3 (intra-slot repetition) 2. Support of sequential mapping for beam mapping/power control parameter set mapping for PUCCH repetitions scheme 3; FG 23-3-2. 23-3-2b; Multi-TRP PUCCH repetition with cyclic mapping; Support of cyclic mapping for beam mapping/power control parameter set mapping [when the number of repetitions is larger than 2] for PUCCH repetition when the number of repetitions is larger than 2; FG 23-3-2. 23-3-2c; Multi-TPC in DL DCI; Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats 1_1 / 1_2; FG 8-9 and FG 23-3-2. 23-3-2d; Spatial relation/power control update for PUCCH group with Multi-TRP repetition; Support of PUCCH resource groups per BWP for simultaneous spatial relation/power control parameter update for Multi-TRP PUCCH repetition.; FG 23-3-2. |
| Samsung [6] | In our view, similar to single-DCI based mTRP PDSCH repetition (TDM schemes) in Rel-16, UE capability signalling for single-DCI based PUCCH/PUSCH repetition can be supported per band level. Proposal 18: Support FG 23-3-2 for multi-TRP PUCCH repetition per band level. |
| NTT DOCOMO, INC. [7] | For M-TRP PUCCH, for FG23-3-2, we have following suggestion. - Component3: the text in bracket should be removed, because it is duplicate with the text at the end of the sentence. - Component4: the text in bracket should be removed. It was agreed that for DCI format 1_0, single TPC field is applied. Proposal 4: Adopt the following for Rel-17 M-TRP PDCCH 23. NR_FeMIMO; 23-3-2; Multi-TRP PUCCH repetition; 1. Support of PUCCH repetition scheme 1 (inter-slot repetition) 2. Support of PUCCH repetition scheme 3 (intra-slot repetition) 3. Support of cyclic mapping for beam mapping/power control parameter set mapping for PUCCH repetitions scheme 1 and 3 when the number of repetitions is larger than 2 4. Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats1_1 / 1_2; Optional with capability signalling. |
| Spreadtrum Communications [8] |  |
| Qualcomm Incorporated [9] | The following aspects need to be considered for UE capability of mTRP PUCCH repetitions: - Separate capabilities are needed for mTRP PUCCH Scheme 1 versus Scheme 3 - Supported PUCCH format(s) need to be indicated for each of the Scheme 1 and Scheme 3 - For the details of the capability for Scheme 3, we need to consider how the capability of sub-slot based PUCCH repetition with sTRP in Rel-17 IIoT AI is indicated. - Updating two beams / power control parameters for a group of PUCCH resources requires a UE capability Proposal 5-1: Adopt the following for Rel-17 mTRP PUCCH UE features. 23. NR_FeMIMO; 23-3-2; Multi-TRP PUCCH repetition; 1. Support of multi-TRP PUCCH repetition scheme 1 (inter-slot repetition) 2. Support of updating two Spatial Relation Info’s / two sets of power control parameters for a PUCCH resource by MAC-CE 3. Supported PUCCH formats for this scheme; Per FS; Note: For more than 2 repetitions, sequential beam mapping is supported as part of this FG. Component 3: A bit map of size 5 for PUCCH format 0-4. At least one value of the bitmap should be set to 1.; Optional. 23. NR_FeMIMO; 23-3-2a; Multi-TRP PUCCH repetition with sub-slots; 1. Support of multi-TRP PUCCH repetition scheme 3 (sub-slot based repetition) 2. Supported PUCCH formats for this scheme; FG 23-3-2; Per FS; Note1: For more than 2 repetitions, sequential beam mapping is supported as part of this FG. Note2: The details of this FG should be decided after FG(s) related to sTRP sub-slot based PUCCH repetition in Rel-17 IIoT AI. Component 3: A bit map of size 5 for PUCCH format 0-4. At least one value of the bitmap should be set to 1.; Optional. 23. NR_FeMIMO; 23-3-2b; Two TPC fields in DL DCI; Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats 1_1 / 1_2; FG 8-9 FG 23-3-2; Per Band; Optional. 23. NR_FeMIMO; 23-3-2c; Cyclic beam mapping; Support of cyclic mapping when the number of repetitions is larger than 2; FG 23-3-2; Per Band; Optional. 23. NR_FeMIMO; 23-3-2d; Updating two Spatial relation or two sets of power control parameters for PUCCH group; Support of updating two Spatial Relation Info’s / two sets of power control parameters for a group of PUCCH resources in a CC by MAC-CE; FG 16-1b-3 FG 23-3-2; Per Band; Optional. |
| OPPO [10] |  |
| Intel Corporation [11] | 23-3-2; Multi-TRP PUCCH repetition; 1. Support of PUCCH repetition scheme 1 (inter-slot repetition) - sequential mapping for any number of repetitions - cyclic mapping for 2 repetitions 2. Support of PUCCH repetition scheme 3 (intra-slot repetition) - sequential mapping for any number of repetitions - cyclic mapping for 2 repetitions 3. Support of cyclic mapping for beam mapping/power control parameter set mapping [when the number of repetitions is larger than 2] for PUCCH repetitions scheme 1 and 4. Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats [1_0 /] 1_1 / 1_2. |
| Apple [12] |  |
| FUTUREWEI [13] |  |
| xiaomi [14] | - On component 3, the following agreement was made in RAN1#104bis-e [5] as bellow. Agreement Confirm the following working assumption (with removing the last bullet): For single DCI based M-TRP PUSCH repetition Type A and B, it is possible to configure either cyclic mapping or sequential mapping of UL beams. - The support of cyclic mapping can be optional UE feature for the cases when the number of repetitions is larger than 2. - FFS: Support of half-half mapping. - FFS: Additional considerations on mapping patterns (including required beam switching gaps) Based on the agreement, the component 3 can be modified as “Support of cyclic mapping for beam mapping/power control parameter set mapping for both PUCCH repetitions scheme 1 and 3 when the number of repetitions is larger than 2”. - On component 4, the following agreement was made in RAN1#106-e [7] as bellow. Agreement For per-TRP closed-loop power control, when the indicated PUCCH transmission in DCI format 1_0 (fallback DCI) is associated with two “closedLoopIndex” values for multi-TRP PUCCH transmission schemes, the single TPC field (the existing TPC field) is applied to both closed loop indices for the scheduled PUCCH. Based on the agreement, DCI format 1_0 can be removed since it is not in line with the above-mentioned agreement. Proposal 10: The proposed FG23-3-2 for M-TRP PUCCH repetition is show as follows. 23. NR_FeMIMO; 23-3-2; Multi-TRP PUCCH repetition; 1. Support of PUCCH repetition scheme 1 (inter-slot repetition) 2. Support of PUCCH repetition scheme 3 (intra-slot repetition) 3. Support of cyclic mapping for beam mapping/power control parameter set mapping [when the number of repetitions is larger than 2] for both PUCCH repetitions scheme 1 and 3 when the number of repetitions is larger than 2 4. Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats [1_0 /]1_1 / 1_2; Optional with capability signalling. |
| LG Electronics [15] |  |
| Lenovo/Motorola Mobility [16] |  |
| Ericsson [17] |  |
| MediaTek Inc. [18] | Many components in FG 23-3-2 are agreed as UE optional features of multi-TRP PUCCH/PUSCH repetitions, so these features should be listed as separate feature groups. Proposal 39: Move Components 2, 3, 4 in FG 23-3-2 into distinct feature groups FG 23-3-2a, FG 23-3-2b, FG 23-3-2c. |
| CMCC [19] | Similar as multi-TRP PUSCH repetition, the support of sequential mapping is a basic capability if the UE supporting multi-TRP PUSCH repetition type A, so “Support of sequential mapping for multi-TRP PUCCH repetition” should be also one of the basic components for FG 23-3-2. Furthermore, the support of cyclic mapping is optional UE feature for the cases when the number of repetitions is larger than 2, so component 3 should be separated from FG 23-3-2. Proposal 4: Add following component in FG 23-3-2 Support of sequential mapping for multi-TRP PUCCH repetition Proposal 5: Separate component 3 from FG 23-3-2. 23. NR_FeMIMO; 23-3-2; Multi-TRP PUCCH repetition Codebook Based; Support of PUCCH repetition scheme 1 (inter-slot repetition); {Supported, Not supported}; Optional with capability signalling. 23. NR_FeMIMO; 23-3-2a; Multi-TRP PUCCH intra-slot repetition; Support of PUCCH repetition scheme 3 (intra-slot repetition); 23-3-2; {Supported, Not supported}; Optional with capability signalling. 23. NR_FeMIMO; 23-3-2b; Multi-TRP PUCCH repetition with cyclic mapping; Support of cyclic mapping for beam mapping/power control parameter set mapping [when the number of repetitions is larger than 2] for PUCCH repetitions scheme 1 and 3 when the number of repetitions is larger than 2; 23-3-2; {Supported, Not supported}; Optional with capability signalling. 23. NR_FeMIMO; 23-3-2c; Multi-TRP PUCCH repetition with second TPC field; Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats [1_0] / 1_1 / 1_2; 23-3-2; {Supported, Not supported}; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [20] |  |

| 23. NR_FeMIMO | 23-4 | IntCell-mTRP | 1. Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info [2. Support of X>1 (max number of PCIs different from serving cell)] [3. Supported max number of RRC-configured PCIs different from serving cell PCI for [FR1/case1] (X1)] [SSB time domain positions or periodicity of additional PCIs is not exactly the same as serving cell PCI)] [4. Supported max number of RRC-configured PCIs different from serving cell PCI for [FR2/case2] (X2)] [SSB time domain positions and periodicity are exactly the same among the additional PCIs and the same as serving cell PCI] [5. default case to be supported, e.g., case2 with X2=1] |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | Inter-cell multi-TRP operation Similar to the discussions on UE feature for <Enhancements on Multi-beam Operation>, we propose the following: Proposal 3-5: Revise component 2/3 in FG 23-4 on RRC-configured PCI(s) different from serving cell PCI as follows and remove component 4/5 - 2. The maximum number of configured additional PCIs is X1 when time domain positions and periodicity of configured SSBs with additional PCIs are the same as time domain positions and periodicity of the serving cell SSBs, with candidate values {0, 1, 2, 3, 4, 5, 6, 7}; - 3. The maximum number of configured additional PCIs is X2 when time domain positions and periodicity of configured SSBs with additional PCIs are different with time domain positions and periodicity of the serving cell SSBs, with candidate values {0, 1, 2, 3, 4, 5, 6, 7}; 23. NR_FeMIMO; 23-4; IntCell-mTRP; 1. Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info 2. The maximum number of configured additional PCIs is X1 when each configuration of SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI; 3. The maximum number of configured additional PCIs is X2 when the configurations of SSB time domain positions and periodicity of the additional PCIs is different with SSB time domain positions and periodicity of the serving cell PCI; [2. Support of X>1 (max number of PCIs different from serving cell)] [3. Supported max number of RRC-configured PCIs different from serving cell PCI for [FR1/case1] (X1)] [SSB time domain positions or periodicity of additional PCIs is not exactly the same as serving cell PCI)] [4. Supported max number of RRC-configured PCIs different from serving cell PCI for [FR2/case2] (X2)] [SSB time domain positions and periodicity are exactly the same among the additional PCIs and the same as serving cell PCI] [5. default case to be supported, e.g., case2 with X2=1]; Optional with capability signalling. |
| ZTE [3] | On component 3 and component 4, the statements of both X1 (case 1) and X2 (case 2) are opposite with the existing agreement in RAN1#106b-e, where - X1 (case 1): SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI - X2 (Case 2): SSB time domain positions and periodicity of the additional PCIs is not according to case 1 Hence the statements of both X1 (case 1) and X2 (case 2) should be revised to be in line with the existing agreement. Besides, we can be general fine with the differentiation of X in FR1 and FR2, but we think it can be distinguished by the column “FR1-FR2 DIFF”, if necessary. Hence we propose to remove the texts “FR1” and “FR2” in component 3 and component 4, respectively. On component 5, it is NOT in line with any agreement in the previous RAN1 meetings so far, hence we propose to remove component 5. Proposal 10: For FG 23-4, - revise the statement of X1 as “SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI” in component 3. - revise the statement of X2 as “SSB time domain positions and periodicity of the additional PCIs is not according to case 1” in component 4. - remove the text “FR1” and “FR2” in component 3 and component 4, respectively. - remove component 5. 23-4; IntCell-mTRP; 1. Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info [2. Support of X>1 (max number of PCIs different from serving cell)] [3. Supported max number of RRC-configured PCIs different from serving cell PCI for [case1] (X1)] [SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI] [4. Supported max number of RRC-configured PCIs different from serving cell PCI for [case2] (X2)] [SSB time domain positions and periodicity of the additional PCIs is not according to case 1] [5. |
| vivo [4] | Following agreements were reached in RAN1#106b Agreement Support two independent X values (X1, X2) are reported as a UE capability for two different assumptions on additional SSB time domain position and periodicity with respect to serving cell SSB. - X1 (Case 1)= The maximum number of configured additional PCIs when each configuration of SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI - X2 (Case 2)= The maximum number of configured additional PCIs when the configurations of SSB time domain positions and periodicity of the additional PCIs is not according to Case 1 - Note: By definition, Case 1 and Case 2 cannot be enabled simultaneously - Supported values for X1 and X2 include at least 0,1,2,3 and 7. FFS on other values - This UE capability has FR1 and FR2 differentiation Following revision on IntCell-mTRP UE feature is proposed: Proposal 5-1: 23. NR_FeMIMO; 23-4; IntCell-mTRP; 1. Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info 2. support of the maximum number for configured additional PCIs for case 1, X1 3. support of the maximum number for configured additional PCIs for case 2, X2 [2. Support of X>1 (max number of PCIs different from serving cell)] [3. Supported max number of RRC-configured PCIs different from serving cell PCI for [FR1/case1] (X1)] [SSB time domain positions or periodicity of additional PCIs is not exactly the same as serving cell PCI)] [4. Supported max number of RRC-configured PCIs different from serving cell PCI for [FR2/case2] (X2)] [SSB time domain positions and periodicity are exactly the same among the additional PCIs and the same as serving cell PCI] [5. default case to be supported, e.g., case2 with X2=1]; Needed of FR1/FR2 differentiation Yes; Note: (Case 1)= The maximum number of configured additional PCIs when each configuration of SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI. (Case 2)= The maximum number of configured additional PCIs when the configurations of SSB time domain positions and periodicity of the additional PCIs is not according to Case 1. By definition, Case 1 and Case 2 cannot be enabled simultaneously; Optional with capability signalling Component1: {support, not support} Component2: {0,1,3,4,7} Component3: {0,1,3,4,7}. |
| CATT [5] |  |
| Samsung [6] | It was agreed to relate X1 to the case where the SSB time domain positions or periodicity of additional PCI(s) are not exactly the same as those of the serving cell PCI, and X2 to the case where the SSB time domain positions or periodicity of additional PCI(s). From our understanding, X1 and X2 are not relevant to FR1 and FR2, respectively. Hence, we suggest to remove “FR1/case1” and “FR2/case2” from components 3 and 4, respectively. Proposal 21: Support to delete “FR1/case1” in the description for Component 3, and “FR2/case2” in the description for Component 4 in FG 23-4. |
| NTT DOCOMO, INC. [7] | For component 2, 3, and 4 which are related to the number of RRC-configured PCIs different from serving cell, component 2 can be deleted and component 3 and 4 can be kept considering the duplication. In addition, based on agreements, component 3 is for case1 and component 4 is for case2, and we think both FR1 and FR2 can be considered for both cases. For component 5, since we have agreed case1 and case2 cannot be enabled simultaneously, it is important to define a default case for inter-cell MTRP operation, otherwise, it is not friendly to deploy this feature in NW. For this default case, we think new component is not needed. Instead, a note is sufficient. Proposal 5: Adopt the following for Rel-17 inter-cell M-TRP. 23. NR_FeMIMO; 23-4; IntCell-mTRP; 1. Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info 2. Supported max number of RRC-configured PCIs different from serving cell PCI for case1 (X1) SSB time domain positions or periodicity of additional PCIs is not exactly the same as serving cell PCI 3. Supported max number of RRC-configured PCIs different from serving cell PCI for case2 (X2) SSB time domain positions and periodicity are exactly the same among the additional PCIs and the same as serving cell PCI Note: default case to be supported, case2 with X2=1; Optional with capability signalling. |
| Spreadtrum Communications [8] |  |
| Qualcomm Incorporated [9] | The following agreement needs to be implemented: Agreement Support two independent X values (X1, X2) are reported as a UE capability for two different assumptions on additional SSB time domain position and periodicity with respect to serving cell SSB. - X1 (Case 1)= The maximum number of configured additional PCIs when each configuration of SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI - X2 (Case 2)= The maximum number of configured additional PCIs when the configurations of SSB time domain positions and periodicity of the additional PCIs is not according to Case 1 - Note: By definition, Case 1 and Case 2 cannot be enabled simultaneously - Supported values for X1 and X2 include at least 0,1,2,3 and 7. FFS on other values - This UE capability has FR1 and FR2 differentiation (FFS : Whether this UE capability is per UE or per band) Proposal 6-1: Adopt the following for Rel-17 inter-cell mTRP UE features (modifications in red). 23. NR_FeMIMO; 23-4; IntCell-mTRP; 1. Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info 2. The maximum number of configured additional PCIs when each configuration of SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI (X1 for Case 1) 3. The maximum number of configured additional PCIs when the configurations of SSB time domain positions and periodicity of the additional PCIs is not according to Case 1 (X2 for Case 2); FG 16-2a; Per Band; Component 2: {0,1,2,3,7} Component 3: {0,1,2,3,7} Note: UE indicating support of this FG indicates a non-zero value for at least one of component 2 or component 3; Optional. |
| OPPO [10] | In the past meeting, two independent X values are agreed as UE capability for two assumptions on SSB time domain position and periodicity with respect to serving cell SSB. The candidate values for X1 and X2 were also discussed including value of 0. From UE perspective, case 2 is optional and X2 can be reported as 0 considering potential complexity for rate-matching. Hence, for UE supporting inter-cell mTRP, X1=1 should be mandatory while X2 can be optional to be reported. For UE without reporting X, the default capability can be {X1=1 and X2=0}. Agreement Support two independent X values (X1, X2) are reported as a UE capability for two different assumptions on additional SSB time domain position and periodicity with respect to serving cell SSB. - X1 (Case 1)= The maximum number of configured additional PCIs when each configuration of SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI - X2 (Case 2)= The maximum number of configured additional PCIs when the configurations of SSB time domain positions and periodicity of the additional PCIs is not according to Case 1 - Note: By definition, Case 1 and Case 2 cannot be enabled simultaneously - Supported values for X1 and X2 includes at least 0,1,2,3 and 7. FFS on other values This UE capability has FR1 and FR2 differentiation (FFS : Whether this UE capability is per UE or per band) Proposal 4: Support {X1=1, X2=0} as the default UE capability for UE supporting inter-cell mTRP. |
| Intel Corporation [11] | Reagrding UE capabilities for inter-cell mTRP we propose to use X1 and X2 as two components with description as agreed in RAN1#106b-e with candidate values [0,1,2,3 and 7] 23-4; Inter cell-mTRP; 1. Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info 2. X1= The maximum number of configured additional PCIs when each configurations of SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI 3. X2 = The maximum number of configured additional PCIs when the configurations of SSB time domain positions and periodicity of the additional PCIs is not according to Case 1 [2. Support of X>1 (max number of PCIs different from serving cell)] [3. Supported max number of RRC-configured PCIs different from serving cell PCI for [FR1/case1] (X1)] [SSB time domain positions or periodicity of additional PCIs is not exactly the same as serving cell PCI)] [4. Supported max number of RRC-configured PCIs different from serving cell PCI for [FR2/case2] (X2)] [SSB time domain positions and periodicity are exactly the same among the additional PCIs and the same as serving cell PCI] [5. default case to be supported, e.g., case2 with X2=1]; Per band. |
| Apple [12] | 23. NR_FeMIMO; 23-4; IntCell-mTRP; 1. Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info including other cell info in RRC TCI state config 2. Supported max number of RRC-configured PCIs different from serving cell PCI for (X1) SSB time domain positions or periodicity of additional PCIs is exactly the same as serving cell PCI) 3. Supported max number of RRC-configured PCIs different from serving cell PCI for (X2) SSB time domain positions and periodicity is not exactly the same among the additional PCIs and the same as serving cell PCI 4. Maximum number of overlapped SSBs in one SSB resource for L1-RSRP measurement; 1. candidate values {Support, Not support} 2. candidate values {0, 1, 2, 3, 7} 3. candidate values {0, 1, 2, 3, 7} 4. candidate values {1, 2, 3, 4, 5, 6, 7, 8}; Optional with capability signalling. |
| FUTUREWEI [13] |  |
| xiaomi [14] | An agreement with respective to the number of additional PCI(s) different from serving cell associated with the TCI state as shown follows [6]: Agreement Support two independent X values (X1, X2) are reported as a UE capability for two different assumptions on additional SSB time domain position and periodicity with respect to serving cell SSB. - X1 (Case 1)= The maximum number of configured additional PCIs when each configuration of SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI - X2 (Case 2)= The maximum number of configured additional PCIs when the configurations of SSB time domain positions and periodicity of the additional PCIs is not according to Case 1 - Note: By definition, Case 1 and Case 2 cannot be enabled simultaneously - Supported values for X1 and X2 includes at least 0,1,2,3 and 7. FFS on other values - This UE capability has FR1 and FR2 differentiation (FFS : Whether this UE capability is per UE or per band) We have already agreed on some candidate values and it should be added to the components 3 and 4 in FG23-4. Proposal 11: Add the agreed candidate values to component 3 and 4 in FG-23-4. Features; Index; Feature group; Components; Note; Mandatory/Optional. 23. NR_FeMIMO; 23-4; IntCell-mTRP; 1. Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info [2. Support of X>1 (max number of PCIs different from serving cell)] [3. Supported max number of RRC-configured PCIs different from serving cell PCI for [FR1/case1] (X1)] [SSB time domain positions or periodicity of additional PCIs is not exactly the same as serving cell PCI), with candidate {01,2,3,7}] [4. Supported max number of RRC-configured PCIs different from serving cell PCI for [FR2/case2] (X2)] [SSB time domain positions and periodicity are exactly the same among the additional PCIs and the same as serving cell PCI, with candidate {01,2,3,7}] [5. default case to be supported, e.g., case2 with X2=1]; Optional with capability signalling. |
| LG Electronics [15] | - Component 3 and 4: X1 and X2 are for case 1 and 2, respectively, but not for FR 1 and FR 2. So, FR 1 and 2 in square brackets should be removed. - Component 2 and 5: these should be removed. Component 3 and 4 are enough. Proposal 7: Adopt the following table for Rel-17 inter-cell M-TRP PDSCH. 23. NR_FeMIMO; 23-4; IntCell-mTRP; 1. Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info 3. Supported max number of RRC-configured PCIs different from serving cell PCI for case1 (X1)] [SSB time domain positions or periodicity of additional PCIs is not exactly the same as serving cell PCI) 4. Supported max number of RRC-configured PCIs different from serving cell PCI for case2 (X2)] [SSB time domain positions and periodicity are exactly the same among the additional PCIs and the same as serving cell PCI; Optional with capability signalling. |
| Lenovo/Motorola Mobility [16] | In RAN1#106bis-e [1] and RAN1#107-e [2], the following preliminary set of UE features related to inter-cell multi-TRP was provided, with a brief version of the table containing the corresponding feature groups provided below: 23-4; IntCell-mTRP; 1. Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info [2. Support of X>1 (max number of PCIs different from serving cell)] [3. Supported max number of RRC-configured PCIs different from serving cell PCI for [FR1/case1] (X1)] [SSB time domain positions or periodicity of additional PCIs is not exactly the same as serving cell PCI)] [4. Supported max number of RRC-configured PCIs different from serving cell PCI for [FR2/case2] (X2)] [SSB time domain positions and periodicity are exactly the same among the additional PCIs and the same as serving cell PCI] [5. Default case to be supported, e.g., case2 with X2=1]; Optional with capability signalling. RAN1 agreed to report 2 independent values, i.e., X1 and X2, for two different assumptions on additional SSB time domain position and periodicity with respect to the serving cell SSB, so we suggest removing the 2nd component of FG 23-4 and remove the brackets of the 3rd and 4th on component of FG 23-4. Remove component 2 and the brackets of component 3 and component 4 of FG 23-4 On the 5th component of FG 23-4 on the default case, since the 1st component has indicated whether inter-cell multi-TRP is supported by the UE, the default case in the 5th component should be the default value of X1 and/or X2 when they are not reported by the UE and the UE indicates that it can support inter-cell multi-TRP operation. Based on the assumption, it can be merged into component 3 and component 4. The default case in component 5 of FG 23-4 should be the default value of X1 and/or X2 when they are not reported by the UE and the UE indicates that it can support inter-cell multi-TRP operation |
| Ericsson [17] | Regarding Component 3 and 4, the carefully drafted formulations in the agreements should be used. - Indication of parameter X1 for Case 1, providing the supported max number of RRC-configured PCIs different from serving cell PCI when each configuration of SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI - Indication of parameter X2 for Case 2, providing the supported max number of RRC-configured PCIs different from serving cell PCI when the configurations of SSB time domain positions and periodicity of the additional PCIs is not according to Case 1 A note can be provided that X2=1 is the default in case RAN2 decided in a signalling structure that requires a default case. Adopt revised formulation proposed in Section 2.2.2 of this contribution for components 3 and 4 of FG 23-4. |
| MediaTek Inc. [18] | We propose to update the description of component 3 and 4 as the current description tend to indicate that these features are applicable to FR1 and FR2 respectively, which is not inline with the WI discussions and definitions. We propose to update the wording such that the FG description. Proposal 40: In order to have more consistent definitions update the definition for FG-23-4 component 3 and 4 to: 3. Supported max number of RRC-configured PCIs different from serving cell PCI for X1 [SSB time domain positions or periodicity of additional PCIs is not exactly the same as serving cell PCI)] 4. Supported max number of RRC-configured PCIs different from serving cell PCI for X2 [SSB time domain positions and periodicity are exactly the same among the additional PCIs and the same as serving cell PCI] 23. NR_FeMIMO; 23-4; IntCell-mTRP; 1. Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info including other cell info in RRC TCI state config [2. Support of X>1 (max number of PCIs different from serving cell)] [3. Supported max number of RRC-configured PCIs different from serving cell PCI for [FR1/case1 (X1)] [SSB time domain positions or periodicity of additional PCIs is not exactly the same as serving cell PCI)] [4. Supported max number of RRC-configured PCIs different from serving cell PCI for [FR2/case2] (X2)] [SSB time domain positions and periodicity are exactly the same among the additional PCIs and the same as serving cell P[5. default case to be supported, e.g., case2 with X2=1]; Optional with capability signalling. |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] |  |

| 23. NR_FeMIMO | 23-5-1 | Group based L1-RSRP reporting enhancements | 1. Support a single L1-RSRP reporting instance consisting of N beam groups and M=2 beams per [pair/group] [in one report instance] 2. [Support of enhanced group-based reporting for Rel-17 M-TRP] 3. [Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs] 4. [Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs] 5. [Support beam measurement on two CMR resource sets] 6. [Support of enhanced group-based reporting for Rel-17 intra-cell and inter-cell mTRP] |  |  |  |  |  |  |  |  | Candidate values: {1,2,3,4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | Beam Management for multi-TRP With the latest draft CRs for Rel-17 FeMIMO, some configurations for Rel-15/16 beam measurement/reporting/failure-recovery are reused for beam measurement/reporting/failure-recovery for multi-TRP scenario in Rel-17. In this case, it is necessary to clarify that the consumed UE processing capability for multi-TRP scenario in Rel-17 are included in the reported UE capability in FG 16-1g/16-1g-1 (accounting for all beam-related measurements). In this way, component 3/4 in FG 23-5-1 and component 3/4/5 in FG 23-5-2 can be removed. We propose the following: Proposal 3-6: Clarify that the supported number of RSs for beam measurement/reporting/failure-recovery for multi-TRP scenario in Rel-17 are included in FG 16-1g/16-1g-1, and remove component 3/4 in FG 23-5-1 and component 3/4/5 in FG 23-5-2. 23. NR_FeMIMO; 23-5-1; Group based L1-RSRP reporting enhancements; 1. Support a single L1-RSRP reporting instance consisting of N beam groups and M=2 beams per [pair/group] [in one report instance] 2. [Support of enhanced group-based reporting for Rel-17 M-TRP] 3. [Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs] 4. [Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs] 5. [Support beam measurement on two CMR resource sets] 6. [Support of enhanced group-based reporting for Rel-17 intra-cell and inter-cell mTRP]; Candidate values: {1,2,3,4}; Optional with capability signalling. |
| ZTE [3] | In our views, the explicit sub-features of supporting Rel-17 group-based reporting and supporting Rel-17 mTRP BFR should be introduced for [23-5-1] and [23-5-2], respectively. - Regarding group based report [23-5-1] - Regarding #. RS for both CMR sets (component-3/4), we are open to have some further discussion. - Regarding scope of group based reporting (component-2/5), we think that it should be supported for mTRP and should have 2 resource sets that can be merged with first component. - Then, for inter-cell mTRP, in RAN1#107, we have a conclusion that on Rel-17 enhancements for inter-cell beam management and inter-cell mTRP, in Rel-17, there is no consensus on supporting event-driven beam reporting. Based on that, the component-6 should be removed. 23-5-1; Group based L1-RSRP reporting enhancements; 1. Support a single L1-RSRP reporting instance consisting of N beam groups and M=2 beams per [pair/group] [in one report instance], based on measurement on two CMR resource sets. 2. [Support of enhanced group-based reporting for Rel-17 M-TRP] 3. [Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs] 4. [Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs] 5. [Support beam measurement on two CMR resource sets] 6. [Support of enhanced group-based reporting for Rel-17 intra-cell and inter-cell mTRP]. |
| vivo [4] |  |
| CATT [5] | Component 2 and 5 are basic features of group based L1-RSRP reporting enhancemetns, and they can be merged to Component 1. For component 3 and 4, the maximum number of (configured) SSB and CSI-RS resources shall depend on UE capability. Therefore, we propose to remove brackets in these two components. With regard to component 6, whethter enhanced group-based reporting can be applied for inter-cell mTRP scenario needs further discussion. Since inter-cell TRP is an important feature in Rel-17, and extending enhanced group-basd reporting framework to inter-cell TRP is technically feasible, we propose to support enhanced group-based reporting for both Rel-17 intra-cell and inter-cell mTRP. Proposal-13: Feature group 23-5-1 can be revised as follows: 23-5-1; Group based L1-RSRP reporting enhancements; 1. Support a single L1-RSRP reporting instance consisting of N beam groups and M=2 beams per [pair/group] [in one report instance] based on two CMR resource sets 2. [Support of enhanced group-based reporting for Rel-17 M-TRP] 3. [Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs] 4. [Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs] 5. [Support beam measurement on two CMR resource sets] 6. [Support of enhanced group-based reporting for Rel-17 intra-cell and inter-cell mTRP]; Candidate values: {1,2,3,4}; Optional with capability signalling. |
| Samsung [6] | For component 6 in FG 23-5-1, group-based beam reporting enhancement(s) was not discussed for inter-cell mTRP; we suggest to remove inter-cell from component 6. Furthermore, the bracket on [in one report instance] should be removed, which is the essence of group based reporting. Proposal 22: Support FG 23-5-1 in principle; support to remove [inter-cell] from Component 6 in FG 23-5-1; support to remove the bracket on [in one report instance] in Component 1 in FG 23-5-1. |
| NTT DOCOMO, INC. [7] | For 23-5-1, for component 1, to align the wording, it is suggested to keep ‘group’ and delete the ‘[pair/][in one reporting instance]’. For other components, since component 2 and 5 are naturally supported for group-based L1-RSRP beam reporting, separate components are not needed and can be removed, while component 3, 4, and 6 can be kept. 23. NR_FeMIMO; 23-5-1; Group based L1-RSRP reporting enhancements; 1. Support a single L1-RSRP reporting instance consisting of N beam groups and M=2 beams per group 2. Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs 3. Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs 4. Support of enhanced group-based reporting for Rel-17 intra-cell and inter-cell mTRP; Candidate values: {1,2,3,4}; Optional with capability signalling. |
| Spreadtrum Communications [8] | We suggest to remove the brackets in component 1, 3, and 4, to reflect the agreements we have achieved. Component 2 should be removed, for the reason that FG 23-5-1 itself is targeting the function. Component 6 should be removed. In Rel-17 for the topic: BM enhancement for M-TRP, only intra-cell is assumed. Proposal 3: For FG 23-5-1, suggest to remove the brackets in component 1, 3 and 4. Proposal 4: For FG 23-5-1, suggest to delete component 2 and 6. |
| Qualcomm Incorporated [9] |  |
| OPPO [10] | For component 2: it shall be removed because enhanced group-based beam reporting was not designed for inter-cell mTRP. Components 3 and 4 shall be kept because they are about the number of CMR resources that the UE is capable of measuring. For component 5: we prefer to remove it since configuring two CMR sets is one basic component for enhanced group-based beam reporting in rel17. For component 6: we prefer to remove it for the same reason as component 2. Proposal 5: On FG23-5-1: - Remove components 2 and 6. - Keep components 3 and 4. - Remove component 5. |
| Intel Corporation [11] | Reagrding UE capabilities for mTRP beam management. We propose the following modifications to the FG description - for FG 23-5-1, Component 1: [in one report instance] is not needed as it is repetitive - for FG 23-5-1, Component 2: Not clear what exactly is intended in this bullet - for FG 23-5-1, Component 3, 4: We think one of them is sufficient (merge) - for FG 23-5-1, Component 5: We think this is part of basic UE FG - for FG 23-5-1, Component 6: Clarify that intra-cell is part of basic FG |
| Apple [12] | 23. NR_FeMIMO; 23-5-1; Group based L1-RSRP Beam reporting enhancements; - Support a single L1-RSRP reporting instance consisting of N of the maximum number of beam groups and M=2 beams per [pair/group] [in one report instance] - Support of enhanced group-based reporting for Rel-17 M-TRP - Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs - Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs - Support of enhanced group-based reporting for Rel-17 intra-cell and inter-cell mTRP; 1. Candidate values: {1,2,3,4} 2. Candidate values: {Support, not support} 3. Candidate values: {1,2,3,4} 4. Candidate values: {8, 16, 24, 32, 48, 64} 5. Candidate values: {intra-cell mTRP, inter-cell mTRP, intra-cell+inter-cell mTRP}; Optional with capability signalling. |
| FUTUREWEI [13] |  |
| xiaomi [14] | For component 1 in FG23-5-1, it is better to update it to - Support a single L1-RSRP reporting instance consisting of N beam groups and M=2 beams per [pair/group] [in one report instance] Proposal 12: Update component 1 in FG23-5-1 as below. - Support a single L1-RSRP reporting instance consisting of N beam groups and M=2 beams per [pair/group] [in one report instance] |
| LG Electronics [15] | - Component 1, 2 and 5: These are the basic feature of this FG, so component 2 and 5 can be removed and merged to component 1. We could add ‘from two CMR resource sets’ in component 1 for better clarity. - Component 6: this should be removed since there is no relevant agreement. Proposal 8: Adopt the following table for Rel-17 M-TRP beam management. 23. NR_FeMIMO; 23-5-1; Group based L1-RSRP reporting enhancements; 1. Support a single L1-RSRP reporting instance consisting of N beam groups and M=2 beams per [pair/group] from two CMR resource sets [in one report instance] 3. [Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs] 4. [Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs]; Candidate values: {1,2,3,4}; Optional with capability signalling. |
| Lenovo/Motorola Mobility [16] | In RAN1#106bis-e [1] and RAN1#107-e [2], the following preliminary set of UE features related to beam management for multi-TRP was provided, with a brief version of the table containing the corresponding feature groups provided below: 23-5-1; Group based L1-RSRP reporting enhancements; 1. Support a single L1-RSRP reporting instance consisting of N beam groups and M=2 beams per [pair/group] [in one report instance] 2. [Support of enhanced group-based reporting for Rel-17 M-TRP] 3. [Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs] 4. [Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs] 5. [Support beam measurement on two CMR resource sets] 6. [Support of enhanced group-based reporting for Rel-17 intra-cell and inter-cell mTRP]; Candidate values: {1,2,3,4}; Optional with capability signalling. On group-based beam reporting of FG 23-5-1, component 2 implies whether beam measurement on both CMR sets is supported. If the UE report to support enhanced group-based reporting for Rel-17 M-TRP, beam measurement on two CMR resource sets can be supported as well. In RAN1-107e meeting, RAN1 has come to the following conclusion: On Rel-17 enhancements for inter-cell beam management and inter-cell mTRP , in Rel-17, there is no consensus that the agreed L1-RSRP measurement/reporting also includes group-based beam report for inter-cell mTRP. Based on that, group-based beam reporting for inter-cell mTRP is not supported, which makes component 5 and component 6 invalid. Therefore, both component 5 and component 6 of FG 23-5-1 should be removed. Remove component 5 and component 6 of FG 23-5-1 |
| Ericsson [17] | Regarding Component 2, this is more of a description of the whole feature rather than a useful component. On Component 5, two CMR resource sets are already captured in 38.214 and is needed to operate this feature. So, we do not see the need to capture this as a component in this FG. Remove Components 2 and 5 from FG 23-5-1. |
| MediaTek Inc. [18] | On component #1, current wording is a bit confused, thus we suggest the following update. Proposal 41: Update component #1 in FG 23-5-1 as follows: - The maximum number of beam groups (M=2 beams per beam group) in one L1-RSRP reporting instance - Component 1: Candidate values: {1,2,3,4} On component #2, this is the same as component #1, we suggest to remove. Proposal 42: Remove component #2 in FG 23-5-1 On components #3~#5, we support and can further discuss the details. Proposal 43: Support components #3~#5 in FG 23-5-1, further discuss the details. On component #6, according to the following RAN1 conclusion, we suggest to remove this component. Conclusion from RAN1#107 On Rel-17 enhancements for inter-cell beam management and inter-cell mTRP, in Rel-17, there is no consensus that the agreed L1-RSRP measurement/reporting also includes group-based beam report for inter-cell mTRP. Proposal 44: Remove component #6 in FG 23-5-1 23. NR_FeMIMO; 23-5-1; Group based L1-RSRP Beam reporting enhancements; - The maximum number of beam groups (M=2 beams per beam group) in one L1-RSRP reporting instance Support a single L1-RSRP reporting instance consisting of N of the maximum number of beam groups and M=2 beams per [pair/group] [in one report instance] - [Support of enhanced group-based reporting for Rel-17 M-TRP] - [Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs] - [Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs] - [Support beam measurement on two CMR resource sets] - [Support of enhanced group-based reporting for Rel-17 intra-cell and inter-cell mTRP]; Componet 1: Candidate values: {1,2,3,4}; Optional with capability signalling. |
| CMCC [19] | For FG 23-5-1, we think components 2 and 5 need to be supported for M-TRP group based L1-RSRP reporting, hence we propose to remove components 2 and 5. For MTRP BFR, we have the following agreements. Agreement(RAN1#104) For M-TRP BFR - Support 2 BFD-RS sets per BWP, and up to N resources per BFD-RS set - FFS: value of N (e.g. fixed in specification, or UE capability) - FFS: number of BFD RSs across all BFD-RS sets per DL BWP (e.g. fixed maximum value or UE capability) Agreement(RAN1#106) The maximum number of BFD-RS resources per set is a UE capability, including a possible candidate value of 1 in Rel.17. Based on the above agreement, we think component 2 should be supported for MTRP BFR, hence it can be removed. For component 1, it can be revised as “Support of the maximum number of BFD-RS resources per set per BWP”. Proposal 6: Remove components 2 and 5 for FG 23-5-1. |
| Nokia/Nokia Shanghai Bell [20] | - Component 1 and 5 essential components. - Component 2 is redundant or equivalent to component 1+5. - Components 3, 4, and 6 are OK. |

| 23. NR_FeMIMO | 23-5-2 | MTRP BFR enhancements | 1. Support of the maximum number of BFD-RS resources per set 2. [Support of Rel-17 M-TRP BFR based on two BFD-RS sets] 3. [Maximum number of CSI-RS resources of both BFR-RS sets across all CCs] 4. [Maximal number of different SSBs of both BFD-RS sets across all CCs] 5. [Maximal number of different CSI-RS and/or SSB resources of both NBI-RS sets across all CCs] 6. [Support up to two PUCCH-SR resources for MTRP BFRQ] 7. [Supported maximum number of BFD-RS resources across two BFD-RS sets per BWP] 8. [Support of [single DCI/multi-DCI] based BFR enhancement] |  |  |  |  |  |  |  |  | [Candidate values: {1, 2,…}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | Beam Management for multi-TRP With the latest draft CRs for Rel-17 FeMIMO, some configurations for Rel-15/16 beam measurement/reporting/failure-recovery are reused for beam measurement/reporting/failure-recovery for multi-TRP scenario in Rel-17. In this case, it is necessary to clarify that the consumed UE processing capability for multi-TRP scenario in Rel-17 are included in the reported UE capability in FG 16-1g/16-1g-1 (accounting for all beam-related measurements). In this way, component 3/4 in FG 23-5-1 and component 3/4/5 in FG 23-5-2 can be removed. We propose the following: Proposal 3-6: Clarify that the supported number of RSs for beam measurement/reporting/failure-recovery for multi-TRP scenario in Rel-17 are included in FG 16-1g/16-1g-1, and remove component 3/4 in FG 23-5-1 and component 3/4/5 in FG 23-5-2. To reflect UE processing capability on number of CCs with beam failure recovery running, there needs be to a UE feature on maximum number of CCs on which spCell/SCell/MTRP BFR in Rel-15/16/17 can be configured. We then propose the following: Proposal 3-7: Add a new component in FG 23-5-2 as follows - 9. The maximum number of CCs configured with BFR (including spCell/SCell/MTRP BFR in Rel-15/16/17), with candidate values {1, 2, 3, 4, 5, 6, 7, 8, 9} 23. NR_FeMIMO; 23-5-2; MTRP BFR enhancements; 1. Support of the maximum number of BFD-RS resources per set 2. [Support of Rel-17 M-TRP BFR based on two BFD-RS sets] 3. [Maximum number of CSI-RS resources of both BFR-RS sets across all CCs] 4. [Maximal number of different SSBs of both BFD-RS sets across all CCs] 5. [Maximal number of different CSI-RS and/or SSB resources of both NBI-RS sets across all CCs] 6. [Support up to two PUCCH-SR resources for MTRP BFRQ] 7. [Supported maximum number of BFD-RS resources across two BFD-RS sets per BWP] 8. [Support of [single DCI/multi-DCI] based BFR enhancement] 9. The maximum number of CCs configured with BFR (including spCell/SCell/MTRP BFR in Rel-15/16/17); [Candidate values: {1, 2,…}] 9. Candidate values: {1, 2, 3, 4, 5, 6, 7, 8, 9}; Optional with capability signalling. |
| ZTE [3] | In our views, the explicit sub-features of supporting Rel-17 group-based reporting and supporting Rel-17 mTRP BFR should be introduced for [23-5-1] and [23-5-2], respectively. - Regarding MTRP BFR enhancement [23-5-2] - For mTRP-BFR, 2 BFD-RS sets should be supported, and so we do not think that we need the component-2, and for clarification, we can refine the component-1 accordingly. - Regarding Component 3/4/5, we are open to have some further discussion, but we do not think that Component-7 is needed under Component-1. - Regarding Component 6, it is needed based on the already RAN1 agreement. - Regarding Component 8, it seems that only m-DCI mTRP BFR is complete, and we are afraid that s-DCI mTRP BFR may be precluded in Rel-17 mTRP BFR. 23-5-2; MTRP BFR enhancements; 1. Support of the maximum number of BFD-RS resources per set for two BFD-RS sets 2. [Support of Rel-17 M-TRP BFR based on two BFD-RS sets] 3. [Maximum number of CSI-RS resources of both BFR-RS sets across all CCs] 4. [Maximal number of different SSBs of both BFD-RS sets across all CCs] 5. [Maximal number of different CSI-RS and/or SSB resources of both NBI-RS sets across all CCs] 6. [Support up to two PUCCH-SR resources for MTRP BFRQ] 7. [Supported maximum number of BFD-RS resources across two BFD-RS sets per BWP] 8. [Support of [single DCI/multi-DCI] based BFR enhancement]. |
| vivo [4] | Agreement Support to configure an association between a BFD-RS set on SpCell and a PUCCH-SR resource / SR configuration for per TRP BFR. - FFS: Configure an association between a BFD-RS set on SCell and a PUCCH-SR resource / SR configuration for per TRP BFR A UE capability signaling is introduced for indicating the support of this association. Above applies only for multi-DCI case.. As mentioned in the agreement achieved in RAN1#106bis-e meeting, the association between PUCCH-SR resource/SR configuration and BFD-RS set is a new UE feature, which should be captured in the list. As for components other than component 1, we support remove them, since they either must have to make it work, or already captured in existing FGs, e.g. R16 FGs. Proposal 6-1: support to update FG 23-5-2 as following: 23. NR_FeMIMO; 23-5-2; Maximum number of BFD-RS resources per set for MTRP BFR enhancements; - Support of the maximum number of BFD-RS resources per set - Support to configure an association between a BFD-RS set on SpCell and a PUCCH-SR resource / SR configuration for per TRP BFR in multi-DCI case; - Candidate values: {1, [2,…]}; Optional with capability signalling. |
| CATT [5] | For component 2, 6 and 7, the following agreements related to the number of BFD-RS sets (component 2), the maximum number of BFD-RS resources across two BFD-RS sets per BWP (component 7), and the number of PUCCH-SR resources (component 6) were achieved in RAN1#104-e, RAN1 #104b-e and RAN1#104-e, respectively. Agreement For M-TRP BFR - Support 2 BFD-RS sets per BWP, and up to N resources per BFD-RS set - FFS: value of N (e.g. fixed in specification, or UE capability) - FFS: number of BFD RSs across all BFD-RS sets per DL BWP (e.g. fixed maximum value or UE capability) Agreement On BFD-RS of TRP-specific BFR - BFD-RS resource number: - The total number of RSs in two BFR-RS sets per DL BWP is a UE capability - On the maximum number of RS per BFD-RS set, down-select from the following two alternatives in RAN1#105-e - Alt1: max value is 2 - Alt2: max value is a UE capability, including possible candidate value of 1 Agreement For BFRQ of M-TRP BFR - Option 3: Up to two dedicated PUCCH-SR resources in a cell group - FFS: Whether PUCCH-SR for SCell can be reused for M-TRP - Support BFRQ MAC-CE that can convey information of failed CC indices, one new candidate beam for the failed TRP/CC (if found), and whether new candidate beam is found - Support at least indication of a single TRP failure - FFS: whether/what information of failed TRP(s) is conveyed in the MAC-CE - FFS: whether/how to support indication of more than one TRP failure, corresponding BFR procedure, and applicable cell type (SCell vs. SpCell) - FFS: UE behavior when TRP failure status is different across cells - FFS: Whether PUCCH SR resource can be configured with 2 spatial relations. Since these three components have already been agreed, we propose to remove brackets in component 2, 6 and 7. For component 8, although implicit BFD-RS set configuration for single DCI mTRP is not supported in Rel-17, explicit BFD-RS set configuration can be supported. Therefore, brackets in component 8 can also be removed. With regard to component 3, 4 and 5, further discussion is needed. Proposal-14: Feature group 23-5-2 can be revised as follows: 23-5-2; MTRP BFR enhancements; 1. Support of the maximum number of BFD-RS resources per set 2. [Support of Rel-17 M-TRP BFR based on two BFD-RS sets] 3. [Maximum number of CSI-RS resources of both BFR-RS sets across all CCs] 4. [Maximal number of different SSBs of both BFD-RS sets across all CCs] 5. [Maximal number of different CSI-RS and/or SSB resources of both NBI-RS sets across all CCs] 6. [Support up to two PUCCH-SR resources for MTRP BFRQ] 7. [Supported maximum number of BFD-RS resources across two BFD-RS sets per BWP] 8. [Support of [single DCI/multi-DCI] based BFR enhancement]; [Candidate values: {1, 2,…}]; Optional with capability signalling. |
| Samsung [6] | Proposal 23: Support FG 23-5-2 for MTRP BFR enhancements. |
| NTT DOCOMO, INC. [7] | For23-5-2. Component 2 is also naturally supported and separate component is not needed. To make the description more complete, ‘two BFD-RS sets’ can be added into component 1. In component 8, it is important to keep both single DCI and multi-DCI as both cases are supported for MTRP BFR. In addition, based on latest agreement, a new component should be added for support of an association configuration between a BFD-RS set on SpCell and a PUCCH-SR resource / SR configuration for multi-DCI based MTRP BFR. 23. NR_FeMIMO; 23-5-2; MTRP BFR enhancements; 1. Support of the maximum number of BFD-RS resources per set, for the two BFD-RS sets 2. Maximum number of CSI-RS resources of both BFR-RS sets across all CCs 3. Maximal number of different SSBs of both BFD-RS sets across all CCs 4. Maximal number of different CSI-RS and/or SSB resources of both NBI-RS sets across all CCs 5. Support up to two PUCCH-SR resources for MTRP BFRQ 6. Supported maximum number of BFD-RS resources across two BFD-RS sets per BWP 7. Support of single DCI/multi-DCI based BFR enhancement 8. support of an association configuration between a BFD-RS set on SpCell and a PUCCH-SR resource / SR configuration for multi-DCI based MTRP BFR; Candidate values: {1, 2,…}; Optional with capability signalling. |
| Spreadtrum Communications [8] |  |
| Qualcomm Incorporated [9] | For FG 23-5-2, we have the following proposals, where the 2nd proposal is based on the agreement below Proposal 7-1: Introduce UE capability on maximum number of configured NBI-RS resources per NBI-RS set - This can be a new component in FG 23-5-2 Proposal 7-2: Introduce UE capability on support of a configured association between a BFD-RS set on SpCell and a PUCCH-SR resource / SR configuration for per-TRP BFR - This can be a new component in FG 23-5-2 Agreement Support to configure an association between a BFD-RS set on SpCell and a PUCCH-SR resource / SR configuration for per TRP BFR. FFS: Configure an association between a BFD-RS set on SCell and a PUCCH-SR resource / SR configuration for per TRP BFR A UE capability signaling is introduced for indicating the support of this association. Above applies only for multi-DCI case |
| OPPO [10] | For component 2: it shall be removed because the basic function of FG 23-5-2 is per-TRP BFR. For components 3, 4, 5 and 7: we are fine to keep them. For component 6: the current description is not accurate and we do not think it is needed too. If it is kept, we suggest to change it to “6. The maximum number SR configuration for MTRP BFR”. For component 8: it is not needed. Proposal 6: On FG23-5-2: - Keep components 3/4/5/7. - Remove component 2 and 8. - For component 6: remove it or change it to “the maximum number of SR configurations for MTRP BFR” |
| Intel Corporation [11] | - for FG 23-5-2, Component 2: This should be part of basic FG 23-5-2; MTRP BFR enhancements; - Support of the maximum number of BFD-RS resources per set - [Support of Rel-17 M-TRP BFR based on two BFD-RS sets] - [Maximum number of CSI-RS resources of both BFR-RS sets across all CCs] - [Maximal number of different SSBs of both BFD-RS sets across all CCs] - [Maximal number of different CSI-RS and/or SSB resources of both NBI-RS sets across all CCs] - [Support up to two PUCCH-SR resources for MTRP BFRQ] - [Supported maximum number of BFD-RS resources across two BFD-RS sets per BWP] - [Support of [single DCI/multi-DCI] based BFR enhancement]; Per band. |
| Apple [12] | 23. NR_FeMIMO; 23-5-2; Maximum number of BFD-RS resources per set for MTRP BFR enhancements; - Support of the maximum number of BFD-RS resources per set - Maximum number of CSI-RS resources of both BFD-RS sets across all CCs - Maximal number of different CSI-RS and/or SSB resources of both NBI-RS sets across all CCs - Support number of PUCCH-SR resources for MTRP BFRQ - Supported maximum number of BFD-RS resources across two BFD-RS sets per BWP - Support of [single DCI/multi-DCI] based BFR enhancement - Support of association between PUCCH-SR and BFD RS set; 1. Candidate values: {1, 2} 2. Candidate values: {1, 2, 3, 4} 3. Candidate values: {8, 16, 24, 32, 48, 64} 4. Candidate values: {0, 1, 2} 5. Candidate values: {1, 2, 3, 4} 6. Candidate values: {single-DCI, multi-DCI, both} 7. Candidate values: {Support, not support}; Optional with capability signalling. |
| FUTUREWEI [13] |  |
| xiaomi [14] | As for the component 1 in FG23-5-2, “Support of the maximum number of BFD-RS resources per set”, according to the agreement in RAN1-106 e-meeting [7] listed below, the candidate value can be ‘1’. Since for cell specific BFR-set, the maximum number of BFD-RS resources per set can be ‘2’, it is reasonable to support ‘2’ as well. While for the candidate value larger than ‘2’, we don’t have essential reasons to support it. RAN1-106 e-meeting-Agreement The maximum number of BFD-RS resources per set is a UE capability, including a possible candidate value of 1 in Rel.17. While for the component 7 in FG23-5-2, “Supported maximum number of BFD-RS resources across two BFD-RS sets per BWP”, we support it in principle according to the agreement in RAN1-104b e-meeting [5]. Since the maximum number of BFD-RS resources across two BFD-RS sets per BWP is {1, 2}, the candidate value of “Support of the maximum number of BFD-RS resources across two BFD-RS sets per BWP” can be {2, 3, 4}. RAN1-104b e-meeting-Agreement On BFD-RS of TRP-specific BFR - BFD-RS resource number: - The total number of RSs in two BFR-RS sets per DL BWP is a UE capability - On the maximum number of RS per BFD-RS set, down-select from the following two alternatives in RAN1#105-e - Alt1: max value is 2 - Alt2: max value is a UE capability, including possible candidate value of 1 Proposal 13: Support component 1 and component 7 in FG23-5-2. - Support component 1 “Support of the maximum number of BFD-RS resources per set” with candidate value {1, 2} only. - Support component 7 by updating to “Support of the maximum number of BFD-RS resources across two BFD-RS sets per BWP” with candidate value {2, 3, 4}. In addition, according to the agreement in RAN1-106b e-meeting [6] listed as below, A UE capability signaling is introduced for indicating the support of the association between a BFD-RS set on SpCell and a PUCCH-SR resource / SR configuration for per TRP BFR. Thus we propose to add one more component into FG23-5-2. RAN1-106b e-meeting-Agreement Support to configure an association between a BFD-RS set on SpCell and a PUCCH-SR resource / SR configuration for per TRP BFR. - FFS: Configure an association between a BFD-RS set on SCell and a PUCCH-SR resource / SR configuration for per TRP BFR A UE capability signaling is introduced for indicating the support of this association. Above applies only for multi-DCI case. Proposal 14: Add component 8 into FG23-5-2, - Component 8, support of the association between a BFD-RS set on SpCell and a PUCCH-SR resource / SR configuration for per TRP BFR. |
| LG Electronics [15] | - Component 3, 4 and 7: Component 7 is agreed in RAN1#104e, so this should be supported. Component 3 and 4 seems not needed since component 1 is sufficient according to the agreement. Agreement (104e) On BFD-RS of TRP-specific BFR - BFD-RS resource number: - The total number of RSs in two BFR-RS sets per DL BWP is a UE capability - On the maximum number of RS per BFD-RS set, down-select from the following two alternatives in RAN1#105-e - Alt1: max value is 2 - Alt2: max value is a UE capability, including possible candidate value of 1 Agreement (106e) The maximum number of BFD-RS resources per set is a UE capability, including a possible candidate value of 1 in Rel.17. Proposal 8: Adopt the following table for Rel-17 M-TRP beam management. 23. NR_FeMIMO; 23-5-2; MTRP BFR enhancements; 1. Support of the maximum number of BFD-RS resources per set 2. [Support of Rel-17 M-TRP BFR based on two BFD-RS sets] 5. [Maximal number of different CSI-RS and/or SSB resources of both NBI-RS sets across all CCs] 6. [Support up to two PUCCH-SR resources for MTRP BFRQ] 7. Supported maximum number of BFD-RS resources across two BFD-RS sets per BWP 8. [Support of [single DCI/multi-DCI] based BFR enhancement]; [Candidate values: {1, 2,…}]; Optional with capability signalling. |
| Lenovo/Motorola Mobility [16] |  |
| Ericsson [17] |  |
| MediaTek Inc. [18] | On components #1, #2, #6 and #7, we are supportive since they are already captured in the RAN1 agreements, and we suggest the following candidate values for components #1 and #7. Proposal 45: Support components #1, #2, #6 and #7 in FG 23-5-2 Proposal 46: Support component #1 in FG 23-5-2 with the candidate values {1,2} Proposal 47: Support component #7 in FG 23-5-2 with the candidate values {2,3,4} On components #3~#5, we are fine in principle, but the details need to be further discussed. On component #8, we suggest to remove this component since we don't see UE implementations for S-DCI and M-DCI MTRP BFR need to be different. Furthermore, S-DCI may not work in Rel-17 MTRP BFR since it still unclear to configure BFD-RS for S-DCI case. Proposal 48: Support to remove component #8 from FG 23-5-2 23. NR_FeMIMO; 23-5-2; Maximum number of BFD-RS resources per set for MTRP BFR enhancements; - Support of the maximum number of BFD-RS resources per set - [Support of Rel-17 M-TRP BFR based on two BFD-RS sets] - [Maximum number of CSI-RS resources of both BFR-RS sets across all CCs] - [Maximal number of different SSBs of both BFD-RS sets across all CCs] - [Maximal number of different CSI-RS and/or SSB resources of both NBI-RS sets across all CCs] - [Support up to two PUCCH-SR resources for MTRP BFRQ] - [Supported maximum number of BFD-RS resources across two BFD-RS sets per BWP] - [Support of [single DCI/multi-DCI] based BFR enhancement]; Component 1: [Candidate values: {1, 2}] Component 7: {2, 3, 4}; Optional with capability signalling. |
| CMCC [19] | Proposal 7: Remove component 2 for FG 23-5-2. Proposal 8: Revise component 1 for FG 23-5-2 as “Support of the maximum number of BFD-RS resources per set per BWP”. |
| Nokia/Nokia Shanghai Bell [20] | Component 8 is a key feature for multi-DCI case. Single-DCI can be captured as a separate FG. |

| 23. NR_FeMIMO | 23-6-1 | SFN scheme A (scheme 1) | 1. [Support of scheme A for PDCCH] [and default QCL assumption with two TCI states for PDCCH] 2. Support of scheme A for PDSCH [scheduled by [single TRP/ Scheme A] PDCCH] [and default QCL assumption with two TCI states for PDSCH] |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| ZTE [3] |  |
| vivo [4] |  |
| CATT [5] | For SFN based HST enhancements, several FGs (23-6-1 ~ 23-6-4) are listed in [1]. Acoording to the agreements in previous meetings, we have the following comments for these FGs. 23-6-1/2 Scheme 1 / TRP based pre-compensation for PDSCH and PDCCH Firstly, based on agreement in RAN1#107e meeting, TRP-based pre-compensation scheme for PDSCH / PDCCH is supported in both FR1 and FR2 with UE capability at least per FR. So our suggestion is to delete the restriction of FR1 in the current UE feature list in [1]. Sencondly, if dynamic switching between single-TRP and SFN PDSCH scheme A/B can be listed separately, the UE features 23-6-1 should be prerequisite feature for 23-6-1a and the UE features 23-6-2 should be prerequisite feature for 23-6-2a. Thridly, based on the agreement in RAN1#107e meeting, the two default beam(s) for PDSCH/PDCCH is determined by the different combinations of transmission scheme for PDSCH and PDCCH, so we support that if UE supports scheme A for PDCCH, the UE can also support default beam(s) with two TCI states for PDCCH. Finally, based on the agreement in RAN1#106e and #106bis-e meeting, the following combinations of the transmission schemes was supported: - Single-TRP PDCCH + Rel-17 Scheme 1 PDSCH - Single-TRP PDCCH + Rel-17 TRP-based pre-compensation PDSCH - Rel-17 scheme 1 PDCCH + single-TRP PDSCH with UE capability It can be seen that only the combination of Rel-17 scheme 1 PDCCH and single-TRP PDSCH is supported with UE capability. So in 23-6-1 for PDSCH, the second component should be only discussed in this combination scheme. Futhermore, in 23-6-2 for PDSCH, whether Rel-17 TRP-based pre-compensation PDCCH can schedule single-TRP PDSCH should be futher discussed in AI 8.1.2.4. Based on the above analysis, we have the following proposal: Proposal-15: For Scheme 1 / TRP based pre-compensation for PDSCH and PDCCH, the UE features 23-6-1 ~ 23-6-2 are revised as follows: 23-6-1; SFN scheme A (scheme 1); 1. [Support of scheme A for PDCCH] [and default QCL assumption with two TCI states for PDCCH] 2. Support of scheme A for PDSCH [scheduled by [single TRP/ Scheme A] PDCCH] [and default QCL assumption with two TCI states for PDSCH]; Optional with capability signalling. |
| Samsung [6] |  |
| NTT DOCOMO, INC. [7] | - During the RAN1 discussion for Rel.17 HST, the assumption of the basic operation is a combination of “1) SFN-PDCCH + SFN-PDSCH”, and some UE vendors claim difficulty of supporting other combinations. Hence, the basic capability of FG23-6-1 and FG23-6-2 should be the combination of “1) SFN-PDCCH + SFN-PDSCH”, and other combinations should be reported in separate components. The other combinations are “2) S-TRP PDCCH + SFN-PDSCH” and “3) SFN-PDCCH + S-TRP PDSCH”. Note that 3) is only supported in SFN scheme A based on the previous agreements. - The most of function between FR23-6-1 and FG23-6-2 are common, and hence we agree FG23-6-1 is pre-requisite feature of FG23-6-2. - We believe FG23-6-1 and FG23-6-2 should be reported per band. - For FG23-6-2, SFN scheme B is supported in both FR1 and FR2 by RAN1#107e agreement. Hence, “[FR1 only]” should be deleted. 23. NR_FeMIMO; 23-6-1; SFN scheme A (scheme 1); 1. Support of scheme A for PDSCH scheduled by scheme A PDCCH 2. Support of scheme A for PDSCH scheduled by single TRP PDCCH] and default QCL assumption with two TCI states for PDSCH 3. Support of single TRP PDSCH scheduled by scheme A PDCCH and default QCL assumption with one TCI states for PDSCH; Per band; Optional with capability signalling. |
| Spreadtrum Communications [8] | For component 1, the second bracket and content in it should be deleted, for the sake of that there is no default QCL assumptions for COREET. In general, UE can support scheme A for PDSCH and not support scheme A for PDCCH. Thus, we prefer to split the first part of component 2 into multiple components. Proposal 5: For component 1 of FG 23-6-1, delete ‘[and default QCL assumption with two TCI states for PDCCH]’ Proposal 6: Suggest to split component 2 of FG 23-6-1 as below: - 2: Support of scheme A for PDSCH scheduled by single TRP PDCCH - 3: Support of scheme A for PDSCH scheduled by Scheme A PDCCH - 4: default QCL assumption with two TCI states for PDSCH |
| Qualcomm Incorporated [9] | Regarding FG 23-6-1 for the support SFN scheme A (a.k.a scheme 1), we agree with the split the FG into two FGs. Where FG 23-6-1 describes UE capability for supporting SFN scheme 1 for both PDDCH and PDSCH or only PDSCH and FG 23-6-1a describes the UE capability of dynamic switching between single TRP and SFN PDSCH. Based on earlier RAN1 agreements, Table 1 summerized the supported combination of transmission schemes for both HST and URLLC use-case (blue color). SFN scheme A (scheme-1) can be supported for both PDSCH + PDCCH, or SFN PDSCH + single TPR PDCCH. That is why FG 23-6-1 should have two components. The first component for the support of SFN for both PDCCH and PDSCH and the second component for the support of SFN only for PDSCH + single TRP (Rel-15) PDCCH. We would like to emphasize that UEs supporting the same SFN scheme for PDCCH and PDSCH does NOT mean that this UE can SFN PDSCH + single TRP PDCCH as this requires the UE to dynamic switch between single TRP and SFN schemes between PDCCH and PDSCH. Agreement Support the following combination of the transmission schemes - Single-TRP PDCCH + Rel-17 Scheme 1 PDSCH - Single-TRP PDCCH + Rel-17 TRP-based pre-compensation PDSCH - FFS: Other combinations of the transmission scheme Note: The PDSCH corresponds to the PDSCH scheduled by DCI formats 1_1 and 1_2. Agreement - UE does not expect to be configured different SFN schemes (scheme 1 or TRP pre-compensation) for both PDCCH and PDSCH.. Table 1: PDSCH and PDCCH transmission mode in SFN mode PDSCH. PDCCH; Rel-15; SFN scheme A; SFN scheme B. Rel-15; N/A; Supported; Supported. SFN scheme A; Supported (URLLC); Supported; No supported. SFN scheme B; Not support; Not supported; Supported. Proposal 8-1: Support the split of FG 23-6-1 into main FG for the support of scheme A for PDDCH/PDSCH and FG 23-6-1A for the support of dynamic switching between single TRP PDSCH and SFN PDSCH scheme A. Proposal 8-2: FG 23-6-1 should have two componeonets: - Component1: The support of Scheme A for both PDSCH and PDCCH - Component 2: Support of Scheme A for PDSCH only, where PDCCH is single TRP. In the RAN1 meeting #106bis-e, it was agreed to support the combination for Rel-17 SNF PDCCH scheme and single-TPR PDSCH as optional UE feature for the URCCL use-case. This combination scheme is captured in blue color in Table -1. A new UE FG should be added to reflect the RAN1 agreement. Agreement Support combination of Rel-17 SFN PDCCH scheme 1 and single-TRP PDSCH - This is optional UE feature - Note: The support of such combination scheme is for URLLC use-case only.. Proposal 8-3: Add new FG 23-6-1b for the support of SFN PDCCH + nonSFN PDSCH with one component of support of scheme-A for PDCCN and single TRP PDSCH. 23. NR_FeMIMO; 23-6-1; SFN scheme A (scheme 1); 1. Support of SFN scheme A for both PDSCH and PDCCH 2. Support of SFN scheme A for PDSCH only with single TRP PDCCH; Per band; For each component: {Support, Not support}; Optional. 23. NR_FeMIMO; 23-6-1b; SFN scheme A for PDCCH for URLLC; Support of scheme A for PDCCH and single TRP PDSCH; Per band; For each component: {Support, Not support}; Optional. |
| OPPO [10] |  |
| Intel Corporation [11] | 23-6-1; SFN scheme A (scheme 1); 1. [Support of scheme A for both PDCCH and PDSCH] [and default QCL assumption with two TCI states for PDCCH] 2. Support of scheme A for PDSCH only [scheduled by [single TRP/ Scheme A] PDCCH] [and default QCL assumption with two TCI states for PDSCH]; Component 1: {Support, No support}. |
| Apple [12] | - FG 23-6-1: scheme A for PDCCH and PDSCH should be separate component, i.e., remove the yellow background 23. NR_FeMIMO; 23-6-1; SFN scheme A (scheme 1); 1. Support of scheme A for PDCCH 2. Support of scheme A for PDSCH; Optional with capability signalling. |
| FUTUREWEI [13] |  |
| xiaomi [14] | As for the component 1 “[Support of scheme A for PDCCH] [and default QCL assumption with two TCI states for PDCCH]” in FG23-6-1, similar to component 2, it is better to indicate the combination of Scheme A PDCCH with scheme A/single-TRP PDSCH in the description. And according to the agreement in RAN1-104b e-meeting [5] listed as below, FG23-6-1a should be supported. RAN1-104b e-meeting-Agreement - Support dynamic (DCI-based) switching of scheme 1 (PDSCH) with single-TRP scheme by TCI state field in DCI format 1_1/1_2 - This feature is UE optional - FFS all other details including RRC signalling, possible RAN4 impact (if any), etc. Proposal 15: Update the component 1 in FG23-6-1 as below 1. [Support of scheme A for PDCCH] scheduling Scheme A/single TRP PDSCH [and default QCL assumption with two TCI states for PDCCH] |
| LG Electronics [15] |  |
| Lenovo/Motorola Mobility [16] |  |
| Ericsson [17] | PDSCH performance is the essential part of HST SFN enhancement, PDCCH alone using SFN is less essential but important for signalling and beam association for PDSCH. FG 23-6-1 each provides the basic function for the supported SFN scheme. The components in the basic feature group of each scheme shall include a complete set of functions for the essential HST configuration. In our understanding the component 2 – Support of scheme A/B for PDSCH and default QCL assumption with two TCI states for PDSCH – is the essential part of HST functionality. The common search space configuration is shared with both legacy UE and HST UE, therefore CORESET for DCI 1_0 associated with CSS schedules the system information and paging need to be Single TRP configuration, which is also essential functionality to be included in the basic feature group. With these in mind, we suggest removing all the bracelets in the component 1,2 in 23-6-1 23. NR_FeMIMO; 23-6-1; SFN scheme A (scheme 1); 1. [Support of scheme A for PDCCH] [and default QCL assumption with two TCI states for PDCCH] 2. Support of scheme A for PDSCH [scheduled by [single TRP/ Scheme A] PDCCH] [and default QCL assumption with two TCI states for PDSCH] 3. Support of dynamic switching between single-TRP and SFN PDSCH scheme A for DCI formats 1_1, 1_2. Remove all bracelets and highlight in 23-6-1 for component 1 and 2. |
| MediaTek Inc. [18] |  |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] | Confirm the FG. Details to be further discussed. |

| 23. NR_FeMIMO | 23-6-1a | Dynamic switching - scheme A | Support of dynamic switching between single-TRP and SFN PDSCH scheme A by TCI state field in DCI formats 1_1, 1_2 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| ZTE [3] |  |
| vivo [4] |  |
| CATT [5] | For SFN based HST enhancements, several FGs (23-6-1 ~ 23-6-4) are listed in [1]. Acoording to the agreements in previous meetings, we have the following comments for these FGs. 23-6-1/2 Scheme 1 / TRP based pre-compensation for PDSCH and PDCCH Firstly, based on agreement in RAN1#107e meeting, TRP-based pre-compensation scheme for PDSCH / PDCCH is supported in both FR1 and FR2 with UE capability at least per FR. So our suggestion is to delete the restriction of FR1 in the current UE feature list in [1]. Sencondly, if dynamic switching between single-TRP and SFN PDSCH scheme A/B can be listed separately, the UE features 23-6-1 should be prerequisite feature for 23-6-1a and the UE features 23-6-2 should be prerequisite feature for 23-6-2a. Thridly, based on the agreement in RAN1#107e meeting, the two default beam(s) for PDSCH/PDCCH is determined by the different combinations of transmission scheme for PDSCH and PDCCH, so we support that if UE supports scheme A for PDCCH, the UE can also support default beam(s) with two TCI states for PDCCH. Finally, based on the agreement in RAN1#106e and #106bis-e meeting, the following combinations of the transmission schemes was supported: - Single-TRP PDCCH + Rel-17 Scheme 1 PDSCH - Single-TRP PDCCH + Rel-17 TRP-based pre-compensation PDSCH - Rel-17 scheme 1 PDCCH + single-TRP PDSCH with UE capability It can be seen that only the combination of Rel-17 scheme 1 PDCCH and single-TRP PDSCH is supported with UE capability. So in 23-6-1 for PDSCH, the second component should be only discussed in this combination scheme. Futhermore, in 23-6-2 for PDSCH, whether Rel-17 TRP-based pre-compensation PDCCH can schedule single-TRP PDSCH should be futher discussed in AI 8.1.2.4. Based on the above analysis, we have the following proposal: Proposal-15: For Scheme 1 / TRP based pre-compensation for PDSCH and PDCCH, the UE features 23-6-1 ~ 23-6-2 are revised as follows: 23-6-1a; Dynamic switching - scheme A; Support of dynamic switching between single-TRP and SFN PDSCH scheme A by TCI state field in DCI formats 1_1, 1_2; 23-6-1; Optional with capability signalling. |
| Samsung [6] |  |
| NTT DOCOMO, INC. [7] | - FG23-6-1a and FG23-6-2a can be reported per UE, as an optional capability. 23. NR_FeMIMO; 23-6-1a; Dynamic switching - scheme A; Support of dynamic switching between single-TRP and SFN PDSCH scheme A by TCI state field in DCI formats 1_1, 1_2; Per UE; Optional with capability signalling. |
| Spreadtrum Communications [8] |  |
| Qualcomm Incorporated [9] | 23. NR_FeMIMO; 23-6-1a; Dynamic switching - scheme A; Support of dynamic switching between single-TRP and SFN PDSCH scheme A by TCI state field in DCI formats 1_1, 1_2; 23-6-1; Per band; {Support, Not support}; Optional. |
| OPPO [10] |  |
| Intel Corporation [11] |  |
| Apple [12] | - FG 23-6-1a: This has been agreed, i.e., remove the yellow background 23. NR_FeMIMO; 23-6-1a; Dynamic switching - scheme A; Support of dynamic switching between single-TRP and SFN PDSCH scheme A by TCI state field in DCI formats 1_1, 1_2. |
| FUTUREWEI [13] |  |
| xiaomi [14] | support FG23-6-1a |
| LG Electronics [15] |  |
| Lenovo/Motorola Mobility [16] |  |
| Ericsson [17] |  |
| MediaTek Inc. [18] |  |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] |  |

| 23. NR_FeMIMO | 23-6-2 | SFN scheme B (TRP based pre-compensation) | 1. [Support of scheme B for PDCCH] 2. Support of scheme B for PDSCH [scheduled by [single TRP/Scheme B] PDCCH] | [23-6-1] |  |  |  |  |  | [FR1 only] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | HST multi-TRP deployment In RAN1#107e meeting, we have agreed that TRP-based pre-compensation scheme (SFN scheme B) for PDSCH / PDCCH is supported in both FR1 and FR2 with UE capability at least per FR, so default QCL assumption with two TCI states for PDCCH and default QCL assumption with two TCI states for PDSCH should be supported for SFN scheme B. Proposal 3-8: Remove [FR1 only] in FG 23-6-2 Proposal 3-9: Revise component 1/2 in FG 23-6-2: - 1. [Support of scheme B for PDCCH] [and default QCL assumption with two TCI states for PDCCH]； - 2. Support of scheme B for PDSCH [scheduled by [ single TRP/Scheme B] PDCCH] [ and default QCL assumption with two TCI states for PDSCH] 23. NR_FeMIMO; 23-6-2; SFN scheme B (TRP based pre-compensation); 1.[Support of scheme B for PDCCH] [and default QCL assumption with two TCI states for PDCCH] 2. Support of scheme B for PDSCH [scheduled by [ single TRP/Scheme B] PDCCH] [ and default QCL assumption with two TCI states for PDSCH]; [23-6-1]; [FR1 only]; Optional with capability signalling. |
| ZTE [3] |  |
| vivo [4] |  |
| CATT [5] | For SFN based HST enhancements, several FGs (23-6-1 ~ 23-6-4) are listed in [1]. Acoording to the agreements in previous meetings, we have the following comments for these FGs. 23-6-1/2 Scheme 1 / TRP based pre-compensation for PDSCH and PDCCH Firstly, based on agreement in RAN1#107e meeting, TRP-based pre-compensation scheme for PDSCH / PDCCH is supported in both FR1 and FR2 with UE capability at least per FR. So our suggestion is to delete the restriction of FR1 in the current UE feature list in [1]. Sencondly, if dynamic switching between single-TRP and SFN PDSCH scheme A/B can be listed separately, the UE features 23-6-1 should be prerequisite feature for 23-6-1a and the UE features 23-6-2 should be prerequisite feature for 23-6-2a. Thridly, based on the agreement in RAN1#107e meeting, the two default beam(s) for PDSCH/PDCCH is determined by the different combinations of transmission scheme for PDSCH and PDCCH, so we support that if UE supports scheme A for PDCCH, the UE can also support default beam(s) with two TCI states for PDCCH. Finally, based on the agreement in RAN1#106e and #106bis-e meeting, the following combinations of the transmission schemes was supported: - Single-TRP PDCCH + Rel-17 Scheme 1 PDSCH - Single-TRP PDCCH + Rel-17 TRP-based pre-compensation PDSCH - Rel-17 scheme 1 PDCCH + single-TRP PDSCH with UE capability It can be seen that only the combination of Rel-17 scheme 1 PDCCH and single-TRP PDSCH is supported with UE capability. So in 23-6-1 for PDSCH, the second component should be only discussed in this combination scheme. Futhermore, in 23-6-2 for PDSCH, whether Rel-17 TRP-based pre-compensation PDCCH can schedule single-TRP PDSCH should be futher discussed in AI 8.1.2.4. Based on the above analysis, we have the following proposal: Proposal-15: For Scheme 1 / TRP based pre-compensation for PDSCH and PDCCH, the UE features 23-6-1 ~ 23-6-2 are revised as follows: 23-6-2; SFN scheme B (TRP based pre-compensation); 1. [Support of scheme B for PDCCH] 2. Support of scheme B for PDSCH [scheduled by [single TRP/Scheme B] PDCCH]; 23-6-1; [FR1 only]; Optional with capability signalling. |
| Samsung [6] |  |
| NTT DOCOMO, INC. [7] | - During the RAN1 discussion for Rel.17 HST, the assumption of the basic operation is a combination of “1) SFN-PDCCH + SFN-PDSCH”, and some UE vendors claim difficulty of supporting other combinations. Hence, the basic capability of FG23-6-1 and FG23-6-2 should be the combination of “1) SFN-PDCCH + SFN-PDSCH”, and other combinations should be reported in separate components. The other combinations are “2) S-TRP PDCCH + SFN-PDSCH” and “3) SFN-PDCCH + S-TRP PDSCH”. Note that 3) is only supported in SFN scheme A based on the previous agreements. - The most of function between FR23-6-1 and FG23-6-2 are common, and hence we agree FG23-6-1 is pre-requisite feature of FG23-6-2. - We believe FG23-6-1 and FG23-6-2 should be reported per band. - For FG23-6-2, SFN scheme B is supported in both FR1 and FR2 by RAN1#107e agreement. Hence, “[FR1 only]” should be deleted. 23. NR_FeMIMO; 23-6-2; SFN scheme B (TRP based pre-compensation); 1. Support of scheme B for PDSCH scheduled by scheme B PDCCH 2. Support of scheme B for PDSCH scheduled by single TRPPDCCH and default QCL assumption with two TCI states for PDSCH; 23-6-1; Per band; Optional with capability signalling. |
| Spreadtrum Communications [8] | Similar to FG23-6-2, we also suggest to split component 2 into multiple components. Proposal 7: Suggest to split component 2 of FG 23-6-2 as below: - 2: Support of scheme B for PDSCH scheduled by single TRP PDCCH - 3: Support of scheme B for PDSCH scheduled by Scheme B PDCCH |
| Qualcomm Incorporated [9] | A similar discussion is applicable to SFN scheme B (TRP pre-compnesnation), where based on RAN1 agreements SFN scheme B is applicable on to HST deployment only. FG 23-6-2 can be split into main FGs. FG 23-6-2 for the support of SFN scheme B for both PDCCH and PDSCH or SFN PDSCH only where PDCCH is single TRP. FG 23-6-2a for the PDSCH dynamic switching between SFN scheme-B and single TPR. In the last RAN1 meeting, the support of SFN scheme-B has been agreed for both FR1 and FR2. Therefore, there is no need for FR1 and FR2 differentiation. In addition, sfn scheme B was supported to reduce UE demod complexity. It doesn’t make sense to force the UE to support complex demod feature (SFN scheme A) as pre-requiest to support a simpler demod feature. Proposal 8-4: Support the split of FG 23-6-2 into main FG for the support of scheme B for PDDCH/PDSCH and FG 23-6-2a for the support of dynamic switching between single TRP PDSCH and SFN PDSCH scheme B. Proposal 8-5: FG 23-6-2 should have two componeonets: - Component 1: Support of SFN Scheme B for both PDSCH and PDCCH - Component 2: Support of SFN Scheme B for PDSCH only, where PDCCH is single TRP. Proposal 8-6: For FG 23-6-2, support the following: - No need to have FR1 and FR2 differentiaion. - Remove the pre-requesite of FG 23-6-1 23. NR_FeMIMO; 23-6-2; SFN scheme B (TRP based pre-compensation); 1. Support of SFN scheme B for both PDSCH and PDCCH 2. Support of SFN scheme B for PDSCH only with single TRP PDCCH; [23-6-1]; Per band; [FR1 only]; {Support, Not support}; Optional. |
| OPPO [10] |  |
| Intel Corporation [11] | Regarding UE capability to support of SFN scheme for PDSCH and PDDCH. Although there is no significant difference from implementation perspective to support SFN scheme for PDSCH and PDCCH, most of the performance benefits for enhanced SFN transmission so far were observed for PDSCH. In this case, UE should be allowed to report support of SFN scheme A only for PDSCH (i.e. without PDCCH) as part of FG 23-6-1 to reduce testing efforts associated with support of SFN scheme A for PDCCH. In this case the additional component or separate FG should be added for SFN scheme A supported in both PDSCH and PDCCH. The default beam assumption should be removed from the basic capability (according to the agreements on separate UE capability) and considered as separate FGs. Similar modifications should be also considered for SFN scheme B, where support of TRP-based pre-compensation seems more essential for PDSCH only. Moreover, since implementation of scheme B is simpler, UE indicating support of SFN scheme A should also indicate support SFN scheme B, which can be achieved by making FG 23-6-1 pre-requisite for FG 23-6-2. 23-6-2; SFN scheme B (TRP based pre-compensation); 1. [Support of scheme B for PDCCH and PDSCH ] 2. Support of scheme B for PDSCH only [scheduled by [single TRP/Scheme B] PDCCH]; [Pre-requisite 23-6-1] Component 1: {Support, No support. |
| Apple [12] | - FG 23-6-2: scheme B for PDCCH and PDSCH should be separate component, i.e., remove the yellow background 23. NR_FeMIMO; 23-6-2; SFN scheme B (TRP based pre-compensation); 1. Support of scheme B for PDCCH 2. Support of scheme B for PDSCH; [23-6-1]; [FR1 only]; Optional with capability signalling. |
| FUTUREWEI [13] |  |
| xiaomi [14] | Same to FG23-6-1, as for the component 1 “[Support of scheme B for PDCCH]” in FG23-6-2, similar to component 2, it is better to indicate the combination of Scheme B PDCCH with Scheme B PDSCH in the description. And according to the agreement in RAN1-105 [10] e-meeting listed as below, FG23-6-2a should be supported. RAN1-105 e-meeting-Agreement For specification based TRP-based frequency offset pre-compensation scheme - Support dynamic (DCI -based) switching with single-TRP scheme by TCI state field in DCI format 1_1/1_2 - This feature is UE optional - UE is not expected to be indicated by MAC CE with single TCI state per any of TCI codepoint , if UE is configured with TRP-based frequency PDSCH by RRC , but not capable to support dynamic switching between TRP-based frequency and single-TRP by TCI state field in DCI Format 1_1/1_2 - Support semi-static (RRC based) switching with Rel-16 schemes 1a, 2a, 2b, 3, 4 - Support semi-static (RRC based) switching with Rel-17 scheme 1 (PDSCH) Proposal 16: Support the component 1 in FG23-6-2 by updating as below 1. [Support of scheme B for PDCCH] scheduling Scheme B PDSCH |
| LG Electronics [15] |  |
| Lenovo/Motorola Mobility [16] | Feature group 23-6-2, the pre-requisite FG 23-6-1 should be removed. This pre-requisite does not match any agreement made so far. Besides, Scheme B is a network-based approach and intended to support UEs that are not capable of supporting Scheme A, and hence including FG 23-6-1 to be a pre-requisite of FG 23-6-2 is unjustified. For FG 23-6-2, remove the pre-requisite FG 23-6-1 - Additionally, it was agreed in RAN1#107-e [3] that SFN Scheme B is supported in both FR1 and FR2 with UE capability at least per FR. Hence, we suggest restricting FG 23-6-2 to FR1, and add a separate FG for support of SFN Scheme B in FR2, such that FG 23-6-2 is a pre-requisite of the FG corresponding to SFN Scheme B support in FR2 Limit capability interpretation for FR of FG 23-6-2 to FR1 Add a new FG for SFN Scheme B support in FR2, with FG 23-6-2 as a pre-requisite |
| Ericsson [17] | PDSCH performance is the essential part of HST SFN enhancement, PDCCH alone using SFN is less essential but important for signalling and beam association for PDSCH. FG 23-6-2 each provides the basic function for the supported SFN scheme. The components in the basic feature group of each scheme shall include a complete set of functions for the essential HST configuration. In our understanding the component 2 – Support of scheme A/B for PDSCH and default QCL assumption with two TCI states for PDSCH – is the essential part of HST functionality. The common search space configuration is shared with both legacy UE and HST UE, therefore CORESET for DCI 1_0 associated with CSS schedules the system information and paging need to be Single TRP configuration, which is also essential functionality to be included in the basic feature group. With these in mind, we suggest removing all the bracelets in the component 1,2 in 23-6-2. 23. NR_FeMIMO; 23-6-2; SFN scheme B (TRP based pre-compensation); - [Support of scheme B for PDCCH] and PDSCH - Support of scheme B for PDSCH [scheduled by [single TRP/Scheme B] PDCCH] - Support of dynamic switching between single-TRP and SFN PDSCH scheme B for by TCI state field in DCI formats 1_1, 1_2. Remove all bracelets and highlight in 23-6-2 for component 1 and 2. |
| MediaTek Inc. [18] |  |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] | Confirm the FG. Details to be further discussed. |

| 23. NR_FeMIMO | 23-6-2a | Dynamic switching - scheme B | Support of dynamic switching between single-TRP and SFN PDSCH scheme B by TCI state field in DCI formats 1_1, 1_2 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| ZTE [3] |  |
| vivo [4] |  |
| CATT [5] | For SFN based HST enhancements, several FGs (23-6-1 ~ 23-6-4) are listed in [1]. Acoording to the agreements in previous meetings, we have the following comments for these FGs. 23-6-1/2 Scheme 1 / TRP based pre-compensation for PDSCH and PDCCH Firstly, based on agreement in RAN1#107e meeting, TRP-based pre-compensation scheme for PDSCH / PDCCH is supported in both FR1 and FR2 with UE capability at least per FR. So our suggestion is to delete the restriction of FR1 in the current UE feature list in [1]. Sencondly, if dynamic switching between single-TRP and SFN PDSCH scheme A/B can be listed separately, the UE features 23-6-1 should be prerequisite feature for 23-6-1a and the UE features 23-6-2 should be prerequisite feature for 23-6-2a. Thridly, based on the agreement in RAN1#107e meeting, the two default beam(s) for PDSCH/PDCCH is determined by the different combinations of transmission scheme for PDSCH and PDCCH, so we support that if UE supports scheme A for PDCCH, the UE can also support default beam(s) with two TCI states for PDCCH. Finally, based on the agreement in RAN1#106e and #106bis-e meeting, the following combinations of the transmission schemes was supported: - Single-TRP PDCCH + Rel-17 Scheme 1 PDSCH - Single-TRP PDCCH + Rel-17 TRP-based pre-compensation PDSCH - Rel-17 scheme 1 PDCCH + single-TRP PDSCH with UE capability It can be seen that only the combination of Rel-17 scheme 1 PDCCH and single-TRP PDSCH is supported with UE capability. So in 23-6-1 for PDSCH, the second component should be only discussed in this combination scheme. Futhermore, in 23-6-2 for PDSCH, whether Rel-17 TRP-based pre-compensation PDCCH can schedule single-TRP PDSCH should be futher discussed in AI 8.1.2.4. Based on the above analysis, we have the following proposal: Proposal-15: For Scheme 1 / TRP based pre-compensation for PDSCH and PDCCH, the UE features 23-6-1 ~ 23-6-2 are revised as follows: 23-6-2a; Dynamic switching - scheme B; Support of dynamic switching between single-TRP and SFN PDSCH scheme B by TCI state field in DCI formats 1_1, 1_2; 23-6-2; Optional with capability signalling. |
| Samsung [6] |  |
| NTT DOCOMO, INC. [7] | - FG23-6-1a and FG23-6-2a can be reported per UE, as an optional capability. 23. NR_FeMIMO; 23-6-2a; Dynamic switching - scheme B; Support of dynamic switching between single-TRP and SFN PDSCH scheme B by TCI state field in DCI formats 1_1, 1_2; Per UE; Optional with capability signalling. |
| Spreadtrum Communications [8] |  |
| Qualcomm Incorporated [9] | 23. NR_FeMIMO; 23-6-2a; Dynamic switching - scheme B; Support of dynamic switching between single-TRP and SFN PDSCH scheme B by TCI state field in DCI formats 1_1, 1_2; 23-6-2; Per band; {Support, Not support}; Optional. |
| OPPO [10] |  |
| Intel Corporation [11] |  |
| Apple [12] | - FG 23-6-2a: This has been agreed, i.e., remove the yellow background 23. NR_FeMIMO; 23-6-2a; Dynamic switching - scheme B; Support of dynamic switching between single-TRP and SFN PDSCH scheme B by TCI state field in DCI formats 1_1, 1_2. |
| FUTUREWEI [13] |  |
| xiaomi [14] | support FG23-6-2a |
| LG Electronics [15] |  |
| Lenovo/Motorola Mobility [16] |  |
| Ericsson [17] |  |
| MediaTek Inc. [18] |  |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] |  |

| 23. NR_FeMIMO | 23-6-3 | Simultaneous activation of two TCI states for PDCCH across multiple CCs (HST/URLLC) | Support of simultaneous activation of two TCI states for CORESETs with the same CORESET ID in all BWPs across a set of configured component carriers by single MAC-CE | 23-6-1 or 23-6-2 |  |  |  | Per UE |  | Yes |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| ZTE [3] | For SFN based HST enhancements, several FGs (23-6-1 ~ 23-6-3) are listed in [2]. We generally agree with the first two. However, the third one, i.e., 23-6-3 on support of simultaneous activation of two TCI states for PDCCH across multiple CCs, is not needed from our view. That’s because whether UE supports such feature can completely be inferred by the existing Rel-16 UE FG 16-1b-1 combined with the Rel-17 FG 23-6-1 or 23-6-2. 16-1b-1; TCI state activation across multiple CCs; - Support of Simultaneous TCI state activation across multiple CCs: PDCCH, PDSCH. As we know, for a UE support FG 16-1b-1, some configured CCs in which TCI states can be simultaneously updated for CORESETs with the same ID should share the same RF chain or similar channel conditions for analog beamforming. This is regardless of the number of activated TCI states for each CORESET. So, as long as UE supports FG 16-1b-1, and also supports FG 23-6-1 or 23-6-2, it is natural for this UE to support simultaneous activation of two TCI states across set of configured component carriers. Proposal 12: Remove FG 23-6-3 on support of simultaneous activation of two TCI states for PDCCH across multiple CCs for HST. |
| vivo [4] |  |
| CATT [5] |  |
| Samsung [6] |  |
| NTT DOCOMO, INC. [7] |  |
| Spreadtrum Communications [8] |  |
| Qualcomm Incorporated [9] | 23. NR_FeMIMO; 23-6-3; Simultaneous activation of two TCI states for PDCCH across multiple CCs (HST/URLLC); Support of simultaneous activation of two TCI states for CORESETs with the same CORESET ID in all BWPs across a set of configured component carriers by single MAC-CE; 23-6-1 or 23-6-2; Per UE; Yes; For each component {Support, Not support}; Optional. |
| OPPO [10] |  |
| Intel Corporation [11] |  |
| Apple [12] |  |
| FUTUREWEI [13] |  |
| xiaomi [14] |  |
| LG Electronics [15] |  |
| Lenovo/Motorola Mobility [16] |  |
| Ericsson [17] |  |
| MediaTek Inc. [18] |  |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] | Confirm the FG. Details to be further discussed. |

| 23. NR_FeMIMO | 23-6-4 | Default beam setup [for HST] | 1. Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold FFS: 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI [when PDSCH is scheduled with offset equal or larger than the threshold] 3. Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold FFS: 4. Support of single-TRP PUCCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured FFS: 5. Support of single-TRP PUSCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured FFS: 6. Support of single-TRP SRS resource transmission using default beam when enhanced SFN PDCCH transmission scheme is configured |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| ZTE [3] |  |
| vivo [4] |  |
| CATT [5] | For the FG 23-6-4, most of components should be supported in principle. Based on the agreement in RAN1#107e meeting, the default beam(s) for PDSCH when TCI field is not present in DCI and PDSCH is scheduled with offset equal or larger than the threshold is determined by two UE capabilities of default beam feature and dynamic switching feature. So our suggeation is that the UE features 23-6-1a/2a should be prerequisite feature for 23-6-4. Proposal-16: For default beam setup for SFN, the UE features 23-6-4 are revised as follows: 23-6-4; Default beam setup for HSTSFN; 1. Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold FFS: 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI [and when PDSCH is scheduled with offset equal or larger than the threshold] 3. Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold FFS: 4. Support of single-TRP PUCCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured FFS: 5. Support of single-TRP PUSCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured FFS: 6. Support of single-TRP SRS resource transmission using default beam when enhanced SFN PDCCH transmission scheme is configured; 23-6-1a/23-6-2a; Optional with capability signalling. |
| Samsung [6] | Regarding FG 23-6-4, since all new default beam behaviors are included in a single FG so far, we would like to suggest to split this FG as three separate feature groups and each feature group can have their own purpose to support how many numbers of default beams and which direction of default beam. Proposal 25: Split FG 23-6-4 as three separate feature groups and all three feature groups are FR2 only: - Components 3 as a single feature group to require a new default beam behavior with a single default DL beam - Components 1 and 2 to require a new default beam behavior with two default DL beams - Components 4, 5, and 6 to require a new default beam behavior with a single default UL beam. |
| NTT DOCOMO, INC. [7] | - Default beam is useful for both HST and URLLC, and we should add URLLC in addition to HST for the name of FG23-6-4. - We agree all of components in FG23-6-4 are needed. Alternatively, we can create more general component(s) which combines the multiple components. 23. NR_FeMIMO; 23-6-4; Default beam setup for HST/URLLC; 1. Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI when PDSCH is scheduled with offset equal or larger than the threshold 3. Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold 4. Support of single-TRP PUCCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 5. Support of single-TRP PUSCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 6. Support of single-TRP SRS resource transmission using default beam when enhanced SFN PDCCH transmission scheme is configured; Optional with capability signalling. |
| Spreadtrum Communications [8] | FG 23-6-4 In general, UE supports DL default beam, but it can not support default beam for UL, and vice versa. Therefore, we suggest to decouple default beam for DL and UL. Proposal 8: Suggest to split the FG 23-6-4 into two FGs, where one FG includes component 1, 2 and 3, and another includes component 4, 5 and 6. |
| Qualcomm Incorporated [9] | Regarding FG 23-6-4, it should be split into multiple FGs. The first FG is for the default beam for PDSCH and CSI-RS and should include components 1, 2 and 3. The second FG 23-6-4A is for UE capality of two QCL TypeD properories for multiple overlapping CORESETs when a CORESET is activated with two TCI as per RAN1-agreement below. And third FG 23-6-4b is for UL default spatial relation and pathloss RS. Agreement When a CORESET is activated with two TCI states which overlaps with another CORESET, support extension of Rel-15 prioritization rule for PDCCH monitoring of PDCCH candidates in overlapping monitoring occasions with different QCL-TypeD - FFS: Prioritization rule considers CORESETs indicated with 1 and/or 2 TCI states - Supports identifying two QCL-TypeD properties for multiple overlapping CORESETs - UE capability is introduced - FFS other details - FFS: Strive to have same / similar solution as discussed under AI 8.1.2.1. Proposal 8-7: Split FG 23-6-4 into three FGs: - FG 23-6-4: The support of default beam for SFN PDSCH and CSI-RS, - FG 23-6-4A: Two QCL TypeD properties for overlapping CORESETs - FG 23-6-4B: Default spatial relation and pathloss RS 23. NR_FeMIMO; 23-6-4; Default DL beam setup; 1. Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI. 3. Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold; Per band; FR2 only; For each component: {Support, Not support}; Optional. 23. NR_FeMIMO; 23-6-4a; Two QCL TypeD properties for SFN PDCCH; Supports identifying two QCL-TypeD properties for multiple overlapping CORESETs when a CORESET is activated with two TCI; Per Band; FR2 only; {Support, Not support}; Optional. 23. NR_FeMIMO; 23-6-4b; Default spatial relation and pathloss RS; 1. Support of single-TRP PUCCH transmission using default beam and PL-RS when enhanced SFN PDCCH transmission scheme is configured 2. Support of single-TRP PUSCH transmission using default beam and PL-RS when enhanced SFN PDCCH transmission scheme is configured 3. Support of single-TRP SRS resource transmission using default beam and PL-RS when enhanced SFN PDCCH transmission scheme is configured; Per Band; FR2 only; For each component: {Support, Not support}; Optional. |
| OPPO [10] |  |
| Intel Corporation [11] | We also note that simultaneous activation of the two TCI states across multiple CCs in FGs 23-6-3 and default beam for DL / UL in FG 23-6-4 can be applicable not only to HST, but also to URLLC scenarios. In this case, it is proposed to update definition of the corresponding FGs by extending description to ‘HST/URLLC’ (or removing ‘HST’). Moreover, the UE features captured as FFS in FG 23-6-4 should be included in the list of UE capabilities either as part of the other components (e.g. default beams DL or default beams for UL) or as a separate component in FG 23-6-4. 23-6-4; Default beam setup [for HST/URLLC]; 1. Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold FFS: 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI [when PDSCH is scheduled with offset equal or larger than the threshold] 3. Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold FFS: 4. Support of single-TRP PUCCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured FFS: 5. Support of single-TRP PUSCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured FFS: 6. Support of single-TRP SRS resource transmission using default beam when enhanced SFN PDCCH transmission scheme is configured. |
| Apple [12] | - FG 23-6-4: Default beam needs more discussion pending the maintenance of Rel-17 HST enhancement. At least, we should separate the DL and UL default beam as what we did in Rel-16 |
| FUTUREWEI [13] |  |
| xiaomi [14] | As for the component 1 in FG 23-6-4 “Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold”, the detailed scenario is not clear. Since according to the agreement in RAN1-106 e-meeting [7] and RAN1-107 e-meeting [4] listed as below, there are two cases for default beam determination when time offset is less than the threshold. Only one scenario is supported or not up to UE capability. Thus it is better to indicate the scenario in the component clearer. RAN1-106 e-meeting-Agreement If enableTwoDefaultTCI-States is configured and at least one TCI codepoint indicates two TCI states and time offset between the reception of the DL DCI and the PDSCH is less than the threshold timeDurationForQCL, default beam(s) for Rel-17 enhanced SFN PDSCH (scheme 1 or if supported TRP-based pre-compensation) reception: - Alt 1: Reuse rule to determine TCI states as defined for Rel-16 PDSCH scheme-1a This is a UE optional feature RAN1-107 e-meeting-Agreement The agreement from RAN1#106b-e meeting is updated as follows When SFN PDSCH is not configured by RRC and there is no TCI codepoint which indicates two TCI states activated for the PDSCH (i.e,. Rel-16 MTRP PDSCH is not configured), for PDSCH reception scheduled by DCI format 1_0, 1_1, 1_2, if the time offset between the reception of the DL DCI and the corresponding PDSCH is smaller than the threshold timeDurationForQCL, - For DCI format 1_1/1_2, support both configurations with and without TCI state field. - [If enableTwoDefaultTCIStates is not configured,] for both cases with and without TCI state field, - If enhanced SFN PDCCH transmission scheme 1 is configured and the lowest CORESET ID in the latest slot is indicated with two TCI states, select the 1st TCI state of the two TCI states of the CORESET as default beam for the PDSCH reception - FFS : Whether above applies for TRP -based pre-compensation if TRP -based pre-compensation is agreed to be support in FR2 - Otherwise, UE applies the one active TCI state of the CORESET with the lowest controlResourceSetId in the latest slot when receiving the PDSCH - It is up to editor how to capture the above agreement While for component 2 “FFS: 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI [when PDSCH is scheduled with offset equal or larger than the threshold]”, according to the agreement in RAN1-106 e-meeting [7] and RAN1-107 e-meeting [4] listed as below, it is better to remove the bracket. RAN1-106 e-meeting-Agreement For PDSCH reception scheduled by DCI format 1_0, [1_1 and 1_2], if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal or larger than the threshold timeDurationForQCL - Support configuration when there is no TCI field in the DCI scheduling PDSCH - UE applies the state(s) of the scheduling CORESET when receiving the PDSCH - if there are two active TCI states for the CORESET, UE applies the both QCL assumption of the CORESET that schedules the PDSCH when receiving the PDSCH - otherwise, UE applies the one active TCI state of the CORESET when receiving the PDSCH - FFS if the time offset between the reception of the DL DCI and the corresponding PDSCH is smaller than the threshold timeDurationForQCL This is a UE optional feature RAN1-107 e-meeting-Agreement When SFN PDSCH and SFN PDCCH are configured by RRC , for PDSCH reception scheduled by DCI formats 1_1 and 1_2, and, if applicable the time offset between the reception of the DL DCI and the corresponding PDSCH is equal or larger than the threshold timeDurationForQCL - Support configuration when there is no TCI field in the DCI scheduling PDSCH - UE applies the TCI state(s) of the scheduling CORESET when receiving the PDSCH - If there are two active TCI states for the CORESET , UE applies both QCL assumptions of the CORESET that schedules the PDSCH when receiving the PDSCH - otherwise, if there is one active TCI state for the CORESET , UE applies the one active TCI state of the CORESET when receiving the PDSCH This feature is UE optional capability - If UE doesn’t support this capability, UE is expected to be configured with TCI state field - UEs supporting this feature and are not capable of dynamic switching between single TRP and SFN , the CORESET that schedules PDSCH by DCI formats 1_1 and 1_2 (FFS DCI format 1_0) should be activated with two TCI states. FFS for maintenance: if SFN PDCCH is not configured Proposal 17: Support the component 1 by updating as below and support component 3, 4, 5&6 in FG23-6-4. 1. Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold, and enableTwoDefaultTCI-States is configured and at least one TCI codepoint indicates two TCI states. FFS: 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI [when PDSCH is scheduled with offset equal or larger than the threshold]. |
| LG Electronics [15] | - Default beam is useful and all of components in FG 23-6-4 were agreed during RAN1 discussion. So, we support all components and ‘FFS’ should be removed. Proposal 9: Adopt the following table for Rel-17 M-TRP HST-SFN. 23. NR_FeMIMO; 23-6-4; Default beam setup [for HST]; 1. Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI [when PDSCH is scheduled with offset equal or larger than the threshold] 3. Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold 4. Support of single-TRP PUCCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 5. Support of single-TRP PUSCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 6. Support of single-TRP SRS resource transmission using default beam when enhanced SFN PDCCH transmission scheme is configured; Optional with capability signalling. |
| Lenovo/Motorola Mobility [16] |  |
| Ericsson [17] |  |
| MediaTek Inc. [18] | UE can only support default DL beam not default UL beam. Also, UE can only support default UL beam not default DL beam. Thus, we would like to propose the following. Proposal 49: FG 23-6-4 should be separated to two FG 23-6-4 (Default DL beam setup for HST) and FG 23-6-4a (Default UL beam setup for HST). Components 1, 2, 3 are included in original FG 23-6-4 and components 4, 5, 6 are included in FG 23-6-4a. 23. NR_FeMIMO; 23-6-4; Default DL beam setup [for HST]; 1. Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold FFS: 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI [when PDSCH is scheduled with offset equal or larger than the threshold] 3. Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold; Optional with capability signalling. 23. NR_FeMIMO; 23-6-4a; Default UL beam setup [for HST]; 1. Support of single-TRP PUCCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 2. Support of single-TRP PUSCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 3. Support of single-TRP SRS resource transmission using default beam when enhanced SFN PDCCH transmission scheme is configured; {Supported, Not supported}; Optional with capability signalling. |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] | - Confirm the FG. - Components 4-6 can be confirmed. |

| 23. NR_FeMIMO | 23-7-1 | Basic Features of CSI Enhancement for Multi-TRP | - Maximum number of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Nmax=1 - FFS: Maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks,max - CSI report mode selection FFS others (supported options; values for X, codebook types, number of ports of CMRs, CMR sharing among NCJT and sTRP measurement hypotheses for FR1, two CMR groups with Ks=K1+K2 CMRs in CSI-RS resource set, reporting of 2 PMIs, 2 RIs and 2 Lis for NCJT measurement hypothesis …) | FFS |  |  |  | [Per band / per BC] |  |  |  | [Component 2 candidate value set: {2, 3, 4, 5, 6, 7, 8}] Note: ‘NCJT’ is not used in RAN1 specifications and will be aligned with 38.214 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | Comparing to conventional single-TRP/CMR based CSI reporting, the CSI calculation with a NCJT measurement hypothesis in Rel-17 FeMIMO can be more complex due to the joint PMI/RI/CQI determination over two CMRs within a resource pair. On the other hand, commercial UE will share a CSI processing engine across multiple CSI measurement hypotheses and mechanisms within plural CCs simultaneously. Consequently, the UE should be allowed to report its CSI processing capability of Rel-17 NCJT measurement hypothesis and other single-TRP/CMR measurement hypothesis separately. The processing capability of single-TRP/CMR based CSI measurement using Type-I PS codebook is reported by different FGs, i.e., FG 2-36 in Rel-15 or FG 16-8 “Active CSI-RS resources and ports for mixed codebook types in any slot” in Rel-16. For Rel-17, it is proposed to address with NCJT CSI measurement with a similar manner, by assuming a CSI quantization/reporting over a CMR resource pair as a “new codebook type”. Hence being similar with FG 23-9-1 for basic features of FeType II PS Codebook, the processing capability of NCJT measurement hypotheses over a resource pair (or pairs) can be reported by a new component in FG 23-7-1, i.e. component 4 shown in the table below with a similar manner of supported combinations. In addition, for hybrid/mixed CSI measurement capability in FG 23-9-5, {codebook 1} can be extended to support simultaneous CSI calculation with NCJT and single-TRP/CMR based measurement hypotheses, since both of them are based on the same Type I SP codebook. If necessary, additional note can be added in FG 23-9-5 to elaborate the counting mechanism of ports/resources for NCJT. {Codebook 2, codebook 3} can be used as usual to support all advanced Type II/eTypeII/FeTypeII codebooks. Note that there is no prefect or ultimately fine reporting mechanism when addressing with supported mixed codebook types in a slot, e.g. in FG 16-8 in Rel-16, considering reporting overhead and UE/gNB implementation due to signaling complexity. The UE needs to consider the worst scenario when reporting CSI processing capability of FG16-8 in Rel-16 or FG 23-9-5 in Rel-17. Proposal 5-1: It is proposed to support a new component in 23-7-1 with supported combinations of {Max # of Tx ports per resource within a resource pair, Max # of resources across all resource pairs and total # of Tx ports of resources across all resource pairs} across all CCs simultaneously, whilst {codebook 1} in 23-9-5 can be used to report combinations for joint NCJT CSI (if applicable) and single-TRP CSI measurement capability based on Type I SP codebook. 23. NR_FeMIMO; 23-7-1; Basic Features of CSI Enhancement for Multi-TRP; - Maximum number of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Nmax=1 - Maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks_max - CSI report mode selection - A list of supported combinations of active NZP CSI-RS resource pairs and associated ports. Each combination is {Max # of Tx ports per resource within a resource pair, Max # of resources and total # of Tx ports across all resource pairs} across all CCs simultaneously.; FFS; Per band and per BC; Component 2 candidate value set: {2, 3, 4, 5, 6, 7, 8} Component 3 candidate value set: {Mode1, Mode2, Mode1&2} Component 4: The candidate value set for Max # of Tx ports per resource within a resource pair is {2, 4, 8, 12, 16, 24, 32}, The candidate value set for Max # of resources is {2:2:64}, The candidate value set for total # of Tx ports across all resource pairs is {4:4:256} Note: ‘NCJT’ is not used in RAN1 specifications and will be aligned with 38.214; Optional with capability signalling. |
| ZTE [3] | Several UE features (23-7-1 ~ 23-7-5) are listed for MTRP CSI enhancements in [2]. We generally agree with these UE features, as they are aligned with previous agreements. Some suggestions are given as follows Proposal 13: For MTRP CSI enhancements - In FG 23-7-1, - a default value of Ks,max should be added as the basic component of this feature. We suggest Ks,max =4 as a compromised value between 2 and 8. - 24 ports of max # of Tx ports per source in a resource set in FG 23-7-2 should be supported by default |
| vivo [4] | In RAN1#107-e meeting, there are some remaining issues on this feature group, such as the maximum number of NZP CSI-RS resources in one CSI-RS resource set and CSI report mode selection. According to the following agreement, we think the value of Ks,max in basic FG is 2 because most Modes can be supported with Ks=2. While, other values should be given in a separate FG. Besides, we are not sure whether Ks,max is relevant to CMR sharing and the number of CPU. Agreement For a CSI-RS resource set with Ks NZP CSI-RS resources configured for CMR and N NZP CSI-RS resource pairs configured for NCJT measurement hypotheses, study following default value of Ks,max, - Alt 1: Ks,max = 4 - Alt 2: Ks,max = 2 - Alt 3: Ks,max = 4 for FR2, and Ks,max = 2 for FR1 - Note that default value means the minimal supported value for Ks,max in UE capability reporting, if UE support this feature. According to the following agreement on CSI feedback mode, we propose Mode1 with X=0 and Mode2 should be included in the basic FG. The Mode1 with X=1/2 can be in a separate FG. Agreement For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, support following two options: - Option 1: the UE can be configured to report X CSIs associated with single-TRP measurement hypotheses and one CSI associated with NCJT measurement hypothesis - X = 0, 1, 2 - If X=2, two CSIs are associated with two different single-TRP measurement hypotheses with CMRs from different CMR groups - Support of X=1,2 is UE optional for the UE supporting option 1 - FFS omission of CSI associated with NCJT measurement hypothesis - Option 2: the UE can be configured to report one CSI associated with the best one among NCJT and single-TRP measurement hypotheses - FFS how to report recommended measurement hypothesis associated with that CSI report Proposal 7-1: update the following FG to support a separate FG for other supported value(s) of Ks,max and additional supported Mode(s). 23. NR_FeMIMO; 23-7-1; Basic Features of CSI Enhancement for Multi-TRP; 1. Maximum number of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Nmax=1 2. Maximum number of NZP CSI-RS resources in one CSI-RS resource set per CMR group: Ks,max=2[2,4] 3. Support Mode 1 with X=0 and Mode 2. 3. FFS others (, codebook types, number of ports of CMRs, CMR sharing among NCJT and sTRP measurement hypotheses for FR1, two CMR groups with Ks=K1+K2 CMRs in CSI-RS resource set, reporting of 2 PMIs, 2 RIs and 2 Lis for NCJT measurement hypothesis …). 23-7-1a; Other supported value(s) of Ks,max; Supported value(s) of Ks,max more than 2. 23-7-1b; Supported Mode(s); Supported additional Modes. |
| CATT [5] | For the Features of CSI Enhancement for Multi-TRP, we support that UE reports the basic capability of the value of Ks,max in FG 23-7-1 and additionally reports the enhanced capability of Ks,max =8 in FG 23-7-3. Moreover, according to the agreement on CSI feedback mode, we propose Mode1 with X=0 and Mode2 should be incldued in the basic FG. The Mode1 with X=1/2 can be in a separate FG. Proposal-21: For CSI enhancement for Multi-TRP, the UE feature 23-7-1 is revised as follows: 23-7-1; Basic Features of CSI Enhancement for Multi-TRP; 1. Maximum number of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Nmax=1 2. FFS: Maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks,max 3. CSI report mode selection (mode 1 with X=0 / mode 2) FFS others (supported options; values for X, codebook types, number of ports of CMRs, CMR sharing among NCJT and sTRP measurement hypotheses for FR1, two CMR groups with Ks=K1+K2 CMRs in CSI-RS resource set, reporting of 2 PMIs, 2 RIs and 2 Lis for NCJT measurement hypothesis …); Optional with capability signalling. 23-7-1a; Supported CSI reporting Mode(s); Supported additional Mode1 with X=1/2; 23-7-1; Optional with capability signalling. |
| Samsung [6] | Regarding FG 23-7-1, we would like to include the following components of UE basic feature Proposal 26: Support 23-7-1, i.e., basic features of CSI enhancement for Multi-TRP. - Support Nmax=1 as a component of UE basic feature for Multi-TRP CSI - Support of a tuple: {the max # of ports per a resource, the max # of resources in a resource set}. - Include candidate value 16 Tx ports per resource, i.e., the tuple {16, Ks}. - Include CSI report modes, i.e., Mode 1 (Option 1) with X=0 and Mode 2 (Option 2), as a component of UE basic feature for mTRP CSI - For FR1 include CMR sharing between NCJT and sTRP measurement hypotheses as a component of UE basic feature |
| NTT DOCOMO, INC. [7] | We think 23-7-3 could be merged into 23-7-1 with introduction of a default value as basic feature of MTRP CSI enhancement. We prefer the default value of Ks,max =4 but we can also accept Ks,max=2. 23. NR_FeMIMO; 23-7-1; Basic Features of CSI Enhancement for Multi-TRP; - Maximum number of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Nmax=1 - Maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks,max - CSI report mode selection FFS others (supported options; values for X, codebook types, number of ports of CMRs, CMR sharing among NCJT and sTRP measurement hypotheses for FR1, two CMR groups with Ks=K1+K2 CMRs in CSI-RS resource set, reporting of 2 PMIs, 2 RIs and 2 Lis for NCJT measurement hypothesis …); FFS; [Per band / per BC]; [Component 2 candidate value set: {4, 5, 6, 7, 8} Note: ‘NCJT’ is not used in RAN1 specifications and will be aligned with 38.214; Optional with capability signalling. |
| Spreadtrum Communications [8] | In LTE NC-JT, only two CMR resources are configured to be measured for one CSI report. Likewise, at least for FR1, Ks,max =2 is enough for NR system. For FR2, Ks,max =4 seems to be capable of providing more measurement results with the assumption of multiple different beams in a CSI report. However, beam measurement and reporting could be firstly carried out before CSI acquisition, and proper beam could be selected out for CSI measurement. Thus, we think Ks,max =2 is enough for both FR1 and FR2. Proposal 10: For component 2 of FG 23-7-1, Maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks,max =2 |
| Qualcomm Incorporated [9] | The following aspects need to be considered when designing UE capability for mTRP CSI: - The basic FG need to consider the overall complexity for CSI processing across all CCs and all CSI report settings even when some of them are not configured with NCJT CSI - The FG needs to be consistent with the Rel-15 and Rel-16 CSI capability framework (e.g., FGs 2-35 and 2-36) - Due to complexity for UE to calculate two PMIs jointly for NCJT, a list of vectors for NCJT capabilities and sTRP capabilities are needed. - The complexity associated with a NCJT CSI can be much more than twice of the complexity associated with a sTRP CSI. For example, if UE evaluates 8 PMI/RI hypos for a given sTRP CSI, UE may need to evaluate up to 8*8=64 hypos for a given NCJT CSI for optimal results and not just 16 hypos, where the exact additional complexity may depend on UE implementation, and hence, it may make sense to separate the CPU/resource/port occupation budget. - It is important that this list is included in the main (basic) FG for mTRP CSI since otherwise a default assumption would be needed, which may not be possible to have since these capabilities are across all CSI report settings and across all CCs (some of which may not be even configured with NCJT CSI). - Note that proposed components 3 and 4 are extensions of Rel-15 components 1 and 2 of FG 2-36, and proposed components 5 and 6 are extensions of Rel-15 componets 8 and 9 of FG 2-35. - Proposed component 3 should be reported per band and separately per band combination. This is same as legacy behavior for reporting triplets for each codebook per band and separately per band combination. - Proposed component 6 should be reported per band combination similar to Rel-15 capability “simultaneousCSI-ReportsAllCC”. - For NCJT CSI reporting, UE should be able to indicate whether option 1, option 2, or both are supported. - For Option 1, UE should be able to indicate which values of X (X=0,1,2) are supported. - Default capability for Option 1 is X=0. 23. NR_FeMIMO; 23-7-1; Basic Features of CSI Enhancement for Multi-TRP; 1. Maximum number of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Nmax=1 2. CSI report mode selection 3. A list of supported combinations, each combination is {max # of ports per resource for NCJT, max # of resources for NCJT, max # of total ports for NCJT, max # of ports per resource for sTRP, max # of resources for sTRP, max # of total ports for sTRP} simultaneously across all CCs 4. Supported codebook modes for NCJT CSI 5. A list of (Y1,Y2): UE can process Y1 NCJT CSI report(s) and Y2 sTRP CSI report(s) simultaneously in a CC 6. A list of (X1,X2): UE can process X1 NCJT CSI report(s) and X2 sTRP CSI report(s) simultaneously across all CCs; Per Band Per Band Combination; Component 2: {Option 1, Option 2, both} If UE supports ‘Option 1’ or ‘both’, X=0 CSIs associated with single-TRP measurement hypotheses is assumed Compenent 3: The list can have maximum of 16 combinations. For each combination, candidate values are - max # of ports per resource for NCJT: {2, 4, 8, 12, 16, 24, 32} - max # of resources for NCJT: {2 to 64} - max # of total ports for NCJT: {4 to 256} - Max # of Tx ports in one resource for sTRP: {2, 4, 8, 12, 16, 24, 32} - Max # resources for sTRP: {1 to 64} - Max # total ports for sTRP: {2 to 256} Note: Component 3 is also reported per band combination (in addition to per band). Compenent 4:{ mode 1, both mode 1 and mode 2} Compoentn 5: The list can have maximum of 16 pairs. - Y1: {1 to 4} - Y2: {1 to 8} Compoentn 6: The list can have maximum of 16 pairs. - X1: {1 to 16} - X2: {1 to 32} Note: When no NCJT CSI is configured/active, sTRP CSI capabilities are based on Rel-15/16 capabilities.; Optional with capability signalling. |
| OPPO [10] | It was agreed that UE can report the maximal supported number of CMRs within a set as Ks,max. In FR1, the same CMRs can be reused for S-TRP and NC-JT, and Ks=2 can satisfy most use cases for M-TRP. In FR2, if UE supports CMR sharing between S-TRP and NC-JT or if UE only supports option 1 with X=0, it can also report Ks,max=2 as the minimal capability. Hence, for UE capability, the minimal supported value of Ks,max should be 2. Proposal 9: Support at least the following basic features for mTRP CSI: 23. NR_FeMIMO; 23-7-1; Basic Features of CSI Enhancement for Multi-TRP; - Maximum number of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Nmax=1 - Maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks,max=2 - Supported number of CSI-RS ports per CMR within a CMR pair - Supported CSI report mode; FFS; [Per band / per BC]; Component 3 candidate value set: {2, 4, 8,12,16, 24,32} Component 4 candidate value set: {mode 1 with X=0, mode 2, both} Note: ‘NCJT’ is not used in RAN1 specifications and will be aligned with 38.214; Optional with capability signalling. Proposal 10: Support the following features as separate FG: - Maximum number of NZP CSI-RS resource pairs used as CMR pairs for NCJT measurement hypothesis: Nmax=2 - Maximum number of NZP CSI-RS resources in one CSI-RS resource set other than 2, with candidate values of {4, 6, 8} - Supported CSI report mode 1 other than X=0, with candidate values of {mode 1 with X=1, mode 1 with X=2, both} - CMR sharing for FR2 |
| Intel Corporation [11] | We have the following comments for the UE feature list for MTRP CSI. - Term NCJT seems confusing and can be avoided in the UE capability description. CMR pair can be used without additional clarifications that it is related to NCJT. - For component 2 in FG 23-7-1, FFS can be removed, the current value range is reasonable. - For component 3 in FG 23-7-1, in our view both Mode 1 and Mode 2 should be supported if UE indicates support of 23-7-1 - We propose to introduce new component for FG 23-7-1 which considers support for values of parameter X (number of Single-TRP CSI measurement hypothesis for CSI report mode 1); in our view X = 0,1,2 should be supported if UE indicates support of FG 23-7-1. - In our view per band signaling for FG 23-7-1 and 23-7-2 corresponds to reasonable tradeoff between UE implementation flexibility and signaling overhead 23-7-1; Basic Features of CSI Enhancement for Multi-TRP; - Maximum number of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Nmax=1 - FFS: Maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks,max - CSI report mode selection: Mode 1 and Mode 2 - Number of Single-TRP CSI measurement hypothesis X = 0,1,2 for CSI report mode 1 FFS others (supported options; values for X, codebook types, number of ports of CMRs, CMR sharing among NCJT and sTRP measurement hypotheses for FR1, two CMR groups with Ks=K1+K2 CMRs in CSI-RS resource set, reporting of 2 PMIs, 2 RIs and 2 Lis for NCJT measurement hypothesis …); [Component 2 candidate value set: {2, 3, 4, 5, 6, 7, 8}] Note: ‘NCJT’ is not used in RAN1 specifications and will be aligned with 38.214; [Per band / per BC]. |
| Apple [12] | - FG23-7-1: we propose to add two components for the maximum total number of ports and the X related to the joint Single-TRP and Multi-TRP measurement reporting 23. NR_FeMIMO; 23-7-1; Basic Features of CSI Enhancement for Multi-TRP; - Maximum number of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Nmax=1 - FFS: Maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks,max - CSI report mode selection - Support of maximum total number of ports across two CMRs in a pair of CMRs for Multi-TRP CSI measurement - Support of X CSI associated with single-TRP measurement hypotheses and one CSI associated with NCJT measurement hypothesis FFS others (supported options; values for X, codebook types, number of ports of CMRs, CMR sharing among NCJT and sTRP measurement hypotheses for FR1, two CMR groups with Ks=K1+K2 CMRs in CSI-RS resource set, reporting of 2 PMIs, 2 RIs and 2 Lis for NCJT measurement hypothesis …); FFS; [Per band / per BC]; [Component 2 candidate value set: {2, 3, 4, 5, 6, 7, 8}] Component 4: {4,8,12,16,24,32,48,64} Component 5: bitmap {0, 1, 2} Note: ‘NCJT’ is not used in RAN1 specifications and will be aligned with 38.214; Optional with capability signalling. |
| FUTUREWEI [13] |  |
| xiaomi [14] |  |
| LG Electronics [15] |  |
| Lenovo/Motorola Mobility [16] | For Feature group 23-7-1, it was agreed in RAN1#106bis-e [4] that for CSI measurement associated with a CSI-ReportingConfig for NCJT, two CMRs within the same CMR pair configured for NCJT measurement hypothesis are restricted within X continuous slot(s) without DL/UL switch between two CMRs, where X=1,2. We propose setting X=2 as a basic feature of CSI Enhancement for Multi-TRP under FG 23-7-1, whereas X=1 is set as an optional feature. For the basic UE FG 23-7-1 for CSI enhancements under multi-TRP, add a component indicating that two CMRs within the same CMR pair configured for NCJT measurement hypothesis are restricted within X=2 continuous slots Add an optional UE FG 23-7-6 for CSI enhancements under multi-TRP, indicating that two CMRs within the same CMR pair configured for NCJT measurement hypothesis are restricted within X=1 slot Regarding the codebook type for FG 23-7-1, it was agreed to limit the codebook type for NCJT CSI reporting to Type-I single-panel codebook. Hence, codebook type should not be discussed under UE features. Also, in our understanding, CMR sharing among NCJT and sTRP measurement hypotheses for FR1 should not incur additional complexity/overhead, and hence no associated UE features are needed. FG 23-7-1 should not include parameters/components corresponding to either codebook type or CMR sharing among NCJT and sTRP measurement hypotheses for FR1 |
| Ericsson [17] | Since ‘CSI report mode selection’ is supported as part of the basic feature for multi-TRP CSI, it is also important to support the different values of X supported in case of mode 1. Hence, we propose to add ‘number of single-TRP CSIs reported with NC-JT CSI’ as a component of FG 23-7-1. Add ‘number of single-TRP CSIs reported with NC-JT CSI’ as a new component of FG 23-7-1. |
| MediaTek Inc. [18] | This baseline FG needs to be consistent with the Rel-15 and Rel-16 CSI capability framework design. As of definitions specified in RAN1 106 bis-e we don’t find this consistency. Hence, we proposal to include additional components to this FG. Furthermore, we propose to merge FG 23-7-2 and FG 23-7-3 to the basic FG, i.e., 23-7-1. This merge would allow for more concise and clear definitions. Proposal 51: Merge FG 23-7-2 and FG 23-7-3 with FG 23-7-1. Proposal 52: Add the following components to the basic FG: - Support of two resource groups and one resource pair in one CSI-RS resource set and support of Type-I single-panel codebook with codebook mode set to ‘Mode 1’ - Maximum value of numberOfSingleTRP-CSI-Mode1: {0, 1, 2} - Maximum number of NZP CSI-RS resources configured in one CSI resource set: {2, 3, 4, 5, 6, 7, 8} - A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is { - Maximum number of Tx ports in one NZP CSI-RS resource associated with a single-TRP measurement hypothesis: {4, 8, 12, 16, 24, 32} - Maximum number of Tx ports in one NZP CSI-RS resource associated with an NCJT measurement hypothesis: {4, 8, 12, 16, 24, 32} - Maximum total number of calculated single-TRP measurement hypotheses: {from 1 to 64} - Maximum total number of calculated NCJT measurement hypotheses: {from 1 to 16} - Maximum total number of Tx ports of NZP CSI-RS resources associated with single-TRP measurement hypotheses: {4, 8, 12, …, 256} - Maximum total number of Tx ports of NZP CSI-RS resources associated with NCJT measurement hypotheses: {4, 8, 12, …, 256} } - UE can process Y1 CSI report(s) for single-TRP and Y2 report(s) for NCJT simultaneously in a CC: - Y1: FFS range, Y2: FFS range - UE can process X1 CSI report(s) for single-TRP and X2 report(s) for NCJT simultaneously across all CCs: - X1: FFS range, X2: FFS range 23. NR_FeMIMO; 23-7-1; Basic Features of CSI Enhancement for Multi-TRP; - Support of two resource groups and one resource pair in one CSI-RS resource set and support of Type-I single-panel codebook with codebook mode set to ‘Mode 1’ - Maximum number of NZP CSI-RS resources in one CSI resource set: Ks,max - CSI report mode selection - Maximum value of numberOfSingleTRP-CSI-Mode1 - A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is { - Maximum number of Tx ports in one NZP CSI-RS resource associated with a single-TRP measurement hypothesis: {4, 8, 12, 16, 24, 32} - Maximum number of Tx ports in one NZP CSI-RS resource associated with an NCJT measurement hypothesis: {4, 8, 12, 16, 24, 32} - Maximum total number of calculated single-TRP measurement hypotheses: {from 1 to 64} - Maximum total number of calculated NCJT measurement hypotheses: {from 1 to 16} - Maximum total number of Tx ports of NZP CSI-RS resources associated with single-TRP measurement hypotheses: {4, 8, 12, …, 256} - Maximum total number of Tx ports of NZP CSI-RS resources associated with NCJT measurement hypotheses: {4, 8, 12, …, 256} } - UE can process Y1 CSI report(s) for single-TRP and Y2 report(s) for NCJT simultaneously in a CC - UE can process X1 CSI report(s) for single-TRP and X2 report(s) for NCJT simultaneously across all CCs; Component 2: Candidate values: {2, 3, 4, 5, 6, 7, 8} Component 3: Candidate values: {Mode 1 only, Both Mode 1 and Mode 2} Component 4: Candidate values: {0, 1, 2}; Optional with capability signalling. |
| CMCC [19] | About the component 2, we think the maximum number of NZP CSI-RS resources should be included as a basic feature for Multi-TRP CSI enhancement. In addition, at least in FR2, if the CMR sharing between Single-TRP and NCJT is not supported, the Ks,max should be equal to or larger than 4, since two NZP CSI-RS resources are used for NCJT measurement hypothesis and another two NZP CSI-RS resources are used for two different Single-TRP measurement hypothesis. Proposal 9: Maximum number of NZP CSI-RS resources should be included in FG 23-7-1 and the Ks,max should be equal to or larger than 4. |
| Nokia/Nokia Shanghai Bell [20] | - Component 1: A basic configuration for a MTRP CSI resource set always requires N=1 Resource Pair, so the reference to Nmax can be dropped. The component can be described as “Support of N=1 Resource Pair in an NZP CSI-RS Resource Set for channel measurement” (and the note on NCJT can be removed as it becomes obsolete). - Component 2: it was agreed to discuss the default value of Ks, max with UE capability. In our view the minimum value should be Ks,max=4 because, if sharedCMR is absent and N = 1, M1 = K1 - 1 and M2 = K2 – 1 (see R1-2112472, Sec. 6.3.1.1.2), hence K1≥2, K2≥2 and the minimum value of Ks=K1+K2=4 Agreement(RAN1#105-e) For a CSI-RS resource set with Ks NZP CSI-RS resources configured for CMR and N NZP CSI-RS resource pairs configured for NCJT measurement hypotheses, study following default value of Ks,max, - Alt 1: Ks,max = 4 - Alt 2: Ks,max = 2 - Alt 3: Ks,max = 4 for FR2, and Ks,max = 2 for FR1 - Note that default value means the minimal supported value for Ks,max in UE capability reporting, if UE support this feature. Conclusion: (RAN1#106-e) Default value of Ks, max can be discussed later with Rel-17 MIMO UE capability. - New Component 3: ”Support of two Resource Groups in an NZP CSI-RS Resource Set for channel measurement, with K1≥1 resources in Group 1 and K2≥1 resources in Group 2”. Candidate values for K1 and K2: {1,2,3,4,5,6,7} - New Component 4: ”Support of csi-ReportMode 1 and 2” - New Component 5: ”Support of numberOfSingleTRP-CSI-Mode1 X=0” |

| 23. NR_FeMIMO | 23-7-2 | Support of max # of Tx ports [per source/across two CMRs] [in a resource set for Multi-TRP CSI] [and max # resources] | [A list of supported combinations, each combination is {max # of Tx ports per source in a resource set for Multi-TRP CSI, max # resources in a resource set for Multi-TRP CSI}] [Note: same number of ports among CMRs] |  |  |  |  |  |  |  |  | [{4, 8, 12, 16, 24, 32}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| ZTE [3] | Several UE features (23-7-1 ~ 23-7-5) are listed for MTRP CSI enhancements in [2]. We generally agree with these UE features, as they are aligned with previous agreements. Some suggestions are given as follows Proposal 13: For MTRP CSI enhancements - FG 23-7-2 is used to support 32 ports per source in a resource set |
| vivo [4] |  |
| CATT [5] |  |
| Samsung [6] | Regarding 23-7-2, similar to one of the components of FG 23-7-1, it can be reported as tuple, i.e., {the max # of ports per a resource, the max # of resources in a resource set}. The main objective of this feature group would be to indicate, as optional feature, the support of >16 ports per a CMR, i.e., >32 ports across two CMRs in a CMR pair. Proposal 27: Support 23-7-2 to be reported as tuples, i.e., {the max # of ports per a resource, the max # of resources in a resource set}. - Support candidate values of the maximum # ports as 24 and 32, i.e., the tuples {24, Ks} and {32, Ks}, as UE optional feature. - Default Ks,max can be set as: - Ks,max=8 when CMR sharing is not enabled. - Ks,max=4 when CMR sharing is enabled |
| NTT DOCOMO, INC. [7] | For 23-7-2, we do not think it is needed but we can live with it. Current candidate values for 23-7-2 is not aligned with the combinations. |
| Spreadtrum Communications [8] |  |
| Qualcomm Incorporated [9] | The following aspects need to be considered when designing UE capability for mTRP CSI: - The basic FG need to consider the overall complexity for CSI processing across all CCs and all CSI report settings even when some of them are not configured with NCJT CSI - The FG needs to be consistent with the Rel-15 and Rel-16 CSI capability framework (e.g., FGs 2-35 and 2-36) - Due to complexity for UE to calculate two PMIs jointly for NCJT, a list of vectors for NCJT capabilities and sTRP capabilities are needed. - The complexity associated with a NCJT CSI can be much more than twice of the complexity associated with a sTRP CSI. For example, if UE evaluates 8 PMI/RI hypos for a given sTRP CSI, UE may need to evaluate up to 8*8=64 hypos for a given NCJT CSI for optimal results and not just 16 hypos, where the exact additional complexity may depend on UE implementation, and hence, it may make sense to separate the CPU/resource/port occupation budget. - It is important that this list is included in the main (basic) FG for mTRP CSI since otherwise a default assumption would be needed, which may not be possible to have since these capabilities are across all CSI report settings and across all CCs (some of which may not be even configured with NCJT CSI). - Note that proposed components 3 and 4 are extensions of Rel-15 components 1 and 2 of FG 2-36, and proposed components 5 and 6 are extensions of Rel-15 componets 8 and 9 of FG 2-35. - Proposed component 3 should be reported per band and separately per band combination. This is same as legacy behavior for reporting triplets for each codebook per band and separately per band combination. - Proposed component 6 should be reported per band combination similar to Rel-15 capability “simultaneousCSI-ReportsAllCC”. - For NCJT CSI reporting, UE should be able to indicate whether option 1, option 2, or both are supported. - For Option 1, UE should be able to indicate which values of X (X=0,1,2) are supported. - Default capability for Option 1 is X=0. 23. NR_FeMIMO; 23-7-2; Supported value of X for Option 1; Supported values of X (reported CSIs associated with single-TRP measurement hypotheses) for Option 1; FG 23-7-1; Per Band; Note: UE supporting this feature indicates ‘Option 1’ or ‘both’ in component 4 of FG 23-7-1. Candidate values: {1, 2, 1and2}; Optional with capability signalling. |
| OPPO [10] |  |
| Intel Corporation [11] | - In our view per band signaling for FG 23-7-1 and 23-7-2 corresponds to reasonable tradeoff between UE implementation flexibility and signaling overhead - For FG 23-7-2, max # of Tx ports per source and max # resources in a resource set for Multi-TRP CSI is enough to indicate UE capabilities for multi-TRP CSI considering that other UE capabilities can be used (e.g. number of CPUs, Type I PMI codebooks capabilities, CSI-RS capabilities). 23-7-2; Support of max # of Tx ports [per source/across two CMRs] [in a resource set for Multi-TRP CSI] [and max # resources] in a resource set for Multi-TRP CSI; [A list of supported combinations, each combination is {max # of Tx ports per source in a resource set for Multi-TRP CSI, max # resources in a resource set for Multi-TRP CSI}] [Note: same number of ports among CMRs]; Max # of Tx ports per source: [{4, 8, 12, 16, 24, 32}] Max # resources: {2, 3, 4, 5, 6, 7, 8}; Per band. |
| Apple [12] |  |
| FUTUREWEI [13] |  |
| xiaomi [14] |  |
| LG Electronics [15] |  |
| Lenovo/Motorola Mobility [16] |  |
| Ericsson [17] | For maximum number of CSI-RS ports per NZP CSI-RS resource, the value of 32 should be supported. For maximum number of CSI-RS ports per NZP CSI-RS resource, support 32 as a candidate value. |
| MediaTek Inc. [18] | This baseline FG needs to be consistent with the Rel-15 and Rel-16 CSI capability framework design. As of definitions specified in RAN1 106 bis-e we don’t find this consistency. Hence, we proposal to include additional components to this FG. Furthermore, we propose to merge FG 23-7-2 and FG 23-7-3 to the basic FG, i.e., 23-7-1. This merge would allow for more concise and clear definitions. Proposal 51: Merge FG 23-7-2 with FG 23-7-1. |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] | - Confirm the FG. Details to be further discussed. - Propose to rename to “Support of more than 16 Tx ports in one resource”. Based on the following agreement, up to 16 Tx ports per resource (i.e. 32 across a Resource Pair) should be supported as part of basic capability Agreement(RAN1#103-e) - The support of larger than 32 ports across two CMRs is optional for a UE supporting Rel. 17 mTRP CSI |

| 23. NR_FeMIMO | 23-7-3 | More than two resources in a resource set for Multi-TRP CSI | FFS exact candidate values, Ks,max is up to 8 |  |  |  |  |  |  |  |  | [candidate values are {2, 3, 4, 5, 6, 7, 8}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| ZTE [3] | Several UE features (23-7-1 ~ 23-7-5) are listed for MTRP CSI enhancements in [2]. We generally agree with these UE features, as they are aligned with previous agreements. Some suggestions are given as follows Proposal 13: For MTRP CSI enhancements - FG 23-7-3 should be removed as it is similar as component 2 of FG 23-7-1 |
| vivo [4] |  |
| CATT [5] |  |
| Samsung [6] | Regarding 23-7-3 it can be merged with 23-7-2. Proposal 28: Merge 23-7-3 with 23-7-2. |
| NTT DOCOMO, INC. [7] | We think 23-7-3 could be merged into 23-7-1 with introduction of a default value as basic feature of MTRP CSI enhancement. We prefer the default value of Ks,max =4 but we can also accept Ks,max=2. |
| Spreadtrum Communications [8] |  |
| Qualcomm Incorporated [9] | The following aspects need to be considered when designing UE capability for mTRP CSI: - The basic FG need to consider the overall complexity for CSI processing across all CCs and all CSI report settings even when some of them are not configured with NCJT CSI - The FG needs to be consistent with the Rel-15 and Rel-16 CSI capability framework (e.g., FGs 2-35 and 2-36) - Due to complexity for UE to calculate two PMIs jointly for NCJT, a list of vectors for NCJT capabilities and sTRP capabilities are needed. - The complexity associated with a NCJT CSI can be much more than twice of the complexity associated with a sTRP CSI. For example, if UE evaluates 8 PMI/RI hypos for a given sTRP CSI, UE may need to evaluate up to 8*8=64 hypos for a given NCJT CSI for optimal results and not just 16 hypos, where the exact additional complexity may depend on UE implementation, and hence, it may make sense to separate the CPU/resource/port occupation budget. - It is important that this list is included in the main (basic) FG for mTRP CSI since otherwise a default assumption would be needed, which may not be possible to have since these capabilities are across all CSI report settings and across all CCs (some of which may not be even configured with NCJT CSI). - Note that proposed components 3 and 4 are extensions of Rel-15 components 1 and 2 of FG 2-36, and proposed components 5 and 6 are extensions of Rel-15 componets 8 and 9 of FG 2-35. - Proposed component 3 should be reported per band and separately per band combination. This is same as legacy behavior for reporting triplets for each codebook per band and separately per band combination. - Proposed component 6 should be reported per band combination similar to Rel-15 capability “simultaneousCSI-ReportsAllCC”. - For NCJT CSI reporting, UE should be able to indicate whether option 1, option 2, or both are supported. - For Option 1, UE should be able to indicate which values of X (X=0,1,2) are supported. - Default capability for Option 1 is X=0. 23. NR_FeMIMO; 23-7-3; More than two resources in a resource set for Multi-TRP CSI; Supported maximum # of CSI-RS resources in a resource set for Multi-TRP CSI (value Ks,max ); FG 23-7-1; Per Band; Candidate values: {3…8}; Optional with capability signalling. |
| OPPO [10] |  |
| Intel Corporation [11] | FG 23-7-3 is not needed considering component 2 for FG 23-7-1. 23-7-3; More than two resources in a resource set for Multi-TRP CSI; FFS exact candidate values, Ks,max is up to 8; [candidate values are {2, 3, 4, 5, 6, 7, 8}]. |
| Apple [12] |  |
| FUTUREWEI [13] |  |
| xiaomi [14] |  |
| LG Electronics [15] |  |
| Lenovo/Motorola Mobility [16] |  |
| Ericsson [17] |  |
| MediaTek Inc. [18] | This baseline FG needs to be consistent with the Rel-15 and Rel-16 CSI capability framework design. As of definitions specified in RAN1 106 bis-e we don’t find this consistency. Hence, we proposal to include additional components to this FG. Furthermore, we propose to merge FG 23-7-2 and FG 23-7-3 to the basic FG, i.e., 23-7-1. This merge would allow for more concise and clear definitions. Proposal 51: Merge FG 23-7-3 with FG 23-7-1. |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] | - Confirm the FG. Details to be further discussed. - Propose to rename to: “Maximum number of resources Ks,max” with description: “Support of up to Ks,max resources in an NZP CSI-RS Resource Set for channel measurement configured with two Resource Groups and N Resource Pairs”. Candidate values should be {4,5,6,7,8}. |

| 23. NR_FeMIMO | 23-7-4 | Support of Nmax=2 for Multi-TRP CSI | Support of maximum number of CMR pairs Nmax=2 configured in NZP-CSI-RS-ResourceSet for a given CSI report setting | 23-7-1 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| ZTE [3] | Several UE features (23-7-1 ~ 23-7-5) are listed for MTRP CSI enhancements in [2]. We generally agree with these UE features, as they are aligned with previous agreements. Some suggestions are given as follows Proposal 13: For MTRP CSI enhancements - FG 23-7-4 is used to support of Nmax=2. |
| vivo [4] |  |
| CATT [5] |  |
| Samsung [6] | Proposal 29: Support 23-7-4, i.e., the maximum number of CMR pairs (Nmax), as UE optional feature - Support Nmax=2 as UE optional feature |
| NTT DOCOMO, INC. [7] |  |
| Spreadtrum Communications [8] |  |
| Qualcomm Incorporated [9] | The following aspects need to be considered when designing UE capability for mTRP CSI: - The basic FG need to consider the overall complexity for CSI processing across all CCs and all CSI report settings even when some of them are not configured with NCJT CSI - The FG needs to be consistent with the Rel-15 and Rel-16 CSI capability framework (e.g., FGs 2-35 and 2-36) - Due to complexity for UE to calculate two PMIs jointly for NCJT, a list of vectors for NCJT capabilities and sTRP capabilities are needed. - The complexity associated with a NCJT CSI can be much more than twice of the complexity associated with a sTRP CSI. For example, if UE evaluates 8 PMI/RI hypos for a given sTRP CSI, UE may need to evaluate up to 8*8=64 hypos for a given NCJT CSI for optimal results and not just 16 hypos, where the exact additional complexity may depend on UE implementation, and hence, it may make sense to separate the CPU/resource/port occupation budget. - It is important that this list is included in the main (basic) FG for mTRP CSI since otherwise a default assumption would be needed, which may not be possible to have since these capabilities are across all CSI report settings and across all CCs (some of which may not be even configured with NCJT CSI). - Note that proposed components 3 and 4 are extensions of Rel-15 components 1 and 2 of FG 2-36, and proposed components 5 and 6 are extensions of Rel-15 componets 8 and 9 of FG 2-35. - Proposed component 3 should be reported per band and separately per band combination. This is same as legacy behavior for reporting triplets for each codebook per band and separately per band combination. - Proposed component 6 should be reported per band combination similar to Rel-15 capability “simultaneousCSI-ReportsAllCC”. - For NCJT CSI reporting, UE should be able to indicate whether option 1, option 2, or both are supported. - For Option 1, UE should be able to indicate which values of X (X=0,1,2) are supported. - Default capability for Option 1 is X=0. 23. NR_FeMIMO; 23-7-4; Support of Nmax=2 for Multi-TRP CSI; Support of maximum number of CMR pairs Nmax=2 configured in NZP-CSI-RS-ResourceSet for a given CSI report setting; 23-7-1; Per band; Optional with capability signalling. |
| OPPO [10] |  |
| Intel Corporation [11] |  |
| Apple [12] |  |
| FUTUREWEI [13] |  |
| xiaomi [14] |  |
| LG Electronics [15] |  |
| Lenovo/Motorola Mobility [16] |  |
| Ericsson [17] |  |
| MediaTek Inc. [18] |  |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] |  |

| 23. NR_FeMIMO | 23-7-5 | CMR sharing | Support a NZP CSI-RS resource referred by both a CMR pair configured for NCJT measurement hypothesis and a CMR configured for Single-TRP measurement hypothesis | 23-7-1 |  |  |  | Per band |  | FR2 only |  | Note: ‘NCJT’ and ‘single-TRP’ are not used in RAN1 specifications and will be aligned with 38.214 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| ZTE [3] |  |
| vivo [4] |  |
| CATT [5] |  |
| Samsung [6] | Regarding 23-7-5, we support it for FR2. Proposal 30: Support 23-7-5, i.e., whether two CMRs from a CMR pair configured for a NCJT measurement hypothesis can be used for Single-TRP measurement hypotheses, as UE optional feature for FR2. |
| NTT DOCOMO, INC. [7] |  |
| Spreadtrum Communications [8] |  |
| Qualcomm Incorporated [9] | The following aspects need to be considered when designing UE capability for mTRP CSI: - The basic FG need to consider the overall complexity for CSI processing across all CCs and all CSI report settings even when some of them are not configured with NCJT CSI - The FG needs to be consistent with the Rel-15 and Rel-16 CSI capability framework (e.g., FGs 2-35 and 2-36) - Due to complexity for UE to calculate two PMIs jointly for NCJT, a list of vectors for NCJT capabilities and sTRP capabilities are needed. - The complexity associated with a NCJT CSI can be much more than twice of the complexity associated with a sTRP CSI. For example, if UE evaluates 8 PMI/RI hypos for a given sTRP CSI, UE may need to evaluate up to 8*8=64 hypos for a given NCJT CSI for optimal results and not just 16 hypos, where the exact additional complexity may depend on UE implementation, and hence, it may make sense to separate the CPU/resource/port occupation budget. - It is important that this list is included in the main (basic) FG for mTRP CSI since otherwise a default assumption would be needed, which may not be possible to have since these capabilities are across all CSI report settings and across all CCs (some of which may not be even configured with NCJT CSI). - Note that proposed components 3 and 4 are extensions of Rel-15 components 1 and 2 of FG 2-36, and proposed components 5 and 6 are extensions of Rel-15 componets 8 and 9 of FG 2-35. - Proposed component 3 should be reported per band and separately per band combination. This is same as legacy behavior for reporting triplets for each codebook per band and separately per band combination. - Proposed component 6 should be reported per band combination similar to Rel-15 capability “simultaneousCSI-ReportsAllCC”. - For NCJT CSI reporting, UE should be able to indicate whether option 1, option 2, or both are supported. - For Option 1, UE should be able to indicate which values of X (X=0,1,2) are supported. - Default capability for Option 1 is X=0. 23. NR_FeMIMO; 23-7-5; CMR sharing; Support a NZP CSI-RS resource referred by both a CMR pair configured for NCJT measurement hypothesis and a CMR configured for Single-TRP measurement hypothesis; FG 23-7-1; Per Band; FR2 only; Optional with capability signalling. |
| OPPO [10] |  |
| Intel Corporation [11] | 23-7-5; CMR sharing; Support a NZP CSI-RS resource referred by both a CMR pair configured for NCJT measurement hypothesis and a CMR configured for Single-TRP measurement hypothesis; Note: ‘NCJT’ and ‘single-TRP’ are not used in RAN1 specifications and will be aligned with 38.214; Per band. |
| Apple [12] |  |
| FUTUREWEI [13] |  |
| xiaomi [14] |  |
| LG Electronics [15] |  |
| Lenovo/Motorola Mobility [16] |  |
| Ericsson [17] |  |
| MediaTek Inc. [18] |  |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] |  |

| 23. NR_FeMIMO | 23-8-1 | SRS triggering offset enhancement | Support of determining aperiodic SRS location based on available slot [and dynamically based on Rel-17 introduced new field in DCI Format 0_1/0_2/1_1/1_2] |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| ZTE [3] | 23-8-1; SRS triggering offset enhancement; Support of determining aperiodic SRS location based on available slot [and dynamically based on Rel-17 introduced new field in DCI Format 0_1/0_2/1_1/1_2]. |
| vivo [4] |  |
| CATT [5] | For Rel-17 flexible AP-SRS triggering, the available slot is the one configured by RRC if only one vlue is configured by RRC and the available slot is dynamically indicated by the introduced new field in DCI format 0_1/0_2/1_1/1_2 if multiple values for available slot are configured by RRC. Current FG 23-8-1 is misleading as if the available slot cannot be determined by RRC. In order to avoid misleading, we prefer to delete the description on how to determine available slot in the component. Since how to determine available slot would be specified in RAN1 specs, including it in UE feature is not needed. Proposal-17: On FG 23-8-1, the following change is adopted: 23-8-1; SRS triggering offset enhancement; Support of determining aperiodic SRS location based on available slot [and dynamically based on Rel-17 introduced new field in DCI Format 0_1/0_2/1_1/1_2]. |
| Samsung [6] |  |
| NTT DOCOMO, INC. [7] | For FG23-8-1, we have the following two comments: - RAN1 has agreed to support cross carrier triggering of Rel-17 enhanced A-SRS, by reusing the same manner as Rel-16 A-SRS for determining reference slot. As no dedicated capability is defined for cross carrier triggering of A-SRS even in Rel-15 and Rel-16, we believe FG23-8-1 should explicitly cover the support of cross carrier triggering of Rel-17 enhanced A-SRS as well. - There is a part with bracket on the newly supported DCI field. We believe it should be captured as it is since FG23-8-1 should cover both the functionality to indicate the next available slot and the one to use the new DCI field for indicating t value. We do not see the need to define them as separate capabilities. 23. NR_FeMIMO; 23-8-1; SRS triggering offset enhancement for both self/cross carrier triggering; Support of determining aperiodic SRS location based on available slot and dynamically based on Rel-17 introduced new field in DCI Format 0_1/0_2/1_1/1_2; Optional with capability signalling. |
| Spreadtrum Communications [8] | Last meeting, this issue has been discussed, and one revised version has been proposed as shown below. 23-8-1; SRS triggering offset enhancement; The max number of available slots for determining aperiodic SRS location. We prefer this version, for the reason that not only it actually reflects the function of flexible SRS triggering, but also it could provide enough flexibility for gNB and also consider UE’s complexity. Proposal 11: Support FG23-8-1 to revise as ‘The max number of available slots for determining aperiodic SRS location’. |
| Qualcomm Incorporated [9] | In the last meeting, the supported maximum number of RRC configured available slots was discussed where candidate values are {none, 1, 2,3 4}. When a maximum of 1 configured values of available slot is supported, there is no need to have DCI bitfield SOI (Slot offset indicator). The description of the feature could be clarified as “the maximum number of configured available slots offsets for determining aperiodic SRS location based on available slot”. Proposal 9-1: Update the description of FG 23-8-1 as “the maximum number of configured available slots offsets for determining aperiodic SRS location based on available slot” with candidate values of {None, 1, 2 ,3 4}. 23. NR_FeMIMO; 23-8-1; SRS triggering offset enhancement; The maximum number of configured available slots offsets for determining aperiodic SRS location based on available slot; Per FS; {none, 1, 2,3 4}; Optional with capability signalling. |
| OPPO [10] | - Suggest to remove “[and dynamically based on Rel-17 introduced new field in DCI Format 0_1/0_2/1_1/1_2]” as the available slot can be indicated by RRC or DCI |
| Intel Corporation [11] | In FG 23-8-1, there was some discussion that whether let the UE reports the maximum number of available slots that can be configured to the UE. We think this is good way and the candidate value of this component could be {none, 1, 2, 3, 4}. If the value of “none” is reported, it means the UE doesn’t support the available slot operation for aperiodic SRS. The component description could be changed to “The max number of configured available slots offsets for determining aperiodic SRS location”. 23-8-1; SRS triggering offset enhancement; The max number of configured available slots offsets for determining aperiodic SRS location Support of determining aperiodic SRS location based on available slot [and dynamically based on Rel-17 introduced new field in DCI Format 0_1/0_2/1_1/1_2]; {none, 1, 2, 3, 4}. |
| Apple [12] |  |
| FUTUREWEI [13] | Some rows in the endorsed R1-2111807 [2] were discussed in RAN1#107-e meeting but no agreement was made. We can continue the discussion in the next meeting. Regarding the following FG: 23. NR_FeMIMO; 23-8-1; SRS triggering offset enhancement; Support of The max number of available slots for determining aperiodic SRS location based on available slot. There are two different interpretations for the intention of this FG: - One is for the UE to report whether to support the Rel-17 available slot offset or not, and if supported, the UE will support any (and all) of 0, 1, 2, 3, or 4 available slot offset values according to gNB configuration, where 0 corresponds to the legacy behavior. - The other is for the UE to report the supported maximum number of available slot offsets, where the reported number could be one of 0, 1, 2, 3, or 4, where 0 corresponds to the case the UE does not support the Rel-17 feature of available slot offset. If the one number reported by the UE is, say, 2, the UE will support any of 0, 1, or 2 available slot offset values according to gNB configuration, but the gNB is not allowed to configure 3 or 4 available slot offset values as it exceeds the UE reported capability. It seems that the second intention provides finer granularity of UE capability and hence more flexibility to UE implementation but does not add much complexity. Therefore, we suggest adopting the second intention. This finer granularity of UE capability has not been discussed in the SRS agenda item, so a new agreement is needed. Also in the last meeting, some companies pointed out that the description is not too clear. A better description is thus suggested below. Proposal 3: Adopt FG 23-8-1 as “The max number of available slots offsets for determining aperiodic SRS location that can be configured for the UE”. |
| xiaomi [14] | From our point of view, we prefer the current version without the highlighted part. It is because the UE supporting this feature does not use the DCI field for dynamic indication when none of the resource sets is configured with “t” values or the maximum number of ‘t’ values configured for all the aperiodic SRS resource sets across all configured BWPs in a CC for SRS transmission is one. Proposal 18: The proposed FG23-8-1 for SRS triggering offset enhancement is 23. NR_FeMIMO; 23-8-1; SRS triggering offset enhancement; Support of determining aperiodic SRS location based on available slot [and dynamically based on Rel-17 introduced new field in DCI Format 0_1/0_2/1_1/1_2]; Optional with capability signalling. |
| LG Electronics [15] | - Support the version in the first round discussion on RAN1 #107-e, as below table. Proposal 10: Adopt the following table for Rel-17 SRS. 23. NR_FeMIMO; 23-8-1; SRS triggering offset enhancement; Support of determining aperiodic SRS location based on available slot; 2-52; Optional with capability signalling. |
| Lenovo/Motorola Mobility [16] |  |
| Ericsson [17] |  |
| MediaTek Inc. [18] |  |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] |  |

| 23. NR_FeMIMO | 23-8-2 | Triggering SRS only in DCI 0_1/0_2 | Support of triggering SRS in DCI 0_1/0_2 without data and without CSI | 2-52 |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| ZTE [3] | Eight UE features (23-8-1 ~ 23-8-8) are listed for SRS enhancements in [1]. We generally agree with having these UE features, as they are aligned with previous agreements. The major remaining issue is the types of these UE features. In general, from NW perspective, it is desirable to avoid too small granularity (e.g., per BC) for the SRS features. For instance, 23-8-2 (Triggering SRS only in DCI 0_1/0_2) is not related with any band-specific or BC-specific characteristics. Hence it should be per UE. Proposal 14: Strive to avoid per BC features for SRS, e.g., the type of 23-8-2 (Triggering SRS only in DCI 0_1/0_2) should be ‘per UE’. |
| vivo [4] |  |
| CATT [5] |  |
| Samsung [6] |  |
| NTT DOCOMO, INC. [7] |  |
| Spreadtrum Communications [8] |  |
| Qualcomm Incorporated [9] | 23. NR_FeMIMO; 23-8-2; Triggering SRS only in DCI 0_1/0_2; Support of triggering SRS in DCI 0_1/0_2 without data and without CSI; 2-52; Per FS; Optional with capability signalling. |
| OPPO [10] |  |
| Intel Corporation [11] |  |
| Apple [12] |  |
| FUTUREWEI [13] |  |
| xiaomi [14] |  |
| LG Electronics [15] |  |
| Lenovo/Motorola Mobility [16] |  |
| Ericsson [17] |  |
| MediaTek Inc. [18] |  |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] |  |

| 23. NR_FeMIMO | 23-8-3 | SRS Antenna switching for >4Rx | 1. Support of SRS antenna switching xTyR with y>4 2. Report whether the antenna switching impact to downlink receiving in a band 3. Report whether the antenna is switched together with UL Tx in another band | 2-55 |  |  |  | Per BC |  |  |  | Component 1 candidate values: FFS | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | For 23-8-3, SRS antenna switching down gradation was agreed in Rel-16. For the >4Rx antenna switching cases, the down gradation can also be supported for the same reason. Following similar principle, the following UE capabilities can be considered: - {t1r1, t1r2, t2r2, t2r4, t4r4, t4r8} - {t1r1, t1r2, t2r2, t2r4, t2r6} - {t1r1, t1r2, t1r4, t2r2, t2r4, t2r6, t2r8} - {t1r1, t1r2, t1r4, t1r6, t1r8} - {t1r1, t1r2, t1r4, t1r6} Proposal 4-1: R17 SRS antenna switching UE capability allow UE to indicate support of one of the following combinations - {t1r1, t1r2, t2r2, t2r4, t4r4, t4r8} - {t1r1, t1r2, t2r2, t2r4, t2r6} - {t1r1, t1r2, t1r4, t2r2, t2r4, t2r6, t2r8} - {t1r1, t1r2, t1r4, t1r6, t1r8} - {t1r1, t1r2, t1r4, t1r6} 23. NR_FeMIMO; 23-8-3; SRS Antenna switching for >4Rx; 1. Support of SRS antenna switching xTyR with y>4 2. Report whether the antenna switching impact to downlink receiving in a band 3. Report whether the antenna is switched together with UL Tx in another band; 2-55; Per BC; Component 1 candidate values: FFS - {t1r1, t1r2, t2r2, t2r4, t4r4, t4r8} - {t1r1, t1r2, t2r2, t2r4, t2r6} - {t1r1, t1r2, t1r4, t2r2, t2r4, t2r6, t2r8} - {t1r1, t1r2, t1r4, t1r6, t1r8} - {t1r1, t1r2, t1r4, t1r6}; Optional with capability signalling. |
| ZTE [3] |  |
| vivo [4] |  |
| CATT [5] | On UE features for up to 8 Rx SRS antenna switching, one open issue is candidate values for component 1 of FG 23-8-3. In our opinion, it is nature for a UE supports 1T8R to support 1T1R, 1T2R, 1T4R and 1T6R, i.e. degrading SRS configuration can be a basic feature. The following candidate values can be included in component 1 of FG 23-8-3: {t1r1-t1r2-t1r4-t1r6, t1r1-t1r2-t1r4-t1r6-t1r8, t1r1-t1r2-t2r2-t2r4-t2r6, t1r1-t1r2-t1r4-t2r2-t2r4-t2r6-t2r8, t1r1-t1r2-t2r2-t2r4-t4r4-t4r8}. Proposal-18: The candidate values for component 1 of FG 23-8-3 are as follows: t1r1-t1r2-t1r4-t1r6, t1r1-t1r2-t1r4-t1r6-t1r8, t1r1-t1r2-t2r2-t2r4-t2r6, t1r1-t1r2-t1r4-t2r2-t2r4-t2r6-t2r8, t1r1-t1r2-t2r2-t2r4-t4r4-t4r8. |
| Samsung [6] |  |
| NTT DOCOMO, INC. [7] | For FG23-8-3, there is an FFS on Component 1 candidate values. The (explicitly) agreed transceiver architectures are 1T6R, 1T8R, 2T6R, 2T8R and 4T8R, which should be captured. A remaining issue would be whether to capture downgrading configurations for each transceiver architecture. For the existing architectures (i.e. with Rx less than 4), some configurations to use less Rx ports than the implemented ones are supported in Rel-16, which achieves more SRS configuration flexibility and smaller SRS overhead for DL CSI acquisition. Since the same benefit is expected even with larger number of Rx ports, we believe it would be good to support such downgrading configurations together. 23. NR_FeMIMO; 23-8-3; SRS Antenna switching for >4Rx; 1. Support of SRS antenna switching xTyR with y>4 2. Report whether the antenna switching impact to downlink receiving in a band 3. Report whether the antenna is switched together with UL Tx in another band; 2-55; Per BC; Component 1 candidate values: - 't1r6' for 1T6R - ‘t2r6' for 2T6R - 't1r1-t1r2-t1r4-t1r6’ for 1T=1R/1T2R/1T4R/1T6R - 't1r1-t1r2-t2r2-t1r4-t2r4-t1r6-t2r6' for 1T=1R/1T2R/2T=2R/1T4R/2T4R/1T6R/2T6R - ‘t1r6-t2r6' for 1T6R/2T6R - ‘t1r8' for 1T8R - ‘t2r8' for 2T8R - 't1r1-t1r2-t1r4-t1r6-t1r8’ for 1T=1R/1T2R/1T4R/1T6R/1T8R - 't1r1-t1r2-t2r2-t1r4-t2r4-t1r6-t2r6-t1r8-t2r8' for 1T=1R/1T2R/2T=2R/1T4R/2T4R/1T6R/2T6R/1T8R/2T8R - ‘t1r8-t2r8' for 1T8R/2T8R - ‘t4r8' for 4T8R - 't1r1-t1r2-t2r2-t1r4-t2r4-t1r6-t2r6t1r8-t2r8-t4r8' for 1T=1R/1T2R/2T=2R/1T4R/2T4R/1T6R/2T6R1T8R/2T8R/4T8R; Optional with capability signalling. |
| Spreadtrum Communications [8] |  |
| Qualcomm Incorporated [9] | The candidate values for component 1 should have independent xTyR configuration = {1T8R, 1T6R, 2T8R, 2T6R, 4T8R}. UE should report one of {1T6R, 1T8R, 2T6R, 2T8R, 4T6R, 4T8R} similar to basic feature FG 2-55. Tthere is no RAN1 agreement to support a downgraded (or combo) SRS switching configuration. If the UE wants to support downgraded configuration, this can be done by the combination of this FG 38-8-3 and Rel-16 (supportedSRS-TxPortSwitchv1610). Proposal 9-2: The candidate values for component 1 should be independent xTyR configuration of {1T8R, 1T6R, 2T8R, 2T6R, 4T8R}. - Note: Downgraded SRS antenna switching configuration could be achieved by the combination of FG 38-8-3 and Rel-16 (supportedSRS-TxPortSwitchv1610). 23. NR_FeMIMO; 23-8-3; SRS Antenna switching for >4Rx; 1. Support of SRS antenna switching xTyR with y>4 2. Report whether the antenna switching impact to downlink receiving in a band 3. Report whether the antenna is switched together with UL Tx in another band; 2-55; Per BC; Component 1 candidate values: {1T8R, 1T6R, 2T8R, 2T6R, 4T8R}; Optional with capability signalling. |
| OPPO [10] | - candidates are - t1r1-t1r2-t1r4-t1r6 (1T6R) - t1r1-t1r2-t1r4-t1r8 (1T8R) - t1r1-t1r2-t2r2-t2r4-t2r6 (2T6R) - t1r1-t1r2-t2r2-t2r4-t2r8 (2T8R) - t1r1-t2r2-t4r4-t4r6 (4T6R) - t1r1-t2r2-t4r4-t4r8 (4T8R) |
| Intel Corporation [11] | FG 23-8-3 defines UE capability for antenna switching with up to 8 Rx. It should be noted that in Rel-16 UE capability, the downgraded antenna switching configuration can also be reported by the UE. For example, the UE of 1T4R can also support 1T2R and 1T1R. Following the same principles, the UE capable of antenna switching with > 4R should also be able to report the supported downgraded configuration xTyR; Supported configuration. 1T8R; 1T8R, 1T6R, 1T4R, 1T2R, 1T1R. 1T6R; 1T6R, 1T4R, 1T2R, 1T1R. 2T8R; 2T8R, 2T6R, 2T4R, 2T2R, 1T8R, 1T6R, 1T4R, 1T2R, 1T1R. 2T6R; 2T6R, 2T4R, 2T2R, 1T6R, 1T4R, 1T2R, 1T1R. 4T8R; 4T8R, 4T4R, 2T8R, 2T6R, 2T4R, 2T2R, 1T8R, 1T6R, 1T4R, 1T2R, 1T1R. 23-8-3; SRS Antenna switching for >4Rx; 1. Support of SRS antenna switching xTyR with y>4 2. Report whether the antenna switching impact to downlink receiving in a band 3. Report whether the antenna is switched together with UL Tx in another band; Component 1 candidate values: FFS {1T8R-1T6R-1T4R-1T2R-1T1R, 1T6R-1T4R-1T2R-1T1R, 2T8R-2T6R-2T4R-2T2R-1T8R-1T6R-1T4R-1T2R-1T1R, 2T6R-2T4R-2T2R-1T6R-1T4R-1T2R-1T1R, 4T8R-4T4R-2T8R-2T6R-2T4R-2T2R-1T8R-1T6R-1T4R-1T2R-1T1R,} Component 2 candidate values: {1 to 32} Component 3 candidate values: {1 to 32}. |
| Apple [12] | - FG23-8-3: For SRS antenna switching, we propose the candidate value for the components 23. NR_FeMIMO; 23-8-3; SRS Antenna switching for >4Rx; 1. Support of SRS antenna switching xTyR with y>4 2. Report whether the antenna switching impact to downlink receiving in a band 3. Report whether the antenna is switched together with UL Tx in another band; 2-55; Per BC; Component 1: a list of TRx pairs, candidates are bitmap {"Not supported", “1T6R”, “1T8R”, “2T6R”, “2T8R”, “4T6R”, “4T8R”} Component 2: Candidate value set: {yes, no} Component 3: Candidate value set: {yes, no}; Optional with capability signalling. |
| FUTUREWEI [13] |  |
| xiaomi [14] | Conclusion In Rel-17, SRS 4T6R is not supported. This combo of UE supported configurations does not have to support all candidate values and all downgraded options, we suggest to have the following candidate values - t1r1-t2r2-t4r4- t4r8 - t1r1-t1r2-t2r2-t2r4- t2r6 - t1r1-t1r2-t2r2-t2r4 –t1r4-t1r6-t2r6-t2r8 - t1r1- t1r2- t1r4-t1r6 - t1r1-t1r2-t1r4-t2r2-t2r4-t2r8 - t1r1-t1r2-t1r4-t1r8 - t1r6 - t1r8 - t2r6 - t2r8 - t4r8 Proposal 19: For component 1 of FG23-8-3, our proposed candidate values for 6/8Rx are as below, - t1r1-t2r2-t4r4- t4r8 - t1r1-t1r2-t2r2-t2r4- t2r6 - t1r1-t1r2-t2r2-t2r4 –t1r4-t1r6-t2r6-t2r8 - t1r1- t1r2- t1r4-t1r6 - t1r1-t1r2-t1r4-t2r2-t2r4-t2r8 - t1r1-t1r2-t1r4-t1r8 - t1r6 - t1r8 - t2r6 - t2r8 - t4r8 |
| LG Electronics [15] |  |
| Lenovo/Motorola Mobility [16] |  |
| Ericsson [17] |  |
| MediaTek Inc. [18] | For antenna switching ≤ 4R in R15/R16, the candidate values for UE to report includes both non-downgraded (e.g., t2r4) and downgraded (e.g., t1r1-t1r2-t2r2-t2r4) options. It is reasonable that R17 also provides similar options for UE to choose from since whether UE can support downgraded options (and which combinations) or not depends on UE’s implementation. However, as number of Rx is up to 8, the combination considering all possible downgraded options can be large, enumerating all possible cases is not a good approach and difficult to get consensus. RAN1 should look for a more efficient way. Proposal 55: As for reporting FG 23-8-3 component-1, we propose UE reports one or more elements from set {t1r1, t2r2, t1r2, t4r4, t2r4, t1r4, t2r6, t1r6, t4r8, t2r8, t1r8}. The detailed asn.1 data structure can be decided by RAN2. 23. NR_FeMIMO; 23-8-3; SRS Antenna switching for >4Rx; 1. Support of SRS antenna switching xTyR with y>4 2. Report whether the uplink TX switching impact to downlink receiving in a band 3. Report whether the UL Tx is switched together with UL Tx in another band; Component 1 candidate values: one or more elements from set {t1r1, t2r2, t1r2, t4r4, t2r4, t1r4, t2r6, t1r6, t4r8, t2r8, t1r8}; Optional with capability signalling. |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] |  |

| 23. NR_FeMIMO | 23-8-4 | Maximum 2 SP and 1 periodic SRS sets for antenna switching | Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for antenna switching | 2-53 |  |  |  |  |  |  |  | Note1: - Applies for all supported xTyR where y<=8 - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - For xTyR where y<=4, if UE does not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS - The two SP-SRS resource sets are not activated at the same time | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| ZTE [3] |  |
| vivo [4] |  |
| CATT [5] |  |
| Samsung [6] |  |
| NTT DOCOMO, INC. [7] |  |
| Spreadtrum Communications [8] |  |
| Qualcomm Incorporated [9] | 23. NR_FeMIMO; 23-8-4; Maximum 2 SP and 1 periodic SRS sets for antenna switching; Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for antenna switching; 2-53; Per FS; Note1: - Applies for all supported xTyR where y<=8 - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - For xTyR where y<=4, if UE does not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS - The two SP-SRS resource sets are not activated at the same time; Optional with capability signalling. |
| OPPO [10] |  |
| Intel Corporation [11] |  |
| Apple [12] |  |
| FUTUREWEI [13] |  |
| xiaomi [14] |  |
| LG Electronics [15] |  |
| Lenovo/Motorola Mobility [16] |  |
| Ericsson [17] |  |
| MediaTek Inc. [18] |  |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] |  |

| 23. NR_FeMIMO | 23-8-5 | Increased repetition for SRS | Support of increased repetition patterns (8, 10, 12, 14 symbols) for SRS resource | 10-11, 2-52 |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| ZTE [3] |  |
| vivo [4] |  |
| CATT [5] |  |
| Samsung [6] |  |
| NTT DOCOMO, INC. [7] |  |
| Spreadtrum Communications [8] |  |
| Qualcomm Incorporated [9] | 23. NR_FeMIMO; 23-8-5; Increased repetition for SRS; Support of increased repetition patterns (8, 10, 12, 14 symbols) for SRS resource; 10-11, 2-52; Per FS; Optional with capability signalling. |
| OPPO [10] |  |
| Intel Corporation [11] |  |
| Apple [12] |  |
| FUTUREWEI [13] |  |
| xiaomi [14] |  |
| LG Electronics [15] |  |
| Lenovo/Motorola Mobility [16] |  |
| Ericsson [17] |  |
| MediaTek Inc. [18] |  |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] |  |

| 23. NR_FeMIMO | 23-8-6 | Partial frequency sounding of SRS | Support of partial frequency sounding for SRS | 2-52 |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| ZTE [3] |  |
| vivo [4] |  |
| CATT [5] |  |
| Samsung [6] |  |
| NTT DOCOMO, INC. [7] |  |
| Spreadtrum Communications [8] |  |
| Qualcomm Incorporated [9] | 23. NR_FeMIMO; 23-8-6; Partial frequency sounding of SRS; Support of partial frequency sounding for SRS; 2-52; Per Band; Optional with capability signalling. |
| OPPO [10] |  |
| Intel Corporation [11] |  |
| Apple [12] |  |
| FUTUREWEI [13] |  |
| xiaomi [14] |  |
| LG Electronics [15] |  |
| Lenovo/Motorola Mobility [16] |  |
| Ericsson [17] |  |
| MediaTek Inc. [18] |  |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] |  |

| 23. NR_FeMIMO | 23-8-7 | Start RB location hopping for partial frequency SRS | Support of start RB location hopping in partial frequency SRS transmission [across different SRS frequency hopping periods for periodic/semi-persistent SRS] | 23-8-6 |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | For 23-8-7, supporting same start RB location hopping approach as for periodic/semi-persistent SRS was agreed for aperiodic SRS. Agreement For aperiodic SRS, support same start RB location hopping approach as for P/SP SRS if there are multiple frequency hopping period in the slot. According to the agreement, the components of 23-8-7 should also include aperiodic SRS, i.e., Support of start RB location hopping in partial frequency SRS transmission across different SRS frequency hopping periods for periodic/semi-persistent/aperiodic SRS. Proposal 4-2: Add aperiodic SRS in the components of FG 23-8-7. 23. NR_FeMIMO; 23-8-7; Start RB location hopping for partial frequency SRS; Support of start RB location hopping in partial frequency SRS transmission [across different SRS frequency hopping periods for periodic/semi-persistent/aperiodic SRS]; 23-8-6; Optional with capability signalling. |
| ZTE [3] | 23-8-7; Start RB location hopping for partial frequency SRS; Support of start RB location hopping in partial frequency SRS transmission [across different SRS frequency hopping periods for periodic/semi-persistent SRS]. |
| vivo [4] |  |
| CATT [5] | According to the agreement on SRS coverage and capacity enhancement in RAN1#107-e meeting, the start RB location hopping for parital frequency SRS is also applicable to aperiodic SRS. Therefore, the components of Feature 23-8-7 need to be revised. Proposal-19: The components of UE features 23-8-7 are revised as follows: 23-8-7; Start RB location hopping for partial frequency SRS; Support of start RB location hopping in partial frequency SRS transmission [across different SRS frequency hopping periods for periodic/semi-persistent/aperiodic SRS]; 23-8-6; Optional with capability signalling. |
| Samsung [6] |  |
| NTT DOCOMO, INC. [7] | For FG23-8-7, the part with bracket does not capture the latest status correctly since RAN1 has agreed to support start RB location hopping even for A-SRS. Therefore, an update to capture it is needed. 23. NR_FeMIMO; 23-8-7; Start RB location hopping for partial frequency SRS; Support of start RB location hopping in partial frequency SRS transmission across different SRS frequency hopping periods for periodic/semi-persistent/aperiodic SRS; 23-8-6; Optional with capability signalling. |
| Spreadtrum Communications [8] |  |
| Qualcomm Incorporated [9] | In the last RAN1 meeting, the support of start RB hopping was agreed for aperiodic SRS when multiple there are multiple legacy frequency hopping period in a slot. Then, the description of the FG 23-8-7 should be updated to remove the restriction of the support of only periodic and semi-persistent SRS. Proposal 9-3: Update the description of FG 23-8-7 as ‘Support of start RB location hopping in partial frequency SRS transmission across different SRS frequency hopping periods’ - Add a note of “start RB location hopping for aperiodic SRS is supported when there are multiple frequency hopping period in the slot.” 23. NR_FeMIMO; 23-8-7; Start RB location hopping for partial frequency SRS; Support of start RB location hopping in partial frequency SRS transmission [across different SRS frequency hopping periods for periodic/semi-persistent SRS]; 23-8-6; Per Band; Optional with capability signalling. |
| OPPO [10] | - Suggest to remove “across different SRS frequency hopping periods for periodic/semi-persistent SRS” since there is not ambiguity and keep the description concise |
| Intel Corporation [11] |  |
| Apple [12] | 23. NR_FeMIMO; 23-8-7; Start RB location hopping for partial frequency SRS; Support of start RB location hopping in partial frequency SRS transmission across different SRS frequency hopping periods; 23-8-6; Optional with capability signalling. |
| FUTUREWEI [13] | Some rows agreed in the endorsed R1-2111807 [2] had yellow background, meaning that they were agreed but still pending further agreement in related agenda items. A few of them can be updated based on new agreements in related agenda items in RAN1#107-e meeting. Regarding the following FG: 23. NR_FeMIMO; 23-8-7; Start RB location hopping for partial frequency SRS; Support of start RB location hopping in partial [(RB level)] frequency SRS transmission [across different SRS frequency hopping periods for periodic/semi-persistent SRS]. For this FG, the yellow highlighted covered P/SP SRS. In RAN1#107-e meeting, we had the following new agreement: Agreement For aperiodic SRS, support same start RB location hopping approach as for P/SP SRS if there are multiple frequency hopping period in the slot Then this feature can be updated to include AP SRS. Note that whether to support this feature for non-hopping SRS is yet to be decided. Proposal 2: Update FG 23-8-7 as “Support of start RB location hopping in partial [(RB level)] frequency SRS transmission [across different SRS frequency hopping periods for periodic/semi-persistent SRS] and across multiple frequency hopping periods in the slot for aperiodic SRS”. |
| xiaomi [14] | Agreement: Adopt the following changes highlighted in chromatic formatting 23. NR_FeMIMO; 23-8-7; Start RB location hopping for partial frequency SRS; Support of start RB location hopping in partial [(RB level)] frequency SRS transmission [across different SRS frequency hopping periods for periodic/semi-persistent SRS]; [23-8-6]; Optional with capability signalling. Our proposal is based on the following agreement made in last RAN1 meeting [4]. Agreement For aperiodic SRS, support same start RB location hopping approach as for P/SP SRS if there are multiple frequency hopping period in the slot Proposal 20: Support of start RB location hopping in partial frequency SRS transmission across different SRS frequency hopping periods for periodic/semi-persistent SRS and aperiodic SRS if multiple frequency hopping period exists in the slot. |
| LG Electronics [15] | - Start RB location hopping was agreed to be supported for aperiodic SRS in RAN1 #107-e. So, square bracket should be removed and ‘for periodic/semi-persistent’ should also be removed. Agreement For aperiodic SRS, support same start RB location hopping approach as for P/SP SRS if there are multiple frequency hopping period in the slot Proposal 10: Adopt the following table for Rel-17 SRS. 23. NR_FeMIMO; 23-8-7; Start RB location hopping for partial frequency SRS; Support of start RB location hopping in partial frequency SRS transmission across different SRS frequency hopping periods; 23-8-6; Optional with capability signalling. |
| Lenovo/Motorola Mobility [16] |  |
| Ericsson [17] |  |
| MediaTek Inc. [18] |  |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] |  |

| 23. NR_FeMIMO | 23-8-8 | Comb-8 SRS | Support of comb-8 for SRS other than for positioning |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| ZTE [3] |  |
| vivo [4] |  |
| CATT [5] |  |
| Samsung [6] |  |
| NTT DOCOMO, INC. [7] |  |
| Spreadtrum Communications [8] |  |
| Qualcomm Incorporated [9] | 23. NR_FeMIMO; 23-8-8; Comb-8 SRS; Support of comb-8 for SRS other than for positioning; Per FS; Optional with capability signalling. |
| OPPO [10] |  |
| Intel Corporation [11] |  |
| Apple [12] |  |
| FUTUREWEI [13] |  |
| xiaomi [14] |  |
| LG Electronics [15] |  |
| Lenovo/Motorola Mobility [16] |  |
| Ericsson [17] |  |
| MediaTek Inc. [18] |  |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] |  |

| 23. NR_FeMIMO | 23-9-1 | Basic Features of Further Enhanced Port-Selection Type II Codebook (FeType-II) | - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with M=1 and R=1 - Support rank 1,2 - Support parameter combinations with M=1 | 2-35 |  |  |  | Per band and per BC |  |  |  | Component 1 candidate values: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| ZTE [3] |  |
| vivo [4] |  |
| CATT [5] |  |
| Samsung [6] |  |
| NTT DOCOMO, INC. [7] |  |
| Spreadtrum Communications [8] |  |
| Qualcomm Incorporated [9] |  |
| OPPO [10] |  |
| Intel Corporation [11] |  |
| Apple [12] |  |
| FUTUREWEI [13] |  |
| xiaomi [14] |  |
| LG Electronics [15] |  |
| Lenovo/Motorola Mobility [16] |  |
| Ericsson [17] |  |
| MediaTek Inc. [18] |  |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] |  |

| 23. NR_FeMIMO | 23-9-2 | Support of Mv=2 for FeType-II | [1. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with M=2 and R=1] 2. Support parameter combinations with M=2 [3. Support of DFT FD bases of size N] | 23-9-1 |  |  |  | [per band and per BC] |  |  |  | [Component 1 candidate values - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}] [Component 4 candidate values: {N=2 only, N=2 and N=4}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | With regarding to FeType II PS codebook, one remaining issue is whether a UE reports the value of N when M=2 in FG 23-9-2. In our understanding, such a FG component is not needed since the window size of N has marginal impact of UE computational complexity due to small size of N (up to 4) and the selection of FD basis is layer common. From UE perspective, if a UE can support FG 23-9-2, it is more desirable to support both values of N (i.e. N=2 and 4) to simplify gNB implementation. Moreover, FG 23-9-1 and FG23-9-2 can be symmetrical from capability design perspective. Such similarity can be helpful and useful when designing hybrid codebook in FG 23-9-5, similar with the treatment of supported values of R as FG 16-8 in Rel-16. Proposal 5-2: There is no need to report values of N in FG 23-9-2. 23. NR_FeMIMO; 23-9-2; Support of M=2 for FeType-II; - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with M=2 and R=1 - Support parameter combinations with M=2 [4. Support of DFT FD bases of size N]; 23-9-1; per band and per BC; Component 1 candidate values - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} [Component 4 candidate values: {N=2 only, N=2 and N=4}]; Optional with capability signalling. |
| ZTE [3] | Four features (23-9-1, 23-9-1a ~ 23-9-4) are developed in [1] for FeType II port selection CSI, which is analogous to eType II CSI. We discuss some further details for 23-9 series in this section. Optional support of Mv = 2 The complexity and UE hardware cost to support Mv=2 can be different for different triplets {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports}. In the basic feature 23-9-1, a list of such triplets are reported for Mv=1. Hence it is needed to report another list of triplets for Mv=2. Otherwise, UE has to consider the worst case when determining whether it can supports Mv=2. For example, if UE can support Mv=2 for small number of Tx ports, but it cannot support Mv=2 for large number of Tx ports, it has to report no support of Mv=2 if the report of Mv=2 does not include the list of triplets. This limits NW to realize the better performance from Mv=2 for small number of Tx ports. Hence we have the following proposal. Proposal 18: Support to report a list of triplets {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for Mv=2 as follows. 23. NR_FeMIMO; 23-9-2; Support of Mv=2 for FeType-II; {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with Mv=2. 23. NR_FeMIMO; 23-9-2; Support of Mv=2 for FeType-II; [1. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with M=2 and R=1] 2. Support parameter combinations with M=2 [3. Support of DFT FD bases of size N]. |
| vivo [4] |  |
| CATT [5] | Mv has been replaced by M for FeType-II. The name of feature group for FG 23-9-2 needs to be revised. The type of Feature 23-9-1 is per band and per BC. The type of FG 23-9-2 should also be per band and per BC, since the codebook sturture are same for M=1 and M=2. According to discussion on codebook parameter combination, the number of ports has been restricted for M=2. Hence, it is not necessary to further restrict the number of ports through UE capabiltity. ‘{Max # of Tx ports in one resource, Max # of resources and total # of Tx ports}’ in the component 1 should be removed. For the component 3 of FG 23-9-2, the value of window size N can be 2 or 4. However, when the number of PMI subband N3<N, the window size is min{N3,N}. Hence, the candidate values of N can be N=2 , or min{N3,N} when N=2 or 4. When M=1 or M=2, rank 3 and 4 has been supported for FeType-II codebook. Rank 3 and 4 is UE optional. It is not necessary to separate capabilities for M=1 and M=2. Hence, the prerequisite feature group of FG 23-9-3 should be FG 23-9-1 or FG 23-9-2. When M=2, the difference between R=1 and R= 2 is PMI granularity in frequency domain. As discussed on the FG 23-9-2, the number of ports has been restricted for M=2. It does not need to further restrict the number of ports through UE capabiltity. Therefore,the component 2 of FG 23-9-4, i.e., {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support port-selection FeType-II with M=2 and R=2 should be removed. We think the type of FG 23-9-4 is same to FG 23-9-2. For mixed codebook types, UE reports support active CSI-RS resources and ports for up to 4 mixed codebook combinations in any slot for UE feature on Rel-16 eType II codebook, and the feature of supporting active CSI-RS resources and ports is per band. Following this UE feature, the type of FG23-9-5 is also per band. According to above discussion, we provide the following proposal. Proposal-20: For CSI enhancement on FDD CSI, the UE feature is revised as follows: 23-9-2; Support of Mv=2 for FeType-II; [1. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to sSupport Port-selection FeType-II with M=2 and R=1] 2. Support parameter combinations with M=2 [3. Support of DFT FD bases of size N]; 23-9-1; [per band and per BC]; [Component 1 candidate values - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}] [Component 4 3 candidate values: {N=2 only, min{N3,N} when N=2 or 4N=2 and N=4}]; Optional with capability signalling. |
| Samsung [6] | Proposal 32: add the following components - FG 23-9-2: support of M=2 - We only support Component 2 as “Support parameter combinations with M=2” and delete other Components. - Support max value of #CSI-RS ports for which M=2 is supported by the UE |
| NTT DOCOMO, INC. [7] | we do not think it is needed but we can live with it. 23. NR_FeMIMO; 23-9-2; Support of Mv=2 for FeType-II; [1. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with M=2 and R=1] 2. Support parameter combinations with M=2 3. Support of DFT FD bases of size N; 23-9-1; [per band and per BC]; [Component 1 candidate values - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}] [Component 4 candidate values: {N=2 only, N=2 and N=4}]; Optional with capability signalling. |
| Spreadtrum Communications [8] | For component 3, we prefer to keep it. If N = 2, UE can select all of the FD bases within the window. On the other hand, if N = 4, UE should calculate the channel quality of 4 FD bases and further down selection is required, which will increase UE complexity. Proposal 12: For FG 23-9-2, confirm the following component 3 - Support of DFT FD bases of size N |
| Qualcomm Incorporated [9] | - In FG23-9-2, it is preferred to introduce CSI-RS triplet for M=2 because M=2 increases SVD complexity compared to M=1. If signaling complexity is an issue, reusing same signaling (reporting “supported” or “not supported”) as Rel-16 codebook (FG16-3a-2) is sufficient. - For the CSI-RS triplets candidate values in component 1 of FG23-9-1, component 2 of FG23-9-5, component 1 of FG23-9-2 and FG23-9-4, the current form in the table is preferred because they are consistent with all other legacy codebooks in Rel-15 and Rel-16. 23. NR_FeMIMO; 23-9-2; Support of Mv=2 for FeType-II; 1. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with M=2 and R=1 2. Support parameter combinations with M=2 3. Support of DFT FD bases of size N; 23-9-1; per band and per BC; Component 1 candidate values - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} Component 3 candidate values: {N=2 only, N=2 and N=4}; Optional with capability signalling. |
| OPPO [10] |  |
| Intel Corporation [11] | In our view the complexity difference for codebook configuration with M = 1 and M = 2 and R = 1 and R = 2 for feType II PS PMI codebook is not significant so there is no need to have triplets {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for each configuration. Triplets from 23-9-1 can be considered for all the M and R configurations. In this case per band capability signaling is enough for FG 23-9-2 and FG 23-9-4. 23-9-2; Support of Mv=2 for FeType-II; [1. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with M=2 and R=1] 1. Support parameter combinations with M=2 [2. Support of DFT FD bases of size N]; [Component 1 candidate values - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}] [Component 4 2 candidate values: {N=2 only, N=2 and N=4}]; [per band and per BC]. |
| Apple [12] | - FG23-9-2: We prefer to have separate triplets reporting for Mv=2 and R=1 23. NR_FeMIMO; 23-9-2; Support of Mv=2 for FeType-II; 1. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with M=2 and R=1 2. Support parameter combinations with M=2 [3. Support of DFT FD bases of size N]; 23-9-1; per band and per BC; [Component 1 candidate values - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}] [Component 4 candidate values: {N=2 only, N=2 and N=4}]; Optional with capability signalling. |
| FUTUREWEI [13] |  |
| xiaomi [14] |  |
| LG Electronics [15] |  |
| Lenovo/Motorola Mobility [16] | - For Port Selection Type-II codebook, we suggest updating the second component in FG 23-9-2 to support parameter combinations with M=2 and R=1. Additionally, we believe the third component should be removed. Update FG 23-9-2 as follows: - 1. Modify the second component to “Support parameter combinations with Mv=2 and R=1” - 2. Remove the third component |
| Ericsson [17] | In accordance with the RAN2 guidance to minimize the number of features that are reported per band combination, we propose FG 23-9-2 is only defined per band. |
| MediaTek Inc. [18] |  |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] | - It is ok to have a separate list of triplets for M=2. - Component 3 regarding the window size N is not needed. - Typo in the FG name where Mν should be replaced by M |

| 23. NR_FeMIMO | 23-9-3 | Support of rank 3, 4 for FeType-II | Support of rank 3, 4 for FeType-II FFS separate capabilities for M = 1 and M = 2 | 23-9-1[, 23-9-2] |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | With regarding to the value of M shown in FG 23-9-3, the main difference between rank 3/4 and rank 1/2 are extra one or two layers selection/quantization with more non-zero coefficients. Such impact has less correlation with the value of M which have been covered by two FGs, i.e. 23-9-1 (supporting M=1) and 23-9-2 (supporting M=2). There is no need to report them again in FG 23-9-3. Proposal 5-3: There is no need to report values of M in FG 23-9-3. 23. NR_FeMIMO; 23-9-3; Support of rank 3, 4 for FeType-II; Support of rank 3, 4 for FeType-II FFS separate capabilities for M = 1 and M = 2; 23-9-1, 23-9-2; Per band; Optional with capability signalling. |
| ZTE [3] | 23. NR_FeMIMO; 23-9-3; Support of rank 3, 4 for FeType-II; Support of rank 3, 4 for FeType-II FFS separate capabilities for M = 1 and M = 2. |
| vivo [4] |  |
| CATT [5] | Mv has been replaced by M for FeType-II. The name of feature group for FG 23-9-2 needs to be revised. The type of Feature 23-9-1 is per band and per BC. The type of FG 23-9-2 should also be per band and per BC, since the codebook sturture are same for M=1 and M=2. According to discussion on codebook parameter combination, the number of ports has been restricted for M=2. Hence, it is not necessary to further restrict the number of ports through UE capabiltity. ‘{Max # of Tx ports in one resource, Max # of resources and total # of Tx ports}’ in the component 1 should be removed. For the component 3 of FG 23-9-2, the value of window size N can be 2 or 4. However, when the number of PMI subband N3<N, the window size is min{N3,N}. Hence, the candidate values of N can be N=2 , or min{N3,N} when N=2 or 4. When M=1 or M=2, rank 3 and 4 has been supported for FeType-II codebook. Rank 3 and 4 is UE optional. It is not necessary to separate capabilities for M=1 and M=2. Hence, the prerequisite feature group of FG 23-9-3 should be FG 23-9-1 or FG 23-9-2. When M=2, the difference between R=1 and R= 2 is PMI granularity in frequency domain. As discussed on the FG 23-9-2, the number of ports has been restricted for M=2. It does not need to further restrict the number of ports through UE capabiltity. Therefore,the component 2 of FG 23-9-4, i.e., {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support port-selection FeType-II with M=2 and R=2 should be removed. We think the type of FG 23-9-4 is same to FG 23-9-2. For mixed codebook types, UE reports support active CSI-RS resources and ports for up to 4 mixed codebook combinations in any slot for UE feature on Rel-16 eType II codebook, and the feature of supporting active CSI-RS resources and ports is per band. Following this UE feature, the type of FG23-9-5 is also per band. According to above discussion, we provide the following proposal. Proposal-20: For CSI enhancement on FDD CSI, the UE feature is revised as follows: 23-9-3; Support of rank 3, 4 for FeType-II; Support of rank 3, 4 for FeType-II FFS separate capabilities for M = 1 and M = 2; 23-9-1[, 23-9-2]; Per band; Optional with capability signalling. |
| Samsung [6] |  |
| NTT DOCOMO, INC. [7] | We’re generally fine with current version. For component 23-9-3, we think it is fine to support separate capabilities for M=1 and M=2. 23. NR_FeMIMO; 23-9-3; Support of rank 3, 4 for FeType-II; Support of rank 3, 4 for FeType-II separate capabilities for M = 1 and M = 2; 23-9-1[, 23-9-2]; Per band; Optional with capability signalling. |
| Spreadtrum Communications [8] |  |
| Qualcomm Incorporated [9] | 23. NR_FeMIMO; 23-9-3; Support of rank 3, 4 for FeType-II; Support of rank 3, 4 for FeType-II FFS separate capabilities for M = 1 and M = 2; 23-9-1[, 23-9-2]; Per band; Optional with capability signalling. |
| OPPO [10] |  |
| Intel Corporation [11] | - FG 23-9-3 should be applied for configurations with M = 1 and M = 2 23-9-3; Support of rank 3, 4 for FeType-II; Support of rank 3, 4 for FeType-II FFS separate capabilities for M = 1 and M = 2; Per band. |
| Apple [12] |  |
| FUTUREWEI [13] |  |
| xiaomi [14] |  |
| LG Electronics [15] |  |
| Lenovo/Motorola Mobility [16] |  |
| Ericsson [17] |  |
| MediaTek Inc. [18] |  |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] | - Separate capabilities to support rank 3,4 with M=1 and M=2 are not needed |

| 23. NR_FeMIMO | 23-9-4 | Support of R = 2 for FeType-II | 1. Support of R = 2 for FeType-II [2. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support to support Port-selection FeType-II with M=2 and R=2] | 23-9-2 |  |  |  | [per band and per BC] |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 23. NR_FeMIMO; 23-9-4; Support of R = 2 for FeType-II; - Support of R = 2 for FeType-II - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support to support Port-selection FeType-II with M=2 and R=2; 23-9-2; Perband; Optional with capability signalling. |
| ZTE [3] | 23. NR_FeMIMO; 23-9-4; Support of R = 2 for FeType-II; 1. Support of R = 2 for FeType-II [2. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support to support Port-selection FeType-II with M=2 and R=2]. |
| vivo [4] |  |
| CATT [5] | Mv has been replaced by M for FeType-II. The name of feature group for FG 23-9-2 needs to be revised. The type of Feature 23-9-1 is per band and per BC. The type of FG 23-9-2 should also be per band and per BC, since the codebook sturture are same for M=1 and M=2. According to discussion on codebook parameter combination, the number of ports has been restricted for M=2. Hence, it is not necessary to further restrict the number of ports through UE capabiltity. ‘{Max # of Tx ports in one resource, Max # of resources and total # of Tx ports}’ in the component 1 should be removed. For the component 3 of FG 23-9-2, the value of window size N can be 2 or 4. However, when the number of PMI subband N3<N, the window size is min{N3,N}. Hence, the candidate values of N can be N=2 , or min{N3,N} when N=2 or 4. When M=1 or M=2, rank 3 and 4 has been supported for FeType-II codebook. Rank 3 and 4 is UE optional. It is not necessary to separate capabilities for M=1 and M=2. Hence, the prerequisite feature group of FG 23-9-3 should be FG 23-9-1 or FG 23-9-2. When M=2, the difference between R=1 and R= 2 is PMI granularity in frequency domain. As discussed on the FG 23-9-2, the number of ports has been restricted for M=2. It does not need to further restrict the number of ports through UE capabiltity. Therefore,the component 2 of FG 23-9-4, i.e., {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support port-selection FeType-II with M=2 and R=2 should be removed. We think the type of FG 23-9-4 is same to FG 23-9-2. For mixed codebook types, UE reports support active CSI-RS resources and ports for up to 4 mixed codebook combinations in any slot for UE feature on Rel-16 eType II codebook, and the feature of supporting active CSI-RS resources and ports is per band. Following this UE feature, the type of FG23-9-5 is also per band. According to above discussion, we provide the following proposal. Proposal-20: For CSI enhancement on FDD CSI, the UE feature is revised as follows: 23-9-4; Support of R = 2 for FeType-II; 1. Support of R = 2 for FeType-II [2. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support to support Port-selection FeType-II with M=2 and R=2]; 23-9-2; [per band and per BC]; [Component 1 candidate values - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}]; Optional with capability signalling. |
| Samsung [6] |  |
| NTT DOCOMO, INC. [7] | we do not think it is needed but we can live with it. 23. NR_FeMIMO; 23-9-4; Support of R = 2 for FeType-II; 1. Support of R = 2 for FeType-II [2. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support to support Port-selection FeType-II with M=2 and R=2]; 23-9-2; [per band and per BC]; Optional with capability signalling. |
| Spreadtrum Communications [8] | - FG23-9-4 needs to be consistent with FG16-3a/3b-1 by introducing the CSI-RS triplets for supporting R=2. - The reason is the following. The UE would need to construct PMIs to calculate CQI. R=2 doubles the number of PMIs, so the complexity of PMI constructions scales up with the value of R. Having the same signaling of CSI-RS triplets {max #Tx ports per resource, max # resources and total #Tx ports} as R=1 would cause underreporting issue. - For the CSI-RS triplets candidate values in component 1 of FG23-9-1, component 2 of FG23-9-5, component 1 of FG23-9-2 and FG23-9-4, the current form in the table is preferred because they are consistent with all other legacy codebooks in Rel-15 and Rel-16. 23. NR_FeMIMO; 23-9-4; Support of R = 2 for FeType-II; 1. Support of R = 2 for FeType-II 2. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support to support Port-selection FeType-II with M=2 and R=2; 23-9-2; per band and per BC; Component 2 candidate values - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}; Optional with capability signalling. |
| Qualcomm Incorporated [9] |  |
| OPPO [10] |  |
| Intel Corporation [11] | - In our view the complexity difference for codebook configuration with M = 1 and M = 2 and R = 1 and R = 2 for feType II PS PMI codebook is not significant so there is no need to have triplets {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for each configuration. Triplets from 23-9-1 can be considered for all the M and R configurations. In this case per band capability signaling is enough for FG 23-9-2 and FG 23-9-4. 23-9-4; Support of R = 2 for FeType-II; 1. Support of R = 2 for FeType-II [2. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support to support Port-selection FeType-II with M=2 and R=2]; [per band and per BC]. |
| Apple [12] | - FG23-9-4: We prefer to have separate triplets reporting for R=2 similar as Rel-16 Type II codebook enhancement 23. NR_FeMIMO; 23-9-4; Support of R = 2 for FeType-II; 1. Support of R = 2 for FeType-II 2. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support to support Port-selection FeType-II with M=2 and R=2; 23-9-2; per band and per BC; Component 2 candidate values: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}; Optional with capability signalling. |
| FUTUREWEI [13] |  |
| xiaomi [14] |  |
| LG Electronics [15] |  |
| Lenovo/Motorola Mobility [16] | - For consistency, an additional component should be added to FG 23-9-4 corresponding to supporting parameter combinations with Mv=2 and R=2/ For FG 23-9-4, add a new component as follows: “Support parameter combinations with Mv=2 and R=2” |
| Ericsson [17] | In accordance with the RAN2 guidance to minimize the number of features that are reported per band combination, we propose FG 23-9-4 is only defined per band. |
| MediaTek Inc. [18] |  |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] | - No need for a separate list of triplets as those indicated for M=2 can be supported for both R=1 and R=2. |

| 23. NR_FeMIMO | 23-9-5 | Active CSI-RS resources and ports for mixed codebook types in any slot | - List of codebook combinations - List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination | [23-9-1] |  |  |  | FFS |  |  |  | [Component 1 candidate values: Codebook 1 = {Type I SP, Type I MP} {Codebook 2, Codebook 3} = {{FeType II PS M=1, NULL},{FeType II PS M=2 R=1, NULL}, {Type II, FeType II PS M=1}, {Type II, FeType II PS M=2 R=1} ,{eType II R=1, FeType II PS M=1},{eType II R=1, FeType II PS M=2 R=1}} Component 2 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}] Note 1：if a UE reports one or more codebook combinations in [23-9-5], then usage of active CSI-RS resources and ports for multiple codebooks in any slot is allowed only within those combinations [Note 2: For coexisting of mixed codebooks in any slot, gNB need to honor 16-8, 23-10 and per-codebook capability 2-36/40/41/43, 16-3a/b and 16-3a-1/16-3b-1, and 23-9-1] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | When the UE supports multiple codebook types independently, this UE may report conservative values assuming that multiple CSI reports associated with different codebook types are triggered simultaneously. To address this issue, FG 23-9-5 is to report a more precise list of supported codebook combinations with a list of triplets. FG 23-9-1 with M=1 and FG 23-9-2 with M=2 have been agreed as two independent FG for FeType-II PS codebook due to the consideration of the dimensions of SVD, which is approximatelyPCSI-RS M. Conceptually, the former is closed to two-stage codebook and the latter is closed to three-stage codebook. Therefore, when UE supports mixed codebook types in any slot, FG 23-9-1 and FG 23-9-2 can be treated as individual codebooks, with a similar manner of values of R as Rel-16 MIMO capability design methodology. Therefore the following codebook combinations are suggested for FG 23-9-5 considering the mixture of legacy Type II codebook and Rel-17 FeTypeII PS codebook in {codebook 2, codebook 3} - Codebook 1 = {Type I SP, Type I MP}, - {Codebook 2, Codebook 3} ={{FeType II PS M=1, NULL},{FeType II PS M=2, NULL},{Type II, FeType II PS M=1},{Type II, FeType II PS M=2},{eType II R=1, FeType II PS M=1},{eType II R=1, FeType II PS M=2},{eType II R=2, FeType II PS M=1},{eType II R=2, FeType II PS M=2}}. Proposal 5-4: To support mixed codebook types in a slot in FG 23-9-5, FG 23-9-1 and FG 23-9-2 can be treated as individual codebooks as Rel-16 UE capability design methodology, e.g. FG 23-9-1 with M=1 or FG 23-9-2 with M-2 are jointly supported with legacy Type II codebooks in {codebook 2, codebook 3}. 23. NR_FeMIMO; 23-9-5; Active CSI-RS resources and ports for mixed codebook types in any slot; - List of codebook combinations - List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination; 23-9-1; per band and per BC; Component 1 candidate values: Codebook 1 = {Type I SP, Type I MP}, {Codebook 2, Codebook 3} ={{FeType II PS M=1, NULL} {FeType II PS M=2, NULL} {Type II, FeType II PS M=1} {Type II, FeType II PS M=2} ,{eType II R=1, FeType II PS M=1} {eType II R=1, FeType II PS M=2} {eType II R=2, FeType II PS M=1} {eType II R=2, FeType II PS M=2}}. Component 2 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}] Note 1：if a UE reports one or more codebook combinations in 23-9-5, then usage of active CSI-RS resources and ports for multiple codebooks in any slot is allowed only within those combinations Note 2: For coexisting of mixed codebooks in any slot, gNB need to honor 16-8, 23-9-5 and per-codebook capability 2-36/40/41/43, 16-3a/b and 16-3a-1/16-3b-1, 23-9-1 and 23-9-2. Note 3: For an active CSI-RS resource associated with a CSI-RS resource set for channel measurement configured with two resource groups and N resource pairs, if it is referred X times by one of the M CSI-RS resource and/or one or two resource pairs, the CSI-RS resource and associated number of CSI-RS ports are counted X times within max number of resources and max number of total ports respectively.; Optional with capability signalling. |
| ZTE [3] |  |
| vivo [4] |  |
| CATT [5] | Mv has been replaced by M for FeType-II. The name of feature group for FG 23-9-2 needs to be revised. The type of Feature 23-9-1 is per band and per BC. The type of FG 23-9-2 should also be per band and per BC, since the codebook sturture are same for M=1 and M=2. According to discussion on codebook parameter combination, the number of ports has been restricted for M=2. Hence, it is not necessary to further restrict the number of ports through UE capabiltity. ‘{Max # of Tx ports in one resource, Max # of resources and total # of Tx ports}’ in the component 1 should be removed. For the component 3 of FG 23-9-2, the value of window size N can be 2 or 4. However, when the number of PMI subband N3<N, the window size is min{N3,N}. Hence, the candidate values of N can be N=2 , or min{N3,N} when N=2 or 4. When M=1 or M=2, rank 3 and 4 has been supported for FeType-II codebook. Rank 3 and 4 is UE optional. It is not necessary to separate capabilities for M=1 and M=2. Hence, the prerequisite feature group of FG 23-9-3 should be FG 23-9-1 or FG 23-9-2. When M=2, the difference between R=1 and R= 2 is PMI granularity in frequency domain. As discussed on the FG 23-9-2, the number of ports has been restricted for M=2. It does not need to further restrict the number of ports through UE capabiltity. Therefore,the component 2 of FG 23-9-4, i.e., {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support port-selection FeType-II with M=2 and R=2 should be removed. We think the type of FG 23-9-4 is same to FG 23-9-2. For mixed codebook types, UE reports support active CSI-RS resources and ports for up to 4 mixed codebook combinations in any slot for UE feature on Rel-16 eType II codebook, and the feature of supporting active CSI-RS resources and ports is per band. Following this UE feature, the type of FG23-9-5 is also per band. According to above discussion, we provide the following proposal. Proposal-20: For CSI enhancement on FDD CSI, the UE feature is revised as follows: 23-9-5; Active CSI-RS resources and ports for mixed codebook types in any slot; 1. List of codebook combinations List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination; [23-9-1]; Per band; [Component 1 candidate values: Codebook 1 = {Type I SP, Type I MP} {Codebook 2, Codebook 3} = {{FeType II PS M=1, NULL},{FeType II PS M=2 R=1, NULL}, {Type II, FeType II PS M=1}, {Type II, FeType II PS M=2 R=1} ,{eType II R=1, FeType II PS M=1},{eType II R=1, FeType II PS M=2 R=1}} Component 2 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}] Note 1：if a UE reports one or more codebook combinations in [23-9-5], then usage of active CSI-RS resources and ports for multiple codebooks in any slot is allowed only within those combinations [Note 2: For coexisting of mixed codebooks in any slot, gNB need to honor 16-8, 23-10 and per-codebook capability 2-36/40/41/43, 16-3a/b and 16-3a-1/16-3b-1, and 23-9-1]; Optional with capability signalling. |
| Samsung [6] |  |
| NTT DOCOMO, INC. [7] |  |
| Spreadtrum Communications [8] |  |
| Qualcomm Incorporated [9] | - FG23-9-5 needs to be consistent with FG16-8 by - Introducing per band and per band combination signaling. - The Note1 and Note2 should be kept to clarify how 23-9-5 is used together with per-codebook capability. - In addition, codebook combination of FeType II PS M=2 R=2 shoud be added for the same reason mentioned in FG23-9-4. - For the CSI-RS triplets candidate values in component 1 of FG23-9-1, component 2 of FG23-9-5, component 1 of FG23-9-2 and FG23-9-4, the current form in the table is preferred because they are consistent with all other legacy codebooks in Rel-15 and Rel-16. 23. NR_FeMIMO; 23-9-5; Active CSI-RS resources and ports for mixed codebook types in any slot; - List of codebook combinations - List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination; 23-9-1, 16-3a, 16-3b, 16-3a-1, 16-3b-1, 2-36, 2-40, 2-41, 2-43; Per band and per BC; [Component 1 candidate values: Codebook 1 = {Type I SP, Type I MP} {Codebook 2, Codebook 3} = {{FeType II PS M=1, NULL},{FeType II PS M=2 R=1, NULL}, {FeType II PS M=2 R=2, NULL}, {Type II, FeType II PS M=1}, {Type II, FeType II PS M=2 R=1} ,{eType II R=1, FeType II PS M=1},{eType II R=1, FeType II PS M=2 R=1}} Component 2 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} Note 1：if a UE reports one or more codebook combinations in [23-9-5], then usage of active CSI-RS resources and ports for multiple codebooks in any slot is allowed only within those combinations Note 2: For coexisting of mixed codebooks in any slot, gNB need to honor 16-8, 23-10 and per-codebook capability 2-36/40/41/43, 16-3a/b and 16-3a-1/16-3b-1, and 23-9-1; Optional with capability signalling. |
| OPPO [10] |  |
| Intel Corporation [11] | 23-9-5; Active CSI-RS resources and ports for mixed codebook types in any slot; - List of codebook combinations - List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination; [Component 1 candidate values: Codebook 1 = {Type I SP, Type I MP} {Codebook 2, Codebook 3} = {{FeType II PS M=1, NULL},{FeType II PS M=2 R=1, NULL}, {Type II, FeType II PS M=1}, {Type II, FeType II PS M=2 R=1} ,{eType II R=1, FeType II PS M=1},{eType II R=1, FeType II PS M=2 R=1}} Component 2 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}] Note 1：if a UE reports one or more codebook combinations in [23-9-5], then usage of active CSI-RS resources and ports for multiple codebooks in any slot is allowed only within those combinations [Note 2: For coexisting of mixed codebooks in any slot, gNB need to honor 16-8, 23-10 and per-codebook capability 2-36/40/41/43, 16-3a/b and 16-3a-1/16-3b-1, and 23-9-1]; FFS. |
| Apple [12] |  |
| FUTUREWEI [13] |  |
| xiaomi [14] |  |
| LG Electronics [15] |  |
| Lenovo/Motorola Mobility [16] |  |
| Ericsson [17] | In accordance with the RAN2 guidance to minimize the number of features that are reported per band combination, we propose FG 23-9-5 is only defined per band. |
| MediaTek Inc. [18] |  |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] |  |

New FGs

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | In order to limit the UE implementation complexity in tracking activated TCI, Similar to FG 2-4 in Rel-15 and FG 16-2a-7, the maximum number of activated TCI states should also be added in HST scenarios. With above analysis, we propose the following: Proposal 3-10: Support adding a new FG 23-6-5 for TCI tracking capability in HST as follow, - FG 23-6-5: Component 1. The maximal total number of activated TCI states per BWP per CC including data and control, with candidate values {1, 2, 4, 8 ,16}; - FG 23-6-5: Component 2. The maximal total number of activated TCI states all BWPs all CCs including data and control, with candidate values {1, 2, 4, 8 ,16, 32}; In addition, a UE should indicate the PDSCH processing capability that highly impact the implementation complexity.we propose to add one components ‘Maximum number of PDSCHs per slot’ per CC of per band and per BC. Proposal 3-11: Support adding a new FG 23-6-6 for PDSCH processing capability in HST as follow, - FG 23-6-6: Component 1. The maximal number of PDSCHs per slot, with candidate values {1, 2, 3, 4 ,7}; 23. NR_FeMIMO; 23-6-5; Maximum number of activated TCI states for HST/[URLLC]; 1. The maximal number of activated TCI states per BWP per CC including data and control 2. The maximal number of activated TCI states all BWPs all CCs including data and control; Candidate values for Component 1: {1,2,4,8,16} Candidate values for Component 2: {1,2,4,8,16, 32}. 23. NR_FeMIMO; 23-6-6; PDSCH processing capability for HST/[URLLC]; 1. The maximal number of PDSCH per slot; Candidate values for Component 1: {1,2,3,4,7}. In RAN1#106bis-e, extension of aperiodic SRS set number for 1T4R, 1T2R and 2T4R was agreed, which is an optional UE feature. Agreement For extension of aperiodic antenna switching SRS configurations for <=4Rx, support N=4 for 1T4R and N=2 for 1T2R/2T4R. - The above extension is UE optional According to the agreement, a new UE feature should be added for the extension. Therefore, we propose Proposal 4-3: Add a new FG 23-8-9 for extension of aperiodic SRS set number for 1T4R, 1T2R and 2T4R. 23. NR_FeMIMO; 23-8-9; Extension of aperiodic SRS set number for 1T4R, 1T2R and 2T4R; Support of 4 aperiodic SRS resource sets for 1T4R and 2 aperiodic SRS resource sets for 1T2R/2T4R; 2-53. |
| ZTE [3] | In addition, inter-span PDCCH repetition was agreed as a UE optional feature in RAN1#106bis-e meeting. Hence, one new FG should be introduced. Proposal 7: Introduce a new FG on support of inter-span PDCCH repetition. 23-2-4; Inter-span PDCCH repetition; Support of inter-span PDCCH repetition. In RAN1#106bis-e, a new UE optional feature is supported based on the following agreement. Agreement For extension of aperiodic antenna switching SRS configurations for <=4Rx, support N=4 for 1T4R and N=2 for 1T2R/2T4R. - The above extension is UE optional Hence a new FG is needed for such extension of aperiodic antenna switching SRS of 1T4R and 1T2R/2T4R. We have the following proposal. Proposal 15: Add the following new FG 23-8-9; Extension of aperiodic SRS configuration for 1T4R, 1T2R and 2T4R; Support of 4 aperiodic SRS resource sets for 1T4R and 2 aperiodic resource sets for 1T2R/2T4R.. Further, one remaining issue in an editor’s note of the Dec. RAN1 specification is whether RPFS is applicable to the case that frequency hopping is not enabled. We think one way to resolve this issue is to introduce a UE capability for the non-FH case. This new UE feature should use 23-8-6 as the prerequisite. If this capability is not reported, the UE cannot support to be configured with partial frequency sounding for non-FH case. Therefore, we have the following proposal. Proposal 16: Add the following new FG with 23-8-6 as the prerequisite. 23-8-6a; Applicability of partial frequency sounding for non-FH case; Support of partial frequency sounding for the case that FH is not enabled.. |
| vivo [4] | In RAN1#106bis-e meeting, a separate capability on number of max SRS sets for 1T4R and 1T2R/2T4R was agreed. Agreement: For extension of aperiodic antenna switching SRS configurations for <=4Rx, support N=4 for 1T4R and N=2 for 1T2R/2T4R. - The above extension is UE optional Therefore, we propose to add the following new FG. Proposal 8-1: add the following new FG to support capability enhancement on number of max SRS sets for 1T4R and 1T2R/2T4R. 23-8-9; Extension of aperiodic antenna switching SRS configurations for 1T4R, 1T2R and 2T4R; Support of 4 aperiodic SRS resource sets for 1T4R and 2 aperiodic resource sets for 1T2R/2T4R.. |
| CATT [5] | In Rel-17, MAC-CE-based beam indication is supported for all the UEs. Whether to support the DCI-based beam indication (i.e. using DCI format 1_1/1_2) depends on UE capability. In addition, for a UE supporting beam indication by DCI format 1_1/1_2, it must support beam indication by using DCI 1_1/1_2 with DL assignment. But the supportive of beam indication by DCI format 1_1/1_2 without DL assignment is UE optional. Therefore two separate UE features regarding the DCI-based beam indication (i.e. use of DCI formats 1_1/1_2 with DL assignment and use of DCI formats 1_1/1_2 with and without DL assignment) are required and should be reported to gNB. If DCI-based beam indication is supported, the beam application time, i.e. the first slot that is at least Y symbols after the last symbol of the acknowledgment of the joint or separate DL/UL beam indication, should be reported. Proposal-1: For DCI-based beam indication, additional UE feaures are added. 23-1-1a; DCI-based beam indication for intra- and inter-cell beam management; For both intra- and inter-cell beam management: 1. [MAC-CE+DCI-based TCI state indication (including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)] 2. [The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS]; 23-1-1; Per band; Optional. 23-1-1b; DCI-based beam indication for intra- and inter-cell beam management; For both intra- and inter-cell beam management: 1. [MAC-CE+DCI-based TCI state indication (including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)] 2. [The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS]; 23-1-1a; Per band; Optional. With respect to PL-RS, there are cases that the configured PL-RS is not QCLed with the spatial relation RS in the TCI state, i.e. beam misalignment. For some UEs, beam misalignment is not support due to its additional UE complexity. Whether a UE supports “beam misalignment or not” between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is a UE capability. The supportive of beam misalignment should be an additional UE capability. Proposal-2: An additional UE feature should be added for the supportive of beam misalignment. 23-1-1c; Support of beam misalignment; For both intra- and inter-cell beam management: 1. Beam misalignment between the DL source RS in the TCI state to provide spatial relation indication and the PL-RS; 23-1-1; Optional. |
| Samsung [6] | Additional UE feature group is needed to indicate support of CSI report based on Mode 1 (Option 1) with X=1, 2 as UE feature. Proposal 31: Support CSI report based on Mode 1 (Option 1) with X=1, 2 as UE optional feature. |
| NTT DOCOMO, INC. [7] |  |
| Spreadtrum Communications [8] | In RAN1#106b-e, we have the following agreement. Agreement Support combination of Rel-17 SFN PDCCH scheme 1 and single-TRP PDSCH - This is optional UE feature - Note: The support of such combination scheme is for URLLC use-case only.. Thus, we suggest to add one new FG to reflect this agreement. Proposal 9: Add one new FG to reflect to support the combination of Rel-17 SFN PDCCH scheme 1 and single-TRP PDSCH - FG 23-6-1b: Single TRP PDSCH scheduled by SFN scheme A PDCCH 23. NR_FeMIMO; 23-6-1b; Single TRP PDSCH scheduled by SFN scheme A PDCCH; Support single TRP PDSCH scheduled by SFN scheme A PDCCH; 23-6-1; Per UE; Optional with capability signalling. |
| Qualcomm Incorporated [9] | In RAN1 meeting #106bis-e, it was agreed to extend the number of aperiodic SRS sets for legacy xTyR configuration as separate UE feature. A separate UE feature 23-8-9 should be added to reflect the RAN1 agreement. Agreement For extension of aperiodic antenna switching SRS configurations for <=4Rx, support N=4 for 1T4R and N=2 for 1T2R/2T4R. - The above extension is UE optional.. Proposal 9-4: Add FG 23-8-9 supporting the extension of maximum number of sets for aperiodic SRS antenna switching configuration for <= 4Rx 23. NR_FeMIMO; 23-8-9; extension of maximum number of SRS sets for antenna switching; Support the extension of number of sets (N) for aperiodic SRS antenna switching to N = 4 for 1T4R, N = 2 for 1T2R/2T4R.; FG 2-55; Per BC; {Support, Not support}; Optional with capability signalling. |
| OPPO [10] |  |
| Intel Corporation [11] | We note that Rel-17 HST would support BFR enhancements for SFN PDCCH. In particular, BLER calculation for hypothetical PCCCH transmission are based on two reference signals, comparing to single reference signal supported in Rel-16. It is therefore proposed to introduce new FG 23-6-5 for this functionality as part of Rel-17 UE capabilities. In RAN1#106e meeting, it was also agreed to support PDCCH monitoring with for overlapping CORESETs configured with different QCL-Type D. Therefore, it is proposed to capture the corresponding functionality as part of new FG 23-6-6. 23-6-5; BFR for SFN PDCCH; Support of BFR using CORESET configured with two TCI states. 23-6-6; PDCCH monitoring of with different QCL-TypeD; Support of PDCCH monitoring of PDCCH candidates in overlapping monitoring occasions with different QCL-TypeD. In Rel-16 UE feature, the feature group on antenna switching includes another two components: txSwitchImpactToRx and txSwitchWithAnotherBand, which indicates the band that could have impact on the antenna switching operation. Following Rel-16 38.306 spec, the candidate values should be some integer value (1 to 32) indicating the entry number of the first-listed band with UL. In RAN1 #106b-e meeting, the below agreement was reached. Since this feature is UE optional, a new FG 23-8-9 should be introduced accordingly. Agreement For extension of aperiodic antenna switching SRS configurations for <=4Rx, support N=4 for 1T4R and N=2 for 1T2R/2T4R. - The above extension is UE optional. 23-8-9; Increased number of aperiodic SRS resource sets for xTyR with y<=4; - Support 4 aperiodic SRS resource sets for 1T4R - Support 2 aperiodic SRS resource sets for 1T2R - Support 2 aperiodic SRS resource sets for 2T4R. |
| Apple [12] | - We also proposed to have the following two FG - FG 23-6-5: Support implicit configuration of RS(s) with two TCI states for beam failure detection - FG23-6-6: Support of HST-SFN scheme 1 PDCCH scheduling Single-TRP PDSCH. This has been agreed. - We propose to introduce FG23-7-6 for the support of CSI-IMR 23. NR_FeMIMO; 23-6-5; Support implicit/explicit configuration of RS(s) with two TCI states for beam failure detection; Support RS(s) with two TCI states configured, either implicitly or explicitly, for beam failure detection enhancement for SFN scheme A for HST; 23-6-1; Y; N/A; Per band; N; N; {Support, Not support}; Optional. 23. NR_FeMIMO; 23-6-6; Support of SFN scheme A PDCCH scheduling Single-TRP PDSCH; Support of SFN scheme A PDCCH scheduling Single-TRP PDSCH; 23-6-1; Y; N/A; Per band; N; N; {Support, Not support}; Optional. 23. NR_FeMIMO; 23-7-6; Support of CSI-IM for CSI enhancement for Multi-TRP; Support CSI-IM for CSI enhancement for Multi-TRP; 23-7-1; Y; N/A; Per UE; N; Y; {Support, Not support}; Optional. |
| FUTUREWEI [13] |  |
| xiaomi [14] |  |
| LG Electronics [15] |  |
| Lenovo/Motorola Mobility [16] |  |
| Ericsson [17] |  |
| MediaTek Inc. [18] | Add the new optional FG 23-3-4 to capture the following agreement reached in RAN1 #106 bis-e: Agreement When 3 BDs are counted for two linked candidates - The third BD is counted in the later span for inter-span PDCCH repetition when r16monitoringcapablity is configured. - Note: Inter-span repetition is UE optional Proposal 36: Add optional UE feature, FG 23-2-4 to indicate support for inter-span PDCCH repetition 23. NR_FeMIMO; 23-2-4; Inter-span PDCCH repetition; Support of Inter-span repetition for PDCCH; 23-2-1; {Supported, Not supported}; Optional with capability signalling. We propose to add a new FG, 23-6-5 to capture the following agreement reach in RAN1 #106 bis-e Agreement Support combination of Rel-17 SFN PDCCH scheme 1 and single-TRP PDSCH - This is optional UE feature - Note: The support of such combination scheme is for URLLC use-case only. Proposal 50: Add the optional UE feature FG 23-6-5 to indicate support combination of Rel-17 SFN PDCCH scheme 1 and single-TRP PDSCH 23. NR_FeMIMO; 23-6-5; Support of Rel-17 SFN PDCCH scheme 1 and single-TRP PDSCH combination; Support combination of Rel-17 SFN PDCCH scheme 1 and single-TRP PDSCH; {Supported, Not supported}; Optional with capability signalling. In the RAN1#106bis-e meeting, we have the following agreement: Agreement For CSI measurement associated with a CSI-ReportingConfig for NCJT, support two CMRs within the same CMR pair configured for NCJT measurement hypothesis to be restricted within X continuous slot(s) without DL/UL switch between two CMRs - X=1, 2 - whereas X=1 implying the same slot and X=2 implying two adjacent slots - FFS other restrictions for FR2 - FFS whether UE capability is needed for X=2. We propose to add a new optional UE FG to indicate support of two resources in a resource pair configured within two adjacent slots, with candidate values {Supported, Not supported}. The reasons are two-fold: First, maintaining phase coherency across two slots increases UE implementation complexity. Second, awake for an additional slot consumes extra power. Proposal 53: Add FG 23-7-4 to indicate support of maximum slot offset X=2 for two resources in a resource pair. We propose to add a new optional UE FG to indicate support for Codebook type I Mode 2 in Multi-TRP CSI, with candidate values {Supported, Not supported}. Proposal 54: Add FG 23-7-5 to indicate support for Codebook type I Mode 2 in Multi-TRP CSI. 23. NR_FeMIMO; 23-7-4; Codebook type I Mode 2 in Multi-TRP CSI; Support for Codebook type I Mode 2 in Multi-TRP CSI; {Support, Not support}; Optional with capability signalling. |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] |  |

Other

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| ZTE [3] |  |
| vivo [4] | Additionally, it was agreed to enhance the number of max SRS sets for periodic SRS and semi-persistent SRS. Agreement For antenna switching SRS, support maximum one SRS resource set for periodic SRS and maximum 2 SRS resource sets for semi-persistent SRS. - Note: the two SP-SRS resource sets are not activated at the same time - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - Applies for all supported xTyR where y<=8 - For each xTyR antenna switching (except for 4T6R if supported), each periodic or semi-persistent resource set contains y/x resources. This feature is UE optional: For UEs that do not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS Thus, based on above two agreements which indicates two separate features, we have four possible combinations between feature A (extension of aperiodic antenna switching SRS configurations for <=4Rx) and feature B (extension of periodic and semi-persistent SRS) - Combination 1: {not support feature A, not support feature B} - Combination 2: {support feature A, support feature B} - Combination 3: {support feature A, not support feature B} - Combination 4: {not support feature A, support feature B} For combination 1 the configurations should Rel-15 spec on the number of resource sets for different resource type SRS, while the number of max resource sets of aperiodic and periodic/semi-persistent SRS is followed feature A and feature B simultaneously for combination 2, i.e. 2 semi-persistent SRS resource sets + 1 periodic SRS resource set + 4 periodic SRS resource sets for 1T4R and 4 periodic SRS resource sets for 1T2R/2T4R. In others words, we can keep same understandings on SRS resource sets and resources configuration for antenna switching usage for both combination 1 and combination 2. For 1T2R/2T4R, current specification of 38.214 limits the number of total SRS resource sets to 2 and the number of max SRS resource set per resource type to 1, i.e. 1 periodic + 1 semi-persistent, or 1 periodic + 1 aperiodic, or 1 semi-persistent + 1 aperiodic. However, the number of SRS resource set configurations are not clear for 1T2R/2T4R if either combination 3 or combination 4 is supported. More specifically, if combination 3 is supported, it represents maximum 2 aperiodic SRS resource sets can be configured as feature A supported, and, due to feature B not supported, periodic and semi-persistent SRS resource set configurations should Rel-15 spec. These ambiguous statements could have been understood in more than one way. For example, 1 periodic SRS resource set + 1 semi-persistent SRS resource set + 2 aperiodic SRS resource set may be configurated simultaneously for 1T2R/2T4R with supporting combination 3, or maybe only maximum 2 aperiodic SRS resource set can be configured. Similarly, for combination 4, it supports maximum 1 SRS resource set for periodic SRS and maximum 2 SRS resource sets for semi-persistent SRS, while an SRS resource configuration of 2 semi-persistent + 1 aperiodic is supported or is wrong understanding (configuration). Therefore, feature combination 3 and 4 may have ambiguities on SRS resource set configurations with different resource types, and accordingly, we propose to have a clear agreement to clarify SRS resource set configurations with different resource types for 1T2R/2T4R. Proposal 8-2: We suggest to make a common understanding/agreement on SRS resource set configurations with different resource types for 1T2R/2T4R if only one antenna switching enhancement feature relative to 1T2R/2T4R is supported. |
| CATT [5] |  |
| Samsung [6] |  |
| NTT DOCOMO, INC. [7] |  |
| Spreadtrum Communications [8] |  |
| Qualcomm Incorporated [9] |  |
| OPPO [10] |  |
| Intel Corporation [11] |  |
| Apple [12] |  |
| FUTUREWEI [13] | Some of the rows for UE features are relevant to Rel-17 M-TRP designs. A general question is whether the rows depend on Rel-16 M-TRP features, and if yes, in which ways and how to capture the dependency in the column for “Prerequisite feature groups”. For example, for the inter-cell M-TRP row with FG 23-4, it relies on M-DCI, and then a question is whether 3GPP should add Rel-16 M-DCI based M-TRP FG 16-2a as specified in TS 38.822 as the prerequisite for Rel-17 inter-cell M-TRP. FG 16-2a is specified as 16-2a; Multi-DCI based multi-TRP; 1. The maximum number of CORESETs configured per BWP per cell in addition to CORESET 0 2. The maximum number of CORESETs configured per CORESETPoolIndex ( if CORESETPoolIndex is not configured, it is assumed CORESETPoolIndex = 0) per BWP per cell in addition to CORESET 0 3. Support fully/partially overlapping PDSCHs in time and non-overlapping in frequency 4. Maximum number of unicast PDSCHs per CORESETPoolIndex per slot. To adopt the dependency on Rel-16 FG 16-2a generally seems to be a reasonable design, given the FeMIMO WID: - Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: - Identify and specify QCL/TCI-related enhancements to enable inter-cell multi-TRP operations, assuming multi-DCI based multi-PDSCH reception Alternatively, to allow more flexibility, 3GPP may decide to make the Rel-17 features separate from the Rel-16 ones. For the inter-cell M-TRP example, dependency on FG 16-2a implies that the Rel-17 inter-cell M-TRP has to support “fully/partially overlapping PDSCHs in time and non-overlapping in frequency”, and the UE does not have the option to support only inter-cell M-TRP multi-PUSCH or some other Rel-17 features without supporting multi-PDSCH. This could be considered as limiting in some scenarios. If that is the case, then 3GPP may not require FG 23-4 to depend on FG 16-2a for more flexibility. Therefore, we can see that more discussions are useful to address the Rel-17 M-TRP dependency on Rel-16 M-TRP FGs. Proposal 1: Discuss whether the Rel-17 M-TRP FGs depend on Rel-16 M-TRP FGs, and if yes, discuss how to capture the dependency in the column for “Prerequisite feature groups”. |
| xiaomi [14] |  |
| LG Electronics [15] |  |
| Lenovo/Motorola Mobility [16] |  |
| Ericsson [17] |  |
| MediaTek Inc. [18] |  |
| CMCC [19] |  |
| Nokia/Nokia Shanghai Bell [20] |  |

# Discussion/Approval Items during RAN1 #107bis-e — First Checkpoint

After review of contributions submitted to RAN1 #107bis-e in this agenda item, the following topics were identified by the moderator for discussion/approval during RAN1 #107bis-e.

General comments

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Samsung | For FG 23-1-1, 23-1-2, 23-1-3, and perhaps (depending on the outcome of the “maintenance” stage 23-1-4), rather using the current grouping based on joint/separate TCI and intra/inter-cell, after the description of each bullet item is stable, it is proposed that the following re-grouping be used: FG 23-1-0: Basic features (supported for UEs that support Rel-17 TCI) FG 23-1-1: Auxiliary feature group 1 FG 23-1-2: Auxiliary feature group 2 ... FG 23-1-n: Auxiliary feature group n It is noted that all Rel-17 UE features are optional in the absolute sense. - FG 23-1-0 includes Rel-17 UE features that are basic for UEs supporting Rel-17 TCI. - {FG 23-1-m, m>0} represent feature groups that can be supported in addition to FG 23-1-0 or conditioned on the support of other MIMO/non-MIMO features (one example would be, as Nokia pointed out, Carrier Aggregation). Each of {FG 23-1-m, m>0} would be a separate FG. For FG 23-1-0, our view is that it should AT LEAST include the following: - 4 activated TCI states and DCI-based beam indication (with and without DL assignment), for BOTH intra-cell and inter-cell beam management - Inter-cell beam measurement/reporting - TBD: Basic supported value(s) of K and Nmax |

# Issue 1: FG 23-1-3

After review of contributions submitted to RAN1 #107bis-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-1-3 | MPE mitigation | 1. Support of enhanced [PHR] reporting which includes pairs of (P-MPR, SSBRI/CRI) 2. Maximum number of reported P-MPR and SSBRI/CRI pairs 3. Maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation |  |  |  |  | Per band |  |  |  | Component 2 candidate values: of {1,2,3, 4} Component 3 candidate values: FFS {4,8,16,32,64} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Support in principle. In our understanding is correct, this parameter only applies to FR2, right? If so, {4,8} seems too small, and should be removed. |
| Samsung | Support. We also agree with ZTE that this only applies to FR2 and {4,8} can be removed. |
| Ericsson | Essentially OK. Propose to remove “in a RRC pool” – the meaning is unclear. |
| Apple | We are okay with the component and candidate value For the reporting type, in addition to per band, we also need to consider the total UE complexity especially for component 3. One solution is to also report per BC in addition to per band, or, we can consider similar solution as Rel-16 FG16-1g The FG should be FR2 only |
| LG | Fine with the proposal. |
| Qualcomm | Fine with the changes in red |
| CATT | Support in principle. We agree with ZTE that {4, 8} can be removed. |
| NTT DOCOMO | Regarding the candidate value of component 2 and 3, we think whether the number is per CC or across all CCs should be clarified. |
| OPPO | Fine with the proposal and the values {4, 8} shall be kept. This FG is for FR2 only. |
| Xiaomi | Fine with the proposal |
| Spreadtrum | Support in principle. We also think the FG is for FR2 only. |
| CMCC | Support the proposal. |
| MediaTek | We suggest some wording changes to component to avoid confusion. 3. Maximum number of candidate RS(s) configured in a RRC configured CSI-RS/SSB resource set pool for MPE mitigation Agreement from RAN1#106bis On Rel.17 enhancements to facilitate MPE mitigation, the candidate resource pool corresponds to a CSI-RS/SSB resource set configured via RRC (details up to RAN2) Share same view that this is an FR2-only feature, but we are also fine to current Component 3 candidate values |
| Intel | Support. The feature is applicable to FR2. |
| Lenovo/MotM | Fine with the proposal. |
| Nokia, NSB | OK |

# Issue 2: FG 23-1-4

After review of contributions submitted to RAN1 #107bis-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-1-4 | MPUE support for UL | [depending on agreement] 1. Supported UE capability value sets and corresponding max number of SRS ports for each UE capability value set |  |  |  |  | FSPC |  |  |  | Candidate values: Up to 4 value sets each with one value of {1,2,3,4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Support in principle. The candidate values seems to be wired. If our understanding is correct, there are only up to 3 types of configured SRS ports, i.e., 1, 2, 4. So, we think that it should be refined as ‘Candidate values: Up to 4 value sets each with one value of {1,2,3,4}’ |
| Samsung | - Since RAN1 can only agree on one UE capability for this feature (i.e. max # SRS ports, and concluded no consensus on the support of other types), “value set” should be revised to “value” in the above description. - Depending on the outcome of a handful of open issues on MP-UE (options for reporting the CRI/correspondence pair, support for ACK scheme, etc. which also depends on whether consensus can be reached or not), it is unclear that one row is sufficient for FG 23-1-4. |
| Ericsson | Do not support. According to RAN2 guidance, features that are specified “per FS” or “per FSPC” should be minimized. This should be signalled “per band”. |
| Apple | We think that the candidate value for the maximum number of SRS ports should be {1, 2, 4} without 3 We also prefer to discuss UE capability to indicate whether UE supports capability report set index report in AP/SP/P CSI report separately, especially to separate the AP CSI report since AP CSI report has much more stringent timing requirement |
| LG | Fine in principle. Regarding the candidate values for max number of SRS ports per UE capability value set, we agree with ZTE/Apple that 3 can be removed. |
| Qualcomm | Suggest to add a candidate value of 0 for UE to indicate this reported panel is DL only, and gNB should not use this reported DL RS to indicate beam for UL CB based Tx. We are fine to add bracket for more discussions later Candidate values: Up to [4] value sets each with one value of {[0],1,2,3,4} |
| NEC | Panel operation is better in band level or band combination level, so we think this feature should be “per band” or “per BC”. And we also think the outcome of open issues on MP-UE may have impact on this FG. |
| CATT | We are okay with this FG. |
| NTT DOCOMO | Agree with ZTE and Apple that the candidate value of number of ports should be {1,2,4} |
| OPPO | 1. There is no 3 SRS ports scenario. So the candidate value for the max number of SRS ports can only be {1, 2, 4} 2. The number of UE capability value sets should be up to 2, not up to 4.. |
| Xiaomi | The maximum number of SRS port can be 1,2 4, and according to the WA (chairman notes of RAN1-107 e-meeting) pasted as below, different number of SRS port will be indicated in different value set, so 3 value sets is OK. Working Assumption Support the UE reporting a list of UE capability value sets - Each UE capability value set comprises the max supported number of SRS ports - For any two different value sets, at least one capability value needs to be different - FFS: If in addition also identical value sets are allowed. Whether the UE capability value set can be common across all BWPs/CCs in same band or BC can be discussed in UE feature session |
| Spreadtrum | Support in principle. We also think ‘3’ should be removed from the candidate values. |
| CMCC | We agree with Qualcomm to add a candidate value for indicating a DL-only panel. |
| MediaTek | Support in principle. However, we share similar view as Samsung, four rows or components may be needed, e.g., - Support 1st UE capability value set and corresponding max number of SRS ports - Support 2nd UE capability value set and corresponding max number of SRS ports - Support 3rd UE capability value set and corresponding max number of SRS ports - Support 4th UE capability value set and corresponding max number of SRS ports Regarding candidate values we suggest removing the value 3 (since 3 port SRS is not needed), hence we propose candiate value of {1,2,4} for all 4 components mentioned above |
| Intel | Candidate values are not clear to us. - There is no 3 SRS antenna ports configuration - According to current agreements in RAN1 only different number of SRS antenna ports should be considered. In this case the candidate values should include only the following combinations: {1,2}, {1,4}, {2,4}, {1,2,4}. Based on the above the following revisions are proposed: Candidate values: {1,2}, {1,4}, {2,4}, {1,2,4}. |
| Lenovo/MotM | If the candidate values correspond to the number of SRS ports, 3 should be removed. |
| Nokia, NSB | Agree in principle, but we agree with Ericsson that signaling should be per band. |

# Issue 3: FG 23-2-2

After review of contributions submitted to RAN1 #107bis-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-2-2 | Two QCL TypeD for [CORESET monitoring in] PDCCH [repetition] | Support of determining two QCL-TypeD for overlapping in time CORESETs in the same CC or for intra-band CA [when UE is configured with PDCCH repetition] | [23-2-1, 23-6-1, 23-6-2] |  |  |  | Per band |  |  | FR2 only |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Support |
| Apple | We are okay with the proposed FG in principle |
| LG | Support |
| QC | Ok in principle. Instead of “for overlapping in time CORESETs”, we can say “time-domain overlapping CORESETs” (editorial). |
| CATT | Support. The primary goal of this enhancement is to support time-overlapping PDCCH repetition in FR2. Thus, the condition of ‘when UE is configured with PDCCH repetition’ shall be kept. We are generally fine with other modifications in this FG. |
| NTT DOCOMO | Support |
| OPPO | Support |
| Xiaomi | We think “PDCCH repetition” is not clear, and it is better to add more explanation of PDCCH repetition. e.g., PDCCH repetition with SFN scheme or non-SFN TDM and/or FDM sheme. |
| Spreadtrum | Support |
| CMCC | Support. QC’s version is also fine for us. |
| Huawei, HiSilicon | We are fine with the proposal. |
| MediaTek | Support. |
| Samsung | Support in principle. Regarding pre-requisite, if we have 23-2-1 only, then additional FG should be needed to receive two QCL-TypeD for SFN PDCCH. |
| Intel | Support |
| Lenovo/MotM | Support |
| Futurewei | Support. Fine with QC’s editorial comment |
| vivo | We have similar concern as xiaomi, this issue was discussed last year is mainly to resolve how to determine the priority of different QCL-typeD reception only in case of FDM-based PDCCH repetition or SFN-based PDCCH transmission in current CC rather than different QCL-typeD overlapping in time due to the configuration multiple respective TDM-based PDCCH repetition in same CC or between CCs from intra-band CA. Another way is to clarify this is if “linked PDCCH cadidates” and SFN. |
| Nokia, NSB | Support |

# Issue 4: FG 23-2-3

After review of contributions submitted to RAN1 #107bis-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Delete FG/row 23-2-3

| 23. NR_FeMIMO | 23-2-3 | PDCCH repetition for Type3 CSS | Support of PDCCH repetition for Type3 CSS |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Support of removing this FG as we don’t see any difference for CSS and USS PDCCH repetition from UE complexity perspective. |
| LG | Support |
| QC | We are not comfortable with removing this FG at this point. It very much depends on the components of baseline FG and other required FGs for different PDCCH monitoring capabilities as we proposed in our contribution. We can decide about removing this FG or not after some level of details for PDCCH repetition UE capability is agreed. |
| CATT | Support the proposal. Compared with PDCCH repetition for USS, PDCCH repetition for Type 3 CSS will not require additional complexity. Therefore, there is no need to introduce additional capability for Type 3 CSS PDCCH repetition. |
| NTT DOCOMO | Support |
| Xiaomi | Depends on the discussion of 23-2-1 |
| CMCC | Supoort. We can put it in FG 23-7-1. |
| Huawei, HiSilicon | We support to remove this FG. |
| MediaTek | Agree with QCs comment |
| Samsung | Support. It can be included in basic feature group 23-2-1. |
| Intel | Support. It can be also clarified that Type 3 CSS is part of 23-2-1 |
| Lenovo/MotM | We are okay with the proposed FG from UE receiving view. If FG 23-2-3 is deleted, we think the text of “including PDCCH repetition for Type 3 CSS” in the bracket of FG 23-2-1 needs being included. |
| Futurewei | Support. Any further adjustment (if needed) can be addressed in 23-2-1. |
| vivo | Agree with QC’s comments |
| Nokia | Ok to remove this FG. |

# Issue 5: FG 23-6-1

After review of contributions submitted to RAN1 #107bis-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-6-1 | SFN scheme A (scheme 1) | 1. [Support of scheme A for PDCCH] scheduling Scheme A PDSCH and [and default QCL assumption with two TCI states for PDCCH] 2. Support of scheme A for PDSCH [scheduled by [single TRP/ Scheme A] PDCCH] [and default QCL assumption with two TCI states for PDSCH] |  |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Okay in principle. For component 2, we think the part “scheduled by single TRP/ Scheme A PDCCH” should be reserved to capture the case when single TRP PDCCH scheduling SFN scheme A PDSCH, which has been endorsed by the following agreement in RAN#107-e meeting. Agreement When SFN PDSCH and SFN PDCCH are configured by RRC , for PDSCH reception scheduled by DCI formats 1_1 and 1_2, and, if applicable the time offset between the reception of the DL DCI and the corresponding PDSCH is equal or larger than the threshold timeDurationForQCL - Support configuration when there is no TCI field in the DCI scheduling PDSCH - UE applies the TCI state(s) of the scheduling CORESET when receiving the PDSCH - If there are two active TCI states for the CORESET , UE applies both QCL assumptions of the CORESET that schedules the PDSCH when receiving the PDSCH - otherwise, if there is one active TCI state for the CORESET , UE applies the one active TCI state of the CORESET when receiving the PDSCH This feature is UE optional capability - If UE doesn’t support this capability, UE is expected to be configured with TCI state field - UEs supporting this feature and are not capable of dynamic switching between single TRP and SFN , the CORESET that schedules PDSCH by DCI formats 1_1 and 1_2 (FFS DCI format 1_0) should be activated with two TCI states. FFS for maintenance: if SFN PDCCH is not configured |
| Ericsson | Support in principle. “the default QCL assumption with two TCI states” is not clear to us, UE shall always support default QCL assumption with 1 TCI state for PDSCH to receive broadcasting information. We suggest revising the component as : 1. [Support of scheme A for PDCCH] scheduling Scheme A PDSCH and [and default QCL assumption with one or two TCI states for PDCCH] 2. Support of scheme A for PDSCH [scheduled by [single TRP/ Scheme A] PDCCH] [and default QCL assumption with one or two TCI states for PDSCH] |
| Apple | - If we take the component 1 as it is, we also need to add another FG/component of “Support of SFN scheme A PDCCH scheduling Single-TRP PDSCH” which was agreed to be optional - For both component 1 and component, we should remove the default beam related description since the content is not clear at all - and default QCL assumption with two TCI states for PDCCH - and default QCL assumption with two TCI states for PDSCH For the reporting type, we prefer to have “per FS”. This is an advanced PDCCH/PDSCH processing capability, UE may not support it in all the BC. Depending on the number of bands and the band itself, UE may or may not support this feature in a particular band. |
| LG | For Component 1: We think the combination of scheme 1 PDCCH and single TRP PDSCH should be added based on the following agreement. Agreement @106b-e Support combination of Rel-17 SFN PDCCH scheme 1 and single-TRP PDSCH - This is optional UE feature - Note: The support of such combination scheme is for URLLC use-case only. And, default QCL assumption for PDCCH is not clear to us because TCI state(s) for PDCCH can be explicitly configured by RRC/MAC-CE. Our proposal for component 1 is as follows. 1. [Support of scheme A for PDCCH] scheduling single TRP/ Scheme A PDSCH and [and default QCL assumption with two TCI states for PDCCH] For Component 2: We also prefer to keep the part “scheduled by single TRP/ Scheme A PDCCH”. And, regarding default QCL assumption in component 2, we think it should be removed because FG 23-6-4 is defined separately for default QCL assumption capability. Our proposal for component 2 is as follows. 2. Support of scheme A for PDSCH scheduled by single TRP/ Scheme A PDCCH [and default QCL assumption with two TCI states for PDSCH] |
| Qualcomm | - Remove the wording of default QCL assumption at both components. - To further make the description clear, suggest the following wording of the two components - Support of scheme A for both PDCCH and PDSCH. - Support of scheme A for PDSCH only. - We are okay with per FS reporting type as well. |
| CATT | Based on the agreement in RAN1#106bis-e meeting, the transmission combination of Rel-17 scheme 1 PDCCH and single-TRP PDSCH also has been supported with UE capability. So our suggestion is to add one component for the UE capability of Rel-17 scheme 1 PDCCH and single-TRP PDSCH. - [Support of scheme A for PDCCH] scheduling Scheme A PDSCH [and default QCL assumption with one or two TCI states for PDCCH] - [Support of scheme A for PDCCH] scheduling single TRP PDSCH [and default QCL assumption with one or two TCI states for PDCCH] - Support of scheme A for PDSCH [scheduled by [single TRP/ Scheme A] PDCCH] [and default QCL assumption one or two TCI states for PDSCH] |
| NTT DOCOMO | Support in principle. We think “per band” reporting is OK. For component 1, “default QCL assumption with two TCI states for PDCCH” is not clear. It should be “default QCL assumption for PDSCH in case of two active TCI states for PDCCH”. For component 2, we think the part “scheduled by single TRP/ Scheme A PDCCH” should be kept, otherwise it is not clear the difference between component 1 and 2. Also, we need to add the following component 3 to cover all supported cases. 3. Support of single TRP PDSCH scheduled by scheme A PDCCH and default QCL assumption with one TCI states for PDSCH |
| OPPO | - We also propose to delete the default QCL part, which can be separate FG. - Support SFN scheme A PDCCH scheduling Single-TRP PDSCH as separate optional FG. - Support to keep tha part “scheduled by single TRP/ Scheme A PDCCH” |
| Xiaomi | First, we suggest to list all the combination of SFN scheme A - Support of scheme A for PDCCH scheduling scheme A PDSCH - Support of scheme A for PDCCH scheduling single TRP PDSCH - Support of single-TRP for PDCCH scheduling scheme A PDSCH Second, we want to clarify that, what is the meaning of default QCL assumption with two TCI states for PDCCH? Does it mean the issue discussed in FG23-2-2, “Two QCL TypeD for CORESET monitoring in PDCCH repetition”? If yes, we suggest to remove it since it can be discussed in FG23-2-2. - While for default QCL assumption with two TCI states for PDSCH, we prefer to move it to FG23-6-4. |
| Spreadtrum | - Suggest to remove default QCL assumption for all components. It is not clear to us. Does it mean the default QCL assumptions for PDSCH or PDCCH? - We also suugest to add one new component to reflect the agreement posted by LG Regarding reporting type, we also ok with FS type |
| CMCC | For component 2, we suggest to revise component 2 as “support of scheme A for PDSCH scheduled by single TRP PDCCH”, since scheme A PDCCH+scheme A PDSCH is already captured in component 1. |
| Huawei/HiSilicon | Regarding the report type of FG23-6 for HST, we prefer FSPC. A finer report type will enable the chipest/UE vendor to implement and operator/gNB vendor to depoly this advanced feature with basic functions as soon as possible. This feature needs more processing requirements in UE implementation as one typical case of mTRP. Like the legacy Rel-16 multi-DCI multi-TRP capability, this FG can also be FSPC. In addition to basic feature, more processing capability to support T-F tracking/channel estimation/demodulation is required for UEs, as there are many different combinations of TCI states for SFN scheme. We also propose to add two features to limit the TF/beam tracking capability and PDSCH processing capability, respectively. - FG 23-6-5 for TCI tracking capability in HST - FG 23-6-5: Component 1. The maximal total number of activated TCI states per BWP per CC including data and control, with candidate values {1, 2, 4, 8 ,16}; - FG 23-6-5: Component 2. The maximal total number of activated TCI states all BWPs all CCs including data and control, with candidate values {1, 2, 4, 8 ,16, 32}; - FG 23-6-6 for PDSCH processing capability in HST - FG 23-6-6: Component 1. The maximal number of PDSCHs per slot, with candidate values {1, 2, 3, 4 ,7}; |
| MediaTek | Support in principal, however, we have the following comments: - Remove default QCL assumption section from both components - A new optional FG to be introduced to capture, A PDCCH scheduling Single-TRP PDSCH scenario. We agree with the proposed “per band” reporting type. |
| Samsung | Support in principle. - Regarding wording, it would be better to add “SFN” in front of scheme A. - Regarding Component 2, we are fine with keeping “scheduled by single TRP/Scheme A PDCCH”. - Regarding Component 1 and 2, we are fine with deleting default QCL assumption with two TCI states. - Regarding Component 1, we would like to add “single-TRP” as a possible scheduling PDSCH. Then it can be changed as “Support of SFN scheme A for PDCCH scheduling single-TRP/SFN scheme A PDSCH” in order to reflect the following agreement in RAN1#106b-e. Agreement Support combination of Rel-17 SFN PDCCH scheme 1 and single-TRP PDSCH - This is optional UE feature Note: The support of such combination scheme is for URLLC use-case only. |
| Intel | 1. Current formulation of 1st component of FG 23-6-1 doesn’t cover other scheduling option supported in the specification, e.g. SFN scheme A PDCCH scheduling single-TRP PDSCH. Also, it is not clear meaning of default QCL assumption with TCI state for PDCCH. 2. In the 2nd component suggest keeping brackets for default QCL assumption (i.e., [and default QCL assumption with two TCI states for PDSCH]), since it is not clear which default QCL assumptions are meant and there are earlier agreements to have some default QCL assumption as separate UE capability. Suggest we start from definition of the FG in more high level e.g. as follows 1. SFN scheme A for PDSCH 2. SFN scheme A for PDSCH and PDCCH The restriction of the combination schemes should be captured in RRC spec as part of configuration parameters definition and not in UE capability! |
| Lenovo/MotM | Support in principle. We prefer to keept the text “scheduling Scheme A PDSCH” in component 1 and “scheduled by single TRP/ Scheme A PDCCH” in component 2 for clear description. |
| vivo | Regarding Component 1, we prefer to add “single TRP” and remove “the default QCL assumption with two TCI states for PDCCH”. Regarding Component 2, we prefer to reserve “scheduled by single TRP/ Scheme A PDCCH” and remove “the default QCL assumption with two TCI states for PDSCH”. 1. Support of scheme A for PDCCH scheduling single TRP/Scheme A PDSCH and [and default QCL assumption with two TCI states for PDCCH] 2. Support of scheme A for PDSCH scheduled by single TRP/ Scheme A PDCCH] [and default QCL assumption with two TCI states for PDSCH] |
| Nokia/NSB | Suport in principle. Component 1, default QCL assumption is applied only for PDSCH, so it is unclear. We have many cases, so we can further discuss on default QCL assumption. Component 2, default QCL with one TCI state is supported by legacy capability. No need for this. 1. [Support of scheme A for PDCCH] scheduling Scheme A PDSCH [and default QCL assumption with two TCI states for PDCCH] 2. Support of scheme A for PDSCH [scheduled by [single TRP/ Scheme A] PDCCH] [and default QCL assumption with two TCI states for PDSCH] 3. Support of single TRP PDSCH scheduled by scheme A PDCCH and default QCL assumption with one TCI states for PDSCH |

# Issue 6: FG 23-6-1a

After review of contributions submitted to RAN1 #107bis-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-6-1a | Dynamic switching - scheme A | Support of dynamic switching between single-TRP and SFN PDSCH scheme A by TCI state field in DCI formats 1_1, 1_2 | 23-6-1 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Okay. |
| Ericsson | Support. |
| Apple | We are okay with this FG |
| Qualcomm | Support FG 23-6-1a. The reporting type should be similar to 23-6-1 |
| CATT | Support. |
| NTT DOCOMO | OK. |
| OPPO | Support |
| Xiaomi | Support |
| Spreadtrum | Support |
| CMCC | Support. |
| Huawei/HiSilicon | Similar to comment to FG 23-6-1, we prefer FSPC. A finer report type will benefit the chipest/UE vendor to implement and operator/gNB vendor to depoly this advanced feature with basic functions as soon as possible. |
| MediaTek | Support |
| Samsung | Support |
| Intel | Support |
| Lenovo/MotM | Support |
| vivo | Support |
| Nokia, NSB | OK |

# Issue 7: FG 23-6-2

After review of contributions submitted to RAN1 #107bis-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-6-2 | SFN scheme B (TRP based pre-compensation) | 1. [Support of scheme B for PDCCH] scheduling Scheme B PDSCH and default QCL assumption with two TCI states for PDCCH 2. Support of scheme B for PDSCH and default QCL assumption with two TCI states for PDSCH [scheduled by [single TRP/Scheme B] PDCCH] | [23-6-1] |  |  |  | Per band |  | [FR1 only] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Okay in principle. For component 2, we think the part “scheduled by single TRP/ Scheme A PDCCH” should be reserved to capture the case when single TRP PDCCH scheduling SFN scheme B PDSCH, which has been endorsed by the following agreement in RAN#107-e meeting. Agreement When SFN PDSCH and SFN PDCCH are configured by RRC , for PDSCH reception scheduled by DCI formats 1_1 and 1_2, and, if applicable the time offset between the reception of the DL DCI and the corresponding PDSCH is equal or larger than the threshold timeDurationForQCL - Support configuration when there is no TCI field in the DCI scheduling PDSCH - UE applies the TCI state(s) of the scheduling CORESET when receiving the PDSCH - If there are two active TCI states for the CORESET , UE applies both QCL assumptions of the CORESET that schedules the PDSCH when receiving the PDSCH - otherwise, if there is one active TCI state for the CORESET , UE applies the one active TCI state of the CORESET when receiving the PDSCH This feature is UE optional capability - If UE doesn’t support this capability, UE is expected to be configured with TCI state field - UEs supporting this feature and are not capable of dynamic switching between single TRP and SFN , the CORESET that schedules PDSCH by DCI formats 1_1 and 1_2 (FFS DCI format 1_0) should be activated with two TCI states. FFS for maintenance: if SFN PDCCH is not configured |
| Ericsson | Support. Same comments as 23-6-1, default QCL assumption with one or two TCI states |
| Apple | - If we take the component 1 as it is, we would like to confirm that scheme B PDCCH cannot schedule Single-TRP PDSCH. Based on the our understanding, it is the case. - For both component 1 and component, we should remove the default beam related description since the content is not clear at all - and default QCL assumption with two TCI states for PDCCH - and default QCL assumption with two TCI states for PDSCH For the reporting type, we prefer to have “per FS”. This is an advanced PDCCH/PDSCH processing capability, UE may not support it in all the BC. Depending on the number of bands and the band itself, UE may or may not support this feature in a particular band. |
| LG | For Component 1: As commented in 23-6-1, default QCL assumption for PDCCH is not clear to us, so we prefer to remove it. Our proposal for component 1 is as follows. 1. [Support of scheme B for PDCCH] scheduling Scheme B PDSCH and default QCL assumption with two TCI states for PDCCH For Component 2: Same comment as component 2 in 23-6-1. Our proposal for compoenent 2 is as follows. 2. Support of scheme B for PDSCH and default QCL assumption with two TCI states for PDSCH scheduled by single TRP/Scheme B PDCCH |
| Qualcomm | - Remove the wording of “default QCL assumption with two TCI states” at both components. Also, we are fine with per FS reporting type. - Remove the pre-requisite 23-6-1. As commented earlier, this scheme is intended for simpler UE demod processing and shouldn’t be not conditioned on support of advanced demod schemes (23-6-1) - To further make the description clear, suggest the following wording of the two components 1. Support of scheme B for both PDCCH and PDSCH. 2. Support of scheme B for PDSCH only. |
| CATT | Similar view as FG 23-6-1. If the transmission combination of Rel-17 TRP based pre-compensation PDCCH and single-TRP PDSCH is supported, the similar component should be added. |
| NTT DOCOMO | Support in principle. We think “per band” reporting is OK. For component 1, “default QCL assumption with two TCI states for PDCCH” is not clear. It should be “default QCL assumption for PDSCH in case of two active TCI states for PDCCH”. For component 2, we think the part “scheduled by single TRP/ Scheme A PDCCH” should be kept, otherwise it is not clear the difference between component 1 and 2. |
| OPPO | - We also propose to delete the default QCL part, which can be separate FG. - Support to keep tha part “scheduled by single TRP/ Scheme B PDCCH” |
| Xiaomi | First, we suggest to list all the combination of SFN scheme B - Support of scheme B for PDCCH scheduling scheme B PDSCH - Support of single-TRP for PDCCH scheduling scheme B PDSCH Second, we want to clarify that, what is the meaning of default QCL assumption with two TCI states for PDCCH? - While for default QCL assumption with two TCI states for PDSCH, we prefer to move it to FG23-6-4. |
| Spreadtrum | - Suggest to remove default QCL assumption for all components. It is not clear to us. Does it mean the default QCL assumptions for PDSCH or PDCCH? - Regarding reporting type, we also ok with FS type |
| CMCC | For component 2, we suggest to revise component 2 as “support of scheme B for PDSCH scheduled by single TRP PDCCH”, since scheme B PDCCH+scheme A PDSCH is already captured in component 1. |
| Huawei/HiSilicon | Similar to comment to FG 23-6-1, we prefer FSPC. A finer report type will benefit the chipest/UE vendor to implement and operator/gNB vendor to depoly this advanced feature with basic functions as soon as possible. On the pre-requisite, we don’t see the reason to force the scheme-capable UEs to support SFN scheme A. In fact, as gNB has pre-compensated the frequency shift, schemeB needs less processing, while in schemeA, UE has to handle the frequency shift by itself. |
| MediaTek | Suggest to remove default QCL assumption section from both components. |
| Samsung | Support in principle. - Regarding wording, it would be better to add “SFN” in front of scheme B. - Regarding Component 2, we are fine with keeping “scheduled by single TRP/Scheme B PDCCH”. - Regarding Component 1 and 2, we are fine with deleting default QCL assumption with two TCI states. |
| Intel | Similar to the comment made for FG 23-6-1, we proposed to start from definition of the FG in more high level e.g. as follows 1. SFN scheme B for PDSCH 2. SFN scheme B for PDSCH and PDCCH The restriction of the combination schemes should be captured in RRC spec as part of configuration parameters definition and not in UE capability! |
| Lenovo/MotM | Scheme B does not require Scheme A as a pre-requisite (no agreement stated or implied that). Therefore we propose removing 23-6-1 as a pre-requisite |
| vivo | Regarding Component 1, we prefer to remove “the default QCL assumption with two TCI states for PDCCH”. Regarding Component 2, we prefer to reserve “scheduled by single TRP/ Scheme B PDCCH” and remove “and default QCL assumption with two TCI states for PDSCH”. 1. Support of scheme B for PDCCH scheduling Scheme B PDSCH and default QCL assumption with two TCI states for PDCCH 2. Support of scheme B for PDSCH and default QCL assumption with two TCI states for PDSCH scheduled by single TRP/Scheme B PDCCH |
| Nokia/NSB | Similar to Scheme A. 1. [Support of scheme B for PDCCH] scheduling Scheme B PDSCH and default QCL assumption with two TCI states for PDCCH 2. Support of scheme B for PDSCH and default QCL assumption with two TCI states for PDSCH [scheduled by [single TRP/Scheme B] PDCCH] |

# Issue 8: FG 23-6-2a

After review of contributions submitted to RAN1 #107bis-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-6-2a | Dynamic switching - scheme B | Support of dynamic switching between single-TRP and SFN PDSCH scheme B by TCI state field in DCI formats 1_1, 1_2 | 23-6-2 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Okay. |
| Ericsson | Support. |
| Apple | We are okay with this FG |
| Qualcomm | Support FG 23-6-2a, the reporting type should be similar to 23-6-2. |
| CATT | Support. |
| NTT DOCOMO | OK. |
| OPPO | Support |
| Xiaomi | Support |
| Spreadtrum | Support |
| CMCC | Support |
| Huawei/HiSilicon | Similar to comment to FG 23-6-1, we prefer FSPC. A finer report type will benefit the chipest/UE vendor to implement and operator/gNB vendor to depoly this advanced feature with basic functions as soon as possible. |
| MediaTek | Support |
| Samsung | Support |
| Intel | Support |
| Lenovo/MotM | Support |
| vivo | Support |
| Nokia, NSB | OK |

# Issue 9: FG 23-6-4

After review of contributions submitted to RAN1 #107bis-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-6-4 | Default beam setup [for SFN HST] | - Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold FFS: 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI [when PDSCH is scheduled with offset equal or larger than the threshold] - Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold FFS: 4. Support of single-TRP PUCCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured FFS: 5. Support of single-TRP PUSCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured FFS: 6. Support of single-TRP SRS resource transmission using default beam when enhanced SFN PDCCH transmission scheme is configured | 23-6-1a, 23-6-2a |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Okay. |
| Ericsson | Support. |
| Apple | We think this FG should be split into DL and UL, or, UE should be able to indicate whether UE support each component independently. We also need to consider additional UE FG - Support implicit/explicit configuration of RS(s) with two TCI states for beam failure detection |
| Qualcomm | - The prerequisite should be FG 23-6-1 and FG 23-6-2 instead of FG 23-6-1a and FG 23-6-2a - Suggest splitting into two FGs, one for DL default beam (components 1, 2 and 3) and the 2nd one for UL default spatial relation and pathloss RS (components 4,5 and 6). |
| CATT | Support. |
| NTT DOCOMO | Support in principle. But, prerequisite feature(s) is not 23-6-1a, 23-6-2a, and it should be FG 23-6-1 only. For example, if UE supports Scheme A and default QCL for schemeA, UE should be able to report FG23-6-4, even without FG 23-6-2. |
| OPPO | Agree with QC that downlink and uplink should be separate FG. |
| Xiaomi | First, the prerequisite should be FG 23-6-1 and FG 23-6-2. Second, we also prefer to split into two FGs, one for DL and the other one for UL. |
| Spreadtrum | We also think the FG should be split into two FGs, one is for DL, and one is for UL. |
| CMCC | Support. |
| Huawei, HiSilicon | Fine. |
| MediaTek | Agree with Qualcomm on spiliting the FGs into two separate DL (components 1, 2 and 3) and UL (components 4,5 and 6) FGs. |
| Samsung | We would like to split this as multiple FGs, at least considering DL and UL separately. Although our preference is to split this as three FGs (component 3 for a single DL default beam, component 1 and 2 for two DL default beams, component 4, 5, and 6 for a single UL default beam), we are fine with splitting just two FGs (component 1, 2, and 3 for DL default beam and component 4, 5, and 6 for UL default beam) to consider DL and UL separately. Also, this is FR2 only. Regarding pre-requisite, it should be FG 23-6-1 and 23-6-2. |
| Intel | Support |
| Lenovo/MotM | Support |
| vivo | Support in principle, and we are also fine to spilit the FGs into DL and UL part. |
| Nokia, NSB | OK |

# Issue 10: FG 23-7-2

After review of contributions submitted to RAN1 #107bis-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-7-2 | Support of more than 16 max # of Tx ports [per source/across two CMRs] [and max # of resources in a resource set for Multi-TRP CSI] [and max # resources] | [A list of supported combinations, each combination is {max # of Tx ports per source in a resource set for Multi-TRP CSI, max # resources in a resource set for Multi-TRP CSI}] [Note: same number of ports among CMRs] |  |  |  |  | Per band |  |  |  | Max # of Tx ports per source: [{4, 8, 12, 16, 24, 32}] Max # resources: {2, 3, 4, 5, 6, 7, 8} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | We think that support of 32 Tx ports per source in a resource set is essential for mTRP-CSI (which is also aligned with the title of this feature 23-7-2), and the other candidate value should be further justified before being approved. So, we think that the candidate value should be updated as ‘Max # of Tx ports per source: { [4, 8, 12, 16, 24,] 32};’ |
| Ericsson | In the third column, it is not clear why ‘more than 16 Tx ports’ has to be mentioned explicitly. This feature should support the cases when the number of Tx ports per source is smaller than 16 as candidate values of 4, 8, and 12 are captured in the candidate values above. Hence, the third column should be revised as ‘Support of max # of Tx ports per resource and max # of resources in a resource set for multi-TRP CSI’ A second question is that such combination of {max # of Tx ports per resource, max # of resources} seems needed to operate the basic feature for Multi-TRP CSI. Otherwise, we’ll have to define some default max # of Tx ports per resource and max # of resources in the basic feature for Multi-TRP CSI. Hence, we suggest to merge this as a component of the basic feature in 23-7-1. |
| Apple | - We should allow UE to report multiple list, e.g., 16, similar as the triplet report for MIMO codebook - One or multiple A list of supported combinations (the maximum number of lists can be 16) - The reporting Type should be “per band and per BC” to minimize the UE under-reporting issue, similar as MIMO codebook |
| LG | It seems that “per source” should be “per resource”. |
| QC | Do not support. As we explained in detail in our contribution, this list has to be included within the basic FG (FG 23-7-1). Otherwise, a default assumption would be needed, which may not be possible to have since these capabilities are across all CSI report settings and across all CCs (some of which may not be even configured with NCJT CSI). Furthermore, we would like to point out the following: - The FG (basic FG) needs to be consistent with the Rel-15 and Rel-16 CSI capability framework (e.g., FGs 2-35 and 2-36) - Due to complexity for UE to calculate two PMIs jointly for NCJT, a list of vectors for NCJT capabilities and sTRP capabilities are needed. - The complexity associated with a NCJT CSI can be much more than twice of the complexity associated with a sTRP CSI. For example, if UE evaluates 8 PMI/RI hypos for a given sTRP CSI, UE may need to evaluate up to 8*8=64 hypos for a given NCJT CSI for optimal results and not just 16 hypos, where the exact additional complexity may depend on UE implementation, and hence, it may make sense to separate the CPU/resource/port occupation budget. - This was discussed in RAN1 before. The reason that relaxed timeline (Z,Z’) were not agreed was the understanding that UE capability framework will provide necessary flexibility to handle the UE complexity. - Note that proposed components 3 and 4 are extensions of Rel-15 components 1 and 2 of FG 2-36. - Proposed component 3 should be reported per band and separately per band combination. This is same as legacy behavior for reporting triplets for each codebook per band and separately per band combination. We propose the following instead: 23-7-1; Basic Features of CSI Enhancement for Multi-TRP; 1. Maximum number of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Nmax=1 2. CSI report mode selection 3. A list of supported combinations, each combination is {max # of ports per resource for NCJT, max # of resources for NCJT, max # of total ports for NCJT, max # of ports per resource for sTRP, max # of resources for sTRP, max # of total ports for sTRP} simultaneously across all CCs 4. Supported codebook modes for NCJT CSI; Per Band; Component 3: The list can have maximum of 16 combinations. For each combination, candidate values are - max # of ports per resource for NCJT: {2, 4, 8, 12, 16, 24, 32} - max # of resources for NCJT: {2 to 64} - max # of total ports for NCJT: {4 to 256} - Max # of Tx ports in one resource for sTRP: {2, 4, 8, 12, 16, 24, 32} - Max # resources for sTRP: {1 to 64} - Max # total ports for sTRP: {2 to 256} Note: Component 3 is also reported per band combination (in addition to per band). Component 4:{ mode 1, both mode 1 and mode 2}; Optional with capability signalling. Furthermore, we think additional components similar to Rel-15 components 8 and 9 of FG 2-35 are also needed, but we can first focus on the above components 3 and 4. |
| CATT | Similar view with ZTE. There is no reason to restrict the number of Tx ports per source in a resource set for NCJT. |
| OPPO | Support to combine this FG to 23-7-1 similar to Rel-15/16, with a combination list for {# of port, # of resource and # of total port number}. |
| Huawei, HiSilicon | - The changes makes FG discussion more complicated. It will lead to under-report actual UE capability since the UE needs to assume the maximal number of port, e.g. 32 always, to determine the max number of resources, e.g. 1 or 2, if the UE can only report individual max #, rather than a combination of max #. The support of 23-7-2 component 1 shall be covered by triplet reporting, e.g. with a list of supported combinations of active NZP CSI-RS resource pairs and associated ports. Each combination is {Max # of Tx ports per resource within a resource pair, Max # of resources and total # of Tx ports across all resource pairs} across all CCs simultaneously. - The note is not needed since it is hard-coded in RAN1 spec for the restriction of port # in a set. |
| MediaTek | - Support to combine this FG with 23-7-1 similar to Rel 15-16. No need to mention “more than 16” as UE can also report smaller values. We suggest rewording as: - “Support of combinations of # of Tx ports per resource and # of resources in a resource set for Multi-TRP CSI” - Typo in the field of candidate values: It should be “per resource”, instead of “per source”. - Agree with Apples comment the reporting type should be “per band and per BC”. |
| Samsung | We are fine with keeping this as separate FG since more than 16 ports per resource was agreed as optional UE feature. In order to align the name of FG and the corresponding candidate value, we also suggest to maintain 24 and 32 only for the max # of TX ports per source (i.e., remove 4, 8, 12, 16). |
| Intel | Support |
| Lenovo/MotM | Agree with Ericsson, QC. This should be merged with FG 23-7-1. We also share the same understanding as LG that “per source” should be “per resource” |
| Nokia/NSB | OK to replace this FG with list of triplets in additional component of 23-7-1. This component should be reported per band and per BC, like for similar triplet lists in Rel-15/16. |
| vivo | This FG seems to reflect the following agreement. Since the description is “support of more than 16 max # of Tx ports [per resource/across two CMRs]”, we don’t understand why the candidate values of in the second last column includes values smaller than 16 such as {4, 8, 12}? We think the candidate values of {4, 8, 12} should be moved to the basic FG 23-7-1. Another thing to clarify, whether the UE is allowed to report multiple combinations {max # of port per resource, max # resources in a resource set for Multi-TRP CSI }. In our mind, the answer is yes since, e.g., {4, 8} and {8, 4} both can be supported by the UE but with different configurations. Agreement For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, [at least for multi-DCI based and single-DCI based schemes (scheme 1a)], NZP CSI-RS resources for channel measurement are associated to different TRPs/TCI states at resource level - CMRs corresponding to different TRPs respectively shall be configured within the same resource set (i.e. scheme 1-2) and have the same number of ports among CMRs. - At least ‘typeI-SinglePanel’ codebook is supported - FFS: Other codebook types - Note that RAN1 shall strive to finalize NCJT CSI enhancement with single reporting setting firstly. - The support of larger than 32 ports across two CMRs is optional for a UE supporting Rel. 17 mTRP CSI |

# Issue 11: FG 23-7-3

After review of contributions submitted to RAN1 #107bis-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Delete FG/row 23-7-3

| 23. NR_FeMIMO | 23-7-3 | More than two resources in a resource set for Multi-TRP CSI | FFS exact candidate values, Ks,max is up to 8 |  |  |  |  |  |  |  |  | [candidate values are {2, 3, 4, 5, 6, 7, 8}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Support the Moderator’s proposal. The corresponding component can be considered in 23-7-1. |
| Ericsson | Fine removing the FG. |
| Apple | We are okay to consider this as part of the FG 23-7-1 |
| QC | Instead of removing things at this point, we suggest to first focus on stabilizing the basic FG (FG 23-7-1). |
| CATT | Support. |
| NTT DOCOMO | We support to merge FG 23-7-3 into FG 23-7-1. |
| CMCC | Supoort. We can put it in FG 23-7-1. |
| Huawei, HiSilicon | It may not be necessary to mix FG 23-7-3 and 23-7-2 since the context of discussion over numbers/FGs can be different. |
| MediaTek | Support |
| Intel | Support |
| Lenovo/MotM | Fine to merge with FG 23-7-1 |
| Nokia/NSB | Ok to include in 23-7-1 |
| vivo | Fine to merge with FG 23-7-1. |

# Issue 12: FG 23-7-5

After review of contributions submitted to RAN1 #107bis-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-7-5 | CMR sharing | Support a NZP CSI-RS resource referred by both a CMR pair configured for NCJT measurement hypothesis and a CMR configured for Single-TRP measurement hypothesis | 23-7-1 |  |  |  | Per band |  | FR2 only |  | Note: ‘NCJT’ and ‘single-TRP’ are not used in RAN1 specifications and will be aligned with 38.214 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | If removing ‘configured for NCJT measurement hypothesis’, and for the sake of presentation, we think that the ‘single-TRP measurement hypothesis’ should also be replaced by ‘CSI determination as a general description. Based on that, we have the following suggestion in purple: Support a NZP CSI-RS resource referred by both a CMR pair configured for NCJT measurement hypothesis and a single CMR configured for CSI determination Single-TRP measurement hypothesis |
| Ericsson | OK. |
| Apple | We prefer to have the description clearer. We are flexible with the wording, e.g., “configured for Rel-17 Multi-TRP CSI enhancement” We also prefer to discuss some other FGs - Support CSI-IM for CSI enhancement for Multi-TRP |
| QC | It seems that “single-TRP” from the description is still there. We suggest to not worry too much about terminology at this point. “NCJT” and “single-TRP” are currently used for concise and easier description. At the end, we can replace these terminologies in a consistent manner if needed. What is more important is the details of UE capability structure and ASN.1 impact. |
| CATT | Support. |
| NTT DOCOMO | We share similar view as ZTE. Removing ‘configured for NCJT measurement hypothesis’ but keeping ‘configured for single-TRP measurement hypothesis’ is confusing. Either removing both or keeping both is fine to us. |
| OPPO | Fin to keep NC-JT in the description. |
| Spreadtrum | Fine to keep the description, there is no ambiguity. |
| CMCC | We are fine to keep NCJT in the description. |
| Huawei, HiSilicon | OK |
| MediaTek | Support |
| Samsung | Support in principle. |
| Intel | Support |
| Lenovo/MotM | Support in principle. The wording “single-TRP measumrement hypothesis” should not be used |
| Nokia/NSB | OK |
| Futurewei | Support in principle. Similar to ZTE’s view, we suggest either removing both “configured for NCJT measurement hypothesis” and “configured for Single-TRP measurement hypothesis” or keeping both to avoid confusion. |
| vivo | We think the original wording is fine. There are many descriptions in the UE feature list use the terms but not used in the spec, such as “multi-DCI based mTRP", etc. |

# Issue 13: FG 23-8-1

After review of contributions submitted to RAN1 #107bis-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-8-1 | SRS triggering offset enhancement | Support of The maximum number of configured available slots offsets for determining aperiodic SRS location based on available slot [and dynamically based on Rel-17 introduced new field in DCI Format 0_1/0_2/1_1/1_2] | 2-52 |  |  |  | Per FS |  |  |  | Candidate component values: {none, 1, 2, 3, 4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Okay with this update in principle. We are wondering what the difference is to report “none” and not to report this FG. If both mean this FG is not supported, “none” should be removed in the candidate list. |
| Ericsson | - Likely “3” is not needed ,1,2 and 4 is sufficient unless there is strong motivation. - On per FS, we disagree unless the reasons is explicitly given as RAN2 guidance as documented in the RAN2 LS (R1-2001513, Guidelines for UE capability definitions, RAN2) states (where 38.306 states "FS" indicates it is signalled per feature set (per band per band combination)): 4 Minimize features “per-BandCombination” and “per-Band-of-a-BandCombination” The number of features that require capability signaling with “per-BandCombination” and “per-Band-of-a-BandCombination” (FeatureSetPerCC) differentiation should be minimized. Such features cause an explosion in the size of the capability signaling and are even more severe when it comes to the number of possible configurations, making it impossible for the gNB to take all combinations into account. It would be of help for RAN2 to be informed of the reasons why “per-BC” or “per-band-of-a-BC” was considered necessary, and whether other alternatives were considered. |
| Apple | We are okay with this FG. - “none” seems to be redundant since not reporting this FG can be used to indicate that UE does not support DCI based available slot indicate for AP-SRS |
| LG | We don’t support current version. There is no relevant agreement regarding UE capability signaling for the max number of configured available slot offsets. But, we already have this agreement below: Agreement - Up to 4 “t” values can be configured per SRS resource set. So, We prefer following version as below: 23. NR_FeMIMO; 23-8-1; SRS triggering offset enhancement; Support of determining aperiodic SRS location based on available slot; 2-52; Optional with capability signalling. |
| Qualcomm | Support. Agree with other companies that ‘none’ is not needed. |
| CATT | Similar view as LG. Besides, the type shall not be per PS unless it is absolutely necessary. |
| DOCOMO | We prefer “per band”. Not sure the reason why it needs to be per FS. We agree none seems redundant. |
| OPPO | Support in principle. However, “none” and “3” seems not necessary. |
| Xiaomi | Support with removing candidate value “none”. |
| Spreadtrum | Generally we are OK. Also fine to remove ‘none’. |
| Huawei, HiSilicon | We are okay with this FG. Agree with other companies that ‘none’ is not needed. |
| MediaTek | Okay. “none” can be removed |
| Intel | Generally fine with moderator’s proposal. Regarding the granularity, we think it’s better to be per band. |
| Lenovo/MotM | We also agree to remove “none”. |
| Futurewei | Support, and fine to remove ‘none’. The wording ‘maximum number of configured available slots offsets’ is a bit odd. When the UE is reporting, there is no available slot offset configured yet. Suggest to change to ‘maximum number of configured available slots offsets that can be configured for the UE’. |
| vivo | Support, “none” can be kept |
| Nokia, NSB | Agree that “none” should be removed and that granularity should be per band. |

# Issue 14: FG 23-8-2

After review of contributions submitted to RAN1 #107bis-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-8-2 | Triggering SRS only in DCI 0_1/0_2 | Support of triggering SRS in DCI 0_1/0_2 without data and without CSI | 2-52 |  |  |  | Per FS |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | We prefer to have it per UE. It is not clear to us why it needs to be defined per FS. |
| Ericsson | - On per FS, we disagree unless the reasons is explicitly given as RAN2 guidance as documented in the RAN2 LS (R1-2001513, Guidelines for UE capability definitions, RAN2) states (where 38.306 states "FS" indicates it is signalled per feature set (per band per band combination)): 4 Minimize features “per-BandCombination” and “per-Band-of-a-BandCombination” The number of features that require capability signaling with “per-BandCombination” and “per-Band-of-a-BandCombination” (FeatureSetPerCC) differentiation should be minimized. Such features cause an explosion in the size of the capability signaling and are even more severe when it comes to the number of possible configurations, making it impossible for the gNB to take all combinations into account. It would be of help for RAN2 to be informed of the reasons why “per-BC” or “per-band-of-a-BC” was considered necessary, and whether other alternatives were considered. |
| Apple | - We prefer to have it “per band” if “per FS” is not agreeable. (1) AP-SRS related FG, i.e., 2-53, is per FS (2) AP-CSI related FG, i.e., 2-35 is per band |
| Qualcomm | - Support |
| CATT | - We prefer to have it “per UE “. |
| NTT DOCOMO | - We prefer “per band”. Not sure the reason why it needs to be per FS. |
| OPPO | - Support |
| Spreadtrum | - We are also fine with ‘per band’. |
| Huawei, HiSilicon | - Agree with ZTE, we prefer to define it as “Per UE”. |
| MediaTek | - Support |
| Intel | - We think it’s better to be per band |
| Lenovo/MotM | - Support |
| Futurewei | - Support. Ok with ‘per UE’. |
| vivo | - Support |
| Nokia, NSB | - Support. Per UE is fine. |

# Issue 15: FG 23-8-3

After review of contributions submitted to RAN1 #107bis-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-8-3 | SRS Antenna switching for >4Rx | 1. Support of SRS antenna switching xTyR with y>4 2. Report whether the antenna switching impact to downlink receiving in a band 3. Report whether the antenna is switched together with UL Tx in another band | 2-55 |  |  |  | Per BC |  |  |  | Component 1 candidate values: FFS one or more elements from set {t1r1, t2r2, t1r2, t4r4, t2r4, t1r4, t2r6, t1r6, t4r8, t2r8, t1r8} Component 2 candidate values: {1 to 32} Component 3 candidate values: {1 to 32} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Okay |
| Ericsson | Don’t understand how the candidate values of 2 and 3 can be an integer between 1 to 32? Better description of the feature items is needed. How can the answer to the question “whether the antenna switching impact to downlink receiving in a band” be e.g. “17” ? Otherwise, support. |
| Apple | - The reporting type should be “per FS”. For Rel-15, we think it is captured incorrectly in 38.306. However, in 38.331, it is captured correctly as per FS, i.e., per band per BC - It should be per FS since depending on the BC, for the same band, the UE antenna/RF configuration can be different - For component 2 and 3 - We should correct the issue we have in Rel-15, i.e., UE should be able to report a list of bands that will have DL/UL impact due to SRS antenna switching. A mistake was made in RAN2, we believe, by allowing UE to report only the first-listed band |
| Qualcomm | The suggested candidate values for component 1 have some overlapping with the reported values using rel-15/16 capability signaling (‘supportedSRS-TxPortSwitch’, ‘supportedSRS-TxPortSwitchv1610’). However, we are fine with the current proposal. |
| CATT | If a UE support SRS antnenna switching for no more than 4Rx, it can report FG 2-55, therefore “t1r1, t2r2, t1r2, t4r4, t2r4, t1r4” should be removed from the candidate values for component 1. Besides, it is our view that it is nature for a UE supports 1T8R to support 1T1R, 1T2R, 1T4R and 1T6R, i.e. degrading SRS configuration can be a basic feature. Therefore the candidate values of component 1 can be: {t1r1-t1r2-t1r4-t1r6, t1r1-t1r2-t1r4-t1r6-t1r8, t1r1-t1r2-t2r2-t2r4-t2r6, t1r1-t1r2-t1r4-t2r2-t2r4-t2r6-t2r8, t1r1-t1r2-t2r2-t2r4-t4r4-t4r8}. On component 2 and 3, we have similar concern as Ericsson. |
| NTT DOCOMO | We believe downgraded configuration should also be included in candidate values, instead of modifying the existing Rel-16 capability which doesn’t consider R more than 4. The following can be added: - 't1r1-t1r2-t1r4-t1r6’ for 1T=1R/1T2R/1T4R/1T6R - 't1r1-t1r2-t2r2-t1r4-t2r4-t1r6-t2r6' for 1T=1R/1T2R/2T=2R/1T4R/2T4R/1T6R/2T6R - 't1r1-t1r2-t1r4-t1r6-t1r8’ for 1T=1R/1T2R/1T4R/1T6R/1T8R - 't1r1-t1r2-t2r2-t1r4-t2r4-t1r6-t2r6-t1r8-t2r8' for 1T=1R/1T2R/2T=2R/1T4R/2T4R/1T6R/2T6R/1T8R/2T8R - 't1r1-t1r2-t2r2-t1r4-t2r4-t1r6-t2r6t1r8-t2r8-t4r8' for 1T=1R/1T2R/2T=2R/1T4R/2T4R/1T6R/2T6R1T8R/2T8R/4T8R |
| Ericsson | We agree with Docomo that downgraded configuration should also be included. Furthermore, a question to UE side vendors which impacts on the UE capability signalling: For this large number of RX chains, is there a need for implementation flexibility to support unbalanced antenna switching. i.e. where the number of RX ports per TX port is not the same for all TX ports??? For example, if the UE reports 2 TX and 6 RX, for an unbalanced implementation as in the figure, a different number of OFDM symbols is needed compared to the balanced case. |
| OPPO | Regarding the report type, we support Apple’s proposal since the related UE capability is reported per band per BC in TS 38.331 (i.e., the RRC signaling structure is BandCombination -> BandParameters -> srs-TxSwitch) Regarding the current version of Component 1, it allowed UE to report the same value as R15/16. Thus, our first preference is to use the combined values (e.g., t1r1-t1r2-t1r4-t1r8) similar to R16 siganling. On the oher hand, we can also accept the current version if majority companies support it. |
| Xiaomi | Okay with the current version of component 1. |
| Huawei, HiSilicon | For component 1, current candidate values allow arbitrary combination of xTyR, which is unnecessary. For instance, how can a UE with 1T8R capability be downgraded to 2T4R or 4T4R? The candidate values should include specific combinations which may be used in commercial network, like what we did in Rel-16, instead of supporting all combinations. |
| MediaTek | - Support component 1 - For component 2 & 3, we are open to discuss changing legacy way of reporting. General comment, supporting downgraded (in other words combo) SRS configuration for antenna switching has not been agreed in RAN1 and hence it should not be enforced through UE feature reporting. |
| Intel | Fine with moderator’s proposal We also request to add some FFS for component 2&3, whether it is impacted when UE reports component 1 as xTyR with x=y. |
| Lenovo/MotM | - Support |
| vivo | support |

# Issue 16: FG 23-8-4

After review of contributions submitted to RAN1 #107bis-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-8-4 | Maximum 2 SP and 1 periodic SRS sets for antenna switching | Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for antenna switching | 2-53 |  |  |  | Per FS |  |  |  | Note1: - Applies for all supported xTyR where y<=8 - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - For xTyR where y<=4, if UE does not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS - The two SP-SRS resource sets are not activated at the same time | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | We are okay. |
| Ericsson | - On per FS, we disagree unless the reasons is explicitly given as RAN2 guidance as documented in the RAN2 LS (R1-2001513, Guidelines for UE capability definitions, RAN2) states (where 38.306 states "FS" indicates it is signalled per feature set (per band per band combination)): 4 Minimize features “per-BandCombination” and “per-Band-of-a-BandCombination” The number of features that require capability signaling with “per-BandCombination” and “per-Band-of-a-BandCombination” (FeatureSetPerCC) differentiation should be minimized. Such features cause an explosion in the size of the capability signaling and are even more severe when it comes to the number of possible configurations, making it impossible for the gNB to take all combinations into account. It would be of help for RAN2 to be informed of the reasons why “per-BC” or “per-band-of-a-BC” was considered necessary, and whether other alternatives were considered. |
| Apple | - We are okay with this FG |
| LG | - Support. |
| Qualcomm | - Support |
| CATT | - The type shall not be per PS unless it is absolutely necessary. |
| NTT DOCOMO | - We are also wondering why it should be per FS. Could someone clarify? To aling with the type of Rel-15 SRS antenna switching capability? |
| OPPO | - Support |
| Xiaomi | - Okay with this. |
| Huawei, HiSilicon | - Support. |
| MediaTek | - Support |
| Intel | We think it’s better to be per band. - In addition, we have proposal to add a new FG on the extended number of aperiodic SRS resource sets for some legacy xTyR configuration (1T4R, 2T4R and 1T2R). We think a new FG should be added to reflect the corresponding RAN1 agreement. |
| Lenovo/MotM | - Support |
| vivo | support |

# Issue 17: FG 23-8-5

After review of contributions submitted to RAN1 #107bis-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-8-5 | Increased repetition for SRS | Support of increased repetition patterns (8, 10, 12, 14 symbols) for SRS resource | 10-11, 2-52 |  |  |  | Per FS |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | For the type column, we think to make it “per band” should be sufficient. |
| Ericsson | - On per FS, we disagree unless the reasons is explicitly given as RAN2 guidance as documented in the RAN2 LS (R1-2001513, Guidelines for UE capability definitions, RAN2) states (where 38.306 states "FS" indicates it is signalled per feature set (per band per band combination)): 4 Minimize features “per-BandCombination” and “per-Band-of-a-BandCombination” The number of features that require capability signaling with “per-BandCombination” and “per-Band-of-a-BandCombination” (FeatureSetPerCC) differentiation should be minimized. Such features cause an explosion in the size of the capability signaling and are even more severe when it comes to the number of possible configurations, making it impossible for the gNB to take all combinations into account. It would be of help for RAN2 to be informed of the reasons why “per-BC” or “per-band-of-a-BC” was considered necessary, and whether other alternatives were considered. |
| Apple | - We are okay with “per band” |
| LG | - Support. |
| Qualcomm | - Support |
| CATT | - We are fine with “per band” for the type of this feature. |
| NTT DOCOMO | - We are also wondering why it should be per FS. Could someone clarify? We prefer per band. |
| OPPO | - Support |
| Huawei, HiSilicon | - Agree with ZTE and Apple, we prefer to define it as “Per band”. |
| MediaTek | - We are also okay with “per band” |
| Intel | - We think it’s better to be per band |
| Lenovo/MotM | - Support |
| Futurewei | - Support. Ok with ‘per band’. |
| vivo | - Support |
| Nokia, NSB | - Per band indication |

# Issue 18: FG 23-8-6

After review of contributions submitted to RAN1 #107bis-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-8-6 | Partial frequency sounding of SRS | Support of partial frequency sounding for SRS | 2-52 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | We are okay. |
| Ericsson | Support. |
| Apple | We are okay with this FG |
| Qualcomm | Support per-band reporting |
| CATT | Support the proposal. |
| NTT DOCOMO | We are ok with this FG |
| OPPO | Support |
| Xiaomi | Okay to support. |
| Huawei, HiSilicon | Support. |
| MediaTek | Support |
| Intel | Fine with moderator’s proposal |
| Lenovo/MotM | - Support |
| Futurewei | Support |
| vivo | Support |
| Nokia, NSB | OK |

# Issue 19: FG 23-8-7

After review of contributions submitted to RAN1 #107bis-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-8-7 | Start RB location hopping for partial frequency SRS | Support of start RB location hopping in partial frequency SRS transmission [across different SRS frequency hopping periods for periodic/semi-persistent/aperiodic SRS] | 23-8-6 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | We are okay. |
| Ericsson | Support. |
| Apple | We are okay with this FG |
| LG | OK |
| Qualcomm | Support the updated description and per-band reporting. |
| CATT | Support the proposal. |
| NTT DOCOMO | We support this FG |
| OPPO | Support |
| Xiaomi | Okay to support. |
| Huawei, HiSilicon | Support. |
| MediaTek | Support |
| Intel | Ok with moderator’s proposal |
| Lenovo/MotM | - Support |
| Futurewei | Ok |
| vivo | Support |
| Nokia, NSB | OK |

# Issue 20: FG 23-8-8

After review of contributions submitted to RAN1 #107bis-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-8-8 | Comb-8 SRS | Support of comb-8 for SRS other than for positioning |  |  |  |  | Per FS |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | We prefer to define this FG as per band similarly as 23-8-6. |
| Ericsson | - On per FS, we disagree unless the reasons is explicitly given as RAN2 guidance as documented in the RAN2 LS (R1-2001513, Guidelines for UE capability definitions, RAN2) states (where 38.306 states "FS" indicates it is signalled per feature set (per band per band combination)): 4 Minimize features “per-BandCombination” and “per-Band-of-a-BandCombination” The number of features that require capability signaling with “per-BandCombination” and “per-Band-of-a-BandCombination” (FeatureSetPerCC) differentiation should be minimized. Such features cause an explosion in the size of the capability signaling and are even more severe when it comes to the number of possible configurations, making it impossible for the gNB to take all combinations into account. It would be of help for RAN2 to be informed of the reasons why “per-BC” or “per-band-of-a-BC” was considered necessary, and whether other alternatives were considered. |
| Apple | - We are okay with “per band” |
| Qualcomm | - Support |
| NEC | - Prefer to be “per band” reporting |
| CATT | - We are fine with “per band” for the type of this feature. |
| NTT DOCOMO | - We are also wondering why it should be per FS. Could someone clarify? We prefer per band. |
| Huawei, HiSilicon | - Agree with ZTE and Apple, we prefer to define it as “Per band”. |
| MediaTek | - Agree, 23-8-8 should be similar as 23-8-6. Okay with “per band” |
| Lenovo/MotM | - Support |
| Intel | - We think it’s better to be per band. |
| Futurewei | - Support, and ok with ‘per band’. |
| vivo | - Support |
| Nokia, NSB | - Per band indication |

# Issue 21: FG 23-9-2

After review of contributions submitted to RAN1 #107bis-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-9-2 | Support of M=2 and R=1 for FeType-II | [1. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with M=2 and R=1] 2. Support parameter combinations with M=2 [3. Support of DFT FD bases of size N] | 23-9-1 |  |  |  | [per band and per BC] |  |  |  | [Component 1 candidate values - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}] [Component 34 candidate values: {N=2 only, N=2 and N=4}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | We prefer to remove the third component or make it FFS. The need to keep it is not clear to us. |
| Apple | The triplet reporting, i.e., component 1, needs to be “per band and per BC” |
| Qualcomm | If component 1 is introduced, the granularity for component 1 should be per band and per BC. |
| CATT | We prefer to remoe the first component, since the number of ports has been restricted for M=2 according to discussion on codebook parameter combination. It is not necessary to further restrict the number of ports through UE capabiltity. |
| NTT DOCOMO | Okay. |
| OPPO | Agree with other companies that component 1 should be per ban per BC as in Rel-16. |
| Spreadtrum | OK |
| Huawei, HiSilicon | The granularity of 23-9-2 shall be also per BC as well, similar with FG 23-9-1, due to triplet-based reporting in Component 1. The component of size N is not needed, in our understanding, since the window size of N has marginal impact of UE computational complexity due to small size of N (up to 4) and the selection of FD basis is layer common. From UE perspective, if a UE can support FG 23-9-2, it is more desirable to support both values of N (i.e. N=2 and 4) to simplify gNB implementation. |
| MediaTek | Agree with Apple, component 1 needs to have a reporting Type “per band and per BC” |
| Samsung | Support Component 1 and 2 only. |
| Intel | We don’t see the need to indicate triplets for different values of M and R considering tradeoff between UE implementation flexibility and UE capability overhead. We suggest to delete component 1 (in this case it is reused from the basic FG 23-9-1). As a compromise we can accept the component 1 for FG 23-9-2, but considering marginal difference in UE complexity for PMI search with R = 1 and R = 2, component 2 for FG 23-9-4 shall be deleted (as part of the compromise). Also, R = 1 shall be deleted if the above proposal for FG 23-2-4 is implemented. Support of M=2 and R=1 for FeType-II; [1. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with M=2 and R=1] 2. Support parameter combinations with M=2 [3. Support of DFT FD bases of size N]. |
| Lenovo/MotM | Agree with ZTE, prefer to remove the third component |
| Nokia/NSB | FG should be per band and per BC, as usual practice for this type of FG. Component 3 is not needed. Similar view as Huawei, if a UE supports this feature both values of N should be supported |
| Qualcomm2 | If signaling complexity/overhead for CSI-RS triplets in both FG23-9-2 and FG23-9-4 is an issue, we prefer to keep triplets for FG23-9-4 and a simple signaling (“support” or “not support”) in FG23-9-2, or we discuss FG23-9-2/4 jointly in the next round. This is similar to Rel-16 eType II codebook, where a simple signaling is for parameter combination of L=6 though PMI searching complexity increase, while CSI-RS triplets are signaled for R=2, because R=2 doubles the PMI computation obviously. Since Rel-17 FeType II has same structure of Rel-16 eType II, we don’t see the need to change this signaling. Otherwise, UE may not be able to reuse the implementation principle of Rel-16 eType II considering that Rel-16 eType II is about to commercialize. |
| vivo | We share similar view as Huawei that component 3 for size N is not needed. |
| Intel2 | In our view the complexity difference for R = 1 and R = 2 is not the same as for Rel-16 eType II PMI codebooks. For Rel-16 eType II PMI codebooks the number of SVD operations is equal to the number of PMI subbands which is twice higher for R = 2 comparing to R = 1. Thus, doubling of complexity is observed for Rel-16. In Rel-17 the number of SVD operations is the same for R = 1 and R = 2, the only difference is the size of FD vectors (W3). Thus, the complexity difference for R = 1 and R = 2 for Rel-17 feType II is small. Also, in our understanding the indication of triplets requires significant number of bits which is important for UE capability signalling since the size of UE capability report is limited in practice. So, in our view it is really important to avoid indication of triplets if it is not necessary. |

# Issue 22: FG 23-9-3

After review of contributions submitted to RAN1 #107bis-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-9-3 | Support of rank 3, 4 for FeType-II | Support of rank 3, 4 for FeType-II FFS separate capabilities for M = 1 and M = 2 | 23-9-1[, 23-9-2] |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Okay |
| Apple | We are okay with this FG |
| Qualcomm | ok |
| CATT | We prefer that the prerequisite feature group of this feature includes FG 23-9-1 and FG 23-9-2. |
| NTT DOCOMO | Okay. |
| OPPO | Fine |
| Huawei, HiSilicon | OK |
| MediaTek | Support |
| Intel | Support the proposal. |
| Lenovo/MotM | Support |
| Nokia/NSB | Ok |
| vivo | Fine |

# Issue 23: FG 23-9-4

After review of contributions submitted to RAN1 #107bis-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-9-4 | Support of R = 2 for FeType-II | 1. Support of R = 2 for FeType-II [2. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support to support Port-selection FeType-II with M=2 and R=2] 3. Support parameter combinations with Mv=2 and R=2 | 23-9-2 |  |  |  | [per band and per BC] |  |  |  | Component 2 candidate values: • Maximum 16 triplets • Max # of Tx ports in one resource: {4,8,12,16,24,32} • Max # resources: {1 to 64} • Max # total ports: {4 to 256} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | We think the first two components should be sufficient to handle the complexity caused by R=2. The reason to have the third component is not clear. Hence we prefer to remove it or FFS it. |
| Ericsson | Not happy with the granularity of #resources and “total ports”. DO we really need this granularity of supported “resources”? ??i.e. 1,2,3,4,5,6,7,8,9,10,11,12,13,14,15,16,….,59,60,61,62,63,64 ?? |
| Apple | - The reporting type needs to be “per band and per BC” - We are fine with FFS component 3 |
| Qualcomm | The granularity for component 2 should be per band and per BC. This is to be consistent with all other codebooks, where the granularity of the CSI-RS triplets are per band and per BC. The per BC capability is needed to resolve the underreporting issue when working under CA. There are 3 modes for FeType II PS codebook, i.e., {M=1}, {M=2, R=1} and {M=2, R=2}, with complexity in increasing order. In our view, the complexity difference between {M=2, R=1} and {M=2, R=2} is larger than the complexity difference between {M=1} and {M=2, R=1}. This is because the number of PMIs doubles when R=2. So, having CSI-RS triplets reported for M=2 R=2 is more important to make this feature fly. Also, this is to be consistent with Rel-16 eType II codebook, where parameter-combo 7-8 (FG 16-3a-2) share same triplets with basic feature while R=2 requires an additional CSI-RS triplets (FG 16-3a-1). |
| CATT | We think the first component is sufficient. Both component 2 and component 3 can be removed. The reason is similar with FG 23-9-2, i.e., it does not need to further restrict the number of ports through UE capabiltity |
| NTT DOCOMO | Share similar view with ZTE. Prefer to remove the third component. |
| OPPO | Component 2 should be per band and per BC as in Rel-16. |
| Huawei, HiSilicon | We prefer to keep it simple with support/not support of R=2, especially that 23-9-2 needs to report triples per band and per BC. We have too many triplets with large reporting overheads. |
| MediaTek | As mentioned by multiple companies we suggest compoenet 2 to be reported “per band per BC” Support component 3, we are also ok with Huawei, HiSilicon proposal of simple with support/not support for candidate values of this component. |
| Intel | As we commented for FG 23-9-2, we propose to delete component 2 for FG 23-9-4. The difference in complexity for PMI search with R = 1 and R = 2 is really small, especially with M = 1. PMI search complexity is increased only due to the increased dimension of frequency-domain vectors, which leads to marginal impact on total PMI search complexity. Complexity of CQI calculation (precoder reconstruction from the PMI) can be the same depending on UE implementation since CQI is not sensitive to different R values due to coarse CQI granularity. Thus, we propose the following changes. 1. Support of R = 2 for FeType-II for parameter combinations with M = 1 2. Support of R = 2 for FeType-II for parameter combinations with M = 2 [2. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support to support Port-selection FeType-II with M=2 and R=2] 3. Support parameter combinations with Mv=2 and R=2 |
| Lenovo/MotM | Prefer keeping component 3. Also agree with Ericsson’s comments regarding the granularity of resources and ports |
| Nokia/NSB | We also prefer to have a single component (merge of 1 and 3) and reuse the triplets in 23-9-2. |
| vivo | We think the component 3 is not needed. If the group think component 2 and component 3 for {M=2, R=2} can share the same values as {M=2, R=1}, we can merge this FG into FG 23-9-2 by giving two candidate values of R={1} and R={1 or 2}. |

# Issue 24: FG 23-9-5

After review of contributions submitted to RAN1 #107bis-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-9-5 | Active CSI-RS resources and ports for mixed codebook types in any slot | - List of codebook combinations - List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination | [23-9-1], 16-3a, 16-3b, 16-3a-1, 16-3b-1, 2-36, 2-40, 2-41, 2-43 |  |  |  | FFS Per band |  |  |  | [Component 1 candidate values: Codebook 1 = {Type I SP, Type I MP} {Codebook 2, Codebook 3} = {{FeType II PS M=1, NULL}, {FeType II PS M=2 R=1, NULL}, {FeType II PS M=2 R=2, NULL}, {Type II, FeType II PS M=1}, {Type II, FeType II PS M=2 R=1} ,{eType II R=1, FeType II PS M=1},{eType II R=1, FeType II PS M=2 R=1}} Component 2 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}] Note 1：if a UE reports one or more codebook combinations in [23-9-5], then usage of active CSI-RS resources and ports for multiple codebooks in any slot is allowed only within those combinations [Note 2: For coexisting of mixed codebooks in any slot, gNB need to honor 16-8, 23-10 and per-codebook capability 2-36/40/41/43, 16-3a/b and 16-3a-1/16-3b-1, and 23-9-1] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Okay |
| Apple | - The reporting type needs to be “per band and per BC” - Note 2 needs some refinement - [Note 2: For coexisting of mixed codebooks in any slot, gNB need to honor 16-8, 23-10 23-9-5 and per-codebook capability 2-36/40/41/43, 16-3a/b and 16-3a-1/16-3b-1, and 23-9-1/23-9-2/23-9-4] |
| Qualcomm | The granularity of both component 1 and 2 should be per band and per BC (same comment to FG 23-9-2 and FG23-9-4) Agree with Apple on the refinement of Note 2. |
| CATT | We are fine. |
| NTT DOCOMO | Okay. |
| OPPO | Support that it shold be per ban per BC. |
| Spreadtrum | Agree with Apple |
| Huawei, HiSilicon | This FG is per band and per BC. It might be more efficient to clarify the design of FG 23-9-2/23-9-4 and then discuss the mixed codebook types, since they are highly related and depending on details of FG 23-9-2/23-9-4. We slightly prefer to support eType R=2 as a part of mixed codebooks for all legacy designs. |
| MediaTek | Agree with Apple |
| Intel | The list of {Codebook 2, Codebook 3} may be refined depending on outcome of discussion for FG 23-9-2 Component 1 and FG 23-9-4 Component 2. |
| Nokia/NSB | FG should be per band and per BC. We are ok to refine the list of {Codebook 2, Codebook 3} after clarifying the design in FG 23-9-2/23-9-4 |

# Discussion/Approval Items during RAN1 #107bis-e — Second Checkpoint

Based on the comments/questions/suggestions received by the first checkpoint, the following are the revised proposals and/or proposed agreements by the moderator. Companies submitted the following views on the moderator’s proposals.

General comments

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Samsung | For FG 23-1-1, 23-1-2, 23-1-3, and perhaps (depending on the outcome of the “maintenance” stage 23-1-4), rather using the current grouping based on joint/separate TCI and intra/inter-cell, after the description of each bullet item is stable, it is proposed that the following re-grouping be used: FG 23-1-0: Basic features (supported for UEs that support Rel-17 TCI) FG 23-1-1: Auxiliary feature group 1 FG 23-1-2: Auxiliary feature group 2 ... FG 23-1-n: Auxiliary feature group n It is noted that all Rel-17 UE features are optional in the absolute sense. - FG 23-1-0 includes Rel-17 UE features that are basic for UEs supporting Rel-17 TCI. - {FG 23-1-m, m>0} represent feature groups that can be supported in addition to FG 23-1-0 or conditioned on the support of other MIMO/non-MIMO features (one example would be, as Nokia pointed out, Carrier Aggregation). Each of {FG 23-1-m, m>0} would be a separate FG. For FG 23-1-0, our view is that it should AT LEAST include the following: - 4 activated TCI states and DCI-based beam indication (with and without DL assignment), for BOTH intra-cell and inter-cell beam management - Inter-cell beam measurement/reporting TBD: Basic supported value(s) of K and Nmax |

# Issue 1: FG 23-1-3

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-1-3 | MPE mitigation | 1. Support of enhanced [PHR] reporting which includes pairs of (P-MPR, SSBRI/CRI) 2. Maximum number of reported P-MPR and SSBRI/CRI pairs [per CC/ across all CCs] 3. Maximum number of candidate RS(s) configured in a RRC pool CSI-RS/SSB resource set for MPE mitigation [per CC/ across all CCs] |  |  |  |  | Per band [and per BC] |  | [FR2 only] |  | Component 2 candidate values: of {1,2,3, 4} Component 3 candidate values: FFS {[4,8,]16,32,64} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Samsung | Overall this looks fine |
| ZTE | Support |
| Apple | We are okay with this FG |
| Qualcomm | For component 2, our understanding is that the corresponding agreement for the report is per CC For component 3, our understanding is the the corresponding agreement for the resource set is per CC For the whole FG, prefer “per band”, and fine for “FR2 only” as well as new candiate value of 4 and 8 in component 3 |
| LG | Fine with the proposal |
| NTT DOCOMO | Support. And for component2 and 3, we are fine with per CC. |
| MediaTek | We support the new candiate values, “Per band” reporting mode and “FR2 only” Regarding Component 3, given that it counts the number of CSI-RS/SSB resources in a resource set, it’s very clear that it’s in one CC so no need to add [per CC/ across all CCs] so we propose removing the newly added [per CC/ across all CCs]: - Maximum number of candidate RS(s) configured in a RRC pool CSI-RS/SSB resource set for MPE mitigation Regarding Component 2, we also think this reflects Maximum number of reported P-MPR and SSBRI/CRI pairs reported in one MAC-CE massage and no limitation should be there regarding Per or across CC, so we propose removing the newly added [per CC/ across all CCs] and update as: - Maximum number of reported P-MPR and SSBRI/CRI pairs in a single report |
| Xiaomi | Fine with the proposal |
| Ericsson | Component 2: Looking closer at the details of the PHR report, component 2 is unnecessary, since it is a MAC CE report. For L1 reports, this type of FG is critical, since the NW explicitly configures the L1 report, and the NW can at most configure as many entries as the UE supports. However, for a MAC CE report, the UE simply includes as many entries as it likes. The length of the MAC CE report is variable by design, so the NW directly sees how many entries are reported. Note that the multiple entry PHR may include several entries per cell. Component 3: I think it’s unfortunate to use the term “CSI-RS/SSB resource set”, since this points to that the candidate RSs are configured in a CSI-RS/SSB resource, which is quite unlikely. The description of the similar FG for BFR looks like this: maxNumberCSI-RS-SSB-CBD Defines maximal number of different CSI-RS [and/or SSB] resources across all CCs, and across MCG and SCG in case of NR-DC, for new beam identifications…. This is sufficient: introduction of additional text is only confusing. Suggest removing the reference to resource set: 3. Maximum number of candidate RS(s) configured in a RRC pool CSI-RS/SSB resource set for MPE mitigation [per CC/ across all CCs] |
| CATT | For component 3, the bracket and the text within the bracket shall be removed, as it is the maximum number of RS in a set which is configured per BWP. |
| Lenovo/MotM | Support |

# Issue 2: FG 23-1-4

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-1-4 | MPUE support for UL | [depending on agreement] 1. Supported UE capability value sets and corresponding max number of SRS ports for each UE capability value set |  |  |  |  | FSPC per band |  |  |  | Candidate values: Up to [4] value sets each with one value of {[0,]1,2,3,4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Samsung | Since RAN1 can only agree on one UE capability for this feature (i.e. max # SRS ports, and concluded no consensus on the support of other types), not cleat why “value set” is needed. It should be eplaced with “value” in the above description. |
| ZTE | We support the version from the moderator, and from our perspective, we can not support any revision of ‘UE capability value set’ that we have agreed. Regarding Samsung’s comment, we think that, based on current RAN1 agreement, we have already mentioned ‘UE capability value set’ which has been captured in 38.214 as well. Therefore, we prefer to use the same terminology of ‘’UE capability value set’ as the moderator recommended, regardless of whether it contains a single value or more. |
| Apple | In general, we are fine But the candidate value seems to need some clarification since there are two things - Maximum number of capability values sets, do we fix it or UE can report - The maximum number of SRS ports, does UE report it per candidate value set or in general |
| Qualcomm | Fine with the component definition and support 0 as a candidate value to indicate the reported panel is DL only, so gNB can consider other DL RS for UL beam indication. |
| LG | Support the current proposal. Regarding the candidate value, we support to include ‘0’ for max number of SRS ports per each value set to indicate that the UE has DL only panel(s) |
| NTT DOCOMO | Support |
| MediaTek | Support the proposal. Whether to support “0” for max number of SRS ports per each value set to indicate that the UE has DL only panel(s) need to be discussed in AI 8.1.1. |
| Xiaomi | Fine with the proposal and support 0 as a candidate value of the max number of SRS ports. While for the max number of capability value sets, we slightly prefer a value less than 4. |
| Ericsson | We agree with Samsung. Discussing “capability value sets” is quite confusing. We could wait until we have confirmed the updated working assumption. An accurate description of the UE feature would be List of supported maximum number of SRS ports. Having 0 as a candidate value for the maximum number of SRS ports is OK. According to the WA, the following note should also be added: Note: the reported list contains only unique values. To Apple: it must be possible for the UE to report different number of values. “up to [4]” seems to cover that. |
| Intel | We still have concern on current forumation of candidate value set since it allows indicating the same candidate value in different value sets. We suggest to add the following note (in green) according to WA, so that RAN2 know the constraint for signalling design. Candidate values: Up to [4] value sets each with one value of {[0,]1,2,3,4} Note: For any two different value sets, the maximum number of SRS ports needs to be different ------- Working Assumption Support the UE reporting a list of UE capability value sets - Each UE capability value set comprises the max supported number of SRS ports - For any two different value sets, at least one capability value needs to be different - FFS: If in addition also identical value sets are allowed. |
| CATT | Okay with the FG. |
| Lenovo/MotM | We support moderator’s vision to align with the RAN1 agreement. |

# Issue 3: FG 23-2-2

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-2-2 | Two QCL TypeD for [CORESET monitoring in] PDCCH [repetition] | Support of determining two QCL-TypeD for time-domain overlapping in time CORESETs in the same CC or for intra-band CA [when UE is configured with PDCCH repetition] with SFN scheme or non-SFN TDM and/or FDM sheme | [23-2-1, 23-6-1, 23-6-2] |  |  |  | Per band |  |  | FR2 only |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Samsung | If the intention of FG 23-2-2 is for identifying two QCL-TypeD for PDCCH repetition, then “with SFN scheme” should be removed. If the intention of FG 23-2-2 is for identifying two QCL-TypeD for both PDCCH repetition and SFN PDCCH, then “repetition” in the name of FG should be removed. |
| ZTE | ‘SFN scheme or’ should be removed as SFN scheme is for HST. When PDCCH repeititon, only non-SFN scheme is assumed |
| Huawei, HiSilicon | It should be decided whether there’s one UE feature “two QCL TypeD” for both SFN and repetition. If not, then “sfn scheme” should be removed from this FG, and a new FG 23-6-x for two QCL TypeD should be added. |
| Apple | Please put the following in bracket [with SFN scheme] SFN PDCCH enhancement is a separate UE FG family, i.e., FG23-6x, and it is also conflicting with the pre-requisite |
| Qualcomm | The part “with SFN scheme or non-SFN TDM and/or FDM sheme” should be deleted. Two QCL TypeD determination procedure is different for PDCCH repetition versus SFN PDCCH. This is the case in terms of RRC configuration, the actual procedure, and the description in 38.213. In this FG, we are only talking about PDCCH repetition (and because of that prerequisite is 23-2-1) |
| LG | Remove “with SFN scheme”. Separate FG for two QCL type D CORESET monitoring for SFN scheme should be defined since repetition scheme and SFN scheme are separate feature. |
| NTT DOCOMO | Share similar view with other companies that “with SFN scheme” should be removed. This is FG is only related to non-SFN repetition. |
| MediaTek | We propose to remove “with SFN scheme or non-SFN TDM and/or FDM sheme” |
| Xiaomi | Based on the comments from other companies above, we are fine to consider only non-SFN scheme here. Thus “with SFN scheme or non-SFN TDM and/or FDM sheme” can be revised to “with non-SFN scheme”. |
| Intel | It doesn’t make sense to have UE capability for SFN PDCCH which is another FG famility. Suggest to delete “with SFN scheme or non-SFN TDM and/or FDM” |
| CATT | Remove ‘with SFN scheme’, since two QCL-TypeD determination for SFN scheme is a separate FG. |
| Lenovo/MotM | We prefer to remove “SFN scheme or” since different UE capability is required for supporting PDCCH with repetition and SFN based enhanced PDCCH and we focus to discuss the non-SFN scheme here. We are fine to introduce a similar FG in HST part. |
| CMCC | We propose to remove “SFN scheme” since we are only discussing PDCCH repetition here and we can have another similar FG for HST-SFN scheme, if needed. |
| OPPO | Suggest to remove ‘with SFN scheme’ |

# Issue 4: FG 23-2-3

No proposal

# Issue 5: FG 23-6-1

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-6-1 | SFN scheme A (scheme 1) | 1. [Support of SFN scheme A for PDCCH] scheduling [single TRP/] Scheme A PDSCH and [and default QCL assumption with one or two TCI states for PDCCH] 2. Support of SFN scheme A for PDSCH only [scheduled by [single TRP/ Scheme A] PDCCH] [and default QCL assumption with one or two TCI states for PDSCH] |  |  |  |  | [Per band or per FS or FSPC] |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Samsung | We prefer to delete [and default QCL assumption with one or two TCI states for PDCCH] in Component 1 and [and default QCL assumption with one or two TCI states for PDSCH] in Component 2 which are unclear as many companies mentioned. Adding “SFN” in front of “Scheme A PDSCH” in Component 1 is needed. |
| Huawei, HiSilicon | On the reporting granularity, we prefer per FSPC considering the implementation complexity and possibility of earlier implementation. In addition, we would like to have two new FGs: maximum total number of activated TCI states for TCI tracking capability, and maximum number of PDSCH per slot. They can also be part of the FG 23-6-1 as additional components: - Component 3. The maximal total number of activated TCI states per BWP per CC including data and control, with candidate values {1, 2, 4, 8 ,16}; - Component 4. The maximal total number of activated TCI states all BWPs all CCs including data and control, with candidate values {1, 2, 4, 8 ,16, 32}; - Component 5. The maximal number of PDSCHs per slot, with candidate values {1, 2, 3, 4 ,7}; |
| ZTE | We can be fine to remove [and default QCL assumption with one or two TCI states for PDCCH] in both component 1 and component 2. Besides, we are confused to the wording “only” in component 2, which should be removed. Because it has agreed in RAN1 that SFN scheme A PDSCH can be scheduled by either single TRP PDCCH or scheme A PDCCH. We suggest to revise these two components as follow: 1. [Support of SFN scheme A for PDCCH] scheduling [single TRP/] Scheme A PDSCH and [and default QCL assumption with one or two TCI states for PDCCH] 2. Support of SFN scheme A for PDSCH only [scheduled by [single TRP/ Scheme A] PDCCH] [and default QCL assumption with one or two TCI states for PDSCH] |
| Apple | - Since both components are support or not. In the end, we need to split this into two FGs - For component 2, minor editorial comment, “only” should be in bracket too [only [ Regarding the marked content, we may not have time to resolve in this meeting, our view - [single TRP/] should be removed since this was agreed to be optional and we need a separate FG for this - Default beam related description in both components should be removed since it is not even clear - Reporting type, we prefer to be at least per FS |
| Ericsson | We agree with Intel and other companies in the first checkpoint that we shall clarify the FG definition on a higher level. In addition, we can split the 2 components into 2 separate FGs, and the report type shall be “Per band”. 23-6-1 SFN scheme A for PDSCH 23-6-1b SFN scheme A for PDSCH and PDCCH |
| Qualcomm | - Please remove the wording [and default QCL assumption with one or two TCI states for PDCCH] from both components as not clear and there is a separate FG for default QCL assumption. - To make the description simple and clear, suggest the following wording of the two components - Support of SFN scheme A for both PDCCH and PDSCH. - Support of SFN scheme A for PDSCH only. - There is no need to include [single TRP/] in the description of both components as the support of SFN PDCCH + single TRP PDSCH is a separate UE featue (per RAN1 agreements) and should be captured in a separate FG. |
| LG | We think that description for defalt QCL assumption in component ½ should be deleted. |
| NTT DOCOMO | As many companies commented in the 1st round, we suggest clarifying the current 2 components to 3 components. This is because we assume component 1 would be the main use-case, on the other hand, component 3 would be not used. 1. Support of scheme A for PDCCH scheduling scheme A PDSCH 2. Support of single-TRP for PDCCH scheduling scheme A PDSCH 3. Support of scheme A for PDCCH scheduling single-TRP PDSCH From our perspective, we are fine to keep the default QCL. For the reporting granularity, our first preference is “per band”, and we can consider “FS”, if it is necessary. However, we don’t believe “FSPC” is needed, and it has larger signaling overhead of UE capability. |
| Mediatek | We propose to have [and default QCL assumption with one or two TCI states for PDCCH] from the descrpriton in both components. We also agree with Qualcomms proposal for the the simple description for these two components: - Support of SFN scheme A for both PDCCH and PDSCH. - Support of SFN scheme A for PDSCH only. |
| Xiaomi | We prefer to remove the decription for default QCL asssumpotion in both components. In addition, we prefer to list all the combination of SFN scheme A, and either component 3 is discussed as a new FG or not is OK for us. 1.Support of SFN scheme A for PDCCH scheduling SFN scheme A PDSCH 2.Support of single-TRP for PDCCH scheduling SFN scheme A PDSCH 3.Support of SFN scheme A for PDCCH scheduling single TRP PDSCH |
| Intel | As commented in the first round of discussion, simpler formulation is more prefferrable at this point. There is no needed to have spec text describing possible scheduling options. Suggest to have the following structure: - SFN scheme A for PDSCH - SFN scheme A for PDSCH and PDCCH |
| CATT | We support to remove the [and default QCL assumption with one or two TCI states for PDCCH/PDSCH].And for the transmission scheme combination, we agree with Xiaomi and DOCOMO. |
| Lenovo/MotM | We have the similar view for deleting default QCL assumption part and are fine with ZTE’s version. For reporting granularity, we prefer per band reporting. |
| OPPO | - Suggest to remove the part of default QCL assumption - Suggest separate FG for scheme A PDCCH scheduling S-TRP PDSCH. - Suggset separate FG for current component 1 and component 2. |

# Issue 6: FG 23-6-1a

Proposed Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-6-1a | Dynamic switching – scheme A | Support of dynamic switching between single-TRP and SFN PDSCH scheme A by TCI state field in DCI formats 1_1, 1_2 | 23-6-1 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

[Please only comment in the table if you are NOT okay with the proposed agreement]

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | The reporting granularity should be the same as FG 23-6-1. At this stage, it can also be [Per band or per FS or FSPC]. |
| ZTE | Okay. |
| Apple | We are fine with this FG |
| Qualcomm | The reporting type should be similar to FG 23-6-1 [Per band or per FS or FSPC] |
| NTT DOCOMO | We are ok. |
| Mediatek | Agree with Huawei, HiSilicon comment. Reporting should be [Per band or per FS or FSPC]. |
| CATT | Support. |

# Issue 7: FG 23-6-2

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-6-2 | SFN scheme B (TRP based pre-compensation) | 1. [Support of SFN scheme B for PDCCH] scheduling Scheme B PDSCH [and default QCL assumption with one or two TCI states for PDCCH] 2. Support of SFN scheme B for PDSCH only [and default QCL assumption with two TCI states for PDSCH] [scheduled by [single TRP/Scheme B] PDCCH] | [23-6-1] |  |  |  | [Per band or per FS or per FSPC] |  | [FR1 only] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Samsung | We prefer to delete [and default QCL assumption with one or two TCI states for PDCCH] in Component 1 and [and default QCL assumption with one or two TCI states for PDSCH] in Component 2 which are unclear as many companies mentioned. Adding “SFN” in front of “Scheme B PDSCH” in Component 1 is needed. |
| Huawei, HiSilicon | The prerequisite of [23-6-1] should be removed. On the reporting granularity, we prefer per FSPC considering the implementation complexity and possibility of earlier implementation. In addition, we would like to have two new FGs: maximum total number of activated TCI states for TCI tracking capability, and maximum number of PDSCH per slot. They can also be part of the FG 23-6-1 as additional components: - Component 3. The maximal total number of activated TCI states per BWP per CC including data and control, with candidate values {1, 2, 4, 8 ,16}; - Component 4. The maximal total number of activated TCI states all BWPs all CCs including data and control, with candidate values {1, 2, 4, 8 ,16, 32}; - Component 5. The maximal number of PDSCHs per slot, with candidate values {1, 2, 3, 4 ,7}; |
| ZTE | Similar to our comments in FG23-6-1, we suggest to revise these two component as follow: 1. [Support of SFN scheme B for PDCCH] scheduling Scheme B PDSCH [and default QCL assumption with one or two TCI states for PDCCH] 2. Support of SFN scheme B for PDSCH only [and default QCL assumption with two TCI states for PDSCH] [scheduled by [single TRP/Scheme B] PDCCH] |
| Apple | - Since both components are support or not. In the end, we need to split this into two FGs - For component 2, minor editorial comment, “only” should be in bracket too [only [ Regarding the marked content, we may not have time to resolve in this meeting, our view - Default beam related description in both component in bracket should be removed since it is not even clear - Reporting type, we prefer to be at least per FS - Pre-requisite should be removed |
| Ericsson | We agree with Intel and other companies in the first checkpoint that we shall clarify the FG definition on a higher level. In addition, we can split the 2 components into 2 separate FGs, and the report type shall be “Per band”. 23-6-2 SFN scheme B for PDSCH 23-6-2b SFN scheme B for PDSCH and PDCCH |
| Qualcomm | - Please remove the wording [and default QCL assumption with one or two TCI states for PDCCH] from both components as not clear and there is a separate FG for default QCL assumption. - To make the description simple and clear, suggest the following wording of the two components - Support of SFN scheme B for both PDCCH and PDSCH. - Support of SFN scheme A for PDSCH only. - As commented earlier, the pre-requistie 23-6-1 should be removed. |
| LG | We think that description for defalt QCL assumption in component ½ should be deleted. |
| NTT DOCOMO | We agree with prerequisite of “23-6-1”. UE amsung of FG23-6-2 is mostly the same as FG23-6-1. We suggest categorizing the 2 components as below. 1. Support of scheme B for PDCCH scheduling scheme B PDSCH 2. Support of single-TRP for PDCCH scheduling scheme B PDSCH Hence, we suggest to update the component 2 as “[scheduled by [single TRP/Scheme B] PDCCH]”. From our perspective, we are fine to keep the default QCL. For the reporting granularity, our first preference is “per band”, and we can consider “FS”, if it is necessary. However, we don’t believe “FSPC” is needed, and it has larger signaling overhead of UE capability. |
| Mediatek | We propose to have [and default QCL assumption with one or two TCI states for PDCCH] from the descrpriton in both components. We also agree with Qualcomms proposal for the the simple description for these two components, hence: - Support of SFN scheme B for both PDCCH and PDSCH. - Support of SFN scheme B for PDSCH only. |
| Xiaomi | We prefer to remove the decription for default QCL asssumpotion in both components. In addition, we prefer to list all the combination of SFN scheme B. 1.Support of SFN scheme B for PDCCH scheduling SFN scheme B PDSCH 2.Support of single-TRP for PDCCH scheduling SFN scheme B PDSCH |
| Intel | Similar to SFN Scheme A suggest the following structure as a starting point: - SFN scheme B for PDSCH - SFN scheme B for PDSCH and PDCCH |
| CATT | We support to remove the [and default QCL assumption with one or two TCI states for PDCCH/PDSCH].And for the transmission scheme combination, we agree with Xiaomi and DOCOMO. |
| Lenovo/MotM | Agree with Huawei to remove the [23-6-1] pre-requisite. Regarding the component description, we prefer if the description is analogous to that in [23-6-1] |
| OPPO | - Suggest to remove the part of default QCL assumption - Perfer separate FG for current component 1 and component 2. Current version is also acceptable. - Suggest to remove the pre-requisite part. |

# Issue 8: FG 23-6-2a

Proposed Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-6-2a | Dynamic switching – scheme B | Support of dynamic switching between single-TRP and SFN PDSCH scheme B by TCI state field in DCI formats 1_1, 1_2 | 23-6-2 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

[Please only comment in the table if you are NOT okay with the proposed agreement]

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | The reporting granularity should be the same as FG 23-6-2. At this stage, it can also be [Per band or per FS or FSPC]. |
| ZTE | Okay. |
| Apple | We are okay with this FG |
| Qualcomm | The reporting type should be similar to FG 23-6-2 [Per band or per FS or FSPC] |
| NTT DOCOMO | Support. |
| Mediatek | Agree with Huawei, HiSilicon comment. Reporting should be [Per band or per FS or FSPC]. |
| CATT | Support. |

# Issue 9: FG 23-6-4

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-6-4 | Default DL beam setup [for SFN HST] | - Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold - FFS: Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI [when PDSCH is scheduled with offset equal or larger than the threshold] - Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold FFS: 4. Support of single-TRP PUCCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured FFS: 5. Support of single-TRP PUSCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured FFS: 6. Support of single-TRP SRS resource transmission using default beam when enhanced SFN PDCCH transmission scheme is configured | 23-6-1, 23-6-2 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-6-4a | Default UL beam setup for SFN | 1. Support of single-TRP PUCCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 2. Support of single-TRP PUSCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 3. Support of single-TRP SRS resource transmission using default beam when enhanced SFN PDCCH transmission scheme is configured | 23-6-1, 23-6-2 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Samsung | Support in principle. Those FGs should be FR2 only. |
| ZTE | Okay. |
| Apple | We are okay in principle. |
| Qualcomm | Support |
| NTT DOCOMO | Not support. The pre-requisit “FG23-6-1, 23-6-2” is not correct. Even if UE supports FG23-6-1 and not support FG23-6-2, it is possible that UE supports default beam (FG23-6-4/23-6-4a) for Scheme A only. If the pre-requisit “FG23-6-1, 23-6-2” is agreed, operators will need to require UE to implement both FG23-6-1 and FG23-6-2 to use default beam, even if either of FG23-6-1 or FG23-6-2 is used. Hence, we suggest to separate the FG23-6-4 and FG23-6-4a for each of Scheme A and Scheme B. For example, - FG23-4: default DL beam for Scheme A (pre-requisit = FG23-6-1) - FG23-4a: default UL beam for Scheme A (pre-requisit = FG23-6-1) - FG23-4b: default DL beam for Scheme B (pre-requisit = FG23-6-2) FG23-4c: default UL beam for Scheme B (pre-requisit = FG23-6-2) |
| MediaTek | Support |
| CATT | Support |
| Lenovo/MotM | We are fine to split into DL FG and UL FG. |
| OPPO | Support. |

# Issue 10: FG 23-7-2

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-7-2 | Support of more than 16 max # of Tx ports [per source/across two CMRs] [and max # of resources in a resource set for Multi-TRP CSI] [and max # resources] | [A One or more list(s) of supported combinations, each combination is {max # of Tx ports per resource in a resource set for Multi-TRP CSI, max # resources in a resource set for Multi-TRP CSI}] [Note: same number of ports among CMRs] |  |  |  |  | Per band per BC |  |  |  | Maximum number of lists: 16 Max # of Tx ports per source: [{[4, 8, 12, 16, 24,] 32}] Max # resources: {2, 3, 4, 5, 6, 7, 8} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note: continue discussion whether to merge this FG with FG 23-7-1

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Samsung | We prefer to keep 23-7-2 as separate FG due to an optional feature based on RAN1 agreement. Regarding candidate value for “Max # of Tx ports per source”, we prefer to delete 4, 8, 12, and 16”. |
| ZTE | Support |
| Apple | The reporting type should be “Per band and Per BC”. “Per band per BC” is essentially “per FS” which is different. |
| Ericsson | We do not agree with ‘per band per BC’ as the reporting type for this feature. Note that RAN2 provided the following Guidance in the RAN2 LS (R1-2001513, Guidelines for UE capability definitions, RAN2) states: - Minimize features “per-BandCombination” and “per-Band-of-a-BandCombination” The number of features that require capability signaling with “per-BandCombination” and “per-Band-of-a-BandCombination” (FeatureSetPerCC) differentiation should be minimized. Such features cause an explosion in the size of the capability signaling and are even more severe when it comes to the number of possible configurations, making it impossible for the gNB to take all combinations into account. It would be of help for RAN2 to be informed of the reasons why “per-BC” or “per-band-of-a-BC” was considered necessary, and whether other alternatives were considered. Why ‘per band per BC’ is necessary here is not well justified. |
| Qualcomm | This is already being discussed as part of 23-7-1. This row should be deleted as w/o including this in the basic FG, 23-7-1 will be ill-defined as mentioned before. |
| LG | In the 3rd column and in the 2nd last column, “per source” should be modified to “per resource”. |
| NTT DOCOMO | It is under discussion in 23-7-1. We should avoid duplications. |
| Mediatek | We agree with the current candidate values. We propose to merge of 23-7-2 to 23-7-1. |
| Intel | We are OK to keep this FG considering that it controls the configuration of MTRP CSI while other proposed components in FG 23-7-1 controls the complexity of NCJT/STR CSIs calculated simultaneously. Considering the above, in our view per band per BC granularity is not needed for this FG. We suggest to keep per band granularity. |
| CATT | Support |
| Lenovo/MotM | Support. Our understanding is that “per source” should be replaced with “per resource”. We would appreciate further clarity on that |
| CMCC | Support. We think this FG shoule be a separate FG since FG 23-7-2 is about the resource configuration of Multi-TRP CSI, while the component in FG 23-7-1 is about the UE complexity of NCJT and STRP Csi calculated simultaneously. |
| Vivo | We are not sure if “one or more list(s) of supported combinations” is correct. Seems that a list of combinations means multiple {max # of Tx ports per resource in a resource set for Multi-TRP CSI, max # resources in a resource set for Multi-TRP CSI}, then what do more lists of combinations mean? Besides, this FG seems redundant of what we have in FG 23-7-1 in latest agreement in “Session_Notes_AI_8-15-1_v06.docx”, e.g., 2. Maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks,max 5. A list of [supported combinations, up to 16, across all CCs simultaneously, where each combination is] a) [Maximum number of Tx ports in one NZP CSI-RS resource associated with a single-TRP measurement hypothesis] b) Maximum number of Tx ports in one NZP CSI-RS resource associated with an NCJT measurement hypothesis c) [Maximum total number of CMRs for calculated single-TRP measurement hypotheses] [per CC/across all CCs] d) Maximum total number of CMRs for calculated NCJT measurement hypotheses [per CC/across all CCs] e) [Maximum total number of Tx ports of NZP CSI-RS resources associated with single-TRP measurement hypotheses] [per CC/across all CCs] f) Maximum total number of Tx ports of NZP CSI-RS resources associated with NCJT measurement hypotheses [per CC/across all CCs] g) [Maximum total number of Tx ports of NZP CSI-RS resources associated with one NCJT measurement] Therefore, we can consider this FG and FG 23-7-1 together. |
| OPPO | We are fine to have this FG. |

# Issue 11: FG 23-7-3

Proposed Agreement: Delete FG/row 23-7-3

| 23. NR_FeMIMO | 23-7-3 | More than two resources in a resource set for Multi-TRP CSI | FFS exact candidate values, Ks,max is up to 8 |  |  |  |  |  |  |  |  | [candidate values are {2, 3, 4, 5, 6, 7, 8}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

[Please only comment in the table if you are NOT okay with the proposed agreement]

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | We prefer to keep 23-7-2 as a part of FG 23-7-1, or we can come back later once FG 23-7-1 is clarified and more stable. |
| ZTE | Support |
| Apple | We are okay |
| Qualcomm | This cannot be simply deleted as it is already agreed. It can be either merged with basic FG or kept as a separate FG. Our preference is a separate FG. Agreement With regarding to the maximal values of Nmax for N, Ks,max for Ks: - Support of Nmax=2 is a UE optional feature - Support of Ks,max=X is a UE optional feature - X can be up to 8 and other candidate values can be discussed as part of UE features - FFS: Default value of Nmax, Ks,max - FFS: Which combinations of N<=Nmax, Ks<=Ks,max are supported Conclusion: Default value of Ks, max can be discussed later with Rel-17 MIMO UE capability. |
| NTT DOCOMO | Okay to delete it, and discuss it in 23-7-1. |
| CATT | Support |
| CMCC | Support, we can discuss it in FG 23-7-1. |
| Vivo | Prefer to consider this FG and FG 23-7-1, FG 23-7-2 together. |
| OPPO | We can delete it if consider it in 23-7-1. |

# Issue 12: FG 23-7-5

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-7-5 | CMR sharing | Support a NZP CSI-RS resource referred by both a CMR pair configured for Rel-17 Multi-TRP CSI enhancement NCJT measurement hypothesis and a single CMR configured for CSI determination Single-TRP measurement hypothesis | 23-7-1 |  |  |  | Per band |  | FR2 only |  | Note: ‘NCJT’ and ‘single-TRP’ are not used in RAN1 specifications and will be aligned with 38.214 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Samsung | We are fine with deleting “NCJT measurement hypothesis” and “Single-TRP measurement hypothesis” and it seems that “for CSI determination” can be also deleted. |
| Huawei, HiSilicon | We are fine in general. To take into account amsung’ comment, how about this change: “Support a NZP CSI-RS resource referred by both a CMR pair and a single CMR configured for Rel-17 Multi-TRP CSI enhancement” |
| ZTE | Support, and we can be flexible for Huawei’s update. |
| Apple | We are okay with this FG or the suggestion from Huawei |
| Ericsson | For the component description, we suggest to use the terminology used in 38.214 instead of CMR pair and CMR. Our suggestion is as follows: “Support a NZP CSI-RS resource referred by both a NZP CSI-RS resource pair (as defined in TS 38.214 Clause 5.2.1.4.1) and a NZP CSI-RS resource in one of the Resource Groups (as defined in TS 38.214 Clause 5.2.1.4.1).” |
| NTT DOCOMO | Okay with E///’s revision. |
| CATT | We are okay with Huawei’s update. |
| Lenovo/MotM | Support, we are also OK with Huawei’s wording |
| CMCC | Support and we are also fine with Huawei’s version. |
| Vivo | We are fine with Huawei’s wording. |
| OPPO | Support Huawei’s wording. |

# Issue 13: FG 23-8-1

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-8-1 | SRS triggering offset enhancement | [Support of /The maximum number of configured available slots offsets for] determining aperiodic SRS location based on available slot [and dynamically based on Rel-17 introduced new field in DCI Format 0_1/0_2/1_1/1_2] | 2-52 |  |  |  | Per FS band |  |  |  | Candidate component values: {none, 1, 2, [3,] 4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | We support to use “The maximum number of configured available slot offsets” as the description of this FG, which reflects a better trade-off between NW’s and UE’s implementation needs. We are okay to remove 3, or put it in brackets. |
| Huawei, HiSilicon | We are okay with this FG. In terms of the yellow part, we prefer ‘The maximum number of configured available slots offsets for’ and removing [3,] is acceptable to us. |
| Apple | We are okay with this FG |
| Ericsson | Support |
| Qualcomm | Fine with per-band reporting type. We don’t support per-UE repoting as it mandates the support across all bands. However, UE may not support this feature for some bands e.g., unlicensed or FDD band. In addition, given the potential UE testing differentiation among licensed, unlicensed, and NTN band, the type for a UE feature should be at least per band (if not with finer granularity type). |
| LG | We still think that there is no clear reason for report of the max number of configured available slot offsets, but we can live with it if majority supports. |
| DOCOMO | We support moderator’s proposal. Either “Support of” or “The maximum number of configured available slots offsets for” is fine, while we feel the latter one could be slightly clearer. |
| MediaTek | Since the available slots are dynamic depending on slot format and thus the actual slots offset is undetermined, we prefer to add a second component for the maximum actual slots offset UE can support: - Component 2: support of the maximum actual slots offset. - FFS: candidate values |
| Xiaomi | Okay with this. |
| CATT | We prefer not to report the maximum number of configured available slot offsets. |
| Lenovo/MotM | We prefer ‘The maximum number of configured available slots offsets for’ and fine to remove [3]. |
| OPPO | We prefer to keep ‘The maximum number of configured available slots offsets for’ and to remove [3]. |

# Issue 14: FG 23-8-2

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-8-2 | Triggering SRS only in DCI 0_1/0_2 | Support of triggering SRS in DCI 0_1/0_2 without data and without CSI | 2-52 |  |  |  | [Per FS UE or per band] |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | We prefer “per UE” as the granularity type. We are open to hear UE vendors’ reasoning on why it needs “per band”. |
| Huawei, HiSilicon | Agree with ZTE, we prefer to define it as “Per UE”. |
| Apple | We are okay with this FG |
| Ericsson | Per UE or per band is ok |
| Qualcomm | Reporting type resouliton should be at least per-band. We don’t support per-UE repoting as it mandates the support across all bands. However, UE may not support this feature for some bands e.g., unlicensed or FDD band. In addition, given the potential UE testing differentiation among licensed, unlicensed, and NTN band, the type for a UE feature should be at least per band (if not with finer granularity type). |
| LG | OK |
| DOCOMO | Support moderator’s proposal. Either “per UE” or “per band” is fine. |
| Xiaomi | We prefer this “per UE” configured. |
| CATT | Similar view as ZTE and Huawei, we prefer to define it as “per UE”. |
| Lenovo/MotM | Similar view with ZTE and Huawei. Prefer to define it as “Per UE”. |
| OPPO | Share the same view as QC |

# Issue 15: FG 23-8-3

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-8-3 | SRS Antenna switching for >4Rx | 1. Support of SRS antenna switching xTyR with y>4 2. Report whether the antenna switching impact to downlink receiving in a band 3. Report whether the antenna is switched together with UL Tx in another band | 2-55 |  |  |  | Per BC |  |  |  | Component 1 candidate values: FFS [one or more elements from set {t1r1, t2r2, t1r2, t4r4, t2r4, t1r4, t2r6, t1r6, t4r8, t2r8, t1r8}] Component 2 candidate values: {1 to 32} FFS Component 3 candidate values: {1 to 32} FFS FFS for component 2&3: impact when UE reports component 1 as xTyR with x=y | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | We are okay with the yellow part. |
| Huawei, HiSilicon | For Compoment 1, as we mentioned before, it is arbitrary combinations of xTyR, and also overlapped with Rel-15 and Rel-16 UE capability reporting. Actually, Rel-15 and Rel-16 UE capability for xTyR are already there for supporting <=4R antenna switching, the only thing we need to introduce for Rel-17 is >4R antenna switching, i.e., 6R and 8R part. UE could report Rel-17 and Rel-15/16 UE capability. So, the clear UE capability report here is: Component 1: one or more elements from set {t1r1, t2r2, t1r2, t4r4, t2r4, t1r4, t2r6, t1r6, t4r8, t2r8, t1r8} |
| Apple | Please mark the reporting type in yellow “per BC”. It cannot be assumed that UE supports the same AS configuration regardless of bands, since the UE antenna/PA/HW configuration can be different for different bands. As we explained, it is per FS now in the spec and it was captured incorrectly in 38.306 |
| Ericsson | We agree with Docomo from first checkpoint (Section 3.15) that downgraded configuration should also be included. Furthermore, a question to UE side vendors which impacts on the UE capability ignaling: For this large number of RX chains, is there a need for implementation flexibility to support unbalanced antenna switching. i.e. where the number of RX ports per TX port is not the same for all TX ports??? For example, if the UE reports 2 TX and 6 RX, for an unbalanced implementation as in the figure, a different number of OFDM symbols is needed compared to the balanced case. |
| Qualcomm | Support |
| DOCOMO | Same comment as what we had in the previous round, copied below: We believe downgraded configuration should also be included in candidate values, instead of modifying the existing Rel-16 capability which doesn’t consider R more than 4. The following can be added: - ‘t1r1-t1r2-t1r4-t1r6’ for 1T=1R/1T2R/1T4R/1T6R - ‘t1r1-t1r2-t2r2-t1r4-t2r4-t1r6-t2r6’ for 1T=1R/1T2R/2T=2R/1T4R/2T4R/1T6R/2T6R - ‘t1r1-t1r2-t1r4-t1r6-t1r8’ for 1T=1R/1T2R/1T4R/1T6R/1T8R - ‘t1r1-t1r2-t2r2-t1r4-t2r4-t1r6-t2r6-t1r8-t2r8’ for 1T=1R/1T2R/2T=2R/1T4R/2T4R/1T6R/2T6R/1T8R/2T8R - ‘t1r1-t1r2-t2r2-t1r4-t2r4-t1r6-t2r6t1r8-t2r8-t4r8’ for 1T=1R/1T2R/2T=2R/1T4R/2T4R/1T6R/2T6R1T8R/2T8R/4T8R |
| MediaTek | Support |
| Xiaomi | Support |
| CATT | As we mentioned before, we prefer to remove “t1r1, t2r2, t1r2, t4r4, t2r4, t1r4” from the candidate values for component 1 to avoid UE capability overlapping. Although we support downgraded configuration, we don’t think capability reporting on arbitrary combination is needed. We suggest to change the candidate values of component 1 to: - t1r1-t1r2-t1r4-t1r6, - t1r1-t1r2-t1r4-t1r6-t1r8, - t1r1-t1r2-t2r2-t2r4-t2r6, - t1r1-t1r2-t1r4-t2r2-t2r4-t2r6-t2r8, - t1r1-t1r2-t2r2-t2r4-t4r4-t4r8 |
| Lenovo/MotM | Since the FG name is “SRS Antenna switching for >4Rx”, we prefer the candicate value for component 1 only including { t2r6, t1r6, t4r8, t2r8, t1r8}. |
| OPPO | Regarding the report type, we support Apple’s proposal since the related UE capability is reported per band per BC in TS 38.331 (i.e., the RRC signaling structure is BandCombination -> BandParameters -> srs-TxSwitch) Regarding the downgraded configuration, since the 6R configuration is special compared to 1R/2R/4R/8R, it is attracting to support txr6 as the default of txr8. Thus, we suggest the following candidate values: (modified from Docomo’s proposal) - t1r1-t1r2-t1r4-t1r6 - t1r1-t1r2-t2r2-t1r4-t2r4-t1r6-t2r6 - t1r1-t1r2-t1r4-t1r8 - t1r1-t1r2-t2r2-t1r4-t2r4-t1r8-t2r8 - t1r1-t1r2-t2r2-t1r4-t2r4-t1r8-t2r8-t4r8 |

# Issue 16: FG 23-8-4

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-8-4 | Maximum 2 SP and 1 periodic SRS sets for antenna switching | Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for antenna switching | 2-53 |  |  |  | Per FS band |  |  |  | Note1: - Applies for all supported xTyR where y<=8 - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - For xTyR where y<=4, if UE does not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS - The two SP-SRS resource sets are not activated at the same time | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Support |
| Huawei, HiSilicon | Not OK to change ‘per FS’ to ‘per Band’, since the supported number of SRS resources are reported per FS as in the prerequisite 2-53, so it should be kept consistent with ‘per FS’. |
| Apple | We are okay with this FG |
| Qualcomm | Don’t support per-band reporting type. It should be per FS similar to the prerequisite 2-53 (per-FS) |
| LG | OK |
| DOCOMO | We support moderator’s proposal. After some consideration, we changed our view. Since the pre-requisit FG2-53 is “per FS”, the new FG23-8-4 should be reported “per FS”. |
| Xiaomi | Okay with this |
| Lenovo/MotM | Fine. |
| OPPO | Ok |

# Issue 17: FG 23-8-5

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-8-5 | Increased repetition for SRS | Support of increased repetition patterns (8, 10, 12, 14 symbols) for SRS resource | 10-11, 2-52 |  |  |  | Per FS band |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Support |
| Huawei, HiSilicon | Support |
| Apple | We are okay with this FG |
| Ericsson | Support |
| Qualcomm | Fine with per-band reporting type. |
| LG | OK |
| DOCOMO | We support moderator’s proposal. |
| Xiaomi | okay |
| CATT | We are fine with the proposal. |
| Lenovo/MotM | Support |
| OPPO | Ok |

# Issue 18: FG 23-8-6

Proposed Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-8-6 | Partial frequency sounding of SRS | Support of partial frequency sounding for SRS | 2-52 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

[Please only comment in the table if you are NOT okay with the proposed agreement]

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | Support |
| MediaTek | Support |
| OPPO | Ok |

# Issue 19: FG 23-8-7

Proposed Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-8-7 | Start RB location hopping for partial frequency SRS | Support of start RB location hopping in partial frequency SRS transmission [across different SRS frequency hopping periods for periodic/semi-persistent/aperiodic SRS] | 23-8-6 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

[Please only comment in the table if you are NOT okay with the proposed agreement]

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | Support |
| MediaTek | Support |
| OPPO | Ok |

# Issue 20: FG 23-8-8

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-8-8 | Comb-8 SRS | Support of comb-8 for SRS other than for positioning |  |  |  |  | Per FS band |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Support |
| Huawei, HiSilicon | Support |
| Apple | We are okay with this FG |
| Ericsson | Support |
| Qualcomm | Fine with per-band reporting type. |
| LG | OK |
| DOCOMO | We support moderator’s proposal. |
| Xiaomi | Okay with this |
| CATT | We are fine with the proposal. |
| Lenovo/MotM | Support |
| OPPO | Ok |

# Issue 21: FG 23-9-2

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-9-2 | Support of M=2 and R=1 for FeType-II | [1. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with M=2 and R=1] 2. Support parameter combinations with M=2 [3. Support of DFT FD bases of size N] | 23-9-1 |  |  |  | [per band and per BC] |  |  |  | [Component 1 candidate values - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}] [Component 34 candidate values: {N=2 only, N=2 and N=4}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Samsung | Support Component 1 and 2 only. |
| ZTE | We are okay except the yellow part. We support to remove component 3. |
| Apple | We are okay with this FG |
| Qualcomm | We prefer to agree FG23-9-2 and FG23-9-4 together. If having triplets for both of them is of big concern, we prefer to keep triplets for FG23-9-4 or open for other methods of overhead reduction. |
| NTT DOCOMO | Okay. |
| MediaTek | Support |
| Intel | We think that limited size for UE capability message can be an issue for deployment of new PMI codebooks. Since multiple companies think that separate indication of triplets is required for different values of M and R, we propose to support additional mechanism to decrease the capability signaling overhead if it is needed for the UE. For example, we can assume that if the list of triplets (component 1) in FG 23-9-2 is empty then the list of triplets is reused from component 1 of FG 23-9-1. Thus, we propose to add the following note. NOTE: if the list of triplets in component 1 of FG 23-9-2 is empty then the list of triplets is reused from component 1 of FG 23-9-1. |
| CATT | Component 1 should be removed since the number of CSI-RS ports has been restricted for M=2. |
| Lenovo/MotM | OK with components 1, 2. Do not support component 3. |
| Vivo | Fine to remove component 3. |
| OPPO | Support. |

# Issue 22: FG 23-9-3

Proposed Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-9-3 | Support of rank 3, 4 for FeType-II | Support of rank 3, 4 for FeType-II FFS separate capabilities for M = 1 and M = 2 | 23-9-1[, 23-9-2] |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

[Please only comment in the table if you are NOT okay with the proposed agreement]

| Company | Comments/Questions/Suggestions |
| --- | --- |

# Issue 23: FG 23-9-4

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-9-4 | Support of R = 2 for FeType-II | 1. Support of R = 2 for FeType-II [2. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support to support Port-selection FeType-II with M=2 and R=2] [3. Support parameter combinations with Mv=2 and R=2] | 23-9-2 |  |  |  | [per band and per BC] |  |  |  | Component 2 candidate values: • Maximum 16 triplets • Max # of Tx ports in one resource: {4,8,12,16,24,32} • Max # resources: [{1 to 64}] • Max # total ports: [{4 to 256}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | We are okay except the yellow part. We support to remove component 3. The only case to support R=2 in the current specification is M=2. Hence we don’t see the need to have component 3 given we already have component 1. Regarding the overhead issue of the triplets in 23-9-2 and 23-9-4, we think it is okay to keep both. But if we have to choose, we prefer to keep the triplets in 23-9-2. We are also fine to discuss M and R together on the supported triplets as QC suggested. |
| Huawei, HiSilicon | We still have concerns over Component 2 which gives too many triplets reported per band and per BC for FG 23-9-x. A middle point, if possible as a compromise, is that we keep component 1 as per band and per BC, rather than normal granuartiy of per band. Therefore 23-9-4 can be the same reporting mechansium/granuraity as 23-9-2 to ON/OFF R=2 on top of 23-9-4. |
| Apple | We are okay with this FG |
| Qualcomm | Support the proposal. We still prefer triplets for 23-9-4 more than FG23-9-2. |
| NTT DOCOMO | Okay. |
| MediaTek | We also prefer keeping triplet for 23-9-4 more than FG23-9-2 if we were to chose only one. |
| Intel | Similar to FG 23-9-2 we propose to add the following note. NOTE: if the list of triplets in component 2 of FG 23-9-2 is empty then the list of triplets is reused from the list of triplets corresponding to FeType-II with M=2 and R=1. |
| CATT | This feature should be combined with FG23-9-2. |
| Lenovo/MotM | Support. We can be flexible regarding the inclusion of Component 3 |
| OPPO | Support. |

# Issue 24: FG 23-9-5

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-9-5 | Active CSI-RS resources and ports for mixed codebook types in any slot | - List of codebook combinations - List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination | [23-9-1], 16-3a, 16-3b, 16-3a-1, 16-3b-1, 2-36, 2-40, 2-41, 2-43 |  |  |  | FFS Per band and per BC |  |  |  | [Component 1 candidate values: Codebook 1 = {Type I SP, Type I MP} {Codebook 2, Codebook 3} = {{FeType II PS M=1, NULL}, {FeType II PS M=2 R=1, NULL}, {FeType II PS M=2 R=2, NULL}, {Type II, FeType II PS M=1}, {Type II, FeType II PS M=2 R=1} ,{eType II R=1, FeType II PS M=1},{eType II R=1, FeType II PS M=2 R=1}} FFS: The list of {Codebook 2, Codebook 3} may be refined depending on outcome of discussion for FG 23-9-2 Component 1 and FG 23-9-4 Component 2 Component 2 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}] Note 1：if a UE reports one or more codebook combinations in [23-9-5], then usage of active CSI-RS resources and ports for multiple codebooks in any slot is allowed only within those combinations [Note 2: For coexisting of mixed codebooks in any slot, gNB need to honor 16-8, 23-10 23-9-5and per-codebook capability 2-36/40/41/43, 16-3a/b and 16-3a-1/16-3b-1, and 23-9-1/23-9-2/23-9-4] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | We are okay. |
| Huawei, HiSilicon | Similar with FG 23-9-4, it may need to yellow {FeType II PS M=2 R=2, NULL} for now in FG 23-9-5. We agree with other changes/FFS. |
| Apple | We are okay with this FG |
| Qualcomm | Support |
| NTT DOCOMO | Okay. |
| MediaTek | Support |
| Intel | The same logic as in our comment for FG 23-9-2 can be applied here as well. So, we propose to add the following note. Note: If the list of triplets for a codebook combination is empty then the list of triplets is reused from the codebook combination indicated previously. If the above note is supported then UE can indicate list of triplets for {Codebook 1, FeType II PS M = 1, Codebook 3} and reuse it for {Codebook 1, FeType II PS M = 2 R = 1, Codebook 3} and/or {Codebook 1, FeType II PS M = 2 R = 2, Codebook 3} |
| Lenovo/MotM | Support |
| vivo | We are fine. |

# Issue 25: FG 23-1-1

[Please indicate which of the following components should be included into FG 23-1-1 and which components should be separate FGs 23-1-1a/b/c …]

[The goal is to copy the agreement from the GTW session into the table for further editing and discussion]

- Joint DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals)

- Common multi-CC TCI update and activation

- For PUCCH, PUSCH, and SRS, association between TCI state and UL PC settings except for PL RS

- Beam misalignment between the DL source RS in the TCI state to provide spatial relation indication and the PL-RS

- TCI state indication [mode]: update and activation [in case of updates]
a) MAC CE based TCI state indication [for one active TCI state]
b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment)
c) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment)

- Reference BWP/CC configured with reference TCI state pool shared by a set of BWP/CC
Note: agree component, final wording may change (e.g., when this is merged with other components/FGs)

- Maximum number of CCs configured with BFR
FFS whether this is a component or just a note in the FG to reuse R16 signaling

- [R17 mechanism reusing R15/16 signalling to indicate R17 TCI for individual DL channel/RS that cannot share the same unified TCI as UE-dedicated PDCCH/PDSCH]

- The maximum number of configured joint TCI state pools across all BWPs and all CCs in a band [in a band combination]
FFS: Whether to make component 6 a prerequisite or merge with 6

- [The maximum number of configured joint TCI states across all BWPs and all CCs in a band] [in a band combination]

- The maximum number of configured joint TCI states [per BWP per CC] [in a band] [in a band combination]

- The maximum number of MAC-CE activated joint TCI states across all CCs [in a band] [in a band combination]
a) The maximum number of MAC-CE activated joint TCI states per CC [in a band] [in a band combination]

- [The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS]

- [The minimum time gap between the beam indication PDCCH and first slot where beam is applied]

| 23. NR_FeMIMO | 23-1-1 | Unified TCI for [intra- and inter-cell] beam management |  |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-1-1a |  |  |  |  |  |  |  |  |  |  |  |  |
| 23. NR_FeMIMO | 23-1-1b |  |  |  |  |  |  |  |  |  |  |  |  |
| 23. NR_FeMIMO | 23-1-1c |  |  |  |  |  |  |  |  |  |  |  |  |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Example | Components 1/3/5/6/7 should be part of the basic FG 23-1-a Components 2/4 should be in a separate FG Component 8 should be in a separate FG Components 9/10/11 should be in a separate FG … |
| Huawei, HiSilicon | We suggest grouping these components into the 2 features that were formulated in the main agenda, i.e., unified TCI (component 1, 2, 3, 4, 6, 7, 8, 9, 10, 11), dynamic TCI update (component 5, 12, 13, 14), in addition to 23-1-3 (MPE mitigation) and 23-1-4 (MPUE), while inter-cell BM is to be treated separately. To save the time of debating which one is basic, we suggest not to define a basic group (leave the choice to market and implementation). In particular, given the explicit agreement below (“Support UE to report whether or not to support TCI update by DCI format 1_1/1_2”), we have strong concern on including component 5, 12, 13, 14 into a basic group of R17 BM if introduced. The intention of the agreement is clear – to have dedicated UE feature for DCI-based TCI indication (as a compromise between proponents and opponents). Arguing that the agreement did not explicitly say dedicated UE feature and trying to bundle DCI-based TCI indication with other components into a basic group is simply trying to overturn the agreement/compromise, which is being unreasonable. Agreement For beam indication with Rel-17 unified TCI, support DCI format 1_1/1_2 without DL assignment: - Use ACK/NACK mechanism analogous to that for SPS PDSCH release with both type-1 and type-2 HARQ-ACK codebook: - Upon a successful reception of the beam indication DCI, the UE reports an ACK - Note that upon a failed reception of the beam indication DCI, a NACK can be reported. - For type-1 HARQ-ACK codebook, a location for the ACK information in the HARQ-ACK codebook is determined based on a virtual PDSCH indicated by the TDRA field in the beam indication DCI, based on the time domain allocation list configured for PDSCH - For type-2 HARQ-ACK codebook, a location for the ACK information in the HARQ-ACK codebook is determined according to the same rule for SPS release - The ACK is reported in a PUCCH k slots after the end of the PDCCH reception where k is indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI format, or provided dl-DataToUL-ACK or dl-DataToUL-ACK-ForDCI-Format1-2-r16 if the PDSCH-to-HARQ_feedback timing indicator field is not present in the DCI - When used for beam indication: - CS-RNTI is used to scramble the CRC for the DCI - The values of the following DCI fields are set as follows: - RV = all ‘1’s - MCS = all ‘1’s - NDI = 0 - Set to all ‘0’s for FDRA Type 0, or all ‘1’s for FDRA Type 1, or all ‘0’s for dynamicSwitch (same as in Table 10.2-4 of TS38.213) - FFS: Whether HPN is also used - Use the existing TCI field (always present) to signal the following: 1) Joint DL/UL TCI state, 2) DL-only TCI state (for separate DL/UL TCI), 3) UL-only TCI state (for separate DL/UL TCI) - FFS: Whether both DL TCI and UL TCI states can be signaled in one instance of beam indication DCI - FFS: Relation with joint vs separate TCI (DL and/or UL) switching, including M/N>1 if supported - In addition, use the following DCI fields as the fields are being used in Rel-16: - Identifier for DCI formats - Carrier indicator - Bandwidth part indicator - TDRA - Downlink assignment index (if configured) - TPC command for scheduled PUCCH - PUCCH resource indicator - PDSCH-to-HARQ_feedback timing indicator (if present) - The remaining unused DCI fields and codepoints are reserved in R17 - Support UE to report whether or not to support TCI update by DCI format 1_1/1_2. - For a UE supporting TCI update by DCI format 1_1/1_2, it must support TCI update by using DCI 1_1/1_2 with DL assignment, and support of the above feature for TCI update by DCI format 1_1/1_2 without DL assignment is UE optional - FFS: How to handle the case when there is only UL data o FFS: The case for UE being indicated with separate UL TCI in DCI format 1_1/1_2 with DL assignment - FFS: When more than one TCI codepoints are activated by MAC CE, the activated TCI state(s) for the lowest codepoint is/are applied - Support of this feature is UE optional - The “lowest codepoint” function can be configured on or off. - FFS: Interaction with the DCI based beam update if needed, whether/how to support the case with M or N > 1 if supported Note: This agreement on DCI beam indication design is not to be used to be against the support of the cases of M/N>1. The support of M/N>1 will be separately discussed and not dependent on the decision here. |
| Samsung | - Components 1, 3, 5, 11, 12, 13 should be included in the basic FG 23-1-a, with the following clarification - Component 12: Rel-17 TCI basic feature supports 4 MAC-CE activated joint TCI state per CC - Component 5: Rel-17 TCI basic feature supports all the 3 beam indication schemes as follows: - When a UE is configured with 1 MAC-CE activated TCI state, MAC-CE based TCI state indication applies - When a UE is configured with >1 MAC-CE activated TCI states, MAC-CE+DCI-based based TCI state indication applies, and the UE is configured with EITHER DCI 1_1/1_2 with DL assignment OR without DL assignment - Component 2, 6, 9 should be in a separate FG 23-1-b - Supported when Carrier Agregation is supported - Component 8 should be in a separate FG 23-1-c - This applies to AP CSI-RS, DMRS for non-UE-dedicated reception, AP SRS. If not supported, the default behavior is that such signals share the same TCI state as UE_dedicated PDCCH/PDSCH @Huawei: Given that all features in release(s) after Rel-15 are understood as UE optional in the absolute sense (e.g. not all NR UEs will support Rel-17 at least for some time), it is our view that there is no overturning/reverting agreement when the support of >1 activated TCI states and MAC-CE+DCI-based TCI state indication is included as a part of Rel-17 TCI basic feature (which is an open issue). Both basic and auxiliary features are still considered optional in the absolute sense. The current discussion point is the level of optionality. |
| ZTE | Components 1/2/3/5 /6/8/11/12/12a/13 should be part of the basic FG 23-1-a - Note that, for component-5, we think that all of three candidates should be considered as a basic feature. Of course, all of them are optional. - Note that, if component-11, 12, 12-a, 13 are precluded from the basic FG 23-1-a, we have to provide default value(s). Components 4 should be in a separate FG Component 7 should be considered as a note of reusing Rel-15/16 UE capability signaling rather than a separate FG. Component 9 should be in a separate FG, and then component 6 (or basic feature as above) can be made as a prerequisite for this component. Components 10/14 should be removed due to the fact that motivations for both of them are unclear even after several round discussion. |
| Apple | Components 1 should be part of the basic FG 23-1-1 Components 2/6/9/10/11 should be in a separate FG for TCI state for CA Component 3/4 should be in a separate FG for power control Components 5/8/13 should be in a separate FG for TCI indication Components 12 should be in a separate FG for TCI activation Components 7 should be in a separate FG for BFR under unified TCI framework |
| Ericsson | The header says Unified TCI for [intra- and inter-cell] beam management. Our response depends on if both intra-cell and inter-cell are considered If both intra-cell and inter-cell is included: - Component 1, 2, 5, 10, 11, 12, 13 should be included in the basic FG 23-1-1 - Component 3 should be in a separate FG - Component 4 should be in a separate FG - Component 6, 9 should be in a separate FG - Component 8 should be in a separate FG If only intra-cell is included: - Component 1, 2, 10, 11, 12, 13 should be included in the basic FG 23-1-1 - Component 3 should be in a separate FG - Component 5 should be in a separate FG - Component 4 should be in a separate FG - Component 6, 9 should be in a separate FG - Component 8 should be in a separate FG In either case, component 7 (no relation to unified TCI) and 14 (replaced by 13) should not be in any FG. |
| Qualcomm | For component 5, suggest the following wording - TCI state indication [mode]:, where indication includes both update and activation [in case of updates] a) MAC CE based TCI state indication [for one active TCI state] b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) c) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) For component 7, we prefer it to be a new component. To our understanding, the FG 16-1f may not allow UE to indicate the following capability (our understanding of component 7), since UE capability report for Case 1 and 2 may conflict with each other. So the general motivation is to have a UE capability directly for the high lighted part, which should be jointly considered with FG 16-1f - UE is only able to perform BFR on one cell per band on FR2 - Case 1: Both PCell and SCell are on FR2 for intra-band CA - For this, UE needs to indicate not support FG 16-1f, i.e. not support SCell BFR at all - Otherwise, NW can configure both PCell BFR + SCell BFR - Case 2: PCell on FR1 and SCell on FR2 for inter-band CA - For this, UE needs to indicate support 1 for FG 16-1f, i.e. support BFR only on one SCell - Otherwise, UE may not be configured with BFR on FR2 at all 16-1f; SCell beam failure recovery; 1. The maximum number of SCells configured for SCell beam failure recovery simultaneously; 2-31; maxNumberSCellBFR-r16; MIMO-ParametersPerBand; No; No; Component-1: candidate value set is {1,2,4,8}; Optional with capability signaling. For component 8, we are fine for the latest wording from multiple companies in offline email discussion - Support of activation/configuration of Rel-17 TCI states for aperiodic CSI-RS, PDCCH, PDSCH, and SRS reusing the Rel-15/16 signaling/configuration design(s) For component 9, support to make 6 a prerequisite for 9, which makes sense only when UE supports 6. Also, prefer not to have band combination. For component 10, prefer to keep it. A note can be added to clarify that if UE supports 10, the component 11 can be ignored or not reported. For component 11, prefer to keep “per BWP per CC”. The “per-CC” in legacy FG 2-4 is not accurate to our understanding. Otherwise, please clarify component 11 is counted “per BWP” or “across all BWPs per CC”. Also prefer “in a band” For component 12, prefer “in a band” For component 13 and 14, slightly prefer 14, which is the exact agreement wording. But 13 is also fine as long as it is clarified. |
| LG | Component 1, 3, 5(a), 8, 11, 12(a), 13 should be included in the basic FG 23-1-1 Component 5(b), 5(c) should be in a separate FG As captured by Huawei/HiSilicon, UE capability reporting of whether to support DCI-based TCI indication with/without DL assignment was already agreed. Component 2, 6, 9, 10 should be in a separate FG for CA Component 4 should be in a separate FG Component 7 should be in a separate FG if needed (depending on the result of FFS) |
| Huawei, HiSilicon | Our concern on including component 5b and 5c in basic feaure group of R17 BM remains. Enjoy the quiet period. |
| NTT Docomo | Components 1/2/3/5/6/8/11/12/12a/13 should be part of the basic FG 23-1-a. Components 4/9 should be separate FG from the basic FG 23-1-a. Component 7 is not needed, we can reuse Rel.15/16 FG. Component 10 is not needed, even in Rel.15, such UE capability was not defined. We assume the value of component 10 will be multiplication of “the value of component 11” and “supported number of BWPs/CCs”. Component 14 is not needed, we believe component 13 is enough for UE to report the minimum timing of the beam application timing. |
| MediaTek | - Component 1, 2, 3, 5, 11, 12, 13 should be included in the basic FG 23-1-1 - Component 4 should in a separate FG - Componnet 6 and 9 should be in a separate FG for TCI pool sharing - Componnet 7 should be in a separate FG - Componnet 8 should be in a separate FG - Component 10 and 14 should be removed. |
| Xiaomi | Component 1 can be part of the basic FG 23-1-1. Component 2/6/9/10/12 can be part of a separate FG for CA. Component 3/4 can be part of a separate FG for power control. Component 5/8/11/12a/13/14 can be part of a separate FG for TCI state indication, activation and configuration. Component 7 can be part of a separate FG for BFR. |
| Intel | The following components should be considered for basic FG for intra-cell operatio - 1, 2, 5, 10, 11, 12, 13 Other components in separate FGs. Remove components 7 and 14. |
| CATT | Components 1/2/3/5a/6/8/9/10/11/12/13 should be part of the basic FG 23-1-1 Components 5b should be in a separate FG 23-1-1a Components 5c should be in a separate FG 23-1-1b Components 4 should be in a separate FG 23-1-1c We prefer to remove component 7 and component 14. Component 7 ([Maximum number of CCs configured with BFR]) is introduced considering that it is no need to configure BFR on every CC, if all CCs sharing the same analog beam. However, although the same beam is used across CCs, the interference may be different on each CC. BFR of one CC would not be applied to the other CCs. The restriction on the number of CCs configured with BFR would impact the UE performance. Regarding component 14 ([The minimum time gap between the beam indication PDCCH and first slot where beam is applied]), the corresponding UE capability (timeDurationForQCL) has been defined in Rel-15. It is not necessary to define a new UE capability in Rel-17. |
| Lenovo/MotM | Componnent 1/3/5/11/12/13 should be part of basic FG 23-1-a Componnent 2/6/9 should be in a separate FG. Componnent 4 should be a separate FG. Remove component 7 ad component 14. |
| OPPO | Componenet 14 shoud be included beucase that is in previous RAN1 agreement. We are fine to reuse the rel-15 timeDurationForQCL here. But we need specificiaton to claify that timeDurationForQCL is reused here. Otherwise the timeDurationForQCL is only applied to rel-15/16 mechanism PDSCH QCL that is indicated by the scheduling DCI, but not applied to the TCI state indicated in unfied TCI framework. Regarding the Componenets and FGs: - Components 1/5a/12 are in the basic FG - Component 3/4 are in a separate FG - Components 5b-c/8/13/14 are in a separate FG |
| Vivo | Component 1, 10, 11, 12 should be the basic feature for intra-cell operation. - Joint DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals) The maximum number of configured joint TCI states across all BWPs and all CCs in a band in a band combination The maximum number of configured joint TCI states per BWP per CC in a band in a band combination The maximum number of MAC-CE activated joint TCI states across all CCs in a band in a band combination The maximum number of MAC-CE activated joint TCI states per BWP per CC in a band in a band combination Component 5 should be revised as following and be a separate FG merged with component 13. TCI state indication mode [mode]: update and activation [in case of updates] a) MAC CE based TCI state indication [for one active TCI state] b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) c) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) In case if the UE report support of mode b) and mode c), the minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS Component 6, 9 can be merged together with following update and component 2 is the pre-requisite for this FG: Support of reference BWP/CC configured with reference TCI state pool shared by a set of BWP/CC The maximum number of configured joint TCI state pools across all BWPs and all CCs in a band. [in a band combination] FFS: Whether to make component 6 a prerequisite or merge with 6 Component 2, 3, 4, 7, 8 are all separate UE features from above three features and can be independent from each other. |

# Issue 26: FG 23-3-1-1

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-3-1 | Multi-TRP PUSCH repetition (type A)[-CB] | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) [for CB] - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] [2. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A] [3. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2] [4. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) ] [5. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.) ] [6. Support of sequential mapping for single for single DCI based M-TRP PUSCH repetition Type A] [7. The maximum number of PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission)] [8. Support of A-CSI report on two PUSCH repetitions] [9. Support of SP-CSI report on two PUSCH repetitions] [2. Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission] FFS: [11. Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources] |  |  |  |  |  |  |  |  | [Candidate component values: {CB, non-CB, both}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-3-1a | Cyclic mapping | Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1b | Second TPC field | Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1c | Two PHR reporting | Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1e | A-CSI report | Support of A-CSI report on two PUSCH repetitions |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1f | SP-CSI report | Support of SP-CSI report on two PUSCH repetitions |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1g | CG PUSCH transmission | Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.) |  |  |  |  |  |  |  |  |  | Optional with capability signalling |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-3-1-1 | Multi-TRP PUSCH repetition (type B) [-CB] | - Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) [for CB] FFS: discuss details (to be discussed after type A is stable FFS: Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources |  |  |  |  |  |  |  |  | [Candidate component values: {CB, non-CB, both}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

[Similar to what was agreed for FG 23-3-1 please indicate which additional FGs 23-3-1-1a/b/c/d/e/f/g are needed for FG 23-3-1-1]

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Samsung | Similar with FG 23-3-1, we think all FGs 23-3-1-1a/b/c/d/e/f/g are needed for FG 23-3-1-1. And modification of 23-3-1a is required which is appropriate for PUSCH repetition type B: 23. NR_FeMIMO; 23-3-1-1a; Cyclic mapping; Support of cyclic mapping when the number of nominal repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type AB; Optional with capability signalling. Additionally, for basic FG of 23-3-1 (and 23-3-1-1 also), we think that the support of up to two SRS resource sets (usage = codebook or nonCodebook) should be included in FFS component based on the following agreements. Agreement (in RAN1#103-e) For single DCI based M-TRP PUSCH repetition schemes, support codebook based PUSCH transmission with following enhancements. - Support the indication of two SRIs. - Alt1: Bit field of SRI shall be enhanced. - Alt2: No changes on SRI field - Support the indication of two TPMIs. - The same number of layers are applied for both TPMIs if two TPMIs are indicated - The number of SRS ports between two TRPs should be same. - FFS: Details on indicating two TPMIs (e.g, one TPMI field or two TPMI fields) - Increase the maximum number of SRS resource sets to two - FFS: configuration details of each SRS resource set (e.g., number of SRS resources in a resource set) Agreement (in RAN1#103-e) For single DCI based M-TRP PUSCH repetition schemes, support non-codebook based PUSCH transmission with following considerations. - Increase the maximum number of SRS resource sets to two, and associated CSI-RS resource can be configured per SRS resource set. - FFS: Enhancements on SRI field in DCI to indicate the two beams for repetitions Therefore, we suggest: 23. NR_FeMIMO; 23-3-1; Multi-TRP PUSCH repetition (type A)[-CB]; 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) [for CB] - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] [2. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A] [3. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2] [4. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) ] [5. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.) ] [6. Support of sequential mapping for single for single DCI based M-TRP PUSCH repetition Type A] [7. The maximum number of PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission)] [8. Support of A-CSI report on two PUSCH repetitions] [9. Support of SP-CSI report on two PUSCH repetitions] [2. Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission] FFS: [11. Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources and the maximum number of SRS resource sets]; [Candidate component values: {CB, non-CB, both}]; Optional with capability signalling. |
| ZTE | Based on the agreements endorsed in RAN1, we think all the additional FG 23-3-1a/b/c/e/f/g are needed for FG23-3-1-1. BTW, please note that there is no FG 23-1-1d for FG 23-3-1 so far. |
| Apple | Some minor editorial comment for FG23-3-1-1 - For the component description, we can also copy the following to mirror FG23-3-1 - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] In our view, FGs 23-3-1-1a/b/c/e/f/g should be separate FGs in addition to 23-3-1-1, and we support all the FGs. |
| Ericsson | We don’t see the need to duplicate 23-3-1a/b/c/e/f/g for Multi-TRP PUSCH repetition (type B). The already agreed 23-3-1a/b/c/e/f/g can be shared between 23-3-1 and 23-3-1-1. So, our suggestion is to not agree duplicate rows and instead agree that 23-3-1a/b/c/e/f/g can be shared between 23-3-1 and 23-3-1-1. |
| Qualcomm | From our point of view, none of them are needed for FG 23-3-1-1 (i.e., FGs 23-3-1-1a/b/c/d/e/f/g can be applicable regardless of Repetition Type A or Type B). We can be open if other companies see a need. What is more important to be discussed is the CB versus NCB differentiation and associated parameters, which has a clrear impact on UE complexity. |
| LG | Additional FGs 23-3-1-1a/b/c/d/e/f/g are needed in the same ways as FG 23-3-1. |
| NTT Docomo | We share similar view with Ericsson/QC that none of them is needed. 23-3-1a/b/c/e/f/g can be shared by repetition Type A and Type B. |
| MediaTek | We also agree that FGs 23-3-1-1a/b/c/e/f/g can be shared between repetition type A and B, so no need for replicating for each repetition type. Seems to be a typo where 23-3-1d is missed from the list (going from 23-3-1c directly to 23-3-1e), the numbering needs to be updated. More discussions are needed to capture NCB parameters. |
| Xiaomi | On component 2 in FG 23-3-1, from our perspective, this component indicates that the dynamic switching between S-TRP and M-TRP is supported or not. If the UE cannot support this component, then M-TRP transmission is always scheduled. So, we think component 2 should be basic FG and suggest to remove this component. On FG 23-3-1-1, we suggest to add FGs 23-3-1-1a/b/c/d/e/f/g corresponding to PUSCH repetition type A. |
| CATT | Similar view as Ericsson that the agreed 23-3-1a/b/c/e/f/g can be shared between 23-3-1 and 23-3-1-1. |
| Lenovo/MotM | We think the additional FG 23-3-1a/b/c/e/f/g are needed for FG23-3-1-1 based on the RAN1 agreement. |
| vivo | We are open to the applicable scope of FG 23-3-1a/b/c/e/f/g. |

# Summary of Final Proposals for Agreements

This Section summarizes the final proposals for agreement in RAN1 #107bis-e by email. There are no tables for comments.

[All comments must be directly made on the RAN1 email reflector]

Companies can continue to update their comments in the previous Sections, however, these are no longer monitored by the moderator. Any such comments will be for archival purposes only and will not influence the outcome of this email discussion. Any objection to any of the proposals in this Section must be voiced directly on the RAN1 email reflector.

Proposed Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-2-2 | Two QCL TypeD for [CORESET monitoring in] PDCCH [repetition] | Support of determining two QCL-TypeD for time-domain overlapping in time CORESETs in the same CC or for intra-band CA [when UE is configured with PDCCH repetition] [with [SFN scheme or] non-SFN TDM and/or FDM sheme] | [23-2-1, 23-6-1, 23-6-2] |  |  |  | Per band |  |  | FR2 only |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-8-1 | SRS triggering offset enhancement | 1. [Support of /The maximum number of configured available slots offsets for] determining aperiodic SRS location based on available slot [and dynamically based on Rel-17 introduced new field in DCI Format 0_1/0_2/1_1/1_2] [2. Maximum actual slots offset] | 2-52 |  |  |  | Per FS band |  |  |  | Candidate 1 component values: {none, 1, 2, [3,] 4} [Candidate 2 component values: FFS] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-8-2 | Triggering SRS only in DCI 0_1/0_2 | Support of triggering SRS in DCI 0_1/0_2 without data and without CSI | 2-52 |  |  |  | [Per FS UE or per band] |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-8-3 | SRS Antenna switching for >4Rx | 1. Support of SRS antenna switching xTyR with y>4 2. Report whether the antenna switching impact to downlink receiving in a band 3. Report whether the antenna is switched together with UL Tx in another band | 2-55 |  |  |  | [Per BC] |  |  |  | Component 1 candidate values: FFS [one or more elements from set {t1r1, t2r2, t1r2, t4r4, t2r4, t1r4, t2r6, t1r6, t4r8, t2r8, t1r8}] Component 2 candidate values: {1 to 32} FFS Component 3 candidate values: {1 to 32} FFS FFS for component 2&3: impact when UE reports component 1 as xTyR with x=y | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-8-4 | Maximum 2 SP and 1 periodic SRS sets for antenna switching | Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for antenna switching | 2-53 |  |  |  | [Per FS band] |  |  |  | Note1: - Applies for all supported xTyR where y<=8 - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - For xTyR where y<=4, if UE does not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS - The two SP-SRS resource sets are not activated at the same time | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-8-5 | Increased repetition for SRS | Support of increased repetition patterns (8, 10, 12, 14 symbols) for SRS resource | 10-11, 2-52 |  |  |  | Per FS band |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-8-6 | Partial frequency sounding of SRS | Support of partial frequency sounding for SRS | 2-52 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-8-7 | Start RB location hopping for partial frequency SRS | Support of start RB location hopping in partial frequency SRS transmission [across different SRS frequency hopping periods for periodic/semi-persistent/aperiodic SRS] | 23-8-6 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-8-8 | Comb-8 SRS | Support of comb-8 for SRS other than for positioning |  |  |  |  | Per FS band |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-9-3 | Support of rank 3, 4 for FeType-II | Support of rank 3, 4 for FeType-II FFS separate capabilities for M = 1 and M = 2 | 23-9-1[, 23-9-2] |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-9-5 | Active CSI-RS resources and ports for mixed codebook types in any slot | - List of codebook combinations - List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination | [23-9-1], 16-3a, 16-3b, 16-3a-1, 16-3b-1, 2-36, 2-40, 2-41, 2-43 |  |  |  | FFS Per band and per BC |  |  |  | [Component 1 candidate values: Codebook 1 = {Type I SP, Type I MP} {Codebook 2, Codebook 3} = {{FeType II PS M=1, NULL}, {FeType II PS M=2 R=1, NULL}[, {FeType II PS M=2 R=2, NULL},] {Type II, FeType II PS M=1}, {Type II, FeType II PS M=2 R=1} ,{eType II R=1, FeType II PS M=1},{eType II R=1, FeType II PS M=2 R=1}} FFS: The list of {Codebook 2, Codebook 3} may be refined depending on outcome of discussion for FG 23-9-2 Component 1 and FG 23-9-4 Component 2 Component 2 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}] Note 1：if a UE reports one or more codebook combinations in [23-9-5], then usage of active CSI-RS resources and ports for multiple codebooks in any slot is allowed only within those combinations [Note 2: For coexisting of mixed codebooks in any slot, gNB need to honor 16-8, 23-10 23-9-5and per-codebook capability 2-36/40/41/43, 16-3a/b and 16-3a-1/16-3b-1, and 23-9-1/23-9-2/23-9-4] [Note: If the list of triplets for a codebook combination is empty then the list of triplets is reused from the codebook combination indicated previously] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# Conclusion

After further discussion on the RAN1 email reflector and during several GTW sessions [22] the following was agreed as part of this email discussion:

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-2-2 | Two QCL TypeD for [CORESET monitoring in] PDCCH [repetition] | Support of determining two QCL-TypeD for time-domain overlapping in time CORESETs in the same CC or for intra-band CA [when UE is configured with PDCCH repetition] [with [SFN scheme or] non-SFN TDM and/or FDM sheme] | [23-2-1, 23-6-1, 23-6-2] |  |  |  | Per band |  |  | FR2 only |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-8-1 | SRS triggering offset enhancement | 1. [Support of /The maximum number of configured available slots offsets for] determining aperiodic SRS location based on available slot [and dynamically based on Rel-17 introduced new field in DCI Format 0_1/0_2/1_1/1_2] [2. Maximum actual slots offset] | 2-52 |  |  |  | Per FS band |  |  |  | Candidate 1 component values: {none, 1, 2, [3,] 4} [Candidate 2 component values: FFS] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-8-2 | Triggering SRS only in DCI 0_1/0_2 | Support of triggering SRS in DCI 0_1/0_2 without data and without CSI | 2-52 |  |  |  | [Per FS UE or per band] |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-8-3 | SRS Antenna switching for >4Rx | 1. Support of SRS antenna switching xTyR with y>4 2. Report whether the antenna switching impact to downlink receiving in a band 3. Report whether the antenna is switched together with UL Tx in another band | 2-55 |  |  |  | [Per BC] |  |  |  | Component 1 candidate values: FFS [one or more elements from set {t1r1, t2r2, t1r2, t4r4, t2r4, t1r4, t2r6, t1r6, t4r8, t2r8, t1r8}] Component 2 candidate values: {1 to 32} FFS Component 3 candidate values: {1 to 32} FFS FFS for component 2&3: impact when UE reports component 1 as xTyR with x=y | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-8-4 | Maximum 2 SP and 1 periodic SRS sets for antenna switching | Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for antenna switching | 2-53 |  |  |  | [Per FS band] |  |  |  | Note1: - Applies for all supported xTyR where y<=8 - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - For xTyR where y<=4, if UE does not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS - The two SP-SRS resource sets are not activated at the same time | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-8-5 | Increased repetition for SRS | Support of increased repetition patterns (8, 10, 12, 14 symbols) for SRS resource | 10-11, 2-52 |  |  |  | Per FS band |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-8-6 | Partial frequency sounding of SRS | Support of partial frequency sounding for SRS | 2-52 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-8-7 | Start RB location hopping for partial frequency SRS | Support of start RB location hopping in partial frequency SRS transmission [across different SRS frequency hopping periods for periodic/semi-persistent/aperiodic SRS] | 23-8-6 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-8-8 | Comb-8 SRS | Support of comb-8 for SRS other than for positioning |  |  |  |  | Per FS band |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-9-3 | Support of rank 3, 4 for FeType-II | Support of rank 3, 4 for FeType-II FFS separate capabilities for M = 1 and M = 2 | 23-9-1[, 23-9-2] |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-9-5 | Active CSI-RS resources and ports for mixed codebook types in any slot | - List of codebook combinations - List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination | [23-9-1], 16-3a, 16-3b, 16-3a-1, 16-3b-1, 2-36, 2-40, 2-41, 2-43 |  |  |  | FFS Per band and per BC |  |  |  | [Component 1 candidate values: Codebook 1 = {Type I SP, Type I MP} {Codebook 2, Codebook 3} = {{FeType II PS M=1, NULL}, {FeType II PS M=2 R=1, NULL}[, {FeType II PS M=2 R=2, NULL},] {Type II, FeType II PS M=1}, {Type II, FeType II PS M=2 R=1} ,{eType II R=1, FeType II PS M=1},{eType II R=1, FeType II PS M=2 R=1}} FFS: The list of {Codebook 2, Codebook 3} may be refined depending on outcome of discussion for FG 23-9-2 Component 1 and FG 23-9-4 Component 2 Component 2 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}] Note 1：if a UE reports one or more codebook combinations in [23-9-5], then usage of active CSI-RS resources and ports for multiple codebooks in any slot is allowed only within those combinations [Note 2: For coexisting of mixed codebooks in any slot, gNB need to honor 16-8, 23-10 23-9-5and per-codebook capability 2-36/40/41/43, 16-3a/b and 16-3a-1/16-3b-1, and 23-9-1/23-9-2/23-9-4] [Note: If the list of triplets for a codebook combination is empty then the list of triplets is reused from the codebook combination indicated previously] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in green, while keeping the yellow highlighting as shown

- The following are the components of the feature “joint TCI for intra-cell operation”

- Joint DL/UL TCI update with their components: (configuration mechanism, [QCL rules,] applicable source and target signals, MAC-CE -based TCI] activation)

- Common multi-CC TCI update and activation (involving RRC common TCI state pool)

- For PUCCH, PUSCH, and SRS, association between TCI state and UL PC settings except for PL RS [for PUCCH, PUSCH, and SRS]

- Beam misalignment between the DL source RS in the TCI state to provide spatial relation indication and the PL-RS

- TCI state indication [mode]: update and activation [in case of updates]
a) MAC CE based TCI state indication [for one active TCI state]
b) [MAC-CE+DCI-based TCI state indication (including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)]
c) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment)

- [TCI states pool for configured reference BWP/CC shared by a set of BWP/CC] FFS: (involving RRC common TCI state pool)

- [Maximum number of CCs configured with BFR]

- [R17 mechanism reusing R15/16 signalling to indicate R17 TCI for individual DL channel/RS that cannot share the same unified TCI as UE-dedicated PDCCH/PDSCH]

- [Configuration of both R17 TCI and R15/16 TCI and spatial relation]

- The maximum number of configured joint TCI state pools across all BWPs and all CCs in a band [in a band combination]

- [The maximum number of configured joint TCI states across all BWPs and all CCs in a band] [in a band combination]

- [The maximum number of configured joint TCI states per CC [in a band]] [in a band combination]

- [The maximum number of MAC-CE activated joint TCI states [per BWP per CC/across all BWPs and all CCs in a band]] in a band combination
a) The maximum number of MAC-CE activated joint TCI states per CC in a band in a band combination

- [Whether a particular DL RS that is a valid target DL RS of a Rel-15/16 TCI state based on the Rel-15/16 QCL rules can be configured as a target DL RS of Rel-17 DL TCI (hence the Rel-17 DL TCI state pool)]

- [The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS]

- [The minimum time gap between the beam indication PDCCH and first slot where beam is applied]

- This list gives no guidance as to how to group these components into FGs. They may be mapped to one or more FGs and one or more of these FGs may be agreed as basic. Specifically, different FGs can be grouped together through notes “a UE supporting this FG must also support FG 23-x-y”. This allows to create “basic” FGs while allowing, e.g., different types for different components/candidate values.

Agreement: Adopt the following changes highlighted in green, while keeping the yellow highlighting as shown

- The following are the components of the feature “joint TCI for intra-cell operation”

- Joint DL/UL TCI update with their components: (configuration mechanism, [QCL rules,] applicable source and target signals, MAC-CE -based TCI] activation)

- Common multi-CC TCI update and activation (involving RRC common TCI state pool)

- For PUCCH, PUSCH, and SRS, association between TCI state and UL PC settings except for PL RS [for PUCCH, PUSCH, and SRS]

- Beam misalignment between the DL source RS in the TCI state to provide spatial relation indication and the PL-RS

- TCI state indication [mode]: update and activation [in case of updates]
a) MAC CE based TCI state indication [for one active TCI state]
b) [MAC-CE+DCI-based TCI state indication (including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)]
c) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment)

- [TCI states pool for configured Reference BWP/CC configured with reference TCI state pool shared by a set of BWP/CC]
Note: agree component, final wording may change (e.g., when this is merged with other components/FGs)

- [Maximum number of CCs configured with BFR] 
FFS whether this is a component or just a note in the FG to reuse R16 signaling

- [R17 mechanism reusing R15/16 signalling to indicate R17 TCI for individual DL channel/RS that cannot share the same unified TCI as UE-dedicated PDCCH/PDSCH]

- [Configuration of both R17 TCI and R15/16 TCI and spatial relation]

- The maximum number of configured joint TCI state pools across all BWPs and all CCs in a band [in a band combination]
FFS: Whether to make component 6 a prerequisite or merge with 6

- [The maximum number of configured joint TCI states across all BWPs and all CCs in a band] [in a band combination]

- [The maximum number of configured joint TCI states [per BWP per CC] [in a band]] [in a band combination]

- [The maximum number of MAC-CE activated joint TCI states [per BWP per CC/across all BWPs and all CCs [in a band]]] [in a band combination]
a) The maximum number of MAC-CE activated joint TCI states per CC [in a band] [in a band combination]

- [Whether a particular DL RS that is a valid target DL RS of a Rel-15/16 TCI state based on the Rel-15/16 QCL rules can be configured as a target DL RS of Rel-17 DL TCI (hence the Rel-17 DL TCI state pool)]

- [The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS]

- [The minimum time gap between the beam indication PDCCH and first slot where beam is applied]

- This list gives no guidance as to how to group these components into FGs. They may be mapped to one or more FGs and one or more of these FGs may be agreed as basic. Specifically, different FGs can be grouped together through notes “a UE supporting this FG must also support FG 23-x-y”. This allows to create “basic” FGs while allowing, e.g., different types for different components/candidate values.

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-2-1 | [Multi-TRP] PDCCH repetition | 1. Support of intra-slot PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) [with [non-SFN scheme] TDM and FDM] [(except FR2)] [including PDCCH repetition for Type 3 CSS] 2. Support of reporting one [or more] number(s) as required number of BDs for the two PDCCH candidates 3. [If 3 or {2, 3} is reported in component 2,] support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET [for the case that the linked PDCCH candidates is 3 BDs] [If 2 or {2,3} is reported in component 2, the individual candidate is monitored when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 2 BDs.] FFS: 43. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier |  |  |  |  |  |  |  |  | Component 1 details FFS Component 2 candidate values: details FFS 2 or 3 Component 3 candidate values: details FFS {monitored, not monitored} Component 4 candidate values: [{0,1,2,3}] details FFS [Note: UE supports PDCCH repetition for the following (basic) PDCCH monitoring capability: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot] Note: for component4, if N PDCCH candidates are overlapped, the number of overlaps is counted as one. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-2-1a | Monitoring of individual candidates | Support of monitoring of individual candidates when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET | [23-2-1] |  |  |  |  |  |  |  | [Note: [If 2 is reported in component 2 of FG 23-2-1,] the individual candidate is monitored when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 2 BDs. If 3 is reported in component 2, support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET] | Optional with capability signalling |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-3-1 | Multi-TRP PUSCH repetition (type A)[-CB] | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) [for CB] - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] [2. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A] [3. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2] [4. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) ] [5. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.) ] [6. support of sequential mapping for single for single DCI based M-TRP PUSCH repetition Type A] [7. The maximum number of PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission)] [8. Support of A-CSI report on two PUSCH repetitions] [9. Support of SP-CSI report on two PUSCH repetitions] [2. Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission] FFS: [11. Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources] |  |  |  |  |  |  |  |  | [Candidate component values: {CB, non-CB, both}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-3-1a | Cyclic mapping | Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1b | Second TPC field | Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1c | Two PHR reporting | Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1e | A-CSI report | Support of A-CSI report on two PUSCH repetitions |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1f | SP-CSI report | Support of SP-CSI report on two PUSCH repetitions |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1g | CG PUSCH transmission | Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.) |  |  |  |  |  |  |  |  |  | Optional with capability signalling |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-3-1-1 | Multi-TRP PUSCH repetition (type B) [-CB] | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) [for CB] FFS: discuss details (to be discussed after type A is stable FFS: Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources |  |  |  |  |  |  |  |  | [Candidate component values: {CB, non-CB, both}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement:  Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-5-1 | Group based L1-RSRP reporting enhancements | 1. Max number N of beam groups (M=2 beams per beam group) in Support a single L1-RSRP reporting instance consisting of N beam groups and M=2 beams per [pair/group] [in one report instance] based on measurement on two CMR resource sets 2. [Support of enhanced group-based reporting for Rel-17 M-TRP] [23. Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs] [34. Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs] 5. [Support beam measurement on two CMR resource sets] 6. [Support of enhanced group-based reporting for Rel-17 intra-cell and inter-cell mTRP] |  |  |  |  |  |  |  |  | Component 1 candidate values: {1,2,3,4} Component 2 candidate values: FFS Component 3 candidate values: FFS FFS: If FG 23-5-a is not introduced, the relationship of this FG with FG 16-1g/16-1g-1 needs to be further clarified | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-5-1a | Resources [for beam management , PL measurement, BFD, RLM, and new beam identification] | Note: Strive to align the final implementation of FG 23-5-1a with related R15/16 implementations If 23-5-1a ends up being identical to one or more of Rel.15/ 16 FGs this row will be deleted |  |  |  |  |  |  |  |  | Component 1 candidate values: FFS | Optional with capability signalling |

Agreement:  Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-5-2 | MTRP BFR enhancements | 1. Support of the maximum number of BFD-RS resources per set [2. Support of Rel-17 M-TRP BFR based on two BFD-RS sets] 3. [Maximum number of CSI-RS resources of both BFR-RS sets across all CCs] 4. [Maximal number of different SSBs of both BFD-RS sets across all CCs] 5. [Maximal number of different CSI-RS and/or SSB resources of both NBI-RS sets across all CCs] [3. [Support of one up to two PUCCH-SR resources for MTRP BFRQ] 47. [Supported maximum number of BFD-RS resources across two BFD-RS sets per BWP] 8. [Support of [single DCI/multi-DCI] based BFR enhancement] |  |  |  |  |  |  |  |  | [Candidate values: {1, 2,…}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-5-2a | PUCCH-SR resources for MTRP BFRQ | 1. Max number of PUCCH-SR resources for MTRP BFRQ [2. Association between BFD-RS resource set on sPCell and a PUCCH SR resource (if component candidate value equals 2)] |  |  |  |  |  |  |  |  | Component candidate values: {[0,1], 2} | Optional with capability signalling |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-7-1 | Basic Features of CSI Enhancement for Multi-TRP | - Support Maximum number of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Support of Nmax=1 - FFS: Maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks,max - CSI report mode [selection] of mode 1 with X=0 [and/or] mode 2 - Maximum value of numberOfSingleTRP-CSI-Mode1 - A list of [supported combinations, up to 16, across all CCs simultaneously, where each combination is] - [Maximum number of Tx ports in one NZP CSI-RS resource associated with a single-TRP measurement hypothesis] - Maximum number of Tx ports in one NZP CSI-RS resource associated with an NCJT measurement hypothesis - [Maximum total number of CMRs for calculated single-TRP measurement hypotheses] [per CC/across all CCs] - Maximum total number of CMRs for calculated NCJT measurement hypotheses [per CC/across all CCs] - [Maximum total number of Tx ports of NZP CSI-RS resources associated with single-TRP measurement hypotheses] [per CC/across all CCs] - Maximum total number of Tx ports of NZP CSI-RS resources associated with NCJT measurement hypotheses [per CC/across all CCs] - [Maximum total number of Tx ports of NZP CSI-RS resources associated with one NCJT measurement] - [A list of (Y1,Y2): UE can process Y1 NCJT CSI report(s) and Y2 sTRP CSI measurement hypothesis report(s) simultaneously in a CC] - [A list of (X1,X2): UE can process X1 NCJT CSI report(s) and X2 sTRP CSI measurement hypothesis report(s) simultaneously across all CCs] FFS others (supported options; values for X, codebook types, number of ports of CMRs, CMR sharing among NCJT and sTRP measurement hypotheses for FR1, two CMR groups with Ks=K1+K2 CMRs in CSI-RS resource set, reporting of 2 PMIs, 2 RIs and 2 Lis for NCJT measurement hypothesis …) | FFS |  |  |  | [Per band and/ per BC] |  |  |  | [Component 2 candidate value set: {[0, 2, 3,] 4, 5, 6, 7, 8}] [Component 3 candidate value set: { mode 1 with X=0, mode 2, both] Component 4 candidate values: {0, 1, 2} Component 5 candidate values: - [{2, 4, 8, 12, 16, 24, 32}] - {2, 4, 8, 12, 16[, 24, 32]} - [{1,2,3,4 … 64}] - {1,2,3,4 … 164} - [{4,5,6, …, 256}] - {2,3,4, …, 256} [Component 6: The list can have maximum of 16 pairs. - Y1: {1 to 4} - Y2: {1 to 8}] [Component 7: The list can have maximum of 16 pairs. - X1: {1 to 16} - X2: {1 to 32}] Note: ‘NCJT’ is not used in RAN1 specifications and will be aligned with 38.214 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-7-1a | Additional CSI report mode 1 selection | Maximum value of numberOfSingleTRP-CSI-Mode1 |  |  |  |  |  |  |  |  | Component 1 candidate value set: { mode 1 with X=1, mode 1 with X=2} | Optional with capability signalling |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-4 | IntCell-mTRP | 1. Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info [2. The maximum number of configured additional PCIs is X1 when each configuration of SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI] [3. The maximum number of configured additional PCIs is X2 when the configurations of SSB time domain positions and periodicity of the additional PCIs is different with SSB time domain positions and periodicity of the serving cell PCI] [2. Support of X>1 (max number of PCIs different from serving cell)] [3. Supported max number of RRC-configured PCIs different from serving cell PCI for [FR1/case1] (X1)] [SSB time domain positions or periodicity of additional PCIs is not exactly the same as serving cell PCI)] [4. Supported max number of RRC-configured PCIs different from serving cell PCI for [FR2/case2] (X2)] [SSB time domain positions and periodicity are exactly the same among the additional PCIs and the same as serving cell PCI] [5. default case to be supported, e.g., case2 with X2=1] |  |  |  |  |  |  |  |  | [Component 2 candidate values: {1,2,3,7}] [Component 3 candidate values: {0,1,2,3,7}] [Note: case1 and case2 cannot be enabled simultaneously] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-6-1 | SFN scheme A (scheme 1) for PDSCH and PDCCH | 1. [Support of SFN scheme A for PDCCH] scheduling [single TRP/] SFN Scheme A PDSCH and [and default QCL assumption with one or two TCI states for PDCCH] 2. Support of SFN scheme A for PDSCH [only scheduled by [single TRP/ Scheme A] PDCCH] [and default QCL assumption with one or two TCI states for PDSCH] |  |  |  |  | [Per band or per FS or FSPC] |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-6-1a | Dynamic switching - scheme A | Support of dynamic switching between single-TRP and SFN PDSCH SFN scheme A by TCI state field in DCI formats 1_1, 1_2 | [23-6-1[b]] |  |  |  | [Per band or per FS or FSPC] |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-1b | SFN scheme A (scheme 1) for PDSCH only | 1. Support of SFN scheme A for PDSCH [only scheduled by [single TRP/ Scheme A] PDCCH] [and default QCL assumption with one or two TCI states for PDSCH] |  |  |  |  | [Per band or per FS or FSPC] |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-2 | SFN scheme B (TRP based pre-compensation) for PDSCH and PDCCH | 1. [Support of SFN scheme B for PDCCH] scheduling SFN Scheme B PDSCH [and default QCL assumption with one or two TCI states for PDCCH] 2. Support of SFN scheme B for PDSCH only [and default QCL assumption with two TCI states for PDSCH] [scheduled by [single TRP/Scheme B] PDCCH] | [23-6-1] |  |  |  | [Per band or per FS or per FSPC] |  | [FR1 only] |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-2a | Dynamic switching - scheme B | Support of dynamic switching between single-TRP and SFN PDSCH SFN scheme B by TCI state field in DCI formats 1_1, 1_2 | [23-6-2[b]] |  |  |  | [Per band or per FS or FSPC] |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-2b | SFN scheme B (TRP based pre-compensation) for PDSCH only | 1. Support of SFN scheme B for PDSCH [only and default QCL assumption with two TCI states for PDSCH] [scheduled by [single TRP/Scheme B] PDCCH] | [23-6-1] |  |  |  | [Per band or per FS or per FSPC] |  |  |  |  | Optional with capability signalling |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-6-4 | Default DL beam setup [for SFN HST] | - Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold - FFS: Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI [when PDSCH is scheduled with offset equal or larger than the threshold] - Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold FFS: 4. Support of single-TRP PUCCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured FFS: 5. Support of single-TRP PUSCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured FFS: 6. Support of single-TRP SRS resource transmission using default beam when enhanced SFN PDCCH transmission scheme is configured | [23-6-1, 23-6-2] |  |  |  | Per band |  | FR2 only |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-6-4a | Default UL beam setup for SFN | 1. Support of single-TRP PUCCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 2. Support of single-TRP PUSCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 3. Support of single-TRP SRS resource transmission using default beam when enhanced SFN PDCCH transmission scheme is configured | [23-6-1, 23-6-2] |  |  |  | Per band |  | FR2 only |  |  | Optional with capability signalling |

Agreement: Replace the existing row for FG 23-1-1 with the following row

| 23. NR_FeMIMO | 23-1-1 | Unified TCI [with joint DL/UL TCI update] for [intra- [and inter-cell] beam management | - Joint DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals) FFS: whether to include the following components 2-14 into this FG or one or more separate FGs FFS: Whether basic FGs are defined, and if so, which components are basic FGs, i.e., a UE that supports FG 23-1-1 must also support said basic FGs FFS: basic FGs for UEs supporting CA FFS: separate FGs for inter/intra/joint/separate - Common multi-CC TCI update and activation - For PUCCH, PUSCH, and SRS, association between TCI state and UL PC settings except for PL RS - The maximum number of configured joint TCI states [per BWP per CC] [in a band] [in a band combination] - The maximum number of MAC-CE activated joint TCI states across all CCs [in a band] [in a band combination] a) The maximum number of MAC-CE activated joint TCI states per CC [in a band] [in a band combination] - [The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS] - Beam misalignment between the DL source RS in the TCI state to provide spatial relation indication and the PL-RS - TCI state indication [mode]: update and activation [in case of updates] a) MAC CE based TCI state indication [for one active TCI state] b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) c) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) - Reference BWP/CC configured with reference TCI state pool shared by a set of BWP/CC Note: agree component, final wording may change (e.g., when this is merged with other components/FGs) - Maximum number of CCs configured with BFR FFS whether this is a component or just a note in the FG to reuse R16 signaling - [R17 mechanism reusing R15/16 signalling to indicate Support of indication/configuration of R17 TCI for individual DL channel/RS that cannot share the same unified TCI as UE-dedicated states for aperiodic CSI-RS, PDCCH, /PDSCH] , and SRS reusing the Rel-15/16 signaling/configuration design(s) Note: This has no impact on detail signaling design for SRS TCI indication - The maximum number of configured joint TCI state pools across all BWPs and all CCs in a band [in a band combination] FFS: Whether to make component 69 a prerequisite or merge with 69 - [Alt. 1: [The maximum number of PDSCH-Configs containing configured joint TCI states across all BWPs and all CCs in a band] [in a band combination] that can referred to from a PDSCH-Config without TCI states Alt. 2: Support PDSCH-Config which contains a reference to another CC/BWP, in which the PDSCH-Config contains the TCI state list] - [The minimum time gap between the beam indication PDCCH and first slot where beam is applied] |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-1-4 | MPUE support for UL | [depending on agreement] 1. Supported UE capability value [sets] and corresponding max number of SRS ports for each UE capability value [set] |  |  |  |  | FSPC per band |  |  |  | Candidate values: [Up to [4] value [sets] each with one value of {[0,]1,2,4}] Note: the reported list contains only unique value [sets] This FG is a working assumption | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-3-2 | Multi-TRP PUCCH repetition-inter-slot | 1. Support of PUCCH repetition scheme 1 (inter-slot repetition) 2. Support of PUCCH repetition scheme 3 (intra-slot repetition) - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions ] [2. Support of up to two PUCCH spatial relation per PUCCH resource] 32. Support of cyclic mapping for beam mapping/power control parameter set mapping [when the number of repetitions is larger than 2] for both PUCCH repetitions scheme 1 and 3 when the number of repetitions is larger than 2 43. Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats [1_0 /] 1_1 / 1_2 | FFS |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-3-x | Multi-TRP PUCCH repetition-intra-slot | Support of PUCCH repetition scheme 3 (intra-slot repetition) - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] | FFS |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-2b |  | Support of cyclic mapping for beam mapping/power control parameter set mapping for PUCCH repetitions scheme 1 and/or 3 when the number of repetitions is larger than 2 | FFS |  |  |  |  |  |  |  | FFS: candidate values | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-2c |  | Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats 1_1 / 1_2 | FFS |  |  |  |  |  |  |  |  | Optional with capability signalling |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-9-2 | Support of M=2 and R=1 for FeType-II | [1. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with M=2 and R=1] 2. Support parameter combinations with M=2 [3. Support of DFT FD bases of size N] | 23-9-1 |  |  |  | [per band and per BC] |  |  |  | [Component 1 candidate values - Maximum 16 8 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}] [Component 34 candidate values: {N=2 only, N=2 and N=4}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-9-4 | Support of R = 2 for FeType-II | 1. Support of R = 2 for FeType-II [2. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support to support Port-selection FeType-II with M=2 and R=2] [3. Support parameter combinations with Mv=2 and R=2] | 23-9-2 |  |  |  | [per band and per BC] |  |  |  | Component 2 candidate values: • Maximum 16 8 triplets • Max # of Tx ports in one resource: {4,8,12,16,24,32} • Max # resources: [{1 to 64}] • Max # total ports: [{4 to 256}] | Optional with capability signalling |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-9-2 | Support of M=2 and R=1 for FeType-II | [1. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with M=2 and R=1] 2. Support parameter combinations with M=2 [3. Support of DFT FD bases of size N] | 23-9-1 |  |  |  | [per band and per BC] |  |  |  | [Component 1 candidate values - Maximum 16 8 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}] [Component 34 candidate values: {N=2 only, N=2 and N=4}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-9-4 | Support of R = 2 for FeType-II | 1. Support of R = 2 for FeType-II [2. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support to support Port-selection FeType-II with M=2 and R=2] [3. Support parameter combinations with Mv=2 and R=2] | 23-9-2 |  |  |  | [per band and per BC] |  |  |  | Component 2 candidate values: • Maximum 16 8 triplets • Max # of Tx ports in one resource: {4,8,12,16,24,32} • Max # resources: [{1 to 64}] • Max # total ports: [{4 to 256}] | Optional with capability signalling |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-1-2 | Inter-cell beam measurement and reporting [(for inter-cell BM [and mTRP])] | 1. Support of [L1-RSRP/beam] measurement and reporting on SSB(s) with PCI(s) different from serving cell PCI FFS: whether to include the following components 2-17 into this FG or one or more separate FGs FFS: Whether basic FGs are defined, and if so, which components are basic FGs, i.e., a UE that supports FG 23-1-2 must also support said basic FGs FFS: basic FGs for UEs supporting CA FFS: separate FGs for inter/intra/joint/separate [2. Support of [L1-RSRP/beam] reporting of Kmax SSBRI-RSRP[,SS-RSRP] [pairs/beams], with where at least one [pair/beam] associated with a PCI different from serving cell PCI can be reported [in one report instance]] [3. The maximum number of [RRC configured/ MAC-CE activated] PCI(s) different from serving cell PCI for [periodic/ semi-persistent] [L1-RSRP/beam] measurement] [in FR1] [4. The maximum number of [RRC configured/ MAC-CE activated] PCI(s) different from serving cell PCI for [periodic/ semi-persistent] [L1-RSRP/beam] measurement] [in FR2] [5. The max number of SSB resources configured to measure L1-RSRP within a slot across all with PCI(s) same as or different from serving cell PCI[across all CC]] [6. The max number of SSB resources configured to measure L1-RSRP across all with PCI(s) same as or different from serving cell PCI [across all CC]] [7. Support on that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement] [8. The maximum number of configured additional PCIs is X1 when time domain positions and periodicity of configured SSBs with additional PCIs are the same as time domain positions and periodicity of the serving cell SSBs] [9. The maximum number of configured additional PCIs is X2 when time domain positions and periodicity of configured SSBs with additional PCIs are different with time domain positions and periodicity of the serving cell SSBs] [10. Supported mode inter-cell measurement: {inside SMTC, both inside and outside SMTC}] [11. Supported mode of measurement over overlapped SSBs: {overlapped, both overlapped and non-overlapped}] [12. Maximum number of Kmax] [13. Maximum number of overlapped SSBs in one SSB resource for L1-RSRP measurement] [14. The number of RRC-configured PCI(s) different from serving cell PCI for aperiodic beam measurement] [15. The maximum total number of SSB/CSI-RS/CSI-IM resources configured to measure within a slot across all CCs in one frequency range for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, RLM and new beam identification for both intra-cell and inter-cell measurement (Similar to FG 16-1g)] [16. The maximum total number of SSB/CSI-RS/CSI-IM resources configured across all CCs in one frequency range for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, RLM and new beam identification for both intra-cell and inter-cell measurement (Similar to FG 16-1g)] [17. Support of [L1-SINR/beam] measurement and reporting on SSB(s) with PCI(s) different from serving cell PCI] | [2-24, 2-29] |  |  |  | [per band] |  |  |  | [2. Candidate value of {1,2,3,4}] [3. candidate values {1, 2, 4, 6}] [4. candidate values {1, 2, 4}] [5. candidate values: {2, 4, 8, FFS}] [6. candidate values: {4, 8, 16, 32, 64, FFS}] [7. candidate values {not support}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 23. NR_FeMIMO | 23-1-2 | Inter-cell beam measurement and reporting [(for inter-cell BM [and mTRP])] | 1. Support of L1-RSRP measurement and reporting on SSB(s) with PCI(s) different from serving cell PCI FFS: whether to include the following components 2-13 into this FG or one or more separate FGs FFS: Whether basic FGs are defined, and if so, which components are basic FGs, i.e., a UE that supports FG 23-1-2 must also support said basic FGs - Support of up to K[=4] SSBRI-RSRP [pairs/beams] in one report [where at least one [pair/beam] associated with a PCI different from serving cell PCI can be reported] FFS: if K is a component candidate value - The maximum number of [RRC configured] PCI(s) different from serving cell PCI for L1-RSRP measurement FFS: whether to split this for FR1 and FR2 FFS: whether/how to capture different values/behaviors for periodic/aperiodic/semi-persistent L1-RSRP measurement - [The max number of SSB resources configured to measure L1-RSRP within a slot with PCI(s) same as or different from serving cell PCI [across all CC]] - [The max number of SSB resources configured to measure L1-RSRP with PCI(s) same as or different from serving cell PCI [across all CC]] - [Support on that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement] - [The maximum number of configured additional PCIs is X1 when time domain positions and periodicity of configured SSBs with additional PCIs are the same as time domain positions and periodicity of the serving cell SSBs] - [The maximum number of configured additional PCIs is X2 when time domain positions and periodicity of configured SSBs with additional PCIs are different with time domain positions and periodicity of the serving cell SSBs] - [Supported mode inter-cell measurement: {inside SMTC, both inside and outside SMTC}] - [Supported mode of measurement over overlapped SSBs: {overlapped, both overlapped and non-overlapped}] - [Maximum number of overlapped SSBs in one SSB resource for L1-RSRP measurement] - [The maximum total number of SSB/CSI-RS/CSI-IM resources configured to measure within a slot across all CCs in one frequency range for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, RLM and new beam identification for both intra-cell and inter-cell measurement (Similar to FG 16-1g)] - [The maximum total number of SSB/CSI-RS/CSI-IM resources configured across all CCs in one frequency range for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, RLM and new beam identification for both intra-cell and inter-cell measurement (Similar to FG 16-1g)] | [2-24, 2-29] |  |  |  | [per band] |  |  |  | [2. Candidate value of {1,2,3,4}] [3. candidate values {1, 2, 4, 6}] [4. candidate values {1, 2, 4}] [5. candidate values: {2, 4, 8, FFS}] [6. candidate values: {4, 8, 16, 32, 64, FFS}] [7. candidate values {not support}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# References

- R1-2112902, Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e, Moderators (AT&T, NTT DOCOMO, INC.)

- R1-2200031	Rel-17 UE features for further NR MIMO enhancements, Huawei/HiSilicon

- R1-2200057	UE features for feMIMO, ZTE

- R1-2200098	Discussion on Rel-17 MIMO UE features, vivo

- R1-2200136	UE features for Rel-17 FeMIMO, CATT

- R1-2200216	UE features for further enhancements on NR-MIMO, Samsung

- R1-2200246	Discussion on Rel.17 FeMIMO UE features, NTT DOCOMO, INC.

- R1-2200285	Discussion on UE features for FeMIMO, Spreadtrum Communications

- R1-2200311	Discussion on FeMIMO UE features, Qualcomm Incorporated

- R1-2200341	UE features for further enhancements on NR-MIMO, OPPO

- R1-2200389	UE Features for FeMIMO, Intel Corporation

- R1-2200430	Views on Rel-17 FeMIMO UE features, Apple

- R1-2200445	MIMO Rel-17 UE features, FUTUREWEI

- R1-2200454	Discussion on UE features for FeMIMO, xiaomi

- R1-2200493	Discussion on Rel-17 UE feature for NR FeMIMO, LG Electronics

- R1-2200535	Discussion on UE features for FeMIMO, Lenovo/Motorola Mobility

- R1-2200536	Discussion on UE features for FeMIMO, Ericsson

- R1-2200544	UE Features for further enhancements on NR MIMO, MediaTek Inc.

- R1-2200599	Discussion on FeMIMO UE features, CMCC

- R1-2200622	On UE features for further enhancements on NR-MIMO, Nokia/Nokia Shanghai Bell

- R1-2111807, Summary of UE features for further enhancements on NR-MIMO, Moderator (AT&T)

- R1-2200762, Session Notes for Agenda Item 8.15.1, Ad-Hoc Chair (AT&T)