3GPP TSG RAN WG1 #107bis-e		                                                      R1-2200771

e-Meeting, January 17th – 25th, 2022

Source:	Moderator (CATT)

Title:	Summary #6 of email thread [107bis-e-R17-IIoT-URLLC-03]

Agenda Item:	8.3.3

Document for:	Discussion and Decision

- Introduction

This document summarizes the discussions in contributions and during RAN1#107bis-e under the following email thread:

[107bis-e-R17-IIoT-URLLC-03] Email discussion on intra-UE multiplexing/prioritization – Yanping (CATT)

- Focus on simultaneous TX of PUCCH/PUSCH and multiplexing/overlapping resolution procedure for intra-UE multiplexing of UCI of different priorities on PUCCH and PUSCH (Capability 1 only)

- 1st check point: January 20

- Final check point: January 25

- Agreements in previous meetings

  - Rel-17 Intra-UE multiplexing & prioritization framework

RAN1#106-e (Aug. 2021)

Working Assumption For handling overlapping PUCCHs/PUSCHs with different priorities in R17 - Step 1: Resolve overlapping PUCCHs and/or PUSCHs with the same priority - Step 2: Resolve overlapping PUCCHs and/or PUSCHs with different priorities Note: Avoid recursive pseudo-code to implement this procedure Note: It is expected that Rel-15 intra-UE UCI multiplexing timeline will be applicable

RAN1#106bis-e (Oct. 2021)

Agreement The following working assumption is confirmed. For handling overlapping PUCCHs/PUSCHs with different priorities in R17 - Step 1: Resolve overlapping PUCCHs and/or PUSCHs with the same priority - Step 2: Resolve overlapping PUCCHs and/or PUSCHs with different priorities Note: Avoid recursive pseudo-code to implement this procedure Note: It is expected that Rel-15 intra-UE UCI multiplexing timeline will be applicable Agreement For both the subslot-based PUCCH and slot-based PUCCH, if simultaneous PUCCH/PUSCH transmission is not enabled, reuse Rel-16 procedure for Step 1

RAN1#107-e (Nov. 2021)

Agreement For handling overlapping PUCCHs/PUSCHs with different priorities, Step 2 consists of the following sub-steps: - Step 2.1: Resolve collision of LP PUCCHs and HP PUCCHs. - Step 2.2: Resolve collision of PUCCHs and PUSCHs of different priorities. Agreement If multiplexing of PUCCHs and/or PUSCHs with different priorities is enabled by RRC, support both of the following UE capabilities to resolve collision of PUCCHs and/or PUSCHs with different priorities in step 2: - Capability #1: It is not expected that Rel-15 multiplexing timeline is not met for all overlapping channels [FFS the overlapping channels are resultant channels after step 1]. UE performs multiplexing or dropping of PUCCHs and/or PUSCHs with different priorities according to Rel-17 rules. - Dynamic enabling/disabling multiplexing for different priorities is not supported for Capability #1 - (Working assumption) Capability #3: Rel-17 multiplexing for different priorities is dynamically enabled/disabled in step 2. - Dynamic indication of enabling/disabling multiplexing for different priorities can be enabled only if multiplexing of PUCCHs/PUSCHs with different priorities is enabled by RRC configuration. - If dynamic multiplexing for different priorities is indicated as enabled for a PUCCH / PUSCH, the UE performs Rel-17 multiplexing operation using the Rel-15 timeline - The gNB is responsible to ensure that all the DCIs associated with all overlapping channels involved in multiplexing in step 2 meet the Rel-15 timeline for multiplexing. - If dynamic multiplexing for different priorities is indicated as disabled for a PUCCH / PUSCH, the UE does not apply the Rel-17 intra-UE multiplexing - If the UL channel associated with the DCI disabling multiplexing collides with another UL channel of a different priority, UE performs R16 PHY prioritization, using Rel-16 timeline. The gNB is responsible to ensure that the UE meets R16 PHY prioritization timeline. - If the UL channel associated with the DCI disabling multiplexing does not collide with another UL channel of a different priority, UE transmits the UL channel as is. - FFS: whether the UL channel associated with the DCI disabling multiplexing can collide with another UL channel of a same priority. - UE does not expect to receive a dynamic indication resulting in demultiplexing of previously multiplexed PUCCHs/PUSCHs channels after the Rel-15 multiplexing deadline has passed - FFS: UE does not expect to receive a dynamic indication resulting in demultiplexing of previously multiplexed PUCCHs/PUSCHs channels without any associated DCIs - Note: demultiplexing of two previously multiplexed channels means decoupling two channels already multiplexed, dropping one channel, and multiplexing the other channel with another channel(s). The above behaviors of Capability#3 at least apply to resolving collision of two UL channels resulting from Step 1 with different priorities. FFS: more than two UL channels. - FFS whether dynamic indication in multiple DCIs associated with a group of overlapping channels have to be consistent - FFS: Configuration of prioritization / multiplexing of channels without dynamic indication - Note: Capability 3 procedure is a super-set of Capability 1 procedure - FFS: Time unit to apply Rel-15 timeline (e.g. slot based, sub-slot based) - FFS: The set of PUSCH and PUCCH that eligible for Rel-15 multiplexing consideration Note: “collision” refers to overlapping PUCCHs, overlapping PUCCH and PUSCH (excluding PUSCH supporting simultaneous transmission with PUCCH), overlapping PUSCHs on a same cell. Note: “Rel-15 multiplexing timeline” means Rel15 timeline calculation in Rel-16 spec, including all the formula and all the values for the variables Note: “Rel-16 prioritization timeline” means Rel-16 cancellation timeline calculation in Rel-16 spec, including all the formula and all the values for the variables

RAN#94-e (Dec. 2021)

RAN to guide RAN1 to focus on the discussions on Capabilility#1 only in Q1 2022 for Rel-17 intra-UE multiplexing framework.

  - Intra-UE multiplexing enhancements of different priorities (on PUCCH & PUSCH)

RAN1#102-e (Aug. 2020)

Agreements: Support multiplexing for following scenarios in R17: - Multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH in R17. - Multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations). - Multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH. For the above multiplexing scenarios, - FFS conditions, if needed, for the multiplexing, e.g - Whether to support multiplexing between different resources not confined within a sub-slot. - Whether to support multiplexing in case a PUCCH overlaps with more than one PUCCH. - Timeline requirements. - FFS: details, if needed, of the multiplexing scheme, e.g. - How to minimize impact on the latency for high-priority HARQ-ACK. - How to determine the PUCCH resource used for multiplexing (e.g. HP or LP PUCCH resource, or a dedicated PUCCH resource for the multiplexing). - How to multiplex the HARQ-ACK bits (e.g. multiplexing, bundling). - How to encode the UCIs with different priorities (e.g. separate coding vs. joint coding) - How to guarantee the target code rate (e.g. payload control, multiplexing priority, LP HARQ-ACK compression/compaction). - Explicit indication for enabling multiplexing. - Multiplexing rule and order (e.g. HP/LP multiplexing is after resolving collision within the same priority). Agreements: Support multiplexing for following scenarios in R17: - Multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). - Multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only) - Multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. - Multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. For the above multiplexing scenarios, - Support separate configurations of at least beta-offset values (FFS for alpha) for multiplexing with different priority combinations. - FFS for other separate configurations. - FFS: value range of beta-offset (e.g. <1). - FFS the conditions, if needed, for multiplexing, e.g. - FFS: Whether to support multiplexing in case a PUCCH/PUSCH overlaps with more than one PUCCH/PUSCH. - Timeline requirements. - FFS: details, if needed, of the multiplexing scheme, e.g. - How to minimize impact on the latency for high-priority HARQ-ACK. - How to multiplex the HARQ-ACK bits (e.g. multiplexing, bundling)? - How to encode the UCIs with different priorities (e.g. separate coding vs. joint coding). - How to guarantee the target code rate (e.g. payload control, multiplexing priority, LP HARQ-ACK compression/compaction). - Explicit indication for multiplexing. - Multiplexing rule and order (e.g. HP/LP multiplexing is after resolving collision within the same priority). - How to handle multiplexing of UCI of different priorities and CG-UCI in a CG-PUSCH

RAN1#103-e (Oct./Nov. 2020)

Agreements: For multiplexing UCIs of different priorities in a PUCCH in R17, - Support of multiplexing between different resources not confined within a sub-slot if conditions are met - FFS: Details - Support multiplexing in case a PUCCH overlaps with more than one PUCCH if conditions are met - FFS details Agreements: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits are more than 2 bits, down-select from the following options in RAN1#104-e: - Option 1: Support joint coding. - Option 2: Support separate coding. - Option 3: Combination of Option1 and 2. - FFS the details For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is 2 bits, provide design details for decision for the following cases in RAN1#104-e: - Multiplexing on a PUCCH format 0 - Multiplexing on a PUCCH format 1 Agreements: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, support a mechanism for gNB to enable/disable the multiplexing. - FFS the type of the mechanism, e.g. DCI indication and/or RRC configuration - FFS: Interaction between the enable/disable mechanism and other multiplexing conditions - FFS for other types of UCI. Agreements: For HARQ-ACK multiplexing on PUSCH of different priority in R17, support a mechanism for gNB to enable/disable the multiplexing. - FFS the type of the mechanism, e.g. DCI indication and/or RRC configuration, beta_offset=0 - FFS: Interaction between the enable/disable mechanism and other multiplexing conditions - FFS for other types of UCI.

RAN1#104-e (Jan/Feb. 2021)

Agreements: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, - Use a PUCCH resource in the second PUCCH-Config (the PUCCH-config containing the PUCCH resource of the HP HARQ-ACK) at least in case the total number of LP and HP HARQ-ACK bits is more than 2. - FFS: The PUCCH resource is configured dedicated for multiplexing of HP HARQ-ACK and LP HARQ-ACK. - FFS in case the total number of LP and HP HARQ-ACK bits is 2. - FFS details Working assumption: Reuse Rel-15 intra-UE PUCCH/PUSCH multiplexing timeline requirements for Rel-17 intra-UE PUCCH/PUSCH multiplexing with different priorities - FFS whether or not to specify a different behavior than Rel-15 when the timeline requirements are not met Agreements: For multiplexing LP HARQ-ACK in a HP PUSCH, support 0< beta-offset <1. - FFS value(s) - FFS to additionally support beta-offset =0 or a value disabling the multiplexing - Aim to NOT increase the corresponding bitwidth in the DCI (compared to Rel-16) Agreements: When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, further study the following options (proponents are encouraged to provide more details and analysis):  Opt.1: The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource.  Opt.1a: The UE does not transmit negative SR.  Opt.1b: For negative SR, the UE transmit only HARQ-ACK on the HARQ-ACK resource.  Opt.1c: For negative SR, the UE transmits SR and HARQ-ACK on the SR resource  FFS: whether with power boost to transmit multiplexed payload or not.  Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource.  Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource.  Opt.2b: Using 4 CS values as for SR+1-bit HARQ-ACK in Rel-15/16. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit.  Opt.2c: If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.  Opt.3: No enhancement over Rel-16.  Other options not excluded.  FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK? Agreements: When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF1, further study the following options (proponents are encouraged to provide more details and analysis):  Opt.1: The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource.  Opt.1a: The UE does not transmit negative SR.  Opt.1b: For negative SR, the UE transmit only HARQ-ACK on the HARQ-ACK resource.  Opt.1c: For negative SR, the UE transmits SR and HARQ-ACK on the SR resource  FFS: whether with power boost to transmit multiplexed payload or not.  Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource.  Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource.  Opt.2b: Applying QPSK for SR+1-bit HARQ-ACK. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit.  FFS on conditions of multiplexing.  Opt.3: For positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource.  Opt.4: For positive SR, transmit SR on the SR resource and drop HARQ-ACK. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource.  Opt.5: No enhancement over Rel-16.  Other options not excluded.  FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK? Agreements: When a PUCCH carrying HP SR with PF1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, further study the following options (proponents are encouraged to provide more details and analysis):  Opt.1: The SR and HARQ-ACK are multiplexed and transmitted on the SR resource.  Opt.1a: For positive SR, the UE transmits the PUCCH in the resource using PUCCH format 1 for SR. The value of cyclic shift of sequence, i.e., , of this PUCCH format 1 is determined by HARQ-ACK, and the bit, i.e., b(0), of this PUCCH format 1 is determined by SR. For negative SR, the UE transmits only a PUCCH with HARQ-ACK information and drops the PUCCH with negative SR.  Opt.1b: SR and HARQ-ACK are multiplexed and modulated to be transmitted on the SR resource  Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource.  Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource.  Opt.2b: Using 4 CS values as for SR+1-bit HARQ-ACK in Rel-15/16. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit.  Opt.2c: If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.  Opt.2d: HP SR and LP HARQ-ACK are multiplexed by the Rel-15 cyclic shift only if latency requirement for HP SR is met. Otherwise, drop the LP HARQ-ACK and only transmit the HP SR on its resource.  Opt.3: For positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource.  Opt.4: No enhancement over Rel-16.  Other options not excluded.  FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK?

RAN1#104bis-e (April 2021)

Agreements: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is more than 2, support separate coding for the two HARQ-ACKs. - FFS for HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s). - (working assumption) Drop CSI (including part 1 and part2, if exist) if CSI would multiplex on a PUCCH which has HP A/N. - FFS Strive to let HP A/N reuse the encoder, rate matching equation, and RE mapping rules in Rel-15 for A/N+CSI-1. - FFS Strive to let LP A/N reuse the encoder, rate matching equation, and mapping rules in Rel-15 for CSI-2. Agreement: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, support separate coding for the two HARQ-ACKs. - It is understood that it is intended that the number of encoding chains for all UCI multiplexing combinations in Rel-17 should not exceed that in Rel-15/16.

RAN1#105-e (May 2021)

Agreement: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is more than 2, - For HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s), support separate coding. Down-select from the two options: - Option 1: Reuse R15 TS 38.212 Clause 5.3.3.1 for 1-bit. Reuse R15 TS 38.212 Clause 5.3.3.2 for 2-bit. - Option 2: Reuse R15 TS 38.212 Clause 5.3.3.3, i.e., padding to 3 bits and using RM coding. - For HP HARQ-ACK or LP HARQ-ACK >2 bit(s), HP HARQ-ACK and LP HARQ-ACK are separately encoded according to R15 TS 38.212 Clause 5.3.3.3 or Clause 5.3.1. - FFS rate matching equation and RE mapping rules for PF2/3/4. Rel-15 is baseline if available. Agreement: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is 2, treat the two bits as HARQ-ACK bits with High priority. · Rel-15 design (for PF0 and PF1) is baseline. · Note: QC has strong concern on above scheme. The scheme cannot provide unequal error protection between the HP bit and LP bit hence could suffer from performance degradation for the HP bit. QC accept the scheme for the sake of progress in RAN 1 with the concern on the performance reserved.

RAN1#106-e (Aug. 2021)

Agreement For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, - HP A/N reuses rate matching equation, and RE mapping rules in Rel-15 for A/N+CSI-1. - LP A/N reuses rate matching equation, and RE mapping rules in Rel-15 for CSI-2. Above applies at least for PUCCH format 3 and 4. Agreement For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, an additional maxCodeRate for LP HARQ-ACK can be configured in the second PUCCH-Config per PUCCH format. Agreement In NR Rel-17, [at least] 2 new set of beta offset values can be configured to the UE to indicate separate beta_offset values for the following cases: - Multiplexing LP HARQ-ACK on HP PUSCH - Multiplexing HP HARQ-ACK on LP PUSCH Agreement For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, - PUCCH resource set determination is based on: UCI payload size = the number of HP UCI bits + the number of LP UCI bits. - FFS PRB number determination for HP A/N and LP A/N, e.g. based on their coding rates. - FFS the impact to the number of LP UCI bits due to missed DCI and potential solutions - Note: the number of LP UCI bits in the above agreement does may not necessarily mean the actual number of LP UCI bits until the second FFS is resolved

RAN1#106bis-e (Oct. 2021)

Agreement For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, in case the total number of LP and HP HARQ-ACK bits is 2: - Use a PUCCH resource in the second PUCCH-Config (the PUCCH-config containing the PUCCH resource of the HP HARQ-ACK). Agreement For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, if HP HARQ-ACK and LP HARQ-ACK would be transmitted on HP/LP PUSCH without CSI, - HP HARQ-ACK and LP HARQ-ACK are separately encoded according to R15 TS 38.212 Clause 5.3.1 and Clause 5.3.3. - Reuse R15 HARQ-ACK rate matching/puncturing and RE mapping for HP HARQ-ACK in principle. FFS details. - For LP HARQ-ACK, reuse R15 Part 1 CSI rate matching and RE mapping. Agreement For determining the PUCCH resource to carry the multiplexed high-priority and low-priority HARQ-ACKs, - The number of RBs for multiplexing HP HARQ-ACK and LP HARQ-ACK on a PUCCH format 3 is determined as following: - If OHP_UCI+OCRC, HP_UCIrHP_UCI∙Qm+OLP_UCI+OCRC, LP_UCIrLP_UCI∙Qm ≤MRBPUCCH∙Nsc, ctrlRB∙Nsymb-UCIPUCCH, the minimum number of RBs is determined as the number of MRB, minPUCCH, satisfying OHP_UCI+OCRC, HP_UCIrHP_UCI∙Qm+OLP_UCI+OCRC, LP_UCIrLP_UCI∙Qm≤MRB,minPUCCH∙Nsc, ctrlRB∙Nsymb-UCIPUCCH and OHP_UCI+OCRC, HP_UCIrHP_UCI∙Qm+OLP_UCI+OCRC, LP_UCIrLP_UCI∙Qm>MRB,minPUCCH-1∙Nsc, ctrlRB∙Nsymb-UCIPUCCH. - Note: rHP_UCI∙rLP_UCI∙Qm is multiplied at both sides to avoid mismatch between gNB and UE due to floating point operation. Editor to capture as suggested. - Otherwise, - Alt1: the number of RBs is MRBPUCCH. FFS: Whether/How LP HARQ-ACK is dropped. - Alt2: the number of RBs is determined by HP ACK payload size. LP HARQ-ACK is fully dropped. - Other alternatives are not precluded. - r_HP_UCI is maxCodeRate configured for HP bits and r_LP_UCI is maxCodeRate configured for LP bits in the second PUCCH-Config (the PUCCH-config containing the PUCCH resource of the HP HARQ-ACK). - FFS whether more than one maxCodeRate can be configured for one priority. - If MRB, minPUCCH is not equal to according to [4, TS 38.211], MRB, minPUCCH is increased to the nearest allowed value of nrofPRBs for PUCCH-format3 provided by the second PUCCH-Config [12, TS 38.331]. - HP coded bits and LP coded bits are not transmitted using the same RE(s) - FFS for PUCCH format 2.

RAN1#107-e (Nov. 2021)

Agreement For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, if HP HARQ-ACK, LP HARQ-ACK, and LP CSI consisting of two parts would be transmitted on LP PUSCH conveying UL-SCH, - The CSI part 2 is dropped. - Reuse R15 HARQ-ACK rate matching/puncturing and RE mapping for HP HARQ-ACK in principle. FFS details. - Reuse R15 CSI part 1 rate matching and RE mapping for LP HARQ-ACK. - Reuse R15 CSI part 2 rate matching and RE mapping for LP CSI part 1. - FFS for LP CSI consisting of single part. Note: Apple raised concern on CSI being dropped unnecessarily which could cause performance and degrade usefulness of URLLC enhancement. Agreement For determining the PUCCH resource to carry the multiplexed high-priority and low-priority HARQ-ACKs, if OHP_UCI+OCRC, HP_UCIrHP_UCI∙Qm+OLP_UCI+OCRC, LP_UCIrLP_UCI∙Qm >MRBPUCCH∙Nsc, ctrlRB∙Nsymb-UCIPUCCH - The number of RBs is MRBPUCCH. Then follow Rel-15 procedure, i.e., LP HARQ-ACK is mapped to the rest REs after HP HARQ-ACK. Agreement For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, - At least for PUCCH format 3/4, use the HP UCI bit number and HP RE number for ∆TF,b,f,c(i)∆TF, b, f,ci formula selection and calculation - For PUCCH format 1, use the total UCI bit number for ∆TF,b,f,c(i) calculation. - FFS for PUCCH format 2.

  - Simultaneous PUCCH/PUSCH transmission

RAN1#102-e (Aug. 2020)

Agreements: Support simultaneous PUCCH/PUSCH transmissions on different cells at least for inter-band CA. - FFS how to trigger this function. - FFS for intra-band CA.

RAN1#104-e (Jan/Feb. 2021)

Agreements: Per UE with the capability of inter-band CA, simultaneous PUCCH/PUSCH transmission of different PHY priorities over different cells can be RRC configured within the same PUCCH group - FFS: dynamic indication

RAN1#106-e (Aug. 2021)

Conclusion Simultaneous PUCCH/PUSCH transmission on the same cell is not supported in Rel-17.

RAN1#107-e (Nov. 2021)

Conclusion There is no consensus in RAN1 to support simultaneous PUCCH/PUSCH transmission of same priority over different cells in Rel-17. Conclusion There is no consensus in RAN1 to support simultaneous PUCCH/PUSCH transmissions on different cells for intra-band CA in Rel-17.

- Discussion

  - Remaining issues of Rel-17 intra-UE multiplexing

Timeline requirement for step 2

      - 1st/2nd round discussion

In the agreement in RAN1#107-e, the following was agreed for multiplexing timeline in step 2 for Capability #1 with FFS highlighted in yellow. Note the following agreement is not complete and the irrelevant parts are omitted.

Agreement If multiplexing of PUCCHs and/or PUSCHs with different priorities is enabled by RRC, support both of the following UE capabilities to resolve collision of PUCCHs and/or PUSCHs with different priorities in step 2: - Capability #1: It is not expected that Rel-15 multiplexing timeline is not met for all overlapping channels [FFS the overlapping channels are resultant channels after step 1]. UE performs multiplexing or dropping of PUCCHs and/or PUSCHs with different priorities according to Rel-17 rules. - Dynamic enabling/disabling multiplexing for different priorities is not supported for Capability #1 - (Working assumption) Capability #3:… - FFS: Time unit to apply Rel-15 timeline (e.g. slot based, sub-slot based) - FFS: The set of PUSCH and PUCCH that eligible for Rel-15 multiplexing consideration Note: “collision” refers to overlapping PUCCHs, overlapping PUCCH and PUSCH (excluding PUSCH supporting simultaneous transmission with PUCCH), overlapping PUSCHs on a same cell. Note: “Rel-15 multiplexing timeline” means Rel-15 timeline calculation in Rel-16 spec, including all the formula and all the values for the variables Note: “Rel-16 prioritization timeline” means Rel-16 cancellation timeline calculation in Rel-16 spec, including all the formula and all the values for the variables

On whether Rel-15 multiplexing timeline applies to resultant channels after step 1 or all overlapping channels before step 1, the views and the reasons based on the input contributions are summarized as follows.

- Option 1: Rel-15 multiplexing timeline applies to the resultant overlapping channels after step 1.

  - Supported by: Huawei [4], CATT [7], DOCOMO [10], OPPO [14]

  - Reasons:

    - Better scheduling flexibility at gNB side for HP channels in case the resultant channel after step 1 is later in time or the resultant channel after step 1 does not overlap with HP channel [4][7][10]. An example in [10] is shown below.

    - Different from Rel-15 where all overlapped original PUCCH/PUSCHs are multiplexed by a single procedure in Clause 9.2.5 of 38.213, Rel-17 introduces two sequential steps (i.e., Step 1 and Step 2) in which the multiplexing procedures of PUCCH/PUSCHs are individually carried out. In this sense, the Rel-15 timeline should be separately applied to the multiplexing procedures of Step 1 (also separately applied to LP PUCCH/PUSCH multiplexing and HP PUCCH/PUSCH multiplexing in Step 1) and Step 2, respectively, where the input channels before each particular step are separately used as the reference of timeline [4].

- Option 2: Rel-15 multiplexing timeline applies to all overlapping channels before step 1.

  - Supported by: Intel [17]

  - Reason:

    - If UE starts bit preparation for LP UCI carrying in LP PUSCH, e.g., perform rate matching according to the number of REs for LP UCI, determine RE locations for LP UCI mapping and also perform rate matching for PUSCH, at t0, and if HP DCI comes after t0, UE needs to redo the multiplexing for LP UCI as shown below.

The benefit of Option 1 is acknowledged by all the companies above including Intel. For the potential issue raised by Intel, it is only valid if UE prepares LP UCI before t1. But from UE processing time perspective, UE does not need to prepare LP UCI so early and Option 1 still provides sufficient processing time for UE.

In addition, it is proposed by Huawei [4] that time unit to adopt Rel-15 timeline should be based on the time unit of HP channel. For the case shown in the figure below, the DCI and PDSCH associated with HP PUCCH in subslot #2 do not need to meet Rel-15 multiplexing timeline with reference to the start of LP PUCCH or HP PUCCH in subslot #1, which provides better scheduling flexibility. However, note that the proposal may impact other discussions, e.g. multiplexing of LP HARQ-ACK overlapping with HP SR and HP HARQ-ACK where some companies proposed to multiplex LP HARQ-ACK in HP PUCCH with HARQ-ACK, and LP PUSCH overlapping with multiple HP PUCCHs with HARQ-ACK where some company proposed to drop LP PUSCH.

LG [26] proposed to allow the case that Rel-15 multiplexing timeline is not met for the overlapping resultant channels after Step 1. Furthermore, if the inter-priority multiplexing timeline requirements are not met, then the UE would proceed with the multiplexing/transmission only for the HP by dropping the LP. However, moderator’s understanding is that the proposal is not inline with the previous agreement for Capability #1.

Based on the above inputs, companies are invited to provide your views on the following proposal. The intention of the proposal is to define the multiplexing timeline based on resultant channels after step 1 instead of channels before step 1. In addition, the proposal that time unit to adopt Rel-15 timeline is the time unit of HP channel in [4] and the proposal to allow the case that Rel-15 multiplexing timeline is not met for the overlapping resultant channels after Step 1 in [26] are also precluded by the proposal.

Proposed Conclusion 1.1.1:

For the timeline requirement of resolving collision of PUCCHs and/or PUSCHs with different priorities in step 2, it is not expected that Rel-15 multiplexing timeline is not met for all overlapping channels where the overlapping channels are the resultant overlapping channels after step 1.

|  | Company |
| --- | --- |
| Support | Nokia/NSB, ZTE, CATT, DOCOMO Huawei/Hisi, Sharp, New H3C,Samsung, Ericsson, vivo,OPPO, LG (can live) |
| Not support | Intel, Sony, QC, Spreadtrum |

| Company | Comments |  |
| --- | --- | --- |
| Intel | We support using all overlapping channels before step 1 as reference for Rel0-15 timeline, to minimize the impact on Rel-15/16 UE implementation. In Rel-17, yes, same and different priorities are handled in two sequential steps, but we’d like to point out, in Rel-15, PUCCH/PUCCH multiplexing and PUCCH/PUSCH multiplexing are also two sequential steps, while Rel-15 timeline uses the earliest start channel of all overlapped PUCCH and PUSCHs as reference, rather than earliest start channel of resultant PUCCH and PUSCHs as reference. In Rel-17, similar logic as Rel-15 can be reused for two sequential steps (step 1 and step 2). According to the figure shown by FL, if we reuse resultant LP PUCCH as reference for timeline (t1), - At t0 (the deadline with reference to LP PUCCH resource with A/N), UE still has to start to determine LP PUCCH resource carrying both LP A/N and CSI (purple block, outcome from step 1). - At t2, assuming UE has determined PUCCH resource for LP. Since t2 is earlier than t1, UE stops processing and waits until t1. - At t1, UE starts step 2 if UE has detected HP DCI before t1. If no HP DCI has been detected, UE resumes LP PUCCH preparation which was stopped at t2. The stop and wait procedure may have large impact on existing UE implementation, because Rel-15 UE implementation can process PUSCH from t0 without such stop and wait step. We’d like to hear more views on the expected impact on existing UE implementation, if we go with timeline using resultant channel after step 1 as reference. |  |
| ZTE | Fine with the proposal, just additional timeline consideration, for URLLC latency requirement, after the inter-priority multiplexing, the end of multiplexing resource is not expected to be later than the end of high priority channel. |  |
| Sony | The main argument against Capability #2 (RRC configured but UE can also perform Rel-16 prioritisation) was that the UE is not aware of all these processing time for multiplexing/prioritisation. It was further argued that the UE will just do what it was told and very likely the UE will perform multiplexing as soon as it realises there are collisions rather than wait for further collisions. If UE is aware of all these multiplexing/processing timelines, then we do not see a reason why it could not perform Rel-16 prioritisation and Rel-17 multiplexing together (i.e. perform Capability #2). Alternatively, we can define explicit timeline/deadline for the UE which are configurable and so the UE will be explicitly aware of the when it needs to do multiplexing. If we are going this route, we might as well allow the UE to perform Rel-16 prioritisation in addition to Rel-17 multiplexing. |  |
| Huawei/Hisi | From the NW vendor perspective, we think the proposal is beneficial to relax the scheduling restriction to gNB and eventually contributes to better performance, but we would like to hear the voice from UE vendors. |  |
| QC | We understand from NW vendor point of view, this proposal relax scheduler restriction so it is beneficial. However, from UE perspective, this problem creates problem. We fully agree with the analysis provided by Intel on the created problem. In the following example provided by Intel. On high level, this proposal basically prohibits UE to start performance UCI multiplexing right after receiving LP DCIs for the two LP channels. UE has to on hold UCI multiplexing procedure until it receives the later HP DCI, to avoid the situation that UE has to discard the UCI mux result between LP UCI+LP PUSCH and redo the UCI mux with LP UCI+HP UCI+LP PUSCH. Now, the question is: UE does not know there will be a later HP DCI, how long UE suppose to wait? Like Sony mentioned, to solve this issue, RAN1 effectively need to introduce a new functionality to ask UE do timeline check. However, this was already discussed extensively in last meeting and it was excluded already. So, at this late stage of Rel-17, the reasonable route is applying Rel-15 timeline on all overlapping channels including HP and LP and close this issue. |  |
| Spreadtrum | We totally agree with Intel, Sony and QC. This proposal requires UE have two timelines in parallel, which is for same priority and the other for different priority, which can certainly cause the stop, wait and restart operation. Clearly, it leads to totally different and more complex intra-UE multiplexing among the overlapped PUCCH/PUSCH. |  |
| Sharp |  | Since the UCI multiplexing with different priorities is only applied in Step 2. The channel considered should be the resultant channel after Step1. Furthermore, the UE should not undo the multiplexing from Step 1 in Step 2. |
| Apple | We agree with Intel, Sony, QC and Spreadtrum. The timeline requirement should apply to all overlapping channels |  |
| Samsung | UE does not need to redo the multiplexing for LP UCI. There is no need for the UE to prepare the LP REs at a deadline with reference to LP PUCCH and UE cannot always use the deadline with reference to the PUCCH because PUSCH can start before the PUCCH. Instead, the deadline for preparing the UCI REs can be and should be with reference to the PUSCH because the UCI is always transmitted on the PUSCH. At the deadline with reference to the PUSCH, the HP DCI should be decoded as well. |  |
| Ericsson | Since Rel-15 multiplexing procedure (with modification) is applied again in Step 2, only input to Step 2 are considered in Rel-15 timeline calculation, i.e., resultant channels after Step 1. |  |
| LG | We originally proposed to allow the case that Rel-15 multiplexing timeline is not met for the overlapping resultant channels after Step 1, so that UE could proceed the multiplexing/ transmission at least for the HP by dropping the LP, even in case when Rel-15 multiplexing timeline is not met for the overlapping resultant channels after Step 1. But, if it is hard to be accepted by the group, then we can live with the proposed conclusion 1.1.1 provided by FL in the context of relaxing gNB’s scheduling burden. |  |

    - [Open] Resultant PUCCH with UCI of different priorities

      - 1st round discussion

There is a note in previous agreement to avoid recursive pseudo-code to implement two steps for Rel-17 intra-UE multiplexing.

Agreement The following working assumption is confirmed. For handling overlapping PUCCHs/PUSCHs with different priorities in R17 - Step 1: Resolve overlapping PUCCHs and/or PUSCHs with the same priority - Step 2: Resolve overlapping PUCCHs and/or PUSCHs with different priorities Note: Avoid recursive pseudo-code to implement this procedure Note: It is expected that Rel-15 intra-UE UCI multiplexing timeline will be applicable

Since companies have different understandings on the note, whether a resultant PUCCH with HP and LP UCI in step 2 is allowed to be overlapped with a HP PUCCH and/or a HP PUSCH, as shown in the figure below, was discussed in RAN1#107-e meeting. But it was not concluded.

The following alternatives were provided in RAN1#107-e and companies’ views based on the input contributions are as follows.

- Alt. 1: the resultant PUCCH with HP and LP UCI in step 2 is not expected to be overlapped with a HP PUSCH.

- Alt. 2: the resultant PUCCH with HP and LP UCI in step 2 is not expected to be overlapped with a HP PUCCH.

- Supported by: Nokia [3], Huawei [4], vivo [5], Ericsson [19]

- Alt. 3: the resultant PUCCH with HP and LP UCI in step 2 is not expected to be overlapped with a HP PUCCH or a HP PUSCH.

- Supported by: OPPO [14], ETRI [15], Intel [17]

- Alt. 4: the resultant PUCCH with HP and LP UCI in step 2 overlapping with a HP PUCCH or a HP PUSCH is allowed.

- Supported by: CATT [7]

It seems to the moderator that the intentions from companies are different. Proponents of Alt. 2 basically would like to avoid recursive processing in step 2. Given that only PUCCH time domain location may change after multiplexing while PUSCH does not, proponents of Alt. 2 would like to preclude the resultant PUCCH overlapping with another PUCCH only but are fine if the resultant PUCCH with HP and LP UCI overlaps with a HP PUSCH. Note that recursive processing of PUCCH collision handling is allowed in Rel-15/16 and step 1 in Rel-17 intra-UE multiplexing, which is handled by the pseudo code defined in clause 9.2.5 of TS38.213. Huawei [4] pointed out that pseudo code in clause 9.2.5 has to be performed twice in Step 1 for LP PUCCH and HP PUCCH separately, the complexity on processing would be further increased if a third round of such operation is additionally performed in Step 2.1; as a result, there would be a risk that the UE can hardly complete the pseudo code operations for up to three times even within the Rel-15 timeline. More views on the feasibility at UE side would be helpful to make a decision.

Proponents of Alt. 3 think that Alt. 3 is required to achieve the agreed Note “Avoid recursive pseudo-code to implement this procedure”. In addition, if the resultant HP PUCCH can overlap with a HP PUSCH which are not overlapped after step 1, additional rule is needed to select a PUSCH to multiplex UCI, e.g., whether to prioritize HP PUSCH or LP PUSCH, and the relation between PUSCH priority and A-CSI.

Proponent of Alt. 4 thinks that it provides best scheduling flexibility and pseudo code in clause 9.2.5 of TS38.213 can be a unified solution to handle different combinations of overlapping of multiple PUCCHs. In addition, it does not mean going back to step 1 so it does not violate the previous Note in the agreement.

Please take the above arguments into account and provide your views on the following alternatives in the tables.

Proposal 1.2.1:

For resolving collision of PUCCHs and/or PUSCHs with different priorities in step 2, down-select from:

- Alt. 1: a resultant PUCCH with HP and LP UCI is not expected to be overlapped with a HP PUSCH.

- Alt. 2: a resultant PUCCH with HP and LP UCI is not expected to be overlapped with a HP PUCCH.

- Alt. 3: a resultant PUCCH with HP and LP UCI is not expected to be overlapped with a HP PUCCH or a HP PUSCH.

- Alt. 4: a resultant PUCCH with HP and LP UCI overlapping with a HP PUCCH or a HP PUSCH is allowed.

|  | Supporting Company |
| --- | --- |
| Alt. 1 |  |
| Alt. 2 | Nokia/NSB, ZTE, DOCOMO Huawei/Hisi, New H3C, LG (can accept) |
| Alt. 3 | Intel, QC |
| Alt. 4 | Sony, CATT, LG |

| Company | Comments |
| --- | --- |
| Intel | It seems different companies have different understanding of “Avoid recursive pseudo-code to implement this procedure”. For example, Interpretation 1: Not go back to step 1 in the middle or after step 2. Interpretation 2: Not reuse Rel-15 pseudo-code in step 2, e.g., only single check of a pair of LP and HP PUCCH is allowed. Interpretation 3: 1+2 Interpretation 4: … Our understanding is interpretation 1. We’re open for the discussion for interpretation 2, if running Rel-15 pseudo-code in step 2 would lead to very tight processing time, but on top of interpretation 2, we still prefer to avoid unnecessary complexity in step 2.2 caused by moving HP PUCCH to overlap with both LP and HP PUSCH. Therefore, we support Alt 3. |
| Sony | Unclear why this is complicated for UE. If the processing timeline is met, why can’t the UE perform the additional multiplexing. Capability#1 is already restrictive enough, we do not see why we need to further impose restriction to the scenarios that can or cannot be multiplex, which would render the feature useless. |
| CATT | We do not think Alt 4 means going back to step 1 and it provides the best scheduling flexibility. |
| Huawei/Hisi | We prefer Alt.2. The operation of the pseudo code in 9.2.5 has to be performed in Step 1 twice, separately for HP only and LP only. We should avoid introducing the recursive procedure in Step 2 for a third time, as a result of which, the complexity would be triple to R15, and in that sense even the R15 timeline can hardly be met. The gNB can easily configure the HP PUCCH resources for hybrid HP and LP, so that after the one-step multiplexing of HP PUCCH and LP PUCCH, the resultant HP channel will not overlap with another HP PUCCH, and the recursion of the pseudo code is not introduced. For PUSCH, on the other hand, since the PUCCH to PUSCH multiplexing will anyway be needed in Step 2.2, and there is no recursion issue as the pseudo code for PUCCH, we believe there is no need to limit that the hybrid HP+LP PUCCH should not overlap with HP PUSCH. |
| LG | We prefer Alt 4, and can also accept Alt 2 as compromise. We don’t see the reason not to allow the overlapping between PUCCH with HP + LP UCIs and HP PUSCH, even though it was already agreed to support {HP AN + LP AN on HP PUSCH}. |

In addition, Ericsson [19] proposed to cancel LP PUCCH if a resultant PUCCH with multiplexed HP and LP UCI in step 2.1 overlaps with a LP PUCCH. It is discussed in section 3.1.3.

      - 2nd round discussion

Based on the GTW discussion on Monday, the proposal 1.2.1 is down-selected to Alt. 2 and Alt. 3 for further discussion. Between the two alternatives, the main discussion point is whether Alt. 3 introduces additional complexity at UE side by performing additional UCI multiplexing in PUSCH. From moderator’s point of view, it does not since step 2.2 needs to be performed between HP PUCCH with LP PUSCH as well. As discussed in section 3.1.6, one PUSCH among a set of candidate PUSCHs would be selected for UCI multiplexing and no additional complexity is expected.

Proposal 1.2.1a:

For resolving collision of PUCCHs and/or PUSCHs with different priorities in step 2, down-select from:

- Alt. 2: a resultant PUCCH with HP and LP UCI is not expected to be overlapped with a HP PUCCH.

- Alt. 3: a resultant PUCCH with HP and LP UCI is not expected to be overlapped with a HP PUCCH or a HP PUSCH.

|  | Supporting Company |
| --- | --- |
| Alt. 2 | Nokia/NSB, Sony, Sharp (with clarification), ZTE, CATT, Ericsson, DOCOMO,vivo Huawei/Hisi, LG |
| Alt. 3 | QC, Spreadtrum, Apple, Intel, Lenovo, Samsung, ITRI ,OPPO |

| Company | Comments |
| --- | --- |
| QC | We support Alt 3. In GTW today, companies support Alt 2 does not require recursive pseudo codes. We’d like to understand how Alt 2 can avoid going back to step 1. In our understanding, here is the flow to run the mux pseudo codes. - Step 1.1: Mux HP PUCCH with HP PUCCH. Mux LP PUCCH with LP PUCCH. - Step 1.2: MUX HP PUCCH with HP PUSCH. Mux LP PUCCH with LP PUSCH. - Step 2.1: MUX HP PUCH with LP PUCCH -> resulting PUCCH is HP because HP PUCCH resource is used. - Go back to run Step 1.2: if resulting HP PUCCH overlap with HP PUSCH - Step 2.2: Mux PUCCH on PUSCH with different priorities. This additional step creates a recursive behavior, because it goes back to step 1. One can argue this is only 1 time “go back”. But strictly speaking, it is still recursive. One more question is that, with simultaneous PUCCH/PUSCH transmission, should we do the “go back” before or after checking simultaneous Tx? Anyway, the additional “go back” step definitely complicates the overall UCI mux procedure. Given we are in maintenance, we prefer to take the simplest solution which is not allowing the “go back”. |
| Sharp | We prefer to clarify the second HP PUCCH: it should be limited to HP PUCCH for HARQ-ACK reporting. For a PUCCH with a positive HP SR, the gNB does not know the SR status in advance. A simple solution could be drop the HP SR or hold the SR to a later instance if the overlapping occurs. Thus, no impact on the specifications. |
| Apple | We support Alt. 3: the UE implementation needs to be have a bounded processing time. In our view: it is difficult to put the design as a gNB implementation choice to guarantee there is no overlapping of resultant HP PUCCH and a processed HP PUCCH and/or HP PUSCH. A simple way to achieve that is through a best effort strategy at UCI multiplexing: - If the resultant HP PUCCH can keep the same foot print as the original HP PUCCH (i.e. the additional payload due to LP UCI does not induce selecting a different PUCCH resource set), then it is guaranteed there won’t be any overlap between the resultant HP PUCCH and any other surviving HP PUCCH/PUSCH after step 1. - If the resultant HP PUCCH would overlap with a HP PUCCH/PUSCH, then the LP UCI over it is dropped (consequently there won’t be any need to change PUCCH resource set selection due to payload size change). Note due to the agreement two meetings ago on PUCCH resource set selection (simple addition of HP UCI/LP UCI bits without considering their code rates), in general in Rel-17 PUCCH resource sets can be quite coarse, we really don’t see the point to highly optimize the design for changing UCI payload when the PUCCH resource set selection itself is flawed or compromised. |
| ZTE | We think the recursive behavior is going back to pseudo codes for PUCCH only. No need to restrict the PUSCH scheduling. For the joint operation with simultaneously transmission, as simultaneously transmission is only for different priorities channels, it will not affect the multiplexing of resulting HP PUCCH and HP PUSCH. |
| Lenovo | Agree with QC’s assessment for recursive operation, so Alt 3 is preferred. Network can schedule a resultant PUCCH properly to avoid overlapping with another HP PUCCH or HP PUSCH. |
| Samsung | We support Alt 3, it is the simplest solution. |
| Ericsson | We support Alt 2. We do not agree with QC interpretation of “Go back to run Step 1.2: if resulting HP PUCCH overlap with HP PUSCH”. Why does the resulting PUCCH has to be called HP PUCCH? Why is this PUCCH – PUSCH multiplexing interpreted as “going back to run Step 1.2”? This step is simply one case of PUCCH-PUSCH multiplexing of Step 2.2. For example, the following two cases should be treated as parallel sub-cases in Step 2.2: - Resultant PUCCH {HP HARQ-ACK, LP HARQ-ACK} multiplex with HP PUSCH; - Resultant PUCCH {HP HARQ-ACK, LP HARQ-ACK} multiplex with LP PUSCH; |
| ITRI | Suppport Alt. 3 to avoid additional complexity on UCI multiplexing. |
| vivo | We share similar view with ZTE and Erission. We think Step 2.2 can be understood as Mux PUCCH on PUSCH with same/different priorities if the PUCCH with LP and HP UCI is called PUCCH with HP. |
| OPPO | If Alt-2 is followed, people can have two paths ahead: - The “resultant PUCCH with HP and LP UCI” is considered a HP PUCCH: the consequence is that the step-2 can land on the overlapping between this HP PUCCH and a HP PUSCH, which should have been solved by step-1. Then Alt-2 could leave the whole step-1/step-2 framework incomplete unless the step-1 is triggered again after step-2. But rerun a step-1 after step-2 is not an agreed framework step and it conflicts the agreed note saying “Avoid recursive pseudo-code to implement this procedure”. - The “resultant PUCCH with HP and LP UCI” is considered neither a HP PUCCH nor a LP PUCCH; instead, it is treated as a “3rd priority PUCCH”: the consequence of having a “3rd priority PUCCH” would complicate the whole framework logic and it does not even solve the original concern, for example, what about the below-mentioned overlapping after step-2: - a resultant PUCCH with HP and LP UCI and another resultant PUCCH with HP and LP UCI So we see both paths are quite challenging. With Alt-3, the “resultant PUCCH with HP and LP UCI” is considered a HP PUCCH, and the Alt-3 itself excludes the condition to trigger step-1 again. |
| Huawei/Hisi | The pseudo codes specifically means the multiple PUCCH multiplexing procedure, since the resultant PUCCH may overlap with a new PUCCH, causing the recursion. For PUCCH and PUSCH multiplexing, there is only one step anyhow, so the additional complexity is very limited. |
| LG | We have similar view with ZTE. As commented in above, we don’t see the reason not to allow the overlapping between PUCCH with HP + LP UCIs and HP PUSCH, even though it was already agreed to support {HP AN + LP AN on HP PUSCH}. |

Based on the above feedback from companies, it is proposed to agree the common part and leave HP PUSCH for further discussion.

Proposal 1.2.1b:

For resolving collision of PUCCHs and/or PUSCHs with different priorities in step 2, a resultant PUCCH with HP and LP UCI is not expected to be overlapped with a HP PUCCH.

- FFS whether a resultant PUCCH with HP and LP UCI can be overlapped with a HP PUSCH.

      - 6th round discussion

Please indicate whether you are fine to confirm the following working assumption.

Proposal 1.2.1c:

Confirm the following working assumption.

Working Assumption (to be confirmed in RAN1#107bis-e)

For resolving collision of PUCCHs with different priorities in step 2.1, if resultant PUCCH with HP and LP UCI collides with LP PUCCH without HARQ ACK, the LP PUCCH is dropped.

- A resultant PUCCH with HP and LP UCI is not expected to be overlapped with a LP PUCCH with HARQ-ACK.

|  | Company |
| --- | --- |
| Support | LG, QC, Samsung Huawei/Hisi, DOCOMO, Intel, Sharp, NEC, ZTE |
| Not support |  |

| Company | Comments |
| --- | --- |

Another remaining issue is the FFS point in the following conclusion.

Conclusion For resolving collision of PUCCHs and/or PUSCHs with different priorities in step 2, a resultant PUCCH with HP and LP UCI is not expected to be overlapped with a HP PUCCH. - FFS whether a resultant PUCCH with HP and LP UCI can be overlapped with a HP PUSCH.

Based on the discussions in previous rounds, the views are split and slightly more companies prefer to allow the overlapping between a resultant PUCCH with HP and LP UCI and HP PUSCH to achieve better scheduling flexibility.

As shown in the figure below, the question is whether the overlapping between resultant PUCCH and HP PUSCH in blue is allowed or not. It can be seen that if it is allowed, HP PUSCH is one of the candidate PUSCHs for UCI multiplexing. But regardless, as long as the resultant PUCCH overlaps with PUSCH that the UE cannot simultaneously transmit, UCI multiplexing in PUSCH is needed. The potential impact is that priority of PUSCH can be considered as an additional factor for PUSCH selection. But as discussed in section 3.1.6, majority companies agree to reuse Rel-15/16 rules without taking priority of PUSCH into account. With this, allowing a resultant PUCCH with HP and LP UCI to be overlapped with a HP PUSCH does not impact the procedure of UCI multiplexing in step 2.2.

Please share your view whether you support/are fine with the following proposal. A sub-bullet is added which hopefully can address/alleviate some of the concerns.

Proposal 1.2.1c:

A resultant PUCCH with HP and LP UCI is allowed to be overlapped with a HP PUSCH.

- PUSCH selection for UCI multiplexing in step 2.2 is performed without taking the priority of PUSCH into account.

|  | Company |
| --- | --- |
| Support | LG (also for LP PUSCH) , Samsung (can live with it) Huawei/Hisi, DOCOMO, NEC, ZTE |
| Not support | QC |

| Company | Comments |
| --- | --- |
| LG | As noted in the sub-bullet, resultant PUCCH with HP and LP UCI is allowed to be overlapped also with LP PUSCH. |
| QC | We still prefer not going back to step 1.2 from step 2.2, because it is conflict with the concept/principle to have separated step 1 and step 2. If we go this way, why RAN1 decided to separate the procedure into step 1 and step 2 for same and different priorities. As a side note: We actually proposed to do intra-UE mux first for PUCCH, then for PUSCH, following Rel-15 framework, regardless of same or different priorities. But that proposal was rejected. Now, suddenly, we are coming back to Rel-15 for PUSCH… |
| Samsung | Although we would like to prioritize HP PUSCH for this case, but we think the proposal is a good compromise to make progress. We think the sub-bullet can be a general rule for PUSCH selection in step 2.2. @QC, we don’t think the proposal goes back to 1.2. After PUCCH multiplexing, the resultant PUCCH would be multiplexed in a PUSCH anyway if there is collision. The proposal doesn’t complicate the procedure. |
| Huawei/Hisi | @QC What we are talking about here is to multiplex HP+LP PUCCH on PUSCH (Case 1). In our understanding, if the HP PUCCH has been multiplexed into HP PUSCH in step 1.2, there is then no HP UCI in Step 2.1 (Case 2), right? |

    - [Open] PUCCH overlapping with multiple PUCCHs of a different priority in step 2

      - 1st/2nd round discussion

Based on previous agreements, overlapping PUCCHs with same priority is resolved in step 1 and overlapping PUCCHs with different priorities are resolved in step 2.1. LP SR and LP CSI are not eligible for inter-priority multiplexing. Whether multiplexing of LP HARQ-ACK with HP SR is supported or not has not been concluded. If not supported, LP HARQ-ACK overlapping with HP SR is not eligible for multiplexing as well.

For a HP PUCCH overlapping with multiple LP PUCCHs, there are three cases as categorized below and illustrated in the figure below. LP UCIs that are not eligible for UCI multiplexing are in grey.

- Case 1: each of the LP PUCCHs includes UCI eligible for UCI multiplexing

- Case 2: none of the LP PUCCHs includes UCI eligible for UCI multiplexing

- Case 3: some of the LP PUCCH(s) include UCI eligible for UCI multiplexing and other LP PUCCH(s) includes UCI not eligible for UCI multiplexing only

OPPO [14], ETRI [15] and LG [26] proposed to preclude the case that a HP PUCCH overlaps with multiple LP PUCCHs. ZTE [6] proposed to preclude multiplexing in case a PUCCH overlapping with multiple PUCCHs in general.

It needs to be discussed that whether/how the above cases are supported.

For Case 1, the same rule/principle as discussed in section 3.1.4 can be adopted, i.e. a UE does not expect to multiplex HARQ-ACK information that the UE would transmit in different LP PUCCHs in a HP PUCCH.

Proposal 1.3.1:

A UE does not expect to multiplex HARQ-ACK information that the UE would transmit in different LP PUCCHs in a HP PUCCH.

|  | Company |
| --- | --- |
| Support | Nokia/NSB, Sharp, Intel, Samsung , CATT, Ericsson (with modification to proposal), DOCOMO, ITRI,OPPO Huawei/Hisi, LG |
| Not support | Sony (need clarification), QC |

| Company | Comments |
| --- | --- |
| Sony | Case 1 seems to happen only if HP PUCCH is slot and LP PUCCH is sub-slot. Is this the expected configuration? Also why would a gNB configures something like this? |
| QC | The intention of the proposal sounds reasonable: exclude those complicated 3 way collisions. However, the wording of the proposal is not clear. It says “A UE does not expect to multiplex …”. But does it mean that it is an error case or it is a legitimate case where UE does not mux but UE fallback to Rel-16 to do prioritization? |
| Spreadtrum | We would like to check the proposal just want to exclude case 1 and 3. It allows the case 2 because it is just cancelation without multiplexing. Is it right understanding? For case 1, we agree that it is untypical case, but there is no fault to exclude it clearly. |
| Sharp | Case 1 should not happen because the subslot for HP PUCCH should be smaller or equal to subslot for LP PUCCH. Thus, the HP HARQ-ACK can only overlap with one LP HARQ-ACK. Case 2 and Case 3 are already covered by existing agreements or not in the scope of the WI, thus, HP HARQ-ACK should drop the LP SR and LP CSI. |
| ZTE | Case 1 is rare, can be an error case. Case 2 and 3 is clear as LP SR and LP CSI is not eligible for multiplexing with different priority channel. So maybe we could split this proposal to two proposals, one is for error case, the other is for non-eligible? |
| Lenovo | Agree with the intention. The case 1 can be considered as an error case. |
| Samsung | Regarding QC’s question, we think the proposal means error case. The wording is aligned with 38.213, in general, “UE does not expect” means error case. |
| Ericsson | The main reason these cases may happen is, HP HARQ-ACK is has larger (sub-)slot duration than LP HARQ-ACK. We suggest to address the problem by the proposal below: Proposal: (sub-)slot duration configured for HP HARQ-ACK is shorter than or equal to the (sub-)slot duration configured for LP HARQ-ACK. |
| vivo | We think QC’ question is valid, we are open to discuss it. If it means error case, it seems to be worse than that in NR 16 where the case is allowed and prioritization is used. |
| Huawei/Hisi | We understand it is error case, and gNB should avoid scheduling such case. For Ericsson’s modification, we think limiting HP PUCCH shorter than LP PUCCH does not resolve the overlapping case of one HP PUCCH vs multiple LP PUCCHs. |
| LG | We are fine the proposal 1.3.1 with slight update below for clarification, and also have same understanding with Samsung on QC’s question. Proposal 1.3.1 (updated): A UE does not expect to multiplex HARQ-ACK information that the UE would transmit in multiple different LP PUCCHs in a HP PUCCH. |

For Case 1, it does not only happen when the (sub-)slot duration of HP HARQ-ACK is longer, it also happens for the case when HP HARQ-ACK is configured with 2-OS sub-slot while LP HARQ-ACK is configured with 7-OS sub-slot. In addition, some companies think proposal 1.3.1 is not clear whether Case 1 is allowed and LP PUCCHs are dropped or Case 1 is an error case so the proposal is revised as follows.

Proposal 1.3.1a:

For resolving collision of LP PUCCHs and HP PUCCHs in step 2.1, a HP PUCCH with HARQ-ACK is not expected to be overlapped with multiple LP PUCCHs with HARQ-ACK.

For Case 2, it is straightforward to drop all the LP PUCCHs.

For Case 3, more discussion is needed. Different options can be considered. For example, PUCCH multiplexing procedure as defined in Rel-16 TS38.213 clause 9.2.5 can be reused and PUCCH resource for UCI multiplexing is determined according to Rel-17 rules. Then the collision of LP CSI and HP HARQ-ACK is first to be resolved and LP CSI is dropped. Then multiplexing of LP HARQ-ACK and HP HARQ-ACK is performed according to Rel-17 rules. Alternatively, multiplexing of LP HARQ-ACK and HP HARQ-ACK can be performed first and if the resultant channel does not overlap with PUCCH carrying LP CSI, LP CSI can be transmitted; otherwise the LP CSI is dropped. It is proposed to discuss the case together with a LP PUCCH overlapping with multiple HP PUCCHs.

A LP PUCCH with HARQ-ACK may overlap with multiple HP PUCCHs and the HP PUCCHs can be within a same HP PUCCH time unit or in different HP PUCCH time units and the HP PUCCH may include HARQ-ACK and/or SR. The solutions for these cases need to be discussed.

Given that the time unit for HP PUCCH is typically shorter than that for LP PUCCH and UCIs of different priorities are expected to be multiplexed in HP PUCCH resource, Huawei [4], vivo [5], CATT [7], Samsung [9], DOCOMO [10], Spreadtrum [11] and LG [26] proposed to use time unit of HP PUCCH as the time unit for multiplexing in step 2.1, while ZTE [6] would like to preclude multiplexing between resources with different time units. Furthermore, the following options were proposed by companies.

If a LP PUCCH overlapping with multiple HP PUCCHs in different HP PUCCH time units,

- Option 1: LP PUCCH is multiplexed in the first overlapping HP PUCCH time unit according to Rel-17 rules.

  - Supported by: Huawei, vivo, Spreadtrum, OPPO

- Option 2: LP PUCCH is multiplexed in the first overlapping HP PUCCH time unit with HP HARQ-ACK according to Rel-17 rules. If LP PUCCH doesn’t overlap with any HP PUCCH with HARQ-ACK, LP PUCCH is multiplexed in the first overlapping HP PUCCH time unit according to Rel-17 rules.

  - Supported by: Samsung, DOCOMO, Sharp, LG

- Option 3: LP PUCCH joins the multiplexing procedure in each of the overlapping HP PUCCH time units for multiplexing from the first overlapping time unit, unless the LP PUCCH is determined to be dropped or multiplexed with other channels.

  - Supported by: CATT

For the case illustrated in the figure below, LP HARQ-ACK would be multiplexed with HP SR for option 1 or with HP HARQ-ACK for option 2 according to Rel-17 rules. If the multiplexing is not supported, LP HARQ-ACK is dropped. The intention of option 2 is to minimize LP HARQ-ACK dropping given that the multiplexing of LP HARQ-ACK and HP SR is still under discussion and may not be supported. However, note that multiplexing of HARQ-ACKs with different priorities in PF2 is also under discussion which may not be supported. The intention of option 3 is to address the case that LP HARQ-ACK multiplexing with HP SR is supported and LP HARQ-ACK is not multiplexed with HP SR in case the SR is negative. Then LP HARQ-ACK would be multiplexed with HP HARQ-ACK according to Rel-17 rules.

For a LP PUCCH with HARQ-ACK overlapping with multiple HP PUCCHs in a same HP PUCCH time unit, the following options were proposed by companies.

- Option 1: Reuse PUCCH multiplexing procedure as defined in Rel-16 TS38.213 clause 9.2.5 and determine a single resource for UCI multiplexing according to Rel-17 rules.

  - Supported by: vivo, CATT

- Option 2: If there is overlapping HP PUCCH with HARQ-ACK, LP HARQ-ACK is multiplexed with HP HARQ-ACK first.

  - Supported by: Samsung, Sharp, LG

- Option 3: LP PUCCH is multiplexed with the first HP PUCCH

  - Supported by: OPPO

Whether/how to support multiplexing of LP HARQ-ACK and HP SR and whether/how to support multiplexing of LP HARQ-ACK and HP HARQ-ACK in PF2 are not concluded yet, which may impact the discussion of a LP PUCCH with HARQ-ACK overlapping with multiple HP PUCCHs. For the case shown in the left figure below, if multiplexing of HP and LP HARQ-ACKs in PF2 is not supported and if the resultant PUCCH format of LP HARQ-ACK and HP HARQ-ACK in sub-slot #2 is PF2, it needs to be discussed whether LP HARQ-ACK is dropped or can be multiplexed with HP HARQ-ACK in sub-slot #2. Similarly, for the case shown in the right figure below, if multiplexing of HP SR and LP HARQ-ACK is not supported or if LP HARQ-ACK is transmitted in LP PUCCH if HP SR is negative, it needs to be discussed whether LP HARQ-ACK is dropped or can be multiplexed with HP HARQ-ACK.

Therefore, it is proposed to discuss the cases when a LP PUCCH with HARQ-ACK overlaps with multiple HP PUCCHs later based on the progress on whether/how to support multiplexing of LP HARQ-ACK and HP SR and whether/how to support multiplexing of LP HARQ-ACK and HP HARQ-ACK in PF2.

      - 3rd round discussion

A HP PUCCH overlapping with multiple LP PUCCHs

Based on the discussions in section 3.1.2, companies would like to avoid recursive processing in step 2.1. So one-step collision handling is considered without using the pseudo code in 38.213 Clause 9.2.5 in the following discussion.

For the three cases for a HP PUCCH overlapping with multiple LP PUCCHs as in section 3.1.3.1, Case 1 was concluded as an error case according to the following agreement.

Agreement For resolving collision of LP PUCCHs and HP PUCCHs in step 2.1, a HP PUCCH with HARQ-ACK is not expected to be overlapped with multiple LP PUCCHs with HARQ-ACK. - It’s up to the editor whether/how to capture this

In this round, we continue the discussions for the other two cases.

For Case 2 where none of the LP PUCCHs overlapping with the HP HARQ-ACK includes UCI eligible for UCI multiplexing, i.e. each LP PUCCH includes SR and/or CSI only, it is straightforward to drop all the LP PUCCHs.

Proposal 1.3.2:

For resolving collision of LP PUCCHs and HP PUCCHs in step 2.1, if a HP PUCCH overlaps with multiple LP PUCCHs and none of the LP PUCCHs includes LP HARQ-ACK, all the LP PUCCHs are dropped.

|  | Company |
| --- | --- |
| Support | DOCOMO, OPPO, Intel (clarificaiton of relation between this proposal and the procedure), Sharp, New H3C, Ericsson (with updates),vivo, Spreadtrum, ZTE, LG, NEC, Nokia/NSB, Sony |
| Not support | Huawei/Hisi, Samsung |

| Company | Comments |
| --- | --- |
| OPPO | Both LP CSI and LP SR cannot be multiplexed with HP HARQ-ACK in PUCCH. |
| Intel | We agree with the spirit of the proposal, i.e., the final result is all LP PUCCHs without HAR-QACK are dropped, if they overlap with HP PUCCH. But it is unclear, what is the relation between this proposal and procedure for step 2.1. For example, if we agree to support “Perform pair-wise collision resolution sequentially in time” in step 2.1, it seems the proposal 1.3.2 is a result of resolving collision. Or, does it imply UE shall first perform proposal 1.3.2, before Perform pair-wise collision resolution sequentially in time” in step 2.1? Similar question for proposal 1.3.3 ~1.3.5. |
| QC | We don’t object the proposal. We just have a question for clarification: If in step 1, LP HARQ-ACK multiplexed with a LP CSI. Then the resulting PUCCH overlap with a HP PUCCH. In step 2.1: should UE mux both LP HARQ-ACK and LP CSI or should UE only drop LP CSI then multiplex LP HARQ-ACK on HP PUCCH? In other words, the question is: whether UE should demultiplex UCIs already multiplexed if part of UCI cannot be multiplexed with an overlapping HP PUCCH? We think this issue should be discussed. Our preference is not allowing demultiplex operation – just drop the resulting PUCCH including both LP HARQ-ACK and LP CSI. Also, a minor comment on the wording of the proposal: the Proposal emphasize “overlapping with multiple LP PUCCH”. But even in the case of HP PUCCH overlap with a single LP PUCCH, and that LP PUCCH is SR or CSI, it has to be drop as it cannot be multiplexed with HP PUCCH. We are wondering why the proposal does not include the single LP PUCCH case? A side comment: we have this agreement already “For resolving collision of LP PUCCHs and HP PUCCHs in step 2.1, a HP PUCCH with HARQ-ACK is not expected to be overlapped with multiple LP PUCCHs with HARQ-ACK.”. But how about a LP PUCCH overlap with multiple HP HARQ-ACK? Should this be allowed or treat as error case? |
| Huawei/Hisi | We do not object the intention of the proposal, but not clear why we need to first discuss the case of multiple LP PUCCHs vs one HP PUCCH (same as QC). As per our view, even for one LP PUCCH w/o HARQ-ACK vs HP PUCCH, the LP PUCCH should be dropped. Thus the proposal can be modified as: For resolving collision of LP PUCCHs and HP PUCCHs in step 2.1, if a HP PUCCH overlaps with multiple a LP PUCCHs and none of the LP PUCCHs includes without LP HARQ-ACK, all the LP PUCCHs are is dropped. Not sure if dropping LP PUCCH without LP HARQ-ACK in case of collision with HP PUCCH is the common understanding, but we should first discuss the pairwise case, after which I think the rule can be a reference for handling the one PUCCH vs multiple PUCCHs case. |
| Samsung | We agree with the intention, but we share similar concern with Intel. We think the proposal is already supported in current spec. - if // this is for cases the UE supports multiplexing information of different priorities in a PUCCH/PUSCH - a PUCCH transmission with HARQ-ACK information, without repetitions, with smaller priority index overlaps with a PUCCH transmission with HARQ-ACK information, without repetitions, with larger priority index, or - a PUCCH transmission with HARQ-ACK information, without repetitions, with smaller or larger priority index overlaps, respectively, with a PUSCH transmission with larger or smaller priority index the UE multiplexes HARQ-ACK information of different priority indexes in a same PUCCH or PUSCH transmission and applies the procedures in clause 9.2.5.3 or 9.3, respectively - else - if the UE would transmit the following channels that would overlap in time where, if a channel transmission is with repetitions, the following are applicable per repetition - a first PUCCH of larger priority index and a second PUCCH of smaller priority index - a first PUCCH of larger priority index and a second PUSCH of smaller priority index when the UE cannot simultaneously transmit the first PUCCH and second PUSCH - a first PUCCH of smaller priority index and a second PUSCH of larger priority index when the UE cannot simultaneously transmit the first PUCCH and second PUSCH - a first PUSCH of larger priority index and a second PUSCH of smaller priority index on a same serving cell, where at least one of the two PUSCHs is a configured grant PUSCH the UE - transmits the PUCCH or the PUSCH of the larger priority index, and - does not transmit a PUCCH or a PUSCH of smaller priority index. |
| Ericsson | The proposal can be generalized to the following. Revised Proposal 1.3.2: For resolving collision of LP PUCCHs and HP PUCCHs in step 2.1, if a HP PUCCH overlaps with one or more LP PUCCHs and none of the LP PUCCHs includes LP HARQ-ACK, all the LP PUCCHs are dropped. |
| LG | Agree with Ericsson’s update. |
| Nokia/NSB | Support the intention of the proposal. And we are fine with Ericsson’ suggested edit. |
| Sony | The case where a HP PUCCH overlaps with one LP PUCCH is more likely than one HP PUCCH overlaps with multiple LP PUCCH. Hence we prefer Ericson’s update. |

For Case 3 where one of the LP PUCCHs includes HARQ-ACK, different options can be considered.

- Option 1: Perform pair-wise collision resolution sequentially in time.

- Option 2: Collision between LP PUCCH with HARQ-ACK (if any) and HP PUCCH is resolved first. All the LP PUCCH(s) overlapping with the resultant PUCCH are dropped.

- Option 3: LP PUCCH(s) without HARQ-ACK are dropped and the collision between the remaining LP PUCCH with HARQ-ACK (if any) and HP PUCCH is resolved.

The options are illustrated using the examples shown below.

The outcomes of the three options are the same for the following scenario assuming the resultant PUCCH with HP HARQ-ACK and LP HARQ-ACK overlaps with LP CSI. Only PUCCH with HP HARQ-ACK and LP HARQ-ACK is transmitted.

If HP HARQ-ACK is replaced by HP SR, assuming multiplexing of LP HARQ-ACK and HP SR is not supported, the outcomes of the three options are also the same, i.e. all the overlapping LP PUCCHs are dropped.

For scenario 1 and 2 shown in the left figure and right figure below:

- For Option 1, only PUCCH with HP HARQ-ACK and LP HARQ-ACK is transmitted and LP CSI is dropped before multiplexing of HP HARQ-ACK and LP HARQ-ACK for Scenario 1. For Scenario 2, both PUCCH with HP HARQ-ACK and LP HARQ-ACK and PUCCH with LP CSI are transmitted since the resultant PUCCH for HP HARQ-ACK and LP HARQ-ACK does not overlap with LP CSI.

- For Option 2, for both scenarios, both PUCCH with HP HARQ-ACK and LP HARQ-ACK and PUCCH with LP CSI are transmitted.

- For Option 3, LP CSI is dropped for both scenarios and only PUCCH with HP HARQ-ACK and LP HARQ-ACK is transmitted.

Among the three options, Option 3 is the worst in terms of LP HARQ-ACK dropping and does not apply to the case that a LP PUCCH overlapping with multiple HP PUCCHs, it is proposed to down-select from the first two options

Proposal 1.3.3:

For resolving collision of LP PUCCHs and HP PUCCHs in step 2.1, if a HP PUCCH overlaps with multiple LP PUCCHs and one of the LP PUCCHs includes LP HARQ-ACK, down-select from:

- Option 1: Perform pair-wise collision resolution sequentially in time.

- Option 2: Collision between LP PUCCH with HARQ-ACK (if any) and HP PUCCH is resolved first. All the LP PUCCH(s) overlapping with the resultant PUCCH are dropped.

|  | Company |
| --- | --- |
| Option 1 | Intel Huawei/Hisi, Ericsson (with revision and potential combination) , ZTE, Sony |
| Option 2 | DOCOMO, Samsung, Nokia/NSB |

| Company | Comments |
| --- | --- |
| DOCOMO | Prefer Option 2 as it can avoid unnecessary dropping. |
| OPPO | Take same logic of Proposal 1.3.2. LP PUCCHs carrying CSI/SR are dropped first, then HP UCI and LP HARQ-ACK are multiplexed. |
| Intel | We support option 1 for simplicity, it is unified single step checking for PUCCH for any UCI type and PUCCH overlapping cases (option 1 applies to proposal 1.3.2 ~1.3.5), for PUCCH with or without repetition (e.g., no need of sub-step 2.1.1 and 2.1.2 as shown in section 3.1.8), no need of additional mechanism to determine adding LP PUCCH into which sub-slot, etc. For opt 2, it is unclear to us, how option 2 works with options in proposal 1.3.5, e.g., the figure shown below. And also, it is also unclear, whether unified solution is applicable for PUCCH with or without repetition, and whether additional mechanism to determine adding LP PUCCH into which sub-slot, etc. We’re reluctant to design different options for different cases or optimize case by case. Unified solution is preferred at this late stage. |
| QC | Slightly prefer option 2 based on same reason mentioned by DOCOMO. |
| Huawei/Hisi | Step 2.1 can refer to the R15 rule of handling multiple overlapping PUCCHs: Step 1) Find a reference PUCCH resource A with the priority order of earliest symbol followed by longest duration in the slot. Step 2) Find a set of PUCCH resources X overlapping with the PUCCH resource A. Step 3) Perform multiplexing over the set X and A to generate a resulting resource and a resulting UCI. Step 4) Loop Step 1) ~ Step 3) until there is not any overlapping PUCCHs in the slot. That means, the earliest and longest PUCCH should be taken as the reference for handling the overlapping, then the resultant PUCCH, if overlapping with a new PUCCH, will be handled in a next round as in Step 4), which is clearly a pair-wise rule. Applying this rule to Scenario 1, the LP CSI is pair-wise overlapping with HP HARQ-ACK and dropped; for Scenario 2, LP HARQ-ACK is handled first, then the resultant channel of HP+LP PUCCH will be compared with LP CSI. |
| Samsung | Option 1 is definitely not acceptable because of the implementation complexity and large spec impact. - Ordering function is needed for Option 1 to perform resolving pair-wise collision. The details include the order is based on the starting symbol or ending symbol, what if the starting symbol/ending symbol is the same, how to define the order? Should UCI be considered? - Recursive procedure is required. If a LP CSI/SR PUCCH doesn’t overlap with a HP PUCCH, but after multiplexing HP HARQ-ACK and LP HARQ-ACK, the LP CSI/SR can be overlapped with the resulting PUCCH. In this case, recursive procedure is required to drop LP CSI/SR. Re-ordering the remaining PUCCHs are also necessary. Option 2 is much simpler with better performance, LP HARQ-ACK is either multiplexed in the HP PUCCH or dropped. After that other overlapping LP PUCCH is dropped. Option 2 can increase the transmission probability of LP CSI/SR as shown in Scenario 2. We think the following case should also be considered, and suggest the following update for the main bullet, For resolving collision of LP PUCCHs and HP PUCCHs in step 2.1, if a HP PUCCH overlaps with one or more multiple LP PUCCHs and one of the LP PUCCHs includes LP HARQ-ACK, down-select from: Regarding Intel’s question, first resolve the collision of LP HARQ-ACK and HP PUCCHs, for example, LP HARQ-ACK is multiplexed with HP HARQ-ACK, then drop LP CSI if it overlaps with a HP PUCCH. Regarding PUCCH repetition, we would like to ask Intel and FL, are we going to separate discuss the issues related to PUCCH repetitions or we joint discuss it? We are open with either way but we think it is easier to focus on cases without PUCCH repetitions first for simplicity. |
| Ericsson | We prefer Option 1 which is a simple and generic rule. Also Revised Proposals 1.3.2 and 1.3.3 can be easily combined. Revised Proposal 1.3.3: For resolving collision of LP PUCCHs and HP PUCCHs in step 2.1, if a HP PUCCH overlaps with one or more LP PUCCHs and one of the LP PUCCHs includes LP HARQ-ACK: - Option 1: Perform pair-wise collision resolution sequentially in time. Combined Revised Proposals 1.3.2 and 1.3.3: For resolving collision of LP PUCCHs and HP PUCCHs in step 2.1, if a HP PUCCH overlaps with one or more LP PUCCHs, perform pair-wise collision resolution sequentially in time. |
| vivo | We slightly prefer option 2 as it can avoid unnecessary dropping |
| ZTE | Performing pair-wise collision resolution sequentially in time is more nature for UE. UE needn’t to wait the DCI for later LP HARQ-ACK. If the resolution is handled sequentially in time, as CSI is not eligible for multiplexing, anyway, CSI will be dropped. For scenario 2, the CSI may survive or not survive, which depends on whether the resultant PUCCH resources overlapping with LP CSI. |
| NEC | We slightly prefer option 2 to avoid unnecessary UCI dropping. |
| Nokia/NSB | Support Option 2 for similar reason as DOCOMO’s. We are also fine with the suggested edit by Samsung. |
| New H3C | We slightly prefer option 2. |
| Sony | Since UE isn’t aware of any processing timeline and would start resolving a collision as soon as it has decoded a DCI and aware of the collision then Option 1 is really the only option. Option 2 seems to suggest the UE is aware of some processing timeline, which was fiercely argued was not possible in the last meeting and so the UE cannot wait till all overlapping channels are present before starting the processing of resolving them. Hence, I do not see how Option 2 is even an option at all unless suddenly this meeting UE vendors are happy with UE being aware of the processing timelines. |

A LP PUCCH overlapping with multiple HP PUCCHs

If a LP PUCCHs overlaps with multiple HP PUCCHs and the LP PUCCH does not include HARQ-ACK, then the LP UCI is not eligible for UCI multiplexing and should be dropped.

Proposal 1.3.4:

For resolving collision of LP PUCCHs and HP PUCCHs in step 2.1, if a LP PUCCH overlaps with multiple HP PUCCHs and the LP PUCCH does not include LP HARQ-ACK, the LP PUCCH is dropped.

|  | Company |
| --- | --- |
| Support | DOCOMO,OPPO, Intel (need clarifcation), QC,vivo, Spreadtrum, ZTE, LG, NEC, Nokia/NSB,New H3C, Sony |
| Not support | Huawei/Hisi, Samsung |

| Company | Comments |
| --- | --- |
| DOCOMO | Support the proposal as there is no agreement to support multiplexing of HP channel and LP channel not including LP HARQ-ACK so far. |
| Intel | Similar comment as proposal 1.3.2. We agree with the spirit of the proposal that a LP PUCCH without HARQ-ACK overlapping with multiple HP PUCCHs is anyway finally dropped. But it is unclear to us, what is the relation between such LP PUCCH dropping and options in proposal 1.3.3 and 1.3.5, and whether we need a separate agreement. - Whether to firstly drop such LP PUCCH before resolving a pair of LP and HP PUCCH in option 1/ resolving overlapping between LP PUCCH and HP PUCCH in option 2, or it can be processed during option 1 and option 2. - For option 1, when UE performs single check for a pair of LP and HP PUCCH, if LP PUCCH does not include LP HARQ-ACK, then, LP PUCCH is dropped, no matter it overlaps with single or two HP PUCCHs. So, it seems no need of special handling. For option 2, I’m not sure whether special handling is needed, it may depend on detailed design for option 2. |
| QC | OK with the proposal. But similar comment on wording as mentioned for proposal 1.3.2: why emphasizing overlapping with multiple HP PUCCH? Even if the LP PUCCH overlap with a single HP PUCCH but the LP PUCCH does not include HARQ-ACK, the LP PUCCH should be dropped. |
| Huawei/Hisi | With the same reason for proposal 1.3.2, if our modified version in 1.3.2 is adopted, there is no need for a new proposal here. |
| Samsung | We think the proposal is already supported in current spec similar as Proposal 1.3.2. |
| Ericsson | Support in principle. However, we prefer Revised Proposal 1.3.2 which covers proposal 1.3.4 already. That is, proposal 1.3.4 is not needed. Revised Proposal 1.3.2: For resolving collision of LP PUCCHs and HP PUCCHs in step 2.1, if a HP PUCCH overlaps with one or more LP PUCCHs and none of the LP PUCCHs includes LP HARQ-ACK, all the LP PUCCHs are dropped. |
| LG | Agree with Ericsson’s direction. |
| NEC | Support in principle. |
| Nokia/NSB | Support the intention of the proposal, along with Ericsson’s suggested edit. |

For the case that a LP PUCCH with HARQ-ACK overlaps with multiple HP PUCCHs, different options can be considered.

- Option 1: Perform pair-wise collision resolution sequentially in time.

- Option 2: Collision between LP PUCCH and first HP PUCCH with HARQ-ACK (if any) is resolved first. If LP HARQ-ACK cannot be multiplexed with the first HP PUCCH with HARQ-ACK, LP HARQ-ACK is dropped.

For the following case that a LP HARQ-ACK overlaps with multiple HP HARQ-ACKs, the outcomes of the two options are the same. If LP HARQ-ACK can be multiplexed with HP HARQ-ACK in sub-slot #1, it is multiplexed; otherwise it is dropped.

For the following case, assuming LP HARQ-ACK cannot be multiplexed with HP SR, LP HARQ-ACK is dropped according to Option 1 and it is multiplexed with HP HARQ-ACK if supported for Option 2.

Proposal 1.3.5:

For resolving collision of LP PUCCHs and HP PUCCHs in step 2.1, if a LP PUCCH overlaps with multiple HP PUCCHs and the LP PUCCHs includes LP HARQ-ACK, down-select from:

- Option 1: Perform pair-wise collision resolution sequentially in time.

- Option 2: Collision between LP PUCCH and first HP PUCCH with HARQ-ACK (if any) is resolved first. If LP HARQ-ACK cannot be multiplexed with the first HP PUCCH with HARQ-ACK, LP HARQ-ACK is dropped.

|  | Company |
| --- | --- |
| Option 1 | OPPO(with clarification), Intel Huawei/Hisi, ZTE, Sony (needs clarification) |
| Option 2 | DOCOMO, QC, Samsung,vivo, LG (with slight modification), Nokia/NSB,New H3C |

| Company | Comments |
| --- | --- |
| DOCOMO | Prefer Option 2 to avoid unnecessary dropping. |
| OPPO | Take same logic of Proposal 1.3.4. LP PUCCHs carrying CSI/SR are dropped first, then perform pair-wise collision resolution between HP PUCCHs and LP PUCCH carrying HARQ-ACK sequentially in time, i.e. the LP HARQ-ACK should be multiplexed with the first HP PUCCH. |
| Intel | Prefer unified procedure for all cases. |
| QC | Prefer option 2. A small question: “first HP PUCCH with HARQ-ACK” meaning earliest in time? Maybe a clarification on this is needed. |
| Huawei/Hisi | A unified solution is preferred as in proposal 1.3.3. E.g., consider HP SR and HP HARQ-ACK are located in different subslots, and we assume the multiplexing processing time unit is subslot (otherwise it needs to further discuss which subslot should be used to place the resultant PUCCH). For Case 1 LP HARQ-ACK is handled first with HP SR in subslot#1; regardless it is multiplexed on subslot#1 or dropped, the resultant channel will not be overlapped with HP HARQ-ACK on subslot#2. For Case 2 LP HARQ-ACK is handled first with HP HARQ-ACK in subslot#1, and it is the same that the resultant channel will not impact HP SR. If we go with Option 2, Case 1 and Case 2 need to design different processing time orders, e.g., in Case 1 subslot#2 is handled first, while in Case 2 subslot#1 is handled first. That will increase the processing complexity for both UE and gNB. |
| Samsung | Same comment for Proposal 1.3.3. |
| Ericsson | We prefer Option 1 which is a simple and generic rule. Note that Revised Proposal 1.3.3 (copied below) already covers Option 1 of Proposal 1.3.5. Thus Proposal 1.3.5 is not needed. Revised Proposal 1.3.3: For resolving collision of LP PUCCHs and HP PUCCHs in step 2.1, if a HP PUCCH overlaps with one or more LP PUCCHs and one of the LP PUCCHs includes LP HARQ-ACK: Option 1: Perform pair-wise collision resolution sequentially in time. |
| vivo | We slightly prefer option 2 |
| LG | We agree with DOCOMO, and suggest slight modification as below considering possibility of multiplexing with HP SR PUCCH (in case of LP AN PF0/1). Option 2: Collision between LP PUCCH and first HP PUCCH with HARQ-ACK (if any) is resolved first. If LP HARQ-ACK cannot be multiplexed with the first HP PUCCH with HARQ-ACK, LP HARQ-ACK is dropped collision between LP PUCCH and first HP PUCCH is resolved. |
| Nokia/NSB | Support Option 2 for similar reason as DOCOMO’s. |
| New H3C | Same view as Proposal 1.3.3. |
| Sony | For the case with two HP HARQ-ACK overlapping one LP HARQ-ACK, we do not agree with the FL that Option 1 and Option 2 provides the same outcome. In Option 1, it is possible that the later HP HARQ-ACK arrives first then later another HP HARQ-ACK is scheduled earlier and both overlaps the LP PUCCH. In this case, Option 1 would lead to the LP HARQ-ACK being multiplexed with the later HP HARQ-ACK. Hence, there seems to be some clarification required. That is what does it mean by “sequentially in time”? Does it mean: - The UE process whatever is being scheduled in time, that is when the UE receives a DL Grant and is aware of a collision it processes that collision first and if another one arrives it process that one. That is the “sequentially in time” follows which UL channel is scheduled first and this may or may not correspond to the time arrival of the a UL channel. - The UE somehow waits for all collision to arrive, then arrange the UL channels according to when they arrive first. That is “sequentially in time” refers to the time the UL channel is transmitted regardless of when they were scheduled. I think this needs clarification because in interpretation 2, it can be argued that this is not possible for UE that is not aware of processing timeline, that is UE has no idea when all UL channels have arrived. |

Based on the above discussions, it seems that companies prefer to discuss unified solutions instead of per-case discussion. In addition, for proposals 1.3.2 and 1.3.4, although the intention seems agreeable and the updates proposed by companies are reasonable, Samsung pointed out that it is already captured in the specification. Therefore, it is proposed to focus on proposal 1.3.3 and 1.3.5 and discuss the unified solutions instead.

First of all, the following proposal is provided which hopefully is the common understanding.

Proposal 1.3.6:

For resolving collision of PUCCHs and/or PUSCHs with different priorities in step 2.1, if a resultant PUCCH with HP and LP UCI overlaps with a LP PUCCH, the LP PUCCH is dropped.

| Company | Comments |
| --- | --- |
| Nokia/NSB | Support |
| New H3C | Support this proposal. |
| DOCOMO | Support |
| LG | Support (but not limited to this case) |

For the unified solution, it is expected that more discussion is needed. But it is beneficial to narrow down the options for further discussion.

Proposal 1.3.7:

For resolving collision of LP PUCCHs and HP PUCCHs in step 2.1, down-select from:

- Option 1: Perform pair-wise collision resolution sequentially in time.

  - FFS details

- Option 2: First resolve collision among a LP PUCCH with HARQ-ACK and HP PUCCH(s), then resolve collision among LP PUCCH(s) without HARQ-ACK and HP PUCCH(s).

  - FFS details

| Company | Comments |
| --- | --- |
| Nokia/NSB | Option 2 (similar comment as for Proposal 1.3.3) |
| New H3C | We slightly prefer Option 2 |
| Sony | Please clarify what it mean by “sequentially in time”. This can be interpreted in 2 ways: - “Sequentially in time” refers to when the scheduling of the UL channel arrives - “Sequentially in time” refers to when the UL channel is transmitted regardless of when it was scheduled in time |
| DOCOMO | Slightly prefer Option 2. |

      - 4th round discussion

Proposal 1.3.6 was extensively discussed without conclusion. Let’s continue discussing the updated proposal from GTW.

[Closed] Proposal 1.3.6a: (Moderator: Please check the updated proposal 1.3.6b below.)

For resolving collision of PUCCHs and/or PUSCHs with different priorities in step 2.1, if resultant HP PUCCH with HP and LP UCI collides with LP PUCCH without HARQ ACK, the LP PUCCH is dropped.

|  | Company |
| --- | --- |
| Support | Sony |
| Not support | QC (can support if concerns addressed) |

| Company | Comments |
| --- | --- |
| Sony | Since we already ruled out any case where a single HP PUCCH overlaps two LP PUCCH carrying HARQ-ACK, the scenario where a resultant HP PUCCH still somehow overlaps a LP PUCCH with HARQ-ACK is not possible. However, it is fine to agree on a solution for a case that will not happen. |
| Apple | Not support. The key issue here is whether the resultant HP PUCCH is allowed to move around w.r.t. the original HP PUCCH. If not, the case won’t happen, as the all the overlapping LP PUCCHs with the original HP PUCCH would be handle in a single step, there is no further checking needed. If allowed, then how to guarantee while the HP PUCCH moves around w.r.t. the original HP PUCCH, it will not overlap with a HP PUCCH, and can only overlap LP PUCCH? Note for HP PUCCHs, they can be SR or HARQ-ACK. To avoid the resultant HP PUCCH overlapping HP PUCCHs, gNB configuration flexibility can be severely limited. Also for URLLC traffic, latency is key. No matter whether it is for DL data or UL data, ultimately a key thing is the latency for HARQ-ACK and SR. Then allowing LP UCI to drive the HP PUCCH around is not good at all. Please recall the timing chart analysis from Rel-16 URLLC study item. The best choice is just to let the resultant PUCCH be the same as the original HP PUCCH, e.g. a best effort attempt to carry LP UCI, if not viable then the LP UCI is omitted. We suggest change as follows: For resolving collision of PUCCHs and/or PUSCHs with different priorities in step 2.1, if resultant HP PUCCH with HP and LP UCI collides with LP PUCCH without HARQ ACK, the LP PUCCH is dropped the OFDM symbols occupied by resultant HP PUCCH is a subset (which can be a full set) of the OFDM symbols occupied by the original HP PUCCH . [Moderator] As commented during GTW and email discussion, the proposal is too restrictive. In addition, even with your proposal, it is still possible that a resultant PUCCH with HP and LP UCIs overlaps with a LP PUCCH as shown below. |
| Lenovo | We are supportive of the intention. However, we think there is a problem in wording. “resultant HP PUCCH with HP and LP UCI” is a PUCCH resulting from resolving all overlapping PUCCHs in step 2.1. Any overlapping of an intermediate HP PUCCH resource (which is determined during the multiplexing procedure) with a LP PUCCH with SR/CSI should be resolved during the step 2.1 procedure (i.e. the LP PUCCH not included for multiplexing but dropped). [Moderator] “resultant PUCCH with HP and LP UCI” is exactly the wording we used in the agreed conclusion below. It cannot be a PUCCH resulting from resolving all overlapping PUCCHs in step 2.1, otherwise the conclusion does not make sense at all. Conclusion For resolving collision of PUCCHs and/or PUSCHs with different priorities in step 2, a resultant PUCCH with HP and LP UCI is not expected to be overlapped with a HP PUCCH. - FFS whether a resultant PUCCH with HP and LP UCI can be overlapped with a HP PUSCH.. In our view, a necessary conclusion is that “Any LP PUCCH without HARQ-ACK overlapping with a HP PUCCH(s) is not multiplexed in step 2.1.” |
| QC | our main concern here is that the proposal would create exponentially growing # sub-paths and UE implementation/testing complexity is unmanageable. • Like illustrated by the following figure, our preference is to reuse Rel-15 Pseudo code for step 2.1, which just keep multiplexing until no overlap. This is the simplest operation. • The current proposal from FL would create 2-sub-path when handling each pair of overlapping HP and LP PUCCH. So the total # sub-path could grow exponentially in theory. • The third proposal which stop the exponentially growing # sub-paths. So it is acceptable to us, if this red text is added in the proposal “The resulting HP PUCCH does not expect to overlap with any other LP PUCCH that can be multiplex with the resulting PUCCH.” One might argue that one slot cannot have more than 1 LP HARQ-ACK PUCCH so the current proposal = the third proposal. While strictly speaking, this might be true for most of the cases but not true for all cases. For example, although it is a corner case, but if LP PUCCH is configured with sub-slot scheduling, while HP PUCCH is configured with slot based scheduling (it might be a weird setup but spec does not prohibit this). The One HP potentially can overlap with up to 7 LP HARQ-ACKs. There might be other corner cases that we missed. As long as spec does not exclude those cases, UE implementation have to consider them. @Sony, I don’t think we fully excluded the case where a HP PUCCH overlap with more than 1 LP HARQ-ACK. We only have the following agreement for HP PUCH with HARQ-ACK :) but not for other HP UCIs. Agreement For resolving collision of LP PUCCHs and HP PUCCHs in step 2.1, a HP PUCCH with HARQ-ACK is not expected to be overlapped with multiple LP PUCCHs with HARQ-ACK. - It’s up to the editor whether/how to capture this Our second concerns is that: how to deal with the following scenario? - If a LP PUCCH has LP HARQ-ACK + LP CSI (result from step 1.1) and it overlap with a HP PUCCH. Based on some companies insisted that LP CSI cannot be muxed with HP PUCCH, what should UE do? Which of the following is UE behavior? - Option 1: UE drop the LP PUCCH including LP HARQ-ACK and CSI - Option 2: UE only drop the LP CSI and mux LP HARQ-ACK on HP PUCCH - Option 3: UE mux both LP CSI and LP HARQ-ACK on HP PUCCH. We still prefer option 3 because we think it is the simplest. One may argue it needs new agreement to support LP CSI mux on HP PUCCH. But then option 1 does not fit into the FL proposal. Option 2 need UE undo what were already muxed in step 1, which is not good neither. So there seems no solution for this issue if we go with FL proposal. |
| Sharp | We agree the principle although not sure if this proposal is necessary. If the resultant HP PUCCH is not expected to overlap with another LP PUCCH with HARQ-ACK, the proposal is already covered by existing agreements on LP CSI and LP SR are not mutltiplexed with HP HARQ-ACK. So, the proposal is a clarification or an observation than something worth agreeing on. |
| Ericsson | While we are fine with the intention of the proposal, we also agree with other companies’ sentiment that it’s more important to agree on some fundamentals first. Proposal 3.1.3.3.A (related to QC concern on corner case): For a given PUCCH group, (sub-)slot duration configured for HP HARQ-ACK is shorter than or equal to the (sub-)slot duration configured for LP HARQ-ACK. Proposal 3.1.3.3.B (related to Apple concern on moving HP PUCCH resource): In Step 2.1, collision resolution is performed sequentially in units of HP (sub-)slot, starting from the first HP (sub-)slot. Proposal 3.1.3.3.C (related to Sony concern): In Step 2.1, when resolving the collision between a HP PUCCH and LP PUCCH in a HP sub-slot, cancel any LP PUCCH that starts in an earlier HP sub-slot and overlaps with a HP PUCCH in current HP sub-slot. Proposal 3.1.3.3.D (similar to Lenovo point): Rel-17 does not support multiplexing a HP PUCCH with an LP PUCCH, if the LP PUCCH does not carry HARQ-ACK. When resolving the collision between a HP PUCCH and an LP PUCCH without HARQ-ACK, the LP PUCCH is dropped. |
| Intel | It seems most controversial points are caused by different sub-slot configuration for LP and HP PUCCH. Can we just exclude the different sub-slot configuration for LP and HP PUCCH, or at least exclude the sub-slot configuration for LP PUCCH for Rel-17 intra-UE multiplexing? If yes, for procedure-wise, there is only one sub-path after 1st resulting HP PUCCH with both LP and HP UCI, i.e., drop a LP PUCCH, if the LP PUCCH overlaps with the resulting HP PUCCH. Considering we already agreed no multiplexing for LP CSI or SR with HP PUCCH, maybe proposal 1.3.6a is not needed. Regarding Apple’s suggestion, we think it would be too restricted, if we forbid HP PUCCH to move, even if the move does not lead to overlap with any other PUCCHs. Regarding Ericsson’s suggestion, we think HP unit may not help to resolve the issues. For example, for proposal 3.1.3.3.A, if HP is sub-slot with 2 symbols, and LP is sub-slot with 7 symbols, it is still possible a HP PUCCH overlaps with two LP HARQ-ACK in 2 sub-slot. And also, resolving the collision by HP unit does not help for such case. |
| Samsung | We think the proposal is for clarification, it is already captured in current spec. For a LP PUCCH without HAQ-ACK, it goes to else and the LP PUCCH is dropped. Lenovo’s proposal is not needed either. - if // this is for cases the UE supports multiplexing information of different priorities in a PUCCH/PUSCH - a PUCCH transmission with HARQ-ACK information, without repetitions, with smaller priority index overlaps with a PUCCH transmission with HARQ-ACK information, without repetitions, with larger priority index, or - a PUCCH transmission with HARQ-ACK information, without repetitions, with smaller or larger priority index overlaps, respectively, with a PUSCH transmission with larger or smaller priority index the UE multiplexes HARQ-ACK information of different priority indexes in a same PUCCH or PUSCH transmission and applies the procedures in clause 9.2.5.3 or 9.3, respectively - else - if the UE would transmit the following channels that would overlap in time where, if a channel transmission is with repetitions, the following are applicable per repetition - a first PUCCH of larger priority index and a second PUCCH of smaller priority index - a first PUCCH of larger priority index and a second PUSCH of smaller priority index when the UE cannot simultaneously transmit the first PUCCH and second PUSCH - a first PUCCH of smaller priority index and a second PUSCH of larger priority index when the UE cannot simultaneously transmit the first PUCCH and second PUSCH - a first PUSCH of larger priority index and a second PUSCH of smaller priority index on a same serving cell, where at least one of the two PUSCHs is a configured grant PUSCH the UE - transmits the PUCCH or the PUSCH of the larger priority index, and - does not transmit a PUCCH or a PUSCH of smaller priority index. Apple’s proposal is too restrictive and cannot be acceptable. For example, a HP HARQ-ACK with PF0, after multiplexing with LP HARQ-ACK, the total payload is larger than 2, PF3/4 should be used and needs to occupy more OFDM symbols. Regarding QC’s first question, we don’t think it will result in total # sub-path could growing exponentially. Consider the simplest example, HP PUCCH time unit and LP PUCCH time unit is the same, there is up to 1 HP HARQ-ACK and 1 LP HARQ-ACK in a PUCCH time unit. After multiplexing HP HARQ-ACK and LP HARQ-ACK, the resultant PUCCH won’t overlap with another LP HARQ-ACK PUCCH. Consider different PUCCH time units case, for a LP PUCCH time unit, there is also up to 1 LP HARQ-ACK in a LP PUCCH time unit. After multiplexing HP HARQ-ACK and LP HARQ-ACK, the resultant PUCCH won’t overlap with another LP HARQ-ACK PUCCH in the LP PUCCH time unit. In addition, we already made the following agreement. The intention of the agreement is to avoid more than one LP HARQ-ACK to be multiplexed in a HP HARQ-ACK. We don’t support multiplexing more than one PUCCH with HARQ-ACK of a same priority in a same PUCCH/PUSCH since Rel-15, this rule should be kept in Rel-17. Hopefully, this is the common understanding. Agreement For resolving collision of LP PUCCHs and HP PUCCHs in step 2.1, a HP PUCCH with HARQ-ACK is not expected to be overlapped with multiple LP PUCCHs with HARQ-ACK. - It’s up to the editor whether/how to capture this. Regarding QC’s second question, we don’t support multiplexing LP CSI/SR with HP UCI until now, it is not possible for us to support it at the very late stage. Regarding the current spec, we share similar view as Yanping, the spec only says multiplexing HARQ-ACK of different priorities, LP CSI/SR is not included and thus should be dropped. Regarding the undo UCI multiplexing, we have similar issue since Rel-15, for example, SR is multiplexed with HARQ-ACK/CSI in a PUCCH, when the PUCCH overlaps with a PUSCH, the SR is dropped. We don’t think it is a valid issue in Rel-17. Further, if CSI consists of two part2, we only support up to 2 coding chains for PF3/4, Part 2 CSI should be dropped anyway, otherwise, we need to support 3 coding chains for PUCCH. To Intel, Sub-slot is essential for HP HARQ-ACK to reduce latency, we cannot accept to exclude sub-slot for HP HARQ-ACK. For LP HARQ-ACK, we don’t think the restriction is necessary but open for further discussion. |
| LG | We also understand the intention, but there seem confusions among the companies due to using “resultant/original” or “slot/sub-slot” and so on. In this sense, it may be better with the following direction, for example: - If a PUCCH containing HP AN and/or HP SR collided with a LP PUCCH not containing LP AN, the LP PUCCH is dropped. |
| OPPO | We share concern as QC. Different rule from Rel-15 procedure leads complexity for implementation and specification work. |
| vivo | We are fine with the proposal. We understand QC’s intension is to reuse Rel-15 Pseudo code for step 2.1, which just keep multiplexing until no overlap. However, we don’t support multiple PUCCHs with LP HARQ-ACK to be multiplexed with one HP PUCCH, we don’t support multiplex LP CSI/SR with HP HARQ-ACK either. So, if there is a LP PUCCH overlapping with the resultant PUCCH with HP and LP UCI, the LP PUCCH is for LP SR/CSI, then LP PUCCH should be dropped. Regarding QC’s second question, we think option 2/3 is fine. We don’t see any issue for option 2. Note that in R15, when a PUCCH with HARQ-ACK/CSI, it can be firstly multiplexed with SR and then is multiplexed with PUSCH, SR is dropped in that case. For option 3, we are discussing is Jia’s session on how to multiplexing LP HARQ-ACK+LP CSI+HP HARQ-ACK. If CSI includes part 2, CSI part 2 is needed to drop considering the limitation of coding chain. |

Based on the feedback above, the proposal is updated as follows. From moderator’s perspective, it is still useful to agree the proposal. Please comment only if you have problem with the proposal and please elaborate the reasons.

[Updated] Proposal 1.3.6b:

For resolving collision of PUCCHs and/or PUSCHs with different priorities in step 2.1, if resultant PUCCH with HP and LP UCI collides with LP PUCCH without HARQ ACK, the LP PUCCH is dropped.

- A resultant PUCCH with HP and LP UCI is not expected to be overlapped with a LP PUCCH with HARQ-ACK.

| Company | Comments |
| --- | --- |
| Samsung | Support |
| New H3C | Support |
| ZTE | Support |
| OPPO | Support |
| Huawei/Hisi | Support. We also agree with Ericsson’s Proposal 3.1.3.3.D: Proposal 3.1.3.3.D (similar to Lenovo point): Rel-17 does not support multiplexing a HP PUCCH with an LP PUCCH, if the LP PUCCH does not carry HARQ-ACK. When resolving the collision between a HP PUCCH and an LP PUCCH without HARQ-ACK, the LP PUCCH is dropped. If Proposal 3.1.3.3.D is agreed, the resultant PUCCH (which is the result of the multiplexing of a HP UCI and LP HARQ-ACK CB). It should not be supposed to overlap with another LP HARQ-ACK CB, since there is no legacy procedure as a reference to merge two independent HARQ-ACK CBs of the same Type and priority into one new HARQ-ACK CB. |
| LG | Support |
| Nokia/NSB | Support |

In this round, the primary target is to reach common understanding on how each option works. So in addition to share your preference between the two options, please also share/discuss how each option works.

To facilitate the discussions, moderator would like to encourage companies to elaborate how to handle the following cases with a unified solution.

Case 1                                                                  Case 2

Case 3                                                                                       Case 4

Proposal 1.3.7:

For resolving collision of LP PUCCHs and HP PUCCHs in step 2.1, down-select from:

- Option 1: Perform pair-wise collision resolution sequentially in time.

  - FFS details

- Option 2: First resolve collision among a LP PUCCH with HARQ-ACK and HP PUCCH(s), then resolve collision among LP PUCCH(s) without HARQ-ACK and HP PUCCH(s).

  - FFS details

|  | Supporting Company |
| --- | --- |
| Option 1 | Sony (needs clarification), Apple (needs clarification), Intel, ZTE Huawei/Hisi, Spreadtrum |
| Option 2 | Lenovo, Samsung, Nokia/NSB |

| Company | Comments |
| --- | --- |
| Sony | Please clarify what it means by “sequentially in time”. This can be interpreted in 2 ways: - “Sequentially in time” refers to when the scheduling of the UL channel arrives - “Sequentially in time” refers to when the UL channel is transmitted regardless of when it was scheduled in time It was argued that the UE is NOT AWARE OF the processing timeline. If this is correct, then it is impossible for UE to wait for all collisions to be scheduled then only proceed with resolving it. That is “sequentially in time” follows interpretation 1 above and Option 2 is impossible if we assume that “UE is NOT AWARE of any processing timelines”. |
| Apple | The earliest HP PUCCH should be used as reference to test overlapping channels. So we suggest to change the wording as: - Option 1: Perform pair-wise collision resolution sequentially in time with the earliest HP PUCCH should be used as reference to test overlapping channels. |
| Lenovo | In our understanding, Option 2 is closer to Rel-15 multiplexing principle with enhancement. LP PUCCH(s) without HARQ-ACK is dropped, only when it overlaps with a HP PUCCH with HP UCI and LP UCI. |
| QC | We don’t have strong preference here. Our understanding of option 1 is that it basically reuses the same principle as in Rel-15 mux pseudo code to process pairwise overlapping sequentially following the starting time of the overlapping channels. Option 2 can avoid unnecessary drop in certain cases. |
| Ericsson | Support intention of Option 1. But Option 1 build on top of these basic concepts. For example, without Proposal 3.1.3.3.D, it’s incorrect to say ‘pair-wise’, since Rel-15 multiplexing takes in a set of overlapping PUCCHs, not a pair. Only with the basic understanding below, the set becomes a pair since only a LP PUCCH carrying HARQ-ACK is considered for multiplexing. Proposal 3.1.3.3.A (related to QC concern on corner case): For a given PUCCH group, (sub-)slot duration configured for HP HARQ-ACK is shorter than or equal to the (sub-)slot duration configured for LP HARQ-ACK. Proposal 3.1.3.3.B (related to Apple concern on moving HP PUCCH resource): In Step 2.1, collision resolution is performed sequentially in units of HP (sub-)slot, starting from the first HP (sub-)slot. Proposal 3.1.3.3.C (related to Sony concern): In Step 2.1, when resolving the collision between a HP PUCCH and LP PUCCH in a HP sub-slot, cancel any LP PUCCH that starts in an earlier HP sub-slot and overlaps with a HP PUCCH in current HP sub-slot. Proposal 3.1.3.3.D (similar to Lenovo point): Rel-17 does not support multiplexing a HP PUCCH with an LP PUCCH, if the LP PUCCH does not carry HARQ-ACK. When resolving the collision between a HP PUCCH and an LP PUCCH without HARQ-ACK, the LP PUCCH is dropped. |
| Intel | We support Opt 1. In our understanding, “sequential in time” means “PUCCH resource sequential in time”, not DCI, it is same as Rel-15. Opt 1 processes PUCCH resources in time order, which is similar logic as Rel-15. But opt 1 is simpler than Rel-15, at one time, we select a pair of overlapped LP and HP PUCCH with earliest starting symbol for each priority. It is noted that, in step 2, we don’t need any additional rule to select the pair in addition to earliest starting symbol, e.g., considering ending symbol/longer or shorter duration, because the following case in Rel-15 does not exist in step 2 (overlapping with same priority is already resolved in step 1). Hope this can answer Samsung’s question in previous round. Regarding Ericsson’s comments, I agree with you that if a PUCCH with earliest starting symbol overlaps with more than one PUCCHs, it is not pair-wise. But I think just simple modification is needed to support pair-wise, e.g., change “finding all PUCCHs overlapping with a PUCCH” with “finding one PUCCH with earliest starting symbol overlapping with a PUCCH”. Opt 2 is a totally new mechanism compared with Rel-15. Opt 2 leads to different processing orders depending on the combination of overlapped LP and HP UCI types and PUCCH resource time order of HP SR and HP HARQ-ACK. Furthermore, option 2 needs further discussion how to handle a LP PUCCH overlapping with both HP HARQ-ACK and HP SR, e.g., first select HP HARQ-ACK to resolve collision, or select first overlapped HP, or process all 3 channels at a time? It would take quite a lot of time to converge. We understand the benefit of option 2 to reduce the dropping probability of LP HARQ-ACK, when LP HARQ-ACK overlaps both positive HP SR and HP HARQ-ACK, and HP SR PUCCH places before HP HARQ-ACK, and the HP HARQ-ACK or LP HARQ-ACK is not with repetition, and the LP HARQ-ACK does not multiplex with any LP PUSCH in step 1. But it is benefit for just this one case out of tens of cases. we don’t prefer to complicate UE implementation and standard effort just because of this one case. |
| Samsung | Option 1 is definitely not acceptable because of the implementation complexity and large spec impact. - Opt 1 is a totally new mechanism compared with Rel-15, it requires large spec impact. Details are not clear, it cannot be supported at the late stage. - Ordering function is needed for Option 1 to perform resolving pair-wise collision. The details include the order is based on the starting symbol or ending symbol, what if the starting symbol/ending symbol is the same, how to define the order? Should UCI be considered? - Recursive procedure is required. If a LP CSI/SR PUCCH doesn’t overlap with a HP PUCCH, but after multiplexing HP HARQ-ACK and LP HARQ-ACK, the LP CSI/SR can be overlapped with the resulting PUCCH. In this case, recursive procedure is required to drop LP CSI/SR. Re-ordering the remaining PUCCHs are also necessary. - The performance is worse. For example, according to Ericsson’s explanation, consider the example below, the LP HARQ-ACK is dropped when it can be multiplexed in the HP HARQ-ACK PUCCH. Option 2 is much simpler with better performance, LP HARQ-ACK is either multiplexed in the HP PUCCH or dropped. After that other overlapping LP PUCCH is dropped. Option 2 can increase the transmission probability of LP CSI/SR as shown in Scenario 2. |
| LG | We have similar understanding (and view) with QC. |
| New H3C | We have similar view with Samsung. |
| OPPO | Support Option 1 |
| DOCOMO | We slightly prefer Option 2 but are fine with Option 1 as well. In our understanding, Option 1 is simpler and closer to the Rel-15 multiplexing pseudo code, while Option 2 would require a new mechanism to process the overlapping channels. |
| ZTE | Option 1 is more close to the principle as in Rel-15 mux |
| Intel2 | To have a full picture, we want to understand how simpler/unified solution could option 2 be for the following cases. For each case, there can be many options. We also want to understand the cons and pros for each option for each case. Then, we can evaluate whether option 2 is better than option 1. Case 1: LP HARQ-ACK overlaps with >1 HP HARQ-ACKs without repetition Case 2: LP HARQ-ACK overlaps with ≥1 HP SR & ≥HP HARQ-ACKs without repetition Case 3: LP HARQ-ACK overlaps with ≥ 1 HP HARQ-ACKs without repetition & ≥ 1 HP HARQ-ACKs with repetition Case 4: LP HARQ-ACK overlaps with ≥ 1 HP HARQ-ACKs with repetition & ≥ 1 HP SR without repetition |
| Huawei/Hisi | Option 1 is closest to Rel-15 behaviour: find an earliest PUCCH as reference PUCCH, resolve the overlapping PUCCH with it, and keep looping until there is not any overlapping PUCCH in the slot. We anyhow need to define the time order for multiplexing of the slot regardless of option 1 and option 2. E.g., for a bunch of staggered overlapping PUCCHs (as pointed out by Intel), it is not likely to find a later located HP HARQ or LP HARQ as a reference to start with as option 2 does. In addition, we agree with Ericsson’s Proposal 3.1.3.3.B. Proposal 3.1.3.3.B (related to Apple concern on moving HP PUCCH resource): In Step 2.1, collision resolution is performed sequentially in units of HP (sub-)slot, starting from the first HP (sub-)slot. If Proposal 3.1.3.3.B is agreed, the Rel-15 pseudo code can be performed per sub-slot granularity but the principle can be reused. Note the legacy R15 procedure is: for each slot: Step 1) Find a reference PUCCH resource A with the priority order of earliest symbol followed by longest duration in the slot. Step 2) Find a set of PUCCH resources X overlapping with the PUCCH resource A. Step 3) Perform multiplexing over the set X and A to generate a resulting resource and a resulting UCI. Step 4) Loop Step 1) ~ Step 3) until there is not any overlapping PUCCHs in the slot. For R17 if the time unit is subslot, the procedure can be: Loop the subslots in time order, for each subslot: Step 1) Find a reference PUCCH resource A with the priority order of earliest symbol followed by longest duration in the subslot. Step 2) Find a set of inter-priority PUCCH resources X overlapping with the PUCCH resource A. Step 3)-1 Exclude the ineligible PUCCH that cannot support inter-priority multiplexing; if there is still overlapping, go to Step 4), otherwise go to Step 3)-2 Step 3)-2 Perform multiplexing over the set X and A to generate a resulting resource and a resulting UCI. Step 4) Loop Step 1) ~ Step 3) until there is not any overlapping PUCCHs in the subslot. Note that for the LP PUCCH which crosses the subslot boundary, regardless it is multiplexed (Case 2) or dropped (Case 1) due to the multiplexing with the HP UCI in the earliest overlapping subslot, it will not further overlap with the HP UCI in the next subslot. |
| Spreadtrum | Support Option 1 as the starting point. More details need to further clarify. - For pair-wise collision resolution, it is not exact. Such as one HP overlapping with two LP, or otherwise. - Sequentially in time, our understanding is the starting symbol of PUCCH resources, as Q in Rel-15 - Also fine with Apple’s proposal, the earliest HP PUCCH is set first in Q. |
| Nokia/NSB | Slight preference for Option 2, but clearly both can work. As noted by QC and some other companies, Option 1 would essentially reuse similar operation as in Rel-15. On the other hand, in contrast to Option 2, Option 1 might lead to unnecessarily dropping LP HARQ-ACK in some cases – as noted by Samsung. |

      - 5th round discussion

We had some initial discussions on the framework of step 2.1 but the options was described in a very high level way and details are needed.

Proposal 1.3.7:

For resolving collision of LP PUCCHs and HP PUCCHs in step 2.1, down-select from:

- Option 1: Perform pair-wise collision resolution sequentially in time.

  - FFS details

- Option 2: First resolve collision among a LP PUCCH with HARQ-ACK and HP PUCCH(s), then resolve collision among LP PUCCH(s) without HARQ-ACK and HP PUCCH(s).

  - FFS details

In this round, the intention is to provide more details options for further discussion.

Question 1.3.8: How to perform pair-wise collision resolution sequentially in time in step 2.1?

Huawei’s proposal in last round is used as a starting point. Proponents of Option 1 please comment whether any update is needed. In addition, companies are encouraged to provide your view how it works for the example cases above.

Loop the subslots in time order, for each subslot: Step 1) Find a reference PUCCH resource A with the priority order of earliest symbol followed by longest duration in the subslot. Step 2) Find a set of inter-priority PUCCH resources X overlapping with the PUCCH resource A. Step 3)-1 Exclude the ineligible PUCCH that cannot support inter-priority multiplexing; if there is still overlapping, go to Step 4), otherwise go to Step 3)-2 Step 3)-2 Perform multiplexing over the set X and A to generate a resulting resource and a resulting UCI. Step 4) Loop Step 1) ~ Step 3) until there is not any overlapping PUCCHs in the subslot.

| Company | Comments |
| --- | --- |
| Sony | Is the intention to agree on these steps for Option 1 or is this for information? Anyway, shouldn’t there be a step that generate a resultant PUCCH and then check if this resultant PUCCH overlaps with any futher PUCCH? That is: Step 1) Find a reference PUCCH resource A with the priority order of earliest symbol followed by longest duration in the subslot. Step 2) Find a set of inter-priority PUCCH resources X overlapping with the PUCCH resource A. Step 3) Exclude any LP PUCCH that with UCI that does not support inter-priority multiplexing from set X; Step 4) Perform multiplexing over the set X and A to generate a resultant PUCCH B with resulting UCI Step 5) Repeat Step 2 to 5 and replace PUCCH resource A with resultant PUCCH B until only a single PUCCH is left |
| Apple | We propose the following change (highlighted in yellow): Loop the subslots of HP PUCCHs in time order, for each subslot: Step 1) Find a reference PUCCH resource A with the HP priority order of earliest symbol followed by longest duration in the subslot. (Note by step 1.1, there won’t be any overlapping HP PUCCHs) Step 2) Find a set of inter-priority PUCCH resources X overlapping with the PUCCH resource A. Step 3)-1 Exclude the ineligible PUCCH that cannot support inter-priority multiplexing; if there is still overlapping, go to Step 4), otherwise go to Step 3)-2 Step 3)-2 Perform multiplexing over the set X and A to generate a resulting resource and a resulting UCI. Step 4) Loop Step 1) ~ Step 3) until there is not any overlapping PUCCHs in the subslot. |
| LG | We have two questions for clarification: - The intention is to follow the existing multiplexing procedure except for the parts in red mark, is it correct understanding? - Is the subslot corresponding to HP PUCCH? |
| DOCOMO | We share the similar view with Sony. It seems overlapping checking of the resultant PUCCH is missing. |
| Huawei/Hisi | @Apple: Yes the subslot should be HP subslot. Whether the reference resources should be HP can be FFS (we are open to that). @LG: Yes the red mark is the changes on top of R15 pseudo code. @Sony DOCOMO: My understanding is that Step 1) and 2) in the start of the loop is to find overlapping PUCCHs with the resultant PUCCH. So basically I think your description is the same meaning. Some changes on top of the Moderator’s version: Loop the subslots [FFS of HP PUCCHs] in time order, for each subslot: Step 1) Find a reference PUCCH resource A with the priority order of earliest symbol followed by longest duration in the subslot. - FFS the reference PUCCH resource A is HP, and the above “followed by longest duration” is not removed Step 2) Find a set of inter-priority PUCCH resources X overlapping with the PUCCH resource A. Step 3)-1 Exclude the ineligible PUCCH that cannot support inter-priority multiplexing; if there is still overlapping, go to Step 3)-2 4), otherwise go to Step 4) 3)-2 Step 3)-2 Perform multiplexing over the set X and A to generate a resulting resource and a resulting UCI. Step 4) Loop Step 1) ~ Step 3) until there is not any overlapping PUCCHs in the subslot. |
| Intel | In general, we agree with steps provided by HW, while we’d like to further discuss whether to loop per sub-slot or per slot, and whether a set of inter-priority PUCCH resources X includes single or multiple PUCCH resources in step 2). For example, if a set of inter-priority PUCCH resources X only includes one PUCCH resource with earliest symbol overlapping with reference PUCH resource A, then, UE resolves collision between one LP PUCCH and one HP PUCCH at a time. So, loop per sub-slot is equivalent to loop per slot. |
| QC | We disagree with Huawei’s proposal. With the agreement made in last GTW, we don’t think there is a loop needed. One resulting PUCCH after mux between HP PUCCH and LP PUCCH cannot continue to mux with another LP PUCCH. The loop is not needed in the pseudo code. |
| Ericsson | The Rel-15 multiplexing steps can be extended for Rel-17 inter-priority operation, as shown by other companies’ suggestions. In our view, the following proposals provide the key points in evolving existing procedure to the Rel-17 case. It’s more useful to agree on proposals like these first Proposal 3.1.3.3.A For a given PUCCH group, (sub-)slot duration configured for HP HARQ-ACK is shorter than or equal to the (sub-)slot duration configured for LP HARQ-ACK. Proposal 3.1.3.3.B In Step 2.1, collision resolution is performed sequentially in units of HP (sub-)slot, starting from the first HP (sub-)slot. Proposal 3.1.3.3.D Rel-17 does not support multiplexing a HP PUCCH with an LP PUCCH, if the LP PUCCH does not carry HARQ-ACK. When resolving the collision between a HP PUCCH and an LP PUCCH without HARQ-ACK, the LP PUCCH is dropped. When using the sequential steps above, PUCCHs of one priority (e.g., HP) are used as reference. One issue is, how to do with a PUCCH of the other priority (e.g., LP PUCCH) that start in an earlier (e.g., HP) sub-slot? The proposal below addresses this scenario in the simplest way. But we are also OK that UE multiplexes the LP PUCCH in the earlier HP sub-slot with an HP PUCCH in an later HP sub-slot. Proposal 3.1.3.3.C In Step 2.1, when resolving the collision between a HP PUCCH and LP PUCCH in a HP sub-slot, cancel any LP PUCCH that starts in an earlier HP sub-slot and overlaps with a HP PUCCH in current HP sub-slot. |
| Huawei/Hisi 2 | @QC there is no recursive for one butch of overlapping channels. But there can be more than one bunches of overlapping channels, right? BTW, even for the one bunch of channels, I think the resultant HP+LP PUCCH is possible to overlap with a LP PUCCH without HARQ-ACK, in which case the LP PUCCH will be dropped though. So a generic procedure is provided. A Note is added to make it clear. Step 1) Find a reference PUCCH resource A with the priority order of earliest symbol followed by longest duration in the subslot. - FFS the reference PUCCH resource A is HP, and the above “followed by longest duration” is not removed Step 2) Find a set of inter-priority PUCCH resources X overlapping with the PUCCH resource A. Step 3)-1 Exclude the ineligible PUCCH that cannot support inter-priority multiplexing; if there is still overlapping, go to Step 3)-2, otherwise go to Step 4) Step 3)-2 Perform multiplexing over the set X and A to generate a resulting resource and a resulting UCI. - Note: the resulting resource is not expected to overlap with another HP/LP PUCCH that would result in further multiplexing Step 4) Loop Step 1) ~ Step 3) until there is not any overlapping PUCCHs in the subslot. |
| vivo | Generally, we agree with steps provided by HW. But we have some questions. ” Exclude” in step 3)-1 is not clear to us. Does it mean dropping or not being considered for multiplexing? In addition, how to determine the “ineligible PUCCH that cannot support inter-priority multiplexing” is not clear to us, either. As shown in the following figure, if LP HARQ-ACK is selected as A, then HP SR and HP HARQ-ACK would be in the set of X. When UE performs Step 3)-1 to exclude the ineligible PUCCH that cannot support inter-priority multiplexing, it is not clear to us, how to determine “the ineligible PUCCH that cannot support inter-priority multiplexing”, since we may not support to multiplex HP SR and LP HARQ-ACK, but we support to multiplex LP HARQ-ACK and HP HARQ-ACK. In this case “the ineligible PUCCH that cannot support inter-priority multiplexing” is HP SR or LP HARQ-ACK? |
| Samsung | We would like to share our understanding of Rel-15 pseudo-code first to better align our understanding how to reuse the pseudo-code. Rel-15 Order rule - a resource with earlier first symbol is placed before a resource with later first symbol - for two resources with same first symbol, the resource with longer duration is placed before the resource with shorter duration - for two resources with same first symbol and same duration, the placement is arbitrary. Some simple examples are given below. Case 1 Case 2 Case 3 Rel-15 pseudo-code Set to the cardinality of Set to be the first symbol of resource in the slot Set to be the number of symbols of resource in the slot Set - index of first resource in set Set - counter of overlapped resources while if and resource overlaps with resource else if determine a single resource for multiplexing UCI associated with resources as described in Clauses 9.2.5.1 and 9.2.5.2 set the index of the single resource to % start from the beginning after reordering unmerged resources at next step % function that re-orders resources in current set Set to the cardinality of else end if end if end while. j is the index of current resource and o is number of the overlapping resource. How the pseudo-code works is summarized below Step 1: select overlapping resources until the next resource does not overlap with the reference resource. : reference resource (the one with smallest index) : current selected resource Step 2a: if there is at least one overlapping resource (if ) 2a-1: determine a single resource after multiplexing/prioritizing the overlapping resources in 2a-2: put the resultant PUCCH (single resource in the pseudo-code) in the set Q and replaces the resultant PUCCH with original PUCCH j 2a-3: deletes other selected overlapping PUCCHs 2a-4: reset the parameters, reorder the resources in the set Q and restart the procedure Step 2b: if there is no overlapping resource (else), take the next resource as reference resource (j=j+1) Step 3: repeat the procedure until there is no overlapping resources UE may not always selects all the overlapping channels at a time. For example, case 3, UE first chooses the SR as reference and select HARQ-ACK, after multiplexing the SR and HARQ-ACK, UE put the resultant resource in the set Q and remove the overlapping resources. Then UE restarts the procedure. The procedure is shown below For case 2, UE first chooses the HARQ-ACK as reference and select SR and CSI, then multiplex HARQ-ACK, SR, CSI in a same PUCCH. To reuse the pseudo-code, we need to resolve the following essential issues. - Determination of the time unit of Q (HP PUCCH time unit, LP PUCCH time unit, slot): Note the pseudo-code is performed per PUCCH time unit, slot for Rel-15, PUCCH time unit for a given priority for Rel-16. - If the time unit of Q is HP/LP PUCCH time unit, how to associate the LP/HP PUCCHs with a HP/LP set Q. For example, if HP PUCCH time unit is chosen, we need to put LP PUCCHs in the HP PUCCH set Q. If a LP PUCCH overlaps with multiple HP PUCCH time units and each HP PUCCH time unit is associated with a HP set Q, how to choose the associated HP set Q. - How to determine a single resource in step 2a-1.The solutions are clear for 2 overlapping channels (LP HARQ-ACK can be multiplexed with HP HARQ-ACK, LP SR/CSI will be dropped). The key issue here is the more than 2 overlapping channels (FL already gives lots of examples of these cases). In Rel-15, details of more than 2 overlapping channel cases are defined in TS38.213 9.2.5.1 UE procedure for multiplexing HARQ-ACK or CSI and SR in a PUCCH and 9.2.5.2 UE procedure for multiplexing HARQ-ACK/SR/CSI in a PUCCH. For Rel-17, one way is to follow the same rule and define rules for each case, it will require quite a lot of work. An easier way proposed by Intel is restricting the number of selected resources when reusing the pseud-code, i.e., two resources at a time. It can avoid the specification work for more than 2 overlapping channels of different priorities. |
| ZTE | Generally fine with HW’s change. The example in vivo’s comment seems valid, from common understanding, if HP SR multiplexing with LP HARQ is not allowed, the HP HARQ can multiplex with LP HARQ to save LP HARQ, but actually the LP HARQ-ACK will be dropped due to HP SR. The procedure of option 1 may need to additionally consider the cases list by Sony and Intel in next question. |

Question 1.3.9: How Option 2 works in step 2.1?

Option 2 in proposal 1.3.7 is used as a starting point. Proponents of Option 1 please comment whether any update is needed.

First resolve collision among a LP PUCCH with HARQ-ACK and HP PUCCH(s), then resolve collision among LP PUCCH(s) without HARQ-ACK and HP PUCCH(s).

| Company | Comments |
| --- | --- |
| New H3C | Regarding Option 2, we propose below procedure for option 2 Step 1) Find a reference PUCCH resource A with the HP priority order of earliest in the subslot. Step 2) Find all of inter-priority PUCCH with HARQ-ACK and resources X overlapping with the PUCCH resource A. Step 3) Exclude the ineligible PUCCH that cannot support inter-priority multiplexing and perform multiplexing over the set X and A to generate a resulting resource B Step 4) Find all of inter-priority PUCCH without HARQ ACK and with resources Y overlapping with a resulting resource B. Step 5) Exclude the ineligible PUCCH that cannot support inter-priority multiplexing and perform multiplexing over the set Y and A to generate a resulting resource C and a resulting UCI. |
| Sharp | Option 1 works, but may cause drop of LP HARQ-ACK if a HP SR overlaps with the PUCCH with LP HARQ-ACK first before a HP PUCCH with HARQ-ACK. Thus, Option 2 is preferred. The multiplxing should follow subslot order in time as Option1. But if overlapping channels includes LP HARQ-ACK, Option 2 is performed. The LP PUCCH with HARQ-ACK should be considered first because all other LP UCIs (CSI and SR) cannot be multiplexed with HP UCI. This includes several cases: - LP A/N + HP SR only: need to wait for conclusions from the other thread. For LP A/N with PF2/3/4, drop the LP A/N. For LP A/N with PF 0/1, potentially carry LP A/N on HP SR PUCCH. - LP A/N + one HP A/N only: follow agreed procedures - LP A/N + multiple HP A/N: follow timeline, the LP A/N is multiplexed with the earliest overlapping HP A/N - LP A/N + resultant HP PUCCH with (HP A/N + HP SR): follow agreed method by treating HP A/N and HP SR as HP UCI. - LP A/N + HP A/N + HP SR, but HP A/N and HP SR do not overlap. Multiplex HP A/N + LP A/N first. If resultant PUCCH overlaps with HP SR, then multiplex together with the order of HP HARQ-ACK + HP SR + LP HARQ-ACK, where the HP HARQ-ACK and HP SR together treated as HP UCI. |
| Intel | We’d like to know more details for how to resolve collision among a LP PUCCH with HARQ-ACK and HP PUCCHs in same or different sub-slot, with and without repetition, e.g., - whether to handle all overlapped channels at a time, or first select two channels to process (if yes, how to select two channels) - whether allow to first process a later sub-slot and then an earlier sub-slot Case 1: LP HARQ-ACK overlaps with >1 HP HARQ-ACKs without repetition Case 2: LP HARQ-ACK overlaps with ≥1 HP SR & ≥HP HARQ-ACKs without repetition, HP SR and HP HARQ-ACK is in same sub-slot, e.g., figure under question 1.3.11. Case 3: LP HARQ-ACK overlaps with ≥1 HP SR & ≥HP HARQ-ACKs without repetition, HP SR and HP HARQ-ACK is in different sub-slot. Case 1; Case 3. Case 4: LP HARQ-ACK overlaps with ≥ 1 HP HARQ-ACKs without repetition & ≥ 1 HP HARQ-ACKs with repetition Case 5: LP HARQ-ACK overlaps with ≥ 1 HP HARQ-ACKs with repetition & ≥ 1 HP SR without repetition Case 4; Case 5. |
| Samsung | LP PUCCH time unit is taken as the processing unit. For a LP PUCCH time unit, Step1) Resolve the collision of LP PUCCH with HARQ-ACK and HP PUCCHs. (For example, a LP PUCCH can be multiplexed in a HP HARQ-ACK PUCCH) Step 2) Resolve the collision of LP PUCCH with SR and/or CSI and HP PUCCH including the resultant PUCCH after Step1. - If there is collision, LP PUCCH with SR and/or CSI is dropped. Please note for a LP PUCCH time unit, there is up to 1 LP HARQ-ACK in a LP PUCCH time unit. After multiplexing HP HARQ-ACK and LP HARQ-ACK, the resultant PUCCH won’t overlap with another LP HARQ-ACK PUCCH in the LP PUCCH time unit. For PUCCH repetitions, a simply solution is given in section 3.1.8 To FL, regarding Intel’s question are we going to discuss jointly with PUCCH repetitions for this issue? |
| Nokia/NSB | We share similar understanding as Samsung on the general procedure to follow under Option 2. Then, the detailed handling could be essentially done based on Sharp’s suggestions 1) to 4). On Sharp’s point number 5) copied below, we are wondering whether this scenario would be valid considering the following conclusion: “For resolving collision of PUCCHs and/or PUSCHs with different priorities in step 2, a resultant PUCCH with HP and LP UCI is not expected to be overlapped with a HP PUCCH.” << 5) LP A/N + HP A/N + HP SR, but HP A/N and HP SR do not overlap. Multiplex HP A/N + LP A/N first. If resultant PUCCH overlaps with HP SR, then multiplex together with the order of HP HARQ-ACK + HP SR + LP HARQ-ACK, where the HP HARQ-ACK and HP SR together treated as HP UCI. >> On the scenarios with HP PUCCH repetition illustrated by Intel copied below, a simple solution would be to drop the LP PUCCH/UCI in this case. But we are open to further discuss these scenarios. Case 4; Case 5. |

Question 1.3.10: Do you have any additional proposal in addition to Option 1 and 2? If yes, please provide details.

| Company | Comments |
| --- | --- |
| Apple | PUCCH reference A should be at HP. |
| LG | We would be fine with Option 1 if the intention is to follow the existing multiplexing procedure except for the parts in red mark. |
| Sharp | The HP subslot can be the multiplexed unit, but the HP PUCCHs in subslots overlapping with a LP PUCCH with LP HARQ-ACK should be considered jointly. |
| QC | Simplifying option 1 by removing the loops seems the way to go. |
| New H3C2 | We have the similar with QC. Actually compared with option 1, option 2 will remove the loops. The detail handling procedure can be FFS. |

Question 1.3.11: How Option 1/2 and additional option if any work in the following case?

| Company | Comments |
| --- | --- |
| Sony | If we use Option 1, the HP SR + HP HARQ-ACK are mux with the LP HARQ-ACK + LP CSI and the resultant PUCCH would be a HP PUCCH within sub-slot #1. That is after resolving the collision for sub-slot #1, they won’t be anymore collision in sub-slot #2. |
| Apple | Using the modified Option 1 proposed by Apple as above: - In sub-slot #1, HP SR PUCCH is selected as PUCCH reference A: - LP HARQ-ACK+LP CSI PUCCH overlaps with the reference PUCCH reference A. - LP CSI is dropped, and LP HARQ-ACK and HP SR are multiplexed into a resultant HP PUCCH, which does not overlap with the HP A/N in sub-slot #1. |
| LG | We have different understanding from Sony and Apple. If HP SR PUCCH is selected as PUCCH resource A, then LP PUCCH is dropped since the LP PUCCH would be PF2/3/4. |
| Sharp | The HP SR + LP HARQ-ACK multiplexed scenarios are not concluded yet, and at least for LP HARQ-ACK with PF2/3/4, HP SR multiplexing with LP HARQ-ACK is not supported. Therefore, it is better to multiplex HP A/N and LP A/N together following Option 2. Thus, for subslot 1, the LP HARQ-ACK should be multiplexed with HP A/N first instead of HP SR. - If the resultant HP PUCCH with HP A/N and LP A/N still overlaps with the HP SR in subslot 1. Follow the HP HARQ-ACK + HP SR + LP HARQ-ACK multiplexing rules, and - If the resultant HP PUCCH with HP A/N and LP A/N does not overlap with the HP SR in subslot 1. Both HP PUCCHs are transmitted in sublot 1. The HP A/N PUCCH is transmitted in subslot 2 since the resultant HP PUCCH with HP A/N and LP A/N will be contained in subslot 1. |
| DOCOMO | Share the same understanding with Apple. Based on the procedure, it should be noted that the resultant HP PUCCH should not be overlapped with other HP PUCCH. - In sub-slot #1, HP SR PUCCH is selected as PUCCH reference A: - LP HARQ-ACK+LP CSI PUCCH overlaps with the reference PUCCH reference A. - LP CSI is dropped, and LP HARQ-ACK and HP SR are multiplexed into a resultant HP PUCCH, which does not overlap with the HP A/N in sub-slot #1. - Note: The resultant HP PUCCH should not overlap with the HP A/N in sub-slot #1 to avoid recursive operation. |
| QC | Before discussing this complicated case, we think we should discuss a simplified case which we mentioned in previous round. How to deal with the following scenario, which is basically the above scenario provided by FL but with the two HP A/N removed? • If a LP PUCCH has LP HARQ-ACK + LP CSI (result from step 1.1) and it overlap with a HP PUCCH. Based on some companies insisted that LP CSI cannot be muxed with HP PUCCH, what should UE do? Which of the following is UE behavior?  Option 1: UE drop the LP PUCCH including LP HARQ-ACK and CSI  Option 2: UE only drop the LP CSI and mux LP HARQ-ACK on HP PUCCH  Option 3: UE mux both LP CSI and LP HARQ-ACK on HP PUCCH. By the way, we don’t think RAN1 need spend effort to study these complicated but corner cases as FL listed here and subsequently. This is an endless path. There are more complicated cases with 4 channels, 5 channels, 6 channels… overlapping. We just need to define the proposal for reasonable scenario and be done with it. |
| Ericsson | Agree with others that Rel-17 multiplexing rule is applied in multiplexing the 3 overlapping PUCCHs in sub-slot#1. No multiplexing is needed in sub-slot#2. |
| New H3C | We have the similar view with Sharp for handling this case with option 2. |
| Huawei/Hisi | Firstly, we agree with QC to add a separate issue to discuss the LP HARQ+LP CSI vs HP SR case and LP HARQ+LP CSI vs HP HARQ-ACK (+HP SR if any) case. For the HP SR case, we think Option 3 from QC is off the table (SR only supports PF0/1). From our view, Option 2 seems more reasonable. For the HP HARQ-ACK (+HP SR if any) case, as the LP CSI will be fully dropped due to the encoding chain number, Option 3 should not be considered either. As a unified solution, Option 2 is preferred. I change the notation to avoid conflict with the Option 1/2 for this proposal. - Option Alt 1: UE drops the LP HARQ-ACK and LP CSI in together - Option Alt 2: UE multiplexes LP HARQ-ACK with HP SR and drops LP CSI For Option 1 of the proposal, there can be different ways if we select different reference PUCCH (as discussed in previous proposal) Alt.1: Earliest HP PUCCH is selected as the reference PUCCH, i.e., HP SR is the reference PUCCH. Then the result is (based on Alt 2) HP SR + LP HARQ-ACK on HP SR while LP CSI dropped, or LP HARQ-ACK + LP CSI dropped. This is aligned with Apple/DOCOMO. Alt.2: Earliest/longest PUCCH is selected as the reference PUCCH, i.e., LP PUCCH is the reference PUCCH. Then the result is: LP HARQ-ACK, HP HARQ-ACK and HP SR on a resultant HP PUCCH using HP HARQ-ACK resource. |
| Intel | For option 1: Alt1 (same as Alt 2 provided by HW): strictly follow Rel-15 procedure for resource determination Reference PUCCH A is LP PUCCH. A set of PUCCH resources X is HP SR and HP HARQ-ACK. Then, the resultant PUCCH resource is HP PUCCH resource (HP HARQ-ACK) carrying HP HARQ-ACK, HP SR and LP HARQ-ACK, while LP CSI is dropped. The procedure is done. UE transmits the resultant HP PUCCH in sub-slot 1 and HP A/N in sub-slot 2. Alt 2: follow Rel-15 procedure with minor modification of only selecting one inter-priority PUCCH resource with earliest starting symbol overlapped with PUCCH reference A as PUCCH resource X (Rel-15 procedure is to select all PUCCHs overlapped with PUCCH reference A) Reference PUCCH A is LP PUCCH. PUCCH resources X is HP SR. Then, UE drops LP HARQ-ACK and LP CSI (assuming multiplexing LP HARQ-ACK with HP SR is not supported). The procedure is done. UE transmits HP SR, HP AN in sub-slot 1 and HP AN in sub-slot 2. Alt 3 (same as Alt 1 provided by HW): Select Earliest HP PUCCH as reference PUCCH A (as suggested by Apple) Reference PUCCH A is HP SR, PUCCH resource X is LP PUCCH. Then, UE drops LP HARQ-ACK and LP CSI (assuming multiplexing LP HARQ-ACK with HP SR is not supported). The procedure is done. UE transmits HP SR, HP AN in sub-slot 1 and HP AN in sub-slot 2. It can be seen that the outcome of Alt 2 and Alt 3 is the same. Alt 1 and Alt 3 is based on sub-slot unit. Alt 2 can work with slot as time unit, or sub-slot as time unit, the outcome is the same. Regarding the scenario mentioned by QC, we prefer Opt 2, i.e., only drop LP CSI. In our understanding, if all overlapping channels before step 1 meets Rel-15 timeline, the presence of all these channels can be seen by UE before the deadline when UE starts 1st step of intra-UE multiplexing procedure. |
| Nokia/NSB | Based on Option 2 as described in our response to Question 1.3.9, LP HARQ-ACK would be multiplexed with HP HARQ-ACK in the first sub-slot; LP CSI is dropped. Nothing else would be needed as the resultant PUCCH carrying HP+LP HARQ-ACKs is not expected to overlap with a PUCCH carrying HP UCI, based on an earlier RAN1 conclusion (see sec. 6). Therefore, we don’t think any handling needs to be captured (in contrast to some other companies’ inputs here). |
| ZTE | My understanding is very similar with Apple’s view if option 1 is applied. |

Question 1.3.12: How Option 1/2 and additional option if any work in the following case?

| Company | Comments |
| --- | --- |
| Sony | Using Option 1, LP CSI will not be considered for mux (based on previous agreements, I don’t think we support mux of LP CSI + HP SR). HP SR will firstly mux with LP HARQ-ACK giving a resultant HP PUCCH. If the resultant HP PUCCH carrying HP SR + LP HARQ-ACK does not overlap with HP HARQ-ACK then procedure ends. If the resultant HP PUCCH carrying HP SR + LP HARQ-ACK overlaps with HP HARQ-ACK then this leads to recursion in the procedure and therefore this should be an error case. |
| Apple | Using the modified Option 1 proposed by Apple as above: Since feature lead does not sub-slots/slots, we assume every PUCCH is with the same sub-slot/slot - HP SR PUCCH is selected as PUCCH reference A: - LP HARQ-ACK PUCCH and LP CSI PUCCH overlap with the reference PUCCH reference A. - LP CSI is dropped, and LP HARQ-ACK and HP SR are multiplexed into a resultant HP PUCCH, which does not overlap with the HP A/N |
| LG | We also have same understanding with Sony and Apple. |
| Sharp | Again, the HP SR + LP HARQ-ACK multiplexed scenarios are not concluded yet, and at least for LP HARQ-ACK with PF2/3/4, HP SR multiplexing with LP HARQ-ACK is not supported. Therefore, it is better to multiplex HP A/N and LP A/N together following Option 2 and time ordering. - The HP SR is dropped by the HP SR. - The LP A/N is multiplexed with HP A/N on a HP PUCCH. - If the resultant HP PUCCH with HP A/N + LP A/N does not overlap with the HP SR, both HP PUCCHs are transmitted. - If the resultant HP PUCCH with HP A/N + LP A/N overlaps with the HP SR, a HP PUCCH is used to report HP A/N + HP SR + LP A/N, where the HP A/N and HP SR together is treated as HP UCI. |
| DOCOMO | Share the same understanding as Sony and Apple. |
| QC | We don’t think RAN1 need spend effort to study these complicated but corner cases as FL listed here and subsequently. This is an endless path. There are more complicated cases with 4 channels, 5 channels, 6 channels… overlapping. We just need to define the proposal for reasonable scenario and be done with it. |
| Ericsson | First, the HP sub-slot grid should be shown. Then the procedure is applied sequentially starting from the first HP sub-slot. If all boxes belong to a same HP sub-slot, then LP CSI is dropped, while the other 3 PUCCHs are multiplexed. |
| New H3C | Based on option 2, the handling procedure is as follows: - If the resultant HP PUCCH with HP A/N + LP A/N does not overlap with the HP SR, both HP PUCCHs are transmitted. - If the resultant HP PUCCH with HP A/N + LP A/N+HP SR overlaps with LP CSI, LP CSI is dropped. |
| Huawei/Hisi | Option 1, and assuming all channels in one subslot: Alt.1: Earliest HP PUCCH is selected as the reference PUCCH. Same understanding with Apple/LG/DOCOMO if LP HARQ-ACK is PF0/1; if LP HARQ-ACK is PF2/3/4, LP HARQ-ACK is dropped. Alt.2: Earliest/longest PUCCH is selected as the reference PUCCH. LP CSI is selected as the reference PUCCH but dropped in the first round; then HP SR is selected as the reference PUCCH, and the result is the same as Alt.1. |
| Intel | For option 1, assuming all channels in one sub-slot. Alt1 (same as Alt 2 provided by HW): strictly follow Rel-15 procedure for resource determination Reference PUCCH A is LP CSI. PUCCH resource X is HP SR. UE drops LP CSI. Then, HP SR becomes reference PUCCH A, PUCCH resource X is LP HARQ-ACK. UE drops LP HARQ-ACK (assuming multiplexing LP HARQ-ACK with HP SR is not supported). Procedure is done. UE transmits HP SR and HP HARQ-ACK. Alt 2: follow Rel-15 procedure with minor modification of only selecting one inter-priority PUCCH resource with earliest starting symbol overlapped with PUCCH reference A as PUCCH resource X (Rel-15 procedure is to select all PUCCHs overlapped with PUCCH reference A) The procedure and outcome are same as Alt1. Alt 3 (same as Alt 1 provided by HW): Select Earliest HP PUCCH as reference PUCCH A (as suggested by Apple) Reference PUCCH A is HP SR, a set of PUCCH resources X is LP CSI and LP HARQ-ACK. Then, UE drops LP HARQ-ACK and LP CSI (assuming multiplexing LP HARQ-ACK with HP SR is not supported). The procedure is done. UE transmits HP SR and HP HARQ-ACK. It can be seen that the outcome of Alt 1/2/3 is the same. Alt 2 can work with slot as time unit, or sub-slot as time unit, the outcome is the same. |
| Nokia/NSB | Based on Option 2 as described in our response to Question 1.3.9, - LP HARQ-ACK would be multiplexed with the HP HARQ-ACK. - if HP SR is positive, LP CSI is dropped There is no need for any handling of possible overlap of the HP SR & HP PUCCH with HP & LP HARQ-ACK, as the overlap is not expected based on the RAN1#107bis-e conclusion. |
| ZTE | Same consequence with Apple mentioned |

Question 1.3.13: How Option 1/2 and additional option if any work in the following case?

| Company | Comments |
| --- | --- |
| Sony | Since the resultant PUCCH carrying HP HARQ-ACK + LP HARQ-ACK does not overlap with LP CSI, isn’t the process already completed? |
| Apple | Using the modified Option 1 proposed by Apple as above: Since feature lead does not sub-slots/slots, we assume every PUCCH is with the same sub-slot/slot - HP HARQ-ACK PUCCH is selected as PUCCH reference A: - LP HARQ-ACK PUCCH and LP CSI PUCCH overlap with the reference PUCCH reference A. - LP CSI is dropped, and LP HARQ-ACK and HP HARQ-ACK are multiplexed into a resultant HP PUCCH |
| LG | The intention of above question seems to ask whether to perform multiplexing for all three PUCCHs at the same time (LP CSI is dropped in this case) or to perform multiplexing for two HARQ-ACK PUCCHs first (LP CSI is not dropped in this case). Is it correct understanding? |
| Sharp | LP CSI cannot be multiplexed with HP A/N. With Option 1, the LP CSI should be dropped. The go to later multiplexing, the HP A/N + LP A/N is reported on the resultant HP PUCCH. With Option 2, the LP HARQ-ACK multiplexing is considered first, and gives the resultant PUCCH with HP A/N + LP A/N. If it does not overlap with the CSI, the CSI can be reported. This actually shows another example that Option 2 is better than Option 1 alone. However, to make things easier, a combined Option 1 + Option 2, can perform collision handling in time, Option 2 handling is only for overlapping with LP A/N. This will provide the same results as Option 1 in this example. |
| QC | We don’t think RAN1 need spend effort to study these complicated but corner cases as FL listed here and subsequently. This is an endless path. There are more complicated cases with 4 channels, 5 channels, 6 channels… overlapping. We just need to define the proposal for reasonable scenario and be done with it. |
| New H3C | For above case, if the resultant PUCCH with HP A/N + LP A/N does not overlap with the CSI, the CSI can be reported. |
| Huawei/Hisi | Option 1, and assuming all channels in one subslot: Alt.1: Earliest HP PUCCH is selected as the reference PUCCH. HP HARQ-ACK selected as reference PUCCH; LP CSI is dropped, and HP HARQ-ACK + LP HARQ-ACK on the resultant PUCCH. Alt.2: Earliest/longest PUCCH is selected as the reference PUCCH. LP CSI is selected as the reference PUCCH but dropped in the first round; then HP HARQ-ACK selected as reference PUCCH, and the result is the same as Alt.1. |
| Intel | For option 1, assuming all channels in one sub-slot. Alt1 (same as Alt 2 provided by HW): strictly follow Rel-15 procedure for resource determination Reference PUCCH A is LP CSI. PUCCH resource X is HP HARQ-ACK. UE drops LP CSI. Then, HP HARQ-ACK becomes reference PUCCH A, PUCCH resource X is LP HARQ-ACK. The resultant channel is a HP PUCCH carrying HP and LP HARQ-ACK. Procedure is done. UE transmits a HP PUCCH carrying HP and LP HARQ-ACK. Alt 2: follow Rel-15 procedure with minor modification of only selecting one inter-priority PUCCH resource with earliest starting symbol overlapped with PUCCH reference A as PUCCH resource X (Rel-15 procedure is to select all PUCCHs overlapped with PUCCH reference A) The procedure and outcome are same as Alt1. Alt 3 (same as Alt 1 provided by HW): Select Earliest HP PUCCH as reference PUCCH A (as suggested by Apple) Reference PUCCH A is HP HARQ-ACK, a set of PUCCH resources X is LP CSI and LP HARQ-ACK. Then, UE drops LP CSI. The resultant channel is a HP PUCCH carrying HP and LP HARQ-ACK. Procedure is done. UE transmits a HP PUCCH carrying HP and LP HARQ-ACK. It can be seen that the outcome of Alt 1/2/3 is the same. Alt 2 can work with slot as time unit, or sub-slot as time unit, the outcome is the same. |
| Nokia/NSB | Based on Option 2 as described in our response to Question 1.3.9, as also mentioned by some other companies (such as Sharp), if the resultant PUCCH with HP A/N + LP A/N does not overlap with the CSI, the CSI can be reported. As our understanding is, that we first consider the multiplexing of HARQ-ACK before considering dropping LP PUCCH with UCI other than HARQ-ACK. |

      - 6th round discussion

3.1.3.5.1 Rel-15 pseudo code for UCI multiplexing of overlapping PUCCHs (for information)

In Rel-15, UCI multiplexing of overlapping PUCCHs without repetition is performed according to pseudo-code defined in TS38.213 Clause 9.2.5. The procedure can be summarized as the following steps.

For PUCCHs without repetition within a slot/sub-slot,

- Step 1: Determine a reference PUCCH resource.

- Step 2: Select O PUCCH resource(s) overlapping with the reference PUCCH resource.

- Step 3: Determine a single PUCCH resource for multiplexing UCI associated with the reference PUCCH resource and O PUCCH resource(s).

- Step 4: Loop Step 1 ~ Step3 until there is no overlapping PUCCHs in the slot/sub-slot.

To be more specific, the reference PUCCH resource in step 1 is resource which is the PUCCH resource with index j-o (the initial value of o is zero) within set Q ordered according to the earliest starting symbol followed by longest duration.

- a resource with earlier first symbol is placed before a resource with later first symbol - for two resources with same first symbol, the resource with longer duration is placed before the resource with shorter duration

For Step 2, all the PUCCH resources overlapping with resource  are selected one by one with increasing order of index j until the next PUCCH resource does not overlap with resource.

if and resource overlaps with resource else

In Step 3, a single PUCCH resource for multiplexing UCI associated with the overlapping channels identified after step 2. The resultant PUCCH resource is added in set Q and the group of overlapping PUCCH resources are removed from set Q.

if determine a single resource for multiplexing UCI associated with resources as described in Clauses 9.2.5.1 and 9.2.5.2 set the index of the single resource to

Some examples provided by Samsung in 5th round illustrate the procedure, which are copied here for reference.

Set to the set of resources for transmission of corresponding PUCCHs in a single slot without repetitions where - a resource with earlier first symbol is placed before a resource with later first symbol - for two resources with same first symbol, the resource with longer duration is placed before the resource with shorter duration - for two resources with same first symbol and same duration, the placement is arbitrary - the above three steps for the set are according to a subsequent pseudo-code for a function - a resource for negative SR transmission that does not overlap with a resource for HARQ-ACK or CSI transmission is excluded from set - if the UE is not provided simultaneousHARQ-ACK-CSI and resources for transmission of HARQ-ACK information include PUCCH format 0 or PUCCH format 2, resources that include PUCCH format 2, or PUCCH format 3, or PUCCH format 4 for transmission of CSI reports are excluded from the set if they overlap with any resource from the resources for transmission of HARQ-ACK information - if the UE is not provided simultaneousHARQ-ACK-CSI and at least one of the resources for transmission of HARQ-ACK information includes PUCCH format 1, PUCCH format 3, or PUCCH format 4 - resources that include PUCCH format 3 or PUCCH format 4 for transmission of CSI reports are excluded from the set - resources that include PUCCH format 2 for transmission of CSI reports are excluded from the set if they overlap with any resource from the resources for transmission of HARQ-ACK information Set to the cardinality of Set to be the first symbol of resource in the slot Set to be the number of symbols of resource in the slot Set - index of first resource in set Set - counter of overlapped resources while if and resource overlaps with resource else if determine a single resource for multiplexing UCI associated with resources as described in Clauses 9.2.5.1 and 9.2.5.2 set the index of the single resource to % start from the beginning after reordering unmerged resources at next step % function that re-orders resources in current set Set to the cardinality of else end if end if end while The function performs the following pseudo-code { while % the next two while loops are to re-order the unmerged resources while if OR end if end while end while }

        - Rel-17 collision resolution of PUCCHs of different priorities in Step 2.1

General steps for step 2.1

Based on the 5th round discussion, it seems that majority companies would like to reuse Rel-15 procedure with some updates.

The procedure provided in section 3.1.3.5.1 for Rel-15 PUCCH collision handling is copied below for reference.

For PUCCHs without repetition within a slot/sub-slot, - Step 1: Determine a reference PUCCH resource. - Step 2: Select O PUCCH resource(s) overlapping with the reference PUCCH resource. - Step 3: Determine a single PUCCH resource for multiplexing UCI associated with the reference PUCCH resource and O PUCCH resource(s). - Step 4: Loop Step 1 ~ Step3 until there is no overlapping PUCCHs in the slot/sub-slot.

The procedure can be reused except that for Rel-17 intra-UE multiplexing, not all UCI(s) are eligible for UCI multiplexing so it is not possible to always determine a sing PUCCH resource to multiplex all the associated UCIs.

In addition, Qualcomm commented that there is no need for a loop since one resultant PUCCH cannot be multiplexed with another PUCCH. Although it is true that we agreed that a resultant PUCCH with HP and LP UCI is not expected to be overlapped with a HP PUCCH or a LP PUCCH with HARQ-ACK, the loop is still needed for the cases as shown in the following figures provided by Huawei.

The following proposal is proposed for step 2.1 by updating the 3rd step compared with Rel-15 considering that not all UCI(s) are eligible for UCI multiplexing in Rel-17 intra-UE multiplexing.

Proposal 1.3.14:

For resolving collision of PUCCHs of different priorities without repetitions within a time unit, Step 2.1 consists of the following sub-steps:

- Step 2.1-1: Determine a reference PUCCH resource

- Step 2.1-2: Select O PUCCH resource(s) overlapping with the reference PUCCH resource.

- Step 2.1-3: Apply Rel-17 intra-UE multiplexing/dropping rules to resolve overlapping among the reference PUCCH resource and O PUCCH resource(s).

- Step 2.1-4: Loop Step 2.1-1) ~ Step 2.1-3) until there are no overlapping PUCCHs in the time unit.

FFS details

|  | Company |
| --- | --- |
| Support | LG, QC Huawei/Hisi, Samsung, DOCOMO, Intel, Sharp, NEC,OPPO, ZTE |
| Not support |  |

| Company | Comments |
| --- | --- |
| QC | Thanks for the discussion and clarification from FL and other companies. We are fine with the proposal. What we meant in last round of email discussion was that in Step 2.1-3, there is no need to have a loop to decide mux over the O PUCCH resources as a resulting PUCCH (from HP and LP HARQ-ACK mux) does not expect to mux with any other PUCCHs in set of O PUCCH resources and even PUCCHs out of the set of O PUCCH resources. Because of this restriction, some of the details of Rel-15 Pseudo code should be updated. But we can leave the details discussion for later meetings. |
| New H3C | We wonder whether the loop is necessary or not. If we know the whole PUCCH resource(s) overlapping situation (mentioned by previous Option 2), we can resolve UCI multiplexing issue of overlapping PUCCHs based on above-mentioned Step2.1-1 to Step 2.1.-3 one time. |
| Samsung | Thanks for FL for the comprehensive analysis, we think the proposal is a good starting point to make progress. @New H3C, we share the same understanding as FL that the loop is necessary. This is to avoid defining UE behavior for all complicated overlapping cases, similar issue exist in Rel-15, and we introduced the loop to solve this issue. |
| Sharp | Depending on the reference PUCCH selection, the loop may not be necessary. Especially, if the HP PUCCH resources is used as reference resource, it can only overlap with one LP PUCCH in a subslot. The only potential loop if the resultant HP PUCCH overlaps with another HP PUCCH (did we already exclude this case?). |

Options for time unit, reference resource, overlapping resource selection

Following the above proposal, we continue discussing how to define the time unit considering the time units for HP and LP can be different, how to determine a reference PUCCH resource and how to select overlapping PUCCH resources. The proposals are categorized into the following options and explained below.

Option 1:

The idea of Option 1 is to reuse Rel-15 pseudo code by restricting to select up to one overlapping PUCCH resource with the reference PUCCH resource so that pair-wise collision handling between PUCCHs of different priorities can be performed. The time unit for Option 1 is FFS.

- The reference PUCCH resource is determined as in Rel-15, i.e. based on the starting symbol and duration

- In step 2.1-2, select up to one PUCCH resource overlapping with the reference PUCCH resource according to Rel-15 pseudo code

- FFS time unit used for step 2.1

For the above example, assuming time unit is slot, the set Q includes LP PUCCH with HARQ-ACK, HP PUCCH with SR and HP PUCCH with HARQ-ACK. Firstly, LP PUCCH with HARQ-ACK is selected as reference PUCCH resource due to earliest starting symbol. Step 2.1-2 stops after finding HP PUCCH with SR because up to one overlapping PUCCH is selected for Option 1 although there is another overlapping HP PUCCH. Then collision handling between LP HARQ-ACK and HP SR is performed according to Rel-17 rule. LP PUCCH is dropped and there are no overlapping PUCCHs within the slot so that Step 2.1 is completed.

For the same example, assuming time slot is HP PUCCH time unit, i.e. sub-slot, depending on how to associate the LP PUCCH with a sub-slot, LP PUCCH is either associated to sub-slot #1 or sub-slot 2. The following alternatives can be considered:

- Alt. 1: A LP PUCCH overlapping with multiple time units is associated with the first time unit with overlapping HP PUCCH(s)

- Alt. 2: A LP PUCCH overlapping with multiple time units is associated with the first time unit with overlapping HP HARQ-ACK if any. Otherwise, a LP PUCCH overlapping with multiple time units is associated with the first time unit with overlapping HP PUCCH(s).

LP PUCCH is associated with sub-slot #1 according to Alt. 1 and associated with sub-slot #2 according to Alt. 2. Then in the sub-slot, collision handling between a LP PUCCH and a HP PUCCH is performed according to Rel-17 rule.

Option 2:

The idea of Option 2 is to reuse Rel-15 pseudo code and use HP time unit for step 2.1. Assuming HP time unit is used for Option 1 as well, the only difference between Option 1 and Option 2 is that Option 1 selects up to one overlapping PUCCH resource(s) while Option 2 selects all the overlapping PUCCH resource(s) as in Rel-15.

- The time unit of HP HARQ-ACK is used for step 2.1

- The reference PUCCH resource is determined as in Rel-15, i.e. based on the starting symbol and duration

- In step 2.1-2, select all the PUCCH resources overlapping with the reference PUCCH resource according to Rel-15 pseudo code

For the above example, LP PUCCH with HARQ-ACK is associated with sub-slot#1 or sub-slot#2 based on the two alternatives as for Option 1. For the former case, LP HARQ-ACK is dropped so that there is no overlapping in both sub-slots. For the latter case, LP PUCCH is the reference PUCCH resource due to earliest starting symbol. Then it needs to be discussed how to handle collision of LP PUCCH with HARQ-ACK and multiple non-overlapping HP PUCCHs, which is discussed later.

Option 3:

In order to avoid discussing the case of collision handling of LP PUCCH with HARQ-ACK and multiple non-overlapping HP PUCCHs, option 3 prioritizes HP PUCCH for reference PUCCH selection.

- The time unit of HP HARQ-ACK is used for step 2.1

- The reference PUCCH resource is determined by prioritizing HP PUCCH over LP PUCCH on top of Rel-15 rules

- In step 2.1-2, select all the PUCCH resources overlapping with the reference PUCCH resource according to Rel-15 pseudo code

For the example under Option 2, assuming LP PUCCH is associated with sub-slot 2, in sub-slot 2, HP PUCCH with SR is determined as the reference PUCCH resource since it is HP PUCCH although LP PUCCH starts earlier. Then collision between LP HARQ-ACK and HP HARQ-ACK is resolved by dropping LP HARQ-ACK so that the collision is resolved in sub-slot #2.

Option 4:

For Option 3, LP HARQ-ACK is dropped in sub-slot #2 since HP SR is earlier than HP HARQ-ACK. In order to avoid LP HARQ-ACK dropping in this case, Option 4 prioritizes HP HARQ-ACK for overlapping PUCCH selection.

In addition, to avoid LP HARQ-ACK dropping in the following case, Option 4 prioritizes LP HARQ-ACK for reference PUCCH determination.

- The time unit of HP HARQ-ACK is used for step 2.1

- The reference PUCCH resource is determined by prioritizing LP PUCCH carrying HARQ-ACK on top of Rel-15 rules

- In step 2.1-2, If a LP PUCCH carrying HARQ-ACK overlaps with multiple HP PUCCHs and one of the HP PUCCH includes HARQ-ACK, only select the HP PUCCH including HARQ-ACK in step 2.1-2; otherwise, select all the PUCCH resources overlapping with the reference PUCCH resource according to Rel-15 pseudo code

Then for the above case, LP PUCCH with HARQ-ACK is determined as reference PUCCH resource although HP SR starts earlier. In addition, HP PUCCH with HARQ-ACK is selected in step 2.1-2, so LP HARQ-ACK is multiplexed with HP HARQ-ACK so that there are no overlapping PUCCHs.

Given that Option 4 intends to avoid LP HARQ-ACK dropping, Alt 2 in Option 1 for sub-slot association for long LP PUCCH is more reasonable. Then for the case under Option 2, in sub-slot #2, LP PUCCH with HARQ-ACK is the reference PUCCH resource and HP PUCCH with HARQ-ACK is selected in step 2.1-2. LP HARQ-ACK is multiplexed with HP HARQ-ACK and the collision is resolved.

It is proposed to agree to use HP time unit for step 2.1 which applies to Option 1 as well although not absolutely necessary. But still as analysed above, it can potentially bring additional benefit.

Proposal 1.3.15:

For resolving collision of PUCCHs of different priorities without repetition within a time unit, the time unit of HP HARQ-ACK is used.

For a LP PUCCH overlapping with multiple time units, down-select from:

- Alt. 1: the LP PUCCH is associated with the first time unit with overlapping HP PUCCH(s)

- Alt. 2: the LP PUCCH is associated with the first time unit with overlapping HP PUCCH with HARQ-ACK if any. Otherwise, the LP PUCCH is associated with the first time unit with overlapping HP PUCCH(s).

[Update based on inputs] Proposal 1.3.15a:

For resolving collision of PUCCHs of different priorities without repetition within a time unit, the time unit of HP HARQ-ACK is used.

For a LP PUCCH overlapping with multiple time units, down-select from:

- Alt. 1: the LP PUCCH is associated with the first time unit with overlapping HP PUCCH(s)

- Alt. 2: the LP PUCCH is associated with the first time unit with overlapping HP PUCCH with HARQ-ACK if any. Otherwise, the LP PUCCH is associated with the first time unit with overlapping HP PUCCH(s).

- Alt. 3: the LP PUCCH is associated with the last time unit with overlapping HP PUCCH(s)

|  | Company |
| --- | --- |
| Support | Samsung Huawei/Hisi,OPPO |
| Not support | QC, Intel (We do not object, but we suggest to discuss after proposal 1.3.16, because it depends on which option is chosen) |

|  | Supporting Company |
| --- | --- |
| Alt. 1 | Huawei/Hisi, DOCOMO (2nd) |
| Alt. 2 | LG, Samsung, DOCOMO (1st), Sharp, NEC,OPPO, ZTE |

| Company | Comments |
| --- | --- |
| LG | Since the overlapping case in above would occur not rarely, Alt 2 is desirable to avoid unnecessary dropping of LP HARQ-ACK PUCCH since it would be guaranteed to be multiplexed with HP HARQ-ACK PUCCH. |
| QC | We still don’t see strong motivation to deviate from Rel-15 procedure, i.e., just using slot as time unit to run the Pseudo code as in Proposal 1.3.14. Basically, our preference is aligned with option 1. But we don’t see why need FFS time slot and why need restriction to allow only 1 PUCCH to overlap with the reference PUCCH (Yes, mux is up to 1 but overlapping >1 is allowed as LP PUCCH without A/N will be dropped). Why the simplest solution, i.e., Proposal 1.3.14 with slot as time unit, cannot work? We don’t see the motivation to take more complicated route with either Alt 1 or Alt 2. Our proposal is very simple: For resolving collision of PUCCHs of different priorities without repetition, the time unit to perform Step 2.1 is the PUCCH slot (as in Rel-15). |
| Huawei/Hisi | As per our understanding, the time unit means in what time range will the UE handle the overlapping PUCCHs. If the time unit is subslot, it means only the overlapping channels within the subslot will be considered for multiplexing, while the PUCCH placed in another subslot will not be counted in. However, Alt.2 needs the UE to search more than one subslot for the HP HARQ-ACK availability if there is a LP PUCCH (regardless it is the reference PUCCH, or a PUCCH overlapping with the reference PUCCH) crossing multiple subslots, but performs the multiplexing by using the PUCCHs within one target subslot. So it should not be categorized to the first paragraph of ‘HP HARQ-ACK time unit’, but rather to ‘LP PUCCH time unit’, or ‘hybrid HP&LP HARQ-ACK time unit’. Between Alt.1 and Alt.2, we prefere Alt.1 since if the LP PUCCH time unit is adopted (consider it is slot based), it is complicated to both spec and implementation. From the spec perspective, an additional step needs to be introduced to determine the target subslot (consider HP is subslot based), which is not one-way operation (go through all overlapping HP PUCCHs and back to the beginning if there is no HP HARQ-ACK); and more additional open issues need to be discussed, such as: 1) how to select the target HP subslot if the HP HARQ-ACKs include repetitions; 2) how to select the target HP subslot if the LP HARQ-ACK cannot be multiplexed into the HP HARQ-ACK subslot, e.g., remaining RE allocated for LP HARQ-ACK = 0, etc. In addition, if the intention of Alt.2 is to only restrict LP PUCCH with HARQ-ACK as the reference PUCCH (i.e., Option 4), addition rules need to be discussed, such as: How to determine the reference PUCCH where none of the LP PUCCHs over the slot include HARQ-ACK (re-select the reference PUCCH by reusing R15 rule, or re-select other LP PUCCH without HARQ-ACK, or re-select a HP PUCCH, etc.) From the implementation perspective, the whole processing has to introduce two steps - the UE/gNB first look through all the overlapped subslots to determine where to multiplex the LP PUCCH, then proceed within per subslot for handling the multiplexing operation. In addition, the subslot processing is from back to front - first handle the multiplexing on the later subslot, then go back to the earlier subslot for handling the HP SR. In the end, as whether to support multiplexing LP PUCCH PF 0/1 with HP SR is still under discussion, the benefit of multiplexing LP PUCCH into the HP HARQ-ACK subslot is not justified - if the multiplexing LP PUCCH PF 0/1 with HP SR is supported, the latency of LP HARQ-ACK is shorter. |
| QC2 | Even if HP UCI time unit is used, i.e., even if the first sentence of the proposal is adopted, we don’t prefer either Alt 1 nor Alt 2, especially Alt 2, for resolving a LP PUCCH overlap with multiple HP time units. The problem of Alt 2 is that it creates different UE behavior depends on HP PUCCH has HARQ-ACK or not. In general, we want a unified procedure regardless of UCI types. Alt 1 is slightly better than Alt 2. But the justification/benefit of Alt 1 is not very clear neither. For example, why not take another alternative (Alt 3) to put LP HARQ-Ack on the last overlapping HP PUCCH, which will give UE more time to process multiplexing. We admit the benefit of Alt 3 is secondary. But there is no clear benefit to put it on first overlapping HP PUCCH neither. Therefore, we suggest add Alt 3 for consideration: • Alt. 3: the LP PUCCH is associated with the last time unit with overlapping HP PUCCH(s) |
| Samsung | Share similar view as LG. For intra UE multiplexing in Rel-17, PUCCH multiplexing focuses on multiplexing of LP HARQ-ACK and HP HARQ-ACK, LP SR/CSI is not considered to multiplex with HP UCI. Clearly, the intention is to avoid LP HARQ-ACK dropping. The same principle should apply for more than two overlapping channels. Compared with Alt1, Alt 2 can help avoid LP HARQ-ACK dropping. @QC, in Rel-16, if sub-slot is configured, sub-slot is used for resolving PUCCHs collision. One benefit of using sub-slot is that latency can be reduced. For example, if slot is used as the time unit and the HP sub-slot is 2OS length. The procedure cannot start until all the candidate PUCCHs have been put in the set Q. It seems that the HP PUCCH in the first HP sub-slot needs to wait for resolving the PUCCHs in the last HP sub-slot. In this case, the HP PUCCH in the first HP sub-slot cannot be transmitted until resolving the collision of the last HP sub-slot. In addition, using slot as the time unit will complicate UE/gNB implementation. The first 3 steps iterate until there is no overlapping PUCCHs, every time after multiplexing/prioritization, the procedure starts from the beginning. Clearly, dividing the PUCCHs into the sets of sub-slot can help reducing the processing time. @Huawei, we already agreed that all the overlapping channels satisfied the Rel-15 timeline. The PDCCH scheduling a HP HARQ-ACK PUCCH in the later sub-slot should also come Tproc,2 before the starting of the LP PUCCH. Regarding multiplexing with SR, we have expressed our strong technical concern on that. We cannot assume LP HARQ-ACK will be multiplexed with SR for now. At least for LP HARQ-ACK with PF2/3/4, the LP HARQ-ACK is dropped when overlapping with SR. We think it is a typical case for LP HARQ-ACK with more than 2 bits. |
| DOCOMO | It is under discussion whether multiplexing of HP SR and LP HARQ-ACK of PF0/1. Depending on the outcome, the LP HARQ-ACK would be dropped by Alt.1. To avoid unnecessary dropping, we prefer Alt.2. However, we understand that Alt.2 would lead to additional UE complexity as HW/HiSi analysed. Thus, we can also accept Alt.1 if majority companies prefer it. |
| Intel | It depends on which option (option 1-4) is chosen. We suggest discussing this proposal after proposal 1.3.16. For option 1, we shar similar view with QC that using slot as time unit is sufficient. The outcome of using slot and HP sub-slot is the same for option 1. But for other options, e.g., option 2/4, time unit of slot may not work. We need to use HP sub-slot as time unit to run Rel-15 Pseudo code. |
| Sharp | The most important use case is to avoid dropping of LP HARQ-ACK. Thus, the multiplexing time unit can be bsed on HP PUCCH subslot. However, if a LP HARQ-ACK PUCCH overlaps with multiple HP PUCCHs, all overlapping HP PUCCHs should be considered based on UCI types. Thus. if a LP HARQ-ACK overlaps multiple HP PUCCHs including HP HARQ-ACKs and HP SRs, the LP HARQ-ACK should be multiplexed with the earliest HP HARQ-ACK. This avoids dropping the LP HARQ-ACK by an earlier HP SR when multiplexing with HP HARQ-ACK is possible in a later subslot. |
| NEC | We prefer Alt.2 to avoid LP HARQ-ACK dropping. |
| OPPO | We prefer to Alt 2 due to it can avoid unnecessary dropping of LP PUCCH |

Proposal 1.3.16:

For resolving collision of PUCCHs of different priorities without repetition within a time unit, down-select from the following options:

- Option 1:

- The reference PUCCH resource is determined as in Rel-15, i.e. based on the starting symbol and duration

- In step 2.1-2, select up to one PUCCH resource overlapping with the reference PUCCH resource according to Rel-15 pseudo code

- Option 2:

- The reference PUCCH resource is determined as in Rel-15, i.e. based on the starting symbol and duration

- In step 2.1-2, select all the PUCCH resources overlapping with the reference PUCCH resource according to Rel-15 pseudo code

- Option 3:

- The reference PUCCH resource is determined by prioritizing HP PUCCH over LP PUCCH on top of Rel-15 rules

- In step 2.1-2, select all the PUCCH resources overlapping with the reference PUCCH resource according to Rel-15 pseudo code

- Option 4:

- The reference PUCCH resource is determined by prioritizing LP PUCCH carrying HARQ-ACK on top of Rel-15 rules

- In step 2.1-2, If a LP PUCCH carrying HARQ-ACK overlaps with multiple HP PUCCHs and one of the HP PUCCH includes HARQ-ACK, only select the HP PUCCH including HARQ-ACK in step 2.1-2; otherwise, select all the PUCCH resources overlapping with the reference PUCCH resource according to Rel-15 pseudo code

|  | Company |
| --- | --- |
| Support | Huawei/Hisi, Samsung, Intel |
| Not support |  |

|  | Supporting Company |
| --- | --- |
| Option 1 | Samsung (2nd), Intel, ZTE |
| Option 2 | LG, QC Huawei/Hisi (2nd), DOCOMO, NEC(2nd preference),OPPO, ZTE(2nd) |
| Option 3 | LG Huawei/Hisi (1st), DOCOMO, Sharp(2nd), NEC(1st preference) |
| Option 4 | Samsung (1st), Sharp (1st) |

| Company | Comments |
| --- | --- |
| LG | Option 1 and Option 4 seem to be undesirable since those would require some changes to Rel-15/16 multiplexing procedure/steps. |
| QC | Rel-15 procedure should be reused. Again, we don’t see strong motivation to deviate from Rel-15. |
| Huawei/Hisi | Option 3 is the simplest way and closest to R15 rule. For Option 1, for the case where the HP PUCCH with HARQ-ACK overlapping with both LP SR and LP HARQ-ACK, the LP SR may be dropped if it is placed in prior to LP HARQ-ACK. For Option 4, it is a hybrid HP&LP time unit operation, and the problems have been given in Proposal 1.3.15. For Option 2, it needs to further discuss the selection of the target HP subslot for the case where one LP PUCCH overlaps with more than one HP PUCCHs in different subslots. But we can accept Option 2 as the 2nd preference. |
| Samsung | Option 4 has the best performance. Option 1 can avoid collision of more than 2 channels. Option 2 and Option 3 can result in LP HARQ-ACK dropping in some cases. |
| DOCOMO | Share similar view with other companies that Option 1 and Option 4 are undesirable. They would require some changes to the current procedure. |
| Intel | All 4 options require changes to Rel-15 Pseudo code, As we explain under proposal 1.13.15, option 1 does not deviate from Rel-15 principle, option 1 is indeed the option with minimized modification of Rel-15 procedure and pseudo-code. The only difference between option 1 and Rel-15 is, we only find 1 PUCCH overlapping with reference PUCCH A, then, we can reuse all the remaining lines of Rel-15 Pseudo code. But for other options, we have to additional discussion on how to resolve collision between 3 overlapping channels (e.g., proposal 1.3.17, 1.3.19), and we have to revise Rel-15 Pseudo code to cover the case of more than one resource surviving after resolving the collision (e.g., LP CSI overlapping with multiple HP PUCCH, LP HARQ-ACK with multiple HP SR, LP HARQ-ACK with HP HARQ-ACK and HP SR). Rel-15 Pseudo code only supports single resource surviving after resolving the collision (thanks Samsung for the comprehensive explanation of Rel-15 Pseudo code step-by-step, which is really helpful to align our understanding for Rel-15)! determine a single resource for multiplexing UCI associated with resources as described in Clauses 9.2.5.1 and 9.2.5.2 set the index of the single resource to Moreover, for other options, it also requires standard effort for using HP sub-slot as time unit and how to associate a LP PUCCH into a HP sub-slot (e.g., proposal 1.3.19). But option 1 DOES NOT need such effort. Comparing the protection of LP HARQ-ACK, option 4 is better than option 1~3 for few cases. But option 4 allows to process PUCCHs from back to front (if a LP PUCCH is placed in a later position in one slot), which is fundamentally different from Rel-15 principle and has large impact on existing UE implementation. |
| Sharp | Option 4 is preferred to avoid dropping of LP HARQ-ACK by a HP SR if the LP HARQ-ACK PUCCH overlaps with both a HP SR and a HP HARQ-ACK, where the HP SR PUCCH is earlier than and does not overlap with a HP HARQ-ACK PUCCH. However, if the subslot ordering is the main concern, we can accept Option 3, by considering the reference as HP PUCCH in each sublot. Another suggestion is: Apply Option 3 using HP PUCCH subslot as multiplexing time unit, but if a LP HARQ-ACK is dropped by an earlier HP SR PUCCH, the LP HARQ-ACK can be hold until the end of the LP HARQ-ACK PUCCH to allow potential multiplexing with a HP HARQ-ACK in a later subslot if presents. |
| OPPO | Rel-15 procedure should be reused firstly. |

PUCCH/UCI dropping/multiplexing

Following the above proposals, we continue discussing how to apply Rel-17 intra-UE multiplexing/dropping rules to resolve overlapping among the reference PUCCH resource and O PUCCH resource(s).

For LP PUCCH(s) without HARQ-ACK, it is agreeable to drop the LP PUCCH before multiplexing in general except more discussion may be needed for the following case, i.e. a HP PUCCH overlapping with multiple LP PUCCHs where one of the LP PUCCH includes HARQ-ACK. Based on the four options above, it happens to Option 3 only since HP PUCCH is prioritized for reference PUCCH determination. For this case, if LP CSI is dropped before multiplexing, even if the resultant PUCCH with HP and LP HARQ-ACKs, LP CSI cannot be transmitted. But to avoid LP CSI/SR dropping seems to be an unnecessary optimization. Therefore, it is proposed to drop LP PUCCH(s) without HARQ-ACK before multiplexing.

Proposal 1.3.17:

To apply Rel-17 intra-UE multiplexing/dropping rules to resolve overlapping among the reference PUCCH resource and O PUCCH resource(s), LP PUCCH(s) without HARQ-ACK are dropped before multiplexing.

[Update based on inputs] Proposal 1.3.17a:

To apply Rel-17 intra-UE multiplexing/dropping rules to resolve overlapping among the reference PUCCH resource and O PUCCH resource(s) in step 2.1-3, LP PUCCH(s) without HARQ-ACK are dropped before multiplexing if multiplexing is to be performed.

|  | Company |
| --- | --- |
| Support | LG (agree in principle, but seems to be consequence of Step 2.1-3 in Proposal 1.3.14), QC, DOCOMO, Sharp, NEC, OPPO, ZTE |
| Not support | Huawei/Hisi, Intel (no need of this proposal, if single O PUCCH resource is chosen, which depends on options in 1.3.16) |

| Company | Comments |
| --- | --- |
| LG | It seems to be consequence of Step 2.1-3 in Proposal 1.3.14. In other words, dropping of LP PUCCH without HARQ-ACK is expected to be done during the Step 2.1-3 in above Proposal 1.3.14. |
| QC | We support this proposal to simply Rel-17 intra-UE mux procedure. |
| Huawei/Hisi | If the LP CSI is replaced with LP SR, we think it is possible to multiplex all three of the channels into one HP resultant PUCCH. Change as follows: To apply Rel-17 intra-UE multiplexing/dropping rules to resolve overlapping among the reference PUCCH resource and O PUCCH resource(s), LP PUCCH(s) with CSI only without HARQ-ACK are dropped before multiplexing |
| Moderator | @LG, the intention of the proposal is to perform dropping before multiplexing and the preclude the optimization to first perform multiplexing of LP and HP HARQ-ACK and determine whether to drop LP CSI based on whether the resultant PUCCH overlaps with LP CSI or not for the case shown in the figure. Hope it clarifies. |
| Samsung | LP CSI can be transmitted if the resultant PUCCH does not overlap with the LP CSI. |
| Intel | If we only have single O PUCCH resource, it seems we don’t need this proposal, e.g., if we go with option 1. Regarding HW’s comment, it seems we don’t have any agreement to support LP SR to multiplex with HP PUCCH? |
| Sharp | Is this already agreed? As least it was a working assumption from many meetings ago that LP CSI is not allowed to multiplex with a HP HARQ-ACK. The same principle should be applied to HP SR. Thus, we only need to confirm the previous working assumption. |
| OPPO | In our understanding, the intention of this proposal is to discuss the order of multiplexing and prioritization in step 2.1-3, so we’d like to modify it clearer: Updated proposal 1.3.17: To apply Rel-17 intra-UE multiplexing/dropping rules to resolve overlapping among the reference PUCCH resource and O PUCCH resource(s) in step 2.1-3, LP PUCCH(s) without HARQ-ACK are dropped before multiplexing |

Then the follow-up discussion is for LP PUCCH with HARQ-ACK and CSI/SR. For the simplest case that the LP PUCCH overlaps with a single HP PUCCH with HARQ-ACK, according to TS38.213 v17.0.0 Clause 9, moderator’s understanding is that only LP HARQ-ACK is multiplexed with HP HARQ-ACK and CSI/SR are dropped.

- if the UE is provided UCI-MuxWithDifferentPriority - first, the UE resolves overlapping for PUCCH and/or PUSCH transmissions of larger priority index as described in clauses 9.2.5 and 9.2.6 - second, the UE resolves the overlapping for PUCCH transmissions of different priority indexes, and - third, the UE resolves the overlapping for PUCCH and PUSCH transmissions of different priority indexes - if the timeline conditions in clause 9.2.5 for multiplexing UCI in a PUCCH or a PUSCH are satisfied - if // this is for cases the UE supports multiplexing information of different priorities in a PUCCH/PUSCH - a PUCCH transmission with HARQ-ACK information, without repetitions, with smaller priority index overlaps with a PUCCH transmission with HARQ-ACK information, without repetitions, with larger priority index, or - a PUCCH transmission with HARQ-ACK information, without repetitions, with smaller or larger priority index overlaps, respectively, with a PUSCH transmission with larger or smaller priority index the UE multiplexes HARQ-ACK information of different priority indexes in a same PUCCH or PUSCH transmission and applies the procedures in clause 9.2.5.3 or 9.3, respectively

If the HP PUCCH includes SR only, it was agreed to drop LP PUCCH carrying HARQ-ACK with PF 2/3/4. For LP PUCCH carrying HARQ-ACK and other UCI type(s), it is suggested to adopt the same rule. The only pending case is LP PUCCH with HARQ-ACK and SR with PF 0/1.

Agreement When a PUCCH carrying HP SR with PF0/1 overlaps with a PUCCH carrying LP HARQ-ACK with PF2/3/4: - For positive SR, transmit SR on the SR PUCCH resource and drop HARQ-ACK. - For negative SR, transmit HARQ-ACK only on the HARQ-ACK PUCCH resource. Note: It was agreed to support multiplexing a LP HARQ-ACK and a HP SR into a PUCCH for some HARQ-ACK/SR PF combinations in Rel-17.

Proposal 1.3.18:

To resolve overlapping between a HP PUCCH and a LP PUCCH with HARQ-ACK and CSI/SR,

- If the HP PUCCH includes HP HARQ-ACK, LP CSI/SR in LP PUCCH is dropped and LP HARQ-ACK is multiplexed with HP HARQ-ACK according to Rel-17 rule;

- If the HP PUCCH includes HP SR only, LP PUCCH with PF2/3/4 is dropped;

  - FFS LP PUCCH with LP HARQ-ACK and LP SR with PF 0/1

|  | Company |
| --- | --- |
| Support | LG (agree in principle, seems to need some clarification on the above agreement), Samsung, Intel, Sharp, ZTE |
| Not support | QC Huawei/Hisi(only agree with CSI),OPPO |

| Company | Comments |
| --- | --- |
| LG | It seems to need clarification on “PUCCH carrying HP SR with PF0/1” and “PUCCH carrying LP HARQ-ACK with PF2/3/4” in the above agreement. In our understanding, those mean “PF0/1 carrying HP SR only” and “PF2/3/4 carrying LP HARQ-ACK only”, respectively. |
| QC | We don’t support this proposal because it unnecessarily complicate things. The proposal says if HP PUCCH includes A/N, do A; otherwise, do B. We think UCI type dependent behaviour should be avoided to simplify Rel-17 intra-UE mux procedure. We prefer a unified rule regardless of what is the UCI type of HP PUCCH, i.e., we prefer a unified solution regardless HP PUCCH has HARQ-ACK or not. Because of the agreement already made for HP PUCCH include SR only, we prefer a unified solution which is dropping LP PUCCH (including HARQ-ACK and CSI/SR), regardless of what is HP UCI type. |
| Samsung | We think the first sub-bullet can be considered as a clarification without spec impact. The current spec does not support multiplexing LP SR/CSI with HP UCI, by default, LP SR/CSI should be dropped. |
| Huawei/Hisi | As mentioned in Proposal 1.3.17, we agree the CSI part, but think LP SR can be jointly encoded with LP HARQ-ACK. Changes as below: If the HP PUCCH includes HARQ-ACK, CSI/SR in LP PUCCH is dropped and LP HARQ-ACK and LP SR if any is multiplexed with HP HARQ-ACK according to Rel-17 rule |
| Intel | In our understanding, the FFS point is, FFS LP PUCCH with HARQ-ACK with PF 0/1 and SR with PF 0/1 |
| Sharp | We support both bullets. First bullet aligns with the current agreements that CSI is not multiplexed with HP UCI. Since LP SR is not supported to multiplex with HP HARQ-ACK, it should also be dropped. |
| OPPO | Rel-15 procedure should be reused and no optimization at this stage. |

Then the next issue is how to handle collision of LP PUCCH with HARQ-ACK and multiple non-overlapping HP PUCCHs after step 2.1-2. It may happen for Option 2 and 4 in proposal 1.3.16. There are following alternatives.

- Alt. 1: LP PUCCH with HARQ-ACK is dropped.

- Alt. 2: LP HARQ-ACK is multiplexed with HP UCI in different HP PUCCHs in a same PUCCH resource.

- Alt. 3: LP HARQ-ACK is multiplexed with HP UCI in one HP PUCCH that inter-priority multiplexing is supported if any; otherwise drop LP PUCCH

Proposal 1.3.19:

To resolve overlapping of LP PUCCH with HARQ-ACK and multiple non-overlapping HP PUCCHs after step 2.1-2,

- Alt. 1: LP PUCCH with HARQ-ACK is dropped.

- Alt. 2: LP HARQ-ACK is multiplexed with HP UCI in different HP PUCCHs in a same PUCCH resource.

- Alt. 3: LP HARQ-ACK is multiplexed with HP UCI in one HP PUCCH that inter-priority multiplexing is supported if any; otherwise drop LP PUCCH

|  | Supporting Company |
| --- | --- |
| Alt. 1 |  |
| Alt. 2 | Huawei/Hisi (clarification needed),OPPO |
| Alt. 3 | LG, QC, Samsung, DOCOMO, Sharp, ZTE |

| Company | Comments |
| --- | --- |
| LG | It is desirable to apply same rule as for the case in Proposal 1.3.15. That is, Alt 2 in Proposal 1.3.15 is to be applied also in this case to avoid unnecessary dropping of LP HARQ-ACK PUCCH. |
| QC | Alt 2 seems not clear to us. Is there any typo in Alt 2? |
| Huawei/Hisi | Clarification needed: whether the multiple HP PUCCHs are within one subslot, or distributed over multiple subslots? I guess the Moderator means the former case, as the HP SR and HP A/N in subslot#1 in the figure below. In our understanding, if all the HP PUCCHs are within one subslot, Alt.2 is preferred, as they can be multiplexed into one resultant PUCCH in together with LP HARQ-ACK as in R15. E.g., the resultant PUCCH in subslot#1 includes HP SR+HP HARQ-ACK, and LP HARQ-ACK. In contrast, for Alt.3 (I guess Moderator means to multiplex LP HARQ only with the 1st HP PUCCH in subslot#1, i.e. HP HARQ), it is a bit different from R15 rule, and it incurs more restricted gNB implementation to avoid the resultant HP PUCCH overlapping with HP SR. |
| Moderator | @QC, Alt. 2 means to multiple all the UCIs in a single PUCCH. For the example provided by Huawei above, a single PUCCH will be determined to multiple LP HARQ-ACK, HP HARQ-ACK and HP SR. Hope it clarifies. @Huawei, your understanding is correct except that for Alt. 3 which HP PUCCH is selected for multiplexing is open for discussion. The first HP PUCCH is one of the options. |
| Samsung | Alt 2 is not acceptable. For multiplexing HARQ-ACK and SR in Rel-15, both positive and negative SRs are considered. Alt 2 only considered positive SR, the details are not clear. We cannot support Alt 2 at this late stage. |
| Intel | We don’t need this proposal, if we go with option 1 in 1.3.16. I really hope companies can realize how option 1 can avoid additional standard impact and save our time for discussions for many these proposals to ensure we can complicate the whole design in Q1. |
| Sharp | We prefer Alt 3. But we still need to clarify which HP HARQ-ACK of the corresponding HP PUCCH is used for multiplexing with the LP PUCCH. - If only RRC enable/disable is used, the LP HARQ-ACK should be multiplexed with the HP HARQ-ACK from the first overlapping HP HARQ-ACK PUCCH only. - If no explicit DCI indication is used, the LP HARQ-ACK should be multiplexed with the HP HARQ-ACK from the overlapping HP HARQ-ACK PUCCH based on explicit DCI indication. - Or to have a consistent solution, the DCI enable/disable is allowed only for the first overlapping HP HARQ-ACK PUCCH. Thus, the UE only needs to monitor the DCI indication associated with the first overlapping HP HARQ-ACK PUCCH. |
| OPPO | In our understanding, Alt 2 is Rel-15 procedure and we support it. |

    - PUSCH overlapping with multiple PUCCHs

      - 1st/2nd round discussion

Multiplexing of multiple HARQ-ACKs in different PUCCHs of a same priority in a PUSCH

Multiplexing multiple HARQ-ACKs in different PUCCHs in a PUSCH of the same priority is not supported in Rel-15/16.

Agreement (RAN1#101) It is an error case for Rel-16 that more than one PUCCH carrying HARQ-ACK overlapping with a PUSCH or another PUCCH with the same priority

TS 38.213 Clause 9 A UE does not expect to multiplex in a PUSCH transmission in one slot with SCS configuration μ1 UCI of same type that the UE would transmit in PUCCHs in different slots with SCS configuration μ2 if μ1<μ2. A UE does not expect to multiplex in a PUSCH transmission or in a PUCCH transmission HARQ-ACK information that the UE would transmit in different PUCCHs.

Sharp [22] proposed to clarify whether a HP PUSCH overlaps with more than one HP PUCCH with HARQ-ACK is supported.

Ericsson [19] proposed that for a given priority index, it is an error case in Rel-17 that more than one PUCCH carrying HARQ-ACK overlap with a PUSCH or another PUCCH as in Rel-16.

For a HP PUSCH overlapping with multiple LP PUCCHs with HARQ-ACK (Case 1) and a LP PUSCH overlapping with multiple HP PUCCHs with HARQ-ACK (Case 2) as shown below, companies’ views are summarized as follows.

Nokia [3] proposed that for both cases, multiplexing of more than one PUCCH carrying HARQ-ACK on a PUSCH of different priorities should not be supported. Huawei [4] proposed that Case 1 should be avoided by gNB and Case 2 is avoided by gNB or LP PUSCH is dropped. ETRI [15] proposed no special handling for Case 1. Intel [17] proposed to drop LP PUSCH for Case 2. Ericsson [19] proposed to cancel LP channel(s) for both cases. Sharp [22] proposed that HP HARQ-ACK may be jointly reported on the LP PUSCH or the LP PUSCH should be dropped for Case 2. LG [26] proposed that a LP PUCCH firstly overlapping (and satisfying the multiplexing timeline) with HP PUSCH is selected for the multiplexing on the HP PUSCH while other LP PUCCHs are dropped for Case 1 and dropping LP PUSCH can be considered for Case 2.

It is proposed to follow the same rule/principle as in Rel-16. Note that the following case is valid although LP PUSCHs overlaps with two HP PUCCHs with HARQ-ACK.

Proposed conclusion 1.4.1:

A UE does not expect to multiplex HARQ-ACK information that the UE would transmit in different PUCCHs of a same priority in a PUSCH.

|  | Company |
| --- | --- |
| Support | Sony, Nokia/NSB, Spreadtrum, Intel, ZTE, Samsung, CATT, Ericsson, DOCOMO, OPPO(clarification) Huawei/Hisi, LG |
| Not support | QC(clarification of the proposal is needed), Sharp, Lenovo, ITRI |

| Company | Comments |
| --- | --- |
| QC | Same comment as for previous proposal. Does the proposal mean this an error case or UE should fallback to Rel-16? |
| Sharp | Instead of saying “UE is not expected to…”, it is better to define the behaviour on how to handle it, e.g., for Case 2 - Simplest way is to drop the LP PUSCH, and transmits the HP PUCCHs, or - If timeline is satisfied, the HP HARQ-ACK can be jointly reported on the LP PUSCH. Note that the same logic was already agreed for deferred HARQ-ACK transmission under HARQ-ACK enhancements. The deferred HARQ-ACK can be jointly reported with another HARQ-ACK of the same priority. |
| Intel | With Rel-15 timeline, more typical case for Rel-17 intra-UE multiplexing is slot-based PUCCH for both LP and HP. Rel-15/16 rule can reused. If sub-slot PUCCH is to be supported for intra-UE multiplexing (we suggest deprioritizing such case), it would be possible that one LP PUSCH overlaps with more than one HP PUCCH with HARQ-ACKs. But, with Rel-15 timeline, there is typically sufficient time for gNB to avoid scheduling a LP PUSCH overlapping with HP HARQ-ACKs in different sub-slot, e.g., a shorter LP PUSCH or move LP PUSCH to next slot. |
| Lenovo | We think UE should be allowed to multiplex HARQ-ACK of multiple HP PUCCHs of sub-slot into a PUSCH of slot, as long as Rel-15 multiplexing timeline condition is met. |
| Ericsson | We are supportive of the proposal, which is an extension of Rel-16 agreement below: Agreement (RAN1#101) It is an error case for Rel-16 that more than one PUCCH carrying HARQ-ACK overlapping with a PUSCH or another PUCCH with the same priority We are fine to treat this as error case, the same as in Rel-16. |
| ITRI | Prefer to define a UE behaviour to handle the case 2 (i.e., drop LP PUSCH). As for the Case 1, Rel-15 rule can be reused, if the LP HARQ-ACKs are scheduled in the same slot. |
| vivo | Share the same view from QC. |
| OPPO | Clarification on proposal. Is it an error case or fall back to R16? We are fine to treat this as error case. |
| LG | To be clear, we think the proposed conclusion 1.4.1 provided by FL is only focusing on the case of overlapping between multiple PUCCHs and a PUSCH with same priority (not between different priorities). In this sense, we are fine with the proposed conclusion 1.4.1 since avoiding any further complications should be pursued at this late stage. |

Multiplexing of multiple HARQ-ACKs in different PUCCHs of different priorities in a PUSCH

Ericsson [19] also discussed the case when two PUCCHs of different priorities carrying HARQ-ACK overlap with a PUSCH as shown below and proposed that the two PUCCHs are multiplexed onto the PUSCH.

Based on the previous agreement, LP/HP HARQ-ACK should be multiplexed in the LP/HP PUSCH in step 1. So after step 1, there is only LP/HP HARQ-ACK overlapping with HP/LP PUSCH (with HP/LP HARQ-ACK). It is expected that LP/HP HARQ-ACK is multiplexed in HP/LP PUSCH according to Rel-17 rules.

Companies are invited to comment whether additional agreement/conclusion is needed for this case.

| Company | Comments |
| --- | --- |
| Sony | Shouldn’t this be resolved in Step 1? That is in the case on the left, the HP HARQ-ACK would mux with HP-PUSCH resulting in only HP-PUSCH after Step 1. Similarly in the case on the right, after Step 1, there will only be one LP-PUSCH and 1 HP-HARQ-ACK. Why is this problem different? |
| QC | We are also puzzled why there is a problem for this case. |
| Spreadtrum | We agree with the assessment of the FL, there is no issue for these two case. |
| ZTE | We don’t need agree one thing twice. |
| Samsung | Agree with FL. No need further discussion. |
| Ericsson | We are fine to align the understanding without any explicit agreement. The understanding is, for the above cases, the HP HARQ-ACK and LP HARQ-ACK are multiplexed onto the overlapping PUSCH. (In contrast, if the two HARQ-ACK have the same priority (both HP, or both LP), then such multiplexing cannot be performed) |
| DOCOMO | We agree with the assessment of the FL. |
| vivo | Agree with FL, LP/HP HARQ-ACK can be multiplexed in HP/LP PUSCH with HP/LP HARQ-ACK. |
| LG | We also have same understanding with FL. |

      - 3rd round discussion

Based on the 1st round discussion, the proposed conclusion 1.4.1 is supported by majority companies. The only controversial part is whether Case 2 where a LP PUSCH overlapping with multiple HP PUCCHs with HARQ-ACK is allowed. Therefore, the proposal is revised to cover other cases and discuss Case 2 separately.

Proposed conclusion 1.4.1a:

A UE does not expect to multiplex HARQ-ACK information that the UE would transmit in different LP PUCCHs in a PUSCH.

A UE does not expect to multiplex HARQ-ACK information that the UE would transmit in different HP PUCCHs in a HP PUSCH.

|  | Company |
| --- | --- |
| Support | DOCOMO, Intel, Sharp Huawei/Hisi, New H3C, Samsung, Spreadtrum, ZTE(with clarification), NEC, ITRI, Sony |
| Not support | Ericsson, LG (need clarification), Nokia/NSB (clarifications needed) |

| Company | Comments |
| --- | --- |
| DOCOMO | It is natural to follow the Rel-16 UE behaviour. |
| OPPO | Clarification on proposal. Is it an error case or fall back to R16? We are fine to treat this as error case. |
| Sharp | Agree in principle. Please clarify whether they are error cases. So, no UE behaviour is defined, and the gNB should guarantee such scheduling will not occur. |
| QC | Same question as OPPO. We this as error case to simplify spec. A side comment: shouldn’t we have similar proposal to exclude multiplex multiple HARQ-ACKs on a PUCCH? |
| Huawei/Hisi | Is the intention to handle the situation of inter-priority multiple PUCCHs colliding one PUSCH? A UE does not expect to multiplex HARQ-ACK information that the UE would transmit in different LP PUCCHs in a HP PUSCH. A UE does not expect to multiplex HARQ-ACK information that the UE would transmit in different HP PUCCHs in a HLP PUSCH. Since the restriction for the same priority has been captured in R15/16 spec. |
| Samsung | We don’t use the wording “error case” in 38.213, “not expect” means the case won’t happen to UE, if it happens, UE treats it as an error case. Hopefully, it is the common understanding. |
| Ericsson | Proposed conclusion 1.4.1a mixes error cases and dropping cases (but not error case). In our understanding, - If the two HARQ-ACKs and the PUSCH have same priority: this is error case, similar to Rel-16. UE does not expect this to happen. If the two HARQ-ACKs have same priority, but the PUSCH has a different priority: this is not error case. Prioritization is performed by UE, i.e., the lower priority channel(s) are dropped. |
| vivo | For the first bullet, if the PUSCH is HP, it is legitimate case and LP PUCCHs are dropped in R16. Clarification is needed. |
| ZTE | I am wondering why the ‘of a same priority A’ is missing? The original 1.4.1 is: UE does not expect to multiplex HARQ-ACK information that the UE would transmit in different PUCCHs of a same priority in a PUSCH. Typo?: in different LP PUCCHs in a LP PUSCH. |
| LG | It seems to need clarification on the meaning of “not expect to multiplex HARQ-ACK information that the UE would transmit in different LP PUCCHs in a PUSCH”. Does it mean “not expect overlapping between multiple LP AN PUCCHs and a PUSCH”? The reason to ask is that the former seems to allow the overlapping itself but not allow multiplexing of multiple LP ANs, while the latter seems to not allow the overlapping itself at all. In our view, overlapping between multiple LP AN PUCCHs and a HP PUSCH is to be allowed considering that those are on different CCs with different SCS, so we suggest slight update as below. Proposed conclusion 1.4.1a (updated): A UE does not expect to multiplex HARQ-ACK information that the UE would transmit in different LP PUCCHs in a LP PUSCH. A UE does not expect to multiplex HARQ-ACK information that the UE would transmit in different HP PUCCHs in a HP PUSCH. |
| NEC | Agree in principle. But we are not clear it means that multiplexing operation for the collision case is not allowed or the collision case is an error case? |
| Nokia/NSB | Multiplexing of more than one PUCCH with HARQ-ACK on a single PUSCH of the same priority is not supported in Rel-16 either and is considered as an error case. So no need to clarify this (and this applies to both of the sentences). Also, would it need to be actually that we try to cover here only the other overlapping cases? If so, we agree with Huawei’ suggested edits. Finally, we have similar comment as LG on the exact implication of the ‘does not expect to multiplex’. |

For a LP PUSCH overlapping with multiple HP PUCCHs with HARQ-ACK, in general we can discuss the following two options.

- Option 1: If a UE would multiplex HARQ-ACK information that the UE would transmit in different HP PUCCHs in a LP PUSCH, LP PUSCH is dropped.

- Option 2: A UE does not expect to multiplex HARQ-ACK information that the UE would transmit in different HP PUCCHs in a LP PUSCH

For single cell case shown below, it is a valid case for Option 1 and LP PUSCH is dropped while it is an error case for Option 2.

Note that for the following CA case, it is valid for both options and the first HP HARQ-ACK is multiplexed in LP PUSCH1 and the second HP HARQ-ACK is multiplexed in LP PUSCH2.

Proposed conclusion 1.4.1b:

For a LP PUSCH overlapping with multiple HP PUCCHs with HARQ-ACK, down-select from:

- Option 1: If a UE would multiplex HARQ-ACK information that the UE would transmit in different HP PUCCHs in a LP PUSCH, LP PUSCH is dropped.

- Option 2: A UE does not expect to multiplex HARQ-ACK information that the UE would transmit in different HP PUCCHs in a LP PUSCH

|  | Supporting Company |
| --- | --- |
| Option 1 | DOCOMO Huawei/Hisi, Ericsson (with revision), Nokia/NSB, ITRI, Sony |
| Option 2 | OPPO (clarification), Intel, QC Huawei/Hisi, Samsung,vivo, Spreadtrum, ZTE, LG |

| Company | Comments |
| --- | --- |
| DOCOMO | We slightly prefer Option 1 for better scheduling flexibility for HP HARQ-ACK. On the other hand, it seems UE complexity may increase to allow the case. If it is problematic, we could compromise to Option 2. |
| OPPO | Clarification on proposal. Is it an error case or fall back to R16? We are fine to treat this as error case. |
| QC | Same comment as OPPO. We prefer to treat this as error case to simplify spec. Clarification of the proposal is needed. |
| Huawei/Hisi | Both OK for us. Note that the three channels are all under R15 timeline, so there is no timeline issue. Note for Capability#1 UE, there is no option to ‘fall back to R16’. For Option 1, the dropping behaviour and timeline is R15-like. |
| Ericsson | Option 1. Revised below to avoid implying that UE perform multiplexing. Revised Option 1: If two or more HP PUCCHs carrying HARQ-ACK overlap with a LP PUSCH, the LP PUSCH is dropped. |
| LG | Agree with OPPO and QC. |
| NEC | Same comment as OPPO and QC. Clarification of the proposal is needed. We prefer a unified handling rule for overlapping between more than one PUCCH with HARQ-ACK and a PUSCH. |
| Nokia/NSB | Option 2 is unclear to us, as it is not clear what it means ‘does not expect to multiplex’. Is this the same as ‘does not expect to overlap’?? |
| Sony | Option 1 allows the scheduler to schedule this overlapping scenario, that is it still allows scheduler to schedule a 2nd HP PUCCH that overlaps with a LP PUSCH and a 1st HP PUCCH. Option 2 prevents such scheduling and therefore limits gNB flexibility. |

      - 4th round discussion

During the GTW discussion, the intention of proposal 1.4.1c was clarified and hopefully that companies now are on the same page. In this round, please focus on whether/how to refine the wording to reflect the intention more accurately. Note that the case that UE would multiplex HARQ-ACK information in different PUCCHs of a priority in a PUSCH of the same priority is an error case as in Rel-16 and will not be further discussed.

[Closed] Proposal 1.4.1c: (Moderator: Please check the updated proposal 1.4.1d below.)

A UE does not expect to multiplex HARQ-ACK information that the UE would transmit in different LP PUCCHs in a HP PUSCH.

(Alternative formulation from Yuan) A UE does not expect the overlapping of HP PUSCH with more than one LP HARQ ACK which would be multiplexed on a HP PUSCH

The following is just to clarify the intention of the above proposal.

According to the above proposal, the following is an error case. According to the above proposal, the following case is valid and the first LP HARQ-ACK is multiplexed in HP PUSCH1 and the second LP HARQ-ACK is multiplexed in HP PUSCH2.

| Company | Comments |
| --- | --- |
| Sony | I think what we want to say is the UE can expect HP PUSCH to overlap more than one LP PUCCH carrying HARQ-ACK but would not expect to multiplex these LP HARQ-ACK into a single HP PUSCH. Suggested proposal. For the case where at least one HP PUSCH overlaps more than one LP PUCCH carrying HARQ-ACKs, each HP PUSCH is not expected to multiplex HARQ-ACKs from more than one LP PUCCH. I think we need further discussion on how to resolve the 2nd drawing involving two HP PUSCHs. Adding the drawings seems to suggest that we already agreed on a solution. For example have we decided what “sequentially in time” means? If “sequentially in time” means when a UL channel is scheduled (rather than when it is transmitted) then we can have the following two case: Case 1: 1st and 2nd LP HARQ-ACKs are scheduled first followed by HP PUSCH1 and lastly HP PUSCH2. If UE is not aware of processing timelines, then the process is as follows: - UE receives the UL Grant for HP PUSCH1 first and after decoding this UL Grant, UE multiplexes the 1st LP HARQ-ACK into HP PUSCH1 - UE then receives UL Grant for HP PUSCH2 and after decoding this 2nd UL Grant, the UE multiplexes the 2nd LP HARQ-ACK into HP PUSCH2 This is as described in the wordings for the drawing. Case 2: 1st and 2nd LP HARQ-ACKs are scheduled first followed by HP PUSCH2 and lastly HP PUSCH1. If UE is not aware of processing timelines, then the process is as follows: - UE receives the UL Grant for HP PUSCH2 first and after decoding this UL Grant, UE realises HP PUSCH2 overlaps with two LP PUCCH carrying HARQ-ACK. Assume the UE multiplexes the 1st LP PUCCH into HP PUSCH2, the UE would then drop the 2nd LP PUCCH. - UE receives the UL Grant for HP PUSCH1 and since the 1st LP PUCCH is already dropped, nothing is multiplexed into HP PUSCH1 Now this is very different to the wordings in the drawing. Hence I suggest we add the following to the 2nd drawing. According to the above proposal, the following case is valid. Here the UE firstly received scheduling for the 1st LP HARQ-ACK and 2nd LP HARQ-ACK, followed by scheduling for HP PUSCH1 and lastly scheduling for HP PUSCH2. The first LP HARQ-ACK is multiplexed in HP PUSCH1 and the second LP HARQ-ACK is multiplexed in HP PUSCH2. |
| Lenovo | Our suggestion is as follows: “A UE does not expect to multiplex multiple HARQ-ACK information that the UE would transmit in different LP PUCCHs in a HP PUSCH.” |
| QC | We think Yuan’s formulation is better and it captures the intention of the proposal precisely. So we support using Yuan’s formulation with minor editorial change. A UE does not expect the overlapping of a HP PUSCH with more than one LP HARQ-ACK PUCCHs which would be multiplexed on the a HP PUSCH |
| Sharp | We understand the concept and would like to clarify the UE behaviour. If it is treated as an error case, it is up to gNB scheduling to avoid it. But if the error occurs, what will be the UE behavior, multiplxed the first LP HARQ-ACK and drop the later LP HARQ-ACK, or drop all LP HARQ=ACKs? The second case does not provide enough information and is not complete. - For HP PUSCH1 and HP PUSCH2, how to determine the ordering? Based on scheduling DCI timing, or starting sym, or based on CC index ordering … - What if the HP PUSCH1 overlaps with the second LP HARQ-ACK only? At least from CC3 point of view, it is already an error case. Do we need to determine the UCI multiplexing on an earlier starting PUSCH based on a later PUSCH? |
| Ericsson | We suggest the following. This means the combinations are not error case from scheduling perspective, and UE behavior is provided (i.e., dropping of LP). Proposal: - If two (or more) LP PUCCHs carrying HARQ-ACK overlap with a HP PUSCH where the HP PUSCH would be selected to multiplex with the two (or more) LP PUCCHs, the LP PUCCHs are dropped. - If two (or more) HP PUCCHs carrying HARQ-ACK overlap with a LP PUSCH where the LP PUSCH would be selected to multiplex with the two (or more) HP PUCCHs, the LP PUSCH is dropped. |
| Intel | In our understanding, the intension of this proposal is, if a HP PUSCH is selected as the PUSCH to multiplex LP HARQ-ACK for more than one LP PUCCH with LP HARQ-ACK, it is error case. For example, if we put HP PUSCH2 on CC2 and HP PUSCH1 on CC3. For 1st LP PUCCH, UE selects HP PUSCH2 according Rel-15 PUSCH selection rule, and for 2nd LP PUCCH, UE selects HP PUSCH2 again according to Rel-15 PUSCH selection rule, then, it is error case. Based on this understanding, we prefer Yuan’s proposal with QC’s revision. Having said that, if majority view is to allow such case and UE drops LP PUCCHs with Rel-15 timeline, we’re also fine. Hope we can progress. |
| Samsung | Regarding Sony’s update, we agree with the intention but the wording is not correct, for LP HARQ-ACK, it may with PDCCH or without PDCCH. Further, UL grant for PUSCH1 doesn’t need to come before UL grant for PUSCH2. We don’t have such restriction as long as the multiplexing timeline is satisfied. We think the current wording from FL is fine. It follows the wording of 38.213, an example is given below. TS 38.213 A UE does not expect to multiplex in a PUSCH transmission or in a PUCCH transmission HARQ-ACK information that the UE would transmit in different PUCCHs.. We can simply update the current spec as below A UE does not expect to multiplex in a PUSCH transmission or in a PUCCH transmission HARQ-ACK information that the UE would transmit in different PUCCHs of a same priority. The 2nd case is considered as valid based on reusing Rel-15/16 multiplexing rules for PUCCH and PUSCHs. Some minor update can be considered to address Sharp’s concern. According to the above proposal, the following case is valid and the first LP HARQ-ACK is multiplexed in HP PUSCH1 and the second LP HARQ-ACK is multiplexed in HP PUSCH2 assuming Rel-15 rules for multiplexing PUCCH and PUSCHs is reused irrespective of the priority. Could Lenovo clarify a bit what is the difference between “more than one” and “multiple”, sorry we are confused about the intention of changing the wording here. To Sharp, we don’t define UE behaviour if an error case happens. It is up to UE implementation. |
| LG | Focusing the formulation itself, QC’s update in above looks better. But from technical perspective, that direction (treating as error case if happens) doesn’t seem to be desirable considering the restriction to HP PUSCH scheduling. Rather than that, as commented by Sharp and Ericsson, it may be better to allow the overlapping case and provide UE behaviour in the case. In this context, we think to multiplex the first LP AN (as Sharp mentioned) would be reasonable rather than dropping all LP ANs. |
| OPPO | QC’s version is clearer and more accurate and we can accept QC’s version. Just for clarification, this proposal is applied in step 2.2 only. |
| vivo | We think Yuan’s formulation is better. We are fine with QC’s version. |

[Updated] Proposal 1.4.1d:

Moderator: please indicate your support between the three options.

- Option 1: A UE does not expect the overlapping of a HP PUSCH with more than one LP HARQ-ACK PUCCHs which would be multiplexed on the HP PUSCH.

- Supported by: QC Huawei/Hisi

- Option 2: If two (or more) LP PUCCHs carrying HARQ-ACK overlap with a HP PUSCH where the HP PUSCH would be selected to multiplex with the two (or more) LP PUCCHs, the LP PUCCHs are dropped.

- Supported by: Ericsson, Nokia/NSB

- Option 3: A UE does not expect to multiplex in a PUSCH transmission or in a PUCCH transmission HARQ-ACK information that the UE would transmit in different PUCCHs of a same priority.

- Supported by: Samsung

| Company | Comments |
| --- | --- |
| Samsung | Option 3 The wording is simply extended based on the current specification by adding “of a same priority” |
| New H3C | We prefer option 1 because option1 can align with the intention of the proposal. |
| DOCOMO | Slightly prefer Option 3 than Option 1. Share the same view as Samsung. |
| ZTE | Either 1 or 2 is fine. Option 1 defines error case. Option 2 defines the clear UE behavior. |
| Intel | A unified solution for both LP PUSCH vs multiple HP HARQ-ACKs, and HP PUSCH vs multiple LP HARQ-ACKs is desirable, and we think gNB can avoid both cases. Therefore, we prefer option 3 for both proposal 1.4.1d and 1.4.1e (option 3 = option 1 in proposal 1.4.1d + option 2 in proposal 1.4.1e). We can live with treating both cases as valid case and drop LP, i.e., option 2 in proposal 1.4.1d + option 1 in proposal 1.4.1e. |
| Huawei/Hisi | Option 1. |
| Spreadtrum | Option 1 |
| LG | If we go with a direction to define UE behavior (rather than to treat as error case), then multiplexing the first LP HARQ-ACK on HP PUSCH is better than Option 2. Otherwise, either Option 1 or Option 3 could be adopted since there doesn’t seem to be much difference. |
| Nokia/NSB | Option 2, as we think preventing by gNB implementation such overlap especially considering SPS operation may not be always possible. |

For proposal 1.4.1b, let’s try to down-select in this round. Please provide your preference in the table. The wording of the proposal can be refined as for proposal 1.4.1c.

[Closed] Proposal 1.4.1b: (Moderator: Please check the updated proposal 1.4.1e below.)

For a LP PUSCH overlapping with multiple HP PUCCHs with HARQ-ACK, down-select from:

- Option 1: If a UE would multiplex HARQ-ACK information that the UE would transmit in different HP PUCCHs in a LP PUSCH, LP PUSCH is dropped.

- Option 2: A UE does not expect to multiplex HARQ-ACK information that the UE would transmit in different HP PUCCHs in a LP PUSCH

|  | Supporting Company |
| --- | --- |
| Option 1 | Lenovo |
| Option 2 | QC, Intel |

For single cell case shown below, it is a valid case for Option 1 and LP PUSCH is dropped while it is an error case for Option 2. Note that for the following CA case, it is valid for both options and the first HP HARQ-ACK is multiplexed in LP PUSCH1 and the second HP HARQ-ACK is multiplexed in LP PUSCH2.

| Company | Comments |
| --- | --- |
| Sony | Please see comments on Proposal 1.4.1c above. |
| QC | We prefer to treat this as error case to simplify spec. For option 2, suggest to use Yuan’s wording too: A UE does not expect the overlapping of a LP PUSCH with more than one HP HARQ-ACK PUCCHs which would be multiplexed on the LP PUSCH |
| Ericsson | We suggest the following. This means the combinations are not error case from scheduling perspective, and UE behavior is provided (i.e., dropping of LP). Proposal: - If two (or more) LP PUCCHs carrying HARQ-ACK overlap with a HP PUSCH where the HP PUSCH would be selected to multiplex with the two (or more) LP PUCCHs, the LP PUCCHs are dropped. - If two (or more) HP PUCCHs carrying HARQ-ACK overlap with a LP PUSCH where the LP PUSCH would be selected to multiplex with the two (or more) HP PUCCHs, the LP PUSCH is dropped. |
| LG | Similarly with the Proposal 1.4.1c in above, considering the restriction to HP PUCCH scheduling, it may be better also for this case to allow the overlapping case and provide UE behaviour in the case (e.g. dropping LP PUSCH), rather than the direction to treat as error case if it happens. Overall, adopting same direction is preferred in our view for both the overlapping of multiple LP PUCCHs vs. HP PUSCH and the overlapping of multiple HP PUCCHs vs. LP PUSCH. |

Proposal 1.4.1e:

Moderator: please indicate your support between the three options.

- Option 1: If two (or more) HP PUCCHs carrying HARQ-ACK overlap with a LP PUSCH where the LP PUSCH would be selected to multiplex with the two (or more) HP PUCCHs, the LP PUSCH is dropped.

  - Supported by: Ericsson, Nokia/NSB

- Option 2: A UE does not expect the overlapping of a LP PUSCH with more than one HP HARQ-ACK PUCCHs which would be multiplexed on the LP PUSCH.

  - Supported by: QC, Huawei/Hisi

- Option 3: A UE does not expect to multiplex in a PUSCH transmission or in a PUCCH transmission HARQ-ACK information that the UE would transmit in different PUCCHs of a same priority.

  - Supported by: Samsung,

| Company | Comments |
| --- | --- |
| Samsung | Option 3 The wording is simply extended based on the current specification by adding “of a same priority” |
| New H3C | Support option 2 |
| DOCOMO | Slightly prefer Option 3 than Option 2. Share the same view as Samsung. |
| ZTE | Either 1 or 2 is fine. |
| Intel | A unified solution for both LP PUSCH vs multiple HP HARQ-ACKs, and HP PUSCH vs multiple LP HARQ-ACKs is desirable, and we think gNB can avoid both cases. Therefore, we prefer option 3 for both proposal 1.4.1d and 1.4.1e (option 3 = option 1 in proposal 1.4.1d + option 2 in proposal 1.4.1e). We can live with treating both cases as valid case and drop LP, i.e., option 2 in proposal 1.4.1d + option 1 in proposal 1.4.1e. |
| Huawei/Hisi | Option 2 |
| Spreadtrum | Option 2. |
| LG | Either Option 2 or Option 3 could be adopted since there doesn’t seem to be much difference. |
| Nokia/NSB | Option 1 – so same handling as for the case above. |

    - LP PUSCH overlapping with HP PUCCH including positive SR

      - 1st/2nd round discussion

In Rel-15/16, SR multiplexing in PUSCH is not supported. In Rel-17, there is no agreement to support SR multiplexing in a PUSCH with different priority until now. Therefore, for overlapping LP PUSCH and HP PUCCH with SR, HP SR cannot be multiplexed in LP PUSCH and the LP PUSCH is expected to be dropped.

In addition, Samsung [9] and Sharp [22] discussed the case of LP PUSCH overlapping with HP HARQ-ACK and HP SR.

Sharp [22] proposed to multiplex both HP SR and HP HARQ-ACK in the LP PUSCH for Case A above. For Case B, Samsung proposed that a UE does not expect to multiplex a HP HARQ-ACK in a LP PUSCH that would be canceled by HP SR. Sharp proposed that the HP SR should be dropped if there is HP HARQ-ACK multiplexing on the LP PUSCH in the above case.

Considering all the cases above, a unified solution to drop LP PUSCH is proposed as follows.

Proposal 1.5.1:

For resolving collision of PUCCHs and PUSCHs of different priorities in step 2.2, LP PUSCH(s) overlapping with HP PUCCH including positive SR are dropped.

|  | Company |
| --- | --- |
| Support | Intel, Nokia/NSB, ZTE, CATT, DOCOMO Huawei/Hisi (with modification), QC, Spreadtrum, New H3C, Samsung, CATT, Ericsson (with modified proposal), ITRI, vivo,OPPO, LG |
| Not support | Sharp, Lenovo |

| Company | Comments |
| --- | --- |
| Intel | HP UCI should always be prioritized over LP channel. |
| Nokia/NSB | Agree with Intel |
| Huawei/Hisi | We should focus on the discussion of pairwise PUCCH and PUSCH of different priorities since we have not achieved a conclusion whether/how to support Case B (to our understanding, Case B is not expected), so we recommend the proposal is modified as Proposal 1.5.1: For resolving collision of pairwise PUCCHs and PUSCHs of different priorities in step 2.2, LP PUSCH(s) overlapping with HP PUCCH including positive SR areis dropped. |
| QC | We support FL proposal. |
| Sharp | We can agree to drop the SR for Case B if HP HARQ-ACK is already multiplexed on the LP PUSCH. Otherwise, if HP HARQ-AKC is multiplexed on the LP PUSCH, the LP PUSCH should be dropped. For Case A, the HP HARQ-ACK and HP SR are already multiplexed in Step 1. Drop the positive HP SR will undo Step 1. There is no spec impact by treating the multiplexed HP bits them together as HP HARQ-ACK. |
| Intel2 | We’d like to point out, it would be more difficult to avoid LP PUSCH overlapping with HP HARQ-ACK and HP SR, compared with LP PUSCH overlapping with 2 HP HARQ-ACKs, because HP SR resource would be quite frequent and gNB does not know whether positive or negative SR at UE side. Therefore, it is reasonable to allow case B, and it is simple to drop LP PUSCH for case B. |
| New H3C | We have the similar with Intel. |
| ZTE | For confirmation, in the Case B, the HP HARQ and HP SR will be survived when the LP PUSCH is cancelled if the proposal is triggered. |
| Lenovo | We think HP SR can be multiplexed in LP PUSCH to avoid dropping of a transport block delivered to PHY for the LP PUSCH. |
| Samsung | Agree with Intel |
| Ericsson | Since there is no plan to design SR multiplexing onto PUSCH, we are fine with intention of above proposal, which is inline with existing MAC processing with LCH prioritization enabled, i.e., HP SR is prioritized over LP PUSCH, and no PDU is generated for the LP PUSCH (see 38.321 section 5.4.4). If a PUSCH has no MAC PDU, then it does not participate in PHY intra-UE multiplexing procedure. Considering that existing Rel-16 MAC spec already ensures that no HP SR overlaps with LP PUSCH with MAC PDU, we suggest to add the following. Then the HP SR overlapping with LP PUSCH may only occur due to {HP SR + HP HARQ-ACK}. Proposal: Rel-17 physical layer expects the same MAC handling of SR prioritization over PUSCH as in Rel-16. |
| LG | We are fine with the proposal 1.5.1 |

      - 3rd round discussion

According to the agreement below, LP PUSCH is dropped in the example shown below.

Agreement For resolving collision of PUCCHs and PUSCHs of different priorities in step 2.2, LP PUSCH(s) overlapping with HP PUCCH including positive SR are dropped.

The remaining issue is that whether LP PUSCH dropping is performed before or after multiplexing of HP HARQ-ACK in LP PUSCH. If it is performed before multiplexing, both HP HARQ-ACK and HP SR are transmitted. Otherwise if it is performed after multiplexing, it is possible that HP-HARQ-ACK is multiplexed in LP PUSCH and then LP PUSCH with HP HARQ-ACK is dropped. It seems that it is more reasonable to perform dropping before multiplexing.

Proposal 1.5.2:

For resolving collision of PUCCHs and PUSCHs of different priorities in step 2.2, LP PUSCH(s) dropped due to overlapping with HP PUCCH including positive SR are excluded from candidate PUSCHs for UCI multiplexing.

|  | Company |
| --- | --- |
| Support | DOCOMO, OPPO, Intel, Sharp, QC Huawei/Hisi, New H3C, Samsung, Ericsson,vivo, Spreadtrum, ZTE, LG, NEC, Nokia/NSB, ITRI, Sony (need clarification) |
| Not support |  |

| Company | Comments |
| --- | --- |
| Sony | I think we really need to clarify the concept of “sequentially in time”. - “Sequentially in time” refers to when the scheduling of the UL channel arrives - “Sequentially in time” refers to when the UL channel is transmitted regardless of when it was scheduled in time It seems companies assume that it is interpretation 2. If it is interpretation 2, then how do we square the circle on the argument that the UE is unaware of processing timeline? If UE is unaware of processing timeline, then in this scenario if HP HARQ-ACK arrives first the UE would have simply multiplex HP HARQ-ACK into LP PUSCH and there is no way it could drop the LP PUSCH when HP SR arrives later. The above is only possible if the HP SR arrives first but there shouldn’t be any mandate for this. Please clarify. |

      - 4th round discussion

Proposal 1.5.2 seems agreeable to the group except that Sony would like some clarification. We can discuss the issue case by case. At least for the concerned case, SR is triggered internally by the UE so it is expected that UE can perform according to the proposal. Comments/answers to the question from Sony are welcome.

Proposal 1.5.2:

For resolving collision of PUCCHs and PUSCHs of different priorities in step 2.2, LP PUSCH(s) dropped due to overlapping with HP PUCCH including positive SR are excluded from candidate PUSCHs for UCI multiplexing.

| Company | Comments |
| --- | --- |
| Sony | I am not sure how the UE can exclude a positive SR if it is unaware of any processing timeline. Hence, I think we need to clarify what is meant by “sequentially in time”. In the scenario above, if we assume UE is not aware of processing timeline, then how does the UE decides to wait until for a HP SR to arrive before it starts the multiplexing processing? Again we have two cases here: Case 1: With the following time sequence: - UE receives UL Grant for LP PUSCH. So UE is aware of a LP PUSCH - UE MAC triggers a HP SR. Here UE is aware that HP SR overlaps LP PUSCH. UE then drops LP PUSCH since HR SR as per proposal. - UE receives DL Grant scheduling HP HARQ-ACK. UE now aware of HP HARQ-ACK but then this HP HARQ-ACK no longer collides with anything Here we have the intended behaviour Case 1: With the following time sequence: - UE receives UL Grant for LP PUSCH. So UE is aware of a LP PUSCH - UE receives DL Grant scheduling HP HARQ-ACK. UE now aware that HP HARQ-ACK overlaps with LP PUSCH. If UE is NOT AWARE of processing timeline, the UE will proceed to multiplex HP HARQ-ACK into LP PUSCH since this is what it was RRC configured to do - UE triggers HP SR that overlaps LP PUSCH, which contains HP HARQ-ACK. So what is UE going to do here? Drop LP PUSCH together with HP HARQ-ACK? Not allowed to trigger HP SR? Hence I think we need clarification first before we can agree with this proposal. |
| Lenovo | As long as UE decides what to drop and what to multiplex jointly for all overlapping PUCCHs/PUSCHs, LP PUSCH(s) to be dropped due to overlapping with HP PUCCH including positive SR is obviously not available for UCI multiplexing. Do we need this proposal? |
| Sharp | Agree with Lenovo. If the LP PUSCH is dropped, why do we need to mention it again? Also there will be no UCI to be multiplex on the LP PUSCH anymore. - The HP HARQ-ACK and HP SR are in the HP PUCCH that drops the LP PUSCH. - LP UCI, if any, should be multiplexed on LP PUSCH already in Step 1 (between channels with the same priority). |
| Ericsson | Support the intention of the proposal, which avoids unnecessary dropping of UCI together with a dropped LP PUSCH. But the timing question from Sony is valid. The proposal seems to imply a Step 2.2.1: Step 2.2.1: Drop any LP PUSCH due to overlapping with HP PUCCH including positive SR; Step 2.2.2: Perform multiplexing of UCI and PUSCH of different priorities. |
| Intel | Support the proposal by FL. In our understanding, Ericsson’s explanation of step 2.2.1 and 2.2.2 is aligned with the proposal by FL, and the question from Sharp and Lenovo is answered. Regarding Sony’s question, in our understanding, all channels should meet Rel-15 timeline, so UE knows whether there is positive SR before UE starts to multiplex HARQ-ACK onto PUSCH. |
| Samsung | Support. Also fine with the sub-steps proposed by Ericsson. To Lenovo, Sharp, the sub-steps are not clear as pointed out by Ericsson, if Step 2.2.2 is performed first, HP HARQ-ACK may be dropped if multiplexed in the LP PUSCH. To Sony, if Rel-17 mux is enabled, UE cancel a whole LP PUCCH/PUSCH transmission if there is collision, partial cancelation is not allowed. Before UCI multiplexing, UE first checks whether a LP PUSCH collides with HP SR if there is collision UE drops the LP PUSCH, if there is no collision with HP SR, UE then performs multiplexing. |
| LG | We also have similar view with Lenovo and Sharp. |
| New H3C | Support FL proposal and the sub-steps proposed by Ericsson. |
| OPPO | Support To Sony, according to proposal, UE will drop LP PUSCH only and transmit HP HARQ-ACK. And it is UE implementation how to handle this case, such as whether /when to trigger HP SR, and how to avoid reverting resulting PUSCH or revert resulting PUSCH |
| DOCOMO | Support the proposal. Share the similar view with Intel. |
| ZTE | Support. Fine with proposal from Ericsson. |
| Spreadtrum | Support the proposal, fine with Ericsson’s steps. |
| Nokia/NSB | Support. We are fine with Ericsson’s suggested sub-steps. |

    - [Open] PUSCH selection for UCI multiplexing in step 2.2

      - 1st/2nd round discussion

According to previous agreement, collision of PUCCHs and PUSCHs of different priorities are resolved in step 2.2. One remaining issue is the PUSCH selection rule in case a PUCCH overlaps with multiple PUSCHs. Note that the case when there are overlapping PUSCHs on the same cell is separated discussed in section 3.1.7.

Huawei [4], CATT [7], Intel [17] and Ericsson [19] proposed to reuse Rel-15/16 rule for PUSCH selection, while Samsung [9], DOCOMO [10], OPPO [14] and Lenovo [24] proposed to take some other factors into account, including:

- PUSCH without UCI > PUSCH with UCI

- HP PUSCH > LP PUSCH

- PUSCH(s) with earliest ending among the PUSCH(s) determined from step 1

Given the divergent views on whether/which additional factor(s) are considered for PUSCH selection for UCI multiplexing, it is proposed to follow Rel-15/16 rules.

Proposal 1.6.1:

For resolving collision of PUCCHs and/or PUSCHs with different priorities in step 2, if there are multiple candidate PUSCHs for UCI multiplexing, Rel-15/16 rule is reused for PUSCH selection.

|  | Company |
| --- | --- |
| Support | Nokia/NSB, Sony, QC, Spreadtrum, Sharp, Intel (with clarifcation for candidate PUSCHs), New H3C, ZTE, CATT, Ericsson (with clarification), DOCOMO (can accept), ITRI,OPPO Huawei/Hisi, LG, Sharp |
| Not support | Samsung |

| Company | Comments |
| --- | --- |
| Nokia/NSB | Reusing Rel-15/16 rules would be the simplest here (least specs & implementation effort). |
| Apple | Some modifications are necessary: excluding PUSCHs for the same priority as PUCCH (we don’t want to repeat the work), conditional on the configuration of simultaneous PUCCH/PUSCH configuration. |
| Intel | We support reusing Rel-15/16 rule for PUSCH selection, but we’d like to clarify, the proposal does not exclude the case that some PUSCH can be excluded from candidate PUSCHs first, i.e., PUSCH in a different band, a LP PUSCH overlapping with HP PUSCH in the same serving cell, a LP PUSCH overlapping with HP PUCCH with repetitions or HP SR. These issues are still under discussion in other sections. |
| ZTE | Fine with the proposal. Regarding the issue of checking multiplexing timeline of PUSCHs, whether the multiplexing timeline checking on PUSCHs is before the PUSCH selection or after the PUSCH selection? |
| Lenovo | Reusing Rel-15/16 rules is fine for PUSCHs of a same priority. For multiple candidate PUSCHs of different priorities, we think a HP PUSCH should be selected so that a resultant PUSCH cannot be cancelled by another HP PUCCH. |
| Samsung | We think “PUSCH without UCI > PUSCH with UCI” should be supported. It can help avoid dropping LP UCI because of the limited encoding chain. This is a new case in Rel-17. In addition, it is beneficial to ensure the reliability of LP UCI if a HP HARQ-ACK is multiplexed in a PUSCH without LP UCI. For “HP PUSCH > LP PUSCH”, if a PUCCH with HP HARQ-ACK and LP HARQ-ACK overlaps with both HP PUSCH and LP PUSCH, Rel-16 rules cannot be reused. For “PUSCH(s) with earliest ending among the PUSCH(s) determined from step 1”, similar issue was discussed in Rel-15 and can thus be deprioritized. We also share similar view as Intel, candidate PUSCH should be clarified. |
| Ericsson | We are supportive of reusing Rel-15/Rel-16 rules as much as possible. On the other hand, we share similar concern as Intel that the set of candidate PUSCH for UCI multiplexing need to be modified in Rel-17, for example, (a) simultaneous PUCCH/PUSCH tx is applicable or not; (b) if Rel-17 supports the combination of the UCI and PUSCH (e.g., Rel-17 does not support the combination of LP CSI and HP PUSCH) |
| DOCOMO | Although we think it is beneficial to take latency condition into account for the PUSCH selection, we can accept the proposal considering the limited time. |
| vivo | We think the proposal is not very clear, Rel-15/16 rule is reused for PUSCH selection means that UE selects a PUSCH among PUSCHs with the same priority or among all PUSCHs regardless of priority. |
| Huawei/Hisi | Regarding the comments from Ericsson, some changes as below: At least for the case where simultaneous PUCCH/PUSCH tx is diabled, Ffor resolving collision of PUCCHs including HARQ-ACK(s) and/or PUSCHs with different priorities in step 2, if there are multiple candidate PUSCHs for UCI multiplexing, Rel-15/16 rule is reused for PUSCH selection. |
| LG | We have similar view with Nokia. BTW, some clarification may be needed in case when simultaneous PUCCH+PUSCH TX is enabled if companies want (in other words, the proposal 1.6.1 can be proposed at least for the case when simultaneous PUCCH+PUSCH TX is disabled). |

      - 6th round discussion

Based on the discussions in previous rounds, the proposal is supported by majority companies except that one company would like to prioritize PUSCH without UCI over PUSCH with UCI. In addition, some companies commented that candidate PUSCH in the previous proposal is not clear. Therefore, the proposal is updated as follows to avoid confusion.

Proposal 1.6.1a:

For resolving collision of PUCCHs and PUSCHs of different priorities in step 2.2, Rel-15/16 rule is reused for PUSCH selection for UCI multiplexing.

[Update based on inputs] Proposal 1.6.1a:

For resolving collision of PUCCHs and PUSCHs of different priorities in step 2.2, Rel-15/16 rule is reused for PUSCH selection for UCI multiplexing.

- FFS whether/how cancellation is performed before UCI multiplexing

- FFS whether/how to handle overlapping PUSCHs on the same cell

|  | Company |
| --- | --- |
| Support | LG, QC, New H3C Huawei/Hisi, DOCOMO, Intel, Sharp, NEC, ZTE |
| Not support | Samsung (needs clarification) |

| Company | Comments |
| --- | --- |
| Samsung | We are not clear when to perform the procedure. We have agreed to drop LP PUSCH when overlapping HP positive SR. We also agreed to exclude PUSCH supporting simultaneous transmission. For other cases where a PUSCH which might be cancelled are still under discussion. We think the key issue in Rel-17 is to exclude unavailable PUSCHs, after that Rel-15/16 rule should apply by default. We suggest the following update. For resolving collision of PUCCHs and PUSCHs of different priorities in step 2.2 after excluding unavailable PUSCH(s) for UCI multiplexing, Rel-15/16 rule is reused for PUSCH selection for UCI multiplexing. - Unavailable PUSCH(s) includes the following - A LP PUSCH overlapping with HP PUCCH with positive SR. - A PUSCH supporting simultaneous transmission with PUCCH. - FFS: other cases. |

    - [Open] Overlapping PUSCHs on the same cell

      - 1st/2nd round discussion

For Rel-17 intra-UE multiplexing in case of overlapping PUSCHs on the same cell, Nokia [3] proposed to not support joint operation of Rel-17 intra-UE multiplexing and PHY prioritization of overlapping CG and DG PUSCHs with different priorities. OPPO [14] proposed that a UE does not expect overlapping PUSCHs in one cell during Rel-17 intra-UE multiplexing.

Based on the agreements from last meeting, the case is allowed as highlighted in cyan.

Agreement If multiplexing of PUCCHs and/or PUSCHs with different priorities is enabled by RRC, support both of the following UE capabilities to resolve collision of PUCCHs and/or PUSCHs with different priorities in step 2: - Capability #1: It is not expected that Rel-15 multiplexing timeline is not met for all overlapping channels [FFS the overlapping channels are resultant channels after step 1]. UE performs multiplexing or dropping of PUCCHs and/or PUSCHs with different priorities according to Rel-17 rules. - Dynamic enabling/disabling multiplexing for different priorities is not supported for Capability #1 - (Working assumption) Capability #3:… - FFS: Time unit to apply Rel-15 timeline (e.g. slot based, sub-slot based) - FFS: The set of PUSCH and PUCCH that eligible for Rel-15 multiplexing consideration Note: “collision” refers to overlapping PUCCHs, overlapping PUCCH and PUSCH (excluding PUSCH supporting simultaneous transmission with PUCCH), overlapping PUSCHs on a same cell. Note: “Rel-15 multiplexing timeline” means Rel-15 timeline calculation in Rel-16 spec, including all the formula and all the values for the variables Note: “Rel-16 prioritization timeline” means Rel-16 cancellation timeline calculation in Rel-16 spec, including all the formula and all the values for the variables

In last RAN1 meeting, the following conclusion was made in Rel-16 URLLC maintenance.

Conclusion In the Rel-16 multiplexing/prioritization procedures described in TS 38.213 section 9, the UE is expected to apply the procedures to the PUSCH(s) for which a transport block is delivered by MAC, while the PUSCH(s) for which a transport block is not delivered is ignored.

CATT [7] proposed that UE only considers PUSCH(s) for which a transport block is delivered by MAC, the PUSCH(s) for which a transport block is not delivered is ignored as in Rel-16. Samsung [9] proposed to resolve overlapping PUSCHs with different priorities on a same cell before step 2.2. Intel [17], Ericsson [19] and NEC [23] proposed to handle overlapping PUSCHs with different priorities before step 1.

Based on the contributions, different options are proposed in proposal 1.7.1 and the differences of the proposals are illustrated using the examples shown below assuming Rel-17 intra-UE multiplexing is enabled.

Example 1

Example 2

If only one MAC PDU is delivered between the LP and HP PUSCHs, the different options are equivalent. For the examples, if only PDU for LP PUSCH is delivered by MAC, then HARQ-ACK would be multiplex on LP PUSCH; if only PDU for HP PUSCH is delivered by MAC, then HARQ-ACK is transmitted on PUCCH.

However, if two PDUs are delivered by MAC, for option 1, HARQ-ACK would be multiplexed on LP PUSCH and then be dropped. For option 2, HARQ-ACK would not be multiplexed on PUSCH for both examples. For option 3, HARQ-ACK would be multiplexed on LP PUSCH and then be dropped in example 1 and HARQ-ACK would not be multiplexed on PUSCH for example 2. For option 2 and 3, in order to perform PUSCH prioritization before multiplexing, MAC should deliver the PDU for HP PUSCH sufficiently early so that UE can have sufficient time for PUCCH preparation.

Please provide your preference among the options.

Proposal 1.7.1:

For Rel-17 intra-UE multiplexing in case of overlapping PUSCHs on the same cell, down-select from:

- Option 1: When UE performs UCI multiplexing on a PUSCH in Rel-17, UE only consider PUSCH(s) for which a transport block is delivered by MAC, the PUSCH(s) for which a transport block is not delivered is ignored.

- Option 2: PUSCH prioritization is performed before Step 1 and when MAC delivers two PDUs to both of the overlapping grants, then the LP PUSCH is cancelled and ignored, while the HP PUSCH is preserved and participates in the multiplexing/prioritization procedure.

- Option 3: PUSCH prioritization is performed before Step 2.2.

- Option 4: others

|  | Supporting Company |
| --- | --- |
| Option 1 | Sony (Need clarification), Intel, CATT Huawei/Hisi |
| Option 2 | Sony (Need clarification), Samsung, vivo |
| Option 3 | Samsung |
| Option 4 | Nokia/NSB, Lenovo (modified option 1),OPPO |

| Company | Comments |
| --- | --- |
| Nokia/NSB | We prefer to not support the joint operation of ‘handling overlapping DG PUSCH and CG PUSCH of different priorities’ and ‘multiplexing and prioritization using Steps 1 and 2 considering UE capability #1’. This would simplify the discussions, something that is preferred especially that we have entered the maintenance phase. So in this respect Option 1, but we think it should be clarified that there should be only a single MAC PDU delivered on a CC (i.e. there should be no case where PHY prioritization of overlapping PUSCHs on a CC is to be handled). |
| Sony | Are Option 1 and Option 2 really different options? Option 1 is for case where only 1 PDU is delivered to L1 whilst Option 2 is for the case where 2 PDU’s are delivered to L1. Shouldn’t we combine Option 1 and Option 2 for different cases? |
| QC | We also feel the proposal can be reformulated. Our view is that, there are 4 options here: Option 1: CG/DG PUSCH prioritization is done before step 1; option 2: CG/DG PUSCH prioritization is done after step 2; option 3: CG/DG PUSCH prioritization is performed before Step 2.2; Option 4: not supporting joint operation of CG/DG prioritization and step 1&2 multiplexing. |
| Intel | We also think, maybe some clarification for Opt 1 and Opt 2 is needed. For both Options, LP PUSCH is excluded before step 1. The difference is, opt 1 is for single MAC PDU, and opt 2 is for two MAC PDUs. - If companies think, only single MAC PDU case is supported for intra-UE multiplexing, opt 1 is the preferred option. In our understanding, with Rel-15 timeline, there is sufficient time to know the presence of HP PUSCH in advance, only single MAC PDU is delivered to PHY. So, we support opt 2. - If companies think, only two MAC PDU case is supported for intra-UE multiplexing, opt 2 is the preferred option. - If companies think, both single and two MAC PDU cases are supported for intra-UE multiplexing, opt 1+ opt 2 is the preferred option. We’re also fine with opt 4, i.e., CG/DG PUSCH prioritization is not supported if intra-UE multiplexing is configured. We can save time for debating whether single or/and two MAC PDUs are supported for this case. |
| ZTE | We slightly prefer solving the CG/DG PUSCH prioritization before step 1. |
| Lenovo | We suggest the following update in option 1: “When UE performs UCI multiplexing on a PUSCH in Rel-17, UE only consider PUSCH(s) for which a transport block is delivered by MAC or PUSCH(s) with SP-CSI or A-CSI, the PUSCH(s) for which neither a transport block is not delivered nor CSI report(s) is included is ignored.” |
| Samsung | Our first preference is Option 3. We are also fine with Option 2 although it seems violate previous agreements. We have two clarification questions on Option 1 and Option 2. Q1: Does it include resolving semi-static DL symbols/SSB as discussed in 3.1.9 Processing order or Rel-17 intra-UE multiplexing and cancellation? In our understanding, if a CG PUSCH collides with semi-static DL symbols, MAC should not deliver a MAC PDU. Q2: Does the MAC layer generate a MAC PDU if a CG PUSCH collides with a DG PDSCH on a same cell? The related issue is discussed in 3.1.9. |
| CATT | We would like to clarify that Option 1 includes two PDU case as explained by FL above the proposal. |
| Ericsson | We support performing CG-vs-DG PUSCH prioritization before step 1. For the options, We support Option 2. Option 1 is the same as the conclusion in Rel-16 maintenance, and can be put under Option 2. Option 1 is adequate if MAC delivers only one PDU, as in Rel-16. Option 2 text provides the additional processing if MAC delivers two PDU. |
| DOCOMO | We share similar view with other companies that the proposal can be reformulated. |
| vivo | Option 2 can avoid unnecessary dropping of LP HARQ-ACK as shown in the following figure. |
| OPPO | We prefer to not support the joint operation of ‘handling overlapping DG PUSCH and CG PUSCH of different priorities’ and ‘intra UE multiplexing’, considering the following points: - Different timeline requirement for ‘handling overlapping DG PUSCH and CG PUSCH of different priorities’ and ‘intra UE multiplexing’, how to handle different timeline requirement? Complex gNB reception. There is blind decoding on whether CG PUSCH exists or not |
| Huawei/Hisi | Agree with Nokia and OPPO that the UE Capability#1 cannot support the prioritization between DG PUSCH vs CG PUSCH both delivered with MAC PDUs (which actually follows the R16-like prioritization timeline as N2+d1+d3), i.e., Option 2 and Option 3. The joint configuration should not be supported. Only Option 1 is the valid case for intra-UE multiplexing Capability#1. |
| LG | We also have similar view with QC. It can be better to reformulate the options including legacy procedure/rule. |

      - 3rd round discussion

It seems that the proposal in 1st round needs some clarifications. The intention of Option 1 was to consider both one MAC PDU and two MAC PDU cases. In this round, one MAC PDU and two MAC PDU cases are separately discussed.

For one MAC PDU case, I hope that companies have the same understanding. Please check the following proposal.

Proposal 1.7.1a:

For Rel-17 intra-UE multiplexing in case of overlapping PUSCHs on the same cell, if only one TB is delivered by MAC, only the PUSCH for which a TB is delivered by MAC is considered in step 1 and step 2.2, while the PUSCH for which a TB is not delivered by MAC is ignored in step 1 and step 2.2.

|  | Company |
| --- | --- |
| Support | DOCOMO, Intel, Sharp Huawei/Hisi, New H3C，Samsung, Ericsson, ZTE, NEC, ITRI, Sony |
| Not support | OPPO, QC, Nokia/NSB (clarification needed) |

| Company | Comments |
| --- | --- |
| OPPO | Not support the joint operation of ‘handling overlapping PUSCH of different priorities’ and ‘intra UE multiplexing’. Further taking CG and/or PUSCH skipping into account, the issue becomes more complex and gNB needs to be blind decode due to gNB does not know which PUSCH’s MAC is delivered. |
| QC | Indeed, considering CG and PUSCH skipping, this joint feature seems unmanageable, based on the lesson learned for UCI mux + PUSCH skipping discussion for Rel-15/16. Same comment as OPPO, let’s not consider this joint feature. |
| Ericsson | Proposal 1.7.1a simply describes the existing Rel-16 case. |
| vivo | We don’t object the proposal. If it is hard to converge on this issue, we are fine to consider single feature first. |
| Nokia/NSB | As previously mentioned, we prefer to not support the joint operation of ‘handling overlapping DG PUSCH and CG PUSCH of different priorities’ and ‘multiplexing and prioritization using Steps 1 and 2 considering UE capability #1’. Or is this for the CG of different priorities, where only a single TB is to be delivered? |

For two MAC PDU case, different options can be considered.

- Option 1: Both PUSCHs are considered in step 1 and step 2.2.

- Option 2: Only HP PUSCH is considered in step 1 and step 2.2.

- Option 3: Both PUSCHs are considered in step 1 and only HP PUSCH is considered in step 1 and step 2.2.

The benefit of Option 1 is that it does not require MAC to deliver the PDU for the later PUSCH too early in order to not perform UCI multiplexing on the earlier PUSCH. The benefit of Option 2 is that HARQ-ACK dropping can be avoided in the two examples shown in section 3.1.7.1. The benefit of Option 3 is that HARQ-ACK dropping can be avoided in example 2 shown in section 3.1.7.1.

Proposal 1.7.1b:

For Rel-17 intra-UE multiplexing in case of overlapping PUSCHs on the same cell, if two TBs are delivered by MAC for both PUSCHs, down-select from:

- Option 1: both PUSCHs are considered in step 1 and step 2.2.

- Option 2: only HP PUSCH is considered in step 1 and step 2.2.

- Option 3: both PUSCHs are considered in step 1 and only HP PUSCH is considered in step 1 and step 2.2.

|  | Supporting Company |
| --- | --- |
| Option 1 |  |
| Option 2 | Samsung, Ericsson, NEC, ITRI |
| Option 3 | DOCOMO (with clarification) |

| Company | Comments |
| --- | --- |
| DOCOMO | Regarding Option 1, we think it is the simplest option, but we prefer other options to avoid HP HARQ-ACK dropping. Regarding Option 2, it would complicate the overall framework. For example, if there is a LP HARQ-ACK overlapping with a HP HARQ-ACK and LP PUSCH as shown in the figure below, the LP HARQ-ACK is not multiplexed in the LP PUSCH in step 1, while it is multiplexed with the HP HARQ-ACK in step 2.1. Therefore, different operation is required for UE in step 1 and step 2.1 in addition to 2.2 depending on the number of PDUs delivered by MAC. Regarding Option 3, the intention should be clarified because it seems the first sentence and the second sentence contradict each other. We understand the intention is that “both PUSCHs are considered in step 1 and only HP PUSCH is considered in step 1 and step 2.2.” If this understanding is correct, we prefer Option 3 because, in contrast to Option 2, different operation is required only in step 2.2. |
| OPPO | Not support the joint operation of ‘handling overlapping DG PUSCH and CG PUSCH of different priorities’ and ‘intra UE multiplexing’ |
| Intel | In our understanding, with Rel-15 timeline, MAC layer would only deliver one MAC PDU to PHY layer, which is same as Rel-15 DG/CG overriding case. So, there is no need of proposal 1.7.1b. We’re also fine to support no joint operation of ‘handling overlapping DG PUSCH and CG PUSCH of different priorities’ and ‘intra UE multiplexing’. Then, Both proposal 1.7.1a and 1.7.1b is not needed. |
| QC | Similar comment as OPPO. Let’s focus on complete the baseline for intra-UE mux and not consider this joint feature. |
| Huawei/Hisi | Option 4: not support simultaneous configuration of intra-UE multiplexing and CG/DG PUSCH prioritization with two MAC PDUs delivered. As explained in the 1st round, CG/DG PUSCH prioritization applies R16-like timeline and should not be compatible with Capability#1. |
| New H3C | We wonder whether this proposal is required or not because based on Rel-15 timeline, MAC layer would only deliver one MAC PDU to PHY layer. Of course we are open to discuss about it. |
| Samsung | Option 2 has the best performance, it can avoid UCI dropping. |
| Ericsson | Option 2 performs DG-vs-CG PUSCH prioritization before Step 1. This ensures that the intra-UE multiplexing procedure stays the same (i.e., always have one PUSCH as input), regardless of one or two PDUs are delivered by MAC. |
| vivo | We prefer option 2. But if it is hard to converge on this issue, we are fine to consider single feature first. |
| NEC | We share same view with Samsung that option 2 can avoid unnecessary LP UCI dropping. |
| Nokia/NSB | None of the above options. The intention of UE capability #1 is to prevent any need for prioritization cancellation. Therefore, the case of overlapping of CG & DG PUSCH of different priorities should not be supported for R17 Intra-UE multiplexing. We have not agreed the support of the combination of the features – please note that the PHY prioritization operation was included to complete the Rel-16 PHY prioritization framework, which is not applicable for UE capability #1 (as also commented in yesterday’s GTW discussions) |

      - 6th round discussion

Based on the previous discussions, the views are divergent and several companies commented to not support joint operation of ‘handling overlapping DG PUSCH and CG PUSCH of different priorities’ and ‘intra UE multiplexing’, which moderator thinks is the way to go in principle.

However, moderator’s understanding is that the issue is not only for ‘handling overlapping DG PUSCH and CG PUSCH of different priorities’. For overlapping HP CG and LP CG PUSCHs on the same serving cell, two MAC PDUs are allowed to be delivered by MAC and the same issues as discussed above exist.

In addition, it is under discussion in [107bis-e-R17-IIoT-URLLC-04] on whether to introduce RRC parameters to enable the UE handling for overlapping CG/DG PUSCH of different priorities and several companies prefer not to. If that is the case, at least joint operation of ‘handling overlapping DG PUSCH and CG PUSCH of different priorities’ and ‘intra UE multiplexing’ cannot be avoided by not configuring/enabling the two features simultaneously.

So based on the above thinking, moderator’s proposal is as follows and companies are invited to provide your views.

Proposed Conclusion 1.7.1c:

It is not expected that MAC PDUs are delivered for two overlapping PUSCHs on a same serving cell involved in Rel-17 intra-UE multiplexing.

|  | Company |
| --- | --- |
| Support | LG, New H3C Huawei/Hisi, DOCOMO, Intel, Sharp, NEC, ZTE |
| Not support | QC (support the intention but not the current formulation of the current proposal) |

| Company | Comments |
| --- | --- |
| QC | We support the intention of the proposal. But the wording of the proposal seems not accurate. One may read this proposal as different priority CG-PUSCH vs DG-PUSCH prioritization is not support in Rel-17. We still suggest to go with the wording to say ‘handling overlapping DG PUSCH and CG PUSCH of different priorities’ and ‘intra UE multiplexing’ is not configured simultaneously. As for HP CG-PUSCH and LP CG-PUSCH overlapping, the handling of it can be up to UE implementation. No specification is needed. |

    - Interaction with PUCCH repetitions

      - 1st/2nd round discussion

Samsung [9] and Intel [17] discussed the joint operation of Rel-17 intra-UE multiplexing.

Samsung proposed that if a LP PUCCH overlaps in time with a HP PUCCH in Step 2.1 and at least one of the PUCCHs is with repetitions, the LP PUCCH is dropped. When there are more than two overlapping PUCCHs and at least one of the PUCCHs is with repetitions, Samsung proposed that Step 2.1 can consist of the following sub-steps to avoid LP HARQ-ACK dropping in the case illustrated in the following figure.

- Sub-step 2.1.1: Resolve collision of PUCCHs without repetitions

- Sub-step 2.1.2: Resolve collision of PUCCHs (with or without repetitions)

For Step 2.2, Samsung’s proposal is as follows to avoid HP HARQ-ACK dropping in the case shown in the following figure.

For handling overlapping PUCCHs/PUSCHs with different priorities in R17, Step 2.2 can consist of the following sub-steps.

- Sub-step 2.2.1: Resolve overlapping PUSCHs with different priorities on a same cell.

- Sub-step 2.2.2: Resolve overlapping PUCCHs with repetitions and PUSCHs.

- Sub-step 2.2.3: Resolve overlapping PUCCHs without repetitions and PUSCHs.

Intel proposed that if a LP PUCCH overlaps with a HP PUCCH with repetition, LP PUCCH should be cancelled. If a LP PUSCH overlaps with HP PUCCH with repetition, then LP PUSCH should be dropped.

The issue was not discussed in previous meetings. Companies are invited to share your views on the following proposals.

Proposed Conclusion 1.8.1:

For resolving collision of two overlapping channels with different priorities in Step 2,

- If a LP PUCCH overlaps with only one HP PUCCH and the LP or HP PUCCH is with repetitions, the LP PUCCH is dropped.

- If a LP PUSCH overlaps with a HP PUCCH with repetitions, the LP PUSCH is dropped.

- If a HP PUSCH overlaps with a LP PUCCH with repetitions, the LP PUCCH is dropped.

Note: the dropping of LP PUCCH/PUSCH is performed per repetition.

|  | Company |
| --- | --- |
| Support | Nokia/NSB (in principle – see comment below), Sharp, Intel (only support 2nd sub-bullet and 3rd sub-bullet), ZTE,Samsung, CATT, Ericsson (further clarification needed), DOCOMO,OPPO, Huawei/Hisi |
| Not support | QC |

| Company | Comments |
| --- | --- |
| Nokia/NSB | We support the proposal if its intention is to only drop the LP PUCCH repetitions that overlap with the HP PUCCH/PUSCH. This is probably what the Note is alluding to, but we would like to confirm our interpretation of the proposal. |
| Sony | Sub-bullet 1: Why is this restricted to “only one HP PUCCH”? Does this “one HP PUCCH” refers to one repetition of a HP PUCCH or two separate HP PUCCH, e.g. PUCCH#1 and PUCCH#2 where either PUCCH#1 or PUCCH#2 or both contains repetition? Sub-bullet 3: The text seems to say the entire LP PUCCH is dropped, i.e. all repetitions but in the NOTE it said dropping is done per repetition. Perhaps we need to clarify that it meant only the LP PUCCH repetitions that overlaps with HP PUSCH are dropped. That is: - If a HP PUSCH overlaps with a LP PUCCH with repetitions, the LP PUCCH repetition that overlaps with HP PUSCH is dropped. |
| QC | 1, We are wondering whether this is Rel-16 maintenance issue or Rel-17 maintenance issue? 2, a minor question on this: “If a LP PUCCH overlaps with only one HP PUCCH and the LP or HP PUCCH is with repetitions, the LP PUCCH is dropped.” Why limiting to only one HP PUCCH? If the LP PUCCH overlap with more than 1 HP PUCCH, then what is UE behaviour? |
| Sharp | Agree in principle. Not sure if we need to describe the cases. The same principle as in current spec can be used to include UCI with different priorities, e.g. “A UE does not multiplex different UCI types or UCIs from overlapping channels with different priorities in a PUCCH transmission with repetitions.” |
| Intel | We support to always drop LP PUCCH, as long as the LP PUCCH overlaps with a HP PUCCH and at least one of the LP and HP PUCCH is with repetition. 1st sub-bullet is to optimize the LP PUCCH dropping by adding additional rule, e.g., when a LP PUCCH overlaps with both HP HARQ-ACK and HP SR with repetition, UE first checks whether LP PUCCH can be multiplexed onto HP HARQ-ACK. It is similar to the issue under section 3.1.3 (only difference is HP SR is without repetition). We don’t prefer such optimization at this late stage. Therefore, we don’t support 1st sub-bullet We support 2nd and 3rd sub-bullet for PUSCH handling. To answer QC’s question for Rel-16 vs Rel-17 issue, in Rel-16, LP is dropped no matter LP or HP is with repetition, but in Rel-17, LP is dropped only if one of them is with repetition. |
| ZTE | Agree Sony, QC, Sharp’s comments, the revision from Sharp sounds better. |
| Samsung | For two overlapping channesl, the proposal is already captured in current spec as following. We are fine to clarify it if companies think it is necessary. if // this is for cases the UE supports multiplexing information of different priorities in a PUCCH/PUSCH - a PUCCH transmission with HARQ-ACK information, without repetitions, with smaller priority index overlaps with a PUCCH transmission with HARQ-ACK information, without repetitions, with larger priority index, or - a PUCCH transmission with HARQ-ACK information, without repetitions, with smaller or larger priority index overlaps, respectively, with a PUSCH transmission with larger or smaller priority index the UE multiplexes HARQ-ACK information of different priority indexes in a same PUCCH or PUSCH transmission and applies the procedures in clause 9.2.5.3 or 9.3, respectively - else - if the UE would transmit the following channels that would overlap in time where, if a channel transmission is with repetitions, the following are applicable per repetition - a first PUCCH of larger priority index and a second PUCCH of smaller priority index - a first PUCCH of larger priority index and a second PUSCH of smaller priority index when the UE cannot simultaneously transmit the first PUCCH and second PUSCH - a first PUCCH of smaller priority index and a second PUSCH of larger priority index when the UE cannot simultaneously transmit the first PUCCH and second PUSCH - a first PUSCH of larger priority index and a second PUSCH of smaller priority index on a same serving cell, where at least one of the two PUSCHs is a configured grant PUSCH the UE - transmits the PUCCH or the PUSCH of the larger priority index, and - does not transmit a PUCCH or a PUSCH of smaller priority index Sharp’s proposal is not acceptable to us, “not expect” means error case. This is too restrictive for HP PUCCH. |
| Ericsson | We agree in principle that reusing existing procedures for repetition cases is the easiest. In this sense, it seems sufficient to conclude that Rel-17 reuse Rel-16 procedure for handle the overlapping cases with repetition. Then for the question of dropping one or dropping all repetitions, our understanding is, Rel-16 performs dropping per repetition, and this should be reused in Rel-17; |
| vivo | For PUCCH/PUSCH repetition, we prefer to reuse the current rule. That is, if PUCCH is with repetition, only prioritization is supported. |
| LG | We also have similar view with QC and Sharp. We are not sure if something beyond current spec needs to additionally be described. |

Proposal 1.8.2:

For resolving collision of PUCCHs and/or PUSCHs with different priorities in Step 2.1, first resolve collision of PUCCHs without repetitions and then resolve collision of PUCCHs (with or without repetitions).

|  | Company |
| --- | --- |
| Support | Nokia/NSB (with small change),Samsung |
| Not support | QC,vivo, Intel |

| Company | Comments |
| --- | --- |
| Nokia/NSB | We talk about step 2.1 and mention resolving PUCCH & PUSCH overlap. ‘and/or PUSCH’ should be removed here, as this is step 2.2 discussed in the proposal below!? |
| QC | We can understand the intention of Proposal 1.8.3 is to avoid drop HP HARQ-ACK. But for this proposal which is targeting the following scenario. If we follow the proposal, HP A/N and LP A/N may be mux on a new resource which overlap with the HP SR repetition. Then SR with repetition wins and HARQ-ACK will be dropped, no? If we do the opposite way to handle SR repetitions and LP HARQ-ACK first, LP HARQ-ACK will be dropped. But at least the HP HARQ-ACK is transmitted. |
| Samsung | The intention of the proposal is to clarify UE behaviour. In the case above mentioned by QC, UE behaviour is not clear. There can be two options. Option 1: LP HARQ-ACK is dropped. Option 2: LP HARQ-ACK is multiplexed with HP HARQ-ACK. The above options depend on the order of how to handle the collision. If UE resolves the collision of PUCCH with repetitions first, LP HARQ-ACK should be dropped. If UE resolves the collision of PUCCH without repetitions first, LP HARQ-ACK is multiplexed in the HP HARQ-ACK PUCCH. UE behaviour should be clarified. Regarding QC’s comment, we don’t agree with “intention of Proposal 1.8.3 is to avoid drop HP HARQ-ACK”, the intention is to avoid dropping LP HARQ-ACK. Regarding “HP A/N and LP A/N may be mux on a new resource which overlap with the HP SR repetition.” may not happen, the common understanding for Proposal 1.2.1a: is the resulting PUCCH won’t overlap with HP SR. The difference is on the HP PUSCH. Regarding “ Then SR with repetition wins and HARQ-ACK will be dropped”, we don’t agree, if the case happens, the SR should be dropped according to 9.2.6. A UE does not multiplex different UCI types in a PUCCH transmission with repetitions over NPUCCHrepeat>1 slots. If a UE would transmit a first PUCCH over more than one slot and at least a second PUCCH over one or more slots, and the transmissions of the first PUCCH and the second PUCCH would overlap in a number of slots then, for each slot of the number of slots and with UCI type priority of HARQ-ACK > SR > CSI with higher priority > CSI with lower priority - the UE does not expect the first PUCCH and any of the second PUCCHs to start at a same slot and include a UCI type with same priority - if the first PUCCH and any of the second PUCCHs include a UCI type with same priority, the UE transmits the PUCCH starting at an earlier slot and does not transmit the PUCCH starting at a later slot - if the first PUCCH and any of the second PUCCHs do not include a UCI type with same priority, the UE transmits the PUCCH that includes the UCI type with higher priority and does not transmit the PUCCH that include the UCI type with lower priority. |
| vivo | We think in step 2.1, it should follow the same procedure for PUCCHs overlapping handling in R15. The issue also exists in R15, as shown in the following figure, where all PUCCHs are with the same priority, CSI is dropped because SR is configured with repetition. The proposal is for optimization and should be deprioritized. |
| Samsung2 | Regarding vivo’s comment “The issue also exists in R15”. We think UE behaviour in Rel-15 is not clear either. Consider the example below, When considering the collision of SR and HARQ-ACK, SR should be dropped. When considering the collision of SR and CSI, CSI should be dropped. When considering the collision of CSI and HARQ-ACK, CSI should be multiplexed with HARQ-ACK. The order how UE handles the collision is not defined. In this case, whether CSI is dropped is not clear. |
| LG | We’d like to understand the motivation. Is the motivation of “first resolve collision of PUCCHs without repetitions” to avoid drop LP HARQ-ACK? |
| Intel | If we agree “Perform pair-wise collision resolution sequentially in time” in section 3.1.3, there is no need to discuss whether first resolve collision with or without repletion in step 2. In step 1, we share same view with Samsung that how to handle the case in the figure shown by Samsung is not clear. But in step 2, there is no such case (3 channels are all overlapped) |
| Samsung3 | To LG, regarding the motivation of the proposal, firstly, UE behaviour should be clarified. Secondly, LP HARQ-ACK dropping should be avoided. |

Proposal 1.8.3:

For resolving collision of PUCCHs and/or PUSCHs with different priorities in Step 2.2, first resolve collision of PUCCHs with repetitions and PUSCHs, and then resolve collision of PUCCHs without repetitions and PUSCHs.

|  | Company |
| --- | --- |
| Support | Nokia/NSB (with small change), Samsung, Intel |
| Not support | LG |

| Company | Comments |
| --- | --- |
| Nokia/NSB | We talk about step 2.2 and mention resolving PUCCH and/or PUSCH overlap. As this is step 2 it should be ‘and/or PUSCH’ (as without PUSCH, there is no step 2.2?). |
| Samsung | Similar as Proposal 1.8.2, UE behaviour should be clarified. Fine with Nokia’s update. |
| LG | We think the case of overlapping between LP PUSCH and two HP PUCCHs, anyhow, needs to be handled without differentiation according to whether repetition is applied for the HP PUCCH. |
| Intel | In our understanding, it is similar to other issues discussed in other sections, e.g., a LP PUSCH overlaps with a HP SR, or a LP PUSCH overlaps with two HP PUCCHs. We need to determine, whether to resolve overlapping and exclude such LP PUSCH first, or perform UCI multiplexing first. A unified solution is preferred. We prefer to resolve overlapping and exclude such LP PUSCH first. |

      - 4th round discussion

In this round, it is proposed to continue discussing proposal 1.8.1 as in previous rounds which was supported by majority companies.

Please comment only if you are not fine with the proposal. Otherwise if you are fine with the proposal, you do not need to comment.

Proposed Conclusion 1.8.1:

For resolving collision of two overlapping channels with different priorities in Step 2,

- If a LP PUCCH overlaps with only one HP PUCCH and the LP or HP PUCCH is with repetitions, the LP PUCCH is dropped.

- If a LP PUSCH overlaps with a HP PUCCH with repetitions, the LP PUSCH is dropped.

- If a HP PUSCH overlaps with a LP PUCCH with repetitions, the LP PUCCH is dropped.

Note: the dropping of LP PUCCH/PUSCH is performed per repetition.

| Company | Comments |
| --- | --- |
| Sony | We re-iterate our previous comment here. Sub-bullet 1: Why is this restricted to “only one HP PUCCH”? Does this “one HP PUCCH” refers to one repetition of a HP PUCCH or two separate HP PUCCH, e.g. PUCCH#1 and PUCCH#2 where either PUCCH#1 or PUCCH#2 or both contains repetition? Sub-bullet 3: The text seems to say the entire LP PUCCH is dropped, i.e. all repetitions but in the NOTE it said dropping is done per repetition. Perhaps we need to clarify that it meant only the LP PUCCH repetitions that overlaps with HP PUSCH are dropped. That is: - If a HP PUSCH overlaps with a LP PUCCH with repetitions, the LP PUCCH repetition that overlaps with HP PUSCH is dropped. |
| QC | 1, We are wondering whether this is Rel-16 maintenance issue or Rel-17 maintenance issue? There is no multiplexing involved at all in this proposal, and reuse Rel-16 spec is sufficient. Even there is any open issue, to us, it is a Rel-16 issue. 2, A minor question on this: “If a LP PUCCH overlaps with only one HP PUCCH and the LP or HP PUCCH is with repetitions, the LP PUCCH is dropped.” Why limiting to only one HP PUCCH? If the LP PUCCH overlap with more than 1 HP PUCCH, then what is UE behaviour? |
| Ericsson | it seems sufficient to conclude that Rel-17 reuse Rel-16 procedure of per-repetition cancellation when one or more of the overlapping channels use repetition. The proposed conclusion is still confusing, e.g., do the LP and HP channels start in the same slot or not? Do the LP and HP channels have same or different number of repetitions? Dropping means drop one or all remaining repetitions? |
| Intel | We don’t support 1st sub-bullet as previously commented. We don’t understand why we need the restriction of ‘only one’ high-lighted below. It highly depends on the procedure option 1 or option 2. - If a LP PUCCH overlaps with only one HP PUCCH and the LP or HP PUCCH is with repetitions, the LP PUCCH is dropped. |
| LG | We have similar comments with QC and Ericsson. |
| Samsung | We think the conclusion only clarifies current behaviour. With or without the conclusion UE behaviour is the same. Regarding “only one” in the first sub-bullet, the intention is to exclude the case below, for more than two overlapping channels, the solutions can be further discussed. For example, LP HARQ-ACK is dropped or multiplexed in the HP HARQ-ACK PUCCH? Regarding QC ‘s question on whether this is Rel-16 or Rel-17 issue, in Rel-16, LP HARQ-ACK is dropped, in Rel-17, UE behaviour is not clear. |
| vivo | Agree with Ericsson. It is ok to conclude that Rel-17 reuse Rel-16 procedure for the repetition case. |
| Intel | Thanks Samsung for the clarification. If 1st sub-bullet is only for two overlapping channel case, as you previously explained, it is already covered by Rel-17 CR, so we don’t need this agreement. Regarding the case of one LP PUCCH overlaps with two HP PUCCH and at least one of PUCCH is with repetition, we’re open for further discussion, it is highly depends on section 3.1.3.3, because if we go with option 1, we don’t need special handling for this case, but if we go with option 2, we may need 2 sub-steps as you proposed (copied below), and for each sub-step, we may need to handle multiple PUCCHs with different priorities. We really think it is a complicated solution rather than simpler solution. - Sub-step 2.1.1: Resolve collision of PUCCHs without repetitions - Sub-step 2.1.2: Resolve collision of PUCCHs (with or without repetitions) |
| Nokia/NSB | We agree with comments by QC and Ericsson, that some more clarification is needed (e.g. per PUCCH repetition or the overall repetition bundle). |

    - Others

      - 1st/2nd round discussion

Number of polar coding chains

Nokia discussed the case when more than 3 polar encoding chains are required for the UE to enable the simultaneous transmission of two PUSCHs including UCIs on different UL CCs, e.g. a low-priority PUSCH containing UCI (e.g., LP HARQ-ACK) on UL CC1 and a high-priority PUSCH with UCI (e.g., HP HARQ-ACK and A-CSI consisting of CSI part 1 & part 2) on UL CC2 and proposed the following clarification in [3].

Proposal 2.4: RAN1 to clarify, that simultaneous PUSCH transmissions of different PHY priorities with multiplexed-UCI (based on Step 1 of the Rel-17 Intra-UE multiplexing framework) on different UL CCs requiring in total more than 3 Polar encoding chains is supported.

Proposed Conclusion 1.9.1:

RAN1 to clarify, that simultaneous PUSCH transmissions of different PHY priorities with multiplexed-UCI (based on Step 1 of the Rel-17 Intra-UE multiplexing framework) on different UL CCs requiring in total more than 3 Polar encoding chains is supported.

|  | Company |
| --- | --- |
| Support | Nokia/NSB, Samsung, CATT Huawei/Hisi, Sharp |
| Not support | OPPO |

| Company | Comments |
| --- | --- |
| Nokia/NSB | It’s good to conclude on this aspect, so that we are sure that companies have the same understanding for the cases where the number of required UCI encoding chains exceeds 3. |
| Samsung | We think similar issue exists in Rel-16, for example, two PUSCHs with different priorities on different cells and both PUSCHs are multiplexed with UCIs. By default, the proposal is supported but fine to clarify it. |
| OPPO | To avoid additional complexity, the same total polar encoding chain number should be kept as R15/16. Even total polar encoding chain number is 3, simultaneous PUSCH transmission of different PHY priorities still works in some cases. In other words, increased total polar encoding chain number is not necessary condition to support simultaneous PUSCH transmission of different PHY priorities in some cases. |
| LG | We also have same understanding with Samsung. It seems there could be the cases requiring more than 3 Polar encoding chains even in Rel-16, so we are not sure if any further clarification is needed. |

Enabling Rel-16 Prioritization when Rel-17 Mux is Enabled

In [8], the following mechanism is proposed for UE to operate Rel-16 prioritization when Rel-17 intra-UE multiplexing with different priorities is RRC enabled. Please refer to the contribution for more details.

Proposal 2: Introduce a time window TWinDrop that overlaps the slot or sub-slot of LP PUCCH/PUSCH and if the first DL transmission scheduling a HP PUCCH/PUSCH that overlaps with the LP PUCCH/PUSCH:

- ends within TWinDrop, the UE uses Rel-16 prioritisation to resolve the collision

- ends outside of TWinDrop, the UE uses Rel-17 Mux to resolve the collision

It is moderator’s understanding that the proposal is not inline with the agreement for Cap#1. Companies please comment if you think otherwise.

| Company | Comments |
| --- | --- |
| Sony | Only Cap#3 (dynamic indication of enabling/disabling of multiplexing) is not discussed. Since this is not dynamic indication, this is inline with Cap#1 discussion. |
| QC | Object. We prefer not to reopen the discussion on dynamic switch between Rel-16 and Rel 17 behaviour, which was already concluded in last meeting. The proposal is just another way to implement it. |
| Samsung | Agree with FL. |
| Nokia/NSB | We have a similar view as the moderator. |
| OPPO | Object. And Agree with FL’s analysis. |
| Huawei/Hisi | Agree |
| LG | We also agree with FL. |

Processing order or Rel-17 intra-UE multiplexing and cancellation

For Rel-16 intra-UE multiplexing we have the following agreements in RAN1#104e.

Agreement To address collision with semi-static DL symbols and SSB, the following easy way is suggested: - Step1: Perform intra UE prioritization (including multiplexing, overriding) according to related working assumption in 102 e-meeting and produce final PUCCHs/PUSCHs. - Step 2: Final PUCCHs/PUSCHs is cancelled by semi-static DL symbols and SSB symbols.

Samsung [9] proposed that for handling overlapping PUCCHs/PUSCHs with different priorities in R17, cancel PUSCHs with semi-static DL conflict before intra-UE multiplexing/prioritization to avoid dropping HARQ-ACK multiplexed in a PUSCH which is cancelled by semi-static DL symbols and SSB symbols.

DOCOMO [10] proposed to discuss processing order of intra-UE multiplexing with different priorities and cancellation due to dynamic SFI/UL CI/semi-static TDD and SSB.

In Rel-16, intra-UE multiplexing is performed before cancellation. So the question is whether we need to change the principle for Rel-17 intra-UE multiplexing. Companies are invited to share your views.

| Company | Comments |
| --- | --- |
| Samsung | Yes. The 2 steps order is Rel-16 degrades UCI performance and should be reconsidered in Rel-17. Even if a PUCCH can be transmitted before multiplexing with CG PUSCH, it may not be transmitted after multiplexing. In addition, we think this issue is related to proposal 1.7.1 and should be discussed together. |
| DOCOMO | We are fine with either of the processing orders unless the processing order is clarified but slightly prefer to reuse the Rel-16 processing order for simplicity. |
| Nokia/NSB | In principle, we prefer to follow similar order as in Rel-16, i.e. intra-UE handling is done before cancellation due to e.g. DL symbols. But we are open for further discussions on this aspect. |
| OPPO | Prefer to follow R16 processing order. |
| Huawei/Hisi | Following the Rel-16 rule is preferred. |
| LG | We also prefer to follow Rel-16 rule/order. |
| Intel | We prefer to follow similar order as in Rel-16. If in Rel-16, it is no problem for HP CG PUSCH which may carry HP UCI, it is not a problem for Rel-17 intra-UE multiplexing. |
| ZTE | Following the Rel-16 rule is preferred |

In addition, Samsung [9] discussed the case that a CG PUSCH is canceled by a DG PDSCH. If a CG PUSCH overlaps with both DG PDSCH and DG PUSCH on a same cell, UE behavior is not clear. An example is shown below where a HP CG PUSCH overlaps with both DG PDSCH and LP DG PUSCH. If the UE first resolves collision between PDSCH and CG PUSCH, the UE can transmit the LP DG PUSCH, otherwise, UE cannot transmit LP DG PUSCH.

Samsung proposed that if a CG PUSCH overlaps with a DG PDSCH on a same cell, a UE cancels/drops the CG PUSCH before resolving collision of the CG PUSCH and other UL channels, if any. Companies are invited to share your views.

| Company | Comments |
| --- | --- |
| OPPO | Same principle as above issue, i.e. follow R16 processing order. |
| Samsung2 | Could oppo clarify a bit more about Rel-16 order for this case? |
| LG | We also have same view with OPPO |
| Intel | In general, similar issue also exists in Rel-16, e.g., if we replace LP DG PUSCH with LP PUCCH. The unified solution should be applied for both Rel-16 and Rel-17 case. It is unclear to us, how to handle this issue in Rel-16. Maybe we need a Rel-16 CR ? |

Power allocation

Samsung [9] proposed that in the power limited scenario of multiple PUSCHs, the power allocation should be reconsidered if a HP HARQ-ACK is multiplexed in a LP PUSCH. The power allocation should be prioritized for the LP PUSCH with HP HARQ-ACK to ensure the reliability of HP HARQ-ACK. The Proposal is that for PUSCH power allocation in case of CA, a LP PUSCH with HP HARQ-ACK should be prioritized over a PUSCH without HP HARQ-ACK. Companies are invited to share your views.

| Company | Comments |
| --- | --- |
| Nokia/NSB | We don’t see this as essential, especially that the multiplexing of HP HARQ-ACK on LP PUSCH could be avoided by gNB implementation in case of power limited CA scenarios. |
| OPPO | Follow current spec. No enhancement at this stage. |
| Huawei/Hisi | We agree that power allocation rules have to be updated to distinguish priorities of PUSCH with inter-priority UCI, including HP PUSCH with LP-HARQ-ACK, and LP PUSCH with HP HARQ-ACK. |
| LG | We also have similar view with Nokia. |
| Intel | We share same view with Nokia. |

  - Simultaneous PUCCH/PUSCH transmission

Separate enabling simultaneous PUCCH/PUSCH transmission for secondary PUCCH cell group

      - 1st/2nd round discussion

A remaining issue for simultaneous PUCCH/PUSCH transmission is whether simultaneous PUCCH/PUSCH operation can be separately / independently configured for the primary and secondary PUCCH cell group, i.e. whether the RRC parameter simultaneousPUCCH-PUSCH-secondaryPUCCHgroup is supported or not. Companies please share your views on the following proposal.

Proposal 2.1.1:

simultaneousPUCCH-PUSCH-secondaryPUCCHgroup is supported to enable simultaneous PUCCH and PUSCH transmissions with different priorities within the secondary PUCCH cell group separately from primary PUCCH cell group.

|  | Company |
| --- | --- |
| Support | Intel, Nokia/NSB, ZTE, CATT, DOCOMO, QC, Sharp, New H3C, Lenovo, Samsung, vivo, LG |
| Not support | Huawei/Hisi, QC, Ericsson |

| Company | Comments |
| --- | --- |
| Huawei/Hisi | The motivation of introducing separate enable/disable flags for primary PUCCH group and secondary PUCCH group is not justified, and a unified RRC parameter is preferred for both, to save spec effort. |
| QC | Besides saving spec effort to simply RRC parameters, it would be a necessary burden for UE to run two different procedures, one disabling simultaneous transmissions, one enabling simultaneous transmissions, for two different PUCCH groups, which does not seems a well justified use case in real deployment. |
| Ericsson | We don’t see the use case of having separate flags for the two PUCCH groups. One flag is sufficient regardless of one or two PUCCH groups. |

PHR for PUCCH

      - 1st/2nd round discussion

It was proposed to support PHR for PUCCH in NR in Rel-17 in RAN1#107-e without conclusion.

DOCOMO [10] proposed to define PHR type for PUCCH transmission with two possible solutions.

- PHR type for PUCCH transmission based on the equation of PUCCH transmission power defined in section 7.2 of 38.213

- Replace PHR for PUSCH of LTE Type 2 PHR by virtual PHR

Qualcomm [12] proposed to support type 2 actual PHR to report PHR for an actual PUCCH transmission on Pcell or a Scell in a PUCH group and type 2 virtual PHR to report PUCCH PHR on Pcell or a Scell without actual PUCCH transmission in a PUCCH group for PUCCH cell switch in NR Rel-17.

Intel [17] proposed to not consider PUCCH PHR in Rel-17 URLLC considering the benefit is not much as that in LTE with simultaneous PUCCH/PUSCH transmission in the same serving cell and to avoid impacting other WGs.

Companies are invited to provide your views on the following proposal.

Proposal 2.2.1:

Support PHR for PUCCH in NR in Rel-17.

|  | Company |
| --- | --- |
| Support | QC |
| Not support | Intel, Nokia/NSB Huawei/Hisi, Sony, New H3C, CATT, Ericsson, DOCOMO (can accept),OPPO, LG, ZTE(can accept) |

| Company | Comments |
| --- | --- |
| Intel | We understand PUCCH PHR would provide more accurate PHR than type-2 PUSCH PHR, but it is unclear how much gain can be achieved, compared with gNB approximately derive the power range for PUCCH transmission based on type-2 PUSCH PHR. We also have concern on standard effort for PUCCH PHR (can we finish all in this meeting to ensure RAN2 has sufficient time to handle new PHR?). For example, - Virtual or real PHR depending on the relation between DCI for PUCCH and DCI for PUSCH carrying PHR? - Different numerology for PUCCH and PUSCH carrier? - How to handle sub-slot PUCCH case? - Condition to report PUCCH PHR, e.g., configured by gNB, depending on whether gNB configures simultaneous PUCCH/PUSCH transmission or PUCCH carrier switching? And also, if a UE multiplexes UCI into a PUSCH, whether UE reports type-2 PHR for PUSCH and/or virtual or real PHR for PUCCH? Elaboration from proponent companies on the above two aspects would be very helpful. |
| Nokia/NSB | Not an essential issue to be solved. And UE will allocate Tx power according to channel priorities and types. |
| Huawei/Hisi | Firstly, we do not observe strong motivation of introducing the new PUCCH PHR report. What are the specific applicable cases? In addition, we have a similar feeling with Intel, that non-trivial RAN2 spec impact is observed, such as type 2 PHR reporting procedure, MAC CE design, etc., and we are not optimistic that the whole PHR report feature can be quickly finished within one or two meetings. |
| QC | We understand companies have concern to introduce new PHR type here. But this is not about introduce a new functionality which needs use cases to justify. This is to fix a hole in current spec for the following scenario. If we don’t do anything, my question is very simple: According to current RAN2 spec, UE need to report PHR for both Pcell and Scell in the following scenario. What should UE report for Scell? Please let us know your solution/answer to this open question. Even if your answer is UE report nothing for Scell, then despite the fact that we intentionally keep this feature broken, RAN1 still need to send a LS to RAN2 to ask RAN2 change spec to reflect this RAN1 decision. Regardless of what decision we take here (including even not report PHR for Scell), RAN1 need to discuss this issue and send LS to RAN2. And RAN2 have to change their spec accordingly. Therefore, we’d better wrap up this issue in this meeting. |
| Intel2 | @QC, in our understanding, UE reports virtual type-2 PHR for Scell according to 38.213, because UE does not transmit PUSCH on Scell. |
| New H3C | We have the similar view with Nokia. |
| DOCOMO | Although we think it is important to support PUCCH PHR, we understand the concern on the RAN2 impact at this late stage. We can compromise given the situation. |
| QC2 | Thank Intel for answering my question. I guess you meant UE report virtual type 1 PUSCH PHR for Scell. So, is it your assumption that gNB can figure out what is PUCCH power headroom based on PUSCH PHR for Scell? I assume the answer is yes. Honestly, I don’t think gNB can do it, because PUCCH and PUSCH are with totally separate power control loops and gNB cannot infer one from the other, given that gNB does not know UE received or missed power control commands for PUCCH and PUSCH respectively. However, maybe there are some magic gNB can play and I don’t know about it. So, I can give up my proposal and accept your proposal, conditioning on the assumption on capability of gNB is formally documented in Chairman’s notes, as the following. Can Intel and other companies accept this note? Conclusion: PHR for PUCCH is not supported in Rel-17 for simultaneous PUCCH/PUSCH transmissions. Note: it is expected that gNB is capable to infer PUCCH power headroom based on PUSCH PHR for the same CC. |
| LG | We also have similar view with Nokia. |
| Intel3 | As we previously commented, based on the PUSCH PHR, gNB can roughly/approximately estimate for PUCCH transmissions. Also, since typically PUCCH transmissions would relatively "narrow BW transmissions", the variability and thus, criticality, would be less than for PUSCH. If gNB can allocate proper power for PUCCH with or without aid of PUSCH PHR in Rel-15 (it depends on gNB implementation), it also works here for simultaneous PUCCH/PUSCH transmission. Thus, we think PUCCH PHR is optimization, not fundamental issue, i.e., the system does not break. We don’t think we need to add the note as you suggested. Besides, you may already notice, RAN1 does not need to send LS to RAN2, there is no hole in the spec. On the contrary, if RAN1 rushed to agree PUCCH PHR, it is very risky that we may create a hole in the spec, because there are too many aspects for PUCCH PHR to be resolved in just one meeting, I don’t know whether we have such magic to ensure we won’t miss anything. Let’s hear more views from other companies. |

Interaction with intra-UE multiplexing/prioritization

      - 1st/2nd round discussion

Nokia [3], Huawei [4], ZTE [6], CATT [7], Samsung [9], DOCOMO [10], Qualcomm [12], Panasonic [13], ETRI [15], Intel [17], Apple [18], Ericsson [19], Quectel [21] and LG [26] discussed the details of joint operation of Rel-17 intra-UE multiplexing and Rel-17 simultaneous PUCCH/PUSCH transmission in their contributions while vivo [5] proposed to not support the joint operation. It is proposed to first discuss the details of joint operation of Rel-17 intra-UE multiplexing and Rel-17 simultaneous PUCCH/PUSCH transmission to see whether we can converge. If the views cannot be converged, the consequence is that the joint operation is not supported in Rel-17.

Based on the previous conclusions, only simultaneous PUCCH/PUSCH transmission of different priorities on different cells for inter-band CA is supported in Rel-17. Consequently, for joint operation of Rel-17 intra-UE multiplexing and Rel-17 simultaneous PUCCH/PUSCH transmission, simultaneous PUCCH/PUSCH transmission is only considered in Step 2.2 only, which seems to be the common understanding. There are generally two options to apply simultaneous PUCCH/PUSCH transmission in step 2.2 and option 2 is supported by more companies.

If the simultaneous PUCCH/PUSCH transmission is enabled, the handling of overlapping PUCCH(s) and PUSCH(s) of different PHY priorities at Step 2.2 is done as follows:

- Option 1: If a PUCCH (that could be multiplexed into PUSCH) does not overlap with a PUSCH on the same band, consider simultaneous transmission of this PUCCH and overlapping PUSCH(s) on a different band; otherwise, i.e. if a PUCCH overlaps with a PUSCH on the same band, consider the PUSCHs on all CCs and bands as candidates for multiplexing the UCI(s) carried in this PUCCH(s) and apply the corresponding Rel-17 (and Rel-15) multiplexing rules. [3]

- Option 2: A PUSCH that can be simultaneously transmitted with a PUCCH is excluded for multiplexing the UCI of the PUCCH.

The difference of the two options is illustrated in the following figure. For option 1, given that LP PUCCH overlaps with HP PUSCH2 on the same band, PUSCHs on all CCs and bands are candidates for UCI multiplexing and HP PUSCH1 is selected due to smaller CC index. But for option 2, given that LP PUCCH can be simultaneously transmitted with HP PUSCH1, HP PUSCH1 is excluded for multiplexing and UCI is multiplexed in HP PUSCH2. The intention of Option 1 is to preserve legacy behavior on handling overlapping PUCCH and PUSCHs for CA cases, where UCI carried on the PUCCH is multiplexed on the PUSCH of the serving cell with lowest index.

In addition, several companies discussed joint operation of Rel-16 intra-UE prioritization and Rel-17 simultaneous PUCCH/PUSCH transmission. Option 2 can be extended to be applied to intra-UE prioritization. The following proposal is proposed to collect companies’ views.

Proposal 2.3.1:

If the simultaneous PUCCH/PUSCH transmission is enabled, a PUSCH that can be simultaneously transmitted with a PUCCH is excluded from overlapping channels for multiplexing the UCI of the PUCCH and for intra-UE prioritization with the PUCCH.

|  | Company |
| --- | --- |
| Support | QC, Intel, ZTE, Lenovo, Apple, Samsung, CATT, Ericsson, DOCOMO Huawei/Hisi (with changes) |
| Not support | Nokia/NSB (see comment below), LG |

| Company | Comments |
| --- | --- |
| Nokia/NSB | As discussed in our Tdoc, and captured by the Moderator above, Option 1 preserves legacy behavior on handling overlapping PUCCH and PUSCHs for CA cases, where UCI carried on the PUCCH is multiplexed on the PUSCH of the serving cell with lowest index. On the other hand, Option 2/Proposal 2.3.1 diverges from the legacy operation. |
| Ericsson | In our view the above proposal can be realized by running Step 2.2 for each band separately, including identifying the PUSCH for UCI mux. Suggest to modify the proposal so that it avoids the question of “what’s a “PUSCH that can be simultaneously transmitted”?” |
| vivo | We think the discussion for single feature of intra-UE multiplexing or simultaneous PUCCH/PUSCH transmission should be prioritized. There are still so many essential issues for single feature, we should make sure that a single feature can work well first. |
| Huawei/Hisi | We think the intention here is the simultaneous PUCCH/PUSCH Tx and intra-UE multiplexing are both enabled? Thus the operation of Step 2 is per band performed as mentioned by Ericsson. Changes as below to make it clear: If the simultaneous PUCCH/PUSCH transmission and intra-UE multiplexing are simultaneously is enabled, the intra-UE multiplexing in Step 2 is performed per band a PUSCH that can be simultaneously transmitted with a PUCCH is excluded from overlapping channels for multiplexing the UCI of the PUCCH and for intra-UE prioritization with the PUCCH. |
| Samsung2 | We have different understanding from Huawei, the proposal is clear that both intra-UE multiplexing and prioritization is considered. If simultaneous PUCCH and PUSCH transmission is configured, but intra-UE multiplexing is not configured, at least for intra band CC, prioritization should be performed. UE behaviour should be clarified for this case as well - a unified solution can be adopted. |
| LG | Not support. We have similar view with Nokia that PUSCH selection for UCI multiplexing could be done among all the PUSCHs over all the CCs (if PUCCH and at least one PUSCH is in a same band). In other words, there is no reason to limit candidate PUSCHs for UCI multiplexing only to those in the same band with PUCCH. |

In addition, Samsung [9] discussed that for a PUSCH supporting simultaneous PUSCH and PUCCH transmissions, the UCI would not be multiplexed in the PUSCH for intra-UE multiplexing of different priorities and PUCCH would not be dropped because of the PUSCH, there is no need to satisfy the timeline requirement for the PUSCH. Without the timeline requirement, the scheduling flexibility can be increased and the latency of HP PUSCH can be reduced. Companies please check the following proposal from Samsung.

Proposed Conclusion 2.3.2:

The timeline conditions of intra-UE multiplexing/prioritization of PUCCHs and PUSCHs with different priorities do not apply to a PUSCH supporting simultaneous PUSCH and PUCCH transmissions.

|  | Company |
| --- | --- |
| Support | Lenovo, Samsung, Ericsson, Nokia/NSB Huawei/Hisi |
| Not support | QC |

| Company | Comments |
| --- | --- |
| QC | It is not clear how to define “a PUSCH supporting simultaneous PUSCH and PUCCH transmissions”. Does this mean each PUSCH will have a RRC parameter to configure it support simultaneous Tx or not? Or UE would have to figure it out based on PUCCH/PUSCH overlapping? If it is the latter, then the PUSCH should also be involved in the timeline as the overlapping is pairwise, which includes the PUCCH and PUSCH. |
| Lenovo | The proposal can be modified as follows: “If the simultaneous PUCCH/PUSCH transmission is enabled, the timeline conditions of intra-UE multiplexing/prioritization of PUCCHs and PUSCHs with different priorities is not applicable to a PUSCH that can be simultaneously transmitted with a PUCCH.” |
| Samsung | The proposal doesn’t change the RRC parameter for configuration simultaneous transmission. Simultaneous transmission is supported per band, UE knows which PUSCHs support simultaneous transmission based on the scheduled cells. For UCI multiplexing in a PUSCH, UE only needs to check the PUSCH NOT supporting simultaneous transmission. Could QC clarify a bit more why the timeline is required? |
| Ericsson | We agree with the principle that mux timeline conditions only apply to those channels that are involved in multiplexing. |
| Nokia/NSB | Agree with Ericsson comment. |
| Huawei/Hisi | Intra-UE multiplexing and simultaneous PUCCH/PUSCH Tx should be different UE capabilities. UE only considers the multiplexing of intra-band in Step 2, if simultaneous PUCCH/PUSCH Tx is enabled. If there is no consensus, we are also fine not to support simultaneous configuration of both features. |
| LG | We have similar comment with QC. BTW, we think the case that timeline condition is not applicable is not limited to the above PUSCH. In general, the timeline condition is not applicable to a PUSCH without UCI multiplexing, regardless whether simultaneous PUCCH+PUSCH TX is enabled or disabled. |

      - 4th round discussion

In this round, it is proposed to continue discussing proposals as in previous rounds which was supported by majority companies.

Please comment only if you are not fine with the proposals. Otherwise if you are fine with the proposals, you do not need to comment.

Proposal 2.3.1:

If the simultaneous PUCCH/PUSCH transmission is enabled, a PUSCH that can be simultaneously transmitted with a PUCCH is excluded from overlapping channels for multiplexing the UCI of the PUCCH and for intra-UE prioritization with the PUCCH.

| Company | Comments |
| --- | --- |
| LG | Not support. We have similar view with Nokia that PUSCH selection for UCI multiplexing could be done among all the PUSCHs over all the CCs (if PUCCH and at least one PUSCH is in a same band). In other words, there is no reason to limit candidate PUSCHs for UCI multiplexing only to those in the same band with PUCCH. |
| OPPO | According to proposal 2.3.1, PUCCH can be multiplexed in PUSCH in the same band only and the PUSCH may not be the best choice, e.g. CG PUSCH or PUSCH with later starting point. So if PUCCH is to be multiplexed in one PUSCH, we’d better to follow Rel-15 rule. So we suggest to modify proposal to align Rel-15 rule and achieve better choice. when the simultaneous PUCCH/PUSCH transmission is enabled, if there is overlapping PUCCH and PUSCH in the same band (in other words, PUCCH will be multiplexed in a PUSCH finally), Run intra UE multiplexing step 2 In addition, if simultaneous PUCCH/PUSCH transmission is enabled, whether the resulting PUCCH of step 2.1 can be simultaneously transmitted with PUSCH? After step 1, - If LP PUCCH overlaps HP PUCCH, run step 2.1, and simultaneous PUCCH/PUSCH transmission not works; - elseif any band has PUCCH/PUSCH overlapping, run step 2.2, and simultaneous PUCCH/PUSCH transmission not works. - Otherwise, simultaneous PUCCH/PUSCH transmission is performed. |
| vivo | Agree with other companies, the ‘PUSCH can be simultaneously transmitted with a PUCCH’ is not clear to us. If most companies support simultaneous configuration of intra-UE multiplexing and simultaneous PUCCH/PUSCH transmission, as a compromise, we have the following suggestion: If the simultaneous PUCCH/PUSCH transmission and intra-UE multiplexing are simultaneously enabled, intra-UE multiplexing/prioritization of different priorities is performed per band. |
| Nokia/NSB | Not support. As we previously noted, with this proposal we diverge from the legacy operation. As discussed in our TDoc, the simultaneous PUSCH / PUCCH operation should only be considered if having only overlapping PUSCH of different priority on the other band. Please check the related discussions in Sec. 2 of R1-2200018. Figure 2.1: Example illustrating the handling of overlapping PUCCH and PUSCHs of different priorities. There is simultaneous PUCCH/PUSCH transmission in this case. Figure 2.2: Example illustrating the handling of overlapping PUCCH and PUSCHs of different priorities. Although enabled, there is no simultaneous PUCCH/PUSCH transmission in this case. Proposal 2.2: If the simultaneous PUCCH/PUSCH transmission is enabled, the handling of overlapping PUCCH(s) and PUSCH(s) of different PHY priorities at Step 2.2 is done as follows: - If a PUCCH (that could be multiplexed into PUSCH) does not overlap with a PUSCH on the same band, consider simultaneous transmission of this PUCCH and overlapping PUSCH(s) on a different band; - otherwise, i.e. if a PUCCH overlaps with a PUSCH on the same band, consider the PUSCHs on all CCs and bands as candidates for multiplexing the UCI(s) carried in this PUCCH(s) and apply the corresponding Rel-17 (and Rel-15) multiplexing rules. |

Proposed Conclusion 2.3.2:

The timeline conditions of intra-UE multiplexing/prioritization of PUCCHs and PUSCHs with different priorities do not apply to a PUSCH supporting simultaneous PUSCH and PUCCH transmissions.

| Company | Comments |
| --- | --- |
| QC | To us, it is still not clear what is the definition of PUSCH supporting simultaneous PUSCH and PUCCH transmission. Maybe it is just wording issue but we think the version provided by Lenovo is better, which is copied below with minor update. “If the simultaneous PUCCH/PUSCH transmission is enabled, the timeline conditions of intra-UE multiplexing/prioritization of PUCCHs and PUSCHs with different priorities is not applicable to a PUSCH that can be is simultaneously transmitted with a PUCCH” |
| vivo | If the simultaneous PUCCH/PUSCH transmission and intra-UE multiplexing are simultaneously enabled, intra-UE multiplexing/prioritization of different priorities is performed per band. We think the PUSCH that can be simultaneously transmitted with a PUCCH is not involved in the timeline definition. |
| LG | We have similar comment with QC, and the QC’s update in above looks clear. BTW, we think the case that timeline condition is not applicable is not limited to the above PUSCH. In general, the timeline condition is not applicable to a PUSCH without UCI multiplexing, regardless whether simultaneous PUCCH+PUSCH TX is enabled or disabled. |

      - 5th round discussion

For the interaction between simultaneous PUCCH/PUSCH transmission and intra-UE intra-UE multiplexing/prioritization, there are two options as summarized in section 3.2.3.1. The difference between the options has been provided in section 3.2.3.1 as well.

- Option 1:

If the simultaneous PUCCH/PUSCH transmission is enabled, the handling of overlapping PUCCH(s) and PUSCH(s) of different PHY priorities at Step 2.2 is done as follows:

- If a PUCCH (that could be multiplexed into PUSCH) does not overlap with a PUSCH on the same band, consider simultaneous transmission of this PUCCH and overlapping PUSCH(s) on a different band;

- otherwise, i.e. if a PUCCH overlaps with a PUSCH on the same band, consider the PUSCHs on all CCs and bands as candidates for multiplexing the UCI(s) carried in this PUCCH(s) and apply the corresponding Rel-17 (and Rel-15) multiplexing rules.

- Option 2: If the simultaneous PUCCH/PUSCH transmission is enabled, a PUSCH that can be simultaneously transmitted with a PUCCH is excluded from overlapping channels for multiplexing the UCI of the PUCCH and for intra-UE prioritization with the PUCCH.

Option 2 covers both Rel-17 intra-UE multiplexing and Rel-16 prioritization while option 1 covers Rel-17 intra-UE multiplexing only.

So Proponents of Option 1 please share how it works with Rel-16 prioritization.

Question 2.3.3: How option 1 works with Rel-16 prioritization?

| Company | Comments |
| --- | --- |
| LG | I guess FL intended to discuss on the following two options here (I copied below). If the simultaneous PUCCH/PUSCH transmission is enabled, the handling of overlapping PUCCH(s) and PUSCH(s) of different PHY priorities at Step 2.2 is done as follows: - Option 1: If a PUCCH (that could be multiplexed into PUSCH) does not overlap with a PUSCH on the same band, consider simultaneous transmission of this PUCCH and overlapping PUSCH(s) on a different band; otherwise, i.e. if a PUCCH overlaps with a PUSCH on the same band, consider the PUSCHs on all CCs and bands as candidates for multiplexing the UCI(s) carried in this PUCCH(s) and apply the corresponding Rel-17 (and Rel-15) multiplexing rules. [3] - Option 2: A PUSCH that can be simultaneously transmitted with a PUCCH is excluded for multiplexing the UCI of the PUCCH. We think Option 2 is straightforward for Rel-16 prioritization (by replacing “excluded for multiplexing the UCI of the PUCCH” into “excluded for prioritization” in above) while Option 1 is reasonable for Rel-17 multiplexing. |
| Moderator | The two options are added above. |
| Huawei/Hisi | We agree with Option 2, but we feel it is not clear how to define ‘a PUSCH that can be simultaneously transmitted with a PUCCH’: whether it can include all non-overlapping PUSCHs with PUCCH, or only include the overlapping PUSCHs but in a different band? To us, the UE behaviour is simple: it performs the intra-UE multiplexing/prioritization only within per band/band combination, instead of checking if there are overlapping PUSCHs in another band/band combination and exclude that PUSCH. So we suggest the following changes: Option 2: If the simultaneous PUCCH/PUSCH transmission is enabled for a group of bands/band combinations, the intra-UE multiplexing/prioritization should be performed per band/banc combination a PUSCH that can be simultaneously transmitted with a PUCCH is excluded from overlapping channels for multiplexing the UCI of the PUCCH and for intra-UE prioritization with the PUCCH. |

Between Option 1 and 2, companies’ views are as follows. Please comment if you change your mind or you have not provided your preference.

|  | Company |
| --- | --- |
| Option 12 | QC, Intel, ZTE, Lenovo, Apple, Samsung, CATT, Ericsson, DOCOMO Huawei/Hisi, Sony,New H3C, Sharp |
| Option 12 | Nokia/NSB, LG |

| Company | Comments |
| --- | --- |
| Apple | Not sure what options we are talking about here. Maybe FL can copy the proposals to 3.2.3.3 for easier discussions? |
| LG | It seems Option 1 and Option 2 are changed each other in the above table. |
| Moderator | Thanks LG for spotting this. Updated above. |

The following proposal was updated based on Qualcomm’s comments in 4th round.

Proposed Conclusion 2.3.2a:

If the simultaneous PUCCH/PUSCH transmission is enabled, the timeline conditions of intra-UE multiplexing/prioritization of PUCCHs and PUSCHs with different priorities is not applicable to a PUSCH that is simultaneously transmitted with a PUCCH.

|  | Company |
| --- | --- |
| Support | Sony, LG (not limited to this case), New H3C, Sharp, DOCOMO Huawei/Hisi, Intel, Samsung, Ericsson, ZTE |
| Not support |  |

| Company | Comments |
| --- | --- |
| LG | As commented earlier, we think the case that timeline condition is not applicable is not limited to the above PUSCH. In general, the timeline condition is not applicable to a PUSCH without UCI multiplexing, regardless whether the PUSCH is simultaneously transmitted with a PUCCH or not. |
| Intel | @LG, does “a PUSCH without UCI multiplexing” include the PUSCH which is one of the candidate PUSCHs for UCI multiplexing but is not finally selected as the PUSCH for multiplexing? For example, if a HP UCI overlaps with two LP PUSCHs in different carriers in same band, UE only choses one LP PUSCH to multiplex HP UCI, but for another LP PUSCH, in our view, the timeline should also be met, which is same as Rel-15. |
| Samsung | To LG, the understanding of “the timeline condition is not applicable to a PUSCH without UCI multiplexing” is NOT correct, FL clarified this issue in previous meeting when discussing the understanding of Rel-15 timeline for UCI multiplexing. |

Maximum number of supported CCs for simultaneous PUCCH and PUSCH transmission

      - 1st/2nd round discussion

DOCOMO [10] raised the issue that the maximum number of supported CCs for simultaneous PUCCH and PUSCH transmission per UE needs to be discussed.

Another aspect to be considered is that the maximum number of supported CCs for simultaneous PUCCH and PUSCH transmission per UE. As UE is able to support simultaneous PUCCH and PUSCH transmission on different PUCCH groups, the number of supported CCs should be defined based on UE capabilities related to PUCCH groups and the new capability. For example, if the new capability is reported per FS with X CCs and two PUCCH groups with different numerology are supported at the same time, the total number of supported CCs that UE can simultaneously transmit PUCCH and PUSCH across CCs can be maximum number of either reported value, i.e. max(X, 2). Note that the number should depend on how the number of supported CCs is reported by the new capability. Proposal 18: Discuss the interaction between capabilities for two PUCCH groups and the new capability for simultaneous PUCCH/PUSCH transmission on different carriers.

Moderator would like to propose to discuss the issue in UE feature session. Please provide your comments if any.

| Company | Comments |
| --- | --- |
| Nokia/NSB | Agree with FL / moderator |
| Intel | We share same view with FL. |
| ZTE | Agree |
| DOCOMO | Fine with the FL proposal. |
| OPPO | Fine with the FL proposal. |
| LG | We also agree with FL. |

- Proposals for GTW sessions

- Reference

- R1-2112761		List of agreements of Rel-17 URLLC/IIoT WI (post RAN1#107-e)	Rapporteur (Nokia)

- R1-2200012		Intra-UE multiplexing and prioritization	New H3C Technologies Co., Ltd.

- R1-2200018		On UL intra-UE prioritization and multiplexing enhancements	Nokia, Nokia Shanghai Bell

- R1-2200039		Intra-UE multiplexing enhancements	Huawei, HiSilicon

- R1-2200082		Remaining issues on intra-UE Multiplexing/Prioritization for Rel-17 URLLC	vivo

- R1-2200109		Discussion on enhanced intra-UE multiplexing	ZTE

- R1-2200148		Remaining issues on intra-UE multiplexing and prioritization	CATT

- R1-2200180		Remaining issues in intra-UE multiplexing & prioritisation	Sony

- R1-2200199		Remaining issues for Intra-UE Multiplexing/Prioritization	Samsung

- R1-2200233		Discussion on intra-UE multiplexing/prioritization for Rel.17 URLLC	NTT DOCOMO, INC.

- R1-2200275		Discussion on intra-UE multiplexing/prioritization	Spreadtrum Communications

- R1-2200296		Intra-UE multiplexing and prioritization for IOT and URLLC	Qualcomm Incorporated

- R1-2200320		Discussion on intra-UE multiplexing with different priorities	Panasonic Corporation

- R1-2200344		Enhancements on intra-UE multiplexing/prioritization	OPPO

- R1-2200358		Intra-UE Multiplexing/Prioritization	ETRI

- R1-2200365		Intra-UE multiplexing and prioritization	InterDigital, Inc.

- R1-2200374		Remaining Open Details of Intra-UE Uplink Channel Multiplexing and Prioritization	Intel Corporation

- R1-2200416		Rel-17 intra-UE Multiplexing/Prioritization	Apple

- R1-2200442		Intra-UE Multiplexing/Prioritization Enhancements for IIoT/URLLC	Ericsson

- R1-2200485		Discussion on some remaining issues for intra-UE multiplexing and prioritization	China Telecom

- R1-2200492		Remaining Issues on Intra-UE Multiplexing/Prioritization	Quectel, Langbo

- R1-2200497		Intra-UE UCI multiplexing and channel collision resolution with different priorities	Sharp

- R1-2200517		Discussion on Intra-UE prioritization and multiplexing	NEC

- R1-2200531		Intra-UE multiplexing enhancement for IIoT/URLLC	Lenovo, Motorola Mobility

- R1-2200562		Discussion on intra-UE multiplexing and prioritization	ITRI

- R1-2200573		Discussion on Intra-UE multiplexing/prioritization	LG Electronics

- R1-2112712		Summary #2 of email thread [107-e-NR-R17-IIoT-URLLC-03]	Moderator (CATT)

- Appendix: Agreements in RAN1#107bis-e

Conclusion

For resolving collision of PUCCHs and/or PUSCHs with different priorities in step 2, a resultant PUCCH with HP and LP UCI is not expected to be overlapped with a HP PUCCH.

- FFS whether a resultant PUCCH with HP and LP UCI can be overlapped with a HP PUSCH.

Agreement

For resolving collision of LP PUCCHs and HP PUCCHs in step 2.1, a HP PUCCH with HARQ-ACK is not expected to be overlapped with multiple LP PUCCHs with HARQ-ACK.

- It’s up to the editor whether/how to capture this

Agreement

For resolving collision of PUCCHs and PUSCHs of different priorities in step 2.2, LP PUSCH(s) overlapping with HP PUCCH which carries positive SR are dropped.

Working Assumption

For resolving collision of PUCCHs and PUSCHs of different priorities in step 2.2, LP PUSCH(s) overlapping with HP PUCCH which carries positive SR are dropped before UCI multiplexing.

- Step 1.2 behavior is not affected by the above

Agreement

simultaneousPUCCH-PUSCH-secondaryPUCCHgroup is supported to enable simultaneous PUCCH and PUSCH transmissions with different priorities within the secondary PUCCH cell group separately from primary PUCCH cell group.

Working Assumption (to be confirmed in RAN1#107bis-e)

For resolving collision of PUCCHs with different priorities in step 2.1, if resultant PUCCH with HP and LP UCI collides with LP PUCCH without HARQ ACK, the LP PUCCH is dropped.

- A resultant PUCCH with HP and LP UCI is not expected to be overlapped with a LP PUCCH with HARQ-ACK.

Agreement

A UE does not expect to multiplex in a PUSCH transmission HARQ-ACK information that the UE would transmit in different PUCCHs of a same priority.

- The above is considered an error case

Agreement

If the simultaneous PUCCH/PUSCH transmission is enabled, a PUSCH that can be simultaneously transmitted with a PUCCH is excluded from overlapping channels for multiplexing the UCI of the PUCCH and for intra-UE prioritization with the PUCCH.

- Note: For intra-UE multiplexing, above is for step 2-2. For intra-UE prioritization, above is applied after step 1.

- FFS: How to capture this in the specifications

Conclusion

If the simultaneous PUCCH/PUSCH transmission is enabled, the timeline conditions of intra-UE multiplexing/prioritization of PUCCHs and PUSCHs with different priorities is not applicable to a PUSCH that can be simultaneously transmitted with a PUCCH.