3GPP TSG-RAN WG1 Meeting #107bis-e	R1-2200778

e-Meeting, January 17th – 25th, 2022

Agenda item:		8.3

Source:		Rapporteur (Nokia)

Title:	List of agreements of Rel-17 URLLC/IIoT WI (post RAN1#107bis-e)

Document for:		Information

# 1	Introduction

This document contains the list of agreements available in the Rel-17 URLLC/IIoT WI achieved (incl. the agreements from RAN1#107bis-e)

The document is structured per WI AI, and for some AIs the agreements are still separated per topic / feature.

# HARQ-ACK enhancements (AI 8.3.1.1)

  - Generic agreements

RAN1#102-e (Aug. 2020)

Agreements: - Simultaneous PUSCH / PUCCH within a cell group (of Sec. 6.13 of R1-2007216) and enhanced (sub-slot) HARQ-ACK multiplexing on PUSCH (of Sec. 4.3 of R1-2007216) can be further discussed as part of AI 8.3.3 in this WI (but not as part of AI 8.3.1.1). Agreements: Study further at least the following schemes: - SPS HARQ skipping for ‘skipped’ SPS PDSCH - PUCCH repetition enhancements (at least for HARQ-ACK), e.g., sub-slot based, etc. - Retransmission of cancelled HARQ - SPS HARQ payload size reduction and / or skipping for ‘non-skipped’SPS PDSCH - Type 1 HARQ codebook based on sub-slot PUCCH config - PUCCH carrier switching for HARQ feedback

RAN1#103-e (Oct./Nov. 2021)

Agreements: For the studies on SPS HARQ skipping for skipped SPS PDSCH, the further discussions should focus on the following reduced sets methods: - ‘NACK skipping’ for (skipped) SPS PDSCH (Alt. 1) - FFS: details including at least when to skip the HARQ-ACK as well as NACK skipping configuration details (per SPS or group of SPS configurations etc.) - Note: this alternative assumes inherently no identification of a skipped SPS PDSCH by the UE - Dynamic indication of skipped SPS PDSCH occasions (Alt. 3) - FFS: details including dynamic indication methods such as e.g. DCI, MAC CE, specific DM-RS instead of SPS DM-RS, … Agreements: For the studies on SPS HARQ payload size reduction (of non-skipped SPS PDSCH), the further discussions should focus on the following reduced sets of methods: - ACK skipping (NACK-only) (Alt. 1) - FFS: Details - NACK skipping (ACK-only) (Alt. 2) - FFS: Details - HARQ bundling / compression (Alt. 3) - FFS: Details including HARQ bundling / compression window, bundling / compression technique - HARQ-ACK disabling /skipping for certain SPS configurations (Alt. 4) - The skipping / disabling is higher-layer configured per SPS configuration - FFS: HARQ-ACK skipping behaviour for Type 1 CB

RAN1#104bis-e (April 2021)

Conclusion: No support for dynamic indication of skipped SPS PDSCH occasions in Rel-17 as part of this WI.

RAN#92-e (June 2021)

Revised Recommendation2: Provide the following RAN guidance on HARQ-ACK enhancement [RAN1] - No further discussions on SPS HARQ-ACK skipping and size reductionbundling/compression.

  - SPS HARQ-ACK feedback in TDD operation / SPS deferral

RAN1#102-e (Aug. 2020)

Agreements: Support Rel-17 enhancements to avoid SPS HARQ-ACK dropping for TDD due to PUCCH collision with at least one DL or flexible symbol. - This topic is to be considered as high priority - FFS detailed solution(s)

RAN1#103-e (Oct./Nov. 2021)

Agreements: To address the issue of SPS HARQ-ACK dropping for TDD systems, focus on the following two options: - Option 1: Deferring HARQ-ACK until a next (e.g., first) available PUCCH - FFS: Details including the definition of a next (e.g, first) available PUCCH, CB construction / multiplexing - Option 2: Dynamic triggering of a one-shot / Type-3 CB type of re-transmission FFS: Details on triggering and/or CB construction (incl. potential Type-3 CB optimizations) / multiplexing

RAN1#104-e (Jan. 2021)

Agreements: - Support deferring SPS HARQ-ACK dropped due to TDD specific collisions until a next available PUCCH in Rel-17 based on semi-static configuration of slot format - FFS: Details (including possible conditions for such a deferring, whether or not to consider semi-statically configured flexible symbols for PUCCH availability, etc.) - Aim for minimal standardization efforts and UE complexity in implementation Agreements: Further down-select between the following two options for SPS HARQ-ACK deferral: - Option 1: Joint RRC configuration of the SPS HARQ-ACK deferral per PUCCH cell group - Note: any SPS HARQ-ACK within a PUCCH cell group in principle is subject to deferral - Option 2: The SPS HARQ-ACK deferral is configured per SPS configuration - Note: part of sps-config, only HARQ-ACK of SPS PDSCH configurations configured for deferral is in principle subject to deferral Agreements: Rel-16 UCI multiplexing / PUCCH overriding rules are reused for deferred SPS HARQ-ACK in the target slot, if applicable. Agreements: For SPS HARQ-ACK, the deferral from the initial slot/sub-slot determined by k1 in the activation DCI to the target slot/sub-slot determined by k1+ k1def, the UE will check the validity of a target slot/sub-slot evaluating from one slot/sub-slot to the next sub/sub-slot (i.e. in principle k1def granularity is 1 slot/sub-slot) - FFS: if there is a limit on the minimum deferral considered the required UE processing (k1def ≥0) - FFS: if there is a limit on the maximum deferral Agreements: For SPS HARQ-ACK deferral, for the determination of valid symbols in the initial slot/sub-slot a collision with semi-static DL symbols, SSB and CORESET#0 is regarded as ‘invalid’ or ‘no symbols for UL transmission’.

RAN1#104bis-e (April 2021)

Agreements: For SPS HARQ-ACK deferral, for the determination of valid symbols in the target slot/sub-slot a collision with semi-static DL symbols, SSB and CORESET#0 is regarded as ‘invalid’ or ‘no symbols for UL transmission’. Agreements: For SPS HARQ-ACK deferral, support a limit on the maximum deferral of SPS HARQ in terms of k1def or k1+ k1def - FFS: limitation given by a maximum value of k1def or a maximum of k1eff =k1+ k1def - FFS how the limitation is determined (e.g. by K1 set(s) or RRC configured limit) Agreements: For SPS HARQ-ACK deferral, there is no lower limit defined for k1def Agreement: Restrict the further discussions on the initial slot handling for SPS HARQ-ACK deferral to the identified alternatives Alt. 1, Alt. 1A and 2. Agreement: For SPS HARQ-ACK deferral, the limit on the maximum deferral of SPS HARQ is defined in terms of k1eff =k1+ k1def. Working assumption: To handle the collision for the same HARQ process due to deferred SPS HARQ-ACK the following behaviour is to be specified: - In case the UE receives PDSCH of a certain HARQ Process ID, the deferred SPS HARQ bit(s) for this HARQ Process ID are dropped. Agreement: For SPS HARQ-ACK deferral, the initial HARQ-ACK transmission occasion is considered to determine the out-of-order HARQ condition

RAN1#106-e (Aug. 2021)

Agreement The SPS HARQ-ACK deferral is enabled per SPS configuration - Note: part of sps-config, only HARQ-ACK of SPS PDSCH configurations enabled for deferral is in principle subject to deferral Agreement Definition of when to defer from the initial slot: - Alt1: Deferral only, if the SPS HARQ-ACK in the initial slot/sub-slot cannot be transmitted as the resulting PUCCH resource for transmission using the PUCCH by SPS-PUCCH-AN-List-r16 or n1PUCCH-AN is not valid Agreement For SPS HARQ-ACK deferral, the maximum deferral value in terms of k1+k1def is RRC configured per SPS configuration. Agreement For SPS HARQ-ACK deferral, only SPS HARQ bits subject to deferral from HARQ-ACK codebook from an initial PUCCH slot are deferred to the target PUCCH slot Agreement For SPS HARQ-ACK deferral, deferred SPS HARQ bits from more than one ‘initial PUCCH slot’ can be jointly deferred to a target PUCCH slot Agreement For SPS HARQ-ACK deferral, the target PUCCH slot is defined as the next PUCCH slot where sps-PUCCH-AN-List-r16 or n1PUCCH-AN PUCCH resource is regarded as valid, or a PUCCH resource (from PUCCH-ResourceSet, i.e. DG PDSCH HARQ multiplexed) is dynamically indicated - The target PUCCH slot determination is based on the total HARQ-ACK payload size including deferred SPS HARQ-ACK information and non-deferred HARQ-ACK information (if any) of a candidate target PUCCH slot - The final PUCCH resource selection in the target PUCCH slot in terms of PUCCH resource set and PUCCH resource ID follows the Rel-16 procedures. Agreement For SPS HARQ-ACK deferral, if after the target PUCCH slot determination the deferred SPS HARQ-ACK cannot be transmitted, the deferred SPS HARQ-ACK bits are not further deferred and are dropped. Agreement For SPS HARQ-ACK deferral, in the target PUCCH slot the deferred SPS HARQ-ACK bits are appended to the initial HARQ bits / Type 1 or Type 2 codebook. Agreement For SPS HARQ-ACK deferral, confirm the RAN1#104b-e working assumption with the following updates in RED: (working assumption) To handle the collision for the same HARQ process due to deferred SPS HARQ-ACK the following behaviour is to be specified: - In case the UE is expected to receives PDSCH of a certain HARQ Process ID according to TS 38.214 Sec. 5.1, the deferred SPS HARQ bit(s) for this HARQ Process ID are dropped. - Note: there is no further discussion on specific handling for the case of DG PDSCH with the same HARQ process ID

RAN1#106bis-e (Oct. 2021)

Conclusion For SPS HARQ-ACK deferral, only SPS HARQ-ACK bits subject to deferral from one or more initial slots which have not reached the maximum deferral value are jointly deferred to the next available PUCCH (other SPS HARQ-ACK is dropped). Agreement For SPS HARQ-ACK deferral, the bit ordering of deferred SPS HARQ-ACK information from one or more initial slots in the target PUCCH slot is based on the Rel.16 SPS HARQ-ACK bit order principle as in clause 9.1.2 of TS38.213 is applied, i.e., based on serving cell index, SPS configuration index, SPS PDSCH slot index. Conclusion For SPS HARQ-ACK deferral, the operation in the ‘initial’ slot is further clarified as: - The UE performs first the (Rel-16) UCI multiplexing operation. If after the UCI multiplexing operation into a PUCCH or PUSCH if any, and if the UE would be transmitting SPS HARQ-ACK using the PUCCH SPS-PUCCH-AN-List-r16 or n1PUCCH-AN which is not valid, the SPS HARQ-ACK configured for deferral is deferred. Agreement The RAN1#106-e agreement on the target slot definition is updated as follows (in RED): Agreement (from RAN1#106-e) For SPS HARQ-ACK deferral, the target PUCCH slot is defined as the next PUCCH slot, where after performing the (Rel-16) UCI multiplexing operation into a PUCCH or PUSCH if any, the UE would be either (i) transmitting HARQ-ACK using a PUCCH/PUSCH other than the PUCCH determined from PUCCH SPS-PUCCH-AN-List-r16 or n1PUCCH-AN or (ii) would be transmitting HARQ-ACK using a PUCCH resource configured in PUCCH SPS-PUCCH-AN-List-r16 or n1PUCCH-AN being regarded as valid. sps-PUCCH-AN-List-r16 or n1PUCCH-AN PUCCH resource is regarded as valid, or a PUCCH resource (from PUCCH-ResourceSet, i.e. DG PDSCH HARQ multiplexed) is dynamically indicated - The target PUCCH slot determination is based on the total HARQ-ACK payload size including deferred SPS HARQ-ACK information and non-deferred HARQ-ACK information (if any) of a candidate target PUCCH slot - The final PUCCH resource selection in the target PUCCH slot in terms of PUCCH resource set and PUCCH resource ID follows the Rel-16 procedures.. Conclusion If the UE is not configured with Rel-17 Intra-UE multiplexing, SPS HARQ for deferral of different PHY priorities can be separately deferred with the target PUCCHs separately determinated according to their respective PHY priorities. - FFS on the PHY priority handling for SPS HARQ deferral if the UE configured with Rel-17 Intra-UE multiplexing

RAN1#107-e (Nov. 2021)

Agreement The maximum SPS HARQ-ACK deferral value in terms of k1+k1def per SPS configuration is RRC configured from a value range of {1…32}. Conclusion For SPS HARQ-ACK deferral, if a UE is not configured with Rel-17 intra-UE multiplexing but configured with Rel-16 PHY prioritization, the UE first performs Rel-16 UCI multiplexing and PHY prioritization in both initial slot and target slot and if a LP SPS HARQ-ACK PUCCH is deprioritized, the LP SPS HARQ-ACK is not deferred. - Note: If the SPS HARQ-ACK is deprioritized in any slot, no further deferral. Agreement Support simultaneous configuration of SPS HARQ-ACK deferral and PUCCH cell switching based on the semi-static time domain pattern: For the target slot determination of SPS HARQ-ACK deferral, - Step 1: the UE first determines a next PUCCH slot on the cell for PUCCH transmission using the semi-static time-domain PUCCH cell pattern and the related rules for semi-static PUCCH cell switching, followed by - Step 2: the UE determines based on the SPS HARQ-ACK deferral rules if this PUCCH slot on the PUCCH cell for transmission is the target PUCCH slot or not. - Note: In step 1, k is increased on PCell/PScell/PUCCH-Scell. “The next PUCCH slot” represents the slot on the PUCCH cell based on PUCCH cell pattern, which is mapped from the PCell/PScell/PUCCH-Scell slot with increased K1. - Note: The maximum deferral limitation checking is based on the effective k + kdef value based on the granularity of PCell / PScell/PUSCCH-Scell Agreement The earlier RAN1 agreements on the valid symbol definition in the initial and target PUCCH slot for SPS HARQ-ACK deferral are further clarified as: - For SPS HARQ-ACK deferral, for the determination of valid symbols in the initial and target PUCCH slot/sub-slot a collision with semi-static DL symbols, SSB and symbols indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set is regarded as ‘invalid’ or ‘no symbols for UL transmission’. Agreement Support simultaneous configuration of Rel-16 Type 3 HARQ-ACK codebook or enhanced Type 3 HARQ-ACK codebook triggering and SPS HARQ-ACK deferral. - In case a R16 Type 3 HARQ-ACK CB or an enhanced Type 3 HARQ-ACK codebook is triggered for transmission in a PUCCH slot, the UE stops the deferral procedure of pending SPS HARQ-ACK in that PUCCH slot and that PUCCH slot is not considered as a potential target slot for SPS HARQ-ACK deferral anymore.

RAN1#107bis-e (Jan. 2022)

Conclusion There is no consensus to support SPS HARQ-ACK deferral for half-duplex CA UEs in Rel-17. Conclusion There is no consensus to support joint operation of SPS HARQ-ACK deferral and PUCCH repetition in Rel-17. Conclusion The operation of simultaneous configuration of Rel-16 Type 3 HARQ-ACK codebook or enhanced Type 3 HARQ-ACK codebook triggering and SPS HARQ-ACK deferral is further clarified as: - If the UE detects a DCI format in a PDCCH reception that triggers a PUCCH transmission with a Type-3 or enhanced Type-3 HARQ-ACK codebook in a slot, the UE stops the procedure to determine the earliest second slot in that slot. - The pending SPS HARQ information for deferral is not appended to the Type-3 or enhanced Type 3 CB in that slot. Conclusion There is no consensus to support joint configuration of PUCCH cell switching based on dynamic indication and SPS HARQ-ACK deferral in Rel-17. Agreement The following TP to 38.213 is endorsed for the editor’s CR. 9.2.5.4 UE procedure for deferring HARQ-ACK for SPS PDSCH If a UE is provided spsHARQdeferral and, after performing the procedures in clauses 9 and 9.2.5 to resolve overlapping among PUCCHs and PUSCHs in a first slot, the UE determines a PUCCH resource for a PUCCH transmission with first HARQ-ACK information bits for SPS PDSCH receptions that the UE would report for a first time, and the PUCCH resource - is provided by SPS-PUCCH-AN-List as described in clause 9.2.1, or by n1PUCCH-AN if SPS-PUCCH-AN-List is not provided - overlaps with a symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated, or indicated for a SS/PBCH block by ssb-PositionsInBurst, or belonging to a CORESET associated with a Type0-PDCCH CSS set the UE - determines an earliest second slot and, after performing the procedures in clauses 9 and 9.2.5 to resolve overlapping among PUCCHs and PUSCHs, a PUSCH or a PUCCH in the earliest second slot to multiplex HARQ-ACK information bits that include second HARQ-ACK information bits from the first HARQ-ACK information bits - if the UE detects a DCI format in a PDCCH reception that triggers a PUCCH transmission with a Type-3 HARQ-ACK codebook in a slot as described in clause 9.1.4, the UE stops the procedure to determine the earliest second slot in the slot - if the UE is provided a periodic cell switching pattern for PUCCH transmissions by pucch-sSCellPattern, the UE determines the earliest second slot and a corresponding cell based on the periodic cell switching pattern as described in clause 9.A. Agreement Support the simultaneous configuration of one-shot triggering of HARQ re-transmission and SPS deferral - One-shot HARQ-ACK re-transmission can trigger re-transmission SPS HARQ-ACK enabled with deferring from the initial SPS HARQ deferral slot. - If the PUCCH slot indicated by the HARQ_retx_offset is the ‘target’ or earliest ‘second’ slot for SPS HARQ-ACK deferral, the HARQ-ACK CB including the deferred SPS HARQ-ACK bits will be retransmitted in the new retransmission PUCCH triggered by one-shot triggering DCI. - For the SPS HARQ-ACK deferral procedure, the PUCCH slot with a one-shot triggered HARQ-ACK CB is regarded as a valid potential target PUCCH slot for SPS HARQ-ACK deferral with same PHY priority (at least for operation with Rel-16 PHY prioritization) as the PHY priority of the triggered one-shot HARQ-ACK re-transmission. - If the PUCCH slot with a one-shot triggered HARQ-ACK CB is determined by the UE as target or earliest second PUCCH slot for SPS HARQ-ACK deferral, the deferred SPS HARQ-ACK in a target slot is appended to the re-transmitted HARQ-ACK CB and initial, new HARQ-ACK (if any) following the operation of SPS HARQ-ACK deferral procedure.

## 2.3 PUCCH repetition enhancements

RAN#89 (Dec. 2020)

RAN conclusion on IIoT scope: - For handling of the PUCCH repetitions it is proposed to proceed as follows: - RAN1 to continue discussion on PUCCH repetition, whether to specify or not, in the IIoT/URLLC WI for single TRP. - The following items are not within scope of the continued discussions in the IIoT/URLLC WI: - DMRS-less PUCCH with UCI payload up to 11 bits - PUSCH-repetition-Type-B like PUCCH repetition - DMRS bundling across PUCCH repetitions - PUCCH repetition issues with multi-TRP to be handled in Fe-MIMO WI. - For the UE CSI/HARQ-ACK feedback enhancements in the IIoT/URLLC WI, RAN1 work to continue the discussions. Status to be checked in March if any RAN level guidance needed. - RAN1 to continue discussion on A-CSI on PUCCH, whether to specify or not.

RAN1#104-e (Jan. 2021)

Agreements: Support sub-slot based PUCCH repetition for HARQ-ACK based on the Rel-16 PUCCH procedure for slot-based PUCCH applied to sub-slot based PUCCH - Note: the intention is to take the Rel-16 slot-based PUCCH by replacing with “sub-slot” appropriately, without further optimization unless necessary - FFS whether or not there is any restriction for the applicability of sub-slot based PUCCH repetition for HARQ-ACK - Dynamic repetition indication is supported also for sub-slot based PUCCH in Rel-17 - FFS: if the method to be specified in Cov. Enh WI for slot-based PUCCH repetition can be directly applied to sub-slot PUCCH or if changes are needed Agreements: Support PUCCH repetition for PUCCH formats 0 and 2 at least for sub-slot based PUCCH repetition. - FFS: Support for slot-based PUCCH repetition

RAN1#106-e (Aug. 2021)

Conclusion The dynamic repetition indication solution for slot-based PUCCH repetition from the RAN1#105-e working assumption from Cov. Enh. WI can be directly applied for dynamic repetition indication for sub-slot based PUCCH repetition. Agreement For sub-slot based PUCCH repetition for HARQ-ACK, semi-static configured PUCCH repetition (i.e. using nrofSlots) and dynamic repetition factor based operation is supported. - Sub-slot based PUCCH repetition based on semi-static configuration (i.e. using nrofSlots) and based on dynamic indication is subject to separate UE capabilities Agreement Support slot-based PUCCH repetition for PUCCH Format 0 and Format 2 also for single TRP operation. - The support is subject to independent UE capability indication

RAN1#106bis-e (Oct. 2021)

Agreement To align with Rel-16 slot-based PUCCH repetition operation, support sub-slot based PUCCH repetition configured with / using nrofSlots (i.e., not using dynamic indication) of all UCI types (incl. HARQ, SR & CSI). Agreement For sub-slot based PUCCH repetition, the following agreement from Cov. Enh. WI for slot-based PUCCH repetition is adopted also for sub-slot based PUCCH repetition: Agreement - for a PUCCH resource, if both a new repetition parameter corresponding to Rel-17 dynamic PUCCH repetition factor indication and the Rel-15/16 nrofSlots are configured, the new repetition parameter overrides nrofSlots.. Agreement For sub-slot based PUCCH repetition, the following agreement from Cov. Enh. WI for slot-based PUCCH repetition is adopted also for sub-slot based PUCCH repetition: Agreement: Dynamic PUCCH repetition factor indication for SR or P/SP-CSI on PUCCH is not supported in Rel-17.. Agreement For PUCCH repetition enhancements: - Support inter-slotFrequencyHopping for PUCCH repetition operation of PUCCH Format 0 and Format 2 for slot-based PUCCH configurations. - Support inter-subslot Frequency Hopping for PUCCH repetition operation of PUCCH Formats 0, 1, 2, 3 and 4 for 7OS slot-based PUCCH configurations. - The UE applies the inter-subslot FH operation from sub-slot to sub-slot, if configured with inter-slotFrequencyHopping in the respective PUCCH_config. - (Working Assumption) Support inter-subslot Frequency Hopping for PUCCH repetition operation of PUCCH Format 0 and Format 2 for 2OS slot-based PUCCH configurations. - The UE applies the inter-subslot FH operation from sub-slot to sub-slot, if configured with inter-slotFrequencyHopping in the respective PUCCH_config. - Note: As for Rel-15, the configuration / enabling of inter-slotFrequencyHopping and intraSlotFrequencyHopping is not supported. Agreement For sub-slot based PUCCH repetition, the following agreement from Cov. Enh. WI for slot-based PUCCH repetition is adopted also for sub-slot based PUCCH repetition: Agreement - In Rel-17, reuse the Rel-16 PUCCH repetition factors 2, 4, 8. - Do not support PUCCH repetition factor larger than 8 In Rel-17..

RAN1#107-e (Nov. 2021)

Agreement Confirm the following RAN1 working assumption from RAN1#106bis-e with the additional agreement on UE capability (in RED): - (Working Assumption) Support inter-subslot Frequency Hopping for PUCCH repetition operation of PUCCH Format 0 and Format 2 for 2OS slot-based PUCCH configurations. - The UE applies the inter-subslot FH operation from sub-slot to sub-slot, if configured with inter-slotFrequencyHopping in the respective PUCCH_config.. - Support single UE capability indication of inter-subslot FH for PUCCH repetition operation.

## 2.4 Type-1 HARQ-ACK Codebook enhancements

RAN1#104bis-e (April 2021)

Agreement: Support Type-1 HARQ-ACK codebook for sub-slot based PUCCH configuration in Rel-17. - The properties of the Type-1 HARQ-ACK codebook for sub-slot PUCCH at least includes that a PDSCH TDRA is associated with a UL /PUCCH sub-slot if the end of the PDSCH overlaps with the associated sub-slot determined by a k1 in the set of sub-slot timing values K1. - FFS: whether the PDSCH TDRA grouping is performed per DL slot or sub-slot - Decide between PDSCH TDRA grouping per DL slot and sub-slot during RAN1#105-e

RAN1#106bis-e (Oct. 2021)

Agreement For Type-1 HARQ-ACK codebook for sub-slot based PUCCH configuration in Rel-17, the TDRA pruning/grouping is performed per DL slot after TDRA determination per sub-slot. - Strive to minimize the impact on relevant pseudo-code

## 2.5 Retransmissions of dropped HARQ-ACK

### 2.4.1 Generic agreements

RAN1#105-e (May 2021)

Working Assumption: For at least HARQ-ACK re-transmission: - Support at least one enhanced Type 3 HARQ-ACK CB with smaller size (compared to Rel-16) in Rel-17 - Definition of enhanced Type 3 CB: - The codebook size of a single triggered enhanced Type 3 HARQ-ACK codebook at least determined by RRC configuration - The codebook construction uses HARQ processes as a bases (i.e. ordered according to HARQ-IDs and serving cells) - Support one-shot triggering (by a DL assignment) of HARQ-ACK re-transmission on a PUCCH resource other than enhanced Type 2 or (enhanced) Type 3 HARQ-ACK CB (i.e. Alt. 3) in Rel-17 - Details are FFS - Enhanced Type 3 HARQ-ACK CB and/or one-shot triggering (by a DL assignment) of HARQ-ACK re-transmission on a PUCCH resource other than enhanced Type 2 or (enhanced) Type 3 HARQ-ACK CB are subject to separate UE capabilities

RAN1#106-e (Aug. 2021)

Agreement Confirm the following RAN1#105-e working assumption: For at least HARQ-ACK re-transmission: - Support at least one enhanced Type 3 HARQ-ACK CB with smaller size (compared to Rel-16) in Rel-17 - Definition of enhanced Type 3 CB: - The codebook size of a single triggered enhanced Type 3 HARQ-ACK codebook at least determined by RRC configuration - The codebook construction uses HARQ processes as a bases (i.e. ordered according to HARQ-IDs and serving cells) - Support one-shot triggering (by a DL assignment) of HARQ-ACK re-transmission on a PUCCH resource other than enhanced Type 2 or (enhanced) Type 3 HARQ-ACK CB (i.e. Alt. 3) in Rel-17 - Details are FFS - Enhanced Type 3 HARQ-ACK CB and/or one-shot triggering (by a DL assignment) of HARQ-ACK re-transmission on a PUCCH resource other than enhanced Type 2 or (enhanced) Type 3 HARQ-ACK CB are subject to separate UE capabilities

RAN1#107-e (Nov. 2021)

Conclusion There is no consensus to support the simultaneous configuration of the Rel-16 Type 3 HARQ-ACK CB and Rel-17 one-shot re-tx HARQ triggering for a UE in Rel-17. Conclusion There is no consensus to support the simultaneous configuration of the Rel-17 Enhanced Type 3 HARQ-ACK CB and Rel-17 one-shot HARQ re-tx triggering for a UE in Rel-17.

### 2.4.2 (Enhanced) Type-3 HARQ-ACK CB related

RAN1#106-e (Aug. 2021)

Agreement Support PHY priority handling for a PUCCH carrying the Rel-17 enhanced Type 3 HARQ-ACK CB of smaller size. - The indicated PHY priority in the triggering DCI defines the PHY priority of the PUCCH carrying the Rel-17 enhanced Type 3 HARQ-ACK CB of smaller size. - The A/N of HARQ processes is mapped to the Rel-17 enhanced Type 3 HARQ-ACK CB of smaller size irrespective of the PHY priority of the ‘A/N’ of the HARQ processes. - FFS: If the HARQ-ACK codebook size or structure is dependent on the PHY priority (e.g. separate configuration of CBG/NDI usage, separate configuration of HARQ IDs / CCs per priority, SPS HARQ-ACK process IDs of specific priority only for a SPS HARQ-ACK only codebook, …). Agreement Support PHY priority handling for a PUCCH carrying the Rel-16 Type 3 HARQ-ACK CB in Rel-17. - The indicated PHY priority in the triggering DCI defines the PHY priority of the PUCCH carrying the Rel-16 Type 3 HARQ-ACK CB. - The A/N of HARQ processes is mapped to the Rel-16 Type 3 HARQ-ACK CB irrespective of the PHY priority of the ‘A/N’ of the HARQ processes. - The support is subject to a Rel-17 UE capability and a UE supporting this capability can be configured in Rel-17 with Rel-16 Type 3 HARQ-ACK CB and PHY prioritization. Agreement For the PHY priority handling of the enhanced Type 3 CB(s) of smaller size, the enhanced Type 3 HARQ-ACK has the same structure, size and content (in terms of HARQ-IDs, CCs) irrespective of the PHY priority. Agreement Support Rel-17 enhanced Type 3 HARQ-ACK CB of smaller size triggering using DCI format 1_2 for a UE supporting DCI format 1_2. - The triggering support for DCI format 1_2 is independently (from triggering using DCI format 1_1) RRC configured to the UE. Agreement Support Rel-16 Type 3 HARQ-ACK CB triggering using DCI format 1_2 in Rel-17 for a UE supporting DCI format 1_2. - The support is subject to a Rel-17 UE capability and a UE supporting this capability can be configured with DCI format 1_2 triggering of the Rel-16 Type 3 HARQ-ACK CB. Agreement For the enhanced Type 3 HARQ-ACK CB of smaller size triggered in a PUCCH slot, the UE is not expecting HARQ-ACK information in a Type 1 or Type 2 HARQ-ACK CB to be transmitted that cannot be mapped to the enhanced Type 3 HARQ-ACK CB of smaller size as the HARQ process is not part of the codebook. Agreement For enh. Type 3 HARQ-ACK CB(s), support dynamic selection based on indication in the triggering DCI of one of at least one enh. Type 3 HARQ-ACK CB(s). - Each of the at least one enh. Type 3 HARQ-ACK CBs is at least defined by RRC configuration This includes the option to configure all DL HARQ processs of all configured CCs as one enh. Type 3 HARQ-ACK CB (resulting in same structure and size as the Rel-16 Type 3 HARQ-ACK CB) - This includes UE capability signaling (value range {1…X}) on the maximum number of supported simultaneously configured enh. Type 3 HARQ-ACK CBs that can be dynamically indicated - Details including the value of X are FFS Agreement The following enhanced Type 3 CB types of smaller size are supported, the CB to contain either: - the HARQ processes of a subset of configured CCs, or - a subset of configured HARQ processes (specific to CCs) FFS: additional enh. Type 3 CB types

RAN1#106bis-e (Oct. 2021)

Conclusion No additional enhanced Type 3 CB ‘types’ (such as activated CCs, of specific SPS configurations, etc.) in terms of RRC configuration are supported. Agreement For one enhanced Type 3 HARQ-ACK CB, the same CBG and NDI configuration applies to both PHY priorities following the RAN1#106-e agreement. Agreement The same set of enhanced Type 3 CBs (incl. CBG and NDI configuration) is applied for triggering using DCI format 1_1 and 1_2. Agreement Reuse the legacy 1-bit ‘one-shot HARQ-ACK request’ for triggering indication of the enhanced Type 3 HARQ-ACK CB of smaller size. - At least if only a single enhanced Type 3 HARQ-ACK CB is configured, the triggering DCI with the triggering bit set to ‘1’ is also able to schedule PDSCH. Agreement The CBG and NDI usage can be independently configured for different enhanced Type 3 HARQ-ACK CBs. Agreement The maximum number of simultaneously configurable enhanced Type 3 CB is indicated by the UE through UE capability signaling from the set of {1, 2, 4, 8}.

RAN1#107-e (Nov. 2021)

Agreement The list enhanced Type 3 HARQ-ACK codebooks is configured per PUCCH cell group (i.e., separately configurable for primary and secondary PUCCH cell group). Agreement Support simultaneous configuration of enhanced Type 3 CB triggering and PUCCH cell switching. Agreement One enhanced Type 3 HARQ-ACK codebook is RRC configured either as: - a subset of CC, i.e., all HARQ processes of the subset of CCs are part of the codebook, OR pdsch-HARQ-ACK-enhType3perCC; Configure the one enhanced Type 3 HARQ-ACK codebook using per CC configuration; (1..maxNrofServingCells) of Integer (0,1). - a subset of configured HARQ processes per CC, i.e., different subsets of HARQ processes can be configured for each CC. pdsch-HARQ-ACK-enhType3perHARQ; Configure the one enhanced Type 3 HARQ-ACK codebook using a per HARQ process and CC configuration; (1..maxNrofServingCells) of Bit String (Size (16)). Agreement If more than one (M>1) enhanced Type 3 HARQ-ACK codebook is configured and the triggering DCI with the ‘one-shot HARQ-ACK request’ set to ‘1’, - If the FDRA field is not valid, i.e. all “1s” or all “0s” as per Rel-16, then PDSCH is not scheduled: - If a new field with N=ceiling(log2 (M)) bits is configured in the triggering DCI, the UE uses this new field to indicate one of M configured e-Type 3 HARQ-ACK CBs - If the new field is not configured in the triggering DCI, the UE uses the MCS field to indicate one of M configured e-Type 3 HARQ-ACK CBs - If the FDRA field is valid, then a PDSCH is scheduled - If a new field with N=ceiling(log2 (M)) bits is configured in the triggering DCI, the UE uses this new field to indicate one of M configured e-Type 3 HARQ-ACK CBs - If the new field is not configured in the triggering DCI, the UE selects the 1st indexed e-Type 3 HARQ-ACK CB in the M configured e-Type 3 HARQ-ACK CBs

RAN1#107bis-e (Jan. 2022)

Agreement Re-add the RRC parameter for the DCI field configuration in row 17 of the Enh. Type-3 HARQ-ACK codebook for the primary PUCCH cell group (that was lost when moving from v006 to v007 in the final RRC parameter discussions in RAN1#107-e, currently we only have the configuration for the secondary PUCCH cell group) i.e., pdsch-HARQ-ACK-enhType3DCIfield; Enables the enhanced Type 3 CB through a new DCI field to indicate the enhanced Type 3 HARQ-ACK codebook in the primary cell group if the more than one enhanced Type 3 HARQ-ACK codebook is configured for the primary PUCCH cell group.; Enabled. Agreement Support separate configuration of the DCI field presence for enh. Type 3 HARQ-ACK CB for DCI format 1_2 (i.e. pdsch-HARQ-ACK-enhType3DCIfieldDCI-1-2 as discussed in RAN1#107-e)

### 2.4.3 One-shot triggering (by a DL assignment) of HARQ-ACK re-transmission on a PUCCH resource

RAN1#106-e (Aug. 2021)

Agreement A single DCI triggering the Rel-17 one-shot triggering (by a DL assignment) of HARQ-ACK re-transmission on a PUCCH resource other than enhanced Type 2 or (enhanced) Type 3 HARQ-ACK CB can trigger the re-transmission of HARQ-ACK information of only a single HARQ-ACK CB. Agreement The Rel-17 one-shot triggering (by a DL assignment) of HARQ-ACK re-transmission on a PUCCH resource other than enhanced Type 2 or (enhanced) Type 3 HARQ-ACK CB is done through an explicit triggering indication in the DCI through a DCI field. Agreement Support PHY priority handling for the Rel-17 one-shot triggering (by a DL assignment) of HARQ-ACK re-transmission on a PUCCH resource other than enhanced Type 2 or (enhanced) Type 3 HARQ-ACK CB. - The indicated PHY priority in the triggering DCI defines the PHY priority of the PUCCH carrying the re-transmitted HARQ-ACK information. - The indicated PHY priority in the triggering DCI is used to determine the HARQ-ACK information to be re-transmitted corresponding to the indicated PHY priority. Agreement For Rel-17 one-shot triggering for HARQ-ACK re-transmission, the UE does not expect more than one triggering DCI for Rel-17 one-shot feedback indicating the same PUCCH slot for the re-transmission of HARQ-ACK CBs of different PUCCH slots to be re-transmitted - Note: i.e. only a single HARQ-ACK codebook / PUCCH occasion can be re-transmitted in a PUCCH slot

RAN1#106bis-e (Oct. 2021)

Agreement Support triggering of one-shot HARQ re-transmission on PUCCH using DCI format 1_2. Agreement For one-shot HARQ re-transmission on PUCCH, the triggering DCI dynamically indicates a ‘HARQ re-tx offset’ which is used to define the offset in number of PUCCH slots/sub-slots between the triggering DCI and the PUCCH slot/sub-slot of the HARQ-ACK codebook to be re-transmitted. For the triggering DCI received in slot/sub-slot m, indicating the HARQ-ACK re-tx in slot/sub-slot m+k and indicating HARQ_retx_offset, the PUCCH slot/sub-slot n of the HARQ-ACK codebook to be re-transmitted is determined as either: - Alt. 1: n = m - HARQ_retx_offset - Alt. 2: n = m + k - HARQ_retx_offset - FFS: value range of the HARQ-retx_offset Agreement For one-shot triggering of HARQ-ACK re-transmission on PUCCH, - in case the dynamic Type 2 HARQ-ACK codebook is configured, the HARQ-ACK codebook per PHY priority on the indicated PUCCH is constructed by appending the Type 2 HARQ-ACK codebook to be re-transmitted to the Type 2 HARQ-ACK codebook of the indicated PUCCH (carrying new, initial HARQ-ACK information) per PHY priority. - in case the semi-static Type 1 HARQ-ACK codebook is configured, the HARQ-ACK codebook per PHY priority on the indicated PUCCH is constructed by appending the Type 1 HARQ-ACK codebook to be re-transmitted to the Type 1 HARQ-ACK codebook of the indicated PUCCH (carrying new, initial HARQ-ACK information) per PHY priority.

RAN1#107-e (Nov. 2021)

Agreement The one-shot HARQ re-transmission on PUCCH is configured per PUCCH cell group (i.e., separately configurable for primary and secondary PUCCH cell group). Agreement For one-shot HARQ re-transmission on PUCCH, the ‘HARQ re-tx offset’ is determined as Alt. 1: n = m - HARQ_retx_offset Conclusion There is no consensus to support the simultaneous configuration of one-shot HARQ-ACK re-transmission and dynamic PUCCH cell switching in Rel-17. Agreement Support simultaneous configuration of one-shot HARQ-ACK re-transmission and semi-static PUCCH cell switching: - the ‘backward HARQ-ACK slot-offset’ is interpreted with the granularity of a PUCCH slot of the respective PHY priority of PCell /PSCell / PUCCH SCell Agreement Apply a 1-bit triggering DCI field for triggering indication of one-shot HARQ re-transmission on PUCCH. - The triggering DCI with the triggering bit set to ‘1’ is not able to schedule PDSCH. - Some unused bit field in the DCI is used to indicate the HARQ slot offset. - FFS: if the ‘one-shot HARQ-ACK request’ field can be reused - FFS: which unsed DCI field in the DCI is used for HARQ slot offset indication - FFS: The indication of whether the PDSCH is not scheduled will reuse Rel-16 type-3 HARQ ACK CB UE behavior Agreement For one-shot triggering of HARQ re-transmission, introduce a new 1-bit DCI field in DCI format 1_1 and in DCI format 1_2 (if DCI format 1_2 is configured with one-shot triggering of HARQ-ACK re-transmission). Working Assumption For one-shot triggering of HARQ re-transmission, in addition to one-shot triggering of HARQ re-transmission after the initial PUCCH transmission slot, the triggering is supported before the initial PUCCH transmission slot - Re-transmission triggering does not change processing for the initial PUCCH transmission (i.e., HARQ multiplexing / dropping / transmission) - The UE expects the PUCCH carrying the HARQ-ACK re-transmission to be scheduled in a slot/sub-slot after the initial PUCCH transmission slot/sub-slot. - The support for the triggering before the initial PUCCH transmission slot is subject to separate UE capability indication Agreement For one-shot HARQ-ACK re-transmission, the value range for HARQ re-tx offset is fixed in the specification

RAN1#107bis-e (Jan. 2022)

Agreement For one-shot HARQ-ACK re-transmission, the value range for HARQ re-tx offset is given by [min_HARQ_retx_offset_value, max_HARQ_retx_offset_value] with an indication of 1 slot / sub-slot within that range. - FFS the fixed value of min_HARQ_retx_offset_value - FFS the fixed value of max_HARQ_retx_offset_value Conclusion For one-shot HARQ re-transmission on PUCCH, the UE determines no PDSCH is scheduled when the triggering bit is set to ‘1’ (i.e. the UE does not need to in addition check any specific resource allocation setting). Agreement RAN1 confirms the following RAN1#107-e working assumption: Working Assumption For one-shot triggering of HARQ re-transmission, in addition to one-shot triggering of HARQ re-transmission after the initial PUCCH transmission slot, the triggering is supported before the initial PUCCH transmission slot - Re-transmission triggering does not change processing for the initial PUCCH transmission (i.e., HARQ multiplexing / dropping / transmission) - The UE expects the PUCCH carrying the HARQ-ACK re-transmission to be scheduled in a slot/sub-slot after the initial PUCCH transmission slot/sub-slot. - The support for the triggering before the initial PUCCH transmission slot is subject to separate UE capability indication. Agreement For one-shot HARQ-ACK re-transmission, - the minimum value for the HARQ re-tx offset min_HARQ_retx_offset_value is -7. - the maximum value for the HARQ re-tx offset max_HARQ_retx_offset_value is 24. - Note: UE capability reporting on the UE supported value range for HARQ_retx_offset in the scope of [min_HARQ_retx_offset_value, max_HARQ_retx_offset_value ] that can be indicated by the gNB for the UE can be further discussed in UE capabilities Agreement For one-shot triggering of HARQ-ACK re-transmission, the HARQ_retx_offset is indicated by the bits of the MCS field for transport block 1. Conclusion There is no consensus on the support of HARQ-ACK CB size indication in the triggering DCI for HARQ-ACK re-transmission Conclusion There is no consensus to support the following in Rel-17: - For one-shot HARQ re-transmission on PUCCH, if certain HARQ process IDs of the requested HARQ CB to be retransmitted is replaced by new HARQ bits, the UE transmits the new content of HARQ process(es) being updated. Agreement For PUCCH repetition and one-shot HARQ-ACK re-transmission, if the gNB triggers the HARQ-ACK CB re-transmission from a PUCCH slot indicated by HARQ_retx_offset where a HARQ-ACK in a first PUCCH is dropped due to overlapping with another, second PUCCH, where the first PUCCH and second PUCCH have the same L1 priority, and at least one of the first PUCCH and the second PUCCH is subject to a repetition, the UE re-transmits the HARQ-ACK CB of the second PUCCH from the slot.

## 2.5 PUCCH cell switching

### 2.5.1 Generic agreements (applicable to both, dynamic & semi-static PUCCH cell switching)

RAN1#103-e (Oct./Nov. 2021)

Agreements: In the studies on PUCCH carrier switching for HARQ-ACK, PUCCH carrier switching for different cells operated is considered only for cells that are part of the active UL CA configuration.

RAN1#104-e (Jan. 2021)

Agreements: For further study on whether and how to support PUCCH carrier switching in a PUCCH group, focus on the following three alternatives: - Alt. 1: PUCCH carrier switching is based dynamic indication in DCI - Alt. 2B: PUCCH carrier switching is based on certain (semi-static) rules - Alt. 2C: PUCCH carrier switching is based on RRC configured PUCCH cell timing pattern of applicable PUCCH cells - Note: In above alternatives, it is assumed that HARQ-ACK corresponding to PDSCH received on a Pcell/PScell or an Scell in a PUCCH group, can be sent on a PUCCH on an Scell also instead of only on Pcell/PScell/PUCCH-SCell in the same PUCCH group, as opposed to Rel-16 where HARQ-ACK corresponding to PDSCH received on a Pcell/PScell or an Scell in a PUCCH group can only be sent on Pcell/PScell/PUCCH-SCell in the same PUCCH group. - Note: Realistic deployment scenarios including TDD configurations should be considered for the study

RAN1#105-e (May 2021)

Agreement: Support PUCCH carrier switching based on dynamic indication in DCI scheduling a PUCCH and semi-static configuration - Details are FFS (including applicability of dynamic and/or semi-static means) - Aim for minimum specification impact - Dynamic indication and/or semi-static configuration are subject to separate UE capabilities - The semi-static PUCCH carrier switching configuration operation is based on RRC configured PUCCH cell timing pattern of applicable PUCCH cells and supports PUCCH carrier switching across cells with different numerologies. - FFS whether additional rules are needed to support PUCCH carrier switching across cells with different numerologies - FFS the maximum number of PUCCH cells - FFS whether and how to support joint operation of dynamic and semi-static carrier switching for a UE - FFS whether and how to support joint operation of PUCCH carrier switching and SPS HARQ-ACK deferral Agreement: For PUCCH carrier switching, the PUCCH resource configuration is per UL BWP (i.e. per candidate cell and UL BWP of that specific candidate cell).

RAN1#106-e (Aug. 2021)

Agreement Update the following RAN1#105-e agreement as (RED): - RAN1#105-e Agreement: For PUCCH carrier switching, the PUCCH resource configuration (i.e. pucch-Config / PUCCH-ConfigurationList) is per UL BWP (i.e. per candidate cell and UL BWP of that specific candidate cell). - FFS: CSI and SR

RAN1#106bis-e (Oct. 2021)

Agreement For PUCCH carrier switching, support PUCCH carrier switching only among different TDD cells with PUCCH configured on the NUL carrier in Rel-17 Agreement For PUCCH cell switching, support independent TPC per PUCCH cell including - Separate P0 / TPC configuration per PUCCH cell - Note: This flexibility is already provided as PUCCH-config is per UL BWP of a PUCCH cell - Accumulating closed loop power control commands only within the same PUCCH target cell by reusing Rel-15 procedure, i.e. - For dynamic PUCCH cell indication, the TPC command in the DCI scheduling the PUCCH only applies for the dynamically indicated PUCCH target cell - For semi-static / time-domain pattern, the TPC command in the DCI scheduling the PUCCH only applies for the determined PUCCH target (using the time-domain pattern) - Separate TPC command indication using DCI format 2_2 for the individual PUCCH cells - Note: this requires configuration of individual TPC command starting points for each PUCCH cell within DCI format 2_2 Agreement For semi-static and dynamic indication of PUCCH cell switching, the PUCCH repetition factor is determined based on the PUCCH format or PUCCH resource on the target PUCCH cell for the first repetition. Agreement PUCCH cell switching between 2 cells is supported in Rel-17.

RAN1#107-e (Nov. 2021)

Conclusion For PUCCH cell switching DCI field size alignment is done by: - For dynamic PUCCH cell switching, the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_1 and 1_2 is determined by the largest K1 set among the K1 sets of all candidate PUCCH cells for PUCCH cell switching based on dynamic indication - i.e., a number of most significant bits with value set to '0' are inserted to smaller field until the bit width of the field for all the PUCCH cells are the same - Note: for semi-static PUCCH cell switching only the K1 set of PCell is needed - For semi-static and dynamic PUCCH cell switching, the bit width of the PRI field in DCI format 1_2 is determined by the largest value of numberOfBitsForPUCCH-ResourceIndicatorDCI-1-2 of all PUCCH cells - i.e., a number of most significant bits with value set to '0' are inserted to smaller field until the bit width of the field for all the PUCCH cells are the same - FFS: If similar handling is applied for ChannelAccess-CPext DCI field (0 or 2 bit) Agreement For PUCCH cell switching and a PUCCH transmission on the alternative PUCCH cell, the alternative PUCCH cell is used to derive the downlink pathloss estimate PLb,f,c(qd), i.e., replace in the main bullet of the PLb,f,c(qd) determination in Sec. 7.2.1 of 38.213 the ‘primary cell’ with ‘cell for PUCCH transmission’ Conclusion There is no consensus to support simultaneous configuration of semi-static PUCCH cell switching and dynamic PUCCH cell switching in Rel-17.

RAN1#107bis-e (Jan. 2022)

Conclusion There is no consensus for introducing further specification support for the following - PUCCH cell switching between cells with shared spectrum channel access (in any mode) - PUCCH cell switching between a cell with licensed spectrum and a cell with shared spectrum channel access (in any mode) Conclusion There is no consensus to support MAC CE activation indicating a set of values of pucch-SpatialRelationInfoId applicable to the alternative PUCCH sSCell for PUCCH cell switching in Rel-17.

### 2.5.2 Semi-static PUCCH cell switching

RAN1#106-e (Aug. 2021)

Agreement Semi-static PUCCH carrier switching is applicable to all UCI types incl. HARQ-ACK, SR and CSI.

RAN1#106bis-e (Oct. 2021)

Agreement For semi-static PUCCH cell switching, PCell / PSCell / PUCCH-SCell is reference cell: - The time domain pattern configurations are based on the numerology of the reference cell. - The PDSCH to HARQ-ACK offset k1 is interpreted based on the numerology and PUCCH configuration of a reference cell to be able to apply the time-domain PUCCH cell switching pattern. - Note: There may not be a need to define a ‘reference cell’ in the specification. This terminology is used for further clarifications of the procedure. Agreement For semi-static PUCCH cell switching, the time-domain pattern configuration is based on the following properties: - A single time-domain pattern is configured per PUCCH cell group - The granularity of the time-domain pattern is one slot of the PCell / PSCell / PUCCH-SCell reference cell - The time-domain pattern is applied periodically - FFS on period / pattern length (e.g., 10ms, RRC configured, …). - The pattern defines for each slot of the PCell / PSCell / PUCCH-SCell reference cell at least the applicable target PUCCH cell Agreement For semi-static PUCCH cell switching, the PUCCH resource indicator (PRI) is interpreted based on the PUCCH configuration of determined target PUCCH cell. Agreement The periodicity / length of the time-domain pattern for semi-static PUCCH cell switching is directly determined by the RRC configuraton of the time domain pattern pucchCellPattern - Note: pucchCellPattern has a variable length of (1… maxNrofSlots) Agreement Down-select in RAN1#107-e from Alt. 1 & Alt. 3 below: For PUCCH carrier switching based on semi-static operation, for the case the PCell slot to be longer than the target PUCCH cell slot or sub-slot (i.e. multiple target PUCCH cell slots overlapping with a single PCell slot), the following PUCCH cell slot is used for UCI transmission: - Alt. 1: the first target PUCCH slot overlapping with the PCell slot - Alt. 3: using a relative slot-offset within the reference cell slot, the relative slot offset is configured in the time domain pattern (i.e. time domain pattern contains ‘cell index’ & ‘slot_offset’ for each reference cell slot) - Note: different relative slot offset can be configured for each reference cell slot in the time domain pattern, details see R1-2108829 Agreement Down-select in RAN1#107-e from Alt. 2 & Alt. 4 below: For PUCCH carrier switching based on semi-static operation, for the case the PCell slot to be shorter than the target PUCCH cell slot, - Alt. 2: the UE does not expect the same UCI type (i.e. HARQ-ACK, SR or CSI) from more than one PCell PUCCH slot to be overlapping with a single dynamically indicated PUCCH cell slot - Note: there can be e.g. HARQ-ACK only be present in either of the overlapping slots, but not in more than one overlapping slot. - Alt. 4: the UE does not expect a semi-static PUCC cell configuration, where a single target PUCCH slot / sub-slot would be overlapping with more than one PCell slot/sub-slot.

RAN1#107-e (Nov. 2021)

Agreement For PUCCH cell switching based on semi-static operation, for the case the PCell slot to be longer than the target PUCCH cell slot or sub-slot (i.e., multiple target PUCCH cell slots overlapping with a single PCell slot), adopt Alt 1, i.e., the first target PUCCH slot overlapping with the PCell slot is used for UCI transmission. Agreement The time-domain pattern for semi-static PUCCH cell switching is separately configurable for the primary and secondary PUCCH cell group. Agreement The time-domain pattern for semi-static PUCCH cell switching is based on the reference SCS configuration provided by tdd-UL-DL-ConfigurationCommon and is common to every configured UL BWP (of PCell / SPCell / PUCCH SCell). Agreement For PUCCH cell switching based on semi-static operation, adopt Alt. 4, i.e., the UE does not expect a semi-static PUCCH cell configuration, where a single target PUCCH slot / sub-slot would be overlapping with more than one PCell slot/sub-slot. Agreement For semi-static PUCCH cell switching, if the alternative PUCCH cell (i.e. PUCCH sCell) is deactivated or the alternative PUCCH Cell is dormant, the UE does not apply time-domain pattern and the UCI is to be transmitted on PCell / SPCell / PUCCH SCell.

RAN1#107bis-e (Jan. 2022)

Agreement For PUCCH cell switching based on semi-static time domain pattern, the Type 1 HARQ-ACK codebook construction is based on the k1 set(s) of the PCell / SPCell / PUCCH SCell.

### 2.5.3 PUCCH cell switching based on dynamic indication

RAN1#105-e (May 2021)

Agreement: For PUCCH carrier switching based on dynamic indication in DCI scheduling a PUCCH (i.e. Alt. 1), the PDSCH to HARQ-ACK offset k1 is interpreted based on the numerology of the dynamically indicated target PUCCH cell.

RAN1#106-e (Aug. 2021)

Agreement In addition to HARQ-Ack of PDSCH dynamically scheduled by a DCI indicating a PUCCH carrier, the dynamic target carrier indication also applies to: - HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI based on the indication in the activation DCI - HARQ-ACK corresponding to the SPS Release DCI based on the indication in the release DCI - triggered PUCCH for Rel-16 Type 3 CB, Rel-17 enh. Type 3 CB of smaller size and Rel-17 one-shot triggering for HARQ-Ack retransmission based on the indication in the triggering DCI - FFS: Additional cases

RAN1#106bis-e (Oct. 2021)

Agreement UE does not expect overlapping PUCCH slots with dynamic PUCCH cell indication on more than one cell, i.e., gNB should only dynamically indicate a single PUCCH cell for a final PUCCH slot. Agreement For PUCCH cell switching based on dynamic indication in the DCI, introduce a new, dedicated DCI field for the DCI scheduling PDSCH to indicate the target PUCCH cell. Agreement In addition, the dynamic target PUCCH cell indication also applies to HARQ-ACK corresponding to SCell dormancy indication without scheduling PDSCH. Agreement Support PUCCH cell switching based on dynamic indication in the DCI using DCI format 1_2 for a UE supporting DCI format 1_2. - The presence of the ‘PUCCH carrier switching’ bitfield in DCI format 1_2 is RRC configured. Conclusion There is no consensus to support multiplexing of HARQ-ACK (without dynamic PUCCH cell indication), SR and P/SP-CSI on the dynamically indicated PUCCH cell (other than PCell / PSCell / PUCCH-SCell) in Rel-17. - FFS: further handling, incl. e.g., UE does not expect overlapping HARQ-ACK (without dynamic PUCCH cell indication), SR and P/SP-CSI or overlapping HARQ-ACK (without dynamic PUCCH cell indication), SR and P/SP-CSI is to be dropped - FFS: overlapping definition for SR and P/SP-CSI in terms of PUCCH slot or PUCCH resource

RAN1#107-e (Nov. 2021)

Conclusion For dynamic PUCCH cell switching, the UE does not expect a PUCCH slot with UCI on PCell /SPCell / PUCCH SCell to overlap with a PUCCH slot with HARQ-ACK on the dynamically indicated alternative PUCCH PUCCH cell. - The UCI on PCell /SPCell / PUCCH SCell dropped due to collision with semi-static DL symbols, SSB, and symbols indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set is exempted and is not multiplexed on the PUCCH on the alternative PUCCH cell.

RAN1#107bis-e (Jan. 2022)

Agreement For dynamic PUCCH cell switching, the Type 1 HARQ-ACK codebook construction is based on the k1 set(s) of the dynamically indicated PUCCH cell.

# CSI enhancements (AI 8.3.1.2)

RAN1#102-e (Aug. 2020)

Agreement: - CSI feedback enhancement for Multi-TRP transmission is not to be discussed further under IIoT/URLLC enhancement WI Agreements: - Baseline assumptions are used as the required minimum to be simulated for the evaluation of candidate CSI enhancement schemes - Reuse the assumptions in TR 38.824 and TR 38.901 as a starting point - Companies shall report additional parameters (e.g., CSI measurement settings, CSI reporting schemes) used in their evaluation - FFS details of baseline assumptions - Companies can bring additional simulation results with other set(s) of assumptions Agreements: - Study/evaluate further on following CSI enhancement schemes in terms of technical benefit, specification and implementation impacts. - New triggering methods for A-CSI and/or SRS - New reporting based on one or more of the following: - Case 1: channel/interference measurement for new CSI reporting, considering aspects such as one or more of the following: - Reporting more accurate interference characteristics - Reduced CSI feedback overhead (e.g., reporting interference measurement only) - Enhanced CSI reporting such as WB/SB CQI - Case 2: other measurement (other than channel/interference) for additional information - E.g., PDCCH/PDSCH decoding, recommended HARQ RV sequence, etc. - It targets to help gNB scheduler for better link adaptation of (re)transmission - [Reduced CSI computation time/complexity] - [CSI feedback for PDCCH] - Other CSI enhancement schemes that enable accurate MCS selection are not precluded - Detailed assumptions of the proposed CSI enhancement schemes should be provided by the proponent, such as - Reporting values - Triggering conditions for the reporting - Associated measurement resource - Uplink resource to be used for the reporting - How to use the reported information at the gNB scheduler - CSI-RS overhead and CSI reporting frequency - CSI reporting latency/timeline - Etc. Agreements: - Consider Table 1 as baseline assumption for system level simulation for evaluating CSI enhancement schemes - The uses cases in Table 1 is for simulation purposes and it does not preclude a CSI enhancement scheme which is beneficial for the other URLLC use cases - No baseline assumption is used for link level simulation - Companies are encouraged to use one of LLS assumption tables in Section A.3 in TR38.824 for any link level simulation Table 1. Baseline SLS assumption for CSI enhancement schemes in URLLC/IIoT Parameters; Values. Performance metric; Option-1 (section 5.1 of TR 38.824) Additional metrics (it is up to company to bring results with additional metric): - MCS prediction error (e.g., difference of a scheduled MCS and an ideal MCS) - DL/UL signaling overhead - CCDF of latency samples from all UEs - BLER of 1st transmission - Resource utilization - Spectral efficiency. Use cases; Following two use cases can be considered for new triggering method and new reporting. Companies are encouraged to evaluate the following cases in descending priority: - Rel-15 enabled use case (e.g. AR/VR) in TR 38.824 - Reliability: 99.999 - Latency: 4ms (200bytes) - Traffic mode: FTP model 3 (100p/s) - Factory automation in TR 38.824 - Reliability: 99.9999 - Latency: 1ms (32bytes) - Traffic mode: Periodic deterministic traffic model with arrival interval 2ms - Rel-15 enabled use case (e.g. AR/VR) in TR 38.824 - Reliability: 99.999 - Latency: 1ms (32bytes) - Traffic mode: FTP model 3 (100p/s) - Assumptions for eMBB and URLLC UEs sharing the same carrier is used (as in A2.5 of TR 38.824). Simulation assumptions; Following simulation assumption is used based on the use case selected: - Rel-15 enabled use case with UMa (Table A.2.4-1 in TR 38.824) - Factory automation at 4GHz (Table A.2.2-1 in TR38.824) with following update: - Channel model is replaced with InF (InF-DH) in TR 38.901 - Companies can bring results with other InF scenarios additionally - Layout is replaced with BS deployment in Table 7.8-7 in TR 38.901. Transmission scheme; Multiple antenna ports Tx scheme - Companies report the details of Tx scheme used.

RAN1#103-e (Oct/Nov 2020)

Agreements - No change of CSI processing time relative to Rel-16 CSI in this WI - CSI processing time specific to a new CSI reporting quantity/type (if supported) can be studied Agreement: - For Case-2 new reporting, continue studying with focus on the new reporting type based on PDSCH decoding for OLLA performance enhancement for initial and re-transmissions of PDSCH. Agreements: For Case-1 New reporting, the following candidate schemes have been identified to address the fast interference change over time. Continue studying with focus on the identified schemes below for further study and evaluation. - Scheme 1a: New reporting quantity based on CQI/SINR statistics, e.g., - CQI/SINR statistics (e.g., mean, variance, etc.) - CSI prediction - Scheme 1b: New reporting quantity of interference statistics (e.g., mean, variance, interference covariance matrix, etc.) - Scheme 1c: New reporting quantity based on modifying existing reporting format, e.g., - CQI reporting considering the worst subbands - Subband CQI granularity enhancement - Scheme 1d: New reporting quantity related to CSI expiration time - Scheme 1e: New reporting quantity with partial information update, e.g., - CSI reporting with interference update only Companies are encouraged to investigate the above schemes, aiming for down-selection in RAN1#104-e

RAN1#104-e (Jan/Feb 2021)

Conclusion: Continue evaluation of new reporting Case 1 and Case 2 for the schemes identified in Appendix B of R1-2102131. - Companies are encouraged to provide their views on each scheme against each criterion in respective Tables in Appendix B. - Companies are encouraged to provide additional evaluation results for as many schemes as possible, based on assumptions agreed in RAN1#102-e. - Aim for down-selection at RAN1#104-b-e by taking into account evaluation results and assessment against criteria from Appendix B.

RAN1#104bis-e (April 2021)

Conclusion: For new reporting Case 1, do not consider further the following schemes: - Case 1-2: CSI prediction - Case 1-4: Interference covariance matrix - Case 1-9: Reference wideband CQI excludes worst sub-bands - Case 1-10: CSI expiration time Agreements: For new reporting Case 2, focus study on reporting of delta-CQI/MCS (Case 2-3): - Note: this delta-CQI/MCS is determined based on UE implementation (for example, using SINR, LLR, raw BER, flipped bits, LDPC iterations, BLEP, # fail parity checks, etc.) - Companies are encouraged to provide more details in their analysis - FFS: Granularity of new report type (e.g. units of CQI or MCS, how many bits) - FFS: Whether quantity reported is relative to the scheduled MCS Agreement: Focus study on the following for new reporting Case 1: - Reporting of new metric, where new metric shall be determined based on network configured channel and interference measurement interval (multiple CMR and/or IMR instances) to enable accurate MCS selection. - Downselect by RAN1#105 to at most a single method from the following options: - Mean-CQI/SINR and stdev-CQI/SINR (FFS details) - CSI based on worst IMR occasion (FFS details) - Interference standard deviation (FFS details) - Worst-M CQI (FFS details) - FFS: Whether network configured channel and interference measurement interval can also be applied to existing CSI type - Increasing granularity of subband CQI (e.g. 3-bits differential subband CQI or 4-bits full subband CQI). - Updating only CQI in a report, where CQI is conditioned on a previous instance in which RI/PMI/(CRI) is updated. - Applicable for same reporting quantity as R16 for CQI. - FFS: Whether network configured channel and interference measurement interval can also be applied - FFS: Whether RI/PMI/(CRI) is transmitted in a report where only CQI is updated - FFS: how to report the updated CQI - FFS: whether the CQI processing time can be is reduced compared to Rel-16 CSI processing delay

RAN#92 (June 2021)

(RP-211297) RAN1 to further investigate the following for CSI enhancements for IIoT/URLLC: - Increasing the number of bits used for the reported subband CQI (3-bits differential subband CQI or 4-bits CQI) - Reporting of delta-MCS: - Report consists of delta-MCS for a TB received with MCS index IMCS: - delta-MCS is calculated from the difference between IMCS_tgt and IMCS, where IMCS_tgt is the largest MCS index such that the estimated BLER for a TB received with this MCS index would be smaller than or equal to a BLER target, and IMCS is the MCS index of the received TB.

RAN1#106-e (Aug. 2021)

Agreement For subband CQI reporting with more than 2 bits per subband - Support 4-bits CQI only Agreement For subband CQI reporting in Rel-17, RRC can configure use of legacy 2-bits D-CQI or 4-bits CQI for each CSI report configuration. - This feature is subject to UE capability - FFS: Whether wideband CQI report can be omitted Conclusion There is no consensus in RAN1 on the support of delta-MCS in Rel-17.

RAN1#106bis-e (Oct. 2021)

Agreement When subband CQI reporting is configured with 4-bits per subband, UE includes wideband CQI in report.

# NR-U Enhancements (AI 8.3.2)

RAN1#102-e (Aug. 2020)

Agreements: - For semi-static channel access mode, - If sensing is needed, it is performed immediately before the configured/scheduled transmission opportunity. - For operation with semi-static channel access, the Rel-16 random starting offsets for UL configured grants with Full BW allocation when UE initiates a COT, is not supported. Agreements: - For semi-static channel access mode, - When gNB operates as an initiating device - The gNB is not allowed to transmit during the idle period of any FFP associated with the gNB in which the gNB initates a COT - When a UE operates as an initiating device - The UE is not allowed to transmit during the idle period of any FFP associated with the UE in which the UE initates a COT - When a UE shares a COT initiated by the gNB during an FFP associated with the gNB - The UE is not allowed to transmit during the idle period of that FFP in which the UE shares the COT initiated by the gNB - When the gNB shares a COT initiated by a UE during an FFP associated with the UE - The gNB is not allowed to transmit during the idle period of that the FFP in which the gNB shares the COT initiated by the UE - FFS whether/how to support additional restrictions to the idle period Agreements: - For semi-static channel access mode, support using the transmission of any scheduled/configured UL channel/signal to initiate a COT by a UE in RRC_CONNECTED mode - FFS the case when the UE is IDLE/INACTIVE mode Agreements: - A UE initiates a COT in an FFP associated with the UE, if the UE transmits a UL transmission burst starting at the beginning of the FFP and ending at any symbol before the FFP’s idle period after a successful CCA of 9us immediately before the UL transmission burst. Agreements: - At least for FBE, configuration of (cg-RetransmissionTimer) should not be mandated when configured grant Type 1 or Type 2 are configured on unlicensed spectrum. Conclusion: Further study and decide how to harmonize the CG features for Rel-16 URLLC and Rel-16 NR-U. Table 1 in R1-2005376 can be used as a starting point for the corresponding discussion and decision. Agreements: - Conditions on the channel access procedures with respect to sensing duration and transmission gap for UE-initiated COT with UE-to-gNB COT sharing is similar as those for gNB initiated COT and gNB-to-UE COT sharing in Rel-16 by exchanging UE and gNB roles. Agreements: - UE-to- gNB COT sharing in semi-static channel access mode is supported. - The gNB determines a COT in an FFP associated to a UE, that is initiated by the UE, if the gNB detects a UL transmission from the UE starting from the beginning of the FFP and ending before the idle period of the FFP. - FFS details - When the gNB determines a UE has initiated a COT in an FFP associated to the UE, the gNB can transmit within the FFP and before the idle period corresponding to the FFP. - FFS whether/how UE to gNB COT sharing when the gap is >16us Agreements: For semi-static channel access mode, o Start of FFP for UE-initiated COT can be different from the start of FFP for gNB-initiated COT. o FFS: FFP Periodicity for UE-initiated COT can be different from the FFP periodicity for gNB-initiated COT. Agreements: - For semi-static channel access mode, - FFP parameters for UE-initiated COT can be provided to the UE by at least dedicated RRC signaling. - FFS on to be provided by SIB-1 - FFS whether the UE FFP periodicity is explicitly configured, or implicitly determined based on other higher layer parameters

RAN1#103-e (Oct./Nov. 2020)

Agreements: - In semi-static channel access mode, a single FFP (periodicity and offset) is associated to an initiating device (gNB or UE) at a given time which can be used for the purpose of channel occupancy. The FFP configuration that is used for initiating channel occupancy purposes, is such that it shall not be changed for at least 200ms Conclusion: - For operation on unlicensed channels and irrespective of the adopted LBT mechanism (LBE or FBE), all transmissions in DL and UL are controlled by gNB similarly to licensed channels, and potential collisions or blocking are controlled/mitigated by gNB. Agreements: - UE-to-gNB COT sharing in semi-static channel access mode with a gap > 16us is supported Conclusion: If a device X at a given time is initiating a COT, the applicable FFP for the device X is the FFP associated with X. If a device X at a given time is sharing a COT initiated by a device Y, the applicable FFP for the device X is the FFP associated with Y. Note 1: One of the devices X and Y is a UE and the other is its serving gNB. Note 2: Whether or not there is additional restriction on idle period is still FFS. Agreements: Down-select one of the following options (target RAN1#104-e): - Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16. - Option 2-a: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and cg-RetransmissionTimer-r16, respectively. - Option 2-b: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and new parameter Y, respectively, where X and Y are different from cg-RetransmissionTimer-r16. - Option 3: CG-UCI based procedures are supported for unlicensed. CG-DFI based procedures are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16 - Note: Procedures based on CG-UCI rely on UE including CG-UCI in CG PUSCH at least as in Rel-16 where the values of the respective fields of CG-UCI are decided by UE. - Note: Procedures based on CG-DFI rely on automatic re-transmission on CG configuration and reception of CG downlink feedback information (DFI) in DCI for re-transmissions. Agreements: - The gNB configures a UE to initiate semi-static CO in an unlicensed channel(s) only if the gNB configures the UE also with the higher layer parameters of the gNB’s initiating semi-static CO in the same channel(s). - Note: UE initiated FBE configuration is configured per serving cell Agreements: In semi-static channel access mode, FFP Period for UE-initiated COT is separately provided from FFP period for gNB-initiated COT. o Note: Any value for the period, shall be at least 1ms and at most 10ms. o Note: Aim for low complexity operation to handle gNB and UE COT interactions Agreements: In semi-static channel access mode, a UE should be able to determine whether a scheduled UL transmission should be transmitted according to shared gNB COT or UE-initiated COT. - UE determines the initiator of a COT based on at least one of the following alternatives: - Alt 1: Introduce additional bit field in the scheduling DCI - Alt 2: Based on ChannelAccess-CPext field in DCI - Alt. 3: Based on a predetermined rule(s) - Alt. 4: Based on RRC signalling - Alt. 5: Based on MAC CE - FFS other alternatives - FFS on overriding possibility and/or the assumption - Note: A scheduled UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT. Agreements: In semi-static channel access mode: - When a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE, down-select one of the following: - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT. - Alt-c: The UE assumption on whether the configured UL transmission is allowed to correspond to UE-initiated COT is based on gNB configuration. - When a configured UL transmission starts after a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE: - If the UE has already initiated the UE FFP, then UE assumes that the configured UL transmission corresponds to UE-initiated COT - Otherwise, If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and if the UE has already determined that gNB has initiated that gNB FFP, then UE assumes that the configured UL transmission corresponds to gNB-initiated COT. - FFS on other conditions for determining the corresponding UE or gNB initiated COT - Note: A configured UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT.

RAN1#104-e (Jan/Feb. 2021)

Agreement: - PUSCH repetition Type B is supported for unlicensed band operation when using NR IIoT Rel-16 based CG - FFS whether/how to enhance Agreement: - In semi-static channel access mode, UE FFP periodicity is chosen from the following set of values in ms: {1, 2, 2.5, 4, 5,10}. - FFS on other values Agreement: - In semi-static channel access mode: - An FFP period for UE-initiated COT is configured as the same, integer multiple of, or inter-factor of the FFP period configured for gNB-initiated COT - FFP period for UE-initiated COT can be configured independently from FFP period of gNB-initiated COT, if the UE indicates the corresponding capability - FFP offset for UE-initiated COT is the starting point of first UE FFP relative to the radio frame X boundary. - The offset value range is 0 ≤ offset ＜FFP period of UE-initiated COT - FFS on X (e.g. X=0, or X= even index number) Agreement: In semi-static channel access mode when a UE can operate as initiating device, - Select one of the following alternatives to determine whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT: - Alt-a: Determination based on the content in the scheduling DCI - FFS on whether the corresponding field(s) can be absent in DCI - If absent, determination based on the rules applied for configured UL transmissions is applied - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period - Alt-b: Determination based on the rules applied for a configured UL transmission Agreement: In semi-static channel access mode when a UE can operate as UE-initiated COT, - Select one of the following alternatives to determine whether a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, is based on UE-initiated COT or sharing a gNB-initiated COT: - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT. Agreement: - In semi-static channel access mode, sharing a UE initiated COT through the gNB to other intra-cell UEs for UL transmissions, is not supported.

RAN1#104bis-e (April 2021)

Agreements: - Support explicit RRC configuration for the UE-FFP parameters including period and offset in RRC connected mode. Agreements: - For semi-static channel access mode, the offset value for configuration of a UE-FFP for a serving cell has a symbol level granularity. Agreement: - For semi-static channel access mode, in addition to the agreed set of period values for configuration of a UE-FFP for a serving cell: - Do not support any additional period value Agreement: - For semi-static channel access mode, the starting point of first UE FFP for a serving cell - is relative to the boundary of the radio frame of even index number (i.e. X=even indexed number in RAN1#104-e agreement). Agreement: - In semi-static channel access mode, the gNB can schedule by a DCI UL transmission(s) in a later g-FFP that is different from the g-FFP that carries the scheduling DCI. - The UL transmission can occur only if the corresponding channel access requirements are met. - FFS on details. Agreement: - In semi-static channel access mode, the gNB can schedule by a DCI DL transmission(s) in a later g-FFP that is different from the g-FFP that carries the scheduling DCI. - The DL transmission can occur only if the corresponding channel access requirements are met. - FFS on details. Agreement: - Select one of the following options (aiming for RAN1#105-e): - Option 1: Do not support PUSCH repetition Type Bwhen using based on NR-U Rel-16 based CG for unlicensed band operation. - Option 2: Support enhancements of PUSCH repetition Type B when using based on NR-U Rel-16based CG for unlicensed band operation. FFS whether/how to enhance Agreements - For PUSCH repetition Type B enhancements on unlicensed spectrum, further study whether PUSCH segmentation should take into account the idle period of an FFP. - FFS on details Agreements - For PUSCH repetition Type B enhancements on unlicensed spectrum, further study whether orphan symbol(s) are transmitted if they are between two actual repetitions that are transmitted. FFS on details Conclusion: - In semi-static channel access mode, a UE as an initiating device, is allowed to transmit during the idle period of any FFP associated with the serving gNB if the UE transmission is based on UE initiated COT - Note: the gNB may disallow UL transmission during symbols of the idle period by configuring them either as semi-static DL symbols, or indicating them as DL with SFI. Agreement: - Option 2-b and option 3 are not considered further for the agreement in RAN1#103-e regarding CG harmonization

RAN1#105-e (May 2021)

Agreement: - Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16. - Note: Procedures based on CG-UCI rely on UE including CG-UCI in CG PUSCH at least as in Rel-16 where the values of the respective fields of CG-UCI are decided by UE. - Note: Procedures based on CG-DFI rely on automatic re-transmission on CG configuration and reception of CG downlink feedback information (DFI) in DCI for re-transmissions Agreement: In semi-static channel access mode when a UE can operate as UE-initiated COT, - To determine whether a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, is based on UE-initiated COT or sharing a gNB-initiated COT: - If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT Agreement: In semi-static channel access mode when a UE can operate as initiating device, - To determine whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT: - Determination based on the content in the scheduling DCI - FFS on whether the corresponding field(s) can be absent in DCI - If absent, determination based on the rules applied for configured UL transmissions is applied - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period

RAN1#106-e (Aug. 2021)

Agreement In semi-static channel access mode, the content in a scheduling DCI that indicates the assumption on the COT-initiator for the scheduled transmission is determined based on the channel access field in the DCI. Agreement In semi-static channel access mode, - The inclusion of the channel access field in Rel-16 DCI 0_1 and 1_1 in Rel-17 DCI 0_2 and 1_2, respectively, is supported. Agreement In semi-static channel access mode, the size of channel access field in a scheduling DCI with format 0_0/1_0, 0_1/1_1, 0_2/1_2 is 2 bits. Agreement In semi-static channel access mode, the content of the channel access field in a DCI scheduling a UL transmission for a UE determines an index to a row in Table 1 TABLE 1 Bit field mapped to index; Channel Access Type; The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211]; Initiator of a channel occupancy associated to UL transmission described in Clause x.x in TS 37.213. 0; No sensing as defined in Clause 4.3 in TS 37.213; 0; gNB. 1; No sensing as defined in Clause 4.3 in TS 37.213; 2; gNB. 2; 9us sensing within a 25us interval as defined in Clause 4.3 in TS 37.213; 0; gNB. 3; 9us sensing as defined in Clause x.x in TS 37.213; 0; UE. - Note: The last row in Table 1 is only applicable when the UE can operate as an initiating device as configured by gNB. - Note 1: The intention of Clause x.x above is to describe the LBT procedure from a UE perspective when this operates as initiating device. - Note 2: A UE operating as initiating device may transmit an UL transmission burst(s) within its u-FFP immediately after sensing the channel to be idle for at least a sensing slot duration if the gap between the UL transmission burst(s) and any previous transmission burst is more than Conclusion Any UL or DL transmission that is expected to occur, should be associated to a Channel Occupancy (CO) with a corresponding FFP. When a transmission is associated to a CO with a corresponding FFP: - The association of the transmission to a CO with corresponding FFP is based on either of the following assumption: - “Initiating COT”: This assumption implies that the transmission would initiate a CO corresponding the FFP. - “Sharing COT”: This assumption implies that the transmission would share a CO corresponding to the FFP. - The association assumption is validated as follows: - “Initiating COT” assumption is validated if the transmission would start at the FFP boundary and would end before idle period of the FFP. - “Sharing COT” assumption is validated if the transmission would start after the FFP boundary and would end before idle period of the FFP and the CO corresponding to the FFP is initiated. - A transmission based on a CO association assumption can occur if the CO association assumption is validated and if the following sensing conditions are met: - For CO association assumption as “Initiating COT”: - If a CCA is successful before the transmission. - For CO association assumption as “Sharing COT” - If the gap between the beginning of the transmission and the end of previous one sharing the same CO in that FFP is more than 16us and if a CCA is successful before the transmission. - IF the gap between the beginning of the transmission and the end of previous one sharing the same CO in that FFP is at most 16us Agreement In semi-static channel access mode, a DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP, shall include at least scheduled DL transmission or a DCI intended for the UE that initiated that FFP. - FFS whether/how the DL transmission burst can include transmission to any other UE in the cell than the COT initiating UE and/or broadcast transmission while ensuring that the COT initiated by the UE is not shared by any other UE in the cell for any UL transmission Agreement In semi-static channel access mode, when the gNB schedules by a DCI a UL transmission in a later g-FFP that is different from the g-FFP that carries the scheduling DCI: - The UE follows the indicated COT initiator as the following: - If the UE validates the indicated COT initiator assumption and satisfies the applicable sensing conditions, the transmission occurs. Otherwise, the transmission is dropped. Agreement - When a UE operates as an initiating device, and the gNB shares a UE’s FFP for DL transmission, regardless of the gap between any UL and DL bursts, no restriction is imposed on the maximum duration of each of the DL bursts such that each can continue until the UE FFP idle period starts. - Note: The applicability of the EDT calculation based on the UE’s transmit power to the UE COT initiation in accordance to the UL-DL gap duration and/or the content of the DL burst is separately discussed Conclusion There is no consensus in RAN1 to support UE-initiated COT for semi-static channel occupancy in IDLE/INACTIVE mode. Agreement Do not support PUSCH repetition Type B based on NR-U Rel-16 CG for unlicensed band operation.

RAN1#106bis-e (Oct. 2021)

Agreement In semi-static channel access mode, for PUSCH repetition Type B: If a nominal repetition overlaps with a set of symbols in an idle period associated to gNB’s FFP in case UE shares gNB-initiated COT for the nominal repetition or associated to UE’s FFP in case UE assumes UE-initiated COT for the nominal repetition, all the symbols in the idle period should be considered as invalid symbols which are not considered for an actual repetition as in Rel-16. - Segmentation before and/or after the idle period is applied when applicable. - FFS on impact of processing timeline for PUSCH on the UE behaviour Agreement In semi-static channel access mode, for PUSCH repetition Type B, orphan symbol(s) are dropped as in Rel-16 Agreement In semi-static channel access mode, the configuration of energy detection threshold to perform sensing at UE is based on maxEnergyDetectionThreshold. - That means that in semi-static channel access mode, configuration of ul-toDL-COT-SharingED-Threshold is not applicable. - As the consequence, energy detection threshold to perform sensing at UE is based on maxEnergyDetectionThreshold if maxEnergyDetectionThreshold is configured. Otherwise (i.e., if maxEnergyDetectionThreshold is not configured), energy detection threshold to perform sensing at UE is based on the UE maximum transmit power. Agreement Support configuration of harq-ProcID-Offset2 for operation in unlicensed spectrum when the cg-RetransmissionTimer-r16 is not configured. Agreement The following RRC parameters are NOT needed when cg-RetransmissionTimer is configured for CG operation with shared spectrum channel access. - pusch-RepTypeIndicator - startingFromRV0 Agreement The RRC parameter of phy-PriorityIndex is applicable for CG operation in unlicensed band. Agreement Introduce new RRC parameters ul-AccessConfigListDCI-0-2 and ul-AccessConfigListDCI-1-2 to support indication of CP extension, LBT type, and CAPC with DCI 0_2 and 1_2 with dynamic channel access. Agreement In semi-static channel access mode, a DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP, shall include scheduled DL transmission or a DCI intended for the UE that initiated that FFP. - A DL transmission to any other UE in the cell than the COT initiating UE and/or a broadcast transmission can be additionally included in the DL transmission burst if the gNB fulfils the following condition: - It is gNB‘s responsibility to ensure that other UEs do not assume gNB-initiated COT based transmission for a UL transmission based on the detection of any transmission in the DL transmission burst. Agreement In semi-static channel access mode for a UE which is allowed to operate as an initiating device, CG-StartingOffsets is not applicable. - Note: That is, CG-StaringOffsets is not applicable at all for a UE configured with UE FFP parameters (e.g. period, offset) regardless whether the UE would initiate its own COT or would share gNB’s COT. Agreement - When performing Intra-UE multiplexing procedure, if a PUCCH with HARQ-ACK overlaps with a CG-PUSCH and the cg-RetransmissionTimer is configured: - If the HARQ-ACK and the CG-PUSCH have the same priority and the CG-PUSCH is selected for HARQ-ACK multiplexing: - If cg-UCI-Multiplexing is enabled for that CG-PUSCH, HARQ-ACK would be multiplexed in CG-PUSCH. - Otherwise, CG-PUSCH would be dropped. - If the HARQ-ACK and the CG-PUSCH have different priority and the CG-PUSCH is selected for HARQ-ACK multiplexing: - If multiplexing HARQ-ACK on the CG-PUSCH with different priroity is not indicated, - The LP channel between PUCCH or CG-PUSCH would be dropped as in Rel-16. - If multiplexing HARQ-ACK on the CG-PUSCH with different priroity is indicated, - If cg-UCI-Multiplexing is enabled for that CG-PUSCH, HARQ-ACK would be multiplexed in CG-PUSCH. - Otherwise, the LP channel would be dropped.

RAN1#107-e (Nov. 2021)

Agreement In semi-static channel access mode, for a transmission burst that includes multiple transmissions, the associated COT-ownership for all transmissions in the transmission burst should be the same. Agreement In semi-static channel access mode, when a UE is enabled to initiate a channel occupancy: - If single DCI schedules multiple UL transmissions, the COT initiator assumption indicated by the single DCI is applied for all the UL transmissions scheduled by the single DCI. Agreement In semi-static channel access mode, when a DCI schedules a UL transmission in the same g-FFP and the UL transmission is not aligned with a u-FFP boundary and the DCI indicates UE initiated COT, the following are applied: - If the UE has initiated the COT in that u-FFP and satisfies the applicable sensing conditions, the UL transmission occurs. Otherwise, the UL transmission is dropped. Agreement The following channel access procedures for consecutive scheduled UL transmissions are applicable to the semi-static channel access mode. - If a UE is scheduled by a gNB to transmit a set of UL transmissions including PUSCH or SRS symbol(s) using a UL grant, the UE shall not apply a CP extension for the remaining UL transmissions in the set after the first UL transmission after accessing the channel. - If a UE is scheduled to transmit a set of consecutive UL transmissions without gaps including PUSCH using one or more UL grant(s), PUCCH using one or more DL grant(s), or SRS with one or more DL grant(s) or UL grant(s) and the UE transmits one of the scheduled UL transmissions in the set after accessing the channel, the UE may continue transmission of the remaining UL transmissions in the set, if any. - Note: The procedures above are based on description in Clause 4.2.1.0.1 of TS 37.213. Agreement In semi-static channel access mode, when the gNB schedules by a DCI a UL transmission and the scheduling DCI and the scheduled UL transmission are in a same g-FFP but on a different RB sets of the g-FFP bandwidth: - If DCI indicates gNB initiated COT, validation of the gNB-initiated COT (based on the detection of DL transmission from the gNB) for the RB sets with scheduled UL can be skipped. Agreement The symbol offset for the UE FFP configuration is determined based on the smallest SCS among configured SCSs in a serving cell. Conclusion PUSCH repetition Type B for DG on unlicensed spectrum in Rel-17 is supported. Conclusion In semi-static channel access mode when a UE can operate as an initiating device, for a configured UL transmission, the required time to determine whether the configured UL transmission could correspond to gNB’s COT or UE’s COT is up to UE implementation. Conclusion In semi-static channel access mode when a UE can operate as an initiating device, for a scheduled UL transmission, when the scheduling DCI and the first symbol of the scheduled UL transmission are in the same g-FFP, the processing time for the scheduled UL transmission satisfies the time required to the UE determine whether the scheduled UL transmission could correspond to the COT initiator assumption indicated in the DCI. Conclusion In semi-static channel access mode when a UE can operate as an initiating device, for a scheduled UL transmission, when the scheduling DCI and the first symbol of the scheduled UL transmission are in different g-FFPs, and if the DCI indicates gNB as the COT initiator: - the required time to determine whether the gNB had initiated a COT before the start of the scheduled UL transmission is up to UE implementation. Agreement For operation in a cell with shared spectrum access, a UE configured with multiple CG configurations does not expect to operate in the cell with more than one active CG configurations for which the cg-RetransmissionTimer is provided in one active CG configuration and not provided in another. - Note: That means that the UE operates with a same CG type (i.e., Rel-16 NR-U CG type or Rel-16 URLLC CG type per previous agreements) per cell in a shared spectrum. Agreement EnableConfiguredUL is not applicable if cg-RetransmissionTimer is not configured in Rel-17. Agreement In semi-static channel access mode, when the cg-RetransmissionTimer-r16 is enabled and a UE operates as an initiating device, the RRC parameter cg-COT-SharingList-16 is reused, and the UE is not expected to provide any relevant information related to CAPC to the gNB. - Channel Occupancy Time (COT) sharing information bit-field in CG-UCI is as the following: - log2C bits if higher layer parameter cg-COT-SharingList is configured, where C is the number of combinations configured in cg-COT-SharingList; - 0 bit otherwise;

RAN1#107bis-e (Jan. 2022)

Agreement In semi-static channel occupancy, the procedures for intra-period scheduled UL applies to cross-carrier scheduling case if the timing of the scheduled UL transmission and the corresponding scheduling DCI are confined within the same gNB period on the carrier on which the UL transmission is scheduled. Otherwise, the procedures for cross-period scheduled UL transmissions should apply for the cross-carrier scheduled UL transmission. Agreement Text proposal 2-2 in section 1.1.1 of R1-2200692 for Clause 4.3.1.2.4.1 and Clause 4.3.1.2.4.2 of TS 37.213 is endorsed for the editor’s CR. - Including eeditorial updates to change “carrier(s)” to “carrier” in several places that was missed. Potential remaining cases can be handled during implementation of editor’s CR. Agreement Text proposal 3-1 in section 1.1.1 of R1-2200692 for Clauses 4.3.1.2.1, 4.3.1.2.2, 4.3.1.2.4.1 and 4.3.1.2.4.2 of TS 37.213 is endorsed for the editor’s CR. Agreement Text proposal 3-2 in section 1.1.1 of R1-2200692 for Clauses 7.3.1.1 of TS 38.212 is endorsed for the editor’s CR. Agreement Text proposal 4-1 in section 1.1.1 of R1-2200692 for Clause 4.3.1.2.1 and Clause 4.3.1.2.2 of TS 37.213 is endorsed for the editor’s CR. Conclusion If UE-initiated COT in semi-static channel access mode is enabled for a UE, when operating on multiple LBT BWs on a carrier, the assumptions regarding the COT initiator for a configured UL transmission may not be aligned across all LBT BWs for the configured UL transmission Agreement The following TPs for TS37.213 are endorsed for the editor’s CR. ----------- TP#8-3: TS 37.213 Clause 4.3.1.2.1 --------------------- < Start of text proposal> ##### 4.3.1.2.1 Channel occupancy initiated by gNB and sensing procedures *** Unchanged text is omitted *** - If the gap between the UL transmission burst(s) and any previous DL transmission burst in that period is more than 16us, the UL transmission burst(s) may be transmitted if the channel is sensed to be idle for at least a sensing slot duration Tsl=9us within a 25us interval ending immediately before the UL transmission burst(s). *** Unchanged text is omitted *** ##### 4.3.1.2.2 Channel occupancy initiated by UE and sensing procedures *** Unchanged text is omitted *** - If the gap between the DL transmission burst(s) and any previous UL transmission burst in that period is more than 16us, the DL transmission burst(s) may be transmitted if the channel is sensed to be idle for at least a sensing slot duration Tsl=9us within a 25us interval ending immediately before the DL transmission burst(s). *** Unchanged text is omitted *** < End of text proposal>. Agreement Text proposal 6-4 (updated) in section 1.1.3 of R1-2200693 for Clause 4.3.1.2.3 and Clause 4.3.2 of TS 37.213 is endorsed for the editor’s CR. Agreement For semi-static channel access mode when a UE is enabled to initiate a COT, for a nominal repetition of a PUSCH transmission Type B across multiple LBT BWs, if the COT initiator assumption is not aligned among the LBT BWs, the nominal repetition is dropped if there are invalid symbols due to overlapping of the nominal repetition with an idle duration of either gNB’s FFP or UE’s FFP. Conclusion For semi-static channel access mode when a UE is enabled to initiate a COT, - For a scheduled UL transmission across multiple LBT BWs, the same COT initiator assumption is made across all of the LBT BWs. - For a UL transmission other than a nominal repetition of a PUSCH transmission Type B that is configured across multiple LBT BWs, if in an LBT BW of the UL transmission overlaps with an idle duration corresponding to a period that its corresponding COT is associated with, the UL transmission is dropped. Conclusion In semi-static channel access mode, when UE is enabled to initiate COT, UL transmission based on UE-initiated COT for random access procedures in RRC-connected mode is supported where the procedures for configured and scheduled UL transmissions are applicable. Agreement In semi-static channel access mode, when UE is enabled to initiate COT, - UL transmission based on UE-initiated COT for contention-free random access procedure in RRC-connected mode is allowed for PRACH and corresponding PUSCH transmissions. - RAR grant corresponding to the PRACH transmission for the contention-free random access procedure indicates the COT initiator associated with the PUSCH transmission scheduled by the RAR grant based on Table 7.3.1.1.1-4A of TS 38.212. - UL transmission based on UE-initiated COT for contention-based random access procedure in RRC-conneted mode is not allowed. Agreement Text proposal 8-2(updated2) in section 1.1.3 of R1-2200693 for Clause 4.3.1.2.1 of TS 37.213 is endorsed for the editor’s CR. Agreement The text proposals captured in in section 1.1.3 of R1-2200693 as Proposal 1-1 and Proposal 1-2 are endorsed for the editor’s CR for TS37.213. Agreement For semi-static channel access mode when a UE is enabled to initiate a COT, if a nominal repetition of a configured PUSCH transmission repetition Type B shares a gNB-initiated COT and overlaps with an idle duration of the corresponding gNB’s FFP as well as an idle duration of a UE’s FFP where the UE has initiated the corresponding COT, the configured UL transmission is dropped.

# Intra-UE multiplexing & priorization enh. (AI 8.3.3)

## 5.1 Rel-17 Intra-UE multiplexing & prioritization framework

RAN1#106-e (Aug. 2021)

Working Assumption For handling overlapping PUCCHs/PUSCHs with different priorities in R17 - Step 1: Resolve overlapping PUCCHs and/or PUSCHs with the same priority - Step 2: Resolve overlapping PUCCHs and/or PUSCHs with different priorities Note: Avoid recursive pseudo-code to implement this procedure Note: It is expected that Rel-15 intra-UE UCI multiplexing timeline will be applicable

RAN1#106bis-e (Oct. 2021)

Agreement The following working assumption is confirmed. For handling overlapping PUCCHs/PUSCHs with different priorities in R17 - Step 1: Resolve overlapping PUCCHs and/or PUSCHs with the same priority - Step 2: Resolve overlapping PUCCHs and/or PUSCHs with different priorities Note: Avoid recursive pseudo-code to implement this procedure Note: It is expected that Rel-15 intra-UE UCI multiplexing timeline will be applicable Agreement For both the subslot-based PUCCH and slot-based PUCCH, if simultaneous PUCCH/PUSCH transmission is not enabled, reuse Rel-16 procedure for Step 1

RAN1#107-e (Nov. 2021)

Agreement For handling overlapping PUCCHs/PUSCHs with different priorities, Step 2 consists of the following sub-steps: - Step 2.1: Resolve collision of LP PUCCHs and HP PUCCHs. - Step 2.2: Resolve collision of PUCCHs and PUSCHs of different priorities. Agreement If multiplexing of PUCCHs and/or PUSCHs with different priorities is enabled by RRC, support both of the following UE capabilities to resolve collision of PUCCHs and/or PUSCHs with different priorities in step 2: - Capability #1: It is not expected that Rel-15 multiplexing timeline is not met for all overlapping channels [FFS the overlapping channels are resultant channels after step 1]. UE performs multiplexing or dropping of PUCCHs and/or PUSCHs with different priorities according to Rel-17 rules. - Dynamic enabling/disabling multiplexing for different priorities is not supported for Capability #1 - (Working assumption) Capability #3: Rel-17 multiplexing for different priorities is dynamically enabled/disabled in step 2. - Dynamic indication of enabling/disabling multiplexing for different priorities can be enabled only if multiplexing of PUCCHs/PUSCHs with different priorities is enabled by RRC configuration. - If dynamic multiplexing for different priorities is indicated as enabled for a PUCCH / PUSCH, the UE performs Rel-17 multiplexing operation using the Rel-15 timeline - The gNB is responsible to ensure that all the DCIs associated with all overlapping channels involved in multiplexing in step 2 meet the Rel-15 timeline for multiplexing. - If dynamic multiplexing for different priorities is indicated as disabled for a PUCCH / PUSCH, the UE does not apply the Rel-17 intra-UE multiplexing - If the UL channel associated with the DCI disabling multiplexing collides with another UL channel of a different priority, UE performs R16 PHY prioritization, using Rel-16 timeline. The gNB is responsible to ensure that the UE meets R16 PHY prioritization timeline. - If the UL channel associated with the DCI disabling multiplexing does not collide with another UL channel of a different priority, UE transmits the UL channel as is. - FFS: whether the UL channel associated with the DCI disabling multiplexing can collide with another UL channel of a same priority. - UE does not expect to receive a dynamic indication resulting in demultiplexing of previously multiplexed PUCCHs/PUSCHs channels after the Rel-15 multiplexing deadline has passed - FFS: UE does not expect to receive a dynamic indication resulting in demultiplexing of previously multiplexed PUCCHs/PUSCHs channels without any associated DCIs - Note: demultiplexing of two previously multiplexed channels means decoupling two channels already multiplexed, dropping one channel, and multiplexing the other channel with another channel(s). The above behaviors of Capability#3 at least apply to resolving collision of two UL channels resulting from Step 1 with different priorities. FFS: more than two UL channels. - FFS whether dynamic indication in multiple DCIs associated with a group of overlapping channels have to be consistent - FFS: Configuration of prioritization / multiplexing of channels without dynamic indication - Note: Capability 3 procedure is a super-set of Capability 1 procedure - FFS: Time unit to apply Rel-15 timeline (e.g. slot based, sub-slot based) - FFS: The set of PUSCH and PUCCH that eligible for Rel-15 multiplexing consideration Note: “collision” refers to overlapping PUCCHs, overlapping PUCCH and PUSCH (excluding PUSCH supporting simultaneous transmission with PUCCH), overlapping PUSCHs on a same cell. Note: “Rel-15 multiplexing timeline” means Rel15 timeline calculation in Rel-16 spec, including all the formula and all the values for the variables Note: “Rel-16 prioritization timeline” means Rel-16 cancellation timeline calculation in Rel-16 spec, including all the formula and all the values for the variables

RAN#94-e (Dec. 2022)

RAN to guide RAN1 to focus on the discussion on Capability#1 only in Q1 2022 for Rel-17 intra-UE multiplexing framework

RAN1#107bis-e (Jan. 2022)

Conclusion For resolving collision of PUCCHs and/or PUSCHs with different priorities in step 2, a resultant PUCCH with HP and LP UCI is not expected to be overlapped with a HP PUCCH. - FFS whether a resultant PUCCH with HP and LP UCI can be overlapped with a HP PUSCH. Agreement For resolving collision of LP PUCCHs and HP PUCCHs in step 2.1, a HP PUCCH with HARQ-ACK is not expected to be overlapped with multiple LP PUCCHs with HARQ-ACK. - It’s up to the editor whether/how to capture this Agreement For resolving collision of PUCCHs and PUSCHs of different priorities in step 2.2, LP PUSCH(s) overlapping with HP PUCCH which carries positive SR are dropped. Working Assumption For resolving collision of PUCCHs and PUSCHs of different priorities in step 2.2, LP PUSCH(s) overlapping with HP PUCCH which carries positive SR are dropped before UCI multiplexing. - Step 1.2 behavior is not affected by the above Agreement For resolving collision of PUCCHs with different priorities in step 2.1, if resultant PUCCH with HP and LP UCI collides with LP PUCCH without HARQ ACK, the LP PUCCH is dropped. - A resultant PUCCH with HP and LP UCI is not expected to be overlapped with a LP PUCCH with HARQ-ACK. Agreement A UE does not expect to multiplex in a PUSCH transmission HARQ-ACK information that the UE would transmit in different PUCCHs of a same priority. - The above is considered an error case Agreement For resolving collision of PUCCHs of different priorities without repetitions within a time unit, Step 2.1 consists of the following sub-steps: - Step 2.1-1: Determine a reference PUCCH resource - Step 2.1-2: Select O PUCCH resource(s) overlapping with the reference PUCCH resource. - Step 2.1-3: Apply Rel-17 intra-UE multiplexing/dropping rules to resolve overlapping among the reference PUCCH resource and O PUCCH resource(s). - Step 2.1-4: Loop Step 2.1-1) ~ Step 2.1-3) until there are no overlapping PUCCHs in the time unit. FFS details Agreement For resolving collision of PUCCHs of different priorities without repetition within a time unit, down-select from the following options: - Option 1: - The reference PUCCH resource is determined as in Rel-15, i.e. based on the starting symbol and duration - In step 2.1-2, select up to one PUCCH resource overlapping with the reference PUCCH resource according to Rel-15 pseudo code - Option 2: - The reference PUCCH resource is determined as in Rel-15, i.e. based on the starting symbol and duration - In step 2.1-2, select all the PUCCH resources overlapping with the reference PUCCH resource according to Rel-15 pseudo code - Option 3: - The reference PUCCH resource is determined by prioritizing HP PUCCH over LP PUCCH on top of Rel-15 rules - In step 2.1-2, select all the PUCCH resources overlapping with the reference PUCCH resource according to Rel-15 pseudo code - Option 4: - The reference PUCCH resource is determined by prioritizing LP PUCCH carrying HARQ-ACK on top of Rel-15 rules - In step 2.1-2, If a LP PUCCH carrying HARQ-ACK overlaps with multiple HP PUCCHs and one of the HP PUCCH includes HARQ-ACK, only select the HP PUCCH including HARQ-ACK in step 2.1-2; otherwise, select all the PUCCH resources overlapping with the reference PUCCH resource according to Rel-15 pseudo code FFS: Details on time units for all options Working Assumption For resolving collision of PUCCHs of different priorities without repetition within a time unit, the time unit of HP HARQ-ACK is used. For a LP PUCCH overlapping with multiple time units, down-select from: - Alt. 1: the LP PUCCH is associated with the first time unit with overlapping HP PUCCH(s) - Alt. 2: the LP PUCCH is associated with the first time unit with overlapping HP PUCCH with HARQ-ACK if any. Otherwise, the LP PUCCH is associated with the first time unit with overlapping HP PUCCH(s). - Alt. 3: the LP PUCCH is associated with the last time unit with overlapping HP PUCCH(s) Agreement To apply Rel-17 intra-UE multiplexing/dropping rules to resolve overlapping among the reference PUCCH resource and O PUCCH resource(s) in step 2.1-3, LP PUCCH(s) without HARQ ACK are dropped before multiplexing if multiplexing is to be performed. Conclusion A resultant PUCCH with HP and LP UCI overlapping with a HP PUSCH is considered an error case Agreement For resolving collision of PUCCHs and PUSCHs of different priorities in step 2.2, Rel-15/16 rule is reused for PUSCH selection for HARQ ACK multiplexing - FFS: Whether/how dropping is performed before UCI multiplexing - Note: The priorities of PUCCH and PUSCH candidates for multiplexing in step 2.2 are different Conclusion A UE is not expected to be enabled with prioritizationBetweenLP-DG-PUSCHandHP-CG-PUSCH or prioritizationBetweenHP-DG-PUSCHandLP-CG-PUSCH for a cell group if UCI-MuxWithDifferentPriority or UCI-MuxWithDifferentPriority-secondaryPUCCHgroup is enabled for the same cell group.

## 5.2 Overlapping CG/DG PUSCH of different priorities

RAN1#102-e (Aug. 2020)

Agreements: Support PHY prioritization for the case where low-priority DG-PUSCH collides with high-priority CG-PUSCH in R17. - FFS details - Clarify R16 baseline if needed.

RAN1#103-e (Oct./Nov. 2020)

Agreements: Support PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on a BWP of a serving cell in R17. - FFS the related cancelation behavior for the PUSCH of lower PHY priority and other details. - First clarify what is the scope of this feature, e.g. if overlapping between more than 2 channels is considered. - FFS the timeline requirements. - First clarify what is the behavior of Rel-16 UE in case of DG/CG/UCI overlapping, with and without uplink skipping enabled. - FFS UE capability for this feature. - Note: The main bullet has been agreed in the WID by RAN Plenary.

RAN1#106bis-e (Oct. 2021)

Agreement For collision between HP CG PUSCH and LP DG PUSCH, if MAC delivers two MAC PDUs to PHY, PHY layer can make the prioritization so that the UE is expected to transmit the CG PUSCH and cancel the DG PUSCH at latest from the first symbol that is overlapping with the CG PUSCH. - Note: For the DG PUSCH, it is up to UE implementation to handle OFDM symbols of the DG PUSCH before the start of HP CG PUSCH which are nonoverlapping with the HP CG PUSCH. - FFS: How to handle the collision when there is repetition for CG and/or DG PUSCH

RAN1#107-e (Nov. 2021)

Agreement For collision of LP DG-PUSCH and HP CG-PUSCH of different priorities, the cancellation is applied per actual repetition, if LP DG-PUSCH and/or HP CG-PUSCH is repeated. Agreement For the overlapping between LP CG and HP DG, if MAC delivers two MAC PDUs to PHY, PHY layer can make the prioritization so that the UE is expected to cancel the overlapping low priority CG PUSCH by the first overlapping symbol at the latest. - On top of Rel-16 cancellation time (N2+d1) for PUCCH/PUCCH or PUCCH/PUSCH collision, additional time d3 is needed (which results N2+d1+d3 in total cancellation time) for LP CG-PUSCH and HP DG-PUSCH collision resolution. - (Working assumption) d3 = {0, 1,…,2μ+1}symbol(s) upon UE capability report, where μ=0,1,2,3 for SCS=15/30/60/120kHz, respectively. Agreement For collision of HP DG-PUSCH and LP CG-PUSCH, the cancellation is applied per actual repetition, if HP DG-PUSCH and/or LP CG-PUSCH is repeated.

RAN1#107bis-e (Jan. 2022)

Agreement The following working assumption is confirmed For the overlapping between LP CG and HP DG, if MAC delivers two MAC PDUs to PHY, PHY layer can make the prioritization so that the UE is expected to cancel the overlapping low priority CG PUSCH by the first overlapping symbol at the latest. - On top of Rel-16 cancellation time (N2+d1) for PUCCH/PUCCH or PUCCH/PUSCH collision, additional time d3 is needed (which results N2+d1+d3 in total cancellation time) for LP CG-PUSCH and HP DG-PUSCH collision resolution. - d3 = {0, 1,…,2μ+1}symbol(s) upon UE capability report, where μ=0,1,2,3 for SCS=15/30/60/120kHz, respectively. Agreement Introduce RRC parameters to enable the UE handling for overlapping CG/DG PUSCH of different priorities, i.e., keep the yellow marked related RRC parameters in rows 68 and 69 from the IIoT&URLLC RRC parameter sheet from R1-2112979.

## 5.3. Intra-UE multiplexing enhancements of different priorities (on PUCCH & PUSCH)

RAN1#102-e (Aug. 2020)

Agreements: Support multiplexing for following scenarios in R17: - Multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH in R17. - Multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations). - Multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH. For the above multiplexing scenarios, - FFS conditions, if needed, for the multiplexing, e.g - Whether to support multiplexing between different resources not confined within a sub-slot. - Whether to support multiplexing in case a PUCCH overlaps with more than one PUCCH. - Timeline requirements. - FFS: details, if needed, of the multiplexing scheme, e.g. - How to minimize impact on the latency for high-priority HARQ-ACK. - How to determine the PUCCH resource used for multiplexing (e.g. HP or LP PUCCH resource, or a dedicated PUCCH resource for the multiplexing). - How to multiplex the HARQ-ACK bits (e.g. multiplexing, bundling). - How to encode the UCIs with different priorities (e.g. separate coding vs. joint coding) - How to guarantee the target code rate (e.g. payload control, multiplexing priority, LP HARQ-ACK compression/compaction). - Explicit indication for enabling multiplexing. - Multiplexing rule and order (e.g. HP/LP multiplexing is after resolving collision within the same priority). Agreements: Support multiplexing for following scenarios in R17: - Multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). - Multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only) - Multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. - Multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. For the above multiplexing scenarios, - Support separate configurations of at least beta-offset values (FFS for alpha) for multiplexing with different priority combinations. - FFS for other separate configurations. - FFS: value range of beta-offset (e.g. <1). - FFS the conditions, if needed, for multiplexing, e.g. - FFS: Whether to support multiplexing in case a PUCCH/PUSCH overlaps with more than one PUCCH/PUSCH. - Timeline requirements. - FFS: details, if needed, of the multiplexing scheme, e.g. - How to minimize impact on the latency for high-priority HARQ-ACK. - How to multiplex the HARQ-ACK bits (e.g. multiplexing, bundling)? - How to encode the UCIs with different priorities (e.g. separate coding vs. joint coding). - How to guarantee the target code rate (e.g. payload control, multiplexing priority, LP HARQ-ACK compression/compaction). - Explicit indication for multiplexing. - Multiplexing rule and order (e.g. HP/LP multiplexing is after resolving collision within the same priority). - How to handle multiplexing of UCI of different priorities and CG-UCI in a CG-PUSCH

RAN1#103-e (Oct./Nov. 2020)

Agreements: For multiplexing UCIs of different priorities in a PUCCH in R17, - Support of multiplexing between different resources not confined within a sub-slot if conditions are met - FFS: Details - Support multiplexing in case a PUCCH overlaps with more than one PUCCH if conditions are met - FFS details Agreements: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits are more than 2 bits, down-select from the following options in RAN1#104-e: - Option 1: Support joint coding. - Option 2: Support separate coding. - Option 3: Combination of Option1 and 2. - FFS the details For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is 2 bits, provide design details for decision for the following cases in RAN1#104-e: - Multiplexing on a PUCCH format 0 - Multiplexing on a PUCCH format 1 Agreements: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, support a mechanism for gNB to enable/disable the multiplexing. - FFS the type of the mechanism, e.g. DCI indication and/or RRC configuration - FFS: Interaction between the enable/disable mechanism and other multiplexing conditions - FFS for other types of UCI. Agreements: For HARQ-ACK multiplexing on PUSCH of different priority in R17, support a mechanism for gNB to enable/disable the multiplexing. - FFS the type of the mechanism, e.g. DCI indication and/or RRC configuration, beta_offset=0 - FFS: Interaction between the enable/disable mechanism and other multiplexing conditions - FFS for other types of UCI.

RAN1#104-e (Jan/Feb. 2021)

Agreements: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, - Use a PUCCH resource in the second PUCCH-Config (the PUCCH-config containing the PUCCH resource of the HP HARQ-ACK) at least in case the total number of LP and HP HARQ-ACK bits is more than 2. - FFS: The PUCCH resource is configured dedicated for multiplexing of HP HARQ-ACK and LP HARQ-ACK. - FFS in case the total number of LP and HP HARQ-ACK bits is 2. - FFS details Working assumption: Reuse Rel-15 intra-UE PUCCH/PUSCH multiplexing timeline requirements for Rel-17 intra-UE PUCCH/PUSCH multiplexing with different priorities - FFS whether or not to specify a different behavior than Rel-15 when the timeline requirements are not met Agreements: For multiplexing LP HARQ-ACK in a HP PUSCH, support 0< beta-offset <1. - FFS value(s) - FFS to additionally support beta-offset =0 or a value disabling the multiplexing - Aim to NOT increase the corresponding bitwidth in the DCI (compared to Rel-16) Agreements: When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, further study the following options (proponents are encouraged to provide more details and analysis):  Opt.1: The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource.  Opt.1a: The UE does not transmit negative SR.  Opt.1b: For negative SR, the UE transmit only HARQ-ACK on the HARQ-ACK resource.  Opt.1c: For negative SR, the UE transmits SR and HARQ-ACK on the SR resource  FFS: whether with power boost to transmit multiplexed payload or not.  Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource.  Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource.  Opt.2b: Using 4 CS values as for SR+1-bit HARQ-ACK in Rel-15/16. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit.  Opt.2c: If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.  Opt.3: No enhancement over Rel-16.  Other options not excluded.  FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK? Agreements: When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF1, further study the following options (proponents are encouraged to provide more details and analysis):  Opt.1: The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource.  Opt.1a: The UE does not transmit negative SR.  Opt.1b: For negative SR, the UE transmit only HARQ-ACK on the HARQ-ACK resource.  Opt.1c: For negative SR, the UE transmits SR and HARQ-ACK on the SR resource  FFS: whether with power boost to transmit multiplexed payload or not.  Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource.  Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource.  Opt.2b: Applying QPSK for SR+1-bit HARQ-ACK. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit.  FFS on conditions of multiplexing.  Opt.3: For positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource.  Opt.4: For positive SR, transmit SR on the SR resource and drop HARQ-ACK. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource.  Opt.5: No enhancement over Rel-16.  Other options not excluded.  FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK? Agreements: When a PUCCH carrying HP SR with PF1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, further study the following options (proponents are encouraged to provide more details and analysis):  Opt.1: The SR and HARQ-ACK are multiplexed and transmitted on the SR resource.  Opt.1a: For positive SR, the UE transmits the PUCCH in the resource using PUCCH format 1 for SR. The value of cyclic shift of sequence, i.e., , of this PUCCH format 1 is determined by HARQ-ACK, and the bit, i.e., b(0), of this PUCCH format 1 is determined by SR. For negative SR, the UE transmits only a PUCCH with HARQ-ACK information and drops the PUCCH with negative SR.  Opt.1b: SR and HARQ-ACK are multiplexed and modulated to be transmitted on the SR resource  Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource.  Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource.  Opt.2b: Using 4 CS values as for SR+1-bit HARQ-ACK in Rel-15/16. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit.  Opt.2c: If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.  Opt.2d: HP SR and LP HARQ-ACK are multiplexed by the Rel-15 cyclic shift only if latency requirement for HP SR is met. Otherwise, drop the LP HARQ-ACK and only transmit the HP SR on its resource.  Opt.3: For positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource.  Opt.4: No enhancement over Rel-16.  Other options not excluded.  FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK?

RAN1#104bis-e (April 2021)

Agreements: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is more than 2, support separate coding for the two HARQ-ACKs. - FFS for HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s). - (working assumption) Drop CSI (including part 1 and part2, if exist) if CSI would multiplex on a PUCCH which has HP A/N. - FFS Strive to let HP A/N reuse the encoder, rate matching equation, and RE mapping rules in Rel-15 for A/N+CSI-1. - FFS Strive to let LP A/N reuse the encoder, rate matching equation, and mapping rules in Rel-15 for CSI-2. Agreement: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, support separate coding for the two HARQ-ACKs. - It is understood that it is intended that the number of encoding chains for all UCI multiplexing combinations in Rel-17 should not exceed that in Rel-15/16.

RAN1#105-e (May 2021)

Agreement: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is more than 2, - For HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s), support separate coding. Down-select from the two options: - Option 1: Reuse R15 TS 38.212 Clause 5.3.3.1 for 1-bit. Reuse R15 TS 38.212 Clause 5.3.3.2 for 2-bit. - Option 2: Reuse R15 TS 38.212 Clause 5.3.3.3, i.e., padding to 3 bits and using RM coding. - For HP HARQ-ACK or LP HARQ-ACK >2 bit(s), HP HARQ-ACK and LP HARQ-ACK are separately encoded according to R15 TS 38.212 Clause 5.3.3.3 or Clause 5.3.1. - FFS rate matching equation and RE mapping rules for PF2/3/4. Rel-15 is baseline if available. Agreement: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is 2, treat the two bits as HARQ-ACK bits with High priority. · Rel-15 design (for PF0 and PF1) is baseline. · Note: QC has strong concern on above scheme. The scheme cannot provide unequal error protection between the HP bit and LP bit hence could suffer from performance degradation for the HP bit. QC accept the scheme for the sake of progress in RAN 1 with the concern on the performance reserved.

RAN1#106-e (Aug. 2021)

Agreement For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, - HP A/N reuses rate matching equation, and RE mapping rules in Rel-15 for A/N+CSI-1. - LP A/N reuses rate matching equation, and RE mapping rules in Rel-15 for CSI-2. Above applies at least for PUCCH format 3 and 4. Agreement For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, an additional maxCodeRate for LP HARQ-ACK can be configured in the second PUCCH-Config per PUCCH format. Agreement In NR Rel-17, [at least] 2 new set of beta offset values can be configured to the UE to indicate separate beta_offset values for the following cases: - Multiplexing LP HARQ-ACK on HP PUSCH - Multiplexing HP HARQ-ACK on LP PUSCH Agreement For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, - PUCCH resource set determination is based on: UCI payload size = the number of HP UCI bits + the number of LP UCI bits. - FFS PRB number determination for HP A/N and LP A/N, e.g. based on their coding rates. - FFS the impact to the number of LP UCI bits due to missed DCI and potential solutions - Note: the number of LP UCI bits in the above agreement does may not necessarily mean the actual number of LP UCI bits until the second FFS is resolved

RAN1#106bis-e (Oct. 2021)

Agreement For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, in case the total number of LP and HP HARQ-ACK bits is 2: - Use a PUCCH resource in the second PUCCH-Config (the PUCCH-config containing the PUCCH resource of the HP HARQ-ACK). Agreement For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, if HP HARQ-ACK and LP HARQ-ACK would be transmitted on HP/LP PUSCH without CSI, - HP HARQ-ACK and LP HARQ-ACK are separately encoded according to R15 TS 38.212 Clause 5.3.1 and Clause 5.3.3. - Reuse R15 HARQ-ACK rate matching/puncturing and RE mapping for HP HARQ-ACK in principle. FFS details. - For LP HARQ-ACK, reuse R15 Part 1 CSI rate matching and RE mapping. Agreement For determining the PUCCH resource to carry the multiplexed high-priority and low-priority HARQ-ACKs, - The number of RBs for multiplexing HP HARQ-ACK and LP HARQ-ACK on a PUCCH format 3 is determined as following: - If OHP_UCI+OCRC, HP_UCIrHP_UCI∙Qm+OLP_UCI+OCRC, LP_UCIrLP_UCI∙Qm ≤MRBPUCCH∙Nsc, ctrlRB∙Nsymb-UCIPUCCH, the minimum number of RBs is determined as the number of MRB, minPUCCH, satisfying OHP_UCI+OCRC, HP_UCIrHP_UCI∙Qm+OLP_UCI+OCRC, LP_UCIrLP_UCI∙Qm≤MRB,minPUCCH∙Nsc, ctrlRB∙Nsymb-UCIPUCCH and OHP_UCI+OCRC, HP_UCIrHP_UCI∙Qm+OLP_UCI+OCRC, LP_UCIrLP_UCI∙Qm>MRB,minPUCCH-1∙Nsc, ctrlRB∙Nsymb-UCIPUCCH. - Note: rHP_UCI∙rLP_UCI∙Qm is multiplied at both sides to avoid mismatch between gNB and UE due to floating point operation. Editor to capture as suggested. - Otherwise, - Alt1: the number of RBs is MRBPUCCH. FFS: Whether/How LP HARQ-ACK is dropped. - Alt2: the number of RBs is determined by HP ACK payload size. LP HARQ-ACK is fully dropped. - Other alternatives are not precluded. - r_HP_UCI is maxCodeRate configured for HP bits and r_LP_UCI is maxCodeRate configured for LP bits in the second PUCCH-Config (the PUCCH-config containing the PUCCH resource of the HP HARQ-ACK). - FFS whether more than one maxCodeRate can be configured for one priority. - If MRB, minPUCCH is not equal to according to [4, TS 38.211], MRB, minPUCCH is increased to the nearest allowed value of nrofPRBs for PUCCH-format3 provided by the second PUCCH-Config [12, TS 38.331]. - HP coded bits and LP coded bits are not transmitted using the same RE(s) - FFS for PUCCH format 2.

RAN1#107-e (Nov. 2021)

Agreement For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, if HP HARQ-ACK, LP HARQ-ACK, and LP CSI consisting of two parts would be transmitted on LP PUSCH conveying UL-SCH, - The CSI part 2 is dropped. - Reuse R15 HARQ-ACK rate matching/puncturing and RE mapping for HP HARQ-ACK in principle. FFS details. - Reuse R15 CSI part 1 rate matching and RE mapping for LP HARQ-ACK. - Reuse R15 CSI part 2 rate matching and RE mapping for LP CSI part 1. - FFS for LP CSI consisting of single part. Note: Apple raised concern on CSI being dropped unnecessarily which could cause performance and degrade usefulness of URLLC enhancement. Agreement For determining the PUCCH resource to carry the multiplexed high-priority and low-priority HARQ-ACKs, if OHP_UCI+OCRC, HP_UCIrHP_UCI∙Qm+OLP_UCI+OCRC, LP_UCIrLP_UCI∙Qm >MRBPUCCH∙Nsc, ctrlRB∙Nsymb-UCIPUCCH - The number of RBs is MRBPUCCH. Then follow Rel-15 procedure, i.e., LP HARQ-ACK is mapped to the rest REs after HP HARQ-ACK. Agreement For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, - At least for PUCCH format 3/4, use the HP UCI bit number and HP RE number for ∆TF,b,f,c(i)∆TF, b, f,ci formula selection and calculation - For PUCCH format 1, use the total UCI bit number for ∆TF,b,f,c(i) calculation. - FFS for PUCCH format 2.

RAN1#107bis-e (Jan. 2022)

Agreement When a PUCCH carrying HP SR with PF0/1 overlaps with a PUCCH carrying LP HARQ-ACK with PF2/3/4: - For positive SR, transmit SR on the SR PUCCH resource and drop HARQ-ACK. - For negative SR, transmit HARQ-ACK only on the HARQ-ACK PUCCH resource. Note: It was agreed to support multiplexing a LP HARQ-ACK and a HP SR into a PUCCH for some HARQ-ACK/SR PF combinations in Rel-17. Agreement - For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a low-priority (LP) PUSCH in R17, if HP HARQ-ACK, LP HARQ-ACK, and LP CSI consisting of two parts would be transmitted on LP PUSCH not conveying UL-SCH, UE follows the same behaviour as that in case of PUSCH conveying UL-SCH. - For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a high-priority (HP) PUSCH in R17, if HP HARQ-ACK, LP HARQ-ACK, and HP CSI consisting of two parts would be transmitted on HP PUSCH not conveying UL-SCH, UE follows the same behaviour as that in case of PUSCH conveying UL-SCH. Agreement The following TP to remove the restriction of disallowing the collision between HP SPS HARQ-ACK with LP PUCCH/PUSCH is endorsed for the editor’s CR on TS38.213. ------------------ Text Proposal for 38.213 Section 9 ------------------ A UE does not expect to be scheduled to transmit a PUCCH or a PUSCH with smaller priority index that would overlap in time with a PUCCH of larger priority index with HARQ-ACK information only in response to a PDSCH reception without a corresponding PDCCH unless the UE is provided UCI-MuxWithDifferentPriority. A UE does not expect to be scheduled to transmit a PUCCH of smaller priority index that would overlap in time with a PUSCH of larger priority index with SP-CSI report(s) without a corresponding PDCCH.. Agreement Support multiplexing of high-priority HARQ-ACK and low-priority HARQ-ACK on PUCCH Format 2. - Extend legacy agreements on PRB number determination for Rel-17 (RAN1#106bis-e and RAN1#107-e) to cover PUCCH Format 2. - Use the HP UCI bit number and HP RE number for ∆TF,b,f,c(i) formula selection and calculation (as for PUCCH formats 3 & 4). - Concatenate the coded HP HARQ-ACK bits and the coded LP HARQ-ACK bits sequentially and apply the procedures described in R15 TS 38.211 to the concatenated coded HARQ-ACK bit sequence. Agreement When a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 2/3/4 overlaps with a PUCCH carrying LP HARQ-ACK, information bits for K HP SRs are appended to HP HARQ-ACK bits, and treat them as HP UCI, where K (K≥1) PUCCHs semi-statically configured for K HP SRs overlap with the original PUCCH carrying the HP HARQ-ACK. - The number of HP UCI bits is , same as Rel-15; - FFS: PF0, PF1 - Reuse other procedures for multiplexing of LP HARQ-ACK and HP HARQ-ACK on PUCCH resource with PF 2/3/4, i.e. separate coding, PRB determination, rate matching and power control. - If the HP HARQ-ACK is a dynamic HARQ-ACK, a PUCCH resource indicated by PRI is used for multiplexing. - If the HP HARQ-ACK is a SPS HARQ-ACK, a PUCCH resource determined from the PUCCH resource(s) provided by sps-PUCCH-AN-List is used for multiplexing. Agreement Introduce separate RRC parameters to configure ‘Multiplexing UCIs of different priorities on PUCCH or PUSCH’ in the primary and secondary PUCCH cell group. Agreement Define a new table for beta-offset values <1. - FFS for the values with the starting point as below. [0.8]. [0.64]. [0.5]. [0.4]. [0.32]. [0.25]. [0.2]. [0.1]. Agreement For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a LP PUSCH in R17, - If HP HARQ-ACK, LP HARQ-ACK, and LP CSI including a single part would be transmitted on LP PUSCH, - Reuse Rel-15 HARQ-ACK rate matching/puncturing and RE mapping for HP HARQ-ACK. - Reuse Rel-15 CSI part 1 rate matching and RE mapping for LP HARQ-ACK. - Reuse Rel-15 CSI part 2 rate matching and RE mapping for the single part of LP CSI. Agreement For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a HP PUSCH in R17, - If HP HARQ-ACK, LP HARQ-ACK, and HP CSI including a single part would be transmitted on HP PUSCH, - Reuse Rel-15 HARQ-ACK rate matching/puncturing and RE mapping for HP HARQ-ACK. - Reuse Rel-15 CSI part 1 rate matching and RE mapping for the single part of HP CSI. - Reuse Rel-15 CSI part 2 rate matching and RE mapping for LP HARQ-ACK. Agreement For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is more than 2, for HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s), support separate coding - Option 1: Reuse R15 TS 38.212 Clause 5.3.3.1 for 1-bit. Reuse R15 TS 38.212 Clause 5.3.3.2 for 2-bit. Apply the Rel-15 placeholder bit handling procedure for PUSCH together with Rel-15 PUCCH scrambling sequence. Agreement In R17, if HP HARQ-ACK, LP HARQ-ACK and HP CSI consisting of two parts would be transmitted on HP PUSCH, - LP HARQ-ACK is dropped. - Reuse R15 HARQ-ACK rate matching/puncturing and RE mapping for HP HARQ-ACK.

## 5.4 	Simultaneous PUCCH/PUSCH transmissions

RAN1#102-e (Aug. 2020)

Agreements: Support simultaneous PUCCH/PUSCH transmissions on different cells at least for inter-band CA. - FFS how to trigger this function. - FFS for intra-band CA.

RAN1#104-e (Jan/Feb. 2021)

Agreements: Per UE with the capability of inter-band CA, simultaneous PUCCH/PUSCH transmission of different PHY priorities over different cells can be RRC configured within the same PUCCH group - FFS: dynamic indication

RAN1#106-e (Aug. 2021)

Conclusion Simultaneous PUCCH/PUSCH transmission on the same cell is not supported in Rel-17.

RAN1#107-e (Nov. 2021)

Conclusion There is no consensus in RAN1 to support simultaneous PUCCH/PUSCH transmission of same priority over different cells in Rel-17. Conclusion There is no consensus in RAN1 to support simultaneous PUCCH/PUSCH transmissions on different cells for intra-band CA in Rel-17.

RAN1#107bis-e (Jan. 2022)

Agreement simultaneousPUCCH-PUSCH-secondaryPUCCHgroup is supported to enable simultaneous PUCCH and PUSCH transmissions with different priorities within the secondary PUCCH cell group separately from primary PUCCH cell group. Agreement If the simultaneous PUCCH/PUSCH transmission is enabled, a PUSCH that can be simultaneously transmitted with a PUCCH is excluded from overlapping channels for multiplexing the UCI of the PUCCH and for intra-UE prioritization with the PUCCH. - Note: For intra-UE multiplexing, above is for step 2-2. For intra-UE prioritization, above is applied after step 1. - FFS: How to capture this in the specifications Conclusion If the simultaneous PUCCH/PUSCH transmission is enabled, the timeline conditions of intra-UE multiplexing/prioritization of PUCCHs and PUSCHs with different priorities is not applicable to a PUSCH that can be simultaneously transmitted with a PUCCH.

# Propagation delay compensation (AI 8.3.4)

RAN1#102-e (Aug. 2020)

Agreements: - Take the following use cases as the representative use cases for further study on propagation delay compensation enhancements in Rel-17. User-specific clock synchronicity accuracy level; Number of devices in one Communication group for clock synchronisation; 5GS synchronicity budget requirement (note); Service area; Scenario. 2; Up to 300 UEs; ≤900 ns; ≤ 1000 m x 100 m; - Control-to-control communication for industrial controller. 4; Up to 100 UEs; <1 µs; < 20 km2; - Smart Grid: synchronicity between PMUs. Agreements: - ± 8∙64∙Tc/2μ8*64*Tc/2 as the TA indicating error is assumed in the evaluation. Agreements: For 5GS synchronicity budget requirement, - One Uu interface is assumed for smart grid. - Two Uu interfaces are assumed for control-to-control. Agreements: For BS transmit timing error, further study the following three options: - Option 1: 65 ns - Option 2:±130ns for the indoor scenario and ±200ns for the smart grid scenario - Option 3:82.5 ns Agreements: The value defined in Table 7.1.2-1 for initial transmit timing error (Te) in TS 38.133 should be considered for evaluation of the time synchronization. Agreements: Asymmetry between downlink and uplink channel for control-to-control scenario is not considered. Agreements: 100 ns is assumed for BS detecting error. Agreements: Timing advance adjustment accuracy defined in Table 7.3.2.2-1 in TS 38.133 is assumed for evaluation of the time synchronization. Agreements: Both 15 kHz and 30 kHz are assumed for both control-to-control and smart grid for evaluation of the time synchronization. Agreements: Send an LS to RAN2 with the content including - Inform RAN2 the two representative use cases concluded in RAN1 for further study; - Ask RAN2 for input about Uu interface error budget for each of the two use cases; Agreements: The following options for propagation delay compensation are further studied in RAN1 - Option 1: TA-based propagation delay - Option 1a: Propagation delay estimation based on legacy Timing advance (potentially with enhanced TA indication granularity). - Option 1b: Propagation delay estimation based on timing advanced enhanced for time synchronization (as 1a but with updated RAN4 requirements to TA adjustment error and Te) - Option 1c: Propagation delay estimation based on a new dedicated signaling with finer delay compensation granularity (Separated signaling from TA so that TA procedure is not affected) - Option 2: RTT based delay compensation: - Propagation delay estimation based on an RAN managed Rx-Tx procedure intended for time synchronization (FFS to expand or separate procedure/signaling to positioning). Draft LS R1-2007445 is approved, with final LS in R1-2007446.

RAN1#103-e (Oct./Nov. 2020)

Agreements: - Take 65 ns as the assumption of transmit timing error for evaluation of the overall time synchronization error for control-to-control. - Asymmetry between downlink and uplink channel for smart grid scenario is not considered. - TA adjustment accuracy is not considered for the evaluation of time synchronization error. - errorBS,DL,TXerrorBS, DL, TX is included in the equation for calculating the overall time synchronization error. Agreements: TA adjustment accuracy is not considered for the evaluation of time synchronization error. Agreements: For evaluation of the overall time synchronization error for smart grid, companies can take one of the following two options as the assumption for BS transmit timing error: - Option 1: 200 ns - Option 2: 65 ns

RAN1#104-e (Jan/Feb. 2021)

Agreements: Take ±100 ns as the assumption for downlink frame timing detection error (errorUE,DL,RXerrorUE, DL, RX) at the UE for evaluation of the overall time synchronization error for TA based propagation delay compensation, if downlink frame timing detection error needs to be considered separately. - Send a LS to RAN4 to ask for clarification on whether downlink frame timing detection error is included in Te or not - In the LS, to include more details about option 1 (included) & option 2 (not included); also including the necessary background - FFS whether to apply the same value to RTT-based propagation delay compensation, and the corresponding condition (if any) if the same value will be applied Decision: As per email posted on feb 5th, the draft LS is endorsed. Final LS is approved in R1-2102245.

RAN1#104bis-e (April 2021)

Agreements: If downlink frame timing detection error needs to be considered separately from propagation delay estimation error, take ±100 ns as the assumption for downlink frame timing detection error (errorUE,DL,RX) at the UE for evaluation of the overall time synchronization error for RTT based propagation delay compensation Agreements: Take the following equation for evaluation of the DL propagation delay estimation error for TA based propagation delay compensation: - Either option 1 or option 2 below will be applied based on the RAN4 reply to RAN1 LS R1-2102245. - FFS whether errorBS,DL,TX in the above equation should be included or not. Agreements: - Observation 1: Propagation delay compensation based on existing Rel-15/Rel-16 TA procedure and associated granularity, with no enhancements in RAN1, is sufficient for meeting the Uu interface synchronicity error budget in LS R2-2010837 for the smart grid scenario. - Observation 2: RAN1 needs to further study and specify the feasible enhancement (if any with RAN1 spec impact) for propagation delay compensation for control-to-control scenario, in order to meet the synchronicity budget of Uu interface in LS R2-2010837. Working assumption: Agreement: Take the following as the evaluation assumptions for both RTT-based PDC and TA-based PDC. - The UE may acquire an up-to-date PD estimation after waking up from DRX. This implies that gNB may signal an update timing advance value or complete a Rx-Tx measurement procedure. - errorUE,DL,RXerrorUE, DL, RX is based on other signals (e.g. CSI-RS) instead of SSB. - errorBS, UL,RXerrorUE, DL, RX iss based on other uplink signals instead of contention based PRACH, e.g. SRS. - Further study and specify new procedure/signaling (if necessary) to ensure that the PD estimation can be acquired after DRX for the adopted PDC method. Agreement: Existing DL reference signal(s) are used for Rx – Tx time difference estimation at UE side for RTT-based propagation delay compensation, if RTT-based propagation delay compensation is supported. - FFS whether PRS can be used for UE Rx – Tx time difference estimation or not - FFS which DL reference signal(s) to be used if/when PRS is not used Conclusion: - Leave it to RAN2 to decide whether to support UE based compensation and/or gNB based compensation for any propagation delay compensation method RAN1 may adopt for Rel-17, if applicable.

RAN1#105-e (May 2021)

Not on the agenda

RAN1#106-e (Aug. 2021)

Agreement SRS can be used for Rx – Tx time difference estimation at gNB side for RTT-based propagation delay compensation, if RTT-based propagation delay compensation is supported. Agreement Send LS to RAN4 to ask for feedback on the following questions: - Question 1: Is it feasible to support a smaller value than the current Te for the use of propagation delay compensation, assuming the existing conditions in TS 38.133 for Te requirement? If not, is it feasible under new conditions (e.g. using TRS instead of SSB)? If the answer is yes, please also provide feedback on how much it can be reduced at most. - Question 2: Is it feasible to introduce enhanced TA command indication granularity? If the answer is yes, please also provide feedback on how much it can be reduced at most (e.g. reduced to (1/16)* (16*64*Tc/2)) similar as the granularity for Rel-16 IAB based on the Timing Delta MAC CE and related condition. - Note 1: The alternatives in the working assumption achieved in RAN1#104bis-e together with the examples in Table 4.2-2 will be included in the LS to give some background for RAN4 - Note 2: The agreement “both SCS 15 kHz and 30 kHz are assumed for both control-to-control and smart grid for evaluation of the time synchronization” achieved in RAN1#102-e will be included in the LS for RAN4 information also. - Note 3: Inform RAN4 that the enhancements on Te and TA command indication granularity for propagation delay compensation may or may not have impact on normal TA related procedure, depending on which candidate option for TA-based PDC is adopted. Note that this is just for RAN4 information. - Note 4: Whether RAN1 will introduce specification enhancements is still undetermined. Agreement If RTT-based propagation delay compensation is supported, - CSI-RS for tracking (TRS) can be used for Rx – Tx time difference estimation at UE side, if PRS is not configured for the UE. - PRS can be used for Rx – Tx time difference estimation at UE side, if PRS is configured for the UE. Agreement Send LS to RAN4 to ask for defining the following for RTT-based propagation delay compensation, if RTT-based propagation delay compensation is supported. - UE Rx-Tx time difference measurement accuracy errorUE,RxTxDiff based on CSI-RS for tracking - gNB Rx-Tx time difference absolute accuracyerrorgNB,RxTxDiff errorUE,RxTxDiff based on SRS Agreement Support the following configurations for RTT-based propagation delay compensation, if RTT-based propagation delay compensation is supported. - At least one CSI-RS for tracking (TRS) configuration for Rx – Tx time difference estimation at UE side if PRS is not configured - At least one SRS configuration for Rx – Tx time difference estimation at gNB side Agreement If RTT-based propagation delay compensation is supported and performed at the UE side, the Rx-Tx measurement report provided from the gNB to the UE should include at least: - gNB Rx-Tx time difference at a given granularity - FFS whether to include SRS-Resource-ID Agreement Take the following two alternatives as the equation for evaluation of the overall time synchronization error for RTT-based propagation delay compensation. RAN1 to select one of the alternatives in RAN1#106bis-e. - Alt. 1: - errorRxTxDiff, indication is to reflect the error due to indication granularity of Rx-Tx time difference - errorgNB,RxTxDiff and errorUE, RxTxDiff reflects the measurement inaccuracy of gNB Rx-Tx time difference, and the measurement inaccuracy of UE Rx-Tx time difference, respectively. - Note: The equation may be updated after clarification on the gNB TX-RX timing difference and UE TX-RX timing difference - Alt. 2: - errorRxTxDiff,indication is to reflect the error due to indication granularity of Rx-Tx time difference - Note: Alt.2 assumes that gNB can coordinate the time of TA procedure and the time of PD compensation, so that the DL frame timing error and BS transmit timing error for propagation delay estimation is correlated to (e.g. the same as) that for the transmission of RRC signaling carrying the reference time clock Note: FFS whether / how to handle inconsistent RTT measurement in gNB and UE due a change of uplink TX timing LS on TA-based propagation delay compensation approved in R1-2108635.

RAN1#106bis-e (Oct. 2021)

Agreement For evaluation of the overall time synchronization error for RTT-based propagation delay compensation, - Alt.1 for RTT-based PDC Agreement For evaluation of the overall time synchronization error for TA-based propagation delay compensation, - Alt.1 for TA-based PDC Agreement For evaluation of the overall time synchronization error for RTT-based propagation delay compensation with Alt.1, it is assumed that - The UE Rx-Tx time difference measurement accuracy based on PRS defined in Table 10.1.25.2-2 in TS 38.133 v17.3.0 is taken as the reference for the UE Rx-Tx time difference measurement accuracy - The gNB Rx-Tx time difference accuracy based on SRS for positioning defined in Table 13.2.2.2-1 in TS 38.133 v17.3.0 is taken as the reference for the gNB Rx-Tx time difference accuracy based on SRS for PDC Agreement For RTT-based PDC, only a single pair of CSI-RS for tracking (TRS)/PRS and SRS configuration, i.e. one CSI-RS for tracking (TRS)/PRS configuration for Rx – Tx time difference estimation at UE side and one SRS configuration for Rx – Tx time difference estimation at gNB side, is configured for PDC in Rel-17, if RTT-based PDC is supported. Agreement If RTT-based propagation delay compensation is supported and performed at the gNB side, the Rx-Tx measurement report provided from the UE to the gNB should include at least: - UE Rx-Tx time difference at a given granularity Conclusion When evaluating enhanced TA-based PDC, there is no need to replace Te by TA adjustment error. Agreement Send an LS to RAN2 and CC RAN4 with the content including: - The latest available status on PDC methods in RAN1, e.g. key agreements achieved for TA-based PDC and RTT-based PDC. LS is endorsed in R1-2110647 Agreement For evaluation and comparison of enhanced TA-based PDC and RTT-based PDC, the timing detection error = 0.5/(RS BW) = 0.5/(N_PRB*12*SCS) can be used to achieve errorUE,DL,RX and errorBS,UL,RX, if needed in the evaluation equation separately, where N_PRB is the number of PRBs of the RS bandwidth used in the detection by UE and gNB, respectively. - Note: Detection error achieved by evaluations is not precluded if available. Agreement If enhanced TA-based PDC with reduced Te based on TRS is supported in Rel-17, one CSI-RS for tracking (TRS) configuration is configured for enhanced TA-based PDC. - FFS whether/how to configure UL signal for enhanced TA-based PDC Agreement If enhanced TA-based PDC with enhanced TA command indication granularity is supported in Rel-17, - The enhanced TA command indication granularity introduced for enhanced PDC is applied for PDC purpose, which doesn’t have impact on normal TA procedure, i.e. normal TA procedure will still follow the existing TA command indication granularity. Agreement If RTT-based propagation delay compensation is supported, the Rx-Tx time difference is reported with granularity 2k*Tc, where k is an integer satisfying 0<=k<=5. - FFS the value of k - FFS the reporting range of Rx-Tx time difference measurement for PDC

RAN1#107-e (Nov. 2021)

Agreement If RTT-based PDC is supported, a single granularity 32Tc (i.e. k=5) is supported for Rx-Tx measurement report. Agreement For Rel-17 - Support RTT-based PDC method - Support PDC method based on legacy TA-based mechanism - No RAN1/RAN4 specification impact expected Agreement For RTT-based PDC, existing definitions of UE Rx – Tx time difference (i.e. section 5.1.30 in TS 38.215) and gNB Rx – Tx time difference (i.e. section 5.2.3 in TS 38.215) are reused, with updates at least to reflect the single pair of TRS/PRS and SRS configured for RTT-based PDC. Agreement Send an LS to RAN2 and RAN4 with the content including: - The agreements made in RAN1#107-e for propagation delay compensation. - Ask RAN4 to define the following for RTT-based propagation delay compensation: - UE Rx-Tx time difference measurement accuracy based on CSI-RS for tracking - UE Rx-Tx time difference measurement accuracy based on PRS (including reuse existing spec if appropriate) - gNB Rx-Tx time difference absolute accuracy based on SRS (including reuse existing spec if appropriate) - Inform RAN4 that enhanced TA-based PDC with reduced Te and enhanced TA command granularity is precluded in RAN1. LS is endorsed in R1-2112834 Conclusion For RTT-based PDC, it is assumed that the transmission of DL TRS/PRS, UL SRS and reference time information are associated with a same TRP. Note: No RAN1 specification impact is expected for this conclusion Agreement For RTT-based propagation delay compensation, the Rx-Tx time difference is reported via RRC signaling. Conclusion The reporting range of Rx-Tx time difference measurement for RTT-based PDC is up to RAN4.

RAN1#107bis-e (Jan. 2022)

Conclusion SRS for positioning is not supported for RTT-based PDC, regardless of whether TRS or PRS is used for RTT-based PDC. Conclusion Measurement gaps should not be mandatory for a UE to process PRS for PDC purposes. Agreement Add “dl-PRS-ResourceRepetitionFactor-r16 and dl-PRS-ResourceTimeGap-r16 in the RRC parameters list for RTT-based PDC Agreement Include dl-PRS-ResourceBandwidth-r16 and dl-PRS-StartPRB-r16 in NR-DL-PRS-Resource-r16 for the PRS configuration for RTT-based PDC. Agreement Add new “usage-pdc-r17” field to SRS-ResourceSet to indicate that this ResourceSet is used for PDC purpose, meanwhile also indicate that this ResourceSet is used for other purpose by usage. Agreement Alt.2: No need to add new “pathlossReferenceRS-PDC-r17” field to SRS-ResourceSet to indicate a reference signal (e.g. a CSI-RS config or a SS block or a DL-PRS config) to be used for SRS path loss estimation. - Note: With Alt.2, the existing RRC parameter PathlossReferenceRS-Config is used to indicate a reference signal (e.g. a CSI-RS config or a SS block) to be used for SRS path loss estimation. Working Assumption Alt.1: Add new “spatialRelationInfo-PDC-r17” field to SRS-Resource to indicate the spatial relation between a reference RS and the target SRS, with spatialRelationInfo-PDC-r17 as below: spatialRelationInfo-PDC-r17 ::= SEQUENCE { referenceSignal CHOICE { ssb-Index SSB-Index, csi-RS-Index NZP-CSI-RS-ResourceId, dl-PRS-PDC nr-DL-PRS-ResourceID-r16 srs SEQUENCE { resourceId SRS-ResourceId, uplinkBWP BWP-Id } } } Note: RAN1 does not pursue further optimization for SRS configuration with legacy usage and meanwhile with PRS as spatial relation source. Conclusion For PDC method based on legacy TA-based mechanism, the TA value for PDC is the timing advance value associated with the PTAG of MCG. Agreement dl-PRS-PointA-r16 is not included for the PRS configuration for RTT-based PDC. - RAN1 specification change is expected Agreement “dl-PRS-ResourcePower-r16” from 37.355 is not included in the RRC parameters list for PRS configuration for RTT-based PDC. Conclusion There is no consensus to introduce new RRC parameter “DL-PRS-PDC-QCL-Info” to specify the QCL indication with other DL reference signals, for DL PRS configuration for PDC.