3GPP TSG RAN WG1 #107b-e			R1-22xxxxx

e-Meeting, January 17th – 25th, 2022

Agenda Item:			8.8.1.3

Source:			Moderator (China Telecom)

Title:			[107bis-e-R17-CovEnh-03] Email discussion regarding joint channel estimation for PUSCH

Document for:		Discussion

- Introduction

In RAN #90 e-meeting, a new Rel-17 work item on NR coverage enhancements was approved [1] and was revised in [2]. The objective of this work item is to specify enhancements for PUSCH, PUCCH and Msg3 PUSCH for both FR1 and FR2 as well as TDD and FDD.

The detailed objectives are as follows.

- Specification of PUSCH enhancements [RAN1, RAN4]

  - Specify the following mechanisms for enhancements on PUSCH repetition type A [RAN1]

    - Increasing the maximum number of repetitions up to a number to be determined during the course of the work.

    - The number of repetitions counted on the basis of available UL slots.

  - Specify mechanism(s) to support TB processing over multi-slot PUSCH [RAN1]

    - TBS determined based on multiple slots and transmitted over multiple slots.

  - Specify mechanism(s) to enable joint channel estimation [RAN1, RAN4]

    - Mechanism(s) to enable joint channel estimation over multiple PUSCH transmissions, based on the conditions to keep power consistency and phase continuity to be investigated and specified if necessary by RAN4 [RAN1, RAN4]

      - Potential optimization of DMRS location/granularity in time domain is not precluded

    - Inter-slot frequency hopping with inter-slot bundling to enable joint channel estimation [RAN1]

- Specification of PUCCH enhancements [RAN1, RAN4]

  - Specify signaling mechanism to support dynamic PUCCH repetition factor indication [RAN1]

  - Specify mechanism to support DMRS bundling across PUCCH repetitions [RAN1, RAN4]

    - When applicable, based on similar mechanism(s) for enabling joint channel estimation for PUSCH

- Specify mechanism(s) to support Type A PUSCH repetitions for Msg3 [RAN1, RAN2]

Editors’ CRs on introduction of coverage enhancements in NR have been approved in RAN#94e [3]. This contribution is a summary of the following email discussion.

[107bis-e-R17-CovEnh-03] Email discussion regarding joint channel estimation for PUSCH – Jianchi (China Telecom)

- 1st check point: January 20

- Final check point: January 25

- Background

2.1 Use cases for joint channel estimation

RAN1 has identified the potential use cases for joint channel estimation for PUSCH.

- Use case 1: back-to-back PUSCH transmissions within one slot.

- Use case 2: non-back-to-back PUSCH transmissions within one slot.

  - Use case 2a: no uplink transmission in the middle of two PUSCH transmissions

  - Use case 2b: other uplink transmissions in the middle of two PUSCH transmissions

- Use case 3: back-to-back PUSCH transmissions across consecutive slots.

- Use case 4: non-back-to-back PUSCH transmissions across consecutive slots.

  - Use case 4a: no uplink transmission in the middle of two PUSCH transmissions

  - Use case 4b: other uplink transmissions in the middle of two PUSCH transmissions

- Use case 5: PUSCH transmissions across non-consecutive slots.

  - Use case 5a: no uplink transmission in the middle of two PUSCH transmissions

  - Use case 5b: other uplink transmissions in the middle of two PUSCH transmissions

Note: RAN1 assumes “back-to-back PUSCH transmission” has zero gap in-between adjacent PUSCH transmissions.

Note: intervening “other uplink transmissions” can be either on the same component carrier or a different component carrier.

In the past RAN1 meetings, it was discussed whether joint channel estimation can be applied to the above uses cases. In addition, during online discussion of RAN1 #106-e meeting, according to the guidance from Chair there would be no further discussion for transmissions with different TBs. Based on the discussion and agreements, the situation is summarized in the following table.

| Use cases | Repetition type A for the same TB | Repetition type B for the same TB | Transmissions with different TBs | TBoMS |
| --- | --- | --- | --- | --- |
| 1: B2B PUSCH transmission within one slot | / | Support | Not support | / |
| 2: Non-B2B PUSCH transmission within one slot | / | Not support | Not support | / |
| 3: B2B PUSCH transmissions across consecutive slots | Support | Support | No further discussion | Support |
| 4: Non-B2B PUSCH transmissions across consecutive slots | Support (4a) | Support (4a) | No further discussion | Support (4a) |
|  | Not support (4b) | Not support (4b) |  | Not support (4b) |
| 5: PUSCH transmissions across non-consecutive slots | Not support | Not support | Not support | Not support |

2.2 The maximum duration

In the LS [4] and agreed way forward [6] in RAN4, RAN4 provided answers to the related questions about the maximum duration.

| RAN1’s questions | RAN4’s answers |
| --- | --- |
| For joint channel estimation, is there a maximum duration during which UE is able to maintain power consistency and phase continuity under certain tolerance level? If any, how long is it? | Yes, there is a maximum duration but RAN4 has not agreed how many slots it is [4]. Depend on the outcome of “What factors determine the maximum duration”. Note: The number of slots for maximum duration means the consecutive slots. In case of non-scheduled gap and/or other channel transmission, the duration of the non-scheduled gap and/or other channel should be counted [6]. |
| What factors determine the maximum duration? | RAN4 has agreed that TA adjustment should be avoided across the PUSCH/PUCCH transmissions (i.e., from start of first transmission until the end of last transmission) for joint channel estimation. RAN4 is still investigating other factors impact in more detail [4]. Phase and power tolerance within the duration [6]. |
| Whether the maximum duration should be the same for different cases for both PUSCH and PUCCH? | Yes [4]. |
| Whether the maximum duration is dependent on the modulation order of transmission, e.g., QPSK, 16QAM, 64QAM? | Considering the scenario of coverage extension, RAN4 recommends to only focus on modulation orders not higher than QPSK, i.e., focus on QPSK (PUCCH and PUSCH), Pi/2 BPSK (PUCCH and PUSCH), BPSK (PUCCH). RAN4 is still discussing whether maximum duration depends on modulation order for the above modulation schemes [4]. No. Note: It has been agreed to only focus on the modulation orders not higher than QPSK [6]. |
| Whether the maximum duration is dependent on UL waveform (DFT-s-OFDM vs. OFDM)? | No [4]. |
| Whether the maximum duration is band specific? | The length of maximum duration is [6]: - Option 1: Band specific - Option 2: FR specific |
| Besides the factors listed above, whether or not the maximum duration is further dependent on UE capabilities (e.g., multiple possible values for a given set of factor(s)), and if so, whether the UE should report such a duration. | Still under discussion in RAN4 [4]. Down select among the following options once we have the results of the simulations [6]: - Option 1: Subject to a single maximum duration value. - The value is defined for a given set of factors which are depends on the conclusion for the other issues under discussion. - Option 2: Subject to multiple maximum duration value and UE could report the supported value(s). |

Based on LS [5], the length(s) of maximum duration is still being analyzed in RAN4. RAN4 is studying the impact of enabling up to 32 slots. Other numbers beyond 32 slots are not analyzed in RAN4. RAN4 is still discussing whether the max duration would be best defined per-FR or per-band.

- Summary of contributions in RAN1#107bis-e

3.1 Time domain window

Issue #1: The start of configured TDW for CG PUSCH

For CG PUSCH, companies (Huawei, HiSilicon, NTT DOCOMO, Spreadtrum) observe that since the initial transmission for CG-PUSCH could be any transmission occasions associated with RV=0 if startingFromRV0 is set to ‘on’ and RV sequence is {0,0,0,0} or {0,3,0,3}, the start of the first configured TDW also could be any transmission occasions associated with RV=0. gNB might end up with false alarm or miss detection of the actual initial transmission. A misalignment issue of configured TDWs between gNB and UE may be occurred if the scheme of TDW determination for dynamic grant is reused.

The following solution is proposed to solve the above problem by companies:

- For CG PUSCH, to avoid any misalignment of configured TDW between a gNB and UEs, the first configured TDWs should always start from the first physical slot of a CG period.

Companies’ TPs are summarized as follows:

NTT DOCOMO proposes to adopt the following TP:

-------------------------- Start of Text Proposal for TS 38.214--------------------------- <Unchanged parts omitted> 6.1.7 UE procedure for determining time domain windows for bundling DM-RS For PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, PUSCH repetition Type A with a configured grant, PUSCH repetition Type B and TB processing over multiple slots, when PUSCH-DMRS-Bundling is enabled, and for PUCCH transmissions of PUCCH repetition, when PUCCH-DMRS-Bundling is enabled, the UE determines one or multiple nominal TDWs, as follows: - For PUSCH transmissions of repetition Type A, PUSCH repetition Type B and TB processing over multiple slots, the duration of each nominal TDW except the last nominal TDW, in number of consecutive slots, is: - Given by PUSCH-TimeDomainWindowLength, if configured. - Computed as min ([maxDMRS-BundlingDuration], M), if PUSCH-TimeDomainWindowLength is not configured, where M is the time duration in consecutive slots of N∙K PUSCH transmissions, and where: - For PUSCH transmissions of PUSCH repetition Type A, N=1 and K is the number of repetitions, as defined in Clause 6.1.2.1. - For PUSCH transmissions of PUSCH repetition Type B, N=1 and K is the number of nominal repetitions, as defined in Clause 6.1.2.1. - For PUSCH transmissions of TB processing over multiple slots, N is the number of slots used for TBS determination and K is the number of repetitions of the number of slots N used for TBS determination, as defined in Clause 6.1.2.1. - For PUCCH transmissions of PUCCH repetition, the duration of each nominal TDW except the last nominal TDW, in number of consecutive slots, is: - Given by PUCCH-TimeDomainWindowLength, if configured. - Computed as min ([maxDMRS-BundlingDuration], M), if PUCCH-TimeDomainWindowLength is not configured, where M is the time duration in consecutive slots from the first slot determined for PUCCH transmissions of PUCCH repetition to the last slot determined for PUCCH transmissions of PUCCH repetition according to clause 9.2.6 of [6, TS 38.213]. - For PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 and PUSCH repetition Type A with a configured grant, when AvailableSlotCounting is enabled, and for TB processing over multiple slots: - The start of the first nominal TDW is the first slot determined for the first PUSCH transmission. - The end of the last nominal TDW is the last slot determined for the last PUSCH transmission. - The start of any other nominal TDWs is the first slot determined for PUSCH transmission after the last slot determined for PUSCH transmission of a previous nominal TDW. - For PUSCH transmissions of a PUSCH repetition Type A with a configured grant, when the UE is not configured with AvailableSlotCounting or when AvailableSlotCounting is disabled, and for PUSCH repetition type B: - The start of the first nominal TDW is the first slot for the first PUSCH transmission occasion. - The end of the last nominal TDW is the last slot for the last PUSCH transmission. - The start of any other nominal TDWs is the first slot after the last slot of a previous nominal TDW. - For PUSCH transmissions of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2 and PUSCH repetition Type A with a configured grant, when the UE is not configured with AvailableSlotCounting or when AvailableSlotCounting is disabled, and for PUSCH repetition type B: - The start of the first nominal TDW is the first slot for the first PUSCH transmission. - The end of the last nominal TDW is the last slot for the last PUSCH transmission. - The start of any other nominal TDWs is the first slot after the last slot of a previous nominal TDW. - For PUSCH transmissions of a PUSCH repetition Type A with a configured grant, when the UE is not configured with AvailableSlotCounting or when AvailableSlotCounting is disabled, and for PUSCH repetition type B: - The start of the first nominal TDW is the first slot for the first PUSCH transmission occasion. - The end of the last nominal TDW is the last slot for the last PUSCH transmission. - The start of any other nominal TDWs is the first slot after the last slot of a previous nominal TDW. - For PUCCH transmissions of a PUCCH repetition: - The start of the first nominal TDW is the first slot determined for the first PUCCH transmission. - The end of the last nominal TDW is the last slot determined for the last PUCCH transmission. - The start of any other nominal TDWs is the first slot determined for PUCCH transmission after the last slot determined for PUCCH transmission of a previous nominal TDW. <Unchanged parts omitted> -------------------------- End of Text Proposal for TS 38.214 --------------------------

Spreadtrum proposes to adopt the following TP (TS 38.214):

6.1.7 UE procedure for determining time domain windows for bundling DM-RS < Unchanged part is omitted > - For PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, when AvailableSlotCounting is enabled, and for TB processing over multiple slots scheduled by DCI format 0_1 or 0_2: - The start of the first nominal TDW is the first slot determined for the first PUSCH transmission. - The end of the last nominal TDW is the last slot determined for the last PUSCH transmission. - The start of any other nominal TDWs is the first slot determined for PUSCH transmission after the last slot determined for PUSCH transmission of a previous nominal TDW. - For PUSCH transmissions of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, when the UE is not configured with AvailableSlotCounting or when AvailableSlotCounting is disabled, and for PUSCH repetition type B scheduled by DCI format 0_1 or 0_2: - The start of the first nominal TDW is the first slot for the first PUSCH transmission. - The end of the last nominal TDW is the last slot for the last PUSCH transmission. - The start of any other nominal TDWs is the first slot after the last slot of a previous nominal TDW. - For PUSCH transmission of a PUSCH repetition Type A with a configured grant, PUSCH transmission of a PUSCH repetition Type B with a configured grant and for TB processing over multiple slots with a configured grant: - The start of the first nominal TDW is the first transmission occasion in Clause 6.1.2.3.1, Clause 6.1.2.3.2 and Clause 6.1.2.3.3. - The end of the last nominal TDW is the last slot determined for the last PUSCH transmission. - The start of any other nominal TDWs is the first slot determined for PUSCH transmission after the last slot determined for PUSCH transmission of a previous nominal TDW. < Unchanged part is omitted >

Issue #2: Candidate values of the window length L of the configured TDW

In RAN1 #107e, the following agreement was achieved for the candidate values of the window length L of the configured TDW.

Agreement: - The candidate values of the window length L of the configured TDW can be any integer value that is larger than 1 and no larger than the maximum duration.

vivo proposes the time domain window length configured by PUSCH-TimeDomainWindowLength should support 1 slot and not exceed 32 slots.

Nokia proposes to introduce only two candidate values for L, one candidate value is equal to the maximum duration and the other candidate value is FFS.

Issue #3: Events that violate power consistency and phase continuity

Issue #3-1: Events for multi-TRP operations

In RAN1 #107e, the following agreement was achieved that UL beam switching for multi-TRP operation constitutes an event if DMRS bundling and UL beam switching for multi-TRP operation are configured simultaneously.

Agreement: - If DMRS bundling and UL beam switching for multi-TRP operation are configured simultaneously, UL beam switching for multi-TRP operation constitutes an event that violates power consistency and phase continuity. - FFS: UL beam switching for multi-TRP operation is regarded as a semi-static event.

Companies (Samsung, xiaomi, CMCC, Intel) think UL beam switching is as a semi-static event, the main reason is that it can be known prior to the first PUSCH transmission, similar to frequency hopping. While companies (ZTE, Panasonic) think since we have agreed that an event is regarded as a dynamic event if it is triggered by a DCI or MAC CE, UL beam switching in multi-TRP operation should be regarded as a dynamic event. Interdigital thinks the event type for UL beam switching depends on whether it is configured by RRC or indicated by DCI, if it is configured by RRC then it is regarded as a semi-static event; else if it is indicated by DCI, then it is regarded as a dynamic event.

vivo proposes that PUCCH repetitions with different sets of power control parameters in multi-TRP operation should be regarded as an event and adopt the following TP.

----------------------------------Start of TP#1 for section 9.2.6 of 38.214 V17.0.0---------------------------------- 6.1.7 UE procedure for determining time domain windows for bundling DM-RS For PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, PUSCH repetition Type A with a configured grant, PUSCH repetition Type B and TB processing over multiple slots, when PUSCH-DMRS-Bundling is enabled, and for PUCCH transmissions of PUCCH repetition, when PUCCH-DMRS-Bundling is enabled, the UE determines one or multiple nominal TDWs, as follows: <<unchanged text omitted>> Events which cause power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or PUCCH transmissions of PUCCH repetition, within the nominal TDW, are: - A downlink slot or downlink reception or downlink monitoring based on tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated for unpaired spectrum. - The gap between any two consecutive PUSCH transmissions, or the gap between any two consecutive PUCCH transmissions, exceeds 13 symbols. - The gap between any two consecutive PUSCH transmissions, or the gap between any two consecutive PUCCH transmissions, does not exceed 13 symbols but other uplink transmissions are scheduled between the two consecutive PUSCH transmissions or the two consecutive PUCCH transmissions. - For PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B or TB processing over multiple slots, a dropping or cancellation of a PUSCH transmission according to clause 9, clause 11.1 and clause 11.2A of [6, TS 38.213]. - For PUCCH transmissions of PUCCH repetition, a dropping or cancellation of a PUCCH transmission according to clause 9.2.6 and clause 11.1 of [6, TS 38.213]. - For any two consecutive PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, and when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook' or 'noncodebook', a different SRS resource set association is used for the two PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, according to Clause 6.1.2.1. - For any two consecutive PUCCH transmissions of PUCCH repetition, and when a PUCCH resource used for repetitions of a PUCCH transmission by a UE includes first and second spatial relations, or first and second sets of power control parameters, as described in [TS 38.321] and in clause 7.2.1 of [TS 38.213], different spatial relations or different power control parameters, are used for the two PUCCH transmissions of PUCCH repetition, according to Clause 9.2.6 of [6, TS 38.213]. - Uplink timing adjustment in response to a timing advance command according to clause 4.2 of [6, TS 38.213]. - Frequency hopping. <<unchanged text omitted>> ----------------------------------End of TP#1 for section 7.1.1 of 38.213 V17.0.0----------------------------------

Issue #3-2: Dynamic & semi-static events

China Telecom observes that according to the current specification, when PUSCH-Window-Restart or PUCCH-Window-Restart is enabled, a new actual TDW is created within the nominal TDW, it seems “UE is mandatory to support restarting DM-RS bundling due to semi-static events” has not been captured into the specification. Moreover, for UE not capable of restarting DM-RS bundling, when PUSCH-Window-Restart or PUCCH-Window-Restart is enabled, it is not clear whether a new actual TDW is created for the following two cases:

- Case 1: A semi-static event is triggered after one or multiple dynamic events. Whether a new actual TDW is created after the semi-static event?

- Case 2: A semi-static event overlaps with a dynamic event. Whether a new actual TDW is created after the semi-static event?

Fig. Illustration of case 1                          Fig. Illustration of case 2

To solve the above problem, China Telecom has the following proposals:

- A new actual TDW is created after a semi-static event no matter whether there are dynamic events before the semi-static event.

- A new actual TDW is created after a semi-static event in case the semi-static event overlaps with a dynamic event.

Fig. Illustration of Solution 1                    Fig. Illustration of Solution 2

China Telecom proposes to adopt the following TP to capture the agreement ‘UE is mandatory to support restarting DM-RS bundling due to semi-static events’ into the specification (TS 38.214):

- A new actual TDW is created when PUSCH-Window-Restart is enabled or in response to frequency hopping or in response to any event not triggered by DCI or MAC-CE. The start of a new actual TDW is the first symbol of the PUSCH transmission after the event which causes power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots within the nominal TDW, and the PUSCH transmission is in a slot for PUSCH transmission of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots. - A new actual TDW is created when PUCCH-Window-Restart is enabled or in response to frequency hopping or in response to any event not triggered by DCI or MAC-CE. The start of a new actual TDW is the first symbol of the PUCCH transmission after the event which causes power consistency and phase continuity not to be maintained across PUCCH transmissions of PUCCH repetition within the nominal TDW, and the PUCCH transmission is in a slot determined for transmission of the PUCCH.

vivo proposes that for PUSCH repetition type B, gap with more than 13 OFDM symbols created by invalid symbol pattern is considered as a semi-static event.

Issue #3-3: Dropping/Collision rules

LG proposes to treat dropping/cancellation based on Rel-17 collision rules as an event.

Intel proposes to adopt the following TP to add the case when PUCCH repetition with low priority is dropped when overlapping with PUSCH transmission with high priority, which is defined in Clause 9 in TS38.213, as an event:

=== Start of text proposal for 38.214 Subclause 6.1.7 === Events which cause power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or PUCCH transmissions of PUCCH repetition, within the nominal TDW, are: - A downlink slot or downlink reception or downlink monitoring based on tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated for unpaired spectrum. - The gap between any two consecutive PUSCH transmissions, or the gap between any two consecutive PUCCH transmissions, exceeds 13 symbols. - The gap between any two consecutive PUSCH transmissions, or the gap between any two consecutive PUCCH transmissions, does not exceed 13 symbols but other uplink transmissions are scheduled between the two consecutive PUSCH transmissions or the two consecutive PUCCH transmissions. - For PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B or TB processing over multiple slots, a dropping or cancellation of a PUSCH transmission according to clause 9, clause 11.1 and clause 11.2A of [6, TS 38.213]. - For PUCCH transmissions of PUCCH repetition, a dropping or cancellation of a PUCCH transmission according to clause 9, clause 9.2.6 and clause 11.1 of [6, TS 38.213]. - For any two consecutive PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, and when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook’ or ‘noncodebook’, a different SRS resource set association is used for the two PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, according to Clause 6.1.2.1. - For any two consecutive PUCCH transmissions of PUCCH repetition, and when a PUCCH resource used for repetitions of a PUCCH transmission by a UE includes first and second spatial relations, different spatial relations are used for the two PUCCH transmissions of PUCCH repetition, according to Clause 9.2.6 of [6, TS 38.213]. - Uplink timing adjustment in response to a timing advance command according to clause 4.2 of [6, TS 38.213]. - Frequency hopping. === end of text proposal for 38.214 Subclause 6.1.7 ===

Issue #3-4: HD-FDD RedCap UE related issues

Huawei proposes that for HD-FDD RedCap UEs, an event is constituted if the scheduled UL symbols overlap with any symbol of an SS/PBCH block provided by ssb-PositionInBurst.

Spreadtrum proposes that a downlink reception or downlink monitoring based on higher layer singling or DCI format is regarded as an event and adopt the following TP (TS 38.214):

6.1.7 UE procedure for determining time domain windows for bundling DM-RS < Unchanged part is omitted > Events which cause power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or PUCCH transmissions of PUCCH repetition, within the nominal TDW, are: < Unchanged part is omitted > - For PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B or TB processing over multiple slots, a dropping or cancellation of a PUSCH transmission according to clause 9, clause 11.1, clause 11.2A and clause 17.2 of [6, TS 38.213]. - For PUCCH transmissions of PUCCH repetition, a dropping or cancellation of a PUCCH transmission according to clause 9.2.6, clause 11.1 and clause 17.2 of [6, TS 38.213]. < Unchanged part is omitted >

Issue #3-5: Clarifications on the events

vivo: For extended CP case, support to define 11 symbols as the maximum gap length to maintain the power consistency and phase continuity.

Ericsson: Revise ‘Frequency hopping’ in the list of events in 38.214 to ‘Change in starting RB for inter-slot frequency hopping’.

LG: Other UL transmission in between PUSCH/PUCCH transmission is an event only on the same carrier.

3.2 TPC command

Issue #4: TPC command

In RAN1 #107e, the following working assumption was achieved for TPC commands.

Working assumption: - The action of group common TPC commands with format 2_2 does not constitute an event that violates power consistency and phase continuity. - If UE is configured to accumulate TPC commands, - If UE receives TPC commands that would take into effect during a configured TDW, UE accumulates TPC commands without taking effect during the current configured TDW. TPC commands take effect after the current configured TDW. - If UE is not configured to accumulate TPC commands - the last TPC command that would take effect within a configured TDW supersedes all previous TPC commands that take effect within that configured TDW and only the last TPC command is applied by the UE after the current configured TDW. - FFS: no more than 1 TPC command is expected to take effect during a configured TDW.

The majority companies (NTT DOCOMO, Samsung, Huawei, HiSilicon, vivo, ZTE, Apple, CMCC, Nokia, NSB, LG, xiaomi, InterDigital) support to confirm the WA. While Intel proposes to drop the whole WA and proposes the action of group common TPC commands to be considered as an event for DMRS bundling. Ericsson mentioned that absolute TPC is not supported for DCI format 2_2 and proposes to remove the the second bullet.

Companies’ further views on the above WA are summarized as follows:

- Remove the FFS bullet.

  - Support: NTT DOCOMO, Huawei, HiSilicon, vivo, CMCC, xiaomi, InterDigital

  - Not Support: LG

- No redefinition of transmission occasion for PUSCH/PUCCH in Rel-17.

  - Support: Nokia, NSB, Apple, LG, Huawei, HiSilicon, Qualcomm, Ericsson

- Replace all the “configured TDW” to “actual TDW”.

  - Support: Huawei, HiSilicon

- Replace the FFS bullet with “No TPC command is expected to take effect during a configured TDW.”

  - Support: ZTE

Sharp has the following proposal:

Proposal: One of the following two alternatives should be specified for accumulation of TPC commands for DCI format 2_2:

- Alt 1: The transmission occasion for TPC command includes repetitions over the configured TDW.

- Alt 2: Reuse the legacy transmission occasion (e.g., the new variable value for accumulation of TPC commands is introduced).

CMCC proposes the following modification:

Working assumption: - The action of group common TPC commands with format 2_2 does not constitute an event that violates power consistency and phase continuity. - If UE is configured to accumulate TPC commands, - If UE receives TPC commands that would take into effect during a configured TDW, UE accumulates TPC commands without taking effect during the current configured TDW. TPC commands take effect after the current configured TDW. - If UE is not configured to accumulate TPC commands - Tthe last TPC command that would take effect within a configured TDW supersedes all previous TPC commands that take effect within that configured TDW and only the last TPC command is applied by the UE after the current configured TDW. - FFS: no more than 1 TPC command is expected to take effect during a configured TDW.

Qualcomm proposes to capture the agreement on deferring TPC updates to the end of nominal TDWs using the following TP in 38.213, Section 7.1.1:

If the UE is provided PUSCH-DMRS-Bundling = ‘enabled’, and transmission occasion i is not the last transmission occasion within a nominal time domain window, then any TPC command values received via DCI format 2_2 contained in the set Di are deleted and added to the set Dj where j is a transmission occasion occurring after the end of the nominal time domain window.

Ericsson proposes to adopt the following TP in 38.213 for accumulated TPC operation:

If the UE is provided PUSCH-DMRS-Bundling = ‘enabled’, and for processing TPC command values provided by DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI, if a transmission occasion i occurs within a nominal time domain window determined as described in [6, TS 38.214], then fb,f,ci,l=fb,f,c(i1,l), where transmission occasion i1 is a first transmission occasion within the nominal time domain window. For a first transmission occasion i2 after the transmission nominal time domain window, fb,f,ci2,l=fb,f,ci1,l+i=i1+1i2m=0CDi-1δPUSCH,b,f,c(m,l), with CDi as defined above.

Intel proposes to adopt the following TP:

=== Start of text proposal for 38.214 Subclause 6.1.7 === Events which cause power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or PUCCH transmissions of PUCCH repetition, within the nominal TDW, are: - A downlink slot or downlink reception or downlink monitoring based on tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated for unpaired spectrum. - The gap between any two consecutive PUSCH transmissions, or the gap between any two consecutive PUCCH transmissions, exceeds 13 symbols. - The gap between any two consecutive PUSCH transmissions, or the gap between any two consecutive PUCCH transmissions, does not exceed 13 symbols but other uplink transmissions are scheduled between the two consecutive PUSCH transmissions or the two consecutive PUCCH transmissions. - For PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B or TB processing over multiple slots, a dropping or cancellation of a PUSCH transmission according to clause 9, clause 11.1 and clause 11.2A of [6, TS 38.213]. - For PUCCH transmissions of PUCCH repetition, a dropping or cancellation of a PUCCH transmission according to clause 9.2.6 and clause 11.1 of [6, TS 38.213]. - For any two consecutive PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, and when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook’ or ‘noncodebook’, a different SRS resource set association is used for the two PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, according to Clause 6.1.2.1. - For any two consecutive PUCCH transmissions of PUCCH repetition, and when a PUCCH resource used for repetitions of a PUCCH transmission by a UE includes first and second spatial relations, different spatial relations are used for the two PUCCH transmissions of PUCCH repetition, according to Clause 9.2.6 of [6, TS 38.213]. - Uplink timing adjustment in response to a timing advance command according to clause 4.2 of [6, TS 38.213]. - Transmit power adjustment in response to a TPC command in DCI format 2_2 according to Clause 7.1 and 7.2 of [6, TS 38.213]. - Frequency hopping. === end of text proposal for 38.214 Subclause 6.1.7 ===

Samsung proposes to adopt the following TP:

[7.1.1, TS 38.213] - is the PUSCH power control adjustment state for active UL BWP of carrier of serving cell and PUSCH transmission occasion if the UE is not provided tpc-Accumulation, where - The values are given in Table 7.1.1-1 - is a sum of TPC command values in a set of TPC command values with cardinality that the UE receives between symbols before PUSCH transmission occasion and symbols before PUSCH transmission occasion on active UL BWP of carrier of serving cell for PUSCH power control adjustment state , where is the smallest integer for which symbols before PUSCH transmission occasion is earlier than symbols before PUSCH transmission occasion - If the UE is provided PUSCH-DMRS-bundling = ‘enable’, and for processing TPC command values provided by DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI, transmission occasion i is a first transmission occasion in a nominal time window [6, TS 38.214], transmission occasion i-i0 is a first transmission occasion in the preceding nominal time domain window, if any, and fb, f, c (i, l ) is same for all transmission occasions in the nominal time domain window. … - is the PUSCH power control adjustment state for active UL BWP of carrier of serving cell and PUSCH transmission occasion if the UE is provided tpc-Accumulation, where - absolute values are given in Table 7.1.1-1. If the UE is provided PUSCH-DMRS-bundling = ‘enable’, and for processing TPC command values provided by DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI, transmission occasion i is a first transmission occasion in a nominal time window [6, TS 38.214], transmission occasion i-i0 is a first transmission occasion in the preceding nominal time domain window, if any, and fb, f, c (i, l ) is same for all transmission occasions in the nominal time domain window.

Huawei proposes to adopt the following TP1 (TS 38.213):

7.1.1 UE behaviour < Unchanged parts are omitted > - is the PUSCH power control adjustment state for active UL BWP of carrier of serving cell and PUSCH transmission occasion if the UE is not provided tpc-Accumulation, where - The values are given in Table 7.1.1-1 - is a sum of TPC command values in a set of TPC command values with cardinality that the UE receives between symbols before PUSCH transmission occasion and symbols before PUSCH transmission occasion on active UL BWP of carrier of serving cell for PUSCH power control adjustment state , where is the smallest integer for which symbols before PUSCH transmission occasion is earlier than symbols before PUSCH transmission occasion - If the UE is provided PUSCH-DMRS-Bundling =‘enabled’, and for processing TPC command values provided by DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI, - fb,f,c'i,l=fb,f,c'i-i0,l+m=0CDi-1δPUSCH,b,f,cm,l, where fb,f,c'0,l=fb,f,c0,l - If the transmission occasion i is the first transmission occasion within an actual time domain window determined as described in [6, TS 38.214], or if the transmission occasion i is a transmission occasion that is not within an actual time domain window, then fb,f,ci,l=fb,f,c'i,l, otherwise fb,f,ci,l=fb,f,c'i1,l where the transmission occasion i1 is the first transmission occasion within the same actual time domain window as the transmission occasion i. < Unchanged parts are omitted >

Huawei proposes to adopt the following TP2 (TS 38.213):

7.1.1 UE behaviour < Unchanged parts are omitted > - is the PUSCH power control adjustment state for active UL BWP of carrier of serving cell and PUSCH transmission occasion if the UE is provided tpc-Accumulation, where - absolute values are given in Table 7.1.1-1 - If the UE is provided PUSCH-DMRS-Bundling =‘enabled’, and for processing TPC command values provided by DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI, - fb,f,c'i,l=δPUSCH,b,f,ci,l, where fb,f,c'0,l = fb,f,c0,l - If the transmission occasion i is the first transmission occasion within an actual time domain window determined as described in [6, TS 38.214], or if the transmission occasion i is a transmission occasion that is not within an actual time domain window, then fb,f,ci,l=fb,f,c'i,l, otherwise fb,f,ci,l=fb,f,c'i1,l where the transmission occasion i1 is the first transmission occasion within the same actual time domain window as the transmission occasion i. < Unchanged parts are omitted >

vivo proposes to adopt the following TP1:

----------------------------------Start of TP#1 for section 7.1.1 of 38.213 V17.0.0---------------------------------- 7.1.1 UE behaviour If a UE transmits a PUSCH on active UL BWP b of carrier f of serving cell c using parameter set configuration with index j and PUSCH power control adjustment state with index l, the UE determines the PUSCH transmission power PPUSCH,b,f,c(i,j,qd,l) in PUSCH transmission occasion i as [dBm] where, - PCMAX,f,c(i)is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS38.101-2] and [8-3, TS38.101-3] for carrier f of serving cell c in PUSCH transmission occasion i. <<unchanged text omitted>> - fb,f,ci,l=fb,f,ci-i0,l+m=0CDi-1δPUSCH,b,f,c(m,l) is the PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion i if the UE is not provided tpc-Accumulation, where - The δPUSCH,b,f,c values are given in Table 7.1.1-1 - m=0CDi-1δPUSCH,b,f,c(m,l) is a sum of TPC command values in a set Di of TPC command values with cardinality CDi that the UE receives between KPUSCHi-i0-1 symbols before PUSCH transmission occasion i-i0 and KPUSCH(i) symbols before PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c for PUSCH power control adjustment state l, where i0>0 is the smallest integer for which KPUSCH(i-i0) symbols before PUSCH transmission occasion i-i0 is earlier than KPUSCH(i) symbols before PUSCH transmission occasion i - If the UE is provided PUSCH-DMRS-Bundling = ‘enabled’, - for a transmission occasion i occurs within a nominal time domain window determined as described in [6, TS 38.214], fb,f,ci,l=fb,f,c(i1,l), where transmission occasion i1 is a first transmission occasion within the nominal time domain window; - for the first transmission occasion i occurs after the nominal time domain window, fb,f,ci,l=fb,f,ci1,l+δaccuTDW, where δaccuTDW is the accumulation of the TPC command values that would take effect in the nominal time domain window. - If a PUSCH transmission is scheduled by a DCI format, KPUSCH(i) is a number of symbols for active UL BWP b of carrier f of serving cell c after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUSCH transmission - If a PUSCH transmission is configured by ConfiguredGrantConfig, KPUSCH(i) is a number of KPUSCH,min symbols equal to the product of a number of symbols per slot, Nsymbslot, and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP b of carrier f of serving cell c - If the UE has reached maximum power for active UL BWP b of carrier f of serving cell c at PUSCH transmission occasion i-i0 and m=0CDi-1δPUSCH,b,f,c(m,l)≥0, then fb,f,ci,l=fb,f,ci-i0,l - If UE has reached minimum power for active UL BWP b of carrier f of serving cell c at PUSCH transmission occasion i-i0 and m=0CDi-1δPUSCH,b,f,c(m,l)≤0, then fb,f,ci,l=fb,f,ci-i0,l - A UE resets accumulation of a PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c to fb,f,ck,l=0, k=0,1,…,i - If a configuration for a corresponding PO_UE_PUSCH,b,f,cj value is provided by higher layers - If a configuration for a corresponding αb,f,cj value is provided by higher layers where l is determined from the value of j as - If j>1 and the UE is provided higher SRI-PUSCH-PowerControl, l is the sri-PUSCH-ClosedLoopIndex value(s) configured in any SRI-PUSCH-PowerControl with the sri-P0-PUSCH-AlphaSetId value corresponding to j - If j>1 and the UE is not provided SRI-PUSCH-PowerControl or j=0, l=0 - If j=1, l is provided by the value of powerControlLoopToUse - fb,f,ci,l=δPUSCH,b,f,ci,l is the PUSCH power control adjustment state for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion if the UE is provided tpc-Accumulation, where - δPUSCH,b,f,c absolute values are given in Table 7.1.1-1 - If the UE is provided PUSCH-DMRS-Bundling = ‘enabled’, - for a transmission occasion i occurs within a nominal time domain window determined as described in [6, TS 38.214], then fb,f,ci,l=fb,f,c(i1,l), where transmission occasion i1 is a first transmission occasion within the nominal time domain window, - for the first transmission occasion i occurs after the nominal time domain window, fb,f,ci,l=δPUSCH,b,f,c(i2,l), where δPUSCH,b,f,ci2,l is the last TPC command value that would take effect in the nominal time domain window. If the UE transmits a PUSCH associated with the first RS resource index qd, the UE applies the first PO_UE_PUSCH,b,f,cj value, the first αb,f,cj value, and fb,f,ci,l for determining PPUSCH,b,f,c(i,j,qd,l). If the UE transmits a PUSCH associated with the second RS resource index qd, the UE applies the second PO_UE_PUSCH,b,f,cj value, the second αb,f,cj value, and fb,f,ci,l or fb,f,ci,0 if twoPUSCH-PC-AdjustmentStates is provided or not provided, respectively, for determining PPUSCH,b,f,c(i,j,qd,l). <<unchanged text omitted>> ----------------------------------End of TP#1 for section 7.1.1 of 38.213 V17.0.0----------------------------------

vivo proposes to adopt the following TP2:

------------------------------------------------Start of TP#2 for section 7.2.1 of 38.213 V17.0.0---------------------------------- 7.2.1 UE behaviour If a UE transmits a PUCCH on active UL BWP b of carrier f in the primary cell c using PUCCH power control adjustment state with index l, the UE determines the PUCCH transmission power PPUCCH,b,f,c(i,qu,qd,l) in PUCCH transmission occasion i as [dBm] where - PCMAX,f,c(i) is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS38.101-2] and [8-3, TS38.101-3] for carrier f of primary cell c in PUCCH transmission occasion i <<unchanged text omitted>> - gb,f,ci,l=gb,f,ci-i0,l+m=0CDi-1δPUCCH,b,f,c(m,l) is the current PUCCH power control adjustment state l for active UL BWP b of carrier f of primary cell c and PUCCH transmission occasion i, where - The δPUCCH,b,f,c values are given in Table 7.1.2-1 - m=0CDi-1δPUCCH,b,f,c(m,l) is a sum of TPC command values in a set Ci of TPC command values with cardinality CCi that the UE receives between KPUCCHi-i0-1 symbols before PUCCH transmission occasion i-i0 and symbols before PUCCH transmission occasion i on active UL BWP b of carrier f of primary cell c for PUCCH power control adjustment state, where i0>0 is the smallest integer for which KPUCCHi-i0 symbols before PUCCH transmission occasion i-i0 is earlier than KPUCCHi symbols before PUCCH transmission occasion i - If the UE is provided PUCCH-DMRS-Bundling = ‘enabled’, - for a transmission occasion i occurs within a nominal time domain window determined as described in [6, TS 38.214], gb,f,ci,l=gb,f,c(i1,l), where transmission occasion i1 is a first transmission occasion within the nominal time domain window; - for the first transmission occasion i occurs after the nominal time domain window, gb,f,ci,l=gb,f,ci1,l+δaccuTDW, where δaccuTDW is the accumulation of the TPC command values that would take effect in the nominal time domain window. - If the PUCCH transmission is in response to a detection by the UE of a DCI format, KPUCCHi is a number of symbols for active UL BWP b of carrier f of primary cell c after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUCCH transmission <<unchanged text omitted>> ------------------------------------------------End of TP#2 for section 7.2.1 of 38.213 V17.0.0----------------------------------

OPPO proposes to adopt the following TP (TS 38.213):

****************************Start of the TP**************************** 7.1 Physical uplink shared channel <text omitted> fb,f,ci,l=fb,f,ci-i0,l+m=0CDi-1δPUSCH,b,f,c(m,l) is the PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion i if the UE is not provided tpc-Accumulation, where - The δPUSCH,b,f,c values are given in Table 7.1.1-1 - For PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, PUSCH repetition Type A with a configured grant, PUSCH repetition Type B and TB processing over multiple slots, when PUSCH-DMRS-Bundling is enabled, if TPC command is received in DCI format 2_2 and the TPC command would take into effect during a nominal TDW, as defined in clause 6.1.7 of [6, TS 38.214], the TPC commands is not applied during the nominal TDW and would be applied after the nominal TDW. - m=0CDi-1δPUSCH,b,f,c(m,l) is a sum of TPC command values in a set Di of TPC command values with cardinality CDi that the UE receives between KPUSCHi-i0-1 symbols before PUSCH transmission occasion i-i0 and KPUSCH(i) symbols before PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c for PUSCH power control adjustment state l, where i0>0 is the smallest integer for which KPUSCH(i-i0) symbols before PUSCH transmission occasion i-i0 is earlier than KPUSCH(i) symbols before PUSCH transmission occasion i - If a PUSCH transmission is scheduled by a DCI format, KPUSCH(i) is a number of symbols for active UL BWP b of carrier f of serving cell c after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUSCH transmission - If a PUSCH transmission is configured by ConfiguredGrantConfig, KPUSCH(i) is a number of KPUSCH,min symbols equal to the product of a number of symbols per slot, Nsymbslot, and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP b of carrier f of serving cell c - If the UE has reached maximum power for active UL BWP b of carrier f of serving cell c at PUSCH transmission occasion i-i0 and m=0CDi-1δPUSCH,b,f,c(m,l)≥0, then fb,f,ci,l=fb,f,ci-i0,l - If UE has reached minimum power for active UL BWP b of carrier f of serving cell c at PUSCH transmission occasion i-i0 and m=0CDi-1δPUSCH,b,f,c(m,l)≤0, then fb,f,ci,l=fb,f,ci-i0,l - A UE resets accumulation of a PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c to fb,f,ck,l=0, k=0,1,…,i - If a configuration for a corresponding PO_UE_PUSCH,b,f,cj value is provided by higher layers - If a configuration for a corresponding αb,f,cj value is provided by higher layers where l is determined from the value of j as - If j>1 and the UE is provided higher SRI-PUSCH-PowerControl, l is the sri-PUSCH-ClosedLoopIndex value(s) configured in any SRI-PUSCH-PowerControl with the sri-P0-PUSCH-AlphaSetId value corresponding to j - If j>1 and the UE is not provided SRI-PUSCH-PowerControl or j=0, l=0 - If j=1, l is provided by the value of powerControlLoopToUse - fb,f,ci,l=δPUSCH,b,f,ci,l is the PUSCH power control adjustment state for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion if the UE is provided tpc-Accumulation, where - δPUSCH,b,f,c absolute values are given in Table 7.1.1-1 - For PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, PUSCH repetition Type A with a configured grant, PUSCH repetition Type B and TB processing over multiple slots, when PUSCH-DMRS-Bundling is enabled, if TPC command(s) is received in DCI format 2_2 , and if one or more TPC command(s) that would take effect in a nominal TDW, as defined in clause 6.1.7 of [6, TS 38.214], are received, only the last TPC command is applied after the nominal TDW while the others are omitted. <text omitted> ****************************End of the TP****************************

LG proposes to adopt the following TP:

================== Start of Text Proposal for TS38.213 ================== 7.1.1 UE behaviour <Other parts are omitted> - fb,f,ci,l=fb,f,ci-i0,l+m=0CDi-1δPUSCH,b,f,c(m,l) is the PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion i if the UE is not provided tpc-Accumulation, where - The δPUSCH,b,f,c values are given in Table 7.1.1-1 - When PUSCH-DMRS-Bundling is enabled and if transmission occasion i is within the first nominal TDW, m=0CDi-1δPUSCH,b,f,c(m,l) is a sum of TPC command values in a set Di of TPC command values with cardinality CDi that the UE receives between KPUSCHi-i1-i0-1 symbols before PUSCH transmission occasion i-i1-i0 and KPUSCH(i-i1) symbols before PUSCH transmission occasion i-i1 on active UL BWP b of carrier f of serving cell c for PUSCH power control adjustment state l, where i1≥0 is the largest integer that transmission occasion i1 and transmission occasion i are within same nominal TDW and i0>0 is the smallest integer for which KPUSCH(i-i1-i0) symbols before PUSCH transmission occasion i-i1-i0 is earlier than KPUSCH(i-i0) symbols before PUSCH transmission occasion i-i0 - When PUSCH-DMRS-Bundling is enabled and if transmission occasion i is not within the first nominal TDW, m=0CDi-1δPUSCH,b,f,c(m,l) is a sum of TPC command values in a set Di of TPC command values with cardinality CDi that the UE receives between KPUSCHi-i1-i0-1 symbols before PUSCH transmission occasion i-i1-i0 and KPUSCH(i-i1) symbols before PUSCH transmission occasion i-i1 on active UL BWP b of carrier f of serving cell c for PUSCH power control adjustment state l, where i1≥0 is the largest integer that transmission occasion i1 and transmission occasion i are within same nominal TDW and i0>0 is the largest integer that transmission occasion i-i1-i0 is within the previous nominal TDW, for which KPUSCH(i-i1-i0) symbols before PUSCH transmission occasion i-i1-i0 is earlier than KPUSCH(i-i0) symbols before PUSCH transmission occasion i-i0 - When PUSCH-DMRS-Bundling is not enabled, m=0CDi-1δPUSCH,b,f,c(m,l) is a sum of TPC command values in a set Di of TPC command values with cardinality CDi that the UE receives between KPUSCHi-i0-1 symbols before PUSCH transmission occasion i-i0 and KPUSCH(i) symbols before PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c for PUSCH power control adjustment state l, where i0>0 is the smallest integer for which KPUSCH(i-i0) symbols before PUSCH transmission occasion i-i0 is earlier than KPUSCH(i) symbols before PUSCH transmission occasion i - If a PUSCH transmission is scheduled by a DCI format, KPUSCH(i) is a number of symbols for active UL BWP b of carrier f of serving cell c after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUSCH transmission <Other parts are omitted> - gb,f,ci,l=gb,f,ci-i0,l+m=0CDi-1δPUCCH,b,f,c(m,l) is the current PUCCH power control adjustment state l for active UL BWP b of carrier f of primary cell c and PUCCH transmission occasion i, where - The δPUCCH,b,f,c values are given in Table 7.1.2-1 - When PUCCH-DMRS-Bundling is enabled, m=0CDi-1δPUCCH,b,f,c(m,l) is a sum of TPC command values in a set Ci of TPC command values with cardinality CCi that the UE receives between KPUCCHi-i0-1 symbols before PUCCH transmission occasion i-i0 and KPUCCHi symbols before PUCCH transmission occasion i on active UL BWP b of carrier f of primary cell c for PUCCH power control adjustment state, where i0>0 is the smallest integer for which KPUCCHi-i0 symbols before PUCCH transmission occasion i-i0 is earlier than KPUCCHi symbols before PUCCH transmission occasion i - m=0CDi-1δPUCCH,b,f,c(m,l) is a sum of TPC command values in a set Ci of TPC command values with cardinality CCi that the UE receives between KPUCCHi-i0-1 symbols before PUCCH transmission occasion i-i0 and KPUCCHi symbols before PUCCH transmission occasion i on active UL BWP b of carrier f of primary cell c for PUCCH power control adjustment state, where i0>0 is the smallest integer for which KPUCCHi-i0 symbols before PUCCH transmission occasion i-i0 is earlier than KPUCCHi symbols before PUCCH transmission occasion i - m=0CDi-1δPUCCH,b,f,c(m,l) is a sum of TPC command values in a set Ci of TPC command values with cardinality CCi that the UE receives between KPUCCHi-i0-1 symbols before PUCCH transmission occasion i-i0 and symbols before PUCCH transmission occasion i on active UL BWP b of carrier f of primary cell c for PUCCH power control adjustment state, where i0>0 is the smallest integer for which KPUCCHi-i0 symbols before PUCCH transmission occasion i-i0 is earlier than KPUCCHi symbols before PUCCH transmission occasion i <Other parts are omitted> ================== End of Text Proposal for TS38.213 ==================

Nokia proposes to adopt the following TP (TS 38.213):

***Unchanged text is omitted *** 7.1.1 UE behaviour ***Unchanged text is omitted *** - fb,f,ci,l=fb,f,ci-i0,l+m=0CDi-1δPUSCH,b,f,c(m,l) is the PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion i if the UE is not provided tpc-Accumulation, where - The δPUSCH,b,f,c values are given in Table 7.1.1-1 - m=0CDi-1δPUSCH,b,f,c(m,l) is a sum of TPC command values in a set Di of TPC command values with cardinality CDi that the UE receives between KPUSCHi-i0-1 symbols before PUSCH transmission occasion i-i0 and KPUSCH(i) symbols before PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c for PUSCH power control adjustment state l, where i0>0 is the smallest integer for which KPUSCH(i-i0) symbols before PUSCH transmission occasion i-i0 is earlier than KPUSCH(i) symbols before PUSCH transmission occasion i - If the UE is provided PUSCH-DMRS-Bundling = ‘enabled’, KPUSCH(i) is a number of symbols for active UL BWP b of carrier f of serving cell c from the first symbol of the nominal time domain window including the transmission occasion i and before a first symbol of the transmission occasion i, where the nominal time domain window is determined as described in [6, TS 38.214] - If the UE is not provided PUSCH-DMRS-Bundling = ‘enabled’, - If a PUSCH transmission is scheduled by a DCI format, KPUSCH(i) is a number of symbols for active UL BWP b of carrier f of serving cell c after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUSCH transmission - If a PUSCH transmission is configured by ConfiguredGrantConfig, KPUSCH(i) is a number of KPUSCH,min symbols equal to the product of a number of symbols per slot, Nsymbslot, and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP b of carrier f of serving cell c - If the UE has reached maximum power for active UL BWP b of carrier f of serving cell c at PUSCH transmission occasion i-i0 and m=0CDi-1δPUSCH,b,f,c(m,l)≥0, then fb,f,ci,l=fb,f,ci-i0,l - If UE has reached minimum power for active UL BWP b of carrier f of serving cell c at PUSCH transmission occasion i-i0 and m=0CDi-1δPUSCH,b,f,c(m,l)≤0, then fb,f,ci,l=fb,f,ci-i0,l - A UE resets accumulation of a PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c to fb,f,ck,l=0, k=0,1,…,i - If a configuration for a corresponding PO_UE_PUSCH,b,f,cj value is provided by higher layers - If a configuration for a corresponding αb,f,cj value is provided by higher layers where l is determined from the value of j as - If j>1 and the UE is provided higher SRI-PUSCH-PowerControl, l is the sri-PUSCH-ClosedLoopIndex value(s) configured in any SRI-PUSCH-PowerControl with the sri-P0-PUSCH-AlphaSetId value corresponding to j - If j>1 and the UE is not provided SRI-PUSCH-PowerControl or j=0, l=0 - If j=1, l is provided by the value of powerControlLoopToUse - If the UE is provided PUSCH-DMRS-Bundling = ‘enabled’ and tpc-Accumulation, fb,f,c(i,l) is the PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion i, which is identical for all transmission occasions belonging to a nominal TDW, and equal to the absolute value δPUSCH,b,f,c carried by the last TPC command before the first symbol of the nominal TDW including transmission occasion i, and provided by DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI - If the UE is not provided PUSCH-DMRS-Bundling = ‘enabled’, fb,f,ci,l=δPUSCH,b,f,ci,l is the PUSCH power control adjustment state for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion if the UE is provided tpc-Accumulation, where - δPUSCH,b,f,c absolute values are given in Table 7.1.1-1 ***Unchanged text is omitted *** 7.2.1 UE behaviour ***Unchanged text is omitted *** - For the PUCCH power control adjustment state for active UL BWP of carrier of primary cell and PUCCH transmission occasion - is a TPC command value included in a DCI format scheduling a PDSCH reception for active UL BWP of carrier of the primary cell that the UE detects for PUCCH transmission occasion , or is jointly coded with other TPC commands in a DCI format 2_2 with CRC scrambled by TPC-PUCCH-RNTI [5, TS 38.212], as described in clause 11.3 - if the UE is provided twoPUCCH-PC-AdjustmentStates and PUCCH-SpatialRelationInfo and if the UE is not provided twoPUCCH-PC-AdjustmentStates or PUCCH-SpatialRelationInfo - If the UE obtains a TPC command value from a DCI format scheduling a PDSCH reception and if the UE is provided PUCCH-SpatialRelationInfo, the UE obtains a mapping, by an index provided by p0-PUCCH-Id, between a set of pucch-SpatialRelationInfoId values and a set of values for closedLoopIndex that provide the value(s). If the UE receives an activation command indicating a value of pucch-SpatialRelationInfoId, the UE determines the value closedLoopIndex that provides the value of through the link to a corresponding p0-PUCCH-Id index - If the UE obtains one TPC command from a DCI format 2_2 with CRC scrambled by a TPC-PUCCH-RNTI, the value is provided by the closed loop indicator field in DCI format 2_2 - is the current PUCCH power control adjustment state for active UL BWP of carrier of primary cell and PUCCH transmission occasion , where - The values are given in Table 7.1.2-1 - is a sum of TPC command values in a set of TPC command values with cardinality that the UE receives between symbols before PUCCH transmission occasion and symbols before PUCCH transmission occasion on active UL BWP of carrier of primary cell for PUCCH power control adjustment state, where is the smallest integer for which symbols before PUCCH transmission occasion is earlier than symbols before PUCCH transmission occasion - If the UE is provided PUCCH-DMRS-Bundling = ‘enabled’, KPUCCH(i) is a number of symbols for active UL BWP b of carrier f of serving cell c from the first symbol of the time domain window including the transmission occasion i to the first symbol of the transmission occasion i, where the time domain window is determined as described in [6, TS 38.214] - If the UE is not provided PUCCH-DMRS-Bundling = ‘enabled’, - If the PUCCH transmission is in response to a detection by the UE of a DCI format, is a number of symbols for active UL BWP of carrier of primary cell after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUCCH transmission - If the PUCCH transmission is not in response to a detection by the UE of a DCI format, is a number of symbols equal to the product of a number of symbols per slot, , and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP of carrier of primary cell - If the UE has reached maximum power for active UL BWP of carrier of primary cell at PUCCH transmission occasion and , then - If UE has reached minimum power for active UL BWP of carrier of primary cell at PUCCH transmission occasion and , then ***Unchanged text is omitted ***

3.3 Inter-slot frequency hopping with inter-slot bundling

FL comments: This issue is discussed under AI 8.8.2.

3.4 RRC parameters

China Telecom proposes to update the description of the RRC parameter PUSCH-Window-Restart as follows.

| WI code | Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_cov_enh-Core | DM-RS bundling for PUSCH | PUSCH-Window-Restart | new | UE bundles PUSCH DM-RS remaining in a nominal time domain window after dynamic event(s) that violate power consistency and phase continuity requirements | ENUMERATED {enabled, disable } |  | in PUSCH-Config | UE-specific |

China Telecom proposes to update the description of the RRC parameter PUCCH-Window-Restart as follows.

| WI code | Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_cov_enh-Core | DM-RS bundling for PUCCH | PUCCH-Window-Restart | new | UE bundles PUCCH DM-RS remaining in a nominal time domain window after dynamic event(s) that violate power consistency and phase continuity requirements | ENUMERATED {enabled, disable } |  | in PUCCH-Config | UE-specific |

vivo proposes to update the following RRC parameter for JCE of PUSCH:

| Parameter Name | RAN2 Parent IE | RAN1 spec/ section | Value range | New or existing? | UE-specific or Cell-specific | Description |
| --- | --- | --- | --- | --- | --- | --- |
| PUSCH-TimeDomainWindowLength | PUSCH-config | 38.214 | FFS INTEGER {1…32} | new | UE-specific | Length of a nominal time domain window in number of consecutive slots for DMRS bundling for PUSCH. |
| PUSCH-inter-bundlingFrequencyHopping | PUSCH-config | 38.214 | ENUMERATED {enabled, disabled} | new | UE-specific | Enabling/disabling of frequency hopping with DMRS bundling for PUSCH repetitions. |
| PUSCH-inter-bundlingFrequencyHoppingLength | PUSCH-config | 38.214 | INTEGER {1...32} | new | UE-specific | Length (in slots) of a configured frequency hop for PUSCH with DMRS bundling. |

vivo proposes to update the following RRC parameter for JCE of PUCCH:

| Parameter Name | RAN2 Parent IE | RAN1 spec/ section | Value range | New or existing? | UE-specific or Cell-specific | Description |
| --- | --- | --- | --- | --- | --- | --- |
| PUCCH-DMRS -Bundling | FFS PUCCH-format1/2/3/4 | 38.213 | ENUMERATED {enabled, disabled} | new | [UE-specific] | Enabling/disabling of DM-RS bundling and time domain window for PUCCH repetitions. |
| PUCCH-TimeDomainWindowLength | [in PUCCH-Config] PUCCH-format1/2/3/4 | 38.213 | FFS INTEGER {1,2,4.8} | new | [UE-specific] | Length of a [nominal] time domain window in slots for DMRS bundling for PUCCH. |
| PUCCH-Window-Restart | PUCCH-config | 38.213 | ENUMERATED {enabled, disabled} | new | [UE-specific] | UE bundles PUCCH DM-RS slots remaining in a bundling nominal time domain window after a slot for which events violate power consistency and phase continuity requirements |

3.5 Others

CA/DC

Sharp: The simultaneous transmissions in multiple cells/carriers should not be supported when DMRS bundling is configured.

MediaTek: JCE is not supported for CA/DC cases in R-17.

LG: Considering PUSCH/PUCCH with high level of repetition number have lower priority, transmission power of PUSCH/PUCCH can be changed due to the uplink transmission of other carrier within the configured TDW. In such case that transmission power change induced by the other uplink transmission should be an event.

Autonomous UE Tx power changes

MediaTek: JCE may be affected by UE autonomous Tx power changes due to PL changes and P-MPR changes satisfying SAR requirements. Our preference would be for RAN4 to not restrict the UE’s ability to perform those functions during JCE. Proposal: No restriction on UE autonomous power adjustments (e.g., due to PL changes or P-MPR changes) for JCE.

Applicability of DMRS Bundling

Qualcomm: Restrict DMRS bundling for PUSCH to only MCS values that correspond to QPSK or lower modulation orders.

PTRS

Qualcomm: Support different criteria for activation of PTRS or its density for the case of joint channel estimation.

Huawei: To support the joint channel estimation in the case of that the phase continuity cannot be maintained by UE, the PT-RS like reference signal can be used to estimate and compensate phase jumps in the future.

Fig. Joint channel estimation can be performed across the phase jumps using PT-RS like reference signal

TA adjustment

LG observes that some clarification is required for the case where the slot for starting DMRS bundling is a reduced slot due to TA command.

- If the TDRA table of PUSCH repetition type A is not satisfied due to TA command, clarification that PUSCH transmission is not performed is required.

- When a reduced slot occurs due to the TA adjustment, clarification regarding the actual TDW boundary according to whether the actual repetition of PUSCH repetition type B is transmitted or not is necessary.

LG proposes to adopt the following TP for clarification that if the TDRA table of PUSCH repetition type A is not satisfied, PUSCH transmission is not performed.

================== Start of Text Proposal for TS38.214 ================== 6.1.2.1 Resource allocation in time domain <Other parts are omitted> For PUSCH repetition Type A, in case K>1, - If the PUSCH is scheduled by DCI format 0_1 or 0_2 - if AvailableSlotCounting is enabled, the same symbol allocation is applied across the N∙K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the N∙K slots determined for the PUSCH transmission, applying the same symbol allocation in each slot. If the UE determines that, for a PUSCH transmission in a slot, the number of symbols available for the PUSCH transmission is smaller than the value provided by SLIV, the UE does not transmit the PUSCH in the slot. - Otherwise, the same symbol allocation is applied across the N∙K consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the N∙K consecutive slots applying the same symbol allocation in each slot. If the UE determines that, for a PUSCH transmission in a slot, the number of symbols available for the PUSCH transmission is smaller than the value provided by SLIV, the UE does not transmit the PUSCH in the slot. - Else if the PUSCH is scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, the same symbol allocation is applied across the N∙K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the N∙K slots determined for the PUSCH transmission, applying the same symbol allocation in each slot. If the UE determines that, for a PUSCH transmission in a slot, the number of symbols available for the PUSCH transmission is smaller than the value provided by SLIV, the UE does not transmit the PUSCH in the slot. For TB processing over multiple slots: - For unpaired spectrum, the same symbol allocation is applied across the N∙K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the N∙K slots determined for the PUSCH transmission, applying the same symbol allocation in each slot. If the UE determines that, for a PUSCH transmission in a slot, the number of symbols available for the PUSCH transmission is smaller than the value provided by SLIV, the UE does not transmit the PUSCH in the slot. - For paired spectrum or supplementary uplink band, the same symbol allocation is applied across the N∙K consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the N∙K consecutive slots applying the same symbol allocation in each slot. If the UE determines that, for a PUSCH transmission in a slot, the number of symbols available for the PUSCH transmission is smaller than the value provided by SLIV, the UE does not transmit the PUSCH in the slot. <Other parts are omitted> ================== End of Text Proposal for TS38.214 ==================

Ericsson: Specify that if a UE transmits PUSCH or PUCCH with intra-slot frequency hopping, it is not expected to maintain power consistency and phase continuity across PUSCH DMRS or across PUCCH DMRS.

Panasonic: To specify that a UE expects to perform the same precoder of precoding cycling within an actual TDW.

LG: Enhance the UL power allocation priority rule in terms of PUSCH/PUCCH for coverage enhancement.

Apple proposes to adopt the following TP for updating the specification to capture that DMRS building can be applied to Repetition type A defined in Rel-15 /16 /17 and Repetition type B with configured grant (TS 38.214):

6.1.7 UE procedure for determining time domain windows for bundling DM-RS For PUSCH transmissions of PUSCH repetition Type A , PUSCH repetition Type B and TB processing over multiple slots, when PUSCH-DMRS-Bundling is enabled, and for PUCCH transmissions of PUCCH repetition, when PUCCH-DMRS-Bundling is enabled, the UE determines one or multiple nominal TDWs, as follows: - For PUSCH transmissions of repetition Type A, PUSCH repetition Type B and TB processing over multiple slots, the duration of each nominal TDW except the last nominal TDW, in number of consecutive slots, is: - Given by PUSCH-TimeDomainWindowLength, if configured.

Spreadtrum proposes to adopt the following TP for separate descriptions of dynamic grant and configured grant for bundling DMRS TDW of PUSCH repetition type B and TB processing over multiple slots (TS 38.214):

6.1.7 UE procedure for determining time domain windows for bundling DM-RS < Unchanged part is omitted > For PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, PUSCH repetition Type A with a configured grant, PUSCH repetition Type B scheduled by DCI format 0_1 or 0_2, PUSCH repetition Type B with a configured grant, TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, and TB processing over multiple slots with a configured grant, when PUSCH-DMRS-Bundling is enabled, and for PUCCH transmissions of PUCCH repetition, when PUCCH-DMRS-Bundling is enabled, the UE determines one or multiple nominal TDWs, as follows: < Unchanged part is omitted >

OPPO proposes to adopt the following TP for clarification on describing first PUSCH transmission, available slot and downlink monitoring (TS 38.214):

****************************Start of the TP**************************** 6.1.7 UE procedure for determining time domain windows for bundling DM-RS <text omitted> - For PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 and PUSCH repetition Type A with a configured grant, when AvailableSlotCounting is enabled, and for TB processing over multiple slots: - The start of the first nominal TDW is the first slot determined for the first PUSCH transmission. - The end of the last nominal TDW is the last slot determined for the last PUSCH transmission. - The start of any other nominal TDWs is the first slot determined for PUSCH transmission after the last slot determined for PUSCH transmission of a previous nominal TDW. - For PUSCH transmissions of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2 and PUSCH repetition Type A with a configured grant, when the UE is not configured with AvailableSlotCounting or when AvailableSlotCounting is disabled, and for PUSCH repetition type B: - The start of the first nominal TDW is the first slot scheduled for the first PUSCH transmission. - The end of the last nominal TDW is the last slot scheduled for the last PUSCH transmission. - The start of any other nominal TDWs is the first slot after the last slot of a previous nominal TDW. - For PUCCH transmissions of a PUCCH repetition: - The start of the first nominal TDW is the first slot determined for the first PUCCH transmission. - The end of the last nominal TDW is the last slot determined for the last PUCCH transmission. - The start of any other nominal TDWs is the first slot determined for PUCCH transmission after the last slot determined for PUCCH transmission of a previous nominal TDW. For PUSCH transmissions of a PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, PUSCH repetition Type A with a configured grant, PUSCH repetition Type B and TB processing over multiple slots, a nominal TDW consists of one or multiple actual TDWs. The UE determines the actual TDWs as follows: - The start of the first actual TDW is the first symbol of the first PUSCH transmission in a slot determined for PUSCH transmission of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots within the nominal TDW. - The end of an actual TDW is - The last symbol of the last PUSCH transmission in a slot determined for PUSCH transmission of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots within the nominal TDW, if the actual TDW reaches the end of the last PUSCH transmission within the nominal TDW. - The last symbol of a PUSCH transmission before the event, if an event occurs which causes power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots within the nominal TDW, and the PUSCH transmission is in a slot determined for PUSCH transmission of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A wth with a configured grant, or PUSCH repetition type B or TB processing over multiple slots. -<text omitted> Events which cause power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or PUCCH transmissions of PUCCH repetition, within the nominal TDW, are: - A downlink slot or downlink reception or downlink monitoring based on tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated for unpaired spectrum. Or, downlink RS or PDCCH monitoring as descried in clause 5, clause 6 and clause 11 of [6, TS 38.213]. - The gap between any two consecutive PUSCH transmissions, or the gap between any two consecutive PUCCH transmissions, exceeds 13 symbols. <text omitted> ****************************End of the TP****************************

- Email discussion (1st round)

4.1 Time domain window

Issue #1: The start of configured TDW for CG PUSCH

Proposal:

- For CG PUSCH, the first configured TDW should always start from the first physical slot of a CG period.

| Companies | Comments |
| --- | --- |

Issue #2: Candidate values of the window length L of the configured TDW

Companies are encourage to provide comments on issue #2 summarized in section 3.1.

| Companies | Comments |
| --- | --- |

Issue #3: Events that violate power consistency and phase continuity

Issue #3-1: Events for multi-TRP operations

Companies are encouraged to provide views on whether UL beam switching for multi-TRP operation is regarded as a semi-static event or dynamic event.

| Semi-static event | Dynamic event |
| --- | --- |
| [company name] |  |

Additional comments, if any?

| Companies | Comments |
| --- | --- |

vivo propose that PUCCH repetitions with different sets of power control parameters in multi-TRP operation should be regarded as an event.

Companies are encouraged to provide comments on the above proposal by vivo.

| Companies | Comments |
| --- | --- |

Issue #3-2: Dynamic & semi-static events

Companies are encouraged to provide answers to the following two questions:

- Case 1: A semi-static event is triggered after one or multiple dynamic events. Whether a new actual TDW is created after the semi-static event?

- Case 2: A semi-static event overlaps with a dynamic event. Whether a new actual TDW is created after the semi-static event?

| Companies | Comments |
| --- | --- |

China Telecom proposes to adopt the following TP to capture the agreement ‘UE is mandatory to support restarting DM-RS bundling due to semi-static events’ into the specification (TS 38.214):

- A new actual TDW is created when PUSCH-Window-Restart is enabled or in response to frequency hopping or in response to any event not triggered by DCI or MAC-CE. The start of a new actual TDW is the first symbol of the PUSCH transmission after the event which causes power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots within the nominal TDW, and the PUSCH transmission is in a slot for PUSCH transmission of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots. - A new actual TDW is created when PUCCH-Window-Restart is enabled or in response to frequency hopping or in response to any event not triggered by DCI or MAC-CE. The start of a new actual TDW is the first symbol of the PUCCH transmission after the event which causes power consistency and phase continuity not to be maintained across PUCCH transmissions of PUCCH repetition within the nominal TDW, and the PUCCH transmission is in a slot determined for transmission of the PUCCH.

Companies are encouraged to provide comments on the above TP by China Telecom.

| Companies | Comments |
| --- | --- |

vivo proposes that for PUSCH repetition type B, gap with more than 13 OFDM symbols created by invalid symbol pattern is considered as a semi-static event.

Companies are encouraged to provide comments on the above proposal by vivo.

| Companies | Comments |
| --- | --- |

Issue #3-3: Dropping/Collision rules

LG proposes to treat dropping/cancellation based on Rel-17 collision rules as an event.

Companies are encouraged to provide comments on the above proposal by LG.

| Companies | Comments |
| --- | --- |

Intel proposes to adopt the following TP to add the case when PUCCH repetition with low priority is dropped when overlapping with PUSCH transmission with high priority, which is defined in Clause 9 in TS38.213, as an event:

- For PUCCH transmissions of PUCCH repetition, a dropping or cancellation of a PUCCH transmission according to clause 9, clause 9.2.6 and clause 11.1 of [6, TS 38.213].

Companies are encouraged to provide comments on the above TP by Intel.

| Companies | Comments |
| --- | --- |

Issue #3-4: HD-FDD RedCap UE related issues

Huawei proposes that for HD-FDD RedCap UEs, an event is constituted if the scheduled UL symbols overlap with any symbol of an SS/PBCH block provided by ssb-PositionInBurst.

Spreadtrum proposes that a downlink reception or downlink monitoring based on higher layer singling or DCI format is regarded as an event.

Companies are encouraged to provide comments on the above proposals by Huawei and Spreadtrum.

| Companies | Comments |
| --- | --- |

Issue #3-5: Clarifications on the events

vivo: For extended CP case, support to define 11 symbols as the maximum gap length to maintain the power consistency and phase continuity.

Ericsson: Revise ‘Frequency hopping’ in the list of events in 38.214 to ‘Change in starting RB for inter-slot frequency hopping’.

LG: Other UL transmission in between PUSCH/PUCCH transmission is an event only on the same carrier.

Companies are encouraged to provide comments on the above proposals by vivo, Ericsson and LG.

| Companies | Comments |
| --- | --- |

4.2 TPC command

FL comments: It seems the majority support to confirm the working assumption. Ericsson pointed out that there may be some confusion in RAN1. Ericsson thinks absolute TPC is not supported for DCI format 2_2 and proposes to remove the second bullet in the working assumption.

Companies are encouraged to provide comments on whether absolute TPC is supported for DCI format 2_2.

| Companies | Comments |
| --- | --- |

FL comments: The working assumption was not captured into the specification due to the different understandings on the definition of transmission occasion during the Editor’s CR phase in RAN1#107e. Based on the contributions in RAN1#107b-e, it seems the majority support no redefinition of transmission occasion for PUSCH/PUCCH in Rel-17. Therefore, the following proposal is proposed.

Proposal:

- No redefinition of transmission occasion for PUSCH/PUCCH in Rel-17.

Companies are encouraged to provide comments on the above proposal.

| Companies | Comments |
| --- | --- |

FL comments: Many companies provided TPs. However, we need to conclude the above issues before discussing TPs.

4.3 RRC parameters

China Telecom proposes to update the description of the RRC parameters PUSCH-Window-Restart and PUCCH-Window-Restart as follows.

- UE bundles PUSCH DM-RS remaining in a nominal time domain window after dynamic event(s) that violate power consistency and phase continuity requirements

- UE bundles PUCCH DM-RS remaining in a nominal time domain window after dynamic event(s) that violate power consistency and phase continuity requirements

Companies are encouraged to provide comments on the above proposal.

| Companies | Comments |
| --- | --- |

FL comments: Regarding the proposal from vivo, RRC parameters about frequency hopping and PUCCH are discussed under AI 8.8.2, while the candidate values of the window length L of the configured TDW is discussed in issue #2.

Any other comments about RRC parameters?

| Companies | Comments |
| --- | --- |

4.4 Others

FL comments: Regarding the issues summarized in section 3.5. FL would like to discuss the following issues first since other issues seem not so critical or they have already been discussed in previous meetings. We can discuss them later if we have sufficient time.

Qualcomm proposes to restrict DMRS bundling for PUSCH to only MCS values that correspond to QPSK or lower modulation orders. In addition, RAN4 has agreed to only focus on the modulation orders not higher than QPSK [6].

Proposal:

- Restrict DMRS bundling for PUSCH to only MCS values that correspond to QPSK or lower modulation orders.

Companies are encouraged to provide comments on the above proposal.

| Companies | Comments |
| --- | --- |

Apple proposes to adopt the following TP for updating the specification to capture that DMRS building can be applied to Repetition type A defined in Rel-15 /16 /17 and Repetition type B with configured grant (TS 38.214):

6.1.7 UE procedure for determining time domain windows for bundling DM-RS For PUSCH transmissions of PUSCH repetition Type A , PUSCH repetition Type B and TB processing over multiple slots, when PUSCH-DMRS-Bundling is enabled, and for PUCCH transmissions of PUCCH repetition, when PUCCH-DMRS-Bundling is enabled, the UE determines one or multiple nominal TDWs, as follows: - For PUSCH transmissions of repetition Type A, PUSCH repetition Type B and TB processing over multiple slots, the duration of each nominal TDW except the last nominal TDW, in number of consecutive slots, is: - Given by PUSCH-TimeDomainWindowLength, if configured.

Companies are encouraged to provide comments on the above proposal.

| Companies | Comments |
| --- | --- |

LG observes that some clarification is required for the case where the slot for starting DMRS bundling is a reduced slot due to TA command.

- If the TDRA table of PUSCH repetition type A is not satisfied due to TA command, clarification that PUSCH transmission is not performed is required.

- When a reduced slot occurs due to the TA adjustment, clarification regarding the actual TDW boundary according to whether the actual repetition of PUSCH repetition type B is transmitted or not is necessary.

Companies are encouraged to provide comments on the above problem by LG.

| Companies | Comments |
| --- | --- |

Any other comments?

| Companies | Comments |
| --- | --- |

- Agreements at RAN1#107-e

Agreement:

Support Option 1’-a

Option 1’-a:

- If L is configured, the maximum value of window length L of the configured TDW should not exceed the maximum duration, which is reported as UE capability as the duration where UE is able to maintain power consistency and phase continuity subject to power consistency and phase continuity requirements.

- If L is not configured, the default value of L = min (maximum duration, duration of all PUSCH repetitions)

Agreement:

- For non-back-to-back PUSCH/PUCCH transmissions across consecutive slots, the other uplink transmission in the middle of two PUSCH/PUCCH transmissions constitutes an event that violates power consistency and phase continuity.

Conclusion:

- Dynamic indication of the window length L of the configured TDW by DCI or indicated by TDRA table with one additional entry is not supported.

Agreement:

This working Assumption is confirmed.

Working assumption: - The start of the first actual TDW is the first available symbol (at least determined by TDRA table) in available slot for the first PUSCH transmission in an available slot within the configured TDW. - The end of the actual TDW is - the last available symbol (at least determined by TDRA table) in available slot for the last PUSCH transmission in an available slot within the configured TDW if the actual TDW reaches the end of the last PUSCH transmission within the configured TDW. - the last available symbol (at least determined by TDRA table) in available slot of the PUSCH transmission right before the event if an event occurs that violates power consistency and phase continuity, and the PUSCH transmission is in an available slot. - For UE capable of restarting DM-RS bundling, the start of the new actual TDW is the first available symbol (at least determined by TDRA table) in available slot for PUSCH transmission after the event violates power consistency and phase continuity, and the PUSCH transmission is in an available slot.

Agreement:

- The action of gNB indicated TA commands constitutes an event that violates power consistency and phase continuity.

Agreement:

- If DM-RS bundling is supported, UE is mandatory to support restarting DM-RS bundling due to semi-static events. UE capability of restarting DMRS bundling is applied only to dynamic events.

  - An event is regarded as a dynamic event if it is triggered by a DCI or MAC-CE, otherwise it is regarded as a semi-static event.

  - Note: At least frequency hopping event is considered as semi-static event.

Working assumption:

- The action of group common TPC commands with format 2_2 does not constitute an event that violates power consistency and phase continuity.

  - If UE is configured to accumulate TPC commands,

    - If UE receives TPC commands that would take into effect during a configured TDW, UE accumulates TPC commands without taking effect during the current configured TDW. TPC commands take effect after the current configured TDW.

  - If UE is not configured to accumulate TPC commands

    - the last TPC command that would take effect within a configured TDW supersedes all previous TPC commands that take effect within that configured TDW and only the last TPC command is applied by the UE after the current configured TDW.

      - FFS: no more than 1 TPC command is expected to take effect during a configured TDW.

Agreement:

The following working Assumption is confirmed.

Working assumption: For joint channel estimation for PUSCH repetition type A of PUSCH repetitions of the same TB, all the repetitions are covered by one or multiple consecutive/non-consecutive configured TDWs. - Each configured TDW consists of one or multiple consecutive physical slots. - The window length L of the configured TDW(s) can be explicitly configured with a single value and L is no longer than the maximum duration. - FFS: The maximum value of L is the duration of all repetitions - FFS: Solutions to error propagation issue if for L is longer than the maximum duration is to be discussed further. - FFS: The window length L is configured per UL BWP - The start of the first configured TDW is the first PUSCH transmission - FFS: The first available slot/symbol, or the first physical slot/symbol for the first PUSCH transmission. - The start of other configured TDWs can be implicitly determined prior to first repetition. - FFS: The configured TDWs are consecutive for paired spectrum/SUL band - FFS: The start of the configured TDWs for unpaired spectrum is implicitly determined based on semi-static DL/UL configuration. - The end of the last configured TDW is the end of the last PUSCH transmission. - FFS: The end of the configured TDW is the last available slot/symbol, or the last physical slot/symbol for the last PUSCH transmission. - Within one configured TDW, one or multiple actual TDWs can be implicitly determined: - The start of the first actual TDW is the first PUSCH transmission within the configured TDW. - FFS: The first available slot/symbol, or the first physical slot/symbol for the first PUSCH transmission. - After one actual TDW starts, UE is expected to maintain the power consistency and phase continuity until one of the following conditions is met, then the actual TDW is ended. - The actual TDW reaches the end of the last PUSCH transmission within the configured TDW. - FFS: The end of the actual TDW is the last available slot/symbol, or the last physical slot/symbol for the last PUSCH transmission. - An event occurs that violates power consistency and phase continuity - FFS: The events may include e.g., a DL slot based on DL/UL configuration for unpaired spectrum, the actual TDW reaches the maximum duration, DL reception/monitoring occasion for unpaired spectrum, high priority transmission, frequency hopping, precoder cycling. - FFS: The end of the actual TDW is the last available slot/symbol of the PUSCH transmission right before an event such that the power consistency and phase continuity are violated. - If the power consistency and phase continuity are violated due to an event, whether a new actual TDW is created is subject to UE capability of supporting restarting DMRS bundling. - If UE is capable of restarting DM-RS bundling, one new actual TDW is created after the event, - FFS: The start of the new actual TDW is the first available slot/symbol for PUSCH transmission after the event. - If UE is not capable of restarting DM-RS bundling, no new actual TDW is created until the end of the configured TDW. - FFS: UE capability of restarting DMRS bundling is applied only to dynamic event or not Note 1: A ‘configured TDW’ refers to a time domain window whose length can be configured to ‘L’ and whose start and end is determined as described above. Note 2: An ‘actual TDW’ refers to a time domain window during whose entire duration the DM-RS bundling is actually applied. An ‘actual TDW’ duration is always less than or equal to the ‘configure TDW’ duration. Note 3: Whether the terms ‘configured TDW’ and ‘actual TDW’ are revised to other terms and if such terminology is used in specifications is to be further discussed.

Agreement:

- The candidate values of the window length L of the configured TDW can be any integer value that is larger than 1 and no larger than the maximum duration.

Agreement: The following agreement is clarified as follows.

- For PUSCH repetition type A counting based on available slots,

  - “The configured TDWs are determined based on available slots” in the agreement means “The start of the configured TDWs is determined based on available slots”

Agreement - For PUSCH repetition type A counting based on physical slots - The configured TDWs are consecutive, where the start of other configured TDWs is the first physical slot right after the last physical slot of a previous configured TDW. - For PUSCH repetition type A counting based on available slots - The configured TDWs are determined based on available slots, where start of a configured TDWs is the first available slot after the last available slot of a previous configured TDW. - Note: The determination of available slots for PUSCH repetition Type A is defined in AI 8.8.1.1.

Agreement:

- UE should not perform UE autonomous TA adjustment during the actual time domain window.

Agreement:

- The TDW determination procedure agreed for PUSCH repetition type A is reused, when applicable, for PUSCH repetition type B and TBoMS with or without repetition.

- No additional specification enhancements for PUSCH repetition type B and TBoMS.

Agreement:

- If DMRS bundling and UL beam switching for multi-TRP operation are configured simultaneously, UL beam switching for multi-TRP operation constitutes an event that violates power consistency and phase continuity.

  - FFS: UL beam switching for multi-TRP operation is regarded as a semi-static event.

- Agreements at RAN1#106b-e

Agreement:

- For PUSCH repetition type A counting based on physical slots

  - The start of the first configured TDW is the first physical slot for the first PUSCH transmission.

  - The end of the last configured TDW is the last physical slot for the last PUSCH transmission.

- For PUSCH repetition type A counting based on available slots

  - The start of the first configured TDW is the first available slot for the first PUSCH transmission.

  - The end of the last configured TDW is the last available slot for the last PUSCH transmission.

  - Note: The determination of available slots for PUSCH repetition Type A is defined in AI 8.8.1.1.

Conclusion:

- Joint channel estimation over PUSCH transmissions across non-consecutive slots is not supported in Rel-17.

Agreement:

Down-select one of the following options in this meeting:

Option 1:

- The maximum value of window length L of the configured TDW should not exceed the maximum duration, which is reported as UE capability as the duration where UE is able to maintain power consistency and phase continuity subject to power consistency and phase continuity requirements.

Option 1’:

- The maximum value of window length L of the configured TDW should not exceed the maximum duration, which is reported as UE capability as the duration where UE is able to maintain power consistency and phase continuity subject to power consistency and phase continuity requirements.

    - If L is not configured, the configured TDW length is equal to all repetitions

    - If L is not configured, default behavior should be defined, e.g., the configured TDW length is equal to all repetitions

Option 3’:

- Whether the window length L of the configured TDW can be longer than maximum duration is subject to UE capability.

  - If UE is capable of L being longer than maximum duration,

    - The maximum value of the window length L of the configured TDW is the duration of all repetitions.

      - FFS: whether L cannot be other values other than the duration of all repetitions, if it is longer than the maximum duration.

    - If L is longer than the maximum duration, UE does not expect dynamic events.

      - FFS: details of dynamic events

Agreement

- For DG-PUSCH, Type1 CG-PUSCH and Type2 CG-PUSCH, the window length L of the configured TDW is at least configured by RRC.

- FFS: For DG-PUSCH and Type2 CG-PUSCH, whether the window length L of the configured TDW can be indicated by DCI or indicated by TDRA table with one additional entry.

Agreement

- The window length L of the RRC configured TDW is configured separately for PUSCH and PUCCH.

  - For PUSCH, L is configured per BWP.

- FFS whether the window length L can be configured with each row in the TDRA table

Agreement

- For PUSCH repetition type A counting based on physical slots

  - The configured TDWs are consecutive, where the start of other configured TDWs is the first physical slot right after the last physical slot of a previous configured TDW.

- For PUSCH repetition type A counting based on available slots

  - The configured TDWs are determined based on available slots, where start of a configured TDWs is the next first available slot after the conclusion last available slot of a previous configured TDW.

  - Note: The determination of available slots for PUSCH repetition Type A is defined in AI 8.8.1.1.

Working assumption:

- The start of the first actual TDW is the first available symbol (at least determined by TDRA table) in available slot for the first PUSCH transmission in an available slot within the configured TDW.

- The end of the actual TDW is

  - the last available symbol (at least determined by TDRA table) in available slot for the last PUSCH transmission in an available slot within the configured TDW if the actual TDW reaches the end of the last PUSCH transmission within the configured TDW.

  - the last available symbol (at least determined by TDRA table) in available slot of the PUSCH transmission right before the event if an event occurs that violates power consistency and phase continuity, and the PUSCH transmission is in an available slot.

- For UE capable of restarting DM-RS bundling, the start of the new actual TDW is the first available symbol (at least determined by TDRA table) in available slot for PUSCH transmission after the event violates power consistency and phase continuity, and the PUSCH transmission is in an available slot.

Agreement

- For back-to-back PUSCH transmissions across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following case:

  - Over back-to-back PUSCH transmissions for one TB processed over multiple slots

    - It’s subject to UE capability

    - if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A

Agreement

- For non-back-to-back PUSCH transmissions across consecutive slots (no uplink transmission in the middle of two PUSCH transmissions), support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following case:

  - Over non-back-to-back PUSCH transmissions for one TB processed over multiple slots

    - It’s subject to UE capability

    - if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A

Agreement

Down-select one of the following options:

- Option 1: If DM-RS bundling is supported, UE is mandatory to support restarting DM-RS bundling due to semi-static events. UE capability of restarting DMRS bundling is applied only to dynamic events.

- Option 2: UE capability of restarting DMRS bundling is applied to both semi-static events and dynamic events.

Agreement

- Support at least the following events that violate power consistency and phase continuity.

  - Dropping/cancellation based on Rel-15/16 collision rules.

  - FFS: Rel-17 collision rules.

  - DL slot or DL reception/monitoring based on semi-static DL/UL configuration for unpaired spectrum.

  - FFS: Other UL transmission in between PUSCH/PUCCH transmissions.

  - Gap between two PUSCH/PUCCH transmissions exceeds 13 symbols.

  - FFS: Transmission parameters need to be changed due to network-indicated operations, including: Tx power, UL beam/TPMI, and RB allocation.

  - FFS: TPC command.

  - FFS: TA adjustment.

  - FFS: The actual TDW reaches the maximum duration.

  - FFS: Frequency hopping.

  - FFS: Precoder cycling.

  - FFS: other events.

  - FFS: whether events are semi-static events or dynamic events.

  - FFS: the time duration of an event.

Agreement

- Introduce two RRC parameters to indicate enabling of DM-RS bundling and the window length of the configured TDW respectively.

Agreement

- Introduce a new RRC parameter for when UE restarts a PUSCH bundling window

- Agreements at RAN1#106-e

Agreement: Confirm the following working assumption.

Working assumption:

- For non-back-to-back PUSCH transmissions (at least for the case of the same TB) across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following cases:

  - Over non-back-to-back PUSCH transmissions (of the same TB) for repetition type A scheduled by dynamic grant or configured grant.

  - Over non-back-to-back PUSCH transmissions (of the same TB) for repetition type B scheduled by dynamic grant or configured grant, if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A.

    - FFS: additional specification enhancements on top of that defined to support repetition Type A

    - Only for single layer transmissions

    - Subject to UE capability

  - FFS: Over non-back-to-back PUSCH transmissions with different TBs

  - FFS: Over non-back-to-back PUSCH transmissions for TBoMS

  - For the non-back-to-back PUSCH transmissions, it is defined as at least when there is no UL transmission between the two successive PUSCH transmissions

  - Subject to UE capability with details FFS (e.g., separate vs. joint capability for type A & type B, w.r.t. OFF power requirements, etc.)

- FFS: Joint channel estimation over non-back-to-back PUSCH transmissions with other uplink transmissions between the two successive PUSCH transmissions across consecutive slot.

Conclusion

- Optimization of DMRS location in time domain for PUSCH is not considered for joint channel estimation in Rel-17.

Agreement

- Joint channel estimation for PUSCH transmissions and the time domain window are jointly enabled or disabled via RRC configuration for a UE.

  - Note: Enabling/disabling of joint channel estimation for PUSCH transmissions means enabling/disabling of DMRS bundling for PUSCH transmissions under the condition of power consistency and phase continuity.

Agreement

Make down-selection between the following two alternatives:

- Alt 1: UE is not expected to receive TPC commands during the current time domain window.

- Alt 2: UE receives and accumulates TPC commands without taking effect during the current time domain window.

Agreement

- UE should not perform TA adjustment during the time domain window.

  - FFS: UE does not expect to receive TA command to indicate TA adjustment during the TDW.

  - FFS: UE ignores any TA command which indicates TA adjustment during the TDW.

  - FFS: UE performs TA adjustment after the TDW if it receives any TA command indicating TA adjustment during the TDW.

Working assumption:

For joint channel estimation for PUSCH repetition type A of PUSCH repetitions of the same TB, all the repetitions are covered by one or multiple consecutive/non-consecutive configured TDWs.

- Each configured TDW consists of one or multiple consecutive physical slots.

- The window length L of the configured TDW(s) can be explicitly configured with a single value and L is no longer than the maximum duration.

  - FFS: The maximum value of L is the duration of all repetitions

  - FFS: Solutions to error propagation issue if for L is longer than the maximum duration is to be discussed further.

  - FFS: The window length L is configured per UL BWP

- The start of the first configured TDW is the first PUSCH transmission

  - FFS: The first available slot/symbol, or the first physical slot/symbol for the first PUSCH transmission.

- The start of other configured TDWs can be implicitly determined prior to first repetition.

  - FFS: The configured TDWs are consecutive for paired spectrum/SUL band

  - FFS: The start of the configured TDWs for unpaired spectrum is implicitly determined based on semi-static DL/UL configuration.

- The end of the last configured TDW is the end of the last PUSCH transmission.

  - FFS: The end of the configured TDW is the last available slot/symbol, or the last physical slot/symbol for the last PUSCH transmission.

- Within one configured TDW, one or multiple actual TDWs can be implicitly determined:

  - The start of the first actual TDW is the first PUSCH transmission within the configured TDW.

    - FFS: The first available slot/symbol, or the first physical slot/symbol for the first PUSCH transmission.

  - After one actual TDW starts, UE is expected to maintain the power consistency and phase continuity until one of the following conditions is met, then the actual TDW is ended.

    - The actual TDW reaches the end of the last PUSCH transmission within the configured TDW.

      - FFS: The end of the actual TDW is the last available slot/symbol, or the last physical slot/symbol for the last PUSCH transmission.

    - An event occurs that violates power consistency and phase continuity

      - FFS: The events may include e.g., a DL slot based on DL/UL configuration for unpaired spectrum, the actual TDW reaches the maximum duration, DL reception/monitoring occasion for unpaired spectrum, high priority transmission, frequency hopping, precoder cycling.

      - FFS: The end of the actual TDW is the last available slot/symbol of the PUSCH transmission right before an event such that the power consistency and phase continuity are violated.

  - If the power consistency and phase continuity are violated due to an event, whether a new actual TDW is created is subject to UE capability of supporting restarting DMRS bundling.

    - If UE is capable of restarting DM-RS bundling, one new actual TDW is created after the event,

      - FFS: The start of the new actual TDW is the first available slot/symbol for PUSCH transmission after the event.

    - If UE is not capable of restarting DM-RS bundling, no new actual TDW is created until the end of the configured TDW.

    - FFS: UE capability of restarting DMRS bundling is applied only to dynamic event or not

Note 1: A ‘configured TDW’ refers to a time domain window whose length can be configured to ‘L’ and whose start and end is determined as described above.

Note 2: An ‘actual TDW’ refers to a time domain window during whose entire duration the DM-RS bundling is actually applied. An ‘actual TDW’ duration is always less than or equal to the ‘configure TDW’ duration.

Note 3: Whether the terms ‘configured TDW’ and ‘actual TDW’ are revised to other terms and if such terminology is used in specifications is to be further discussed.

- Agreements at RAN1#105-e

Agreement:

- Joint channel estimation over non-back-to-back PUSCH transmissions within one slot is not supported.

Agreement:

- Definition of the maximum duration: a maximum time duration during which UE is able to maintain power consistency and phase continuity subject to power consistency and phase continuity requirements.

- FFS whether or not such a definition is necessary for RAN1 specifications.

  - Note: whether such a definition is to be specified in RAN4 specifications is up to RAN4.

- FFS the maximum duration may be reported by UE.

- Note: it is understood that for a UE, the maximum duration is no less than the time domain window duration

Agreement: Send LS to RAN4 asking the following questions

- For joint channel estimation, is there a maximum duration during which UE is able to maintain power consistency and phase continuity under certain tolerance level? If any, how long is it?

  - What factors determine the maximum duration?

  - Whether the maximum duration should be the same for different cases for both PUSCH and PUCCH?

  - Whether the maximum duration is dependent on the modulation order of transmission, e.g., QPSK, 16QAM, 64QAM?

  - Whether the maximum duration is dependent on UL waveform (DFT-s-OFDM vs. OFDM)?

  - Whether the maximum duration is band specific?

  - Besides the factors listed above, whether or not the maximum duration is further dependent on UE capabilities (e.g., multiple possible values for a given set of factor(s)), and if so, whether the UE should report such a duration

Agreement:

- Optimization of DMRS granularity in time domain for PUSCH is not considered for joint channel estimation in Rel-17.

Agreement:

- For back-to-back PUSCH transmissions within one slot, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following cases:

  - Over back-to-back PUSCH transmissions (of the same TB) for repetition type B scheduled by dynamic grant or configured grant, if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A with consecutive slots

    - FFS: additional specification enhancements on top of that defined to support repetition Type A

    - Only for single layer transmissions

    - Subject to UE capability

- Joint channel estimation over back-to-back PUSCH transmissions with different TBs within one slot is not supported.

Working assumption:

- For non-back-to-back PUSCH transmissions (at least for the case of the same TB) across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following cases:

  - Over non-back-to-back PUSCH transmissions (of the same TB) for repetition type A scheduled by dynamic grant or configured grant.

  - Over non-back-to-back PUSCH transmissions (of the same TB) for repetition type B scheduled by dynamic grant or configured grant, if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A.

    - FFS: additional specification enhancements on top of that defined to support repetition Type A

    - Only for single layer transmissions

    - Subject to UE capability

  - FFS: Over non-back-to-back PUSCH transmissions with different TBs

  - FFS: Over non-back-to-back PUSCH transmissions for TBoMS

  - For the non-back-to-back PUSCH transmissions, it is defined as at least when there is no UL transmission between the two successive PUSCH transmissions

  - Subject to UE capability with details FFS (e.g., separate vs. joint capability for type A & type B, w.r.t. OFF power requirements, etc.)

- FFS: Joint channel estimation over non-back-to-back PUSCH transmissions with other uplink transmissions between the two successive PUSCH transmissions across consecutive slot.

Agreement:

- Joint channel estimation for PUSCH transmissions is enabled or disabled via RRC configuration for a UE

  - FFS: whether additional dynamic signaling is needed to enable/disable joint channel estimation for PUSCH transmissions

  - Note: the enabling of such a feature is subject to certain prerequisites

  - FFS RRC parameter details (including explicit vs. implicit configuration)

- FFS For joint channel estimation for PUSCH, the time domain window is not explicitly enabled or disabled separately from joint channel estimation.

Note: Enabling/disabling of joint channel estimation for PUSCH transmissions means enabling/disabling of DMRS bundling for PUSCH transmissions under the condition of power consistency and phase continuity.

Agreement:

For joint channel estimation for PUSCH repetition type A of PUSCH repetitions of the same TB, down select one of the following alternatives for the time domain window.

- Alt 1: All the repetitions are covered by one single time domain window

  - The start of the window is the first PUSCH transmission

  - FFS: how to handle non-consecutive physical slots for UL transmission, e.g., due to DL/UL configuration for unpaired spectrum

  - FFS: frequency hopping and precoder cycling

- Alt 2: All the repetitions are covered by one or multiple time domain windows

  - For the start of each window,

    - The start of the first window is the first PUSCH transmission.

    - FFS: how to determine the start of other windows, e.g., whether multiple windows are consecutive or non-consecutive, whether the start of the window depends on DL/UL configuration for unpaired spectrum

  - For the length of each window,

    - FFS Each window consists of at least two adjacent physical slots for UL transmission.

    - The length of each window is no longer than the maximum duration.

    - FFS: how to determine the length of each window

    - FFS: whether the length of each window depends on DL/UL configuration for unpaired spectrum

  - FFS: how to handle non-consecutive physical slots for UL transmission, e.g., due to DL/UL configuration for unpaired spectrum.

  - FFS: frequency hopping and precoder cycling

- Other alternatives are not precluded.

- Agreements at RAN1#104b-e

Agreements:

- For joint channel estimation, specify a time domain window during which a UE is expected to maintain power consistency and phase continuity among PUSCH transmissions subject to power consistency and phase continuity requirements.

  - FFS how the time domain window is determined (e.g., via explicit configuration and/or implicitly derived) and whether or not to have the possibility of enabling/disabling the time domain window

  - FFS the units the time domain window (e.g. repetitions, slots, and/or symbols)

    - FFS : association between the potential use case(s) and units of the time window

  - FFS: single or multiple time domain windows

- FFS: relation with UE capability

- FFS: whether the term "time domain window" is used in the specification or replaced by other technical terms

- FFS whether or not to further consider impacting of timing advance

Agreements:

- A new DMRS pattern equally spaced among PUSCH transmissions is not considered for joint channel estimation in Rel-17.

Agreements:

- For inter-slot frequency hopping with inter-slot bundling, down select on the following two options:

  - Option 1: The bundle size (time domain hopping interval) equals to the time domain window size.

  - Option 2: The bundle size (time domain hopping interval) can be different from the time domain window size.

    - FFS: Whether the bundle size (time domain hopping interval) is explicitly configured or implicitly determined.

    - FFS: Whether/How the bundle size (time domain hopping interval) is defined separately for FDD and TDD.

    - FFS: relation between the bundle size (time domain hopping interval) and the time domain window size

Conclusion:

- For optimization of DMRS granularity in time domain with joint channel estimation, the proponents are encouraged to provide more simulation results in next meeting

Agreements:

- For the time domain window for joint channel estimation, down select on the following two options:

  - Option 1: The unit of the time domain window is defined separately for the following PUSCH transmissions:

    - PUSCH repetition type A

    - PUSCH repetition type B, if agreed

    - TBoMS, if agreed

    - Different TB, if agreed

  - Option 2: The unit of the time domain window is the same for the following PUSCH transmission:

    - PUSCH repetition type A

    - PUSCH repetition type B, if agreed

    - TBoMS, if agreed

    - Different TB, if agreed

Agreement:

- For back-to-back PUSCH transmissions across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following cases:

  - Over back-to-back PUSCH transmissions (of the same TB) for repetition type B scheduled by dynamic grant or configured grant, if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A.

    - FFS: additional specification enhancements on top of that defined to support repetition Type A

    - Only for single layer transmissions

    - Subject to UE capability

  - FFS: Over back-to-back PUSCH transmissions with different TBs

- Agreements at RAN1#104e

Agreements:

- Following potential use cases are considered for joint channel estimation for PUSCH:

  - Use case 1: back-to-back PUSCH transmissions within one slot.

  - Use case 2: non-back-to-back PUSCH transmissions within one slot.

  - Use case 3: back-to-back PUSCH transmissions across consecutive slots.

  - Use case 4: non-back-to-back PUSCH transmissions across consecutive slots.

  - Use case 5: PUSCH transmissions across non-consecutive slots.

Note: RAN1 assumes “back-to-back PUSCH transmission” has zero gap in-between adjacent PUSCH transmissions.

Agreements:

- For back-to-back PUSCH transmissions across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation at least for the following case:

  - Over back-to-back PUSCH transmissions (of the same TB) for repetition type A scheduled by dynamic grant or configured grant

  - FFS details (including possible other cases)

Agreements:

- For joint channel estimation, define a time domain window is introduced to facilitate further discussion, during which UE is expected to maintain power consistency and phase continuity among PUSCH transmissions subject to power consistency and phase continuity requirements.

  - FFS: whether the window should be specified

  - FFS: the length of the time domain window is defined by a set of repetitions/slots/symbols

  - FFS: single or multiple time domain windows

- FFS: relation with UE capability

- FFS: the time domain window may or may not be configured or specified.

- FFS: whether the term "time domain window" is used in the specification or replaced by other technical terms

- FFS: Whether the window is determined by the power consistency and phase continuity requirements and/or by other factors is to be decided.

Agreements:

- Companies are encouraged to study optimization of DMRS granularity in time domain with joint channel estimation, including:

  - Use cases

  - Simulations results

  - Enhanced schemes, e.g.,

    - Different DMRS density for different PUSCH transmissions

    - No DMRS for some PUSCH transmissions

  - If applicable, impact of dynamic changes, e.g., cancellation of a repetition and companies report the evaluation method.

- Companies are encouraged to study optimization of DMRS location in time domain with joint channel estimation, including:

  - Use cases

  - Simulations results

  - Enhanced schemes, e.g.,

    - DMRS equally spaced among PUSCH transmissions

    - DMRS located in special slots

    - Orphan symbol上 used for DMRS

  - If applicable, impact of dynamic changes, e.g., cancellation of a repetition and companies report the evaluation method.

- Note: the simulation assumptions for DM-RS in TR 38.830 are used as baseline for performance evaluation on optimization of DMRS location/granularity in time domain.

  - Take into account impairments such as frequency offset, and report corresponding parametrization together with the results. Further discuss impairment details.

Working assumption:

- For back-to-back PUSCH transmissions across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following case:

  - Over back-to-back PUSCH transmissions for TB processing one TB processed over multiple slots

    - It’s subject to UE capability

Agreements:

- For joint channel estimation.

  - Take into account the residual frequency error, e.g., +/- 0.1 ppm as upper bound.

  - Companies can report other values and frequency error model.

- Reference

- 3GPP RP-202928, “New WID on NR coverage enhancements”, China Telecom, RAN#90e, December 7th – 11th, 2020.

- 3GPP RP-211566, “Revised WID on NR coverage enhancements”, China Telecom, RAN#92e, June 14th – 18th, 2021.

- 3GPP RP-212973, “Introduction of coverage enhancements in NR”, RAN1, RAN#94e, December 6th – 17th, 2021.

- 3GPP R4-2114991, “LS on joint channel estimation for PUSCH and PUCCH (R1-2106212, R4-2111706)”, Qualcomm, RAN4#100-e, Aug. 2021.

- 3GPP R4-2120002, Reply LS on PUCCH and PUSCH repetition, RAN4, Qualcomm, RAN4#101-e, November 1th – 12th, 2021.

- 3GPP R4-2120003, WF on phase continuity and power consistency for PUCCH and PUSCH transmissions, Huawei, HiSilicon, RAN4#101-e, November 1th – 12th, 2021.

- R1-2200053	Discussion on joint channel estimation for PUSCH	Huawei, HiSilicon

- R1-2200088	Remaining issues on joint channel estimation for PUSCH	vivo

- R1-2200113	Discussion on remaining issues for joint channel estimation for PUSCH	ZTE

- R1-2200162	Joint channel estimation for PUSCH coverage enhancements	Nokia, Nokia Shanghai Bell

- R1-2200207	Joint channel estimation for PUSCH	Samsung

- R1-2200238	Joint channel estimation for PUSCH	NTT DOCOMO, INC.

- R1-2200268	Discussion on joint channel estimation for PUSCH	Panasonic Corporation

- R1-2200279	Discussion on joint channel estimation for PUSCH	Spreadtrum Communications

- R1-2200303	Joint channel estimation for PUSCH	Qualcomm Incorporated

- R1-2200336	Consideration on Joint channel estimation for PUSCH	OPPO

- R1-2200381	Remaining details on joint channel estimation for PUSCH	Intel Corporation

- R1-2200422	Discussion on joint channel estimation for PUSCH	Apple

- R1-2200467	Discussion on joint channel estimation for PUSCH	xiaomi

- R1-2200486	Remaining issues on joint channel estimation	China Telecom

- R1-2200499	Joint channel estimation for PUSCH	Sharp

- R1-2200520	Discussions on joint channel estimation	InterDigital, Inc.

- R1-2200553	Discussion on Joint channel estimation over multi-slot	MediaTek Inc.

- R1-2200590	Remaining issues on joint channel estimation for PUSCH	CMCC

- R1-2200613	Discussions on joint channel estimation for PUSCH	LG Electronics

- R1-2200657	Remaining Issues for Joint Channel Estimation for PUSCH	Ericsson