3GPP TSG-RAN WG1 Meeting #109-e	R1-2203080

e-Meeting, May 9 – May 20, 2022

Agenda Item:	8.2.2

Source:	Huawei, HiSilicon, SIA

Title:	Remaining issues of PDCCH monitoring enhancement for 52-71GHz spectrum

Document for:	Discussion and Decision

Introduction

In this contribution, we focus on the remaining issues of the PDCCH monitoring enhancement for 52-71GHz spectrum.

Discussion

Multi-slot PDCCH monitoring capability

The following agreement regarding multi-slot monitoring capabilities was achieved in the previous meeting.

Agreement (RAN1#108-e) Revise the RAN1#107bis-e agreement/working assumption: For search space set configuration of multi-slot PDCCH monitoring: - monitoringSlotPeriodicityAndOffset and duration are appended with "-r17", and - For monitoringPeriodicityAndOffset-r17 - The values represent slots - Add periodicity values {32,64,128,5120,10240,20480} to the existing values in monitoringSlotPeriodicityAndOffset - Note: Total list of supported periodicity values that can be configured for SCS 480kHz and 960kHz: {1,2,4,5,8,10,16,20,32,40,64,80,128,160,320,640,1280,2560,5120,10240,20480} - For each periodicity value Xp - The value range for the offset O is {0 .. Xp-1} slots Offset O values that can be configured: {0, 4, 8, …, 4(Xp-1)4} slots - Note: There may be no need to introduce the term "Xp" in the specifications - The configured periodicity at least for Group (1) SSs is restricted to be an integer multiple of XsL slots - FFS: details of offset The configured offset is restricted to be an integer multiple of L slots - For duration-r17 - The values represent slots - The value range is {8, 12, …, 20476} - If duration-r17 is absent, the UE assumes the duration in slots is equal to L - The UE ignores duration-r17 for DCI format 2_0 - The configured duration is restricted to be an integer multiple of XsL slots at least for Group (1) SSs - The maximum duration is one less slot group of L slots than the configured periodicity - FFS: need to revise the definition of durationduration-r17 is the total number of slots in consecutive groups of L slots in which a Search Space can exist in every period as given by monitoringPeriodicityAndOffset-r17 - monitoringSymbolsWithinSlot applies to each slot in a slot group configured for multi-slot PDCCH monitoring - Note: This parameter can be directly re-used from earlier releases. - Introduce new parameter monitoringSlotsWithinSlotGroup-r17 - Working assumption: - The size is 8 bitsTwo sizes L are supported for this parameter: L=4 bits and L=8 bits - Each bit in monitoringSlotsWithinSlotGroup-r17 represents a slot in a slot group of L slots - The parameter monitoringSlotsWithinSlotGroup-r17 is applied in each of the L slots as determined by monitoringSlotPeriodicityAndOffset-r17 and duration-r17. - A slot in the slot each group of L slots is configured for multi-slot PDCCH monitoring if the corresponding bit in the slot group monitoringSlotsWithinSlotGroup-r17 is set to '1' - Note: Further configuration of the monitoring symbols in such a slot is done by monitoringSymbolsWithinSlot - For Group (1) SSs - The slots indicated in the bitmap should be consecutive per group of L slots - The number of slots configured for multi-slot PDCCH monitoring per slot group of Xs slots should be no larger than Ys according to at least one of the Xs,Ys supported by a UE - Working assumption: For Group (2) SSs - For Type0/0A/2 CSS - The slots indicated in monitoringSlotsWithinSlotGroup-r17 are not restricted to be consecutive - The number of slots configured for multi-slot PDCCH monitoring in monitoringSlotsWithinSlotGroup-r17 can be up to L - For Type1 CSS without dedicated RRC - The number of slots configured for multi-slot PDCCH monitoring in monitoringSlotsWithinSlotGroup-r17 per slot group of Xs slots should be no larger than M, where M is FFS

For Type1 CSS without dedicated RRC, the maximum number of slots configured for multi-slot PDCCH monitoring in monitoringSlotsWithinSlotGroup-r17 per slot group of Xs slots is still FFS.

A Type1-PDCCH CSS set is configured by ra-SearchSpace in PDCCH-ConfigCommon for a DCI format 1_0 with CRC scrambled by a RA-RNTI, a MsgB-RNTI, or a TC-RNTI on the primary cell. UE attempts to detect Type1-PDCCH CSS set within the RAR window (or MsgB window). If the number of slots configured for multi-slot PDCCH monitoring in monitoringSlotsWithinSlotGroup-r17 per slot group of Xs slots is limited to be no larger than M, then the maximum number of slots that can be used for a Type1-PDCCH CSS set reduces to MXs×R, where R denotes the number of slots of a given RAR window (or MsgB window). As a result, in order to maintain the opportunity for the Type1-PDCCH CSS sets, a larger M, e.g, M=Xs, is preferred.

- For Type1 CSS without dedicated RRC, the number of slots configured for multi-slot PDCCH monitoring in monitoringSlotsWithinSlotGroup-r17 per slot group of Xs slots is up to Xs slots.

DCI processing

In the previous meeting, it has been discussed whether/how to extend the following Rel-15 rule from slot-based PDCCH monitoring to multi-slot PDCCH monitoring:

TS 38.213 10.1 UE procedure for determining physical downlink control channel assignment … If a UE is provided - one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, or a CSS set by PDCCH-Config, and - a SI-RNTI, a P-RNTI, a RA-RNTI, a MsgB-RNTI, a SFI-RNTI, an INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per slot. …

During the discussion, it seems that there are two interpretations of the legacy rule as follows:

- Interpretation A: For any one of the RNTIs on the list, the UE is not expected to process information from more than one DCI with CRC scrambled with that specific RNTI;

- Interpretation B: The UE is not expected to process information from more than one DCI with CRC scrambled with any one of the RNTIs on the list.

It is necessary to align our understanding of the legacy rule before discussing its extension to multi-slot monitoring. We believe that the specification wording is clearly aligned with Interpretation A and Interpretation B is just a special and a more restricted version of Interpretation A.

Observation 1: For the legacy DCI processing rule the following interpretation should be adopted:

If a UE is provided

-	one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, or a CSS set by PDCCH-Config, and

-	a SI-RNTI, a P-RNTI, a RA-RNTI, a MsgB-RNTI, a SFI-RNTI, an INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI

the UE is not expected to process information from more than one DCI with CRC scrambled with that specific RNTI.

A direct extension from the above slot-based monitoring to multi-slot monitoring case is as follows:

- If a UE is provided

-	one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, or a CSS set by PDCCH-Config, and

-	a SI-RNTI, a P-RNTI, a RA-RNTI, a MsgB-RNTI, a SFI-RNTI, an INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI

then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with that RNTI per slot group.

SSSG switching

It has been discussed in RAN1 107b-e and 108-e whether SSSG switching is only supported between SSSGs with the same Xs,Ys PDCCH monitoring combination. We do not see why the two different SSSGs have to support the same Xs,Ys combination. In our view, this is an unnecessarily conservative approach to guarantee that the distance between two consecutive Ys SS slots on either side of the switching boundary is not less than Xs. For instance, (4,1) can switch to (8,1) as long as the distance between the last monitoring slot in (4,1) to the first monitoring slot in (8,1) is not less than 4. This can be easily supported by gNB implementation. Alternatively, UE can just drop monitoring the first SS in the “switched to” SSSG if the distance between two consecutive Ys SS slots on either side of the switching boundary is not less than Xs.

- SSSG switching is supported between SSSGs that correspond to the same or different Xs,Ys PDCCH monitoring combinations.

Cross-carrier/multi-cell operation

Agreement (RAN1#108-e) - For serving cells configured with 480 or 960 kHz SCS, the serving cells with the same SCS and Xs value are grouped together to determine a total BD/CCE budget for that group and the per-cell BD/CCE budget within the group. - Support UE capability signaling for 4 additional cases: - Case 4: Capability on the number of CCs with Rel-17 monitoring capability only - Range of pdcch-BlindDetectionCA-R17: {[4, 5, …, 16]} - Case 5: Capability on the number of CCs with Rel-15 monitoring capability and Rel-17 monitoring capability on different serving cells - pdcch-BlindDetectionCA-R15 for Rel-15 PDCCH monitoring capability - pdcch-BlindDetectionCA-R17 for Rel-17 PDCCH monitoring capability - Range of pdcch-BlindDetectionCA-R17 and pdcch-BlindDetectionCA-R15: {[1, 2, …, 15]} - Range of pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R17: {[4, 5, …, 16]} - Case 6: Capability on the number of CCs with Rel-16 monitoring capability and Rel-17 monitoring capability on different serving cells - pdcch-BlindDetectionCA-R16 for Rel-16 PDCCH monitoring capability - pdcch-BlindDetectionCA-R17 for Rel-17 PDCCH monitoring capability - Range of pdcch-BlindDetectionCA-R17 and pdcch-BlindDetectionCA-R16: {[1, 2, …, 15]} - Range of pdcch-BlindDetectionCA-R16 + pdcch-BlindDetectionCA-R17: {[3, 4, …, 16]} - Case 7: Capability on the number of CCs with Rel-15 monitoring capability, Rel-16 monitoring capability and Rel-17 monitoring capability on different serving cells - pdcch-BlindDetectionCA-R15 for Rel-15 PDCCH monitoring capability - pdcch-BlindDetectionCA-R16 for Rel-16 PDCCH monitoring capability - pdcch-BlindDetectionCA-R17 for Rel-17 PDCCH monitoring capability - Range of pdcch-BlindDetectionCA-R17, pdcch-BlindDetectionCA-R16, and pdcch-BlindDetectionCA-R15: {[1, 2, …, 15]} - Range of pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R16 + pdcch-BlindDetectionCA-R17: {[4, 5, …, 16]} - For the case with Rel-15 monitoring capability, Rel-16 monitoring capability and Rel-17 monitoring capability on different serving cells (case 7) or any combination of 2 of the capabilities (i.e. case 5, and case 6), the UE will report one or more combination of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16, pdcch-BlindDetectionCA-R17) as UE capability. If UE reports more than one combination of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16, pdcch-BlindDetectionCA-R17), as in Rel-16, the gNB configures which combination for the UE to use for scaling PDCCH monitoring capability if the number of CCs configured is larger than the reported capability. - FFS: Extension to NR-DC scenario

The extension to NR-DC scenario is FFS in the above agreement. This is discussed in the following lines. Using a similar approach as the above agreement for multi-carrier scenario for a single cell group, when a UE is configured for NR-DC operation with a total of NNR-DCDL,cells downlink cells on both the MCG and the SCG, UE capability signaling and the corresponding new RRC parameters for the following 4 additional cases should be supported:

- Case 4: All the downlink cells have SCS configuration μ∈{5,6}.

- Case 5: NNR-DC,r17DL,cells downlink cells have SCS configuration μ∈{5,6}, the UE is provided monitoringCapabilityConfig = r15monitoringcapability for NNR-DC,r15DL,cells downlink cells, the UE is not provided monitoringCapabilityConfig = r16monitoringcapability for any downlink cell.

- Case 6: NNR-DC,r17DL,cells downlink cells have SCS configuration μ∈{5,6}, the UE is provided monitoringCapabilityConfig = r16monitoringcapability for NNR-DC,r16DL,cells downlink cells, and the UE is not provided monitoringCapabilityConfig = r15monitoringcapability for any downlink cell.

- Case 7: NNR-DC,r17DL,cells downlink cells have SCS configuration μ∈{5,6}, the UE is provided monitoringCapabilityConfig = r15monitoringcapability for NNR-DC,r15DL,cells downlink cells and monitoringCapabilityConfig = r16monitoringcapability for NNR-DC,r16DL,cells downlink cells.

For each of the above cases, the reference numbers of cells for both the MCG and the SCG for PDCCH blind detection are RRC configured and UE capability to indicate the maximum values of the configured parameters should be introduced. To save some space, the detailed proposal regarding above four cases are provided in the appendix. In addition, a corresponding proposal is provided in our companion paper [2] on UE feature.

- When a UE is configured for NR-DC operation with a total of NNR-DCDL,cells downlink cells on both the MCG and the SCG, support UE capability signaling and the corresponding new RRC parameters for the following 4 additional cases:

- Case 4: All the downlink cells have SCS configuration μ∈{5,6}.

- Case 5: NNR-DC,r17DL,cells downlink cells have SCS configuration μ∈{5,6}, the UE is provided monitoringCapabilityConfig = r15monitoringcapability for NNR-DC,r15DL,cells downlink cells, the UE is not provided monitoringCapabilityConfig = r16monitoringcapability for any downlink cell.

- Case 6: NNR-DC,r17DL,cells downlink cells have SCS configuration μ∈{5,6}, the UE is provided monitoringCapabilityConfig = r16monitoringcapability for NNR-DC,r16DL,cells downlink cells, and the UE is not provided monitoringCapabilityConfig = r15monitoringcapability for any downlink cell.

- Case 7: NNR-DC,r17DL,cells downlink cells have SCS configuration μ∈{5,6}, the UE is provided monitoringCapabilityConfig = r15monitoringcapability for NNR-DC,r15DL,cells downlink cells and monitoringCapabilityConfig = r16monitoringcapability for NNR-DC,r16DL,cells downlink cells.

The detailed proposal regarding above four cases including UE capabilities and RRC parameters is provided in Appendix of this paper R1-2203080.

Conclusions

In this paper, remaining issues of the PDCCH monitoring enhancement for 52-71GHz spectrum are discussed, and the following are proposed:

Observation 1: For the legacy DCI processing rule the following interpretation should be adopted:

If a UE is provided

-	one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, or a CSS set by PDCCH-Config, and

-	a SI-RNTI, a P-RNTI, a RA-RNTI, a MsgB-RNTI, a SFI-RNTI, an INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI

the UE is not expected to process information from more than one DCI with CRC scrambled with that specific RNTI.

- For Type1 CSS without dedicated RRC, the number of slots configured for multi-slot PDCCH monitoring in monitoringSlotsWithinSlotGroup-r17 per slot group of Xs slots is up to Xs slots.

- If a UE is provided

-	one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, or a CSS set by PDCCH-Config, and

-	a SI-RNTI, a P-RNTI, a RA-RNTI, a MsgB-RNTI, a SFI-RNTI, an INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI

then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with that RNTI per slot group.

- SSSG switching is supported between SSSGs that correspond to the same or different Xs,Ys PDCCH monitoring combinations.

- When a UE is configured for NR-DC operation with a total of NNR-DCDL,cells downlink cells on both the MCG and the SCG, support UE capability signaling and the corresponding new RRC parameters for the following 4 additional cases:

- Case 4: All the downlink cells have SCS configuration μ∈{5,6}.

- Case 5: NNR-DC,r17DL,cells downlink cells have SCS configuration μ∈{5,6}, the UE is provided monitoringCapabilityConfig = r15monitoringcapability for NNR-DC,r15DL,cells downlink cells, the UE is not provided monitoringCapabilityConfig = r16monitoringcapability for any downlink cell.

- Case 6: NNR-DC,r17DL,cells downlink cells have SCS configuration μ∈{5,6}, the UE is provided monitoringCapabilityConfig = r16monitoringcapability for NNR-DC,r16DL,cells downlink cells, and the UE is not provided monitoringCapabilityConfig = r15monitoringcapability for any downlink cell.

- Case 7: NNR-DC,r17DL,cells downlink cells have SCS configuration μ∈{5,6}, the UE is provided monitoringCapabilityConfig = r15monitoringcapability for NNR-DC,r15DL,cells downlink cells and monitoringCapabilityConfig = r16monitoringcapability for NNR-DC,r16DL,cells downlink cells.

The detailed proposal regarding above four cases including UE capabilities and RRC parameters is provided in Appendix of this paper R1-2203080.

- Reference

- R1-2202713, “Feature lead summary #4 for B52.6 GHz PDCCH monitoring enhancements,” RAN1 #108-e, Moderator (Lenovo).

- R1- 2203083, “Rel-17 UE features for extension to 71 GHz,” RAN1 #109-e, Huawei, HiSilicon.

- Appendix

When a UE is configured for NR-DC operation with a total of NNR-DCDL,cells downlink cells on both the MCG and the SCG, support UE capability signaling and the corresponding new RRC parameters for the following 4 additional cases:

- Case 4: All the downlink cells have SCS configuration μ∈{5,6},

  - Support UE capabilities pdcch-BlindDetectionMCG-UE-r17 and pdcch-BlindDetectionSCG-UE-r17 on the maximum values for the new RRC parameters pdcch-BlindDetection4 for the MCG and pdcch-BlindDetection4 for the SCG respectively, where

    - RRC parameters pdcch-BlindDetection4 for the MCG and for the SCG indicate the reference numbers of cells with SCS configuration μ∈{5,6} for PDCCH blind detection, such that:

      - pdcch-BlindDetection4 for the MCG + pdcch-BlindDetection4 for the SCG <= pdcch-BlindDetectionCA-r17, if the UE reports pdcch-BlindDetectionCA-r17, or

      - pdcch-BlindDetection4 for the MCG + pdcch-BlindDetection4 for the SCG <=NNR-DCDL,cells if the UE does not report pdcch-BlindDetectionCA-r17.

    - If the UE reports pdcch-BlindDetectionCA-r17:

      - the value range of pdcch-BlindDetectionMCG-UE-r17 or of pdcch-BlindDetectionSCG-UE-r17 is {0, 1, …, pdcch-BlindDetectionCA-r17 - 1}, and

      - pdcch-BlindDetectionMCG-UE-r17 + pdcch-BlindDetectionSCG-UE-r17 >= pdcch-BlindDetectionCA-r17.

    - Otherwise, if NNR-DC,max,r17DL,cells is a maximum total number of downlink cells that have SCS configuration μ∈{5,6} and the UE is configured on both the MCG and the SCG for NR-DC as indicated in UE-NR-Capability:

      - the value range of pdcch-BlindDetectionMCG-UE-r17 or of pdcch-BlindDetectionSCG-UE-r17 is {1,2,3}, and

      - pdcch-BlindDetectionMCG-UE-r17 + pdcch-BlindDetectionSCG-UE-r17 >= NNR-DC,max,r17DL,cells.

- Case 5: NNR-DC,r17DL,cells downlink cells have SCS configuration μ∈{5,6}, the UE is provided monitoringCapabilityConfig = r15monitoringcapability for NNR-DC,r15DL,cells downlink cells, the UE is not provided monitoringCapabilityConfig = r16monitoringcapability for any downlink cell,

  - Support UE capabilities pdcch-BlindDetectionMCG-UE-r15-r17 and pdcch-BlindDetectionSCG-UE-r15-r17 on the maximum values for the existing RRC parameters c for the MCG and pdcch-BlindDetection3 for the SCG respectively, where

    - (Existing) RRC parameters pdcch-BlindDetection3 for the MCG and for the SCG indicate the reference numbers of cells with monitoringCapabilityConfig = r15monitoringcapability for PDCCH blind detection, such that

      - pdcch-BlindDetection3 for the MCG + pdcch-BlindDetection3 for the SCG <= pdcch-BlindDetectionCA-r15, if the UE reports pdcch-BlindDetectionCA-r15, or

      - pdcch-BlindDetection3 for the MCG + pdcch-BlindDetection3 for the SCG <=NNR-DC,r15DL,cells, if the UE does not report pdcch-BlindDetectionCA-r15.

    - If the UE reports pdcch-BlindDetectionCA-r15:

      - the value range of pdcch-BlindDetectionMCG-UE-r15-r17 or of pdcch-BlindDetectionSCG-UE-r15-r17 is {0, 1, …, pdcch-BlindDetectionCA-r15 - 1}, and

      - pdcch-BlindDetectionMCG-UE-r15-r17 + pdcch-BlindDetectionSCG-UE-r15-r17 >= pdcch-BlindDetectionCA-r15.

    - Otherwise, if NNR-DC,max,r15DL,cells is a maximum total number of downlink cells for which the UE is provided monitoringCapabilityConfig = r15monitoringcapability and the UE is configured on both the MCG and the SCG for NR-DC as indicated in UE-NR-Capability:

      - the value range of pdcch-BlindDetectionMCG-UE-r15-r17 or of pdcch-BlindDetectionSCG-UE-r15-r17 is {0,1,2}, and

      - pdcch-BlindDetectionMCG-UE-r15-r17 + pdcch-BlindDetectionSCG-UE-r15-r17 >= NNR-DC,max,r15DL,cells.

  - Support UE capabilities pdcch-BlindDetectionMCG-UE-r17-r15 and pdcch-BlindDetectionSCG-UE-r17-r15 on the maximum values for the new RRC parameters pdcch-BlindDetection4 for the MCG and pdcch-BlindDetection4 for the SCG respectively, where

    - RRC parameters pdcch-BlindDetection4 for the MCG and for the SCG indicate the reference numbers of cells with SCS configuration μ∈{5,6} for PDCCH blind detection, such that:

      - pdcch-BlindDetection4 for the MCG + pdcch-BlindDetection4 for the SCG <= pdcch-BlindDetectionCA-r17, if the UE reports pdcch-BlindDetectionCA-r17, or

      - pdcch-BlindDetection4 for the MCG + pdcch-BlindDetection4 for the SCG <=NNR-DC,r17DL,cells if the UE does not report pdcch-BlindDetectionCA-r17.

    - If the UE reports pdcch-BlindDetectionCA-r17:

      - the value range of pdcch-BlindDetectionMCG-UE-r17-r15 or of pdcch-BlindDetectionSCG-UE-r17-r15 is {0, 1, …, pdcch-BlindDetectionCA-r17 - 1}, and

      - pdcch-BlindDetectionMCG-UE-r17-r15 + pdcch-BlindDetectionSCG-UE-r17-r15 >= pdcch-BlindDetectionCA-r17.

    - Otherwise, if NNR-DC,max,r17DL,cells is a maximum total number of downlink cells has SCS configuration μ∈{5,6} and the UE is configured on both the MCG and the SCG for NR-DC as indicated in UE-NR-Capability:

      - the value range of pdcch-BlindDetectionMCG-UE-r17-r15 or of pdcch-BlindDetectionSCG-UE-r17-r15 is {0,1,2}, and

      - pdcch-BlindDetectionMCG-UE-r17-r15 + pdcch-BlindDetectionSCG-UE-r17-r15 >= NNR-DC,max,r17DL,cells.

- Case 6: NNR-DC,r17DL,cells downlink cells have SCS configuration μ∈{5,6}, the UE is provided monitoringCapabilityConfig = r16monitoringcapability for NNR-DC,r16DL,cells downlink cells, and the UE is not provided monitoringCapabilityConfig = r15monitoringcapability for any downlink cell,

  - Support UE capabilities pdcch-BlindDetectionMCG-UE-r16-r17 and pdcch-BlindDetectionSCG-UE-r16-r17 on the maximum values for existing parameters pdcch-BlindDetection2 for the MCG and pdcch-BlindDetection2 for the SCG respectively, where

    - (Existing) RRC parameters pdcch-BlindDetection2 for the MCG and for the SCG indicate the reference numbers of cells with monitoringCapabilityConfig = r16monitoringcapability for PDCCH blind detection, such that

      - pdcch-BlindDetection2 for the MCG + pdcch-BlindDetection2 for the SCG <= pdcch-BlindDetectionCA-r16, if the UE reports pdcch-BlindDetectionCA-r16, or

      - pdcch-BlindDetection2 for the MCG + pdcch-BlindDetection2 for the SCG <=NNR-DC,r16DL,cells, if the UE does not report pdcch-BlindDetectionCA-r16.

    - If the UE reports pdcch-BlindDetectionCA-r16:

      - the value range of pdcch-BlindDetectionMCG-UE-r16-r17 or of pdcch-BlindDetectionSCG-UE-r16-r17 is {0, 1, …, pdcch-BlindDetectionCA-r16 - 1}, and

      - pdcch-BlindDetectionMCG-UE-r16-r17 + pdcch-BlindDetectionSCG-UE-r16-r17 >= pdcch-BlindDetectionCA-r16.

    - Otherwise, if NNR-DC,max,r16DL,cells is a maximum total number of downlink cells for which the UE is provided monitoringCapabilityConfig = r16monitoringcapability and the UE is configured on both the MCG and the SCG for NR-DC as indicated in UE-NR-Capability:

      - the value range of pdcch-BlindDetectionMCG-UE-r16-r17 or of pdcch-BlindDetectionSCG-UE-r16-r17 is {0,1}, and

      - pdcch-BlindDetectionMCG-UE-r16-r17 + pdcch-BlindDetectionSCG-UE-r16-r17 >= NNR-DC,max,r16DL,cells.

  - Support UE capabilities pdcch-BlindDetectionMCG-UE-r17-r16 and pdcch-BlindDetectionSCG-UE-r17-r16 on the maximum values for the new parameters pdcch-BlindDetection4 for the MCG and pdcch-BlindDetection4 for the SCG respectively, where

    - RRC parameters pdcch-BlindDetection4 for the MCG and for the SCG indicate the reference numbers of cells with SCS configuration μ∈{5,6} for PDCCH blind detection, such that

      - pdcch-BlindDetection4 for the MCG + pdcch-BlindDetection4 for the SCG <= pdcch-BlindDetectionCA-r17, if the UE reports pdcch-BlindDetectionCA-r17, or

      - pdcch-BlindDetection4 for the MCG + pdcch-BlindDetection4 for the SCG <=NNR-DC,r17DL,cells if the UE does not report pdcch-BlindDetectionCA-r17.

    - If the UE reports pdcch-BlindDetectionCA-r17:

      - the value range of pdcch-BlindDetectionMCG-UE-r17-r16 or of pdcch-BlindDetectionSCG-UE-r17-r16 is {0, 1, …, pdcch-BlindDetectionCA-r17-1}, and

      - pdcch-BlindDetectionMCG-UE-r17 + pdcch-BlindDetectionSCG-UE-r17 >= pdcch-BlindDetectionCA-r17.

    - Otherwise, if NNR-DC,max,r17DL,cells is a maximum total number of downlink cells has SCS configuration μ∈{5,6} and the UE is configured on both the MCG and the SCG for NR-DC as indicated in UE-NR-Capability:

      - the value range of pdcch-BlindDetectionMCG-UE-r17-r16 or of pdcch-BlindDetectionSCG-UE-r17-r16 is {0,1,2}, and

      - pdcch-BlindDetectionMCG-UE-r17-r16 + pdcch-BlindDetectionSCG-UE-r17-r16 >= NNR-DC,max,r17DL,cells.

- Case 7: NNR-DC,r17DL,cells downlink cells have SCS configuration μ∈{5,6}, the UE is provided monitoringCapabilityConfig = r15monitoringcapability for NNR-DC,r15DL,cells downlink cells and monitoringCapabilityConfig = r16monitoringcapability for NNR-DC,r16DL,cells downlink cells,

  - Support UE capabilities pdcch-BlindDetectionMCG-UE-r15-r16r17 and pdcch-BlindDetectionSCG-UE-r15-r16r17 on the maximum values for the existing parameters pdcch-BlindDetection3 for the MCG and pdcch-BlindDetection3 for the SCG respectively, where

    - (Existing) RRC parameters pdcch-BlindDetection3 for the MCG and for the SCG indicate the reference numbers of cells with monitoringCapabilityConfig = r15monitoringcapability for PDCCH blind detection, such that

      - pdcch-BlindDetection3 for the MCG + pdcch-BlindDetection3 for the SCG <= pdcch-BlindDetectionCA-r15, if the UE reports pdcch-BlindDetectionCA-r15, or

      - pdcch-BlindDetection3 for the MCG + pdcch-BlindDetection3 for the SCG <=NNR-DC,r15DL,cells, if the UE does not report pdcch-BlindDetectionCA-r15.

    - If the UE reports pdcch-BlindDetectionCA-r15:

      - the value range of pdcch-BlindDetectionMCG-UE-r15-r16r17 or of pdcch-BlindDetectionSCG-UE-r15-r16r17 is {0, 1, …, pdcch-BlindDetectionCA-r15-1}, and

      - pdcch-BlindDetectionMCG-UE-r15-r16r17 + pdcch-BlindDetectionSCG-UE-r15-r16r17 >= pdcch-BlindDetectionCA-r15

    - Otherwise, if NNR-DC,max,r15DL,cells is a maximum total number of downlink cells for which the UE is provided monitoringCapabilityConfig = r15monitoringcapability and the UE is configured on both the MCG and the SCG for NR-DC as indicated in UE-NR-Capability:

      - the value range of pdcch-BlindDetectionMCG-UE-r15-r16r17 or of pdcch-BlindDetectionSCG-UE-r15-r16r17 is {0,1,2}, and

      - pdcch-BlindDetectionMCG-UE-r15-r16r17 + pdcch-BlindDetectionSCG-UE-r15-r16r17 >= NNR-DC,max,r15DL,cells.

  - Support UE capabilities pdcch-BlindDetectionMCG-UE-r16-r15r17 and pdcch-BlindDetectionSCG-UE-r16-r15r17 on the maximum values for the existing parameters pdcch-BlindDetection2 for the MCG and pdcch-BlindDetection2 for the SCG respectively, where

    - (Existing) RRC parameters pdcch-BlindDetection2 for the MCG and for the SCG indicate the reference numbers of cells with monitoringCapabilityConfig = r16monitoringcapability for PDCCH blind detection, such that

      - pdcch-BlindDetection2 for the MCG + pdcch-BlindDetection2 for the SCG <= pdcch-BlindDetectionCA-r16, if the UE reports pdcch-BlindDetectionCA-r16, or

      - pdcch-BlindDetection2 for the MCG + pdcch-BlindDetection2 for the SCG <=NNR-DC,r16DL,cells, if the UE does not report pdcch-BlindDetectionCA-r16.

    - If the UE reports pdcch-BlindDetectionCA-r16:

      - the value range of pdcch-BlindDetectionMCG-UE-r16-r15r17 or of pdcch-BlindDetectionSCG-UE-r16-r15r17 is {0, 1, …, pdcch-BlindDetectionCA-r16-1}, and

      - pdcch-BlindDetectionMCG-UE-r16-r15r17 + pdcch-BlindDetectionSCG-UE-r16-r15r17 >= pdcch-BlindDetectionCA-r16.

    - Otherwise, if NNR-DC,max,r16DL,cells is a maximum total number of downlink cells for which the UE is provided monitoringCapabilityConfig = r16monitoringcapability and the UE is configured on both the MCG and the SCG for NR-DC as indicated in UE-NR-Capability:

      - the value range of pdcch-BlindDetectionMCG-UE-r16-r15r17 or of pdcch-BlindDetectionSCG-UE-r16-r15r17 is {0,1}, and

      - pdcch-BlindDetectionMCG-UE-r16-r15r17 + pdcch-BlindDetectionSCG-UE-r16-r15r17 >= NNR-DC,max,r16DL,cells.

  - Support UE capabilities pdcch-BlindDetectionMCG-UE-r17-r15r16 and pdcch-BlindDetectionSCG-UE-r17-r15r16 on the maximum values for the new parameters pdcch-BlindDetection4 for the MCG and pdcch-BlindDetection4 for the SCG respectively, where

    - RRC parameter pdcch-BlindDetection4 for the MCG and for the SCG indicate the reference numbers of cells with SCS configuration μ∈{5,6} for PDCCH blind detection, such that

      - pdcch-BlindDetection4 for the MCG + pdcch-BlindDetection4 for the SCG <= pdcch-BlindDetectionCA-r17, if the UE reports pdcch-BlindDetectionCA-r17, or

      - pdcch-BlindDetection4 for the MCG + pdcch-BlindDetection4 for the SCG <=NNR-DC,r17DL,cells if the UE does not report pdcch-BlindDetectionCA-r17.

    - If the UE reports pdcch-BlindDetectionCA-r17:

      - the value range of pdcch-BlindDetectionMCG-UE-r17-r15r16 or of pdcch-BlindDetectionSCG-UE-r17-r15r16 is {0, 1, …, pdcch-BlindDetectionCA-r17-1}, and

      - pdcch-BlindDetectionMCG-UE-r17-r15r16 + pdcch-BlindDetectionSCG-UE-r17-r15r16 >= pdcch-BlindDetectionCA-r17.

    - Otherwise, if NNR-DC,max,r17DL,cells is a maximum total number of downlink cells has SCS configuration μ∈{5,6} and the UE is configured on both the MCG and the SCG for NR-DC as indicated in UE-NR-Capability:

      - the value range of pdcch-BlindDetectionMCG-UE-r17-r15r16 or of pdcch-BlindDetectionSCG-UE-r17-r15r16 is {0,1,2}, and

      - pdcch-BlindDetectionMCG-UE-r17-r15r16 + pdcch-BlindDetectionSCG-UE-r17-r15r16 >= NNR-DC,max,r17DL,cells.