# 3GPP TSG RAN WG1 #109-e		 	                                   R1-2203243

# e-Meeting, May 9th – May 20th, 2022

# Source:	ZTE

# Title:	Discussion on other issues for Rel-18 NTN

# Agenda Item:	9.12.4

# Document for:  Discussion

Introduction

In RAN#95e [1], the WID of Rel-18 NR-NTN was approved and the performance enhancement objective in terms of throughput is listed as below.

- Study detailed regulatory requirement (e.g. accuracy, privacy, reliability, latency) for network-verified UE location for potential use cases/services (i.e. emergency call, lawful intercept, public warning, charging/billing) (at RAN plenary, from RAN#95 to RAN#96). [RAN]

  - Including further clarification on network verified UE location and its relationship to network-based positioning [RAN]

  - Study and evaluate, if needed, solutions for network to verify UE reported location information [RAN2,RAN1,RAN3]

In this contribution, the performance of legacy network based positioning methods is evaluated in NTN.

Application of legacy positioning methods in NTN

              - In legacy TN, the RAT dependent network based positioning methods may require multiple BS/TRP to work as anchor nodes for positioning. For example, in multi-RTT methods, network should collect the RTTs between target UE and different BSs to estimate the UE position as shown in Figure 1.

Figure 1 Illustration of multi-RTT method

              - However, in NTN, multiple available satellites at same time cannot be ensured currently. The legacy methods in TN, which utilize multiple BSs for positioning, are hard to be directly adopted in NTN. But thanks to the mobility of satellite, multiple RTTs can be obtained by only one satellite as shown in Figure 2 if the processing time of positioning is allowed to be long. For the single satellite based positioning, although multiple RTTs can be obtained by setting the satellite position at different time as anchor nodes, the performance cannot be as good as in legacy TN since the anchor nodes are not well spread in space. The performance should be evaluated to see whether the regulatory requirements can be satisfied.

Figure 2 Illustration of positioning by single satellite

Observation 1: Single satellite based verification solution should be considered in Rel-18 since multiple available satellite cannot be guaranteed at the same time.

Observation 2: The performance of single-satellite based RAT-dependent positioning method is not as good as in legacy TN scenario.

Proposal 1: The performance of single-satellite based RAT-dependent positioning method should be evaluated to see whether regulatory requirements are satisfied.

Evaluation of single-satellite based multi-RTT method

              - In the evaluation, only one serving satellite is considered to measure the RTTs at different time instant. Since 3D positioning is to be performed, 4 RTT measurements are assumed. Moreover, in order to simplify the expression, the center of nadir beam is assumed as the origin and the orbit of satellite is within the y-z plane. The detailed parameters for evaluation are listed in Table 1.

              - Table 1 Parameters for evaluation

| - Parameter | - Value |
| --- | --- |
| Orbit | LEO-600 |
| Number of measurements | 4 |
| Period of measurement | [10 ms, 100 ms, 1 s, 10 s, 30 s, 60 s] |
| Initial satellite location | Above nadir point |
| UE location | Nadir point is set as origin UE is randomly located in a square region: x = 5km + [-100,100]m, y = 500m + [-100,100]m, z=0 |
| Max RTT measurement error | [1 ns, 5 ns, 50 ns] |
| Monte Carlo simulation times | 100 |

              - For the scenarios listed in Table 1, the evaluated positioning error are illustrated in Figure 3. From the figure, it can be observed that the average positioning error can be smaller than 1km if the period of measurement is long enough and measurement error is negligible. However, if 50ns measurement error is assumed, the average positioning error can be as large as 10km even if 60s measurement period is adopted. Therefore, single-satellite based positioning method can be used for the scenarios where there is no high requirement on accuracy, e.g., identify UE belonging when it locates near the country boarder. For the use case where high positioning accuracy is needed, e.g., emergency call, this method cannot be used.

Figure 3 Evaluation of positioning error of single-satellite based multi-RTT method

Proposal 2: The average positioning error of single-satellite based positioning method can be as large as several kilometers.

Proposal 3: The single-satellite based positioning method can be considered for coarse verification of UE position.

Conclusions

In this contribution, analysis on improved GNSS operation for IoT-NTN is conducted with following proposals and observations:

Observation 1: Single satellite based verification solution should be considered in Rel-18 since multiple available satellite cannot be guaranteed at the same time.

Observation 2: The performance of single-satellite based RAT-dependent positioning method is not as good as in legacy TN scenario.

Proposal 1: The performance of single-satellite based RAT-dependent positioning method should be evaluated to see whether regulatory requirements are satisfied.

Proposal 2: The average positioning error of single-satellite based positioning method can be as large as several kilometers.

Proposal 3: The single-satellite based positioning method can be considered for coarse verification of UE position.

- Reference

- RP-220953 NR NTN (Non-Terrestrial Networks) enhancements, RAN#95e