3GPP TSG RAN WG1 #109-e	                                         	R1-2203339

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.6.2

Source:	Spreadtrum Communications

Title:	Discussion on evaluation needs and assumptions for eRedCap

Document for:	Discussion and decision

Introduction

When the foundation has been laid in R17, enhancements can be considered to improve the support for the mentioned use cases and also to expand RedCap into a new range of use cases such as smart grid. To further expand the market for RedCap use cases with relatively low cost, low energy consumption, and low data rate requirements, e.g., industrial wireless sensor network use cases, some further complexity reduction enhancements should be considered.

According to the R18 RedCap SID [1], at least the following solutions should be studied.

- Study further UE complexity reduction techniques based on Rel-17 evaluation methodology in TR 38.875 [RAN1] - Consider network impact, coexistence of Rel-17 and Rel-18 RedCap and non-RedCap UEs in a cell, UE impact, specification impact - Potential solutions, which may complement each other, for reducing device complexity are focusing on: - UE bandwidth reduction to 5MHz in FR1, - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI - reduced UE peak data rate in FR1, - Possibly including restricted bandwidth for PDSCH and/or PUSCH - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI

In this contribution, we discuss the evaluation needs and assumptions for R18 RedCap.

Cost evaluation based on R17 evaluation methodology

Cost evaluation baseline and methodology

During the R17 study item phase, the comparison reference device and comparison method of cost evaluation are very clear. The comparison reference device is NR devices captured in chapter 6.1 of TR 38.875 [2], and the detailed cost for the reference NR devices are copied below [2]. The RF-to-baseband cost ratio was assumed to be 40:60 for an FR1 UE.

Table 6.1-1: Detailed cost breakdown for the reference NR devices Functional block; FR1 FDD (2Rx); FR1 TDD (4Rx). RF. Antenna array for FR2. Power amplifier; ~25%; ~25%. Filters; ~10%; ~15%. RF transceiver (including LNAs, mixer, and local oscillator); ~45%; ~55%. Duplexer / Switch; ~20%; ~5%. Baseband. ADC / DAC; ~10%; ~9%. FFT/IFFT; ~4%; ~4%. Post-FFT data buffering; ~10%; ~10%. Receiver processing block; ~24%; ~29%. LDPC decoding; ~10%; ~9%. HARQ buffer; ~14%; ~12%. DL control processing & decoder; ~5%; ~4%. Synchronization / cell search block; ~9%; ~9%. UL processing block; ~5%; ~5%. MIMO specific processing blocks; ~9%; ~9%.

Based on the above reference device, the comparison method is introducing one or more features on top of the reference NR device, as the feature(s) will reduce the cost of some components, then, by comparing the rest cost of each component with the reference device, we can get the cost reduction of the feature(s).

For R18 RedCap cost evaluation, as there is an intermediate device (i.e., R17 RedCap) between R18 RedCap and the established NR devices, there are many possibilities in comparison reference devices and comparison methods. For example, the reference device can be established NR devices or R17 RedCap devices. Obviously, different comparison reference devices and comparison methods will lead to different evaluation results. Therefore, before the detailed cost evaluation, a unified evaluation baseline and methodology is required.

Proposal 1: For R18 cost evaluation, a unified evaluation baseline and methodology is required.

Based on the previous discussion (e.g., R18 RWS) and our understanding, there are three options for R18 evaluation, and they are listed below:

Table 2.1-1: Possible cost evaluation methodologies for R18 based on R17 methodology

| Options | Reference device | Comparison method |
| --- | --- | --- |
| Option 1 | The current reference device i.e., reference NR devices established in TR 38.875 | R17 single feature vs. R18 single feature e.g., 100MHz vs.20MHz vs.5MHz |
| Option 2 | The current reference device i.e., reference NR devices established in TR 38.875 | Compared to the simplest R17 RedCap e.g., all R17 RedCap features vs. all R17 RedCap features+R18 feature(s) |
| Option 3 | Establish a new reference R17 RedCap devices | Evaluation on R18 feature(s) based on the new reference R17 RedCap devices |

Next, we are going to explain and analyze these three options one by one, and share our preference for evaluation options.

Option 1: Based on the current reference device, R17 single feature vs. R18 single feature

This option is exactly follow the same as R17, i.e., when considering a cost/complexity reduction feature, only the effect of this feature will be taken into account, the other capabilities are not changed.

Take further BW reduction to 5MHz in FDD as an example, with option 1, we just need to have the comparison of 100Mhz vs. 20MHz vs. 5MHz, the corresponding cost evaluation details are as follows.

Table 2.1.1-1: Example for cost evaluation based on option 1

| FDD |  | Reference NR device （TR 38.875） | R17 feature: 20MHz | R18 fearure: 5MHz |
| --- | --- | --- | --- | --- |
| RF 40% | Power amplifier | 25.00% | 0.00% | 0.00% |
|  | Filters | 10.00% | 0.00% | 0.00% |
|  | RF transceiver (including LNAs, mixer, and local oscillator) | 45.00% | 0.00% | 0.00% |
|  | Duplexer / Switch | 20.00% | 0.00% | 0.00% |
|  | Reduction in RF |  | 0.00% | 0.00% |
| Baseband 60% | ADC / DAC | 10.00% | 80.00% | 92.00% |
|  | FFT/IFFT | 4.00% | 75.00% | 90.00% |
|  | Post-FFT data buffering | 10.00% | 80.00% | 92.00% |
|  | Receiver processing block | 24.00% | 80.00% | 92.00% |
|  | LDPC decoding | 10.00% | 80.00% | 92.00% |
|  | Memory(HARQ buffer) | 14.00% | 80.00% | 92.00% |
|  | DL control processing & decoder | 5.00% | 0.00% | 0.00% |
|  | Synchronization / cell search block | 9.00% | 0.00% | 0.00% |
|  | UL processing block | 5.00% | 60.00% | 70.00% |
|  | MIMO specific processing blocks | 9.00% | 0.00% | 0.00% |
|  | Reduction in BB |  | 60.00% | 69.91% |
| Cost reduction |  |  | 36.24% | 41.80% |

The above comparison is actually comparing the cost of the following devices.

- Device 1: 100MHz+2Rx+2MIMO+FD-FDD+256QAM

- Device 2: 20MHz+2Rx+2MIMO+FD-FDD+256QAM

- Device 3: 5MHz+2Rx+2MIMO+FD-FDD+256QAM

Obviously, the Device 1 is the reference NR devices established in TR 38.875, and Device 2 can be regard as a high-end R17 RedCap. However, the device 3 cannot be the target R18 RedCap, since device 3 does not meet the main goal of R18 RedCap. In addition, we understand that there is no demand for device 3-liked RedCap in the market. Based on this, it can be considered that further complexity reduction based on a high-end RedCap (the Device 2 here) is not reasonable.

Observation 1: Further complexity reduction based on a high-end RedCap is not reasonable, since it does not meet the main goal of R18 RedCap

In addition, based on option1, it is very hard to evaluate other features(e.g., reduced peak data rate), since the intermediate device is unclear here, i.e., we don’t have a UE with 100MHz+2Rx+2MIMO+FD-FDD+256QAM capabilities, but the peak data rate is limited to a R17 RedCap level (e.g., ~80Mbps).

Observation 2: The comparison method of “R17 single feature vs. R18 single feature” is not suitable for the feature of reduced peak data rate, since the intermediate device is unclear.

Option 2: Based on the current reference device, compared to the simplest R17 RedCap

Under this method, we need to combine the R17 RedCap feature(s) to construct an R17 RedCap as a reference point for R18 comparison. As most of the features for R17 RedCap are optional (2Rx/MIMO, 256QAM, HD-FDD), a wide variety of combined features can be constructed.  In our understanding, only comparison with the simplest R17 RedCap (20MHz+1Rx+1MIMO+FD-FDD+64QAM) can show the meaningful cost reduction on top of R17 RedCap, since the cost reductions on top of the non-simplest RedCap may not be as low as the simplest R17 RedCap.

Observation 3: Comparison with the simplest R17 RedCap (20MHz+1Rx+1MIMO+FD-FDD+64QAM) can show the meaningful cost reduction on top of R17 RedCap.

Based on option 2 and take further BW reduction to 5MHz in FDD as an example, the cost evaluation and comparison for R18 complexity reduction would be:

Table 2.1.2-1: Illustration for cost evaluation based on option 2

|  | Reference NR devices established in TR 38.875 | R17 simplest RedCap | R18 RedCap |
| --- | --- | --- | --- |
| Capabilities | 100Mhz+2Rx+2MIMO+FD-FDD+256QAM | 20Mhz+1Rx+1MIMO+HD-FDD+64QAM | 5Mhz*1+1Rx+1MIMO+HD-FDD+64QAM |
| Cost | A*2 | B | C |
| Note1: 5MHz means reduce the BW from 100MHz to 20MHz, and further reduce the BW from 20MHz to 5MHz. Note2: According to TR38.875, the value of A is 100%. |  |  |  |

Then, by comparing the value of A/B/C, we can get the cost reduction ratio of BW reduction to 5MHz, the corresponding cost evaluation example is as follows:

Table 2.1.2-2: Illustration for cost evaluation based on option 2

| FDD |  | Reference NR device （TR 38.875） | R17 simplest RedCap: 20MHz+1Rx+1MIMO+HD-FDD+64QAM | R18 RedCap: 5Mhz+1Rx+1MIMO+HD-FDD+64QAM |
| --- | --- | --- | --- | --- |
| RF 40% | Power amplifier | 25% | 0% | 0% |
|  | Filters | 10% | 50% | 50.00% |
|  | RF transceiver (including LNAs, mixer, and local oscillator) | 45% | 53% | 52.50% |
|  | Duplexer / Switch | 20% | 80% | 80% |
|  | Reduction in RF |  | 45% | 44.63% |
| Baseband 60% | ADC / DAC | 10% | 92% | 96% |
|  | FFT/IFFT | 4% | 88% | 95% |
|  | Post-FFT data buffering | 10% | 90% | 96% |
|  | Receiver processing block | 24% | 95% | 98% |
|  | LDPC decoding | 10% | 93% | 97% |
|  | Memory(HARQ buffer) | 14% | 93% | 97% |
|  | DL control processing & decoder | 5% | 20% | 20% |
|  | Synchronization / cell search block | 9% | 50% | 50% |
|  | UL processing block | 5% | 60% | 72% |
|  | MIMO specific processing blocks | 9% | 88% | 88% |
|  | Reduction in BB |  | 83% | 86.82% |
| Cost reduction |  |  | 67.8% | 69.94% |
| Cost (rest) |  | 100% | 32.2% | 30.06% |

According to the above, B=32.2%, C=30.06%. Then, compared with R15/16 NR UEs, the additional cost reductions from 20MHz to 5MHz is 2.14%. While compared with R17 simplest RedCap UE, the cost reduction can be roughly calculated as (32.2%-30.06%)/(32.2%)=6.6%.

With option 2, the evaluation workload is relatively small.

Observation 4: With option2 (Based on the current reference device, compared to the simplest R17 RedCap), the evaluation workload is relatively small.

Option 3: Establish a new reference R17 RedCap devices

For option 3, establish a new reference R17 RedCap device is needed. As discussed in previous, the reference R17 RedCap devices should be the R17 simplest RedCap. Then, after combine all R17 features, the cost ratio of each components was changed, i.e., the cost ratio of each component of reference R17 RedCap device is quite different as those of reference NR devices, even the ratio of baseband to RF has changed. An example of new reference R17 RedCap device is as follows (FDD):

Table 2.1.3-1: new reference R17 RedCap device establishment

| FDD |  | Reference NR device (TR 38.875) | R17 simplest RedCap: 20MHz+1Rx+1MIMO+HD-FDD+64QAM | Cost ratio transition for R17 RedCap |  | New reference R17 RedCap device |
| --- | --- | --- | --- | --- | --- | --- |
| RF 40% |  |  |  |  |  |  |
|  | Power amplifier | 25% | 0% | 10.00% | 31.06% | 45% |
|  | Filters | 10% | 50% | 2.00% | 6.21% | 9% |
|  | RF transceiver (including LNAs, mixer, and local oscillator) | 45% | 53% | 8.55% | 26.56% | 39% |
|  | Duplexer / Switch | 20% | 80% | 1.60% | 4.97% | 7% |
|  | Cost reduction in RF |  | 45% |  |  |  |
| Baseband 60% | ADC / DAC | 10% | 92% | 0.48% | 1.49% | 5% |
|  | FFT/IFFT | 4% | 88% | 0.29% | 0.89% | 3% |
|  | Post-FFT data buffering | 10% | 90% | 0.60% | 1.86% | 6% |
|  | Receiver processing block | 24% | 95% | 0.72% | 2.24% | 7% |
|  | LDPC decoding | 10% | 93% | 0.42% | 1.30% | 4% |
|  | Memory(HARQ buffer) | 14% | 93% | 0.59% | 1.83% | 6% |
|  | DL control processing & decoder | 5% | 20% | 2.40% | 7.45% | 24% |
|  | Synchronization / cell search block | 9% | 50% | 2.70% | 8.39% | 27% |
|  | UL processing block | 5% | 60% | 1.20% | 3.73% | 12% |
|  | MIMO specific processing blocks | 9% | 88% | 0.65% | 2.01% | 6% |
|  | Cost reduction in BB |  | 83% |  |  |  |

The RF-to-baseband cost ratio changed from 40:60 to around 70:30. Then, based on the established new reference R17 RedCap device, further cost reduction can be the following:

Table 2.1.3-2: cost evaluation based on reference R17 RedCap device

| FDD |  | New reference R17 RedCap device | R18 feature: 5MHz |
| --- | --- | --- | --- |
| RF 70% |  |  |  |
|  | Power amplifier | 45% | 0% |
|  | Filters | 9% | 0% |
|  | RF transceiver (including LNAs, mixer, and local oscillator) | 39% | 0% |
|  | Duplexer / Switch | 7% | 0% |
|  | Total |  | 0.00% |
| Baseband 30% | ADC / DAC | 5% | 55% |
|  | FFT/IFFT | 3% | 55% |
|  | Post-FFT data buffering | 6% | 55% |
|  | Receiver processing block | 7% | 55% |
|  | LDPC decoding | 4% | 55% |
|  | Memory(HARQ buffer) | 6% | 55% |
|  | DL control processing & decoder | 24% | 0% |
|  | Synchronization / cell search block | 27% | 0% |
|  | UL processing block | 12% | 30% |
|  | MIMO specific processing blocks | 6% | 0% |
|  | Total |  | 20.54% |
| Cost reduction |  |  | 6.41% |

Based on option 3, the cost reduction of further BW reduction to 5MHz is 6.41%. This result is quite similar to those based on option2, since the compared devices for both option 2 and option 3 are same, i.e., the R17 simplest RedCap. Compared to option 2, the evaluation workload of option3 is relatively great, since a new reference R17 RedCap device is needed and the value of each component need to calibrate.

Observation 5: As the compared device for both option2 and option3 are the same, the cost evaluation results based on option3 are similar to those based on option2.

Observation 6: Compared to option 2, the evaluation workload of option3 is relatively heavy.

Summary of options for cost evaluation baseline and methodology

Based on the analyses in chapter 2.1.1-2.1.3, the comparison of these three evaluation options are listed below:

Table 2.1.4-1: summary of cost evaluation baseline and methodology

|  | Option 1 | Option 2 | Option 3 |
| --- | --- | --- | --- |
| Match the goal of R18 | No*1 | Yes | Yes |
| Evaluation workload | Low | Low | Middle*1 |
| Note1: Further cost/complexity reductions are compared to R17 high-end RedCap Note2: Establish a new R17 RedCap reference device is needed. |  |  |  |

Based on the summary and the previous observations, we propose to adopt option2, i.e., the cost reduction for a R18 feature should compare to the simplest R17 RedCap, i.e., comparison of all R17 RedCap features and all R17 RedCap features+R18 feature(s) on top of the reference NR devices established in TR 38.875.

Proposal 2: The cost evaluation for R18 feature(s) should be carried out by comparing to the simplest R17 RedCap.

- I.e., comparison of (all R17 RedCap features) and (all R17 RedCap features+R18 feature(s)).

Cost evaluation cases and assumptions

Classification and combinations for R18 complexity solutions/features

According to the SID and based on our understanding, the solutions listed in the SID [1] can be divided into two categories, a) main solutions, and b) secondary/supplementary solutions:

  - Main solutions

    - UE bandwidth reduction to 5MHz in FR.

    - Reduced UE peak data rate in FR1.

  - Secondary/supplementary solutions

    - Relaxed UE processing timeline.

For these solutions, we understand that it is unnecessary to combine the two main solutions, i.e., combine the UE bandwidth reduction to 5MHz and reduced UE peak data rate in FR1, since a similar effect can be achieved by both solutions.

Further, based on the SID [1], the cost of the following combinations can be evaluated.

- Combination 1: Combination of 5MHz BW and processing time relaxation

- Combination 2: Combination of reduced peak data rate and processing time relaxation

Cost evaluation cases

Based on the analysis in 2.1.1, the cases for cost evaluation in R18 are as follows

  - Case 1: UE bandwidth reduction to 5MHz

  - Case 2: Reduced peak data rate

  - Case 3: Processing timeline relaxation

  - Case 4: UE bandwidth reduction to 5MHz + Processing timeline relaxation

  - Case 5: Reduced peak data rate + Processing timeline relaxation

Proposal 3: Consider the following cases for R18 cost evaluation:

  - Case 1: UE bandwidth reduction to 5MHz

  - Case 2: Reduced peak data rate

  - Case 3: Processing timeline relaxation

  - Case 4: UE bandwidth reduction to 5MHz + Processing timeline relaxation

  - Case 5: Reduced peak data rate + Processing timeline relaxation

Cost evaluation assumptions

For cost evaluation, we have the following assumption:

- For UE bandwidth reduction to 5MHz, we assume the same maximum UE bandwidth in a band applies to both RF and baseband and applies to both data and control channels. Furthermore, this maximum UE bandwidth is assumed for both DL and UL.

- For reduced peak data rate, we assume the target peak data rate for R18 RedCap is around 10Mbps. In addition, we think this solution is mainly reduce the cost of data processing related components in BB, e.g., Receiver processing block, LDPC decoding, Memory(HARQ  buffer), UL processing block. Then, we assume the same maximum UE bandwidth is 20MHz in a band applies to both RF and baseband, i.e., the baseband BW for both control channel and data channel is 20MHz.  Furthermore, 20MHz bandwidth is assumed for both DL and UL.

- For processing timeline relaxation, the values of N1 and N2 to be studied are assumed to be doubled compared to those of capability 1.

Based on the above, we have the following proposal:

Proposal 4: The cost evaluations for R18 are based on the following assumptions:

  - For UE bandwidth reduction to 5MHz, the same maximum UE bandwidth in a band applies to both RF and baseband and applies to both data and control channels. Furthermore, this maximum UE bandwidth is assumed for both DL and UL.

  - For reduced peak data rate, the target peak data rate for R18 RedCap is around 10Mbps, and the same maximum UE bandwidth is 20MHz in a band applies to both RF and baseband, i.e., the baseband BW for both control channel and data channel is 20MHz. Furthermore, 20MHz bandwidth is assumed for both DL and UL.

  - For processing timeline relaxation, the values of N1 and N2 to be studied are assumed to be doubled compared to those of capability 1.

Initial cost evaluation results and observations

Based on the cost evaluation baseline and methodology option 2 and 3, and based on the evaluation assumptions discussed in chapter 2.2.3, we evaluated the cost reduction for the five cases proposed in chapter 2.2.2, the initial cost evaluation results are listed in the following table:

Table 2.1.4-1: summary of cost evaluation baseline and methodology

| Cases*1 | Based on Option 2*2 |  |  |  | Based on option3*3 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDD |  | TDD |  | FDD | TDD |
|  | Additional cost reduction | cost reduction on top of R17 | Additional cost reduction | cost reduction on top of R17 |  |  |
| Case 1 | 2.14% | 6.63% | 1.33% | 4.73% | 6.41% | 4.92% |
| Case 2 | 1.75% | 5.44% | 1.20% | 4.27% | 5.17% | 4.63% |
| Case 3 | 1.01% | 3.15% | 0.83% | 2.97% | 3.33% | 3.03% |
| Case 4 | 2.99% | 8.87% | 1.99% | 7.08% | 9.25% | 7.54% |
| Case 5 | 2.35% | 7.31% | 1.71% | 6.08% | 7.46% | 6.66% |
| Note 1: The details of cases can be found in chapter 2.2.2. Note 2/3: Compared to simplest R17 RedCap. |  |  |  |  |  |  |

From the above table, we have the following observations:

Observation 7: Based on R17 evaluation methodology, the cost reduction of UE bandwidth reduction to 5MHz and reduced peak data rate are limited and almost the same.

Observation 8: Based on R17 evaluation methodology, the combined features will bring relatively large cost benefits. The cost reductions of combination 1(UE bandwidth reduction to 5MHz + Processing timeline relaxation) and combination 2(Reduced peak data rate + Processing timeline relaxation) are almost the same.

Other aspects for R18 cost evaluation.

Non-liner relationship for the cost

Based on our initial cost evaluation results, the cost reduction for R18 features are limited. The main reason is that after combine all R17 features, the rest of the costs of each component in RF and BB are quite small. In addition, the lower the capability of component is, the more non-linear the relationship between capability and cost becomes, and the lower the non-linear curvature. Then, there are no much rooms for cost reduction in RF and BB on top of R17 RedCap.

Observation 9: The further cost reduction may be limited, since the lower the capability of component is, the more non-linear the relationship between capability and cost becomes, and the lower the nonlinear curvature.

The cost of memory

R17 evaluation methodology only focus on RF and BB, due to the reason we mentioned in chapter 3.1, there is no much room for cost reduction if we still focus on these two parts.

However, it is worth noticing that there are some other components (e.g., memory, different from RF and BB) will be affect by R18 features. BW reduction to 5MH and reduced peak data rate are both lead to a lower peak data rate, which means lower memory requirements, then the less cost of memory. Based on the product experience, if the peak data rate can be reduced from ~80Mbps to 10Mbps, the memory cost can be reduced by around 10% in overall. The cost reduction for memory is the other important motivation for R18 RedCap.

In order to illustrate the principle of R18 cost reduction, and to convey correct and positive information to vertical industries, we think it is necessary to consider the memory cost and capture the impact of R18 features on memory cost in the corresponding TR.

Observation 10: There are some other components (e.g., memory) will be affect by R18 features

Observation 11: The cost reduction for memory is the other important motivation for R18 RedCap.

Proposal 5: The memory cost should be considered in R18 and capture the impact of R18 features on memory cost in the corresponding TR.

Conclusion

Based on the analyses and discussions, we have the following proposals and observations:

- Proposals

Proposal 1: For R18 cost evaluation, a unified evaluation baseline and methodology is required.

Proposal 2: The cost evaluation for R18 feature(s) should be carried out by comparing to the simplest R17 RedCap.

- I.e., comparison of (all R17 RedCap features) and (all R17 RedCap features+R18 feature(s)).

Proposal 3: Consider the following cases for R18 cost evaluation:

  - Case 1: UE bandwidth reduction to 5MHz

  - Case 2: Reduced peak data rate

  - Case 3: Processing timeline relaxation

  - Case 4: UE bandwidth reduction to 5MHz + Processing timeline relaxation

  - Case 5: Reduced peak data rate + Processing timeline relaxation

Proposal 4: The cost evaluations for R18 are based on the following assumptions:

  - For UE bandwidth reduction to 5MHz, the same maximum UE bandwidth in a band applies to both RF and baseband and applies to both data and control channels. Furthermore, this maximum UE bandwidth is assumed for both DL and UL.

  - For reduced peak data rate, the target peak data rate for R18 RedCap is around 10Mbps, and the same maximum UE bandwidth is 20MHz in a band applies to both RF and baseband, i.e., the baseband BW for both control channel and data channel is 20MHz. Furthermore, 20MHz bandwidth is assumed for both DL and UL.

  - For processing timeline relaxation, the values of N1 and N2 to be studied are assumed to be doubled compared to those of capability 1.

Proposal 5: The memory cost should be considered in R18 and capture the impact of R18 features on memory cost in the corresponding TR.

- Observations

Observation 1: Further complexity reduction based on a high-end RedCap is not reasonable, since it does not meet the main goal of R18 RedCap

Observation 2: The comparison method of “R17 single feature vs. R18 single feature” is not suitable for the feature of reduced peak data rate, since the intermediate device is unclear.

Observation 3: Comparison with the simplest R17 RedCap (20MHz+1Rx+1MIMO+FD-FDD+64QAM) can show the meaningful cost reduction on top of R17 RedCap.

Observation 4: With option2 (Based on the current reference device, compared to the simplest R17 RedCap), the evaluation workload is relatively small.

Observation 5: As the compared device for both option2 and option3 are the same, the cost evaluation results based on option3 are similar to those based on option2.

Observation 6: Compared to option 2, the evaluation workload of option3 is relatively heavy.

Observation 7: Based on R17 evaluation methodology, the cost reduction of UE bandwidth reduction to 5MHz and reduced peak data rate are limited and almost the same.

Observation 8: Based on R17 evaluation methodology, the combined features will bring relatively large cost benefits. The cost reductions of combination 1(UE bandwidth reduction to 5MHz + Processing timeline relaxation) and combination 2(Reduced peak data rate + Processing timeline relaxation) are almost the same.

Observation 9: The further cost reduction may be limited, since the lower the capability of component is, the more non-linear the relationship between capability and cost becomes, and the lower the nonlinear curvature.

Observation 10: There are some other components (e.g., memory) will be affect by R18 features

Observation 11: The cost reduction for memory is the other important motivation for R18 RedCap.

Reference

- RP-213661, New SID on Study on further NR RedCap UE complexity reduction. RAN#94e, December 6 - 17, 2021.

- TR 38.875, Study on support of reduced capability NR devices- Release 17, V17.0.0 (2021-03).