3GPP TSG RAN WG1 #109-e                                                                R1-2203430

e-Meeting, May 9th – 20th, 2022

Source:	CATT

Title:	Remaining issues on initial access aspects for up to 71GHz operation

Agenda Item:	8.2.1

Document for:   Discussion and Decision

- Introduction

In RAN1#108b-e meeting [1], the following agreements were made regarding issues on initial access aspects for up to 71GHz operation:

Working assumption - Use 1 bit for Q in MIB - SubcarrierSpacingCommon field will be used to convey value of NSSBQCL={32, 64} for operation with shared spectrum channel access - Note that this is revising the working assumption made in RAN1#107-e on “use 2 bits for Q, {SubcarrierSpacingCommon, spare bit in MIB}” Conclusion Update the ssb-PositionQCL in RRC to {32, 64} values. - For reference, the following are list of RRC IEs that references ssb-PositionQCL in release 16. - SIB2:: ssb-PositionQCL-Common-r16 - SIB3:: ssb-PositionQCL-r16 - SIB4:: ssb-PositionQCL-Common-r16 - SIB4:: ssb-PositionQCL-r16 - MeasObjectNR:: ssb-PositionQCL-Common-r16 - MeasObjectNR:: ssb-PositionQCL-r16 - ServingCellConfigCommon:: ssb-PositionQCL-r16 Conclusion - For operation with shared spectrum channel access, support default DBTW length of 5ms if discoveryBurstWindowLength is not provided in FR2-2. - No change to specification is needed Working Assumption - The following table is used for set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {120, 120}, {480, 480}, and {960, 960} kHz for FR2-2. - FFS: whether/how to define X, if defined, candidate values {56, 76} - Text Proposal #4-2D (for 38.213, Section 13) in section 3 of R1-2202503 is endorsed - Note: this working assumption can be revisited once RAN4 finalizes the further details of the channelization. Index; SS/PBCH block and CORESET multiplexing pattern; Number of RBs NRBCORESET; Number of Symbols NsymbCORESET; Offset (RBs). 0; 1; 24; 2; 0. 1; 1; 24; 2; 4. 2; 1; 48; 1; 0. 3; 1; 48; 1; 14. 4; 1; 48; 1; 28. 5; 1; 48; 2; 0. 6; 1; 48; 2; 14. 7; 1; 48; 2; 28. 8; 1; 96; 1; 0. 9; 1; 96; 1; X. 10; 1; 96; 2; 0. 11; 1; 96; 2; X. 12; 3; 24; 2; -20 if kSSB=0 -21 if kSSB > 0. 13; 3; 24; 2; 24. 14; 3; 48; 2; -20 if kSSB=0 -21 if kSSB > 0. 15; 3; 48; 2; 48.

In this contribution, the remaining issues on some aspects of initial access for up to 71GHz operation are discussed.

- Discussion on initial access for up to 71GHz operation

  - Q indication in MIB

Regarding Q indication in MIB, the following is agreed in RAN1#108-e:

Working assumption

- Use 1 bit for Q in MIB

  - SubcarrierSpacingCommon field will be used to convey value of NSSBQCL={32, 64} for operation with shared spectrum channel access

  - Note that this is revising the working assumption made in RAN1#107-e on “use 2 bits for Q, {SubcarrierSpacingCommon, spare bit in MIB}”

The reason of revising previous working assumption made in RAN1#107-e is because in RAN2#116b-e, RAN2 does not agree to use the spare bit in MIB for the signaling of FR2-2 QCL assumptions for SSB. Given RAN2 decision to not to use the spare bit in MIB, RAN1 agreed the working assumption to use 1 bit for Q in MIB for supporting {32, 64} only. However, some companies still have reservation for this WF as they would prefer more bits for Q value indication with methods such as repurposing other bit for such purposes. However, all alternatives require further discussion and may have impact on other issues. Given the available time in the maintenance period of the WID, it is proposed to confirm the working assumption.

Proposal 1: Confirm the working assumption reached in RAN1#108-e regarding the Q indication in MIB.

  - Configuration for resource blocks and slot symbols of CORESET for Type0-PDCCH search space set

The following agreement is reached in RAN1#108-e for the configuration of CORESET for Type0-PDCCH search space set:

Working Assumption

- The following table is used for set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {120, 120}, {480, 480}, and {960, 960} kHz for FR2-2.

  - FFS: whether/how to define X, if defined, candidate values {56, 76}

- Text Proposal #4-2D (for 38.213, Section 13) in section 3 of R1-2202503 is endorsed

Note: this working assumption can be revisited once RAN4 finalizes the further details of the channelization.

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs NRBCORESET | Number of Symbols NsymbCORESET | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 4 |
| 2 | 1 | 48 | 1 | 0 |
| 3 | 1 | 48 | 1 | 14 |
| 4 | 1 | 48 | 1 | 28 |
| 5 | 1 | 48 | 2 | 0 |
| 6 | 1 | 48 | 2 | 14 |
| 7 | 1 | 48 | 2 | 28 |
| 8 | 1 | 96 | 1 | 0 |
| 9 | 1 | 96 | 1 | X |
| 10 | 1 | 96 | 2 | 0 |
| 11 | 1 | 96 | 2 | X |
| 12 | 3 | 24 | 2 | -20 if kSSB=0 -21 if kSSB > 0 |
| 13 | 3 | 24 | 2 | 24 |
| 14 | 3 | 48 | 2 | -20 if kSSB=0 -21 if kSSB > 0 |
| 15 | 3 | 48 | 2 | 48 |

One remaining issue is the definition of X and corresponding the value of X.  Regarding the offset value, 76 are preferred since it will enable more efficient resource allocation. In addition, there is no need to define X in the specification. Even if the intention is to support both value this is not preferred since there will complicate the configuration. Therefore, it is proposed to replace X with 76.

Proposal 2:  Regarding resource blocks and slot symbols of CORESET for Type0-PDCCH search space set, it is proposed to replace X in the table with 76.

Conclusion

This contribution discussed the remaining aspects of initial access for the support of NR operation up to 71 GHz. We have the following proposals,

Proposal 1:  Confirm the working assumption reached in RAN1#108-e regarding the Q indication in MIB.

Proposal 2:  Regarding resource blocks and slot symbols of CORESET for Type0-PDCCH search space set, it is proposed to replace X in the table with 76.

Reference

- 3GPP TSG RAN WG1 Meeting RAN1 #108b-e chairman notes, February 21st – March 3rd, 2022