3GPP TSG RAN WG1 #109-e	        		     R1-2203868

e-Meeting, May 9th – 20th, 2022

Agenda Item:		8.16.7

Source:			Samsung

Title:			UE features for UE power saving enhancements

Document for:		Discussion and decision

# Introduction

This contribution includes comments for Rel-17 UE features for supporting UE power saving enhancements, as drafted in [1].

# 2. Comments to FG 29-1: Paging enhancement

The following agreement was made for FG 29-1 in RAN1#109-e, one remaining issue is whether to support a third component 3 as highlighted in blue.

Agreement

•            FG 29-1 is updated as follows

| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support receiving paging early indication in DCI format 2_7 2. Support receiving UE subgroup indication in DCI format 2_7 [3. For type 2A CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, 2 or 2A CSS configurations within a single span of three consecutive OFDM symbols within a slot] |  | Yes |  | UE does not support paging enhancement | Per band | N/A | N/A | N/A | For component 2, it is up to RAN2 whether/how to separate the capability for UE subgroup indication Leave RAN2 to decide whether ‘optional with capability signalling’ or ‘optional without capability signalling’ Leave RAN2 to decide whether Need for the gNB to know if the feature is supported is Yes or No | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The purpose of component 3 is to capture the PDCCH monitoring of type2A-CSS based on legacy UE capability FG3-1. However, we don’t see the need to repeat the legacy paging reception criterion for FG 29-1. FG 29-1 reuses the legacy paging PDCCH to provide the paging early indication, and there is no any new design of paging PDCCH reception from FG 29-1.

Conclusion 1: No need to add component 3 to capture the PDCCH monitoring of type2A-CSS based on legacy UE capability FG3-1

# 5. Reference

[1] R1-2201516, Summary on UE features for UE power saving enhancements	Moderator (NTT DOCOMO, INC.)