3GPP TSG RAN WG1 #109-e		R1-2204065

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.8.2

Source:	Lenovo

Title:	Discussion on L1/L2 signaling for side control information of network-controlled repeaters

Document for:	Discussion

# Introduction

In RANP#94e, a SID on network-controlled repeater [1] is approved, and the objective is as following:

The study on NR network-controlled repeaters is to focus on the following scenarios and assumptions:

- Network-controlled repeaters are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands, while during the study FR2 deployments may be prioritized for both outdoor and O2I scenarios.

- For only single hop stationary network-controlled repeaters

- Network-controlled repeaters are transparent to UEs

- Network-controlled repeater can maintain the gNB-repeater link and repeater-UE link simultaneously

NOTE1: Cost efficiency is a key consideration point for network-controlled repeaters.

Study and identify which side control information below is necessary for network-controlled repeaters including assumption of max transmission power [RAN1]

- Beamforming information

- Timing information to align transmission / reception boundaries of network-controlled repeater

- Information on UL-DL TDD configuration

- ON-OFF information for efficient interference management and improved energy efficiency

- Power control information for efficient interference management (as the 2nd priority)

Study and identify L1/L2 signaling (including its configuration) to carry the side control information [RAN1]

Study the following aspects of network-controlled repeater management

- Identification and authorization of network-controlled repeaters [RAN2, RAN3]

NOTE2: Coordination with SA3 may be needed.

In this contribution, we discuss the issues related to side control information for network-controlled repeater.

In this contribution, we discuss the issues related to L1/L2 signaling for side control information of network-controlled repeater.

# Discussion

One of the issues that need to be addressed during the SI phase of network-controlled repeater is the signalling of the side control information, discussed in our companion contribution [2], between the network and the repeater. This includes the study of the different methods for signalling each feature/ control parameter and the ways to configure this signaling and the level of the required protocol stack at the repeater side. RAN1 should investigate whether a full UE protocol stack is needed, or a limited protocol stack is possible by using legacy configuration or using OAM-based configuration.

## Initial access for network-controlled repeater

In R16/R17 IAB network, the IAB node contains IAB-MT and IAB-DU components. When it is firstly set on, the MT part works similar to a UE for performing random access procedure. As the IAB-MT terminates a protocol stack including L1/L2 and RRC, after initial access, the IAB donor or other parent IAB node identifies the new device as an IAB node. In contrast, the network-controlled repeater has only L1/L2. Msg 1/2/3/4 in random access procedure can be finished within L1 and L2. However, identification of network-controlled repeater can’t be done by legacy procedure. Therefore, there may be a need of some signaling from the repeater to the network to indicate that it is a repeater rather than a UE.

Proposal 1: Study mechanisms to indicate the network-controlled repeater type.

## Differentiation between network-controlled repeater and normal UE

There may be some side control information transmitted from gNB to network-controlled repeater. Meanwhile, the gNB may also serve some UEs with direct link between gNB and these UEs. The information transmitted to repeater and to UE may be different. For UE, the information may be scheduling information in PDCCH or some group common DCI carrying slot format, etc. For repeater, the information is the side control information. The differentiation between repeater and UE can reuse the differentiation mechanism between different UEs in legacy network. Different UEs can be associated with different UE IDs in legacy network. A special UE ID can be configured to a repeater. With this configuration, the gNB can identify whether a node is a repeater or a UE and can make proper scheduling.

Proposal 2: Study the mechanism to differentiate repeater and UE. Differentiation mechanisms for different UEs can be a starting point.

## L1/L2 signaling

The side control information may contain timing, TDD configuration, beam indication and on/off indication. In legacy release, for some procedure/channel, signaling from physical layer to RRC layer are necessary. For beam indication for PDSCH, there may be some RRC configuration, and further MAC CE configuration and then some physical layer configuration in DCI to determine the beam for PDSCH. For other channels such as PDCCH/PUSCH/PUCCH/CSI-RS/SRS, there are also combination of RRC, MAC CE and DCI signaling. For TDD configuration, TDD config common and TDD config dedicated are also configured by RRC signaling, and DCI 2-0 is by a group common DCI in physical layer. If reuse legacy TDD configuration for indicating TDD pattern between gNB and repeater, RRC in network-controlled repeater may be necessary. There can also be other mechanisms such as envelop the RRC signalling to L1/L2 signaling. This may lead to some spec work.

Proposal 3: Study how to transmit legacy RRC signaling to repeater.

In legacy UE configuration, RRC is used to carry different UE dedicated configuration including spatial information configuration such as CSI-RS and measurement configuration. To allow the repeater to apply a proper spatial, time, power control on the forwarding link a full UE stack might be necessary at the repeater side. On the other hand, to simplify the repeater functionality, layer 1 and possibly layer 2 can be considered for the protocol stack. This may entail designing a new DCI format specific to the repeater to carry configuration information. Furthermore, as the frequency scheduling related parameters are irrelevant to the repeater, the repeater is not expected to decode and use this information, thus the repeater may be configured to decode a repeater-specific DCI format, that carry only repeater relevant control information, or performing limited blind search on limited candidates instead of reusing the legacy UE DCI formats. Having fixed size DCI or limited DCI formats with only repeater relevant parameters helps in fast decoding of the control information and hence fast application of the configuration on the forwarding link.

Proposal 4: Study whether a new repeater-specific DCI format is needed to carry repeater configuration information related to RRC configurations.

# Conclusion

In this contribution, we discussed the issues related to L1/L2 signaling for side control information for network-controlled repeater, and our proposals are as following:

Proposal 1: Study mechanisms to indicate the network-controlled repeater type.

Proposal 2: Study the mechanism to differentiate repeater and UE. Differentiation mechanisms for different UEs can be a starting point.

Proposal 3: Study how to transmit legacy RRC signaling to repeater.

Proposal 4: Study whether a new repeater-specific DCI format is needed to carry repeater configuration information related to RRC configurations.

# References

RP-213700, RANP#94e, “Study on NR Network-controlled Repeaters”

R1-2204064, RAN1#109e “Discussion on side control information for network-controlled repeater”, Lenovo