3GPP TSG RAN WG1 Meeting #109-e	R1-2204099

e-Meeting, May 9th – May 20th, 2022

Agenda Item:	9.1.2

Source:	Futurewei

Title:	CSI enhancement for high/medium UE velocities and CJT

Document for:	Discussion and decision

# Introduction

In RAN#94-e meeting, the Rel. 18 NR_MIMO_evo_DL_UL WID [1] was approved. There are seven objectives in the new WID. The first and fourth objectives in [1] are:

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows: - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis - UE reporting of time-domain channel properties measured via CSI-RS for tracking - Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

In this contribution, we present our views on CSI enhancement for high/medium UE velocities and evaluation of coherent-JT (CJT), and proposals for moving forward.

# Use Cases for CSI Enhancement for High/medium UE Velocities

The introduction of cellular V2X technologies in 3GPP standards and the ever-increasing trend for autonomous driving will spur in-vehicle entertainment and work to become a mainstream application. This in turn will generate high data rate traffic with high mobility along the roadways.  Some examples of the use case for high data rate with high mobility for users in moving vehicles includes entertainment with HD video and video conferencing with HD video, etc.  Regarding the data rate requirement, depending on the video format, the required data rate can range from around 50 Mbps to around 100 Mbps.  For example, 50 Mbps for 2Kx2K video format and 100 Mbps for 4K UHD format.  Regarding the UE velocity, it can range from 30 km/h to 120 km/h.

It is known that MU-MIMO enabled by large array base-station deployments delivers high cell spectral efficiency in low-mobility conditions but can sharply degrade at high/medium UE velocities. The main reason for the performance degradation is due to the outdated CSI feedback and/or the lack of reliable channel estimates.  Therefore, in our view, the main use cases for CSI enhancement for high/medium UE velocities are for UE velocity in the range of 30-120 km/h.

Based on the above analysis, we have the following observation:

Observation 1: The main use cases for CSI enhancement for high/medium UE velocities are for UE velocity in the range of 30-120 km/h.

# Views on CSI Enhancement for High/medium UE Velocities

The Rel-16/17 Type-II codebook provides high-resolution CSI which is essential to MU-MIMO performance enhancements.  Specifically, the Rel-16 Type-II codebook employs compression in both the spatial and frequency domain by utilizing the spatial and frequency domain basis.  Furthermore, the Rel-17 enhanced Type-II port selection codebook exploits the angle-delay reciprocity in DL and UL which is applicable for both TDD and FDD, therefore can further reduce the CSI feedback overhead from the UE.

However, in high/medium UE velocity scenario, although the Rel-16/17 Type-II codebook can provide high-resolution CSI at the CSI measurement time instant, due to the fast variation of the channel, the CSI feedback can be outdated when the actual data transmission occurs, and data scheduling/MU-MIMO UE pairing based on the outdated CSI feedback could result in significant performance degradation.

To combat the fast variation of the channel due to high/medium UE velocities, CSI enhancements are needed.  In our view, the following approaches can be considered:

- Approach #1: UE feeds back the measured CSI and time-domain channel properties such as Doppler. The time-domain channel properties can be, e.g., measured via CSI-RS for tracking. The gNB, once receives the CSI feedback and the time-domain channel properties reported from the UE, adjusts/predicts the CSI for data scheduling/MU-MIMO UE pairing in the future and performs data transmission accordingly.

- Approach #2: UE predicts the CSI in the future based on its measurement and feeds back the predicted CSI.  The gNB, once receives the CSI feedback, perform data scheduling/MU-MIMO UE pairing according to the CSI feedback and perform data transmission in a future time instant. In this approach, due to the UE’s lack of knowledge on the future time instant when the data transmission occurs, the time instant the UE targets at for the predicted CSI could be well off from the real time instant where the data transmission occurs.  In addition, in this approach, the computational burden for CSI prediction is on the UE side, which might increase UE’s complexity on CSI measurement/reporting.

In our view, Approach #1 is preferred.

Based on the above analysis, we propose the following:

Proposal 1: For CSI enhancement for high/medium UE velocities, the group focus on the approach where UE feeds back the measured CSI and time-domain channel properties.

In Rel-16/17, most of the evaluations of CSI enhancements (e.g., Type-II codebook enhancements) are based on system level simulations (SLS).  For CSI enhancement for high/medium UE velocities in Rel-18, as we discussed above, it is expected that most of the proposed schemes will involve CSI prediction.  Regarding evaluation of the proposed schemes, in our view, in addition to SLS, link level simulations (LLS) should also be allowed to be used for evaluation.  The reason is that it is much easier to evaluate, and straightforward to investigate CSI predicting performance without having to use SLS.

Based on the above analysis, we propose the following:

Proposal 2: For evaluation of CSI enhancement for high/medium UE velocities, in addition to SLS, LLS can be used.

# Views on Evaluation of CSI enhancement for CJT

For CSI enhancement for CJT, it is expected that multiple alternative schemes will be proposed.  To evaluate the performance of different schemes, SLS most likely will be needed.  It is also noted that according to the WID [1], enhancements of CSI acquisition for CJT are targeted at both FDD and TDD, therefore SLS evaluations of both FDD CJT and TDD CJT are needed.  Furthermore, it is noted that the SRS enhancement targeting TDD CJT is treated under agenda item 9.1.3.2.   A question will then arise on where to handle the SLS evaluation of FDD CJT and TDD CJT schemes.  In our views, there could be two ways:

- Way #1: The SLS evaluations of FDD CJT and TDD CJT schemes are both handled in agenda item 9.1.2 for CSI enhancement.

- Way #2: The SLS evaluations of FDD CJT schemes are handled in agenda item 9.1.2 while the SLS evaluations TDD CJT schemes are handled in agenda item 9.1.3.2 for SRS enhancement targeting TDD CJT and 8 TX operation.

Considering the fact that a large portion of the FDD CJT and TDD CJT evaluation methodologies should be the same, to avoid duplicating the related efforts in both agenda items 9.1.2 and 9.1.3.2, we prefer Way #1.

Note that the joint evaluation effort described in Way #1 is only for the study and justification phase of CJT. For specification of enhancements, the work for FDD CJT and TDD CJT should be separate in 2 agenda items.

Based on the above analysis, we propose the following:

Proposal 3: For the study and justification phase of CJT, the SLS evaluations of FDD CJT and TDD CJT schemes are both handled in agenda item 9.1.2 for CSI enhancement.  For specification of enhancements, the work for FDD CJT and TDD CJT should be separate in 2 agenda items: 9.1.2 and 9.1.3.2, respectively.

# Conclusions

In this contribution, we present our views on CSI enhancement for high/medium UE velocities and evaluation of CJT.  Based on the discussions in the previous sections we have the following observation and proposals:

Observation 1: The main use cases for CSI enhancement for high/medium UE velocities are for UE velocity in the range of 30-120 km/h.

Proposal 1: For CSI enhancement for high/medium UE velocities, the group focus on the approach where UE feeds back the measured CSI and time-domain channel properties.

Proposal 2: For evaluation of CSI enhancement for high/medium UE velocities, in addition to SLS, LLS can be used.

Proposal 3: For the study and justification phase of CJT, the SLS evaluations of FDD CJT and TDD CJT schemes are both handled in agenda item 9.1.2 for CSI enhancement.  For specification of enhancements, the work for FDD CJT and TDD CJT should be separate in 2 agenda items: 9.1.2 and 9.1.3.2, respectively.

# References

RP-213598, “New WID: MIMO Evolution for Downlink and Uplink”, Samsung, RAN#94-e.