3GPP TSG RAN WG1 #109	R1-2204431

e-Meeting, May 9th – 20th, 2022

Agenda item:		9.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On subband non-overlapping full duplex for NR

Document for:		Discussion and Decision

# Introduction

RAN has agreed in RP-220633 a new Study Item on evolution of NR duplex operation with the following objectives [1]:

The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In this contribution we discuss the subband non-overlapping full duplex (SBFD) scenario. We present challenges and new interference types introduced due to SBFD and include considerations on feasibility of SBFD with special attention to adjacent-channel and co-channel inter-subband cross-link interference (CLI), including gNB self-interference. We provide high-level considerations on system operation and potential enhancements to enable SBFD operation in NR, including efficient support of legacy operation.

# Discussion

## Adjacent-channel CLI

Inter-operator adjacent-channel cross-link interference (CLI) has been studied in Rel-16 for dynamic TDD. Coexistence studies concluded that performance degradation is observed in macro-to-macro FR1 scenario from the gNB-to-gNB interference if not using the same TDD configuration for all operators. For macro-to-macro in FR2, micro-to-micro and indoor scenarios, some performance degradation is observed unless carefully planned. Therefore, dynamic TDD can be used at least for low power and/or indoor gNBs, if care is taken [2]. These conclusions shall be taken as a starting point for the Rel-18 study on evolution of duplex operation. For more details on the conclusions from earlier NR releases, please also refer to our companion contribution on dynamic TDD enhancements [6]. We envision that, from an inter-operator adjacent-channel CLI perspective, SBFD will introduce similar coexistence issues as dynamic TDD. Hence, conclusions from the Rel-16 coexistence studies could be assumed to hold also for SBFD, i.e., feasible deployment scenarios for SBFD should be deployments with low power and/or indoor gNBs.

Alternatively, new coexistence studies for macro deployments (e.g. Dense Urban scenario) between e.g. static TDD and SBFD will need to be conducted in RAN4, with the possibility that RAN4 ends up with the same or similar conclusions as in the Rel-16 coexistence studies for dynamic TDD. In case the baseline assumptions for the SBFD coexisting studies are modified as compared to Rel-16 (e.g. assumptions on gNB collocation, grid shift, etc.), new coexistence studies may need to be done for both dynamic TDD and SBFD scenarios.

Proposal 1: Based on Rel-16 coexistence study, RAN1 could conclude that feasible deployment scenarios for SBFD are deployments with low power and/or indoor gNBs. Alternatively, RAN4 may need to conduct new coexistence studies for SBFD and dynamic TDD, including macro deployments (e.g. Dense Urban scenario).

## Co-channel CLI

SBFD introduces new CLI type, namely co-channel inter-subband CLI. This interference can be better classified as:

  - gNB self-interference

  - intra-cell UE-to-UE co-channel inter-subband CLI

  - inter-cell UE-to-UE co-channel inter-subband CLI

  - gNB-to-gNB co-channel inter-subband CLI

Besides these new CLI types, in case of different frequency domain partitioning in neighbor cells, the system may also suffer from co-channel intra-subband CLI, i.e. CLI from transmissions on overlapping frequency resources:

- gNB-to-gNB inter-cell co-channel intra-subband CLI

- UE-to-UE inter-cell co-channel intra-subband CLI

Figure 1 Co-channel interference types in SBFD deployment

However, inter-cell co-channel intra-subband CLI (5 and 6 in Figure 1b) is the same as with dynamic TDD. We propose that potential enhancements to combat inter-cell co-channel intra-subband CLI are firstly studied under dynamic TDD enhancements, while the study on SBFD focuses on evaluating the performance impact of co-channel inter-subband CLI (1-4 in Figure 1a), including the impact on legacy operation, and on potential enhancements.

Proposal 2: The study on SBFD focuses on evaluating the performance impact of co-channel inter-subband CLI, including the impact on legacy operation, and potential enhancements.

RAN1 should derive proper models for co-channel inter-subband CLI so that its impact is carefully considered when evaluating the performance of SBFD as compared to static and dynamic TDD.

For UE-to-UE co-channel inter-subband CLI (both intra- and inter-cell), RAN1 should:

- Agree on a model for in-channel unwanted emission, e.g. based on the in-band emissions (IBE) model in TS 38.101, clause 6.4.2.3.

- Agree on a model for in-channel receiver selectivity. As no requirements for UE in-channel selectivity (ICS) currently exist in RAN4 specifications, RAN1 should send an LS to RAN4 asking what to assume for the UE in-channel receiver selectivity.

- Based on the reply from RAN4, RAN1 should then conclude on the model and parametrization for UE-to-UE co-channel inter-subband CLI to be used in the evaluations of SBFD.

It should be noted that by “in-channel” it is meant within the bandwidth of the Tx/Rx analog and digital filters at the transmitter/receiver. RAN4 should provide considerations on the ICS and in-channel unwanted transmissions performance of both legacy UE and SBFD optimized UE, depending on e.g. different assumptions on whether in-band emissions/in-channel selectivity are considered to be within or outside of the bandwidth of the UE Tx/Rx digital filter.

Proposal 3: RAN1 sends an LS to RAN4 asking for confirmation that the IBE model in TS in 38.101 can be used to model the UE in-channel unwanted emission, and what to assume for the UE in-channel receiver selectivity. RAN1 requests RAN4 to provide information on the performance of both legacy UE and SBFD optimized UE.

For gNB-to-gNB co-channel inter-subband CLI, RAN1 should similarly agree on models for both the gNB in-band unwanted emissions and the gNB in-channel receiver selectivity. No gNB IBE requirements exist in the RAN4 technical specifications. For gNB ICS, there are some requirements specified in TS 38.104, Section 7.8. However, as also discussed in [5] and [10], we think that the specified requirements are overly conservative and thus RAN4 should discuss new values to be used in system level evaluations, taking realistic gNB capabilities into account.

Proposal 4: RAN1 sends an LS to RAN4 asking for assumptions for the gNB in-band unwanted emissions and the gNB in-channel receiver selectivity. RAN1 requests RAN4 to provide information on the performance of both legacy gNB and SBFD optimized gNB.

gNB self-interference is handled in the next section.

## Considerations on feasibility of gNB self-interference

Self-interference is one of the most challenging interference types introduced with SBFD. Solutions to handle gNB self-interference with full duplex are e.g. physical isolation, analog/RF cancellation, and active digital cancellation. As active digital cancellation is typically used to suppress self-interference on overlapping frequency resources, in this contribution we focus on physical Tx-Rx isolation and analog/RF cancellation as primary techniques to combat gNB self-interference with SBFD.

Figure 2 shows a simplified example of legacy gNB RF architecture, and possible enhancements to achieve the required self-interference suppression needs for SBFD. With legacy RF architecture (1), the power is limited by circulator isolation and antenna matching. Additional RF cancellation is needed to suppress in-band emissions and relax receiver linearity requirements. However, RF cancellation solutions (2) are currently not commercially viable due to their high cost and complexity. Also, the extra noise from additional RF components may further degrade the uplink performance, even when SBFD is not being used. On top of that, the complexity and cost of RF cancellation solutions do not scale nicely with the number of TRXs. For all these reasons, Tx-Rx isolation from separate DL and UL antenna panels (3) is probably the most feasible solution for practical implementation of SBFD at the gNB. However, use of separate DL and UL antenna panels have significant impacts on the antenna size with inevitable consequences for the gNB volume and weight constraints. In other words, for the same antenna size/gNB volume, at least a 3dB array loss may be experienced as compared to the baseline of using the full available array for Tx or Rx. Moreover, the Tx-Rx reciprocity may be somewhat compromised, which may impact the ability to optimize throughput using advanced massive MIMO techniques. These aspects have also been highlighted in [4].

Legacy

RF cancel path

Dual antennas

Legacy

RF cancel path

Dual antennas

Figure 2 Example of legacy and enhanced gNB RF architecture for better support of SBFD

As an example, we have conducted simulations in a 3D Urban Micro (UMi) scenario with 200 meters inter-site distance, 46 dBm gNB Tx Power @3.5GHz carrier frequency, full buffer traffic model and 10 UEs per cell. We observe a ca. 30% loss in both DL sector and DL cell edge spectral efficiency when going from 32 ports to 16 ports transmission.

Proposal 5: When evaluating the performance of SBFD, the impacts of the specific BS RF architecture assumptions on the gNB volume and complexity needs to be carefully considered. Performance results should be benchmarked for comparable values of the gNB volume and complexity. The impact needs to be clearly stated when presenting performance results for SBFD in the technical report.

With all this said, we have conducted some preliminary simulations for SBFD in a small cell outdoor scenario, more specifically scenario 2a in TR 36.872, with 1 cluster of 4 small cells per macro cell area (500 meters inter-site distance). Only the small-cell layer is simulated. We assume 30 dBm gNB Tx power and full-buffer traffic, FR1 @3.5 GHz carrier frequency, 30 kHz SCS and 20 MHz carrier bandwidth, corresponding to 50 PRBs. More detailed simulation assumptions can be found in Appendix A. For simplicity, we consider SBFD with fixed DL-UL frequency domain partitioning in all symbols/slots with 24 PRBs used for DL, 2 PRBs of guard band between DL and UL, and 24 PRBs used for UL, as illustrated in a. In Figure 3b, we compare the normalized UL spectral efficiency of TDD and SBFD for different values of gNB self-interference cancellation (IC). On top of the self-interference cancellation, all simulations assume a co-channel inter-subband interference ratio (ISIR) of 45 dB, which includes for both gNB in-band unwanted emissions and gNB in-channel receiver selectivity. The gNB self-interference cancellation (IC) models the reduction of in-band interference due to Tx-Rx isolation and/or analog/RF cancellation.

DL (24 RBs)

UL (24 RBs)

Guard (2 PRBs)

time

frequency

FFFFF frame structure, same UL-DL split in F slots for all the small cells

DL (24 RBs)

UL (24 RBs)

Guard (2 PRBs)

time

frequency

FFFFF frame structure, same UL-DL split in F slots for all the small cells

Figure 3 SBFD UL-DL PRB partitioning and normalized UL SE for different assumptions of the gNB self-interference cancellation (IC)

These preliminary results show that, assuming an inter-subband interference ratio (ISIR) of 45 dB and a conservative yet realistic assumption of 50 dB IC, considering practical gNB RF architecture, SBFD brings 85% worse UL spectral efficiency than TDD. Even for small cell deployments, at least 80 dB of IC (this includes both Tx-Rx isolation and/or analog/RF cancellation) is needed to reach close-to-ideal performance. The requirement will be even more stringent for macro-type of base stations with e.g. 46 dBm Tx power.

Proposal 6: RAN1 sends an LS to RAN4 asking to provide a realistic range of values to be assumed in RAN1 evaluations for the in-band self-interference ratio and the self-interference cancellation for different assumptions on the BS RF architecture.

In general, we think that, after considering the potential impacts of inter-operator adjacent-channel CLI on the feasibility of the simulated deployment scenarios, and after agreeing on realistic models to properly consider the effects of gNB-to-gNB and UE-to-UE inter-subband CLI (including self-interference at the gNB), SBFD should provide significant gains, as compared to fixed TDD and dynamic TDD under similar assumptions and constraints, in order to justify the complexity of introducing support for SBFD in NR.

Proposal 7: Sufficiently large gain under realistic assumptions should be observed from SBFD as compared to fixed and dynamic TDD to justify the complexity of introducing support for SBFD.

## System design considerations

Frame structure and slot types

From an inter-operator adjacent-channel perspective, gNB-to-gNB CLI is typically more critical than UE-to-UE CLI [1]. Therefore, a legacy TDD operator may be more affected if a neighbor operator is using SBFD during UL legacy slots, as compared to DL legacy slots. Moreover, as one of the primary targets of introducing SBFD in unpaired spectrum is to increase UL capacity and UL coverage, enabling SBFD operation during DL legacy slots/symbols should have higher priority as compared to UL legacy slots/symbols.

Proposal 8: The SI on evolution of duplex operation should focus on enabling SBFD operation in DL legacy symbols/slots. Enabling SBFD operation in UL legacy symbols/slots should only be considered with lower priority.

This could mean that e.g. SBFD operation may not be allowed during UL legacy slots/symbols, in which case, it would be possible to operate the system with both SBFD and UL-only slots. In general, we think that it should be possible to operate with both SBFD slots/symbols and with DL-only and UL-only slots/symbols, as this may also facilitate the deployment of legacy devices in unpaired carriers when also using SBFD feature.

Proposal 9: In case SBFD is supported, system operation with SBFD symbols/slots in DL legacy slots/symbols, UL-only symbols/slots and DL-only symbols/slots should be allowed.

Initial access and Idle mode/connected mode support in SBFD slots/symbols

Another aspect that RAN1 needs to discuss is initial access when SBFD is deployed, including initial access by legacy devices. Since initial access can benefit from SBFD by e.g., having more RACH occasions, we think RAN1 should discuss if the initial access procedure should only be allowed in UL-only and DL-only slots/symbols, or also in SBFD slots/symbols. And in the latter case, whether initial access on SBFD slots/symbols should only be enabled for SBFD-aware UEs, or also for legacy UEs.

Proposal 10: Solutions for enabling initial access transmissions in SBFD slots/symbols should be studied, at least for SBFD-aware devices.

The advantages of UEs being aware of SBFD slots/symbols, including the DL-UL frequency domain partitioning, should also be discussed by RAN1. Both idle mode and RRC connected mode should be considered. One potential advantage for idle mode UE could be enabling initial access on SBFD slots/symbols, which may reduce the latency of the overall random-access procedure and reduce collision. One potential advantage for RRC connected UE could be allowing dynamic adaptation of semi-static allocated frequency resources, such as CG-PUSCH, based on the occurrence of SBFD slots/symbols and the corresponding DL-UL frequency domain partitioning. Based on the agreed advantages, RAN1 should also discuss the required enhancements to the current frame structure signaling (UL-DL TDD configuration) and slot format indicator/configuration.

Proposal 11: RAN1 should discuss the advantages in case idle and/or RRC connected UEs are aware of the occurrence of SBFD slots/symbols, including the UL-DL frequency domain partitioning. Based on the recognized advantages, RAN1 should identify the required enhancements to the legacy frame format signaling framework (UL-DL TDD configuration, SFI, etc.).

Power control in SBFD slots/symbols

As discussed in our companion contribution on dynamic TDD enhancements [6], boosting the UE Tx power in slots/symbols affected by gNB-to-gNB CLI from overlapping frequency resources can be a simple yet effective way to handle cross-link interference. Similar solutions may also be applicable with SBFD operation e.g. to compensate for the decrease in UL SINR due to gNB self-interference in SBFD symbols/slots. Therefore, we propose the following:

Proposal 12: The potential benefits of boosting the UE Tx power in SBFD slots/symbols shall be further investigated as a potential method to boost the UL received SINR in slots/symbols affected by gNB self-interference.

SBFD and Bandwidth Parts

During the Rel-18 RAN workshop and the following round of email discussions, few companies have proposed that, when deploying SBFD, the UL subband could be placed in the middle of the unpaired TDD carrier, so that the DL subband can act as an additional guard band towards adjacent legacy TDD carriers. See, for example, [7], [8] and [9]. One advantage of placing the UL subband in the middle of the TDD carrier may be the reduced gNB-to-gNB adjacent-channel CLI. However, as we also discuss in [10], the shape of typical gNB transmitter emissions is relatively flat across the adjacent channel, in which case, the CLI reduction could be negligible or at least limited to a few decibels. On the other hand, one potential drawback of placing the UL subband in the middle of the unpaired TDD carrier could be the higher number of PRBs that may be needed as guard band to limit intra-cell CLI (including self-interference at the gNB). Essentially, two guard bands between DL and UL subbands need to be deployed.

Proposal 13: RAN1 should evaluate different options with respect to where, within the unpaired TDD carrier, the DL and UL subbands may be configured during SBFD symbols/slots, and provide a preliminary analysis of the pros and cons of each option.

During the Rel-18 RAN workshop and the following round of email discussions, some companies seemed to assume that coupling the UE bandwidth part (BWP) to the corresponding UL/DL subband (gNB) in SBFD slots/symbols may be beneficial to mitigate/suppress UE-to-UE CLI (see e.g. [11]). However, unless the UL-DL frequency domain partitioning with SBFD is only semi-statically modified, this may require much faster UE digital filter bandwidth adaptation as currently possible with BWP switching. Based on current specifications, the BWP switch delay (in FR1) is between 0.75 ms and 3 ms, depending on the SCS and the UE capability. Note that semi-static UL-DL frequency domain partitioning basically implies it won’t be possible to operate with both SBFD, UL-only and DL-only symbols/slots, as by proposal 9. Furthermore, significant UE DL throughput reduction may be experienced in case the DL BWP for legacy UEs need to be constrained to only one of the two DL subbands (i.e. to ensure no UL PRBs are present in the DL BWP.)

Proposal 14: RAN1 should discuss the implications of the UE UL/DL BWP not being perfectly overlapped with the UL/DL subband in SBFD slots/symbols. RAN1 may additionally study the feasibility and potential advantages of BWP feature enhancements.

# Conclusion

In this contribution, we discussed subband non-overlapping full duplex (SBFD) for NR, and included considerations on new interference types and feasibility of SBFD. Based on the discussions, we have the following proposals:

Proposal 1: Based on Rel-16 coexistence study, RAN1 could conclude that feasible deployment scenarios for SBFD are deployments with low power and/or indoor gNBs. Alternatively, RAN4 may need to conduct new coexistence studies for SBFD and dynamic TDD, including macro deployments (e.g. Dense Urban scenario).

Proposal 2: The study on SBFD focuses on evaluating the performance impact of co-channel inter-subband CLI, including the impact on legacy operation, and potential enhancements.

Proposal 3: RAN1 sends an LS to RAN4 asking for confirmation that the IBE model in TS in 38.101 can be used to model the UE in-channel unwanted emission, and what to assume for the UE in-channel receiver selectivity . RAN1 requests RAN4 to provide information on the performance of both legacy UE and SBFD optimized UE.

Proposal 4: RAN1 sends an LS to RAN4 asking for assumptions for the gNB in-band unwanted emissions and the gNB in-channel receiver selectivity. RAN1 requests RAN4 to provide information on the performance of both legacy gNB and SBFD optimized gNB.

Proposal 5: When evaluating the performance of SBFD, the impacts of the specific BS RF architecture assumptions on the gNB volume and complexity needs to be carefully considered. Performance results should be benchmarked for comparable values of the gNB volume and complexity. The impact needs to be clearly stated when presenting performance results for SBFD in the technical report.

Proposal 6: RAN1 sends an LS to RAN4 asking to provide a realistic range of values to be assumed in RAN1 evaluations for the in-band self-interference ratio and the self-interference cancellation for different assumptions on the BS RF architecture.

Proposal 7: Sufficiently large gain under realistic assumptions should be observed from SBFD as compared to fixed and dynamic TDD to justify the complexity of introducing support for SBFD.

Proposal 8: The SI on evolution of duplex operation should focus on enabling SBFD operation in DL legacy symbols/slots. Enabling SBFD operation in UL legacy symbols/slots should only be considered with lower priority.

Proposal 9: In case SBFD is supported, system operation with SBFD symbols/slots in DL legacy slots/symbols, UL-only symbols/slots and DL-only symbols/slots should be allowed.

Proposal 10: Solutions for enabling initial access transmissions in SBFD slots/symbols should be studied, at least for SBFD-aware devices.

Proposal 11: RAN1 should discuss the advantages in case idle and/or RRC connected UEs are aware of the occurrence of SBFD slots/symbols, including the UL-DL frequency domain partitioning. Based on the recognized advantages, RAN1 should identify the required enhancements to the legacy frame format signaling framework (UL-DL TDD configuration, SFI, etc.).

Proposal 12: The potential benefits of boosting the UE Tx power in SBFD slots/symbols shall be further investigated as a potential method to boost the UL received SINR in slots/symbols affected by gNB self-interference.

Proposal 13: RAN1 should evaluate different options with respect to where, within the unpaired TDD carrier, the DL and UL subbands may be configured during SBFD symbols/slots, and provide a preliminary analysis of the pros and cons of each option.

Proposal 14: RAN1 should discuss the implications of the UE UL/DL BWP not being perfectly overlapped with the UL/DL subband in SBFD slots/symbols. RAN1 may additionally study the feasibility and potential advantages of BWP feature enhancements.

# References

- RP-220633, Study on Evolution of NR Duplex Operation, CMCC

- 3GPP TR 38.828, CLI handling and RIM for NR, V16.1.0 (2019-09)

- 3GPP TS 38.101-1, UE radio transmission and reception; Part 1: Range 1 standalone, V17.5.0 (2022-03)

- RWS-210295, On full Duplex, Ericsson

- R1-2204430, On the evaluation methodology for NR duplexing enhancements, Nokia, Nokia Shanghai Bell

- R1-2204432, Dynamic TDD enhancements, Nokia, Nokia Shanghai Bell

- RWS-210180, XDD (cross-division duplex) for enhanced duplexing operation in 5G Advanced, Samsung

- RWS-210026, On NR Full Duplex, Qualcomm

- RP-211997, Discussion on evolution of duplex operation, CMCC

- R1-2204433, RF considerations of dynamic TDD and SBFD, Nokia, Nokia Shanghai Bell

- RWS-210054, R18 Flexible/Full Duplex considerations, Spreadtrum Communications

# Annex A: Detailed simulation assumptions for SBFD

|  | Macro cell | Small cell |
| --- | --- | --- |
| Layout | Hexagonal grid, 3 sectors per site and 7 Macro sites. ISD = 500m | Clusters uniformly random within macro geographical area; small cells uniformly random dropping within cluster area |
| System bandwidth per carrier | 20 MHz | 20 MHz |
| Carrier frequency | 3.5 GHz. Separate carriers assumed for the small cell and Macro cell layer. No adjacent-channel interference assumed between the two carriers |  |
| Total BS TX power | 46 dBm | 30 dBm |
| Channel model (UE-to-gNB) | UMa in TR 38.901 | UMi in TR 38.901 |
| Channel model (UE-to-UE) | UMi in TR 38.901 |  |
| UE-to-UE min. coupling loss | 40 dB |  |
| Antenna Height | 25m | 10m |
| UE antenna Height | 1.5m |  |
| Antenna gain + connector loss | 8 dBi | 0 dBi (isotropic antenna element) |
| Antenna gain of UE | 0 dBi |  |
| Antenna configuration | {M, N, P, Mg, Ng} = {4, 4, 2, 1, 1}; (dH, dV) = (0.5, 0.5)λ | {M, N, P, Mg, Ng} = {2, 2, 2, 1, 1}; (dH, dV) = (0.5, 0.5)λ |
| Antenna configuration of UE | {M, N, P, Mg, Ng} = {1, 2, 2, 1, 1}; (dH, dV) = (0.5, 0.5)λ |  |
| Antenna element radiation pattern | According to TR 38.901 | Isotropic |
| Antenna element radiation pattern of UE | Isotropic |  |
| Number of clusters/buildings per macro cell geographical area | 1 |  |
| Number of small cells per cluster | 4 |  |
| Number of UEs | 300 UEs in total. 14 UEs per macro cell geographical area on average. |  |
| UE dropping | 2/3 UEs randomly and uniformly dropped within the clusters, 1/3 UEs randomly and uniformly dropped throughout the macro geographical area. 100% UEs are outdoor. |  |
| Radius for small cell dropping in a cluster | 50m |  |
| Radius for UE dropping in a cluster | 70m |  |
| Minimum distance (2D distance) | Small cell – small cell: 20m |  |
|  | Small cell – UE: 5m |  |
|  | Macro – small cell cluster center: 105m |  |
|  | Macro – UE : 35m |  |
|  | cluster center – cluster center: 2*Radius for small cell dropping in a cluster |  |
| Traffic model | Full buffer traffic in both DL and UL. |  |
| TDD frame configuration | {DDDDU} | FFFFF with 48% DL PRBs, 48% UL PRBs and 4% guard PRBs. F = SBFD slot |
| UE receiver | MMSE-IRC as baseline |  |
| BS noise figure | 5 dB | 9 dB |
| UE noise figure | 9dB |  |
| UE speed | Static with fast fading speed = 3km/h |  |
| Cell selection criteria | RSRP with cell range extension of 7.5 dB for the small cells. |  |