3GPP TSG RAN WG1 #109-e	R1-2204506

e-Meeting, May 9th – 20th, 2022

Source:	Sharp

Title:	Unified TCI framework extension for multi-TRP

Agenda Item:	9.1.1.1

Document for:	Discussion and Decision

Introduction

In RAN#94e, a new work item “MIMO Evolution for Downlink and Uplink” was approved, and the following objective was agreed for unified TCI framework extension [1].

- 2. Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework.

In this contribution, we share our views on extension of unified TCI framework for indication of multiple DL and UL TCI states.

Discussions

The necessity of indication of multiple DL and UL TCI states

In Rel-17, more than one TCI states with a single indication has been discussed and not been introduced. Figure 1 shows an example of use cases of indication of multiple DL and UL TCI states for multi-TRP operation.

DCI

Figure 1. Indication of multiple TCI states of unified TCI framework

One of the use cases of the extension of unified TCI framework is that single DCI indicates multiple active TCI states in multi-TRP operation. In our view, this scenario is one promising scenario in ideal backhaul scenario in Rel-18 as the extension of Rel-17 unified TCI framework (i.e. M > 1 and N > 1).

Proposal 1: RAN1 should introduce indication of multiple TCI states using Rel-17 unified TCI framework.

The maximum number of DL and UL TCI states

Up to Rel-17 specification, the maximum number of TRPs is 2. The motivation of this extension is to switch multiple active beam pair links with a single indication in multi-TRP case. Therefore, from our perspective, it would be enough the number of TCI states in an indication is the maximum number of TRPs in multi-TRP operation.

Proposal 2: The maximum number of DL and UL TCIs should be the same as the maximum number of TRPs in multi-TRP operation.

- If the number of TRPs in multi-TRP operation is not increased, M and N is up to 2.

Conclusion

In this contribution, we have the following proposals:

Proposal 1: RAN1 should introduce indication of multiple TCI states using Rel-17 unified TCI framework.

Proposal 2: The maximum number of DL and UL TCIs should be the same as the maximum number of TRPs in multi-TRP operation.

- If the number of TRPs in multi-TRP operation is not increased, M and N is up to 2.

References

RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Dec. 2021.