3GPP TSG RAN WG1 #109-e			R1-2204548

e-Meeting, May 9th – 20th, 2022

Source:	WILUS Inc.

Title:	Remaining issues on enhancements for PUSCH repetition Type A

Agenda item:	8.8.1

Document for:	Discussion/Decision

Introduction

In the RAN1#106b-e meeting [1] and RAN1#107b-e meeting [2], the following agreement and conclusion were made on enhancements on PUSCH repetition type A.

Agreement at RAN1#106b-e: - Only tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst are considered for the determination of available slots. - Any other RRC configuration is not considered for the determination of available slots. Conclusion at RAN1#107b-e: - The CovEnh discussion on the available slot counting for inter-cell mTRPs is deferred until further progress on the collision handling between UL channels/signals and multiple SSBs for inter-cell mTRPs is made in feMIMO session.

In this contribution, we provide our views on determination of available slots for enhancements on PUSCH repetition Type A.

Discussion on determination of available slots

Use of SSBs across multiple TRPs for the available slot determination.

In NR Rel-17 feMIMO WI, it was agreed to support different SSB sets with different PCIs transmitted from mTRPs in a same serving cell. In other words, different SSB sets can be configured by different sets of ssb-PositionsInBurst from mTRPs in a same serving cell. Accordingly, whether/how to consider different SSBs across mTRPs for the available slot determination should be discussed. Consequently, it was concluded at the RAN1#107b-e meeting that available slot counting for inter-cell mTRPs is deferred until further progress on the collision handling between UL channels/signals and multiple SSBs for inter-cell mTRPs is made in feMIMO WI.

In the previous RAN1#108-e meeting [3], the following agreement was made in feMIMO WI.

Agreement at RAN1#108-e: For inter-cell mTRP, UE does not transmit PUCCH/PUSCH/PRACH in a slot or SRS in the symbols if in time domain the PUCCH/PUSCH/PRACH/SRS overlaps with an SSB of a serving cell PCI or an SSB associated with the active additional PCI.

Accordingly, we provide our views on potential options for available slot determination over SSBs across mTRPs.

- Option 1: SSBs with PCI different from the serving cell PCI are only used for actual PUSCH repetition determination after the available slots are determined.

With this option, SSBs with serving cell PCI are used for available slot determination in Step 1, while SSBs with PCI different from the serving cell are considered in Step 2.

In the former case, symbol(s) configured for SSBs with the serving cell PCI can be regarded as unavailable symbol(s) for PUSCH repetition Type A. Thus, if a collision occurs between symbol(s) for PUSCH repetition Type A and symbol(s) configured for SSBs with the serving cell PCI in a slot, the slot is not counted as available slot for PUSCH repetition Type A and the PUSCH repetition should be deferred to the next available slot for UL transmission.

In the latter case, Rel-17 dropping rule can be applied to the case of collision between symbol(s) for PUSCH repetition Type A and symbol(s) configured for SSBs with PCI different from the serving cell. According to Rel-17 dropping rule, a UE does not transmit PUSCH in a slot if in time domain the PUSCH overlaps with an SSB associated with the active additional PCI.

- Option 2: Both SSBs with serving cell PCI and SSBs with PCI different from the serving cell PCI are used for the available slot determination.

In our view, Option 2 can be divided into two sub-options, i.e., whether to consider SSBs from inter-cell mTRP or not.

  - Option 2-1: Different SSBs with PCI different from the serving cell for inter-cell mTRP operation.

In this case, SSBs include SSBs with PCI different from the serving cell in the different cell. Based on current agreements, only SSBs in the serving cell are considered for available slot determination, while SSBs in the different cell (e.g., SSB by SMTC) are not considered. Therefore, Rel-17 dropping rule can be applied to the case of collision between symbol(s) for PUSCH repetition Type A and symbol(s) configured for SSBs with PCI different from the serving cell from inter-cell mTRP. According to Rel-17 dropping rule, a UE does not transmit PUSCH in a slot if in time domain the PUSCH overlaps with an SSB associated with the active additional PCI.

  - Option 2-2: Different SSBs with different PCIs transmitted from mTRPs in a same serving cell.

In this case, SSBs include both SSBs with serving cell PCI and SSBs with PCI different from the serving cell in a same serving cell. SSBs in a serving cell can be considered for available slot determination regardless of PCI to prioritize the reception of SSBs in a serving cell. Thus, if a collision occurs between symbol(s) for PUSCH repetition Type A and symbol(s) configured for SSBs transmitted from mTRPs in a same serving cell in a slot, the slot is not counted as available slot for PUSCH repetition Type A, and the PUSCH repetition should be deferred to the next available slot for UL transmission.

- Option 3: SSBs with the same PCI as the SSB used to determine the spatial relation of the PUSCH, are used to determine the available slots. Other set of SSBs are used for actual PUSCH repetition determination after the available slots are determined.

With this option, regardless of PCI same or different from the serving cell, and mTRPs in a same serving cell or inter-cell, SSBs with PCI that spatially related to the PUSCH are used for available slot determination. In this case, SSBs with PCI different from the serving cell for inter-cell mTRP (i.e., Option 2-1) can be included. As discussed in Option 2-1, it would be better to not consider SSBs from inter-cell mTRPs for the available slot determination.

Based on above discussion, we propose that both SSBs with serving cell PCI and SSBs with PCI (i.e., active additional PCI) different from the serving cell PCI are used for the available slot determination (i.e., Option 2). In Option 2, different SSBs with different PCIs transmitted from mTRPs in a same serving cell can be used for the available slot determination (i.e., Option 2-2).

- Proposal 1: We propose that both SSBs with serving cell PCI and SSBs with PCI (i.e., active additional PCI) different from the serving cell PCI are used for the available slot determination (i.e., Option 2).

  - Different SSBs with different PCIs transmitted from mTRPs in a same serving cell.

Conclusion

In this contribution, we discussed a counting rule on the available slot for PUSCH repetition Type A and summarize our views as the following:

- Proposal 1: We propose that both SSBs with serving cell PCI and SSBs with PCI (i.e., active additional PCI) different from the serving cell PCI are used for the available slot determination (i.e., Option 2).

  - Different SSBs with different PCIs transmitted from mTRPs in a same serving cell.

References

- Final Report of 3GPP TSG RAN WG1 #106b-e v1.0.0

- Final Report of 3GPP TSG RAN WG1 #107b-e v1.0.0

- Draft Report of 3GPP TSG RAN WG1 #108-e v0.1.0