3GPP TSG RAN WG1 #109-e                                                                                        R1-2204867

E-Meeting, May 9th – 20th, 2022

Agenda item:	9.4.2

Source:	Robert Bosch GmbH

Title:	On co-channel coexistence between LTE and NR sidelink

Document for:	Decision

# Introduction

In Rel-18 WI [1], an important V2X deployment scenario aspect is consider to study and, if necessary, specify LTE V2X and NR V2X coexistence in the same frequency channel. For the two different types of devices to coexist while using a common carrier frequency, it is important that there is mechanism to efficiently utilize resource allocation by the two technologies without negatively impacting the operation of each technology. This requirement was also mentioned as part of the input from 5G Automotive Association to the Rel-18 RAN Workshop [2].

- Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4]

- Reuse the in-device coexistence framework defined in Rel-16 as much as possible

In this TDoc, we present our direction and proposals for this important, automotive related, aspect.

# LTE/NR-V2X sidelink co-channel coexistence

Co-channel coexistence of LTE and NR sidelink is not supported in Rel. 16 V2X, since each of these RATs is utilizing a different channel and/or resource pool [3]. Since the available ITS frequency spectrum for C-V2X direct communication is currently very limited, e.g., the spectrum in US has been reduced to 30MHz only, LTE-NR V2X coexistence is becoming very crucial to speed up NR-V2X deployment.  Additionally, some use cases which have been built up based on LTE and NR V2X, may need to coexist concurrently, which will require such dynamic, quasi-dynamic or even static coexistence.

The coexistence between NR and LTE V2X will also support an eventual and smooth transition from LTE-V2X to NR-V2X in future. Therefore, such a coexistence mechanism need to consider the traffic density and channel load of each RAT to achieve this task efficiently.

In a more advanced version of this coexistence, a harmonized and fair co-channel sharing between 3GPP RATs and non-3GPP technology (e.g., 802.11p/bd) is recommended from automotive and regulatory perspective. Therefore, it is recommended to study extended the coexistence mechanism in this WI to consider cross-technology coexistence with the minimum possible specification impact. This will guarantee a future proof specifications and readiness to varieties of regulations in the different geographical regions.

In this TDoc, we are proposing to strive for an LTE-NR coexisting mechanism that guarantees:

- no changes on the lower layers of existing LTE V2X devices

- a solution that can be easily retrofitted to R16/R17 (if needed)

- avoiding adoption of LTE protocol stack in NR devices as much as possible

Our proposals are centric around 3 possible schemes:

- Scheme 1: Static

- Scheme 2: Quasi-dynamic/quasi-static

- Scheme 3: Dynamic spectrum sharing

Wherein all previous schemes may require, at least, NR devices to be fully synchronized to the LTE devices existing in the same network.

Objective 1: Co-channel coexistence between LTE- and NR-V2X may be based on static, quasi-dynamic, and/or dynamic schemes.

Proposal 1: LTE-NR coexistence schemes needs to guarantee:

- no changes on the lower layers of existing LTE V2X devices

- a solution that can be easily retrofitted to R16/R17 (if needed)

- avoiding adoption of LTE protocol stack in NR devices as much as possible

## Scheme 1: Static co-channel coexistence

During the development of Rel-18 scope for NR sidelink enhancements considering LTE-NR co-channel coexistence, the following alternative was proposed:

Alt 1: Resource pool separation between the two RATs

Alt 2: Dynamic resource sharing using overlapping resource pools between the two RATs.

In this case, a static configuration and/or pre-configuration may consider different, non-overlapping, resource pools where LTE and NR sidelink may coexist either in FDM or TDM fashions. In this case, different numerologies (between LTE and NR sidelink) can be configured. Since this solution is static, it may fit both out of coverage and in network coverage. In the latter, the sidelink configuration can be controlled by the network (via eNB/gNB) allowing different shares of each RAT.

Figure 1: TDM and FDM possible co-channel coexistence (note: same and different numerologies in the figure are for illustration only and they may not happen simultaneously in the same resource pool)

Objective 2: Static FDM/TDM resource pool configuration for NR/LTE coexistence fits both in coverage and out of coverage scenarios

Proposal 2: Study resource pool separation between LTE and NR sidelink using static FDM and TDM (pre-)configuration

## Scheme 2: Quasi-dynamic co-channel coexistence

In this section, we propose to study a possible quasi-static or quasi-dynamic co-channel coexistence mechanism between LTE and NR sidelink. In this case, also different resource pools can be (pre-)configured, but considering overlapping portions. It could also be assumed that each RAT may have its preferred non-overlapping section, where in the overlapping part may be used based on detect-and-vacate approach (proposed by 5GAA in [4]). In this case, the detect-and-vacate may use one of the possible energy detection schemes in 3GPP, i.e., RSSI based measurements and/or CBR measurements. Both RAT may be configured to vacate such a shared set of resources if the other RAT exists with, e.g., a certain CBR threshold.

One advantage in this scheme is that vacating the shared resources will be in favor of the high density RAT, i.e., low density RAT will vacate for the higher density one. Another advantage is that mutual decoding of control channels between the two RATs are not required. See Figure 2 (adopted from [4] with modification to fit the discussion here).

Figure 2: different RAT-based preferred resources and complemented by a shared portion by mutual detect-and-vacate approach (adopted from [4]).

Since the detect-and-vacate quasi-dynamic resource sharing does not require decoding of the opposite RAT, it was proposed by 5GAA in [4] to be used to allow coexistence between 3GPP RATs and non-3GPP techniques, namely IEEE 802.11b. Therefore, this scheme, if considered, may be extended to consider coexistence to another technology additionally.

Objective 3: Detect-and-vacate on shared areas can be decided based on RSSI-based CBR measurement, i.e., without decoding the reservation of the other RAT.

Objective 3: Detect-and-vacate has been proposed as a solution to coexist with non-3GPP technologies.

Proposal 3: Study possible quasi-dynamic co-channel coexistence between LTE and NR V2X based on detect-and-vacate of shared resources.

Proposal 4: Study if extending quasi-dynamic/detect-and-vacate coexistence mechanism may support coexistence with non-3GPP technologies.

## Scheme 3: Fully dynamic spectrum sharing

Similar to the static and quasi-dynamic schemes, a dynamic technique also requires NR-V2X devices to be fully synchronized to LTE-V2X ones. Additionally, a fully dynamic scheme may consider fully overlapping resource patterns. In this case, and assuming that LTE devices are not required to be adapted for these schemes, it is important to study:

- How to discover LTE sidelink transmission (i.e., Whether decoding/sensing of LTE transmission is necessary in this case) or whether energy detection only is possible or is sufficient

- How to enhance NR V2X Rel-16 pre-emption and/or re-evaluation to avoid concurrent LTE V2X transmissions.

Note: in [5], different possible energy detection schemes were considered to allow coexistence between LTE-V2X and non-3GPP schemes. Therefore, in our understanding, a suitable energy detection scheme may also allow seamless coexistence between LTE and NR-V2X.

Objective 5: Dynamic spectrum sharing-like techniques may require fully overlapping, but synchronized, resources

Proposal 5: For dynamic spectrum sharing between LTE and NR V2X study whether:

- Discovery/sensing of LTE sidelink transmission (by NR V2X devices) is possible or energy detection only is sufficient

- NR V2X Rel-16 pre-emption and/or re-evaluation can be enhanced to avoid collision with concurrent LTE V2X transmissions.

# Conclusions

In this contribution the following proposals have been made:

Proposal 1: LTE-NR coexistence schemes needs to guarantee:

- no changes on the lower layers of existing LTE V2X devices

- a solution that can be easily retrofitted to R16/R17 (if needed)

- avoiding adoption of LTE protocol stack in NR devices as much as possible

Proposal 2: Study resource pool separation between LTE and NR sidelink using static FDM and TDM (pre-)configuration

Proposal 3: Study possible quasi-dynamic co-channel coexistence between LTE and NR V2X based on detect-and-vacate of shared resources.

Proposal 4: Study if extending quasi-dynamic/detect-and-vacate coexistence mechanism may support coexistence with non-3GPP technologies.

Proposal 5: For dynamic spectrum sharing between LTE and NR V2X study whether:

- Discovery/sensing of LTE sidelink transmission (by NR V2X devices) is possible or energy detection only is sufficient

- NR V2X Rel-16 pre-emption and/or re-evaluation can be enhanced to avoid collision with concurrent LTE V2X transmissions.

# References

[1] RP-220300, “WID revision: NR sidelink evolution,” OPPO, 3GPP TSG RAN Meeting #95e, March 2022

[2] RWS-210360, “5GAA input to 3GPP Rel.18 Workshop,” 5GAA, RAN-Rel-18 workshop, June 2021

[3] RP-200129, “Revised WID on 5G V2X with NR sidelink,” LG Electronics, 3GPP TSG RAN Meeting #87e, March 2020.

[4] 5GAA Position Paper, “Coexistence of C-V2X and ITS-G5 at 5.9GHz,” 5GAA, 05 April 2018

[5] White paper, “ITS-G5 and Sidelink LTE-V2X Co-Channel Coexistence Mitigation Methods,”