3GPP TSG RAN WG1 #110bis-e	R1-2208371

e-Meeting, October 10th – 19th, 2022

Agenda Item:	9.4.1.2

Source:	FUTUREWEI

Title:	Discussion on physical channel design for sidelink on unlicensed spectrum

Document for:	Discussion and decision

# Introduction

At RAN#94-e, a WI on sidelink evolution was agreed for Rel-18 (RP-213678) [1]. In this WI, an objective on sidelink in unlicensed spectrum is included as:

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4]

- Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation

  - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation

    - No specific enhancements for Rel-17 resource allocation mechanisms

    - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval.

- Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum

  - The existing NR sidelink and NR-U channel structure shall be reused as the baseline.

- No specific enhancements for existing NR SL feature

- The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

In this contribution, we discuss the resource allocation and physical channel design for sidelink transmission in unlicensed spectrum.

# Discussions

## PSCCH and PSSCH in SL-U

In RAN1#110 the following agreements established the framework for PSCCH and PSSCH design.

Agreement

For PSCCH and PSSCH in SL-U:

- Both R16/R17 NR SL contiguous RB-based and interlace RB-based transmissions similar to R16 NR-U are supported

Agreement

For PSCCH and PSSCH in SL-U:

- For interlace RB-based transmission

  - Frequency domain resource allocation granularity is one sub-channel for PSSCH transmission

  - 1 sub-channel equals K interlace

    - FFS: whether K is fixed as 1 or (pre-)configured

  - Discuss whether one or both of the following alternatives are supported

    - Alt 1: 1 sub-channel is confined within 1 RB set

    - Alt 2: 1 sub-channel spans 1 or multiple RB set(s) belonging to a resource pool

One of the selection criteria for the resource granularity is its flexibility to allow resource allocation from a very small to a very large size.

In TS 38.214, Clause 7, an “RB set” is defined in TS 38.214 as the number of contiguous RBs between two consecutive guard bands when the UE is configured with any of IntraCellGuardBandsPerSCS.

“For operation with shared spectrum channel access for FR1, when the UE is configured with any of IntraCellGuardBandsPerSCS for UL carrier and for DL carrier with SCS configuration μ, the UE is provided with  NRB-set,x-1 intra-cell guard bands on a carrier with μ.”

“The intra-cell guard bands separate NRB-set,x RB sets, each defined by start and end CRB, RB s,xstart,μ and RB s,xend,μ, respectively.”

When the UE is not configured with IntraCellGuardBandsPerSCS, there is just a RB set per carrier.

“When the UE is not configured with IntraCellGuardBandsPerSCS for μ, the UE determines the CRB indices for the intra-cell guard band(s), if any, and corresponding RB set(s) according to the nominal intra-cell guard band and RB set pattern as specified in [8, TS 38.101-1] corresponding to μ and carrier size Ngrid,xsize,μ. For either or both DL and UL, if the nominal intra-cell guard band and RB set pattern as specified in [8, TS 38.101-1] contains no intra-cell guard bands, the number of RB sets for the carrier is NRB-set,x=1”

The number of RBs per RB set is also defined in TS 38.214: “For μ=0, the UE expects the number of RBs within an RB set is between 100 and 110. For μ=1, the UE expects the number of RBs within a RB set is between 50 and 55 except for at most one RB set which may contain 56 RBs.”

Therefore, if a SL-U sub-channel is confined in an RB set, the sub-channel size is for μ=0, between 18 MHz and 19.8MHz, which corresponds to a NR-U channel of 20 MHz raster. For SL-U wideband access a NR-U like multiple channel access may be used, where a NR-U channel would correspond to a SL-U subchannel.

Therefore, the use of a sub-channel per RB set allows large size resource allocations.

To achieve small size resource allocation while using a sub-channel of 100-110 RBs, one or more interlace indices can be used. For instance, for scs15 (15 kHz SCS) TS38.331 defines for PUCCH that interlace index can take values in {0...9} and for scs30 (30 kHz SCS) values in {0…4}. In TS38.211 the number of common resource blocks in an interlace is defined for 15 kHz SCS as 10 and for 30 kHz SCS as 5.  Using a single interlace index in a 20 MHz sub-channel, one can select about 2MHz of resources, which would be the minimum resource size. Using multiple (K) interlace indices, the resource size can be increased between 2MHz to 20 MHz in steps of 2MHz.

Resource sizes lower than 2MHz can be obtained if interlaces larger than 10 are defined.

Proposal 1: Use a (pre-)configured K number of interlaces per sub-channel, where a sub-channel is configured within one RB set.

## PSFCH and SL HARQ in SL-U

In RAN1#109 it was agreed that at least R16 NR SL PSFCH format 0 is supported.

Agreement

For PSFCH and SL-HARQ in SL-U:

- At least R16 NR SL PSFCH format 0 is supported

- FFS whether to introduce new PSFCH format

- FFS: how to meet OCB and PSD requirement for PSFCH transmission, e.g., using interlaced RB transmission, whether/how to avoid too small PSFCH capacity, etc.

- FFS: the locations of PSFCH resources, e.g., (pre-)configured, dynamically indicated, etc.

- FFS: whether/how to address PSFCH transmission dropping due to LBT failure, e.g., whether to have multiple PSFCH occasions for a PSSCH and the related PSSCH-PSFCH mapping relationship, impact on SL HARQ-ACK reporting to the gNB for Mode 1, etc.

- FFS: whether/how to address PSFCH and related PSSCH in different COTs

However, format 0 uses a single PRB, which cannot satisfy the OCB requirements as was shown in a previous contribution [R1-2205745].

RAN1#110 adopted a couple agreements to satisfy the OCB requirements for the PSFCH transmission.

Agreement

To meet OCB and PSD requirement for PSFCH transmission, at least RB-based interlace is supported at least for 15 kHz and 30 kHz SCS, FFS details.

Agreement

Regarding PSFCH transmission, at least the followings alternatives can be further studied

- Alt 1: each PSFCH transmission occupies a common interlace and zero or one or more dedicated PRB(s)

- Alt 2: each PSFCH transmission occupies an interlace, and may or may not further apply code domain enhancement (e.g., OCC, PRB-level cyclic shifts)

- Alt 3: each PSFCH transmission occupies some dedicated PRBs and some common PRBs

- FFS details of above alternatives

The agreement focuses on three alternatives for OCB requirement. In RAN1#110 many companies observed that PSFCH format 0 is very similar to PUCCH format 0, and as in NR-U, for OCB solution an interlaced approach can be used. If an interlace of M=10 (for 15kHz SCS) is used the transmission takes place in one PRB every 10 PRBs. The interlace index m takes values in {0,1, 2, …, M-1} and consists of common resource blocks {m, m+M, m+2M, m+3M, …}.  The use of an interlace for each PSFCH is a straightforward way achieve OCB at the expense of lowering channel capacity. Alt 3 and Alt1 are somewhat equivalent. Alt 1 is compatible with previous agreement (“at least RB-based interlace is supported”), while Alt 3 requires an additional design. Therefore, we do not think that Alt 3 is necessary.

Proposal 2: For PSFCH use only interlace-based solutions for the OCB requirement (either Alt 1 or Alt2).

At this time RAN1 did not decide yet if the PSFCH transmission should be considered short control transmission and therefore exempt from LBT.  However, an agreement was reached for the case when a LBT prior to PSFCH is required.

Agreement

If RAN1 decides that LBT is performed for PSFCH transmission, for the time and frequency domain locations of PSFCH resources, at least the followings alternatives can be further studied

- Alt 1: PSFCH resources are (pre-)configured

- Alt 2: PSFCH resources are dynamically indicated

- Combination of above alternatives are not precluded

- FFS details of above alternatives

In the RAN1#110 meeting most companies suggested that to cope to LBT failure, additional PSFCH opportunities with respect to the licensed case should be considered. We note that both Alt 1 and Alt 2 of the above agreement offer such possibility.

Even when the LBT is required, the LBT may not be necessary if the gap between consecutive transmissions in the same COT is less than 16us in FR1. To shorten the gap, and retain the channel control, a cyclic prefix extension (CPE) may be considered [TS 38.211, TS 38.212].

Proposal 3: For COT sharing purposes use the CPE to avoid LBT prior to PSFCH transmissions.

Another potential issue may occur if the SL transmission is close to the end of COT, which may not allow enough time for the PSFCH transmission in the same COT. The simplest solution to this potential problem is to not allow PSSCH transmission close to the end of COT. Another solution may be to allow PSFCH transmissions outside of the COT. Such solution would imply the use of PSFCH (pre-)configured resources rather than a dynamic indication from the COT initiator beyond COT termination.

Proposal 4: The transmission of PSSCH and the dynamically indicated PSFCH opportunity should belong to the same COT.  Consider the cross-COT PSFCH using only (pre-)configured resources.

## S-SSB in SL-U

In RAN1#110 it was decided to support additional S-SSB transmissions.

Agreement

If RAN1 decides that LBT is performed for S-SSB transmission, in addition to the S-SSB occasions in R16/R17 NR SL design, support additional candidate S-SSB occasions

- FFS the number and locations of additional candidate S-SSB occasions

- FFS when a UE transmits S-SSB on such additional candidate S-SSB occasions, and the related Rx UE’s behavior

The periodicity of S-SSB with SL synchronization signals is fixed to be 160ms. The number of S-SSB transmissions in each period is (pre-)configured. In Rel-16, the following number of S-SSB transmissions in one 160ms period for (pre-)configuration has been specified, which is SCS dependent and frequency band dependent.

- For FR1:

  - For 15kHz SCS, {1}

  - For 30kHz SCS, {1, 2}

  - For 60kHz SCS, {1, 2, 4}

- For FR2:

  - For 60kHz SCS, {1, 2, 4, 8, 16, 32}

  - For 120kHz SCS, {1, 2, 4, 8, 16, 32, 64}

Alternatively, more S-SSB transmissions can be supported for sidelink unlicensed access to minimize the LBT failure impact, if LBT is required before S-SSB transmission. For example, support 4, 8, and 16 S-SSB transmissions within a 160ms period for 15kHz, 30kHz, and 60kHz SCS, respectively, in FR1. Increasing two to four times the number of S-SSB transmissions in a period allows dealing with LBT failures or other RAT interference. Further increasing the number of transmissions may increase unnecessary the overhead, interference, and power consumption.

Proposal 5: If RAN1 decides that LBT is performed for S-SSB transmission, support up to 4, 8, and 16 S-SSB transmissions within a 160ms period for 15kHz, 30kHz, and 60kHz SCS, respectively.

One of the issues to solve in the S-SSB design in shared spectrum is the requirement of occupied channel bandwidth (OCB).

Agreement

For S-SSB and synchronization in SL-U:

- No changes on R16 NR SL S-PSS/S-SSS sequence generation

- Continue studying the 4 options from the previous agreement and whether/how temporary exemption of OCB requirement is applicable for S-SSB transmission, e.g., how to meet the minimum of 2 MHz requirement under 15 kHz SCS.

In licensed bands, the S-SSB has one slot time duration and occupies 11 PRB (132 subcarriers) of frequency resources. For 15kHz SCS the OCB is 1.980 MHz. Therefore, the S-SSB without additional changes does not satisfy the OCB requirement for 20MHz channels. ETSI EN 301.893 defines exempt short transmissions of the OCB requirements:

“During Channel Occupancy Time (COT) equipment may operate temporarily with an Occupied Channel Bandwidth of less than 80% of its Nominal Channel Bandwidth with a minimum of 2MHz.”

However, at 15kHz SCS a slot duration lasts 1ms; therefore the following observation can be made:

Observation 1: One slot S-SSB (1ms at 15kHz SCS) does not qualify as temporary transmission, which should be much shorter than COT duration (3-4ms).

This observation implies that a solution for OCB requires a new S-SSB design in shared spectrum at least for 15kHz SCS S-SSB.

RAN1 agreed to continue studying four options for S-SSB design that focus on satisfying the OCB condition.

Option 1 uses interlace PRB for S-SSB transmission. The interlace(s) index is preferable to be known in advance at the receiving SL UE to reduce synchronization complexity and latency.  This implies that the interlace index is the same for all S-SSB transmitters.

Option 2 allows multiplexing S-SSB with other sidelink signals, which implies that other signals may be transmitted in the same resource pool as S-SSB. If a S-SSB is transmitted in a separate resource pool, these signals must be available for transmission at the same time as S-SSB, which is difficult to guarantee. It seems that this option would require S-SSB transmission in a shared pool, which is a departure from sidelink in licensed bands where the S-SSB transmission takes place in a separate resource pool.

Option 3 uses repetition in frequency domain to achieve OCB. For instance, for 20 MHz channels, the S-SSB may be transmitted the lower 2MHz and the upper 2MHz of the channel bandwidth. To have a reduced synchronization latency and a reduced implementation complexity, at least one of these copies must have a known frequency location, preferrable at the bottom of the SL-U frequency band transmissions similar to licensed band solution.

Option 4 uses a reduced number of symbols while increasing the BW of the S-SSB. S-SSB in licensed bands occupies 11RB and 11 or 13 symbols (Figure 1), while SSB occupies 20RBs and 4 symbols.  A simple reordering in frequency may reduce the number of S-SSB transmitted symbols while increasing the BW.  Thus, the S-SSB may be reduced to 6 symbols only if just duplicate the bandwidth without any other changes and qualify for an exempt of the OCB due to its short duration of 504us at 15kHz SCS (including symbol duplication for AGC purposes), which is less than 584us used for short control signalling LBT exempt. Our preference is to keep the S-SSB design as much as possible compatible with licensed design of S-SSB. Therefore, we do not think that a reduction in number of SSB symbols is necessary.

Figure 1, Sidelink SSB configuration for licensed bands, longer than 584us duration

Figure 2, Possible S-SSB configuration for a reduced number of symbols, less than 584us duration

Proposal 6: For shared spectrum, SL-U supports either Option 1 or Option 3 to satisfy the S-SSB OCB requirements at least for 15kHz SCS. For higher SCS consider S-SSB transmission under OCB and LBT short transmission exemption.

The increase of the number of S-SSB opportunities was supported by most companies in RAN1#110 meeting because it helps mitigate LBT failures and other RAT interference. But, increasing S-SSB opportunities protects against other RAT interference but not for possible collisions between S-SSB transmissions. For periodic transmissions of S-SSB, once a collision between two transmissions takes place, it will repeat periodically. Additional solutions may be further considered to increase the S-SSB robustness against interference with other S-SSB transmissions.  A simple solution could be to use different interlace indices for different S-SSB transmissions, where different interlaces are orthogonal to each other in frequency, or another solution is just to identify when such collisions occur and then change the S-SSB transmission parameters.

Proposal 7: For shared spectrum, SL-U should mitigate potential S-SSB collisions.

# Conclusion

Proposal 1: Use a (pre-)configured K number of interlaces per sub-channel, where a sub-channel is configured within one RB set.

Proposal 2: For PSFCH use only interlace-based solutions for the OCB requirement (either Alt 1 or Alt2).

Proposal 3: For COT sharing purposes use the CPE to avoid LBT prior to PSFCH transmissions.

Proposal 4: The transmission of PSSCH and the dynamically indicated PSFCH opportunity should belong to the same COT.  Consider the cross-COT PSFCH using only (pre-)configured resources.

Proposal 5: If RAN1 decides that LBT is performed for S-SSB transmission, support up to 4, 8, and 16 S-SSB transmissions within a 160ms period for 15kHz, 30kHz, and 60kHz SCS, respectively.

Observation 1: One slot S-SSB (1ms at 15kHz SCS) does not qualify as temporary transmission, which should be much shorter than COT duration (3-4ms).

Proposal 6: For shared spectrum, SL-U supports either Option 1 or Option 3 to satisfy the S-SSB OCB requirements at least for 15kHz SCS. For higher SCS consider S-SSB transmission under OCB and LBT short transmission exemption.

Proposal 7: For shared spectrum, SL-U should mitigate potential S-SSB collisions.

# References

RP-213678, “NR sidelink evolution”, RAN#94-e, Dec. 2021.

ETSI EN 301 893 5 GHz RLAN; Harmonized Standard covering the essential requirements

RAN1#109 Chair Notes

RAN1#110 Chair Notes