3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2208423

e-Meeting, October 10 – 19, 2022

Agenda Item:	9.8.2

Source:	Huawei, HiSilicon

Title:	Configuration for side control information signalling of network-controlled repeaters

Document for:	Discussion and Decision

# Introduction

The work item of NR network-controlled repeaters (NCR) was approved at RAN#97e [1]. The detailed objectives related to RAN1 include:

- Specify the signalling and behavior of the following side control information for controlling the NCR-Fwd

  - Beamforming

  - UL-DL TDD operation

  - ON-OFF information

- Note: Power control aspect will be checked in RAN#98e.

- Specify control plane signalling and procedures

  - The configuration of signalling for side control information indication

    - NOTE: Down-selection of solutions in section 7.2 of TR 38.867 is needed

In this contribution, we discuss the configuration of signaling for side control information indication.

# Configuration of signaling for side control information indication

Based on the study in [2], the following options are considered for the necessary configuration of signaling:

The NCR-MT can obtain the necessary configuration for receiving the L1/L2 signaling of the side control information. - Option 1: The necessary configuration is from RRC. - Option 2: The necessary configuration is from OAM or hard-coded. - Option 3: The necessary configuration is partially configured by RRC and partially configured by OAM or hard-coded. The necessary configurations from RRC and/or OAM (or hard-coded) contain: - The configurations of PHY channels to carry the L1/L2 signaling including - The configurations for receiving PDCCH and PDSCH. - The configurations for transmitting PUCCH, if needed. - The configurations for transmitting PUSCH, if needed. - The configurations of L1/L2 signaling including - The configurations for DCI. - The configurations for UCI, if needed. - The configurations for MAC CE, if needed. For the parameters in the necessary configurations for L1/L2 signaling, the existing parameters for PDCCH, PDSCH, PUCCH, PUSCH, DCI, UCI and MAC CE in Rel-17 are the baseline for further discussion.

In NR, cell common RRC signaling is used to configure all the PHY channels and L1/L2 signaling for UEs during initial access, and dedicated RRC signaling can reconfigure the PHY channels and L1/L2 signaling for a specific UE. The cell common configuration is received by all the UEs in a cell such as to establish a link connection. The reconfigured parameters can be different for different UEs and may also change over time.

For NCR, NCR-MT is able to receive the cell common configuration (e.g., PBCH, SIB1) and access the gNB following a normal UE initial access procedure. NCR-MT can also receive RRC reconfigurations via dedicated RRC signaling. In comparison, hard-coded configuration is fixed all the time which does not provide configuration flexibility in practical deployment. OAM based configuration is expected to be set for a long time duration or to be changed manually for each NCR which may bring heavy workload.

Observation 1: For the necessary configuration for receiving the L1/L2 signaling of the side control information, OAM based configuration is expected to be fixed for long time durations, and has to be changed manually, while hard-coded configuration is fixed all the time and hence does not provide configuration flexibility.

Thus, we propose:

Proposal 1: The necessary configuration for receiving the PHY channels and L1/L2 signaling of the side control information is configured via RRC.

# Conclusions

The contribution provides our considerations on NCR side control information, and the observations are listed as following:

Observation 1: For the necessary configuration for receiving the L1/L2 signaling of the side control information, OAM based configuration is expected to be fixed for long time durations, and has to be changed manually, while hard-coded configuration is fixed all the time and hence does not provide configuration flexibility.

The proposals are:

Proposal 1: The necessary configuration for receiving the PHY channels and L1/L2 signaling of the side control information is configured via RRC.

# References

- RP-222673, “New WID on NR network-controlled repeaters”, ZTE, Sanechips, RAN#97e, September 12-16, 2022.

- TR 38.867, “Study on NR network-controlled repeaters”.